3GPP TSG-RAN2 Meeting #116bis electronic	R2-2201236

Online, Jan17th - Jan25th, 2022

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the eNPN UE Capability

Agenda item:		8.16.3

Document for: 	Discussion and Decision

- Introduction

In the previous meetings, many agreements have been achieved for the CH, on-boarding and IMS features. In this paper, we focus on the UE capabilities for the eNPN.

- Discussion

2.1 UE capability for the CH feature and On-boarding feature

About the CH feature, it will affect the UE’s initial access, once UE selected a network, there would be no different processing from the home network. Besides, RAN2 has confirmed that there is no impact on connected mode mobility when accessing an SNPN through CHs. Similarly, for the on-boarding, it will only affect UE’s initial access and it was also agreed that no UE impact on connected mode mobility for onboarding. Thus there is no need to introduce any UE capabilities for the CH and On-boarding feature.

Proposal 1: No UE capabilities for the CH and On-boarding feature.

2.2 UE capability for the IMS feature

About the IMS feature, the below capability was defined in the current spec.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| VoiceOverNR Indicates whether the UE supports IMS voice over NR. It is mandated to the UE if the UE is capable of IMS voice over NR. Otherwise, the UE does not include this field. If this field is included and the UE is capable of E-UTRA with EPC, the UE shall support IMS voice over E-UTRA via EPC. | UE | No | No | Yes |

Besides, IMS emergency call was also defined as conditionally mandatory feature without UE radio access capability parameters:

| IMS emergency call | It is mandatory to support IMS emergency call for UEs which are IMS voice capable in NR. |
| --- | --- |

To indicate supporting of IMS feature for the eNPN, these 2 capabilities can be reused.

Proposal 2: No new UE capabilities for the eNPN IMS feature, the legacy capabilities (e.g. VoiceOverNR/IMS emergency call) can be reused.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: No UE capabilities for the CH and On-boarding feature.

Proposal 2: No new UE capabilities for the eNPN IMS feature, the legacy capabilities (e.g. VoiceOverNR/IMS emergency call) can be reused.