3GPP TSG-RAN WG2 #116bis-e                                                                                   R2-2201247

eMeeting, 17th – 25th January, 2022

Agenda Item:	    8.22.2

Source: 	    MediaTek

Title:		    Discussion on pre-configured MG

Document for:	    Discussion and decision

# 1 Introduction

This paper focus on the pre-configured MG design (objective 1 in Rel-17 MG enhancement WI).

- Pre-configured MG pattern(s) (fast MG configuration) [RAN4, RAN2]

  - Procedures and signaling for pre-configured MG pattern(s) [RAN2]

    - Specification of protocol impacts of the mechanisms of activation/deactivation of MG following a DCI or timer based BWP switch, e.g., per BWP MG configuration based on RAN4 input

In RAN2#116, the following agreements are made and an LS [1] is sent to RAN4.

At least case 5 is supported for pre-configured gap. FFS for case 4.

- Case 4: NW signals the pre-configured gap (A+B in Q1) via RRC, then UE follows BWP status (B) to activates/deactivates gap upon BWP switching

- Case 5: NW signals the pre-configured gap (A in Q1) via RRC, then UE determines whether the pre-configured gap should be activated or not upon BWP switching.  For example, if it is overlapped with SSB, then pre-configured gap is deactivated, otherwise it is activated.

RAN2 hasn't seen any usefulness of MAC-CE based activation/deactivation and prefers to not support it.

And there is an incoming LS from RAN4 [2] to provide further details on pre-configured MG. In this contribution, we discuss the remaining issue of pre-configured MG based on RAN4 input and propose some way forward.

# 2 Discussion

In the incoming RAN4 LS [2], the RAN4 inform RAN2 their agreement on use case of pre-configured MG.

Agreements: - Pre-configured MG for SSB measurements should be supported. - It is feasible to configure Pre-MG for Rel-16 PRS measurements. - Pre-MG used for PRS measurement in Rel-16 shall be always activated, if PRS measurement in Rel-16 is configured. - It is up to NW to configure either Pre-MG which shall be always activated or legacy MG for PRS measurement. - The pre-configured MG for the positioning measurements beyond Rel-16 will NOT be discussed under this WI (NR_MG_enh-Core). - It is feasible to configure Pre-MG for CSI-RS L3 inter-frequency measurement. - Pre-MG used for CSI-RS L3 inter-frequency measurement shall be always activated, if inter-frequency CSI-RS L3 measurements are configured. - It is up to NW to configure either Pre-MG which shall be always activated or legacy MG for CSI-RS L3 inter-frequency measurement. - In RAN4#101e meeting, RAN4 continue focus on the single carrier case and evaluate the additional working efforts to support the CA case with pre-configured MG activation/deactivation based on BWP switching on a single CC. It will be decided in further meeting whether CA case with pre-configured MG activation/deactivation based on BWP switching on a single CC will be supported in this release. - In current Pre-MG discussion, MR-DC case in Rel-17 is deprioritized. If any critical technical issues identified in the next meeting, it can be revisited.

From our reading, there is no RAN2 impact on this part. RAN4 may define some rules that the pre-configured MG is always activated if pre-configured MG is used for CSI-RS or PRS measurement but there seems no RAN2 specification action.

On Reconfiguration and activation/deactivation, RAN4 has the following agreement.

- Besides the per BWP indication, no additional explicit signalling is needed to indicate the initial pre-MG activation/deactivation status at or during pre-MG being configured. - FFS: Initial status (activation/deactivation) of Pre-MG is determined based on the same rules used for autonomous activation/deactivation of Pre-MG.

- NW can control activation/deactivation of pre-configured MG for the specific BWP via RRC message ONLY. - Additional activation/deactivation conditions are not considered in application to network-controlled pre-MG activation/deactivation. (i.e. MAC CE based activation/deactivation is not supported) - Specific conditions can be further handled as a part of discussion on rules of UE autonomous activation/deactivation. - The criteria (rules) for pre-MG autonomous (de)activation shall be defined in RAN4.

So, RAN4 agree that NO MAC CE control for pre-configured MG and no additional controlling parameter is needed. We understand that this aligned with RAN2 conclusion last time that RAN2 does not prefer MAC CE control. So, we would like to confirm this.

Proposal 1: RAN2 confirms that MAC-CE based activation/deactivation for pre-configured MG is NOT supported.

RAN2 discussed whether to support per BWP status indicator for pre-configured MG (case 4 the agreement) but no consensus is reached. Based on the latest LS from RAN4, it can be found that the RAN4 still assume that RRC message could control the activation/deactivation of pre-configured MG. In addition, RAN4 already agreed that there will be separate capabilities for two different controlling mechanisms as below.

- Define separate UE capabilities for different pre-MG activation/deactivation mechanisms (i.e. NW-Controlled activation/deactivation mechanism and UE autonomous activation/deactivation mechanism).

While we do agree that it would be simpler to support only the implicit rule, we do not find strong reason to revert the RAN4 agreement. So, we still suggest adding the per BWP indicator under the assumption that all the rules on activated or deactivate the pre-configured MG will be specified in RAN4.

Proposal 2: Introduce pre-configured MG status indicator (activated or not) in each downlink BWP.

TP for pre-configured MG is provided in Annex. One can easily found that adding per BWP indication is quite simple and does really cause too much change in RAN2 specification. We believe that TP could be endorse as baseline running CR discussion for pre-configured MG.

Proposal 3: RAN2 to endorse the ASN.1 from Annex as baseline for running CR discussion.

# 3 Conclusions

Base on the discussion in section 2, we propose the following:

Proposal 1: RAN2 confirms that MAC-CE based activation/deactivation for pre-configured MG is NOT supported.

Proposal 2: Introduce pre-configured MG status indicator (activated or not) in each downlink BWP.

Proposal 3: RAN2 to endorse the ASN.1 from Annex as baseline for running CR discussion.

# 4 References

[1] R2-2111601, “Reply LS on Pre-configured MG”, RAN2

[2] R4-2120302, “LS on R17 NR MG enhancements – Pre-configured MG”, RAN4

# Annex  – Text proposal

### 6.3.2	Radio resource control information elements

<Skip>

#### –	BWP-DownlinkDedicated

The IE BWP-DownlinkDedicated is used to configure the dedicated (UE specific) parameters of a downlink BWP.

BWP-DownlinkDedicated information element

-- ASN1START

-- TAG-BWP-DOWNLINKDEDICATED-START

BWP-DownlinkDedicated ::=           SEQUENCE {

pdcch-Config                        SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

pdsch-Config                        SetupRelease { PDSCH-Config }                                     OPTIONAL,   -- Need M

sps-Config                          SetupRelease { SPS-Config }                                       OPTIONAL,   -- Need M

radioLinkMonitoringConfig           SetupRelease { RadioLinkMonitoringConfig }                        OPTIONAL,   -- Need M

...,

[[

sps-ConfigToAddModList-r16          SPS-ConfigToAddModList-r16                                        OPTIONAL,   -- Need N

sps-ConfigToReleaseList-r16         SPS-ConfigToReleaseList-r16                                       OPTIONAL,   -- Need N

sps-ConfigDeactivationStateList-r16 SPS-ConfigDeactivationStateList-r16                               OPTIONAL,   -- Need R

beamFailureRecoverySCellConfig-r16  SetupRelease {BeamFailureRecoverySCellConfig-r16}                 OPTIONAL,   -- Cond SCellOnly

sl-PDCCH-Config-r16                 SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

sl-V2X-PDCCH-Config-r16             SetupRelease { PDCCH-Config }                                     OPTIONAL    -- Need M

]],

[[

activatedMeasGapList                SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17            OPTIONAL    -- Need R

]]

}

SPS-ConfigToAddModList-r16 ::=          SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-Config

SPS-ConfigToReleaseList-r16 ::=         SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16

SPS-ConfigDeactivationState-r16 ::=     SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16

SPS-ConfigDeactivationStateList-r16 ::= SEQUENCE (SIZE (1..maxNrofSPS-DeactivationState)) OF SPS-ConfigDeactivationState-r16

-- TAG-BWP-DOWNLINKDEDICATED-STOP

-- ASN1STOP

BWP-DownlinkDedicated field descriptions

activatedMeasGapList Indicates the pre-configured measurement gaps to be activated while this BWP is activated according to 38.133 FFS which section.

beamFailureRecoverySCellConfig Configuration of candidate RS for beam failure recovery in SCells.

pdcch-Config UE specific PDCCH configuration for one BWP.

pdsch-Config UE specific PDSCH configuration for one BWP.

sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time. Network can only configure SPS in one BWP using either this field or sps-ConfigToAddModList.

sps-ConfigDeactivationStateList Indicates a list of the deactivation states in which each state can be mapped to a single or multiple SPS configurations to be deactivated, see clause 10.2 in TS 38.213 [13]. If a state is mapped to multiple SPS configurations, each of these SPS configurations is configured with the same harq-CodebookID.

sps-ConfigToAddModList Indicates a list of one or more DL SPS configurations to be added or modified in one BWP. Except for reconfiguration with sync, the NW does not reconfigure a SPS configuration when it is active (see TS 38.321 [3]).

sps-ConfigToReleaseList Indicates a list of one or more DL SPS configurations to be released. The NW may release a SPS configuration at any time.

radioLinkMonitoringConfig UE specific configuration of radio link monitoring for detecting cell- and beam radio link failure occasions. The maximum number of failure detection resources should be limited up to 8 for both cell and beam radio link failure detection. For SCells, only periodic 1-port CSI-RS can be configured in IE RadioLinkMonitoringConfig.

sl-PDCCH-Config Indicates the UE specific PDCCH configurations for receiving the SL grants (via SL-RNTI or SL-CS-RNTI) for NR sidelink communication.

sl-V2X-PDCCH-Config Indicates the UE specific PDCCH configurations for receiving SL grants (i.e. sidelink SPS) for V2X sidelink communication.

| Conditional Presence | Explanation |
| --- | --- |
| ScellOnly | The field is optionally present, Need M, in the BWP-DownlinkDedicated of an Scell. It is absent otherwise. |

<Skip>

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                              OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                              OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                            OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                            OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                                  OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                                  OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                                       OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                                      OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                                       OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                                OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                                                   OPTIONAL    -- Need R

]]

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

interFrequencyConfig-NoGap-r16 If the field is set to true, UE is configured to perform SSB based inter-frequency measurement without measurement gaps when the inter-frequency SSB is completely contained in the active DL BWP of the UE, as specified in TS 38.133 [14], clause 9.3. Otherwise, the SSB based inter-frequency measurement is performed within measurement gaps.

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

measGapSharingConfig Specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/release of measurement gaps.

MeasGapConfig information element

-- ASN1START

-- TAG-MEASGAPCONFIG-START

MeasGapConfig ::=                   SEQUENCE {

gapFR2                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

...,

[[

gapFR1                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

gapUE                               SetupRelease { GapConfig }                                              OPTIONAL    -- Need M

]]

}

GapConfig ::=                       SEQUENCE {

gapOffset                           INTEGER (0..159),

mgl                                 ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                                ENUMERATED {ms20, ms40, ms80, ms160},

mgta                                ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator                ENUMERATED {pCell, pSCell, mcg-FR2}                                 OPTIONAL   -- Cond NEDCorNRDC

]],

[[

refFR2ServCellAsyncCA-r16           ServCellIndex                                                       OPTIONAL,   -- Cond AsyncCA

mgl-r16                             ENUMERATED {ms10, ms20}                                             OPTIONAL    -- Cond PRS

]],

[[

isPreConfigGap-r17					ENUMERATED {true}                                                   OPTIONAL   -- Need R

]]

}

-- TAG-MEASGAPCONFIG-STOP

-- ASN1STOP

MeasGapConfig field descriptions

gapFR1 Indicates measurement gap configuration that applies to FR1 only. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 measurement gap). In NE-DC, gapFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap). In NR-DC, gapFR1 can only be set up in the measConfig associated with MCG. gapFR1 can not be configured together with gapUE. The applicability of the FR1 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapFR2 Indicates measurement gap configuration applies to FR2 only. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap). In NR-DC, gapFR2 can only be set up in the measConfig associated with MCG. gapFR2 cannot be configured together with gapUE. The applicability of the FR2 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapUE Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). In (NG)EN-DC, gapUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE measurement gap). In NE-DC, gapUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap). In NR-DC, gapUE can only be set up in the measConfig associated with MCG. If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. The applicability of the per UE measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapOffset Value gapOffset is the gap offset of the gap pattern with MGRP indicated in the field mgrp. The value range is from 0 to mgrp-1.

isPreConfigGap Indicates whether this measurement gap is a pre-configured measurement gap.

mgl Value mgl is the measurement gap length in ms of the measurement gap. The measurement gap length is according to in Table 9.1.2-1 in TS 38.133 [14]. Value ms1dot5 corresponds to 1.5 ms, ms3 corresponds to 3 ms and so on. If mgl-r16 is present, UE shall ignore the mgl (without suffix).

mgrp Value mgrp is measurement gap repetition period in (ms) of the measurement gap. The measurement gap repetition period is according to Table 9.1.2-1 in TS 38.133 [14].

mgta Value mgta is the measurement gap timing advance in ms. The applicability of the measurement gap timing advance is according to clause 9.1.2 of TS 38.133 [14]. Value ms0 corresponds to 0 ms, ms0dot25 corresponds to 0.25 ms and ms0dot5 corresponds to 0.5 ms. For FR2, the network only configures 0 ms and 0.25 ms.

refFR2ServCellAsyncCA Indicates the FR2 serving cell identifier whose SFN and subframe is used for FR2 gap calculation for this gap pattern with asynchronous CA involving FR2 carrier(s).

refServCellIndicator Indicates the serving cell whose SFN and subframe are used for gap calculation for this gap pattern. Value pCell corresponds to the PCell, pSCell corresponds to the PSCell, and mcg-FR2 corresponds to a serving cell on FR2 frequency in MCG.

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present when configuring FR2 gap pattern to UE in: - (NG)EN-DC or NR SA with asynchronous CA involving FR2 carrier(s); - NE-DC or NR-DC with asynchronous CA involving FR2 carrier(s), if the field refServCellIndicator is set to mcg-FR2. In case the gap pattern to UE in NE-DC and NR-DC is already configured and the serving cell used for the gap calculation corresponds to a serving cell on FR2 frequency in MCG, then the field is optionally present, need M. Otherwise, it is absent, Need R. |
| NEDCorNRDC | This field is mandatory present when configuring gap pattern to UE in NE-DC or NR-DC. In case the gap pattern to UE in NE-DC and NR-DC is already configured, then the field is absent, need M. Otherwise, it is absent. |
| PRS | This field is optionally present, Need R, when configuring gap pattern to UE for measurements of DL-PRS configured via LPP (TS 37.355 [49]). Otherwise, it is absent. |