3GPP TSG-RAN WG2 Meeting #116bis electronic   						    	R2-2201373

E-Meeting, 17th Jan – 25th Jan, 2022

Source: 	Xiaomi Communications

Title:  	MAC impact of RAN1 Rel-17 HARQ deferral

Agenda Item:	8.5.1

Document for:	Discussion and Decision

# Introduction

According to the RAN1 discussion on the HARQ feedback enhancement in Rel-17 IIOT work item, RAN1 agreed to introduce the SPS HARQ deferral in RAN1#104e as follows [1]:

Agreements: - Support deferring SPS HARQ-ACK dropped due to TDD specific collisions until a next available PUCCH in Rel-17 based on semi-static configuration of slot format Agreements: Rel-16 UCI multiplexing / PUCCH overriding rules are reused for deferred SPS HARQ-ACK in the target slot, if applicable. Agreements: For SPS HARQ-ACK, the deferral from the initial slot/sub-slot determined by k1 in the activation DCI to the target slot/sub-slot determined by k1+ k1def, the UE will check the validity (TBD) of a target slot/sub-slot evaluating from one slot/sub-slot to the next sub/sub-slot (i.e. in principle k1def granularity is 1 slot/sub-slot) - FFS: if there is a limit on the minimum deferral considered the required UE processing (k1def ≥0 >1) - FFS: if there is a limit on the maximum deferral given in terms of e.g. k1def <=X or k1+k1def <=X Agreements: For SPS HARQ-ACK deferral, for the determination of valid symbols in the initial slot/sub-slot a collision with semi-static DL symbols, SSB and CORESET#0 is regarded as ‘invalid’ or ‘no symbols for UL transmission’.

In this contribution, we discuss the MAC specification impact caused by the Rel-17 SPS HARQ deferral.

# Discussion

## DRX impact

1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission: 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; NOTE 3: When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating a non-numerical k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback. 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. 3> if the PDSCH-to-HARQ_feedback timing indicate a non-numerical k1 value as specified in TS 38.213 [6]: 4> start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

According to the current MAC specification as quoted above, the drx-HARQ-RTT-TimerDL is started after a

(real) transmission of the DL HARQ feedback. If the HARQ feedback is postponed by a “non-numerical k1”, only the (real) transmission of the DL HARQ feedback as indicated in a later PDCCH requesting the HARQ-ACK feedback will start the drx-HARQ-RTT-TimerDL.

Observation: In Rel-16, only the (real) transmission of the HARQ-ACK feedback due to the non-numerical k1 starts the drx-HARQ-RTT-TimerDL.

Proposal 1: Only the real transmission of the HARQ-ACK feedback due to the Rel-17 SPS HARQ deferral starts the drx-HARQ-RTT-TimerDL, same as the Rel-16 non-numerical k1.

The text proposal is provided in the Annex.

Proposal 2: To capture the text proposal provided in the Annex to the MAC running CR.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation: In Rel-16, only the (real) transmission of the HARQ-ACK feedback due to the non-numerical k1 starts the drx-HARQ-RTT-TimerDL.

Proposal 1: Only the real transmission of the HARQ-ACK feedback due to the Rel-17 SPS HARQ deferral starts the drx-HARQ-RTT-TimerDL, same as the Rel-16 non-numerical k1.

Proposal 2: To capture the text proposal provided in the Annex to the MAC running CR.

# Reference

[1] RAN1#104e meeting minutes.

[2] 3GPP TS 38.321, “NR; Medium Access Control (MAC) protocol specification”.

# Annex

--------------------------------Start of change------------------------------------------------

## 5.7	Discontinuous Reception (DRX)

…

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

NOTE x:	When SPS HARQ feedback is postponed by the transmission collision, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is deferred to the next available resource as specified in TS 38.213 [6].

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

--------------------------------End of change------------------------------------------------