3GPP TSG-WG2 Meeting #116bis-e	R2-2201451

# Online, 17 – 25 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | XXXX | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR - Support of Non-Terrestrial Network in NB-IoT and eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2022-01-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of Non-Terrestrial Network in NB-IoT and eMTC connected to EPC |  |  |  |  |  |  |  |  |
| Summary of change: |  | This running CR captures the agreements for support of non-terrestrial networks (NTN) in NB-IoT/eMTC from RAN2 115-e meeting. This running CR also captures RAN1 parameters in R1-2112975 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN is not supported in NB-IoT/eMTC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.3, 5.2.2.3, 5.2.2.4, 5.2.2.7, 5.2.2.x (new), 5.3.2.1, 5.3.11, 6.2.2, 6.3.2, 6.3.5, 6.3.6, 6.4, 6.6, 6.7.1, 6.7.2, 6.7.3.2, 6.7.3.6, 6.7.5, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2108922: First endorsed version after RAN2#115-e R2-2110478: Ported on September 2021 specification, before RAN2#116-e R2-2111436: Endorsed version after RAN2#116-e R2-2201451: Ported on December 2021 specification and capturing RAN1 parameters in R1-2112975, before RAN2#116b-e |  |  |  |  |  |  |  |  |

Beginning of change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	3GPP TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer ".

[4]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); UE Procedures in Idle Mode".

[5]	3GPP TS 36.306 "Evolved Universal Terrestrial Radio Access (E-UTRA); UE Radio Access Capabilities".

[6]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[7]	3GPP TS 36.322:"Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) Specification".

[9]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

[10]	3GPP TS 22.011: "Service accessibility".

[11]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[12]	3GPP2 C.S0002-F v1.0: "Physical Layer Standard for cdma2000 Spread Spectrum Systems".

[13]	ITU-T Recommendation X.680 (07/2002) "Information Technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation" (Same as the ISO/IEC International Standard 8824-1).

[14]	ITU-T Recommendation X.681 (07/2002) "Information Technology - Abstract Syntax Notation One (ASN.1): Information object specification" (Same as the ISO/IEC International Standard 8824-2).

[15]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[16]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[17]	3GPP TS 25.101: "Universal Terrestrial Radio Access (UTRA); User Equipment (UE) radio transmission and reception (FDD)".

[18]	3GPP TS 25.102: "Universal Terrestrial Radio Access (UTRA); User Equipment (UE) radio transmission and reception (TDD)".

[19]	3GPP TS 25.331:"Universal Terrestrial Radio Access (UTRA); Radio Resource Control (RRC); Protocol specification".

[20]	3GPP TS 45.005: "Radio transmission and reception".

[21]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[22]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[23]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[24]	3GPP2 C.S0057-E v1.0: "Band Class Specification for cdma2000 Spread Spectrum Systems".

[25]	3GPP2 C.S0005-F v1.0: "Upper Layer (Layer 3) Signaling Standard for cdma2000 Spread Spectrum Systems".

[26]	3GPP2 C.S0024-C v2.0: "cdma2000 High Rate Packet Data Air Interface Specification".

[27]	3GPP TS 23.003: "Numbering, addressing and identification".

[28]	3GPP TS 45.008: "Radio subsystem link control".

[29]	3GPP TS 25.133: "Requirements for Support of Radio Resource Management (FDD)".

[30]	3GPP TS 25.123: "Requirements for Support of Radio Resource Management (TDD)".

[31]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access (E-UTRA); Architecture description".

[32]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[33]	3GPP2 A.S0008-C v4.0: "Interoperability Specification (IOS) for High Rate Packet Data (HRPD) Radio Access Network Interfaces with Session Control in the Access Network"

[34]	3GPP2 C.S0004-F v1.0: "Signaling Link Access Control (LAC) Standard for cdma2000 Spread Spectrum Systems"

[35]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[36]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol".

[37]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[38]	3GPP TS 23.038: "Alphabets and Language".

[39]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access (E-UTRAN); S1 Application Protocol (S1 AP)".

[40]	3GPP TS 25.304: "Universal Terrestrial Radio Access (UTRAN); User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode".

[41]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[42]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[43]	3GPP TS 45.005: "GSM/EDGE Radio transmission and reception".

[44]	3GPP2 C.S0087-A v2.0: "E-UTRAN - cdma2000 HRPD Connectivity and Interworking Air Interface Specification"

[45]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control (RRC) protocol".

[46]	3GPP TS 25.223: "Spreading and modulation (TDD)".

[47]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[48]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer - Measurements".

[49]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core network protocols; Stage 3".

[50]	3GPP TS 45.010:	"Radio subsystem synchronization".

[51]	3GPP TS 23.272: "Circuit Switched Fallback in Evolved Packet System; Stage 2".

[52]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[53]	3GPP2 C.S0097-0 v3.0: "E-UTRAN - cdma2000 1x Connectivity and Interworking Air Interface Specification".

[54]	3GPP TS 36.355: "LTE Positioning Protocol (LPP)".

[55]	3GPP TS 36.216: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer for relaying operation".

[56]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[57]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[58]	3GPP TS 32.422: "Telecommunication management; Subsriber and equipment trace; Trace control and confiuration management".

[59]	3GPP TS 22.368: "Service Requirements for Machine Type Communications; Stage 1".

[60]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[61]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[62]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service (MBMS); Stage 1".

[63]	3GPP TR 36.816: "Evolved Universal Terrestrial Radio Access (E-UTRA); Study on signalling and procedure for interference avoidance for in-device coexistence".

[64]	IS-GPS-200F: "Navstar GPS Space Segment/Navigation User Segment Interfaces".

[65]	3GPP TS 25.307: "Requirement on User Equipments (UEs) supporting a release-independent frequency band".

[66]	3GPP TS 24.312: "Access Network Discovery and Selection Function (ANDSF) Management Object (MO)".

[67]	IEEE 802.11-2012, Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.

[68]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[69]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects; Stage 3".

[70]	3GPP TS 24.333: "Proximity-services (ProSe) Management Objects (MO)".

[71]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2- Measurements".

[72]	3GPP TS 24.105: "Application specific Congestion control for Data Communication (ACDC) Management Object (MO)".

[73]	3GPP TS 23.179: "Functional architecture and information flows to support mission critical communication services; Stage 2".

[74]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks".

[75]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses; Stage-2".

[76]	Wi-Fi Alliance® Technical Committee, Hotspot 2.0 Technical Task Group Hotspot 2.0 (Release 2) Technical Specification Version 3.11.

[77]	3GPP TS 22.101: "Service aspects; Service principles".

[78]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[79]	3GPP TS 36.307: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) supporting a release-independent frequency band".

[80]	Military Standard WGS84 Metric MIL-STD-2401 (11 January 1994): "Military Standard Department of Defence World Geodetic System (WGS)".

[81]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[82]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[83]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) Specification".

[84]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[85]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone ".

[86]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[87]	3GPP TS 38.306: "NR; UE Radio Access Capabilities".

[88]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[89]	3GPP TS 38.215: "NR; Physical layer measurements".

[90]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[91]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[92]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[93]	Bluetooth Special Interest Group: "Bluetooth Core Specification v5.0", December 2016.

[94]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access".

[95]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[96]	3GPP TS 22.261: "Service requirements for the 5G System".

[97]	3GPP TS 37.324: "Service Data Adaptation Protocol (SDAP) specification".

[98]	ATIS 0700041: "WEA 3.0: Device-Based Geo-Fencing".

[99]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction ".

[100]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone ".

[101]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[102]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[103]	3GPP TS 38.314: "NR; layer 2 measurements".

[104]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services ".

[105]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[106]	3GPP TS 38.300: "NR; Overall description; Stage 2".

[107]	3GPP TS 38.174: "NR; Integrated access and backhaul radio transmission and reception".

[108]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[x]	NIMA TR 8350.2, Third Edition, Amendment 1, 3 January 2000: “DEPARTMENT OF DEFENSE WORLD GEODETIC SYSTEM 1984”, https://gis-lab.info/docs/nima-tr8350.2-wgs84fin.pdf.

# 3	Definitions, symbols and abbreviations

Next change

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Anchor carrier: In NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Bandwidth Reduced: Refers to operation in downlink and uplink with a limited channel bandwidth of 6 PRBs.

CEIL: Mathematical function used to 'round up' i.e. to the nearest integer having a higher or equal value.

Cellular IoT EPS Optimisation: Provides improved support of small data transfer, as defined in TS 24.301 [35].

Commercial Mobile Alert System: Public Warning System that delivers Warning Notifications provided by Warning Notification Providers to CMAS capable UEs.

Common access barring parameters: The common access barring parameters refer to the access class barring parameters that are broadcast in SystemInformationBlockType2 outside the list of PLMN specific parameters (i.e. in ac-BarringPerPLMN-List).

Control plane CIoT 5GS optimisation: Enables support of efficient transport of user data (IP, Ethernet or unstructured) or SMS messages over control plane via the AMF without triggering data radio bearer establishment, as defined in TS 24.501 [95].

Control plane CIoT EPS optimisation: Enables support of efficient transport of user data (IP, non-IP or SMS) over control plane via the MME without triggering data radio bearer establishment, as defined in TS 24.301 [35].

Control plane EDT: Early Data Transmission used with the Control plane CIoT EPS optimisation or Control plane CIoT 5GS optimisation.

CSG member cell: A cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN and for which the CSG whitelist of the UE includes an entry comprising cell's CSG ID and the respective PLMN identity.

DAPS bearer: A bearer whose radio protocols are located in both the source eNB and the target eNB during a DAPS handover to use both source eNB and target eNB resources.

Dual Connectivity: A UE in RRC_CONNECTED is configured with Dual Connectivity when configured with a Master and a Secondary Cell Group.

Early Data Transmission: Allows one uplink data transmission optionally followed by one downlink data transmission during the random access procedure as specified in TS 36.300 [9]. The S1 connection is established or resumed upon reception of the uplink data and may be released or suspended along with the transmission of the downlink data. Early data transmission refers to both CP-EDT and UP-EDT.

Early Security Reactivation: Re-activation of AS security prior to the transmission of RRCConnectionResumeRequest message when a UE is provided with an NCC value during suspension.

E-UTRA-NR Dual Connectivity: A form of dual connectivity in which a UE in RRC_CONNECTED is configured with MCG cells using E-UTRA and SCG cells using NR as defined in TS 37.340 [81].

EU-Alert: Public Warning System that delivers Warning Notifications provided by Warning Notification Providers using the same AS mechanisms as defined for CMAS.

Field: The individual contents of an information element are referred as fields.

FLOOR: Mathematical function used to 'round down' i.e. to the nearest integer having a lower or equal value.

Information element: A structural element containing a single or multiple fields is referred as information element.

Korean Public Alert System (KPAS): Public Warning System that delivers Warning Notifications provided by Warning Notification Providers using the same AS mechanisms as defined for CMAS.

Master Cell Group: For a UE not configured with DC, the MCG comprises all serving cells. For a UE configured with DC, the MCG concerns a subset of the serving cells comprising of the PCell and zero or more secondary cells.

Mixed Operation Mode: In NB-IoT FDD, multi-carrier operation where the anchor carrier is in standalone mode while the non-anchor carrier is in inband or guardand mode, and vice versa. See TS 36.300 [9].

MBMS service: MBMS bearer service as defined in TS 23.246 [56] (i.e. provided via an MRB or an SC-MRB).

NB-IoT: NB-IoT allows access to network services via E-UTRA with a channel bandwidth limited to 200 kHz.

NB-IoT UE: A UE that uses NB-IoT.

NCSG: Network controlled small gap as defined in TS 36.133 [16].

NR-E-UTRA Dual Connectivity (NE-DC): A form of dual connectivity in which a UE in RRC_CONNECTED is configured with MCG cells using NR and SCG cells using E-UTRA as defined in TS 37.340 [81].

Non-anchor carrier: In NB-IoT, a carrier where the UE does not assume that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

NR Carrier Frequency: Frequency referring to the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [104], between two or more nearby UEs, using NR technology but not traversing any network node.

Primary Cell: The cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure, or the cell indicated as the primary cell in the handover procedure.

Primary Secondary Cell: The SCG cell in which the UE is instructed to perform random access or initial PUSCH transmission if random access procedure is skipped when performing the SCG change procedure.

Primary Timing Advance Group: Timing Advance Group containing the PCell or the PSCell.

PUCCH SCell: An SCell configured with PUCCH.

RLC bearer configuration: The lower layer part of the radio bearer configuration comprising the RLC and logical channel configurations.

Secondary Cell: A cell, operating on a secondary frequency, which may be configured once an RRC connection is established and which may be used to provide additional radio resources. Except for the case of (NG)EN-DC, the PSCell is considered to be an SCell.

Secondary Cell Group: For a UE configured with DC, the subset of serving cells not part of the MCG, i.e. comprising of the PSCell and zero or more other secondary cells.

Secondary Timing Advance Group: Timing Advance Group neither containing the PCell nor the PSCell. A secondary timing advance group contains at least one cell with configured uplink.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/ DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of one or more cells comprising of the primary cell and all secondary cells.

Sidelink: UE to UE interface for sidelink communication, V2X sidelink communication and sidelink discovery. The sidelink corresponds to the PC5 interface as defined in TS 23.303 [68].

Sidelink communication: AS functionality enabling ProSe Direct Communication as defined in TS 23.303 [68], between two or more nearby UEs, using E-UTRA technology but not traversing any network node. In this version, the terminology "sidelink communication" without "V2X" prefix only concerns PS unless specifically stated otherwise.

Sidelink discovery: AS functionality enabling ProSe Direct Discovery as defined in TS 23.303 [68], using E-UTRA technology but not traversing any network node.

Sidelink operation: Includes sidelink communication, V2X sidelink communication and sidelink discovery.

Split SRB: in MR-DC, an SRB between the MN and the UE, allowing selection of either the direct path or the path via the SN as well as duplication of RRC PDUs across both paths as defined in TS 37.340 [81].

Timing Advance Group: A group of serving cells that is configured by RRC and that, for the cells with an UL configured, use the same timing reference cell and the same Timing Advance value. A Timing Advance Group only includes cells of the same cell group i.e. it either includes MCG cells or SCG cells.

Transmission using PUR: Allows one uplink data transmission using preconfigured uplink resource from RRC_IDLE mode as specified in TS 36.300 [9]. Transmission using PUR refers to both CP transmission using PUR and UP transmission using PUR.

UE Inactive AS Context: UE Inactive AS Context is stored when the connection is suspended and restored when the connection is resumed. It includes information as defined in clause 5.3.8.7.

UE in CE: Refers to a UE that is capable of using coverage enhancement, and requires coverage enhancement mode to access a cell or is configured in a coverage enhancement mode.

User plane CIoT 5GS optimisation: Enables support for change from 5GMM-IDLE mode to 5GMM-CONNECTED mode without the need for using the Service Request procedure, as defined in TS 24.501 [95].

User plane CIoT EPS optimisation: Enables support for change from EMM-IDLE mode to EMM-CONNECTED mode without the need for using the Service Request procedure, as defined in TS 24.301 [35].

User plane EDT: Early Data Transmission used with the User plane CIoT EPS optimisation or User plane CIoT 5GS optimisation.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [78], between nearby UEs, using E-UTRA technology but not traversing any network node.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 36.300 [9] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] or TS 36.300 [9].

1xRTT	CDMA2000 1x Radio Transmission Technology

AB	Access Barring

ACDC	Application specific Congestion control for Data Communication

ACK	Acknowledgement

AILC	Assistance Information bit for Local Cache

AM	Acknowledged Mode

ANDSF	Access Network Discovery and Selection Function

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

AUL	Autonomous Uplink

BCCH	Broadcast Control Channel

BCD	Binary Coded Decimal

BCH	Broadcast Channel

BL	Bandwidth reduced Low complexity

BLER	Block Error Rate

BR	Bandwidth Reduced

BR-BCCH	Bandwidth Reduced Broadcast Control Channel

CA	Carrier Aggregation

CAS	Cell Acquisition Subframes

CBR	Channel Busy Ratio

CCCH	Common Control Channel

CCO	Cell Change Order

CE	Coverage Enhancement

CFI	Control Format Indicator

CG	Cell Group

CHO	Conditional Handover

CIoT	Cellular IoT

CMAS	Commercial Mobile Alert Service

CP	Control Plane

CPC	Conditional PSCell Change

CP-EDT	Control Plane EDT

C-RNTI	Cell RNTI

CRS	Cell-specific Reference Signal

CSFB	CS fallback

CSG	Closed Subscriber Group

CSI	Channel State Information

DAPS	Dual Active Protocol Stack

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCI	Downlink Control Information

DCN	Dedicated Core Networks

DFN	Direct Frame Number

DL	Downlink

DL-SCH	Downlink Shared Channel

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

EAB	Extended Access Barring

eDRX	Extended DRX

EDT	Early Data Transmission

EHPLMN	Equivalent Home Public Land Mobile Network

eIMTA	Enhanced Interference Management and Traffic Adaptation

ENB	Evolved Node B

EN-DC	E-UTRA NR Dual Connectivity with E-UTRAN connected to EPC

EPC	Evolved Packet Core

EPDCCH	Enhanced Physical Downlink Control Channel

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRA/5GC	E-UTRA connected to 5GC

E-UTRA/EPC	E-UTRA connected to EPC

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For Further Study

GERAN	GSM/EDGE Radio Access Network

GNSS	Global Navigation Satellite System

G-RNTI	Group RNTI

GSM	Global System for Mobile Communications

GWUS	Group Wake Up Signal

HARQ	Hybrid Automatic Repeat Request

HFN	Hyper Frame Number

HPLMN	Home Public Land Mobile Network

HRPD	CDMA2000 High Rate Packet Data

HSDN	High Speed Dedicated Network

H-SFN	Hyper SFN

IAB	Integrated Access and Backhaul

IAB-DU	IAB-node DU

IAB-MT	IAB Mobile Termination

IDC	In-Device Coexistence

IE	Information element

IMEI	International Mobile Equipment Identity

IMSI	International Mobile Subscriber Identity

IoT	Internet of Things

ISM	Industrial, Scientific and Medical

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

LAA	Licensed-Assisted Access

LWA	LTE-WLAN Aggregation

LWAAP	LTE-WLAN Aggregation Adaptation Protocol

LWIP	LTE-WLAN Radio Level Integration with IPsec Tunnel

MAC	Medium Access Control

MBMS	Multimedia Broadcast Multicast Service

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MCG	Master Cell Group

MCOT	Maximum Channel Occupancy Time

MCPTT	Mission Critical Push To Talk

MDT	Minimization of Drive Tests

MIB	Master Information Block

MO	Mobile Originating

MPDCCH	MTC Physical Downlink Control Channel

MRB	MBMS Point to Multipoint Radio Bearer

MR-DC	Multi-Radio Dual Connectivity

MRO	Mobility Robustness Optimisation

MSI	MCH Scheduling Information

MT	Mobile Terminating

MTSI	Multimedia Telephony Service for IMS

MUST	MultiUser Superposition Transmission

N/A	Not Applicable

NACC	Network Assisted Cell Change

NAICS	Network Assisted Interference Cancellation/Suppression

NAS	Non Access Stratum

NB-IoT	NarrowBand Internet of Things

NE-DC	NR E-UTRA Dual Connectivity

(NG)EN-DC	E-UTRA NR Dual Connectivity (i.e. covering both EN-DC and NGEN-DC)

NGEN-DC	E-UTRA NR Dual Connectivity with E-UTRAN connected to 5GC

NPBCH	Narrowband Physical Broadcast channel

NPDCCH	Narrowband Physical Downlink Control channel

NPDSCH	Narrowband Physical Downlink Shared channel

NPRACH	Narrowband Physical Random Access channel

NPSS	Narrowband Primary Synchronization Signal

NPUSCH	Narrowband Physical Uplink Shared channel

NR	NR Radio Access

NRS	Narrowband Reference Signal

NSSAI	Network Slice Selection Assistance Information

NSSS	Narrowband Secondary Synchronization Signal

OS	OFDM Symbol

P2X	Pedestrian-to-Everything

PCCH	Paging Control Channel

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

PMK	Pairwise Master Key

PO	Paging Occasion

posSIB	Positioning SIB

ProSe	Proximity based Services

PS	Public Safety (in context of sidelink), Packet Switched (otherwise)

PSCell	Primary Secondary Cell

PSK	Pre-Shared Key

PTAG	Primary Timing Advance Group

PUCCH	Physical Uplink Control Channel

PUR	Preconfigured Uplink Resource

QCI	QoS Class Identifier

QoE	Quality of Experience

QoS	Quality of Service

RACH	Random Access CHannel

RAI	Release Assistance Indication

RAT	Radio Access Technology

RB	Radio Bearer

RCLWI	RAN Controlled LTE-WLAN Integration

RLC	Radio Link Control

RLOS	Restricted Local Operator Services

RMTC	RSSI Measurement Timing Configuration

RN	Relay Node

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

ROHC	RObust Header Compression

RPLMN	Registered Public Land Mobile Network

RRC	Radio Resource Control

RSCP	Received Signal Code Power

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSS	Resynchronisation signal

RSSI	Received Signal Strength Indicator

SAE	System Architecture Evolution

SAP	Service Access Point

SBAS	Satellite Based Augmentation System

SC	Sidelink Control

SCell	Secondary Cell

SCG	Secondary Cell Group

SC-MRB	Single Cell MRB

SC-RNTI	Single Cell RNTI

SD-RSRP	Sidelink Discovery Reference Signal Received Power

SFN	System Frame Number

SI	System Information

SIB	System Information Block

SI-RNTI	System Information RNTI

SL	Sidelink

SLSS	Sidelink Synchronisation Signal

SMC	Security Mode Control

SMTC	SS/PBCH Block Measurement Timing Configuration

SPDCCH	Short PDCCH

SPS	Semi-Persistent Scheduling

SPT	Short Processing Time

SPUCCH	Short PUCCH

SR	Scheduling Request

SRB	Signalling Radio Bearer

S-RSRP	Sidelink Reference Signal Received Power

SSAC	Service Specific Access Control

SSTD	SFN and Subframe Timing Difference

STAG	Secondary Timing Advance Group

S-TMSI	SAE Temporary Mobile Station Identifier

STTI	Short TTI

TA	Tracking Area

TAG	Timing Advance Group

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TM	Transparent Mode

TPC-RNTI	Transmit Power Control RNTI

T-RPT	Time Resource Pattern of Transmission

TTI	Transmission Time Interval

TTT	Time To Trigger

UDC	Uplink Data Compression

UE	User Equipment

UICC	Universal Integrated Circuit Card

UL	Uplink

UL-SCH	Uplink Shared Channel

UM	Unacknowledged Mode

UP	User Plane

UP-EDT	User Plane EDT

UTC	Coordinated Universal Time

UTRAN	Universal Terrestrial Radio Access Network

V2X	Vehicle-to-Everything

VoLTE	Voice over Long Term Evolution

WLAN	Wireless Local Area Network

WT	WLAN Termination

WUS	Wake-up Signal

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

Next change

#### 5.2.1.3	System information validity and notification of changes

Change of system information (other than for ETWS, CMAS, EAB, UAC, and satellite ephemeris parameters and other than for AB parameters for NB-IoT) only occurs at specific radio frames, i.e. the concept of a modification period is used. System information may be transmitted a number of times with the same content within a modification period, as defined by its scheduling. The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. If H-SFN is provided in SystemInformationBlockType1-BR, modification period boundaries for BL UEs and UEs in CE are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0. For NB-IoT, H-SFN is always provided and the modification period boundaries are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0.

Editor’s Note: Agreement: System information update notification procedure is not used to inform TAC updates, at least for TAC additions (FFS removals).

To enable system information update notification for RRC_IDLE UEs configured to use a DRX cycle longer than the modification period, an eDRX acquisition period is defined. The boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 256 =0. For NB-IoT, the boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 =0.

NOTE 1:	If the UE in RRC_IDLE is configured to use extended DRX cycle, e.g., in the order of several minutes or longer, in case the eNB is reset the UE SFN may not be synchronized to the new eNB SFN. The UE is expected to recover, e.g., acquire MIB within a reasonable time, to avoid repeated paging failures.

NOTE 1a:	For the UE in RRC_INACTIVE, the idle mode extended DRX cycle, if configured, is used to compare with the modification period.

When the network changes (some of the) system information, it first notifies the UEs about this change, i.e. this may be done throughout a modification period. In the next modification period, the network transmits the updated system information. During a modification period where ETWS or CMAS transmission is started or stopped, the SI messages carrying the SIBs scheduled in schedulingInfoListExt and/or SI messages carrying the posSIBs scheduled in posSchedulingInfoList may change, so the UE might not be able to successfully receive those SIBs and/or posSIBs in the remainder of the current modification period and next modification period according to the scheduling information received prior to the change. These general principles are illustrated in figure 5.2.1.3-1, in which different colours indicate different system information. Upon receiving a change notification, the UE not configured to use a DRX cycle that is longer than the modification period acquires the new system information immediately from the start of the next modification period. Upon receiving a change notification applicable to eDRX, a UE in RRC_IDLE configured to use a DRX cycle that is longer than the modification period acquires the updated system information immediately from the start of the next eDRX acquisition period. The UE applies the previously acquired system information until the UE acquires the new system information. The possible boundaries of modification for SystemInformationBlockType1-BR are defined by SFN values for which SFN mod 512 = 0 except for notification of ETWS/CMAS for which the eNB may change SystemInformationBlockType1-BR content at any time. For NB-IoT, the possible boundaries of modification for SystemInformationBlockType1-NB are defined by SFN values for which (H-SFN * 1024 + SFN) mod 4096 = 0.

Figure 5.2.1.3-1: Change of system Information

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. If the UE is in RRC_CONNECTED or is not configured to use a DRX cycle longer than the modification period in RRC_IDLE, and receives a Paging message including the systemInfoModification, it knows that the system information will change at the next modification period boundary. A UE in RRC_IDLE that is configured to use a DRX cycle longer than the modification period, and receives in an eDRX acquisition period at least one Paging message including the systemInfoModification-eDRX, shall acquire the updated system information at the next eDRX acquisition period boundary. Although the UE may be informed about changes in system information, no further details are provided e.g. regarding which system information will change, except if systemInfoValueTagSI is received by BL UEs or UEs in CE.

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information. In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification, EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

NOTE 2:	Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

SystemInformationBlockType1 (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) includes a value tag systemInfoValueTag, that indicates if a change has occurred in the SI messages. UEs may use systemInfoValueTag, e.g. upon return from out of coverage, to verify if the previously stored SI messages are still valid. MasterInformationBlock and RSS (if transmitted, see TS 36.211 [21]) may indicate using systemInfoUnchanged-BR that a change has not occurred in the SIB1-BR and SI messages of the current cell at least over the SI validity time, and the BL UEs or UEs in CE may use the systemInfoUnchanged-BR, e.g. upon return from out of coverage, to verify if the previously stored SIB1-BR and SI messages are still valid. Additionally, for other than BL UEs or UEs in CE or NB-IoT UEs, the UE considers stored system information to be invalid after 3 hours from the moment it was successfully confirmed as valid, unless specified otherwise. BL UE or UE in CE considers stored system information to be invalid after 24 hours from the moment it was successfully confirmed as valid, unless the UE is configured by parameter si-ValidityTime to consider stored system information to be invalid 3 hours after validity confirmation. NB-IoT UE considers stored system information to be invalid after 24 hours from the moment it was successfully confirmed as valid. If a BL UE, UE in CE or NB-IoT UE in RRC_CONNECTED state considers the stored system information invalid, the UE shall continue using the stored system information while in RRC_CONNECTED state in the serving cell.

For BL UEs or UEs in CE or NB-IoT UEs, the change of specific SI message can additionally be indicated by a SI message specific value tag systemInfoValueTagSI. If systemInfoValueTag included in the SystemInformationBlockType1-BR (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) is different from the one of the stored system information and if systemInfoValueTagSI is included in the SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) for a specific SI message and is different from the stored one, the UE shall consider this specific SI message to be invalid. If only systemInfoValueTag is included and is different from the stored one, the BL UE or UE in CE should consider any stored system information except SystemInformationBlockType10, SystemInformationBlockType11, SystemInformationBlockType12, SystemInformationBlockType14, SystemInformationBlockType25 and SystemInformationBlockTypeXX to be invalid; the NB-IoT UE should consider any stored system information except SystemInformationBlockType14-NB and SystemInformationBlockTypeXX-NB to be invalid.

On MBMS-dedicated cell and on FeMBMS/Unicast-mixed cell, the change of system information and ETWS/CMAS notification is indicated by using Direct Indication FeMBMS defined in 6.6a. The modification periodicity follows MCCH modification periodicity as defined in 5.8.1.3.

E-UTRAN may not update systemInfoValueTag upon change of some system information e.g. ETWS information, CMAS information, RLOS indication (i.e., rlos-Enabled), regularly changing parameters like time information (SystemInformationBlockType8, SystemInformationBlockType16, hyperSFN-MSB in SystemInformationBlockType1-NB), EAB and AB parameters, UAC parameters, positioning system information blocks, or satellite ephemeris parameters. Similarly, E-UTRAN may not include the systemInfoModification within the Paging message upon change of some system information.

The UE that is not configured to use a DRX cycle longer than the modification period verifies that stored system information remains valid by either checking systemInfoValueTag in SystemInformationBlockType1 (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) after the modification period boundary, or attempting to find the systemInfoModification indication at least modificationPeriodCoeff times during the modification period in case no paging is received, in every modification period. If no paging message is received by the UE during a modification period, the UE may assume that no change of system information will occur at the next modification period boundary. If UE in RRC_CONNECTED, during a modification period, receives one paging message, it may deduce from the presence/ absence of systemInfoModification whether a change of system information other than ETWS information, CMAS information, EAB and UAC parameters will occur in the next modification period or not.

When the RRC_IDLE UE is configured with a DRX cycle that is longer than the modification period, and at least one modification period boundary has passed since the UE last verified validity of stored system information, the UE verifies that stored system information remains valid by checking the systemInfoValueTag before establishing or resuming an RRC connection.

ETWS and/or CMAS capable UEs in RRC_CONNECTED, other than BL UEs and UEs in CE, shall attempt to read paging at least once every defaultPagingCycle to check whether ETWS and/or CMAS notification is present or not.

Next change

#### 5.2.2.3	System information required by the UE

The UE shall:

1>	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

2>	if in RRC_IDLE:

3>	if the UE is a NB-IoT UE:

4>	the MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB and SystemInformationBlockType1-NB as well as SystemInformationBlockType2-NB through SystemInformationBlockType5-NB, SystemInformationBlockType22-NB;

3>	else:

4>	the MasterInformationBlock and SystemInformationBlockType1 (or SystemInformationBlockType1-BR depending on whether the UE is a BL UE or the UE in CE) as well as SystemInformationBlockType2 through SystemInformationBlockType8 and SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC), SystemInformationBlockType29 (only for BL UE or the UE in CE depending on support of resource reservation), SystemInformationBlockType21, SystemInformationBlockType26 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), and SystemInformationBlockType28 (if UE is capable of NR sidelink communication and is configured by upper layers to receive or transmit NR sidelink communication);

2>	if in RRC_INACTIVE:

3>	the MasterInformationBlock and SystemInformationBlockType1 as well as SystemInformationBlockType2 through SystemInformationBlockType8 (depending on support of the concerned RATs), SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType25, SystemInformationBlockType29 (only for BL UE or the UE in CE depending on support of resource reservation), SystemInformationBlockType21, SystemInformationBlockType26 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), and SystemInformationBlockType28 (if UE is capable of NR sidelink communication and is configured by upper layers to receive or transmit NR sidelink communication);

2>	if in RRC_CONNECTED; and

2>	the UE is not a BL UE; and

2>	the UE is not in CE; and

2>	the UE is not a NB-IoT UE:

3>	the MasterInformationBlock, SystemInformationBlockType1 and SystemInformationBlockType2 as well as SystemInformationBlockType8 (depending on support of CDMA2000), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC);

2>	if in RRC_CONNECTED and T311 is running; and

2>	the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE;

3>	the MasterInformationBlock (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT), SystemInformationBlockType25 (only for BL UE or the UE in CE depending on support of E-UTRA/5GC), SystemInformationBlockType29 (only for BL UE or the UE in CE depending on support of resource reservation) and for NB-IoT SystemInformationBlockType22-NB;

1>	delete any stored system information after 3 hours or 24 hours from the moment it was confirmed to be valid as defined in 5.2.1.3, unless specified otherwise;

1>	consider any stored system information except SystemInformationBlockType10, SystemInformationBlockType11, systemInformationBlockType12, systemInformationBlockType14 (systemInformationBlockType14-NB in NB-IoT) and systemInformationBlockType25 to be invalid if systemInfoValueTag included in the SystemInformationBlockType1 (MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) is different from the one of the stored system information and in case of NB-IoT UEs, BL UEs and UEs in CE, systemInfoValueTagSI is not broadcasted. Otherwise consider system information validity as defined in 5.2.1.3;

Editor’s Note: Agreement: Update to serving cell ephemeris information does not affect the system information value tag and does not trigger System information modification procedure. How to trigger re-read of this information is FFS. FFS if the UE shall reacquire the new SIB when SI update is triggered.

#### 5.2.2.4	System information acquisition by the UE

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1 or BR-BCCH configuration defined in 9.1.1.8;

1>	if the procedure is triggered by a system information change notification:

2>	if the UE uses an idle DRX cycle longer than the modification period:

3>	start acquiring the required system information, as defined in 5.2.2.3, from the next eDRX acquisition period boundary;

2>	else

3>	start acquiring the required system information, as defined in 5.2.2.3, from the beginning of the modification period following the one in which the change notification was received;

NOTE 1:	The UE continues using the previously received system information until the new system information has been acquired.

1>	if the UE is in RRC_IDLE and enters a cell for which the UE does not have stored a valid version of the system information required in RRC_IDLE, as defined in 5.2.2.3:

2>	acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_IDLE, as defined in 5.2.2.3;

1>	following successful handover completion to a PCell for which the UE does not have stored a valid version of the system information required in RRC_CONNECTED, as defined in 5.2.2.3:

2>	acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_CONNECTED, as defined in 5.2.2.3;

2>	upon acquiring the concerned system information:

3>	discard the corresponding radio resource configuration information included in the radioResourceConfigCommon previously received in a dedicated message, if any;

1>	following a request from CDMA2000 upper layers:

2>	acquire SystemInformationBlockType8, as defined in 5.2.3;

1>	neither initiate the RRC connection establishment/resume procedure nor initiate transmission of the RRCConnectionReestablishmentRequest message until the UE has a valid version of the MasterInformationBlock (MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) and SystemInformationBlockType1 (SystemInformationBlockType1-NB in NB-IoT) messages as well as SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT), and for NB-IoT, SystemInformationBlockType22-NB;

1>	not initiate the RRC connection establishment/resume procedure subject to EAB until the UE has a valid version of SystemInformationBlockType14, if broadcast;

1>	if the UE is ETWS capable:

2>	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3>	discard any previously buffered warningMessageSegment;

3>	clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

2>	when the UE acquires SystemInformationBlockType1 following ETWS indication, upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3>	if schedulingInfoList indicates that SystemInformationBlockType10 is present:

4>	if the UE is in CE:

5>	start acquiring SystemInformationBlockType10;

4>	else

5>	start acquiring SystemInformationBlockType10 immediately;

3>	if schedulingInfoList indicates that SystemInformationBlockType11 is present:

4>	start acquiring SystemInformationBlockType11 immediately;

NOTE 2:	UEs shall start acquiring SystemInformationBlockType10 and SystemInformationBlockType11 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

1>	if the UE is CMAS capable:

2>	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3>	discard any previously buffered warningMessageSegment;

3>	clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment;

2>	when the UE acquires SystemInformationBlockType1 following CMAS indication, upon entering a cell during RRC_IDLE, following successful handover and upon connection re-establishment:

3>	if schedulingInfoList indicates that SystemInformationBlockType12 is present:

4>	acquire SystemInformationBlockType12;

NOTE 3:	UEs shall start acquiring SystemInformationBlockType12 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

1>	if the UE is interested to receive MBMS services:

2>	if the UE is capable of MBMS reception as specified in 5.8:

3>	if schedulingInfoList indicates that SystemInformationBlockType13 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType13;

3>	else if SystemInformationBlockType13 is present in SystemInformationBlockType1-MBMS and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType13 from SystemInformationBlockType1-MBMS;

2>	if the UE is capable of SC-PTM reception as specified in 5.8a:

3>	if schedulingInfoList indicates that SystemInformationBlockType20 (SystemInformationBlockType20-NB in NB-IoT) is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType20 (SystemInformationBlockType20-NB in NB-IoT);

2>	if the UE is capable of MBMS Service Continuity:

3>	if schedulingInfoList indicates that SystemInformationBlockType15 (SystemInformationBlockType15-NB in NB-IoT) is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType15 (SystemInformationBlockType15-NB in NB-IoT);

1>	if the UE is EAB capable:

2>	when the UE does not have stored a valid version of SystemInformationBlockType14 upon entering RRC_IDLE, or when the UE acquires SystemInformationBlockType1 following EAB parameters change notification, or upon entering a cell during RRC_IDLE, or before establishing an RRC connection if using eDRX with DRX cycle longer than the modification period:

3>	if schedulingInfoList indicates that SystemInformationBlockType14 is present:

4>	start acquiring SystemInformationBlockType14 immediately;

3>	else:

4>	discard SystemInformationBlockType14, if previously received;

NOTE 4:	EAB capable UEs start acquiring SystemInformationBlockType14 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

NOTE 5:	EAB capable UEs maintain an up to date SystemInformationBlockType14 in RRC_IDLE.

1>	if the UE is capable of sidelink communication and is configured by upper layers to receive or transmit sidelink communication:

2>	if the cell used for sidelink communication meets the S-criteria as defined in TS 36.304 [4]; and

2>	if schedulingInfoList indicates that SystemInformationBlockType18 is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType18;

1>	if the UE is capable of sidelink discovery and is configured by upper layers to receive or transmit sidelink discovery announcements on the primary frequency:

2>	if schedulingInfoList of the serving cell/ PCell indicates that SystemInformationBlockType19 is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType19;

1>	if the UE is capable of sidelink discovery and, for each of the one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19 and for which the UE is configured by upper layers to receive sidelink discovery announcements on:

2>	if SystemInformationBlockType19 of the serving cell/ PCell does not provide the corresponding reception resources; and

2>	if schedulingInfoList of the cell on the concerned frequency indicates that SystemInformationBlockType19 is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType19;

1>	if the UE is capable of sidelink discovery and, for each of the one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19 and for which the UE is configured by upper layers to transmit sidelink discovery announcements on:

2>	if SystemInformationBlockType19 of the serving cell/ PCell includes discTxResourcesInterFreq which is set to acquireSI-FromCarrier; and

2>	if schedulingInfoList of the cell on the concerned frequency indicates that SystemInformationBlockType19 is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType19;

1>	if the UE is a NB-IoT UE connected to EPC and if ab-Enabled included in MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB is set to TRUE:

2>	not initiate the RRC connection establishment/resume procedure for all access causes except mobile terminating calls until the UE has acquired the SystemInformationBlockType14-NB;

1>	if the UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication on a frequency:

2>	if schedulingInfoList on the serving cell/PCell indicates that SystemInformationBlockType21 is present and the UE does not have stored valid version of this system information block:

3>	acquire SystemInformationBlockType21 from serving cell/PCell;

2>	if schedulingInfoList on the serving cell/PCell indicates that SystemInformationBlockType26 is present and the UE does not have stored valid version of this system information block;

3>	acquire SystemInformationBlockType26 from serving cell/PCell;

1>	if the UE is capable of V2X sidelink communication and is configured by upper layers to receive V2X sidelink communication on a frequency, which is not primary frequency:

2>	if neither SystemInformationBlockType21 nor SystemInformationBlockType26 of the serving cell/ PCell provide reception resource pool for V2X sidelink communication for the concerned frequency; and

2>	if the cell used for V2X sidelink communication on the concerned frequency meets the S-criteria as defined in TS 36.304 [4]:

3>	if schedulingInfoList on the concerned frequency indicates that SystemInformationBlockType21 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType21 from the concerned frequency;

3>	if schedulingInfoList on the concerned frequency indicates that SystemInformationBlockType26 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType26 from the concerned frequency;

1>	if the UE is capable of V2X sidelink communication and is configured by upper layers to transmit V2X sidelink communication on a frequency, which is not primary frequency and is not included in v2x-InterFreqInfoList in SystemInformationBlockType21 nor SystemInformationBlockType26 of the serving cell/PCell:

2>	if the cell used for V2X sidelink communication on the concerned frequency meets the S-criteria as defined in TS 36.304 [4]:

3>	if schedulingInfoList on the concerned frequency indicates that SystemInformationBlockType21 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType21 from the concerned frequency;

3>	if schedulingInfoList on the concerned frequency indicates that SystemInformationBlockType26 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType26 from the concerned frequency;

1>	if the NB-IoT UE supports NPRACH resources using preamble format 2:

2>	if schedulingInfoList indicates that SystemInformationBlockType23-NB is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType23-NB;

1>	following a request from positioning upper layers:

2>	acquire SystemInformationBlockPos, as defined in 5.2.3;

1>	if the UE is capable of NR sidelink communication and is configured by upper layers to receive or transmit NR sidelink communication on a frequency:

2>	if schedulingInfoList on the serving cell/PCell indicates that SystemInformationBlockType28 is present and the UE does not have stored valid version of this system information block:

3>	acquire SystemInformationBlockType28 from serving cell/PCell;

1>	if the UE connected to 5GC is a BL UE or a UE in CE:

2>	when the UE does not have stored a valid version of SystemInformationBlockType25 upon entering RRC_IDLE, or when the UE acquires SystemInformationBlockType1-BR following UAC parameters change notification, or upon entering a cell during RRC_IDLE, or before establishing, resuming or re-establishing an RRC connection if using an eDRX cycle longer than the modification period:

3>	if schedulingInfoList indicates that SystemInformationBlockType25 is present:

4>	start acquiring SystemInformationBlockType25 immediately before establishing, resuming or re-establishing an RRC connection;

3>	else:

4>	discard SystemInformationBlockType25, if previously received;

NOTE 5a:	When connected to 5GC, BL UEs or a UEs in CE start acquiring SystemInformationBlockType25 as described above even when systemInfoValueTag in SystemInformationBlockType1-BR has not changed.

NOTE 5b:	When connected to 5GC, BL UEs or a UEs in CE maintain an up to date SystemInformationBlockType25 in RRC_IDLE.

1>	if the UE is a NB-IoT UE connected to 5GC and if ab-Enabled5GC included in MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB is set to TRUE:

2>	not initiate the RRC connection establishment/ resume/ re-establishment procedure for all access causes until the UE has acquired the SystemInformationBlockType14-NB;

The UE may apply the received SIBs or posSIBs immediately, i.e. the UE does not need to delay using a SIB or posSIB until all SI messages have been received. The UE may delay applying the received SIBs until completing lower layer procedures associated with a received or a UE originated RRC message, e.g. an ongoing random access procedure.

NOTE 6:	While attempting to acquire a particular SIB/posSIB, if the UE detects from schedulingInfoList/ posSchedulingInfoList that it is no longer present, the UE should stop trying to acquire the particular SIB/ posSIB.

Editor’s Note: Agreement: Update to serving cell ephemeris information does not affect the system information value tag and does not trigger System information modification procedure. How to trigger re-read of this information is FFS. FFS if the UE shall reacquire the new SIB when SI update is triggered.

Next change

#### 5.2.2.7	Actions upon reception of the SystemInformationBlockType1 message

Upon receiving the SystemInformationBlockType1 or SystemInformationBlockType1-BR either via broadcast or via dedicated signalling, the UE shall:

1>	if the upper layers indicate the selected core network type as 5GC:

2>	if the cellAccessRelatedInfoList-5GC contains an entry with the plmn-Identity or plmn-Index of the selected PLMN:

3>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfoList-5GC containing the selected PLMN;

1>	else if the cellAccessRelatedInfoList contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfoList containing the selected PLMN;

1>	if in RRC_IDLE or in RRC_CONNECTED while T311 is running; and

1>	if the UE is a category 0 UE according to TS 36.306 [5]; and

1>	if category0Allowed is not included in SystemInformationBlockType1:

2>	consider the cell as barred in accordance with TS 36.304 [4];

1>	if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2>	disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

1>	else:

2>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE and it is not a downlink only band; or

2>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE and they are not downlink only bands:

3>	forward the cellIdentity to upper layers;

3>	forward the trackingAreaCode to upper layers;

3>	forward the trackingAreaList to upper layers, if present;

3>	forward the PLMN identity to upper layers;

3>	if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers:

4>	if the serving cell does not belong to the configured ran-NotificationAreaInfo:

5>	initiate an RNA update as specified in 5.3.17.2;

3>	forward the ims-EmergencySupport to upper layers, if present;

3>	forward the eCallOverIMS-Support to upper layers, if present;

3>	if the UE is capable of 5G NAS:

4>	forward the ims-EmergencySupport5GC to upper layers, if present;

4>	forward the eCallOverIMS-Support5GC to upper layers, if present;

4>	forward cp-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN;

4>	forward up-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN;

3>	if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo or the multiBandInfoList-v10j0 is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo or multiBandInfoList-v10j0:

4>	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo or multiBandInfolist-v10j0;

4>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

5>	apply the additionalPmax;

4>	else:

5>	apply the p-Max;

3>	if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred for IAB-MT in accordance with TS 36.304 [4];

3>	else:

4>	apply the additionalSpectrumEmission in SystemInformationBlockType2 and the p-Max;

2>	else:

3>	consider the cell as barred in accordance with TS 36.304 [4]; and

3>	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

Upon receiving the SystemInformationBlockType1-NB, the UE shall:

1>	if the upper layers indicate the selected core network type as 5GC:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the cellAccessRelatedInfo-5GC;

1>	else:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList and cellIdentity for the cell as received in the cellAccessRelatedInfo;

1>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

1>	if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

3>	forward the trackingAreaList to upper layers, if present;

2>	if attachWithoutPDN-Connectivity is received for the selected PLMN:

3>	forward the attachWithoutPDN-Connectivity to upper layers;

2>	else

3>	indicate to upper layers that attachWithoutPDN-Connectivity is not present;

2>	if the UE is capable of 5G NAS:

3>	forward ng-U-DataTransfer to upper layers, if present for the selected PLMN;

3>	forward up-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN;

2>	if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo:

3>	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo;

3>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

4>	apply the additionalPmax;

3>	else:

4>	apply the p-Max;

2>	else:

3>	apply the additionalSpectrumEmission in SystemInformationBlockType2-NB and the p-Max;

1>	else:

2>	consider the cell as barred in accordance with TS 36.304 [4]; and

2>	perform barring as if intraFreqReselection is set to notAllowed.

No UE requirements related to the contents of SystemInformationBlockType1-MBMS apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

Next change

#### 5.2.2.x	Actions upon reception of SystemInformationBlockTypeXX

Editor’s Note: Editor: FFS if specific UE requirements need to be captured, e.g. handling of the UL synchronisation validity timer

Next change

### 5.3.2	Paging

#### 5.3.2.1	General

Figure 5.3.2.1-1: Paging

The purpose of this procedure is:

-	to transmit CN initiated paging information to a UE in RRC_IDLE or RRC_INACTIVE and/ or;

-	to transmit RAN initiated paging information to a UE in RRC_INACTIVE and/or;

-	to inform UEs in RRC_IDLE, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about a system information change and/ or;

-	to inform UEs in RRC_IDLE other than NB-IoT UEs, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about an ETWS primary notification and/ or ETWS secondary notification and/ or;

-	to inform UEs in RRC_IDLE other than NB-IoT UEs, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about a CMAS notification and/ or;

-	to inform UEs other than NB-IoT UEs in RRC_IDLE, and other than UEs connected to 5GC about an EAB parameters modification and/ or;

-	to inform UEs other than NB-IoT UEs in RRC_IDLE, and UEs in RRC_INACTIVE to perform E-UTRAN inter-frequency redistribution procedure.

Editor’s Note: Agreement: System information update notification procedure is not used to inform TAC updates, at least for TAC additions (FFS removals).

The paging information of CN initiated paging is provided to upper layers, which in response may initiate RRC connection establishment, e.g. to receive an incoming call.

Next change

### 5.3.11	Radio link failure related actions

Editor’s Note: Agreement: No enhancement to R16 RLF and RRC connection Re-establishment procedures are introduced in R17.  (this does not include handling of UL synchronisation loss which is FFS and does not include non continuous coverage).

#### 5.3.11.1	Detection of physical layer problems in RRC_CONNECTED

The UE shall:

1>	if any DAPS bearer is configured, upon receiving N310 consecutive "out-of-sync" indications for the source PCell from lower layers and T304 is running:

2>	start timer T310 for the source PCell;

1>	upon receiving N310 consecutive "out-of-sync" indications for the PCell from lower layers while neither T300, T301, T304, T311, nor T316 is running:

2>	start timer T310;

1>	upon receiving N313 consecutive "out-of-sync" indications for the PSCell from lower layers while T307 is not running:

2>	start T313;

NOTE:	Physical layer monitoring and related autonomous actions do not apply to SCells except for the PSCell.

#### 5.3.11.1a	Early detection of physical layer problems in RRC_CONNECTED

The UE shall:

1>	upon receiving N310 consecutive "early-out-of-sync" indications for the PCell from lower layers:

2>	start timer T314 with the timer value set to the value of T310;

#### 5.3.11.1b	Detection of physical layer improvements in RRC_CONNECTED

The UE shall:

1>	upon receiving N311 consecutive "early-in-sync" indications for the PCell from lower layers:

2>	start timer T315 with the timer value set to the value of T310;

#### 5.3.11.2	Recovery of physical layer problems

Upon receiving N311 consecutive "in-sync" indications for the PCell from lower layers while T310 is running, the UE shall:

1>	stop timer T310;

1>	stop timer T312, if running;

NOTE 1:	In this case, the UE maintains the RRC connection without explicit signalling, i.e. the UE maintains the entire radio resource configuration.

NOTE 2:	Periods in time where neither "in-sync" nor "out-of-sync" is reported by layer 1 do not affect the evaluation of the number of consecutive "in-sync" or "out-of-sync" indications.

Upon receiving N314 consecutive "in-sync" indications for the PSCell from lower layers while T313 is running, the UE shall:

1>	stop timer T313;

#### 5.3.11.2a	Recovery of early detection of physical layer problems

Upon receiving N311 consecutive "in-sync" indications for the PCell from lower layers while T314 is running, the UE shall:

1>	stop timer T314;

#### 5.3.11.2b	Cancellation of physical layer improvements in RRC_CONNECTED

Upon receiving N311 consecutive "in-sync" indications for the PCell from lower layers while T315 is running, the UE shall:

1>	stop timer T315;

#### 5.3.11.3	Detection of radio link failure

The UE shall:

1>	in case any DAPS bearer is configured, only the target PCell is considered in the following;

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	discard any segments of segmented RRC messages received;

2>	store the following radio link failure information in the VarRLF-Report (VarRLF-Report-NB in NB-IoT) by setting its fields as follows:

3>	clear the information included in VarRLF-Report (VarRLF-Report-NB in NB-IoT), if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	except for NB-IoT, set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

3>	except for NB-IoT, if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

3>	except for NB-IoT, if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise , except for NB-IoT, to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	except for NB-IoT, set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

3>	except for NB-IoT, if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

5>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

5>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO NR:

5>	include the previousNR-PCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3.

3>	except for NB-IoT, if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1:

4>	include the drb-EstablishedWithQCI-1;

3>	except for NB-IoT, set the connectionFailureType to rlf;

3>	except for NB-IoT, set the c-RNTI to the C-RNTI used in the PCell;

3>	except for NB-IoT, set the rlf-Cause to the trigger for detecting radio link failure;

2>	if the UE is configured with (NG)EN-DC; and

2>	if T316 is configured; and

2>	if SCG transmission is not suspended; and

2>	if neither NR PSCell change nor NR PSCell addition is ongoing (i.e. T304 for the NR PSCell is not running as specified in TS 38.331 [82], clause 5.3.5.5.2, in (NG)EN-DC):

3>	initiate the MCG failure information procedure as specified in 5.6.26 to report MCG radio link failure;

2>	else:

3>	if AS security has not been activated:

4>	if the UE is a NB-IoT UE:

5>	if the UE is connected to EPC and the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation; or

5>	if the UE is connected to 5GC, the UE supports RRC connection re-establishment for the Control Plane CIoT 5GS optimisation and the UE is configured with a truncated 5G-S-TMSI:

6>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

5>	else:

6>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

4>	else:

5>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

In case of DC or NE-DC, the UE shall:

1>	upon T313 expiry; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC, which is allowed to be sent on PSCell, that the maximum number of retransmissions has been reached for an SCG, for a split DRB or for a split SRB:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	if the UE is configured with DC; or

2>	if the UE is configured with NE-DC and MCG transmission is not suspended:

3>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

2>	else:

3>	initiate the connection re-establishment procedure as specified in TS 38.331 [82], clause 5.3.7.

In case of CA PDCP duplication, the UE shall:

1>	upon indication from an RLC entity, which is restricted to be sent on SCell only, that the maximum number of retransmissions has been reached:

2>	initiate the failure information procedure as specified in 5.6.21 to report RLC failure of type duplication;

If any DAPS bearer is configured and T304 is running, the UE shall:

1>	upon T310 expiry for the source PCell; or

1>	upon random access problem indication from source MCG MAC; or

1>	upon indication from source MCG RLC, which is allowed to be sent on source PCell, that the maximum number of retransmissions has been reached for an DRB:

2>	consider radio link failure to be detected for the source MCG;

2>	suspend the transmission of all DRBs in the source MCG;

2>	reset MAC for the source MCG;

2>	release the source connection;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report (VarRLF-Report-NB in NB-IoT), 48 hours after the radio link failure is detected, upon power off or upon detach, and for NB-IoT, upon entering another RAT.

#### 5.3.11.3a	Detection of early-out-of-sync event

The UE shall:

1>	upon T314 expiry;

2>	consider "early-out-of-sync" event to be detected and initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10;

#### 5.3.11.3b	Detection of early-in-sync event

The UE shall:

1>	upon T315 expiry;

2>	consider "early-in-sync" event to be detected and initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10;

Next change

## 6.2	RRC messages

NOTE:	The messages included in this clause reflect the current status of the discussions. Additional messages may be included at a later stage.

### 6.2.1	General message structure

#### –	EUTRA-RRC-Definitions

This ASN.1 segment is the start of the E-UTRA RRC PDU definitions.

-- ASN1START

EUTRA-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- ASN1STOP

#### –	BCCH-BCH-Message

The BCCH-BCH-Message class is the set of RRC messages that may be sent from the E-UTRAN to the UE via BCH on the BCCH logical channel.

-- ASN1START

BCCH-BCH-Message ::= SEQUENCE {

message					BCCH-BCH-MessageType

}

BCCH-BCH-MessageType ::=						MasterInformationBlock

-- ASN1STOP

#### –	BCCH-BCH-Message-MBMS

The BCCH-BCH-Message-MBMS class is the set of RRC messages that may be sent from the E-UTRAN to the UE via BCH on the BCCH logical channel in an MBMS-dedicated cell.

-- ASN1START

BCCH-BCH-Message-MBMS::= SEQUENCE {

message					BCCH-BCH-MessageType-MBMS-r14

}

BCCH-BCH-MessageType-MBMS-r14 ::=					MasterInformationBlock-MBMS-r14

-- ASN1STOP

#### –	BCCH-DL-SCH-Message

The BCCH-DL-SCH-Message class is the set of RRC messages that may be sent from the E-UTRAN to the UE via DL-SCH on the BCCH logical channel.

-- ASN1START

BCCH-DL-SCH-Message ::= SEQUENCE {

message					BCCH-DL-SCH-MessageType

}

BCCH-DL-SCH-MessageType ::= CHOICE {

c1						CHOICE {

systemInformation						SystemInformation,

systemInformationBlockType1				SystemInformationBlockType1

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	BCCH-DL-SCH-Message-BR

The BCCH-DL-SCH-Message-BR class is the set of RRC messages that may be sent from the E-UTRAN to the UE via DL-SCH on the BR-BCCH logical channel.

-- ASN1START

BCCH-DL-SCH-Message-BR ::= SEQUENCE {

message					BCCH-DL-SCH-MessageType-BR-r13

}

BCCH-DL-SCH-MessageType-BR-r13 ::= CHOICE {

c1						CHOICE {

systemInformation-BR-r13				SystemInformation-BR-r13,

systemInformationBlockType1-BR-r13		SystemInformationBlockType1-BR-r13

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	BCCH-DL-SCH-Message-MBMS

The BCCH-DL-SCH-Message-MBMS class is the set of RRC messages that may be sent from the E-UTRAN to the UE via DL-SCH on the BCCH logical channel in an MBMS-dedicated cell.

-- ASN1START

BCCH-DL-SCH-Message-MBMS ::= SEQUENCE {

message						BCCH-DL-SCH-MessageType-MBMS-r14

}

BCCH-DL-SCH-MessageType-MBMS-r14 ::= CHOICE {

c1									CHOICE {

systemInformation-MBMS-r14					SystemInformation-MBMS-r14,

systemInformationBlockType1-MBMS-r14		SystemInformationBlockType1-MBMS-r14

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	MCCH-Message

The MCCH-Message class is the set of RRC messages that may be sent from the E-UTRAN to the UE on the MCCH logical channel.

-- ASN1START

MCCH-Message ::=		SEQUENCE {

message					MCCH-MessageType

}

MCCH-MessageType ::= CHOICE {

c1							CHOICE {

mbsfnAreaConfiguration-r9		MBSFNAreaConfiguration-r9

},

later						CHOICE {

c2								CHOICE{

mbmsCountingRequest-r10			MBMSCountingRequest-r10

},

messageClassExtension	SEQUENCE {}

}

}

-- ASN1STOP

#### –	PCCH-Message

The PCCH-Message class is the set of RRC messages that may be sent from the E-UTRAN to the UE on the PCCH logical channel.

-- ASN1START

PCCH-Message ::= SEQUENCE {

message					PCCH-MessageType

}

PCCH-MessageType ::= CHOICE {

c1						CHOICE {

paging									Paging

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	DL-CCCH-Message

The DL-CCCH-Message class is the set of RRC messages that may be sent from the E-UTRAN to the UE on the downlink CCCH logical channel.

-- ASN1START

DL-CCCH-Message ::= SEQUENCE {

message					DL-CCCH-MessageType

}

DL-CCCH-MessageType ::= CHOICE {

c1						CHOICE {

rrcConnectionReestablishment			RRCConnectionReestablishment,

rrcConnectionReestablishmentReject		RRCConnectionReestablishmentReject,

rrcConnectionReject						RRCConnectionReject,

rrcConnectionSetup						RRCConnectionSetup

},

messageClassExtension	CHOICE {

c2						CHOICE {

rrcEarlyDataComplete-r15			RRCEarlyDataComplete-r15,

spare3	NULL, spare2 NULL, spare1 NULL

},

messageClassExtensionFuture-r15			SEQUENCE {}

}

}

-- ASN1STOP

#### –	DL-DCCH-Message

The DL-DCCH-Message class is the set of RRC messages that may be sent from the E-UTRAN to the UE or from the E-UTRAN to the RN on the downlink DCCH logical channel.

-- ASN1START

DL-DCCH-Message ::= SEQUENCE {

message					DL-DCCH-MessageType

}

DL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

csfbParametersResponseCDMA2000			CSFBParametersResponseCDMA2000,

dlInformationTransfer					DLInformationTransfer,

handoverFromEUTRAPreparationRequest		HandoverFromEUTRAPreparationRequest,

mobilityFromEUTRACommand				MobilityFromEUTRACommand,

rrcConnectionReconfiguration			RRCConnectionReconfiguration,

rrcConnectionRelease					RRCConnectionRelease,

securityModeCommand						SecurityModeCommand,

ueCapabilityEnquiry						UECapabilityEnquiry,

counterCheck							CounterCheck,

ueInformationRequest-r9					UEInformationRequest-r9,

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10,

rnReconfiguration-r10					RNReconfiguration-r10,

rrcConnectionResume-r13					RRCConnectionResume-r13,

dlDedicatedMessageSegment-r16			DLDedicatedMessageSegment-r16,

spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	UL-CCCH-Message

The UL-CCCH-Message class is the set of RRC messages that may be sent from the UE to the E-UTRAN on the uplink CCCH logical channel.

-- ASN1START

UL-CCCH-Message ::= SEQUENCE {

message					UL-CCCH-MessageType

}

UL-CCCH-MessageType ::= CHOICE {

c1						CHOICE {

rrcConnectionReestablishmentRequest		RRCConnectionReestablishmentRequest,

rrcConnectionRequest					RRCConnectionRequest

},

messageClassExtension	CHOICE {

c2						CHOICE {

rrcConnectionResumeRequest-r13		RRCConnectionResumeRequest-r13

},

messageClassExtensionFuture-r13	CHOICE {

c3						CHOICE {

rrcEarlyDataRequest-r15			RRCEarlyDataRequest-r15,

spare3	NULL, spare2	NULL, spare1	NULL

},

messageClassExtensionFuture-r15		SEQUENCE {}

}

}

}

-- ASN1STOP

#### –	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the E-UTRAN or from the RN to the E-UTRAN on the uplink DCCH logical channel.

-- ASN1START

UL-DCCH-Message ::= SEQUENCE {

message			UL-DCCH-MessageType

}

UL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

csfbParametersRequestCDMA2000				CSFBParametersRequestCDMA2000,

measurementReport							MeasurementReport,

rrcConnectionReconfigurationComplete		RRCConnectionReconfigurationComplete,

rrcConnectionReestablishmentComplete		RRCConnectionReestablishmentComplete,

rrcConnectionSetupComplete					RRCConnectionSetupComplete,

securityModeComplete						SecurityModeComplete,

securityModeFailure							SecurityModeFailure,

ueCapabilityInformation						UECapabilityInformation,

ulHandoverPreparationTransfer				ULHandoverPreparationTransfer,

ulInformationTransfer						ULInformationTransfer,

counterCheckResponse						CounterCheckResponse,

ueInformationResponse-r9					UEInformationResponse-r9,

proximityIndication-r9						ProximityIndication-r9,

rnReconfigurationComplete-r10				RNReconfigurationComplete-r10,

mbmsCountingResponse-r10					MBMSCountingResponse-r10,

interFreqRSTDMeasurementIndication-r10		InterFreqRSTDMeasurementIndication-r10

},

messageClassExtension	CHOICE {

c2							CHOICE {

ueAssistanceInformation-r11			UEAssistanceInformation-r11,

inDeviceCoexIndication-r11			InDeviceCoexIndication-r11,

mbmsInterestIndication-r11			MBMSInterestIndication-r11,

scgFailureInformation-r12			SCGFailureInformation-r12,

sidelinkUEInformation-r12			SidelinkUEInformation-r12,

wlanConnectionStatusReport-r13		WLANConnectionStatusReport-r13,

rrcConnectionResumeComplete-r13		RRCConnectionResumeComplete-r13,

ulInformationTransferMRDC-r15		ULInformationTransferMRDC-r15,

scgFailureInformationNR-r15			SCGFailureInformationNR-r15,

measReportAppLayer-r15				MeasReportAppLayer-r15,

failureInformation-r15				FailureInformation-r15,

ulDedicatedMessageSegment-r16		ULDedicatedMessageSegment-r16,

purConfigurationRequest-r16			PURConfigurationRequest-r16,

failureInformation-r16				FailureInformation-r16,

mcgFailureInformation-r16			MCGFailureInformation-r16,

ulInformationTransferIRAT-r16		ULInformationTransferIRAT-r16

},

messageClassExtensionFuture-r11

SEQUENCE {}

}

}

-- ASN1STOP

#### –	SC-MCCH-Message

The SC-MCCH-Message class is the set of RRC messages that may be sent from the E-UTRAN to the UE on the SC-MCCH logical channel.

-- ASN1START

SC-MCCH-Message-r13 ::= SEQUENCE {

message					SC-MCCH-MessageType-r13

}

SC-MCCH-MessageType-r13 ::= CHOICE {

c1						CHOICE {

scptmConfiguration-r13						SCPTMConfiguration-r13

},

messageClassExtension	CHOICE {

c2							CHOICE {

scptmConfiguration-BR-r14				SCPTMConfiguration-BR-r14,

spare									NULL

},

messageClassExtensionFuture-r14	SEQUENCE {}

}

}

-- ASN1STOP

### 6.2.2	Message definitions

#### –	CounterCheck

The CounterCheck message is used by the E-UTRAN to indicate the current COUNT MSB values associated to each DRB and to request the UE to compare these to its COUNT MSB values and to report the comparison results to E-UTRAN.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

CounterCheck message

-- ASN1START

CounterCheck ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

counterCheck-r8						CounterCheck-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

CounterCheck-r8-IEs ::=	SEQUENCE {

drb-CountMSB-InfoList				DRB-CountMSB-InfoList,

nonCriticalExtension				CounterCheck-v8a0-IEs				OPTIONAL

}

CounterCheck-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				CounterCheck-v1530-IEs				OPTIONAL

}

CounterCheck-v1530-IEs ::= SEQUENCE {

drb-CountMSB-InfoListExt-r15		DRB-CountMSB-InfoListExt-r15		OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

DRB-CountMSB-InfoList ::=		SEQUENCE (SIZE (1..maxDRB)) OF DRB-CountMSB-Info

DRB-CountMSB-InfoListExt-r15 ::=	SEQUENCE (SIZE (1..maxDRBExt-r15)) OF DRB-CountMSB-Info

DRB-CountMSB-Info ::=	SEQUENCE {

drb-Identity					DRB-Identity,

countMSB-Uplink					INTEGER(0..33554431),

countMSB-Downlink				INTEGER(0..33554431)

}

-- ASN1STOP

CounterCheck field descriptions

count-MSB-Downlink If configured with E-UTRA PDCP, it indicates the value of 25 MSBs from downlink COUNT associated to this DRB. If configured with NR PDCP, it indicates the value of 25 MSBs from RX_NEXT – 1 (specified in TS 38.323 [83]) associated to this DRB.

count-MSB-Uplink If configured with E-UTRA PDCP, it indicates the value of 25 MSBs from uplink COUNT associated to this DRB. If configured with NR PDCP, it indicates the value of 25 MSBs from TX_NEXT – 1 (specified in TS 38.323 [83]) associated to this DRB.

drb-CountMSB-InfoList Indicates the MSBs of the COUNT values of the DRBs.

#### –	CounterCheckResponse

The CounterCheckResponse message is used by the UE to respond to a CounterCheck message.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

CounterCheckResponse message

-- ASN1START

CounterCheckResponse ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

counterCheckResponse-r8				CounterCheckResponse-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

CounterCheckResponse-r8-IEs ::=	SEQUENCE {

drb-CountInfoList					DRB-CountInfoList,

nonCriticalExtension				CounterCheckResponse-v8a0-IEs		OPTIONAL

}

CounterCheckResponse-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				CounterCheckResponse-v1530-IEs		OPTIONAL

}

CounterCheckResponse-v1530-IEs ::= SEQUENCE {

drb-CountInfoListExt-r15			DRB-CountInfoListExt-r15			OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

DRB-CountInfoList ::=			SEQUENCE (SIZE (0..maxDRB)) OF DRB-CountInfo

DRB-CountInfoListExt-r15 ::=	SEQUENCE (SIZE (1..maxDRBExt-r15)) OF DRB-CountInfo

DRB-CountInfo ::=	SEQUENCE {

drb-Identity					DRB-Identity,

count-Uplink					INTEGER(0..4294967295),

count-Downlink					INTEGER(0..4294967295)

}

-- ASN1STOP

CounterCheckResponse field descriptions

count-Downlink If configured with E-UTRA PDCP, it indicates the value of downlink COUNT associated to this DRB. If configured with NR PDCP, it indicates the value of RX_NEXT – 1 (specified in TS 38.323 [83]) associated to this DRB.

count-Uplink If configured with E-UTRA PDCP, it indicates the value of uplink COUNT associated to this DRB. If configured with NR PDCP, it indicates the value of TX_NEXT – 1 (specified in TS 38.323 [83]) associated to this DRB.

drb-CountInfoList Indicates the COUNT values of the DRBs.

#### –	CSFBParametersRequestCDMA2000

The CSFBParametersRequestCDMA2000 message is used by the UE to obtain the CDMA2000 1xRTT Parameters from the network. The UE needs these parameters to generate the CDMA2000 1xRTT Registration message used to register with the CDMA2000 1xRTT Network which is required to support CSFB to CDMA2000 1xRTT.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

CSFBParametersRequestCDMA2000 message

-- ASN1START

CSFBParametersRequestCDMA2000 ::= SEQUENCE {

criticalExtensions					CHOICE {

csfbParametersRequestCDMA2000-r8	CSFBParametersRequestCDMA2000-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

CSFBParametersRequestCDMA2000-r8-IEs ::= SEQUENCE {

nonCriticalExtension				CSFBParametersRequestCDMA2000-v8a0-IEs	OPTIONAL

}

CSFBParametersRequestCDMA2000-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	CSFBParametersResponseCDMA2000

The CSFBParametersResponseCDMA2000 message is used to provide the CDMA2000 1xRTT Parameters to the UE so the UE can register with the CDMA2000 1xRTT Network to support CSFB to CDMA2000 1xRTT.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

CSFBParametersResponseCDMA2000 message

-- ASN1START

CSFBParametersResponseCDMA2000 ::= SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

csfbParametersResponseCDMA2000-r8		CSFBParametersResponseCDMA2000-r8-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

CSFBParametersResponseCDMA2000-r8-IEs ::= SEQUENCE {

rand								RAND-CDMA2000,

mobilityParameters					MobilityParametersCDMA2000,

nonCriticalExtension				CSFBParametersResponseCDMA2000-v8a0-IEs	OPTIONAL

}

CSFBParametersResponseCDMA2000-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	DLDedicatedMessageSegment

The DLDedicatedMessageSegment message is used to transfer one segment of the RRCConnectionResume or RRCConnectionReconfiguration messages.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

DLDedicatedMessageSegment message

-- ASN1START

DLDedicatedMessageSegment-r16 ::=		SEQUENCE {

criticalExtensions						CHOICE {

dlDedicatedMessageSegment-r16			DLDedicatedMessageSegment-r16-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

DLDedicatedMessageSegment-r16-IEs ::=	SEQUENCE {

segmentNumber-r16						INTEGER (0..4),

rrc-MessageSegmentContainer-r16			OCTET STRING,

rrc-MessageSegmentType-r16				ENUMERATED {notLastSegment, lastSegment},

lateNonCriticalExtensions				OCTET STRING					OPTIONAL,

nonCriticalExtension					SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

DLDedicatedMessageSegment field descriptions

segmentNumber Identifies the sequence number of a segment within the encoded DL DCCH message. The network transmits the segments with continuously increasing segmentNumber order so that the UE's RRC layer may expect to obtain them from lower layers in the correct order. Hence, the UE is not required to perform segment re-ordering on RRC level.

rrc-MessageSegmentContainer Includes a segment of the encoded DL DCCH message. The size of the included segment in this container should be small enough so the resulting encoded RRC message PDU is less than or equal to the PDCP SDU size limit.

rrc-MessageSegmentType Indicates whether the included DL DCCH message segment is the last segment of the message or not.

#### –	DLInformationTransfer

The DLInformationTransfer message is used for the downlink transfer of NAS, non-3GPP dedicated information, IAB-DU specific F1-C related information, or time reference information.

NOTE:	The UE may use the time reference information provided in the timeReferenceInfo IE for numerous purposes, possibly involving upper layers e.g. to synchronise the UE clock.

Signalling radio bearer: SRB2 or SRB1. If only timeReferenceInfo is included in the message, SRB1 is used. Otherwise, SRB1 is used only if SRB2 not established yet, and if SRB2 is suspended, E-UTRAN does not send this message until SRB2 is resumed. If only dedicatedInfoF1c is included, SRB2 is used.

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

DLInformationTransfer message

-- ASN1START

DLInformationTransfer ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

dlInformationTransfer-r8			DLInformationTransfer-r8-IEs,

dlInformationTransfer-r15			DLInformationTransfer-r15-IEs,

spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

DLInformationTransfer-r8-IEs ::=	SEQUENCE {

dedicatedInfoType					CHOICE {

dedicatedInfoNAS					DedicatedInfoNAS,

dedicatedInfoCDMA2000-1XRTT			DedicatedInfoCDMA2000,

dedicatedInfoCDMA2000-HRPD			DedicatedInfoCDMA2000

},

nonCriticalExtension				DLInformationTransfer-v8a0-IEs		OPTIONAL

}

DLInformationTransfer-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				DLInformationTransfer-v1610-IEs		OPTIONAL

}

DLInformationTransfer-r15-IEs ::=	SEQUENCE {

dedicatedInfoType-r15				CHOICE {

dedicatedInfoNAS-r15				DedicatedInfoNAS,

dedicatedInfoCDMA2000-1XRTT-r15		DedicatedInfoCDMA2000,

dedicatedInfoCDMA2000-HRPD-r15		DedicatedInfoCDMA2000

}																		OPTIONAL,	-- Need ON

timeReferenceInfo-r15				TimeReferenceInfo-r15				OPTIONAL,	-- Need ON

nonCriticalExtension				DLInformationTransfer-v8a0-IEs		OPTIONAL

}

DLInformationTransfer-v1610-IEs ::= SEQUENCE {

dedicatedInfoF1c-r16				DedicatedInfoF1c-r16				OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	FailureInformation

The FailureInformation message is used to provide information regarding failures detected by the UE, e.g. radio link failure for one of the RLC entities configured with PDCP duplication or failure of a DAPS HO.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

FailureInformation message

-- ASN1START

FailureInformation-r15 ::=		SEQUENCE {

failedLogicalChannelInfo-r15	FailedLogicalChannelInfo-r15		OPTIONAL

-- nonCriticalExtension is removed in this version as OPTIONAL was missing

}

FailureInformation-r16 ::=		SEQUENCE {

criticalExtensions					CHOICE {

failureInformation-r16				FailureInformation-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

FailedLogicalChannelInfo-r15 ::=	SEQUENCE {

failedLogicalChannelIdentity-r15		SEQUENCE {

cellGroupIndication-r15				ENUMERATED {mn, sn},

logicalChannelIdentity-r15			INTEGER (1..10)				OPTIONAL,

logicalChannelIdentityExt-r15		INTEGER (32..38)			OPTIONAL

},

failureType	ENUMERATED {duplication, spare3, spare2, spare1}

}

FailureInformation-r16-IEs ::=	SEQUENCE {

failedLogicalChannelIdentity-r16	FailedLogicalChannelIdentity-r16	OPTIONAL,

failureType-r16						ENUMERATED {duplication, dapsHO-failure,

spare2, spare1}					OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

FailedLogicalChannelIdentity-r16 ::=	SEQUENCE {

cellGroupIndication-r16				ENUMERATED {mn, sn},

logicalChannelIdentity-r16			INTEGER (1..10)				OPTIONAL,

logicalChannelIdentityExt-r16		INTEGER (32..38)			OPTIONAL

}

-- ASN1STOP

FailureInformation field descriptions

cellGroupIndication This field indicates the cell group (MCG, SCG) of the RLC entity for which the PDCP duplication failure occurred.

failureType This field indicates the type of failure reported. Value duplication indicates that a radio link failure for one of the RLC entities configured with PDCP duplication has been detected. Value dapsHO-failure indicates that timer T304 expired during a DAPS HO.

logicalChannelIdentity, logicalChannelIdentityExt This field indicates the logical channel identity of the RLC entity for which the PDCP duplication failure occurred.

NOTE:	The UE may apply the FailureInformation-r16 message to report a failure defined in REL-15, but only if it is configured with a feature incorporating a failure that can only be reported by the FailureInformation-r16 message.

#### –	HandoverFromEUTRAPreparationRequest (CDMA2000)

The HandoverFromEUTRAPreparationRequest message is used to trigger the handover preparation procedure with a CDMA2000 RAT. This message is also used to trigger a tunneled preparation procedure with a CDMA2000 1xRTT RAT to obtain traffic channel resources for the enhanced CS fallback to CDMA2000 1xRTT, which may also involve a concurrent preparation for handover to CDMA2000 HRPD. Also, this message is used to trigger the dual Rx/Tx redirection procedure with a CDMA2000 1xRTT RAT.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

HandoverFromEUTRAPreparationRequest message

-- ASN1START

HandoverFromEUTRAPreparationRequest ::= SEQUENCE {

rrc-TransactionIdentifier		RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

handoverFromEUTRAPreparationRequest-r8		HandoverFromEUTRAPreparationRequest-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverFromEUTRAPreparationRequest-r8-IEs ::= SEQUENCE {

cdma2000-Type					CDMA2000-Type,

rand							RAND-CDMA2000				OPTIONAL,	-- Cond cdma2000-Type

mobilityParameters				MobilityParametersCDMA2000	OPTIONAL,	-- Cond cdma2000-Type

nonCriticalExtension			HandoverFromEUTRAPreparationRequest-v890-IEs	OPTIONAL

}

HandoverFromEUTRAPreparationRequest-v890-IEs ::= SEQUENCE {

lateNonCriticalExtension		OCTET STRING				OPTIONAL,

nonCriticalExtension			HandoverFromEUTRAPreparationRequest-v920-IEs	OPTIONAL

}

HandoverFromEUTRAPreparationRequest-v920-IEs ::= SEQUENCE {

concurrPrepCDMA2000-HRPD-r9		BOOLEAN					OPTIONAL,	-- Cond cdma2000-Type

nonCriticalExtension			HandoverFromEUTRAPreparationRequest-v1020-IEs	OPTIONAL

}

HandoverFromEUTRAPreparationRequest-v1020-IEs ::= SEQUENCE {

dualRxTxRedirectIndicator-r10		ENUMERATED {true}		OPTIONAL,	-- Cond cdma2000-1XRTT

redirectCarrierCDMA2000-1XRTT-r10	CarrierFreqCDMA2000		OPTIONAL,	-- Cond dualRxTxRedirect

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

-- ASN1STOP

HandoverFromEUTRAPreparationRequest field descriptions

concurrPrepCDMA2000-HRPD Value TRUE indicates that upper layers should initiate concurrent preparation for handover to CDMA2000 HRPD in addition to preparation for enhanced CS fallback to CDMA2000 1xRTT.

dualRxTxRedirectIndicator Value TRUE indicates that the second radio of the dual Rx/Tx UE is being redirected to CDMA2000 1xRTT, as specified in TS 23.272 [51].

redirectCarrierCDMA2000-1XRTT Used to indicate the CDMA2000 1xRTT carrier frequency where the UE is being redirected to.

| Conditional presence | Explanation |
| --- | --- |
| cdma2000-1XRTT | The field is optionally present, need ON, if the cdma2000-Type = type1XRTT; otherwise it is not present. |
| cdma2000-Type | The field is mandatory present if the cdma2000-Type = type1XRTT; otherwise it is not present. |
| dualRxTxRedirect | The field is optionally present, need ON, if dualRxTxRedirectIndicator is present; otherwise it is not present. |

#### –	InDeviceCoexIndication

The InDeviceCoexIndication message is used to inform E-UTRAN about IDC problems which can not be solved by the UE itself, as well as to provide information that may assist E-UTRAN when resolving these problems.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

InDeviceCoexIndication message

-- ASN1START

InDeviceCoexIndication-r11 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

inDeviceCoexIndication-r11				InDeviceCoexIndication-r11-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

InDeviceCoexIndication-r11-IEs ::=	SEQUENCE {

affectedCarrierFreqList-r11			AffectedCarrierFreqList-r11					OPTIONAL,

tdm-AssistanceInfo-r11				TDM-AssistanceInfo-r11						OPTIONAL,

lateNonCriticalExtension			OCTET STRING								OPTIONAL,

nonCriticalExtension				InDeviceCoexIndication-v11d0-IEs			OPTIONAL

}

InDeviceCoexIndication-v11d0-IEs ::=	SEQUENCE {

ul-CA-AssistanceInfo-r11			SEQUENCE {

affectedCarrierFreqCombList-r11		AffectedCarrierFreqCombList-r11		OPTIONAL,

victimSystemType-r11				VictimSystemType-r11

}																			OPTIONAL,

nonCriticalExtension				InDeviceCoexIndication-v1310-IEs		OPTIONAL

}

InDeviceCoexIndication-v1310-IEs ::=	SEQUENCE {

affectedCarrierFreqList-v1310			AffectedCarrierFreqList-v1310		OPTIONAL,

affectedCarrierFreqCombList-r13			AffectedCarrierFreqCombList-r13		OPTIONAL,

nonCriticalExtension					InDeviceCoexIndication-v1360-IEs	OPTIONAL

}

InDeviceCoexIndication-v1360-IEs ::=	SEQUENCE {

hardwareSharingProblem-r13				ENUMERATED {true}					OPTIONAL,

nonCriticalExtension					InDeviceCoexIndication-v1530-IEs	OPTIONAL

}

InDeviceCoexIndication-v1530-IEs ::=	SEQUENCE {

mrdc-AssistanceInfo-r15					MRDC-AssistanceInfo-r15				OPTIONAL,

nonCriticalExtension					InDeviceCoexIndication-v1610-IEs	OPTIONAL

}

InDeviceCoexIndication-v1610-IEs::=	SEQUENCE {

victimSystemType-v1610					VictimSystemType-v1610			OPTIONAL,

nonCriticalExtension					SEQUENCE {}							OPTIONAL

}

AffectedCarrierFreqList-r11 ::=	SEQUENCE (SIZE (1..maxFreqIDC-r11)) OF AffectedCarrierFreq-r11

AffectedCarrierFreqList-v1310 ::= SEQUENCE (SIZE (1..maxFreqIDC-r11)) OF AffectedCarrierFreq-v1310

AffectedCarrierFreq-r11 ::=	SEQUENCE {

carrierFreq-r11				MeasObjectId,

interferenceDirection-r11	ENUMERATED {eutra, other, both, spare}

}

AffectedCarrierFreq-v1310 ::=	SEQUENCE {

carrierFreq-v1310				MeasObjectId-v1310								OPTIONAL

}

AffectedCarrierFreqCombList-r11 ::=	SEQUENCE (SIZE (1..maxCombIDC-r11)) OF AffectedCarrierFreqComb-r11

AffectedCarrierFreqCombList-r13 ::= SEQUENCE (SIZE (1..maxCombIDC-r11)) OF AffectedCarrierFreqComb-r13

AffectedCarrierFreqComb-r11 ::=	SEQUENCE (SIZE (2..maxServCell-r10)) OF MeasObjectId

AffectedCarrierFreqComb-r13 ::= SEQUENCE (SIZE (2..maxServCell-r13)) OF MeasObjectId-r13

TDM-AssistanceInfo-r11 ::=	CHOICE {

drx-AssistanceInfo-r11				SEQUENCE {

drx-CycleLength-r11					ENUMERATED {sf40, sf64, sf80, sf128, sf160,

sf256, spare2, spare1},

drx-Offset-r11						INTEGER (0..255)	OPTIONAL,

drx-ActiveTime-r11					ENUMERATED {sf20, sf30, sf40, sf60, sf80,

sf100, spare2, spare1}

},

idc-SubframePatternList-r11			IDC-SubframePatternList-r11,

...

}

IDC-SubframePatternList-r11 ::=	SEQUENCE (SIZE (1..maxSubframePatternIDC-r11)) OF IDC-SubframePattern-r11

IDC-SubframePattern-r11 ::= CHOICE {

subframePatternFDD-r11				BIT STRING (SIZE (4)),

subframePatternTDD-r11				CHOICE {

subframeConfig0-r11					BIT STRING (SIZE (70)),

subframeConfig1-5-r11				BIT STRING (SIZE (10)),

subframeConfig6-r11					BIT STRING (SIZE (60))

},

...

}

VictimSystemType-r11 ::= SEQUENCE {

gps-r11							ENUMERATED {true}				OPTIONAL,

glonass-r11						ENUMERATED {true}				OPTIONAL,

bds-r11							ENUMERATED {true}				OPTIONAL,

galileo-r11						ENUMERATED {true}				OPTIONAL,

wlan-r11						ENUMERATED {true}				OPTIONAL,

bluetooth-r11					ENUMERATED {true}				OPTIONAL

}

VictimSystemType-v1610 ::= SEQUENCE {

navic-r16						ENUMERATED {true}				OPTIONAL

}

MRDC-AssistanceInfo-r15 ::= SEQUENCE {

affectedCarrierFreqCombInfoListMRDC-r15		SEQUENCE (SIZE (1..maxCombIDC-r11)) OF AffectedCarrierFreqCombInfoMRDC-r15,

...,

[[	affectedCarrierFreqCombInfoListMRDC-v1610		SEQUENCE (SIZE (1..maxCombIDC-r11)) OF VictimSystemType-v1610			OPTIONAL

]]

}

AffectedCarrierFreqCombInfoMRDC-r15 ::= SEQUENCE {

victimSystemType-r15					VictimSystemType-r11,

interferenceDirectionMRDC-r15			ENUMERATED {eutra-nr, nr, other, eutra-nr-other,

nr-other, spare3, spare2, spare1},

affectedCarrierFreqCombMRDC-r15		SEQUENCE {

affectedCarrierFreqCombEUTRA-r15		AffectedCarrierFreqComb-r15		OPTIONAL,

affectedCarrierFreqCombNR-r15			AffectedCarrierFreqCombNR-r15

}				OPTIONAL

}

AffectedCarrierFreqComb-r15 ::= SEQUENCE (SIZE (1..maxServCell-r13)) OF MeasObjectId-r13

AffectedCarrierFreqCombNR-r15 ::= SEQUENCE (SIZE (1..maxServCellNR-r15)) OF ARFCN-ValueNR-r15

-- ASN1STOP

InDeviceCoexIndication field descriptions

AffectedCarrierFreq If carrierFreq-v1310 is included, carrierFreq-r11 is ignored by eNB.

affectedCarrierFreqCombList Indicates a list of E-UTRA carrier frequencies that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from E-UTRA when configured with UL CA. affectedCarrierFreqCombList-r13 is used when more than 5 serving cells are configured or affected combinations contain MeasObjectId larger than 32. If affectedCarrierFreqCombList-r13 is included, affectedCarrierFreqCombList-r11 shall not be included.

affectedCarrierFreqCombMRDC Indicates a set of at least one NR carrier frequency and optionally one or more E-UTRA carrier frequency that is affected by IDC problems due to Inter-Modulation Distortion and harmonics when configured with MR-DC.

affectedCarrierFreqList List of E-UTRA carrier frequencies affected by IDC problems. If E-UTRAN includes affectedCarrierFreqList-v1310 it includes the same number of entries, and listed in the same order, as in affectedCarrierFreqList-r11.

drx-ActiveTime Indicates the desired active time that the E-UTRAN is recommended to configure. Value in number of subframes. Value sf20 corresponds to 20 subframes, sf30 corresponds to 30 subframes and so on.

drx-CycleLength Indicates the desired DRX cycle length that the E-UTRAN is recommended to configure. Value in number of subframes. Value sf40 corresponds to 40 subframes, sf64 corresponds to 64 subframes and so on.

drx-Offset Indicates the desired DRX starting offset that the E-UTRAN is recommended to configure. The UE shall set the value of drx-Offset smaller than the value of drx-CycleLength. The starting frame and subframe satisfy the relation: [(SFN * 10) + subframe number] modulo (drx-CycleLength) = drx-Offset.

hardwareSharingProblem Indicates whether the UE has hardware sharing problems that the UE cannot solve by itself. The field is present (i.e. value true), if the UE has such hardware sharing problems. Otherwise the field is absent.

idc-SubframePatternList A list of one or more subframe patterns indicating which HARQ process E-UTRAN is requested to abstain from using. Value 0 indicates that E-UTRAN is requested to abstain from using the subframe. For FDD, the radio frame in which the pattern starts (i.e. the radio frame in which the first/leftmost bit of the subframePatternFDD corresponds to subframe #0) occurs when SFN mod 2 = 0. For TDD, the first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. The UE shall indicate a subframe pattern that follows HARQ time line, as specified in TS 36.213 [23], i.e, if a subframe is set to 1 in the subframe pattern, also the corresponding subframes carrying the potential UL grant, as specified in TS 36.213 [23], clause 8.0, the UL HARQ retransmission, as specified in TS 36.213 [23], clause 8.0, and the DL/UL HARQ feedback, as specified in TS 36.213 [23], clauses 7.3, 8.3 and 9.1.2, shall be set to 1.

interferenceDirection Indicates the direction of IDC interference. Value eutra indicates that only E-UTRA is victim of IDC interference, value other indicates that only another radio is victim of IDC interference and value both indicates that both E-UTRA and another radio are victims of IDC interference. The other radio refers to either the ISM radio or GNSS (see TR 36.816 [63]).

interferenceDirectionMRDC Indicates the direction of IDC interference. Value eutra-nr indicates E-UTRA and NR is victim, value nr indicates NR, value other indicates other radio system and so on. The other radio refers to either the ISM radio or GNSS (see TR 36.816 [63]).

victimSystemType Indicate the list of victim system types to which IDC interference is caused from E-UTRA when configured with UL CA or from E-UTRA and NR when configured with MR-DC. gps, glonass, bds, galileo, and navic indicate the type of GNSS. Value wlan indicates WLAN and value bluetooth indicates Bluetooth.

#### –	InterFreqRSTDMeasurementIndication

The InterFreqRSTDMeasurementIndication message is used to indicate that the UE is going to either start or stop OTDOA inter-frequency RSTD measurement which requires measurement gaps as specified in TS 36.133 [16], clause 8.1.2.6. The InterFreqRSTDMeasurementIndication message is also used to indicate to the network that the UE is going to start/stop OTDOA intra-frequency RSTD measurements which require measurement gaps. The InterFreqRSTDMeasurementIndication message is also used to indicate to the network the measurement gap that the category M1 or M2 UE prefers to perform RSTD measurements with dense PRS configuration, as specified in TS 36.133 [16], Table 8.1.2.1-3.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

InterFreqRSTDMeasurementIndication message

-- ASN1START

InterFreqRSTDMeasurementIndication-r10 ::=			SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

interFreqRSTDMeasurementIndication-r10	InterFreqRSTDMeasurementIndication-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

InterFreqRSTDMeasurementIndication-r10-IEs ::=		SEQUENCE {

rstd-InterFreqIndication-r10		CHOICE {

start								SEQUENCE {

rstd-InterFreqInfoList-r10				RSTD-InterFreqInfoList-r10

},

stop								NULL

},

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

RSTD-InterFreqInfoList-r10 ::=	SEQUENCE (SIZE(1..maxRSTD-Freq-r10)) OF RSTD-InterFreqInfo-r10

RSTD-InterFreqInfo-r10 ::=		SEQUENCE {

carrierFreq-r10					ARFCN-ValueEUTRA,

measPRS-Offset-r10				INTEGER (0..39),

...,

[[	carrierFreq-v1090			ARFCN-ValueEUTRA-v9e0				OPTIONAL

]],

[[	measPRS-Offset-r15		CHOICE {

rstd0-r15				INTEGER (0..79),

rstd1-r15				INTEGER (0..159),

rstd2-r15				INTEGER (0..319),

rstd3-r15				INTEGER (0..639),

rstd4-r15				INTEGER (0..1279),

rstd5-r15				INTEGER (0..159),

rstd6-r15				INTEGER (0..319),

rstd7-r15				INTEGER (0..639),

rstd8-r15				INTEGER (0..1279),

rstd9-r15				INTEGER (0..319),

rstd10-r15				INTEGER (0..639),

rstd11-r15				INTEGER (0..1279),

rstd12-r15				INTEGER (0..319),

rstd13-r15				INTEGER (0..639),

rstd14-r15				INTEGER (0..1279),

rstd15-r15				INTEGER (0..639),

rstd16-r15				INTEGER (0..1279),

rstd17-r15				INTEGER (0..639),

rstd18-r15				INTEGER (0..1279),

rstd19-r15				INTEGER (0..639),

rstd20-r15				INTEGER (0..1279)

}														OPTIONAL

]]

}

-- ASN1STOP

InterFreqRSTDMeasurementIndication field descriptions

carrierFreq The EARFCN value of the carrier received from upper layers for which the UE needs to perform the inter-frequency RSTD measurements. If the UE includes carrierFreq-v1090, it shall set carrierFreq-r10 to maxEARFCN. In case the UE starts intra-frequency RSTD measurements the carrierFreq indicates the carrier frequency of the serving cell.

measPRS-Offset Indicates the requested gap offset for performing inter-frequency or intra-frequency RSTD measurements. It is the smallest subframe offset from the beginning of subframe 0 of SFN=0 of the serving cell of the requested gap for measuring PRS positioning occasions in the carrier frequency carrierFreq for which the UE needs to perform the inter-frequency or intra-frequency RSTD measurements. The PRS positioning occasion information is received from upper layers. The value of measPRS-Offset-r10 is obtained by mapping the starting subframe of the PRS positioning occasion in the measured cell onto the corresponding subframe in the serving cell and is calculated as the serving cell's number of subframes from SFN=0 mod 40. If measPRS-Offset-r15 is included, the field further indicates the requested gap pattern that the category M1 or M2 UE prefers to perform RSTD measurements with dense PRS configuration, as specified in TS 36.133 [16], Table 8.1.2.1-3, where value rstd0 corresponds to Gap Pattern Id rstd0, value rstd1 corresponds to Gap Pattern Id rstd1 and so on. The value of measPRS-Offset-r15 is obtained by mapping the starting subframe of the PRS positioning occasion in the measured cell onto the corresponding subframe in the serving cell and is calculated as the serving cell's number of subframes from SFN=0 mod MGRP corresponding to the requested Gap pattern Id. If measPRS-Offset-r15 is included, measPRS-Offset-r10 is ignored. The UE shall take into account any additional time required by the UE to start PRS measurements on the other carrier when it does this mapping for determining the measPRS-Offset. NOTE: Figure 6.2.2-1 illustrates the measPRS-Offset field.

rstd-InterFreqIndication Indicates the inter-frequency or intra-frequency RSTD measurement action, i.e. the UE is going to start or stop inter-frequency or intra-frequency RSTD measurement.

Figure 6.2.2-1 (informative): Exemplary calculation of measPRS-Offset field.

#### –	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by E-UTRAN to configure the UE to perform logging of measurement results while in RRC_IDLE or to perform logging of measurement results for MBSFN while in both RRC_IDLE and RRC_CONNECTED. It is used to transfer the logged measurement configuration for network performance optimisation, see TS 37.320 [60].

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

LoggedMeasurementConfiguration message

-- ASN1START

LoggedMeasurementConfiguration-r10 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r10-IEs ::= SEQUENCE {

traceReference-r10				TraceReference-r10,

traceRecordingSessionRef-r10	OCTET STRING (SIZE (2)),

tce-Id-r10						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r10			AbsoluteTimeInfo-r10,

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,	-- Need OR

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

nonCriticalExtension			LoggedMeasurementConfiguration-v1080-IEs	OPTIONAL

}

LoggedMeasurementConfiguration-v1080-IEs ::= SEQUENCE {

lateNonCriticalExtension-r10	OCTET STRING						OPTIONAL,

nonCriticalExtension			LoggedMeasurementConfiguration-v1130-IEs	OPTIONAL

}

LoggedMeasurementConfiguration-v1130-IEs ::= SEQUENCE {

plmn-IdentityList-r11			PLMN-IdentityList3-r11		OPTIONAL,	-- Need OR

areaConfiguration-v1130			AreaConfiguration-v1130		OPTIONAL,	-- Need OR

nonCriticalExtension			LoggedMeasurementConfiguration-v1250-IEs	OPTIONAL

}

LoggedMeasurementConfiguration-v1250-IEs ::= SEQUENCE {

targetMBSFN-AreaList-r12	TargetMBSFN-AreaList-r12		OPTIONAL,	-- Need OP

nonCriticalExtension			LoggedMeasurementConfiguration-v1530-IEs					OPTIONAL

}

LoggedMeasurementConfiguration-v1530-IEs ::= SEQUENCE {

bt-NameList-r15					BT-NameList-r15					OPTIONAL,	--Need OR

wlan-NameList-r15				WLAN-NameList-r15				OPTIONAL,	--Need OR

nonCriticalExtension			SEQUENCE {}						OPTIONAL

}

TargetMBSFN-AreaList-r12 ::=			SEQUENCE (SIZE (0..maxMBSFN-Area)) OF TargetMBSFN-Area-r12

TargetMBSFN-Area-r12 ::=				SEQUENCE {

mbsfn-AreaId-r12					MBSFN-AreaId-r12		OPTIONAL,	-- Need OR

carrierFreq-r12						ARFCN-ValueEUTRA-r9,

...

}

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ identities.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

targetMBSFN-AreaList Used to indicate logging of MBSFN measurements and further restrict the area and frequencies for which the UE performs measurement logging for MBSFN. If both MBSFN area id and carrier frequency are present, a specific MBSFN area is indicated. If only carrier frequency is present, all MBSFN areas on that carrier frequency are indicated. If there is no entry in the list, any MBSFN area is indicated.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [58].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [58]

#### –	MasterInformationBlock

The MasterInformationBlock includes the system information transmitted on BCH.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

MasterInformationBlock

-- ASN1START

MasterInformationBlock ::=			SEQUENCE {

dl-Bandwidth						ENUMERATED {

n6, n15, n25, n50, n75, n100},

phich-Config						PHICH-Config,

systemFrameNumber					BIT STRING (SIZE (8)),

schedulingInfoSIB1-BR-r13			INTEGER (0..31),

systemInfoUnchanged-BR-r15			BOOLEAN,

spare								BIT STRING (SIZE (4))

}

-- ASN1STOP

MasterInformationBlock field descriptions

dl-Bandwidth Parameter: transmission bandwidth configuration, NRB in downlink, see TS 36.101 [42], table 5.6-1. n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on.

phich-Config Specifies the PHICH configuration. If the UE is a BL UE or UE in CE, it shall ignore this field.

schedulingInfoSIB1-BR Indicates the index to the tables that define SystemInformationBlockType1-BR scheduling information. The tables are specified in TS 36.213 [23], Table 7.1.6-1 and Table 7.1.7.2.7-1. Value 0 means that SystemInformationBlockType1-BR is not scheduled.

systemFrameNumber Defines the 8 most significant bits of the SFN. As indicated in TS 36.211 [21], 6.6.1, the 2 least significant bits of the SFN are acquired implicitly in the P-BCH decoding, i.e. timing of 40ms P-BCH TTI indicates 2 least significant bits (within 40ms P-BCH TTI, the first radio frame: 00, the second radio frame: 01, the third radio frame: 10, the last radio frame: 11). One value applies for all serving cells of a Cell Group (i.e. MCG or SCG). The associated functionality is common (i.e. not performed independently for each cell).

systemInfoUnchanged-BR Value TRUE indicates that no change has occurred in the SIB1-BR and SI messages at least over the SI validity time. NOTE: Value of systemInfoUnchanged-BR is also carried in RSS (if transmitted), see TS 36.211 [21].

#### –	MasterInformationBlock-MBMS

The MasterInformationBlock-MBMS includes the system information transmitted on BCH.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

MasterInformationBlock-MBMS

-- ASN1START

MasterInformationBlock-MBMS-r14 ::=			SEQUENCE {

dl-Bandwidth-MBMS-r14						ENUMERATED {

n6, n15, n25, n50, n75, n100},

systemFrameNumber-r14						BIT STRING (SIZE (6)),

additionalNonMBSFNSubframes-r14				INTEGER (0..3),

semiStaticCFI-MBMS-r16						INTEGER (0..3),

spare										BIT STRING (SIZE (11))

}

-- ASN1STOP

MasterInformationBlock-MBMS field descriptions

additionalNonMBSFNSubframes Configures additional non-MBSFN subframes where SystemInformationBlockType1-MBMS and SystemInformation-MBMS may be transmitted. Value 0, 1, 2, 3 mean zero, one, two, three additional non-MBSFN subframes are configured after each subframe which has PBCH.

dl-Bandwidth-MBMS Parameter: transmission bandwidth configuration, NRB in downlink, see TS 36.101 [42], table 5.6-1. n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on.

semiStaticCFI-MBMS Indicates semi-static value of CFI as specified in TS 36.213 [23], clause 9.1.3. If value 0 is indicated, CFI is obtained from PCFICH, otherwise the UE may assume the CFI in CAS is given by this field.

systemFrameNumber Defines the 6 most significant bits of the SFN of the MBMS-dedicated cell. As indicated in TS 36.211 [21], clause 6.6.1, the 4 least significant bits of the SFN are acquired implicitly in the P-BCH decoding, i.e. timing of 160ms P-BCH TTI indicates 4 least significant bits (within 40ms P-BCH TTI, the first radio frame: 00, the fourth radio frame: 01, the eighth radio frame: 10, the last radio frame: 11).

#### –	MBMSCountingRequest

The MBMSCountingRequest message is used by E-UTRAN to count the UEs that are receiving or interested to receive specific MBMS services.

Signalling radio bearer: N/A

RLC-SAP: UM

Logical channel: MCCH

Direction: E-UTRAN to UE

MBMSCountingRequest message

-- ASN1START

MBMSCountingRequest-r10 ::=		SEQUENCE {

countingRequestList-r10			CountingRequestList-r10,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

CountingRequestList-r10 ::=	SEQUENCE (SIZE (1..maxServiceCount)) OF CountingRequestInfo-r10

CountingRequestInfo-r10 ::=		SEQUENCE {

tmgi-r10							TMGI-r9,

...

}

-- ASN1STOP

#### –	MBMSCountingResponse

The MBMSCountingResponse message is used by the UE to respond to an MBMSCountingRequest message.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

MBMSCountingResponse message

-- ASN1START

MBMSCountingResponse-r10 ::=			SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

countingResponse-r10				MBMSCountingResponse-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

MBMSCountingResponse-r10-IEs ::=	SEQUENCE {

mbsfn-AreaIndex-r10				INTEGER (0..maxMBSFN-Area-1)						OPTIONAL,

countingResponseList-r10		CountingResponseList-r10			OPTIONAL,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

CountingResponseList-r10 ::=		SEQUENCE (SIZE (1..maxServiceCount)) OF CountingResponseInfo-r10

CountingResponseInfo-r10 ::=		SEQUENCE {

countingResponseService-r10	INTEGER (0..maxServiceCount-1),

...

}

-- ASN1STOP

MBMSCountingResponse field descriptions

countingResponseList List of MBMS services which the UE is receiving or interested to receive. Value 0 for field countingResponseService corresponds to the first entry in countingRequestList within MBMSCountingRequest, value 1 corresponds to the second entry in this list and so on.

mbsfn-AreaIndex Index of the entry in field mbsfn-AreaInfoList within SystemInformationBlockType13. Value 0 corresponds to the first entry in 1st mbsfn-AreaInfoList within SystemInformationBlockType13, value 1 corresponds to the second entry in the same list, or when no more entry are present within the same mbsfn-AreaInfoList, then the first entry in the subsequent mbsfn-AreaInfoList within the same SystemInformationBlockType13 and so on.

#### –	MBMSInterestIndication

The MBMSInterestIndication message is used to inform E-UTRAN that the UE is receiving/ interested to receive or no longer receiving/ interested to receive MBMS via an MRB or SC-MRB including MBMS service(s) in receive only mode.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

MBMSInterestIndication message

-- ASN1START

MBMSInterestIndication-r11 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

interestIndication-r11				MBMSInterestIndication-r11-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

MBMSInterestIndication-r11-IEs ::=	SEQUENCE {

mbms-FreqList-r11					CarrierFreqListMBMS-r11				OPTIONAL,

mbms-Priority-r11					ENUMERATED {true}					OPTIONAL,

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				MBMSInterestIndication-v1310-IEs	OPTIONAL

}

MBMSInterestIndication-v1310-IEs ::=	SEQUENCE {

mbms-Services-r13					MBMS-ServiceList-r13				OPTIONAL,

nonCriticalExtension				MBMSInterestIndication-v1540-IEs		OPTIONAL

}

MBMSInterestIndication-v1540-IEs ::=	 SEQUENCE {

mbms-ROM-InfoList-r15			SEQUENCE (SIZE(1..maxMBMS-ServiceListPerUE-r13)) OF MBMS-ROM-Info-r15																	OPTIONAL,

nonCriticalExtension				MBMSInterestIndication-v1610-IEs	OPTIONAL

}

MBMSInterestIndication-v1610-IEs ::=	SEQUENCE {

mbms-ROM-InfoList-r16				SEQUENCE (SIZE(1..maxMBMS-ServiceListPerUE-r13)) OF MBMS-ROM-Info-r16		OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

MBMS-ROM-Info-r15 ::= SEQUENCE {

mbms-ROM-Freq-r15						ARFCN-ValueEUTRA-r9,

mbms-ROM-SubcarrierSpacing-r15		ENUMERATED {kHz15, kHz7dot5, kHz1dot25},

mbms-Bandwidth-r15					ENUMERATED {n6, n15, n25, n50, n75, n100}

}

MBMS-ROM-Info-r16 ::= SEQUENCE {

mbms-ROM-Freq-r16					ARFCN-ValueEUTRA-r9,

mbms-ROM-SubcarrierSpacing-r16		ENUMERATED {kHz2dot5, kHz0dot37},

mbms-Bandwidth-r16					ENUMERATED {n6, n15, n25, n50, n75, n100}

}

-- ASN1STOP

MBMSInterestIndication field descriptions

mbms-Bandwidth Indicates the UE received MBMS service frequency bandwidth configuration, NRB in downlink, see TS 36.101 [42], table 5.6-1. n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on.

mbms-FreqList List of MBMS frequencies on which the UE is receiving or interested to receive MBMS via an MRB or SC-MRB.

mbms-Priority Indicates whether the UE prioritises MBMS reception above unicast reception. The field is present (i.e. value true), if the UE prioritises reception of all listed MBMS frequencies above reception of any of the unicast bearers. Otherwise the field is absent.

mbms-ROM-Freq The value indicates the carrier frequency used by the UE to receive MBMS service(s) in receive only mode.

mbms-ROM-InfoList List of receive only mode MBMS service(s) related parameters which the UE is receiving or interested to receive.

mbms-ROM-SubcarrierSpacing The value indicates subcarrier spacing for MBSFN subframes received by UE in receive only mode and kHz15 refers to 15kHz, kHz7dot5 refers to 7.5kHz subcarrier spacing and so on as defined in TS 36.211 [21], clause 6.12.

#### –	MBSFNAreaConfiguration

The MBSFNAreaConfiguration message contains the MBMS control information applicable for an MBSFN area. For each MBSFN area included in SystemInformationBlockType13 E-UTRAN configures an MCCH (i.e. the MCCH identifies the MBSFN area) and signals the MBSFNAreaConfiguration message.

Signalling radio bearer: N/A

RLC-SAP: UM

Logical channel: MCCH

Direction: E-UTRAN to UE

MBSFNAreaConfiguration message

-- ASN1START

MBSFNAreaConfiguration-r9 ::=		SEQUENCE {

commonSF-Alloc-r9					CommonSF-AllocPatternList-r9,

commonSF-AllocPeriod-r9				ENUMERATED {

rf4, rf8, rf16, rf32, rf64, rf128, rf256},

pmch-InfoList-r9					PMCH-InfoList-r9,

nonCriticalExtension				MBSFNAreaConfiguration-v930-IEs	OPTIONAL

}

MBSFNAreaConfiguration-v930-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				MBSFNAreaConfiguration-v1250-IEs	OPTIONAL

}

MBSFNAreaConfiguration-v1250-IEs ::= SEQUENCE {

pmch-InfoListExt-r12				PMCH-InfoListExt-r12				OPTIONAL,	-- Need OR

nonCriticalExtension				MBSFNAreaConfiguration-v1430-IEs	OPTIONAL

}

MBSFNAreaConfiguration-v1430-IEs ::= SEQUENCE {

commonSF-Alloc-v1430					CommonSF-AllocPatternList-v1430,

nonCriticalExtension				MBSFNAreaConfiguration-v1610-IEs							OPTIONAL

}

MBSFNAreaConfiguration-v1610-IEs ::= SEQUENCE {

commonSF-Alloc-v1610					CommonSF-AllocPatternList-v1610		OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

CommonSF-AllocPatternList-r9 ::=	SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF MBSFN-SubframeConfig

CommonSF-AllocPatternList-v1430 ::=	SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF MBSFN-SubframeConfig-v1430

CommonSF-AllocPatternList-v1610 ::=	SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF MBSFN-SubframeConfig-v1610

-- ASN1STOP

MBSFNAreaConfiguration field descriptions

commonSF-Alloc Indicates the subframes allocated to the MBSFN area. E-UTRAN always sets this field to cover at least the subframes configured by SystemInformationBlockType13 for this MCCH, regardless of whether any MBMS sessions are ongoing. E-UTRAN includes commonSF-Alloc-v1610 only when the cell is a MBMS-dedicated cell. If E-UTRAN includes commonSF-Alloc-v1430 and/or commonSF-Alloc-v1610, it includes the same number of entries, and listed in the same order, as in commonSF-Alloc-r9.

commonSF-AllocPeriod Indicates the period during which resources corresponding with field commonSF-Alloc are divided between the (P)MCH that are configured for this MBSFN area. The subframe allocation patterns, as defined by commonSF-Alloc, repeat continously during this period. Value rf4 corresponds to 4 radio frames, rf8 corresponds to 8 radio frames and so on. The commonSF-AllocPeriod starts in the radio frames for which: SFN mod commonSF-AllocPeriod = 0.

pmch-InfoList EUTRAN may include pmch-InfoListExt even if pmch-InfoList does not include maxPMCH-PerMBSFN entries. EUTRAN configures at most maxPMCH-PerMBSFN entries i.e. across pmch-InfoList and pmch-InfoListExt.

#### –	MCGFailureInformation

The MCGFailureInformation message is used to provide information regarding E-UTRA MCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

MCGFailureInformation message

-- ASN1START

MCGFailureInformation-r16 ::=				SEQUENCE {

criticalExtensions						CHOICE {

mcgFailureInformation					MCGFailureInformation-r16-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

MCGFailureInformation-r16-IEs ::=			SEQUENCE {

failureReportMCG-r16						FailureReportMCG-r16					OPTIONAL,

lateNonCriticalExtension				OCTET STRING						OPTIONAL,

nonCriticalExtension					SEQUENCE {}							OPTIONAL

}

FailureReportMCG-r16 ::=					SEQUENCE {

failureType-r16								ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, t312-Expiry, spare4,

spare3, spare2, spare1}			OPTIONAL,

measResultFreqListEUTRA-r16					MeasResultList3EUTRA-r15			OPTIONAL,

measResultFreqListNR-r16					MeasResultFreqListFailNR-r15		OPTIONAL,

measResultFreqListGERAN-r16					MeasResultList2GERAN-r10			OPTIONAL,

measResultFreqListUTRA-r16					MeasResultList2UTRA-r9 				OPTIONAL,

measResultSCG-r16							OCTET STRING						OPTIONAL,

...

}

-- ASN1STOP

MCGFailureInformation field descriptions

measResultFreqListEUTRA The field contains available results of measurements on EUTRA frequencies the UE is configured to measure by measConfig.

measResultFreqListGERAN The field contains available results of measurements on GERAN frequencies the UE is configured to measure by measConfig.

measResultFreqListNR The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultFreqListUTRA The field contains available results of measurements on UTRA frequencies the UE is configured to measure by measConfig.

measResultSCG Includes the NR MeasResultSCG-Failure IE as specified in TS 38.331 [82]. The field contains available results of measurements on NR frequencies the UE is configured to measure by the NR RRCConfiguration message.

#### –	MeasReportAppLayer

The MeasReportAppLayer message is used for sending application layer measurement report.

Signalling radio bearer: SRB4

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

MeasReportAppLayer message

-- ASN1START

MeasReportAppLayer-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

measReportAppLayer-r15				MeasReportAppLayer-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

MeasReportAppLayer-r15-IEs ::=		SEQUENCE {

measReportAppLayerContainer-r15		OCTET STRING (SIZE(1..8000))			OPTIONAL,

serviceType-r15						ENUMERATED {qoe, qoemtsi, spare6, spare5, spare4, spare3, spare2, spare1}		OPTIONAL,

nonCriticalExtension				MeasReportAppLayer-v1590-IEs				OPTIONAL

}

MeasReportAppLayer-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

MeasReportAppLayer field descriptions

measReportAppLayerContainer The field contains container of application layer measurements, see Annex L (normative) in TS 26.247 [90] and clause 16.5 in TS 26.114 [99].

serviceType Indicates the type of application layer measurement. Value qoe indicates Quality of Experience Measurement Collection for streaming services, value qoemtsi indicates Quality of Experience Measurement Collection for MTSI.

#### –	MeasurementReport

The MeasurementReport message is used for the indication of measurement results.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

MeasurementReport message

-- ASN1START

MeasurementReport ::=				SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

measurementReport-r8				MeasurementReport-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

MeasurementReport-r8-IEs ::=		SEQUENCE {

measResults							MeasResults,

nonCriticalExtension				MeasurementReport-v8a0-IEs			OPTIONAL

}

MeasurementReport-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	MobilityFromEUTRACommand

The MobilityFromEUTRACommand message is used to command handover or a cell change from E-UTRA to another RAT (3GPP or non-3GPP), or enhanced CS fallback to CDMA2000 1xRTT.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

MobilityFromEUTRACommand message

-- ASN1START

MobilityFromEUTRACommand ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

mobilityFromEUTRACommand-r8			MobilityFromEUTRACommand-r8-IEs,

mobilityFromEUTRACommand-r9			MobilityFromEUTRACommand-r9-IEs,

spare2 NULL, spare1					NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

MobilityFromEUTRACommand-r8-IEs ::=	SEQUENCE {

cs-FallbackIndicator				BOOLEAN,

purpose								CHOICE{

handover							Handover,

cellChangeOrder						CellChangeOrder

},

nonCriticalExtension				MobilityFromEUTRACommand-v8a0-IEs	OPTIONAL

}

MobilityFromEUTRACommand-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				MobilityFromEUTRACommand-v8d0-IEs	OPTIONAL

}

MobilityFromEUTRACommand-v8d0-IEs ::= SEQUENCE {

bandIndicator						BandIndicatorGERAN			OPTIONAL,	-- Cond GERAN

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

MobilityFromEUTRACommand-r9-IEs ::=	SEQUENCE {

cs-FallbackIndicator				BOOLEAN,

purpose								CHOICE{

handover							Handover,

cellChangeOrder						CellChangeOrder,

e-CSFB-r9							E-CSFB-r9,

...

},

nonCriticalExtension				MobilityFromEUTRACommand-v930-IEs	OPTIONAL

}

MobilityFromEUTRACommand-v930-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				MobilityFromEUTRACommand-v960-IEs	OPTIONAL

}

MobilityFromEUTRACommand-v960-IEs ::= SEQUENCE {

bandIndicator					BandIndicatorGERAN					OPTIONAL,	-- Cond GERAN

nonCriticalExtension			MobilityFromEUTRACommand-v1530-IEs	OPTIONAL

}

MobilityFromEUTRACommand-v1530-IEs ::= SEQUENCE {

smtc-r15							MTC-SSB-NR-r15				OPTIONAL,		-- Need OP

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

Handover ::=						SEQUENCE {

targetRAT-Type						ENUMERATED {

utra, geran, cdma2000-1XRTT, cdma2000-HRPD,

nr, eutra, spare2, spare1, ...},

targetRAT-MessageContainer			OCTET STRING,

nas-SecurityParamFromEUTRA			OCTET STRING (SIZE (1))	OPTIONAL,	-- Cond UTRAGERANEPC

systemInformation					SI-OrPSI-GERAN				OPTIONAL	-- Cond PSHO

}

CellChangeOrder ::=				SEQUENCE {

t304								ENUMERATED {

ms100, ms200, ms500, ms1000,

ms2000, ms4000, ms8000, ms10000-v1310},

targetRAT-Type						CHOICE {

geran							SEQUENCE {

physCellId						PhysCellIdGERAN,

carrierFreq						CarrierFreqGERAN,

networkControlOrder				BIT STRING (SIZE (2))		OPTIONAL,	-- Need OP

systemInformation				SI-OrPSI-GERAN				OPTIONAL	-- Need OP

},

...

}

}

SI-OrPSI-GERAN ::=					CHOICE {

si									SystemInfoListGERAN,

psi									SystemInfoListGERAN

}

E-CSFB-r9 ::=						SEQUENCE {

messageContCDMA2000-1XRTT-r9		OCTET STRING		OPTIONAL,	-- Need ON

mobilityCDMA2000-HRPD-r9			ENUMERATED {

handover, redirection

}					OPTIONAL,	-- Need OP

messageContCDMA2000-HRPD-r9		OCTET STRING			OPTIONAL,	-- Cond concHO

redirectCarrierCDMA2000-HRPD-r9	CarrierFreqCDMA2000		OPTIONAL	-- Cond concRedir

}

-- ASN1STOP

MobilityFromEUTRACommand field descriptions

bandIndicator Indicates how to interpret the ARFCN of the BCCH carrier.

carrierFreq contains the carrier frequency of the target GERAN cell.

cs-FallbackIndicator Value true indicates that the CS fallback procedure to UTRAN or GERAN is triggered.

messageContCDMA2000-1XRTT This field contains a message specified in CDMA2000 1xRTT standard that either tells the UE to move to specific 1xRTT target cell(s) or indicates a failure to allocate resources for the enhanced CS fallback to CDMA2000 1xRTT.

messageContCDMA2000-HRPD This field contains a message specified in CDMA2000 HRPD standard that either tells the UE to move to specific HRPD target cell(s) or indicates a failure to allocate resources for the handover to CDMA2000 HRPD.

mobilityCDMA2000-HRPD This field indicates whether or not mobility to CDMA2000 HRPD is to be performed by the UE and it also indicates the type of mobility to CDMA2000 HRPD that is to be performed; If this field is not present the UE shall perform only the enhanced CS fallback to CDMA2000 1xRTT.

nas-SecurityParamFromEUTRA If the targetRAT-Type is set to "eutra" and the source CN is 5GC, this field is used to deliver the key synchronisation and key freshness for the Key freshness for the 5GS to EPS handovers as specified in TS 33.501 [86] and the content of the parameter is defined in TS 24.501 [95]. Otherwise, this field is used to deliver the key synchronisation and Key freshness for the E-UTRAN to UTRAN handovers as specified in TS 33.401 [32] and the content of the parameter is defined in TS24.301 [35].

networkControlOrder Parameter NETWORK_CONTROL_ORDER in TS 44.060 [36].

purpose Indicates which type of mobility procedure the UE is requested to perform. EUTRAN always applies value e-CSFB in case of enhanced CS fallback to CDMA2000 (e.g. also when that procedure results in handover to CDMA2000 1XRTT only, in handover to CDMA2000 HRPD only or in redirection to CDMA2000 HRPD only),

redirectCarrierCDMA2000-HRPD The redirectCarrierCDMA2000-HRPD indicates a CDMA2000 carrier frequency and is used to redirect the UE to a HRPD carrier frequency.

smtc The SSB periodicity/offset/duration configuration of target cell for inter-RAT handover to NR. It is based on timing reference of EUTRA PCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SystemInfoListGERAN If purpose = CellChangeOrder and if the field is not present, the UE has to acquire SI/PSI from the GERAN cell.

t304 Timer T304 as described in clause 7.3. Value ms100 corresponds with 100 ms, ms200 corresponds with 200 ms and so on. EUTRAN includes extended value ms10000-v1310 only when UE supports CE.

targetRAT-Type Indicates the target RAT type.

targetRAT-MessageContainer The field contains a message specified in another standard, as indicated by the targetRAT-Type, and carries information about the target cell identifier(s) and radio parameters relevant for the target radio access technology. NOTE 1. A complete message is included, as specified in the other standard.

| Conditional presence | Explanation |
| --- | --- |
| concHO | The field is mandatory present if the mobilityCDMA2000-HRPD is set to "handover"; otherwise the field is optional present, need ON. |
| concRedir | The field is mandatory present if the mobilityCDMA2000-HRPD is set to "redirection"; otherwise the field is not present. |
| GERAN | The field should be present if the purpose is set to "handover" and the targetRAT-Type is set to "geran"; otherwise the field is not present |
| PSHO | The field is mandatory present in case of PS handover toward GERAN; otherwise the field is optionally present, but not used by the UE |
| UTRAGERANEPC | The field is mandatory present if the targetRAT-Type is set to "utra" or "geran" or if the targetRAT-Type is set to "eutra" and the source CN is 5GC; otherwise the field is not present |

NOTE 1:	The correspondence between the value of the targetRAT-Type, the standard to apply and the message contained within the targetRAT-MessageContainer is shown in the table below:

| targetRAT-Type | Standard to apply | targetRAT-MessageContainer |
| --- | --- | --- |
| cdma2000-1XRTT | C.S0001 or later, C.S0007 or later, C.S0008 or later |  |
| cdma2000-HRPD | C.S0024 or later |  |
| eutra | TS 36.331 (clause 5.4.2) | RRCConnectionReconfiguration |  |
| geran | GSM TS 04.18, version 8.5.0 or later, or TS 44.018 (clause 9.1.15) TS 44.060, version 6.13.0 or later (clause 11.2.43) TS 44.060, version 7.6.0 or later (clause 11.2.46) | HANDOVER COMMAND PS HANDOVER COMMAND DTM HANDOVER COMMAND |
| nr | TS 38.331 (clause 6.2.2) | RRCReconfiguration |
| utra | TS 25.331 (clause 10.2.16a) | Handover TO UTRAN command |

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: E-UTRAN to UE

Paging message

-- ASN1START

Paging ::=					SEQUENCE {

pagingRecordList				PagingRecordList					OPTIONAL,	-- Need ON

systemInfoModification			ENUMERATED {true}					OPTIONAL,	-- Need ON

etws-Indication					ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v890-IEs						OPTIONAL

}

Paging-v890-IEs ::=			SEQUENCE {

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			Paging-v920-IEs						OPTIONAL

}

Paging-v920-IEs ::=			SEQUENCE {

cmas-Indication-r9				ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1130-IEs					OPTIONAL

}

Paging-v1130-IEs ::=			SEQUENCE {

eab-ParamModification-r11		ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1310-IEs					OPTIONAL

}

Paging-v1310-IEs ::=			SEQUENCE {

redistributionIndication-r13	ENUMERATED {true}					OPTIONAL,	-- Need ON

systemInfoModification-eDRX-r13	ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1530-IEs					OPTIONAL

}

Paging-v1530-IEs ::=			SEQUENCE {

accessType						ENUMERATED {non3GPP}				OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1610-IEs					OPTIONAL

}

Paging-v1610-IEs ::=			SEQUENCE {

pagingRecordList-v1610			PagingRecordList-v1610				OPTIONAL,	-- Need ON

uac-ParamModification-r16		ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

PagingRecordList ::=				SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord

PagingRecordList-v1610 ::=			SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-v1610

PagingRecord ::=					SEQUENCE {

ue-Identity							PagingUE-Identity,

cn-Domain							ENUMERATED	{ps, cs},

...

}

PagingRecord-v1610 ::=				SEQUENCE {

accessType-r16						ENUMERATED {non3GPP}			OPTIONAL,		-- Need ON

mt-EDT-r16							ENUMERATED {true}				OPTIONAL		-- Need ON

}

PagingUE-Identity ::=				CHOICE {

s-TMSI								S-TMSI,

imsi								IMSI,

...,

ng-5G-S-TMSI-r15					NG-5G-S-TMSI-r15,

fullI-RNTI-r15						I-RNTI-r15

}

IMSI ::=							SEQUENCE (SIZE (6..21)) OF IMSI-Digit

IMSI-Digit ::=						INTEGER (0..9)

-- ASN1STOP

Editor’s Note: Agreement: System information update notification procedure is not used to inform TAC updates, at least for TAC additions (FFS removals).

| Paging field descriptions |
| --- |
| accessType It indicates whether Paging is originated due to the PDU sessions from the non-3GPP access when E-UTRA is connected to 5GC. E-UTRAN does not include both accessType (i.e., without suffix) and accessType-r16 in a single paging message. |
| cmas-Indication If present: indication of a CMAS notification. |
| cn-Domain Indicates the origin of paging. |
| eab-ParamModification If present: indication of an EAB parameters (SIB14) modification. |
| etws-Indication If present: indication of an ETWS primary notification and/ or ETWS secondary notification. |
| imsi The International Mobile Subscriber Identity, a globally unique permanent subscriber identity, see TS 23.003 [27]. The first element contains the first IMSI digit, the second element contains the second IMSI digit and so on. |
| mt-EDT Indication of mobile terminating EDT. |
| pagingRecordList If E-UTRAN includes pagingRecordList-v1610, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix). |
| redistributionIndication If present: indication to trigger E-UTRAN inter-frequency redistribution procedure as specified in TS 36.304 [4], clause 5.2.4.10. |
| systemInfoModification If present: indication of a BCCH modification other than SIB10, SIB11, SIB12, SIB14 and SIBXX. This indication does not apply to UEs using eDRX cycle longer than the BCCH modification period. |
| systemInfoModification-eDRX If present: indication of a BCCH modification other than SIB10, SIB11, SIB12, SIB14 and SIBXX. This indication applies only to UEs using eDRX cycle longer than the BCCH modification period. |
| uac-ParamModification If present: indication of UAC parameters (SIB25) modification. |  |
| ue-Identity Provides the NAS identity of the UE that is being paged. The IMSI is not applicable for E-UTRA/5GC. |

#### –	ProximityIndication

The ProximityIndication message is used to indicate that the UE is entering or leaving the proximity of one or more CSG member cell(s).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ProximityIndication message

-- ASN1START

ProximityIndication-r9 ::= SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

proximityIndication-r9				ProximityIndication-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

ProximityIndication-r9-IEs ::= SEQUENCE {

type-r9								ENUMERATED {entering, leaving},

carrierFreq-r9						CHOICE {

eutra-r9							ARFCN-ValueEUTRA,

utra-r9								ARFCN-ValueUTRA,

...,

eutra2-v9e0							ARFCN-ValueEUTRA-v9e0

},

nonCriticalExtension				ProximityIndication-v930-IEs							OPTIONAL

}

ProximityIndication-v930-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

ProximityIndication field descriptions

carrierFreq Indicates the RAT and frequency of the CSG member cell(s), for which the proximity indication is sent. For E-UTRA and UTRA frequencies, the UE shall set the ARFCN according to a band it previously considered suitable for accessing (one of) the CSG member cell(s), for which the proximity indication is sent.

type Used to indicate whether the UE is entering or leaving the proximity of CSG member cell(s).

#### –	PURConfigurationRequest

The PURConfigurationRequest message is used by BL UE or UE in CE to indicate to the E-UTRAN that the UE is interested to be configured with PUR and provide PUR related information to E-UTRAN.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

PURConfigurationRequest message

-- ASN1START

PURConfigurationRequest-r16 ::=		SEQUENCE {

criticalExtensions					CHOICE {

purConfigurationRequest				PURConfigurationRequest-r16-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

PURConfigurationRequest-r16-IEs ::=	SEQUENCE {

pur-ConfigRequest-r16				CHOICE {

pur-ReleaseRequest					NULL,

pur-SetupRequest					SEQUENCE {

requestedNumOccasions-r16			ENUMERATED {one, infinite},

requestedPeriodicityAndOffset-r16	PUR-PeriodicityAndOffset-r16 	OPTIONAL,

requestedTBS-r16					ENUMERATED {b328, b344, b376, b392, b408,

b424, b440, b456, b472, b488, b504, b536,

b568, b584, b616, b648, b680, b712, b744,

b776, b808, b840, b872, b904, b936, b968,

b1000, b1032, b1064, b1096, b1128, b1160,

b1192, b1224, b1256, b1288, b1320, b1352,

b1384, b1416, b1480, b1544, b1608, b1672,

b1736, b1800, b1864, b1928, b1992, b2024,

b2088, b2152, b2216, b2280, b2344, b2408,

b2472, b2536, b2600, b2664, b2728, b2792,

b2856, b2984},

rrc-ACK-r16							ENUMERATED {true}			OPTIONAL

}

}																		OPTIONAL,

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

PURConfigurationRequest field descriptions

requestedNumOccasions Indicates the requested number of PUR grant occasions. Value one corresponds to one occasion and value infinite corresponds to infinite occasions.

requestedPeriodicityAndOffset Indicates the requested periodicity for the PUR occasions and time offset until the first PUR occasion.

requestedTBS Indicates the requested TBS for the PUR. b328 corresponds to 328 bits, b344 corresponds to 344 bits and so on. The maximum requested TBS is limited to the UL TBS size supported by the UE.

rrc-ACK Indicates RRC response message is preferred by the UE for acknowledging the reception of a transmission using PUR.

#### –	RNReconfiguration

The RNReconfiguration is a command to modify the RN subframe configuration and/or to convey changed system information.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to RN

RNReconfiguration message

-- ASN1START

RNReconfiguration-r10 ::=		SEQUENCE {

rrc-TransactionIdentifier		RRC-TransactionIdentifier,

criticalExtensions				CHOICE {

c1								CHOICE {

rnReconfiguration-r10		RNReconfiguration-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture		SEQUENCE {}

}

}

RNReconfiguration-r10-IEs ::=		SEQUENCE {

rn-SystemInfo-r10					RN-SystemInfo-r10				OPTIONAL,	-- Need ON

rn-SubframeConfig-r10				RN-SubframeConfig-r10			OPTIONAL,	-- Need ON

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

RN-SystemInfo-r10 ::=			SEQUENCE {

systemInformationBlockType1-r10		OCTET STRING (CONTAINING SystemInformationBlockType1)	OPTIONAL,	-- Need ON

systemInformationBlockType2-r10		SystemInformationBlockType2		OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

#### –	RNReconfigurationComplete

The RNReconfigurationComplete message is used to confirm the successful completion of an RN reconfiguration.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: RN to E-UTRAN

RNReconfigurationComplete message

-- ASN1START

RNReconfigurationComplete-r10 ::=		SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

c1										CHOICE{

rnReconfigurationComplete-r10			RNReconfigurationComplete-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

RNReconfigurationComplete-r10-IEs ::=	SEQUENCE {

lateNonCriticalExtension				OCTET STRING				OPTIONAL,

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

-- ASN1STOP

#### –	RRCConnectionReconfiguration

The RRCConnectionReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, conditional reconfigurations (conditional handover), radio resource configuration (including RBs, MAC main configuration and physical channel configuration) including any associated dedicated NAS information and security configuration.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionReconfiguration message

-- ASN1START

RRCConnectionReconfiguration ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcConnectionReconfiguration-r8		RRCConnectionReconfiguration-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfiguration-r8-IEs ::= SEQUENCE {

measConfig							MeasConfig						OPTIONAL,	-- Need ON

mobilityControlInfo					MobilityControlInfo				OPTIONAL,	-- Cond HO

dedicatedInfoNASList				SEQUENCE (SIZE(1..maxDRB)) OF

DedicatedInfoNAS			OPTIONAL,	-- Cond nonHO

radioResourceConfigDedicated		RadioResourceConfigDedicated	OPTIONAL, -- Cond HO-toEUTRA

securityConfigHO					SecurityConfigHO				OPTIONAL,	-- Cond HO-toEPC

nonCriticalExtension				RRCConnectionReconfiguration-v890-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v890-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING RRCConnectionReconfiguration-v8m0-IEs)	OPTIONAL,

nonCriticalExtension				RRCConnectionReconfiguration-v920-IEs	OPTIONAL

}

-- Late non-critical extensions:

RRCConnectionReconfiguration-v8m0-IEs ::= SEQUENCE {

-- Following field is only for pre REL-10 late non-critical extensions

lateNonCriticalExtension			OCTET STRING								OPTIONAL,

nonCriticalExtension				RRCConnectionReconfiguration-v10i0-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v10i0-IEs ::= SEQUENCE {

antennaInfoDedicatedPCell-v10i0	AntennaInfoDedicated-v10i0		OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v10l0-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v10l0-IEs ::= SEQUENCE {

mobilityControlInfo-v10l0			MobilityControlInfo-v10l0			OPTIONAL,

sCellToAddModList-v10l0			SCellToAddModList-v10l0			OPTIONAL,	-- Need ON

-- Following field is only for late non-critical extensions from REL-10 to REL-11

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				RRCConnectionReconfiguration-v12f0-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v12f0-IEs ::= SEQUENCE {

scg-Configuration-v12f0			SCG-Configuration-v12f0		OPTIONAL,	-- Cond nonFullConfig

-- Following field is only for late non-critical extensions from REL-12

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				RRCConnectionReconfiguration-v1370-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v1370-IEs ::= SEQUENCE {

radioResourceConfigDedicated-v1370	RadioResourceConfigDedicated-v1370	OPTIONAL, -- Need ON

sCellToAddModListExt-v1370			SCellToAddModListExt-v1370	OPTIONAL,	-- Need ON

nonCriticalExtension					RRCConnectionReconfiguration-v13c0-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v13c0-IEs ::= SEQUENCE {

radioResourceConfigDedicated-v13c0	RadioResourceConfigDedicated-v13c0	OPTIONAL, -- Need ON

sCellToAddModList-v13c0				SCellToAddModList-v13c0		OPTIONAL,	-- Need ON

sCellToAddModListExt-v13c0			SCellToAddModListExt-v13c0	OPTIONAL,	-- Need ON

scg-Configuration-v13c0				SCG-Configuration-v13c0		OPTIONAL,	-- Need ON

-- Following field is only for late non-critical extensions from REL-13 onwards

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

-- Regular non-critical extensions:

RRCConnectionReconfiguration-v920-IEs ::= SEQUENCE {

otherConfig-r9						OtherConfig-r9				OPTIONAL,	-- Need ON

fullConfig-r9						ENUMERATED {true}			OPTIONAL,	-- Cond HO-Reestab

nonCriticalExtension				RRCConnectionReconfiguration-v1020-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v1020-IEs ::= SEQUENCE {

sCellToReleaseList-r10			SCellToReleaseList-r10			OPTIONAL,	-- Need ON

sCellToAddModList-r10				SCellToAddModList-r10				OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v1130-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v1130-IEs ::= SEQUENCE {

systemInformationBlockType1Dedicated-r11	OCTET STRING (CONTAINING SystemInformationBlockType1)																			OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v1250-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v1250-IEs ::= SEQUENCE {

wlan-OffloadInfo-r12				CHOICE {

release								NULL,

setup									SEQUENCE {

wlan-OffloadConfigDedicated-r12		WLAN-OffloadConfig-r12,

t350-r12								ENUMERATED {min5, min10, min20, min30, min60,

min120, min180, spare1}	OPTIONAL	-- Need OR

}

}																	OPTIONAL,		-- Need ON

scg-Configuration-r12				SCG-Configuration-r12		OPTIONAL,	-- Cond nonFullConfig

sl-SyncTxControl-r12				SL-SyncTxControl-r12			OPTIONAL,	-- Need ON

sl-DiscConfig-r12					SL-DiscConfig-r12				OPTIONAL,	-- Need ON

sl-CommConfig-r12					SL-CommConfig-r12				OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v1310-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v1310-IEs ::= SEQUENCE {

sCellToReleaseListExt-r13			SCellToReleaseListExt-r13		OPTIONAL,	-- Need ON

sCellToAddModListExt-r13			SCellToAddModListExt-r13		OPTIONAL,	-- Need ON

lwa-Configuration-r13				LWA-Configuration-r13			OPTIONAL,	-- Need ON

lwip-Configuration-r13				LWIP-Configuration-r13			OPTIONAL,	-- Need ON

rclwi-Configuration-r13				RCLWI-Configuration-r13			OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v1430-IEs						OPTIONAL

}

RRCConnectionReconfiguration-v1430-IEs ::= SEQUENCE {

sl-V2X-ConfigDedicated-r14		SL-V2X-ConfigDedicated-r14		OPTIONAL,	-- Need ON

sCellToAddModListExt-v1430		SCellToAddModListExt-v1430		OPTIONAL,	-- Need ON

perCC-GapIndicationRequest-r14	ENUMERATED{true}					OPTIONAL,	-- Need ON

systemInformationBlockType2Dedicated-r14	OCTET STRING (CONTAINING SystemInformationBlockType2)											OPTIONAL,	-- Cond nonHO

nonCriticalExtension			RRCConnectionReconfiguration-v1510-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v1510-IEs ::= SEQUENCE {

nr-Config-r15					CHOICE {

release							NULL,

setup							SEQUENCE {

endc-ReleaseAndAdd-r15	BOOLEAN,

nr-SecondaryCellGroupConfig-r15	OCTET STRING				OPTIONAL,	-- Need ON

p-MaxEUTRA-r15					P-Max						OPTIONAL	-- Need ON

}

}																	OPTIONAL,	-- Need ON

sk-Counter-r15					INTEGER (0.. 65535)					OPTIONAL,	-- Need ON

nr-RadioBearerConfig1-r15		OCTET STRING						OPTIONAL,	-- Need ON

nr-RadioBearerConfig2-r15		OCTET STRING						OPTIONAL,	-- Need ON

tdm-PatternConfig-r15			TDM-PatternConfig-r15			OPTIONAL,	-- Cond FDD-PCell

nonCriticalExtension			RRCConnectionReconfiguration-v1530-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v1530-IEs ::= SEQUENCE {

securityConfigHO-v1530				SecurityConfigHO-v1530			OPTIONAL,	-- Cond HO-5GC

sCellGroupToReleaseList-r15		SCellGroupToReleaseList-r15			OPTIONAL,	-- Need ON

sCellGroupToAddModList-r15		SCellGroupToAddModList-r15			OPTIONAL,	-- Need ON

dedicatedInfoNASList-r15		SEQUENCE (SIZE(1..maxDRB-r15)) OF

DedicatedInfoNAS			OPTIONAL,	-- Cond nonHO

p-MaxUE-FR1-r15					P-Max								OPTIONAL,	-- Need OR

smtc-r15						MTC-SSB-NR-r15						OPTIONAL,	-- Need OP

nonCriticalExtension			RRCConnectionReconfiguration-v1610-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v1610-IEs ::= SEQUENCE {

conditionalReconfiguration-r16			ConditionalReconfiguration-r16	OPTIONAL, -- Need ON

daps-SourceRelease-r16					ENUMERATED{true}				OPTIONAL, -- Need ON

tdm-PatternConfig2-r16						TDM-PatternConfig-r15			OPTIONAL, -- Need ON

sl-ConfigDedicatedForNR-r16					OCTET STRING					OPTIONAL, -- Need OR

sl-SSB-PriorityEUTRA-r16					INTEGER (1..8)					OPTIONAL, -- Need OR

nonCriticalExtension						SEQUENCE {}						OPTIONAL

}

SL-SyncTxControl-r12 ::=			SEQUENCE {

networkControlledSyncTx-r12				ENUMERATED {on, off}		OPTIONAL	-- Need OP

}

PSCellToAddMod-r12 ::=				SEQUENCE {

sCellIndex-r12						SCellIndex-r10,

cellIdentification-r12				SEQUENCE {

physCellId-r12						PhysCellId,

dl-CarrierFreq-r12					ARFCN-ValueEUTRA-r9

}																	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigCommonPSCell-r12		RadioResourceConfigCommonPSCell-r12	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigDedicatedPSCell-r12	RadioResourceConfigDedicatedPSCell-r12	OPTIONAL,	-- Cond SCellAdd2

...,

[[	antennaInfoDedicatedPSCell-v1280		AntennaInfoDedicated-v10i0	OPTIONAL	-- Need ON

]],

[[	sCellIndex-r13					SCellIndex-r13	OPTIONAL		-- Need ON

]],

[[	radioResourceConfigDedicatedPSCell-v1370	RadioResourceConfigDedicatedPSCell-v1370	OPTIONAL	-- Need ON

]],

[[	radioResourceConfigDedicatedPSCell-v13c0	RadioResourceConfigDedicatedPSCell-v13c0	OPTIONAL	-- Need ON

]]

}

PSCellToAddMod-v12f0 ::=				SEQUENCE {

radioResourceConfigCommonPSCell-r12		RadioResourceConfigCommonPSCell-v12f0	OPTIONAL

}

PSCellToAddMod-v1440 ::=				SEQUENCE {

radioResourceConfigCommonPSCell-r14		RadioResourceConfigCommonPSCell-v1440	OPTIONAL

}

PowerCoordinationInfo-r12 ::= SEQUENCE {

p-MeNB-r12							INTEGER (1..16),

p-SeNB-r12							INTEGER (1..16),

powerControlMode-r12				INTEGER (1..2)

}

SCellToAddModList-r10 ::=		SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellToAddMod-r10

SCellToAddModList-v10l0 ::=		SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellToAddMod-v10l0

SCellToAddModList-v13c0 ::=		SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellToAddMod-v13c0

SCellToAddModList-r16 ::=		SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellToAddMod-r16

SCellToAddModListExt-r13 ::=	SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellToAddModExt-r13

SCellToAddModListExt-v1370 ::=	SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellToAddModExt-v1370

SCellToAddModListExt-v13c0 ::=	SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellToAddMod-v13c0

SCellToAddModListExt-v1430 ::=	SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellToAddModExt-v1430

SCellGroupToAddModList-r15 ::=	SEQUENCE (SIZE (1..maxSCellGroups-r15)) OF SCellGroupToAddMod-r15

SCellToAddMod-r10 ::=			SEQUENCE {

sCellIndex-r10						SCellIndex-r10,

cellIdentification-r10				SEQUENCE {

physCellId-r10						PhysCellId,

dl-CarrierFreq-r10					ARFCN-ValueEUTRA

}																OPTIONAL,	-- Cond SCellAdd

radioResourceConfigCommonSCell-r10		RadioResourceConfigCommonSCell-r10	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigDedicatedSCell-r10	RadioResourceConfigDedicatedSCell-r10	OPTIONAL,	-- Cond SCellAdd2

...,

[[	dl-CarrierFreq-v1090				ARFCN-ValueEUTRA-v9e0	OPTIONAL	-- Cond EARFCN-max

]],

[[	antennaInfoDedicatedSCell-v10i0		AntennaInfoDedicated-v10i0	OPTIONAL	-- Need ON

]],

[[	srs-SwitchFromServCellIndex-r14		INTEGER (0.. 31) OPTIONAL	-- Need ON

]],

[[	sCellState-r15						ENUMERATED {activated, dormant}	OPTIONAL	-- Need ON

]]

}

SCellToAddMod-v10l0 ::=			SEQUENCE {

radioResourceConfigCommonSCell-v10l0		RadioResourceConfigCommonSCell-v10l0	OPTIONAL

}

SCellToAddMod-v13c0 ::=			SEQUENCE {

radioResourceConfigDedicatedSCell-v13c0	RadioResourceConfigDedicatedSCell-v13c0	OPTIONAL

}

SCellToAddMod-r16 ::=			SEQUENCE {

sCellIndex-r16						SCellIndex-r13,

cellIdentification-r16				SEQUENCE {

physCellId-r16						PhysCellId,

dl-CarrierFreq-r16					ARFCN-ValueEUTRA-r9

}																OPTIONAL,	-- Cond SCellAdd

radioResourceConfigCommonSCell-r16		RadioResourceConfigCommonSCell-r10	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigDedicatedSCell-r16	RadioResourceConfigDedicatedSCell-r10	OPTIONAL,	-- Cond SCellAdd2

antennaInfoDedicatedSCell-r16		AntennaInfoDedicated-v10i0	OPTIONAL,	-- Need ON

srs-SwitchFromServCellIndex-r16			INTEGER (0.. 31) OPTIONAL,	-- Need ON

sCellState-r16							ENUMERATED {activated, dormant}	OPTIONAL, 	-- Need ON

...

}

SCellToAddModExt-r13 ::=			SEQUENCE {

sCellIndex-r13						SCellIndex-r13,

cellIdentification-r13				SEQUENCE {

physCellId-r13						PhysCellId,

dl-CarrierFreq-r13					ARFCN-ValueEUTRA-r9

}																OPTIONAL,	-- Cond SCellAdd

radioResourceConfigCommonSCell-r13		RadioResourceConfigCommonSCell-r10	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigDedicatedSCell-r13	RadioResourceConfigDedicatedSCell-r10	OPTIONAL,	-- Cond SCellAdd2

antennaInfoDedicatedSCell-r13			AntennaInfoDedicated-v10i0		OPTIONAL	-- Need ON

}

SCellToAddModExt-v1370 ::=			SEQUENCE {

radioResourceConfigCommonSCell-v1370		RadioResourceConfigCommonSCell-v10l0	OPTIONAL

}

SCellToAddModExt-v1430 ::=			SEQUENCE {

srs-SwitchFromServCellIndex-r14			INTEGER (0.. 31)			OPTIONAL,	-- Need ON

...,

[[	sCellState-r15					ENUMERATED {activated, dormant}		OPTIONAL	-- Need ON

]]

}

SCellGroupToAddMod-r15 ::=			SEQUENCE {

sCellGroupIndex-r15					SCellGroupIndex-r15,

sCellConfigCommon-r15				SCellConfigCommon-r15			OPTIONAL,	-- Need ON

sCellToReleaseList-r15				SCellToReleaseListExt-r13		OPTIONAL,	-- Need ON

sCellToAddModList-r15				SCellToAddModListExt-r13		OPTIONAL	-- Need ON

}

SCellToReleaseList-r10 ::=			SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellIndex-r10

SCellToReleaseListExt-r13 ::=			SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellIndex-r13

SCellGroupToReleaseList-r15 ::=			SEQUENCE (SIZE (1..maxSCellGroups-r15)) OF SCellGroupIndex-r15

SCellGroupIndex-r15 ::=			INTEGER (1..maxSCellGroups-r15)

SCellConfigCommon-r15 ::= SEQUENCE {

radioResourceConfigCommonSCell-r15		RadioResourceConfigCommonSCell-r10	OPTIONAL,	-- Need ON

radioResourceConfigDedicatedSCell-r15	RadioResourceConfigDedicatedSCell-r10	OPTIONAL,-- Need ON

antennaInfoDedicatedSCell-r15			AntennaInfoDedicated-v10i0		OPTIONAL	-- Need ON

}

SCG-Configuration-r12 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

scg-ConfigPartMCG-r12				SEQUENCE {

scg-Counter-r12						INTEGER (0.. 65535)			OPTIONAL,	-- Need ON

powerCoordinationInfo-r12			PowerCoordinationInfo-r12	OPTIONAL,	-- Need ON

...

}																OPTIONAL,	-- Need ON

scg-ConfigPartSCG-r12				SCG-ConfigPartSCG-r12		OPTIONAL	-- Need ON

}

}

SCG-Configuration-v12f0 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

scg-ConfigPartSCG-v12f0				SCG-ConfigPartSCG-v12f0		OPTIONAL	-- Need ON

}

}

SCG-Configuration-v13c0 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

scg-ConfigPartSCG-v13c0				SCG-ConfigPartSCG-v13c0		OPTIONAL	-- Need ON

}

}

SCG-ConfigPartSCG-r12 ::=			SEQUENCE {

radioResourceConfigDedicatedSCG-r12	RadioResourceConfigDedicatedSCG-r12	OPTIONAL,	-- Need ON

sCellToReleaseListSCG-r12			SCellToReleaseList-r10		OPTIONAL,	-- Need ON

pSCellToAddMod-r12					PSCellToAddMod-r12			OPTIONAL,	-- Need ON

sCellToAddModListSCG-r12			SCellToAddModList-r10		OPTIONAL,	-- Need ON

mobilityControlInfoSCG-r12			MobilityControlInfoSCG-r12	OPTIONAL,	-- Need ON

...,

[[

sCellToReleaseListSCG-Ext-r13			SCellToReleaseListExt-r13		OPTIONAL,	-- Need ON

sCellToAddModListSCG-Ext-r13			SCellToAddModListExt-r13		OPTIONAL	-- Need ON

]],

[[

sCellToAddModListSCG-Ext-v1370		SCellToAddModListExt-v1370	OPTIONAL	-- Need ON

]],

[[

pSCellToAddMod-v1440				PSCellToAddMod-v1440		OPTIONAL	-- Need ON

]],

[[	sCellGroupToReleaseListSCG-r15	SCellGroupToReleaseList-r15	OPTIONAL,	-- Need ON

sCellGroupToAddModListSCG-r15	SCellGroupToAddModList-r15	OPTIONAL	-- Need ON

]],

[[	-- NE-DC addition for setup/ modification and release SN configured measurements

measConfigSN-r15				MeasConfig						OPTIONAL,	-- Need ON

-- NE-DC additions concerning DRBs/ SRBs are within RadioResourceConfigDedicatedSCG

tdm-PatternConfigNE-DC-r15		TDM-PatternConfig-r15			OPTIONAL	-- Cond FDD-PSCell

]],

[[	p-MaxEUTRA-r15					P-Max							OPTIONAL	-- Need ON

]]

}

SCG-ConfigPartSCG-v12f0 ::=			SEQUENCE {

pSCellToAddMod-v12f0				PSCellToAddMod-v12f0		OPTIONAL,	-- Need ON

sCellToAddModListSCG-v12f0			SCellToAddModList-v10l0		OPTIONAL	-- Need ON

}

SCG-ConfigPartSCG-v13c0 ::=			SEQUENCE {

sCellToAddModListSCG-v13c0			SCellToAddModList-v13c0		OPTIONAL,	-- Need ON

sCellToAddModListSCG-Ext-v13c0		SCellToAddModListExt-v13c0	OPTIONAL	-- Need ON

}

SecurityConfigHO ::=				SEQUENCE {

handoverType						CHOICE {

intraLTE							SEQUENCE {

securityAlgorithmConfig				SecurityAlgorithmConfig		OPTIONAL,	-- Cond fullConfig

keyChangeIndicator					BOOLEAN,

nextHopChainingCount				NextHopChainingCount

},

interRAT							SEQUENCE {

securityAlgorithmConfig				SecurityAlgorithmConfig,

nas-SecurityParamToEUTRA			OCTET STRING (SIZE(6))

}

},

...

}

SecurityConfigHO-v1530 ::=		SEQUENCE {

handoverType-v1530				CHOICE {

intra5GC-r15						SEQUENCE {

securityAlgorithmConfig-r15			SecurityAlgorithmConfig		OPTIONAL,	-- Cond HO-toEUTRA

keyChangeIndicator-r15				BOOLEAN,

nextHopChainingCount-r15			NextHopChainingCount,

nas-Container-r15					OCTET STRING	OPTIONAL	-- Need ON

},

fivegc-ToEPC-r15					SEQUENCE {

securityAlgorithmConfig-r15			SecurityAlgorithmConfig,

nextHopChainingCount-r15			NextHopChainingCount

},

epc-To5GC-r15					SEQUENCE {

securityAlgorithmConfig-r15		SecurityAlgorithmConfig,

nas-Container-r15				OCTET STRING

}

},

...

}

-- ASN1STOP

RRCConnectionReconfiguration field descriptions

conditionalReconfiguration This field is used to configure the UE with a conditional reconfiguration. The reconfiguration is applied when the execution condition(s) is fulfilled. The field is absent if daps-HO is configured for any DRB or if MobilityControlInfo is included in the RRCConnectionReconfiguration message. The conditionalReconfiguration is not configured in the RRCConnectionReconfiguration message included in a conditionalReconfiguration.

daps-SourceRelease A one-shot field that indicates that the UE shall release the resources associated with source PCell at a DAPS HO, including reconfiguration of the PDCP entity to release DAPS.

dedicatedInfoNASList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list. If dedicatedInfoNASList-r15 is present, UE shall ignore the dedicatedInfoNASList (without suffix).

endc-ReleaseAndAdd A one-shot field indicating whether the UE simultaneously releases and adds all the NR SCG related configuration within nr-Config, i.e. the configuration set by the NR RRCReconfiguration message (e.g. secondaryCellGroup, SRB3 and measConfig).

fullConfig Indicates the full configuration option is applicable for the RRC Connection Reconfiguration message for intra-system intra-RAT handover. For inter-RAT handover from NR to E-UTRA, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent when the RRCConnectionReconfiguration message is generated by the E-UTRA SCG.

keyChangeIndicator If UE is connected to EPC, true is used only in an intra-cell handover when a KeNB key is derived from a KASME key taken into use through the latest successful NAS SMC procedure, as described in TS 33.401 [32] for KeNB re-keying. false is used in an intra-LTE handover when the new KeNB key is obtained from the current KeNB key or from the NH as described in TS 33.401 [32]. If UE is connected to 5GC, with keyChangeIndicator-r15, true is used in an intra-cell handover when a KeNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, as described in TS 33.501 [86] for KeNB re-keying. False is used for intra-system handover when the new KeNB key is obtained from the current KeNB key or from the NH as described in TS 33.501 [86]. True is also used in NG based handover procedure with KAMF change, when a KeNB key is derived from the new KAMF key as described in TS 33.501 [86].

lwa-Configuration This field is used to provide parameters for LWA configuration. E-UTRAN does not simultaneously configure LWA with DC, LWIP or RCLWI for a UE.

lwip-Configuration This field is used to provide parameters for LWIP configuration. E-UTRAN does not simultaneously configure LWIP with DC, LWA or RCLWI for a UE.

measConfig Measurements that E-UTRAN may configure when the UE is not configured with NE-DC.

measConfigSN Measurements that E-UTRAN may configure when the UE is configured with NE-DC and for which reports are carried within an NR RRC message.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although, if included, it affects activation of AS- security after handover within E-UTRA/5GC. The content is defined in TS 24.501 [95]. In case of NG based handover, the content of nas-Container is. the Intra N1 mode NAS transparent container IE. In case of inter-system handover to from 5GS to EPS, the content of NAS-Container is. the S1 mode to N1 mode NAS transparent container IE.

nas-securityParamToEUTRA This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although, if included, it affects activation of AS- security after inter-RAT handover to E-UTRA/EPC or inter-system handover to E-UTRA/EPC. The content is defined in TS 24.301 [35]. This field is not used for handover from 5GC.

networkControlledSyncTx This field indicates whether the UE shall transmit synchronisation information (i.e. become synchronisation source). Value On indicates the UE to transmit synchronisation information while value Off indicates the UE to not transmit such information.

nextHopChainingCount Parameter NCC: See TS 33.401 [32] if UE is connected to EPC, else see 33.501 [86] if UE is connected to 5GC.

nr-Config Includes the NR related configurations. This field is used to configure (NG)EN-DC configuration, possibly in conjunction with fields sk-Counter and nr-RadioBearerConfig1/ 2. NOTE 1.

nr-RadioBearerConfig1, nr-RadioBearerConfig2 Includes the NR RadioBearerConfig IE as specified in TS 38.331 [82]. The field includes the configuration of RBs configured with NR PDCP.

nr-SecondaryCellGroupConfig Includes the NR RRCReconfiguration message as specified in TS 38.331 [82]. In this version of the specification, the NR RRC message only includes fields secondaryCellGroup, conditionalReconfiguration, otherConfig, bap-Config, iab-IP-AddressConfigurationList and/ or measConfig. If nr-SecondaryCellGroupConfig is configured, the network always includes this field upon MN handover to initiate an NR SCG reconfiguration with sync and key change.

perCC-GapIndicationRequest Indicates that UE shall include perCC-GapIndicationList and numFreqEffective in the RRCConnectionReconfigurationComplete message. numFreqEffectiveReduced may also be included if frequencies are configured for reduced measurement performance.

p-MaxEUTRA Indicates the maximum power available for LTE.

p-MaxUE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited on cell- or cell-group level. The field is optionally present, if (NG)EN-DC (nr-Config-r15) has been configured. It is absent otherwise.

p-MeNB Indicates the guaranteed power for the MeNB, as specified in TS 36.213 [23]. The value N corresponds to N-1 in TS 36.213 [23].

powerControlMode Indicates the power control mode used in DC. Value 1 corresponds to DC power control mode 1 and value 2 indicates DC power control mode 2, as specified in TS 36.213 [23].

p-SeNB Indicates the guaranteed power for the SeNB as specified in TS 36.213 [23], Table 5.1.4.2-1. The value N corresponds to N-1 in TS 36.213 [23].

rclwi-Configuration WLAN traffic steering command as specified in 5.6.16.2. E-UTRAN does not simultaneously configure RCLWI with DC, LWA or LWIP for a UE.

sCellConfigCommon Indicates the common configuration for the SCell group.

sCellGroupIndex Indicates the identity of SCell groups for which a common configuration is provided.

sCellIndex The sCellIndex is unique within the scope of the UE. In case of DC, an SCG cell can not use the same value as used for an MCG cell. For pSCellToAddMod, if sCellIndex-r13 is present the UE shall ignore sCellIndex-r12.

sCellGroupToAddModList, sCellGroupToAddModListSCG Indicates the SCell group to be added or modified. E-UTRAN only configures at most 4 SCell groups per UE over all cell groups. SCell groups can only be configured for LTE SCells, and all SCells in an SCell group must belong to the same cell group.

sCellGroupToReleaseList Indicates the SCell group to be released.

sCellState A one-shot field that indicates whether the SCell shall be considered to be in activated or dormant state upon SCell configuration.

sCellToAddModList, sCellToAddModListExt Indicates the SCell to be added or modified. E-UTRAN uses field sCellToAddModList-r10 to add or modify SCells (with sCellIndex-r10) for a UE that does not support carrier aggregation with more than 5 component carriers. If E-UTRAN includes sCellToAddModListExt-v1430 it includes the same number of entries, and listed in the same order, as in sCellToAddModListExt-r13. If E-UTRAN includes sCellToAddModList-v10l0 it includes the same number of entries, and listed in the same order, as in sCellToAddModList-r10. If E-UTRAN includes sCellToAddModListExt-v1370 it includes the same number of entries, and listed in the same order, as in sCellToAddModListExt-r13. If E-UTRAN includes sCellToAddModListExt-v13c0 it includes the same number of entries, and listed in the same order, as in sCellToAddModListExt-r13.

sCellToAddModListSCG, sCellToAddModListSCG-Ext Indicates the SCG cell to be added or modified. The field is used for SCG cells other than the PSCell (which is added/ modified by field pSCellToAddMod). E-UTRAN uses field sCellToAddModListSCG-r12 to add or modify SCells (with sCellIndex-r10) for a UE that does not support carrier aggregation with more than 5 component carriers. If E-UTRAN includes sCellToAddModListSCG-v10l0 it includes the same number of entries, and listed in the same order, as in sCellToAddModListSCG-r12. If E-UTRAN includes sCellToAddModListSCG-Ext-v1370 it includes the same number of entries, and listed in the same order, as in sCellToAddModListSCG-Ext-r13. If E-UTRAN includes sCellToAddModListSCG-Ext-v13c0 it includes the same number of entries, and listed in the same order, as in sCellToAddModListSCG-Ext-r13.

sCellToReleaseList, sCellToReleaseListExt Indicates the SCell to be released. E-UTRAN uses field sCellToReleaseList-r10 to release SCells for a UE that does not support carrier aggregation with more than 5 component carriers.

sCellToReleaseListSCG, sCellToReleaseListSCG-Ext Indicates the SCG cell to be released. The field is also used to release the PSCell e.g. upon change of PSCell, upon system information change for the PSCell. E-UTRAN uses field sCellToReleaseListSCG-r12 to release SCells for a UE that does not support carrier aggregation with more than 5 component carriers.

scg-Configuration Covers the SCG configuration as used in case of DC and NE-DC. When the UE is configured with NE-DC, E-UTRAN neither applies value release nor configures scg-ConfigPartMCG. When resuming a connection with NE-DC, this field is included, containing at least the mobilityControlInfoSCG.

scg-Counter A counter used upon initial configuration of SCG security as well as upon refresh of S-KeNB. E-UTRAN includes the field upon SCG change when one or more SCG DRBs are configured. Otherwise E-UTRAN does not include the field.

securityConfigHO This field contains the parameters required to update the security keys at handover. If E-UTRAN includes the securityConfigHO (i.e., without suffix), the choice intraLTE is used for handover within E-UTRA/EPC while the choice interRAT is used for handover from GERAN or UTRAN to E-UTRA/EPC. If E-UTRAN includes the securityConfigHO-v1530 (i.e., with suffix), the choice intra5GC is used for handover from NR or E-UTRA/5GC to E-UTRA/5GC while the choice fivegc-ToEPC is used for inter-system handover from NR or E-UTRA/5GC to E-UTRA/EPC and the choice epc-To5GC is used for inter-system handover from E-UTRA/EPC to E-UTRA/5GC.

sk-Counter A one-shot counter used upon initial configuration of S-KgNB as well as upon refresh of S-KgNB. E-UTRAN always provides this field either upon initial configuration of an NR SCG, or upon configuration of the first (SN terminated) RB using S-KgNB, whichever happens first.

sl-ConfigDedicatedForNR Container for providing the dedicated configurations for NR sidelink communication, the octet string contains the NR RRCReconfiguration message as specified in TS 38.331 [82]. In this version of the specification, the NR RRC message only includes fields related to NR sidelink communication, i.e. sl-ConfigDedicatedNR, measConfig and/or otherConfig. If the UE is configured by the current Pcell with sl-ScheduledConfig set to setup (i.e., NR sidelink communication mode 1), the network only includes sl-PrioritizationThres and sl-ConfiguredGrantConfig that only includes the configurations of configured sidelink grant Type 1 in the field sl-ScheduledConfig.

sl-SSB-PriorityEUTRA Indicates the priority of LTE PSSS/SSSS/PSBCH transmission and reception. NOTE 3.

sl-V2X-ConfigDedicated Indicates sidelink configuration for non-P2X related V2X sidelink communication as well as P2X related V2X sidelink communication.

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell addition and SN change. It is based on timing reference of EUTRA PCell. NOTE 2. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

srs-SwitchFromServCellIndex Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less cell. During SRS transmission on a PUSCH-less cell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH in the same CG to allow the PUSCH-less cell to transmit SRS. The PUSCH-less cell is always a TDD cell but the serving cell with PUSCH may be either a FDD or TDD cell.

systemInformationBlockType1Dedicated This field is used to transfer SystemInformationBlockType1 or SystemInformationBlockType1-BR to the UE.

systemInformationBlockType2Dedicated This field is used to transfer BR version of SystemInformationBlockType2 to BL UEs or UEs in CE or SystemInformationBlockType2 to non-BL UEs.

t350 Timer T350 as described in clause 7.3. Value minN corresponds to N minutes.

tdm-PatternConfig This field is used when power control or IMD issues require single UL transmission in (NG)EN-DC as specified in TS 38.101-3 [101] and TS 38.213 [88].

tdm-PatternConfig2 This field is used for dual UL transmission in EN-DC with LTE FDD PCell and for single UL transmission in EN-DC with LTE FDD/TDD PCell, as specified in TS 38.101-3 [101] and TS 38.213 [88]. The network sets at most one of tdm-PatternConfig and tdm-PatternConfig2 to setup. When this field is configured in EN-DC with LTE TDD PCell, it is not applicable if TDD configuration is sa0 or sa6 in SIB1.

tdm-PatternConfigNE-DC This field is used when power control or IMD issues require single UL transmission in NE-DC as specified in TS 38.101-3 [101] and TS 38.213 [88].

| Conditional presence | Explanation |
| --- | --- |
| EARFCN-max | The field is mandatory present if dl-CarrierFreq-r10 is included and set to maxEARFCN. Otherwise the field is not present. |
| FDD-PCell | This field is optionally present, need ON, for a FDD PCell if there is no SCell with configured uplink. Otherwise, the field is not present. |
| FDD-PSCell | This field is optionally present, need ON, for a FDD PSCell if there is no SCell with configured uplink. Otherwise, the field is not present. |
| fullConfig | This field is mandatory present for handover within E-UTRA when the fullConfig is included; otherwise it is optionally present, Need OP. |
| HO | The field is mandatory present in case of handover within E-UTRA or to E-UTRA and in a message contained in a NR DLInformationTransferMRDC message; otherwise the field is not present. The field is not present if source PCell resources after a DAPS handover have not been released. |
| HO-Reestab | The field is mandatory present in case of inter-system handover within E-UTRA or handover from NR to E-UTRA/EPC; it is optionally present, need ON, in case of intra-system handover within E-UTRA or upon the first reconfiguration after RRC connection re-establishment; or for intra-system handover from NR to E-UTRA, otherwise the field is not present. |
| HO-5GC | The field is mandatory present in case of handover within E-UTRA/5GC, handover to E-UTRA/5GC, handover from NR to E-UTRA/EPC, or handover from E-UTRA/5GC to E-UTRA/EPC, otherwise the field is not present. |
| HO-toEPC | The field is mandatory present in case of handover within E-UTRA/EPC or to E-UTRA/EPC, except handover from NR or E-UTRA/5GC, otherwise the field is not present. |
| HO-toEUTRA | The field is mandatory present in case of handover to E-UTRA or for reconfigurations when fullConfig is included; otherwise the field is optionally present, need ON. |
| nonFullConfig | The field is not present when the fullConfig is included or in case of handover to E-UTRA; otherwise it is optional present, need ON. |
| nonHO | The field is not present in case of handover within E-UTRA or to E-UTRA; otherwise it is optional present, need ON. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is not present. |
| SCellAdd2 | The field is mandatory present upon SCell addition; otherwise it is optionally present, need ON. |

NOTE 1:	Fields sk-Counter and nr-RadioBearerConfig1/ 2 are placed outside nr-Config, as these may be configured while the UE is not configured with (NG)EN-DC.

NOTE 2:	It is not specified whether the timing reference for the SMTC configuration is the source EUTRA PCell or the target EUTRA PCell in case the NR PSCell addition or SN change takes place simultaneously with handover. As a consequence, explicit SMTC configuration is only supported when the source EUTRA PCell and the target EUTRA PCell of the handover are SFN/subframe-synchronized.

NOTE 3:	For UEs in RRC_IDLE, RRC_INACTIVE or out-of coverage, and for the case that sl-SSB-PriorityEUTRA is absent, it is up to UE implementation to decide the priority of LTE PSSS/SSSS/PSBCH transmission and reception.

#### –	RRCConnectionReconfigurationComplete

The RRCConnectionReconfigurationComplete message is used to confirm the successful completion of an RRC connection reconfiguration.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

RRCConnectionReconfigurationComplete message

-- ASN1START

RRCConnectionReconfigurationComplete ::= SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcConnectionReconfigurationComplete-r8

RRCConnectionReconfigurationComplete-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfigurationComplete-r8-IEs ::= SEQUENCE {

nonCriticalExtension				RRCConnectionReconfigurationComplete-v8a0-IEs	OPTIONAL

}

RRCConnectionReconfigurationComplete-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				RRCConnectionReconfigurationComplete-v1020-IEs	OPTIONAL

}

RRCConnectionReconfigurationComplete-v1020-IEs ::= SEQUENCE {

rlf-InfoAvailable-r10				ENUMERATED {true}				OPTIONAL,

logMeasAvailable-r10				ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				RRCConnectionReconfigurationComplete-v1130-IEs	OPTIONAL

}

RRCConnectionReconfigurationComplete-v1130-IEs ::= SEQUENCE {

connEstFailInfoAvailable-r11		ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				RRCConnectionReconfigurationComplete-v1250-IEs	OPTIONAL

}

RRCConnectionReconfigurationComplete-v1250-IEs ::= SEQUENCE {

logMeasAvailableMBSFN-r12			ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				RRCConnectionReconfigurationComplete-v1430-IEs						OPTIONAL

}

RRCConnectionReconfigurationComplete-v1430-IEs ::= SEQUENCE {

perCC-GapIndicationList-r14			PerCC-GapIndicationList-r14		OPTIONAL,

numFreqEffective-r14				INTEGER (1..12)					OPTIONAL,

numFreqEffectiveReduced-r14			INTEGER (1..12)					OPTIONAL,

nonCriticalExtension				RRCConnectionReconfigurationComplete-v1510-IEs						OPTIONAL

}

RRCConnectionReconfigurationComplete-v1510-IEs ::= SEQUENCE {

scg-ConfigResponseNR-r15			OCTET STRING					OPTIONAL,

nonCriticalExtension				RRCConnectionReconfigurationComplete-v1530-IEs						OPTIONAL

}

RRCConnectionReconfigurationComplete-v1530-IEs ::= SEQUENCE {

logMeasAvailableBT-r15				ENUMERATED {true}				OPTIONAL,

logMeasAvailableWLAN-r15			ENUMERATED {true}				OPTIONAL,

flightPathInfoAvailable-r15			ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

RRCConnectionReconfigurationComplete field descriptions

numFreqEffective This field is used to indicate the number of effective frequencies that a UE measures in series according to TS 36.133 [16]. Simultaneous measurement in parallel on multiple frequencies can be equivalent to a single effective frequency. The frequencies configured for reduced measurement performance should not be included.

numFreqEffectiveReduced This field is used to indicate the number of effective frequencies that a UE measures in series according to TS 36.133 [16] for frequencies configured for reduced measurement performance. Simultaneous measurement in parallel on multiple frequencies can be equivalent to a single effective frequency.

perCC-GapIndicationList This field is used to indicate per CC measurement gap preference by the UE.

scg-ConfigResponseNR Includes the NR RRCReconfigurationComplete message as defined in TS 38.331 [82].

#### –	RRCConnectionReestablishment

The RRCConnectionReestablishment message is used to re-establish SRB1.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: E-UTRAN to UE

RRCConnectionReestablishment message

-- ASN1START

RRCConnectionReestablishment ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcConnectionReestablishment-r8		RRCConnectionReestablishment-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4	NULL,

spare3 NULL, spare2 NULL, spare1	NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReestablishment-r8-IEs ::= SEQUENCE {

radioResourceConfigDedicated		RadioResourceConfigDedicated,

nextHopChainingCount				NextHopChainingCount,

nonCriticalExtension				RRCConnectionReestablishment-v8a0-IEs	OPTIONAL

}

RRCConnectionReestablishment-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	RRCConnectionReestablishmentComplete

The RRCConnectionReestablishmentComplete message is used to confirm the successful completion of an RRC connection re-establishment.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

RRCConnectionReestablishmentComplete message

-- ASN1START

RRCConnectionReestablishmentComplete ::= SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcConnectionReestablishmentComplete-r8

RRCConnectionReestablishmentComplete-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReestablishmentComplete-r8-IEs ::= SEQUENCE {

nonCriticalExtension				RRCConnectionReestablishmentComplete-v920-IEs	OPTIONAL

}

RRCConnectionReestablishmentComplete-v920-IEs ::= SEQUENCE {

rlf-InfoAvailable-r9				ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				RRCConnectionReestablishmentComplete-v8a0-IEs	OPTIONAL

}

RRCConnectionReestablishmentComplete-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				RRCConnectionReestablishmentComplete-v1020-IEs	OPTIONAL

}

RRCConnectionReestablishmentComplete-v1020-IEs ::= SEQUENCE {

logMeasAvailable-r10				ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				RRCConnectionReestablishmentComplete-v1130-IEs	OPTIONAL

}

RRCConnectionReestablishmentComplete-v1130-IEs ::= SEQUENCE {

connEstFailInfoAvailable-r11		ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				RRCConnectionReestablishmentComplete-v1250-IEs	OPTIONAL

}

RRCConnectionReestablishmentComplete-v1250-IEs ::= SEQUENCE {

logMeasAvailableMBSFN-r12			ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				RRCConnectionReestablishmentComplete-v1530-IEs	OPTIONAL

}

RRCConnectionReestablishmentComplete-v1530-IEs ::= SEQUENCE {

logMeasAvailableBT-r15				ENUMERATED {true}				OPTIONAL,

logMeasAvailableWLAN-r15			ENUMERATED {true}				OPTIONAL,

flightPathInfoAvailable-r15			ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

RRCConnectionReestablishmentComplete field descriptions

rlf-InfoAvailable This field is used to indicate the availability of radio link failure or handover failure related measurements

#### –	RRCConnectionReestablishmentReject

The RRCConnectionReestablishmentReject message is used to indicate the rejection of an RRC connection re-establishment request.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: E-UTRAN to UE

RRCConnectionReestablishmentReject message

-- ASN1START

RRCConnectionReestablishmentReject ::= SEQUENCE {

criticalExtensions					CHOICE {

rrcConnectionReestablishmentReject-r8

RRCConnectionReestablishmentReject-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReestablishmentReject-r8-IEs ::= SEQUENCE {

nonCriticalExtension				RRCConnectionReestablishmentReject-v8a0-IEs	OPTIONAL

}

RRCConnectionReestablishmentReject-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	RRCConnectionReestablishmentRequest

The RRCConnectionReestablishmentRequest message is used to request the reestablishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to E-UTRAN

RRCConnectionReestablishmentRequest message

-- ASN1START

RRCConnectionReestablishmentRequest ::= SEQUENCE {

criticalExtensions					CHOICE {

rrcConnectionReestablishmentRequest-r8

RRCConnectionReestablishmentRequest-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReestablishmentRequest-r8-IEs ::= SEQUENCE {

ue-Identity							ReestabUE-Identity,

reestablishmentCause				ReestablishmentCause,

spare								BIT STRING (SIZE (2))

}

ReestabUE-Identity ::=				SEQUENCE {

c-RNTI								C-RNTI,

physCellId							PhysCellId,

shortMAC-I							ShortMAC-I

}

ReestablishmentCause ::=			ENUMERATED {

reconfigurationFailure, handoverFailure,

otherFailure, spare1

}

-- ASN1STOP

RRCConnectionReestablishmentRequest field descriptions

physCellId The Physical Cell Identity of the PCell the UE was connected to prior to the failure.

reestablishmentCause Indicates the failure cause that triggered the re-establishment procedure. eNB is not expected to reject a RRCConnectionReestablishmentRequest due to unknown cause value being used by the UE.

ue-Identity UE identity included to retrieve UE context and to facilitate contention resolution by lower layers.

#### –	RRCConnectionReject

The RRCConnectionReject message is used to reject the RRC connection establishment or to reject the EDT procedure.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: E-UTRAN to UE

RRCConnectionReject message

-- ASN1START

RRCConnectionReject ::=				SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionReject-r8				RRCConnectionReject-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReject-r8-IEs ::=		SEQUENCE {

waitTime							INTEGER (1..16),

nonCriticalExtension				RRCConnectionReject-v8a0-IEs		OPTIONAL

}

RRCConnectionReject-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				RRCConnectionReject-v1020-IEs		OPTIONAL

}

RRCConnectionReject-v1020-IEs ::=	SEQUENCE {

extendedWaitTime-r10				INTEGER (1..1800)					OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReject-v1130-IEs		OPTIONAL

}

RRCConnectionReject-v1130-IEs ::=	SEQUENCE {

deprioritisationReq-r11				SEQUENCE {

deprioritisationType-r11			ENUMERATED {frequency, e-utra},

deprioritisationTimer-r11			ENUMERATED {min5, min10, min15, min30}

}																		OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReject-v1320-IEs							OPTIONAL

}

RRCConnectionReject-v1320-IEs ::=	SEQUENCE {

rrc-SuspendIndication-r13				ENUMERATED {true}				OPTIONAL,	--	Need ON

nonCriticalExtension					SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

RRCConnectionReject field descriptions

deprioritisationReq Indicates whether the current frequency or RAT is to be de-prioritised. The UE shall be able to store a depriotisation request for up to 8 frequencies (applicable when receiving another frequency specific deprioritisation request before T325 expiry).

deprioritisationTimer Indicates the period for which either the current carrier frequency or E-UTRA is deprioritised. Value minN corresponds to N minutes.

extendedWaitTime Value in seconds for the wait time for Delay Tolerant access requests.

rrc-SuspendIndication If present, this field indicates that the UE should remain suspended and not release its stored context.

waitTime Wait time value in seconds.

#### –	RRCConnectionRelease

The RRCConnectionRelease message is used to command the release of an RRC connection, or to complete an UP-EDT procedure.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionRelease message

-- ASN1START

RRCConnectionRelease ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionRelease-r8				RRCConnectionRelease-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionRelease-r8-IEs ::=		SEQUENCE {

releaseCause						ReleaseCause,

redirectedCarrierInfo				RedirectedCarrierInfo				OPTIONAL,	-- Need ON

idleModeMobilityControlInfo			IdleModeMobilityControlInfo			OPTIONAL,	-- Need OP

nonCriticalExtension				RRCConnectionRelease-v890-IEs		OPTIONAL

}

RRCConnectionRelease-v890-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING RRCConnectionRelease-v9e0-IEs)	OPTIONAL,

nonCriticalExtension				RRCConnectionRelease-v920-IEs		OPTIONAL

}

-- Late non critical extensions

RRCConnectionRelease-v9e0-IEs ::= SEQUENCE {

redirectedCarrierInfo-v9e0			RedirectedCarrierInfo-v9e0			OPTIONAL,	-- Cond NoRedirect-r8

idleModeMobilityControlInfo-v9e0	IdleModeMobilityControlInfo-v9e0	OPTIONAL,	-- Cond IdleInfoEUTRA

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- Regular non critical extensions

RRCConnectionRelease-v920-IEs ::=	SEQUENCE {

cellInfoList-r9					CHOICE {

geran-r9						CellInfoListGERAN-r9,

utra-FDD-r9						CellInfoListUTRA-FDD-r9,

utra-TDD-r9						CellInfoListUTRA-TDD-r9,

...,

utra-TDD-r10					CellInfoListUTRA-TDD-r10

}															OPTIONAL,	-- Cond Redirection

nonCriticalExtension			RRCConnectionRelease-v1020-IEs		OPTIONAL

}

RRCConnectionRelease-v1020-IEs ::=	SEQUENCE {

extendedWaitTime-r10				INTEGER (1..1800)		OPTIONAL,	-- Need ON

nonCriticalExtension			RRCConnectionRelease-v1320-IEs				OPTIONAL

}

RRCConnectionRelease-v1320-IEs::=	SEQUENCE {

resumeIdentity-r13					ResumeIdentity-r13				OPTIONAL,	-- Need OR

nonCriticalExtension				RRCConnectionRelease-v1530-IEs	OPTIONAL

}

RRCConnectionRelease-v1530-IEs ::=	SEQUENCE {

drb-ContinueROHC-r15				ENUMERATED {true}			OPTIONAL,	-- Cond UP-EDTorPUR

nextHopChainingCount-r15			NextHopChainingCount		OPTIONAL,	-- Cond EarlySec

measIdleConfig-r15					MeasIdleConfigDedicated-r15	OPTIONAL,	-- Need ON

rrc-InactiveConfig-r15				RRC-InactiveConfig-r15		OPTIONAL,	-- Need OR

cn-Type-r15							ENUMERATED {epc,fivegc}		OPTIONAL,	-- Need OR

nonCriticalExtension				RRCConnectionRelease-v1540-IEs			OPTIONAL

}

RRCConnectionRelease-v1540-IEs ::=	SEQUENCE {

waitTime							INTEGER (1..16)		OPTIONAL, -- Cond 5GC

nonCriticalExtension				RRCConnectionRelease-v15b0-IEs	OPTIONAL

}

RRCConnectionRelease-v15b0-IEs ::=	SEQUENCE {

noLastCellUpdate-r15				ENUMERATED {true}		OPTIONAL,	-- Need OP

nonCriticalExtension				RRCConnectionRelease-v1610-IEs	OPTIONAL

}

RRCConnectionRelease-v1610-IEs ::=	SEQUENCE {

fullI-RNTI-r16						I-RNTI-r15					OPTIONAL, -- Need OR

shortI-RNTI-r16					ShortI-RNTI-r15 				OPTIONAL, -- Need OR

pur-Config-r16						SetupRelease {PUR-Config-r16}	OPTIONAL, -- Need ON

rrc-InactiveConfig-v1610			RRC-InactiveConfig-v1610	OPTIONAL,  -- Cond BLCE-IDLEeDRX

releaseIdleMeasConfig-r16			ENUMERATED {true}			OPTIONAL, -- Need ON

altFreqPriorities-r16				ENUMERATED {true}			OPTIONAL, -- Need ON

t323-r16							ENUMERATED {

min5, min10, min20, min30, min60, min120, min180,

min720}						OPTIONAL, -- Need OR

nonCriticalExtension				RRCConnectionRelease-v1650-IEs		OPTIONAL

}

RRCConnectionRelease-v1650-IEs ::=	SEQUENCE {

mpsPriorityIndication-r16				ENUMERATED {true}	OPTIONAL, -- Cond Redirection2

nonCriticalExtension					SEQUENCE {}		OPTIONAL

}

ReleaseCause ::=				ENUMERATED {loadBalancingTAUrequired,

other, cs-FallbackHighPriority-v1020, rrc-Suspend-v1320}

RedirectedCarrierInfo ::=			CHOICE {

eutra								ARFCN-ValueEUTRA,

geran								CarrierFreqsGERAN,

utra-FDD							ARFCN-ValueUTRA,

utra-TDD							ARFCN-ValueUTRA,

cdma2000-HRPD						CarrierFreqCDMA2000,

cdma2000-1xRTT						CarrierFreqCDMA2000,

...,

utra-TDD-r10						CarrierFreqListUTRA-TDD-r10,

nr-r15								CarrierInfoNR-r15

}

RedirectedCarrierInfo-v9e0 ::=			SEQUENCE {

eutra-v9e0								ARFCN-ValueEUTRA-v9e0

}

RRC-InactiveConfig-r15::=		SEQUENCE {

fullI-RNTI-r15					I-RNTI-r15,

shortI-RNTI-r15					ShortI-RNTI-r15,

ran-PagingCycle-r15				ENUMERATED {	rf32, rf64, rf128, rf256}	OPTIONAL,	--Need OR

ran-NotificationAreaInfo-r15	RAN-NotificationAreaInfo-r15		OPTIONAL,	--Need ON

periodic-RNAU-timer-r15			ENUMERATED {min5, min10, min20, min30, min60,

min120, min360, min720}		OPTIONAL,	--Need OR

nextHopChainingCount-r15		NextHopChainingCount		OPTIONAL,	--Cond INACTIVE

dummy							SEQUENCE{}		OPTIONAL

}

RRC-InactiveConfig-v1610::=		SEQUENCE {

ran-PagingCycle-v1610				ENUMERATED {rf512, rf1024}

}

RAN-NotificationAreaInfo-r15	::= CHOICE {

cellList-r15				PLMN-RAN-AreaCellList-r15,

ran-AreaConfigList-r15		PLMN-RAN-AreaConfigList-r15

}

PLMN-RAN-AreaCellList-r15	::=	SEQUENCE (SIZE (1..maxPLMN-r15)) OF PLMN-RAN-AreaCell-r15

PLMN-RAN-AreaCell-r15	::=		SEQUENCE {

plmn-Identity-r15				PLMN-Identity	OPTIONAL,

ran-AreaCells-r15				SEQUENCE (SIZE (1..32)) OF CellIdentity

}

PLMN-RAN-AreaConfigList-r15	::=	SEQUENCE (SIZE (1..maxPLMN-r15)) OF PLMN-RAN-AreaConfig-r15

PLMN-RAN-AreaConfig-r15	::=	SEQUENCE {

plmn-Identity-r15			PLMN-Identity	OPTIONAL,

ran-Area-r15				SEQUENCE (SIZE (1..16)) OF	RAN-AreaConfig-r15

}

RAN-AreaConfig-r15	::=	SEQUENCE {

trackingAreaCode-5GC-r15	TrackingAreaCode-5GC-r15,

ran-AreaCodeList-r15		SEQUENCE (SIZE (1..32)) OF RAN-AreaCode-r15	OPTIONAL	--Need OR

}

CarrierFreqListUTRA-TDD-r10 ::=			SEQUENCE (SIZE (1..maxFreqUTRA-TDD-r10)) OF ARFCN-ValueUTRA

IdleModeMobilityControlInfo ::=		SEQUENCE {

freqPriorityListEUTRA				FreqPriorityListEUTRA			OPTIONAL,		-- Need ON

freqPriorityListGERAN				FreqsPriorityListGERAN			OPTIONAL,		-- Need ON

freqPriorityListUTRA-FDD			FreqPriorityListUTRA-FDD		OPTIONAL,		-- Need ON

freqPriorityListUTRA-TDD			FreqPriorityListUTRA-TDD		OPTIONAL,		-- Need ON

bandClassPriorityListHRPD			BandClassPriorityListHRPD		OPTIONAL,		-- Need ON

bandClassPriorityList1XRTT			BandClassPriorityList1XRTT		OPTIONAL,		-- Need ON

t320								ENUMERATED {

min5, min10, min20, min30, min60, min120, min180,

spare1}						OPTIONAL,		-- Need OR

...,

[[	freqPriorityListExtEUTRA-r12		FreqPriorityListExtEUTRA-r12		OPTIONAL		-- Need ON

]],

[[	freqPriorityListEUTRA-v1310			FreqPriorityListEUTRA-v1310			OPTIONAL,		-- Need ON

freqPriorityListExtEUTRA-v1310		FreqPriorityListExtEUTRA-v1310		OPTIONAL		-- Need ON

]],

[[	freqPriorityListNR-r15				FreqPriorityListNR-r15		OPTIONAL		-- Need ON

]]

}

IdleModeMobilityControlInfo-v9e0 ::=	SEQUENCE {

freqPriorityListEUTRA-v9e0			SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA-v9e0

}

FreqPriorityListEUTRA ::=			SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA

FreqPriorityListExtEUTRA-r12 ::=		SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA-r12

FreqPriorityListEUTRA-v1310 ::=			SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA-v1310

FreqPriorityListExtEUTRA-v1310 ::=		SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA-v1310

FreqPriorityEUTRA ::=				SEQUENCE {

carrierFreq							ARFCN-ValueEUTRA,

cellReselectionPriority				CellReselectionPriority

}

FreqPriorityEUTRA-v9e0 ::=			SEQUENCE {

carrierFreq-v9e0					ARFCN-ValueEUTRA-v9e0		OPTIONAL	-- Cond EARFCN-max

}

FreqPriorityEUTRA-r12 ::=				SEQUENCE {

carrierFreq-r12							ARFCN-ValueEUTRA-r9,

cellReselectionPriority-r12				CellReselectionPriority

}

FreqPriorityEUTRA-v1310 ::=				SEQUENCE {

cellReselectionSubPriority-r13				CellReselectionSubPriority-r13		OPTIONAL		-- Need ON

}

FreqPriorityListNR-r15 ::=		SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR-r15

FreqPriorityNR-r15 ::=			SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

cellReselectionPriority-r15			CellReselectionPriority,

cellReselectionSubPriority-r15		CellReselectionSubPriority-r13		OPTIONAL		-- Need OR

}

FreqsPriorityListGERAN ::=			SEQUENCE (SIZE (1..maxGNFG)) OF FreqsPriorityGERAN

FreqsPriorityGERAN ::=				SEQUENCE {

carrierFreqs						CarrierFreqsGERAN,

cellReselectionPriority				CellReselectionPriority

}

FreqPriorityListUTRA-FDD ::=		SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF FreqPriorityUTRA-FDD

FreqPriorityUTRA-FDD ::=			SEQUENCE {

carrierFreq							ARFCN-ValueUTRA,

cellReselectionPriority				CellReselectionPriority

}

FreqPriorityListUTRA-TDD ::=		SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF FreqPriorityUTRA-TDD

FreqPriorityUTRA-TDD ::=			SEQUENCE {

carrierFreq							ARFCN-ValueUTRA,

cellReselectionPriority				CellReselectionPriority

}

BandClassPriorityListHRPD ::=		SEQUENCE (SIZE (1..maxCDMA-BandClass)) OF BandClassPriorityHRPD

BandClassPriorityHRPD ::=			SEQUENCE {

bandClass							BandclassCDMA2000,

cellReselectionPriority				CellReselectionPriority

}

BandClassPriorityList1XRTT ::=	SEQUENCE (SIZE (1..maxCDMA-BandClass)) OF BandClassPriority1XRTT

BandClassPriority1XRTT ::=			SEQUENCE {

bandClass							BandclassCDMA2000,

cellReselectionPriority				CellReselectionPriority

}

CellInfoListGERAN-r9 ::=		SEQUENCE (SIZE (1..maxCellInfoGERAN-r9)) OF CellInfoGERAN-r9

CellInfoGERAN-r9 ::=				SEQUENCE {

physCellId-r9						PhysCellIdGERAN,

carrierFreq-r9						CarrierFreqGERAN,

systemInformation-r9				SystemInfoListGERAN

}

CarrierInfoNR-r15	::= SEQUENCE {

carrierFreq-r15					ARFCN-ValueNR-r15,

subcarrierSpacingSSB-r15			ENUMERATED {kHz15, kHz30, kHz120, kHz240},

smtc-r15							MTC-SSB-NR-r15				OPTIONAL		-- Need OP

}

CellInfoListUTRA-FDD-r9 ::=			SEQUENCE (SIZE (1..maxCellInfoUTRA-r9)) OF CellInfoUTRA-FDD-r9

CellInfoUTRA-FDD-r9 ::=				SEQUENCE {

physCellId-r9						PhysCellIdUTRA-FDD,

utra-BCCH-Container-r9				OCTET STRING

}

CellInfoListUTRA-TDD-r9 ::=			SEQUENCE (SIZE (1..maxCellInfoUTRA-r9)) OF CellInfoUTRA-TDD-r9

CellInfoUTRA-TDD-r9 ::=				SEQUENCE {

physCellId-r9						PhysCellIdUTRA-TDD,

utra-BCCH-Container-r9				OCTET STRING

}

CellInfoListUTRA-TDD-r10 ::=		SEQUENCE (SIZE (1..maxCellInfoUTRA-r9)) OF CellInfoUTRA-TDD-r10

CellInfoUTRA-TDD-r10 ::=			SEQUENCE {

physCellId-r10						PhysCellIdUTRA-TDD,

carrierFreq-r10						ARFCN-ValueUTRA,

utra-BCCH-Container-r10				OCTET STRING

}

-- ASN1STOP

RRCConnectionRelease field descriptions

altFreqPriorities Indicates that the UE shall apply the alternative cell reselectionpriorities, when available. This field is not configured together with idleModeMobilityControlInfo.

carrierFreq or bandClass The carrier frequency (UTRA, E-UTRA, and NR) and band class (HRPD and 1xRTT) for which the associated cellReselectionPriority is applied. For NR, the ARFCN-ValueNR corresponds to a GSCN value as specified in TS 38.101 [85].

carrierFreqs The list of GERAN carrier frequencies organised into one group of GERAN carrier frequencies.

cellInfoList Used to provide system information of one or more cells on the redirected inter-RAT carrier frequency. The system information can be used if, upon redirection, the UE selects an inter-RAT cell indicated by the physCellId and carrierFreq (GERAN and UTRA TDD) or by the physCellId (other RATs). The choice shall match the redirectedCarrierInfo. In particular, E-UTRAN only applies value utra-TDD-r10 in case redirectedCarrierInfo is set to utra-TDD-r10.

cellList Indicates a list of cells configured as RAN area. For each element, in the absence of plmn-Identity the UE considers the registered PLMN. Total number of cells across all PLMNs does not exceed 32.

cn-Type The cn-Type is used to indicate that the UE is redirected from 5GC to EPC or 5GC when redirectedCarrierInfo indicates E-UTRA frequency.

drb-ContinueROHC This field indicates whether to continue or reset the header compression protocol context for the DRBs configured with the header compression protocol. Presence of the field indicates that the header compression protocol context continues when UE initiates UP-EDT in the same cell, while absence indicates that the header compression protocol context is reset.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

extendedWaitTime Value in seconds for the wait time for Delay Tolerant access requests.

freqPriorityListX Provides a cell reselection priority for each frequency, by means of separate lists for each RAT (including E-UTRA). The UE shall be able to store at least 3 occurrences of FreqsPriorityGERAN. If E-UTRAN includes freqPriorityListEUTRA-v9e0 and/or freqPriorityListEUTRA-v1310 it includes the same number of entries, and listed in the same order, as in freqPriorityListEUTRA (i.e. without suffix). Field freqPriorityListExt includes additional neighbouring inter-frequencies, i.e. extending the size of the inter-frequency carrier list using the general principles specified in 5.1.2. EUTRAN only includes freqPriorityListExtEUTRA if freqPriorityListEUTRA (i.e without suffix) includes maxFreq entries. If E-UTRAN includes freqPriorityListExtEUTRA-v1310 it includes the same number of entries, and listed in the same order, as in freqPriorityListExtEUTRA-r12.

idleModeMobilityControlInfo Provides dedicated cell reselection priorities. Used for cell reselection as specified in TS 36.304 [4]. For E-UTRA and UTRA frequencies, a UE that supports multi-band cells for the concerned RAT considers the dedicated priorities to be common for all overlapping bands (i.e. regardless of the ARFCN that is used).

measIdleConfig Indicates a one-shot measurement configuration to be stored and used by the UE while in RRC_IDLE or RRC_INACTIVE.

mpsPriorityIndication Indicates the UE can set the establishment cause to highPriorityAccess for a new connection to a new RAT following a redirect to E-UTRA. If the target RAT is NR, see TS 38.331 [82]. The eNB/ng-eNB sets the indication only for UEs authorized to receive MPS treatment as indicated by ARP and/or QoS characteristics at the eNB/ng-eNB, and it is applicable only for this instance of release with redirection to carrier/RAT included in the redirectedCarrierInfo field in the RRCConnectionRelease message.

noLastCellUpdate Presence of the field indicates that the last used cell for (G)WUS shall not be updated.

periodic-RNAU-timer Refers to the timer that triggers the periodic RNAU procedure in UE. Value min5 corresponds to 5 minutes, value min10 corresponds to 10 minutes and so on.

ran-Area Indicates whether TA code(s) or RAN area code(s) are used for the RAN notification area. The network uses only TA code(s) or RAN area code(s) to configure a UE. Total number of TACs across all PLMNs does not exceed 16. Total number of RAN-AreaCode across all PLMNs does not exceed 32.

ran-NotificationAreaInfo Network ensures that the UE in RRC_INACTIVE always has a valid ran-NotificationAreaInfo.

ranAreaConfigList Indicates a list of RAN area codes or RA code(s) as RAN area. For each element, in the absence of plmn-Identity the UE considers the registered PLMN.

ran-pagingCycle Refers to the UE specific cycle for RAN-initiated paging. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on.

redirectedCarrierInfo The redirectedCarrierInfo indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an E-UTRA or an inter-RAT carrier frequency, by means of the cell selection upon leaving RRC_CONNECTED as specified in TS 36.304 [4]. The value geran can only be included after successful security activation when UE is connected to 5GC.

releaseCause The releaseCause is used to indicate the reason for releasing the RRC Connection. The cause value cs-FallbackHighPriority is only applicable when redirectedCarrierInfo is present with the value set to utra-FDD, utra-TDD or utra-TDD-r10. E-UTRAN should not set the releaseCause to loadBalancingTAURequired or to cs-FallbackHighPriority if the extendedWaitTime is present. The network should not set the releaseCause to loadBalancingTAURequired if the UE is connected to 5GC. The network does not set the releaseCause to rrc-Suspend if the UE is configured with a DAPS bearer, i.e. if source PCell resources after a DAPS handover have not been released.

releaseIdleMeasConfig Indicates that the UE shall release the idle/inactive measurement configurations, if configured.

rrc-InactiveConfig Indicates configuration for the RRC_INACTIVE state. The network does not configure this field when the UE is redirected to an inter-RAT carrier frequency or if the UE is configured with a DAPS bearer.

smtc The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing

subcarrierSpacingSSB Indicate subcarrier spacing of SSB of redirected target NR frequency. Only the values 15 or 30 (<6GHz), 120 kHz or 240 kHz (>6GHz) are applicable.

systemInformation Container for system information of the GERAN cell i.e. one or more System Information (SI) messages as defined in TS 44.018 [45], table 9.1.1.

t320 Timer T320 as described in clause 7.3. Value minN corresponds to N minutes.

t323 Timer T323 as described in clause 7.3. Value minN corresponds to N minutes.

utra-BCCH-Container Contains System Information Container message as defined in TS 25.331 [19].

waitTime Wait time value in seconds.

| Conditional presence | Explanation |
| --- | --- |
| 5GC | The field is optionally present, Need ON, if the UE is connected to 5GC; otherwise the field is not present. |
| BLCE-IDLEeDRX | The field is optionally present, Need OR, if the UE is a BL UE or UE in CE and the UE is connected to 5GC and IDLE mode eDRX is configured and ran-PagingCycle-r15 is absent; otherwise the field is not present. |  |
| EARFCN-max | The field is mandatory present if the corresponding carrierFreq (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| EarlySec | When the UE is connected to 5GC, the field is mandatory present. When the UE is connected to EPC, the field is optionally present, Need ON, if the UE supports UP-EDT or UP transmission using PUR or early security reactivation and releaseCause is set to rrc-Suspend; otherwise the field is not present. |  |
| IdleInfoEUTRA | The field is optionally present, Need OP, if the IdleModeMobilityControlInfo (i.e. without suffix) is included and includes freqPriorityListEUTRA; otherwise the field is not present. |
| INACTIVE | The field is mandatory present in this release. |
| NoRedirect-r8 | The field is optionally present, Need OP, if the redirectedCarrierInfo (i.e. without suffix) is not included; otherwise the field is not present. |
| Redirection | The field is optionally present, Need ON, if the redirectedCarrierInfo is included and set to geran, utra-FDD, utra-TDD or utra-TDD-r10; otherwise the field is not present. |
| Redirection2 | The field is optionally present, Need OR, if redirectedCarrierInfo is included; otherwise the field is not present. |
| UP-EDTorPUR | The field is optionally present, Need ON, if the UE supports UP-EDT or UP transmission using PUR and releaseCause is set to rrc-Suspend; otherwise the field is not present. |

#### –	RRCConnectionRequest

The RRCConnectionRequest message is used to request the establishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to E-UTRAN

RRCConnectionRequest message

-- ASN1START

RRCConnectionRequest ::=			SEQUENCE {

criticalExtensions					CHOICE {

rrcConnectionRequest-r8				RRCConnectionRequest-r8-IEs,

rrcConnectionRequest-r15			RRCConnectionRequest-5GC-r15-IEs

}

}

RRCConnectionRequest-r8-IEs ::=		SEQUENCE {

ue-Identity							InitialUE-Identity,

establishmentCause					EstablishmentCause,

spare								BIT STRING (SIZE (1))

}

RRCConnectionRequest-5GC-r15-IEs ::=	SEQUENCE {

ue-Identity								InitialUE-Identity-5GC,

establishmentCause						EstablishmentCause-5GC,

spare									BIT STRING (SIZE (1))

}

InitialUE-Identity ::=				CHOICE {

s-TMSI								S-TMSI,

randomValue							BIT STRING (SIZE (40))

}

InitialUE-Identity-5GC ::=			CHOICE {

ng-5G-S-TMSI-Part1					BIT STRING (SIZE (40)),

randomValue							BIT STRING (SIZE (40))

}

EstablishmentCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1}

EstablishmentCause-5GC ::=			ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, spare2, spare1}

-- ASN1STOP

RRCConnectionRequest field descriptions

establishmentCause Provides the establishment cause for the RRC connection request as provided by the upper layers. W.r.t. the cause value names: highPriorityAccess concerns AC11..AC15, 'mt' stands for 'Mobile Terminating' and 'mo' for 'Mobile Originating. eNB is not expected to reject a RRCConnectionRequest due to unknown cause value being used by the UE. The cause value of delayTolerantAccess is not used for E-UTRA/5GC in this release.

randomValue Integer value in the range 0 to 240 1.

ng-5G-S-TMSI-Part1 The rightmost 40 bits of 5G-S-TMSI.

ue-Identity UE identity included to facilitate contention resolution by lower layers.

#### –	RRCConnectionResume

The RRCConnectionResume message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionResume message

-- ASN1START

RRCConnectionResume-r13 ::=		SEQUENCE {

rrc-TransactionIdentifier		RRC-TransactionIdentifier,

criticalExtensions				CHOICE {

c1								CHOICE {

rrcConnectionResume-r13			RRCConnectionResume-r13-IEs,

spare3							NULL,

spare2							NULL,

spare1							NULL

},

criticalExtensionsFuture		SEQUENCE {}

}

}

RRCConnectionResume-r13-IEs ::=		SEQUENCE {

radioResourceConfigDedicated-r13		RadioResourceConfigDedicated	OPTIONAL,	-- Need ON

nextHopChainingCount-r13				NextHopChainingCount,

measConfig-r13							MeasConfig						OPTIONAL,	-- Need ON

antennaInfoDedicatedPCell-r13			AntennaInfoDedicated-v10i0		OPTIONAL,	-- Need ON

drb-ContinueROHC-r13					ENUMERATED {true}				OPTIONAL,	-- Need OP

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

rrcConnectionResume-v1430-IEs			RRCConnectionResume-v1430-IEs	OPTIONAL

}

RRCConnectionResume-v1430-IEs ::= SEQUENCE {

otherConfig-r14						OtherConfig-r9					OPTIONAL,		-- Need ON

rrcConnectionResume-v1510-IEs		RRCConnectionResume-v1510-IEs	OPTIONAL

}

RRCConnectionResume-v1510-IEs ::= SEQUENCE {

sk-Counter-r15						INTEGER (0.. 65535)				OPTIONAL,	-- Need ON

nr-RadioBearerConfig1-r15			OCTET STRING					OPTIONAL,	-- Need ON

nr-RadioBearerConfig2-r15			OCTET STRING					OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionResume-v1530-IEs	OPTIONAL

}

RRCConnectionResume-v1530-IEs ::= SEQUENCE {

fullConfig-r15						ENUMERATED {true}				OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionResume-v1610-IEs	OPTIONAL

}

RRCConnectionResume-v1610-IEs ::=	SEQUENCE {

idleModeMeasurementReq-r16			ENUMERATED {true}				OPTIONAL,	-- Need ON

restoreMCG-SCells					ENUMERATED {true}				OPTIONAL,	-- Need ON

restoreSCG							ENUMERATED {true}				OPTIONAL,	-- Cond EarlySec

sCellToAddModList-r16				SCellToAddModList-r16			OPTIONAL,	-- Cond EarlySec

sCellToReleaseList-r16				SCellToReleaseListExt-r13		OPTIONAL,	-- Need ON

sCellGroupToReleaseList-r16			SCellGroupToReleaseList-r15		OPTIONAL,	-- Need ON

sCellGroupToAddModList-r16			SCellGroupToAddModList-r15		OPTIONAL,	-- Cond EarlySec

nr-SecondaryCellGroupConfig			OCTET STRING					OPTIONAL,	-- Cond RestoreSCG

p-MaxEUTRA-r16						P-Max							OPTIONAL,	-- Cond SCG

p-MaxUE-FR1-r16						P-Max							OPTIONAL,	-- Cond SCG

tdm-PatternConfig-r16				TDM-PatternConfig-r15			OPTIONAL,	-- Cond FDD-PCell

tdm-PatternConfig2-r16				TDM-PatternConfig-r15			OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

RRCConnectionResume field descriptions

drb-ContinueROHC This field indicates whether to continue or reset the header compression protocol context for the DRBs configured with EUTRA PDCP and the header compression protocol. Presence of the field indicates that the header compression protocol context continues while absence indicates that the header compression protocol context is reset.

fullConfig Indicates that the full configuration option is applicable for the RRCConnectionResume message.

idleModeMeasurementReq This field indicates that the UE shall report the idle/inactive measurements to the network in the RRCConnectionResumeComplete message

p-MaxEUTRA Indicates the maximum power available for E-UTRA.

p-MaxUE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited on cell- or cell-group level.

nr-RadioBearerConfig1, nr-RadioBearerConfig2 Includes the NR RadioBearerConfig IE as specified in TS 38.331 [82]. The field includes the configuration of RBs configured with NR PDCP.

nr-SecondaryCellGroupConfig Includes the NR RRCReconfiguration message as specified in TS 38.331 [82]. In this version of the specification, the NR RRC message only includes fields secondaryCellGroup, with at least reconfigurationWithSync, otherConfig and/ or measConfig.

restoreMCG-Scells Indicates that the UE shall restore the MCG Scell configurations from the UE AS Context or UE Inactive AS Context, if configured.

restoreSCG If included, the UE shall restore the SCG configurations from the UE AS Context or UE Inactive AS Context.

sCellGroupToAddModList Indicates the SCell group to be added or modified.

sCellGroupToReleaseList Indicates the SCell group to be released.

sCellToAddModList List of SCells to be added or modified.

sCellToReleaseList List of SCells to be released.

sk-Counter A one-shot counter used upon initial configuration of S-KgNB as well as upon refresh of S-KgNB. E-UTRAN provides this field when the UE is configured with an (SN-terminated) RB using S-KgNB or NR SCG is configured.

tdm-PatternConfig This field is used when power control or IMD issues require single UL transmission in (NG)EN-DC as specified in TS 38.101-3 [101] and TS 38.213 [88].

tdm-PatternConfig2 This field is used for dual UL transmission in EN-DC with LTE FDD PCell and for single UL transmission in EN-DC with LTE FDD/TDD PCell, as specified in TS 38.101-3 [101] and TS 38.213 [88]. The network sets at most one of tdm-PatternConfig and tdm-PatternConfig2 to setup. When this field is configured in EN-DC with LTE TDD PCell, it is not applicable if TDD configuration is sa0 or sa6 in SIB1.

| Conditional presence | Explanation |
| --- | --- |
| EarlySec | For EPC, the field is optionally present, Need ON, if the UE supports early security reactivation; otherwise the field is not present. For 5GC, the field is optionally present, Need ON. |
| RestoreSCG | The field is mandatory present if restoreSCG is configured. It is optionally present, Need ON, otherwise. For EPC, this field can be present only if the UE supports early security reactivation. |
| FDD-PCell | This field is optionally present, need ON, for an FDD PCell if there is no SCell with configured uplink. Otherwise, the field is not present, need OR. |
| SCG | This field is optionally present, need OR, if nr-SecondaryCellGroupConfig is present, otherwise it is absent, need OR. |

#### –	RRCConnectionResumeComplete

The RRCConnectionResumeComplete message is used to confirm the successful completion of an RRC connection resumption.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

RRCConnectionResumeComplete message

-- ASN1START

RRCConnectionResumeComplete-r13 ::= SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions							CHOICE {

rrcConnectionResumeComplete-r13				RRCConnectionResumeComplete-r13-IEs,

criticalExtensionsFuture					SEQUENCE {}

}

}

RRCConnectionResumeComplete-r13-IEs ::= SEQUENCE {

selectedPLMN-Identity-r13				INTEGER (1..maxPLMN-r11)					OPTIONAL,

dedicatedInfoNAS-r13					DedicatedInfoNAS							OPTIONAL,

rlf-InfoAvailable-r13					ENUMERATED {true}							OPTIONAL,

logMeasAvailable-r13					ENUMERATED {true}							OPTIONAL,

connEstFailInfoAvailable-r13			ENUMERATED {true}							OPTIONAL,

mobilityState-r13						ENUMERATED {normal, medium, high, spare}	OPTIONAL,

mobilityHistoryAvail-r13				ENUMERATED {true}							OPTIONAL,

logMeasAvailableMBSFN-r13				ENUMERATED {true}							OPTIONAL,

lateNonCriticalExtension				OCTET STRING								OPTIONAL,

nonCriticalExtension					RRCConnectionResumeComplete-v1530-IEs		OPTIONAL

}

RRCConnectionResumeComplete-v1530-IEs ::= SEQUENCE {

logMeasAvailableBT-r15			ENUMERATED {true}				OPTIONAL,

logMeasAvailableWLAN-r15		ENUMERATED {true}				OPTIONAL,

idleMeasAvailable-r15			ENUMERATED {true}				OPTIONAL,

flightPathInfoAvailable-r15		ENUMERATED {true}				OPTIONAL,

nonCriticalExtension			RRCConnectionResumeComplete-v1610-IEs	OPTIONAL

}

RRCConnectionResumeComplete-v1610-IEs ::= SEQUENCE {

measResultListIdle-r16				MeasResultListIdle-r15			OPTIONAL,

measResultListExtIdle-r16				MeasResultListExtIdle-r16			OPTIONAL,

measResultListIdleNR-r16			MeasResultListIdleNR-r16		OPTIONAL,

scg-ConfigResponseNR-r16			OCTET STRING					OPTIONAL,

nonCriticalExtension				SEQUENCE{}						OPTIONAL

}

-- ASN1STOP

RRCConnectionResumeComplete field descriptions

idleMeasAvailable Indication that the UE has idle/inactive measurement report available.

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityList fields included in SIB1. 1 if the 1st PLMN is selected from the 1st plmn-IdentityList included in SIB1, 2 if the 2nd PLMN is selected from the same plmn-IdentityList, or when no more PLMN are present within the same plmn-IdentityList, then the PLMN listed 1st in the subsequent plmn-IdentityList within the same SIB1 and so on. The selectedPLMN-Identity is referred to the PLMN list for 5GC if the UE is in RRC_INACTIVE state.

#### –	RRCConnectionResumeRequest

The RRCConnectionResumeRequest message is used to request the resumption of a suspended RRC connection or to perform UP-EDT.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to E-UTRAN

RRCConnectionResumeRequest message

-- ASN1START

RRCConnectionResumeRequest-r13 ::=	SEQUENCE {

criticalExtensions						CHOICE {

rrcConnectionResumeRequest-r13			RRCConnectionResumeRequest-r13-IEs,

rrcConnectionResumeRequest-r15			RRCConnectionResumeRequest-5GC-r15-IEs

}

}

RRCConnectionResumeRequest-r13-IEs ::=		SEQUENCE {

resumeIdentity-r13								CHOICE {

resumeID-r13									ResumeIdentity-r13,

truncatedResumeID-r13							BIT STRING (SIZE (24))

},

shortResumeMAC-I-r13							BIT STRING (SIZE (16)),

resumeCause-r13									ResumeCause,

spare											BIT STRING (SIZE (1))

}

RRCConnectionResumeRequest-5GC-r15-IEs ::=		SEQUENCE {

resumeIdentity-r15								CHOICE {

fullI-RNTI-r15									I-RNTI-r15,

shortI-RNTI-r15									ShortI-RNTI-r15

},

shortResumeMAC-I-r15							BIT STRING (SIZE (16)),

resumeCause-r15									ResumeCause-r15,

spare											BIT STRING (SIZE (1))

}

ResumeCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280,

mt-EDT-v1610

}

ResumeCause-r15 ::=			ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, rna-Update, mo-VoiceCall, spare1

}

-- ASN1STOP

RRCConnectionResumeRequest field descriptions

resumeCause Provides the resume cause for the RRC connection resume request as provided by the upper layers. The network is not expected to reject a RRCConnectionResumeRequest due to unknown cause value being used by the UE.

resumeIdentity UE identity to facilitate UE context retrieval at eNB

shortResumeMAC-I Authentication token to facilitate UE authentication at eNB

#### –	RRCConnectionSetup

The RRCConnectionSetup message is used to establish SRB1.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: E-UTRAN to UE

RRCConnectionSetup message

-- ASN1START

RRCConnectionSetup ::=				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionSetup-r8				RRCConnectionSetup-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionSetup-r8-IEs ::=		SEQUENCE {

radioResourceConfigDedicated		RadioResourceConfigDedicated,

nonCriticalExtension				RRCConnectionSetup-v8a0-IEs			OPTIONAL

}

RRCConnectionSetup-v8a0-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				RRCConnectionSetup-v1610-IEs		OPTIONAL

}

RRCConnectionSetup-v1610-IEs ::=	SEQUENCE {

dedicatedInfoNAS-r16				DedicatedInfoNAS					OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

RRCConnectionSetup field descriptions

dedicatedInfoNAS Downlink NAS PDU in case of mobile terminated CP-EDT. E-UTRAN may include this field only if the RRCConnectionSetup is in response to RRCEarlyDataRequest with establishment cause mt-Access.

#### –	RRCConnectionSetupComplete

The RRCConnectionSetupComplete message is used to confirm the successful completion of an RRC connection establishment.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

RRCConnectionSetupComplete message

-- ASN1START

RRCConnectionSetupComplete ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcConnectionSetupComplete-r8		RRCConnectionSetupComplete-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionSetupComplete-r8-IEs ::= SEQUENCE {

selectedPLMN-Identity				INTEGER (1..maxPLMN-r11),

registeredMME						RegisteredMME						OPTIONAL,

dedicatedInfoNAS					DedicatedInfoNAS,

nonCriticalExtension				RRCConnectionSetupComplete-v8a0-IEs	OPTIONAL

}

RRCConnectionSetupComplete-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING							OPTIONAL,

nonCriticalExtension				RRCConnectionSetupComplete-v1020-IEs	OPTIONAL

}

RRCConnectionSetupComplete-v1020-IEs ::= SEQUENCE {

gummei-Type-r10						ENUMERATED {native, mapped}				OPTIONAL,

rlf-InfoAvailable-r10				ENUMERATED {true}						OPTIONAL,

logMeasAvailable-r10				ENUMERATED {true}						OPTIONAL,

rn-SubframeConfigReq-r10			ENUMERATED {required, notRequired}		OPTIONAL,

nonCriticalExtension				RRCConnectionSetupComplete-v1130-IEs	OPTIONAL

}

RRCConnectionSetupComplete-v1130-IEs ::= SEQUENCE {

connEstFailInfoAvailable-r11		ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				RRCConnectionSetupComplete-v1250-IEs		OPTIONAL

}

RRCConnectionSetupComplete-v1250-IEs ::= SEQUENCE {

mobilityState-r12					ENUMERATED {normal, medium, high, spare}	OPTIONAL,

mobilityHistoryAvail-r12			ENUMERATED {true}							OPTIONAL,

logMeasAvailableMBSFN-r12			ENUMERATED {true}							OPTIONAL,

nonCriticalExtension				RRCConnectionSetupComplete-v1320-IEs		OPTIONAL

}

RRCConnectionSetupComplete-v1320-IEs ::= SEQUENCE {

ce-ModeB-r13						ENUMERATED {supported}						OPTIONAL,

s-TMSI-r13							S-TMSI										OPTIONAL,

attachWithoutPDN-Connectivity-r13	ENUMERATED {true}							OPTIONAL,

up-CIoT-EPS-Optimisation-r13		ENUMERATED {true}							OPTIONAL,

cp-CIoT-EPS-Optimisation-r13		ENUMERATED {true}							OPTIONAL,

nonCriticalExtension				RRCConnectionSetupComplete-v1330-IEs		OPTIONAL

}

RRCConnectionSetupComplete-v1330-IEs ::= SEQUENCE {

ue-CE-NeedULGaps-r13				ENUMERATED {true}							OPTIONAL,

nonCriticalExtension				RRCConnectionSetupComplete-v1430-IEs		OPTIONAL

}

RRCConnectionSetupComplete-v1430-IEs ::= SEQUENCE {

dcn-ID-r14							INTEGER (0..65535)							OPTIONAL,

nonCriticalExtension				RRCConnectionSetupComplete-v1530-IEs		OPTIONAL

}

RRCConnectionSetupComplete-v1530-IEs ::= SEQUENCE {

logMeasAvailableBT-r15				ENUMERATED {true}						OPTIONAL,

logMeasAvailableWLAN-r15			ENUMERATED {true}						OPTIONAL,

idleMeasAvailable-r15				ENUMERATED {true}						OPTIONAL,

flightPathInfoAvailable-r15			ENUMERATED {true}						OPTIONAL,

connectTo5GC-r15					ENUMERATED {true}						OPTIONAL,

registeredAMF-r15					RegisteredAMF-r15						OPTIONAL,

s-NSSAI-list-r15					SEQUENCE(SIZE (1..maxNrofS-NSSAI-r15)) OF S-NSSAI-r15 OPTIONAL,

ng-5G-S-TMSI-Bits-r15				CHOICE {

ng-5G-S-TMSI-r15					NG-5G-S-TMSI-r15,

ng-5G-S-TMSI-Part2-r15				BIT STRING (SIZE (8))

}																			OPTIONAL,

nonCriticalExtension				RRCConnectionSetupComplete-v1540-IEs	OPTIONAL

}

RRCConnectionSetupComplete-v1540-IEs ::= SEQUENCE {

gummei-Type-v1540					ENUMERATED {mappedFrom5G-v1540}		OPTIONAL,

guami-Type-r15						ENUMERATED {native, mapped}			OPTIONAL,

nonCriticalExtension				RRCConnectionSetupComplete-v1610-IEs	OPTIONAL

}

RRCConnectionSetupComplete-v1610-IEs ::= SEQUENCE {

rlos-Request-r16					ENUMERATED {true}					OPTIONAL,

cp-CIoT-5GS-Optimisation-r16		ENUMERATED {true}					OPTIONAL,

up-CIoT-5GS-Optimisation-r16		ENUMERATED {true}					OPTIONAL,

pur-ConfigID-r16					PUR-ConfigID-r16					OPTIONAL,

lte-M-r16							ENUMERATED {true}					OPTIONAL,

iab-NodeIndication-r16				ENUMERATED {true}					OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

RegisteredMME ::=					SEQUENCE {

plmn-Identity						PLMN-Identity						OPTIONAL,

mmegi								BIT STRING (SIZE (16)),

mmec								MMEC

}

RegisteredAMF-r15	::=				SEQUENCE {

plmn-Identity-r15					PLMN-Identity						OPTIONAL,

amf-Identifier-r15					AMF-Identifier-r15

}

-- ASN1STOP

| RRCConnectionSetupComplete field descriptions |
| --- |
| attachWithoutPDN-Connectivity This field is used to indicate that the UE performs an Attach without PDN connectivity procedure, as indicated by the upper layers and specified in TS 24.301 [35]. |
| cp-CIoT-5GS-Optimisation This field is included when the UE supports the Control plane CIoT 5GS optimisation, as indicated by the upper layers, see TS 24.501 [95]. |
| cp-CIoT-EPS-Optimisation This field is included when the UE supports the Control plane CIoT EPS Optimisation, as indicated by the upper layers, see TS 24.301 [35]. |
| ce-ModeB Indicates whether the UE supports operation in CE mode B, as specified in TS 36.306 [5]. |
| connectTo5GC This field is not used in the specification. It shall not be sent by the UE. |
| dcn-ID The Dedicated Core Network Identity, see TS 23.401 [41]. |
| guami-Type This field is used to indicate whether the GUAMI included is native (derived from native 5G-GUTI) or mapped (from EPS, derived from EPS GUTI) as specified in TS 24.501 [95]. |
| gummei-Type This field is used to indicate whether the GUMMEI included is native (assigned by EPC) or mapped. The value native indicates the GUMMEI is native, mapped indicates the GUMMEI is mapped from 2G/3G identifiers, and mappedFrom5G indicates the GUMMEI is mapped from 5G identifiers. A UE that sets gummei-Type-v1540 to mappedFrom5G shall also include gummei-Type-r10 and set it to native. |
| iab-NodeIndication This field is used to indicate that the connection is being established by an IAB-node as specified in TS 38.300 [106]. |
| idleMeasAvailable Indication that the UE has idle/inactive measurement report available. |
| lte-M Indicates the UE is category M. |  |
| mmegi Provides the Group Identity of the registered MME within the PLMN, as provided by upper layers, see TS 23.003 [27]. |
| mobilityState This field indicates the UE mobility state (as defined in TS 36.304 [4], clause 5.2.4.3) just prior to UE going into RRC_CONNECTED state. The UE indicates the value of medium and high when being in Medium-mobility and High-mobility states respectively. Otherwise the UE indicates the value normal. |
| ng-5G-S-TMSI-Part2 The leftmost 8 bits of 5G-S-TMSI. |
| registeredAMF This field is used to transfer the GUAMI of the AMF where the UE is registered, as provided by upper layers, see TS 23.003 [27]. |
| registeredMME This field is used to transfer the GUMMEI of the MME where the UE is registered, as provided by upper layers. |
| rlos-Request Indicates whether the UE is initiating RLOS as specified in TS 23.401 [41]. |  |
| rn-SubframeConfigReq If present, this field indicates that the connection establishment is for an RN and whether a subframe configuration is requested or not. |
| selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityList fields included in SIB1. 1 if the 1st PLMN is selected from the 1st plmn-IdentityList included in SIB1, 2 if the 2nd PLMN is selected from the same plmn-IdentityList, or when no more PLMN are present within the same plmn-IdentityList, then the PLMN listed 1st in the subsequent plmn-IdentityList within the same SIB1 and so on. |
| s-NSSAI-List This field is a list of S-NSSAI as indicated by the upper layers. The UE can report up to eight S-NSSAI per NSSAI, see TS 23.003 [27]. |
| ue-CE-NeedULGaps Indicates whether the UE needs uplink gaps during continuous uplink transmission in FDD as specified in TS 36.211 [21] and TS 36.306 [5]. |
| up-CIoT-5GS-Optimisation This field is included when the UE supports the User plane CIoT 5GS optimisation, as indicated by the upper layers, see TS 24.501 [95]. |
| up-CIoT-EPS-Optimisation This field is included when the UE supports the User plane CIoT EPS Optimisation, as indicated by the upper layers, see TS 24.301 [35]. |

#### –	RRCEarlyDataComplete

The RRCEarlyDataComplete message is used to confirm the successful completion of the CP-EDT procedure.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: E-UTRAN to UE

RRCEarlyDataComplete message

-- ASN1START

RRCEarlyDataComplete-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

rrcEarlyDataComplete-r15			RRCEarlyDataComplete-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCEarlyDataComplete-r15-IEs ::=	SEQUENCE {

dedicatedInfoNAS-r15				DedicatedInfoNAS					OPTIONAL,	-- Need ON

extendedWaitTime-r15				INTEGER (1..1800)					OPTIONAL,	-- Need ON

idleModeMobilityControlInfo-r15		IdleModeMobilityControlInfo			OPTIONAL,	-- Need OP

idleModeMobilityControlInfoExt-r15	IdleModeMobilityControlInfo-v9e0	OPTIONAL,	-- Cond IdleInfoEUTRA

redirectedCarrierInfo-r15			RedirectedCarrierInfo-r15-IEs		OPTIONAL,	-- Need ON

nonCriticalExtension				RRCEarlyDataComplete-v1590-IEs	OPTIONAL

}

RRCEarlyDataComplete-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

RedirectedCarrierInfo-r15-IEs ::=	CHOICE {

eutra-r15					ARFCN-ValueEUTRA-r9,

geran-r15					CarrierFreqsGERAN,

utra-FDD-r15				ARFCN-ValueUTRA,

cdma2000-HRPD-r15			CarrierFreqCDMA2000,

cdma2000-1xRTT-r15			CarrierFreqCDMA2000,

utra-TDD-r15				CarrierFreqListUTRA-TDD-r10

}

-- ASN1STOP

RRCEarlyDataComplete field descriptions

extendedWaitTime Value in seconds for the wait time for Delay Tolerant access requests.

| Conditional presence | Explanation |
| --- | --- |
| IdleInfoEUTRA | The field is optionally present, Need OP, if the IdleModeMobilityControlInfo-r15 is included and includes freqPriorityListEUTRA; otherwise the field is not present. |

#### –	RRCEarlyDataRequest

The RRCEarlyDataRequest message is used to initiate CP-EDT.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to E-UTRAN

RRCEarlyDataRequest message

-- ASN1START

RRCEarlyDataRequest-r15 ::=		SEQUENCE {

criticalExtensions				CHOICE {

rrcEarlyDataRequest-r15			RRCEarlyDataRequest-r15-IEs,

criticalExtensionsFuture		CHOICE {

rrcEarlyDataRequest-5GC-r16		RRCEarlyDataRequest-5GC-r16-IEs,

criticalExtensionsFuture-r16	SEQUENCE {}

}

}

}

RRCEarlyDataRequest-r15-IEs ::=	SEQUENCE {

s-TMSI-r15						S-TMSI,

establishmentCause-r15			ENUMERATED {mo-Data, delayTolerantAccess},

dedicatedInfoNAS-r15				DedicatedInfoNAS,

nonCriticalExtension				RRCEarlyDataRequest-v1590-IEs			OPTIONAL

}

RRCEarlyDataRequest-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

nonCriticalExtension					RRCEarlyDataRequest-v1610-IEs	OPTIONAL

}

RRCEarlyDataRequest-v1610-IEs ::=	SEQUENCE {

establishmentCause-v1610			ENUMERATED {mt-Access, spare3, spare2, spare1},

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

RRCEarlyDataRequest-5GC-r16-IEs ::=	SEQUENCE {

ng-5G-S-TMSI-r16					NG-5G-S-TMSI-r15,

establishmentCause-r16				ENUMERATED {mo-Data, spare3, spare2, spare1},

dedicatedInfoNAS-r16				DedicatedInfoNAS,

lateNonCriticalExtension			OCTET STRING			OPTIONAL,

nonCriticalExtension				SEQUENCE {}			OPTIONAL

}

-- ASN1STOP

RRCEarlyDataRequest field descriptions

establishmentCause Provides the establishment cause for the RRC Early Data Request as provided by the upper layers. W.r.t. the cause value names: 'mo' stands for 'Mobile Originating'. eNB is not expected to reject a RRCEarlyDataRequest due to unknown cause value being used by the UE. If establishmentCause-v1610 is included, E-UTRAN ignores establishmentCause-r15.

#### –	SCGFailureInformation

The SCGFailureInformation message is used to provide information regarding E-UTRA SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

SCGFailureInformation message

-- ASN1START

SCGFailureInformation-r12 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformation-r12			SCGFailureInformation-r12-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformation-r12-IEs ::=	SEQUENCE {

failureReportSCG-r12				FailureReportSCG-r12			OPTIONAL,

nonCriticalExtension				SCGFailureInformation-v12d0a-IEs	OPTIONAL

}

SCGFailureInformation-v12d0a-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING SCGFailureInformation-v12d0b-IEs)						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- Late non-critical extensions:

SCGFailureInformation-v12d0b-IEs ::= SEQUENCE {

failureReportSCG-v12d0				FailureReportSCG-v12d0				OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- Regular non-critical extensions:

FailureReportSCG-r12 ::=			SEQUENCE {

failureType-r12						ENUMERATED {t313-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure },

measResultServFreqList-r12			MeasResultServFreqList-r10			OPTIONAL,

measResultNeighCells-r12			MeasResultList2EUTRA-r9				OPTIONAL,

...,

[[	failureType-v1290				ENUMERATED {maxUL-TimingDiff-v1290}	OPTIONAL

]],

[[	measResultServFreqListExt-r13	MeasResultServFreqListExt-r13		OPTIONAL

]]

}

FailureReportSCG-v12d0 ::= SEQUENCE {

measResultNeighCells-v12d0			MeasResultList2EUTRA-v9e0			OPTIONAL

}

-- ASN1STOP

#### –	SCGFailureInformationNR

The SCGFailureInformationNR message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

SCGFailureInformationNR message

-- ASN1START

SCGFailureInformationNR-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformationNR-r15			SCGFailureInformationNR-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformationNR-r15-IEs ::=	SEQUENCE {

failureReportSCG-NR-r15				FailureReportSCG-NR-r15				OPTIONAL,

nonCriticalExtension					SCGFailureInformationNR-v1590-IEs	OPTIONAL

}

SCGFailureInformationNR-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension					OCTET STRING					OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, dummy},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL,

failureType-v1610				ENUMERATED {t312-Expiry, scg-lbtFailure,

beamFailureRecoveryFailure, bh-RLF-r16, spare4,

spare3, spare2, spare1}	OPTIONAL

]]

}

MeasResultFreqListFailNR-r15 ::=	SEQUENCE (SIZE (1..maxFreqNR-r15)) OF MeasResultFreqFailNR-r15

MeasResultFreqFailNR-r15 ::=		SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

measResultCellList-r15				MeasResultCellListNR-r15			OPTIONAL,

...

}

-- ASN1STOP

SCGFailureInformationNR field descriptions

failureType Indicates the cause of the SCG failure. When the field failureType-v1610 is included, the network ignores the field failureType-r15.

measResultFreqListNR The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG Includes the NR MeasResultSCG-Failure IE as specified in TS 38.331 [82]. The field contains available results of measurements on NR frequencies the UE is configured to measure by the NR RRCConfiguration message.

#### –	SCPTMConfiguration

The SCPTMConfiguration message contains the control information applicable for MBMS services transmitted via SC-MRB.

Signalling radio bearer: N/A

RLC-SAP: UM

Logical channel: SC-MCCH

Direction: E-UTRAN to UE

SCPTMConfiguration message

-- ASN1START

SCPTMConfiguration-r13 ::=		SEQUENCE {

sc-mtch-InfoList-r13			SC-MTCH-InfoList-r13,

scptm-NeighbourCellList-r13		SCPTM-NeighbourCellList-r13			OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SCPTMConfiguration-v1340			OPTIONAL

}

SCPTMConfiguration-v1340 ::= SEQUENCE {

p-b-r13								INTEGER (0..3)			OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

-- ASN1STOP

SCPTMConfiguration field descriptions

sc-mtch-InfoList Provides the configuration of each SC-MTCH in the current cell.

scptm-NeighbourCellList List of neighbour cells providing MBMS services via SC-MRB. When absent, the UE shall assume that MBMS services listed in the SCPTMConfiguration message are not provided via SC-MRB in any neighbour cell.

p-b Parameter: for the PDSCH scrambled by G-RNTI, see TS 36.213 [23], Table 5.2-1.

#### –	SCPTMConfiguration-BR

The SCPTMConfiguration-BR message contains the control information applicable for MBMS services transmitted via SC-MRB for BL UEs or UEs in CE.

Signalling radio bearer: N/A

RLC-SAP: UM

Logical channel: SC-MCCH

Direction: E-UTRAN to UE

SCPTMConfiguration-BR message

-- ASN1START

SCPTMConfiguration-BR-r14 ::=	SEQUENCE {

sc-mtch-InfoList-r14			SC-MTCH-InfoList-BR-r14,

scptm-NeighbourCellList-r14		SCPTM-NeighbourCellList-r13			OPTIONAL,	-- Need OP

p-b-r14							INTEGER (0..3)						OPTIONAL,	-- Need OR

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SCPTMConfiguration-BR-v1610			OPTIONAL

}

SCPTMConfiguration-BR-v1610 ::=	SEQUENCE {

sc-MTCH-InfoList-MultiTB-r16	SC-MTCH-InfoList-BR-r14,

multiTB-Gap-r16					ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, spare}

OPTIONAL,	-- Need OR,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

SCPTMConfiguration-BR field descriptions

p-b Parameter: for the PDSCH scrambled by G-RNTI, see TS 36.213 [23], Table 5.2-1.

multiTB-Gap Indicates scheduling gaps in sub-frames for SC-MTCH using multi-TB scheduling. Value sf2 corresponds to 2 sub-frames, value sf4 corresponds to 4 sub-frames and so on. If the field is absent, there is no scheduling gap.

sc-mtch-InfoList Provides the configuration of each SC-MTCH not using multi-TB scheduling in the current cell for BL UEs or UEs in CE.

sc-MTCH-InfoList-MultiTB Provides the configuration of each SC-MTCH using multi-TB scheduling in the current cell for BL UEs or UEs in CE. When this field is included, the total number of SC-MTCH configurations in sc-mtch-InfoList and sc-MTCH-InfoList-MultiTB cannot be more than maxSC-MTCH-BR-r14.

scptm-NeighbourCellList List of neighbour cells providing MBMS services via SC-MRB. When absent, the BL UE or UE in CE shall assume that MBMS services listed in the SCPTMConfiguration-BR message are not provided via SC-MRB in any neighbour cell.

#### –	SecurityModeCommand

The SecurityModeCommand message is used to command the activation of AS security.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

SecurityModeCommand message

-- ASN1START

SecurityModeCommand ::=				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

securityModeCommand-r8				SecurityModeCommand-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SecurityModeCommand-r8-IEs ::=		SEQUENCE {

securityConfigSMC					SecurityConfigSMC,

nonCriticalExtension				SecurityModeCommand-v8a0-IEs		OPTIONAL

}

SecurityModeCommand-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

SecurityConfigSMC ::=					SEQUENCE {

securityAlgorithmConfig					SecurityAlgorithmConfig,

...

}

-- ASN1STOP

#### –	SecurityModeComplete

The SecurityModeComplete message is used to confirm the successful completion of a security mode command.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

SecurityModeComplete message

-- ASN1START

SecurityModeComplete ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

securityModeComplete-r8				SecurityModeComplete-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

SecurityModeComplete-r8-IEs ::=		SEQUENCE {

nonCriticalExtension				SecurityModeComplete-v8a0-IEs		OPTIONAL

}

SecurityModeComplete-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	SecurityModeFailure

The SecurityModeFailure message is used to indicate an unsuccessful completion of a security mode command.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

SecurityModeFailure message

-- ASN1START

SecurityModeFailure ::=				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

securityModeFailure-r8				SecurityModeFailure-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

SecurityModeFailure-r8-IEs ::=		SEQUENCE {

nonCriticalExtension				SecurityModeFailure-v8a0-IEs		OPTIONAL

}

SecurityModeFailure-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	SidelinkUEInformation

The SidelinkUEInformation message is used for the indication of sidelink information to the eNB.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

SidelinkUEInformation message

-- ASN1START

SidelinkUEInformation-r12 ::=	SEQUENCE {

criticalExtensions				CHOICE {

c1								CHOICE {

sidelinkUEInformation-r12		SidelinkUEInformation-r12-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SidelinkUEInformation-r12-IEs ::=	SEQUENCE {

commRxInterestedFreq-r12			ARFCN-ValueEUTRA-r9				OPTIONAL,

commTxResourceReq-r12				SL-CommTxResourceReq-r12		OPTIONAL,

discRxInterest-r12					ENUMERATED {true}				OPTIONAL,

discTxResourceReq-r12				INTEGER (1..63)					OPTIONAL,

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				SidelinkUEInformation-v1310-IEs	OPTIONAL

}

SidelinkUEInformation-v1310-IEs ::=	SEQUENCE {

commTxResourceReqUC-r13				SL-CommTxResourceReq-r12				OPTIONAL,

commTxResourceInfoReqRelay-r13		SEQUENCE {

commTxResourceReqRelay-r13			SL-CommTxResourceReq-r12			OPTIONAL,

commTxResourceReqRelayUC-r13		SL-CommTxResourceReq-r12			OPTIONAL,

ue-Type-r13							ENUMERATED {relayUE, remoteUE}

}																			OPTIONAL,

discTxResourceReq-v1310			SEQUENCE {

carrierFreqDiscTx-r13			INTEGER (1..maxFreq)					OPTIONAL,

discTxResourceReqAddFreq-r13	SL-DiscTxResourceReqPerFreqList-r13		OPTIONAL

}																			OPTIONAL,

discTxResourceReqPS-r13			SL-DiscTxResourceReq-r13					OPTIONAL,

discRxGapReq-r13				SL-GapRequest-r13							OPTIONAL,

discTxGapReq-r13				SL-GapRequest-r13							OPTIONAL,

discSysInfoReportFreqList-r13	SL-DiscSysInfoReportFreqList-r13			OPTIONAL,

nonCriticalExtension			SidelinkUEInformation-v1430-IEs				OPTIONAL

}

SidelinkUEInformation-v1430-IEs ::=	SEQUENCE {

v2x-CommRxInterestedFreqList-r14	SL-V2X-CommFreqList-r14					OPTIONAL,

p2x-CommTxType-r14					ENUMERATED {true}						OPTIONAL,

v2x-CommTxResourceReq-r14			SL-V2X-CommTxFreqList-r14				OPTIONAL,

nonCriticalExtension				SidelinkUEInformation-v1530-IEs			OPTIONAL

}

SidelinkUEInformation-v1530-IEs ::=	SEQUENCE {

reliabilityInfoListSL-r15			SL-ReliabilityList-r15					OPTIONAL,

nonCriticalExtension				SEQUENCE {}								OPTIONAL

}

SL-CommTxResourceReq-r12 ::=		SEQUENCE {

carrierFreq-r12						ARFCN-ValueEUTRA-r9						OPTIONAL,

destinationInfoList-r12				SL-DestinationInfoList-r12

}

SL-DiscTxResourceReqPerFreqList-r13 ::=	SEQUENCE (SIZE (1..maxFreq)) OF SL-DiscTxResourceReq-r13

SL-DiscTxResourceReq-r13 ::=		SEQUENCE {

carrierFreqDiscTx-r13				INTEGER (1..maxFreq)					OPTIONAL,

discTxResourceReq-r13				INTEGER (1..63)

}

SL-DestinationInfoList-r12 ::=	SEQUENCE (SIZE (1..maxSL-Dest-r12)) OF SL-DestinationIdentity-r12

SL-DestinationIdentity-r12 ::=	BIT STRING (SIZE (24))

SL-DiscSysInfoReportFreqList-r13 ::=	SEQUENCE (SIZE (1.. maxSL-DiscSysInfoReportFreq-r13)) OF SL-DiscSysInfoReport-r13

SL-V2X-CommFreqList-r14 ::=	SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF INTEGER (0..maxFreqV2X-1-r14)

SL-V2X-CommTxFreqList-r14 ::=	SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF SL-V2X-CommTxResourceReq-r14

SL-V2X-CommTxResourceReq-r14 ::=	SEQUENCE {

carrierFreqCommTx-r14				INTEGER (0.. maxFreqV2X-1-r14)			OPTIONAL,

v2x-TypeTxSync-r14					SL-TypeTxSync-r14						OPTIONAL,

v2x-DestinationInfoList-r14			SL-DestinationInfoList-r12				OPTIONAL

}

-- ASN1STOP

SidelinkUEInformation field descriptions

carrierFreqCommTx Indicates the index of the frequency on which the UE is interested to transmit V2X sidelink communication. The value 1 corresponds to the frequency of first entry in v2x-InterFreqInfoList broadcast in SIB21, the value 2 corresponds to the frequency of second entry in v2x-InterFreqInfoList broadcast in SIB21 and so on. If SIB26 is broadcast and the number of entries included in v2x-InterFreqInfoList of SIB21 is N, the value N+1 corresponds to the frequency of the first entry which is included in v2x-InterFreqInfoList broadcast in SIB26 and has a frequency not included in SIB21, the value N+2 corresponds to the frequency of the second entry which is included in v2x-InterFreqInfoList broadcast in SIB26 and has a frequency not included in SIB21, and so on. The value 0 corresponds the PCell's frequency.

carrierFreqDiscTx Indicates the frequency by the index of the entry in field discInterFreqList within SystemInformationBlockType19. Value 1 corresponds to the first entry in discInterFreqList within SystemInformationBlockType19, value 2 corresponds to the second entry in this list and so on.

commRxInterestedFreq Indicates the frequency on which the UE is interested to receive sidelink communication.

commTxResourceReq Indicates the frequency on which the UE is interested to transmit non-relay related sidelink communication as well as the one-to-many sidelink communication transmission destination(s) for which the UE requests E-UTRAN to assign dedicated resources. NOTE 1.

commTxResourceReqRelay Indicates the relay related one-to-many sidelink communication transmission destination(s) for which the sidelink relay UE requests E-UTRAN to assign dedicated resources.

commTxResourceReqRelayUC Indicates the relay related one-to-one sidelink communication transmission destination(s) for which the sidelink relay UE or sidelink remote UE requests E-UTRAN to assign dedicated resources i.e. either contains the unicast destination identity of the sidelink relay UE or of the sidelink remote UE.

commTxResourceReqUC Indicates the frequency on which the UE is interested to transmit non-relay related one-to-one sidelink communication as well as the sidelink communication transmission destination(s) for which the UE requests E-UTRAN to assign dedicated resources. NOTE 1.

destinationInfoList Indicates the destination(s) for relay or non-relay related one-to-one or one-to-many sidelink communication. For one-to-one sidelink communication the destination is identified by the ProSe UE ID for unicast communication, while for one-to-many the destination it is identified by the ProSe Layer-2 Group ID as specified in TS 23.303 [68].

discRxInterest Indicates that the UE is interested to monitor sidelink discovery announcements.

discSysInfoReportFreqList Indicates, for one or more frequencies, a list of sidelink discovery related parameters acquired from system Information of cells on configured inter-frequency carriers.

discTxResourceReq Indicates the number of separate discovery message(s) the UE wants to transmit every discovery period. This field concerns the resources the UE requires every discovery period for transmitting sidelink discovery announcement(s).

discTxResourceReqAddFreq Indicates, for any frequencies in addition to the one covered by discTxResourceReq, the number of separate discovery message(s) the UE wants to transmit every discovery period. This field concerns the resources the UE requires every discovery period for transmitting sidelink discovery announcement(s).

discTxResourceReqPS Indicates the number of separate PS related discovery message(s) the UE wants to transmit every discovery period. This field concerns the resources the UE requires every discovery period for transmitting PS related sidelink discovery announcement(s).

p2x-CommTxType Indicates that the requested transmission resource pool is for P2X related V2X sidelink communication.

reliabilityInfoListSL Indicates the reliability(ies) (i.e., PPPRs as specified in TS 36.300 [9]), associated with the reported traffic to be transmitted for V2X sidelink communication.

v2x-CommRxInterestedFreqList Indicates the index(es) of the frequency(ies) on which the UE is interested to receive V2X sidelink communication. The value 1 corresponds to the frequency of first entry in v2x-InterFreqInfoList broadcast in SIB21, the value 2 corresponds to the frequency of second entry in v2x-InterFreqInfoList broadcast in SIB21 and so on. If SIB26 is broadcast and the number of entries included in v2x-InterFreqInfoList of SIB21 is N, the value N+1 corresponds to the frequency of the first entry which is included in v2x-InterFreqInfoList broadcast in SIB26 and has a frequency not included in SIB21, the value N+2 corresponds to the frequency of the second entry which is included in v2x-InterFreqInfoList broadcast in SIB26 and has a frequency not included in SIB21, and so on. The value 0 corresponds the PCell's frequency.

v2x-DestinationInfoList Indicates the destination(s) for V2X sidelink communication.

v2x-TypeTxSync Indicates the synchronization reference used by the UE.

NOTE 1:	When configuring commTxResourceReq, commTxResourceReqUC, commTxResourceReqRelay and commTxResourceReqRelayUC, E-UTRAN configures at most maxSL-Dest-r12 destinations in total (i.e. as included in the four fields together).

#### –	SystemInformation

The SystemInformation message is used to convey one or more System Information Blocks or Positioning System Information Blocks. All the SIBs or posSIBs included are transmitted with the same periodicity. SystemInformation-BR and SystemInformation-MBMS use the same structure as SystemInformation.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH and BR-BCCH

Direction: E-UTRAN to UE

SystemInformation message

-- ASN1START

SystemInformation-BR-r13 ::=	SystemInformation

SystemInformation-MBMS-r14 ::=	SystemInformation

SystemInformation ::=				SEQUENCE {

criticalExtensions					CHOICE {

systemInformation-r8				SystemInformation-r8-IEs,

criticalExtensionsFuture-r15		CHOICE {

posSystemInformation-r15			PosSystemInformation-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

}

SystemInformation-r8-IEs ::=		SEQUENCE {

sib-TypeAndInfo						SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

sib2								SystemInformationBlockType2,

sib3								SystemInformationBlockType3,

sib4								SystemInformationBlockType4,

sib5								SystemInformationBlockType5,

sib6								SystemInformationBlockType6,

sib7								SystemInformationBlockType7,

sib8								SystemInformationBlockType8,

sib9								SystemInformationBlockType9,

sib10								SystemInformationBlockType10,

sib11								SystemInformationBlockType11,

...,

sib12-v920							SystemInformationBlockType12-r9,

sib13-v920							SystemInformationBlockType13-r9,

sib14-v1130							SystemInformationBlockType14-r11,

sib15-v1130							SystemInformationBlockType15-r11,

sib16-v1130							SystemInformationBlockType16-r11,

sib17-v1250							SystemInformationBlockType17-r12,

sib18-v1250							SystemInformationBlockType18-r12,

sib19-v1250							SystemInformationBlockType19-r12,

sib20-v1310							SystemInformationBlockType20-r13,

sib21-v1430							SystemInformationBlockType21-r14,

sib24-v1530							SystemInformationBlockType24-r15,

sib25-v1530							SystemInformationBlockType25-r15,

sib26-v1530							SystemInformationBlockType26-r15,

sib26a-v1610							SystemInformationBlockType26a-r16,

sib27-v1610							SystemInformationBlockType27-r16,

sib28-v1610							SystemInformationBlockType28-r16,

sib29-v1610							SystemInformationBlockType29-r16,

sibXX-v17xx							SystemInformationBlockTypeXX-r17

},

nonCriticalExtension				SystemInformation-v8a0-IEs		OPTIONAL

}

SystemInformation-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

PosSystemInformation-r15-IEs ::= SEQUENCE {

posSIB-TypeAndInfo-r15			SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

posSib1-1-r15					SystemInformationBlockPos-r15,

posSib1-2-r15					SystemInformationBlockPos-r15,

posSib1-3-r15					SystemInformationBlockPos-r15,

posSib1-4-r15					SystemInformationBlockPos-r15,

posSib1-5-r15					SystemInformationBlockPos-r15,

posSib1-6-r15					SystemInformationBlockPos-r15,

posSib1-7-r15					SystemInformationBlockPos-r15,

posSib2-1-r15					SystemInformationBlockPos-r15,

posSib2-2-r15					SystemInformationBlockPos-r15,

posSib2-3-r15					SystemInformationBlockPos-r15,

posSib2-4-r15					SystemInformationBlockPos-r15,

posSib2-5-r15					SystemInformationBlockPos-r15,

posSib2-6-r15					SystemInformationBlockPos-r15,

posSib2-7-r15					SystemInformationBlockPos-r15,

posSib2-8-r15					SystemInformationBlockPos-r15,

posSib2-9-r15					SystemInformationBlockPos-r15,

posSib2-10-r15					SystemInformationBlockPos-r15,

posSib2-11-r15					SystemInformationBlockPos-r15,

posSib2-12-r15					SystemInformationBlockPos-r15,

posSib2-13-r15					SystemInformationBlockPos-r15,

posSib2-14-r15					SystemInformationBlockPos-r15,

posSib2-15-r15					SystemInformationBlockPos-r15,

posSib2-16-r15					SystemInformationBlockPos-r15,

posSib2-17-r15					SystemInformationBlockPos-r15,

posSib2-18-r15					SystemInformationBlockPos-r15,

posSib2-19-r15					SystemInformationBlockPos-r15,

posSib3-1-r15					SystemInformationBlockPos-r15,

...,

[[

posSib1-8-v1610					SystemInformationBlockPos-r15,

posSib2-20-v1610				SystemInformationBlockPos-r15,

posSib2-21-v1610				SystemInformationBlockPos-r15,

posSib2-22-v1610				SystemInformationBlockPos-r15,

posSib2-23-v1610				SystemInformationBlockPos-r15,

posSib2-24-v1610					SystemInformationBlockPos-r15,

posSib2-25-v1610					SystemInformationBlockPos-r15,

posSib4-1-v1610					SystemInformationBlockPos-r15,

posSib5-1-v1610					SystemInformationBlockPos-r15

]]

},

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

nonCriticalExtension			SEQUENCE {}								OPTIONAL

}

-- ASN1STOP

#### –	SystemInformationBlockType1

SystemInformationBlockType1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. SystemInformationBlockType1-BR uses the same structure as SystemInformationBlockType1.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH and BR-BCCH

Direction: E-UTRAN to UE

SystemInformationBlockType1 message

-- ASN1START

SystemInformationBlockType1-BR-r13 ::=	SystemInformationBlockType1

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

SystemInformationBlockType1-v890-IEs::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING SystemInformationBlockType1-v8h0-IEs)			OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-v920-IEs	OPTIONAL

}

-- Late non critical extensions

SystemInformationBlockType1-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList					MultiBandInfoList		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v9e0-IEs	OPTIONAL

}

SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE {

freqBandIndicator-v9e0				FreqBandIndicator-v9e0		OPTIONAL,	-- Cond FBI-max

multiBandInfoList-v9e0				MultiBandInfoList-v9e0		OPTIONAL,	-- Cond mFBI-max

nonCriticalExtension				SystemInformationBlockType1-v10j0-IEs	OPTIONAL

}

SystemInformationBlockType1-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v10l0-IEs					OPTIONAL

}

SystemInformationBlockType1-v10l0-IEs ::= SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v10x0-IEs		OPTIONAL

}

SystemInformationBlockType1-v10x0-IEs ::=	SEQUENCE {

-- This field is only for late non-critical extensions from Rel-10 or Rel-11 onwards

lateNonCriticalExtension			OCTET STRING								OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-v12j0-IEs		OPTIONAL

}

SystemInformationBlockType1-v12j0-IEs ::=	SEQUENCE {

schedulingInfoList-v12j0			SchedulingInfoList-v12j0	OPTIONAL,	-- Need OR

schedulingInfoListExt-r12			SchedulingInfoListExt-r12	OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v15g0-IEs	OPTIONAL

}

SystemInformationBlockType1-v15g0-IEs ::= SEQUENCE {

bandwidthReducedAccessRelatedInfo-v15g0	SEQUENCE {

posSchedulingInfoList-BR-r15	SchedulingInfoList-BR-r13

}																OPTIONAL,	-- Need OR

nonCriticalExtension						SEQUENCE {}		OPTIONAL

}

-- Regular non critical extensions

SystemInformationBlockType1-v920-IEs ::=	SEQUENCE {

ims-EmergencySupport-r9				ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfo-v920				CellSelectionInfo-v920		OPTIONAL,	-- Cond RSRQ

nonCriticalExtension				SystemInformationBlockType1-v1130-IEs	OPTIONAL

}

SystemInformationBlockType1-v1130-IEs ::=	SEQUENCE {

tdd-Config-v1130				TDD-Config-v1130			OPTIONAL,	-- Cond TDD-OR

cellSelectionInfo-v1130			CellSelectionInfo-v1130		OPTIONAL,	-- Cond WB-RSRQ

nonCriticalExtension			SystemInformationBlockType1-v1250-IEs	OPTIONAL

}

SystemInformationBlockType1-v1250-IEs ::=	SEQUENCE {

cellAccessRelatedInfo-v1250					SEQUENCE {

category0Allowed-r12						ENUMERATED {true}		OPTIONAL	-- Need OP

},

cellSelectionInfo-v1250					CellSelectionInfo-v1250		OPTIONAL,	-- Cond RSRQ2

freqBandIndicatorPriority-r12			ENUMERATED {true}			OPTIONAL,	-- Cond mFBI

nonCriticalExtension			SystemInformationBlockType1-v1310-IEs	OPTIONAL

}

SystemInformationBlockType1-v1310-IEs ::=	SEQUENCE {

hyperSFN-r13								BIT STRING (SIZE (10))		OPTIONAL,	-- Need OR

eDRX-Allowed-r13							ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13	OPTIONAL,	-- Need OP

bandwidthReducedAccessRelatedInfo-r13	SEQUENCE {

si-WindowLength-BR-r13					ENUMERATED {

ms20, ms40, ms60, ms80, ms120,

ms160, ms200, spare},

si-RepetitionPattern-r13				ENUMERATED {everyRF, every2ndRF, every4thRF,

every8thRF},

schedulingInfoList-BR-r13				SchedulingInfoList-BR-r13	OPTIONAL,	-- Cond SI-BR

fdd-DownlinkOrTddSubframeBitmapBR-r13	CHOICE {

subframePattern10-r13					BIT STRING (SIZE (10)),

subframePattern40-r13					BIT STRING (SIZE (40))

}																	OPTIONAL,	-- Need OP

fdd-UplinkSubframeBitmapBR-r13			BIT STRING (SIZE (10))		OPTIONAL,	-- Need OP

startSymbolBR-r13						INTEGER (1..4),

si-HoppingConfigCommon-r13				ENUMERATED {on,off},

si-ValidityTime-r13						ENUMERATED {true}	OPTIONAL,			-- Need OP

systemInfoValueTagList-r13				SystemInfoValueTagList-r13	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond BW-reduced

nonCriticalExtension						SystemInformationBlockType1-v1320-IEs	OPTIONAL

}

SystemInformationBlockType1-v1320-IEs ::=	SEQUENCE {

freqHoppingParametersDL-r13				SEQUENCE {

mpdcch-pdsch-HoppingNB-r13				ENUMERATED {nb2, nb4}		OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeA-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13					ENUMERATED {int1, int5, int10, int20}

}																	OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeB-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13					ENUMERATED { int5, int10, int20, int40}

}																	OPTIONAL,	-- Need OR

mpdcch-pdsch-HoppingOffset-r13			INTEGER (1..maxAvailNarrowBands-r13)	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond Hopping

nonCriticalExtension						SystemInformationBlockType1-v1350-IEs					OPTIONAL

}

SystemInformationBlockType1-v1350-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-r13				CellSelectionInfoCE1-r13	OPTIONAL,	-- Need OP

nonCriticalExtension					SystemInformationBlockType1-v1360-IEs				OPTIONAL

}

SystemInformationBlockType1-v1360-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-v1360				CellSelectionInfoCE1-v1360	OPTIONAL,	-- Cond QrxlevminCE1

nonCriticalExtension						SystemInformationBlockType1-v1430-IEs		OPTIONAL

}

SystemInformationBlockType1-v1430-IEs ::=	SEQUENCE {

eCallOverIMS-Support-r14				ENUMERATED {true}			OPTIONAL,	-- Need OR

tdd-Config-v1430						TDD-Config-v1430			OPTIONAL,	-- Cond TDD-OR

cellAccessRelatedInfoList-r14			SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF

CellAccessRelatedInfo-r14	OPTIONAL,	-- Need OR

nonCriticalExtension					SystemInformationBlockType1-v1450-IEs				OPTIONAL

}

SystemInformationBlockType1-v1450-IEs ::=	SEQUENCE {

tdd-Config-v1450						TDD-Config-v1450		OPTIONAL,	-- Cond TDD-OR

nonCriticalExtension					SystemInformationBlockType1-v1530-IEs					OPTIONAL

}

SystemInformationBlockType1-v1530-IEs ::=	SEQUENCE {

hsdn-Cell-r15						ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfoCE-v1530			CellSelectionInfoCE-v1530	OPTIONAL,	-- Need OP

crs-IntfMitigConfig-r15				CHOICE {

crs-IntfMitigEnabled					NULL,

crs-IntfMitigNumPRBs			ENUMERATED {n6, n24}

}	OPTIONAL,	-- Need OR

cellBarred-CRS-r15					ENUMERATED {barred, notBarred},

plmn-IdentityList-v1530				PLMN-IdentityList-v1530		OPTIONAL,	-- Need OR

posSchedulingInfoList-r15			PosSchedulingInfoList-r15	OPTIONAL,	-- Need OR

cellAccessRelatedInfo-5GC-r15		SEQUENCE {

cellBarred-5GC-r15					ENUMERATED {barred, notBarred},

cellBarred-5GC-CRS-r15				ENUMERATED {barred, notBarred},

cellAccessRelatedInfoList-5GC-r15	SEQUENCE (SIZE (1..maxPLMN-r11)) OF

CellAccessRelatedInfo-5GC-r15

}				OPTIONAL,	-- Need OP

ims-EmergencySupport5GC-r15			ENUMERATED {true}			OPTIONAL,	-- Need OR

eCallOverIMS-Support5GC-r15			ENUMERATED {true}			OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v1540-IEs		OPTIONAL

}

SystemInformationBlockType1-v1540-IEs ::=	SEQUENCE {

si-posOffset-r15								ENUMERATED {true}		OPTIONAL,	-- Need ON

nonCriticalExtension							SystemInformationBlockType1-v1610-IEs	OPTIONAL

}

SystemInformationBlockType1-v1610-IEs ::=	SEQUENCE {

eDRX-Allowed-5GC-r16					ENUMERATED {true}		OPTIONAL,	-- Need OR

transmissionInControlChRegion-r16	ENUMERATED {true}		OPTIONAL,	-- Cond BW-reduced

campingAllowedInCE-r16				ENUMERATED {true}			OPTIONAL,	-- Need OR

plmn-IdentityList-v1610				PLMN-IdentityList-v1610		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v17xx-IEs	OPTIONAL

}

SystemInformationBlockType1-v17xx-IEs ::=	SEQUENCE {

cellAccessRelatedInfoList-NTN-r17		SEQUENCE (SIZE (1..maxPLMN-r11)) OF

CellAccessRelatedInfo-NTN-r17	OPTIONAL, -- Need OR

nonCriticalExtension					SEQUENCE {}			OPTIONAL

}

PLMN-IdentityList ::=					SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellReservedForOperatorUse				ENUMERATED {reserved, notReserved}

}

PLMN-IdentityList-v1530 ::=				SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-v1530

PLMN-IdentityInfo-v1530 ::=				SEQUENCE {

cellReservedForOperatorUse-CRS-r15		ENUMERATED {reserved, notReserved}

}

PLMN-IdentityList-r15::=			SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-r15

PLMN-IdentityList-v1610::=	SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-v1610

PLMN-IdentityInfo-r15 ::=			SEQUENCE {

plmn-Identity-5GC-r15				CHOICE{

plmn-Identity-r15					PLMN-Identity,

plmn-Index-r15						INTEGER (1..maxPLMN-r11)

},

cellReservedForOperatorUse-r15			ENUMERATED {reserved, notReserved},

cellReservedForOperatorUse-CRS-r15		ENUMERATED {reserved, notReserved}

}

PLMN-IdentityInfo-v1610 ::=	SEQUENCE {

cp-CIoT-5GS-Optimisation-r16	ENUMERATED {true}			OPTIONAL,	-- Need OR

up-CIoT-5GS-Optimisation-r16	ENUMERATED {true}			OPTIONAL,	-- Need OR

iab-Support-r16				ENUMERATED {true}			OPTIONAL	-- Need OR

}

SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo

SchedulingInfoList-v12j0 ::=	SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-v12j0

SchedulingInfoListExt-r12 ::=	SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfoExt-r12

SchedulingInfo ::=	SEQUENCE {

si-Periodicity				SI-Periodicity-r12,

sib-MappingInfo				SIB-MappingInfo

}

SchedulingInfo-v12j0 ::=	SEQUENCE {

sib-MappingInfo-v12j0		SIB-MappingInfo-v12j0				OPTIONAL	-- Need OR

}

SchedulingInfoExt-r12 ::=	SEQUENCE {

si-Periodicity-r12			SI-Periodicity-r12,

sib-MappingInfo-r12			SIB-MappingInfo-v12j0

}

SchedulingInfoList-BR-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13

SchedulingInfo-BR-r13 ::=	SEQUENCE {

si-Narrowband-r13		INTEGER (1..maxAvailNarrowBands-r13),

si-TBS-r13				ENUMERATED {b152, b208, b256, b328, b408, b504, b600, b712, b808, b936}

}

SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type

SIB-MappingInfo-v12j0 ::=	SEQUENCE (SIZE (1..maxSIB-1)) OF SIB-Type-v12j0

SIB-Type ::=						ENUMERATED {

sibType3, sibType4, sibType5, sibType6,

sibType7, sibType8, sibType9, sibType10,

sibType11, sibType12-v920, sibType13-v920,

sibType14-v1130, sibType15-v1130,

sibType16-v1130, sibType17-v1250, sibType18-v1250,

..., sibType19-v1250, sibType20-v1310, sibType21-v1430,

sibType24-v1530, sibType25-v1530, sibType26-v1530,

sibType26a-v1610, sibType27-v1610, sibType28-v1610,

sibType29-v1610, sibTypeXX-v17xx}

SIB-Type-v12j0 ::=			ENUMERATED {

sibType19-v1250, sibType20-v1310, sibType21-v1430,

sibType24-v1530, sibType25-v1530, sibType26-v1530,

sibType26a-v1610, sibType27-v1610, sibType28-v1610,

sibType29-v1610, sibTypeXX-v17xx, spare5,

spare4, spare3, spare2, spare1, ...}

SI-Periodicity-r12 ::=		ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512}

SystemInfoValueTagList-r13 ::=		SEQUENCE (SIZE (1..maxSI-Message)) OF SystemInfoValueTagSI-r13

SystemInfoValueTagSI-r13 ::=		INTEGER (0..3)

CellSelectionInfo-v920 ::=			SEQUENCE {

q-QualMin-r9						Q-QualMin-r9,

q-QualMinOffset-r9					INTEGER (1..8)						OPTIONAL	-- Need OP

}

CellSelectionInfo-v1130 ::=			SEQUENCE {

q-QualMinWB-r11						Q-QualMin-r9

}

CellSelectionInfo-v1250 ::=			SEQUENCE {

q-QualMinRSRQ-OnAllSymbols-r12		Q-QualMin-r9

}

CellAccessRelatedInfo-r14 ::=	SEQUENCE {

plmn-IdentityList-r14				PLMN-IdentityList,

trackingAreaCode-r14				TrackingAreaCode,

cellIdentity-r14					CellIdentity

}

CellAccessRelatedInfo-5GC-r15 ::=	SEQUENCE {

plmn-IdentityList-r15			PLMN-IdentityList-r15,

ran-AreaCode-r15					RAN-AreaCode-r15 OPTIONAL,	-- Need OR

trackingAreaCode-5GC-r15			TrackingAreaCode-5GC-r15,

cellIdentity-5GC-r15				CellIdentity-5GC-r15

}

CellIdentity-5GC-r15 ::= CHOICE{

cellIdentity-r15	CellIdentity,

cellId-Index-r15	INTEGER (1..maxPLMN-r11)

}

CellAccessRelatedInfo-NTN-r17 ::=	SEQUENCE {

plmn-IdentityList-r17				PLMN-IdentityList,

trackingAreaList-r17				TrackingAreaList-r17,

cellIdentity-r17					CellIdentity,

cellReservedForOperatorUse-CRS-r17	ENUMERATED {reserved, notReserved}

}

TrackingAreaList-r17 ::= SEQUENCE (SIZE (1..maxTAC-r17)) OF  TrackingAreaCode

PosSchedulingInfoList-r15 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF PosSchedulingInfo-r15

PosSchedulingInfo-r15 ::=	SEQUENCE {

posSI-Periodicity-r15		ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

posSIB-MappingInfo-r15		PosSIB-MappingInfo-r15

}

PosSIB-MappingInfo-r15 ::= SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r15

PosSIB-Type-r15 ::= SEQUENCE {

encrypted-r15		ENUMERATED { true }				OPTIONAL,		-- Need OP

gnss-id-r15			GNSS-ID-r15						OPTIONAL,		-- Need OP

sbas-id-r15			SBAS-ID-r15						OPTIONAL,		-- Need OP

posSibType-r15		ENUMERATED {	posSibType1-1,

posSibType1-2,

posSibType1-3,

posSibType1-4,

posSibType1-5,

posSibType1-6,

posSibType1-7,

posSibType2-1,

posSibType2-2,

posSibType2-3,

posSibType2-4,

posSibType2-5,

posSibType2-6,

posSibType2-7,

posSibType2-8,

posSibType2-9,

posSibType2-10,

posSibType2-11,

posSibType2-12,

posSibType2-13,

posSibType2-14,

posSibType2-15,

posSibType2-16,

posSibType2-17,

posSibType2-18,

posSibType2-19,

posSibType3-1,

...,

posSibType1-8-v1610,

posSibType2-20-v1610,

posSibType2-21-v1610,

posSibType2-22-v1610,

posSibType2-23-v1610,

posSibType2-24-v1610,

posSibType2-25-v1610,

posSibType4-1-v1610,

posSibType5-1-v1610

},

...

}

-- ASN1STOP

Editor’s Note: Agreement: Satellite assistance information will be used by the UE for predicting coverage discontinuity. The details of the assistance information is FFS. FFS whether any applicable agreements made in NR-NTN can be reused. Editor: this agreement is captured here until it is decided in which SIB the assistance information is located.

Editor’s Note: Agreement: RAN2 assumes that Satellite assistance information, e.g. for cell selection reselection, for serving cell is provided to UE. Editor: this agreement is captured here until it is decided in which SIB the assistance information is located.

Editor’s Note: Agreement: FFS if Satellite assistance information for neighbour cell(s) is provided to UE for cell selection/reselection (justification would be needed). Editor: this agreement is captured here until it is decided in which SIB the assistance information is located.

Editor’s Note: Satellite Ephemeris Parameters (not same as for L1 pre-compensation, for the constellation, not just single satellite) is needed for the UE for predicting coverage discontinuity. Other info, e.g. beam info, elevation angle, reference location or corresponding is FFS. Editor: this agreement is captured here until it is decided in which SIB the assistance information is located.

Editor’s Note: Providing the start-time of (incoming) satellite’s coverage and end-time of serving satellite’s coverage is needed for Quasi-Earth Fixed satellites. Editor: this agreement is captured here until it is decided in which SIB the incoming satellite assistance information is located.

Editor’s Note: Agreement: When the network stops broadcasting a TAC, the UE needs to know it. FFS how this is done. Editor: this agreement is captured here until the FFS is resolved.

Editor’s Note: Agreement: It is feasible to use the legacy barring bit to block legacy UEs, and it is possible to have a new bit that assumes the functionality of the old bit. It is FFS if it is needed to use the barring bit or whether other mechanism can be assumed (new band etc).

Editor’s Note: Agreement: K_mac value is broadcasted by network. Editor: this agreement is captured here until it is decided in which SIB the information is located.

| SystemInformationBlockType1 field descriptions |
| --- |
| bandwithReducedAccessRelatedInfo Access related information for BL UEs and UEs in CE. NOTE 3. |
| campingAllowedInCE Indicates whether non-BL UE is allowed to camp in the non-standalone BL cell in enhanced coverage mode when S-criterion for normal coverage is fulfilled. The field is not applicable for standalone BL cell. |
| category0Allowed The presence of this field indicates category 0 UEs are allowed to access the cell. |
| cellAccessRelatedInfoList This field contains a list allowing signalling of access related information per PLMN. One PLMN can be included in only one entry of this list. NOTE 4. |
| cellAccessRelatedInfoList-5GC This field contains a PLMN list and a list allowing signalling of access related information per PLMN for PLMNs that provides connectivity to 5GC. One PLMN can be included in only one entry of this list. NOTE4 |
| cellAccessRelatedInfoList-NTN This field contains a PLMN list and a list allowing signalling of access related information per PLMN for PLMNs that provides connectivity via NTN. One PLMN can be included in only one entry of this list. NOTE4. If this field is included, cellAccessRelatedInfoList is not included and the UE ignores the corresponding parameters signalled in cellAccesRelateInfo. |
| cellBarred, cellBarred-CRS barred means the cell is barred, as defined in TS 36.304 [4]. |
| cellBarred-5GC, cellBarred-5GC-CRS barred means the cell is barred for connectivity to 5GC, as defined in TS 36.304 [4]. |
| cellIdentity Indicates the cell identity. NOTE 2. |
| cellId-Index The index of the cell ID in the PLMN lists for EPC, indicates UE the corresponding cell ID is used for 5GC. Value 1 indicates the cell ID of the 1st PLMN list for EPC in the SIB1. Value 2 indicates the cell ID of the 2nd PLMN list for EPC, and so on. |
| cellReservedForOperatorUse, cellReservedForOperatorUse-CRS As defined in TS 36.304 [4]. |
| cellSelectionInfoCE Cell selection information for BL UEs and UEs in CE. If absent, coverage enhancement S criteria is not applicable. NOTE 3. |
| cellSelectionInfoCE1 Cell selection information for BL UEs and UEs in CE supporting CE Mode B. E-UTRAN includes this IE only if cellSelectionInfoCE is present in SystemInformationBlockType1-BR. NOTE 3. |  |
| cp-CIoT-5GS-Optimisation Indicates whether the UE is allowed to establish the connection with Control plane CIoT 5GS optimisation, see TS 24.501 [95]. |
| crs-IntfMitigConfig crs-IntfMitigEnabled indicates CRS interference mitigation is enabled for the cell, as specified in TS 36.133 [16], clause 3.6.1.1. For BL UEs supporting ce-CRS-IntfMitig, presence of crs-IntfMitigNumPRBs indicates CRS interference mitigation is enabled in the cell, as specified in TS 36.133 [16], clauses 3.6.1.2 and 3.6.1.3, and the value of crs-IntfMitigNumPRBs indicates number of PRBs, i.e. 6 or 24 PRBs, for CRS transmission in the central cell BW when CRS interference mitigation is enabled. For UEs not supporting this feature, the behaviour is undefined if this field is configured and the field cellBarred in SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs or UEs in CE) is set to notbarred. |  |
| csg-Identity Identity of the Closed Subscriber Group the cell belongs to. |
| csg-Indication If set to TRUE the UE is only allowed to access the cell if it is a CSG member cell, if selected during manual CSG selection or to obtain limited service, see TS 36.304 [4]. |
| eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services via EPC for UEs as defined in TS 23.401 [41]. If absent, eCall over IMS via EPC is not supported by the network in the cell. NOTE 2. |
| eCallOverIMS-Support5GC Indicates whether the cell supports eCall over IMS services via 5GC as defined in TS 23.401 [41]. If absent, eCall over IMS via 5GC is not supported by the network in the cell. NOTE 2. |
| eDRX-Allowed The presence of this field indicates if idle mode extended DRX is allowed in the cell for the UE connected to EPC. The UE shall stop using extended DRX in idle mode if eDRX-Allowed is not present when connected to EPC. |
| eDRX-Allowed-5GC The presence of this field indicates if idle mode extended DRX is allowed in the cell for the UE connected to 5GC. The UE shall stop using extended DRX in idle mode if eDRX-Allowed-5GC is not present when connected to 5GC. |
| encrypted The presence of this field indicates that the posSibType is encrypted as specified in TS 36.355 [54]. |
| fdd-DownlinkOrTddSubframeBitmapBR The set of valid subframes for FDD downlink or TDD transmissions, see TS 36.213 [23]. If this field is present, SystemInformationBlockType1-BR-r13 is transmitted in RRCConnectionReconfiguration, and if RRCConnectionReconfiguration does not include systemInformationBlockType2Dedicated, UE may assume the valid subframes in fdd-DownlinkOrTddSubframeBitmapBR are not indicated as MBSFN subframes. If this field is not present, the set of valid subframes is the set of non-MBSFN subframes as indicated by mbsfn-SubframeConfigList. If neither this field nor mbsfn-SubframeConfigList is present, all subframes are considered as valid subframes for FDD downlink transmission, all DL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD DL transmission, and all UL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD UL transmission. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| fdd-UplinkSubframeBitmapBR The set of valid subframes for FDD uplink transmissions for BL UEs, see TS 36.213 [23]. If the field is not present, then UE considers all uplink subframes as valid subframes for FDD uplink transmissions. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| freqBandIndicatorPriority If the field is present and supported by the UE, the UE shall prioritize the frequency bands in the multiBandInfoList field in decreasing priority order. Only if the UE does not support any of the frequency band in multiBandInfoList, the UE shall use the value in freqBandIndicator field. Otherwise, the UE applies frequency band according to the rules defined in multiBandInfoList. NOTE 2. |
| freqBandInfo A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs and TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs, for the frequency band in freqBandIndicator. If E-UTRAN includes freqBandInfo-v10l0 it includes the same number of entries, and listed in the same order, as in freqBandInfo-r10. |
| freqHoppingParametersDL Downlink frequency hopping parameters for BR versions of SI messages, MPDCCH/PDSCH of paging, MPDCCH/PDSCH of RAR/Msg4 and unicast MPDCCH/PDSCH. If not present, the UE is not configured downlink frequency hopping. |
| gnss-ID The presence of this field indicates that the posSibType is for a specific GNSS. |
| hsdn-Cell This field indicates this is a HSDN cell as specified in TS 36.304 [4]. |
| hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around. |
| iab-Support This field combines both the support of IAB-node and the cell status for IAB-node. If the field is present, the cell supports IAB-nodes and the cell is also considered as a candidate for cell (re)selection for IAB-nodes; if the field is absent, the cell does not support IAB and/or the cell is barred for IAB-node. |  |
| ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services via EPC for UEs in limited service mode. If absent, IMS emergency call via EPC is not supported by the network in the cell for UEs in limited service mode. NOTE 2. |
| ims-EmergencySupport5GC Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode via 5GC. If absent, IMS emergency call via 5GC is not supported by the network in the cell for UEs in limited service mode. NOTE 2. |
| intraFreqReselection Used to control cell reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 36.304 [4]. NOTE 2. |
| multiBandInfoList A list of additional frequency band indicators, as defined in TS 36.101 [42], table 5.5-1, that the cell belongs to. If the UE supports the frequency band in the freqBandIndicator field it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList field. If E-UTRAN includes multiBandInfoList-v9e0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). See Annex D for more descriptions. The UE shall ignore the rule defined in this field description if freqBandIndicatorPriority is present and supported by the UE. |
| multiBandInfoList-v10j0 A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs and TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs, for the frequency bands in multiBandInfoList (i.e. without suffix) and multiBandInfoList-v9e0. If E-UTRAN includes multiBandInfoList-v10j0, it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList-v10j0. |
| plmn-IdentityList List of PLMN identities. The first listed PLMN-Identity is the primary PLMN. If plmn-IdentityList-v1530 is included, E-UTRAN includes the same number of entries, and listed in the same order, as in plmn-IdentityList (without suffix). If plmn-IdentityList-v1610 is included, E-UTRAN includes the same number of entries, and listed in the same order, as in plmn-IdentityList-r15. NOTE 2. |
| plmn-Index Index of the PLMN in the plmn-IdentityList fields included in SIB1 for EPC, indicating the same PLMN ID is connected to 5GC. Value 1 indicates the 1st PLMN in the 1st plmn-IdentityList included in SIB1, value 2 indicates the 2nd PLMN in the same plmn-IdentityList, or when no more PLMNs are present within the same plmn-IdentityList, then the PLMN listed 1st in the subsequent plmn-IdentityList within the same SIB1 and so on. NOTE 6. |
| p-Max Value applicable for the cell. If absent the UE applies the maximum power according to its capability as specified in TS 36.101 [42], clause 6.2.2. NOTE 2. This field is ignored by IAB-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [107]. |
| posSchedulingInfoList-BR Indicates additional scheduling information of positioning SI messages for BL UEs and UEs in CE. E-UTRAN always includes this field if posSchedulingInfoList-r15 is included in SystemInformationBlockType1-BR, and includes the same number of entries, and listed in the same order, as in posSchedulingInfoList-r15. |
| posSIB-MappingInfo List of the posSIBs mapped to this SystemInformation message. |
| posSibType The positioning SIB type is defined in TS 36.355 [54]. |
| q-QualMin Parameter "Qqualmin" in TS 36.304 [4]. If cellSelectionInfo-v920 is not present, the UE applies the (default) value of negative infinity for Qqualmin. NOTE 1. |
| q-QualMinRSRQ-OnAllSymbols If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. NOTE 1. |
| q-QualMinOffset Parameter "Qqualminoffset" in TS 36.304 [4]. Actual value Qqualminoffset = field value [dB]. If cellSelectionInfo-v920 is not present or the field is not present, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell. |
| q-QualMinWB If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16]. NOTE 1. |
| q-RxLevMinOffset Parameter Qrxlevminoffset in TS 36.304 [4]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell. |
| sbas-ID The presence of this field indicates that the posSibType is for a specific SBAS. |
| schedulingInfoList Indicates scheduling information of SI messages. The schedulingInfoList-v12j0 (if present) provides additional SIBs mapped into the SI message scheduled via schedulingInfoList (without suffix). If E-UTRAN includes schedulingInfoList-v12j0, it includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| schedulingInfoListExt Indicates scheduling information of additional SI messages. The UE concatenates the entries of schedulingInfoListExt to the entries in schedulingInfoList, according to the general concatenation principles for list extension as defined in 5.1.2. If the schedulingInfoListExt is present, E-UTRAN ensures that the total number of entries of this field plus schedulingInfoList (without suffix) shall not exceed the value of maxSI-Message. |
| sib-MappingInfo List of the SIBs mapped to this SystemInformation message. There is no mapping information of SIB2; it is always present in the first SystemInformation message listed in the schedulingInfoList (without suffix) list. If present, sib-MappingInfo-v12j0 indicates one or more additional SIBs mapped to the concerned SI message listed in the schedulingInfoList (without suffix) list. If schedulingInfoList-v12j0 or schedulingInfoListExt-r12 is present, E-UTRAN does not include any value indicating SIB of type 19 or higher in sib-MappingInfo (without suffix). If schedulingInfoList-v12j0 is present, E-UTRAN ensures that the total number of entries of this field plus sib-MappingInfo (without suffix) shall not exceed the value of maxSIB-1. |
| si-HoppingConfigCommon Frequency hopping activation/deactivation for BR versions of SI messages and MPDCCH/PDSCH of paging. |
| si-Narrowband This field indicates the index of a narrowband used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.211 [21], clause 6.4.1 and TS 36.213 [23], clause 7.1.6. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..maxAvailNarrowBands-r13-1) as specified in TS 36.211 [21]. |
| si-RepetitionPattern Indicates the radio frames within the SI window used for SI message transmission. Value everyRF corresponds to every radio frame, value every2ndRF corresponds to every 2 radio frames, and so on. The first transmission of the SI message is transmitted from the first radio frame of the SI window. |
| si-Periodicity, posSI-Periodicity Periodicity of the SI-message in radio frames, such that rf8 denotes 8 radio frames, rf16 denotes 16 radio frames, and so on. If the si-posOffset is configured, the posSI-Periodicity of rf8 cannot be used. |
| si-posOffset This field, if present and set to true indicates that the SI messages in PosSchedulingInfoList are scheduled with an offset of 8 radio frames compared to SI messages in SchedulingInfoList. si-posOffset may be present only if the shortest configured SI message periodicity for SI messages in SchedulingInfoList is 80ms. |
| si-TBS This field indicates the transport block size information used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.213 [23], Table 7.1.7.2.1-1, for a 6 PRB bandwidth and a QPSK modulation. |
| schedulingInfoList-BR Indicates additional scheduling information of SI messages for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| si-ValidityTime Indicates system information validity timer. If set to TRUE, the timer is set to 3h, otherwise the timer is set to 24h. |
| si-WindowLength, si-WindowLength-BR Common SI scheduling window for all SIs. Unit in milliseconds, where ms1 denotes 1 millisecond, ms2 denotes 2 milliseconds and so on. In case si-WindowLength-BR-r13 is present and the UE is a BL UE or a UE in CE, the UE shall use si-WindowLength-BR-r13 and ignore the original field si-WindowLength (without suffix). UEs other than BL UEs or UEs in CE shall ignore the extension field si-WindowLength-BR-r13. |
| startSymbolBR For BL UEs and UEs in CE, indicates the OFDM starting symbol for any MPDCCH, PDSCH scheduled on the same cell except the PDSCH carrying SystemInformationBlockType1-BR, see TS 36.213 [23]. Values 1, 2, and 3 are applicable for dl-Bandwidth greater than 10 resource blocks. Values 2, 3, and 4 are applicable otherwise. |
| systemInfoValueTagList Indicates SI message specific value tags for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| systemInfoValueTagSI SI message specific value tag as specified in clause 5.2.1.3. Common for all SIBs within the SI message other than MIB, SIB1, SIB10, SIB11, SIB12 and SIB14. |
| systemInfoValueTag Common for all SIBs other than MIB, MIB-MBMS, SIB1, SIB1-MBMS, SIB10, SIB11, SIB12 and SIB14. Change of MIB, MIB-MBMS, SIB1 and SIB1-MBMS is detected by acquisition of the corresponding message. |
| tdd-Config Specifies the TDD specific physical channel configurations. NOTE 2. |
| trackingAreaCode/trackingAreaCode-5GC A trackingAreaCode that is common for all the PLMNs listed. NOTE2. NOTE 5. |
| trackingAreaList A list of additional tracking area codes that is common for all the PLMNs listed. NOTE2. |
| transmissionInControlChRegion Indicates, for BL UEs and UEs in CE, LTE control channel region may be used for DL broadcast transmission. NOTE 3. |
| up-CIoT-5GS-Optimisation Indicates whether the UE is allowed to resume the connection with User plane CIoT 5GS optimisation, see TS 24.501 [95]. |

NOTE 1:	The value the UE applies for parameter "Qqualmin" in TS 36.304 [4] depends on the q-QualMin fields signalled by E-UTRAN and supported by the UE. In case multiple candidate options are available, the UE shall select the highest priority candidate option according to the priority order indicated by the following table (top row is highest priority).

| q-QualMinRSRQ-OnAllSymbols | q-QualMinWB | Value of parameter "Qqualmin" in TS 36.304 [4] |
| --- | --- | --- |
| Included | Included | q-QualMinRSRQ-OnAllSymbols – (q-QualMin – q-QualMinWB) |
| Included | Not included | q-QualMinRSRQ-OnAllSymbols |
| Not included | Included | q-QualMinWB |
| Not included | Not included | q-QualMin |

NOTE 2:	E-UTRAN sets this field to the same value for all instances of SIB1 message that are broadcasted within the same cell.

NOTE 3:	E-UTRAN configures this field only in the BR version of SIB1 message.

NOTE 4:	E-UTRAN configures at most 6 EPC PLMNs in total (i.e. across all the PLMN lists except for PLMN lists in cellAccessRelatedInfoList-5GC in SIB1). E-UTRAN configures at most 6 5GC PLMNs in total (i.e. across all the PLMN lists in cellAccessRelatedInfoList-5GC in SIB1).

NOTE 5:	E-UTRAN configures only one value for this parameter per PLMN.

NOTE 6:	E-UTRAN configures plmn-Index only if the cellBarred is set to notBarred.

| Conditional presence | Explanation |
| --- | --- |
| BW-reduced | The field is optional present, Need OR, if schedulingInfoSIB1-BR in MIB is set to a value greater than 0. Otherwise the field is not present. |
| FBI-max | The field is mandatory present if freqBandIndicator (i.e. without suffix) is set to maxFBI. Otherwise the field is not present. |
| mFBI | The field is optional present, Need OR, if multiBandInfoList is present. Otherwise the field is not present. |
| mFBI-max | The field is mandatory present if one or more entries in multiBandInfoList (i.e. without suffix, introduced in -v8h0) is set to maxFBI. Otherwise the field is not present. |
| RSRQ | The field is mandatory present if SIB3 is being broadcast and threshServingLowQ is present in SIB3; otherwise optionally present, Need OP. |
| RSRQ2 | The field is mandatory present if q-QualMinRSRQ-OnAllSymbols is present in SIB3; otherwise it is not present and the UE shall delete any existing value for this field. |
| Hopping | The field is mandatory present if si-HoppingConfigCommon field is broadcasted and set to on. Otherwise the field is optionally present, need OP. |
| QrxlevminCE1 | The field is optionally present, Need OR, if q-RxLevMinCE1-r13 is set below -140 dBm. Otherwise the field is not present. |
| TDD | This field is mandatory present for TDD; it is not present for FDD and the UE shall delete any existing value for this field. |
| TDD-OR | The field is optional present for TDD, need OR; it is not present for FDD. |
| WB-RSRQ | The field is optionally present, need OP if the measurement bandwidth indicated by allowedMeasBandwidth in systemInformationBlockType3 is 50 resource blocks or larger; otherwise it is not present. |
| SI-BR | The field is mandatory present if schedulingInfoSIB1-BR is included in MIB with a value greater than 0. Otherwise the field is not present. |

#### –	SystemInformationBlockType1-MBMS

SystemInformationBlockType1-MBMS contains information relevant for receiving service from MBMS-dedicated cell and defines the scheduling of other system information.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: E-UTRAN to UE

SystemInformationBlockType1-MBMS message

-- ASN1START

SystemInformationBlockType1-MBMS-r14 ::=	SEQUENCE {

cellAccessRelatedInfo-r14				SEQUENCE {

plmn-IdentityList-r14					PLMN-IdentityList-MBMS-r14,

trackingAreaCode-r14						TrackingAreaCode,

cellIdentity-r14							CellIdentity

},

freqBandIndicator-r14					FreqBandIndicator-r11,

multiBandInfoList-r14					MultiBandInfoList-r11				OPTIONAL, -- Need OR

schedulingInfoList-MBMS-r14			SchedulingInfoList-MBMS-r14,

si-WindowLength-r14						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,ms40, ms80},

systemInfoValueTag-r14					INTEGER (0..31),

nonMBSFN-SubframeConfig-r14				NonMBSFN-SubframeConfig-r14		OPTIONAL, --Need OR

pdsch-ConfigCommon-r14					PDSCH-ConfigCommon,

systemInformationBlockType13-r14		SystemInformationBlockType13-r9	OPTIONAL, --Need OR

cellAccessRelatedInfoList-r14		SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF

CellAccessRelatedInfo-r14	OPTIONAL,	-- Need OR

nonCriticalExtension					SEQUENCE {}							OPTIONAL

}

PLMN-IdentityList-MBMS-r14 ::=				SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-Identity

SchedulingInfoList-MBMS-r14 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-MBMS-r14

SchedulingInfo-MBMS-r14 ::=	SEQUENCE {

si-Periodicity-r14						ENUMERATED {

rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo-r14						SIB-MappingInfo-MBMS-r14

}

SIB-MappingInfo-MBMS-r14 ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type-MBMS-r14

SIB-Type-MBMS-r14 ::=					ENUMERATED {

sibType10, sibType11, sibType12-v920, sibType13-v920,

sibType15-v1130, sibType16-v1130, ...}

NonMBSFN-SubframeConfig-r14 ::=			SEQUENCE {

radioFrameAllocationPeriod-r14		ENUMERATED {rf4, rf8, rf16, rf32, rf64, rf128, rf512},

radioFrameAllocationOffset-r14		INTEGER (0..7),

subframeAllocation-r14				BIT STRING (SIZE(9))

}

-- ASN1STOP

SystemInformationBlockType1-MBMS field descriptions

cellAccessRelatedInfoList This field contains a list allowing signalling of access related information per PLMN. One PLMN can be included in only one entry of this list. NOTE 2.

cellIdentity Indicates the cell identity. NOTE 1.

freqBandIndicator A list of as defined in TS 36.101 [42], table 6.2.4-1, for the frequency band in freqBandIndicator.

multiBandInfoList A list of additional frequency band indicators, as defined in TS 36.101 [42], table 5.5-1, that the cell belongs to. If the UE supports the frequency band in the freqBandIndicator field it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList field.

nonMBSFN-SubframeConfig Defines the non-MBSFN subframes within the radio frame allocation period defined by the radioFrameAllocationPeriod and the radioFrameAllocationOffset.

plmn-IdentityList List of PLMN identities. The first listed PLMN-Identity is the primary PLMN. NOTE 1.

radioFrameAllocationPeriod, radioFrameAllocationOffset Radio-frames that contain non-MBSFN subframes occur when equation SFN mod radioFrameAllocationPeriod = radioFrameAllocationOffset is satisfied. Value rf4 for radioframeAllocationPeriod denotes 4 radio frames, rf8 detones 8 radion frames, and so on.

schedulingInfoList-MBMS Indicates additional scheduling information of SI messages on MBMS-dedicated cell.

sib-MappingInfo List of the SIBs mapped to this SystemInformation message.

si-Periodicity Periodicity of the SI-message in radio frames, such that rf16 denotes 16 radio frames, rf32 denotes 32 radio frames, and so on.

si-WindowLength Common SI scheduling window for all SIs. Unit in milliseconds, where ms1 denotes 1 millisecond, ms2 denotes 2 milliseconds and so on.

subframeAllocation Defines the subframes that are allocated for non-MBSFN within the radio frame allocation period defined by the radioFrameAllocationPeriod and the radioFrameAllocationOffset. "0" denotes that the corresponding subframe is a MBSFN subframe. "1" denotes that the corresponding subframe is a non-MBSFN subframe. If E-UTRAN configures a value other than "0" for additionalNonMBSFNSubframes within MasterInformationBlock-MBMS, subframeAllocation configuration should also indicate subframes pointed out by additionalNonMBSFNSubframes as non-MBSFN subframes.

systemInformationBlockType13 E-UTRAN does not configure this field if schedulingInfoList–MBMS indicates that SystemInformationBlockType13 is present.

systemInfoValueTag Common for all SIBs other than MIB, SIB1, SIB10, SIB11, SIB12 and SIB14. Change of MIB and SIB1 is detected by acquisition of the corresponding message.

trackingAreaCode A trackingAreaCode that is common for all the PLMNs listed. NOTE1.

NOTE 1:	E-UTRAN sets this field to the same value for all instances of SIB1-MBMS message that are broadcasted within the same cell.

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the eNB.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UEAssistanceInformation message

-- ASN1START

UEAssistanceInformation-r11 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

ueAssistanceInformation-r11			UEAssistanceInformation-r11-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

UEAssistanceInformation-r11-IEs ::=		SEQUENCE {

powerPrefIndication-r11				ENUMERATED	{normal, lowPowerConsumption}	OPTIONAL,

lateNonCriticalExtension			OCTET STRING								OPTIONAL,

nonCriticalExtension				UEAssistanceInformation-v1430-IEs			OPTIONAL

}

UEAssistanceInformation-v1430-IEs ::=	SEQUENCE {

bw-Preference-r14						BW-Preference-r14						OPTIONAL,

sps-AssistanceInformation-r14			SEQUENCE {

trafficPatternInfoListSL-r14			TrafficPatternInfoList-r14			OPTIONAL,

trafficPatternInfoListUL-r14			TrafficPatternInfoList-r14			OPTIONAL

}			OPTIONAL,

rlm-Report-r14							SEQUENCE {

rlm-Event-r14							ENUMERATED {earlyOutOfSync, earlyInSync},

excessRep-MPDCCH-r14					ENUMERATED {excessRep1, excessRep2}	OPTIONAL

}																				OPTIONAL,

delayBudgetReport-r14					DelayBudgetReport-r14					OPTIONAL,

nonCriticalExtension					UEAssistanceInformation-v1450-IEs		OPTIONAL

}

UEAssistanceInformation-v1450-IEs ::=	SEQUENCE {

overheatingAssistance-r14				OverheatingAssistance-r14				OPTIONAL,

nonCriticalExtension					UEAssistanceInformation-v1530-IEs		OPTIONAL

}

UEAssistanceInformation-v1530-IEs ::=	SEQUENCE {

sps-AssistanceInformation-v1530			SEQUENCE {

trafficPatternInfoListSL-v1530			TrafficPatternInfoList-v1530

}			OPTIONAL,

nonCriticalExtension					UEAssistanceInformation-v1610-IEs						OPTIONAL

}

UEAssistanceInformation-v1610-IEs ::=	SEQUENCE {

overheatingAssistance-v1610				OverheatingAssistance-v1610			OPTIONAL,

nonCriticalExtension						SEQUENCE {}							OPTIONAL

}

BW-Preference-r14 ::= SEQUENCE {

dl-Preference-r14		ENUMERATED	{mhz1dot4, mhz5, mhz20 }				OPTIONAL,

ul-Preference-r14		ENUMERATED	{mhz1dot4, mhz5}						OPTIONAL

}

TrafficPatternInfoList-r14 ::= SEQUENCE (SIZE (1..maxTrafficPattern-r14)) OF TrafficPatternInfo-r14

TrafficPatternInfo-r14 ::=	SEQUENCE {

trafficPeriodicity-r14			ENUMERATED {

sf20, sf50, sf100, sf200, sf300, sf400, sf500,

sf600, sf700, sf800, sf900, sf1000},

timingOffset-r14				INTEGER (0..10239),

priorityInfoSL-r14				SL-Priority-r13								OPTIONAL,

logicalChannelIdentityUL-r14	INTEGER (3..10)								OPTIONAL,

messageSize-r14					BIT STRING (SIZE (6))

}

TrafficPatternInfoList-v1530 ::= SEQUENCE (SIZE (1..maxTrafficPattern-r14)) OF TrafficPatternInfo-v1530

TrafficPatternInfo-v1530 ::=	SEQUENCE {

trafficDestination-r15			SL-DestinationIdentity-r12					OPTIONAL,

reliabilityInfoSL-r15			SL-Reliability-r15							OPTIONAL

}

DelayBudgetReport-r14::=	CHOICE {

type1							ENUMERATED {

msMinus1280, msMinus640, msMinus320, msMinus160,

msMinus80, msMinus60, msMinus40, msMinus20, ms0, ms20,

ms40, ms60, ms80, ms160, ms320, ms640, ms1280},

type2							ENUMERATED {

msMinus192, msMinus168,msMinus144, msMinus120,

msMinus96, msMinus72, msMinus48, msMinus24, ms0, ms24,

ms48, ms72, ms96, ms120, ms144, ms168, ms192}

}

OverheatingAssistance-r14 ::=	SEQUENCE {

reducedUE-Category			SEQUENCE {

reducedUE-CategoryDL		INTEGER (0..19),

reducedUE-CategoryUL		INTEGER (0..21)

}		OPTIONAL,

reducedMaxCCs				SEQUENCE {

reducedCCsDL				INTEGER (0..31),

reducedCCsUL				INTEGER (0..31)

}		OPTIONAL

}

OverheatingAssistance-v1610 ::=	SEQUENCE {

overheatingAssistanceForSCG-r16			OCTET STRING

}

-- ASN1STOP

UEAssistanceInformation field descriptions

delayBudgetReport Indicates the UE-preferred adjustment to connected mode DRX or coverage enhancement configuration.

dl-Preference Indicates UE's preference on configuration of maximum PDSCH bandwidth. The value mhz1dot4 corresponds to CE mode usage in 1.4MHz bandwidth, mhz5 corresponds to CE mode usage in 5MHz bandwidth, and mhz20 corresponds to CE mode usage in 20MHz bandwidth or normal coverage.

excessRep-MPDCCH Indicates the excess number of repetitions on MPDCCH. Value excessRep1 and excessRep2 indicate the excess number of repetitions defined in TS 36.133 [16].

logicalChannelIdentityUL Indicates the logical channel identity associated with the reported traffic pattern in the uplink logical channel.

messageSize Indicates the maximum TB size based on the observed traffic pattern. The value refers to the index of TS 36.321 [6], table 6.1.3.1-1.

overheatingAssistanceForSCG Includes the NR OverheatingAssistance IE as specified in TS 38.331 [82]. The field indicates UE's preference on reduced configuration for NR SCG to address overheating.

powerPrefIndication Value lowPowerConsumption indicates the UE prefers a configuration that is primarily optimised for power saving. Otherwise the value is set to normal.

priorityInfoSL Indicates the traffic priority (i.e., PPPP) associated with the reported traffic pattern for V2X sidelink communication.

reducedCCsDL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink SCells indicated by the field, to address overheating. This maximum number includes both SCells of E-UTRA and PSCell/SCells of NR in (NG)EN-DC.

reducedCCsUL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink SCells indicated by the field, to address overheating. This maximum number includes both SCells of E-UTRA and PSCell/SCells of NR in (NG)EN-DC.

reducedUE-CategoryDL, reducedUE-CategoryUL Indicates that UE prefers a configuration corresponding to the reduced UE category, to address overheating. The reduced UE DL category and reduced UE UL category should be indicated according to supported combinations for UE UL and DL Categories, see TS 36.306 [5], Table 4.1A-6.

reliabilityInfoSL Indicates the traffic reliability (i.e., PPPR) associated with the reported traffic pattern for V2X sidelink communication.

rlm-Event This field provides the RLM event ("early-out-of-sync" or "early-in-sync").

rlm-Report This field provides the RLM report for BL UEs and UEs in CE.

sps-AssistanceInformation Indicates the UE assistance information to assist E-UTRAN to configure SPS.

timingOffset This field indicates the estimated timing for a packet arrival in a SL/UL logical channel. Specifically, the value indicates the timing offset with respect to subframe#0 of SFN#0 in milliseconds.

trafficDestination Indicates the destination associated with the reported traffic pattern for V2X sidelink communication.

trafficPatternInfoListSL This field provides the traffic characteristics of sidelink logical channel(s) that are setup for V2X sidelink communication. If trafficPatternInfoListSL-v1530 is included, it includes the same number of entries, and listed in the same order, as in trafficPatternInfoListSL-r14.

trafficPatternInfoListUL This field provides the traffic characteristics of uplink logical channel(s).

trafficPeriodicity This field indicates the estimated data arrival periodicity in a SL/UL logical channel. Value sf20 corresponds to 20 ms, sf50 corresponds to 50 ms and so on.

type1 Indicates the preferred amount of increment/decrement to the connected mode DRX cycle length with respect to the current configuration. Value in number of milliseconds. Value ms40 corresponds to 40 milliseconds, msMinus40 corresponds to -40 milliseconds and so on.

type2 Indicates the preferred amount of increment/decrement to the coverage enhancement configuration with respect to the current configuration so that the Uu air interface delay changes by the indicated amount. Value in number of milliseconds. Value ms24 corresponds to 24 milliseconds, msMinus24 corresponds to -24 milliseconds and so on.

ul-Preference Indicates UE's preference on configuration of maximum PUSCH bandwidth. The value mhz1dot4 corresponds to CE mode usage in 1.4MHz bandwidth, and mhz5 corresponds to CE mode usage in 5MHz bandwidth.

#### –	UECapabilityEnquiry

The UECapabilityEnquiry message is used to request the transfer of UE radio access capabilities for E-UTRA as well as for other RATs.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

UECapabilityEnquiry message

-- ASN1START

UECapabilityEnquiry ::=				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

ueCapabilityEnquiry-r8				UECapabilityEnquiry-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

UECapabilityEnquiry-r8-IEs ::=		SEQUENCE {

ue-CapabilityRequest				UE-CapabilityRequest,

nonCriticalExtension				UECapabilityEnquiry-v8a0-IEs		OPTIONAL

}

UECapabilityEnquiry-v8a0-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				UECapabilityEnquiry-v1180-IEs		OPTIONAL

}

UECapabilityEnquiry-v1180-IEs ::=	SEQUENCE {

requestedFrequencyBands-r11			SEQUENCE (SIZE (1..16)) OF FreqBandIndicator-r11	OPTIONAL,

nonCriticalExtension				UECapabilityEnquiry-v1310-IEs						OPTIONAL

}

UECapabilityEnquiry-v1310-IEs ::=	SEQUENCE {

requestReducedFormat-r13			ENUMERATED {true}					OPTIONAL,	-- Need ON

requestSkipFallbackComb-r13			ENUMERATED {true}					OPTIONAL,	-- Need ON

requestedMaxCCsDL-r13				INTEGER (2..32)						OPTIONAL,	-- Need ON

requestedMaxCCsUL-r13				INTEGER (2..32)						OPTIONAL,	-- Need ON

requestReducedIntNonContComb-r13	ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension				UECapabilityEnquiry-v1430-IEs		OPTIONAL

}

UECapabilityEnquiry-v1430-IEs ::=	SEQUENCE {

requestDiffFallbackCombList-r14		BandCombinationList-r14				OPTIONAL,	-- Need ON

nonCriticalExtension				UECapabilityEnquiry-v1510-IEs		OPTIONAL

}

UECapabilityEnquiry-v1510-IEs ::=	SEQUENCE {

requestedFreqBandsNR-MRDC-r15		OCTET STRING						OPTIONAL,

nonCriticalExtension				UECapabilityEnquiry-v1530-IEs		OPTIONAL

}

UECapabilityEnquiry-v1530-IEs ::=	SEQUENCE {

requestSTTI-SPT-Capability-r15		ENUMERATED {true}					OPTIONAL,

eutra-nr-only-r15					ENUMERATED {true}					OPTIONAL,

nonCriticalExtension				UECapabilityEnquiry-v1550-IEs		OPTIONAL

}

UECapabilityEnquiry-v1550-IEs ::=	SEQUENCE {

requestedCapabilityNR-r15			OCTET STRING						OPTIONAL,

nonCriticalExtension				UECapabilityEnquiry-v1560-IEs		OPTIONAL

}

UECapabilityEnquiry-v1560-IEs ::=	SEQUENCE {

requestedCapabilityCommon-r15		OCTET STRING						OPTIONAL,

nonCriticalExtension				UECapabilityEnquiry-v1610-IEs		OPTIONAL

}

UECapabilityEnquiry-v1610-IEs ::=	SEQUENCE {

rrc-SegAllowed-r16					ENUMERATED {enabled}			OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

UE-CapabilityRequest ::=			SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF RAT-Type

-- ASN1STOP

UECapabilityEnquiry field descriptions

eutra-nr-only Indicates that the UE is requested to provide UE capabilities related to (NG)EN-DC only as specified in TS38.331 [82].

requestDiffFallbackCombList List of CA band combinations for which the UE is requested to provide different capabilities for their fallback band combinations in conjunction with the capabilities supported for the CA band combinations in this list. The UE shall exclude fallback band combinations for which their supported UE capabilities are the same as the CA band combination indicated in this list.

requestReducedFormat Indicates that the UE is requested to provide supported CA band combinations in the supportedBandCombinationReduced-r13 instead of the supportedBandCombination-r10. The E-UTRAN includes this field if requestSkipFallbackComb or requestDiffFallbackCombList is included in the message.

requestSkipFallbackComb Indicates that the UE shall explicitly exclude fallback CA band combinations in capability signalling.

ue-CapabilityRequest List of the RATs for which the UE is requested to transfer the UE radio access capabilities i.e. E-UTRA, UTRA, GERAN-CS, GERAN-PS, CDMA2000. A separate RAT-Type value applies for some EUTRA-NR capabilities that are transferred by a separate UE capability container, used in case of MRDC.

requestedFrequencyBands List of frequency bands for which the UE is requested to provide supported CA band combinations and non CA bands.

requestedFreqBandsNR-MRDC Interpreted as FreqBandList IE as specified in TS 38.331 [82]. It concerns a list of NR and/ or E-UTRA frequency bands for which the UE is requested to provide its supported NR CA and/or MR-DC band combinations (i.e. within the UE capability containers for NR and MR-DC, as requested by E-UTRAN) and feature sets corresponding to the MR-DC band combinations (i.e. within the UE capability containers for LTE and NR, as requested by E-UTRAN).

requestedCapabilityCommon Contains the filter common for all requested MR-DC related capability containers as defined by UE-CapabilityRequestFilterCommon IE in TS 38.331 [82].

requestedCapabilityNR Interpreted as UE-CapabilityRequestFilterNR IE as specified in TS 38.331 [82], in which the field frequencyBandListFilter is omitted.

requestedMaxCCsDL, requestedMaxCCsUL Indicates the maximum number of CCs for which the UE is requested to provide supported CA band combinations and non-CA bands.

requestReducedIntNonContComb Indicates that the UE shall explicitly exclude supported intra-band non-contiguous CA band combinations other than included in capability signalling as specified in TS 36.306 [5], clause 4.3.5.21.

requestSTTI-SPT-Capability Indicates that the UE is requested to provide its supported short TTI and SPT capabilities in capability signalling.

rrc-SegAllowed A one-shot field that indicates that the UE is enabled to segment the response message into a series of ULDedicatedMessageSegment messages.

#### –	UECapabilityInformation

The UECapabilityInformation message is used to transfer of UE radio access capabilities requested by the E-UTRAN.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UECapabilityInformation message

-- ASN1START

UECapabilityInformation ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

ueCapabilityInformation-r8			UECapabilityInformation-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

UECapabilityInformation-r8-IEs ::=	SEQUENCE {

ue-CapabilityRAT-ContainerList		UE-CapabilityRAT-ContainerList,

nonCriticalExtension				UECapabilityInformation-v8a0-IEs	OPTIONAL

}

UECapabilityInformation-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				UECapabilityInformation-v1250-IEs	OPTIONAL

}

UECapabilityInformation-v1250-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r12				UE-RadioPagingInfo-r12				OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

UECapabilityInformation field descriptions

ue-RadioPagingInfo This field contains UE capability information used for paging.

#### –	ULDedicatedMessageSegment

The ULDedicatedMessageSegment message is used to transfer segments of the UECapabilityInformation message.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ULDedicatedMessageSegment message

-- ASN1START

ULDedicatedMessageSegment-r16 ::=	SEQUENCE {

criticalExtensions					CHOICE {

ulDedicatedMessageSegment-r16		ULDedicatedMessageSegment-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

ULDedicatedMessageSegment-r16-IEs ::=	SEQUENCE {

segmentNumber-r16						INTEGER (0..15),

rrc-MessageSegmentContainer-r16			OCTET STRING,

rrc-MessageSegmentType-r16				ENUMERATED {notLastSegment, lastSegment},

lateNonCriticalExtension				OCTET STRING						OPTIONAL,

nonCriticalExtension					SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

ULDedicatedMessageSegment field descriptions

segmentNumber Identifies the sequence number of a segment within the encoded UL DCCH message.

rrc-MessageSegmentContainer Includes a segment of the encoded UL DCCH message. The size of the included segment in this container should be small enough that the resulting encoded RRC message PDU is less than or equal to the PDCP SDU size limit.

rrc-MessageSegmentType Indicates whether the included UL DCCH message segment is the last segment or not.

#### –	UEInformationRequest

The UEInformationRequest is the command used by E-UTRAN to retrieve information from the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

UEInformationRequest message

-- ASN1START

UEInformationRequest-r9		::=				SEQUENCE {

rrc-TransactionIdentifier		RRC-TransactionIdentifier,

criticalExtensions				CHOICE {

c1								CHOICE {

ueInformationRequest-r9				UEInformationRequest-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

UEInformationRequest-r9-IEs ::=		SEQUENCE {

rach-ReportReq-r9					BOOLEAN,

rlf-ReportReq-r9					BOOLEAN,

nonCriticalExtension				UEInformationRequest-v930-IEs		OPTIONAL

}

UEInformationRequest-v930-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				UEInformationRequest-v1020-IEs		OPTIONAL

}

UEInformationRequest-v1020-IEs ::=	SEQUENCE {

logMeasReportReq-r10				ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension				UEInformationRequest-v1130-IEs		OPTIONAL

}

UEInformationRequest-v1130-IEs ::= SEQUENCE {

connEstFailReportReq-r11			ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension				UEInformationRequest-v1250-IEs		OPTIONAL

}

UEInformationRequest-v1250-IEs ::= SEQUENCE {

mobilityHistoryReportReq-r12		ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension				UEInformationRequest-v1530-IEs		OPTIONAL

}

UEInformationRequest-v1530-IEs ::= SEQUENCE {

idleModeMeasurementReq-r15			ENUMERATED {true}					OPTIONAL,	-- Need ON

flightPathInfoReq-r15				FlightPathInfoReportConfig-r15		OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

UEInformationRequest field descriptions

rach-ReportReq This field is used to indicate whether the UE shall report information about the random access procedure.

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer the information requested by the E-UTRAN.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UEInformationResponse message

-- ASN1START

UEInformationResponse-r9	::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

ueInformationResponse-r9			UEInformationResponse-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

UEInformationResponse-r9-IEs ::=		SEQUENCE {

rach-Report-r9							RACH-Report-r16		OPTIONAL,

rlf-Report-r9							RLF-Report-r9			OPTIONAL,

nonCriticalExtension					UEInformationResponse-v930-IEs			OPTIONAL

}

-- Late non critical extensions

UEInformationResponse-v9e0-IEs ::= SEQUENCE {

rlf-Report-v9e0						RLF-Report-v9e0					OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- Regular non critical extensions

UEInformationResponse-v930-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING UEInformationResponse-v9e0-IEs)	OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1020-IEs		OPTIONAL

}

UEInformationResponse-v1020-IEs ::= SEQUENCE {

logMeasReport-r10					LogMeasReport-r10					OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1130-IEs		OPTIONAL

}

UEInformationResponse-v1130-IEs ::= SEQUENCE {

connEstFailReport-r11				ConnEstFailReport-r11				OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1250-IEs		OPTIONAL

}

UEInformationResponse-v1250-IEs ::= SEQUENCE {

mobilityHistoryReport-r12			MobilityHistoryReport-r12			OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1530-IEs		OPTIONAL

}

UEInformationResponse-v1530-IEs ::= SEQUENCE {

measResultListIdle-r15				MeasResultListIdle-r15			OPTIONAL,

flightPathInfoReport-r15			FlightPathInfoReport-r15		OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1610-IEs		OPTIONAL

}

UEInformationResponse-v1610-IEs ::= SEQUENCE {

rach-Report-v1610					RACH-Report-v1610				OPTIONAL,

measResultListExtIdle-r16			MeasResultListExtIdle-r16		OPTIONAL,

measResultListIdleNR-r16			MeasResultListIdleNR-r16		OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

RACH-Report-r16 ::=					SEQUENCE {

numberOfPreamblesSent-r16			NumberOfPreamblesSent-r11,

contentionDetected-r16				BOOLEAN

}

RACH-Report-v1610 ::=	SEQUENCE {

initialCEL-r16 					INTEGER (0..3),

edt-Fallback-r16					BOOLEAN

}

RLF-Report-r9 ::=					SEQUENCE {

measResultLastServCell-r9			SEQUENCE {

rsrpResult-r9						RSRP-Range,

rsrqResult-r9						RSRQ-Range						OPTIONAL

},

measResultNeighCells-r9				SEQUENCE {

measResultListEUTRA-r9				MeasResultList2EUTRA-r9			OPTIONAL,

measResultListUTRA-r9				MeasResultList2UTRA-r9			OPTIONAL,

measResultListGERAN-r9				MeasResultListGERAN				OPTIONAL,

measResultsCDMA2000-r9				MeasResultList2CDMA2000-r9		OPTIONAL

}	OPTIONAL,

...,

[[	locationInfo-r10				LocationInfo-r10					OPTIONAL,

failedPCellId-r10					CHOICE {

cellGlobalId-r10					CellGlobalIdEUTRA,

pci-arfcn-r10						SEQUENCE {

physCellId-r10						PhysCellId,

carrierFreq-r10						ARFCN-ValueEUTRA

}

}																	OPTIONAL,

reestablishmentCellId-r10		CellGlobalIdEUTRA					OPTIONAL,

timeConnFailure-r10				INTEGER (0..1023)					OPTIONAL,

connectionFailureType-r10		ENUMERATED {rlf, hof}				OPTIONAL,

previousPCellId-r10				CellGlobalIdEUTRA					OPTIONAL

]],

[[	failedPCellId-v1090				SEQUENCE {

carrierFreq-v1090				ARFCN-ValueEUTRA-v9e0

}																	OPTIONAL

]],

[[	basicFields-r11					SEQUENCE {

c-RNTI-r11						C-RNTI,

rlf-Cause-r11					ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, t312-Expiry-r12},

timeSinceFailure-r11			TimeSinceFailure-r11

}																	OPTIONAL,

previousUTRA-CellId-r11			SEQUENCE {

carrierFreq-r11					ARFCN-ValueUTRA,

physCellId-r11					CHOICE {

fdd-r11							PhysCellIdUTRA-FDD,

tdd-r11							PhysCellIdUTRA-TDD

},

cellGlobalId-r11				CellGlobalIdUTRA				OPTIONAL

}																	OPTIONAL,

selectedUTRA-CellId-r11			SEQUENCE {

carrierFreq-r11					ARFCN-ValueUTRA,

physCellId-r11					CHOICE {

fdd-r11							PhysCellIdUTRA-FDD,

tdd-r11							PhysCellIdUTRA-TDD

}

}																	OPTIONAL

]],

[[	failedPCellId-v1250				SEQUENCE {

tac-FailedPCell-r12				TrackingAreaCode

}																	OPTIONAL,

measResultLastServCell-v1250	RSRQ-Range-v1250					OPTIONAL,

lastServCellRSRQ-Type-r12		RSRQ-Type-r12						OPTIONAL,

measResultListEUTRA-v1250		MeasResultList2EUTRA-v1250			OPTIONAL

]],

[[	drb-EstablishedWithQCI-1-r13	ENUMERATED {qci1}					OPTIONAL

]],

[[	measResultLastServCell-v1360	RSRP-Range-v1360					OPTIONAL

]],

[[	logMeasResultListBT-r15			LogMeasResultListBT-r15				OPTIONAL,

logMeasResultListWLAN-r15		LogMeasResultListWLAN-r15			OPTIONAL

]],

[[	measResultListNR-r16			MeasResultCellListNR-r15			OPTIONAL,

previousNR-PCellId-r16			CellGlobalIdNR-r16					OPTIONAL,

failedNR-PCellId-r16			CHOICE {

cellGlobalId-r16				CellGlobalIdNR-r16,

pci-arfcn-r16					SEQUENCE {

physCellId-r16					PhysCellIdNR-r15,

carrierFreq-r16					ARFCN-ValueNR-r15

}

}																	OPTIONAL,

reconnectCellId-r16				CHOICE {

nrReconnectCellId-r16			CellGlobalIdNR-r16,

eutraReconnectCellId-r16		SEQUENCE {

cellGlobalId-r16				CellGlobalIdEUTRA,

trackingAreaCode-EPC-r16		TrackingAreaCode			OPTIONAL,

trackingAreaCode-5GC-r16		TrackingAreaCode-5GC-r15	OPTIONAL

}

}																	OPTIONAL,

timeUntilReconnection-r16		TimeUntilReconnection-r16			OPTIONAL

]],

[[	measResultListNR-v1640			SEQUENCE {

carrierFreqNR-r16				ARFCN-ValueNR-r15

}																	OPTIONAL,

measResultListExtNR-r16			MeasResultFreqListNR-r16		OPTIONAL

]]

}

RLF-Report-v9e0 ::=				SEQUENCE {

measResultListEUTRA-v9e0			MeasResultList2EUTRA-v9e0

}

MeasResultList2EUTRA-r9 ::=				SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2EUTRA-r9

MeasResultList2EUTRA-v9e0 ::=			SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2EUTRA-v9e0

MeasResultList2EUTRA-v1250 ::=			SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2EUTRA-v1250

MeasResult2EUTRA-r9 ::=				SEQUENCE {

carrierFreq-r9						ARFCN-ValueEUTRA,

measResultList-r9					MeasResultListEUTRA

}

MeasResult2EUTRA-v9e0 ::=			SEQUENCE {

carrierFreq-v9e0					ARFCN-ValueEUTRA-v9e0		OPTIONAL

}

MeasResult2EUTRA-v1250 ::=			SEQUENCE {

rsrq-Type-r12						RSRQ-Type-r12		OPTIONAL

}

MeasResultList2UTRA-r9 ::=			SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2UTRA-r9

MeasResult2UTRA-r9 ::=				SEQUENCE {

carrierFreq-r9						ARFCN-ValueUTRA,

measResultList-r9					MeasResultListUTRA

}

MeasResultList2CDMA2000-r9 ::=		SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2CDMA2000-r9

MeasResult2CDMA2000-r9 ::=			SEQUENCE {

carrierFreq-r9						CarrierFreqCDMA2000,

measResultList-r9					MeasResultsCDMA2000

}

LogMeasReport-r10 ::=				SEQUENCE {

absoluteTimeStamp-r10				AbsoluteTimeInfo-r10,

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10		OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

logMeasInfoList-r10					LogMeasInfoList-r10,

logMeasAvailable-r10				ENUMERATED {true}				OPTIONAL,

...,

[[	logMeasAvailableBT-r15			ENUMERATED {true}				OPTIONAL,

logMeasAvailableWLAN-r15		ENUMERATED {true}				OPTIONAL

]]

}

LogMeasInfoList-r10 ::=		SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF LogMeasInfo-r10

LogMeasInfo-r10 ::=		SEQUENCE {

locationInfo-r10					LocationInfo-r10				OPTIONAL,

relativeTimeStamp-r10				INTEGER (0..7200),

servCellIdentity-r10				CellGlobalIdEUTRA,

measResultServCell-r10				SEQUENCE {

rsrpResult-r10						RSRP-Range,

rsrqResult-r10						RSRQ-Range

},

measResultNeighCells-r10			SEQUENCE {

measResultListEUTRA-r10				MeasResultList2EUTRA-r9		OPTIONAL,

measResultListUTRA-r10				MeasResultList2UTRA-r9		OPTIONAL,

measResultListGERAN-r10				MeasResultList2GERAN-r10	OPTIONAL,

measResultListCDMA2000-r10			MeasResultList2CDMA2000-r9	OPTIONAL

}	OPTIONAL,

...,

[[	measResultListEUTRA-v1090			MeasResultList2EUTRA-v9e0	OPTIONAL

]],

[[	measResultListMBSFN-r12				MeasResultListMBSFN-r12		OPTIONAL,

measResultServCell-v1250			RSRQ-Range-v1250			OPTIONAL,

servCellRSRQ-Type-r12				RSRQ-Type-r12				OPTIONAL,

measResultListEUTRA-v1250			MeasResultList2EUTRA-v1250	OPTIONAL

]],

[[	inDeviceCoexDetected-r13			ENUMERATED {true}			OPTIONAL

]],

[[	measResultServCell-v1360			RSRP-Range-v1360			OPTIONAL

]],

[[	logMeasResultListBT-r15				LogMeasResultListBT-r15		OPTIONAL,

logMeasResultListWLAN-r15			LogMeasResultListWLAN-r15	OPTIONAL

]],

[[	anyCellSelectionDetected-r15		ENUMERATED {true}			OPTIONAL

]],

[[	measResultListNR-r16				MeasResultCellListNR-r15	OPTIONAL

]],

[[	measResultListNR-v1640			SEQUENCE {

carrierFreqNR-r16				ARFCN-ValueNR-r15

}																	OPTIONAL,

measResultListExtNR-r16			MeasResultFreqListNR-r16		OPTIONAL

]]

}

MeasResultListMBSFN-r12 ::=			SEQUENCE (SIZE (1..maxMBSFN-Area)) OF MeasResultMBSFN-r12

MeasResultMBSFN-r12 ::=			SEQUENCE {

mbsfn-Area-r12					SEQUENCE {

mbsfn-AreaId-r12				MBSFN-AreaId-r12,

carrierFreq-r12					ARFCN-ValueEUTRA-r9

},

rsrpResultMBSFN-r12				RSRP-Range,

rsrqResultMBSFN-r12				MBSFN-RSRQ-Range-r12,

signallingBLER-Result-r12		BLER-Result-r12					OPTIONAL,

dataBLER-MCH-ResultList-r12		DataBLER-MCH-ResultList-r12		OPTIONAL,

...

}

DataBLER-MCH-ResultList-r12 ::=		SEQUENCE (SIZE (1.. maxPMCH-PerMBSFN)) OF DataBLER-MCH-Result-r12

DataBLER-MCH-Result-r12 ::=			SEQUENCE {

mch-Index-r12						INTEGER (1..maxPMCH-PerMBSFN),

dataBLER-Result-r12					BLER-Result-r12

}

BLER-Result-r12 ::=					SEQUENCE {

bler-r12							BLER-Range-r12,

blocksReceived-r12					SEQUENCE {

n-r12								BIT STRING (SIZE (3)),

m-r12								BIT STRING (SIZE (8))

}

}

BLER-Range-r12 ::=						INTEGER(0..31)

MeasResultList2GERAN-r10 ::=			SEQUENCE (SIZE (1..maxCellListGERAN)) OF MeasResultListGERAN

MeasResultFreqListNR-r16::=		SEQUENCE (SIZE (1..maxFreq-1-r16)) OF MeasResultFreqFailNR-r15

ConnEstFailReport-r11 ::=				SEQUENCE {

failedCellId-r11					CellGlobalIdEUTRA,

locationInfo-r11					LocationInfo-r10					OPTIONAL,

measResultFailedCell-r11			SEQUENCE {

rsrpResult-r11						RSRP-Range,

rsrqResult-r11						RSRQ-Range						OPTIONAL

},

measResultNeighCells-r11			SEQUENCE {

measResultListEUTRA-r11				MeasResultList2EUTRA-r9			OPTIONAL,

measResultListUTRA-r11				MeasResultList2UTRA-r9			OPTIONAL,

measResultListGERAN-r11				MeasResultListGERAN				OPTIONAL,

measResultsCDMA2000-r11				MeasResultList2CDMA2000-r9		OPTIONAL

}	OPTIONAL,

numberOfPreamblesSent-r11			NumberOfPreamblesSent-r11,

contentionDetected-r11				BOOLEAN,

maxTxPowerReached-r11				BOOLEAN,

timeSinceFailure-r11				TimeSinceFailure-r11,

measResultListEUTRA-v1130			MeasResultList2EUTRA-v9e0			OPTIONAL,

...,

[[	measResultFailedCell-v1250		RSRQ-Range-v1250					OPTIONAL,

failedCellRSRQ-Type-r12			RSRQ-Type-r12						OPTIONAL,

measResultListEUTRA-v1250		MeasResultList2EUTRA-v1250			OPTIONAL

]],

[[	measResultFailedCell-v1360		RSRP-Range-v1360					OPTIONAL

]],

[[	logMeasResultListBT-r15			LogMeasResultListBT-r15				OPTIONAL,

logMeasResultListWLAN-r15		LogMeasResultListWLAN-r15			OPTIONAL

]],

[[	measResultListNR-r16			MeasResultCellListNR-r15			OPTIONAL

]],

[[	measResultListNR-v1640			SEQUENCE {

carrierFreqNR-r16				ARFCN-ValueNR-r15

}																	OPTIONAL,

measResultListExtNR-r16			MeasResultFreqListNR-r16		OPTIONAL

]]

}

NumberOfPreamblesSent-r11::=			INTEGER (1..200)

TimeSinceFailure-r11 ::=				INTEGER (0..172800)

TimeUntilReconnection-r16 ::=			INTEGER (0..172800)

MobilityHistoryReport-r12 ::=	VisitedCellInfoList-r12

FlightPathInfoReport-r15 ::=		SEQUENCE {

flightPath-r15	SEQUENCE (SIZE (1..maxWayPoint-r15)) OF WayPointLocation-r15	OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

WayPointLocation-r15 ::=			SEQUENCE {

wayPointLocation-r15						LocationInfo-r10,

timeStamp-r15							AbsoluteTimeInfo-r10		OPTIONAL

}

-- ASN1STOP

UEInformationResponse field descriptions

absoluteTimeStamp Indicates the absolute time when the logged measurement configuration logging is provided, as indicated by E-UTRAN within absoluteTimeInfo.

anyCellSelectionDetected This field is used to indicate the detection of any cell selection state, as defined in TS 36.304 [4]. The UE sets this field when performing the logging of measurement results in RRC_IDLE and there is no suitable cell or no acceptable cell.

bler Indicates the measured BLER value. The coding of BLER value is defined in TS 36.133 [16].

blocksReceived Indicates total number of MCH blocks, which were received by the UE and used for the corresponding BLER calculation, within the measurement period as defined in TS 36.133 [16].

carrierFreq In case the UE includes carrierFreq-v9e0 and/ or carrierFreq-v1090, the UE shall set the corresponding entry of carrierFreq-r9 and/ or carrierFreq-r10 respectively to maxEARFCN. For E-UTRA and UTRA frequencies, the UE sets the ARFCN according to the band used when obtaining the concerned measurement results.

carrierFreqNR In case the UE includes measResultListNR, the UE uses this field to indicate the ARFCN value according to the band used when obtaining the concrned measurement results

connectionFailureType This field is used to indicate whether the connection failure is due to radio link failure or handover failure.

contentionDetected This field is used to indicate that contention was detected for at least one of the transmitted preambles, see TS 36.321 [6].

c-RNTI This field indicates the C-RNTI used in the PCell upon detecting radio link failure or the C-RNTI used in the source PCell upon handover failure.

dataBLER-MCH-ResultList Includes a BLER result per MCH on subframes using dataMCS, with the applicable MCH(s) listed in the same order as in pmch-InfoList within MBSFNAreaConfiguration.

drb-EstablishedWithQCI-1 This field is used to indicate the radio link failure occurred while a bearer with QCI value equal to 1 was configured, see TS 24.301 [35].

edt-Fallback Value TRUE indicates the last successfully completed random access procedure was initiated with EDT PRACH resource and succeeded after receiving EDT fallback indication from lower layers.

failedCellId This field is used to indicate the cell in which connection establishment failed.

failedPCellId This field is used to indicate the PCell in which RLF is detected or the target PCell of the failed handover. The UE sets the EARFCN according to the band used for transmission/ reception when the failure occurred.

inDeviceCoexDetected Indicates that measurement logging is suspended due to IDC problem detection.

initialCEL Indicates the initial CE level used for the last successfully completed random access procedure for BL UEs and UEs in CE.

logMeasResultListBT This field refers to the Bluetooth measurement results.

logMeasResultListWLAN This field refers to the WLAN measurement results.

maxTxPowerReached This field is used to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6].

mch-Index Indicates the MCH by referring to the entry as listed in pmch-InfoList within MBSFNAreaConfiguration.

measResultFailedCell This field refers to the last measurement results taken in the cell, where connection establishment failure happened. For UE supporting CE Mode B, when CE mode B is not restricted by upper layers, measResultFailedCell-v1360 is reported if the measured RSRP is less than -140 dBm.

measResultLastServCell This field refers to the last measurement results taken in the PCell, where radio link failure or handover failure happened. For BL UEs or UEs in CE, when operating in CE Mode B, measResultLastServCell-v1360 is reported if the measured RSRP is less than -140 dBm.

measResultListEUTRA If measResultListEUTRA-v9e0, measResultListEUTRA-v1090 or measResultListEUTRA-v1130 is included, the UE shall include the same number of entries, and listed in the same order, as in measResultListEUTRA-r9, measResultListEUTRA-r10 and/ or measResultListEUTRA-r11 respectively.

measResultListEUTRA-v1250 If included in RLF-Report-r9 the UE shall include the same number of entries, and listed in the same order, as in measResultListEUTRA-r9; If included in LogMeasInfo-r10 the UE shall include the same number of entries, and listed in the same order, as in measResultListEUTRA-r10; If included in ConnEstFailReport-r11 the UE shall include the same number of entries, and listed in the same order, as in measResultListEUTRA-r11;

measResultListIdle This field indicates the E-UTRA measurement results done during RRC_IDLE and RRC_INACTIVE at network request.

measResultListIdleNR This field indicates the NR measurement results done during RRC_IDLE and RRC_INACTIVE at network request.

measResultListNR, measResultListExtNR Includes NR measurement results, with measResultListNR including results of a first NR frequency and measResultListExtNR including results of additinal NR frequencies, if available.

measResultServCell This field refers to the log measurement results taken in the Serving cell. For UE supporting CE Mode B, when CE mode B is not restricted by upper layers, measResultServCell-v1360 is reported if the measured RSRP is less than -140 dBm.

mobilityHistoryReport This field is used to indicate the time of stay in 16 most recently visited E-UTRA cells or of stay out of E-UTRA.

numberOfPreamblesSent This field is used to indicate the number of RACH preambles that were transmitted. Corresponds to parameter PREAMBLE_TRANSMISSION_COUNTER in TS 36.321 [6].

previousPCellId This field is used to indicate the source PCell of the last handover (source PCell when the last RRCConnectionReconfiguration message including mobilityControlInfo was received).

previousUTRA-CellId This field is used to indicate the source UTRA cell of the last successful handover to E-UTRAN, when RLF occurred at the target PCell. The UE sets the ARFCN according to the band used for transmission/ reception on the concerned cell.

reconnectCellId This field is used to indicate the cell in which the UE comes back to connected after connection failure and after failing to perform reestablishment. If the UE comes back to RRC CONNECTED in an NR cell then nrReconnectCellID is included and if the UE comes back to RRC CONNECTED in an LTE cell then eutraReconnectCellID is included.

reestablishmentCellId This field is used to indicate the cell in which the re-establishment attempt was made after connection failure.

relativeTimeStamp Indicates the time of logging measurement results, measured relative to the absoluteTimeStamp. Value in seconds.

rlf-Cause This field is used to indicate the cause of the last radio link failure that was detected. In case of handover failure information reporting (i.e., the connectionFailureType is set to 'hof'), the UE is allowed to set this field to any value.

selectedUTRA-CellId This field is used to indicate the UTRA cell that the UE selects after RLF is detected, while T311 is running. The UE sets the ARFCN according to the band selected for transmission/ reception on the concerned cell.

signallingBLER-Result Includes a BLER result of MBSFN subframes using signallingMCS.

tac-FailedPCell This field is used to indicate the Tracking Area Code of the PCell in which RLF is detected.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [58].

timeConnFailure This field is used to indicate the time elapsed since the last HO initialization until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (establishment) failure. Value in seconds. The maximum value 172800 means 172800s or longer.

timeStamp Includes time stamps for the waypoints that describe planned locations for the UE.

timeUntilReconnection This field is used to indicate the time that elapsed between the connection (radio link or handover) failure and the next time the UE comes to RRC CONNECTED in an NR or EUTRA cell, after failing to perform reestablishment. Value in seconds. The maximum value 172800 means 172800s or longer.

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [58].

wayPointLocation Includes location coordinates for a UE for Aerial UE operation. The waypoints describe planned locations for the UE.

#### –	ULHandoverPreparationTransfer (CDMA2000)

The ULHandoverPreparationTransfer message is used for the uplink transfer of handover related CDMA2000 information when requested by the higher layers.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ULHandoverPreparationTransfer message

-- ASN1START

ULHandoverPreparationTransfer ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

ulHandoverPreparationTransfer-r8		ULHandoverPreparationTransfer-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

ULHandoverPreparationTransfer-r8-IEs ::= SEQUENCE {

cdma2000-Type						CDMA2000-Type,

meid								BIT STRING (SIZE (56))					OPTIONAL,

dedicatedInfo						DedicatedInfoCDMA2000,

nonCriticalExtension				ULHandoverPreparationTransfer-v8a0-IEs	OPTIONAL

}

ULHandoverPreparationTransfer-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

ULHandoverPreparationTransfer field descriptions

meid The 56 bit mobile identification number provided by the CDMA2000 Upper layers.

#### –	ULInformationTransfer

The ULInformationTransfer message is used for the uplink transfer of NAS, non-3GPP dedicated information, or IAB-DU specific F1-C related information.

Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet). If SRB2 is suspended, the UE does not send this message until SRB2 is resumed. If only dedicatedInfoF1c is included, SRB2 is used.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ULInformationTransfer message

-- ASN1START

ULInformationTransfer ::=			SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

ulInformationTransfer-r8			ULInformationTransfer-r8-IEs,

ulInformationTransfer-r16			ULInformationTransfer-r16-IEs,

spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

ULInformationTransfer-r8-IEs ::=	SEQUENCE {

dedicatedInfoType					CHOICE {

dedicatedInfoNAS					DedicatedInfoNAS,

dedicatedInfoCDMA2000-1XRTT			DedicatedInfoCDMA2000,

dedicatedInfoCDMA2000-HRPD			DedicatedInfoCDMA2000

},

nonCriticalExtension				ULInformationTransfer-v8a0-IEs		OPTIONAL

}

ULInformationTransfer-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

ULInformationTransfer-r16-IEs ::=	SEQUENCE {

dedicatedInfoType-r16				CHOICE {

dedicatedInfoNAS-r16				DedicatedInfoNAS,

dedicatedInfoCDMA2000-1XRTT-r16		DedicatedInfoCDMA2000,

dedicatedInfoCDMA2000-HRPD-r16		DedicatedInfoCDMA2000

}																		OPTIONAL,

dedicatedInfoF1c-r16				DedicatedInfoF1c-r16				OPTIONAL,

nonCriticalExtension				ULInformationTransfer-v8a0-IEs	OPTIONAL

}

-- ASN1STOP

#### –	ULInformationTransferIRAT

The ULInformationTransferIRAT message is used for the uplink transfer of information terminated by E-UTRAN but specified by anoher RAT. In this release of the specification, the message is used for sidelink information specified by TS 38.331.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ULInformationTransferIRAT message

-- ASN1START

ULInformationTransferIRAT-r16 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

ulInformationTransferIRAT-r16			ULInformationTransferIRAT-r16-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

ULInformationTransferIRAT-r16-IEs ::=	SEQUENCE {

ul-DCCH-MessageNR-r16			OCTET STRING						OPTIONAL,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

ULInformationTransferIRAT field descriptions

ul-DCCH-MessageNR Includes the UL-DCCH-Message as defined in TS 38.331 [82]. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport, NR RRC SidelinkUEInformationNR and the NR RRC UEAssistanceInformation messages.

#### –	ULInformationTransferMRDC

The ULInformationTransferMRDC message is used for the uplink transfer of MR DC information (i.e. for the case the SCG employs another RAT e.g. for transferring the NR RRC Measurement Report message).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ULInformationTransferMRDC message

-- ASN1START

ULInformationTransferMRDC-r15 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

ulInformationTransferMRDC-r15			ULInformationTransferMRDC-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

ULInformationTransferMRDC-r15-IEs ::=	SEQUENCE {

ul-DCCH-MessageNR-r15			OCTET STRING						OPTIONAL,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

ULInformationTransferMRDC field descriptions

ul-DCCH-MessageNR Includes the UL-DCCH-Message as defined in TS 38.331 [82]. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport, NR RRC UEAssistanceInformation, NR RRC IABOtherInformation, NR RRC FailureInformation, and the NR RRC RRCReconfigurationComplete messages.

#### –	WLANConnectionStatusReport

The WLANConnectionStatusReport message is used to inform the successful connection to WLAN or failure of the WLAN connection or connection attempt(s).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

WLANConnectionStatusReport message

-- ASN1START

WLANConnectionStatusReport-r13 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

wlanConnectionStatusReport-r13		WLANConnectionStatusReport-r13-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

WLANConnectionStatusReport-r13-IEs ::=	SEQUENCE {

wlan-Status-r13					WLAN-Status-r13,

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

nonCriticalExtension			WLANConnectionStatusReport-v1430-IEs	OPTIONAL

}

WLANConnectionStatusReport-v1430-IEs ::=	SEQUENCE {

wlan-Status-v1430				WLAN-Status-v1430,

nonCriticalExtension			SEQUENCE {}								OPTIONAL

}

-- ASN1STOP

WLANConnectionStatusReport field descriptions

wlan-Status Indicates the connection status to WLAN and the cause of failures. If the wlan-Status-v1430 is included, E-UTRAN ignores the wlan-Status-r13.

## 6.3	RRC information elements

-- ASN1START

TypeFFS ::= NULL					-- To be removed

-- ASN1STOP

### 6.3.0	Parameterized types

#### –	SetupRelease

SetupRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.8 for guidelines.

-- ASN1START

SetupRelease { ElementTypeParam } ::= CHOICE {

release			NULL,

setup			ElementTypeParam

}

-- ASN1STOP

### 6.3.1	System information blocks

#### –	SystemInformationBlockPos

The IE SystemInformationBlockPos contains positioning assistance data as defined in TS 36.355 [54].

SystemInformationBlockPos information element

-- ASN1START

SystemInformationBlockPos-r15 ::= SEQUENCE {

assistanceDataSIB-Element-r15		OCTET STRING,

lateNonCriticalExtension			OCTET STRING							OPTIONAL,

...

}

-- ASN1STOP

SystemInformationBlockPos field descriptions

assistanceDataSIB-Element Parameter AssistanceDataSIBelement defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

#### –	SystemInformationBlockType2

The IE SystemInformationBlockType2 contains radio resource configuration information that is common for all UEs.

NOTE:	UE timers and constants related to functionality for which parameters are provided in another SIB are included in the corresponding SIB.

SystemInformationBlockType2 information element

-- ASN1START

SystemInformationBlockType2 ::=		SEQUENCE {

ac-BarringInfo						SEQUENCE {

ac-BarringForEmergency				BOOLEAN,

ac-BarringForMO-Signalling			AC-BarringConfig				OPTIONAL,	-- Need OP

ac-BarringForMO-Data				AC-BarringConfig				OPTIONAL	-- Need OP

}																		OPTIONAL,	-- Need OP

radioResourceConfigCommon			RadioResourceConfigCommonSIB,

ue-TimersAndConstants				UE-TimersAndConstants,

freqInfo							SEQUENCE {

ul-CarrierFreq						ARFCN-ValueEUTRA				OPTIONAL,	-- Need OP

ul-Bandwidth						ENUMERATED {n6, n15, n25, n50, n75, n100}

OPTIONAL,	-- Need OP

additionalSpectrumEmission			AdditionalSpectrumEmission

},

mbsfn-SubframeConfigList			MBSFN-SubframeConfigList			OPTIONAL,	-- Need OR

timeAlignmentTimerCommon			TimeAlignmentTimer,

...,

lateNonCriticalExtension		OCTET STRING (CONTAINING SystemInformationBlockType2-v8h0-IEs)						OPTIONAL,

[[	ssac-BarringForMMTEL-Voice-r9		AC-BarringConfig				OPTIONAL,	-- Need OP

ssac-BarringForMMTEL-Video-r9		AC-BarringConfig				OPTIONAL	-- Need OP

]],

[[	ac-BarringForCSFB-r10				AC-BarringConfig			OPTIONAL	-- Need OP

]],

[[	ac-BarringSkipForMMTELVoice-r12		ENUMERATED {true}			OPTIONAL,	-- Need OP

ac-BarringSkipForMMTELVideo-r12		ENUMERATED {true}			OPTIONAL,	-- Need OP

ac-BarringSkipForSMS-r12			ENUMERATED {true}			OPTIONAL,	-- Need OP

ac-BarringPerPLMN-List-r12			AC-BarringPerPLMN-List-r12	OPTIONAL	-- Need OP

]],

[[	voiceServiceCauseIndication-r12		ENUMERATED {true}			OPTIONAL	-- Need OP

]],

[[	acdc-BarringForCommon-r13			ACDC-BarringForCommon-r13		OPTIONAL,	-- Need OP

acdc-BarringPerPLMN-List-r13		ACDC-BarringPerPLMN-List-r13	OPTIONAL	-- Need OP

]],

[[

udt-RestrictingForCommon-r13		UDT-Restricting-r13				OPTIONAL,	-- Need OR

udt-RestrictingPerPLMN-List-r13		UDT-RestrictingPerPLMN-List-r13	OPTIONAL,	-- Need OR

cIoT-EPS-OptimisationInfo-r13		CIOT-EPS-OptimisationInfo-r13	OPTIONAL,	-- Need OP

useFullResumeID-r13					ENUMERATED {true}				OPTIONAL	-- Need OP

]],

[[	unicastFreqHoppingInd-r13			ENUMERATED {true}				OPTIONAL	-- Need OP

]],

[[	mbsfn-SubframeConfigList-v1430		MBSFN-SubframeConfigList-v1430	OPTIONAL,	-- Need OP

videoServiceCauseIndication-r14		ENUMERATED {true}				OPTIONAL	-- Need OP

]],

[[	plmn-InfoList-r15					PLMN-InfoList-r15				OPTIONAL	-- Need OP

]],

[[	cp-EDT-r15							ENUMERATED {true}				OPTIONAL,	-- Need OR

up-EDT-r15							ENUMERATED {true}				OPTIONAL,	-- Need OR

idleModeMeasurements-r15			ENUMERATED {true}				OPTIONAL,	-- Need OR

reducedCP-LatencyEnabled-r15		ENUMERATED {true}				OPTIONAL	-- Need OR

]],

[[	mbms-ROM-ServiceIndication-r15	ENUMERATED {true}				OPTIONAL	-- Need OR

]],

[[	rlos-Enabled-r16					ENUMERATED {true}				OPTIONAL,	-- Need OR

earlySecurityReactivation-r16		ENUMERATED {true}				OPTIONAL,	-- Need OR

cp-EDT-5GC-r16						ENUMERATED {true}				OPTIONAL,	-- Need OR

up-EDT-5GC-r16						ENUMERATED {true}				OPTIONAL,	-- Need OR

cp-PUR-EPC-r16						ENUMERATED {true}				OPTIONAL,	-- Need OR

up-PUR-EPC-r16						ENUMERATED {true}				OPTIONAL,	-- Need OR

cp-PUR-5GC-r16						ENUMERATED {true}				OPTIONAL,	-- Need OR

up-PUR-5GC-r16						ENUMERATED {true}				OPTIONAL,	-- Need OR

mpdcch-CQI-Reporting-r16			ENUMERATED {fourBits, both}		OPTIONAL,	-- Need OR

rai-ActivationEnh-r16				ENUMERATED {true}				OPTIONAL,	-- Need OR

idleModeMeasurementsNR-r16			ENUMERATED {true}				OPTIONAL	-- Need OR

]],

[[	ta-Report-r17						ENUMERATED {true}				OPTIONAL	-- Need OR

-- Editor’s Note: FFS on where this parameter is placed

]]

}

SystemInformationBlockType2-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList				SEQUENCE (SIZE (1..maxMultiBands)) OF AdditionalSpectrumEmission	OPTIONAL,	-- Need OR

nonCriticalExtension			SystemInformationBlockType2-v9e0-IEs	OPTIONAL

}

SystemInformationBlockType2-v9e0-IEs ::= SEQUENCE {

ul-CarrierFreq-v9e0					ARFCN-ValueEUTRA-v9e0		OPTIONAL,	-- Cond ul-FreqMax

nonCriticalExtension				SystemInformationBlockType2-v9i0-IEs					OPTIONAL

}

SystemInformationBlockType2-v9i0-IEs ::= SEQUENCE {

-- Following field is for any non-critical extensions from REL-9

nonCriticalExtension			OCTET STRING (CONTAINING SystemInformationBlockType2-v10m0-IEs)						OPTIONAL,

dummy		SEQUENCE {}		OPTIONAL

}

SystemInformationBlockType2-v10m0-IEs ::= SEQUENCE {

freqInfo-v10l0						SEQUENCE {

additionalSpectrumEmission-v10l0			AdditionalSpectrumEmission-v10l0

}														OPTIONAL,

multiBandInfoList-v10l0				SEQUENCE (SIZE (1..maxMultiBands)) OF

AdditionalSpectrumEmission-v10l0				OPTIONAL,

nonCriticalExtension		SystemInformationBlockType2-v10n0-IEs		OPTIONAL

}

SystemInformationBlockType2-v10n0-IEs ::= SEQUENCE {

-- Following field is for non-critical extensions up-to REL-12

lateNonCriticalExtension	OCTET STRING								OPTIONAL,

nonCriticalExtension		SystemInformationBlockType2-v13c0-IEs		OPTIONAL

}

SystemInformationBlockType2-v13c0-IEs ::= SEQUENCE {

uplinkPowerControlCommon-v13c0	UplinkPowerControlCommon-v1310			OPTIONAL,	-- Need OR

-- Following field is for non-critical extensions from REL-13

nonCriticalExtension			SEQUENCE {}	OPTIONAL

}

AC-BarringConfig ::=				SEQUENCE {

ac-BarringFactor					ENUMERATED {

p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

ac-BarringTime						ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512},

ac-BarringForSpecialAC				BIT STRING (SIZE(5))

}

MBSFN-SubframeConfigList ::=		SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF MBSFN-SubframeConfig

MBSFN-SubframeConfigList-v1430 ::=		SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF MBSFN-SubframeConfig-v1430

AC-BarringPerPLMN-List-r12 ::=		SEQUENCE (SIZE (1.. maxPLMN-r11)) OF AC-BarringPerPLMN-r12

AC-BarringPerPLMN-r12 ::=			SEQUENCE {

plmn-IdentityIndex-r12					INTEGER (1..maxPLMN-r11),

ac-BarringInfo-r12						SEQUENCE {

ac-BarringForEmergency-r12			BOOLEAN,

ac-BarringForMO-Signalling-r12		AC-BarringConfig	OPTIONAL,	-- Need OP

ac-BarringForMO-Data-r12			AC-BarringConfig	OPTIONAL	-- Need OP

}															OPTIONAL,	-- Need OP

ac-BarringSkipForMMTELVoice-r12		ENUMERATED {true}		OPTIONAL,	-- Need OP

ac-BarringSkipForMMTELVideo-r12		ENUMERATED {true}		OPTIONAL,	-- Need OP

ac-BarringSkipForSMS-r12			ENUMERATED {true}		OPTIONAL,	-- Need OP

ac-BarringForCSFB-r12				AC-BarringConfig		OPTIONAL,	-- Need OP

ssac-BarringForMMTEL-Voice-r12		AC-BarringConfig		OPTIONAL,	-- Need OP

ssac-BarringForMMTEL-Video-r12		AC-BarringConfig		OPTIONAL	-- Need OP

}

ACDC-BarringForCommon-r13 ::=			SEQUENCE {

acdc-HPLMNonly-r13						BOOLEAN,

barringPerACDC-CategoryList-r13			BarringPerACDC-CategoryList-r13

}

ACDC-BarringPerPLMN-List-r13 ::=		SEQUENCE (SIZE (1.. maxPLMN-r11)) OF ACDC-BarringPerPLMN-r13

ACDC-BarringPerPLMN-r13 ::=			SEQUENCE {

plmn-IdentityIndex-r13				INTEGER (1..maxPLMN-r11),

acdc-OnlyForHPLMN-r13				BOOLEAN,

barringPerACDC-CategoryList-r13		BarringPerACDC-CategoryList-r13

}

BarringPerACDC-CategoryList-r13 ::= SEQUENCE (SIZE (1..maxACDC-Cat-r13)) OF BarringPerACDC-Category-r13

BarringPerACDC-Category-r13 ::= SEQUENCE {

acdc-Category-r13				INTEGER (1..maxACDC-Cat-r13),

acdc-BarringConfig-r13			SEQUENCE {

ac-BarringFactor-r13			ENUMERATED {

p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

ac-BarringTime-r13				ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512}

}										OPTIONAL	-- Need OP

}

UDT-Restricting-r13	::= SEQUENCE {

udt-Restricting-r13					ENUMERATED {true}			OPTIONAL, --Need OR

udt-RestrictingTime-r13				ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512} OPTIONAL --Need OR

}

UDT-RestrictingPerPLMN-List-r13 ::=	SEQUENCE (SIZE (1..maxPLMN-r11)) OF UDT-RestrictingPerPLMN-r13

UDT-RestrictingPerPLMN-r13 ::= SEQUENCE {

plmn-IdentityIndex-r13			INTEGER (1..maxPLMN-r11),

udt-Restricting-r13				UDT-Restricting-r13			OPTIONAL	--Need OR

}

CIOT-EPS-OptimisationInfo-r13 ::=	SEQUENCE (SIZE (1.. maxPLMN-r11)) OF CIOT-OptimisationPLMN-r13

CIOT-OptimisationPLMN-r13::= SEQUENCE {

up-CIoT-EPS-Optimisation-r13		ENUMERATED {true}			OPTIONAL,	-- Need OP

cp-CIoT-EPS-Optimisation-r13		ENUMERATED {true}			OPTIONAL,	-- Need OP

attachWithoutPDN-Connectivity-r13	ENUMERATED {true}			OPTIONAL	-- Need OP

}

PLMN-InfoList-r15 ::=				SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-Info-r15

PLMN-Info-r15 ::=			SEQUENCE {

upperLayerIndication-r15			ENUMERATED {true}			OPTIONAL		-- Need OR

}

-- ASN1STOP

| SystemInformationBlockType2 field descriptions |
| --- |
| ac-BarringFactor If the random number drawn by the UE is lower than this value, access is allowed. Otherwise the access is barred. The values are interpreted in the range [0,1): p00 = 0, p05 = 0.05, p10 = 0.10,…, p95 = 0.95. Values other than p00 can only be set if all bits of the corresponding ac-BarringForSpecialAC are set to 0. |
| ac-BarringForCSFB Access class barring for mobile originating CS fallback. |
| ac-BarringForEmergency Access class barring for AC 10. |
| ac-BarringForMO-Data Access class barring for mobile originating calls. |
| ac-BarringForMO-Signalling Access class barring for mobile originating signalling. |
| ac-BarringForSpecialAC Access class barring for AC 11-15. The first/ leftmost bit is for AC 11, the second bit is for AC 12, and so on. |
| ac-BarringTime Mean access barring time value in seconds. |
| acdc-BarringConfig Barring configuration for an ACDC category. If the field is absent, access to the cell is considered as not barred for the ACDC category in accordance with clause 5.3.3.13. |
| acdc-Category Indicates the ACDC category as defined in TS 24.105 [72]. |
| acdc-OnlyForHPLMN Indicates whether ACDC is applicable for UEs not in their HPLMN for the corresponding PLMN. TRUE indicates that ACDC is applicable only for UEs in their HPLMN for the corresponding PLMN. FALSE indicates that ACDC is applicable for both UEs in their HPLMN and UEs not in their HPLMN for the corresponding PLMN. |
| additionalSpectrumEmission The UE requirements related to IE AdditionalSpectrumEmission are defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs and TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs. NOTE 1. |
| attachWithoutPDN-Connectivity If present, the field indicates that attach without PDN connectivity as specified in TS 24.301 [35] is supported for this PLMN. |
| barringPerACDC-CategoryList A list of barring information per ACDC category according to the order defined in TS 22.011 [10]. The first entry in the list corresponds to the highest ACDC category of which applications are the least restricted in access attempts at a cell, the second entry in the list corresponds to the ACDC category of which applications are restricted more than applications of the highest ACDC category in access attempts at a cell, and so on. The last entry in the list corresponds to the lowest ACDC category of which applications are the most restricted in access attempts at a cell. |
| cIoT-EPS-OptimisationInfo A list of CIoT EPS related parameters. Value 1 indicates parameters for the PLMN listed 1st in the 1st plmn-IdentityList included in SIB1. Value 2 indicates parameters for the PLMN listed 2nd in the same plmn-IdentityList, or when no more PLMN are present within the same plmn-IdentityList, then the value indicates paramters for PLMN listed 1st in the subsequent plmn-IdentityList within the same SIB1 and so on. NOTE 1. |
| cp-CIoT-EPS-Optimisation This field indicates if the UE is allowed to establish the connection with Control plane CIoT EPS Optimisation, see TS 24.301 [35]. |
| cp-EDT This field indicates whether the UE is allowed to initiate CP-EDT when connected to EPC, see 5.3.3.1b. |
| cp-EDT-5GC This field indicates whether the UE is allowed to initiate CP-EDT when connected to 5GC, see 5.3.3.1b. |
| cp-PUR-5GC This field indicates whether CP transmission using PUR is supported in the cell when connected to 5GC, see 5.3.3.1c. |
| cp-PUR-EPC This field indicates whether CP transmission using PUR is supported in the cell when connected to EPC, see 5.3.3.1c. |
| dummy This field is not used in the specification. If received it shall be ignored by the UE. |
| earlySecurityReactivation If present, this field indicates that early security reactivation when resuming a suspended RRC connection as specified in 5.3.3.18 is supported. |
| idleModeMeasurements This field indicates that a UE that is configured for EUTRA idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform EUTRA idle/inactive measurements. |
| idleModeMeasurementsNR This field indicates that a UE that is configured for NR idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform NR idle/inactive measurements. |
| mbms-ROM-ServiceIndication This field indicates whether the UE is allowed to send MBMSInterestIndication message for the purpose of indicating receive only mode MBMS service parameters. |  |
| mbsfn-SubframeConfigList Defines the subframes that are reserved for MBSFN in downlink. NOTE 1. If the cell is a FeMBMS/Unicast mixed cell, EUTRAN includes mbsfn-SubframeConfigList-v1430. If a FeMBMS/Unicast mixed cell does not use sub-frames #4 or #9 as MBSFN sub-frames, mbsfn-SubframeConfigList-v1430 is still included and indicates all sub-frames as non-MBSFN sub-frames. |
| mpdcch-CQI-Reporting This field indicates if downlink channel quality reporting during random access procedure is allowed, see TS 36.321 [6]. Value 'fourBits' indicates 4-bit CQI reporting is allowed and value 'both' indicates both 2-bit and 4-bit reporting are allowed. |
| multiBandInfoList A list of AdditionalSpectrumEmission i.e. one for each additional frequency band included in multiBandInfoList in SystemInformationBlockType1, listed in the same order. If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList. |
| plmn-IdentityIndex Index of the PLMN across the plmn-IdentityList fields included in SIB1. Value 1 indicates the PLMN listed 1st in the 1st plmn-IdentityList included in SIB1. Value 2 indicates the PLMN listed 2nd in the same plmn-IdentityList, or when no more PLMN are present within the same plmn-IdentityList, then the PLMN listed 1st in the subsequent plmn-IdentityList within the same SIB1 and so on. NOTE 1. |
| plmn-InfoList If E-UTRAN includes this field, it includes the same number of entries, and listed in the same order as PLMNs across the plmn-IdentityList fields included in SIB1. I.e. the first entry corresponds to the first entry of the combined list that results from concatenating the entries included in the second to the original plmn-IdentityList field. |
| rai-ActivationEnh Indicates whether UE connected to EPC is allowed to report the AS release assistance indication using the DCQR and AS RAI MAC CE in the cell as specified in TS 36.321 [6]. |  |
| reducedCP-LatencyEnabled If present, reduced control plane latency is enabled. UEs supporting reduced CP latency transmit Msg3 according to timing as specified in TS 36.213 [23] when transmitting RRCConnectionResumeRequest in Msg3. |  |
| rlos-Enabled Indicates whether access to RLOS is allowed as specified in TS 23.401 [41]. |  |
| ssac-BarringForMMTEL-Video Service specific access class barring for MMTEL video originating calls. |
| ssac-BarringForMMTEL-Voice Service specific access class barring for MMTEL voice originating calls. |
| ta-Report Indicates whether UE specific TA reporting in Msg3 or Msg5 at initial access is enabled as specified in TS 36.321 [6]. |
| udt-Restricting Value TRUE indicates that the UE should indicate to the higher layers to restrict unattended data traffic TS 22.101 [77] irrespective of the UE being in RRC_IDLE or RRC_CONNECTED. The UE shall not indicate to the higher layers if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11]. |
| udt-RestrictingTime If present and when the udt-Restricting changes from TRUE, the UE runs a timer for a period equal to rand * udt-RestrictingTime, where rand is a random number drawn that is uniformly distributed in the range 0 ≤ rand < 1 value in seconds. The timer stops if udt-Restricting changes to TRUE. Upon timer expiry, the UE indicates to the higher layers that the restriction is alleviated. |
| unicastFreqHoppingInd This field indicates if the UE is allowed to indicate support of frequency hopping for unicast MPDCCH/PDSCH/PUSCH as described in TS 36.321 [6]. This field is included only in the BR version of SI message carrying SystemInformationBlockType2. |
| ul-Bandwidth Parameter: transmission bandwidth configuration, NRB, in uplink, see TS 36.101 [42], table 5.6-1. Value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on. If for FDD this parameter is absent, the uplink bandwidth is equal to the downlink bandwidth. For TDD this parameter is absent and it is equal to the downlink bandwidth. NOTE 1. |
| ul-CarrierFreq For FDD: If absent, the (default) value determined from the default TX-RX frequency separation defined in TS 36.101 [42], table 5.7.3-1, applies. For TDD: This parameter is absent and it is equal to the downlink frequency. NOTE 1. |
| up-CIoT-EPS-Optimisation This field indicates if the UE is allowed to resume the connection with User plane CIoT EPS Optimisation, see TS 24.301 [35]. |
| up-EDT This field indicates whether the UE is allowed to initiate UP-EDT when connected to EPC, see 5.3.3.1b. |
| up-EDT-5GC This field indicates whether the UE is allowed to initiate UP-EDT when connected to 5GC, see 5.3.3.1b. |
| up-PUR-5GC This field indicates whether UP transmission using PUR is supported in the cell when connected to 5GC, see 5.3.3.1c. |
| up-PUR-EPC This field indicates whether UP transmission using PUR is supported in the cell when connected to EPC, see 5.3.3.1c. |
| upperLayerIndication Indication to be provided to upper layers. |
| useFullResumeID This field indicates if the UE indicates full resume ID of 40 bits in RRCConnectionResumeRequest. |
| videoServiceCauseIndication Indicates whether the UE is requested to use the establishment cause mo-VoiceCall for mobile originating MMTEL video calls. |
| voiceServiceCauseIndication Indicates whether UE is requested to use the establishment cause mo-VoiceCall for mobile originating MMTEL voice calls. |

| Conditional presence | Explanation |
| --- | --- |
| ul-FreqMax | The field is mandatory present if ul-CarrierFreq (i.e. without suffix) is present and set to maxEARFCN. Otherwise the field is not present. |

NOTE 1:	E-UTRAN sets this field to the same value for all instances of SI message that are broadcasted within the same cell.

#### –	SystemInformationBlockType3

The IE SystemInformationBlockType3 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SystemInformationBlockType3 information element

-- ASN1START

SystemInformationBlockType3 ::=		SEQUENCE {

cellReselectionInfoCommon			SEQUENCE {

q-Hyst								ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars			SEQUENCE {

mobilityStateParameters				MobilityStateParameters,

q-HystSF						SEQUENCE {

sf-Medium						ENUMERATED {

dB-6, dB-4, dB-2, dB0},

sf-High							ENUMERATED {

dB-6, dB-4, dB-2, dB0}

}

}																OPTIONAL		-- Need OP

},

cellReselectionServingFreqInfo		SEQUENCE {

s-NonIntraSearch					ReselectionThreshold		OPTIONAL,		-- Need OP

threshServingLow					ReselectionThreshold,

cellReselectionPriority				CellReselectionPriority

},

intraFreqCellReselectionInfo		SEQUENCE {

q-RxLevMin							Q-RxLevMin,

p-Max								P-Max						OPTIONAL,		-- Need OP

s-IntraSearch						ReselectionThreshold		OPTIONAL,		-- Need OP

allowedMeasBandwidth				AllowedMeasBandwidth		OPTIONAL,		-- Need OP

presenceAntennaPort1				PresenceAntennaPort1,

neighCellConfig						NeighCellConfig,

t-ReselectionEUTRA					T-Reselection,

t-ReselectionEUTRA-SF				SpeedStateScaleFactors		OPTIONAL		-- Need OP

},

...,

lateNonCriticalExtension				OCTET STRING (CONTAINING SystemInformationBlockType3-v10j0-IEs)	OPTIONAL,

[[	s-IntraSearch-v920					SEQUENCE {

s-IntraSearchP-r9					ReselectionThreshold,

s-IntraSearchQ-r9					ReselectionThresholdQ-r9

}																OPTIONAL,		-- Need OP

s-NonIntraSearch-v920				SEQUENCE {

s-NonIntraSearchP-r9				ReselectionThreshold,

s-NonIntraSearchQ-r9				ReselectionThresholdQ-r9

}																OPTIONAL,		-- Need OP

q-QualMin-r9						Q-QualMin-r9				OPTIONAL,		-- Need OP

threshServingLowQ-r9				ReselectionThresholdQ-r9	OPTIONAL		-- Need OP

]],

[[	q-QualMinWB-r11						Q-QualMin-r9				OPTIONAL	-- Cond WB-RSRQ

]],

[[	q-QualMinRSRQ-OnAllSymbols-r12			Q-QualMin-r9				OPTIONAL			-- Cond RSRQ

]],

[[	cellReselectionServingFreqInfo-v1310 CellReselectionServingFreqInfo-v1310	OPTIONAL,		-- Need OP

redistributionServingInfo-r13			RedistributionServingInfo-r13 OPTIONAL,	--Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13		OPTIONAL,		-- Need OP

t-ReselectionEUTRA-CE-r13				T-ReselectionEUTRA-CE-r13	OPTIONAL		-- Need OP

]],

[[	cellSelectionInfoCE1-r13				CellSelectionInfoCE1-r13	OPTIONAL	-- Need OP

]],

[[	cellSelectionInfoCE1-v1360			CellSelectionInfoCE1-v1360	OPTIONAL		-- Cond QrxlevminCE1

]],

[[	cellReselectionInfoCommon-v1460		CellReselectionInfoCommon-v1460	OPTIONAL	-- Need OR

]],

[[	cellReselectionInfoHSDN-r15			CellReselectionInfoHSDN-r15	OPTIONAL,		-- Need OR

cellSelectionInfoCE-v1530			CellSelectionInfoCE-v1530		OPTIONAL,		-- Need OP

crs-IntfMitigNeighCellsCE-r15		ENUMERATED {enabled}		OPTIONAL		-- Need OP

]],

[[	cellReselectionServingFreqInfo-v1610	CellReselectionServingFreqInfo-v1610	OPTIONAL	-- Need OR

]]

}

RedistributionServingInfo-r13 ::=		SEQUENCE {

redistributionFactorServing-r13		INTEGER(0..10),

redistributionFactorCell-r13		ENUMERATED{true}				OPTIONAL,	--Need OP

t360-r13							ENUMERATED {min4, min8, min16, min32,infinity,

spare3,spare2,spare1},

redistrOnPagingOnly-r13				ENUMERATED {true}		OPTIONAL	--Need OP

}

CellReselectionServingFreqInfo-v1310 ::=	SEQUENCE {

cellReselectionSubPriority-r13				CellReselectionSubPriority-r13

}

CellReselectionServingFreqInfo-v1610 ::= SEQUENCE {

altCellReselectionPriority-r16		CellReselectionPriority		OPTIONAL, -- Need OR

altCellReselectionSubPriority-r16	CellReselectionSubPriority-r13	OPTIONAL -- Need OR

}

-- Late non critical extensions

SystemInformationBlockType3-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType3-v10l0-IEs					OPTIONAL

}

SystemInformationBlockType3-v10l0-IEs ::= SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

CellReselectionInfoCommon-v1460 ::=	SEQUENCE {

s-SearchDeltaP-r14					ENUMERATED {dB6, dB9, dB12, dB15}

}

CellReselectionInfoHSDN-r15 ::= SEQUENCE {

cellEquivalentSize-r15				INTEGER(2..16)

}

-- ASN1STOP

| SystemInformationBlockType3 field descriptions |
| --- |
| allowedMeasBandwidth If absent, the value corresponding to the downlink bandwidth indicated by the dl-Bandwidth included in MasterInformationBlock applies. |
| altCellReselectionPriority Alternative cell reselection priorities to be used by the UEs for which the altFreqPriorities is set to true in the RRCConnectionRelease message. |
| altCellReselectionSubPriority Alternative cell reselection sub-priorities to be used by the UEs for which the altFreqPriorities is set to true in the RRCConnectionRelease message. |
| cellEquivalentSize The number of cell count used for mobility state estimation for this cell as specified in TS 36.304 [4]. |
| cellSelectionInfoCE Parameters included in coverage enhancement S criteria for BL UEs and UEs in CE, applicable for intra-frequency neighbour cells. If absent, coverage enhancement S criteria is not applicable. |
| cellSelectionInfoCE1 Parameters included in coverage enhancement S criteria for BL UEs and UEs in CE supporting CE Mode B, applicable for intra-frequency neighbour cells. E-UTRAN includes this IE only if cellSelectionInfoCE in SIB3 is present. |  |
| cellReselectionInfoCommon Cell re-selection information common for cells. |
| cellReselectionServingFreqInfo Information common for Cell re-selection to inter-frequency and inter-RAT cells. |
| crs-IntfMitigNeighCellsCE For BL UEs supporting ce-CRS-IntfMitig, this field indicates CRS interference mitigation, as specified in TS 36.133 [16], clause 3.6.1.2 and 3.6.1.3, is enabled in any of the intra-frequency neibhour cells, and the UE shall perform intra-frequency neighbour cell RRM measurements in the center 6 PRBs. |
| freqBandInfo A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs and TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs, applicable for the intra-frequency neighouring E-UTRA cells if the UE selects the frequency band from freqBandIndicator in SystemInformationBlockType1. If E-UTRAN includes freqBandInfo-v10l0 it includes the same number of entries, and listed in the same order, as in freqBandInfo-r10. |
| intraFreqcellReselectionInfo Cell re-selection information common for intra-frequency cells. |
| multiBandInfoList-v10j0 A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs and TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs, applicable for the intra-frequency neighouring E-UTRA cells if the UE selects the frequency bands in multiBandInfoList (i.e. without suffix) or multiBandInfoList-v9e0. If E-UTRAN includes multiBandInfoList-v10j0, it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList-v10j0. |
| p-Max Value applicable for the intra-frequency neighbouring E-UTRA cells. If absent the UE applies the maximum power according to its capability as specified in TS 36.101 [42], clause 6.2.2. This field is ignored by IAB-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [107]. |
| redistrOnPagingOnly If this field is present and the UE is redistribution capable, the UE shall only wait for the paging message to trigger E-UTRAN inter-frequency redistribution procedure as specified in clause 5.2.4.10 of TS 36.304 [4]. |
| q-Hyst Parameter Qhyst in TS 36.304 [4], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on. |
| q-HystSF Parameter "Speed dependent ScalingFactor for Qhyst" in TS 36.304 [4]. The sf-Medium and sf-High concern the additional hysteresis to be applied, in Medium and High Mobility state respectively, to Qhyst as defined in TS 36.304 [4]. In dB. Value dB-6 corresponds to -6dB, dB-4 corresponds to -4dB and so on. |
| q-QualMin Parameter "Qqualmin" in TS 36.304 [4], applicable for intra-frequency neighbour cells. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin. NOTE 1. |
| q-QualMinRSRQ-OnAllSymbols If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. NOTE 1. |
| q-QualMinWB If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16]. NOTE 1. |
| q-RxLevMin Parameter "Qrxlevmin" in TS 36.304 [4], applicable for intra-frequency neighbour cells. |
| redistributionFactorCell If redistributionFactorCell is present, redistributionFactorServing is only applicable for the serving cell otherwise it is applicable for serving frequency |
| redistributionFactorServing Parameter redistributionFactorServing in TS 36.304 [4]. |
| s-IntraSearch Parameter "SIntraSearchP" in TS 36.304 [4]. If the field s-IntraSearchP is present, the UE applies the value of s-IntraSearchP instead. Otherwise if neither s-IntraSearch nor s-IntraSearchP is present, the UE applies the (default) value of infinity for SIntraSearchP. |
| s-IntraSearchP Parameter "SIntraSearchP" in TS 36.304 [4]. See descriptions under s-IntraSearch. |
| s-IntraSearchQ Parameter "SIntraSearchQ" in TS 36.304 [4]. If the field is not present, the UE applies the (default) value of 0 dB for SIntraSearchQ. |
| s-NonIntraSearch Parameter "SnonIntraSearchP" in TS 36.304 [4]. If the field s-NonIntraSearchP is present, the UE applies the value of s-NonIntraSearchP instead. Otherwise if neither s-NonIntraSearch nor s-NonIntraSearchP is present, the UE applies the (default) value of infinity for SnonIntraSearchP. |
| s-NonIntraSearchP Parameter "SnonIntraSearchP" in TS 36.304 [4]. See descriptions under s-NonIntraSearch. |
| s-NonIntraSearchQ Parameter "SnonIntraSearchQ" in TS 36.304 [4]. If the field is not present, the UE applies the (default) value of 0 dB for SnonIntraSearchQ. |
| s-SearchDeltaP Parameter "SSearchDeltaP" in TS 36.304 [4]. This parameter is only applicable for UEs supporting relaxed monitoring as specified in TS 36.306 [5]. Value dB6 corresponds to 6 dB, dB9 corresponds to 9 dB and so on. |
| speedStateReselectionPars Speed dependent reselection parameters, see TS 36.304 [4]. If this field is absent, i.e, mobilityStateParameters is also not present, UE behaviour is specified in TS 36.304 [4]. |
| t360 Parameter "T360" in TS 36.304 [4]. Value min4 corresponds to 4 minutes, value min8 corresponds to 8 minutes, and so on. |
| threshServingLow Parameter "ThreshServing, LowP" in TS 36.304 [4]. |
| threshServingLowQ Parameter "ThreshServing, LowQ" in TS 36.304 [4]. |
| t-ReselectionEUTRA Parameter "TreselectionEUTRA" in TS 36.304 [4]. |
| t-ReselectionEUTRA-SF Parameter "Speed dependent ScalingFactor for TreselectionEUTRA" in TS 36.304 [4]. If the field is not present, the UE behaviour is specified in TS 36.304 [4]. |

NOTE 1:	The value the UE applies for parameter "Qqualmin" in TS 36.304 [4] depends on the q-QualMin fields signalled by E-UTRAN and supported by the UE. In case multiple candidate options are available, the UE shall select the highest priority candidate option according to the priority order indicated by the following table (top row is highest priority).

| q-QualMinRSRQ-OnAllSymbols | q-QualMinWB | Value of parameter "Qqualmin" in TS 36.304 [4] |
| --- | --- | --- |
| Included | Included | q-QualMinRSRQ-OnAllSymbols – (q-QualMin – q-QualMinWB) |
| Included | Not included | q-QualMinRSRQ-OnAllSymbols |
| Not included | Included | q-QualMinWB |
| Not included | Not included | q-QualMin |

| Conditional presence | Explanation |
| --- | --- |
| QrxlevminCE1 | The field is optionally present, Need OR, if q-RxLevMinCE1-r13 is set below -140 dBm. Otherwise the field is not present. |
| RSRQ | The field is optionally present, Need OR, if threshServingLowQ is present in SIB3; otherwise it is not present. |
| WB-RSRQ | The field is optionally present, need OP if the measurement bandwidth indicated by allowedMeasBandwidth is 50 resource blocks or larger; otherwise it is not present. |

#### –	SystemInformationBlockType4

The IE SystemInformationBlockType4 contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters as well as blacklisted cells.

SystemInformationBlockType4 information element

-- ASN1START

SystemInformationBlockType4 ::=		SEQUENCE {

intraFreqNeighCellList				IntraFreqNeighCellList		OPTIONAL,	-- Need OR

intraFreqBlackCellList				IntraFreqBlackCellList				OPTIONAL,	-- Need OR

csg-PhysCellIdRange					PhysCellIdRange				OPTIONAL,	-- Cond CSG

...,

lateNonCriticalExtension				OCTET STRING				OPTIONAL,

[[	intraFreqNeighHSDN-CellList-r15		IntraFreqNeighHSDN-CellList-r15	OPTIONAL	-- Need OR

]],

[[	rss-ConfigCarrierInfo-r16			RSS-ConfigCarrierInfo-r16		OPTIONAL,	-- Cond RSS

intraFreqNeighCellList-v1610		IntraFreqNeighCellList-v1610	OPTIONAL	-- Cond RSS

]]

}

IntraFreqNeighCellList ::=		SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo

IntraFreqNeighCellList-v1610 ::=	SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo-v1610

IntraFreqNeighHSDN-CellList-r15 ::= SEQUENCE (SIZE (1..maxCellIntra)) OF PhysCellIdRange

IntraFreqNeighCellInfo ::=		SEQUENCE {

physCellId								PhysCellId,

q-OffsetCell							Q-OffsetRange,

...

}

IntraFreqNeighCellInfo-v1610 ::=	SEQUENCE {

rss-MeasPowerBias-r16				RSS-MeasPowerBias-r16

}

IntraFreqBlackCellList ::=		SEQUENCE (SIZE (1..maxCellBlack)) OF PhysCellIdRange

-- ASN1STOP

| SystemInformationBlockType4 field descriptions |
| --- |
| csg-PhysCellIdRange Set of physical cell identities reserved for CSG cells on the frequency on which this field was received. The received csg-PhysCellIdRange applies if less than 24 hours has elapsed since it was received and the UE is camped on a cell of the same primary PLMN where this field was received. The 3 hour validity restriction (clause 5.2.1.3) does not apply to this field. The UE shall not apply any stored csg-PhysCellIdRange when it is in any cell selection state defined in TS 36.304 [4]. |
| intraFreqBlackCellList List of blacklisted intra-frequency neighbouring cells. |
| intraFreqNeighCellList List of intra-frequency neighbouring cells with specific cell re-selection parameters. intraFreqNeighCellList-v1610 indicates list of RSS assistance information which is used for the corresponding physCellId. If E-UTRAN includes intraFreqNeighCellList-v1610, it includes the same number of entries, and listed in the same order, as in intraFreqNeighCellList (i.e. without suffix). If intraFreqNeighCellList-v1610 is absent, measurement based on RSS is not applicable for all the neighbour cells in intraFreqNeighCellList (i.e. without suffix). |
| intraFreqNeighHSDN-CellList List of intra-frequency neighbouring HSDN cells as specified in TS 36.304 [4]. |
| q-OffsetCell Parameter "Qoffsets,n" in TS 36.304 [4]. |
| rss-ConfigCarrierInfo RSS configurations for this carrier frequency. If absent and rss-MeasConfig is included in SIB2, RSS is collocated (time and frequency domain) in all cells. |  |

| Conditional presence | Explanation |
| --- | --- |
| CSG | This field is optional, need OP, for non-CSG cells, and mandatory for CSG cells. |
| RSS | This field is optional, need OP, if rss-MeasConfig is included in SIB2. Otherwise the field is not present, and the UE shall delete any existing value for this field. |

#### –	SystemInformationBlockType5

The IE SystemInformationBlockType5 contains information relevant for inter-frequency cell re-selection (i.e. information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection) and information relevant for E-UTRA and NR idle/inactive measurements. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SystemInformationBlockType5 information element

-- ASN1START

SystemInformationBlockType5 ::=		SEQUENCE {

interFreqCarrierFreqList			InterFreqCarrierFreqList,

...,

lateNonCriticalExtension				OCTET STRING	(CONTAINING SystemInformationBlockType5-v8h0-IEs)				OPTIONAL,

[[	interFreqCarrierFreqList-v1250	InterFreqCarrierFreqList-v1250		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-r12	InterFreqCarrierFreqListExt-r12	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqListExt-v1280	InterFreqCarrierFreqListExt-v1280	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqList-v1310		InterFreqCarrierFreqList-v1310		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1310	InterFreqCarrierFreqListExt-v1310	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqList-v1350		InterFreqCarrierFreqList-v1350	OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1350	InterFreqCarrierFreqListExt-v1350	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqListExt-v1360	InterFreqCarrierFreqListExt-v1360	OPTIONAL	-- Need OR

]],

[[	scptm-FreqOffset-r14				INTEGER (1..8)					OPTIONAL	-- Need OP

]],

[[	interFreqCarrierFreqList-v1530		InterFreqCarrierFreqList-v1530		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1530	InterFreqCarrierFreqListExt-v1530	OPTIONAL,	-- Need OR

measIdleConfigSIB-r15				MeasIdleConfigSIB-r15			OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqList-v1610		InterFreqCarrierFreqList-v1610		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1610	InterFreqCarrierFreqListExt-v1610	OPTIONAL,	-- Need OR

measIdleConfigSIB-NR-r16			MeasIdleConfigSIB-NR-r16			OPTIONAL	-- Need OR

]]

}

-- Late non critical extensions

SystemInformationBlockType5-v8h0-IEs ::=	SEQUENCE {

interFreqCarrierFreqList-v8h0 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v8h0				OPTIONAL,	-- Need OP

nonCriticalExtension			SystemInformationBlockType5-v9e0-IEs							OPTIONAL

}

SystemInformationBlockType5-v9e0-IEs ::=	SEQUENCE {

interFreqCarrierFreqList-v9e0	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v9e0				OPTIONAL,	-- Need OR

nonCriticalExtension			SystemInformationBlockType5-v10j0-IEs	OPTIONAL

}

SystemInformationBlockType5-v10j0-IEs ::=	SEQUENCE {

interFreqCarrierFreqList-v10j0	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v10j0				OPTIONAL,	-- Need OR

nonCriticalExtension			SystemInformationBlockType5-v10l0-IEs		OPTIONAL

}

SystemInformationBlockType5-v10l0-IEs ::=	SEQUENCE {

interFreqCarrierFreqList-v10l0	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v10l0				OPTIONAL,	-- Need OR

nonCriticalExtension			SystemInformationBlockType5-v13a0-IEs			OPTIONAL

}

SystemInformationBlockType5-v13a0-IEs ::=	SEQUENCE {

-- Late non critical extensions from REL-10 upto REL-12

lateNonCriticalExtension		OCTET STRING					OPTIONAL,	-- Need OR

interFreqCarrierFreqList-v13a0	InterFreqCarrierFreqList-v13a0	OPTIONAL,	-- Need OR

-- Late non critical extensions from REL-13

nonCriticalExtension			SEQUENCE {}						OPTIONAL

}

InterFreqCarrierFreqList ::=		SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqList-v1250 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1250

InterFreqCarrierFreqList-v1310 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1310

InterFreqCarrierFreqList-v1350 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1350

InterFreqCarrierFreqList-v13a0 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1360

InterFreqCarrierFreqList-v1530 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1530

InterFreqCarrierFreqList-v1610 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610

InterFreqCarrierFreqListExt-r12 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-r12

InterFreqCarrierFreqListExt-v1280 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v10j0

InterFreqCarrierFreqListExt-v1310 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1310

InterFreqCarrierFreqListExt-v1350 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1350

InterFreqCarrierFreqListExt-v1360 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1360

InterFreqCarrierFreqListExt-v1530 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1530

InterFreqCarrierFreqListExt-v1610 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610

InterFreqCarrierFreqInfo ::=	SEQUENCE {

dl-CarrierFreq						ARFCN-ValueEUTRA,

q-RxLevMin							Q-RxLevMin,

p-Max								P-Max							OPTIONAL,		-- Need OP

t-ReselectionEUTRA					T-Reselection,

t-ReselectionEUTRA-SF				SpeedStateScaleFactors			OPTIONAL,		-- Need OP

threshX-High						ReselectionThreshold,

threshX-Low							ReselectionThreshold,

allowedMeasBandwidth				AllowedMeasBandwidth,

presenceAntennaPort1				PresenceAntennaPort1,

cellReselectionPriority				CellReselectionPriority			OPTIONAL,		-- Need OP

neighCellConfig						NeighCellConfig,

q-OffsetFreq						Q-OffsetRange					DEFAULT dB0,

interFreqNeighCellList				InterFreqNeighCellList			OPTIONAL,		-- Need OR

interFreqBlackCellList				InterFreqBlackCellList			OPTIONAL,		-- Need OR

...,

[[	q-QualMin-r9					Q-QualMin-r9					OPTIONAL,		-- Need OP

threshX-Q-r9					SEQUENCE {

threshX-HighQ-r9				ReselectionThresholdQ-r9,

threshX-LowQ-r9					ReselectionThresholdQ-r9

}																OPTIONAL		-- Cond RSRQ

]],

[[	q-QualMinWB-r11					Q-QualMin-r9					OPTIONAL	-- Cond WB-RSRQ

]]

}

InterFreqCarrierFreqInfo-v8h0 ::=		SEQUENCE {

multiBandInfoList					MultiBandInfoList				OPTIONAL	-- Need OR

}

InterFreqCarrierFreqInfo-v9e0 ::=	SEQUENCE {

dl-CarrierFreq-v9e0					ARFCN-ValueEUTRA-v9e0	OPTIONAL,	-- Cond dl-FreqMax

multiBandInfoList-v9e0				MultiBandInfoList-v9e0	OPTIONAL	-- Need OR

}

InterFreqCarrierFreqInfo-v10j0 ::=	SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL	-- Need OR

}

InterFreqCarrierFreqInfo-v10l0 ::=	SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL	-- Need OR

}

InterFreqCarrierFreqInfo-v1250 ::=		SEQUENCE {

reducedMeasPerformance-r12		ENUMERATED {true}		OPTIONAL,		-- Need OP

q-QualMinRSRQ-OnAllSymbols-r12	Q-QualMin-r9					OPTIONAL	-- Cond RSRQ2

}

InterFreqCarrierFreqInfo-r12 ::=		SEQUENCE {

dl-CarrierFreq-r12					ARFCN-ValueEUTRA-r9,

q-RxLevMin-r12						Q-RxLevMin,

p-Max-r12							P-Max							OPTIONAL,		-- Need OP

t-ReselectionEUTRA-r12				T-Reselection,

t-ReselectionEUTRA-SF-r12			SpeedStateScaleFactors			OPTIONAL,		-- Need OP

threshX-High-r12					ReselectionThreshold,

threshX-Low-r12						ReselectionThreshold,

allowedMeasBandwidth-r12			AllowedMeasBandwidth,

presenceAntennaPort1-r12			PresenceAntennaPort1,

cellReselectionPriority-r12			CellReselectionPriority			OPTIONAL,		-- Need OP

neighCellConfig-r12					NeighCellConfig,

q-OffsetFreq-r12					Q-OffsetRange					DEFAULT dB0,

interFreqNeighCellList-r12			InterFreqNeighCellList			OPTIONAL,		-- Need OR

interFreqBlackCellList-r12			InterFreqBlackCellList			OPTIONAL,		-- Need OR

q-QualMin-r12						Q-QualMin-r9					OPTIONAL,		-- Need OP

threshX-Q-r12						SEQUENCE {

threshX-HighQ-r12					ReselectionThresholdQ-r9,

threshX-LowQ-r12					ReselectionThresholdQ-r9

}																	OPTIONAL,	-- Cond RSRQ

q-QualMinWB-r12						Q-QualMin-r9					OPTIONAL,	-- Cond WB-RSRQ

multiBandInfoList-r12				MultiBandInfoList-r11			OPTIONAL,	-- Need OR

reducedMeasPerformance-r12			ENUMERATED {true}				OPTIONAL,	-- Need OP

q-QualMinRSRQ-OnAllSymbols-r12		Q-QualMin-r9					OPTIONAL,	-- Cond RSRQ2

...

}

InterFreqCarrierFreqInfo-v1310	::=	SEQUENCE {

cellReselectionSubPriority-r13		CellReselectionSubPriority-r13		OPTIONAL,		-- Need OP

redistributionInterFreqInfo-r13		RedistributionInterFreqInfo-r13		OPTIONAL, --Need OP

cellSelectionInfoCE-r13				CellSelectionInfoCE-r13			OPTIONAL,	-- Need OP

t-ReselectionEUTRA-CE-r13			T-ReselectionEUTRA-CE-r13		OPTIONAL	-- Need OP

}

InterFreqCarrierFreqInfo-v1350	::= SEQUENCE {

cellSelectionInfoCE1-r13			CellSelectionInfoCE1-r13			OPTIONAL	-- Need OP

}

InterFreqCarrierFreqInfo-v1360	::= SEQUENCE {

cellSelectionInfoCE1-v1360		CellSelectionInfoCE1-v1360	OPTIONAL	-- Cond QrxlevminCE1

}

InterFreqCarrierFreqInfo-v1530	::= SEQUENCE {

hsdn-Indication-r15					BOOLEAN,

interFreqNeighHSDN-CellList-r15		InterFreqNeighHSDN-CellList-r15		OPTIONAL,	-- Need OR

cellSelectionInfoCE-v1530			CellSelectionInfoCE-v1530			OPTIONAL	-- Need OP

}

InterFreqCarrierFreqInfo-v1610	::= SEQUENCE {

altCellReselectionPriority-r16		CellReselectionPriority		OPTIONAL,	-- Need OR

altCellReselectionSubPriority-r16	CellReselectionSubPriority-r13	OPTIONAL,	-- Need OR

rss-ConfigCarrierInfo-r16				RSS-ConfigCarrierInfo-r16		OPTIONAL,	-- Cond RSS

interFreqNeighCellList-v1610			InterFreqNeighCellList-v1610	OPTIONAL	-- Cond RSS

}

InterFreqNeighCellList ::=			SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo

InterFreqNeighCellList-v1610 ::=		SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1610

InterFreqNeighHSDN-CellList-r15 ::= SEQUENCE (SIZE (1..maxCellInter)) OF PhysCellIdRange

InterFreqNeighCellInfo ::=			SEQUENCE {

physCellId							PhysCellId,

q-OffsetCell						Q-OffsetRange

}

InterFreqNeighCellInfo-v1610 ::= SEQUENCE {

rss-MeasPowerBias-r16			RSS-MeasPowerBias-r16

}

InterFreqBlackCellList ::=			SEQUENCE (SIZE (1..maxCellBlack)) OF PhysCellIdRange

RedistributionInterFreqInfo-r13 ::=		SEQUENCE {

redistributionFactorFreq-r13			RedistributionFactor-r13	OPTIONAL,	--Need OP

redistributionNeighCellList-r13			RedistributionNeighCellList-r13		OPTIONAL	--Need OP

}

RedistributionNeighCellList-r13 ::=		SEQUENCE (SIZE (1..maxCellInter)) OF RedistributionNeighCell-r13

RedistributionNeighCell-r13 ::=		SEQUENCE {

physCellId-r13									PhysCellId,

redistributionFactorCell-r13					RedistributionFactor-r13

}

RedistributionFactor-r13 ::=	INTEGER(1..10)

-- ASN1STOP

| SystemInformationBlockType5 field descriptions |
| --- |
| altCellReselectionPriority Alternative cell reselection priorities to be used by the UEs for which the altFreqPriorities is set to true in the RRCConnectionRelease message. |
| altCellReselectionSubPriority Alternative cell reselection sub-priorities to be used by the UEs for which the altFreqPriorities is set to true in the RRCConnectionRelease message. |
| cellSelectionInfoCE Parameters included in coverage enhancement S criteria for BL UEs and UEs in CE, applicable for inter-frequency neighbour cells. If absent, coverage enhancement S criteria is not applicable. |
| cellSelectionInfoCE1 Parameters included in coverage enhancement S criteria for BL UEs and UEs in CE supporting CE Mode B. E-UTRAN includes this IE only in an entry of InterFreqCarrierFreqList-v1350 or InterFreqCarrierFreqListExt-v1350 if cellSelectionInfoCE is present in the corresponding entry of InterFreqCarrierFreqList-v1310 or InterFreqCarrierFreqListExt-v1310 is present. |  |
| freqBandInfo A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs and TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs, for the frequency band represented by dl-CarrierFreq for which cell reselection parameters are common. If E-UTRAN includes freqBandInfo-v10l0 it includes the same number of entries, and listed in the same order, as in freqBandInfo-r10. |
| hsdn-Indication Indicates whether there are deployed HSDN cells or not on the the DL carrier frequency indicated by dl-CarrierFreq-r12. |
| interFreqBlackCellList List of blacklisted inter-frequency neighbouring cells. |
| interFreqCarrierFreqList List of neighbouring inter-frequencies. E-UTRAN does not configure more than one entry for the same physical frequency regardless of the E-ARFCN used to indicate this. If E-UTRAN includes interFreqCarrierFreqList-v8h0, interFreqCarrierFreqList-v9e0, InterFreqCarrierFreqList-v1250, InterFreqCarrierFreqList-v1310, InterFreqCarrierFreqList-v1350, InterFreqCarrierFreqList-v13a0, InterFreqCarrierFreqList-v1530 and/or InterFreqCarrierFreqList-v1610, it includes the same number of entries, and listed in the same order, as in interFreqCarrierFreqList (i.e. without suffix). See Annex D for more descriptions. |
| interFreqCarrierFreqListExt List of additional neighbouring inter-frequencies, i.e. extending the size of the inter-frequency carrier list using the general principles specified in 5.1.2. E-UTRAN does not configure more than one entry for the same physical frequency regardless of the E-ARFCN used to indicate this. EUTRAN may include interFreqCarrierFreqListExt even if interFreqCarrierFreqList (i.e without suffix) does not include maxFreq entries. If E-UTRAN includes InterFreqCarrierFreqListExt-v1310, InterFreqCarrierFreqListExt-v1350, InterFreqCarrierFreqListExt-v1360, InterFreqCarrierFreqListExt-v1530 and/or InterFreqCarrierFreqList-v1610, it includes the same number of entries, and listed in the same order, as in interFreqCarrierFreqListExt-r12. |
| interFreqNeighCellList List of inter-frequency neighbouring cells with specific cell re-selection parameters. interFreqNeighCellList-v1610 indicates list of RSS assistance information which is used for the corresponding physCellId. If E-UTRAN includes interFreqNeighCellList-v1610 in interFreqCarrierFreqList-v1610 / interFreqCarrierFreqListExt-v1610, it includes the same number of entries, and listed in the same order, as in interFreqNeighCellList (i.e. without suffix) / interFreqNeighCellList-r12. If interFreqNeighCellList-v1610 is absent in interFreqCarrierFreqList-v1610/ interFreqCarrierFreqListExt-v1610, measurement based on RSS is not applicable for all the neighbour cells in interFreqNeighCellList (i.e. without suffix) / interFreqNeighCellList-r12. |
| interFreqNeighHSDN-CellList List of inter-frequency neighbouring HSDN cells as specified in TS 36.304 [4]. |
| measIdleConfigSIB Indicates E-UTRA measurement configuration to be stored and used by the UE while in RRC_IDLE or RRC_INACTIVE. |
| measIdleConfigSIB-NR Indicates the NR measurement configuration to be stored and used by the UE while in RRC_IDLE or RRC_INACTIVE. |
| multiBandInfoList Indicates the list of frequency bands in addition to the band represented by dl-CarrierFreq for which cell reselection parameters are common. E-UTRAN indicates at most maxMultiBands frequency bands (i.e. the total number of entries across both multiBandInfoList and multiBandInfoList-v9e0 is below this limit). |
| multiBandInfoList-v10j0 A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs and TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs, for the frequency bands in multiBandInfoList (i.e. without suffix) and multiBandInfoList-v9e0. If E-UTRAN includes multiBandInfoList-v10j0, it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList-v10j0. |
| p-Max Value applicable for the neighbouring E-UTRA cells on this carrier frequency. If absent the UE applies the maximum power according to its capability as specified in TS 36.101 [42], clause 6.2.2. This field is ignored by IAB-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [107]. |
| q-OffsetCell Parameter "Qoffsets,n" in TS 36.304 [4]. |
| q-OffsetFreq Parameter "Qoffsetfrequency" in TS 36.304 [4]. |
| q-QualMin Parameter "Qqualmin" in TS 36.304 [4]. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin. NOTE 1. |
| q-QualMinRSRQ-OnAllSymbols If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. NOTE 1. |
| q-QualMinWB If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16]. NOTE 1. |
| redistributionFactorFreq Parameter redistributionFactorFreq in TS 36.304 [4]. |
| redistributionFactorCell Parameter redistributionFactorCell in TS 36.304 [4]. |
| reducedMeasPerformance Value TRUE indicates that the neighbouring inter-frequency is configured for reduced measurement performance, see TS 36.133 [16]. If the field is not included, the neighbouring inter-frequency is configured for normal measurement performance, see TS 36.133 [16]. |
| rss-ConfigCarrierInfo RSS configuration for this carrier frequency. If absent and rss-MeasConfig is included in SIB2, RSS is collocated (time and frequency domain) in all cells on this carrier. |
| scptm-FreqOffset Parameter QoffsetSCPTM in TS 36.304 [4]. Actual value QoffsetSCPTM = field value * 2 [dB]. If the field is not present, the UE uses infinite dBs for the SC-PTM frequency offset with cell ranking as specified in TS 36.304 [4]. |
| threshX-High Parameter "ThreshX, HighP" in TS 36.304 [4]. |
| threshX-HighQ Parameter "ThreshX, HighQ" in TS 36.304 [4]. |
| threshX-Low Parameter "ThreshX, LowP" in TS 36.304 [4]. |
| threshX-LowQ Parameter "ThreshX, LowQ" in TS 36.304 [4]. |
| t-ReselectionEUTRA Parameter "TreselectionEUTRA" in TS 36.304 [4]. |
| t-ReselectionEUTRA-SF Parameter "Speed dependent ScalingFactor for TreselectionEUTRA" in TS 36.304 [4]. If the field is not present, the UE behaviour is specified in TS 36.304 [4]. |

NOTE 1:	The value the UE applies for parameter "Qqualmin" in TS 36.304 [4] depends on the q-QualMin fields signalled by E-UTRAN and supported by the UE. In case multiple candidate options are available, the UE shall select the highest priority candidate option according to the priority order indicated by the following table (top row is highest priority).

| q-QualMinRSRQ-OnAllSymbols | q-QualMinWB | Value of parameter "Qqualmin" in TS 36.304 [4] |
| --- | --- | --- |
| Included | Included | q-QualMinRSRQ-OnAllSymbols – (q-QualMin – q-QualMinWB) |
| Included | Not included | q-QualMinRSRQ-OnAllSymbols |
| Not included | Included | q-QualMinWB |
| Not included | Not included | q-QualMin |

| Conditional presence | Explanation |
| --- | --- |
| dl-FreqMax | The field is mandatory present if, for the corresponding entry in InterFreqCarrierFreqList (i.e. without suffix), dl-CarrierFreq (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| QrxlevminCE1 | The field is optionally present, Need OR, if q-RxLevMinCE1-r13 is set below -140 dBm. Otherwise the field is not present. |
| RSRQ | The field is mandatory present if threshServingLowQ is present in systemInformationBlockType3; otherwise it is not present. |
| RSRQ2 | The field is mandatory present for all EUTRA carriers listed in SIB5 if q-QualMinRSRQ-OnAllSymbols is present in SIB3; otherwise it is not present and the UE shall delete any existing value for this field. |
| RSS | This field is optional, need OP, if rss-MeasConfig is included in SIB2. Otherwise the field is not present, and the UE shall delete any existing value for this field. |
| WB-RSRQ | The field is optionally present, need OP if the measurement bandwidth indicated by allowedMeasBandwidth is 50 resource blocks or larger; otherwise it is not present. |

#### –	SystemInformationBlockType6

The IE SystemInformationBlockType6 contains information relevant only for inter-RAT cell re-selection i.e. information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

SystemInformationBlockType6 information element

-- ASN1START

SystemInformationBlockType6 ::=		SEQUENCE {

carrierFreqListUTRA-FDD				CarrierFreqListUTRA-FDD			OPTIONAL,		-- Need OR

carrierFreqListUTRA-TDD				CarrierFreqListUTRA-TDD			OPTIONAL,		-- Need OR

t-ReselectionUTRA					T-Reselection,

t-ReselectionUTRA-SF				SpeedStateScaleFactors			OPTIONAL,		-- Need OP

...,

lateNonCriticalExtension			OCTET STRING	(CONTAINING SystemInformationBlockType6-v8h0-IEs)					OPTIONAL,

[[	carrierFreqListUTRA-FDD-v1250 SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF

CarrierFreqInfoUTRA-v1250		OPTIONAL,	-- Cond UTRA-FDD

carrierFreqListUTRA-TDD-v1250 SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF

CarrierFreqInfoUTRA-v1250		OPTIONAL,	-- Cond UTRA-TDD

carrierFreqListUTRA-FDD-Ext-r12	CarrierFreqListUTRA-FDD-Ext-r12 OPTIONAL,	-- Cond UTRA-FDD

carrierFreqListUTRA-TDD-Ext-r12	CarrierFreqListUTRA-TDD-Ext-r12 OPTIONAL		-- Cond UTRA-TDD

]]

}

SystemInformationBlockType6-v8h0-IEs ::=	SEQUENCE {

carrierFreqListUTRA-FDD-v8h0 SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF CarrierFreqInfoUTRA-FDD-v8h0 OPTIONAL,	-- Cond UTRA-FDD

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

CarrierFreqInfoUTRA-v1250 ::=		SEQUENCE {

reducedMeasPerformance-r12		ENUMERATED {true}		OPTIONAL		-- Need OP

}

CarrierFreqListUTRA-FDD ::=		SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF CarrierFreqUTRA-FDD

CarrierFreqUTRA-FDD ::=				SEQUENCE {

carrierFreq							ARFCN-ValueUTRA,

cellReselectionPriority				CellReselectionPriority			OPTIONAL,		-- Need OP

threshX-High						ReselectionThreshold,

threshX-Low							ReselectionThreshold,

q-RxLevMin							INTEGER (-60..-13),

p-MaxUTRA							INTEGER (-50..33),

q-QualMin							INTEGER (-24..0),

...,

[[	threshX-Q-r9					SEQUENCE {

threshX-HighQ-r9				ReselectionThresholdQ-r9,

threshX-LowQ-r9					ReselectionThresholdQ-r9

}																OPTIONAL		-- Cond RSRQ

]]

}

CarrierFreqInfoUTRA-FDD-v8h0 ::=			SEQUENCE {

multiBandInfoList					SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicator-UTRA-FDD				OPTIONAL	-- Need OR

}

CarrierFreqListUTRA-FDD-Ext-r12 ::=	SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF

CarrierFreqUTRA-FDD-Ext-r12

CarrierFreqUTRA-FDD-Ext-r12 ::=				SEQUENCE {

carrierFreq-r12						ARFCN-ValueUTRA,

cellReselectionPriority-r12			CellReselectionPriority			OPTIONAL,	-- Need OP

threshX-High-r12					ReselectionThreshold,

threshX-Low-r12						ReselectionThreshold,

q-RxLevMin-r12						INTEGER (-60..-13),

p-MaxUTRA-r12						INTEGER (-50..33),

q-QualMin-r12						INTEGER (-24..0),

threshX-Q-r12						SEQUENCE {

threshX-HighQ-r12				ReselectionThresholdQ-r9,

threshX-LowQ-r12				ReselectionThresholdQ-r9

}																OPTIONAL,		-- Cond RSRQ

multiBandInfoList-r12				SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicator-UTRA-FDD				OPTIONAL,	-- Need OR

reducedMeasPerformance-r12			ENUMERATED {true}				OPTIONAL,	-- Need OP

...

}

CarrierFreqListUTRA-TDD ::=		SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF CarrierFreqUTRA-TDD

CarrierFreqUTRA-TDD ::=				SEQUENCE {

carrierFreq							ARFCN-ValueUTRA,

cellReselectionPriority				CellReselectionPriority			OPTIONAL,		-- Need OP

threshX-High						ReselectionThreshold,

threshX-Low							ReselectionThreshold,

q-RxLevMin							INTEGER (-60..-13),

p-MaxUTRA							INTEGER (-50..33),

...

}

CarrierFreqListUTRA-TDD-Ext-r12 ::=	SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF

CarrierFreqUTRA-TDD-r12

CarrierFreqUTRA-TDD-r12 ::=	SEQUENCE {

carrierFreq-r12						ARFCN-ValueUTRA,

cellReselectionPriority-r12			CellReselectionPriority			OPTIONAL,		-- Need OP

threshX-High-r12					ReselectionThreshold,

threshX-Low-r12						ReselectionThreshold,

q-RxLevMin-r12						INTEGER (-60..-13),

p-MaxUTRA-r12						INTEGER (-50..33),

reducedMeasPerformance-r12			ENUMERATED {true}				OPTIONAL,	-- Need OP

...

}

FreqBandIndicator-UTRA-FDD ::=				INTEGER (1..86)

-- ASN1STOP

SystemInformationBlockType6 field descriptions

carrierFreqListUTRA-FDD List of carrier frequencies of UTRA FDD. E-UTRAN does not configure more than one entry for the same physical frequency regardless of the ARFCN used to indicate this. If E-UTRAN includes carrierFreqListUTRA-FDD-v8h0 and/or carrierFreqListUTRA-FDD-v1250, it includes the same number of entries, and listed in the same order, as in carrierFreqListUTRA-FDD (i.e. without suffix). See Annex D for more descriptions.

carrierFreqListUTRA-FDD-Ext List of additional carrier frequencies of UTRA FDD. E-UTRAN does not configure more than one entry for the same physical frequency regardless of the ARFCN used to indicate this. EUTRAN may include carrierFreqListUTRA-FDD-Ext even if carrierFreqListUTRA-FDD (i.e without suffix) does not include maxUTRA-FDD-Carrier entries.

carrierFreqListUTRA-TDD List of carrier frequencies of UTRA TDD. E-UTRAN does not configure more than one entry for the same physical frequency regardless of the ARFCN used to indicate this If E-UTRAN includes carrierFreqListUTRA-TDD-v1250, it includes the same number of entries, and listed in the same order, as in carrierFreqListUTRA-TDD (i.e. without suffix).

carrierFreqListUTRA-TDD-Ext List of additional carrier frequencies of UTRA TDD. E-UTRAN does not configure more than one entry for the same physical frequency regardless of the ARFCN used to indicate this. EUTRAN may include carrierFreqListUTRA-TDD-Ext even if carrierFreqListUTRA-TDD (i.e without suffix) does not include maxUTRA-TDD-Carrier entries.

multiBandInfoList Indicates the list of frequency bands in addition to the band represented by carrierFreq in the CarrierFreqUTRA-FDD for which UTRA cell reselection parameters are common.

p-MaxUTRA The maximum allowed transmission power on the (uplink) carrier frequency, see TS 25.304 [40]. In dBm

q-QualMin Parameter "Qqualmin" in TS 25.304 [40]. Actual value = field value [dB].

q-RxLevMin Parameter "Qrxlevmin" in TS 25.304 [40]. Actual value = field value * 2+1 [dBm].

reducedMeasPerformance Value TRUE indicates that the UTRA carrier frequency is configured for reduced measurement performance, see TS 36.133 [16]. If the field is not included, the UTRA carrier frequency is configured for normal measurement performance, see TS 36.133 [16].

t-ReselectionUTRA Parameter "TreselectionUTRAN" in TS 36.304 [4].

t-ReselectionUTRA-SF Parameter "Speed dependent ScalingFactor for TreselectionUTRA" in TS 36.304 [4]. If the field is not present, the UE behaviour is specified in TS 36.304 [4].

threshX-High Parameter "ThreshX, HighP" in TS 36.304 [4].

threshX-HighQ Parameter "ThreshX, HighQ" in TS 36.304 [4].

threshX-Low Parameter "ThreshX, LowP" in TS 36.304 [4].

threshX-LowQ Parameter "ThreshX, LowQ" in TS 36.304 [4].

| Conditional presence | Explanation |
| --- | --- |
| RSRQ | The field is mandatory present if the threshServingLowQ is present in systemInformationBlockType3; otherwise it is not present. |
| UTRA-FDD | The field is optionally present, need OR, if the carrierFreqListUTRA-FDD is present. Otherwise it is not present. |
| UTRA-TDD | The field is optionally present, need OR, if the carrierFreqListUTRA-TDD is present. Otherwise it is not present. |

#### –	SystemInformationBlockType7

The IE SystemInformationBlockType7 contains information relevant only for inter-RAT cell re-selection i.e. information about GERAN frequencies relevant for cell re-selection. The IE includes cell re-selection parameters for each frequency.

SystemInformationBlockType7 information element

-- ASN1START

SystemInformationBlockType7 ::=		SEQUENCE {

t-ReselectionGERAN					T-Reselection,

t-ReselectionGERAN-SF				SpeedStateScaleFactors				OPTIONAL,	-- Need OR

carrierFreqsInfoList				CarrierFreqsInfoListGERAN			OPTIONAL,	-- Need OR

...,

lateNonCriticalExtension				OCTET STRING				OPTIONAL

}

CarrierFreqsInfoListGERAN ::=			SEQUENCE (SIZE (1..maxGNFG)) OF CarrierFreqsInfoGERAN

CarrierFreqsInfoGERAN ::=			SEQUENCE {

carrierFreqs						CarrierFreqsGERAN,

commonInfo							SEQUENCE {

cellReselectionPriority				CellReselectionPriority			OPTIONAL,	-- Need OP

ncc-Permitted						BIT STRING (SIZE (8)),

q-RxLevMin							INTEGER (0..45),

p-MaxGERAN							INTEGER (0..39)					OPTIONAL,	-- Need OP

threshX-High						ReselectionThreshold,

threshX-Low							ReselectionThreshold

},

...

}

-- ASN1STOP

SystemInformationBlockType7 field descriptions

carrierFreqs The list of GERAN carrier frequencies organised into one group of GERAN carrier frequencies.

carrierFreqsInfoList Provides a list of neighbouring GERAN carrier frequencies, which may be monitored for neighbouring GERAN cells. The GERAN carrier frequencies are organised in groups and the cell reselection parameters are provided per group of GERAN carrier frequencies.

commonInfo Defines the set of cell reselection parameters for the group of GERAN carrier frequencies.

ncc-Permitted Field encoded as a bit map, where bit N is set to "0" if a BCCH carrier with NCC = N-1 is not permitted for monitoring and set to "1" if the BCCH carrier with NCC = N-1 is permitted for monitoring; N = 1 to 8; bit 1 of the bitmap is the leading bit of the bit string.

p-MaxGERAN Maximum allowed transmission power for GERAN on an uplink carrier frequency, see TS 45.008 [28]. Value in dBm. Applicable for the neighbouring GERAN cells on this carrier frequency. If pmaxGERAN is absent, the maximum power according to the UE capability is used.

q-RxLevMin Parameter "Qrxlevmin" in TS 36.304 [4], minimum required RX level in the GSM cell. The actual value of Qrxlevmin in dBm = (field value * 2) 115.

threshX-High Parameter "ThreshX, HighP" in TS 36.304 [4].

threshX-Low Parameter "ThreshX, LowP" in TS 36.304 [4].

t-ReselectionGERAN Parameter "TreselectionGERAN" in TS 36.304 [4].

t-ReselectionGERAN-SF Parameter "Speed dependent ScalingFactor for TreselectionGERAN" in TS 36.304 [4]. If the field is not present, the UE behaviour is specified in TS 36.304 [4].

#### –	SystemInformationBlockType8

The IE SystemInformationBlockType8 contains information relevant only for inter-RAT cell re-selection i.e. information about CDMA2000 frequencies and CDMA2000 neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SystemInformationBlockType8 information element

-- ASN1START

SystemInformationBlockType8 ::=		SEQUENCE {

systemTimeInfo						SystemTimeInfoCDMA2000				OPTIONAL,	-- Need OR

searchWindowSize					INTEGER (0..15)						OPTIONAL,	-- Need OR

parametersHRPD						SEQUENCE {

preRegistrationInfoHRPD				PreRegistrationInfoHRPD,

cellReselectionParametersHRPD		CellReselectionParametersCDMA2000	OPTIONAL -- Need OR

}																		OPTIONAL,	-- Need OR

parameters1XRTT						SEQUENCE {

csfb-RegistrationParam1XRTT			CSFB-RegistrationParam1XRTT		OPTIONAL,	-- Need OP

longCodeState1XRTT					BIT STRING (SIZE (42))			OPTIONAL,	-- Need OR

cellReselectionParameters1XRTT		CellReselectionParametersCDMA2000	OPTIONAL -- Need OR

}																		OPTIONAL,	-- Need OR

...,

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

[[	csfb-SupportForDualRxUEs-r9			BOOLEAN							OPTIONAL,	-- Need OR

cellReselectionParametersHRPD-v920	CellReselectionParametersCDMA2000-v920	OPTIONAL,	-- Cond NCL-HRPD

cellReselectionParameters1XRTT-v920	CellReselectionParametersCDMA2000-v920	OPTIONAL,	-- Cond NCL-1XRTT

csfb-RegistrationParam1XRTT-v920	CSFB-RegistrationParam1XRTT-v920		OPTIONAL,	-- Cond REG-1XRTT

ac-BarringConfig1XRTT-r9			AC-BarringConfig1XRTT-r9	OPTIONAL	-- Cond REG-1XRTT

]],

[[	csfb-DualRxTxSupport-r10			ENUMERATED {true}			OPTIONAL	-- Cond REG-1XRTT

]],

[[	sib8-PerPLMN-List-r11				SIB8-PerPLMN-List-r11		OPTIONAL	-- Need OR

]]

}

CellReselectionParametersCDMA2000 ::= SEQUENCE {

bandClassList						BandClassListCDMA2000,

neighCellList						NeighCellListCDMA2000,

t-ReselectionCDMA2000			T-Reselection,

t-ReselectionCDMA2000-SF			SpeedStateScaleFactors				OPTIONAL	-- Need OP

}

CellReselectionParametersCDMA2000-r11 ::= SEQUENCE {

bandClassList						BandClassListCDMA2000,

neighCellList-r11					SEQUENCE (SIZE (1..16)) OF NeighCellCDMA2000-r11,

t-ReselectionCDMA2000				T-Reselection,

t-ReselectionCDMA2000-SF			SpeedStateScaleFactors				OPTIONAL	-- Need OP

}

CellReselectionParametersCDMA2000-v920 ::= SEQUENCE {

neighCellList-v920						NeighCellListCDMA2000-v920

}

NeighCellListCDMA2000 ::=			SEQUENCE (SIZE (1..16)) OF NeighCellCDMA2000

NeighCellCDMA2000 ::=	SEQUENCE {

bandClass							BandclassCDMA2000,

neighCellsPerFreqList				NeighCellsPerBandclassListCDMA2000

}

NeighCellCDMA2000-r11 ::=	SEQUENCE {

bandClass							BandclassCDMA2000,

neighFreqInfoList-r11				SEQUENCE (SIZE (1..16)) OF NeighCellsPerBandclassCDMA2000-r11

}

NeighCellsPerBandclassListCDMA2000 ::= SEQUENCE (SIZE (1..16)) OF NeighCellsPerBandclassCDMA2000

NeighCellsPerBandclassCDMA2000 ::=	SEQUENCE {

arfcn								ARFCN-ValueCDMA2000,

physCellIdList						PhysCellIdListCDMA2000

}

NeighCellsPerBandclassCDMA2000-r11 ::=	SEQUENCE {

arfcn								ARFCN-ValueCDMA2000,

physCellIdList-r11					SEQUENCE (SIZE (1..40)) OF PhysCellIdCDMA2000

}

NeighCellListCDMA2000-v920 ::=		SEQUENCE (SIZE (1..16)) OF NeighCellCDMA2000-v920

NeighCellCDMA2000-v920 ::=			SEQUENCE {

neighCellsPerFreqList-v920			NeighCellsPerBandclassListCDMA2000-v920

}

NeighCellsPerBandclassListCDMA2000-v920 ::= SEQUENCE (SIZE (1..16)) OF NeighCellsPerBandclassCDMA2000-v920

NeighCellsPerBandclassCDMA2000-v920 ::=	SEQUENCE {

physCellIdList-v920					PhysCellIdListCDMA2000-v920

}

PhysCellIdListCDMA2000 ::=			SEQUENCE (SIZE (1..16)) OF PhysCellIdCDMA2000

PhysCellIdListCDMA2000-v920 ::=		SEQUENCE (SIZE (0..24)) OF PhysCellIdCDMA2000

BandClassListCDMA2000 ::=			SEQUENCE (SIZE (1..maxCDMA-BandClass)) OF BandClassInfoCDMA2000

BandClassInfoCDMA2000 ::=	SEQUENCE {

bandClass							BandclassCDMA2000,

cellReselectionPriority				CellReselectionPriority				OPTIONAL,	-- Need OP

threshX-High						INTEGER (0..63),

threshX-Low							INTEGER (0..63),

...

}

AC-BarringConfig1XRTT-r9 ::=		SEQUENCE {

ac-Barring0to9-r9					INTEGER (0..63),

ac-Barring10-r9						INTEGER (0..7),

ac-Barring11-r9						INTEGER (0..7),

ac-Barring12-r9						INTEGER (0..7),

ac-Barring13-r9						INTEGER (0..7),

ac-Barring14-r9						INTEGER (0..7),

ac-Barring15-r9						INTEGER (0..7),

ac-BarringMsg-r9					INTEGER (0..7),

ac-BarringReg-r9					INTEGER (0..7),

ac-BarringEmg-r9					INTEGER (0..7)

}

SIB8-PerPLMN-List-r11 ::=			SEQUENCE (SIZE (1..maxPLMN-r11)) OF SIB8-PerPLMN-r11

SIB8-PerPLMN-r11 ::=				SEQUENCE {

plmn-Identity-r11					INTEGER (1..maxPLMN-r11),

parametersCDMA2000-r11				CHOICE {

explicitValue						ParametersCDMA2000-r11,

defaultValue						NULL

}

}

ParametersCDMA2000-r11 ::=			SEQUENCE {

systemTimeInfo-r11					CHOICE	{

explicitValue						SystemTimeInfoCDMA2000,

defaultValue						NULL

}																OPTIONAL,	-- Need OR

searchWindowSize-r11				INTEGER (0..15),

parametersHRPD-r11					SEQUENCE {

preRegistrationInfoHRPD-r11			PreRegistrationInfoHRPD,

cellReselectionParametersHRPD-r11	CellReselectionParametersCDMA2000-r11	OPTIONAL -- Need OR

}		OPTIONAL,	-- Need OR

parameters1XRTT-r11					SEQUENCE {

csfb-RegistrationParam1XRTT-r11		CSFB-RegistrationParam1XRTT			OPTIONAL, -- Need OP

csfb-RegistrationParam1XRTT-Ext-r11	CSFB-RegistrationParam1XRTT-v920	OPTIONAL, -- Cond REG-1XRTT-PerPLMN

longCodeState1XRTT-r11				BIT STRING (SIZE (42))	OPTIONAL, -- Cond PerPLMN-LC

cellReselectionParameters1XRTT-r11	CellReselectionParametersCDMA2000-r11	OPTIONAL, -- Need OR

ac-BarringConfig1XRTT-r11			AC-BarringConfig1XRTT-r9			OPTIONAL, -- Cond REG-1XRTT-PerPLMN

csfb-SupportForDualRxUEs-r11		BOOLEAN								OPTIONAL, -- Need OR

csfb-DualRxTxSupport-r11			ENUMERATED {true}			OPTIONAL -- Cond REG-1XRTT-PerPLMN

}		OPTIONAL,	-- Need OR

...

}

-- ASN1STOP

SystemInformationBlockType8 field descriptions

ac-BarringConfig1XRTT Contains the access class barring parameters the UE uses to calculate the access class barring factor, see C.S0097 [53].

ac-Barring0to9 Parameter used for calculating the access class barring factor for access overload classes 0 through 9. It is the parameter "PSIST" in C.S0004 [34] for access overload classes 0 through 9.

ac-BarringEmg Parameter used for calculating the access class barring factor for emergency calls and emergency message transmissions for access overload classes 0 through 9. It is the parameter "PSIST_EMG" in C.S0004 [34].

ac-BarringMsg Parameter used for modifying the access class barring factor for message transmissions. It is the parameter "MSG_PSIST" in C.S0004 [34].

ac-BarringN Parameter used for calculating the access class barring factor for access overload class N (N = 10 to 15). It is the parameter "PSIST" in C.S0004 [34] for access overload class N.

ac-BarringReg Parameter used for modifying the access class barring factor for autonomous registrations. It is the parameter "REG_PSIST" in C.S0004 [34].

bandClass Identifies the Frequency Band in which the Carrier can be found. Details can be found in C.S0057 [24, Table 1.5].

bandClassList List of CDMA2000 frequency bands.

cellReselectionParameters1XRTT Cell reselection parameters applicable only to CDMA2000 1xRTT system.

cellReselectionParameters1XRTT-Ext Cell reselection parameters applicable for cell reselection to CDMA2000 1XRTT system.

cellReselectionParameters1XRTT-v920 Cell reselection parameters applicable for cell reselection to CDMA2000 1XRTT system. The field is not present if cellReselectionParameters1XRTT is not present; otherwise it is optionally present.

cellReselectionParametersHRPD Cell reselection parameters applicable for cell reselection to CDMA2000 HRPD system

cellReselectionParametersHRPD-Ext Cell reselection parameters applicable for cell reselection to CDMA2000 HRPD system.

cellReselectionParametersHRPD-v920 Cell reselection parameters applicable for cell reselection to CDMA2000 HRPD system. The field is not present if cellReselectionParametersHRPD is not present; otherwise it is optionally present.

csfb-DualRxTxSupport Value TRUE indicates that the network supports dual Rx/Tx enhanced 1xCSFB, which enables UEs capable of dual Rx/Tx enhanced 1xCSFB to switch off their 1xRTT receiver/transmitter while camped in E-UTRAN [51].

csfb-RegistrationParam1XRTT Contains the parameters the UE will use to determine if it should perform a CDMA2000 1xRTT Registration/Re-Registration. This field is included if either CSFB or enhanced CS fallback to CDMA2000 1xRTT is supported.

csfb-SupportForDualRxUEs Value TRUE indicates that the network supports dual Rx CSFB [51].

longCodeState1XRTT The state of long code generation registers in CDMA2000 1XRTT system as defined in C.S0002 [12], clause 1.3, at ms, where t equals to the cdma-SystemTime. This field is required for reporting CGI for 1xRTT, SRVCC handover and enhanced CS fallback to CDMA2000 1xRTT operation. Otherwise this IE is not needed. This field is excluded when estimating changes in system information, i.e. changes of longCodeState1XRTT should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1.

neighCellList List of CDMA2000 neighbouring cells. The total number of neighbouring cells in neighCellList for each RAT (1XRTT or HRPD) is limited to 32.

neighCellList-v920 Extended List of CDMA2000 neighbouring cells. The combined total number of CDMA2000 neighbouring cells in both neighCellList and neighCellList-v920 is limited to 32 for HRPD and 40 for 1xRTT.

neighCellsPerFreqList List of carrier frequencies and neighbour cell ids in each frequency within a CDMA2000 Band, see C.S0002 [12] or C.S0024 [26].

neighCellsPerFreqList-v920 Extended list of neighbour cell ids, in the same CDMA2000 Frequency Band as the corresponding instance in "NeighCellListCDMA2000".

parameters1XRTT Parameters applicable for interworking with CDMA2000 1XRTT system.

parametersCDMA2000 Provides the corresponding SIB8 parameters for the CDMA2000 network associated with the PLMN indicated in plmn-Identity. A choice is used to indicate whether for this PLMN the parameters are signalled explicitly or set to the (default) values common for all PLMNs i.e. the values not included in sib8-PerPLMN-List.

parametersHRPD Parameters applicable only for interworking with CDMA2000 HRPD systems.

physCellIdList Identifies the list of CDMA2000 cell ids, see C.S0002 [12] or C.S0024 [26].

physCellIdList-v920 Extended list of CDMA2000 cell ids, in the same CDMA2000 ARFCN as the corresponding instance in "NeighCellsPerBandclassCDMA2000".

plmn-Identity Indicates the PLMN associated with this CDMA2000 network. Value 1 indicates the PLMN listed 1st in the 1st plmn-IdentityList included in SIB1, value 2 indicates the PLMN listed 2nd in the same plmn-IdentityList, or when no more PLMN are present within the same plmn_identityList, then the PLMN listed 1st in the subsequent plmn-IdentityList within the same SIB1 and so on. A PLMN which identity is not indicated in the sib8-PerPLMN-List, does not support inter-working with CDMA2000.

preRegistrationInfoHRPD The CDMA2000 HRPD Pre-Registration Information tells the UE if it should pre-register with the CDMA2000 HRPD network and identifies the Pre-registration zone to the UE.

searchWindowSize The search window size is a CDMA2000 parameter to be used to assist in searching for the neighbouring pilots. For values see C.S0005 [25], Table 2.6.6.2.1-1, and C.S0024 [26], Table 8.7.6.2-4. This field is required for a UE with rx-ConfigHRPD= single and/ or rx-Config1XRTT= single to perform handover, cell re-selection, UE measurement based redirection and enhanced 1xRTT CS fallback from E-UTRAN to CDMA2000 according to this specification and TS 36.304 [4].

sib8-PerPLMN-List This field provides the values for the interworking CDMA2000 networks corresponding, if any, to the UE's RPLMN.

systemTimeInfo Information on CDMA2000 system time. This field is required for a UE with rx-ConfigHRPD= single and/ or rx-Config1XRTT= single to perform handover, cell re-selection, UE measurement based redirection and enhanced 1xRTT CS fallback from E-UTRAN to CDMA2000 according to this specification and TS 36.304 [4]. This field is excluded when estimating changes in system information, i.e. changes of systemTimeInfo should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1. For the field included in ParametersCDMA2000, a choice is used to indicate whether for this PLMN the parameters are signalled explicitly or set to the (default) value common for all PLMNs i.e. the value not included in sib8-PerPLMN-List.

threshX-High Parameter "ThreshX, HighP" in TS 36.304 [4]. This specifies the high threshold used in reselection towards this CDMA2000 band class expressed as an unsigned binary number equal to FLOOR (-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in C.S0005 [25].

threshX-Low Parameter "ThreshX, LowP" in TS 36.304 [4]. This specifies the low threshold used in reselection towards this CDMA2000 band class expressed as an unsigned binary number equal to FLOOR (-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in C.S0005 [25].

t-ReselectionCDMA2000 Parameter "TreselectionCDMA_HRPD" or "TreselectionCDMA_1xRTT" in TS 36.304 [4].

t-ReselectionCDMA2000-SF Parameter "Speed dependent ScalingFactor for TreselectionCDMA-HRPD" or TreselectionCDMA-1xRTT" in TS 36.304 [4]. If the field is not present, the UE behaviour is specified in TS 36.304 [4].

| Conditional presence | Explanation |
| --- | --- |
| NCL-1XRTT | The field is optional present, need OR, if cellReselectionParameters1xRTT is present; otherwise it is not present. |
| NCL-HRPD | The field is optional present, need OR, if cellReselectionParametersHRPD is present; otherwise it is not present. |
| PerPLMN-LC | The field is optional present, need OR, when systemTimeInfo is included in SIB8PerPLMN for this CDMA2000 network; otherwise it is not present. |
| REG-1XRTT | The field is optional present, need OR, if csfb-RegistrationParam1XRTT is present; otherwise it is not present. |
| REG-1XRTT-PerPLMN | The field is optional present, need OR, if csfb-RegistrationParam1XRTT is included in SIB8PerPLMN for this CDMA2000 network; otherwise it is not present. |

#### –	SystemInformationBlockType9

The IE SystemInformationBlockType9 contains a home eNB name (HNB Name).

SystemInformationBlockType9 information element

-- ASN1START

SystemInformationBlockType9 ::=		SEQUENCE {

hnb-Name							OCTET STRING (SIZE(1..48))		OPTIONAL,	-- Need OR

...,

lateNonCriticalExtension				OCTET STRING				OPTIONAL

}

-- ASN1STOP

SystemInformationBlockType9 field descriptions

hnb-Name Carries the name of the home eNB, coded in UTF-8 with variable number of bytes per character, see TS 22.011 [10].

#### –	SystemInformationBlockType10

The IE SystemInformationBlockType10 contains an ETWS primary notification.

SystemInformationBlockType10 information element

-- ASN1START

SystemInformationBlockType10 ::=	SEQUENCE {

messageIdentifier					BIT STRING (SIZE (16)),

serialNumber						BIT STRING (SIZE (16)),

warningType							OCTET STRING (SIZE (2)),

dummy								OCTET STRING (SIZE (50))	OPTIONAL,		-- Need OP

...,

lateNonCriticalExtension			OCTET STRING				OPTIONAL

}

-- ASN1STOP

SystemInformationBlockType10 field descriptions

messageIdentifier Identifies the source and type of ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.44) contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.1, while the trailing bit contains bit 0 of the second octet of the same equivalent IE.

serialNumber Identifies variations of an ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.45), contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.2, while the trailing bit contains bit 0 of the second octet of the same equivalent IE.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

warningType Identifies the warning type of the ETWS primary notification and provides information on emergency user alert and UE popup. The first octet (which is equivalent to the first octet of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.50) contains the first octet of the equivalent IE defined in and encoded according to TS 23.041 [37], clause 9.3.24, and so on.

#### –	SystemInformationBlockType11

The IE SystemInformationBlockType11 contains an ETWS secondary notification.

SystemInformationBlockType11 information element

-- ASN1START

SystemInformationBlockType11 ::=	SEQUENCE {

messageIdentifier					BIT STRING (SIZE (16)),

serialNumber						BIT STRING (SIZE (16)),

warningMessageSegmentType			ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber			INTEGER (0..63),

warningMessageSegment				OCTET STRING,

dataCodingScheme					OCTET STRING (SIZE (1))		OPTIONAL,	-- Cond Segment1

...,

lateNonCriticalExtension				OCTET STRING				OPTIONAL

}

-- ASN1STOP

SystemInformationBlockType11 field descriptions

dataCodingScheme Identifies the alphabet/coding and the language applied variations of an ETWS notification. The octet (which is equivalent to the octet of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.52), contains the octet of the equivalent IE defined in TS 23.041 [37], clause 9.4.3.2.3, and encoded according to TS 23.038 [38].

messageIdentifier Identifies the source and type of ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.44), contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.1, while the trailing bit contains bit 0 of second octet of the same equivalent IE.

serialNumber Identifies variations of an ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.45) contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.2, while the trailing bit contains bit 0 of second octet of the same equivalent IE.

warningMessageSegment Carries a segment of the Warning Message Contents IE defined in TS 36.413 [39], clause 9.2.1.53. The first octet of the Warning Message Contents IE is equivalent to the first octet of the CB data IE defined in and encoded according to TS 23.041 [37], clause 9.4.2.2.5, and so on.

warningMessageSegmentNumber Segment number of the ETWS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, one corresponds to the second segment, and so on.

warningMessageSegmentType Indicates whether the included ETWS warning message segment is the last segment or not.

| Conditional presence | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB11, otherwise it is not present. |

#### –	SystemInformationBlockType12

The IE SystemInformationBlockType12 contains a CMAS notification.

SystemInformationBlockType12 information element

-- ASN1START

SystemInformationBlockType12-r9 ::=	SEQUENCE {

messageIdentifier-r9				BIT STRING (SIZE (16)),

serialNumber-r9						BIT STRING (SIZE (16)),

warningMessageSegmentType-r9		ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber-r9		INTEGER (0..63),

warningMessageSegment-r9			OCTET STRING,

dataCodingScheme-r9					OCTET STRING (SIZE (1))		OPTIONAL,	-- Cond Segment1

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	warningAreaCoordinatesSegment-r15		OCTET STRING	OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

SystemInformationBlockType12 field descriptions

dataCodingScheme Identifies the alphabet/coding and the language applied variations of a CMAS notification. The octet (which is equivalent to the octet of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.52), contains the octet of the equivalent IE defined in TS 23.041 [37], clause 9.4.3.2.3, and encoded according to TS 23.038 [38].

messageIdentifier Identifies the source and type of CMAS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.44) contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.1, while the trailing bit contains bit 0 of second octet of the same equivalent IE.

serialNumber Identifies variations of a CMAS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.45), contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.2, while the trailing bit contains bit 0 of second octet of the same equivalent IE.

warningAreaCoordinatesSegment If present, carries a segment, with one or more octets, of the geographical area where the CMAS warning message is valid as defined in [98]. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 [37] and so on.

warningMessageSegment Carries a segment, with one or more octets, of the Warning Message Contents IE defined in TS 36.413 [39]. The first octet of the Warning Message Contents IE is equivalent to the first octet of the CB data IE defined in and encoded according to TS 23.041 [37], clause 9.4.2.2.5, and so on.

warningMessageSegmentNumber Segment number of the CMAS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, one corresponds to the second segment, and so on. If warning area coordinates are provided for the warning message, then this field applies to both warning message segment and warning area coordinates segment.

warningMessageSegmentType Indicates whether the included CMAS warning message segment is the last segment or not. If warning area coordinates are provided for the warning message, then this field applies to both warning message segment and warning area coordinates segment.

| Conditional presence | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB12, otherwise it is not present. |

#### –	SystemInformationBlockType13

The IE SystemInformationBlockType13 contains the information required to acquire the MBMS control information associated with one or more MBSFN areas.

SystemInformationBlockType13 information element

-- ASN1START

SystemInformationBlockType13-r9 ::=	SEQUENCE {

mbsfn-AreaInfoList-r9				MBSFN-AreaInfoList-r9,

notificationConfig-r9				MBMS-NotificationConfig-r9,

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[

notificationConfig-v1430			MBMS-NotificationConfig-v1430		OPTIONAL

]],

[[

mbsfn-AreaInfoList-r16				MBSFN-AreaInfoList-r16		OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

SystemInformationBlockType13 field descriptions

notificationConfig Indicates the MBMS notification related configuration parameters. The UE shall ignore this field when dl-Bandwidth included in MasterInformationBlock is set to n6.

#### –	SystemInformationBlockType14

The IE SystemInformationBlockType14 contains the EAB parameters.

SystemInformationBlockType14 information element

-- ASN1START

SystemInformationBlockType14-r11 ::=	SEQUENCE {

eab-Param-r11							CHOICE {

eab-Common-r11							EAB-Config-r11,

eab-PerPLMN-List-r11					SEQUENCE (SIZE (1..maxPLMN-r11)) OF EAB-ConfigPLMN-r11

}														OPTIONAL, -- Need OR

lateNonCriticalExtension				OCTET STRING			OPTIONAL,

...,

[[	eab-PerRSRP-r15					ENUMERATED {thresh0, thresh1, thresh2, thresh3}	OPTIONAL	-- Need OR

]]

}

EAB-ConfigPLMN-r11 ::=				SEQUENCE {

eab-Config-r11						EAB-Config-r11				OPTIONAL -- Need OR

}

EAB-Config-r11 ::=					SEQUENCE {

eab-Category-r11					ENUMERATED {a, b, c},

eab-BarringBitmap-r11				BIT STRING (SIZE (10))

}

-- ASN1STOP

SystemInformationBlockType14 field descriptions

eab-BarringBitmap Extended access class barring for AC 0-9. The first/ leftmost bit is for AC 0, the second bit is for AC 1, and so on.

eab-Category Indicates the category of UEs for which EAB applies. Value a corresponds to all UEs, value b corresponds to the UEs that are neither in their HPLMN nor in a PLMN that is equivalent to it, and value c corresponds to the UEs that are neither in the PLMN listed as most preferred PLMN of the country where the UEs are roaming in the operator-defined PLMN selector list on the USIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN, see TS 22.011 [10].

eab-Common The EAB parameters applicable for all PLMN(s).

eab-PerPLMN-List The EAB parameters per PLMN, listed in the same order as the PLMN(s) listed across the plmn-IdentityList fields in SystemInformationBlockType1.

eab-PerRSRP Access barring per RSRP. Value thresh0 means access to the cell is barred when in enhanced coverage as specified in TS 36.304 [4] and does not apply to UEs satisfying S criteria for normal coverage. Value thresh1 is compared to the first entry configured in rsrp-ThresholdsPrachInfoList, value thresh2 is compared to the second entry configured in rsrp-ThresholdsPrachInfoList and so on.

#### –	SystemInformationBlockType15

The IE SystemInformationBlockType15 contains the MBMS Service Area Identities (SAI) of the current and/ or neighbouring carrier frequencies.

SystemInformationBlockType15 information element

-- ASN1START

SystemInformationBlockType15-r11 ::=	SEQUENCE {

mbms-SAI-IntraFreq-r11					MBMS-SAI-List-r11				OPTIONAL,	-- Need OR

mbms-SAI-InterFreqList-r11				MBMS-SAI-InterFreqList-r11		OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...,

[[	mbms-SAI-InterFreqList-v1140		MBMS-SAI-InterFreqList-v1140	OPTIONAL	-- Cond InterFreq

]],

[[	mbms-IntraFreqCarrierType-r14		MBMS-CarrierType-r14			OPTIONAL,	-- Need OR

mbms-InterFreqCarrierTypeList-r14

MBMS-InterFreqCarrierTypeList-r14	OPTIONAL	-- Need OR

]]

}

MBMS-SAI-List-r11 ::=					SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF MBMS-SAI-r11

MBMS-SAI-r11 ::=						INTEGER (0..65535)

MBMS-SAI-InterFreqList-r11 ::=			SEQUENCE (SIZE (1..maxFreq)) OF MBMS-SAI-InterFreq-r11

MBMS-SAI-InterFreqList-v1140 ::=		SEQUENCE (SIZE (1..maxFreq)) OF MBMS-SAI-InterFreq-v1140

MBMS-SAI-InterFreq-r11 ::=				SEQUENCE {

dl-CarrierFreq-r11						ARFCN-ValueEUTRA-r9,

mbms-SAI-List-r11						MBMS-SAI-List-r11

}

MBMS-SAI-InterFreq-v1140 ::=			SEQUENCE {

multiBandInfoList-r11				MultiBandInfoList-r11			OPTIONAL	-- Need OR

}

MBMS-InterFreqCarrierTypeList-r14 ::=	SEQUENCE (SIZE (1..maxFreq)) OF MBMS-CarrierType-r14

MBMS-CarrierType-r14 ::=				SEQUENCE {

carrierType-r14							ENUMERATED {mbms, fembmsMixed, fembmsDedicated},

frameOffset-r14							INTEGER (0..3)					OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType15 field descriptions

carrierType Indicates whether the carrier is pre-Rel-14 MBMS carrier (mbms) or FeMBMS/Unicast mixed carrier (fembmsMixed) or MBMS-dedicated carrier (fembmsDedicated).

frameOffset For MBMS-dedicated carrier, the frameOffset gives the radio frame which contains PBCH by SFN mod 4 = frameOffset.

mbms-InterFreqCarrierTypeList Indicates whether this is an feMBMS carrier. The field is included only if mbms-SAI-InterFreqList-r11 is included. The number of entries is the same in both fields and carrier type relates to the frequency indicated in mbms-SAI-InterFreqList-r11 in the corresponding entry index.

mbms-IntraFreqCarrierType Contains indication whether the carrier is pre-Rel-14 MBMS carrier, FeMBMS/Unicast mixed carrier or MBMS-dedicated carrier.

mbms-SAI-InterFreqList Contains a list of neighboring frequencies including additional bands, if any, that provide MBMS services and the corresponding MBMS SAIs.

mbms-SAI-IntraFreq Contains the list of MBMS SAIs for the current frequency. A duplicate MBMS SAI indicates that this and all following SAIs are not offered by this cell but only by neighbour cells on the current frequency. For MBMS service continuity, the UE shall use all MBMS SAIs listed in mbms-SAI-IntraFreq to derive the MBMS frequencies of interest.

mbms-SAI-List Contains a list of MBMS SAIs for a specific frequency.

multiBandInfoList A list of additional frequency bands applicable for the cells participating in the MBSFN transmission.

| Conditional presence | Explanation |
| --- | --- |
| InterFreq | The field is optionally present, need OR, if the mbms-SAI-InterFreqList-r11 is present. Otherwise it is not present. |

#### –	SystemInformationBlockType16

The IE SystemInformationBlockType16 contains information related to GPS time and Coordinated Universal Time (UTC). The UE may use the parameters provided in this system information block to obtain the UTC, the GPS and the local time.

NOTE:	The UE may use the time information for numerous purposes, possibly involving upper layers e.g. to assist GPS initialisation, to synchronise the UE clock (a.o. to determine MBMS session start/ stop).

SystemInformationBlockType16 information element

-- ASN1START

SystemInformationBlockType16-r11 ::=		SEQUENCE {

timeInfo-r11							SEQUENCE {

timeInfoUTC-r11						INTEGER (0..549755813887),

dayLightSavingTime-r11				BIT STRING (SIZE (2))		OPTIONAL,	-- Need OR

leapSeconds-r11						INTEGER (-127..128)			OPTIONAL,	-- Need OR

localTimeOffset-r11					INTEGER (-63..64)			OPTIONAL	-- Need OR

}																	OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING				OPTIONAL,

...,

[[	timeReferenceInfo-r15				TimeReferenceInfo-r15	OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

SystemInformationBlockType16 field descriptions

dayLightSavingTime It indicates if and how daylight saving time (DST) is applied to obtain the local time. The semantics is the same as the semantics of the Daylight Saving Time IE in TS 24.301 [35] and TS 24.008 [49]. The first/leftmost bit of the bit string contains the b2 of octet 3, i.e. the value part of the Daylight Saving Time IE, and the second bit of the bit string contains b1 of octet 3.

leapSeconds Number of leap seconds offset between GPS Time and UTC. UTC and GPS time are related i.e. GPS time -leapSeconds = UTC time.

localTimeOffset Offset between UTC and local time in units of 15 minutes. Actual value = field value * 15 minutes. Local time of the day is calculated as UTC time + localTimeOffset.

timeInfoUTC Coordinated Universal Time corresponding to the SFN boundary at or immediately after the ending boundary of the SI-window in which SystemInformationBlockType16 is transmitted. The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). NOTE 1. This field is excluded when estimating changes in system information, i.e. changes of timeInfoUTC should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1.

NOTE 1:	The UE may use this field together with the leapSeconds field to obtain GPS time as follows: GPS Time (in seconds) = timeInfoUTC (in seconds) - 2,524,953,600 (seconds) + leapSeconds, where 2,524,953,600 is the number of seconds between 00:00:00 on Gregorian calendar date 1 January, 1900 and 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time).

#### –	SystemInformationBlockType17

The IE SystemInformationBlockType17 contains information relevant for traffic steering between E-UTRAN and WLAN.

SystemInformationBlockType17 information element

-- ASN1START

SystemInformationBlockType17-r12 ::=	SEQUENCE {

wlan-OffloadInfoPerPLMN-List-r12		SEQUENCE (SIZE (1..maxPLMN-r11)) OF

WLAN-OffloadInfoPerPLMN-r12			OPTIONAL, -- Need OR

lateNonCriticalExtension				OCTET STRING				OPTIONAL,

...

}

WLAN-OffloadInfoPerPLMN-r12 ::=			SEQUENCE {

wlan-OffloadConfigCommon-r12		WLAN-OffloadConfig-r12		OPTIONAL,	-- Need OR

wlan-Id-List-r12					WLAN-Id-List-r12			OPTIONAL,	-- Need OR

...

}

WLAN-Id-List-r12 ::=				SEQUENCE (SIZE (1..maxWLAN-Id-r12)) OF WLAN-Identifiers-r12

WLAN-Identifiers-r12 ::=			SEQUENCE {

ssid-r12						OCTET STRING (SIZE (1..32))		OPTIONAL,	-- Need OR

bssid-r12						OCTET STRING (SIZE (6))			OPTIONAL,	-- Need OR

hessid-r12						OCTET STRING (SIZE (6))			OPTIONAL,	-- Need OR

...

}

-- ASN1STOP

SystemInformationBlockType17 field descriptions

bssid Basic Service Set Identifier (BSSID) defined in IEEE 802.11-2012 [67].

hessid Homogenous Extended Service Set Identifier (HESSID) defined in IEEE 802.11-2012 [67].

ssid Service Set Identifier (SSID) defined in IEEE 802.11-2012 [67].

wlan-OffloadInfoPerPLMN-List The WLAN offload configuration per PLMN includes the same number of entries, listed in the same order as the PLMN(s) listed across the plmn-IdentityList fields in SystemInformationBlockType1.

#### –	SystemInformationBlockType18

The IE SystemInformationBlockType18 indicates E-UTRAN supports the sidelink UE information procedure and may contain sidelink communication related resource configuration information.

SystemInformationBlockType18 information element

-- ASN1START

SystemInformationBlockType18-r12 ::= SEQUENCE {

commConfig-r12						SEQUENCE {

commRxPool-r12						SL-CommRxPoolList-r12,

commTxPoolNormalCommon-r12			SL-CommTxPoolList-r12			OPTIONAL,	-- Need OR

commTxPoolExceptional-r12			SL-CommTxPoolList-r12			OPTIONAL,	-- Need OR

commSyncConfig-r12					SL-SyncConfigList-r12		OPTIONAL	-- Need OR

}																		OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

...,

[[	commTxPoolNormalCommonExt-r13			SL-CommTxPoolListExt-r13	OPTIONAL,	-- Need OR

commTxResourceUC-ReqAllowed-r13			ENUMERATED {true}		OPTIONAL,	-- Need OR

commTxAllowRelayCommon-r13				ENUMERATED {true}			OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

SystemInformationBlockType18 field descriptions

commRxPool Indicates the resources by which the UE is allowed to receive sidelink communication while in RRC_IDLE and while in RRC_CONNECTED.

commSyncConfig Indicates the configuration by which the UE is allowed to receive and transmit synchronisation information. E-UTRAN configures commSyncConfig including txParameters when configuring UEs by dedicated signalling to transmit synchronisation information.

commTxAllowRelayCommon Indicates whether the UE is allowed to transmit relay related sidelink communication data using the transmission pools included in SystemInformationBlockType18 i.e. either via commTxPoolNormalCommon, commTxPoolNormalCommonExt or via commTxPoolExceptional.

commTxPoolExceptional Indicates the resources by which the UE is allowed to transmit sidelink communication in exceptional conditions, as specified in 5.10.4.

commTxPoolNormalCommon Indicates the resources by which the UE is allowed to transmit sidelink communication while in RRC_IDLE or when in RRC_CONNECTED while transmitting sidelink via a frequency other than the primary.

commTxPoolNormalCommonExt Indicates transmission resource pool(s) in addition to the pool(s) indicated by field commTxPoolNormalCommon, by which the UE is allowed to transmit sidelink communication while in RRC_IDLE or when in RRC_CONNECTED while transmitting sidelink via a frequency other than the primary. E-UTRAN configures commTxPoolNormalCommonExt only when it configures commTxPoolNormalCommon.

commTxResourceUC-ReqAllowed Indicates whether the UE is allowed to request transmission pools for non-relay related one-to-one sidelink communication.

#### –	SystemInformationBlockType19

The IE SystemInformationBlockType19 indicates E-UTRAN supports the sidelink UE information procedure and may contain sidelink discovery related resource configuration information.

SystemInformationBlockType19 information element

-- ASN1START

SystemInformationBlockType19-r12 ::= SEQUENCE {

discConfig-r12						SEQUENCE {

discRxPool-r12						SL-DiscRxPoolList-r12,

discTxPoolCommon-r12				SL-DiscTxPoolList-r12			OPTIONAL,	-- Need OR

discTxPowerInfo-r12				SL-DiscTxPowerInfoList-r12	OPTIONAL,	-- Cond Tx

discSyncConfig-r12					SL-SyncConfigList-r12		OPTIONAL	-- Need OR

}																		OPTIONAL,	-- Need OR

discInterFreqList-r12				SL-CarrierFreqInfoList-r12		OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

...,

[[	discConfig-v1310				SEQUENCE {

discInterFreqList-v1310			SL-CarrierFreqInfoList-v1310	OPTIONAL,	-- Need OR

gapRequestsAllowedCommon		ENUMERATED {true}			OPTIONAL	-- Need OR

}																OPTIONAL,	-- Need OR

discConfigRelay-r13				SEQUENCE {

relayUE-Config-r13				SL-DiscConfigRelayUE-r13,

remoteUE-Config-r13				SL-DiscConfigRemoteUE-r13

}																OPTIONAL,	-- Need OR

discConfigPS-13					SEQUENCE {

discRxPoolPS-r13				SL-DiscRxPoolList-r12,

discTxPoolPS-Common-r13			SL-DiscTxPoolList-r12		OPTIONAL	-- Need OR

}																OPTIONAL	-- Need OR

]]

}

SL-CarrierFreqInfoList-r12 ::=	SEQUENCE (SIZE (1..maxFreq)) OF SL-CarrierFreqInfo-r12

SL-CarrierFreqInfoList-v1310 ::=	SEQUENCE (SIZE (1..maxFreq)) OF SL-CarrierFreqInfo-v1310

SL-CarrierFreqInfo-r12::=		SEQUENCE {

carrierFreq-r12					ARFCN-ValueEUTRA-r9,

plmn-IdentityList-r12			PLMN-IdentityList4-r12			OPTIONAL	-- Need OP

}

SL-DiscConfigRelayUE-r13	::= SEQUENCE {

threshHigh-r13			RSRP-RangeSL4-r13						OPTIONAL,	-- Need OR

threshLow-r13			RSRP-RangeSL4-r13						OPTIONAL,	-- Need OR

hystMax-r13				ENUMERATED {dB0, dB3, dB6, dB9, dB12, dBinf}	OPTIONAL,	-- Cond ThreshHigh

hystMin-r13				ENUMERATED {dB0, dB3, dB6, dB9, dB12}	OPTIONAL	-- Cond ThreshLow

}

SL-DiscConfigRemoteUE-r13	::= SEQUENCE {

threshHigh-r13			RSRP-RangeSL4-r13						OPTIONAL,	-- Need OR

hystMax-r13				ENUMERATED {dB0, dB3, dB6, dB9, dB12}	OPTIONAL,	-- Cond ThreshHigh

reselectionInfoIC-r13	ReselectionInfoRelay-r13

}

ReselectionInfoRelay-r13 ::=	SEQUENCE {

q-RxLevMin-r13					Q-RxLevMin,

-- Note that the mapping of invidual values may be different for PC5, but the granularity/

-- number of values is same as for Uu

filterCoefficient-r13			FilterCoefficient,

minHyst-r13					ENUMERATED {dB0, dB3,

dB6, dB9, dB12, dBinf}	OPTIONAL	-- Need OR

}

SL-CarrierFreqInfo-v1310::=	SEQUENCE {

discResourcesNonPS-r13			SL-ResourcesInterFreq-r13		OPTIONAL,	-- Need OR

discResourcesPS-r13				SL-ResourcesInterFreq-r13		OPTIONAL,	-- Need OR

discConfigOther-r13			SL-DiscConfigOtherInterFreq-r13		OPTIONAL,	-- Need OR

...

}

PLMN-IdentityList4-r12 ::=	SEQUENCE (SIZE (1..maxPLMN-r11)) OF	PLMN-IdentityInfo2-r12

PLMN-IdentityInfo2-r12 ::=		CHOICE	{

plmn-Index-r12					INTEGER (1..maxPLMN-r11),

plmnIdentity-r12				PLMN-Identity

}

SL-DiscTxResourcesInterFreq-r13 ::=	CHOICE {

acquireSI-FromCarrier-r13		NULL,

discTxPoolCommon-r13			SL-DiscTxPoolList-r12,

requestDedicated-r13			NULL,

noTxOnCarrier-r13				NULL

}

SL-DiscConfigOtherInterFreq-r13::=	SEQUENCE {

txPowerInfo-r13					SL-DiscTxPowerInfoList-r12			OPTIONAL,	-- Cond Tx

refCarrierCommon-r13			ENUMERATED {pCell}					OPTIONAL,	-- Need OR

discSyncConfig-r13				SL-SyncConfigListNFreq-r13			OPTIONAL,	-- Need OR

discCellSelectionInfo-r13		CellSelectionInfoNFreq-r13			OPTIONAL	-- Need OR

}

SL-ResourcesInterFreq-r13 ::= SEQUENCE {

discRxResourcesInterFreq-r13	SL-DiscRxPoolList-r12				OPTIONAL,	-- Need OR

discTxResourcesInterFreq-r13	SL-DiscTxResourcesInterFreq-r13		OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType19 field descriptions

discCellSelectionInfo Parameters that may be used by the UE to select/ reselect a cell on the concerned non serving frequency. If absent, the UE acquires the information from the target cell on the concerned frequency. See TS 36.304 [4], clause 11.4.

discInterFreqList Indicates the neighbouring frequencies on which sidelink discovery announcement is supported. May also provide further information i.e. reception resource pool and/ or transmission resource pool, or an indication how resources could be obtained.

discRxPool Indicates the resources by which the UE is allowed to receive non-PS related sidelink discovery announcements while in RRC_IDLE and while in RRC_CONNECTED.

discRxPoolPS Indicates the resources by which the UE is allowed to receive PS related sidelink discovery announcements while in RRC_IDLE and while in RRC_CONNECTED.

discRxResourcesInterFreq Indicates the resource pool configuration for receiving discovery announcements on a carrier frequency.

discSyncConfig Indicates the configuration by which the UE is allowed to receive and transmit synchronisation information. E-UTRAN configures discSyncConfig including txParameters when configuring UEs by dedicated signalling to transmit synchronisation information.

discTxPoolCommon Indicates the resources by which the UE is allowed to transmit non-PS related sidelink discovery announcements while in RRC_IDLE.

discTxPoolPS-Common Indicates the resources by which the UE is allowed to transmit PS related sidelink discovery announcements while in RRC_IDLE.

discTxResourcesInterFreq For the concerned frequency, either provides the UE with a pool of sidelink discovery announcement transmission resources the UE is allowed to use while in RRC_IDLE, or indicates whether such transmission is allowed, and if so how the UE may obtain the required resources. Value noTxOnCarrier indicates that the UE is not allowed to transmit sidelink discovery announcements on the concerned frequency. Value acquireSI-FromCarrier indicates that the required resources are to be obtained by autonomously acquiring SIB19 and other relevant SIBs from the concerned frequency. Value requestDedicated indicates, that for the concerned carrier, the required sidelink discovery resources are to be obtained by means of a dedicated resource request using the SidelinkUEInformation message.

plmn-IdentityList List of PLMN identities for the neighbouring frequency indicated by carrierFreq. Absence of the field indicates the same PLMN identities as listed across the plmn-IdentityList fields (without suffix) in SystemInformationBlockType1.

plmn-Index Index of the corresponding entry across the plmn-IdentityList fields (without suffix) within SystemInformationBlockType1.

refCarrierCommon Indicates if the PCell (RRC_CONNECTED)/ serving cell (RRC_IDLE) is to be used as reference for DL measurements and synchronization, instead of the DL frequency paired with the one used to transmit sidelink discovery announcements on, see TS 36.213 [23], clause 14.3.1.

reselectionInfoIC Includes the parameters used by the UE when selecting/ reselecting a sidelink relay UE.

SL-CarrierFreqInfoList-v1310 If included, the UE shall include the same number of entries, and listed in the same order, as in SL-CarrierFreqInfoList-r12.

threshHigh, threshLow (relayUE) Indicates when a sidelink remote UE or sidelink relay UE that is in network coverage may use the broadcast PS related sidelink discovery Tx resource pool, if broadcast, or request Tx resources by dedicated signalling otherwise. For remote UEs, this parameter is used similarly for relay related sidelink communication.

| Conditional presence | Explanation |
| --- | --- |
| ThreshHigh | The field is mandatory present if threshHigh is included in the corresponding IE. Otherwise the field is not present and UE shall delete any existing value for this field. |
| ThreshLow | The field is mandatory present if threshLow is included. Otherwise the field is not present UE shall delete any existing value for this field. |
| Tx | The field is mandatory present if discTxPoolCommon is included. Otherwise the field is optional present, need OR. |

#### –	SystemInformationBlockType20

The IE SystemInformationBlockType20 contains the information required to acquire the control information associated transmission of MBMS using SC-PTM.

SystemInformationBlockType20 information element

-- ASN1START

SystemInformationBlockType20-r13 ::=	SEQUENCE {

sc-mcch-RepetitionPeriod-r13		ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256},

sc-mcch-Offset-r13				INTEGER (0..10),

sc-mcch-FirstSubframe-r13		INTEGER (0..9),

sc-mcch-duration-r13			INTEGER (2..9)	OPTIONAL,

sc-mcch-ModificationPeriod-r13	ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256,

rf512, rf1024, r2048, rf4096, rf8192, rf16384, rf32768,

rf65536},

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

...,

[[	br-BCCH-Config-r14					SEQUENCE {

dummy								ENUMERATED {rf1},

dummy2								ENUMERATED {rf1},

mpdcch-Narrowband-SC-MCCH-r14		INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-NumRepetition-SC-MCCH-r14	ENUMERATED {r1, r2, r4, r8, r16,

r32, r64, r128, r256},

mpdcch-StartSF-SC-MCCH-r14			CHOICE {

fdd-r14								ENUMERATED {v1, v1dot5, v2, v2dot5, v4,

v5, v8, v10},

tdd-r14								ENUMERATED {v1, v2, v4, v5, v8, v10, v20}

},

mpdcch-PDSCH-HoppingConfig-SC-MCCH-r14	ENUMERATED {off, ce-ModeA, ce-ModeB},

sc-mcch-CarrierFreq-r14				ARFCN-ValueEUTRA-r9,

sc-mcch-Offset-BR-r14				INTEGER (0..10),

sc-mcch-RepetitionPeriod-BR-r14		ENUMERATED {rf32, rf128, rf512, rf1024,

rf2048, rf4096, rf8192, rf16384},

sc-mcch-ModificationPeriod-BR-r14	ENUMERATED { rf32, rf128, rf256, rf512, rf1024,

rf2048, rf4096, rf8192, rf16384, rf32768,

rf65536, rf131072, rf262144, rf524288,

rf1048576}

}																	OPTIONAL,	-- Need OR

sc-mcch-SchedulingInfo-r14			SC-MCCH-SchedulingInfo-r14		OPTIONAL,	-- Need OP

pdsch-maxNumRepetitionCEmodeA-SC-MTCH-r14

ENUMERATED { r16, r32 }		OPTIONAL,	-- Need OR

pdsch-maxNumRepetitionCEmodeB-SC-MTCH-r14

ENUMERATED {

r192, r256, r384, r512, r768, r1024,

r1536, r2048}				OPTIONAL	-- Need OR

]],

[[	sc-mcch-RepetitionPeriod-v1470		ENUMERATED {rf1}				OPTIONAL,	-- Need OR

sc-mcch-ModificationPeriod-v1470	ENUMERATED {rf1}				OPTIONAL	-- Need OR

]]

}

SC-MCCH-SchedulingInfo-r14::=	SEQUENCE	{

onDurationTimerSCPTM-r14			ENUMERATED {psf10, psf20, psf100, psf300,

psf500, psf1000, psf1200, psf1600},

drx-InactivityTimerSCPTM-r14		ENUMERATED {psf0, psf1, psf2, psf4, psf8, psf16,

psf32, psf64, psf128, psf256, ps512,

psf1024, psf2048, psf4096, psf8192, psf16384},

schedulingPeriodStartOffsetSCPTM-r14	CHOICE {

sf10									INTEGER(0..9),

sf20									INTEGER(0..19),

sf32									INTEGER(0..31),

sf40									INTEGER(0..39),

sf64									INTEGER(0..63),

sf80									INTEGER(0..79),

sf128									INTEGER(0..127),

sf160									INTEGER(0..159),

sf256									INTEGER(0..255),

sf320									INTEGER(0..319),

sf512									INTEGER(0..511),

sf640									INTEGER(0..639),

sf1024									INTEGER(0..1023),

sf2048									INTEGER(0..2047),

sf4096									INTEGER(0..4095),

sf8192									INTEGER(0..8191)

},

...

}

-- ASN1STOP

SystemInformationBlockType20 field descriptions

br-BCCH-Config-r14 The field is present if SystemInformationBlockType20 is sent on BR-BCCH. Otherwise the field is absent.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

drx-InactivityTimerSCPTM Timer for listening to SC-MCCH scheduling in TS 36.321 [6]. Value in number of MPDCCH sub-frames. Value psf0 corresponds to 0 MPDCCH sub-frame, psf1 corresponds to 1 MPDCCH sub-frame and so on.

mpdcch-Narrowband-SC-MCCH Narrowband for MPDCCH for SC-MCCH, see TS 36.213 [23].

mpdcch-NumRepetitions-SC-MCCH The maximum number of MPDCCH repetitions the UE needs to monitor for SC-MCCH, see TS 36.213 [23].

mpdcch-StartSF-SC-MCCH Configuration of the starting subframes of the MPDCCH search space for SC-MCCH, see TS 36.213 [23].

mpdcch-PDSCH-HoppingConfig-SC-MCCH Frequency hopping configuration for MPDCCH/PDSCH for SC-MCCH, see TS 36.213 [23].

onDurationTimerSCPTM Indicates the duration in subframes during which SC-MCCH may be scheduled in MPDCCH sub-frames, see TS 36.321 [6].

pdsch-maxNumRepetitionCEmodeA-SC-MTCH Maximum value to indicate the set of PDSCH repetition numbers for SC-MTCH to UEs in CE mode A, see TS 36.213 [23].

pdsch-maxNumRepetitionCEmodeB-SC-MTCH Maximum value to indicate the set of PDSCH repetition numbers for SC-MTCH CE to UEs in mode B, see TS 36.213 [23].

schedulingPeriodStartOffsetSCPTM SCPTM-SchedulingCycle and SCPTM-SchedulingOffset in TS 36.321 [6]. The value of SCPTM-SchedulingCycle is in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. The value of SCPTM-SchedulingOffset is in number of sub-frames.

sc-mcch-CarrierFreq Downlink carrier used for all multicast SC-MCCH transmissions.

sc-mcch-duration Indicates, starting from the subframe indicated by sc-mcch-FirstSubframe, the duration in subframes during which SC-MCCH may be scheduled in PDCCH sub-frames, see TS 36.321 [6]. Absence of this IE means that SC-MCCH is only scheduled in the subframe indicated by sc-mcch-FirstSubframe.

sc-mcch-ModificationPeriod Defines periodically appearing boundaries, i.e. radio frames for which SFN mod sc-mcch-ModificationPeriod = 0. The contents of different transmissions of SC-MCCH information can only be different if there is at least one such boundary in-between them. Value rf2 corresponds to 2 radio frames, value rf4 corresponds to 4 radio frames and so on. In case sc-mcch-ModificationPeriod-v1470 is configured, the UE shall ignore the configuration of sc-mcch-ModificationPeriod-r13.

sc-mcch-ModificationPeriod-BR Defines periodically appearing boundaries for BL UE or UE in CE, i.e. radio frames for which (H-SFN*1024 + SFN) mod sc-mcch-ModificationPeriod-BR = 0 if hyperSFN is present in SystemInformationBlockType1-BR or radio frames for which SFN mod sc-mcchModificationPeriod-BR = 0 otherwise.The contents of different transmissions of SC-MCCH information can only be different if there is at least one such boundary in-between them. Value rf32 corresponds to 32 radio frames, value rf128 corresponds to 128 radio frames and so on.

sc-mcch-FirstSubframe Indicates the first subframe in which SC-MCCH is scheduled

sc-mcch-Offset Indicates, together with the sc-mcch-RepetitionPeriod, the radio frames in which SC-MCCH is scheduled i.e. SC-MCCH is scheduled in radio frames for which: SFN mod sc-mcch-RepetitionPeriod = sc-mcch-Offset.

sc-mcch-Offset-BR Indicates, together with the sc-mcch-RepetitionPeriod-BR, the boundary of the SC-MCCH repetition period for BL UE or UE in CE: (H-SFN*1024 + SFN) mod sc-mcch-RepetitionPeriod-BR = sc-mcch-Offset-BR if hyperSFN is present in SystemInformationBlockType1-BR or radio frames for which (SFN mod mod sc-mcch-RepetitionPeriod-BR) = sc-mcch-Offset-BR otherwise.

sc-mcch-RepetitionPeriod Defines the interval between transmissions of SC-MCCH information, in radio frames. Value rf2 corresponds to 2 radio frames, rf4 corresponds to 4 radio frames and so on. In case sc-mcch-RepetitionPeriod-v1470 is configured, the UE shall ignore the configuration of sc-mcch-RepetitionPeriod-r13.

sc-mcch-RepetitionPeriod-BR Defines the interval between transmissions of SC-MCCH information for BL UE or UE in CE, in radio frames. Value rf32 corresponds to 32 radio frames, rf128 corresponds to 128 radio frames and so on.

sc-mcch-SchedulingInfo DRX information for the SC-MCCH. If this field is absent, DRX is not used for SC-MCCH reception.

#### –	SystemInformationBlockType21

The IE SystemInformationBlockType21 contains V2X sidelink communication configuration.

SystemInformationBlockType21 information element

-- ASN1START

SystemInformationBlockType21-r14 ::= SEQUENCE {

sl-V2X-ConfigCommon-r14				SL-V2X-ConfigCommon-r14				OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

...,

[[	anchorCarrierFreqListNR-r16		SL-NR-AnchorCarrierFreqList-r16		OPTIONAL	-- Need OR

]]

}

SL-V2X-ConfigCommon-r14 ::=		SEQUENCE {

v2x-CommRxPool-r14					SL-CommRxPoolListV2X-r14			OPTIONAL,	-- Need OR

v2x-CommTxPoolNormalCommon-r14		SL-CommTxPoolListV2X-r14			OPTIONAL,	-- Need OR

p2x-CommTxPoolNormalCommon-r14		SL-CommTxPoolListV2X-r14			OPTIONAL,	-- Need OR

v2x-CommTxPoolExceptional-r14		SL-CommResourcePoolV2X-r14			OPTIONAL,	-- Need OR

v2x-SyncConfig-r14					SL-SyncConfigListV2X-r14			OPTIONAL,	-- Need OR

v2x-InterFreqInfoList-r14			SL-InterFreqInfoListV2X-r14			OPTIONAL,	-- Need OR

v2x-ResourceSelectionConfig-r14		SL-CommTxPoolSensingConfig-r14		OPTIONAL,	-- Need OR

zoneConfig-r14						SL-ZoneConfig-r14					OPTIONAL,	-- Need OR

typeTxSync-r14						SL-TypeTxSync-r14					OPTIONAL,	-- Need OR

thresSL-TxPrioritization-r14		SL-Priority-r13						OPTIONAL,	-- Need OR

anchorCarrierFreqList-r14			SL-AnchorCarrierFreqList-V2X-r14	OPTIONAL,	-- Need OR

offsetDFN-r14						INTEGER (0..1000)					OPTIONAL,	-- Need OR

cbr-CommonTxConfigList-r14			SL-CBR-CommonTxConfigList-r14		OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType21 field descriptions

anchorCarrierFreqList Indicates EUTRA carrier frequencies which may include inter-carrier resource configuration for V2X sidelink communication.

anchorCarrierFreqListNR Indicates NR carrier frequencies which may include inter-carrier resource configuration for V2X sidelink communication.

cbr-CommonTxConfigList Indicates the common list of CBR ranges and the list of PSSCH transmissions parameter configurations available to configure congestion control to the UE for V2X sidelink communication.

offsetDFN Indicates the timing offset for the UE to determine DFN timing when GNSS is used for timing reference for the PCell. Value 0 corresponds to 0 milliseconds, value 1 corresponds to 0.001 milliseconds, value 2 corresponds to 0.002 milliseconds, and so on.

p2x-CommTxPoolNormalCommon Indicates the resources by which the UE is allowed to transmit P2X related V2X sidelink communication. zoneID is not configured in the pools in this field.

thresSL-TxPrioritization Indicates the threshold used to determine whether SL V2X transmission is prioritized over uplink transmission if they overlap in time (see TS 36.321 [6]). This value shall overwrite thresSL-TxPrioritization configured in SL-V2X-Preconfiguration if any.

typeTxSync Indicates the prioritized synchronization type (i.e. eNB or GNSS) for performing V2X sidelink communication on the carrier frequency on which this field is broadcast.

v2x-CommRxPool Indicates the resources by which the UE is allowed to receive V2X sidelink communication while in RRC_IDLE and in RRC_CONNECTED.

v2x-CommTxPoolExceptional Indicates the resources by which the UE is allowed to transmit V2X sidelink communication in exceptional conditions, as specified in 5.10.13.

v2x-CommTxPoolNormalCommon Indicates the resources by which the UE is allowed to transmit non-P2X related V2X sidelink communication when in RRC_IDLE or when in RRC_CONNECTED while transmitting V2X sidelink communication via a frequency other than the primary. E-UTRAN configures one resource pool per zone.

v2x-InterFreqInfoList Indicates synchronization and resource allocation configurations of neighboring frequencies for V2X sidelink communication.

v2x-ResourceSelectionConfig Indicates V2X sidelink communication configurations used for UE autonomous resource selection.

v2x-SyncConfig Indicates the configuration by which the UE is allowed to receive and transmit synchronisation information for V2X sidelink communication. E-UTRAN configures v2x-SyncConfig including txParameters when configuring UEs to transmit synchronisation information.

zoneConfig Indicates zone configurations used for V2X sidelink communication in 5.10.13.2.

#### –	SystemInformationBlockType24

The IE SystemInformationBlockType24 contains information relevant for inter-RAT cell re-selection (i.e. information about NR frequencies and NR neighbouring cells relevant for cell re-selection), which can also be used for NR idle/inactive measurements. The IE includes cell re-selection parameters common for a frequency.

SystemInformationBlockType24 information element

-- ASN1START

SystemInformationBlockType24-r15 ::=	SEQUENCE {

carrierFreqListNR-r15				CarrierFreqListNR-r15				OPTIONAL,		-- Need OR

t-ReselectionNR-r15					T-Reselection,

t-ReselectionNR-SF-r15				SpeedStateScaleFactors				OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	carrierFreqListNR-v1610			CarrierFreqListNR-v1610		OPTIONAL    -- Need OR

]]

}

CarrierFreqListNR-r15 ::=		SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15

CarrierFreqListNR-v1610 ::=		SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1610

CarrierFreqNR-r15 ::=				SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

multiBandInfoList-r15				MultiFrequencyBandListNR-r15		OPTIONAL,	-- Need OR

multiBandInfoListSUL-r15			MultiFrequencyBandListNR-r15		OPTIONAL,	-- Need OR

measTimingConfig-r15				MTC-SSB-NR-r15						OPTIONAL,	-- Need OR

subcarrierSpacingSSB-r15			ENUMERATED {kHz15, kHz30, kHz120, kHz240},

ss-RSSI-Measurement-r15				SS-RSSI-Measurement-r15		OPTIONAL,		-- Cond RSRQ2

cellReselectionPriority-r15			CellReselectionPriority		OPTIONAL,		-- Need OP

cellReselectionSubPriority-r15		CellReselectionSubPriority-r13	OPTIONAL,	-- Need OR

threshX-High-r15					ReselectionThreshold,

threshX-Low-r15						ReselectionThreshold,

threshX-Q-r15						SEQUENCE {

threshX-HighQ-r15				ReselectionThresholdQ-r9,

threshX-LowQ-r15				ReselectionThresholdQ-r9

}																OPTIONAL,	-- Cond RSRQ

q-RxLevMin-r15						INTEGER (-70..-22),

q-RxLevMinSUL-r15					INTEGER (-70..-22)				OPTIONAL,		-- Need OR

p-MaxNR-r15							P-MaxNR-r15,

ns-PmaxListNR-r15					NS-PmaxListNR-r15					OPTIONAL,	-- Need OR

q-QualMin-r15						INTEGER (-43..-12)				OPTIONAL,		-- Need OP

deriveSSB-IndexFromCell-r15			BOOLEAN,

maxRS-IndexCellQual-r15				MaxRS-IndexCellQualNR-r15		OPTIONAL,		-- Need OR

threshRS-Index-r15					ThresholdListNR-r15				OPTIONAL,		-- Need OR

...,

[[	multiBandNsPmaxListNR-v1550		MultiBandNsPmaxListNR-1-v1550	OPTIONAL,	-- Need OR

multiBandNsPmaxListNR-SUL-v1550	MultiBandNsPmaxListNR-v1550		OPTIONAL,	-- Need OR

ssb-ToMeasure-r15				SSB-ToMeasure-r15				OPTIONAL		-- Need OR

]]

}

CarrierFreqNR-v1610 ::=		SEQUENCE {

smtc2-LP-r16						MTC-SSB2-LP-NR-r16				OPTIONAL,	  -- Need OR

ssb-PositionQCL-CommonNR-r16	SSB-PositionQCL-RelationNR-r16	OPTIONAL,	-- Cond SharedSpectrum2

whiteCellListNR-r16				WhiteCellListNR-r16					OPTIONAL,	-- Cond SharedSpectrum

highSpeedCarrierNR-r16			ENUMERATED {true}				OPTIONAL	  -- Need OR

}

MultiBandNsPmaxListNR-1-v1550	::=	SEQUENCE (SIZE (1.. maxMultiBandsNR-1-r15)) OF NS-PmaxListNR-r15

MultiBandNsPmaxListNR-v1550	::=	SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF NS-PmaxListNR-r15

WhiteCellListNR-r16 ::=			SEQUENCE (SIZE (1..maxCellWhiteNR-r16)) OF PhysCellIdNR-r15

-- ASN1STOP

SystemInformationBlockType24 field descriptions

carrierFreqListNR List of carrier frequencies of NR carriers. These frequencies correspond to GSCN values as specified in TS 38.101 [85]. If the carrierFreqListNR-v1610 is present, it contains the same number of entries, listed in the same order as in the carrierFreqListNR (without suffix).

cellReselectionPriority The field concerns the absolute priority of the concerned carrier frequency as used by the cell reselection procedure. Corresponds with parameter "priority" in TS 36.304 [4].

deriveSSB-IndexFromCell The field indicates whether the UE may use, to derive the SSB index of a cell on the indicated SSB frequency and subcarrier spacing, the timing of any detected cell with the same SSB frequency and subcarrier spacing. If this field is set to TRUE, the UE assumes SFN and frame boundary alignment across cells on the same NR carrier frequency as specified in TS 36.133 [16].

highSpeedCarrierNR If the field is present, the UE shall apply the enhanced inter-RAT NR measurement requirements to support high speed up to 500 km/h as specified in TS 36.133 [16] to the NR carrier.

maxRS-IndexCellQual Number of SS blocks to average for cell measurement derivation. Corresponds to the parameter nrofSS-BlocksToAverage in TS 38.304 [92].

measTimingConfig Used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs. If the field is absent, the UE assumes that SSB periodicity is 5ms in this frequency.

multiBandInfoList Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE shall select the first listed band which it supports in the multiBandInfoList field to represent the NR neighbour carrier frequency. The network always includes this field.

multiBandInfoListSUL Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE shall select the first listed band which it supports in the multiBandInfoListSUL field to represent the NR neighbour carrier frequency.

multiBandNsPmaxListNR Indicates the NS-PmaxListNR configuration for the NR frequency band(s) listed in multiBandInfoList. The first entry corresponds to the second listed band in multiBandInfoList, and second entry corresponds to the third listed band in multiBandInfoList, and so on.

multiBandNsPmaxListNR-SUL Indicates the NS-PmaxListNR configuration for the NR SUL frequency band(s) listed in multiBandInfoListSUL. The first entry corresponds to the first listed band in multiBandInfoListSUL, and second entry corresponds to the second listed band in multiBandInfoListSUL, and so on.

ns-PmaxListNR Indicates a list of additionalPmax and additionalSpectrumEmission, corresponds to the first listed band in the multiBandInfoList.

p-MaxNR Indicates the maximum power for NR (see TS 38.104 [91]) the UE can use in NR SCG.

q-QualMin Parameter "Qqualmin" in TS 36.304 [4], applicable for NR neighbour cells. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin. The actual value Qqualmin = field value [dB].

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [92], applicable for NR neighbour cells. The actual value Qrxlevmin = field value * 2 [dBm].

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [92], applicable for NR neighbouring cells. The actual value Qrxlevmin = field value * 2 [dBm].

smtc2-LP Measurement timing configuration for inter-RAT neighbour cells in NR with a Long Periodicity (LP) indicated by periodicity in smtc2-LP. The timing offset and duration are equal to the offset and duration indicated in measTimingConfig in CarrierFreqNR. The periodicity in smtc2-LP can only be set to a value strictly larger than the periodicity in measTimingConfig in CarrierFreqNR (e.g. if measTimingConfig indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if measTimingConfig indicates sf160, smtc2-LP cannot be configured). The pci-List, if present, includes the physical cell identities of the inter-RAT neighbour cells with Long Periodicity. If smtc2-LP is absent, the UE assumes that there are no inter-RAT neighbour cells with a Long Periodicity.

ssb-PositionQCL-CommonNR Indicates the QCL relationship between SS/PBCH blocks for NR neighbor cells on the indicated frequency as specified in TS 38.213 [88], clause 4.1.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [89]). When the field is absent the UE measures on all SS-blocks.

ss-RSSI-Measurements Indicates the SSB-based RSSI measurement configuration. If the field is absent, the UE behaviour is defined in TS 38.215 [89], clause 5.1.3.

threshRS-Index List of thresholds for consolidation of L1 measurements per RS index. Corresponds to the parameter absThreshSS-BlocksConsolidation in TS 38.304 [92].

threshX-High Parameter "ThreshX, HighP" in TS 36.304 [4].

threshX-HighQ Parameter "ThreshX, HighQ" in TS 36.304 [4].

threshX-Low Parameter "ThreshX, LowP" in TS 36.304 [4].

threshX-LowQ Parameter "ThreshX, LowQ" in TS 36.304 [4].

t-ReselectionNR Parameter "TreselectionNR" in TS 36.304 [4].

t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 36.304 [4]. If the field is not present, the UE behaviour is specified in TS 36.304 [4].

whiteCellListNR List of whitelisted neighbouring NR cells.

| Conditional presence | Explanation |
| --- | --- |
| RSRQ | The field is mandatory present if the threshServingLowQ is present in systemInformationBlockType3; otherwise it is not present. |
| RSRQ2 | The field is optional Need OP if the threshServingLowQ is present in systemInformationBlockType3; otherwise it is not present. |
| SharedSpectrum | The field is optional Need OP if NR operates with shared spectrum channel access; otherwise, it is not present. |
| SharedSpectrum2 | The field is mandatory present if NR operates with shared spectrum channel access; otherwise, it is not present. |

#### –	SystemInformationBlockType25

The IE SystemInformationBlockType25 contains the UAC parameters.

SystemInformationBlockType25 information element

-- ASN1START

SystemInformationBlockType25-r15 ::=	SEQUENCE {

uac-BarringForCommon-r15				UAC-BarringPerCatList-r15				OPTIONAL,	-- Need OP

uac-BarringPerPLMN-List-r15			UAC-BarringPerPLMN-List-r15				OPTIONAL,	-- Need OP

uac-BarringInfoSetList-r15			UAC-BarringInfoSetList-r15,

uac-AC1-SelectAssistInfo-r15		CHOICE {

plmnCommon-r15							UAC-AC1-SelectAssistInfo-r15,

individualPLMNList-r15	SEQUENCE (SIZE (2..maxPLMN-r11)) OF UAC-AC1-SelectAssistInfo-r15

}			OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING								OPTIONAL,

...,

[[	ab-PerRSRP-r16					ENUMERATED {thresh0, thresh1, thresh2, thresh3}	OPTIONAL	-- Need OR

]],

[[

uac-AC1-SelectAssistInfo-r16 SEQUENCE (SIZE (2..maxPLMN-r11)) OF UAC-AC1-SelectAssistInfo-r16 OPTIONAL    -- Need OR

]]

}

UAC-BarringPerPLMN-List-r15::=	SEQUENCE (SIZE (1.. maxPLMN-r11)) OF UAC-BarringPerPLMN-r15

UAC-BarringPerPLMN-r15 ::=	SEQUENCE {

plmn-IdentityIndex-r15		INTEGER (1.. maxPLMN-r11),

uac-AC-BarringListType-r15		CHOICE{

uac-ImplicitAC-BarringList-r15		SEQUENCE (SIZE(maxAccessCat-1-r15)) OF UAC-BarringInfoSetIndex-r15,

uac-ExplicitAC-BarringList-r15		UAC-BarringPerCatList-r15

}					OPTIONAL	-- Need OR

}

UAC-BarringPerCatList-r15 ::= SEQUENCE (SIZE (1..maxAccessCat-1-r15)) OF UAC-BarringPerCat-r15

UAC-BarringPerCat-r15 ::= SEQUENCE {

accessCategory-r15					INTEGER (1..maxAccessCat-1-r15),

uac-barringInfoSetIndex-r15		UAC-BarringInfoSetIndex-r15

}

UAC-BarringInfoSetIndex-r15 ::=	INTEGER (1..maxBarringInfoSet-r15)

UAC-BarringInfoSetList-r15 ::=		SEQUENCE (SIZE (1..maxBarringInfoSet-r15)) OF UAC-BarringInfoSet-r15

UAC-BarringInfoSet-r15 ::= SEQUENCE {

uac-BarringFactor-r15		ENUMERATED {

p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

uac-BarringTime-r15			ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512},

uac-BarringForAccessIdentity-r15			BIT STRING (SIZE(7))

}

UAC-AC1-SelectAssistInfo-r15::=	ENUMERATED {a, b, c}

UAC-AC1-SelectAssistInfo-r16::= ENUMERATED {a, b, c, notConfigured}

-- ASN1STOP

SystemInformationBlockType25 field descriptions

accessCategory The Access Category according to TS 22.261 [96].

ab-PerRSRP Access barring per RSRP. Value thresh0 means access to the cell is barred when UE is in enhanced coverage as specified in TS 36.304 [4] and does not apply to UEs satisfying S criteria for normal coverage. Value thresh1 is compared to the first entry configured in rsrp-ThresholdsPrachInfoList, value thresh2 is compared to the second entry configured in rsrp-ThresholdsPrachInfoList and so on. E-UTRA/5GC includes this field only in the BR version of SystemInformationBlockType25.

uac-AC-BarringListType Access control parameters for each access category valid only for a specific PLMN. UE behaviour upon absence of this field is specified in clause 5.3.16.2.

uac-AC1-SelectAssistInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [96]. If plmnCommon is chosen, the UAC-AC1-SelectAssistInfo is applicable to all the PLMNs in cellAccessRelatedInfoList-5GC. If individualPLMNList is chosen, the 1st entry in the list corresponds to the first PLMN in cellAccessRelatedInfoList-5GC, the 2nd entry in the list corresponds to the second PLMN in cellAccessRelatedInfoList-5GC and so on. If uac-AC1-SelectAssistInfo-r16 is present, the UE shall ignore the uac-AC1-SelectAssistInfo-r15. Value notConfigured indicates that Access Category1 is not configured for the corresponding PLMN. The corresponding UAC-AC1-SelectAssistInfo for the selected PLMN is forwarded to upper layers, if present and set to a, b or c.

uac-BarringFactor Represents the probability that access attempt would be allowed during access barring check.

uac-BarringForAccessIdentity Indicates whether access attempt is allowed for each Access Identity. The leftmost bit, bit 0 in the bit string corresponds to Access Identity 1, bit 1 in the bit string corresponds to Access Identity 2, bit 2 in the bit string corresponds to Access Identity 11, bit 3 in the bit string corresponds to Access Identity 12 and so on. Value 0 means that access attempt is allowed for the corresponding access identity.

uac-BarringForCommon Common access control parameters for each access category. Common values are used for all PLMNs, unless overwritten by the PLMN specific configuration provided in uac-BarringPerPLMN-List. The parameters are specified by providing an index to the set of configurations (uac-BarringInfoSetList). UE behaviour upon absence of this field is specified in clause 5.3.16.2.

uac-barringInfoSetIndex Index of the entry in field uac-BarringInfoSetList. Value 1 corresponds to the first entry in uac-BarringInfoSetList, value 2 corresponds to the second entry in this list and so on. An index value referring to an entry not included in uac-BarringInfoSetList indicates no barring.

uac-BarringInfoSetList List of access control parameter sets. Each access category can be configured with access parameters corresponding to a particular set by uac-barringInfoSetIndex. Association of an access category with an index that has no corresponding entry in the uac-BarringInfoSetList is valid configuration and indicates no barring.

uac-BarringPerPLMN-List Access control parameters for each access category valid only for a specific PLMN.

uac-BarringTime The average time in seconds before a new access attempt is to be performed after an access attempt was barred at access barring check for the same access category, see 5.3.16.5.

#### –	SystemInformationBlockType26

The IE SystemInformationBlockType26 contains V2X sidelink communication configurations which can be used jointly with those included in SystemInformationBlockType21.

SystemInformationBlockType26 information element

-- ASN1START

SystemInformationBlockType26-r15 ::= SEQUENCE {

v2x-InterFreqInfoList-r15			SL-InterFreqInfoListV2X-r14			OPTIONAL,	-- Need OR

cbr-pssch-TxConfigList-r15			SL-CBR-PPPP-TxConfigList-r15		OPTIONAL,	-- Need OR

v2x-PacketDuplicationConfig-r15		SL-V2X-PacketDuplicationConfig-r15	OPTIONAL,	-- Need OR

syncFreqList-r15					SL-V2X-SyncFreqList-r15				OPTIONAL,	-- Need OR

slss-TxMultiFreq-r15				ENUMERATED{true}					OPTIONAL,	-- Need OR

v2x-FreqSelectionConfigList-r15		SL-V2X-FreqSelectionConfigList-r15	OPTIONAL,	-- Need OR

threshS-RSSI-CBR-r15				INTEGER (0..45)						OPTIONAL,	-- Need OR

...,

lateNonCriticalExtension			OCTET STRING						OPTIONAL

}

-- ASN1STOP

SystemInformationBlockType26 field descriptions

cbr-pssch-TxConfigList Indicates the mapping between PPPPs, CBR ranges by using indexes of the entry in cbr-RangeCommonConfigList included in SIB21, and PSSCH transmission parameters and CR limit by using indexes of the entry in sl-CBR-PSSCH-TxConfigList included in SIB21. The configurations in this field apply to all the resource pools on all the carrier frequencies included in SIB26 for V2X sidelink communication transmission. The mcs-PSSCH-RangeList-r15 included in this field also applies to all the resource pools on all the carrier frequencies included in SIB21 for V2X sidelink communication transmission.

slss-TxMultiFreq Value TRUE indicates the UE transmits SLSS on multiple carrier frequencies for V2X sidelink communication. If this field is absent, the UE transmits SLSS only on the synchronisation carrier frequency.

syncFreqList Indicates a list of candidate carrier frequencies that can be used for the synchronisation of V2X sidelink communication.

threshS-RSSI-CBR Indicates the S-RSSI threshold for determining the contribution of a sub-channel to the CBR measurement, as specified in TS 36.214 [48]. Value 0 corresponds to -112 dBm, value 1 to -110 dBm, value n to (-112 + n*2) dBm, and so on. If included, the threshS-RSSI-CBR in SL-CommResourcePoolV2X in SIB26 is absent.

v2x-FreqSelectionConfigList Indicates the configuration information for the carrier selection for V2X sidelink communication transmission on the carrier frequency where the field is broadcast.

v2x-PacketDuplicationConfig Indicates the configuration information for sidelink packet duplication for V2X sidelink communication.

v2x-InterFreqInfoList If this field includes a carrier frequency which is included in SIB21 and some configuration(s) for that carrier are already included in SIB21, the corresponding configuration(s) for that carrier frequency are not included in this field.

#### –	SystemInformationBlockType26a

The IE SystemInformationBlockType26a contains NR bands list which can be used for EN-DC operation with the serving cell.

SystemInformationBlockType26a information element

-- ASN1START

SystemInformationBlockType26a-r16 ::= SEQUENCE {

plmn-InfoList-r16						PLMN-InfoList-r16,

bandListENDC-r16						BandListENDC-r16,

lateNonCriticalExtension				OCTET STRING				OPTIONAL,

...

}

BandListENDC-r16 ::=		SEQUENCE (SIZE (1.. maxBandsENDC-r16)) OF FreqBandIndicatorNR-r15

PLMN-InfoList-r16 ::=		SEQUENCE (SIZE (0..maxPLMN-r11)) OF PLMN-Info-r16

PLMN-Info-r16 ::=			SEQUENCE {

nr-BandList-r16				BIT STRING (SIZE(maxBandsENDC-r16))	OPTIONAL		-- Need OR

}

-- ASN1STOP

SystemInformationBlockType26a field descriptions

bandListENDC A list of NR bands which can be configured as SCG in EN-DC operation with serving cell for the forwarding of upperLayerIndication to upper layers.

plmn-InfoList This field includes the same number of entries, and listed in the same order as PLMNs across the plmn-IdentityList fields plmn-IdentityList and plmn-IdentityList-r14 included in SIB1. I.e. the first entry corresponds to the first entry of the combined list that results from concatenating the entries included in the second to the original plmn-IdentityList field in SIB1. If the size of the field is set to 0, all bands in bandListENDC apply for all PLMNs listed in SIB1.

nr-BandList This field indicates a list of bands and is encoded as a bitmap, where the bit N is set to "1" if the current serving cell supports EN-DC operation with the N-th NR band in bandListENDC. The bits which have no corresponding bands in bandListENDC shall be set to 0; bit 1 of the bitmap is the leading bit of the bit string.

#### –	SystemInformationBlockType27

The IE SystemInformationBlockType27 contains information relevant only for inter-RAT cell selection i.e. assistance information about NB-IoT frequencies for cell selection.

SystemInformationBlockType27 information element

-- ASN1START

SystemInformationBlockType27-r16 ::=	SEQUENCE {

carrierFreqListNBIOT-r16				CarrierFreqListNBIOT-r16		OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...

}

CarrierFreqListNBIOT-r16 ::=				SEQUENCE (SIZE (1.. maxFreqNBIOT-r16)) OF	CarrierFreqNBIOT-r16

CarrierFreqNBIOT-r16 ::=		SEQUENCE {

carrierFreq-r16					ARFCN-ValueEUTRA-r9,

carrierFreqOffset-r16			ENUMERATED {v-10, v-9, v-8dot5, v-8, v-7, v-6, v-5, v-4dot5,

v-4,v-3, v-2, v-1, v-0dot5,	v0, v1, v2, v3, v3dot5,

v4, v5, v6, v7, v7dot5, v8, v9}

}

-- ASN1STOP

SystemInformationBlockType27 field descriptions

carrierFreqListNBIOT Provides a list of neighbouring NB-IoT carrier frequencies, which may be searched for neighbouring NB-IoT cells.

carrierFreq Provides the ARFCN applicable for the NB-IoT carrier frequency as defined in TS 36.101 [42], Table 5.7.3-1.

carrierFreqOffset Offset of the NB-IoT channel number to EARFCN as defined in TS 36.101 [42], clause 5.7.3F. Value v-10 means -10, v-9 means -9, and so on. The values v-8dot5, v-4dot5, v3dot5 and v7dot5 are only applicable for a carrier in a TDD band.

#### –	SystemInformationBlockType28

The IE SystemInformationBlockType28 contains NR sidelink communication configuration.

SystemInformationBlockType28 information element

-- ASN1START

SystemInformationBlockType28-r16 ::= SEQUENCE {

segmentNumber-r16					INTEGER (0..63),

segmentType-r16						ENUMERATED {notLastSegment,lastSegment},

segmentContainer-r16				OCTET STRING,

lateNonCriticalExtension			OCTET STRING				OPTIONAL,

...

}

-- ASN1STOP

SystemInformationBlockType28 field descriptions

segmentContainer Container for the configuration for NR sidelink communication, this field includes a segment of SIB12-IEs as specified in TS 38.331 [82]. The size of the included segment in this container should be small enough that the SIB message size is less than or equal to the maximum size of a LTE SI i.e. 2216 bits.

segmentNumber This field identifies the sequence number of a segment of SIB12-IEs IE as specified in TS 38.331 [82]. A segment number of zero corresponds to the first segment, a segment number of one corresponds to the second segment, and so on.

segmentType This field indicates whether the included segment is the last segment or not.

#### –	SystemInformationBlockType29

The IE SystemInformationBlockType29 contains common resource reservation, e.g. for coexistence with NR.

SystemInformationBlockType29 information element

-- ASN1START

SystemInformationBlockType29-r16 ::= SEQUENCE {

resourceReservationConfigCommonDL-r16	ResourceReservationConfigDL-r16	OPTIONAL,	-- Need OR

resourceReservationConfigCommonUL-r16	ResourceReservationConfigUL-r16	OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...

}

-- ASN1STOP

#### –	SystemInformationBlockTypeXX

The IE SystemInformationBlockTypeXX contains satellite assistance information.

SystemInformationBlockTypeXX information element

-- ASN1START

SystemInformationBlockTypeXX-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...

}

ServingSatelliteInfo-r17 ::= 	SEQUENCE {

ephemerisStateVectors-r17		EphemerisStateVectors-r17		OPTIONAL,	-- Need OR

ephemerisOrbitalParameters-r17	EphemerisOrbitalParameters-r17	OPTIONAL,	-- Need OR

nta-CommonParameters-17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827) 		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29470)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512) 				OPTIONAL,	-- Need OP

t-Service-r17              		TimeUTC-r17						OPTIONAL,	-- Need OR

...

}

EphemerisStateVectors-r17 ::= 	SEQUENCE {

positionX-r17					PositionStateVector-r17,

positionY-r17					PositionStateVector-r17,

positionZ-r17					PositionStateVector-r17,

velocityVX-r17					VelocityStateVector-r17,

velocityVY-r17					VelocityStateVector-r17,

velocityVZ-r17					VelocityStateVector-r17

}

EphemerisOrbitalParameters-r17 ::= SEQUENCE {

semiMajorAxis-r17					INTEGER (0..8589934591),

eccentricity-r17					INTEGER (0..524287),

periapsis-r17						INTEGER (0..16777215),

longitude-r17						INTEGER (0..2097151),

inclination-r17						INTEGER (-524288..524287),

anomaly-r17							INTEGER (0..16777215)

}

PositionStateVector-r17 ::= INTEGER (-3355432..33554431)

VelocityStateVector-r17 ::= INTEGER (-131072..131071)

-- ASN1STOP

SystemInformationBlockTypeXX field descriptions

anomaly Satellite orbital parameter: Mean anomaly M at epoch time, see NIMA TR 8350.2 [X]. Unit in radian. Value range 0...2π by step of 2π * 2-24. Actual value = IE value * ( 2 * π * 2-24).

eccentricity Satellite orbital parameter: eccentricity e, see NIMA TR 8350.2 [X]. Value range 0...0.015 by step of 0.015 * 2-19. Actual value = IE value * (0.015 * 2-19).

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. If the field is absent, the UE uses the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIBXX is transmitted.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in number of slots for a given subcarrier spacing. If the field if absent, the UE uses the (default) value of 0.

k-Offset Scheduling offset used in the timing relationships in NTN, see TS 36.213 [23]. Unit in ms.

inclination Satellite orbital parameter: inclination i, see NIMA TR 8350.2 [X]. Unit in radian. Value range -π/2…π/2 by step of π/2 * 2-19. Actual value = IE value * ((π/2) * 2-19).

longitude Satellite orbital parameter: longitude of ascending node , see NIMA TR 8350.2 [X]. Unit in radian. Value range 0...2π by step of 2π * 2-21. Actual value = IE value * (2 * π * 2-21).

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Value range 0..270.73 ms by step of 32.55208 ×10-3 μs. Actual value = IE value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Value range of -53.33…53.33 by step of 0.2 ×10-3. Actual value = IE value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Value range 0…0.60 by step of 0.2 ×10-4. Actual value = IE value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

periapsis Satellite orbital parameter: argument of periapsis , see NIMA TR 8350.2 [X]. Unit in radian. Value range 0...2π by step of 2π * 2-24. Actual value = IE value * (2 * π * 2-24).

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit in meter. Value range -42200000…42200000 by step of 1.3. Actual value = IE value * 1.3.

semiMajorAxis Satellite orbital parameter: semi major axis , see NIMA TR 8350.2 [X]. Unit in meter. Value range 6500000….43000000 by step of 2-33. Actual value = (43000000 – 6500000) + IE value * 2-33.

t-Service Time information on when a NTN quasi-Earth fixed cell is going to stop serving the area it is currently covering.

ul-SyncValidationDuration Validity duration of the satellite ephemeris data and common TA parameters, i.e. maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris, see TS 36.213 [23]. Unit in s. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on.

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit in meter/second. Value range -8000…8000 by step of 0.06. Actual value = IE value * 0.06.

Editor’s Note: FFS whether to refer to RAN1 specs if details on these IEs are explained in RAN1 specs. Otherwise, need to ask RAN1 for better description of these parameters.

### 6.3.2	Radio resource control information elements

#### –	Alpha

The IE Alpha is used to indicate parameter α, see TS 36.213 [23], clause 5.1.1.1 and 5.1.3.1. Value al0 corresponds to 0, al04 corresponds to value 0.4, al05 to 0.5, al06 to 0.6, al07 to 0.7, al08 to 0.8, al09 to 0.9 and al1 corresponds to 1.

Alpha information element

-- ASN1START

Alpha-r12 ::= ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}

-- ASN1STOP

#### –	AntennaInfo

The IE AntennaInfoCommon and the AntennaInfoDedicated are used to specify the common and the UE specific antenna configuration respectively.

AntennaInfo information elements

-- ASN1START

AntennaInfoCommon ::=				SEQUENCE {

antennaPortsCount					ENUMERATED {an1, an2, an4, spare1}

}

AntennaInfoDedicated ::=			SEQUENCE {

transmissionMode					ENUMERATED {

tm1, tm2, tm3, tm4, tm5, tm6,

tm7, tm8-v920},

codebookSubsetRestriction			CHOICE {

n2TxAntenna-tm3						BIT STRING (SIZE (2)),

n4TxAntenna-tm3						BIT STRING (SIZE (4)),

n2TxAntenna-tm4						BIT STRING (SIZE (6)),

n4TxAntenna-tm4						BIT STRING (SIZE (64)),

n2TxAntenna-tm5						BIT STRING (SIZE (4)),

n4TxAntenna-tm5						BIT STRING (SIZE (16)),

n2TxAntenna-tm6						BIT STRING (SIZE (4)),

n4TxAntenna-tm6						BIT STRING (SIZE (16))

}		OPTIONAL,															-- Cond TM

ue-TransmitAntennaSelection			CHOICE{

release							NULL,

setup							ENUMERATED {closedLoop, openLoop}

}

}

AntennaInfoDedicated-v920 ::=		SEQUENCE {

codebookSubsetRestriction-v920		CHOICE {

n2TxAntenna-tm8-r9					BIT STRING (SIZE (6)),

n4TxAntenna-tm8-r9					BIT STRING (SIZE (32))

}		OPTIONAL															-- Cond TM8

}

AntennaInfoDedicated-r10 ::=		SEQUENCE {

transmissionMode-r10				ENUMERATED {

tm1, tm2, tm3, tm4, tm5, tm6, tm7, tm8-v920,

tm9-v1020, tm10-v1130, spare6, spare5, spare4,

spare3, spare2, spare1},

codebookSubsetRestriction-r10		BIT STRING			OPTIONAL,			-- Cond TMX

ue-TransmitAntennaSelection		CHOICE{

release							NULL,

setup							ENUMERATED {closedLoop, openLoop}

}

}

AntennaInfoDedicated-v10i0::=	SEQUENCE {

maxLayersMIMO-r10			ENUMERATED {twoLayers, fourLayers, eightLayers}		OPTIONAL	-- Need OR

}

AntennaInfoDedicated-v1250 ::=		SEQUENCE {

alternativeCodebookEnabledFor4TX-r12	BOOLEAN

}

AntennaInfoDedicated-v1430 ::=		SEQUENCE {

ce-UE-TxAntennaSelection-config-r14			ENUMERATED {on}		OPTIONAL	-- Need OR

}

AntennaInfoDedicatedSTTI-r15 ::=	CHOICE {

release								NULL,

setup								SEQUENCE {

transmissionModeDL-MBSFN-r15		ENUMERATED {tm9, tm10}		OPTIONAL,	-- Need OR

transmissionModeDL-nonMBSFN-r15	ENUMERATED {tm1, tm2, tm3, tm4, tm6, tm8, tm9,

tm10}		OPTIONAL,	--_Need OR

codebookSubsetRestriction			CHOICE {

n2TxAntenna-tm3-r15					BIT STRING (SIZE (2)),

n4TxAntenna-tm3-r15					BIT STRING (SIZE (4)),

n2TxAntenna-tm4-r15					BIT STRING (SIZE (6)),

n4TxAntenna-tm4-r15					BIT STRING (SIZE (64)),

n2TxAntenna-tm5-r15					BIT STRING (SIZE (4)),

n4TxAntenna-tm5-r15					BIT STRING (SIZE (16)),

n2TxAntenna-tm6-r15					BIT STRING (SIZE (4)),

n4TxAntenna-tm6-r15					BIT STRING (SIZE (16)),

n2TxAntenna-tm8-r15					BIT STRING (SIZE (6)),

n4TxAntenna-tm8-r15					BIT STRING (SIZE (64)),

n2TxAntenna-tm9and10-r15			BIT STRING (SIZE (6)),

n4TxAntenna-tm9and10-r15			BIT STRING (SIZE (96)),

n8TxAntenna-tm9and10-r15			BIT STRING (SIZE (109))

}		OPTIONAL,															-- Cond TM

maxLayersMIMO-STTI-r15			ENUMERATED {twoLayers, fourLayers}	OPTIONAL, -- Need OR

slotSubslotPDSCH-TxDiv-2Layer-r15	BOOLEAN,

slotSubslotPDSCH-TxDiv-4Layer-r15	BOOLEAN

}

}

AntennaInfoDedicated-v1530 ::=		CHOICE {

release								NULL,

setup								CHOICE {

ue-TxAntennaSelection-SRS-1T4R-Config-r15			NULL,

ue-TxAntennaSelection-SRS-2T4R-NrOfPairs-r15			ENUMERATED {two, three}

}

}

-- ASN1STOP

AntennaInfo field descriptions

alternativeCodebookEnabledFor4TX Indicates whether code book in TS 36.213 [23] Table 7.2.4-0A to Table 7.2.4-0D is being used for deriving CSI feedback and reporting. E-UTRAN only configures the field if the UE is configured with a) tm8 with 4 CRS ports, tm9 or tm10 with 4 CSI-RS ports and b) PMI/RI reporting.

antennaPortsCount Parameter represents the number of cell specific antenna ports where an1 corresponds to 1, an2 to 2 antenna ports etc. see TS 36.211 [21], clause 6.2.1.

ce-ue-TxAntennaSelection-config Configuration of UL closed-loop transmit antenna selection for non-BL UE in CE Mode A, see TS 36.212 [22].

codebookSubsetRestriction Parameter: codebookSubsetRestriction, see TS 36.213 [23], clause 7.2 and TS 36.211 [21], clause 6.3.4.2.3. The number of bits in the codebookSubsetRestriction for applicable transmission modes is defined in TS 36.213 [23], Table 7.2-1b. If the UE is configured with transmissionMode tm8, E-UTRAN configures the field codebookSubsetRestriction if PMI/RI reporting is configured. If the UE is configured with transmissionMode tm9, E-UTRAN configures the field codebookSubsetRestriction if PMI/RI reporting is configured and if the number of CSI-RS ports is greater than 1. E-UTRAN does not configure the field codebookSubsetRestriction in other cases where the UE is configured with transmissionMode tm8 or tm9. Furthermore, E-UTRAN does not configure the field codebookSubsetRestriction if the UE is configured with eMIMO-Type unless it is set to beamformed, alternativeCodebookEnabledBeamformed is set to FALSE and csi-RS-ConfigNZPIdListExt is not configured.

maxLayersMIMO Indicates the maximum number of layers for spatial multiplexing used to determine the rank indication bit width and Kc determination of the soft buffer size for the corresponding serving cell according to TS 36.212 [22]. EUTRAN configures this field only when transmissionMode is set to tm3, tm4, tm9 or tm10 for the corresponding serving cell. When configuring the field for a serving cell which transmissionMode is set to tm3 or tm4, EUTRAN only configures value fourLayers: For a serving cell which transmissionMode is set to tm9 or tm10, EUTRAN only configures the field only if intraBandContiguousCC-InfoList or FeatureSetDL-PerCC is indicated for the band and the band combination of the corresponding serving cell or the UE supports maxLayersMIMO-Indication.

maxLayersMIMO-STTI Indicates the maximum number of layers, for each serving cell, to be used when determining if the shifted DMRS pattern is applicable TS 36.211 [21], clause 6.10.3.2.

slotSubslotPDSCH-TxDiv-2Layer, slotSubslotPDSCH-TxDiv-4Layer Indicates the table to be used in case of dynamic TX diversity fallback for TM9 and 10 for up to 2-layer/4-layer slot or subslot PDSCH operation, see TS 36.212 [22], clause 5.3.3.1.22.

transmissionMode Points to one of Transmission modes defined in TS 36.213 [23], clause 7.1, where tm1 refers to transmission mode 1, tm2 to transmission mode 2 etc.

transmissionModeDL-MBSFN Indicates, for MBSFN, the transmission mode as defined in TS 36.213 [23], clause 7.1, where tm1 refers to transmission mode 1, tm2 to transmission mode 2 etc for slot or subslot operation. In case of FDD, TM8 is not applicable.

transmissionModeDL-nonMBSFN Indicates, for non-MBSFN, the transmission mode as defined in TS 36.213 [23], clause 7.1, where tm1 refers to transmission mode 1, tm2 to transmission mode 2 etc. for slot or subslot operation. In case of FDD, TM8 is not applicable.

ue-TransmitAntennaSelection For value setup, the field indicates whether UE transmit antenna selection control is closed-loop or open-loop as described in TS 36.213 [23], clause 8.7.

ue-TxAntennaSelection-SRS-1T4R-Config Configuration of UL closed-loop transmit antenna selection for UE to select one antenna among four antennas to transmit SRS for the corresponding serving cell as described in TS 36.213 [23]. When ue-TxAntennaSelection-SRS-1T4R-Config and ue-TransmitAntennaSelection are configured simultaneously for a given serving cell, the UE selects one of the first two antennas for PUSCH transmission and selects one antenna among four antennas at each SRS instance for SRS transmission for the corresponding serving cell as described in TS 36.213 [23].

ue-TxAntennaSelection-SRS-2T4R-NrOfPairs Presence of the field indicates configuration of UL closed-loop transmit antenna selection for UE to select two antennas among four antennas to transmit SRS simultaneously for the corresponding serving cell as described in TS 36.213 [23]. Further, the field indicates the number of antenna pairs to select from for SRS transmission for a given serving cell as described in TS 36.213 [23]. Value two indicates the UE to select one antenna pair between two antenna pairs to transmit SRS simultaneously at each SRS instance for the corresponding serving cell. Value three indicates the UE to select one antenna pair among three antenna pairs to transmit SRS simultaneously at each SRS instance for the corresponding serving cell. EUTRAN does not simultaneously configure ue-TransmitAntennaSelection and ue-TxAntennaSelection-SRS-2T4R-NrOfPairs for a given serving cell.

| Conditional presence | Explanation |
| --- | --- |
| TM | The field is mandatory present if the transmissionMode is set to tm3, tm4, tm5 or tm6. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| TM8 | The field is optional present, need OR, if AntennaInfoDedicated is included and transmissionMode is set to tm8. If AntennaInfoDedicated is included and transmissionMode is set to a value other than tm8, the field is not present and the UE shall delete any existing value for this field. Otherwise the field is not present. |
| TMX | The field is mandatory present if the transmissionMode-r10 is set to tm3, tm4, tm5 or tm6. The field is optionally present, need OR, if the transmissionMode-r10 is set to tm8 or tm9. Otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	AntennaInfoUL

The IE AntennaInfoUL is used to specify the UL antenna configuration.

AntennaInfoUL information elements

-- ASN1START

AntennaInfoUL-r10 ::=		SEQUENCE {

transmissionModeUL-r10				ENUMERATED {tm1, tm2, spare6, spare5,

spare4, spare3, spare2, spare1}	OPTIONAL,	-- Need OR

fourAntennaPortActivated-r10			ENUMERATED {setup}			OPTIONAL		-- Need OR

}

AntennaInfoUL-STTI-r15 ::=	SEQUENCE {

transmissionModeUL-STTI-r15			ENUMERATED {tm1, tm2}		OPTIONAL	-- Need OR

}

-- ASN1STOP

AntennaInfoUL field descriptions

fourAntennaPortActivated Parameter indicates if four antenna ports are used. See TS 36.213 [23], clause 8.2. E-UTRAN optionally configures fourAntennaPortActivated only if transmissionModeUL is set to tm2.

transmissionModeUL Points to one of UL Transmission modes defined in TS 36.213 [23], clause 8.0, where tm1 refers to transmission mode 1, tm2 to transmission mode 2 etc.

transmissionModeUL-STTI Indicates the UL transmission mode as defined in TS 36.213 [23], clause 8.0, where tm1 refers to transmission mode 1 and tm2 to transmission mode 2 for slot or subslot operation.

#### –	AUL-Config

The IE AUL-Config is used to specify the autonomous uplink configuration.

AUL-Config information element

-- ASN1START

AUL-Config-r15 ::=	CHOICE {

release							NULL,

setup							SEQUENCE {

aul-CRNTI-r15							C-RNTI,

aul-Subframes-r15						BIT STRING (SIZE (40)),

aul-HARQ-Processes-r15					INTEGER (1..16),

transmissionModeUL-AUL-r15				ENUMERATED {tm1,tm2},

aul-StartingFullBW-InsideMCOT-r15		BIT STRING (SIZE (5)),

aul-StartingFullBW-OutsideMCOT-r15		BIT STRING (SIZE (7)),

aul-StartingPartialBW-InsideMCOT-r15	ENUMERATED {o34, o43, o52, o61, oOS1},

aul-StartingPartialBW-OutsideMCOT-r15	ENUMERATED {o16, o25, o34, o43, o52, o61, oOS1},

aul-RetransmissionTimer-r15				ENUMERATED {psf4, psf5, psf6, psf8, psf10, psf12, psf20, psf28, psf37, psf44, psf68, psf84, psf100,

psf116, psf132, psf164, psf324},

endingSymbolAUL-r15						INTEGER(12..13),

subframeOffsetCOT-Sharing-r15			INTEGER(2..4),

contentionWindowSizeTimer-r15			ENUMERATED {n0, n5, n10}

}

}

-- ASN1STOP

AUL-Config field descriptions

aul-CRNTI AUL C-RNTI, see TS 36.321 [6].

aul-HARQ-Processes This field indicates which HARQ process IDs are configured for AUL operation as described in TS 36.321 [6]. In case tm1 is configured for the transmissionModeUL-AUL the number of configured HARQ processes equals to field value. In case tm2 is configured for the transmissionModeUL-AUL the number of configured HARQ processes equals to double of the field value. The largest value of the HARQ process ID is equal to the number of configured HARQ processes - 1.

aul-RetransmissionTimer This timer is used to restrict both new transmission and retransmission for the same HARQ process for AUL operation as described in TS 36.321 [6]. Value psf4 corresponds to 4 PDCCH subframes etc.

aul-StartingFullBW-InsideMCOT This field indicates the AUL-specific set of PUSCH starting offset values for the AUL transmission inside of eNB obtained MCOT when a UE configured with AUL configuration is allocated to occupy the full channel bandwidth as described in TS 36.213 [23], clause 8.0. The first/leftmost bit corresponds to value 34, second bit corresponds to value 43, third bit corresponds to value 52, fourth bit corresponds to value 61 and last bit corresponds to value OS#1.

aul-StartingFullBW-OutsideMCOT This field indicates the AUL-specific set of PUSCH starting offset values for the AUL transmission outside of eNB obtained MCOT when a UE configured with AUL configuration is allocated to occupy the full channel bandwidth as described in TS 36.213 [23], clause 8.0. The first/leftmost bit corresponds to value 16, second bit corresponds to value 25, third bit corresponds to value 34, fourth bit corresponds to value 43, fifth bit corresponds to value 52, sixth bit corresponds to value 61 and last bit corresponds to value OS#1.

aul-StartingPartialBW-InsideMCOT This field indicates the exact AUL-specific PUSCH starting offset value for the AUL transmission inside of eNB obtained MCOT when a UE configured with AUL configuration is allocated to occupy partial channel bandwidth as described in TS 36.213 [23], clause 8.0. The value o34 corresponds to 34, and the value o43 corresponds to 43 and so on.

aul-StartingPartialBW-OutsideMCOT This field indicates the exact AUL-specific PUSCH starting offset value for the AUL transmission outside of eNB obtained MCOT when a UE configured with AUL configuration is allocated to occupy partial channel bandwidth as described in TS 36.213 [23], clause 8.0. The value o16 corresponds to 16, the value o25 corresponds to 25 and so on.

aul-Subframes This field indicates which subframes are allowed for AUL operation as described in TS 36.321 [6]. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod 4 = 0. Value 0 in the bitmap indicates that the corresponding subframe is not allowed for AUL. Value 1 in the bitmap indicates that the corresponding subframe is allowed for AUL.

contentionWindowSizeTimer This field indicates contention window size adjustment timer as described in TS 37.213 [94], clause 4.2.2. The value n0 corresponds to 0ms, value n5 corresponds to 5ms, value n10 corresponds to 10ms. The value is set to n0 or n5 if the absence of other technologies on the same carrier cannot be guaranteed. The value is set to n0 or n10 if the absence of other technologies on the same carrier can be guaranteed.

endingSymbolAUL This field indicates PUSCH ending symbol of the last AUL subframe in an AUL burst as described in TS 36.211 [21], clause 4.1.3.

subframeOffsetCOT-Sharing This field is COT sharing indication parameter X indicating if subframe n+X is an applicable subframe for UL to DL sharing as described in TS 37.213 [94], clause 4.1.3.

transmissionModeUL-AUL This field indicates which UL transmission mode is used for AUL as described in TS 36.213 [23], clause 8.0, where tm1 refers to transmission mode 1, tm2 to transmission mode 2.

#### –	CQI-ReportAperiodic

The IE CQI-ReportAperiodic is used to specify the aperiodic CQI reporting configuration.

CQI-ReportAperiodic information elements

-- ASN1START

CQI-ReportAperiodic-r10	::=		CHOICE {

release								NULL,

setup								SEQUENCE {

cqi-ReportModeAperiodic-r10			CQI-ReportModeAperiodic,

aperiodicCSI-Trigger-r10			SEQUENCE {

trigger1-r10					BIT STRING (SIZE (8)),

trigger2-r10					BIT STRING (SIZE (8))

}																	OPTIONAL	-- Need OR

}

}

CQI-ReportAperiodic-v1250	::=		CHOICE {

release								NULL,

setup								SEQUENCE {

aperiodicCSI-Trigger-v1250			SEQUENCE {

trigger-SubframeSetIndicator-r12	ENUMERATED {s1, s2},

trigger1-SubframeSetIndicator-r12	BIT STRING (SIZE (8)),

trigger2-SubframeSetIndicator-r12	BIT STRING (SIZE (8))

}

}

}

CQI-ReportAperiodic-v1310	::=		CHOICE {

release								NULL,

setup								SEQUENCE {

aperiodicCSI-Trigger-v1310			SEQUENCE {

trigger1-r13					BIT STRING (SIZE (32)),

trigger2-r13					BIT STRING (SIZE (32)),

trigger3-r13					BIT STRING (SIZE (32)),

trigger4-r13					BIT STRING (SIZE (32)),

trigger5-r13					BIT STRING (SIZE (32)),

trigger6-r13					BIT STRING (SIZE (32))

}																	OPTIONAL,	-- Need ON

aperiodicCSI-Trigger2-r13		CHOICE {

release							NULL,

setup						SEQUENCE {

trigger1-SubframeSetIndicator-r13	BIT STRING (SIZE (32)),

trigger2-SubframeSetIndicator-r13	BIT STRING (SIZE (32)),

trigger3-SubframeSetIndicator-r13	BIT STRING (SIZE (32)),

trigger4-SubframeSetIndicator-r13	BIT STRING (SIZE (32)),

trigger5-SubframeSetIndicator-r13	BIT STRING (SIZE (32)),

trigger6-SubframeSetIndicator-r13	BIT STRING (SIZE (32))

}

}																	OPTIONAL	-- Need ON

}

}

CQI-ReportAperiodicProc-r11	::=		SEQUENCE {

cqi-ReportModeAperiodic-r11			CQI-ReportModeAperiodic,

trigger01-r11						BOOLEAN,

trigger10-r11						BOOLEAN,

trigger11-r11						BOOLEAN

}

CQI-ReportAperiodicProc-v1310	::=		SEQUENCE {

trigger001-r13						BOOLEAN,

trigger010-r13						BOOLEAN,

trigger011-r13						BOOLEAN,

trigger100-r13						BOOLEAN,

trigger101-r13						BOOLEAN,

trigger110-r13						BOOLEAN,

trigger111-r13						BOOLEAN

}

CQI-ReportAperiodicHybrid-r14	::=		SEQUENCE {

triggers-r14						CHOICE {

oneBit-r14							SEQUENCE {

trigger1-Indicator-r14				BIT STRING (SIZE (8))

},

twoBit-r14							SEQUENCE {

trigger01-Indicator-r14				BIT STRING (SIZE (8)),

trigger10-Indicator-r14				BIT STRING (SIZE (8)),

trigger11-Indicator-r14				BIT STRING (SIZE (8))

},

threeBit-r14						SEQUENCE {

trigger001-Indicator-r14			BIT STRING (SIZE (32)),

trigger010-Indicator-r14			BIT STRING (SIZE (32)),

trigger011-Indicator-r14			BIT STRING (SIZE (32)),

trigger100-Indicator-r14			BIT STRING (SIZE (32)) ,

trigger101-Indicator-r14			BIT STRING (SIZE (32)),

trigger110-Indicator-r14			BIT STRING (SIZE (32)),

trigger111-Indicator-r14			BIT STRING (SIZE (32))

}

}																OPTIONAL	-- Need OR

}

CQI-ReportModeAperiodic ::=				ENUMERATED {

rm12, rm20, rm22, rm30, rm31,

rm32-v1250, rm10-v1310, rm11-v1310

}

-- ASN1STOP

CQI-ReportAperiodic field descriptions

aperiodicCSI-Trigger Indicates for which serving cell(s) the aperiodic CSI report is triggered when one or more SCells are configured. trigger1-r10 corresponds to the CSI request field 10 while trigger1-r13 corresponds to the CSI request field 010, trigger2-r10 corresponds to the CSI request field 11 while trigger2-r13 corresponds to the CSI request field 011, trigger3-r13 corresponds to the CSI request field 100, see TS 36.213 [23], table 7.2.1-1A and table 7.2.1-1D, and so on. The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex=1 etc. Each bit has either value 0 (means no aperiodic CSI report is triggered) or value 1 (means the aperiodic CSI report is triggered). At most 5 bits can be set to value 1 in the bit string in aperidociCSI-Trigger-r10 and in aperiodicCSI-Trigger-v1250 and at most 32 bits can be set to value 1 in the bit string in aperiodicCSI-Trigger-v1310. E-UTRAN configures value 1 only for cells configured with transmissionMode set in range tm1 to tm9. One value applies for all serving cells configured with transmissionMode set in range tm1 to tm9 and belonging to the same PUCCH group (the associated functionality is common i.e. not performed independently for each cell).

trigger-SubframeSetIndicator For a serving cell configured with csi-MeasSubframeSets-r12, indicates for which CSI subframe set the aperiodic CSI report is triggered for the serving cell if the aperiodic CSI is triggered by the CSI request field 01 or 001, see TS 36.213 [23], table 7.2.1-1C or table 7.2.1.-1E. Value s1 corresponds to CSI subframe set 1 and value s2 corresponds to CSI subframe set 2.

trigger001 Indicates whether or not reporting for this CSI-process or reporting for this CSI-process corresponding to a CSI subframe set is triggered by CSI request field set to 001, for a CSI request applicable for the serving cell on the same frequency as the CSI process, see TS 36.213 [23], table 7.2.1-1D and 7.2.1-E.

trigger001-IndicatorN.. trigger111-IndicatorN Indicates for which eMIMO-Type the aperiodic CSI report is triggered (the corresponding CSI process, CSI subframe set}-pair(s) and/or a serving cell) as applicable, See TS 36.213 [23], table 7.2.1-1A, 7.2.1-1B, and 7.2.1-1C.

trigger01 Indicates whether or not reporting for this CSI-process or reporting for this CSI-process corresponding to a CSI subframe set is triggered by CSI request field set to 01, for a CSI request applicable for the serving cell on the same frequency as the CSI process, see TS 36.213 [23], table 7.2.1-1D and 7.2.1-1E.

trigger010, trigger011, trigger100, trigger101, Trigger110, Trigger111 Indicates whether or not reporting for this CSI-process or reporting for this CSI-process corresponding to a CSI subframe set is triggered by CSI request field set to 010, 011, 100, 101, 110 or 111, see TS 36.213 [23], table 7.2.1-1D and 7.2.1-1E.

trigger10, trigger11 Indicates whether or not reporting for this CSI-process or reporting for this CSI-process corresponding to a CSI subframe set is triggered by CSI request field set to 10 or 11, see TS 36.213 [23], table 7.2.1-1B. EUTRAN configures at most 5 CSI processes, across all serving frequencies within each CG, to be triggered by a CSI request field set to value 10. The same restriction applies for value 11. In case E-UTRAN simultaneously triggers CSI requests for more than 5 CSI processes some limitations apply, see TS 36.213 [23].

trigger1-SubframeSetIndicator If signalled in the aperiodicCSI-Trigger-v1250, indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field 10, see TS 36.213 [23], table 7.2.1-1C, or by the CSI request field 010, see TS 36.213 [23], table 7.2.1-1E.The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex=1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

trigger2-SubframeSetIndicator If signalled in the aperiodicCSI-Trigger-v1250, indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field 11, see TS 36.213 [23], table 7.2.1-1C, or by the CSI request field 011, see TS 36.213 [23], table 7.2.1-1E.The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex=1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

trigger3-SubframeSetIndicator Indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field100, see TS 36.213 [23], table 7.2.1-1E.The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex =1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

trigger4-SubframeSetIndicator Indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field 101, see TS 36.213 [23], table 7.2.1-1E.The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex =1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

trigger5-SubframeSetIndicator Indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field 110, see TS 36.213 [23], table 7.2.1-1E.The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex =1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

trigger6-SubframeSetIndicator Indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field 111, see TS 36.213 [23], table 7.2.1-1E.The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex =1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

#### –	CQI-ReportBoth

The IE CQI-ReportBoth is used to specify the CQI reporting configuration common to both periodic and aperiodic configurations.

CQI-ReportBoth information elements

-- ASN1START

CQI-ReportBoth-r11 ::=			SEQUENCE {

csi-IM-ConfigToReleaseList-r11		CSI-IM-ConfigToReleaseList-r11	OPTIONAL,	-- Need ON

csi-IM-ConfigToAddModList-r11		CSI-IM-ConfigToAddModList-r11	OPTIONAL,	-- Need ON

csi-ProcessToReleaseList-r11		CSI-ProcessToReleaseList-r11	OPTIONAL,	-- Need ON

csi-ProcessToAddModList-r11			CSI-ProcessToAddModList-r11		OPTIONAL	-- Need ON

}

CQI-ReportBoth-v1250 ::=			SEQUENCE {

csi-IM-ConfigToReleaseListExt-r12		CSI-IM-ConfigId-v1250	OPTIONAL,	-- Need ON

csi-IM-ConfigToAddModListExt-r12		CSI-IM-ConfigExt-r12	OPTIONAL	-- Need ON

}

CQI-ReportBoth-v1310 ::=			SEQUENCE {

csi-IM-ConfigToReleaseListExt-r13	CSI-IM-ConfigToReleaseListExt-r13	OPTIONAL,	-- Need ON

csi-IM-ConfigToAddModListExt-r13	CSI-IM-ConfigToAddModListExt-r13	OPTIONAL	-- Need ON

}

CSI-IM-ConfigToAddModList-r11 ::=		SEQUENCE (SIZE (1..maxCSI-IM-r11)) OF CSI-IM-Config-r11

CSI-IM-ConfigToAddModListExt-r13 ::=	SEQUENCE (SIZE (1..maxCSI-IM-v1310)) OF CSI-IM-ConfigExt-r12

CSI-IM-ConfigToReleaseList-r11 ::=		SEQUENCE (SIZE (1..maxCSI-IM-r11)) OF CSI-IM-ConfigId-r11

CSI-IM-ConfigToReleaseListExt-r13 ::=	SEQUENCE (SIZE (1..maxCSI-IM-v1310)) OF CSI-IM-ConfigId-v1310

CSI-ProcessToAddModList-r11 ::=		SEQUENCE (SIZE (1..maxCSI-Proc-r11)) OF CSI-Process-r11

CSI-ProcessToReleaseList-r11 ::=	SEQUENCE (SIZE (1..maxCSI-Proc-r11)) OF CSI-ProcessId-r11

CQI-ReportBothProc-r11 ::=			SEQUENCE {

ri-Ref-CSI-ProcessId-r11			CSI-ProcessId-r11				OPTIONAL,		-- Need OR

pmi-RI-Report-r11					ENUMERATED {setup}				OPTIONAL		-- Need OR

}

-- ASN1STOP

CQI-ReportBoth field descriptions

csi-IM-ConfigToAddModList For a serving frequency E-UTRAN configures one or more CSI-IM-Config only when transmission mode 10 is configured for the serving cell on this carrier frequency.

csi-ProcessToAddModList For a serving frequency E-UTRAN configures one or more CSI-Process only when transmission mode 10 is configured for the serving cell on this carrier frequency.

cqi-ReportModeAperiodic Parameter: reporting mode. Value rm12 corresponds to Mode 1-2, rm20 corresponds to Mode 2-0, rm22 corresponds to Mode 2-2 etc. PUSCH reporting modes are described in TS 36.213 [23], clause 7.2.1. The UE shall ignore cqi-ReportModeAperiodic-r10 when transmission mode 10 is configured for the serving cell on this carrier frequency. The UE shall ignore cqi-ReportModeAperiodic-r10 configured for the PCell/ PSCell when the transmission bandwidth of the PCell/PSCell in downlink is 6 resource blocks.

pmi-RI-Report See TS 36.213 [23], clause 7.2. The presence of this field means PMI/RI reporting is configured; otherwise the PMI/RI reporting is not configured. EUTRAN configures this field only when transmissionMode is set to tm8, tm9 or tm10. The UE shall ignore pmi-RI-Report-r9/ pmi-RI-Report-r10 when transmission mode 10 is configured for the serving cell on this carrier frequency.

ri-Ref-CSI-ProcessId CSI process whose RI value the UE inherits when reporting RI, in the same subframe, for CSI reporting. E-UTRAN ensures that the CSI process that inherits the RI value is configured in accordance with the conditions specified in TS 36.213 [23], clauses 7.2.1 and 7.2.2.

#### –	CQI-ReportConfig

The IE CQI-ReportConfig is used to specify the CQI reporting configuration.

CQI-ReportConfig information elements

-- ASN1START

CQI-ReportConfig ::=				SEQUENCE {

cqi-ReportModeAperiodic			CQI-ReportModeAperiodic	OPTIONAL,			-- Need OR

nomPDSCH-RS-EPRE-Offset				INTEGER (-1..6),

cqi-ReportPeriodic				CQI-ReportPeriodic	OPTIONAL				-- Need ON

}

CQI-ReportConfig-v920 ::=		SEQUENCE {

cqi-Mask-r9						ENUMERATED {setup}		OPTIONAL,		-- Cond cqi-Setup

pmi-RI-Report-r9				ENUMERATED {setup}		OPTIONAL		-- Cond PMIRI

}

CQI-ReportConfig-r10 ::=	SEQUENCE {

cqi-ReportAperiodic-r10				CQI-ReportAperiodic-r10			OPTIONAL,	-- Need ON

nomPDSCH-RS-EPRE-Offset			INTEGER (-1..6),

cqi-ReportPeriodic-r10				CQI-ReportPeriodic-r10			OPTIONAL,	-- Need ON

pmi-RI-Report-r9					ENUMERATED {setup}				OPTIONAL,	-- Cond PMIRIPCell

csi-SubframePatternConfig-r10		CHOICE {

release							NULL,

setup							SEQUENCE {

csi-MeasSubframeSet1-r10			MeasSubframePattern-r10,

csi-MeasSubframeSet2-r10			MeasSubframePattern-r10

}

}																	OPTIONAL	-- Need ON

}

CQI-ReportConfig-v1130 ::=	SEQUENCE {

cqi-ReportPeriodic-v1130			CQI-ReportPeriodic-v1130,

cqi-ReportBoth-r11					CQI-ReportBoth-r11

}

CQI-ReportConfig-v1250 ::=		SEQUENCE {

csi-SubframePatternConfig-r12		CHOICE {

release							NULL,

setup							SEQUENCE {

csi-MeasSubframeSets-r12			BIT STRING (SIZE (10))

}

}																OPTIONAL,	-- Need ON

cqi-ReportBoth-v1250					CQI-ReportBoth-v1250		OPTIONAL,	-- Need ON

cqi-ReportAperiodic-v1250	CQI-ReportAperiodic-v1250		OPTIONAL,	-- Need ON

altCQI-Table-r12			ENUMERATED {

allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}		OPTIONAL		-- Need OP

}

CQI-ReportConfig-v1310 ::=			SEQUENCE {

cqi-ReportBoth-v1310				CQI-ReportBoth-v1310		OPTIONAL,	-- Need ON

cqi-ReportAperiodic-v1310			CQI-ReportAperiodic-v1310	OPTIONAL,		-- Need ON

cqi-ReportPeriodic-v1310			CQI-ReportPeriodic-v1310	OPTIONAL		-- Need ON

}

CQI-ReportConfig-v1320 ::=			SEQUENCE {

cqi-ReportPeriodic-v1320			CQI-ReportPeriodic-v1320	OPTIONAL	-- Need ON

}

CQI-ReportConfig-v1430 ::=			SEQUENCE {

cqi-ReportAperiodicHybrid-r14		CQI-ReportAperiodicHybrid-r14	OPTIONAL	-- Need ON

}

CQI-ReportConfig-v1530 ::=		SEQUENCE {

altCQI-Table-1024QAM-r15		ENUMERATED {

allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}		OPTIONAL		-- Need OP

}

CQI-ReportConfig-r15 ::=	CHOICE {

release					NULL,

setup					SEQUENCE {

cqi-ReportConfig-r10			CQI-ReportConfig-r10			OPTIONAL,		-- Need ON

cqi-ReportConfig-v1130			CQI-ReportConfig-v1130			OPTIONAL,		-- Need ON

cqi-ReportConfigPCell-v1250		CQI-ReportConfig-v1250			OPTIONAL,		-- Need ON

cqi-ReportConfig-v1310			CQI-ReportConfig-v1310			OPTIONAL,		-- Need ON

cqi-ReportConfig-v1320			CQI-ReportConfig-v1320			OPTIONAL,		-- Need ON

cqi-ReportConfig-v1430			CQI-ReportConfig-v1430			OPTIONAL,		-- Need ON

altCQI-Table-1024QAM-r15		ENUMERATED {allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}		OPTIONAL		-- Need OP

}

}

CQI-ReportConfigSCell-r10 ::=				SEQUENCE {

cqi-ReportModeAperiodic-r10			CQI-ReportModeAperiodic OPTIONAL,			-- Need OR

nomPDSCH-RS-EPRE-Offset-r10				INTEGER (-1..6),

cqi-ReportPeriodicSCell-r10			CQI-ReportPeriodic-r10			OPTIONAL,	-- Need ON

pmi-RI-Report-r10					ENUMERATED {setup}				OPTIONAL	-- Cond PMIRISCell

}

CQI-ReportConfigSCell-r15 ::=			SEQUENCE {

cqi-ReportPeriodicSCell-r15				CQI-ReportPeriodicSCell-r15		OPTIONAL,	-- Need ON

altCQI-Table-1024QAM-r15				ENUMERATED {allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}		OPTIONAL		-- Need OP

}

-- ASN1STOP

CQI-ReportConfig field descriptions

altCQI-Table, altCQI-Table-1024QAM Indicates the applicability of the alternative CQI table (i.e. Table 7.2.3-2 and Table 7.2.3-4 in TS 36.213 [23]) for both aperiodic and periodic CSI reporting for the concerned serving cell. Value allSubframes means the alternative CQI table applies to all the subframes and CSI processes, if configured, and value csi-SubframeSet1 means the alternative CQI table applies to CSI subframe set1, and value csi-SubframeSet2 means the alternative CQI table applies to CSI subframe set2. EUTRAN sets the value to csi-SubframeSet1 or csi-SubframeSet2 only if transmissionMode is set in range tm1 to tm9 and csi-SubframePatternConfig-r10 is configured for the concerned serving cell and different CQI tables apply to the two CSI subframe sets; otherwise EUTRAN sets the value to allSubframes. EUTRAN does not configure altCQI-Table-r12 in CQI-ReportConfig-v1250 and altCQI-Table-1024QAM-r15 in CQI-ReportConfig-v1530 or in CQI-ReportConfigSCell-r15 in the same serving cell simultaneously. If altCQI-Table-r12 and altCQI-Table-1024QAM-r15 are absent, the UE shall use Table 7.2.3-1 in TS 36.213 [23] for all subframes and CSI processes, if configured.

cqi-Mask Limits CQI/PMI/PTI/RI reports to the on-duration period of the DRX cycle, see TS 36.321 [6]. One value applies for all CSI processes and all serving cells (the associated functionality is common i.e. not performed independently for each cell).

cqi-ReportAperiodic E-UTRAN does not configure CQI-ReportAperiodic when transmission mode 10 is configured for all serving cells. E-UTRAN configures cqi-ReportAperiodic-v1250 only if cqi-ReportAperiodic-r10 and csi-MeasSubframeSets-r12 are configured. E-UTRAN configures cqi-ReportAperiodic-v1310 only if cqi-ReportAperiodic-r10 is configured.

cqi-ReportModeAperiodic Parameter: reporting mode. Value rm12 corresponds to Mode 1-2, rm20 corresponds to Mode 2-0, rm22 corresponds to Mode 2-2 etc. PUSCH reporting modes are described in TS 36.213 [23], clause 7.2.1. The UE shall ignore cqi-ReportModeAperiodic-r10 when transmission mode 10 is configured for the serving cell on this carrier frequency. The UE shall ignore cqi-ReportModeAperiodic-r10 configured for the PCell/ PSCell when the transmission bandwidth of the PCell/PSCell in downlink is 6 resource blocks.

cqi-ReportPeriodic E-UTRAN does not configure CQI-ReportPeriodic for sTTI within CQI-ReportConfig.

csi-MeasSubframeSets Indicates the two CSI subframe sets. Value 0 means the subframe belongs to CSI subframe set 1 and value 1 means the subframe belongs to CSI subframe set 2. CSI subframe set 1 refers to CCSI,0 in TS 36.213 [23], clause 7.2, and CSI subframe set 2 refers to CCSI,1 in TS 36.213 [23], clause 7.2. EUTRAN does not configure csi-MeasSubframeSet1-r10 and csi-MeasSubframeSet2-r10 if either csi-MeasSubframeSets-r12 for PCell or eimta-MainConfigPCell-r12 is configured.

csi-MeasSubframeSet1, csi-MeasSubframeSet2 Indicates the CSI measurement subframe sets. csi-MeasSubframeSet1 refers to CCSI,0 in TS 36.213 [23], clause 7.2 and csi-MeasSubframeSet2 refers to CCSI,1 in TS 36.213 [23], clause 7.2. E-UTRAN only configures the two CSI measurement subframe sets for the PCell.

nomPDSCH-RS-EPRE-Offset Parameter: see TS 36.213 [23], clause 7.2.3. Actual value = field value * 2 [dB].

pmi-RI-Report See TS 36.213 [23], clause 7.2. The presence of this field means PMI/RI reporting is configured; otherwise the PMI/RI reporting is not configured. EUTRAN configures this field only when transmissionMode is set to tm8, tm9 or tm10. The UE shall ignore pmi-RI-Report-r9/ pmi-RI-Report-r10 when transmission mode 10 is configured for the serving cell on this carrier frequency.

| Conditional presence | Explanation |
| --- | --- |
| cqi-Setup | This field is not present for an Scell except for the PSCell, while it is conditionally present for the PCell and the PSCell according to the following. The field is optional present, need OR, if the cqi-ReportPeriodic in the cqi-ReportConfig is set to setup. If the field cqi-ReportPeriodic is present and set to release, the field is not present and the UE shall delete any existing value for this field. Otherwise the field is not present. |
| PMIRI | The field is optional present, need OR, if cqi-ReportPeriodic is included and set to setup, or cqi-ReportModeAperiodic is included. If the field cqi-ReportPeriodic is present and set to release and cqi-ReportModeAperiodic is absent, the field is not present and the UE shall delete any existing value for this field. Otherwise the field is not present. |
| PMIRIPCell | The field is optional present, need OR, if cqi-ReportPeriodic is included in the CQI-ReportConfig-r10 and set to setup, or cqi-ReportAperiodic is included in the CQI-ReportConfig-r10 and set to setup. If the field cqi-ReportPeriodic is present in the CQI-ReportConfig-r10 and set to release and cqi-ReportAperiodic is included in the CQI-ReportConfig-r10 and set to release, the field is not present and the UE shall delete any existing value for this field. Otherwise the field is not present. |
| PMIRISCell | The field is optional present, need OR, if cqi-ReportPeriodicSCell is included and set to setup, or cqi-ReportModeAperiodic-r10 is included in the CQI-ReportConfigSCell. If the field cqi-ReportPeriodicSCell is present and set to release and cqi-ReportModeAperiodic-r10 is absent in the CQI-ReportConfigSCell, the field is not present and the UE shall delete any existing value for this field. Otherwise the field is not present. |

#### –	CQI-ReportPeriodic

The IE CQI-ReportPeriodic is used to specify the periodic CQI reporting configuration elements.

CQI-ReportPeriodic information elements

-- ASN1START

CQI-ReportPeriodic ::=		CHOICE {

release							NULL,

setup							SEQUENCE {

cqi-PUCCH-ResourceIndex				INTEGER (0..1185),

cqi-pmi-ConfigIndex					INTEGER (0..1023),

cqi-FormatIndicatorPeriodic			CHOICE {

widebandCQI							NULL,

subbandCQI							SEQUENCE {

k									INTEGER (1..4)

}

},

ri-ConfigIndex						INTEGER (0..1023)	OPTIONAL,				-- Need OR

simultaneousAckNackAndCQI			BOOLEAN

}

}

CQI-ReportPeriodic-r10 ::=		CHOICE {

release								NULL,

setup								SEQUENCE {

cqi-PUCCH-ResourceIndex-r10			INTEGER (0..1184),

cqi-PUCCH-ResourceIndexP1-r10		INTEGER (0..1184)				OPTIONAL,	-- Need OR

cqi-pmi-ConfigIndex				INTEGER (0..1023),

cqi-FormatIndicatorPeriodic-r10		CHOICE {

widebandCQI-r10						SEQUENCE {

csi-ReportMode-r10		ENUMERATED {submode1, submode2}		OPTIONAL	-- Need OR

},

subbandCQI-r10						SEQUENCE {

k								INTEGER (1..4),

periodicityFactor-r10				ENUMERATED {n2, n4}

}

},

ri-ConfigIndex					INTEGER (0..1023)		OPTIONAL,				-- Need OR

simultaneousAckNackAndCQI		BOOLEAN,

cqi-Mask-r9						ENUMERATED {setup}		OPTIONAL,				-- Need OR

csi-ConfigIndex-r10				CHOICE {

release							NULL,

setup							SEQUENCE {

cqi-pmi-ConfigIndex2-r10		INTEGER (0..1023),

ri-ConfigIndex2-r10				INTEGER (0..1023)		OPTIONAL		-- Need OR

}

}		OPTIONAL																-- Need ON

}

}

CQI-ReportPeriodic-v1130 ::=	SEQUENCE {

simultaneousAckNackAndCQI-Format3-r11		ENUMERATED {setup}		OPTIONAL,	-- Need OR

cqi-ReportPeriodicProcExtToReleaseList-r11	CQI-ReportPeriodicProcExtToReleaseList-r11	OPTIONAL,	-- Need ON

cqi-ReportPeriodicProcExtToAddModList-r11	CQI-ReportPeriodicProcExtToAddModList-r11	OPTIONAL	-- Need ON

}

CQI-ReportPeriodic-v1310 ::=	SEQUENCE {

cri-ReportConfig-r13			CRI-ReportConfig-r13				OPTIONAL,	-- Need OR

simultaneousAckNackAndCQI-Format4-Format5-r13		ENUMERATED {setup}		OPTIONAL-- Need OR

}

CQI-ReportPeriodic-v1320 ::=	SEQUENCE {

periodicityFactorWB-r13			ENUMERATED {n2, n4}			OPTIONAL		-- Need OR

}

CQI-ReportPeriodicSCell-r15 ::=	CHOICE {

release								NULL,

setup								SEQUENCE {

cqi-pmi-ConfigIndexDormant-r15		INTEGER (0..1023),

ri-ConfigIndexDormant-r15			INTEGER (0..1023)		OPTIONAL,		-- Need OR

csi-SubframePatternDormant-r15		CHOICE {

release							NULL,

setup							SEQUENCE {

csi-MeasSubframeSet1-r15			MeasSubframePattern-r10,

csi-MeasSubframeSet2-r15			MeasSubframePattern-r10

}

}																OPTIONAL,	-- Need ON

cqi-FormatIndicatorDormant-r15	CHOICE {

widebandCQI-r15				SEQUENCE {

csi-ReportMode-r15			ENUMERATED {submode1, submode2}	OPTIONAL	-- Need OR

},

subbandCQI-r15				SEQUENCE {

k-r15						INTEGER (1..4),

periodicityFactor-r15		ENUMERATED {n2, n4}

}

}															OPTIONAL		-- Need OR

}

}

CQI-ReportPeriodicProcExtToAddModList-r11 ::=		SEQUENCE (SIZE (1..maxCQI-ProcExt-r11)) OF CQI-ReportPeriodicProcExt-r11

CQI-ReportPeriodicProcExtToReleaseList-r11 ::=	SEQUENCE (SIZE (1..maxCQI-ProcExt-r11)) OF CQI-ReportPeriodicProcExtId-r11

CQI-ReportPeriodicProcExt-r11 ::=		SEQUENCE {

cqi-ReportPeriodicProcExtId-r11	CQI-ReportPeriodicProcExtId-r11,

cqi-pmi-ConfigIndex-r11			INTEGER (0..1023),

cqi-FormatIndicatorPeriodic-r11	CHOICE {

widebandCQI-r11				SEQUENCE {

csi-ReportMode-r11			ENUMERATED {submode1, submode2}	OPTIONAL	-- Need OR

},

subbandCQI-r11				SEQUENCE {

k							INTEGER (1..4),

periodicityFactor-r11		ENUMERATED {n2, n4}

}

},

ri-ConfigIndex-r11				INTEGER (0..1023)					OPTIONAL,	-- Need OR

csi-ConfigIndex-r11				CHOICE {

release							NULL,

setup							SEQUENCE {

cqi-pmi-ConfigIndex2-r11		INTEGER (0..1023),

ri-ConfigIndex2-r11				INTEGER (0..1023)		OPTIONAL		-- Need OR

}

}																OPTIONAL,		-- Need ON

...,

[[	cri-ReportConfig-r13			CRI-ReportConfig-r13				OPTIONAL	-- Need ON

]],

[[	periodicityFactorWB-r13			ENUMERATED {n2, n4}			OPTIONAL		-- Need ON

]]

}

CQI-ShortConfigSCell-r15 ::= CHOICE {

release								NULL,

setup								SEQUENCE {

cqi-pmi-ConfigIndexShort-r15	INTEGER (0..1023),

ri-ConfigIndexShort-r15			INTEGER (0..1023)		OPTIONAL,		-- Need OR

cqi-FormatIndicatorShort-r15	CHOICE {

widebandCQI-Short-r15			SEQUENCE {

csi-ReportModeShort-r15		ENUMERATED {submode1, submode2}	OPTIONAL	-- Need OR

},

subbandCQI-Short-r15		SEQUENCE {

k-r15						INTEGER (1..4),

periodicityFactor-r15		ENUMERATED {n2, n4}

}

}															OPTIONAL		-- Need OR

}

}

CRI-ReportConfig-r13 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

cri-ConfigIndex-r13					CRI-ConfigIndex-r13,

cri-ConfigIndex2-r13				CRI-ConfigIndex-r13	OPTIONAL	-- Need OR

}

}

CRI-ConfigIndex-r13 ::=				INTEGER (0..1023)

-- ASN1STOP

CQI-ReportPeriodic field descriptions

cqi-FormatIndicatorPeriodic Parameter: PUCCH CQI Feedback Type, see TS 36.213 [23], table 7.2.2-1. Depending on transmissionMode, reporting mode is implicitly given from the table.

cqi-Mask Limits CQI/PMI/PTI/RI reports to the on-duration period of the DRX cycle, see TS 36.321 [6]. One value applies for all CSI processes and all serving cells (the associated functionality is common i.e. not performed independently for each cell).

cqi-pmi-ConfigIndex Parameter: CQI/PMI Periodicity and Offset Configuration Index ICQI/PMI, see TS 36.213 [23], tables 7.2.2-1A and 7.2.2-1C. If subframe patterns for CSI (CQI/PMI/PTI/RI) reporting are configured (i.e. csi-SubframePatternConfig is configured), the parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet1 or corresponding to the CSI subframe set 1 indicated by csi-MeasSubframeSets-r12.

cqi-pmi-ConfigIndex2 Parameter: CQI/PMI Periodicity and Offset Configuration Index ICQI/PMI, see TS 36.213 [23], tables 7.2.2-1A and 7.2.2-1C. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet2 or corresponding to the CSI subframe set 2 indicated by csi-MeasSubframeSets-r12.

cqi-PUCCH-ResourceIndex, cqi-PUCCH-ResourceIndexP1 Parameter for antenna port P0 and for antenna port P1 respectively, see TS 36.213 [23], clause 7.2. E-UTRAN does not apply value 1185. One value applies for all CSI processes.

cqi-ReportAperiodic E-UTRAN does not configure CQI-ReportAperiodic when transmission mode 10 is configured for all serving cells. E-UTRAN configures cqi-ReportAperiodic-v1250 only if cqi-ReportAperiodic-r10 and csi-MeasSubframeSets-r12 are configured. E-UTRAN configures cqi-ReportAperiodic-v1310 only if cqi-ReportAperiodic-r10 is configured.

cqi-ReportModeAperiodic Parameter: reporting mode. Value rm12 corresponds to Mode 1-2, rm20 corresponds to Mode 2-0, rm22 corresponds to Mode 2-2 etc. PUSCH reporting modes are described in TS 36.213 [23], clause 7.2.1. The UE shall ignore cqi-ReportModeAperiodic-r10 when transmission mode 10 is configured for the serving cell on this carrier frequency. The UE shall ignore cqi-ReportModeAperiodic-r10 configured for the PCell/ PSCell when the transmission bandwidth of the PCell/PSCell in downlink is 6 resource blocks.

CQI-ReportPeriodicProcExt A set of periodic CQI related parameters for which E-UTRAN may configure different values for each CSI process. For a serving frequency E-UTRAN configures one or more CQI-ReportPeriodicProcExt only when transmission mode 10 is configured for the serving cell on this carrier frequency.

cri-ConfigIndex Parameter: cri-ConfigIndex ICRIsee TS 36.213 [23]. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet1. EUTRAN configures the field if subframe patterns for CSI (CQI/PMI/PTI/RI/CRI) reporting are configured (i.e. csi-SubframePatternConfig is configured).

cri-ConfigIndex2 Parameter: cri-ConfigIndex ICRIsee TS 36.213 [23]. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet2 or corresponding to the CSI subframe set 2 indicated by csi-MeasSubframeSets. E-UTRAN configures cri-ConfigIndex2 only if cri-ConfigIndex is configured.

cri-ReportConfig E-UTRAN configures the field only if the UE is configured with eMIMO-Type set to "beamformed" and if multiple references to RS configuration using non-zero power transmission are configured (i.e. if csi-RS-ConfigNZPIdListExt is configured).

csi-ConfigIndex E-UTRAN configures csi-ConfigIndex only for PCell and only if csi-SubframePatternConfig is configured. The UE shall release csi-ConfigIndex if csi-SubframePatternConfig is released.

csi-ProcessToAddModList For a serving frequency E-UTRAN configures one or more CSI-Process only when transmission mode 10 is configured for the serving cell on this carrier frequency.

csi-ReportMode Parameter: PUCCH_format1-1_CSI_reporting_mode, see TS 36.213 [23], clause 7.2.2.

K Parameter: K, see TS 36.213 [23], clause 7.2.2.

nomPDSCH-RS-EPRE-Offset Parameter: see TS 36.213 [23], clause 7.2.3. Actual value = field value * 2 [dB].

periodicityFactor, periodicityFactorWB Parameter: , see TS 36.213 [23], clause 7.2.2. EUTRAN configures field periodicityFactorWB only when the UE is configured with eMIMO-Type set to nonPrecoded and with cqi-FormatIndicatorPeriodic set to widebandCQI.

ri-ConfigIndex Parameter: RI Config Index IRI, see TS 36.213 [23], clause 7.2.2-1B. If subframe patterns for CSI (CQI/PMI/PTI/RI/CRI) reporting are configured (i.e. csi-SubframePatternConfig is configured), the parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet1.

ri-ConfigIndex2 Parameter: RI Config Index IRI, see TS 36.213 [23], clause 7.2.2-1B. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet2 or corresponding to the CSI subframe set 2 indicated by csi-MeasSubframeSets-r12. E-UTRAN configures ri-ConfigIndex2 only if ri-ConfigIndex is configured.

simultaneousAckNackAndCQI Parameter: Simultaneous-AN-and-CQI, see TS 36.213 [23], clause 10.1. TRUE indicates that simultaneous transmission of ACK/NACK and CQI is allowed. One value applies for all CSI processes. For SCells except for the PSCell and PUCCH SCell this field is not applicable and the UE shall ignore the value.

simultaneousAckNackAndCQI-Format3 Indicates that the UE shall perform simultaneous transmission of HARQ A/N and periodic CQI report multiplexing on PUCCH format 3, see TS 36.213 [23], clauses 7.2 and 10.1.1. E-UTRAN configures this information only when pucch-Format is set to format3. One value applies for all CSI processes. For SCells except for the PSCell and PUCCH SCell this field is not applicable and the UE shall ignore the value.

simultaneousAckNackAndCQI-Format4-Format5 Indicates that the UE shall perform simultaneous transmission of HARQ A/N and periodic CSI report multiplexing on PUCCH format 4 and format 5, see TS 36.213 [23], clause 10.1.1. E-UTRAN configures this information only when pucch-Format is set to format4 or format5. One value applies for all CSI processes. For SCells except for the PSCell and PUCCH SCell this field is not applicable and the UE shall ignore the value.

#### –	CQI-ReportPeriodicProcExtId

The IE CQI-ReportPeriodicProcExtId is used to identify a periodic CQI reporting configuration that E-UTRAN may configure in addition to the configuration specified by the IE CQI-ReportPeriodic-r10. These additional configurations are specified by the IE CQI-ReportPeriodicProcExt-r11. The identity is unique within the scope of a carrier frequency.

CQI-ReportPeriodicProcExtId information elements

-- ASN1START

CQI-ReportPeriodicProcExtId-r11 ::=					INTEGER (1..maxCQI-ProcExt-r11)

-- ASN1STOP

#### –	CrossCarrierSchedulingConfig

The IE CrossCarrierSchedulingConfig is used to specify the configuration when the cross carrier scheduling is used in a cell.

CrossCarrierSchedulingConfig information elements

-- ASN1START

CrossCarrierSchedulingConfig-r10 ::=		SEQUENCE {

schedulingCellInfo-r10				CHOICE {

own-r10								SEQUENCE {					-- No cross carrier scheduling

cif-Presence-r10						BOOLEAN

},

other-r10								SEQUENCE {					-- Cross carrier scheduling

schedulingCellId-r10				ServCellIndex-r10,

pdsch-Start-r10						INTEGER (1..4)

}

}

}

CrossCarrierSchedulingConfig-r13 ::=		SEQUENCE {

schedulingCellInfo-r13				CHOICE {

own-r13								SEQUENCE {					-- No cross carrier scheduling

cif-Presence-r13						BOOLEAN

},

other-r13							SEQUENCE {					-- Cross carrier scheduling

schedulingCellId-r13				ServCellIndex-r13,

pdsch-Start-r13						INTEGER (1..4),

cif-InSchedulingCell-r13				INTEGER (1..7)

}

}

}

CrossCarrierSchedulingConfigLAA-UL-r14 ::=		SEQUENCE {

schedulingCellId-r14							ServCellIndex-r13,

cif-InSchedulingCell-r14						INTEGER (1..7)

}

-- ASN1STOP

CrossCarrierSchedulingConfig field descriptions

cif-Presence The field is used to indicate whether carrier indicator field is present (value TRUE) or not (value FALSE) in PDCCH/ EPDCCH DCI formats, see TS 36.212 [22], clause 5.3.3.1.

cif-InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate this cell, see TS 36.212 [22], clause 5.3.3.1. In case of carrier indicator field is present, the CIF value is 0.

pdsch-Start The starting OFDM symbol of PDSCH for the concerned SCell, see TS 36.213 [23]. clause 7.1.6.4. Values 1, 2, 3 are applicable when dl-Bandwidth for the concerned SCell is greater than 10 resource blocks, values 2, 3, 4 are applicable when dl-Bandwidth for the concerned SCell is less than or equal to 10 resource blocks, see TS 36.211 [21], Tables 6 and 7-1.

schedulingCellId Indicates which cell signals the downlink allocations and uplink grants, if applicable, for the concerned SCell. In case the UE is configured with DC, the scheduling cell is part of the same cell group (i.e. MCG or SCG) as the scheduled cell. In case the UE is configured with crossCarrierSchedulingConfigLAA-UL, schedulingCellId indicated in crossCarrierSchedulingConfigLAA-UL only indicates which cell signals the uplink grants.

#### –	CRS-ChEstMPDCCH-Config

The IE CRS-ChEstMPDCCH-Config is used to configure and enable use of CRS for MPDCCH performance improvement, see TS 36.211 [21], clause 6.8B.5 and TS 36.213 [23], clause 9.1.5.

CRS-ChEstMPDCCH-Config information elements

-- ASN1START

CRS-ChEstMPDCCH-ConfigCommon-r16 ::=		SEQUENCE {

powerRatio-r16		ENUMERATED {dB-4dot77, dB-3, dB-1dot77, dB0, dB1, dB2, dB3, dB4dot77}

}

CRS-ChEstMPDCCH-ConfigDedicated-r16 ::=		SEQUENCE {

powerRatio-r16				ENUMERATED {dB-4dot77, dB-3, dB-1dot77, dB0, dB1, dB2, dB3,

dB4dot77}	OPTIONAL, -- Cond setup

localizedMappingType-r16	ENUMERATED {predefined, csi-Based, reciprocityBased}

DEFAULT	predefined

}

-- ASN1STOP

CRS-ChEstMPDCCH-Config field descriptions

powerRatio Power ratio in dB between DMRS and CRS antenna ports of MPDCCH, see TS 36.213 [23], clause 5.2. Value dB-4dot77 corresponds to -4.77 dB, value dB-3 corresponds to -3 dB and so on.

localizedMappingType DMRS mapping type for MPDCCH performance improvement with localized MPDCCH allocation for CE mode A or B in RRC_CONNECTED, see TS 36.213 [23], clause 9.1.5. Value predefined corresponds to predefined mapping, value csi-Based corresponds to CSI-based mapping, and value reciprocityBased corresponds to reciprocity based mapping. Reciprocity based mapping is only applicable for TDD.

| Conditional presence | Explanation |
| --- | --- |
| setup | The field is mandatory present if CRS-ChEstMPDCCH-ConfigDedicated is set to setup and this field has not been configured in CRS-ChEstMPDCCH-ConfigCommon; otherwise the field is optional, need ON. |

#### –	CSI-IM-Config

The IE CSI-IM-Config is the CSI Interference Measurement (IM) configuration that E-UTRAN may configure on a serving frequency, see TS 36.213 [23], clause 7.2.6.

CSI-IM-Config information elements

-- ASN1START

CSI-IM-Config-r11 ::=		SEQUENCE {

csi-IM-ConfigId-r11			CSI-IM-ConfigId-r11,

resourceConfig-r11			INTEGER (0..31),

subframeConfig-r11			INTEGER (0..154),

...,

[[	interferenceMeasRestriction-r13		BOOLEAN		OPTIONAL	-- Need ON

]]

}

CSI-IM-ConfigExt-r12 ::=		SEQUENCE {

csi-IM-ConfigId-v1250			CSI-IM-ConfigId-v1250,

resourceConfig-r12			INTEGER (0..31),

subframeConfig-r12			INTEGER (0..154),

...,

[[	interferenceMeasRestriction-r13	BOOLEAN				OPTIONAL,	-- Need ON

csi-IM-ConfigId-v1310		CSI-IM-ConfigId-v1310	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

CSI-IM-Config field descriptions

resourceConfig Parameter: CSI reference signal configuration, see TS 36.213 [23], clause 7.2.6 and TS 36.211 [21], tables 6.10.5.2-1 and 6.10.5.2-2 for 4 REs.

subframeConfig Parameter: , see TS 36.213 [23], clause 7.2.6 and TS 36.211 [21], table 6.10.5.3-1.

#### –	CSI-IM-ConfigId

The IE CSI-IM-ConfigId is used to identify a CSI-IM configuration that is configured by the IE CSI-IM-Config. The identity is unique within the scope of a carrier frequency.

CSI-IM-ConfigId information elements

-- ASN1START

CSI-IM-ConfigId-r11 ::=					INTEGER (1..maxCSI-IM-r11)

CSI-IM-ConfigId-r12 ::=					INTEGER (1..maxCSI-IM-r12)

CSI-IM-ConfigId-v1250 ::=				INTEGER (maxCSI-IM-r12)

CSI-IM-ConfigId-v1310 ::=				INTEGER (minCSI-IM-r13..maxCSI-IM-r13)

CSI-IM-ConfigId-r13 ::=					INTEGER (1..maxCSI-IM-r13)

-- ASN1STOP

#### –	CSI-Process

The IE CSI-Process is the CSI process configuration that E-UTRAN may configure on a serving frequency.

CSI-Process information elements

-- ASN1START

CSI-Process-r11 ::=		SEQUENCE {

csi-ProcessId-r11			CSI-ProcessId-r11,

csi-RS-ConfigNZPId-r11		CSI-RS-ConfigNZPId-r11,

csi-IM-ConfigId-r11			CSI-IM-ConfigId-r11,

p-C-AndCBSRList-r11			P-C-AndCBSR-Pair-r13a,

cqi-ReportBothProc-r11		CQI-ReportBothProc-r11					OPTIONAL,		-- Need OR

cqi-ReportPeriodicProcId-r11	INTEGER (0..maxCQI-ProcExt-r11)		OPTIONAL,		-- Need OR

cqi-ReportAperiodicProc-r11	CQI-ReportAperiodicProc-r11				OPTIONAL,		-- Need OR

...,

[[	alternativeCodebookEnabledFor4TXProc-r12	ENUMERATED {true}	OPTIONAL,	-- Need ON

csi-IM-ConfigIdList-r12		CHOICE {

release						NULL,

setup						SEQUENCE (SIZE (1..2)) OF CSI-IM-ConfigId-r12

}																OPTIONAL,	-- Need ON

cqi-ReportAperiodicProc2-r12	CHOICE {

release						NULL,

setup						CQI-ReportAperiodicProc-r11

}																OPTIONAL	-- Need ON

]],

[[	cqi-ReportAperiodicProc-v1310	CHOICE {

release							NULL,

setup							CQI-ReportAperiodicProc-v1310

}																OPTIONAL,		-- Need ON

cqi-ReportAperiodicProc2-v1310	CHOICE {

release							NULL,

setup							CQI-ReportAperiodicProc-v1310

}																OPTIONAL,		-- Need ON

eMIMO-Type-r13					CSI-RS-ConfigEMIMO-r13			OPTIONAL		-- Need ON

]],

[[	dummy				CSI-RS-ConfigEMIMO-v1430		OPTIONAL,		-- Need ON

eMIMO-Hybrid-r14				CSI-RS-ConfigEMIMO-Hybrid-r14		OPTIONAL,	-- Need ON

advancedCodebookEnabled-r14		BOOLEAN							OPTIONAL		-- Need ON

]],

[[	eMIMO-Type-v1480				CSI-RS-ConfigEMIMO-v1480		OPTIONAL		-- Need ON

]],

[[	feCOMP-CSI-Enabled-v1530		BOOLEAN							OPTIONAL,		-- Need ON

eMIMO-Type-v1530				CSI-RS-ConfigEMIMO-v1530		OPTIONAL		-- Need ON

]]

}

-- ASN1STOP

CSI-Process field descriptions

advancedCodebookEnabled Value TRUE indicates that the UE should use the advanced code book defined in TS 36.213 [23]. EUTRAN does not configure the field when the UE is configured with eMIMO-Type is set to beamformed, when the UE is configured with eMIMO-Hybrid or when the UE is configured with semiOpenLoop.

alternativeCodebookEnabledFor4TXProc Indicates whether code book in TS 36.213 [23] Table 7.2.4-0A to Table 7.2.4-0D is being used for deriving CSI feedback and reporting for a CSI process. EUTRAN may configure the field only if the number of CSI-RS ports for non-zero power transmission CSI-RS configuration is 4.

cqi-ReportAperiodicProc If csi-MeasSubframeSets-r12 is configured for the same frequency as the CSI process, cqi-ReportAperiodicProc applies for CSI subframe set 1. If csi-MeasSubframeSet1-r10 or csi-MeasSubframeSet2-r10 are configured for the same frequency as the CSI process, cqi-ReportAperiodicProc applies for CSI subframe set 1 or CSI subframe set 2. Otherwise, cqi-ReportAperiodicProc applies for all subframes. E-UTRAN configures cqi-ReportAperiodicProc-v1310 only if cqi-ReportAperiodicProc-r11 is configured

cqi-ReportAperiodicProc2 cqi-ReportAperiodicProc2 is configured only if csi-MeasSubframeSets-r12 is configured for the same frequency as the CSI process. cqi-ReportAperiodicProc2 is for CSI subframe set 2. E-UTRAN shall set cqi-ReportModeAperiodic-r11 in cqi-ReportAperiodicProc2 the same as in cqi-ReportAperiodicProc. E-UTRAN configures cqi-ReportAperiodicProc2-v1310 only if cqi-ReportAperiodicProc2-r12 is configured.

cqi-ReportBothProc Includes CQI configuration parameters applicable for both aperiodic and periodic CSI reporting, for which CSI process specific values may be configured. E-UTRAN configures the field if and only if cqi-ReportPeriodicProcId is included and/ or if cqi-ReportAperiodicProc is included.

cqi-ReportPeriodicProcId Refers to a periodic CQI reporting configuration that is configured for the same frequency as the CSI process. Value 0 refers to the set of parameters defined by the REL-10 CQI reporting configuration fields, while the other values refer to the additional configurations E-UTRAN assigns by CQI-ReportPeriodicProcExt-r11 (and as covered by CQI-ReportPeriodicProcExtId).

csi-IM-ConfigId Refers to a CSI-IM configuration that is configured for the same frequency as the CSI process. If csi-IM-ConfigId-v1250 or csi-IM-ConfigId-v1310 is configured, the UE only considers this extension (i.e., UE ignores csi-IM-ConfigId-r11 and csi-IM-ConfigId-r12).

csi-IM-ConfigIdList Refers to one or two CSI-IM configurations that are configured for the same frequency as the CSI process. csi-IM-ConfigIdList can include 2 entries only if csi-MeasSubframeSets-r12 is configured for the same frequency as the CSI process.

csi-RS-ConfigNZPId Refers to a CSI RS configuration using non-zero power transmission that is configured for the same frequency as the CSI process.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

eMIMO-Type Parameter: eMIMO-Type, see TS 36.213 [23], TS 36.211 [21]. If eMIMO-Type is set to nonPrecoded, the codebooks used for deriving CSI feedback are in TS 36.213 [23], Table 7.2.4-10 to Table 7.2.4-17. Choice values nonPrecoded and beamformed correspond to 'CLASS A' and 'CLASS B' respectively, see TS 36.212 [22] and TS 36.213 [23].

feCOMP-CSI-Enabled Parameter: FeCoMPCSIEnabled, see TS 36.213 [23], clause 7.1.10. Refers to CSI feedback based on FeCoMP. E-UTRAN only configures the field when the UE is configured with eMIMO-Type-r13 set to beamformed with two NZP CSI-RS resources using the IE CSI-RS-ConfigBeamformed-r13 which contains the two NZP CSI-RS reources configued with csi-RS-ConfigNZPIdListExt-r13.

p-C-AndCBSRList The UE shall ignore p-C-AndCBSRList-r11 if configured with eMIMO-Type unless it is set to beamformed, alternativeCodebookEnabledBeamformed (in CSI-RS-ConfigBeamformed) is set to FALSE and csi-RS-ConfigNZPIdListExt is not configured,

#### –	CSI-ProcessId

The IE CSI-ProcessId is used to identify a CSI process that is configured by the IE CSI-Process. The identity is unique within the scope of a carrier frequency.

CSI-ProcessId information elements

-- ASN1START

CSI-ProcessId-r11 ::=		INTEGER (1..maxCSI-Proc-r11)

-- ASN1STOP

#### –	CSI-RS-Config

The IE CSI-RS-Config is used to specify the CSI (Channel-State Information) reference signal configuration.

CSI-RS-Config information elements

-- ASN1START

CSI-RS-Config-r10 ::=		SEQUENCE {

csi-RS-r10					CHOICE {

release						NULL,

setup						SEQUENCE {

antennaPortsCount-r10			ENUMERATED {an1, an2, an4, an8},

resourceConfig-r10				INTEGER (0..31),

subframeConfig-r10				INTEGER (0..154),

p-C-r10							INTEGER (-8..15)

}

}																OPTIONAL,			-- Need ON

zeroTxPowerCSI-RS-r10		ZeroTxPowerCSI-RS-Conf-r12			OPTIONAL			-- Need ON

}

CSI-RS-Config-v1250 ::=		SEQUENCE {

zeroTxPowerCSI-RS2-r12		ZeroTxPowerCSI-RS-Conf-r12			OPTIONAL,			-- Need ON

ds-ZeroTxPowerCSI-RS-r12		CHOICE {

release							NULL,

setup							SEQUENCE {

zeroTxPowerCSI-RS-List-r12	SEQUENCE (SIZE (1..maxDS-ZTP-CSI-RS-r12)) OF ZeroTxPowerCSI-RS-r12

}

}															OPTIONAL				-- Need ON

}

CSI-RS-Config-v1310 ::=		SEQUENCE {

eMIMO-Type-r13				CSI-RS-ConfigEMIMO-r13			OPTIONAL	-- Need ON

}

CSI-RS-Config-v1430 ::=		SEQUENCE {

dummy								CSI-RS-ConfigEMIMO-v1430			OPTIONAL,	-- Need ON

eMIMO-Hybrid-r14					CSI-RS-ConfigEMIMO-Hybrid-r14		OPTIONAL,	-- Need ON

advancedCodebookEnabled-r14		BOOLEAN								OPTIONAL	-- Need ON

}

CSI-RS-Config-v1480 ::=		SEQUENCE {

eMIMO-Type-v1480				CSI-RS-ConfigEMIMO-v1480			OPTIONAL	-- Need ON

}

CSI-RS-Config-v1530 ::=		SEQUENCE {

eMIMO-Type-v1530				CSI-RS-ConfigEMIMO-v1530			OPTIONAL	-- Need ON

}

CSI-RS-Config-r15 ::=		CHOICE {

release					NULL,

setup					SEQUENCE {

csi-RS-Config-r10			CSI-RS-Config-r10				OPTIONAL,		-- Need ON

csi-RS-Config-v1250		CSI-RS-Config-v1250			OPTIONAL,		-- Need ON

csi-RS-Config-v1310		CSI-RS-Config-v1310			OPTIONAL,		-- Need ON

csi-RS-Config-v1430		CSI-RS-Config-v1430			OPTIONAL		-- Need ON

}

}

ZeroTxPowerCSI-RS-Conf-r12 ::=	CHOICE {

release							NULL,

setup							ZeroTxPowerCSI-RS-r12

}

ZeroTxPowerCSI-RS-r12 ::=	SEQUENCE {

zeroTxPowerResourceConfigList-r12		BIT STRING (SIZE (16)),

zeroTxPowerSubframeConfig-r12			INTEGER (0..154)

}

-- ASN1STOP

CSI-RS-Config field descriptions

advancedCodebookEnabled Value TRUE indicates that the UE should use the advanced code book defined in TS 36.213 [23]. EUTRAN does not configure the field when the UE is configured with eMIMO-Type is set to beamformed, when the UE is configured with eMIMO-Hybrid or when the UE is configured with semiOpenLoop.

antennaPortsCount Parameter represents the number of antenna ports used for transmission of CSI reference signals where value an1 corresponds to 1 antenna port, an2 to 2 antenna ports and so on, see TS 36.211 [21], clause 6.10.5.

ds-ZeroTxPowerCSI-RS Parameter for additional zeroTxPowerCSI-RS for a serving cell, concerning the CSI-RS included in discovery signals.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

eMIMO-Type Parameter: eMIMO-Type, see TS 36.213 [23], TS 36.211 [21]. If eMIMO-Type is set to nonPrecoded, the codebooks used for deriving CSI feedback are in TS 36.213 [23], Table 7.2.4-10 to Table 7.2.4-17. Choice values nonPrecoded and beamformed correspond to 'CLASS A' and 'CLASS B' respectively, see TS 36.212 [22] and TS 36.213 [23].

p-C Parameter: , see TS 36.213 [23], clause 7.2.5. The UE shall ignore p-C-r10 if configured with eMIMO-Type unless it is set to beamformed, alternativeCodebookEnabledBeamformed (in CSI-RS-ConfigBeamformed) is set to FALSE and csi-RS-ConfigNZPIdListExt is not configured.

resourceConfig Parameter: CSI reference signal configuration, see TS 36.211 [21], tables 6.10.5.2-1 and 6.10.5.2-2.

subframeConfig Parameter: , see TS 36.211 [21], table 6.10.5.3-1.

zeroTxPowerCSI-RS2 Parameter for additional zeroTxPowerCSI-RS for a serving cell. E-UTRAN configures the field only if csi-MeasSubframeSets-r12 and TM 1 – 9 are configured for the serving cell.

zeroTxPowerResourceConfigList Parameter: ZeroPowerCSI-RS, see TS 36.213 [23], clause 7.2.7.

zeroTxPowerSubframeConfig Parameter: , see TS 36.211 [21], table 6.10.5.3-1.

#### –	CSI-RS-ConfigBeamformed

The IE CSI-RS-ConfigBeamformed is used to specify the beamforming configuration of EBF/ FD-MIMO.

CSI-RS-ConfigBeamformed information elements

-- ASN1START

CSI-RS-ConfigBeamformed-r13 ::=			SEQUENCE	{

csi-RS-ConfigNZPIdListExt-r13			SEQUENCE (SIZE (1..7)) OF CSI-RS-ConfigNZPId-r13	OPTIONAL,	-- Need OR

csi-IM-ConfigIdList-r13					SEQUENCE (SIZE (1..8)) OF CSI-IM-ConfigId-r13	OPTIONAL,	-- Need OR

p-C-AndCBSR-PerResourceConfigList-r13	SEQUENCE (SIZE (1..8)) OF P-C-AndCBSR-Pair-r13	OPTIONAL,	-- Need OR

ace-For4Tx-PerResourceConfigList-r13	SEQUENCE (SIZE (1..7)) OF BOOLEAN	OPTIONAL,	-- Need OR

alternativeCodebookEnabledBeamformed-r13	ENUMERATED {true}	OPTIONAL,	-- Need OR

channelMeasRestriction-r13				ENUMERATED {on}			OPTIONAL	-- Need OR

}

CSI-RS-ConfigBeamformed-r14 ::=		SEQUENCE {

csi-RS-ConfigNZPIdListExt-r14			SEQUENCE (SIZE (1..7)) OF CSI-RS-ConfigNZPId-r13	OPTIONAL,	-- Need OR

csi-IM-ConfigIdList-r14					SEQUENCE (SIZE (1..8)) OF CSI-IM-ConfigId-r13	OPTIONAL,	-- Need OR

p-C-AndCBSR-PerResourceConfigList-r14	SEQUENCE (SIZE (1..8)) OF P-C-AndCBSR-Pair-r13	OPTIONAL,	-- Need OR

ace-For4Tx-PerResourceConfigList-r14	SEQUENCE (SIZE (1..7)) OF BOOLEAN	OPTIONAL,	-- Need OR

alternativeCodebookEnabledBeamformed-r14	ENUMERATED {true}	OPTIONAL,	-- Need OR

channelMeasRestriction-r14				ENUMERATED {on}			OPTIONAL,	-- Need OR

csi-RS-ConfigNZP-ApList-r14				SEQUENCE (SIZE (1..8)) OF CSI-RS-ConfigNZP-r11

OPTIONAL,	-- Need OR

nzp-ResourceConfigOriginal-v1430	CSI-RS-Config-NZP-v1430		OPTIONAL,	-- Need OR

csi-RS-NZP-Activation-r14				CSI-RS-ConfigNZP-Activation-r14	OPTIONAL		-- Need OR

}

CSI-RS-ConfigBeamformed-v1430::=		SEQUENCE {

csi-RS-ConfigNZP-ApList-r14				SEQUENCE (SIZE (1..8)) OF CSI-RS-ConfigNZP-r11

OPTIONAL,	-- Need OR

nzp-ResourceConfigOriginal-v1430	CSI-RS-Config-NZP-v1430		OPTIONAL,	-- Need OR

csi-RS-NZP-Activation-r14				CSI-RS-ConfigNZP-Activation-r14	OPTIONAL		-- Need OR

}

CSI-RS-Config-NZP-v1430::=		SEQUENCE {

transmissionComb-r14					NZP-TransmissionComb-r14	OPTIONAL,	-- Need OR

frequencyDensity-r14					NZP-FrequencyDensity-r14	OPTIONAL	-- Need OR

}

CSI-RS-ConfigNZP-Activation-r14::=		SEQUENCE {

csi-RS-NZP-mode-r14						ENUMERATED {semiPersistent, aperiodic},

activatedResources-r14					INTEGER (0..4)

}

-- ASN1STOP

CSI-RS-ConfigBeamformed field descriptions

ace-For4Tx-PerResourceConfigList The field indicates the alternativeCodeBookEnabledFor4TX-r12 per CSI-RS resource. E-UTRAN configures the field only if csi-RS-ConfigNZPIdListExt is configured.

activatedResources The number of activated CSI-RS resources, which concerns a subset of the aperiodic CSI-RS resources (for both semi-persistent and aperiodic mode). E-UTRAN configures at most the minimum between nMaxResource as configured by MIMO-UE-ParametersPerTM-r1430 and the number of resources as configured by csi-RS-ConfigNZP-ApList-r14.

alternativeCodebookEnabledBeamformed The field indicates whether code book in TS 36.213 [23], Table 7.2.4-18 to Table 7.2.4-20, is being used for deriving CSI feedback and reporting for a CSI process. E-UTRAN configures the field only for a process referring to a single RS configuration using non-zero power transmission (i.e a process for which csi-RS-ConfigNZPIdListExt is not configured). Field alternativeCodebookEnabledBeamformed corresponds to parameter alternativeCodebookEnabledCLASSB_K1 in TS 36.212 [22] and TS 36.213 [23].

csi-IM-ConfigIdList E-UTRAN configures the field csi-IM-ConfigIdList only if the IE is included in CSI-Process is configured (i.e. when TM10 is configured for the serving cell).

CSI-RS-ConfigBeamformed If csi-RS-ConfigNZPIdListExt-r13 is configured, E-UTRAN configures the same total number of entries for NZP, csi-IM-ConfigIdList-r13 and p-C-AndCBSR-PerResourceConfigList-r13.

csi-RS-ConfigNZP-ApList The field is used to configure NZP configurations for aperiodic or semi-persistent CSI RS reporting for which MAC controls activation. EUTRAN configures this field only when the UE is configured to use 2, 4 or and 8 ports CSI-RS, in which case EUTRAN configures the number of entries to be the same as the number of NZP resource configurations. For all these entries the UE shall ignore field subframeConfig. EUTRAN always configures this field together with csi-RS-NZP-Activation. Furthermore, for a given process, E-UTRAN does not simultaneously configure the periodic NZP configuration(s) and NZP CSI RS configurations for aperiodic or semi-persistent reporting.

csi-RS-ConfigNZP-EMIMO The field is used to configure NZP configurations additional to the one defined by the original NZP configuration as included in CSI-RS-Config/ CSI-Process when using 12 and 16 ports CSI-RS.

csi-RS-ConfigNZPIdListExt (in CSI-RS-ConfigBeamformed) Indicates the NZP configuration(s)in addition to the original NZP configuration, as defined by csi-RS-Config-r10 (TM9) or csi-RS-ConfigNZPId-r11 (TM10). I.e. extends the size of the NZP configuration list (originally a single entry i.e. list of size 1) using the general principles specified in 5.1.2.

p-C-AndCBSR-PerResourceConfigList E-UTRAN does not configure the field p-C-AndCBSR-PerResourceConfigList if the UE is configured with eMIMO-Type set to beamformed, alternativeCodebookEnabledBeamformed is set to FALSE and csi-RS-ConfigNZPIdListExt is not configured.

#### –	CSI-RS-ConfigEMIMO

The IE CSI-RS-ConfigEMIMO is used to specify the CSI (Channel-State Information) reference signal configuration for EBF/ FD-MIMO.

CSI-RS-ConfigEMIMO information elements

-- ASN1START

CSI-RS-ConfigEMIMO-r13 ::=	CHOICE {

release						NULL,

setup						CHOICE {

nonPrecoded-r13				CSI-RS-ConfigNonPrecoded-r13,

beamformed-r13				CSI-RS-ConfigBeamformed-r13

}

}

CSI-RS-ConfigEMIMO-v1430 ::=	CHOICE {

release						NULL,

setup						CHOICE {

nonPrecoded-v1430				CSI-RS-ConfigNonPrecoded-v1430,

beamformed-v1430				CSI-RS-ConfigBeamformed-v1430

}

}

CSI-RS-ConfigEMIMO-v1480 ::=	CHOICE {

release						NULL,

setup						CHOICE {

nonPrecoded-v1480				CSI-RS-ConfigNonPrecoded-v1480,

beamformed-v1480				CSI-RS-ConfigBeamformed-v1430

}

}

CSI-RS-ConfigEMIMO-v1530 ::=	CHOICE {

release						NULL,

setup						CHOICE {

nonPrecoded-v1530				CSI-RS-ConfigNonPrecoded-v1530

}

}

CSI-RS-ConfigEMIMO2-r14 ::=	CHOICE {

release						NULL,

setup						CSI-RS-ConfigBeamformed-r14

}

CSI-RS-ConfigEMIMO-Hybrid-r14 ::=	CHOICE {

release						NULL,

setup						SEQUENCE {

periodicityOffsetIndex-r14			INTEGER (0..1023)				OPTIONAL,	-- Need OR

eMIMO-Type2-r14						CSI-RS-ConfigEMIMO2-r14		OPTIONAL	-- Need ON

}

}

-- ASN1STOP

CSI-RS-ConfigEMIMO field descriptions

periodicityOffsetIndex This parameter is associated with the first EMIMO configuration of the hybrid eMIMO configuration.

#### –	CSI-RS-ConfigNonPrecoded

The IE CSI-RS-ConfigNonPrecoded is used to specify the non-precoded EBF/ FD-MIMO configuration.

-- ASN1START

CSI-RS-ConfigNonPrecoded-r13 ::=		SEQUENCE {

p-C-AndCBSRList-r13						P-C-AndCBSR-Pair-r13			OPTIONAL,	-- Need OR

codebookConfigN1-r13					ENUMERATED {n1, n2, n3, n4, n8},

codebookConfigN2-r13					ENUMERATED {n1, n2, n3, n4, n8},

codebookOverSamplingRateConfig-O1-r13	ENUMERATED {n4, n8}				OPTIONAL,	-- Need OR

codebookOverSamplingRateConfig-O2-r13	ENUMERATED {n4, n8}				OPTIONAL,	-- Need OR

codebookConfig-r13						INTEGER (1..4),

csi-IM-ConfigIdList-r13					SEQUENCE (SIZE (1..2)) OF CSI-IM-ConfigId-r13	OPTIONAL,	-- Need OR

csi-RS-ConfigNZP-EMIMO-r13				CSI-RS-ConfigNZP-EMIMO-r13		OPTIONAL	-- Need ON

}

CSI-RS-ConfigNonPrecoded-v1430::=		SEQUENCE {

csi-RS-ConfigNZP-EMIMO-v1430			CSI-RS-ConfigNZP-EMIMO-v1430	OPTIONAL,	-- Need ON

codebookConfigN1-v1430					ENUMERATED {n5, n6, n7, n10, n12, n14, n16},

codebookConfigN2-v1430					ENUMERATED {n5, n6, n7},

nzp-ResourceConfigTM9-Original-v1430	CSI-RS-Config-NZP-v1430

}

CSI-RS-ConfigNonPrecoded-v1480::=		SEQUENCE {

csi-RS-ConfigNZP-EMIMO-v1480			CSI-RS-ConfigNZP-EMIMO-v1430	OPTIONAL,	-- Need ON

codebookConfigN1-v1480					ENUMERATED {n5, n6, n7, n10, n12, n14, n16}		OPTIONAL,	-- Need OR

codebookConfigN2-r1480					ENUMERATED {n5, n6, n7}			OPTIONAL,	-- Need OR

nzp-ResourceConfigTM9-Original-v1480	CSI-RS-Config-NZP-v1430

}

CSI-RS-ConfigNonPrecoded-v1530 ::=			SEQUENCE {

p-C-AndCBSRList-r15						P-C-AndCBSR-Pair-r15			OPTIONAL	-- Need OR

}

-- ASN1STOP

CSI-RS-ConfigNonPrecoded field descriptions

codebookConfig Indicates a sub-set of the codebook entry, see TS 36.213 [23].

codebookConfigNx Indicates the number of antenna ports per polarization in dimension x as used for transmission of CSI reference signals. Value n1 corresponds to 1, value n2 corresponds to 2 and so on, see TS 36.213 [23]. E-UTRAN configures the field in accordance with the restrictions as specified in TS 36.213 [23]. If codebookConfigNx in CSI-RS-ConfigNonPrecoded-v1480 is configured, the UE shall ignore the field codebookConfigNx in CSI-RS-ConfigNonPrecoded-r13.

codebookOverSamplingRateConfig-Ox Indicates the spatial over-sampling rate in dimension x as used for transmission of CSI reference signals. Value n4 corresponds to 4 and value n8 corresponds to 8, see TS 36.213 [23].

csi-IM-ConfigId(List) E-UTRAN configures the field csi-IM-ConfigIdList only if the IE is included in CSI-Process is configured (i.e. when TM10 is configured for the serving cell).

csi-RS-ConfigNZP-EMIMO The field is used to configure NZP configurations additional to the one defined by the original NZP configuration as included in CSI-RS-Config/ CSI-Process when using more than 8 ports CSI-RS as defined in TS 36.211 [21], table 6.10.5-1.

#### –	CSI-RS-ConfigNZP

The IE CSI-RS-ConfigNZP is the CSI-RS resource configuration using non-zero power transmission that E-UTRAN may configure on a serving frequency.

CSI-RS-ConfigNZP information elements

-- ASN1START

CSI-RS-ConfigNZP-r11 ::=		SEQUENCE {

csi-RS-ConfigNZPId-r11			CSI-RS-ConfigNZPId-r11,

antennaPortsCount-r11			ENUMERATED {an1, an2, an4, an8},

resourceConfig-r11				INTEGER (0..31),

subframeConfig-r11				INTEGER (0..154),

scramblingIdentity-r11			INTEGER (0..503),

qcl-CRS-Info-r11				SEQUENCE {

qcl-ScramblingIdentity-r11		INTEGER (0..503),

crs-PortsCount-r11				ENUMERATED {n1, n2, n4, spare1},

mbsfn-SubframeConfigList-r11	CHOICE {

release						NULL,

setup						SEQUENCE {

subframeConfigList			MBSFN-SubframeConfigList

}

}																OPTIONAL	-- Need ON

}																	OPTIONAL,	-- Need OR

...,

[[	csi-RS-ConfigNZPId-v1310		CSI-RS-ConfigNZPId-v1310		OPTIONAL	-- Need ON

]],

[[	transmissionComb-r14			NZP-TransmissionComb-r14		OPTIONAL,	-- Need OR

frequencyDensity-r14			NZP-FrequencyDensity-r14		OPTIONAL	-- Need OR

]],

[[	mbsfn-SubframeConfigList-v1430	CHOICE {

release						NULL,

setup						SEQUENCE {

subframeConfigList-v1430	MBSFN-SubframeConfigList-v1430

}

}																OPTIONAL	-- Need OP

]]

}

CSI-RS-ConfigNZP-EMIMO-r13 ::=	CHOICE {

release						NULL,

setup						SEQUENCE {

nzp-resourceConfigList-r13		SEQUENCE (SIZE (1..2)) OF NZP-ResourceConfig-r13,

cdmType-r13						ENUMERATED {cdm2, cdm4}	OPTIONAL	-- Need OR

}

}

CSI-RS-ConfigNZP-EMIMO-v1430 ::=	SEQUENCE {

-- All extensions are for Non-Precoded so could be grouped by setup/ release choice

nzp-resourceConfigListExt-r14	SEQUENCE (SIZE (0..4)) OF NZP-ResourceConfig-r13,

cdmType-v1430					ENUMERATED {cdm8 }			OPTIONAL	-- Need OR

}

NZP-ResourceConfig-r13 ::=	SEQUENCE {

resourceConfig-r13				ResourceConfig-r13,

...,

[[	transmissionComb-r14		NZP-TransmissionComb-r14		OPTIONAL,	-- Need OR

frequencyDensity-r14		NZP-FrequencyDensity-r14		OPTIONAL	-- Need OR

]]

}

ResourceConfig-r13 ::=				INTEGER (0..31)

NZP-TransmissionComb-r14 ::=			INTEGER (0..2)

NZP-FrequencyDensity-r14 ::=			ENUMERATED {d1, d2, d3}

-- ASN1STOP

CSI-RS-ConfigNZP field descriptions

antennaPortsCount Parameter represents the number of antenna ports used for transmission of CSI reference signals where an1 corresponds to 1, an2 to 2 antenna ports etc. see TS 36.211 [21], clause 6.10.5.

cdmType Parameter: CDMType, see TS 36.211 [21], clause 6.10.5.2.

csi-RS-ConfigNZPId Refers to a CSI RS configuration using non-zero power transmission that is configured for the same frequency as the CSI process. UE shall ignore CSI-RS-ConfigNZPId-r11 if CSI-RS-ConfigNZPId-v1310 is signalled.

frequencyDensity Indicates the freqency-domain density reduction. E-UTRAN configures the values in accordance with the restrictions specified in TS 36.213 [23].

mbsfn-SubframeConfigList Indicates the MBSFN configuration for the CSI-RS resources. If qcl-CRS-Info-r11 is absent, the field is released.

nzp-resourceConfigList Indicate a list of non-zero power transmission CSI-RS resources using parameter resourceConfig.

qcl-CRS-Info Indicates CRS antenna ports that is quasi co-located with the CSI-RS antenna ports, see TS 36.213 [23], clause 7.2.5. EUTRAN configures this field if and only if the UE is configured with qcl-Operation set to typeB.

resourceConfig Parameter: CSI reference signal configuration, see TS 36.211 [21], table 6.10.5.2-1 and 6.10.5.2-2.

subframeConfig Parameter: , see TS 36.211 [21], table 6.10.5.3-1.

scramblingIdentity Parameter: Pseudo-random sequence generator parameter, , see TS 36.213 [23], clause 7.2.5.

transmissionComb Indicates the transmission combining offset. E-UTRAN configures the values in accordance with the restrictions specified in TS 36.213 [23].

#### –	CSI-RS-ConfigNZPId

The IE CSI-RS-ConfigNZPId is used to identify a CSI-RS resource configuration using non-zero transmission power, as configured by the IE CSI-RS-ConfigNZP. The identity is unique within the scope of a carrier frequency.

CSI-RS-ConfigNZPId information elements

-- ASN1START

CSI-RS-ConfigNZPId-r11 ::=					INTEGER (1..maxCSI-RS-NZP-r11)

CSI-RS-ConfigNZPId-v1310 ::=				INTEGER (minCSI-RS-NZP-r13..maxCSI-RS-NZP-r13)

CSI-RS-ConfigNZPId-r13 ::=					INTEGER (1..maxCSI-RS-NZP-r13)

-- ASN1STOP

#### –	CSI-RS-ConfigZP

The IE CSI-RS-ConfigZP is the CSI-RS resource configuration, for which UE assumes zero transmission power, that E-UTRAN may configure on a serving frequency.

CSI-RS-ConfigZP information elements

-- ASN1START

CSI-RS-ConfigZP-r11 ::=		SEQUENCE {

csi-RS-ConfigZPId-r11		CSI-RS-ConfigZPId-r11,

resourceConfigList-r11		BIT STRING (SIZE (16)),

subframeConfig-r11			INTEGER (0..154),

...

}

CSI-RS-ConfigZP-ApList-r14 ::=	CHOICE {

release							NULL,

setup							SEQUENCE (SIZE (1.. maxCSI-RS-ZP-r11)) OF CSI-RS-ConfigZP-r11

}

-- ASN1STOP

CSI-RS-ConfigZP field descriptions

CSI-RS-ConfigZP-ApList Indicates the aperiodic zero power CSI-RS present in a given subframe. See 36.213 [23], Table 7.1.9-2. First entry in the list corresponds to aperiodic trigger 00, second entry in the list corresponds to aperiodic trigger 01 and so on.

resourceConfigList Parameter: ZeroPowerCSI-RS, see TS 36.213 [23], clause 7.2.7.

subframeConfig Parameter: , see TS 36.211 [21], table 6.10.5.3-1.

#### –	CSI-RS-ConfigZPId

The IE CSI-RS-ConfigZPId is used to identify a CSI-RS resource configuration for which UE assumes zero transmission power, as configured by the IE CSI-RS-ConfigZP. The identity is unique within the scope of a carrier frequency.

CSI-RS-ConfigZPId information elements

-- ASN1START

CSI-RS-ConfigZPId-r11 ::=					INTEGER (1..maxCSI-RS-ZP-r11)

-- ASN1STOP

#### –	DataInactivityTimer

The IE DataInactivityTimer is used to control Data inactivity operation. Corresponds to the timer for data inactivity monitoring in TS 36.321 [6]. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on.

DataInactivityTimer information element

-- ASN1START

DataInactivityTimer-r14 ::=				ENUMERATED {

s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60,

s80, s100, s120, s150, s180}

-- ASN1STOP

#### –	DMRS-Config

The IE DMRS-Config is the DMRS configuration that E-UTRAN may configure on a serving frequency.

DMRS-Config information elements

-- ASN1START

DMRS-Config-r11 ::=		CHOICE {

release						NULL,

setup						SEQUENCE {

scramblingIdentity-r11		INTEGER (0..503),

scramblingIdentity2-r11		INTEGER (0..503)

}

}

DMRS-Config-v1310 ::=			SEQUENCE {

dmrs-tableAlt-r13				ENUMERATED {true}			OPTIONAL	-- Need OR

}

-- ASN1STOP

DMRS-Config field descriptions

scramblingIdentity, scramblingIdentity2 Parameter: , see TS 36.211 [21], clause 6.10.3.1.

dmrs-tableAlt The field indicates whether to use an alternative table for DMRS upon PDSCH transmission, see TS 36.213 [23].

#### –	DRB-Identity

The IE DRB-Identity is used to identify a DRB used by a UE.

DRB-Identity information elements

-- ASN1START

DRB-Identity ::=					INTEGER (1..32)

-- ASN1STOP

#### –	EPDCCH-Config

The IE EPDCCH-Config specifies the subframes and resource blocks for EPDCCH monitoring that E-UTRAN may configure for a serving cell.

EPDCCH-Config information element

-- ASN1START

EPDCCH-Config-r11 ::=		SEQUENCE{

config-r11		CHOICE {

release						NULL,

setup						SEQUENCE {

subframePatternConfig-r11	CHOICE {

release						NULL,

setup						SEQUENCE {

subframePattern-r11			MeasSubframePattern-r10

}

}																OPTIONAL, -- Need ON

startSymbol-r11				INTEGER (1..4)						OPTIONAL, -- Need OP

setConfigToReleaseList-r11	EPDCCH-SetConfigToReleaseList-r11	OPTIONAL, -- Need ON

setConfigToAddModList-r11	EPDCCH-SetConfigToAddModList-r11	OPTIONAL -- Need ON

}

}

}

EPDCCH-SetConfigToAddModList-r11 ::= SEQUENCE (SIZE(1..maxEPDCCH-Set-r11)) OF EPDCCH-SetConfig-r11

EPDCCH-SetConfigToReleaseList-r11 ::= SEQUENCE (SIZE(1..maxEPDCCH-Set-r11)) OF EPDCCH-SetConfigId-r11

EPDCCH-SetConfig-r11 ::=		SEQUENCE {

setConfigId-r11					EPDCCH-SetConfigId-r11,

transmissionType-r11			ENUMERATED {localised, distributed},

resourceBlockAssignment-r11		SEQUENCE{

numberPRB-Pairs-r11				ENUMERATED {n2, n4, n8},

resourceBlockAssignment-r11		BIT STRING (SIZE(4..38))

},

dmrs-ScramblingSequenceInt-r11	INTEGER (0..503),

pucch-ResourceStartOffset-r11	INTEGER (0..2047),

re-MappingQCL-ConfigId-r11		PDSCH-RE-MappingQCL-ConfigId-r11	OPTIONAL, -- Need OR

...,

[[	csi-RS-ConfigZPId2-r12		CHOICE {

release							NULL,

setup							CSI-RS-ConfigZPId-r11

}															OPTIONAL	-- Need ON

]],

[[	numberPRB-Pairs-v1310			CHOICE {

release							NULL,

setup							ENUMERATED {n6}

}															OPTIONAL,	-- Need ON

mpdcch-config-r13				CHOICE {

release							NULL,

setup							SEQUENCE {

csi-NumRepetitionCE-r13			ENUMERATED {sf1, sf2, sf4, sf8, sf16, sf32},

mpdcch-pdsch-HoppingConfig-r13	ENUMERATED {on,off},

mpdcch-StartSF-UESS-r13			CHOICE {

fdd-r13							ENUMERATED {v1, v1dot5, v2, v2dot5, v4,

v5, v8, v10},

tdd-r13							ENUMERATED {v1, v2, v4, v5, v8, v10,

v20, spare1}

},

mpdcch-NumRepetition-r13		ENUMERATED {r1, r2, r4, r8, r16,

r32, r64, r128, r256},

mpdcch-Narrowband-r13			INTEGER (1.. maxAvailNarrowBands-r13)

}

}															OPTIONAL	-- Need ON

]]

}

EPDCCH-SetConfigId-r11 ::=	INTEGER (0..1)

-- ASN1STOP

EPDCCH-Config field descriptions

csi-NumRepetitionCE Number of subframes for CSI reference resource, see TS 36.213 [23]. Value sf1 corresponds to 1 subframe, sf2 corresponds to 2 subframes and so on.

csi-RS-ConfigZPId2 Indicates the rate matching parameters in addition to those indicated by re-MappingQCL-ConfigId. E-UTRAN configures this field only when tm10 is configured.

dmrs-ScramblingSequenceInt The DMRS scrambling sequence initialization parameter or defined in TS 36.211 [21], clause 6.10.3A.1.

EPDCCH-SetConfig Provides EPDCCH configuration set. See TS 36.213 [23], clause 9.1.4. E-UTRAN configures at least one EPDCCH-SetConfig when EPDCCH-Config is configured. For BL UEs or UEs in CE, EUTRAN does not configure more than one EPDCCH-SetConfig.

mpdcch-Narrowband Parameter: , see TS 36.211 [21], clause 6.8B.5. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..maxAvailNarrowBands-r13-1) as specified in TS 36.211 [21].

mpdcch-NumRepetition Maximum numbers of repetitions for UE-SS for MPDCCH, see TS 36.213 [23].

mpdcch-pdsch-HoppingConfig Frequency hopping activation/deactivation for unicast MPDCCH/PDSCH, see TS 36.211 [21]. E-UTRAN does not configure the value on if freqHoppingParametersDL is not present in SystemInformationBlockType1.

mpdcch-StartSF-UESS Starting subframe configuration for an MPDCCH UE-specific search space, see TS 36.213 [23]. Value v1 corresponds to 1, value v1dot5 corresponds to 1.5, and so on.

numberPRB-Pairs Indicates the number of physical resource-block pairs used for the EPDCCH set. Value n2 corresponds to 2 physical resource-block pairs; n4 corresponds to 4 physical resource-block pairs and so on. Value n8 is not supported if dl-Bandwidth is set to 6 resource blocks. EUTRAN only configures values up to n6 for BL UEs or UEs in CE. Value n6 is only applicable to BL UEs or UEs in CE.

pucch-ResourceStartOffset PUCCH format 1a, 1b and 3 resource starting offset for the EPDCCH set. See TS 36.213 [23], clause 10.1.

re-MappingQCL-ConfigId Indicates the starting OFDM symbol, the related rate matching parameters and quasi co-location assumption for EPDCCH when the UE is configured with tm10. This field provides the identity of a configured PDSCH-RE-MappingQCL-Config. E-UTRAN configures this field only when tm10 is configured.

resourceBlockAssignment Indicates the index to a specific combination of physical resource-block pair for EPDCCH set. See TS 36.213 [23], clause 9.1.4.4. The size of resourceBlockAssignment is specified in TS 36.213 [23], clause 9.1.4.4, and based on numberPRB-Pairs and the signalled value of dl-Bandwidth. If numberPRB-Pairs-v1310 field is present, the total number of physical resource-block pairs is 6 and it is composed of one subset of 2 physical resource-block pairs and another subset of 4 physical resource-block pairs, and the resourceBlockAssignment field defines the subset of 2 physical resource-block pairs.

setConfigId Indicates the identity of the EPDCCH configuration set.

startSymbol Indicates the OFDM starting symbol for any EPDCCH and PDSCH scheduled by EPDCCH on the same cell, see TS 36.213 [23], clause 9.1.4.1. If not present, the UE shall release the configuration and shall derive the starting OFDM symbol of EPDCCH and PDSCH scheduled by EPDCCH from PCFICH. Values 1, 2, and 3 are applicable for dl-Bandwidth greater than 10 resource blocks. Values 2, 3, and 4 are applicable otherwise. E-UTRAN does not configure the field for UEs configured with tm10.

subframePatternConfig Configures the subframes which the UE shall monitor the UE-specific search space on EPDCCH, except for pre-defined rules in TS 36.213 [23], clause 9.1.4. If the field is not configured when EPDCCH is configured, the UE shall monitor the UE-specific search space on EPDCCH in all subframes except for pre-defined rules in TS 36.213 [23], clause 9.1.4.

transmissionType Indicates whether distributed or localized EPDCCH transmission mode is used as defined in TS 36.211 [21], clause 6.8A.1.

#### –	EIMTA-MainConfig

The IE EIMTA-MainConfig is used to specify the eIMTA-RNTI used for eIMTA and the subframes used for monitoring PDCCH with eIMTA-RNTI. The IE EIMTA-MainConfigServCell is used to specify the eIMTA related parameters applicable for the concerned serving cell.

EIMTA-MainConfig information element

-- ASN1START

EIMTA-MainConfig-r12 ::=	CHOICE {

release							NULL,

setup							SEQUENCE {

eimta-RNTI-r12				C-RNTI,

eimta-CommandPeriodicity-r12	ENUMERATED {sf10, sf20, sf40, sf80},

eimta-CommandSubframeSet-r12	BIT STRING (SIZE(10))

}

}

EIMTA-MainConfigServCell-r12 ::=	CHOICE {

release								NULL,

setup								SEQUENCE {

eimta-UL-DL-ConfigIndex-r12			INTEGER (1..5),

eimta-HARQ-ReferenceConfig-r12		ENUMERATED {sa2, sa4, sa5},

mbsfn-SubframeConfigList-v1250		CHOICE {

release								NULL,

setup								SEQUENCE {

subframeConfigList-r12				MBSFN-SubframeConfigList

}

}

}

}

-- ASN1STOP

EIMTA-MainConfig field descriptions

eimta-CommandPeriodicity Configures the periodicity to monitor PDCCH with eIMTA-RNTI, see TS 36.213 [23], clause 13.1. Value sf10 corresponds to 10 subframes, sf20 corresponds to 20 subframes and so on.

eimta-CommandSubframeSet Configures the subframe(s) to monitor PDCCH with eIMTA-RNTI within the periodicity configured by eimta-CommandPeriodicity. The 10 bits correspond to all subframes in the last radio frame within each periodicity. The left most bit is for subframe 0 and so on. Each bit can be of value 0 or 1. The value of 1 means that the corresponding subframe is configured for monitoring PDCCH with eIMTA-RNTI, and the value of 0 means otherwise. In case of TDD as PCell, only the downlink and the special subframes indicated by the UL/ DL configuration in SIB1 can be configured for monitoring PDCCH with eIMTA-RNTI. In case of FDD as PCell, any of the ten subframes can be configured for monitoring PDCCH with eIMTA-RNTI.

eimta-HARQ-ReferenceConfig Indicates UL/ DL configuration used as the DL HARQ reference configuration for this serving cell. Value sa2 corresponds to Configuration2, sa4 to Configuration4 etc, as specified in TS 36.211 [21], table 4.2-2. E-UTRAN configures the same value for all serving cells residing on same frequency band.

eimta-UL-DL-ConfigIndex Index of I, see TS 36.212 [22], clause 5.3.3.1.4. E-UTRAN configures the same value for all serving cells residing on same frequency band.

mbsfn-SubframeConfigList Configure the MBSFN subframes for the UE on this serving cell. An uplink subframe indicated by the DL/UL subframe configuration in SIB1 can be configured as MBSFN subframe.

#### –	GWUS-Config

The IE GWUS-Config is used to specify the Group WUS configuration. For the UEs supporting GWUS, E-UTRAN uses GWUS to indicate that the UE shall attempt to receive paging in that cell, see TS 36.304 [4].

GWUS-Config information element

-- ASN1START

GWUS-Config-r16 ::=				SEQUENCE {

groupAlternation-r16			ENUMERATED {true}				OPTIONAL,	-- Need OR

commonSequence-r16				ENUMERATED {g0, g126}			OPTIONAL,	-- Need OR

timeParameters-r16				GWUS-TimeParameters-r16			OPTIONAL,	-- Cond NoWUSr15

resourceConfigDRX-r16			GWUS-ResourceConfig-r16,

resourceConfig-eDRX-Short-r16	GWUS-ResourceConfig-r16			OPTIONAL,	-- Need OP

resourceConfig-eDRX-Long-r16	GWUS-ResourceConfig-r16			OPTIONAL,	-- Cond TimeOffset

probThreshList-r16				GWUS-ProbThreshList-r16		OPTIONAL, 	-- Cond ProbabilityBased

groupNarrowBandList-r16			GWUS-GroupNarrowBandList-r16	OPTIONAL -- Need OR

}

GWUS-TimeParameters-r16 ::=		SEQUENCE {

maxDurationFactor-r16			ENUMERATED {one32th, one16th, one8th, one4th},

numPOs-r16						ENUMERATED {n1, n2, n4, spare1}			DEFAULT n1,

timeOffsetDRX-r16				ENUMERATED {ms40, ms80, ms160, ms240},

timeOffset-eDRX-Short-r16		ENUMERATED {ms40, ms80, ms160, ms240},

timeOffset-eDRX-Long-r16		ENUMERATED {ms1000, ms2000}				OPTIONAL,	-- Need OP

numDRX-CyclesRelaxed-r16		ENUMERATED {n1, n2, n4, n8}				OPTIONAL,	-- Need OR

powerBoost-r16					ENUMERATED {dB0, dB1dot8, dB3, dB4dot8}	OPTIONAL,	-- Need OR

...

}

GWUS-ResourceConfig-r16 ::=		SEQUENCE {

resourceMappingPattern-r16		CHOICE {

resourceLocationWithWUS			ENUMERATED {primary, secondary, primary3FDM},

resourceLocationWithoutWUS		ENUMERATED {n0, n2}

},

numGroupsList-r16				GWUS-NumGroupsList-r16			OPTIONAL,	-- Need OP

groupsForServiceList-r16		GWUS-GroupsForServiceList-r16	OPTIONAL	-- Cond ProbabilityBased

}

GWUS-GroupsForServiceList-r16 ::=	SEQUENCE (SIZE (1..maxGWUS-ProbThresholds-r16)) OF INTEGER (1..maxGWUS-Groups-1-r16)

GWUS-GroupNarrowBandList-r16 ::=	SEQUENCE (SIZE (1..maxAvailNarrowBands-r13)) OF BOOLEAN

GWUS-NumGroupsList-r16 ::= 		SEQUENCE (SIZE (1..maxGWUS-Resources-r16)) OF GWUS-NumGroups-r16

GWUS-ProbThreshList-r16 ::=		SEQUENCE (SIZE (1..maxGWUS-ProbThresholds-r16)) OF GWUS-PagingProbThresh-r16

GWUS-NumGroups-r16 ::=			ENUMERATED {n1, n2, n4, n8}

GWUS-PagingProbThresh-r16 ::=	ENUMERATED {p20, p30, p40, p50, p60, p70, p80, p90}

-- ASN1STOP

GWUS-Config field descriptions

commonSequence Presence of the field indicates common WUS sequence is configured. Value g0 indicates common WUS sequence for the shared WUS resource corresponds to g = 0, and value g126 indicates common WUS sequence for the shared WUS resource corresponds to g = 126, see TS 36.211 [21].

groupAlternation Presence of the field enables WUS group alternation between the two or more WUS resources for the gap type, see TS 36.304 [4].

groupNarrowBandList List indicating which paging narrowbands support group WUS see TS 36.304 [4]. First entry in the list indicates WUS support for first paging narrowband, second entry in the list indicates WUS support for second paging narrowband, and so on. If E-UTRAN includes groupNarrowBandList, the number of entries is equal to the value of paging-narrowBands. If this list is absent, group WUS is supported on all paging narrowbands. E-UTRAN does not configure this field when RRC_INACTIVE is used in the cell.

groupsForServiceList Number of WUS groups for each paging probability group see TS 36.304 [4]. The first entry corresponds to the first probability group, the second entry corresponds to the second paging probability group, and so on. Total number of WUS groups in this list cannot be more than the total number of WUS groups in numGroupsList. If E-UTRAN includes groupsForServiceList, it includes the same number of entries and listed in the same order as in probThreshList.

numGroupsList List of WUS groups for each WUS resource see TS 36.304 [4]. First entry corresponds to the first resource, second entry corresponds to the second resource, and so on. numGroupsList is mandatory present in resourceConfigDRX. If numGroupsList is not present in resourceConfig-eDRX-Short, parameter for DRX WUS resource applies for short eDRX WUS resource. If numGroupsList is not present in resourceConfig-eDRX-Long, parameter for short eDRX WUS resource applies for long eDRX WUS resource.

probThreshList Paging probability thresholds corresponding to the paging probability groups, see TS 36.304 [4]. Value p20 corresponds to 20%, value p30 corresponds to 30%, and so on.

resourceConfigDRX, resourceConfig-eDRX-Short, resourceConfig-eDRX-Long WUS resource configured for each gap type see TS 36.304 [4]. If resourceConfig-eDRX-Short is not present, DRX WUS parameters apply for short eDRX WUS resource. If resourceConfig-eDRX-Long is not present, short eDRX WUS parameters apply for long eDRX WUS resource.

resourceMappingPattern Identifies the WUS resource mapping to time/frequency as defined in TS 36.304 [4]. If wus-Config-r15 is present in SystemInformationBlockType2, the field is set to value resourceLocationWithWUS; otherwise the field is set to value resourceLocationWithoutWUS.

timeParameters Time domain WUS configuration information. For individual field descriptions, see WUS-Config. If the field is absent, the parameters in wus-Config apply.

| Conditional presence | Explanation |
| --- | --- |
| NoWUSr15 | The field is mandatory present if wus-Config-r15 is not present in SystemInformationBlockType2; otherwise the field is not present. |
| ProbabilityBased | The field is mandatory present if paging probability based WUS group selection is configured; otherwise the field is not present and the UE shall delete any existing value for this field. |
| TimeOffset | The field is optionally present, Need OP, if timeOffset-eDRX-Long is present in timeParameters; otherwise the field is not present, and the UE shall delete any existing value for this field. |

#### –	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

LogicalChannelConfig ::=			SEQUENCE {

ul-SpecificParameters				SEQUENCE {

priority							INTEGER (1..16),

prioritisedBitRate					ENUMERATED {

kBps0, kBps8, kBps16, kBps32, kBps64, kBps128,

kBps256, infinity, kBps512-v1020, kBps1024-v1020,

kBps2048-v1020, spare5, spare4, spare3, spare2,

spare1},

bucketSizeDuration					ENUMERATED {

ms50, ms100, ms150, ms300, ms500, ms1000, spare2,

spare1},

logicalChannelGroup					INTEGER (0..3)			OPTIONAL			-- Need OR

}		OPTIONAL,																	-- Cond UL

...,

[[	logicalChannelSR-Mask-r9			ENUMERATED {setup}		OPTIONAL		-- Cond SRmask

]],

[[	logicalChannelSR-Prohibit-r12		BOOLEAN					OPTIONAL		-- Need ON

]],

[[	laa-UL-Allowed-r14					BOOLEAN					OPTIONAL,		-- Need ON

bitRateQueryProhibitTimer-r14	ENUMERATED {

s0, s0dot4, s0dot8, s1dot6, s3, s6, s12,

s30}				OPTIONAL		--Need OR

]],

[[	allowedTTI-Lengths-r15		CHOICE	{

release			NULL,

setup			SEQUENCE {

shortTTI-r15		BOOLEAN,

subframeTTI-r15		BOOLEAN

}

}										OPTIONAL,								-- Need ON

logicalChannelSR-Restriction-r15 CHOICE	{

release			NULL,

setup			ENUMERATED {spucch, pucch}

}										OPTIONAL,								-- Need ON

channelAccessPriority-r15			CHOICE {

release									NULL,

setup									INTEGER (1..4)

}										OPTIONAL,		-- Need ON

lch-CellRestriction-r15				BIT STRING (SIZE (maxServCell-r13)) OPTIONAL -- Need ON

]],

[[

bitRateMultiplier-r16		ENUMERATED {x40, x70, x100, x200}	OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

LogicalChannelConfig field descriptions

allowedTTI-Lengths Indicates the allowed TTI lengths for the logical channel. If not configured, the UE is allowed to transmit the logical channel using any TTI length.

bitRateMultiplier Bit rate multiplier for recommended bit rate MAC CE as specified in TS 36.321 [6]. Value x40 indicates bit rate multiplier 40, value x70 indicates bit rate multiplier 70 and so on.

bitRateQueryProhibitTimer The timer is used for bit rate recommendation query in TS 36.321 [6], clause 5.18, in seconds. Value s0 means 0s, s0dot4 means 0.4s and so on.

bucketSizeDuration Bucket Size Duration for logical channel prioritization in TS 36.321 [6]. Value in milliseconds. Value ms50 corresponds to 50 ms, ms100 corresponds to 100 ms and so on.

channelAccessPriority Indicates the channel access priority class for the logical channel. UE shall select the lowest channel access priority class (i.e. highest signalled value) of the logical channel with MAC SDU multiplexed into the MAC PDU. MAC CEs except padding BSR apply the highest channel access priority class (i.e. lowest signalled value) , as defined in TS 36.300 [9].

laa-UL-Allowed Indicates whether the data of a logical channel is allowed to be transmitted via UL of LAA SCells. Value TRUE indicates that the logical channel is allowed to be sent via UL of LAA SCells. Value FALSE indicates that the logical channel is not allowed to be sent via UL of LAA SCells.

lch-CellRestriction Indicates cells which are restricted for the logical channel, The bit is set to 1 if the cell is restricted and to 0 if the cell is not restricted, for each cell. The least significant bit corresponds to the serving cell with index 0, the next bit corresponds to the serving cell with index 1, and so on. If the cell is restricted for the logical channel, then data for the logical channel is not allowed to be sent using that cell. If the field is not included, no cells are restricted. See also TS 36.321 [6], clause 5.4.3.1. The restriction is only active when PDCP duplication using CA is activated.

logicalChannelGroup Mapping of logical channel to logical channel group for BSR reporting in TS 36.321 [6].

logicalChannelSR-Mask Controlling SR triggering on a logical channel basis when an uplink grant is configured. See TS 36.321 [6].

logicalChannelSR-Prohibit Value TRUE indicates that the logicalChannelSR-ProhibitTimer is enabled for the logical channel. E-UTRAN only (optionally) configures the field (i.e. indicates value TRUE) if logicalChannelSR-ProhibitTimer is configured. See TS 36.321 [6].

logicalChannelSR-Restriction Defines the restricted SR configuration for the logical channel. Value spucch indicates that the SR cannot be sent on SPUCCH and value pucch indicates that the SR cannot be sent on PUCCH. If not configured, the UE is allowed to transmit the SR on any SR resource.

prioritisedBitRate Prioritized Bit Rate for logical channel prioritization in TS 36.321 [6]. Value in kilobytes/second. Value kBps0 corresponds to 0 kB/second, kBps8 corresponds to 8 kB/second, kBps16 corresponds to 16 kB/second and so on. Infinity is the only applicable value for SRB1 and SRB2

priority Logical channel priority in TS 36.321 [6]. Value is an integer.

shortTTI, subframeTTI For short TTIs and subframe TTIs respectively: Value TRUE indicates that the UE is allowed to transmit using this TTI length for the logical channel and the value FALSE indicates that the UE is not allowed to transmit using this TTI length for the logical channel. If not configured for a TTI length, then the UE is allowed to transmit this logical channel using this TTI length.

| Conditional presence | Explanation |
| --- | --- |
| SRmask | The field is optionally present if ul-SpecificParameters is present, need OR; otherwise it is not present. |
| UL | The field is mandatory present for UL logical channels; otherwise it is not present. |

#### –	LWA-Configuration

The IE LWA-Configuration is used to setup/modify/release LTE-WLAN Aggregation.

-- ASN1START

LWA-Configuration-r13 ::=		CHOICE {

release							NULL,

setup							SEQUENCE {

lwa-Config-r13					LWA-Config-r13

}

}

LWA-Config-r13 ::=	SEQUENCE {

lwa-MobilityConfig-r13			WLAN-MobilityConfig-r13		OPTIONAL,	-- Need ON

lwa-WT-Counter-r13				INTEGER (0..65535)			OPTIONAL,	-- Need ON

...,

[[	wt-MAC-Address-r14		OCTET STRING (SIZE (6))	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

LWA-Configuration field descriptions

lwa-MobilityConfig Indicates the parameters used for WLAN mobility.

lwa-WT-Counter Indicates the parameter used by UE for WLAN authentication.

wt-MAC-Address Indicates the WT MAC address of the WT handling the LWA operation for the UE. The UE uses this MAC address in uplink transmissions to enable routing of LWA uplink data from the AP to the WT. E-UTRAN configures the field only if ul-LWA-Config-r14 is configured for at least one LWA bearer.

#### –	LWIP-Configuration

The IE LWIP-Configuration is used to add, modify or release DRBs that are using LWIP Tunnel.

-- ASN1START

LWIP-Configuration-r13 ::=		CHOICE {

release							NULL,

setup							SEQUENCE {

lwip-Config-r13					LWIP-Config-r13

}

}

LWIP-Config-r13 ::=	SEQUENCE {

lwip-MobilityConfig-r13			WLAN-MobilityConfig-r13		OPTIONAL,	-- Need ON

tunnelConfigLWIP-r13			TunnelConfigLWIP-r13		OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

LWIP-Configuration field descriptions

lwip-MobilityConfig Indicates the WLAN mobility set for LWIP.

tunnelConfigLWIP Indicates the parameters used for establishing the LWIP tunnel.

#### –	MAC-MainConfig

The IE MAC-MainConfig is used to specify the MAC main configuration for signalling and data radio bearers. All MAC main configuration parameters can be configured independently per Cell Group (i.e. MCG or SCG), unless explicitly specified otherwise.

MAC-MainConfig information element

-- ASN1START

MAC-MainConfig ::=					SEQUENCE {

ul-SCH-Config						SEQUENCE {

maxHARQ-Tx							ENUMERATED {

n1, n2, n3, n4, n5, n6, n7, n8,

n10, n12, n16, n20, n24, n28,

spare2, spare1}		OPTIONAL,	-- Need ON

periodicBSR-Timer					PeriodicBSR-Timer-r12	OPTIONAL,	-- Need ON

retxBSR-Timer						RetxBSR-Timer-r12,

ttiBundling							BOOLEAN

}																OPTIONAL,	-- Need ON

drx-Config							DRX-Config					OPTIONAL,	-- Need ON

timeAlignmentTimerDedicated			TimeAlignmentTimer,

phr-Config							CHOICE {

release								NULL,

setup								SEQUENCE {

periodicPHR-Timer					ENUMERATED {sf10, sf20, sf50, sf100, sf200,

sf500, sf1000, infinity},

prohibitPHR-Timer					ENUMERATED {sf0, sf10, sf20, sf50, sf100,

sf200, sf500, sf1000},

dl-PathlossChange					ENUMERATED {dB1, dB3, dB6, infinity}

}

}																OPTIONAL,	-- Need ON

...,

[[	sr-ProhibitTimer-r9					INTEGER (0..7)			OPTIONAL	-- Need ON

]],

[[	mac-MainConfig-v1020				SEQUENCE {

sCellDeactivationTimer-r10			ENUMERATED {

rf2, rf4, rf8, rf16, rf32, rf64, rf128,

spare}			OPTIONAL,	-- Need OP

extendedBSR-Sizes-r10				ENUMERATED {setup}		OPTIONAL,	-- Need OR

extendedPHR-r10						ENUMERATED {setup}		OPTIONAL	-- Need OR

}															OPTIONAL	-- Need ON

]],

[[	stag-ToReleaseList-r11				STAG-ToReleaseList-r11	OPTIONAL,	-- Need ON

stag-ToAddModList-r11				STAG-ToAddModList-r11	OPTIONAL,	-- Need ON

drx-Config-v1130					DRX-Config-v1130		OPTIONAL	-- Need ON

]],

[[	e-HARQ-Pattern-r12					BOOLEAN					OPTIONAL,	-- Need ON

dualConnectivityPHR					CHOICE {

release								NULL,

setup								SEQUENCE {

phr-ModeOtherCG-r12					ENUMERATED {real, virtual}

}

}														OPTIONAL,	-- Need ON

logicalChannelSR-Config-r12		CHOICE {

release								NULL,

setup								SEQUENCE {

logicalChannelSR-ProhibitTimer-r12		ENUMERATED {sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}

}

}															OPTIONAL		-- Need ON

]],

[[	drx-Config-v1310					DRX-Config-v1310		OPTIONAL,		-- Need ON

extendedPHR2-r13					BOOLEAN		OPTIONAL,		-- Need ON

eDRX-Config-CycleStartOffset-r13	CHOICE {

release							NULL,

setup

CHOICE {

sf5120									INTEGER(0..1),

sf10240									INTEGER(0..3)

}

}										OPTIONAL	-- Need ON

]],

[[	drx-Config-r13						CHOICE {

release								NULL,

setup								DRX-Config-r13

}															OPTIONAL	-- Need ON

]],

[[	skipUplinkTx-r14					CHOICE {

release								NULL,

setup								SEQUENCE {

skipUplinkTxSPS-r14					ENUMERATED {true}		OPTIONAL,	-- Need OR

skipUplinkTxDynamic-r14				ENUMERATED {true}		OPTIONAL	-- Need OR

}

}															OPTIONAL,	-- Need ON

dataInactivityTimerConfig-r14		CHOICE {

release								NULL,

setup								SEQUENCE {

dataInactivityTimer-r14				DataInactivityTimer-r14

}

}												OPTIONAL	-- Need ON

]],

[[	rai-Activation-r14					ENUMERATED {true}			OPTIONAL	-- Need OR

]],

[[	shortTTI-AndSPT-r15				CHOICE {

release							NULL,

setup							SEQUENCE {

drx-Config-r15					DRX-Config-r15				OPTIONAL, -- Need ON

periodicBSR-Timer-r15			ENUMERATED {

sf1, sf5, sf10, sf16, sf20, sf32, sf40,

sf64, sf80, sf128, sf160, sf320, sf640,

sf1280, sf2560, infinity}

OPTIONAL, -- Need ON

proc-Timeline-r15				ENUMERATED {nplus4set1, nplus6set1,

nplus6set2, nplus8set2 }	OPTIONAL, -- Need ON

ssr-ProhibitTimer-r15			INTEGER (0..7)				OPTIONAL -- Need ON

}

}																	OPTIONAL, -- Need ON

mpdcch-UL-HARQ-ACK-FeedbackConfig-r15	BOOLEAN		OPTIONAL,	-- Need ON

dormantStateTimers-r15				CHOICE {

release								NULL,

setup								SEQUENCE {

sCellHibernationTimer-r15			ENUMERATED {

rf2, rf4, rf8, rf16, rf32, rf64, rf128, spare}		OPTIONAL,	-- Need OR

dormantSCellDeactivationTimer-r15	ENUMERATED {

rf2, rf4, rf8, rf16, rf32, rf64,

rf128, rf320, rf640, rf1280, rf2560,

rf5120, rf10240, spare3, spare2, spare1}			OPTIONAL	-- Need OR

}

}																 OPTIONAL	-- Need ON

]],

[[	ce-ETWS-CMAS-RxInConn-r16				ENUMERATED {true}		OPTIONAL	-- Need OR

]] ,

[[ ta-ReportConfig-r17					SetupRelease {TA-ReportConfig-r17}OPTIONAL	-- Need ON

]]

}

MAC-MainConfigSCell-r11 ::=			SEQUENCE {

stag-Id-r11							STAG-Id-r11		OPTIONAL,	-- Need OP

...

}

DRX-Config ::=						CHOICE {

release								NULL,

setup								SEQUENCE {

onDurationTimer						ENUMERATED {

psf1, psf2, psf3, psf4, psf5, psf6,

psf8, psf10, psf20, psf30, psf40,

psf50, psf60, psf80, psf100,

psf200},

drx-InactivityTimer					ENUMERATED {

psf1, psf2, psf3, psf4, psf5, psf6,

psf8, psf10, psf20, psf30, psf40,

psf50, psf60, psf80, psf100,

psf200, psf300, psf500, psf750,

psf1280, psf1920, psf2560, psf0-v1020,

spare9, spare8, spare7, spare6,

spare5, spare4, spare3, spare2,

spare1},

drx-RetransmissionTimer				ENUMERATED {

psf1, psf2, psf4, psf6, psf8, psf16,

psf24, psf33},

longDRX-CycleStartOffset		CHOICE {

sf10							INTEGER(0..9),

sf20							INTEGER(0..19),

sf32							INTEGER(0..31),

sf40							INTEGER(0..39),

sf64							INTEGER(0..63),

sf80							INTEGER(0..79),

sf128							INTEGER(0..127),

sf160							INTEGER(0..159),

sf256							INTEGER(0..255),

sf320							INTEGER(0..319),

sf512							INTEGER(0..511),

sf640							INTEGER(0..639),

sf1024							INTEGER(0..1023),

sf1280							INTEGER(0..1279),

sf2048							INTEGER(0..2047),

sf2560							INTEGER(0..2559)

},

shortDRX							SEQUENCE {

shortDRX-Cycle						ENUMERATED	{

sf2, sf5, sf8, sf10, sf16, sf20,

sf32, sf40, sf64, sf80, sf128, sf160,

sf256, sf320, sf512, sf640},

drxShortCycleTimer					INTEGER (1..16)

}		OPTIONAL													-- Need OR

}

}

DRX-Config-v1130 ::=					SEQUENCE {

drx-RetransmissionTimer-v1130			ENUMERATED {psf0-v1130}	OPTIONAL,	--Need OR

longDRX-CycleStartOffset-v1130			CHOICE {

sf60-v1130								INTEGER(0..59),

sf70-v1130								INTEGER(0..69)

}																OPTIONAL,	--Need OR

shortDRX-Cycle-v1130					ENUMERATED	{sf4-v1130}	OPTIONAL	--Need OR

}

DRX-Config-v1310 ::=					SEQUENCE {

longDRX-CycleStartOffset-v1310		SEQUENCE {

sf60-v1310								INTEGER(0..59)

}																OPTIONAL	--Need OR

}

DRX-Config-r13 ::=					SEQUENCE {

onDurationTimer-v1310					ENUMERATED {psf300, psf400, psf500, psf600,

psf800, psf1000, psf1200, psf1600}

OPTIONAL,	--Need OR

drx-RetransmissionTimer-v1310			ENUMERATED {psf40, psf64, psf80, psf96, psf112,

psf128, psf160, psf320}

OPTIONAL,	--Need OR

drx-ULRetransmissionTimer-r13			ENUMERATED {psf0, psf1, psf2, psf4, psf6, psf8, psf16,

psf24, psf33, psf40, psf64, psf80, psf96,

psf112, psf128, psf160, psf320}

OPTIONAL	--Need OR

}

DRX-Config-r15 ::=					SEQUENCE {

drx-RetransmissionTimerShortTTI-r15		ENUMERATED {

tti10, tti20, tti40, tti64, tti80, tti96,

tti112,tti128, tti160, tti320} OPTIONAL, --Need OR

drx-UL-RetransmissionTimerShortTTI-r15	ENUMERATED {

tti0, tti1, tti2, tti4, tti6, tti8, tti16,

tti24, tti33, tti40, tti64, tti80, tti96, tti112,

tti128, tti160, tti320}	OPTIONAL --Need OR

}

PeriodicBSR-Timer-r12 ::=					ENUMERATED {

sf5, sf10, sf16, sf20, sf32, sf40, sf64, sf80,

sf128, sf160, sf320, sf640, sf1280, sf2560,

infinity, spare1}

RetxBSR-Timer-r12 ::=							ENUMERATED {

sf320, sf640, sf1280, sf2560, sf5120,

sf10240, spare2, spare1}

STAG-ToReleaseList-r11 ::=	SEQUENCE (SIZE (1..maxSTAG-r11)) OF STAG-Id-r11

STAG-ToAddModList-r11 ::=	SEQUENCE (SIZE (1..maxSTAG-r11)) OF STAG-ToAddMod-r11

STAG-ToAddMod-r11 ::=		SEQUENCE {

stag-Id-r11					STAG-Id-r11,

timeAlignmentTimerSTAG-r11	TimeAlignmentTimer,

...

}

STAG-Id-r11::=				INTEGER (1..maxSTAG-r11)

TA-ReportConfig-r17 ::=		TypeFFS

-- ASN1STOP

Editor’s Note: Agreement: RAN2 assumes that sr-ProhibitTimer need to be extended. Postpone treatment of sr-ProhibitTimer values until the NR NTN details have been decided.

Editor’s Note: Agreement: Support TA reporting in RRC connected mode in IoT NTN. Agreement: Support event-triggered for TA reporting in connected mode. Wait for NR NTN agreements for other triggers

| MAC-MainConfig field descriptions |  |
| --- | --- |
| ce-ETWS-CMAS-RxInConn Indicates UE shall monitor for ETWS/CMAS notification on control channels associated with the shared data channel in RRC_CONNECTED as specified in TS 36.213 [23], clause 7.1. |  |  |
| dl-PathlossChange DL Pathloss Change and the change of the required power backoff due to power management (as allowed by P-MPRc, see TS 36.101 [42]) for PHR reporting in TS 36.321 [6]. Value in dB. Value dB1 corresponds to 1 dB, dB3 corresponds to 3 dB and so on. The same value applies for each serving cell (although the associated functionality is performed independently for each cell). |  |
| dormantSCellDeactivationTimer SCell deactivation timer for UEs supporting dormant state as specified in TS 36.321 [6]. Value in number of radio frames. Value rf4 corresponds to 4 radio frames, value rf8 corresponds to 8 radio frames and so on. E-UTRAN only configures the field if the UE is configured with one or more SCells other than the PSCell and PUCCH SCell. The same value applies for each SCell of a Cell Group (i.e. MCG or SCG) (although the associated functionality is performed independently for each SCell). Field dormantSCellDeactivationTimer does not apply for the PUCCH SCell. |  |
| drx-Config Used to configure DRX as specified in TS 36.321 [6]. E-UTRAN configures the values in DRX-Config-v1130 only if the UE indicates support for IDC indication. E-UTRAN configures drx-Config-v1130, drx-Config-v1310 and drx-Config-r13 only if drx-Config (without suffix) is configured. E-UTRAN configures drx-Config-r13 only if UE supports CE or if the UE is configured with uplink of an LAA SCell. |  |
| drx-InactivityTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH sub-frames. Value psf0 corresponds to 0 PDCCH sub-frame and behaviour as specified in 7.3.2 applies, value psf1 corresponds to 1 PDCCH sub-frame, psf2 corresponds to 2 PDCCH sub-frames and so on. |  |
| drx-RetransmissionTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH sub-frames. Value psf0 corresponds to 0 PDCCH sub-frame and behaviour as specified in 7.3.2 applies, value psf1 corresponds to 1 PDCCH sub-frame, psf2 corresponds to 2 PDCCH sub-frames and so on. In case drx-RetransmissionTimer-v1130 or drx-RetransmissionTimer-v1310 is signalled, the UE shall ignore drx-RetransmissionTimer (i.e. without suffix). |  |
| drx-RetransmissionTimerShortTTI Timer for DRX in TS 36.321 [6]. Value in number of short TTIs when short TTI is configured. Value tti10 corresponds to 10 TTIs, value tti20 corresponds to 20 TTIs and so on. |  |
| drx-ULRetransmissionTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH sub-frames. Value psf0 correponds to 0 PDCCH sub-frame and behaviour as specified in 7.3.2 applies, value psf1 corresponds to 1 PDCCH sub-frame, psf2 corresponds to 2 PDCCH sub-frames and so on. |  |
| drx-UL-RetransmissionTimerShortTTI Timer for DRX in TS 36.321 [6]. Value in number of short TTIs when short TTI is configured. Value tti0 corresponds to 0 TTIs and behaviour as specified in 7.3.2 applies, value tti1 corresponds to 1 TTI and so on. |  |
| drxShortCycleTimer Timer for DRX in TS 36.321 [6]. Value in multiples of shortDRX-Cycle. A value of 1 corresponds to shortDRX-Cycle, a value of 2 corresponds to 2 * shortDRX-Cycle and so on. |  |
| dualConnectivityPHR Indicates if power headroom shall be reported using Dual Connectivity Power Headroom Report MAC Control Element defined in TS 36.321 [6] (value setup). For both LTE DC and (NG)EN-DC, if PHR functionality is configured, E-UTRAN always configures the value setup for this field and configures phr-Config and dualConnectivityPHR. For LTE DC, E-UTRAN configures the field for both CGs while for (NG)EN-DC, E-UTRAN configures the field only for MCG. E-UTRAN does not configure this field when a DAPS bearer is configured. |  |
| e-HARQ-Pattern TRUE indicates that enhanced HARQ pattern for TTI bundling is enabled for FDD. E-UTRAN enables this field only when ttiBundling is set to TRUE. |  |
| eDRX-Config-CycleStartOffset Indicates longDRX-Cycle and drxStartOffset in TS 36.321 [6]. The value of longDRX-Cycle is in number of sub-frames. The value of drxStartOffset, in number of subframes, is indicated by the value of eDRX-Config-CycleStartOffset multiplied by 2560 plus the offset value configured in longDRX-CycleStartOffset. E-UTRAN only configures value setup when the value in longDRX-CycleStartOffset is sf2560. |  |
| extendedBSR-Sizes If value setup is configured, the BSR index indicates extended BSR size levels as defined in TS 36.321 [6], Table 6.1.3.1-2. |  |
| extendedPHR Indicates if power headroom shall be reported using the Extended Power Headroom Report MAC control element defined in TS 36.321 [6] (value setup). E-UTRAN always configures the value setup if more than one and up to eight Serving Cell(s) with uplink is configured and none of the serving cells with uplink configured has a servingCellIndex higher than seven and if PUCCH on SCell is not configured and if dual connectivity is not configured. E-UTRAN configures extendedPHR only if phr-Config is configured. E-UTRAN does not configure this field when a DAPS bearer is configured. The UE shall release extendedPHR if phr-Config is released. |  |
| extendedPHR2 Indicates if power headroom shall be reported using the Extended Power Headeroom Report MAC Control Element defined in TS 36.321 [6] (value setup). E-UTRAN always configures the value setup if any of the serving cells with uplink configured has a servingCellIndex higher than seven in case dual connectivity is not configured or if PUCCH SCell (with any number of serving cells with uplink configured) is configured. E-UTRAN configures extendedPHR2 only if phr-Config is configured. E-UTRAN does not configure this field when a DAPS bearer is configured. The UE shall release extendedPHR2 if phr-Config is released. |  |
| logicalChannelSR-ProhibitTimer Timer used to delay the transmission of an SR for logical channels enabled by logicalChannelSR-Prohibit. Value sf20 corresponds to 20 subframes, sf40 corresponds to 40 subframes, and so on. See TS 36.321 [6]. |  |
| longDRX-CycleStartOffset longDRX-Cycle and drxStartOffset in TS 36.321 [6] unless eDRX-Config-CycleStartOffset is configured. The value of longDRX-Cycle is in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. If shortDRX-Cycle is configured, the value of longDRX-Cycle shall be a multiple of the shortDRX-Cycle value. The value of drxStartOffset value is in number of sub-frames. In case longDRX-CycleStartOffset-v1130 is signalled, the UE shall ignore longDRX-CycleStartOffset (i.e. without suffix). In case longDRX-CycleStartOffset-v1310 is signalled, the UE shall ignore longDRX-CycleStartOffset (i.e. without suffix). |  |
| maxHARQ-Tx Maximum number of transmissions for UL HARQ in TS 36.321 [6]. |  |
| mpdcch-UL-HARQ-ACK-FeedbackConfig TRUE indicates E-UTRAN may send UL HARQ-ACK feedback or UL grant corresponding to a new transmission for early termination of PUSCH transmission, or positive acknowledgement of completed PUSCH transmissions as specified in TS 36.321 [6] and TS 36.212 [22]. In case of acknowledgement of RRC Connection Release, MPDCCH monitoring is terminated. |
| onDurationTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH sub-frames. Value psf1 corresponds to 1 PDCCH sub-frame, psf2 corresponds to 2 PDCCH sub-frames and so on. In case onDurationTimer-v1310 is signalled, the UE shall ignore onDurationTimer (i.e. without suffix). |  |
| periodicBSR-Timer Timer for BSR reporting in TS 36.321 [6]. Value in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. |  |
| periodicPHR-Timer Timer for PHR reporting in TS 36.321 [6]. Value in number of sub-frames. Value sf10 corresponds to 10 subframes, sf20 corresponds to 20 subframes and so on. |  |
| phr-ModeOtherCG Indicates the mode (i.e. real or virtual) used for the PHR of the activated cells that are part of the other Cell Group (i.e. MCG or SCG), when DC is configured. |  |
| proc-Timeline Minimum processing timeline for short TTI with subslot operation. Value nplus4set1 indicates processing time n+4 for set 1, value nplus6set1 indicates processing time n+6 for set 1, value nplus6set2 indicates processing time n+6 for set and value nplus8set2 indicates processing time n+8 for set 2. See also UE capability min-Proc-TimelineSubslot for sTTI. |  |
| prohibitPHR-Timer Timer for PHR reporting in TS 36.321 [6]. Value in number of sub-frames. Value sf0 corresponds to 0 subframes and behaviour as specified in 7.3.2 applies, sf100 corresponds to 100 subframes and so on. |  |
| rai-Activation Activation of release assistance indication (RAI) in TS 36.321 [6] for BL UEs. |  |
| retxBSR-Timer Timer for BSR reporting in TS 36.321 [6]. Value in number of sub-frames. Value sf640 corresponds to 640 sub-frames, sf1280 corresponds to 1280 sub-frames and so on. |  |
| sCellDeactivationTimer SCell deactivation timer in TS 36.321 [6]. Value in number of radio frames. Value rf4 corresponds to 4 radio frames, value rf8 corresponds to 8 radio frames and so on. E-UTRAN only configures the field if the UE is configured with one or more SCells other than the PSCell and PUCCH SCell. If the field is absent, the UE shall delete any existing value for this field and assume the value to be set to infinity. The same value applies for each SCell of a Cell Group (i.e. MCG or SCG) (although the associated functionality is performed independently for each SCell). Field sCellDeactivationTimer does not apply for the PUCCH SCell. |  |
| sCellHibernationTimer SCell hibernation timer for UEs supporting dormant SCell state as specified in TS 36.321 [6]. Value in number of radio frames. Value rf4 corresponds to 4 radio frames, value rf8 corresponds to 8 radio frames and so on. E-UTRAN only configures the field if the UE is configured with one or more SCells other than the PSCell and PUCCH SCell. The same value applies for each SCell of a Cell Group (i.e. MCG or SCG) (although the associated functionality is performed independently for each SCell). Field sCellHibernationTimer does not apply for the PUCCH SCell. |  |
| shortDRX-Cycle Short DRX cycle in TS 36.321 [6]. Value in number of sub-frames. Value sf2 corresponds to 2 sub-frames, sf5 corresponds to 5 subframes and so on. In case shortDRX-Cycle-v1130 is signalled, the UE shall ignore shortDRX-Cycle (i.e. without suffix). Short DRX cycle is not configured for UEs in CE. |  |
| skipUplinkTxDynamic If configured, the UE skips UL transmissions for an uplink grant other than a configured uplink grant if no data is available for transmission in the UE buffer as described in TS 36.321 [6]. |  |
| skipUplinkTxSPS If configured, the UE skips UL transmissions for a configured uplink grant if no data is available for transmission in the UE buffer as described in TS 36.321 [6]. E-UTRAN always configures skipUplinkTxSPS when there is at least one SPS configuration with semiPersistSchedIntervalUL shorter than sf10 or when at least one SPS-ConfigUL-STTI is configured for the cell group. |  |
| sr-ProhibitTimer Timer for SR transmission on PUCCH in TS 36.321 [6]. Value in number of SR period(s) of shortest SR period of any serving cell with PUCCH. Value 0 means that behaviour as specified in 7.3.2 applies. Value 1 corresponds to one SR period, Value 2 corresponds to 2*SR periods and so on. SR period is defined in TS 36.213 [23], table 10.1.5-1. |  |
| ssr-ProhibitTimer Timer for prohibiting SR transmission on SPUCCH in TS 36.321 [6]. Value in number of SR period(s) of shortest SR period of any serving cell with SPUCCH. Value 0 means that behaviour as specified in 7.3.2 applies. Value 1 corresponds to one SR period, value 2 corresponds to 2 SR periods and so on. SR period is defined in TS 36.213 [23], table 10.1.5-1. |  |
| stag-Id Indicates the TAG of an SCell, see TS 36.321 [6]. Uniquely identifies the TAG within the scope of a Cell Group (i.e. MCG or SCG). If the field is not configured for an SCell (e.g. absent in MAC-MainConfigSCell), the SCell is part of the PTAG. |  |
| stag-ToAddModList, stag-ToReleaseList Used to configure one or more STAGs. E-UTRAN ensures that a STAG contains at least one SCell with configured uplink. If, due to SCell release a reconfiguration would result in an 'empty' TAG, E-UTRAN includes release of the concerned TAG. |  |
| ta-ReportConfig Used to configure UE specific TA reporting as specified in TS 36.321 [6]. |  |
| timeAlignmentTimerSTAG Indicates the value of the time alignment timer for an STAG, see TS 36.321 [6]. |  |
| ttiBundling TRUE indicates that TTI bundling TS 36.321 [6] is enabled while FALSE indicates that TTI bundling is disabled. TTI bundling can be enabled for FDD and for TDD for configurations 0, 1 and 6 and additionally for configurations 2 and 3 when symPUSCH-UpPTS-r14 is configured. The functionality is performed independently per Cell Group (i.e. MCG or SCG), but E-UTRAN does not configure TTI bundling for the SCG. For a TDD PCell, E-UTRAN does not simultaneously enable TTI bundling and semi-persistent scheduling in this release of specification. Furthermore, for a Cell Group, E-UTRAN does not simultaneously configure TTI bundling and SCells with configured uplink, and E-UTRAN does not simultaneously configure TTI bundling and eIMTA. |  |

#### –	P-C-AndCBSR

The IE P-C-AndCBSR is used to specify the power control and codebook subset restriction configuration.

P-C-AndCBSR information elements

-- ASN1START

P-C-AndCBSR-r11 ::=	SEQUENCE {

p-C-r11						INTEGER (-8..15),

codebookSubsetRestriction-r11	BIT STRING

}

P-C-AndCBSR-r13 ::=	SEQUENCE {

p-C-r13						INTEGER (-8..15),

cbsr-Selection-r13			CHOICE{

nonPrecoded-r13				SEQUENCE {

codebookSubsetRestriction1-r13				BIT STRING,

codebookSubsetRestriction2-r13				BIT STRING

},

beamformedK1a-r13			SEQUENCE {

codebookSubsetRestriction3-r13				BIT STRING

},

beamformedKN-r13			SEQUENCE {

codebookSubsetRestriction-r13				BIT STRING

}

},

...

}

P-C-AndCBSR-r15 ::=	SEQUENCE {

p-C-r15						INTEGER (-8..15),

codebookSubsetRestriction4-r15	BIT STRING

}

P-C-AndCBSR-Pair-r13a ::=	SEQUENCE (SIZE (1..2)) OF P-C-AndCBSR-r11

P-C-AndCBSR-Pair-r13 ::=	SEQUENCE (SIZE (1..2)) OF P-C-AndCBSR-r13

P-C-AndCBSR-Pair-r15 ::=	SEQUENCE (SIZE (1..2)) OF P-C-AndCBSR-r15

-- ASN1STOP

P-C-AndCBSR field descriptions

cbsr-Selection Indicates which codebook subset restriction parameter(s) are to be used. E-UTRAN applies values nonPrecoded when eMIMO-Type is set to nonPrecoded. E-UTRAN applies value beamformedK1a when eMIMO-Type is set to beamformed, alternativeCodebookEnabledBeamformed is set to TRUE and csi-RS-ConfigNZPIdListExt is not configured. E-UTRAN applies value beamformedKN when csi-RS-ConfigNZPIdListExt is configured. E-UTRAN applies value beamformedKN when eMIMO-Type is set to beamformed, csi-RS-ConfigNZPIdListExt is not configured and alternativeCodebookEnabledBeamformed is set to FALSE.

codebookSubsetRestriction Parameter: codebookSubsetRestriction, see TS 36.213 [23] and TS 36.211 [21]. The number of bits in the codebookSubsetRestriction for applicable transmission modes is defined in TS 36.213 [23].

codebookSubsetRestriction1 Parameter: codebookSubsetRestriction1, see TS 36.213 [23], Table 7.2-1d. The number of bits in the codebookSubsetRestriction1 for applicable transmission modes is defined in TS 36.213 [23].

codebookSubsetRestriction2 Parameter: codebookSubsetRestriction2, see TS 36.213 [23], Table 7.2-1e. The number of bits in the codebookSubsetRestriction2 for applicable transmission modes is defined in TS 36.213 [23].

codebookSubsetRestriction3 Parameter: codebookSubsetRestriction3, see TS 36.213 [23], Table 7.2-1f. The UE shall ignore codebookSubsetRestriction-r11 or codebookSubsetRestriction-r10 if codebookSubsetRestriction3-r13 is configured. The number of bits in the codebookSubsetRestriction3 for applicable transmission modes is defined in TS 36.213 [23].

codebookSubsetRestriction4 Parameter: codebookSubsetRestriction4, see TS 36.213 [23], Table 7.2. The number of bits in the codebookSubsetRestriction4 for applicable transmission modes is defined in TS 36.213 [23].

p-C Parameter: , see TS 36.213 [23], clause 7.2.5.

P-C-AndCBSR-Pair E-UTRAN includes a single entry if the UE is configured with TM9. If the UE is configured with TM10 and E-UTRAN includes 2 entries, this indicates that the subframe patterns configured for CSI (CQI/PMI/PTI/RI/CRI) reporting (i.e. as defined by field csi-MeasSubframeSet1 and csi-MeasSubframeSet2, or as defined by csi-MeasSubframeSets-r12) are to be used for this CSI process, while including a single entry indicates that the subframe patterns are not to be used for this CSI process. For a UE configured with TM10, E-UTRAN does not include 2 entries with csi-MeasSubframeSet1 and csi-MeasSubframeSet2 for CSI processes concerning a secondary frequency. Furthermore, E-UTRAN includes 2 entries when configuring both cqi-pmi-ConfigIndex and cqi-pmi-ConfigIndex2.

#### –	PDCCH-ConfigSCell

The IE PDCCH-ConfigSCell specifies PDCCH monitoring parameters that E-UTRAN may configure for a serving cell.

PDCCH-ConfigSCell information element

-- ASN1START

PDCCH-ConfigSCell-r13 ::=		SEQUENCE {

skipMonitoringDCI-format0-1A-r13	ENUMERATED {true}				OPTIONAL	-- Need OR

}

PDCCH-ConfigLAA-r14 ::=		SEQUENCE {

maxNumberOfSchedSubframes-Format0B-r14	ENUMERATED {sf2, sf3, sf4}	OPTIONAL,		-- Need OR

maxNumberOfSchedSubframes-Format4B-r14	ENUMERATED {sf2, sf3, sf4}	OPTIONAL,		-- Need OR

skipMonitoringDCI-Format0A-r14				ENUMERATED {true}		OPTIONAL,		-- Need OR

skipMonitoringDCI-Format4A-r14				ENUMERATED {true}		OPTIONAL,		-- Need OR

pdcch-CandidateReductions-Format0A-r14

PDCCH-CandidateReductions-r13			OPTIONAL,		-- Need ON

pdcch-CandidateReductions-Format4A-r14

PDCCH-CandidateReductionsLAA-UL-r14		OPTIONAL,	-- Need ON

pdcch-CandidateReductions-Format0B-r14

PDCCH-CandidateReductionsLAA-UL-r14		OPTIONAL,	-- Need ON

pdcch-CandidateReductions-Format4B-r14

PDCCH-CandidateReductionsLAA-UL-r14	OPTIONAL		-- Need ON

}

PDCCH-CandidateReductionValue-r13 ::= ENUMERATED {n0, n33, n66, n100}

PDCCH-CandidateReductionValue-r14 ::= ENUMERATED {n0, n50, n100, n150}

PDCCH-CandidateReductions-r13 ::= CHOICE {

release							NULL,

setup							SEQUENCE {

pdcch-candidateReductionAL1-r13		PDCCH-CandidateReductionValue-r13,

pdcch-candidateReductionAL2-r13		PDCCH-CandidateReductionValue-r13,

pdcch-candidateReductionAL3-r13		PDCCH-CandidateReductionValue-r13,

pdcch-candidateReductionAL4-r13		PDCCH-CandidateReductionValue-r13,

pdcch-candidateReductionAL5-r13		PDCCH-CandidateReductionValue-r13

}

}

PDCCH-CandidateReductionsLAA-UL-r14 ::= CHOICE {

release							NULL,

setup							SEQUENCE {

pdcch-candidateReductionAL1-r14		PDCCH-CandidateReductionValue-r13,

pdcch-candidateReductionAL2-r14		PDCCH-CandidateReductionValue-r13,

pdcch-candidateReductionAL3-r14		PDCCH-CandidateReductionValue-r14,

pdcch-candidateReductionAL4-r14		PDCCH-CandidateReductionValue-r14,

pdcch-candidateReductionAL5-r14		PDCCH-CandidateReductionValue-r14

}

}

-- ASN1STOP

PDCCH-ConfigSCell field descriptions

maxNumberOfSchedSubframes-Format0B Indicates maximum number of schedulable subframes for DCI format 0B as specified in TS 36.213 [23]. Value sf2 corresponds to 2 subframes, value sf3 corresponds to 3 subframes and so on.

maxNumberOfSchedSubframes-Format4B Indicates maximum number of schedulable subframes for DCI format 4B as specified in TS 36.213 [23]. Value sf2 corresponds to 2 subframes, value sf3 corresponds to 3 subframes and so on.

skipMonitoringDCI-format0-1A Indicates whether the UE is configured to omit monitoring DCI fromat 0/1A, see TS 36.213 [23], clause 9.1.1.

skipMonitoringDCI-Format0A Indicates whether the UE is configured to omit monitoring DCI fromat 0A as specified in TS 36.213 [23].

skipMonitoringDCI-Format4A Indicates whether the UE is configured to omit monitoring DCI fromat 4A as specified in TS 36.213 [23].

pdcch-candidateReductionALx Indicates reduced (E)PDCCH monitoring requirements on UE specific search space of the x-th aggregation level, see TS 36.213 [23], clause 9.1.1. Value n0 corresponds to 0%, value n33 corresponds to 33% and so on.

pdcch-CandidateReductions-Formatx Indicates number of blind detections on UE specific search space for each aggregation layer as specified in TS 36.213 [23]. The field can only be present when the UE is configured with uplink of an LAA SCell. If pdcch-CandidateReductions-Formatx is not configured, pdcch-CandidateReductions-r13 applies to the corresponding DCIs (if configured).

#### –	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for data radio bearers.

PDCP-Config information element

-- ASN1START

PDCP-Config ::=						SEQUENCE {

discardTimer						ENUMERATED {

ms50, ms100, ms150, ms300, ms500,

ms750, ms1500, infinity

}															OPTIONAL,			-- Cond Setup

rlc-AM								SEQUENCE {

statusReportRequired				BOOLEAN

}															OPTIONAL,			-- Cond Rlc-AM-UM

rlc-UM								SEQUENCE {

pdcp-SN-Size						ENUMERATED {len7bits, len12bits}

}															OPTIONAL,			-- Cond Rlc-UM

headerCompression					CHOICE {

notUsed								NULL,

rohc								SEQUENCE {

maxCID								INTEGER (1..16383)				DEFAULT 15,

profiles							SEQUENCE {

profile0x0001						BOOLEAN,

profile0x0002						BOOLEAN,

profile0x0003						BOOLEAN,

profile0x0004						BOOLEAN,

profile0x0006						BOOLEAN,

profile0x0101						BOOLEAN,

profile0x0102						BOOLEAN,

profile0x0103						BOOLEAN,

profile0x0104						BOOLEAN

},

...

}

},

...,

[[	rn-IntegrityProtection-r10		ENUMERATED {enabled}	OPTIONAL	-- Cond RN

]],

[[	pdcp-SN-Size-v1130				ENUMERATED {len15bits}	OPTIONAL	-- Cond Rlc-AM2

]],

[[	ul-DataSplitDRB-ViaSCG-r12		BOOLEAN		OPTIONAL,	-- Need ON

t-Reordering-r12				ENUMERATED {

ms0, ms20, ms40, ms60, ms80, ms100, ms120, ms140,

ms160, ms180, ms200, ms220, ms240, ms260, ms280, ms300,

ms500, ms750, spare14, spare13, spare12, spare11, spare10,

spare9, spare8, spare7, spare6, spare5, spare4, spare3,

spare2, spare1}					OPTIONAL	-- Cond SetupS

]],

[[	ul-DataSplitThreshold-r13		CHOICE {

release						NULL,

setup						ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800,

b25600, b51200, b102400, b204800, b409600, b819200,

spare1}

}																OPTIONAL,	-- Need ON

pdcp-SN-Size-v1310				ENUMERATED {len18bits}	OPTIONAL,	-- Cond Rlc-AM3

statusFeedback-r13				CHOICE {

release						NULL,

setup						SEQUENCE {

statusPDU-TypeForPolling-r13		ENUMERATED {type1, type2}		OPTIONAL,	-- Need ON

statusPDU-Periodicity-Type1-r13		ENUMERATED {

ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70, ms80, ms90,

ms100, ms150, ms200, ms300, ms500, ms1000, ms2000, ms5000,

ms10000, ms20000, ms50000}		OPTIONAL,	-- Need ON

statusPDU-Periodicity-Type2-r13		ENUMERATED {

ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70, ms80, ms90,

ms100, ms150, ms200, ms300, ms500, ms1000, ms2000, ms5000,

ms10000, ms20000, ms50000}		OPTIONAL,	-- Need ON

statusPDU-Periodicity-Offset-r13	ENUMERATED {

ms1, ms2, ms5, ms10, ms25, ms50, ms100, ms250, ms500,

ms2500, ms5000, ms25000}	OPTIONAL	-- Need ON

}

}																OPTIONAL	-- Need ON

]],

[[	ul-LWA-Config-r14			CHOICE {

release						NULL,

setup						SEQUENCE {

ul-LWA-DRB-ViaWLAN-r14		BOOLEAN,

ul-LWA-DataSplitThreshold-r14	ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400,

b12800, b25600, b51200, b102400, b204800, b409600,

b819200 }			OPTIONAL	-- Need OR

}

}														OPTIONAL,		-- Need ON

uplinkOnlyHeaderCompression-r14		CHOICE {

notUsed-r14							NULL,

rohc-r14								SEQUENCE {

maxCID-r14								INTEGER (1..16383)		DEFAULT 15,

profiles-r14							SEQUENCE {

profile0x0006-r14						BOOLEAN

},

...

}

}													OPTIONAL -- Need ON

]],

[[	uplinkDataCompression-r15	SEQUENCE {

bufferSize-r15				ENUMERATED {kbyte2, kbyte4, kbyte8, spare1},

dictionary-r15				ENUMERATED {sip-SDP, operator}	OPTIONAL, -- Need OR

...

}															OPTIONAL,-- Cond Rlc-AM4

pdcp-DuplicationConfig-r15	CHOICE {

release						NULL,

setup						SEQUENCE {

pdcp-Duplication-r15		ENUMERATED {configured, activated}

}

}													OPTIONAL -- Need ON

]],

[[

ethernetHeaderCompression-r16	SetupRelease {EthernetHeaderCompression-r16}	OPTIONAL -- Need ON

]],

[[	discardTimerExt-r17			SetupRelease {DiscardTimerExt-r17}	OPTIONAL	-- Need ON

]]

}

EthernetHeaderCompression-r16	::=	SEQUENCE {

ehc-Common-r16		SEQUENCE {

ehc-CID-Length-r16		ENUMERATED {bits7, bits15}

},

ehc-Downlink-r16	SEQUENCE {

drb-ContinueEHC-DL-r16		ENUMERATED {true}		OPTIONAL -- Need OR

}	OPTIONAL,-- Need ON

ehc-Uplink-r16		SEQUENCE {

maxCID-EHC-UL-r16				INTEGER (1..32767),

drb-ContinueEHC-UL-r16		ENUMERATED {true}		OPTIONAL -- Need OR

}	OPTIONAL,   -- Need ON

...

}

DiscardTimerExt-r17 ::= ENUMERATED {ffs}

-- ASN1STOP

Editor’s Note: Agreement: The PDCP discardTimer should be extended to support eMTC over NTN. Agreement: If PDCP discardTimer is agreed to be extended to support eMTC over NTN, how to extend the timer value can wait for the conclusion for RLC t-reordering timer

PDCP-Config field descriptions

bufferSize Indicates the buffer size applied for UDC specified in TS 36.323 [8]. Value kbyte2 means 2048 bytes, kbyte4 means 4096 bytes and so on.

dictionary Indicates which pre-defined dictionary is used for UDC as specified in TS 36.323 [8]. The value sip-SDP means that UE shall prefill the buffer with standard dictionary for SIP and SDP defined in TS 36.323 [8], and the value operator means that UE shall prefill the buffer with operator-defined dictionary.

discardTimer Indicates the discard timer value specified in TS 36.323 [8]. Value in milliseconds. Value ms50 means 50 ms, ms100 means 100 ms and so on.

discardTimerExt Indicates the discard timer value specified in TS 36.323 [8]. Value in milliseconds. Value FFS means FFS ms and so on. The UE shall use the extended value discardTimerExt, if present, and ignore the value signaled by discardTimer.

drb-ContinueEHC-DL Indicates whether the PDCP entity continues or resets the downlink EHC header compression protocol during PDCP re-establishment, as specified in TS 36.323 [8]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

drb-ContinueEHC-UL Indicates whether the PDCP entity continues or resets the uplink EHC header compression protocol during PDCP re-establishment, as specified in TS 36.323 [8]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

ehc-CID-Length Indicates the length of the CID field for EHC packet. Once the field ethernetHeaderCompression-r16 is configured for a DRB, the value of the field ehc-CID-Length for this DRB is not reconfigured to a different value.

ehc-Common Indicates the configurations that apply for both downlink and uplink.

ehc-Downlink Indicates the configurations that apply for only downlink. If the field is configured, then Ethernet header compression is configured for downlink. Otherwise, it is not configured for downlink.

ehc-Uplink Indicates the configurations that apply for only uplink. If the field is configured, then Ethernet header compression is configured for uplink. Otherwise, it is not configured for uplink.

ethernetHeaderCompression This field configures Ethernet Header Compression. This field can only be configured for DRB. E-UTRAN does not reconfigure ethernetHeaderCompression for an MCG DRB except for upon handover and upon the first reconfiguration after RRC connection re-establishment. E-UTRAN does not reconfigure ethernetHeaderCompression for a SCG DRB except for upon SCG change involving PDCP re-establishment. E-UTRAN does not configure this field if uplinkDataCompression is configured. E-UTRAN does not configure this field for split and LWA DRBs.

headerCompression E-UTRAN does not reconfigure header compression for an MCG DRB except for upon handover and upon the first reconfiguration after RRC connection re-establishment, and without any drb-ContinueROHC. E-UTRAN does not reconfigure header compression for a SCG DRB except for upon SCG change involving PDCP re-establishment. E-UTRAN does not configure header compression while t-Reordering is configured except for DAPS bearers. E-UTRAN only configures this field when neither uplinkOnlyHeaderCompression nor uplinkDataCompression is configured. If headerCompression is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. ROHC and EHC can be both configured simultaneously for a DRB.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 36.323 [8]. The total value of MAX_CIDs across all bearers for the UE should be less than or equal to the value of maxNumberROHC-ContextSessions parameter as indicated by the UE.

maxCID-EHC-UL Indicates the value of the MAX_CID_EHC_UL parameter as specified in TS 36.323 [8]. The total value of MAX_CID_EHC_UL across all bearers for the UE should be less than or equal to the value of maxNumberEHC-Contexts parameter as indicated by the UE.

pdcp-Duplication Parameter for configuring PDCP duplication as specified in TS 36.323 [8]. Value configured indicates that PDCP duplication is configured but initially deactivated and value activated indicates that PDCP duplication is configured and activated upon configuration. For EN-DC, E-UTRAN configures PDCP duplication for MCG DRB only if PDCP duplication is not configured for any split DRB. PDCP duplication is not supported during a DAPS handover.

pdcp-SN-Size Indicates the PDCP Sequence Number length in bits. For RLC UM: value len7bits means that the 7-bit PDCP SN format is used and len12bits means that the 12-bit PDCP SN format is used. For RLC AM: value len15bits means that the 15-bit PDCP SN format is used, value len18bits means that the 18-bit PDCP SN format is used, otherwise if the field is not included upon setup of the PCDP entity 12-bit PDCP SN format is used, as specified in TS 36.323 [8].

profiles The profiles used by both compressor and decompressor in both UE and E-UTRAN. The field indicates which of the ROHC profiles specified in TS 36.323 [8] are supported, i.e. value true indicates that the profile is supported. Profile 0x0000 shall always be supported when the use of ROHC is configured. If support of two ROHC profile identifiers with the same 8 LSB's is signalled, only the profile corresponding to the highest value shall be applied.

statusFeedback Indicates whether the UE shall send PDCP Status Report periodically or by E-UTRAN polling as specified in TS 36.323 [8]. E-UTRAN configures this field only for LWA DRB.

statusPDU-TypeForPolling Indicates the PDCP Control PDU option when it is triggered by E-UTRAN polling. Value type1 indicates using the legacy PDCP Control PDU for PDCP status reporting and value type2 indicates using the LWA specific PDCP Control PDU for LWA status reporting as specified in TS 36.323 [8].

statusPDU-Periodicity-Type1 Indicates the value of the PDCP Status reporting periodicity for type1 Status PDU, as specified in TS 36.323 [8]. Value in milliseconds. Value ms5 means 5 ms, ms10 means 10 ms and so on.

statusPDU-Periodicity-Type2 Indicates the value of the PDCP Status reporting periodicity for type2 Status PDU, as specified in TS 36.323 [8]. Value in milliseconds. Value ms5 means 5 ms, ms10 means 10 ms and so on.

statusPDU-Periodicity-Offset Indicates the value of the offset for type2 Status PDU periodicity, as specified in TS 36.323 [8]. Value in milliseconds. Value ms1 means 1 ms, ms2 means 2 ms and so on.

t-Reordering Indicates the value of the reordering timer, as specified in TS 36.323 [8]. Value in milliseconds. Value ms0 means 0 ms and behaviour as specified in 7.3.2 applies, ms20 means 20 ms and so on.

rn-IntegrityProtection Indicates that integrity protection or verification shall be applied for all subsequent packets received and sent by the RN on the DRB.

statusReportRequired Indicates whether or not the UE shall send a PDCP Status Report upon re-establishment of the PDCP entity, upon PDCP data recovery, upon uplink data switching during DAPS handover and upon release of the source cell after DAPS handover as specified in TS 36.323 [8]. If the UE supports DAPS handover, for RLC UM radio bearers, the field has the value FALSE if it has not been configured.

ul-DataSplitDRB-ViaSCG Indicates whether the UE shall send PDCP PDUs via SCG as specified in TS 36.323 [8]. E-UTRAN only configures the field (i.e. indicates value TRUE) for split DRBs. For PDCP duplication, if this field is set to TRUE, the primary RLC entity is SCG RLC entity and the secondary RLC entity is MCG RLC entity. If this field is not configured or set to FALSE, the primary RLC entity is MCG RLC entity and the secondary RLC entity is SCG RLC entity.

ul-DataSplitThreshold Indicates the threshold value for uplink data split operation specified in TS 36.323 [8]. Value b100 means 100 Bytes, b200 means 200 Bytes and so on. E-UTRAN only configures this field for split DRBs.

ul-LWA-DRB-ViaWLAN Indicates whether the UE shall send PDCP PDUs via the LWAAP entity as specified in TS 36.323 [8]. E-UTRAN only configures this field (i.e. indicates value TRUE) for LWA DRBs.

ul-LWA-DataSplitThreshold Indicates the threshold value for uplink data split operation as specified in TS 36.323 [8]. Value b0 means 0 Bytes, b100 means 100 Bytes and so on. E-UTRAN only configures this field for LWA DRBs.

uplinkDataCompression Indicates the UDC configuration that the UE shall apply. E-UTRAN does not configure uplinkDataCompression for a DRB, if ethernetHeaderCompression, headerCompression or uplinkOnlyHeaderCompression is already configured for the DRB. E-UTRAN does not configure uplinkDataCompression for the split and LWA DRBs.The maximum number of DRBs where uplinkDataCompression can be applied is two. In this version of the specification, for existing DRBs, E-UTRAN can only (re)configure uplinkDataCompression via handover procedure or the first RRCConnectionReconfiguration message after RRC connection re-establishment.

uplinkOnlyHeaderCompression Indicates the ROHC configuration that the UE shall apply uplink-only ROHC operations, see TS 36.323 [8]. E-UTRAN only configures this field when headerCompression is not configured. E-UTRAN does not reconfigure header compression for an MCG DRB except for upon handover and upon the first reconfiguration after RRC connection re-establishment. E-UTRAN does not reconfigure header compression for a SCG DRB except for upon SCG change involving PDCP re-establishment. For split and LWA DRBs E-UTRAN configures only notUsed.

| Conditional presence | Explanation |
| --- | --- |
| Rlc-AM-UM | The field is mandatory present upon setup of a PDCP entity for a radio bearer configured with RLC AM. The field is optional, need ON, in case of reconfiguration of a PDCP entity at handover, at the first reconfiguration after RRC re-establishment or at SCG change involving PDCP re-establishment or PDCP data recovery for a radio bearer configured with RLC AM. If the UE supports DAPS handover, this field is optional, need ON, for a radio bearer configured with RLC UM. Otherwise the field is not present. |
| Rlc-AM2 | The field is optionally present, need OP, upon setup of a PDCP entity for a radio bearer configured with RLC AM. Otherwise the field is not present. |
| Rlc-AM3 | The field is optionally present, need OP, upon setup of a PDCP entity for a radio bearer configured with RLC AM, if pdcp-SN-Size-v1130 is absent. Otherwise the field is not present. |
| Rlc-AM4 | The field is optionally present, need ON, upon setup of a PDCP entity for a radio bearer configured with RLC AM. The field is optional, need OP, in case of reconfiguration of a PDCP entity at handover, or at the first reconfiguration after RRC re-establishment. Otherwise the field is not present and the UE shall continue to use the existing value. |
| Rlc-UM | The field is mandatory present upon setup of a PDCP entity for a radio bearer configured with RLC UM. It is optionally present, Need ON, upon handover within E-UTRA, upon the first reconfiguration after re-establishment and upon SCG change involving PDCP re-establishment. Otherwise the field is not present. |
| RN | The field is optionally present when signalled to the RN, need OR. Otherwise the field is not present. |
| Setup | The field is mandatory present in case of radio bearer setup. Otherwise the field is optionally present, need ON. |
| SetupS | The field is mandatory present in case of setup of or reconfiguration to a split DRB or LWA DRB as well as in case of setup of or reconfiguration to a DRB associated with at least one RLC entity configured with rlc-OutOfOrderDelivery. The field is optionally present upon reconfiguration of a split DRB or LWA DRB or upon DRB type change from split to MCG DRB or from LWA to LTE only as well as upon reconfiguration of a DRB associated with at least one RLC entity configured with rlc-OutOfOrderDelivery, need ON. Otherwise the field is not present. |

#### –	PDSCH-Config

The IE PDSCH-ConfigCommon and the IE PDSCH-ConfigDedicated are used to specify the common and the UE specific PDSCH configuration respectively.

PDSCH-Config information element

-- ASN1START

PDSCH-ConfigCommon ::=		SEQUENCE {

referenceSignalPower				INTEGER (-60..50),

p-b									INTEGER (0..3)

}

PDSCH-ConfigCommon-v1310 ::=	SEQUENCE {

pdsch-maxNumRepetitionCEmodeA-r13	ENUMERATED {

r16, r32 }					OPTIONAL,	-- Need OR

pdsch-maxNumRepetitionCEmodeB-r13	ENUMERATED {

r192, r256, r384, r512, r768, r1024,

r1536, r2048}					OPTIONAL	-- Need OR

}

PDSCH-ConfigDedicated::=		SEQUENCE {

p-a									ENUMERATED {

dB-6, dB-4dot77, dB-3, dB-1dot77,

dB0, dB1, dB2, dB3}

}

PDSCH-ConfigDedicated-v1130 ::=		SEQUENCE {

dmrs-ConfigPDSCH-r11				DMRS-Config-r11					OPTIONAL,	-- Need ON

qcl-Operation						ENUMERATED {typeA, typeB}			OPTIONAL,	-- Need OR

re-MappingQCLConfigToReleaseList-r11	RE-MappingQCLConfigToReleaseList-r11	OPTIONAL,	-- Need ON

re-MappingQCLConfigToAddModList-r11		RE-MappingQCLConfigToAddModList-r11		OPTIONAL	-- Need ON

}

PDSCH-ConfigDedicated-v1280 ::=		SEQUENCE {

tbsIndexAlt-r12						ENUMERATED {a26, a33}				OPTIONAL	-- Need OR

}

PDSCH-ConfigDedicated-v1310 ::=		SEQUENCE {

dmrs-ConfigPDSCH-v1310				DMRS-Config-v1310					OPTIONAL	-- Need ON

}

PDSCH-ConfigDedicated-v1430 ::=		SEQUENCE {

ce-PDSCH-MaxBandwidth-r14			ENUMERATED {bw5, bw20}				OPTIONAL,	-- Need OP

ce-PDSCH-TenProcesses-r14			ENUMERATED {on}						OPTIONAL,	-- Need OR

ce-HARQ-AckBundling-r14				ENUMERATED {on}						OPTIONAL,	-- Need OR

ce-SchedulingEnhancement-r14		ENUMERATED {range1, range2}			OPTIONAL,	-- Need OR

tbsIndexAlt2-r14						ENUMERATED {b33}				OPTIONAL	-- Need OR

}

PDSCH-ConfigDedicated-v1530 ::=		SEQUENCE {

qcl-Operation-v1530						ENUMERATED {typeC}				OPTIONAL,	-- Need OR

tbs-IndexAlt3-r15							ENUMERATED {a37}			OPTIONAL,	-- Need OR

ce-CQI-AlternativeTableConfig-r15			ENUMERATED {on}				OPTIONAL,	-- Need OR

ce-PDSCH-64QAM-Config-r15					ENUMERATED {on}				OPTIONAL,	-- Need OR

ce-PDSCH-FlexibleStartPRB-AllocConfig-r15	ENUMERATED {on}				OPTIONAL,	-- Need OR

altMCS-TableScalingConfig-r15		ENUMERATED {oDot5, oDot625, oDot75, oDot875}	OPTIONAL	-- Need OR

}

PDSCH-ConfigDedicated-v1610 ::=		SEQUENCE {

ce-PDSCH-MultiTB-Config-r16		SetupRelease {CE-PDSCH-MultiTB-Config-r16}

}

PDSCH-ConfigDedicatedSCell-v1430 ::=		SEQUENCE {

tbsIndexAlt2-r14						ENUMERATED {b33}				OPTIONAL	-- Need OR

}

CE-PDSCH-MultiTB-Config-r16 ::= SEQUENCE {

interleaving-r16						ENUMERATED {on}		OPTIONAL,		-- Need OR

harq-AckBundling-r16					ENUMERATED {on}		OPTIONAL		-- Need OR

}

RE-MappingQCLConfigToAddModList-r11 ::=		SEQUENCE (SIZE (1..maxRE-MapQCL-r11)) OF PDSCH-RE-MappingQCL-Config-r11

RE-MappingQCLConfigToReleaseList-r11 ::=	SEQUENCE (SIZE (1..maxRE-MapQCL-r11)) OF PDSCH-RE-MappingQCL-ConfigId-r11

PDSCH-RE-MappingQCL-Config-r11 ::=		SEQUENCE {

pdsch-RE-MappingQCL-ConfigId-r11	PDSCH-RE-MappingQCL-ConfigId-r11,

optionalSetOfFields-r11				SEQUENCE {

crs-PortsCount-r11					ENUMERATED {n1, n2, n4, spare1},

crs-FreqShift-r11					INTEGER (0..5),

mbsfn-SubframeConfigList-r11		CHOICE {

release								NULL,

setup								SEQUENCE {

subframeConfigList					MBSFN-SubframeConfigList

}

}																	OPTIONAL,	-- Need ON

pdsch-Start-r11						ENUMERATED {reserved, n1, n2, n3, n4, assigned}

}																		OPTIONAL,	-- Need OP

csi-RS-ConfigZPId-r11				CSI-RS-ConfigZPId-r11,

qcl-CSI-RS-ConfigNZPId-r11			CSI-RS-ConfigNZPId-r11				OPTIONAL,	-- Need OR

...,

[[	mbsfn-SubframeConfigList-v1430	CHOICE {

release						NULL,

setup						SEQUENCE {

subframeConfigList-v1430	MBSFN-SubframeConfigList-v1430

}

}																	OPTIONAL	-- Need OP

]],

[[	codewordOneConfig-v1530				CHOICE {

release						NULL,

setup						SEQUENCE {

crs-PortsCount-v1530					ENUMERATED {n1, n2, n4, spare1},

crs-FreqShift-v1530						INTEGER (0..5),

mbsfn-SubframeConfigList-v1530			MBSFN-SubframeConfigList	OPTIONAL,

mbsfn-SubframeConfigListExt-v1530		MBSFN-SubframeConfigList-v1430 OPTIONAL,

pdsch-Start-v1530						ENUMERATED {reserved, n1, n2, n3, n4, assigned},

csi-RS-ConfigZPId-v1530					CSI-RS-ConfigZPId-r11,

qcl-CSI-RS-ConfigNZPId-v1530			CSI-RS-ConfigNZPId-r11	OPTIONAL

}

}																OPTIONAL	-- Cond TypeC

]]

}

-- ASN1STOP

| PDSCH-Config field descriptions |
| --- |
| altMCS-TableScalingConfig Presence of the field indicates activation of 6-bit MCS table (i.e., altMCS-Table) for UE indicating support for altMCS-Table, see TS 36.212 [22] and TS 36.213 [23]. The indicated value configures the parameter altMCS-Table-Scaling where value oDot5 corresponds to scaling factor 0.5, value oDot625 corresponds to scaling factor 0.625 and so on, see TS 36.213 [23]. |
| ce-CQI-AlternativeTableConfig Configures the UE supporting alternative CQI table to use the alternative CQI table in CE mode A. See TS 36.213 [23]. |
| ce-HARQ-AckBundling Activation of PDSCH HARQ-ACK bundling in half duplex FDD in CE mode A, see TS 36.212 [22] and TS 36.213 [23]. |
| ce-PDSCH-64QAM-Config Activation of 64 QAM for non-repeated unicast PDSCH in CE mode A. |
| ce-PDSCH-FlexibleStartPRB-AllocConfig Activation of flexible starting PRB for PDSCH resource allocation in CE mode A or B. E-UTRAN does not configure this field when E-UTRA system bandwidth is 1.4 MHz. |
| ce-PDSCH-MaxBandwidth Maximum PDSCH channel bandwidth in CE mode A and B, see TS 36.212 [22] and TS 36.213 [23]. Value bw5 corresponds to 5 MHz, and value bw20 corresponds to 20 MHz. If this field is absent, the UE shall release any existing value and set the maximum PDSCH channel bandwidth in CE mode A and B to 1.4 MHz. Parameter: transmission bandwidth configuration, see TS 36.101 [42], table 5.6-1. The max bandwidth can by configured to 5MHz for BL UEs and 5MHz or 20MHz for UEs in CE. |
| ce-PDSCH-MultiTB-Config Indicates whether DL multi-TB scheduling is enabled, i.e., a single DCI can schedule up to 8 PDSCH transport blocks in CE mode A and up to 4 PDSCH transport blocks in CE mode B. See TS 36.213 [23], clause 7.1.11. |  |
| ce-PDSCH-TenProcesses Configuration of 10 (instead of 8) DL HARQ processes in FDD in CE mode A, see TS 36.212 [22] and TS 36.213 [23]. |
| ce-SchedulingEnhancement Activation of dynamic HARQ-ACK delay for HD-FDD for PDSCH in CE mode A controlled by the DCI, see TS 36.212 [22] and TS 36.213 [23]. Value range1 corresponds to the first range of HARQ-ACK delays, and value range2 corresponds to second range of HARQ-ACK delays. |
| codewordOneConfig The field corresponds to codeword 1, see TS 36.213 [23], clause 7.1.10. If absent, the UE applies the values from the serving cell configured on the same frequency. |
| harq-AckBundling Indicates whether HARQ-ACK bundling for DL multi-TB scheduling is enabled, see TS 36.213 [23], clause 7.3. |
| interleaving Indicates whether interleaving for DL multi-TB scheduling is enabled, see TS 36.213 [23], clause 7.1.11. |
| mbsfn-SubframeConfigList Indicates the MBSFN configuration for the CSI-RS resources. If optionalSetOfFields is absent, the fields mbsfn-SubframeConfigList-r11 and mbsfn-SubframeConfigList-v1430 are released. |
| optionalSetOfFields If absent, the UE releases the configuration provided previously, if any, and applies the values from the serving cell configured on the same frequency. If the UE is configured with qcl-Operation-v1530, this field corresponds to codeword 0, see TS 36.213 [23], clause 7.1.10. |
| p-a Parameter: , see TS 36.213 [23], clause 5.2. Value dB-6 corresponds to -6 dB, dB-4dot77 corresponds to -4.77 dB etc. |
| p-b Parameter: , see TS 36.213 [23], clause Table 5.2-1. |
| pdsch-maxNumRepetitionCEmodeA Maximum value to indicate the set of PDSCH repetition numbers for CE mode A, see TS 36.211 [21] and TS 36.213 [23]. |
| pdsch-maxNumRepetitionCEmodeB Maximum value to indicate the set of PDSCH repetition numbers for CE mode B, see TS 36.211 [21] and TS 36.213 [23]. |
| pdsch-Start The starting OFDM symbol of PDSCH for the concerned serving cell, see TS 36.213 [23], clause 7.1.6.4. Values 1, 2, 3 are applicable when dl-Bandwidth for the concerned serving cell is greater than 10 resource blocks, values 2, 3, 4 are applicable when dl-Bandwidth for the concerned serving cell is less than or equal to 10 resource blocks, see TS 36.211 [21], Table 6.7-1. Value n1 corresponds to 1, value n2 corresponds to 2 and so on. If the field pdsch-Start-v1530 is also configured, E-UTRAN ensures that this value is the same as pdsch-Start (i.e., without suffix). |
| qcl-CSI-RS-ConfigNZPId Indicates the CSI-RS resource that is quasi co-located with the PDSCH antenna ports, see TS 36.213 [23], clause 7.1.9. E-UTRAN configures this field if and only if the UE is configured with qcl-Operation set to typeB or qcl-Operation-v1530 set to typeC. If the UE is configured with qcl-Operation-v1530 set to typeC, the field qcl-CSI-RS-ConfigNZPId-r11 corresponds to codeword 0, and the field qcl-CSI-RS-ConfigNZPId-v1530 corresponds to codeword 1, see TS 36.213 [23], clause 7.1.10.. |
| qcl-Operation Indicates the quasi co-location behaviour to be used by the UE, type A, type B, or type C, as described in TS 36.213 [23], clause 7.1.10. In case qcl-Operation-v1530 is present, the UE shall ignore the field qcl-Operation (without suffix). E-UTRAN configures qcl-Operation-v1530 only when transmission mode 10 is configured for the serving cell on this carrier frequency and QCL type C is configured. |
| referenceSignalPower Parameter: Reference-signal power, which provides the downlink reference-signal EPRE, see TS 36.213 [23], clause 5.2. The actual value in dBm. |
| re-MappingQCLConfigToAddModList, re-MappingQCLConfigToReleaseList For a serving frequency E-UTRAN configures at least one PDSCH-RE-MappingQCL-Config when transmission mode 10 is configured for the serving cell on this carrier frequency. Otherwise it does not configure this field. |
| tbsIndexAlt Indicates the applicability of the alternative TBS index for the ITBS 26 and 33 (see TS 36.213 [23], Table 7.1.7.2.1-1), to all subframes scheduled by DCI format 2C or 2D. Value a26 refers to the alternative TBS index ITBS 26A, and value a33 refers to the alternative TBS index ITBS 33A. If this field is not configured, the UE shall use ITBS 26 specified in Table 7.1.7.2.1-1 in TS 36.213 [23] for all subframes instead. If neither this field nor tbsIndexAlt2 configures an alternative TBS index for ITBS 33, the UE shall use ITBS 33 specified in Table 7.1.7.2.1-1 in TS 36.213 [23] for all subframes instead. |
| tbsIndexAlt2 Indicates the applicability of the alternative TBS index for the ITBS 33 (see TS 36.213 [23], Table 7.1.7.2.1-1) to all subframes. Value b33 refers to the alternative TBS index ITBS 33B. If neither this field nor tbsIndexAlt configures an alternative TBS index for ITBS 33, the UE shall use ITBS 33 specified in Table 7.1.7.2.1-1 in TS 36.213 [23] for all subframes instead. |
| tbs-IndexAlt3 Indicates the applicability of the alternative TBS index for the ITBS 37 (see TS 36.213 [23], Table 7.1.7.2.1-1) to all subframes. Value a37 refers to the alternative TBS index ITBS 37A. |

| Conditional presence | Explanation |
| --- | --- |
| TypeC | The field is optional, need ON when qcl-Operation is configured with typeC. Otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	PDSCH-RE-MappingQCL-ConfigId

The IE PDSCH-RE-MappingQCL-ConfigId is used to identify a set of PDSCH parameters related to resource element mapping and quasi co-location, as configured by the IE PDSCH-RE-MappingQCL-Config. The identity is unique within the scope of a carrier frequency.

PDSCH-RE-MappingQCL-ConfigId information elements

-- ASN1START

PDSCH-RE-MappingQCL-ConfigId-r11 ::=		INTEGER (1..maxRE-MapQCL-r11)

-- ASN1STOP

#### –	PerCC-GapIndicationList

The IE PerCC-GapIndicationList is used to specify the UE measurement gap preference.

PerCC-GapIndication information elements

-- ASN1START

PerCC-GapIndicationList-r14 ::=	SEQUENCE (SIZE (1..maxServCell-r13)) OF PerCC-GapIndication-r14

PerCC-GapIndication-r14 ::=			SEQUENCE {

servCellId-r14								ServCellIndex-r13,

gapIndication-r14							ENUMERATED {gap, ncsg, nogap-noNcsg}

}

-- ASN1STOP

PerCC-GapIndication field descriptions

servCellId This field identifies the serving cell for which the measurement gap perference is provided.

gapIndication This field is used to indicate the measurement gap preference per component carrier (serving cell) by the UE both in non-CA and CA configurations. Value gap indicates that a measurement gap is needed for the associated servCellId, value nogap-noNcsg indicates that neither a measurement gap nor a ncsg is needed for the associated servCellId, value ncsg indicates that ncsg is needed for the associated servCellId. The UE shall indicate the per CC measurement gap preference consistently for the same non-CA or CA configuration and measurement configuration during the same RRC connection.

#### –	PHICH-Config

The IE PHICH-Config is used to specify the PHICH configuration.

PHICH-Config information element

-- ASN1START

PHICH-Config ::=					SEQUENCE {

phich-Duration						ENUMERATED {normal, extended},

phich-Resource						ENUMERATED {oneSixth, half, one, two}

}

-- ASN1STOP

PHICH-Config field descriptions

phich-Duration Parameter: PHICH-Duration, see TS 36.211 [21], Table 6.9.3-1.

phich-Resource Parameter: Ng, see TS 36.211 [21], clause 6.9. Value oneSixth corresponds to 1/6, half corresponds to 1/2 and so on.

#### –	PhysicalConfigDedicated

The IE PhysicalConfigDedicated is used to specify the UE specific physical channel configuration.

PhysicalConfigDedicated information element

-- ASN1START

PhysicalConfigDedicated ::=		SEQUENCE {

pdsch-ConfigDedicated				PDSCH-ConfigDedicated			OPTIONAL,		-- Need ON

pucch-ConfigDedicated				PUCCH-ConfigDedicated			OPTIONAL,		-- Need ON

pusch-ConfigDedicated				PUSCH-ConfigDedicated			OPTIONAL,		-- Need ON

uplinkPowerControlDedicated			UplinkPowerControlDedicated		OPTIONAL,		-- Need ON

tpc-PDCCH-ConfigPUCCH				TPC-PDCCH-Config				OPTIONAL,		-- Need ON

tpc-PDCCH-ConfigPUSCH				TPC-PDCCH-Config				OPTIONAL,		-- Need ON

cqi-ReportConfig					CQI-ReportConfig				OPTIONAL,		-- Cond CQI-r8

soundingRS-UL-ConfigDedicated		SoundingRS-UL-ConfigDedicated	OPTIONAL,		-- Need ON

antennaInfo							CHOICE {

explicitValue						AntennaInfoDedicated,

defaultValue						NULL

}																	OPTIONAL,	-- Cond AI-r8

schedulingRequestConfig				SchedulingRequestConfig			OPTIONAL,		-- Need ON

...,

[[	cqi-ReportConfig-v920				CQI-ReportConfig-v920		OPTIONAL,		-- Cond CQI-r8

antennaInfo-v920					AntennaInfoDedicated-v920	OPTIONAL		-- Cond AI-r8

]],

[[	antennaInfo-r10					CHOICE {

explicitValue-r10				AntennaInfoDedicated-r10,

defaultValue					NULL

}																OPTIONAL,	-- Cond AI-r10

antennaInfoUL-r10				AntennaInfoUL-r10				OPTIONAL,		-- Need ON

cif-Presence-r10				BOOLEAN							OPTIONAL,		-- Need ON

cqi-ReportConfig-r10			CQI-ReportConfig-r10			OPTIONAL,	-- Cond CQI-r10

csi-RS-Config-r10				CSI-RS-Config-r10				OPTIONAL,		-- Need ON

pucch-ConfigDedicated-v1020		PUCCH-ConfigDedicated-v1020		OPTIONAL,		-- Need ON

pusch-ConfigDedicated-v1020		PUSCH-ConfigDedicated-v1020		OPTIONAL,		-- Need ON

schedulingRequestConfig-v1020	SchedulingRequestConfig-v1020	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicated-v1020

SoundingRS-UL-ConfigDedicated-v1020		OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedAperiodic-r10

SoundingRS-UL-ConfigDedicatedAperiodic-r10	OPTIONAL,		-- Need ON

uplinkPowerControlDedicated-v1020

UplinkPowerControlDedicated-v1020	OPTIONAL		-- Need ON

]],

[[	additionalSpectrumEmissionCA-r10			CHOICE {

release									NULL,

setup									SEQUENCE {

additionalSpectrumEmissionPCell-r10		AdditionalSpectrumEmission

}

}			OPTIONAL	-- Need ON

]],

[[	-- DL configuration as well as configuration applicable for DL and UL

csi-RS-ConfigNZPToReleaseList-r11

CSI-RS-ConfigNZPToReleaseList-r11	OPTIONAL,		-- Need ON

csi-RS-ConfigNZPToAddModList-r11

CSI-RS-ConfigNZPToAddModList-r11	OPTIONAL,		-- Need ON

csi-RS-ConfigZPToReleaseList-r11

CSI-RS-ConfigZPToReleaseList-r11	OPTIONAL,		-- Need ON

csi-RS-ConfigZPToAddModList-r11	CSI-RS-ConfigZPToAddModList-r11	OPTIONAL,		-- Need ON

epdcch-Config-r11				EPDCCH-Config-r11				OPTIONAL,		-- Need ON

pdsch-ConfigDedicated-v1130		PDSCH-ConfigDedicated-v1130		OPTIONAL,		-- Need ON

-- UL configuration

cqi-ReportConfig-v1130			CQI-ReportConfig-v1130			OPTIONAL,		-- Need ON

pucch-ConfigDedicated-v1130		PUCCH-ConfigDedicated-v1130		OPTIONAL,		-- Need ON

pusch-ConfigDedicated-v1130		PUSCH-ConfigDedicated-v1130		OPTIONAL,		-- Need ON

uplinkPowerControlDedicated-v1130

UplinkPowerControlDedicated-v1130	OPTIONAL		-- Need ON

]],

[[	antennaInfo-v1250				AntennaInfoDedicated-v1250		OPTIONAL,	-- Cond AI-r10

eimta-MainConfig-r12			EIMTA-MainConfig-r12			OPTIONAL,		-- Need ON

eimta-MainConfigPCell-r12		EIMTA-MainConfigServCell-r12	OPTIONAL,		-- Need ON

pucch-ConfigDedicated-v1250		PUCCH-ConfigDedicated-v1250		OPTIONAL,		-- Need ON

cqi-ReportConfigPCell-v1250		CQI-ReportConfig-v1250			OPTIONAL,		-- Need ON

uplinkPowerControlDedicated-v1250

UplinkPowerControlDedicated-v1250	OPTIONAL,		-- Need ON

pusch-ConfigDedicated-v1250		PUSCH-ConfigDedicated-v1250		OPTIONAL,		-- Need ON

csi-RS-Config-v1250					CSI-RS-Config-v1250			OPTIONAL		-- Need ON

]],

[[	pdsch-ConfigDedicated-v1280			PDSCH-ConfigDedicated-v1280	OPTIONAL		-- Need ON

]],

[[	pdsch-ConfigDedicated-v1310			PDSCH-ConfigDedicated-v1310	OPTIONAL,		-- Need ON

pucch-ConfigDedicated-r13			PUCCH-ConfigDedicated-r13	OPTIONAL,		-- Need ON

pusch-ConfigDedicated-r13			PUSCH-ConfigDedicated-r13	OPTIONAL,		-- Need ON

pdcch-CandidateReductions-r13

PDCCH-CandidateReductions-r13	OPTIONAL,		-- Need ON

cqi-ReportConfig-v1310					CQI-ReportConfig-v1310	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicated-v1310

SoundingRS-UL-ConfigDedicated-v1310		OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedUpPTsExt-r13

SoundingRS-UL-ConfigDedicatedUpPTsExt-r13		OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedAperiodic-v1310

SoundingRS-UL-ConfigDedicatedAperiodic-v1310	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13

SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13		OPTIONAL,		-- Need ON

csi-RS-Config-v1310				CSI-RS-Config-v1310				OPTIONAL,		-- Need ON

ce-Mode-r13					CHOICE {

release						NULL,

setup						ENUMERATED {ce-ModeA,ce-ModeB}

}																OPTIONAL,		-- Need ON

csi-RS-ConfigNZPToAddModListExt-r13	CSI-RS-ConfigNZPToAddModListExt-r13	OPTIONAL,	-- Need ON

csi-RS-ConfigNZPToReleaseListExt-r13	CSI-RS-ConfigNZPToReleaseListExt-r13	OPTIONAL	-- Need ON

]],

[[	cqi-ReportConfig-v1320					CQI-ReportConfig-v1320	OPTIONAL		-- Need ON

]],

[[	typeA-SRS-TPC-PDCCH-Group-r14	CHOICE {

release							NULL,

setup							SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config-r14

}																OPTIONAL,		-- Need ON

must-Config-r14						CHOICE{

release								NULL,

setup								SEQUENCE {

k-max-r14						ENUMERATED {l1, l3},

p-a-must-r14					ENUMERATED {

dB-6, dB-4dot77, dB-3, dB-1dot77,

dB0, dB1, dB2, dB3}	OPTIONAL		-- Need ON

}

}															OPTIONAL,		-- Need ON

pusch-EnhancementsConfig-r14		PUSCH-EnhancementsConfig-r14		OPTIONAL,	-- Need ON

ce-pdsch-pusch-EnhancementConfig-r14		ENUMERATED {on}	OPTIONAL,	-- Need OR

antennaInfo-v1430				AntennaInfoDedicated-v1430		OPTIONAL,	-- Need ON

pucch-ConfigDedicated-v1430		PUCCH-ConfigDedicated-v1430		OPTIONAL,	-- Need ON

pdsch-ConfigDedicated-v1430		PDSCH-ConfigDedicated-v1430		OPTIONAL,		-- Need ON

pusch-ConfigDedicated-v1430		PUSCH-ConfigDedicated-v1430	OPTIONAL,		-- Need ON

soundingRS-UL-PeriodicConfigDedicatedList-r14			SEQUENCE (SIZE (1..2)) OF SoundingRS-UL-ConfigDedicated	OPTIONAL,		-- Cond PeriodicSRSPCell

soundingRS-UL-PeriodicConfigDedicatedUpPTsExtList-r14	SEQUENCE (SIZE (1..4)) OF SoundingRS-UL-ConfigDedicatedUpPTsExt-r13	OPTIONAL,		-- Cond PeriodicSRSExt

soundingRS-UL-AperiodicConfigDedicatedList-r14			SEQUENCE (SIZE (1..2)) OF SoundingRS-UL-ConfigDedicatedAperiodic-r10	OPTIONAL,		-- Cond AperiodicSRS

soundingRS-UL-ConfigDedicatedApUpPTsExtList-r14	SEQUENCE (SIZE (1..4)) OF SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13	OPTIONAL,		-- Cond AperiodicSRSExt

csi-RS-Config-v1430				CSI-RS-Config-v1430				OPTIONAL,		-- Need ON

csi-RS-ConfigZP-ApList-r14				CSI-RS-ConfigZP-ApList-r14	OPTIONAL,	-- Need ON

cqi-ReportConfig-v1430					CQI-ReportConfig-v1430	OPTIONAL,	-- Need ON

semiOpenLoop-r14						BOOLEAN					OPTIONAL	-- Need ON

]],

[[	csi-RS-Config-v1480					CSI-RS-Config-v1480			OPTIONAL		-- Need ON

]],

[[	physicalConfigDedicatedSTTI-r15		PhysicalConfigDedicatedSTTI-r15	OPTIONAL,-- Need ON

pdsch-ConfigDedicated-v1530			PDSCH-ConfigDedicated-v1530		OPTIONAL,-- Need ON

pusch-ConfigDedicated-v1530			PUSCH-ConfigDedicated-v1530		OPTIONAL,-- Need ON

cqi-ReportConfig-v1530				CQI-ReportConfig-v1530			OPTIONAL,-- Need ON

antennaInfo-v1530					AntennaInfoDedicated-v1530		OPTIONAL,-- Need ON

csi-RS-Config-v1530					CSI-RS-Config-v1530				OPTIONAL,-- Need ON

uplinkPowerControlDedicated-v1530

UplinkPowerControlDedicated-v1530	OPTIONAL,		-- Need ON

semiStaticCFI-Config-r15		CHOICE{

release							NULL,

setup							CHOICE{

cfi-Config-r15					CFI-Config-r15,

cfi-PatternConfig-r15			CFI-PatternConfig-r15

}

}														OPTIONAL,	 -- Need ON

blindPDSCH-Repetition-Config-r15	CHOICE{

release							NULL,

setup							SEQUENCE {

blindSubframePDSCH-Repetitions-r15		BOOLEAN,

blindSlotSubslotPDSCH-Repetitions-r15	BOOLEAN,

maxNumber-SubframePDSCH-Repetitions-r15	ENUMERATED {n4,n6}	OPTIONAL,	-- Need ON

maxNumber-SlotSubslotPDSCH-Repetitions-r15	ENUMERATED {n4,n6}		OPTIONAL, -- Need ON

rv-SubframePDSCH-Repetitions-r15	ENUMERATED {dlrvseq1, dlrvseq2}	OPTIONAL, -- Need ON

rv-SlotsublotPDSCH-Repetitions-r15	ENUMERATED {dlrvseq1, dlrvseq2}	OPTIONAL, -- Need ON

numberOfProcesses-SubframePDSCH-Repetitions-r15	INTEGER(1..16)	OPTIONAL, -- Need ON

numberOfProcesses-SlotSubslotPDSCH-Repetitions-r15	INTEGER(1..16)	OPTIONAL, -- Need ON

mcs-restrictionSubframePDSCH-Repetitions-r15	ENUMERATED {n0, n1}	OPTIONAL, -- Need ON

mcs-restrictionSlotSubslotPDSCH-Repetitions-r15	ENUMERATED {n0, n1}	OPTIONAL -- Need ON

}

}														OPTIONAL	 -- Need ON

]],

[[	spucch-Config-v1550				SPUCCH-Config-v1550			OPTIONAL -- Need ON

]],

[[	pdsch-ConfigDedicated-v1610		PDSCH-ConfigDedicated-v1610		OPTIONAL,  -- Need ON

pusch-ConfigDedicated-v1610		PUSCH-ConfigDedicated-v1610		OPTIONAL,  -- Need ON

ce-CSI-RS-Feedback-r16			ENUMERATED {enabled}			OPTIONAL,  -- Need OR

resourceReservationConfigDedicatedDL-r16	SetupRelease {ResourceReservationConfigDedicatedDL-r16}		OPTIONAL,  -- Need ON

resourceReservationConfigDedicatedUL-r16	SetupRelease {ResourceReservationConfigDedicatedUL-r16}		OPTIONAL,  -- Need ON

soundingRS-UL-ConfigDedicatedAdd-r16		SetupRelease {SoundingRS-UL-ConfigDedicatedAdd-r16}

OPTIONAL, --  Need ON

uplinkPowerControlAddSRS-r16	SetupRelease {UplinkPowerControlAddSRS-r16}	OPTIONAL,  -- Need ON

soundingRS-VirtualCellID-r16		SetupRelease {SoundingRS-VirtualCellID-r16}	OPTIONAL,  -- Need ON

widebandPRG-r16					SetupRelease {WidebandPRG-r16}				OPTIONAL   -- Need ON

]],

[[	pusch-ConfigDedicated-v17xx		PUSCH-ConfigDedicated-v17xx 	OPTIONAL,	-- Need OR

pucch-ConfigDedicated-v17xx		PUCCH-ConfigDedicated-v17xx 	OPTIONAL	-- Need OR

]]

}

PhysicalConfigDedicated-v1370 ::=	SEQUENCE {

pucch-ConfigDedicated-v1370			PUCCH-ConfigDedicated-v1370		OPTIONAL		-- Cond PUCCH-Format4or5

}

PhysicalConfigDedicated-v13c0 ::=	SEQUENCE {

pucch-ConfigDedicated-v13c0			PUCCH-ConfigDedicated-v13c0

}

PhysicalConfigDedicatedSCell-r10 ::=		SEQUENCE {

-- DL configuration as well as configuration applicable for DL and UL

nonUL-Configuration-r10					SEQUENCE {

antennaInfo-r10

AntennaInfoDedicated-r10	OPTIONAL,		-- Need ON

crossCarrierSchedulingConfig-r10

CrossCarrierSchedulingConfig-r10	OPTIONAL,		-- Need ON

csi-RS-Config-r10						CSI-RS-Config-r10		OPTIONAL,		-- Need ON

pdsch-ConfigDedicated-r10				PDSCH-ConfigDedicated	OPTIONAL		-- Need ON

}																	OPTIONAL,	-- Cond SCellAdd

-- UL configuration

ul-Configuration-r10					SEQUENCE {

antennaInfoUL-r10						AntennaInfoUL-r10		OPTIONAL,		-- Need ON

pusch-ConfigDedicatedSCell-r10

PUSCH-ConfigDedicatedSCell-r10		OPTIONAL,	-- Cond PUSCH-SCell1

uplinkPowerControlDedicatedSCell-r10

UplinkPowerControlDedicatedSCell-r10	OPTIONAL,		-- Need ON

cqi-ReportConfigSCell-r10			CQI-ReportConfigSCell-r10	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicated-r10

SoundingRS-UL-ConfigDedicated	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicated-v1020

SoundingRS-UL-ConfigDedicated-v1020	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedAperiodic-r10

SoundingRS-UL-ConfigDedicatedAperiodic-r10	OPTIONAL	-- Need ON

}																	OPTIONAL,	-- Cond CommonUL

...,

[[	-- DL configuration as well as configuration applicable for DL and UL

csi-RS-ConfigNZPToReleaseList-r11

CSI-RS-ConfigNZPToReleaseList-r11	OPTIONAL,		-- Need ON

csi-RS-ConfigNZPToAddModList-r11

CSI-RS-ConfigNZPToAddModList-r11	OPTIONAL,		-- Need ON

csi-RS-ConfigZPToReleaseList-r11

CSI-RS-ConfigZPToReleaseList-r11	OPTIONAL,		-- Need ON

csi-RS-ConfigZPToAddModList-r11

CSI-RS-ConfigZPToAddModList-r11	OPTIONAL,		-- Need ON

epdcch-Config-r11					EPDCCH-Config-r11			OPTIONAL,		-- Need ON

pdsch-ConfigDedicated-v1130			PDSCH-ConfigDedicated-v1130	OPTIONAL,		-- Need ON

-- UL configuration

cqi-ReportConfig-v1130				CQI-ReportConfig-v1130		OPTIONAL,		-- Need ON

pusch-ConfigDedicated-v1130

PUSCH-ConfigDedicated-v1130		OPTIONAL,	-- Cond PUSCH-SCell1

uplinkPowerControlDedicatedSCell-v1130

UplinkPowerControlDedicated-v1130	OPTIONAL		-- Need ON

]],

[[	antennaInfo-v1250					AntennaInfoDedicated-v1250	OPTIONAL,		-- Need ON

eimta-MainConfigSCell-r12

EIMTA-MainConfigServCell-r12	OPTIONAL,		-- Need ON

cqi-ReportConfigSCell-v1250			CQI-ReportConfig-v1250		OPTIONAL,		-- Need ON

uplinkPowerControlDedicatedSCell-v1250

UplinkPowerControlDedicated-v1250	OPTIONAL,		-- Need ON

csi-RS-Config-v1250					CSI-RS-Config-v1250			OPTIONAL		-- Need ON

]],

[[	pdsch-ConfigDedicated-v1280			PDSCH-ConfigDedicated-v1280	OPTIONAL		-- Need ON

]],

[[	pucch-Cell-r13						ENUMERATED {true}		OPTIONAL,	-- Cond PUCCH-SCell1

pucch-SCell							CHOICE{

release								NULL,

setup								SEQUENCE {

pucch-ConfigDedicated-r13

PUCCH-ConfigDedicated-r13	OPTIONAL,		-- Need ON

schedulingRequestConfig-r13

SchedulingRequestConfigSCell-r13	OPTIONAL,		-- Need ON

tpc-PDCCH-ConfigPUCCH-SCell-r13

TPC-PDCCH-ConfigSCell-r13	OPTIONAL,		-- Need ON

pusch-ConfigDedicated-r13

PUSCH-ConfigDedicated-r13	OPTIONAL,	-- Cond PUSCH-SCell

uplinkPowerControlDedicated-r13

UplinkPowerControlDedicatedSCell-v1310	OPTIONAL	-- Need ON

}

}																OPTIONAL,	-- Need ON

crossCarrierSchedulingConfig-r13

CrossCarrierSchedulingConfig-r13	OPTIONAL,	-- Cond Cross-Carrier-Config

pdcch-ConfigSCell-r13				PDCCH-ConfigSCell-r13		OPTIONAL,		-- Need ON

cqi-ReportConfig-v1310				CQI-ReportConfig-v1310		OPTIONAL,		-- Need ON

pdsch-ConfigDedicated-v1310			PDSCH-ConfigDedicated-v1310	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicated-v1310

SoundingRS-UL-ConfigDedicated-v1310		OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedUpPTsExt-r13

SoundingRS-UL-ConfigDedicatedUpPTsExt-r13	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedAperiodic-v1310

SoundingRS-UL-ConfigDedicatedAperiodic-v1310	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13

SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13	OPTIONAL,		-- Need ON

csi-RS-Config-v1310					CSI-RS-Config-v1310			OPTIONAL,		-- Need ON

laa-SCellConfiguration-r13			LAA-SCellConfiguration-r13	OPTIONAL,		-- Need ON

csi-RS-ConfigNZPToAddModListExt-r13	CSI-RS-ConfigNZPToAddModListExt-r13	OPTIONAL,	-- Need ON

csi-RS-ConfigNZPToReleaseListExt-r13	CSI-RS-ConfigNZPToReleaseListExt-r13	OPTIONAL	-- Need ON

]],

[[	cqi-ReportConfig-v1320				CQI-ReportConfig-v1320	OPTIONAL		-- Need ON

]],

[[	laa-SCellConfiguration-v1430		LAA-SCellConfiguration-v1430

OPTIONAL,	-- Need ON

typeB-SRS-TPC-PDCCH-Config-r14		SRS-TPC-PDCCH-Config-r14	OPTIONAL,	-- Need ON

uplinkPUSCH-LessPowerControlDedicated-v1430		UplinkPUSCH-LessPowerControlDedicated-v1430 OPTIONAL,		-- Need ON

soundingRS-UL-PeriodicConfigDedicatedList-r14					SEQUENCE (SIZE (1..2)) OF SoundingRS-UL-ConfigDedicated						OPTIONAL,		-- Cond PeriodicSRS

soundingRS-UL-PeriodicConfigDedicatedUpPTsExtList-r14					SEQUENCE (SIZE (1..4)) OF SoundingRS-UL-ConfigDedicatedUpPTsExt-r13						OPTIONAL,		-- Cond PeriodicSRSExt

soundingRS-UL-AperiodicConfigDedicatedList-r14					SEQUENCE (SIZE (1..2)) OF SoundingRS-AperiodicSet-r14						OPTIONAL,		-- Cond AperiodicSRS

soundingRS-UL-ConfigDedicatedApUpPTsExtList-r14					SEQUENCE (SIZE (1..4)) OF SoundingRS-AperiodicSetUpPTsExt-r14			OPTIONAL,		-- Cond AperiodicSRSExt

must-Config-r14						CHOICE{

release								NULL,

setup								SEQUENCE {

k-max-r14						ENUMERATED {l1, l3},

p-a-must-r14					ENUMERATED {

dB-6, dB-4dot77, dB-3, dB-1dot77,

dB0, dB1, dB2, dB3}	OPTIONAL		-- Need ON

}

}															OPTIONAL,		-- Need ON

pusch-ConfigDedicated-v1430			PUSCH-ConfigDedicatedSCell-v1430	OPTIONAL,	-- Need ON

csi-RS-Config-v1430						CSI-RS-Config-v1430			OPTIONAL,	-- Need ON

csi-RS-ConfigZP-ApList-r14				CSI-RS-ConfigZP-ApList-r14		OPTIONAL,	-- Need ON

cqi-ReportConfig-v1430					CQI-ReportConfig-v1430	OPTIONAL,	-- Need ON

semiOpenLoop-r14						BOOLEAN						OPTIONAL,	-- Need ON

pdsch-ConfigDedicatedSCell-v1430		PDSCH-ConfigDedicatedSCell-v1430		OPTIONAL		-- Need ON

]],

[[	csi-RS-Config-v1480					CSI-RS-Config-v1480				OPTIONAL	-- Need ON

]],

[[	physicalConfigDedicatedSTTI-r15		PhysicalConfigDedicatedSTTI-r15	OPTIONAL,	-- Need ON

pdsch-ConfigDedicated-v1530			PDSCH-ConfigDedicated-v1530		OPTIONAL,	-- Need ON

dummy								CQI-ReportConfig-v1530			OPTIONAL,	-- Need ON

cqi-ReportConfigSCell-r15			CQI-ReportConfigSCell-r15		OPTIONAL,	-- Need ON

cqi-ShortConfigSCell-r15			CQI-ShortConfigSCell-r15		OPTIONAL,	-- Need ON

csi-RS-Config-v1530					CSI-RS-Config-v1530				OPTIONAL,	-- Need ON

uplinkPowerControlDedicatedSCell-v1530

UplinkPowerControlDedicated-v1530	OPTIONAL,		-- Need ON

laa-SCellConfiguration-v1530		LAA-SCellConfiguration-v1530	OPTIONAL,	-- Need ON

pusch-ConfigDedicated-v1530			PUSCH-ConfigDedicatedScell-v1530	OPTIONAL,	-- Cond AUL

semiStaticCFI-Config-r15		CHOICE{

release							NULL,

setup							CHOICE{

cfi-Config-r15					CFI-Config-r15,

cfi-PatternConfig-r15			CFI-PatternConfig-r15

}

}														OPTIONAL,	 -- Need ON

blindPDSCH-Repetition-Config-r15	CHOICE{

release							NULL,

setup							SEQUENCE {

blindSubframePDSCH-Repetitions-r15		BOOLEAN,

blindSlotSubslotPDSCH-Repetitions-r15	BOOLEAN,

maxNumber-SubframePDSCH-Repetitions-r15	ENUMERATED {n4,n6}	OPTIONAL,	-- Need ON

maxNumber-SlotSubslotPDSCH-Repetitions-r15	ENUMERATED {n4,n6}		OPTIONAL, -- Need ON

rv-SubframePDSCH-Repetitions-r15	ENUMERATED {dlrvseq1, dlrvseq2}	OPTIONAL, -- Need ON

rv-SlotsublotPDSCH-Repetitions-r15	ENUMERATED {dlrvseq1, dlrvseq2}	OPTIONAL, -- Need ON

numberOfProcesses-SubframePDSCH-Repetitions-r15	INTEGER(1..16)	OPTIONAL, -- Need ON

numberOfProcesses-SlotSubslotPDSCH-Repetitions-r15	INTEGER(1..16)	OPTIONAL, -- Need ON

mcs-restrictionSubframePDSCH-Repetitions-r15	ENUMERATED {n0, n1}	OPTIONAL, -- Need ON

mcs-restrictionSlotSubslotPDSCH-Repetitions-r15	ENUMERATED {n0, n1}	OPTIONAL -- Need ON

}

}														OPTIONAL	 -- Need ON

]],

[[	spucch-Config-v1550				SPUCCH-Config-v1550			OPTIONAL -- Need ON

]],

[[	soundingRS-UL-ConfigDedicatedAdd-r16		SetupRelease {SoundingRS-UL-ConfigDedicatedAdd-r16}

OPTIONAL, -- Need ON

uplinkPowerControlAddSRS-r16				SetupRelease {UplinkPowerControlAddSRS-r16}

OPTIONAL, -- Need ON

soundingRS-VirtualCellID-r16				SetupRelease {SoundingRS-VirtualCellID-r16}

OPTIONAL,  -- Need ON

widebandPRG-r16							SetupRelease {WidebandPRG-r16}		OPTIONAL -- Need ON

]]

}

PhysicalConfigDedicatedSCell-v1370 ::=	SEQUENCE {

pucch-SCell-v1370				CHOICE{

release							NULL,

setup							SEQUENCE {

pucch-ConfigDedicated-v1370		PUCCH-ConfigDedicated-v1370		OPTIONAL	-- Cond PUCCH-Format4or5

}

}

}

PhysicalConfigDedicatedSCell-v13c0 ::=	SEQUENCE {

pucch-SCell-v13c0				CHOICE{

release							NULL,

setup							SEQUENCE {

pucch-ConfigDedicated-v13c0		PUCCH-ConfigDedicated-v13c0

}

}

}

CFI-Config-r15	::= SEQUENCE {

cfi-SubframeNonMBSFN-r15		INTEGER (1..4)						OPTIONAL,	 -- Need ON

cfi-SlotSubslotNonMBSFN-r15		INTEGER (1..3)					OPTIONAL,	 -- Need ON

cfi-SubframeMBSFN-r15			INTEGER (1..2)						OPTIONAL,	 -- Need ON

cfi-SlotSubslotMBSFN-r15		INTEGER (1..2)						OPTIONAL	 -- Need ON

}

CFI-PatternConfig-r15	::= SEQUENCE {

cfi-PatternSubframe-r15		SEQUENCE (SIZE(10)) OF INTEGER (1..4)	OPTIONAL,	 -- Need ON

cfi-PatternSlotSubslot-r15	SEQUENCE (SIZE(10)) OF INTEGER (1..3)	OPTIONAL	 -- Need ON

}

LAA-SCellConfiguration-r13 ::=			SEQUENCE {

subframeStartPosition-r13				ENUMERATED {s0, s07},

laa-SCellSubframeConfig-r13				BIT STRING (SIZE(8))

}

LAA-SCellConfiguration-v1430 ::=		SEQUENCE {

crossCarrierSchedulingConfig-UL-r14	CHOICE {

release									NULL,

setup									SEQUENCE {

crossCarrierSchedulingConfigLAA-UL-r14		CrossCarrierSchedulingConfigLAA-UL-r14

}

}													OPTIONAL,	-- Cond Cross-Carrier-ConfigUL

lbt-Config-r14								LBT-Config-r14			OPTIONAL,		-- Need ON

pdcch-ConfigLAA-r14							PDCCH-ConfigLAA-r14	OPTIONAL,		-- Need ON

absenceOfAnyOtherTechnology-r14			ENUMERATED {true}		OPTIONAL,		-- Need OR

soundingRS-UL-ConfigDedicatedAperiodic-v1430

SoundingRS-UL-ConfigDedicatedAperiodic-v1430	OPTIONAL		-- Need ON

}

LAA-SCellConfiguration-v1530 ::=		SEQUENCE {

aul-Config-r15							AUL-Config-r15		OPTIONAL,		-- Need ON

pusch-ModeConfigLAA-r15					PUSCH-ModeConfigLAA-r15	OPTIONAL	-- Need OR

}

PUSCH-ModeConfigLAA-r15 ::=			SEQUENCE {

laa-PUSCH-Mode1	BOOLEAN,

laa-PUSCH-Mode2	BOOLEAN,

laa-PUSCH-Mode3	BOOLEAN

}

LBT-Config-r14 ::=		CHOICE{

maxEnergyDetectionThreshold-r14				INTEGER(-85..-52),

energyDetectionThresholdOffset-r14			INTEGER(-13..20)

}

CSI-RS-ConfigNZPToAddModList-r11 ::=	SEQUENCE (SIZE (1..maxCSI-RS-NZP-r11)) OF CSI-RS-ConfigNZP-r11

CSI-RS-ConfigNZPToAddModListExt-r13 ::=	SEQUENCE (SIZE (1..maxCSI-RS-NZP-v1310)) OF CSI-RS-ConfigNZP-r11

CSI-RS-ConfigNZPToAddModList-r15 ::=	SEQUENCE (SIZE (1..maxCSI-RS-NZP-r13)) OF CSI-RS-ConfigNZP-r11

CSI-RS-ConfigNZPToReleaseList-r11 ::=	SEQUENCE (SIZE (1..maxCSI-RS-NZP-r11)) OF CSI-RS-ConfigNZPId-r11

CSI-RS-ConfigNZPToReleaseListExt-r13 ::=	SEQUENCE (SIZE (1..maxCSI-RS-NZP-v1310)) OF CSI-RS-ConfigNZPId-v1310

CSI-RS-ConfigNZPToReleaseList-r15 ::=	SEQUENCE (SIZE (1..maxCSI-RS-NZP-r13)) OF CSI-RS-ConfigNZPId-r13

CSI-RS-ConfigZPToAddModList-r11 ::=	SEQUENCE (SIZE (1..maxCSI-RS-ZP-r11)) OF CSI-RS-ConfigZP-r11

CSI-RS-ConfigZPToReleaseList-r11 ::=	SEQUENCE (SIZE (1..maxCSI-RS-ZP-r11)) OF CSI-RS-ConfigZPId-r11

PhysicalConfigDedicatedSTTI-r15 ::=	CHOICE {

release					NULL,

setup					SEQUENCE {

antennaInfoDedicatedSTTI-r15		AntennaInfoDedicatedSTTI-r15		OPTIONAL, -- Need ON

antennaInfoUL-STTI-r15				AntennaInfoUL-STTI-r15				OPTIONAL, -- Need ON

pucch-ConfigDedicated-v1530			PUCCH-ConfigDedicated-v1530			OPTIONAL, -- Need ON

schedulingRequestConfig-v1530		SchedulingRequestConfig-v1530		OPTIONAL, -- Need ON

uplinkPowerControlDedicatedSTTI-r15	UplinkPowerControlDedicatedSTTI-r15	OPTIONAL,	--Need ON

cqi-ReportConfig-r15				CQI-ReportConfig-r15				OPTIONAL, -- Need ON

csi-RS-Config-r15					CSI-RS-Config-r15					OPTIONAL, -- Need ON

csi-RS-ConfigNZPToReleaseList-r15	CSI-RS-ConfigNZPToReleaseList-r15	OPTIONAL, -- Need ON

csi-RS-ConfigNZPToAddModList-r15	CSI-RS-ConfigNZPToAddModList-r15	OPTIONAL, -- Need ON

csi-RS-ConfigZPToReleaseList-r15	CSI-RS-ConfigZPToReleaseList-r11	OPTIONAL, -- Need ON

csi-RS-ConfigZPToAddModList-r11		CSI-RS-ConfigZPToAddModList-r11		OPTIONAL, -- Need ON

csi-RS-ConfigZP-ApList-r15			CSI-RS-ConfigZP-ApList-r14			OPTIONAL, -- Need ON

eimta-MainConfig-r12				EIMTA-MainConfig-r12				OPTIONAL, -- Need ON

eimta-MainConfigServCell-r15		EIMTA-MainConfigServCell-r12		OPTIONAL, -- Need ON

semiOpenLoopSTTI-r15				BOOLEAN,

slotOrSubslotPDSCH-Config-r15		SlotOrSubslotPDSCH-Config-r15		OPTIONAL, -- Need ON

slotOrSubslotPUSCH-Config-r15		SlotOrSubslotPUSCH-Config-r15		OPTIONAL, -- Need ON

spdcch-Config-r15					SPDCCH-Config-r15					OPTIONAL, -- Need ON

spucch-Config-r15					SPUCCH-Config-r15					OPTIONAL, -- Need ON

srs-DCI7-TriggeringConfig-r15		BOOLEAN,

shortProcessingTime-r15				BOOLEAN,

shortTTI-r15						ShortTTI-r15						OPTIONAL -- Need ON

}

}

SoundingRS-AperiodicSet-r14 ::= SEQUENCE{

srs-CC-SetIndexList-r14

SEQUENCE (SIZE (1..4)) OF SRS-CC-SetIndex-r14

OPTIONAL,	-- Cond SRS-Trigger-TypeA

soundingRS-UL-ConfigDedicatedAperiodic-r14

SoundingRS-UL-ConfigDedicatedAperiodic-r10

}

SoundingRS-AperiodicSetUpPTsExt-r14 ::= SEQUENCE{

srs-CC-SetIndexList-r14

SEQUENCE (SIZE (1..4)) OF SRS-CC-SetIndex-r14

OPTIONAL,	-- Cond SRS-Trigger-TypeA

soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r14

SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13

}

ShortTTI-r15 ::=					SEQUENCE {

dl-STTI-Length-r15					ShortTTI-Length-r15			OPTIONAL,	-- Need OR

ul-STTI-Length-r15					ShortTTI-Length-r15			OPTIONAL	-- Need OR

}

ShortTTI-Length-r15 ::=					ENUMERATED {slot, subslot}

SoundingRS-VirtualCellID-r16 ::=			SEQUENCE {

srs-VirtualCellID-r16						INTEGER (0..503),

srs-VirtualCellID-AllSRS-r16				BOOLEAN

}

WidebandPRG-r16 ::= SEQUENCE {

widebandPRG-Subframe-r16				BOOLEAN,

widebandPRG-SlotSubslot-r16			BOOLEAN

}

ResourceReservationConfigDedicatedDL-r16 ::= SEQUENCE {

resourceReservationDedicatedDL-r16			ResourceReservationConfigDL-r16	OPTIONAL -- Need OP

}

ResourceReservationConfigDedicatedUL-r16 ::= SEQUENCE {

resourceReservationDedicatedUL-r16			ResourceReservationConfigUL-r16	OPTIONAL -- Need OP

}

-- ASN1STOP

| PhysicalConfigDedicated field descriptions |
| --- |
| absenceOfAnyOtherTechnology Presence of this field indicates absence on a long term basis (e.g. by level of regulation) of any other technology sharing the carrier; absence of this field indicates the potential presence of any other technology sharing the carrier, as specified in TS 37.213 [94]. |
| additionalSpectrumEmissionPCell E-UTRAN does not configure this field in this release of the specification. |
| antennaInfo A choice is used to indicate whether the antennaInfo is signalled explicitly or set to the default antenna configuration as specified in clause 9.2.4. |
| blindSlotSubslotPDSCH-Repetitions Enables HARQ-less/blind slot or subslot PDSCH repetitions for a UE in a given cell, i.e. back to back slot/subslot PDSCH transmissions for the same transport block. The number of slot/subslot PDSCH transmissions is indicated in the DCI. |
| blindSubframePDSCH-Repetitions Enables HARQ-less/blind subframe PDSCH repetitions for a UE in a given cell, i.e. back to back PDSCH transmissions for the same transport block. The number of PDSCH transmissions is indicated in the DCI. |
| ce-CSI-RS-Feedback Indicates whether CSI-RS-based CSI feedback is enabled for non-BL UE in CE mode A, see TS 36.213 [23], clause 7.2.2. |  |
| ce-Mode Indicates the CE mode as specified in TS 36.213 [23]. |
| ce-pdsch-pusch-Enhancement-Config Activation of new numbers of repetitions for PUSCH and modulation restrictions for PDSCH/PUSCH in CE mode A, see TS 36.212 [22] and TS 36.213 [23]. |
| cfi-SlotSubslotNonMBSFN Indicates the semi-static control format indicator for slot/subslot operation in non-MBSFN subframes. |
| cfi-SlotSubslotMBSFN Indicates the semi-static control format indicator for slot/subslot operation in MBSFN subframes. |
| cfi-SubframeMBSFN Indicates the semi-static control format indicator for subframe operation in MBSFN subframes. |
| cfi-SubframeNonMBSFN Indicates the semi-static control format indicator for subframe operation in non-MBSFN subframes. |
| cqi-ShortConfigSCell Indicates whether the CSI (CQI/PMI/RI/PTI/CRI) reporting resource configured by cqi-ShortConfigSCell is available upon receiving the SCell activation command for this SCell. E-UTRAN only configures this field when transmission mode 1-8 is configured for the serving cell on this carrier frequency. |
| csi-RS-Config For a serving frequency E-UTRAN does not configure csi-RS-Config (includes zeroTxPowerCSI-RS) when transmission mode 10 is configured for the serving cell on this carrier frequency. |
| csi-RS-ConfigNZPToAddModList For a serving frequency E-UTRAN configures one or more CSI-RS-ConfigNZP only when transmission mode 9 or 10 is configured for the serving cell on this carrier frequency. For a serving frequency, EUTRAN configures a maximum number of CSI-RS-ConfigNZP in accordance with transmission mode (including CSI processes), eMIMO (including class) and associated UE capabilities (e.g. k-Max, n-MaxList). |
| csi-RS-ConfigZP-ApList The aperiodic ZP CSI-RS for PDSCH rate matching. The field subframeConfig is applicable to semi-persistent CSI RS reporting. In other cases, the UE shall ignore field subframeConfig. |
| csi-RS-ConfigZPToAddModList For a serving frequency E-UTRAN configures one or more CSI-RS-ConfigZP only when transmission mode 10 is configured for the serving cell on this carrier frequency. |
| dl-STTI-Length, ul-STTI-Length Indicates the DL and UL short TTI lengths. Value slot corresponds to 7 OFDM symbols and value subslot corresponds to 2 or 3 OFDM symbols. E-UTRAN configures the same value for all serving cells sending PUCCH feedback on the same cell. If one SCell is configured with short TTI in the group of cells configured to send PUCCH on the same cell, the cell carrying PUCCH shall be configured with short TTI. E-UTRAN can configure different value of dl-STTI-Length and ul-STTI-Length for serving cells sending PUCCH feedback on different cells. E-UTRAN does not configure the combination {slot,subslot} for {DL,UL}. |
| dummy This field is not used in the specification. If received it shall be ignored by the UE. |
| eimta-MainConfigPCell, eimta-MainConfigSCell If E-UTRAN configures eimta-MainConfigPCell or eimta-MainConfigSCell for one serving cell in a frequency band, E-UTRAN configures eimta-MainConfigPCell or eimta-MainConfigSCell for all serving cells residing on the frequency band. E-UTRAN configures eimta-MainConfigPCell or eimta-MainConfigSCell only if eimta-MainConfig is configured. |
| energyDetectionThresholdOffset Indicates the offset to the default maximum energy detection threshold value. Unit in dB. Value -13 corresponds to -13dB, value -12 corresponds to -12dB, and so on (i.e. in steps of 1dB) as specified in TS 37.213 [94]. |
| epdcch-Config indicates the EPDCCH-Config for the cell. E-UTRAN does not configure EPDCCH-Config for an SCell that is configured with value other for schedulingCellInfo in CrossCarrierSchedulingConfig. |
| k-max Indicates the maximum number of interfering spatial layers signaled in the assistance information for MUST. Value l1 corresponds to 1 layer, Value l3 corresponds to 3 layers. |
| laa-PUSCH-Mode1, laa-PUSCH-Mode2, laa-PUSCH-Mode3 Indicates whether LAA PUSCH mode 1, 2 and/or 3 is configured as specified in TS 36.212 [22], clause 5.3.3.1. |
| laa-SCellSubframeConfig A bit-map indicating LAA SCell subframe configuration, "1" denotes that the corresponding subframe is allocated as MBSFN subframe. The bitmap is interpreted as follows: Starting from the first/leftmost bit in the bitmap, the allocation applies to subframes #1, #2, #3, #4, #6, #7, #8, and #9. |
| maxEnergyDetectionThreshold Indicates the absolute maximum energy detection threshold value. Unit in dBm. Value -85 corresponds to -85 dBm, value -84 corresponds to -84 dBm, and so on (i.e. in steps of 1dBm) as specified in TS 36.213 [23]. If the field is not configured, the UE shall use a default maximum energy detection threshold value as specified in TS 37.213 [94]. |
| maxNumber-SlotSubslotPDSCH-Repetitions Indicates the maximum number of PDSCH transmissions for slot or subslot PDSCH repetitions. |
| maxNumber-SubframePDSCH-Repetitions Indicates the maximum number of PDSCH transmissions for subframe PDSCH repetitions. |
| mcs-restrictionSlotSubslotPDSCH-Repetitions Indicates the MCS restriction in terms of number of non-addressable MSB in the MCS bit-field for slot or subslot PDSCH repetition applicable when k > 1. |
| mcs-restrictionSubframePDSCH-Repetitions Indicates MCS restriction in terms of number of non-addressable MSB in the MCS bit-field for subframe PDSCH repetition applicable when k > 1. |
| numberOfProcesses-SlotSubslotPDSCH-Repetitions Indicates the number of HARQ processes for slot/subslot PDSCH repetition applicable when k > 1 configured per serving cell. |
| numberOfProcesses-SubframePDSCH-Repetitions Indicates the number of HARQ processes for subframe PDSCH repetition applicable when k > 1 configured per serving cell. |
| p-a-must Parameter: , see TS 36.213 [23], clause 5.2. Value dB-6 corresponds to -6 dB, dB-4dot77 corresponds to -4.77 dB etc. |
| pdsch-ConfigDedicated-v1130 For a serving frequency, E-UTRAN configures pdsch-ConfigDedicated-v1130 only when transmission mode 10 is configured for the serving cell on this carrier frequency. |
| pdsch-ConfigDedicated-v1280 For a serving frequency, E-UTRAN configures pdsch-ConfigDedicated-v1280 only when transmission mode 9 or 10 is configured for the serving cell on this carrier frequency. |
| pucch-Cell If present, PUCCH feedback of this SCell is sent on the PUCCH SCell. If absent, PUCCH feedback of this SCell is sent on PCell or PSCell, or if the cell concerns the PUCCH SCell, on the concerned cell. If this field is not modified upon change of PUCCH SCell, the UE shall always send the PUCCH feedback of the concerned SCell using the configured PUCCH SCell. |
| pucch-ConfigDedicated E-UTRAN configures pucch-ConfigDedicated-r13 only if pucch-ConfigDedicated (i.e., without suffix) is not configured. UE shall ignore pucch-ConfigDedicated-v1020 when pucch-ConfigDedicated-r13 is configured. |
| pucch-SCell If present, the concerned SCell is the PUCCH SCell. E-UTRAN only configures this field upon SCell addition i.e. this field is only released when the SCell is released. The field is not applicable for an LAA SCell in this release. |
| pusch-ConfigDedicated-r13 E-UTRAN configures pusch-ConfigDedicated-r13 only if pusch-ConfigDedicated is not configured. |
| pusch-ConfigDedicated-v1250 E-UTRAN configures pusch-ConfigDedicated-v1250 only if tpc-SubframeSet is configured. |
| pusch-EnhancementsConfig Indicates that the UE shall transmit in the PUSCH enhancement mode if pusch-EnhancementsConfig is set to setup, see TS 36.211 [21] and TS 36.213 [23]. |
| resourceReservationConfigDedicatedDL Indicates whether the DL resource reservation is enabled for the UE, e.g. for NR coexistence. If the field is set to setup and resourceReservationDedicatedDL is not included, then resourceReservationConfigCommonDL in SystemInformationBlockType29 applies. |
| resourceReservationConfigDedicatedUL Indicates whether the UL resource reservation is enabled for the UE, e.g. for NR coexistence. If the field is set to setup and resourceReservationDedicatedUL is not included, then resourceReservationConfigCommonUL in SystemInformationBlockType29 applies. |
| rv-SlotsublotPDSCH-Repetitions Indicates the RV cycling sequence for slot or subslot PDSCH repetition. Value dlrvseq1 = {0, 0, 0, 0} and value dlrvseq2 = {0, 2, 3, 1}. |
| rv-SubframePDSCH-Repetitions Indicates the RV cycling sequence for subframe PDSCH repetition. Value dlrvseq1 = {0, 0, 0, 0} and value dlrvseq2 = {0, 2, 3, 1}. |
| semiOpenLoop, semiOpenLoopSTTI Value TRUE indicates that semi-open-loop transmission is used for deriving CSI reporting and corresponding PDSCH transmission (DMRS). |
| shortProcessingTime Indicates whether short processing time is configured as specific in TS 36.321 [6]. An SCell can only be configured with short processing if the cell carrying PUCCH for that SCell is configured with short processing time. |
| soundingRS-UL-PeriodicConfigDedicatedList Indicates periodic soundingRS configuration except for the extension sounding symbols of the UpPTs subframe. E-UTRAN configures this field in PhysicalConfigDedicated only for the UE indicating support of ce-SRS-Enhancement-r14 or ce-SRS-EnhancementWithoutComb4-r14. E-UTRAN configures this field in PhysicalConfigDedicatedSCell-r10 only for the UE indicating support of srs-UpPTS-6sym-r14. |
| soundingRS-UL-PeriodicConfigDedicatedUpPTsExtList Indicates periodic soundingRS configuration in extension sounding symbols of the UpPTs subframe. E-UTRAN configures this field in PhysicalConfigDedicated only for the UE indicating support of ce-SRS-Enhancement-r14 or ce-SRS-EnhancementWithoutComb4-r14. E-UTRAN configures this field in PhysicalConfigDedicatedSCell-r10 only for the UE indicating support of srs-UpPTS-6sym-r14. |
| soundingRS-UL-AperiodicConfigDedicatedList Indicates aperiodic soundingRS configuration except for the extension sounding symbols of the UpPTs subframe. E-UTRAN configures this field in PhysicalConfigDedicated only for the UE indicating support of ce-SRS-Enhancement-r14 or ce-SRS-EnhancementWithoutComb4-r14. E-UTRAN configures this field in PhysicalConfigDedicatedSCell-r10 only for the UE indicating support of srs-UpPTS-6sym-r14. |
| soundingRS-UL-DedicatedApUpPTsExtList Indicates aperiodic soundingRS configuration in extension sounding symbols of the UpPTs subframe. E-UTRAN configures this field in PhysicalConfigDedicated only for the UE indicating support of ce-SRS-Enhancement-r14 or ce-SRS-EnhancementWithoutComb4-r14. E-UTRAN configures this field in PhysicalConfigDedicatedSCell-r10 only for the UE indicating support of srs-UpPTS-6sym-r14. |
| srs-CC-SetIndexList Indicates the srs-CC-SetIndex list which the soundingRS-UL-ConfigDedicatedAperiodic and soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt belongs to. |
| srs-DCI7-TriggeringConfig Indicates whether SRS triggering via DCI7 is configured. |
| srs-VirtualCellID Indicates the virtual cell ID for SRS. |
| srs-VirtualCellID-AllSRS Value TRUE indicates the configured virtual cell ID is applied to all SRS symbols. Value FALSE indicates the configured virtual cell ID is applied only to additional SRS symbols. |
| subframeStartPosition Indicates possible starting positions of transmission in the first subframe of the DL transmission burst, see TS 36.211 [21]. Value s0 means the starting position is subframe boundary, s07 means the starting position is either subframe boundary or slot boundary. |
| tpc-PDCCH-ConfigPUCCH PDCCH configuration for power control of PUCCH using format 3/3A, see TS 36.212 [22]. |
| tpc-PDCCH-ConfigPUSCH PDCCH configuration for power control of PUSCH using format 3/3A, see TS 36.212 [22]. |
| typeA-SRS-TPC-PDCCH-Group Indicates Type A trigger configuration for SRS transmission on a PUSCH-less SCell. E-UTRAN configures the UE with either typeA-SRS-TPC-PDCCH-Group or typeB-SRS-TPC-PDCCH-Group, if any. |
| uplinkPowerControlDedicated E-UTRAN configures uplinkPowerControlDedicated-v1130 only if uplinkPowerControlDedicated (without suffix) is configured. |
| uplinkPowerControlDedicatedSCell E-UTRAN configures uplinkPowerControlDedicatedSCell-v1130 only if uplinkPowerControlDedicatedSCell-r10 is configured for this serving cell. |
| widebandPRG-SlotSubslot Indicates whether the precoding resource block group size is the whole scheduled bandwidth for slot or subslot PDSCH operation as specified in TS 36.213 [23]. |
| widebandPRG-Subframe Indicates whether the precoding resource block group size is the whole scheduled bandwidth for subframe PDSCH operation as specified in TS 36.213 [23]. |

| Conditional presence | Explanation |
| --- | --- |
| AI-r8 | The field is optionally present, need ON, if antennaInfoDedicated-r10 is absent. Otherwise the field is not present |
| AI-r10 | The field is optionally present, need ON, if antennaInfoDedicated is absent. Otherwise the field is not present |
| AperiodicSRS | If soundingRS-UL-ConfigDedicatedAperiodic-r10 is absent, the field is optional, Need ON. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| AperiodicSRSExt | If soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13 is absent, the field is optional, Need ON. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| AUL | The field is optionally present, need ON, if aul-config-r15 is present. Otherwise the field is not present. |
| CommonUL | The field is mandatory present if ul-Configuration of RadioResourceConfigCommonSCell-r10 is present; otherwise it is optional, need ON. |
| CQI-r8 | The field is optionally present, need ON, if cqi-ReportConfig-r10 is absent. Otherwise the field is not present |
| CQI-r10 | The field is optionally present, need ON, if cqi-ReportConfig is absent. Otherwise the field is not present |
| Cross-Carrier-Config | The field is optionally present, need ON, if crossCarrierSchedulingConfig-r10 is absent. Otherwise the field is not present |
| Cross-Carrier-ConfigUL | The field is optionally present, need ON, if crossCarrierSchedulingConfig-r10 and crossCarrierSchedulingConfig-r13 are absent or schedulingCellInfo is set to 'own'. Otherwise the field is not present. |
| PeriodicSRS | If soundingRS-UL-ConfigDedicated-r10 is absent, the field is optional, Need ON. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| PeriodicSRSPCell | If soundingRS-UL-ConfigDedicated is absent, the field is optional, Need ON. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| PeriodicSRSExt | If soundingRS-UL-ConfigDedicatedUpPTsExt-r13 is absent, the field is optional, Need ON. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| PUCCH-Format4or5 | The field is mandatory present with pucch-Format-v1370 set to setup if pucch-ConfigDedicated-r13 is configured and pucch-ConfigDedicated-r13 indicates PUCCH format 4 or PUCCH format 5; otherwise it is not present and the UE shall delete any existing value for this field. |
| PUCCH-SCell1 | The field is optionally present, need OR, for SCell not configured with pucch-configDedicated-r13. Otherwise it is not present. |
| PUSCH-SCell | The field is optionally present, need ON, if pusch-ConfigDedicatedSCell-r10 and pusch-ConfigDedicated-v1130 are absent. Otherwise the field is not present |
| PUSCH-SCell1 | The field is optionally present, need ON, for SCell not configured with pucch-configDedicated-r13. Otherwise it is not present. |
| SCellAdd | The field is mandatory present if cellIdentification is present; otherwise it is optional, need ON. |
| SRS-Trigger-TypeA | The field is mandatory present if typeA-SRS-TPC-PDCCH-Group-r14 is present. Otherwise the field is not present and the UE shall delete any existing value for this field. |

NOTE 1:	During handover, the UE performs a MAC reset, which involves reverting to the default CQI/ SRS/ SR configuration in accordance with clause 5.3.13 and TS 36.321 [6], clauses 5.9 and 5.2. Hence, for these parts of the dedicated radio resource configuration, the default configuration (rather than the configuration used in the source PCell) is used as the basis for the delta signalling that is included in the message used to perform handover.

NOTE 2:	Since delta signalling is not supported for the common SCell configuration, E-UTRAN can only add or release the uplink of an SCell by releasing and adding the concerned SCell.

#### –	P-Max

The IE P-Max is used to limit the UE's uplink transmission power on a carrier frequency and is used to calculate the parameter Pcompensation defined in TS 36.304 [4]. Corresponds to parameter PEMAX or PEMAX,c in TS 36.101 [42]. The UE transmit power on one serving cell shall not exceed the configured maximum UE output power of the serving cell determined by this value as specified in TS 36.101 [42], clauses 6.2.5 or 6.2.5A, or, when transmitting sidelink discovery announcements within the coverage of the concerned cell, as specified in TS 36.101 [42], clause 6.2.5D.

P-Max information element

-- ASN1START

P-Max ::=				INTEGER (-30..33)

-- ASN1STOP

#### –	PRACH-Config

The IE PRACH-ConfigSIB and IE PRACH-Config are used to specify the PRACH configuration in the system information and in the mobility control information, respectively.

PRACH-Config information elements

-- ASN1START

PRACH-ConfigSIB ::=				SEQUENCE {

rootSequenceIndex					INTEGER (0..837),

prach-ConfigInfo					PRACH-ConfigInfo

}

PRACH-ConfigSIB-v1310 ::=			SEQUENCE {

rsrp-ThresholdsPrachInfoList-r13		RSRP-ThresholdsPrachInfoList-r13,

mpdcch-startSF-CSS-RA-r13			CHOICE {

fdd-r13								ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8,

v10},

tdd-r13								ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare}

}																		OPTIONAL,	-- Cond MP

prach-HoppingOffset-r13				INTEGER (0..94)						OPTIONAL,	-- Need OR

prach-ParametersListCE-r13			PRACH-ParametersListCE-r13

}

PRACH-ConfigSIB-v1530 ::=			SEQUENCE {

edt-PRACH-ParametersListCE-r15		SEQUENCE (SIZE(1..maxCE-Level-r13)) OF EDT-PRACH-ParametersCE-r15

}

PRACH-ConfigSIB-v17xx ::=			SEQUENCE {

ce-PRACH-TxDuration-r17				ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128}

OPTIONAL	-- Cond NTN

}

PRACH-Config ::=					SEQUENCE {

rootSequenceIndex					INTEGER (0..837),

prach-ConfigInfo					PRACH-ConfigInfo					OPTIONAL	-- Need ON

}

PRACH-Config-v1310 ::=				SEQUENCE {

rsrp-ThresholdsPrachInfoList-r13		RSRP-ThresholdsPrachInfoList-r13		OPTIONAL,	-- Cond MP

mpdcch-startSF-CSS-RA-r13			CHOICE {

fdd-r13								ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8,

v10},

tdd-r13								ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare}

}																		OPTIONAL,	-- Cond MP

prach-HoppingOffset-r13				INTEGER (0..94)						OPTIONAL,	-- Need OR

prach-ParametersListCE-r13			PRACH-ParametersListCE-r13			OPTIONAL,	-- Cond MP

initial-CE-level-r13					INTEGER (0..3)		OPTIONAL	-- Need OR

}

PRACH-Config-v1430 ::=				SEQUENCE {

rootSequenceIndexHighSpeed-r14				INTEGER (0..837),

zeroCorrelationZoneConfigHighSpeed-r14		INTEGER (0..12),

prach-ConfigIndexHighSpeed-r14				INTEGER (0..63),

prach-FreqOffsetHighSpeed-r14				INTEGER (0..94)

}

PRACH-Config-v17xx ::=				SEQUENCE {

ce-PRACH-TxDuration-r17				ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128}

OPTIONAL	-- Cond NTN

}

PRACH-ConfigSCell-r10 ::=				SEQUENCE {

prach-ConfigIndex-r10					INTEGER (0..63)

}

PRACH-ConfigInfo ::=				SEQUENCE {

prach-ConfigIndex					INTEGER (0..63),

highSpeedFlag						BOOLEAN,

zeroCorrelationZoneConfig			INTEGER (0..15),

prach-FreqOffset					INTEGER (0..94)

}

PRACH-ParametersListCE-r13 ::=	SEQUENCE (SIZE(1..maxCE-Level-r13)) OF PRACH-ParametersCE-r13

PRACH-ParametersCE-r13 ::=			SEQUENCE {

prach-ConfigIndex-r13					INTEGER (0..63),

prach-FreqOffset-r13						INTEGER (0..94),

prach-StartingSubframe-r13				ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128,

sf256}				OPTIONAL,	-- Need OP

maxNumPreambleAttemptCE-r13

ENUMERATED {n3, n4, n5, n6, n7, n8, n10}	OPTIONAL,	-- Need OP

numRepetitionPerPreambleAttempt-r13		ENUMERATED {n1,n2,n4,n8,n16,n32,n64,n128},

mpdcch-NarrowbandsToMonitor-r13			SEQUENCE (SIZE(1..2)) OF

INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-NumRepetition-RA-r13				ENUMERATED {r1, r2, r4, r8, r16,

r32, r64, r128, r256},

prach-HoppingConfig-r13					ENUMERATED {on,off}

}

EDT-PRACH-ParametersCE-r15 ::=		SEQUENCE {

edt-PRACH-ParametersCE-r15		SEQUENCE {

prach-ConfigIndex-r15				INTEGER (0..63),

prach-FreqOffset-r15					INTEGER (0..94),

prach-StartingSubframe-r15			ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256}			OPTIONAL,	-- Need OP

mpdcch-NarrowbandsToMonitor-r15		SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)

}	OPTIONAL -- Need OR

}

RSRP-ThresholdsPrachInfoList-r13 ::= SEQUENCE (SIZE(1..3)) OF RSRP-Range

-- ASN1STOP

PRACH-Config field descriptions

ce-PRACH-TxDuration Duration of PRACH segment transmission in NTN transmission, see TS 36.213 [23]. Unit in duration of one preamble transmission including guard period (TCP+TSEQ+TGP). Value n1 corresponds to the duration of 1 preamble transmission, value n2 corresponds to the duration of 2 preambles transmission and so on.

edt-PRACH-ParametersListCE Configures PRACH parameters for each CE level applicable to a UE performing EDT. If included, the number of entries is same as number of entries in prach-ParametersListCE. The first entry in the list is the PRACH parameters for CE level 0, the second entry in the list is the PRACH parameters for CE level 1, and so on. The parameters maxNumPreambleAttemptCE, numRepetitionPerPreambleAttempt, mpdcch-NumRepetition-RA, prach-HoppingConfig included in prach-ParametersListCE for CE level X are also applicable for EDT.

initial-CE-level Indicates initial PRACH CE level at random access, see TS 36.321 [6]. If not configured, UE selects PRACH CE level based on measured RSRP level, see TS 36.321 [6].

highSpeedFlag Parameter: High-speed-flag, see TS 36.211 [21], clause 5.7.2]. TRUE corresponds to Restricted set and FALSE to Unrestricted set.

maxNumPreambleAttemptCE Maximum number of preamble transmission attempts per CE level. See TS 36.321 [6]. If the field is absent, the UE shall use the default value n3.

mpdcch-NarrowbandsToMonitor Narrowbands to monitor for MPDCCH for RAR, see TS 36.213 [23], clause 6.2. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..maxAvailNarrowBands-r13-1) as specified in TS 36.211 [21].

mpdcch-NumRepetition-RA Maximum number of repetitions for MPDCCH common search space (CSS) for RAR, Msg3 and Msg4, see TS 36.211 [21].

mpdcch-startSF-CSS-RA Starting subframe configuration for MPDCCH common search space (CSS), including RAR, Msg3 retransmission, PDSCH with contention resolution and PDSCH with CCCH MAC SDU, see TS 36.211 [21] and TS 36.213 [23]. Value v1 corresponds to 1, value v1dot5 corresponds to 1.5, and so on.

numRepetitionPerPreambleAttempt Number of PRACH repetitions per attempt for each CE level, See TS 36.211 [21].

prach-ConfigIndex Parameter: prach-ConfigurationIndex, see TS 36.211 [21], clause 5.7.1.

prach-ConfigIndexHighSpeed Parameter: prach-ConfigurationIndexHighSpeed, see TS 36.211 [21], clause 5.7.1. If this field is present, the UE shall ignore prach-ConfigIndex.

prach-FreqOffset Parameter: prach-FrequencyOffset, see TS 36.211 [21], clause 5.7.1. For TDD the value range is dependent on the value of prach-ConfigIndex.

prach-FreqOffsetHighSpeed Parameter: prach-FrequencyOffsetHighSpeed, see TS 36.211 [21], clause 5.7.1. For TDD the value range is dependent on the value of prach-ConfigIndexHighSpeed. If this field is present, the UE shall ignore prach-FreqOffset.

prach-HoppingConfig Coverage level specific frequency hopping configuration for PRACH.

prach-HoppingOffset Parameter: PRACH frequency hopping offset, expressed as a number of resource blocks, see TS 36.211 [21], clause 5.7.1.

prach-ParametersListCE Configures PRACH parameters for each CE level. The first entry in the list is the PRACH parameters of CE level 0, the second entry in the list is the PRACH parameters of CE level 1, and so on.

prach-StartingSubframe PRACH starting subframe periodicity, expressed in number of subframes available for preamble transmission (PRACH opportunities), see TS 36.211 [21]. Value sf2 corresponds to 2 subframes, sf4 corresponds to 4 subframes and so on. EUTRAN configures the PRACH starting subframe periodicity larger than or equal to the number of PRACH repetitions per attempt for each CE level (numRepetitionPerPreambleAttempt). If the field is absent, the value is determined implicitly in TS 36.211 [21], clause 5.7.1.

rootSequenceIndex Parameter: RACH_ROOT_SEQUENCE, see TS 36.211 [21], clause 5.7.1.

rootSequenceIndexHighSpeed The field indicates starting logical root sequence index used to derive the 64 random access preambles based on restricted set type B in high speed scenario, see TS 36.211 [21], clause 5.7.2. If this field is present, the UE shall generate random access preambles based on restricted set type B and ignore rootSequenceIndex.

rsrp-ThresholdsPrachInfoList The criterion for BL UEs and UEs in CE to select PRACH resource set. Up to 3 RSRP threshold values are signalled to determine the CE level for PRACH, see TS 36.213 [23]. The first element corresponds to RSRP threshold 1, the second element corresponds to RSRP threshold 2 and so on, see TS 36.321 [6]. The UE shall ignore this field if only one CE level, i.e. CE level 0, is configured in prach-ParametersListCE. The number of RSRP thresholds present in rsrp-ThresholdsPrachInfoList is equal to the number of CE levels configured in prach-ParametersListCE minus one. A UE that supports powerClass-14dBm shall correct the RSRP threshold values before applying them as follows: RSRP threshold = Signalled RSRP threshold - min{0, (14-min(23, P-Max))} where P-Max is the value of p-Max field in SystemInformationBlockType1-BR.

zeroCorrelationZoneConfig Parameter: NCS configuration, see TS 36.211 [21], clause 5.7.2: table 5.7.2-2, for preamble format 0..3 and TS 36.211 [21], clause 5.7.2: table 5.7.2-3, for preamble format 4.

zeroCorrelationZoneConfigHighSpeed The field indicates NCS configuration for the restricted set type B in high speed scenario, see TS 36.211 [21], clause 5.7.2. If this field is present, the UE shall generate random access preambles based on restricted set type B and ignore zeroCorrelationZoneConfig.

| Conditional presence | Explanation |
| --- | --- |
| MP | The field is mandatory present. |
| NTN | The field is optionally present, need OR, for NTN. Otherwise, the field is not present and the UE shall delete any existing value for this field. |

#### –	PresenceAntennaPort1

The IE PresenceAntennaPort1 is used to indicate whether all the neighbouring cells use Antenna Port 1. When set to TRUE, the UE may assume that at least two cell-specific antenna ports are used in all neighbouring cells.

PresenceAntennaPort1 information element

-- ASN1START

PresenceAntennaPort1 ::=				BOOLEAN

-- ASN1STOP

#### –	PUCCH-Config

The IE PUCCH-ConfigCommon and IE PUCCH-ConfigDedicated are used to specify the common and the UE specific PUCCH configuration respectively.

PUCCH-Config information elements

-- ASN1START

PUCCH-ConfigCommon ::=				SEQUENCE {

deltaPUCCH-Shift					ENUMERATED {ds1, ds2, ds3},

nRB-CQI								INTEGER (0..98),

nCS-AN								INTEGER (0..7),

n1PUCCH-AN							INTEGER (0..2047)

}

PUCCH-ConfigCommon-v1310 ::=		SEQUENCE {

n1PUCCH-AN-InfoList-r13					N1PUCCH-AN-InfoList-r13		OPTIONAL,	-- Need OR

pucch-NumRepetitionCE-Msg4-Level0-r13	ENUMERATED {n1, n2, n4, n8}		OPTIONAL,	-- Need OR

pucch-NumRepetitionCE-Msg4-Level1-r13	ENUMERATED {n1, n2, n4, n8}		OPTIONAL,	-- Need OR

pucch-NumRepetitionCE-Msg4-Level2-r13	ENUMERATED {n4, n8, n16, n32}	OPTIONAL,	-- Need OR

pucch-NumRepetitionCE-Msg4-Level3-r13	ENUMERATED {n4, n8, n16, n32}	OPTIONAL	-- Need OR

}

PUCCH-ConfigCommon-v1430 ::=		SEQUENCE {

pucch-NumRepetitionCE-Msg4-Level3-r14	ENUMERATED {n64, n128}	OPTIONAL	-- Need OR

}

PUCCH-ConfigCommon-v17xx ::=		SEQUENCE {

ce-PUCCH-TxDuration-r17				ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128}

OPTIONAL	-- Cond NTN

}

PUCCH-ConfigDedicated ::=			SEQUENCE {

ackNackRepetition					CHOICE{

release								NULL,

setup								SEQUENCE {

repetitionFactor					ENUMERATED {n2, n4, n6, spare1},

n1PUCCH-AN-Rep						INTEGER (0..2047)

}

},

tdd-AckNackFeedbackMode				ENUMERATED {bundling, multiplexing}	OPTIONAL	-- Cond TDD

}

PUCCH-ConfigDedicated-v1020 ::=		SEQUENCE {

pucch-Format-r10					CHOICE {

format3-r10						PUCCH-Format3-Conf-r13,

channelSelection-r10				SEQUENCE {

n1PUCCH-AN-CS-r10					CHOICE {

release								NULL,

setup								SEQUENCE {

n1PUCCH-AN-CS-List-r10				SEQUENCE (SIZE (1..2)) OF N1PUCCH-AN-CS-r10

}

}																OPTIONAL	-- Need ON

}

}																		OPTIONAL,	-- Need OR

twoAntennaPortActivatedPUCCH-Format1a1b-r10		ENUMERATED {true}		OPTIONAL,	-- Need OR

simultaneousPUCCH-PUSCH-r10						ENUMERATED {true}		OPTIONAL,	-- Need OR

n1PUCCH-AN-RepP1-r10							INTEGER (0..2047)		OPTIONAL	-- Need OR

}

PUCCH-ConfigDedicated-v1130 ::=		SEQUENCE {

n1PUCCH-AN-CS-v1130					CHOICE {

release								NULL,

setup								SEQUENCE {

n1PUCCH-AN-CS-ListP1-r11			SEQUENCE (SIZE (2..4)) OF INTEGER (0..2047)

}

}																		OPTIONAL,	-- Need ON

nPUCCH-Param-r11					CHOICE {

release								NULL,

setup								SEQUENCE {

nPUCCH-Identity-r11					INTEGER (0..503),

n1PUCCH-AN-r11						INTEGER (0..2047)

}

}																		OPTIONAL	-- Need ON

}

PUCCH-ConfigDedicated-v1250 ::=		SEQUENCE {

nkaPUCCH-Param-r12					CHOICE {

release								NULL,

setup								SEQUENCE {

nkaPUCCH-AN-r12						INTEGER (0..2047)

}

}

}

PUCCH-ConfigDedicated-r13 ::=		SEQUENCE {

--Release 8

ackNackRepetition-r13				CHOICE{

release								NULL,

setup								SEQUENCE {

repetitionFactor-r13				ENUMERATED {n2, n4, n6, spare1},

n1PUCCH-AN-Rep-r13				INTEGER (0..2047)

}

},

tdd-AckNackFeedbackMode-r13			ENUMERATED {bundling, multiplexing}	OPTIONAL,	-- Cond TDD

--Release 10

pucch-Format-r13					CHOICE {

format3-r13								SEQUENCE {

n3PUCCH-AN-List-r13	SEQUENCE (SIZE (1..4)) OF INTEGER (0..549)	OPTIONAL,	-- Need ON

twoAntennaPortActivatedPUCCH-Format3-r13		CHOICE {

release											NULL,

setup											SEQUENCE {

n3PUCCH-AN-ListP1-r13	SEQUENCE (SIZE (1..4)) OF INTEGER (0..549)

}

}																OPTIONAL	-- Need ON

},

channelSelection-r13				SEQUENCE {

n1PUCCH-AN-CS-r13					CHOICE {

release								NULL,

setup								SEQUENCE {

n1PUCCH-AN-CS-List-r13				SEQUENCE (SIZE (1..2)) OF N1PUCCH-AN-CS-r10,

dummy1			SEQUENCE (SIZE (2..4)) OF INTEGER (0..2047)

}

}																OPTIONAL	-- Need ON

},

format4-r13							SEQUENCE {

format4-resourceConfiguration-r13			SEQUENCE (SIZE (4)) OF Format4-resource-r13,

format4-MultiCSI-resourceConfiguration-r13	SEQUENCE (SIZE (1..2)) OF Format4-resource-r13 OPTIONAL	-- Need OR

},

format5-r13				SEQUENCE {

format5-resourceConfiguration-r13			SEQUENCE (SIZE (4)) OF Format5-resource-r13,

format5-MultiCSI-resourceConfiguration-r13	Format5-resource-r13 OPTIONAL	-- Need OR

}

}																		OPTIONAL,	-- Need OR

twoAntennaPortActivatedPUCCH-Format1a1b-r13		ENUMERATED {true}		OPTIONAL,	-- Need OR

simultaneousPUCCH-PUSCH-r13						ENUMERATED {true}		OPTIONAL,	-- Need OR

n1PUCCH-AN-RepP1-r13							INTEGER (0..2047)		OPTIONAL,	-- Need OR

--Release 11

nPUCCH-Param-r13					CHOICE {

release								NULL,

setup								SEQUENCE {

nPUCCH-Identity-r13					INTEGER (0..503),

n1PUCCH-AN-r13						INTEGER (0..2047)

}

}																		OPTIONAL,	-- Need ON

--Release 12

nkaPUCCH-Param-r13					CHOICE {

release								NULL,

setup								SEQUENCE {

nkaPUCCH-AN-r13						INTEGER (0..2047)

}

}																		OPTIONAL,	-- Need ON

--Release 13

spatialBundlingPUCCH-r13			BOOLEAN,

spatialBundlingPUSCH-r13			BOOLEAN,

harq-TimingTDD-r13					BOOLEAN,

codebooksizeDetermination-r13		ENUMERATED {dai,cc}					OPTIONAL,	-- Need OR

maximumPayloadCoderate-r13			INTEGER (0..7)						OPTIONAL,	-- Need OR

pucch-NumRepetitionCE-r13			CHOICE {

release						NULL,

setup						CHOICE {

modeA						SEQUENCE {

pucch-NumRepetitionCE-format1-r13					ENUMERATED {r1, r2, r4, r8},

pucch-NumRepetitionCE-format2-r13					ENUMERATED {r1, r2, r4, r8}

},

modeB						SEQUENCE {

pucch-NumRepetitionCE-format1-r13					ENUMERATED {r4, r8, r16, r32},

pucch-NumRepetitionCE-format2-r13					ENUMERATED {r4, r8, r16, r32}

}

}

}																		OPTIONAL	--Need ON

}

PUCCH-ConfigDedicated-v1370 ::=		SEQUENCE {

pucch-Format-v1370					CHOICE {

release	NULL,

setup	PUCCH-Format3-Conf-r13

}

}

PUCCH-ConfigDedicated-v13c0 ::=		SEQUENCE {

channelSelection-v13c0				SEQUENCE {

n1PUCCH-AN-CS-v13c0					CHOICE {

release								NULL,

setup								SEQUENCE {

n1PUCCH-AN-CS-ListP1-v13c0			SEQUENCE (SIZE (2..4)) OF INTEGER (0..2047)

}

}

}

}

PUCCH-Format3-Conf-r13 ::=	SEQUENCE {

n3PUCCH-AN-List-r13	SEQUENCE (SIZE (1..4)) OF INTEGER (0..549)	OPTIONAL,	-- Need ON

twoAntennaPortActivatedPUCCH-Format3-r13		CHOICE {

release											NULL,

setup											SEQUENCE {

n3PUCCH-AN-ListP1-r13	SEQUENCE (SIZE (1..4)) OF INTEGER (0..549)

}

}																OPTIONAL	-- Need ON

}

PUCCH-ConfigDedicated-v1430 ::=		SEQUENCE {

pucch-NumRepetitionCE-format1-r14		ENUMERATED {r64,r128}		OPTIONAL	-- Need OR

}

PUCCH-ConfigDedicated-v1530 ::=		SEQUENCE {

n1PUCCH-AN-SPT-r15					INTEGER (0..2047)			OPTIONAL,	-- Need OR

codebooksizeDeterminationSTTI-r15	ENUMERATED {dai,cc}			OPTIONAL	-- Need OR

}

PUCCH-ConfigDedicated-v17xx ::=		SEQUENCE {

ce-PUCCH-TxDurationConfig-r17		SetupRelease {CE-PUCCH-TxDurationConfig-r17} OPTIONAL	-- Need ON

}

CE-PUCCH-TxDurationConfig-r17 ::=	SEQUENCE {

ce-PUCCH-TxDuration-r17				ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128}

}

Format4-resource-r13	::=				SEQUENCE {

startingPRB-format4-r13						INTEGER (0..109),

numberOfPRB-format4-r13					INTEGER (0..7)

}

Format5-resource-r13	::=				SEQUENCE {

startingPRB-format5-r13						INTEGER (0..109),

cdm-index-format5-r13						INTEGER (0..1)

}

N1PUCCH-AN-CS-r10	::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..2047)

N1PUCCH-AN-InfoList-r13 ::= SEQUENCE (SIZE(1..maxCE-Level-r13)) OF INTEGER (0..2047)

-- ASN1STOP

PUCCH-Config field descriptions

ackNackRepetition Parameter indicates whether ACK/NACK repetition is configured, see TS 36.213 [23], clause 10.1.

cdm-index-format5 Parameter see TS 36.211 [21], clause 5.4.2c, for determining PUCCH resource(s) of PUCCH format 5.

ce-PUCCH-TxDuration Duration of PUCCH segment transmission in NTN transmission, see TS 36.213 [23]. Unit in subframe. Value sf2 corresponds to 2 subframes, value sf4 corresponds to 4 subframes and so on.

codebooksizeDetermination, codebooksizeDeterminationSTTI Parameter indicates whether HARQ codebook size is determined with downlink assignment indicator based solution or number of configured CCs, see TS 36.212 [22], clauses 5.2.2.6, 5.2.3.1 and 5.3.3.1.2, and TS 36.213 [23], clauses 10.1.2.2.3, 10.1.3.2.3, 10.1.3.2.3.1, 10.1.3.2.3.2 and 10.1.3.2.4.

deltaPUCCH-Shift Parameter: , see TS 36.211 [21], clause 5.4.1, where ds1 corresponds to value 1, ds2 corresponds to value 2 etc.

dummy1 This field is not used in the specification. If received it shall be ignored by the UE.

harq-TimingTDD Parameter indicates for a TDD SCell when aggregated with a TDD PCell of different UL/DL configurations whether deriving the HARQ timing for such a cell is done in the same way as the DL HARQ timing of an FDD SCell with a TDD PCell, see TS 36.213 [23], clause 10.2.

maximumPayloadCoderate Maximum payload or code rate for multi P-CSI on each PUCCH resource, see TS 36.213 [23], clause 10.1.1.

n1PUCCH-AN Parameter: , see TS 36.213 [23], clause 10.1. n1PUCCH-AN-r11 indicates UE-specific PUCCH AN resource offset, see TS 36.213 [23], clause 10.1.

n1PUCCH-AN-CS-List Parameter: for antenna port for PUCCH format 1b with channel selection, see TS 36.213 [23], clauses 10.1.2.2.1 and 10.1.3.2.1.

n1PUCCH-AN-CS-ListP1 Parameter: for antenna port for PUCCH format 1b with channel selection, see TS 36.213 [23], clause 10.1. E-UTRAN configures this field only when pucch-Format is set to channelSelection.

n1PUCCH-AN-Rep, n1PUCCH-AN-RepP1 Parameter: for antenna port P0 and for antenna port P1 respectively, see TS 36.213 [23], clause 10.1.

n3PUCCH-AN-List, n3PUCCH-AN-ListP1 Parameter: for antenna port P0 and for antenna port P1 respectively, see TS 36.213 [23], clause 10.1.

n1PUCCH-AN-SPT Parameter: , see TS 36.213 [23], clause 10.1. Indicates UE-specific PUCCH AN resource offset for short processing time.

nCS-An Parameter: see TS 36.211 [21], clause 5.4.

nkaPUCCH-AN Parameter: , see TS 36.213 [23], clause 10.1.3. nkaPUCCH-AN-r12 indicates PUCCH format 1a/1b starting offset for the subframe set , see TS 36.213 [23], clause 10.1.3. E-UTRAN configures nkaPUCCH-AN only if eimta-MainConfig is configured.

nPUCCH-Identity Parameter: , see TS 36.211 [21], clause 5.5.1.5.

nRB-CQI Parameter: , see TS 36.211 [21], clause 5.4.

numberOfPRB-format4 Parameter see TS 36.213 [23], Table 10.1.1-2, for determining PUCCH resource(s) of PUCCH format 4.

n1PUCCH-AN-InfoList Starting offsets of the PUCCH resource(s) indicated by SIB1-BR. The first entry in the list is the starting offset of the PUCCH resource(s) of CE level 0, the second entry in the list is the starting offset of the PUCCH resource(s) of CE level 1, and so on. If E-UTRAN includes n1PUCCH-AN-InfoList, it includes the same number of entries as in prach-ParametersListCE. See TS 36.213 [23].

pucch-Format Parameter indicates one of the PUCCH formats for transmission of HARQ-ACK, see TS 36.213 [23], clause 10.1. For TDD, if the UE is configured with PCell only, the channelSelection indicates the transmission of HARQ-ACK multiplexing as defined in Tables 10.1.3-5, 10.1.3-6, and 10.1.3-7 in TS 36.213 [23] for PUCCH, and in 7.3 in TS 36.213 [23] for PUSCH. E-UTRAN only configures pucch-Format-v1370 when pucch-Format-r13 is configured and set to format4 or format5.

pucch-NumRepetitionCE Number of PUCCH repetitions for PUCCH format 1/1a and for PUCCH format 2/2a/2b for CE modes A and B, see TS 36.211 [21] and TS 36.213 [23]. The UE shall ignore pucch-NumRepetitionCE-format2-r13, if received, for CE mode B in this release of specification. For UE in CE mode B supporting extended PUCCH repetition, if pucch-NumRepetitionCE-format1-r14 is included then the UE shall ignore pucch-NumRepetitionCE-format1-r13.

pucch-NumRepetitionCE-Msg4-Level0, pucch-NumRepetitionCE-Msg4-Level1, pucch-NumRepetitionCE-Msg4-Level2, pucch-NumRepetitionCE-Msg4-Level3 Number of repetitions for PUCCH carrying HARQ response to PDSCH containing Msg4 for PRACH CE levels 0, 1, 2 and 3, see TS 36.211 [21] and TS 36.213 [23]. Value n1 corresponds to 1 repetition, value n2 corresponds to 2 repetitions, and so on. For BL UEs or non-BL UEs in enhanced coverage supporting extended PUCCH repetition, if pucch-NumRepetitionCE-Msg4-Level3-r14 is included then the UE shall ignore pucch-NumRepetitionCE-Msg4-Level3-r13.

repetitionFactor Parameter see TS 36.213 [23], clause 10.1, where n2 corresponds to repetition factor 2, n4 to 4.

simultaneousPUCCH-PUSCH Parameter indicates whether simultaneous PUCCH and PUSCH or simultaneous SPUCCH and SlotOrSubslotPUSCH transmissions are configured, see TS 36.213 [23], clauses 10.1 and 5.1.1. E-UTRAN configures this field for the PCell, only when the nonContiguousUL-RA-WithinCC-Info is set to supported in the band on which PCell is configured. Likewise, E-UTRAN configures this field for the PSCell, only when the nonContiguousUL-RA-WithinCC-Info is set to supported in the band on which PSCell is configured. Likewise, E-UTRAN configures this field for the PUCCH SCell, only when the nonContiguousUL-RA-WithinCC-Info is set to supported in the band on which PUCCH SCell is configured.

spatialBundlingPUCCH Parameter indicates whether spatial bundling is enabled or not for PUCCH, see TS 36.212 [22], clause 5.2.3.1.

spatialBundlingPUSCH Parameter indicates whether spatial bundling is enabled or not for PUSCH, see see TS 36.212 [22], clause 5.2.2.6.

startingPRB-format4 Parameter see TS 36.211 [21], clause5.4.3 for determining PUCCH resource(s) of PUCCH format 4.

startingPRB-format5 Parameter see TS 36.211 [21], clause 5.4.3 for determining PUCCH resource(s) of PUCCH format 5.

tdd-AckNackFeedbackMode Parameter indicates one of the TDD ACK/NACK feedback modes used, see TS 36.213 [23], clauses 7.3 and 10.1.3. The value bundling corresponds to use of ACK/NACK bundling whereas, the value multiplexing corresponds to ACK/NACK multiplexing as defined in Tables 10.1.3-2, 10.1.3-3, and 10.1.3-4 in TS 36.213 [23]. The same value applies to both ACK/NACK feedback modes on PUCCH as well as on PUSCH.

twoAntennaPortActivatedPUCCH-Format1a1b Indicates whether two antenna ports are configured for PUCCH format 1a/1b for HARQ-ACK, see TS 36.213 [23], clause 10.1. The field also applies for PUCCH format 1a/1b transmission when format3 is configured, see TS 36.213 [23], clauses 10.1.2.2.2 and 10.1.3.2.2.

twoAntennaPortActivatedPUCCH-Format3 Indicates whether two antenna ports are configured for PUCCH format 3 for HARQ-ACK, see TS 36.213 [23], clause 10.1.

| Conditional presence | Explanation |
| --- | --- |
| NTN | The field is optionally present, Need OR, for NTN. Otherwise, the field is not present and the UE shall delete any existing value for this field. |
| TDD | The field is mandatory present for TDD if the pucch-Format is not present. If the pucch-Format is present, the field is not present and the UE shall delete any existing value for this field. It is not present for FDD and the UE shall delete any existing value for this field. |

#### –	PUR-Config

The IE PUR-Config is used to specify the PUR configuration.

PUR-Config information element

-- ASN1START

PUR-Config-r16 ::=		SEQUENCE {

pur-ConfigID-r16				PUR-ConfigID-r16			OPTIONAL,	-- Need OR

pur-ImplicitReleaseAfter-r16	ENUMERATED {n2, n4, n8, spare}	OPTIONAL,	-- Need OR

pur-StartTimeParameters-r16		SEQUENCE {

periodicityAndOffset-r16		PUR-PeriodicityAndOffset-r16,

startSFN-r16					INTEGER (0..1023),

startSubFrame-r16				INTEGER (0..9),

hsfn-LSB-Info-r16				BIT STRING (SIZE(1))

}		OPTIONAL,	--Need ON

pur-NumOccasions-r16			ENUMERATED {one, infinite},

pur-RNTI-r16					C-RNTI						OPTIONAL,	-- Need ON

pur-TimeAlignmentTimer-r16		INTEGER (1..8)				OPTIONAL,	-- Need OR

pur-RSRP-ChangeThreshold-r16	SetupRelease {PUR-RSRP-ChangeThreshold-r16} OPTIONAL,	-- Need ON

pur-ResponseWindowTimer-r16		ENUMERATED {sf240, sf480, sf960, sf1920, sf3840, sf5760, sf7680, sf10240}		OPTIONAL,	-- Need ON

pur-MPDCCH-Config-r16			PUR-MPDCCH-Config-r16		OPTIONAL,	-- Need ON

pur-PDSCH-FreqHopping-r16		BOOLEAN,

pur-PUCCH-Config-r16			PUR-PUCCH-Config-r16		OPTIONAL,	-- Need ON

pur-PUSCH-Config-r16			PUR-PUSCH-Config-r16		OPTIONAL,	-- Need ON

...

}

PUR-MPDCCH-Config-r16 ::=		SEQUENCE {

mpdcch-FreqHopping-r16			BOOLEAN,

mpdcch-Narrowband-r16			INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-PRB-PairsConfig-r16		SEQUENCE{

numberPRB-Pairs-r16				ENUMERATED {n2, n4, n6, spare1},

resourceBlockAssignment-r16		BIT STRING (SIZE(4))

},

mpdcch-NumRepetition-r16		ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

mpdcch-StartSF-UESS-r16			CHOICE {

fdd								ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10},

tdd							ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare1}

},

mpdcch-Offset-PUR-SS-r16	ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth}

}

PUR-PUCCH-Config-r16 ::=			SEQUENCE {

n1PUCCH-AN-r16						INTEGER (0..2047)			OPTIONAL,	-- Need ON

pucch-NumRepetitionCE-Format1-r16	ENUMERATED {n1, n2, n4, n8}	OPTIONAL	-- Need ON

}

PUR-PUSCH-Config-r16 ::=		SEQUENCE {

pur-GrantInfo-r16				CHOICE {

ce-ModeA						SEQUENCE {

numRUs-r16						BIT STRING (SIZE(2)),

prb-AllocationInfo-r16			BIT STRING (SIZE(10)),

mcs-r16							BIT STRING (SIZE(4)),

numRepetitions-r16				BIT STRING (SIZE(3))

},

ce-ModeB						SEQUENCE {

subPRB-Allocation-r16			BOOLEAN,

numRUs-r16						BOOLEAN,

prb-AllocationInfo-r16			BIT STRING (SIZE(8)),

mcs-r16							BIT STRING (SIZE(4)),

numRepetitions-r16				BIT STRING (SIZE(3))

}

}	OPTIONAL,	-- Need ON

pur-PUSCH-FreqHopping-r16		BOOLEAN,

p0-UE-PUSCH-r16					INTEGER (-8..7),

alpha-r16						Alpha-r12,

pusch-CyclicShift-r16			ENUMERATED {n0, n6},

pusch-NB-MaxTBS-r16				BOOLEAN,

locationCE-ModeB-r16			INTEGER (0..5)	OPTIONAL -- Cond SubPRB

}

PUR-RSRP-ChangeThreshold-r16 ::= SEQUENCE {

increaseThresh-r16				RSRP-ChangeThresh-r16,

decreaseThresh-r16				RSRP-ChangeThresh-r16	OPTIONAL		--Need OP

}

RSRP-ChangeThresh-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

-- ASN1STOP

PUR-Config field descriptions

alpha Parameter: αc(3). See TS 36.213 [23], clause 5.1.1.1.

hsfn-LSB-Info Indicates the LSB of the H-SFN corresponding to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config.

locationCE-ModeB PRB location within the narrowband when PUSCH sub-PRB resource allocation is enabled for PUR grant in CE mode B.

mpdcch-FreqHopping Frequency hopping activation/deactivation for MPDCCH. See TS 36.213 [23].

mpdcch-Narrowband Indicates the index of a narrowband on which the UE monitors for MPDCCH, see TS 36.213 [23], clause 9.1.5. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..maxAvailNarrowBands-r13-1) as specified in TS 36.211 [21].

mpdcch-NumRepetition Maximum number of repetitions levels for UE-SS for MPDCCH, see TS 36.213 [23].

mpdcch-Offset-PUR-SS Starting subframes configuration of the MPDCCH search space for PUR, see TS 36.213 [23].

mpdcch-PRB-PairsConfig Indicates the configuration of physical resource-block pairs used for MPDCCH. See TS 36.213 [23]. mpdcch-PRB-Pairs indicates the number of PRB pairs. Value n2 corresponds to 2 PRB pairs; n4 corresponds to 4 PRB pairs and so on. resourceBlockAssignment indicates the index to a specific combination of PRB pair for MPDCCH set. See TS 36.213 [23], clause 9.1.4.4.

mpdcch-StartSF-UESS Starting subframe configuration for an MPDCCH PUR search space, see TS 36.213 [23]. Value v1 corresponds to 1, value v1dot5 corresponds to 1.5, and so on.

n1PUCCH-AN Indicates UE-specific PUCCH AN resource offset, see TS 36.213 [23], clause 10.1.

p0-UE-PUSCH Parameter: P0_UE_PUSCH,c (3). See TS 36.213 [23], clause 5.1.1.1, unit dB.

pucch-NumRepetitionCE-Format1 Number of PUCCH repetitions for PUCCH format 1/1a, see TS 36.211 [21] and TS 36.213 [23]. When pur-GrantInfo is set to ce-ModeA, value n1 corresponds to 1 repetition, value n2 corresponds to 2 repetitions, and so on. When pur-GrantInfo is set to ce-ModeB, actual value corresponds to 4 * indicated value.

pusch-CyclicShift Parameter: ncs,λ. See TS 36.211 [21] clause 5.5.2.1.1. Value n0 corresponds to 0 and n6 corresponds to 6.

pusch-NB-MaxTBS Activation of 2984 bits maximum PUSCH TBS in 1.4 MHz in CE mode A, see TS 36.212 [22] and TS 36.213 [23].

pur-GrantInfo Indicates UL grant for transmission using PUR. Field set to ce-ModeA indicates the PUR grant is for CE Mode A and the field set to ce-ModeB indicates the PUR grant is for CE Mode B. numRUs indicates DCI field for PUSCH number of resource units, see TS 36.213 [23] clause 8.1.6. prbAllocationInfo indicates DCI field for PUSCH resource block assignment, see TS 36.212 [22], clause 5.3.3.1.10 (CE Mode A) and clause 5.3.3.1.11 (CE Mode B). mcs indicates DCI field for PUSCH modulation and coding scheme, see TS 36.213 [23] clause 8.6. numRepetitions indicates DCI field for PUSCH repetition number, see TS 36.213 [23] clause 8.0. For CE Mode A, numRUs set to '00' indicates use of full-PRB resource allocation, otherwise sub-PRB resource allocation as defined in TS 36.213 [23], clause 8.1.6. For CE Mode B, subPRB-Allocation indicates whether sub-PRB resource allocation is used.

pur-ImplicitReleaseAfter Number of consecutive PUR occasions that can be skipped before implicit release, as specified in 5.3.3.20. Value n2 corresponds to 2 PUR occasions, value n4 corresponds to 4 PUR occasions and so on.

pur-NumOccasions Number of PUR occasions. Value one corresponds to 1 PUR occasion, and value infinite corresponds to an infinite number of PUR occasions.

pur-PDSCH-FreqHopping Frequency hopping activation/deactivation for PDSCH. See TS 36.213 [23].

pur-PeriodicityAndOffset Indicates the periodicity for the PUR occasions and time offset until the first PUR occasion.

pur-PUSCH-FreqHopping Frequency hopping activation/deactivation for PUSCH. See TS 36.213 [23].

pur-ResponseWindowTimer PUR MPDCCH search space window duration. See TS 36.321 [6] and TS 36.213 [23]. Value in subframes. Value sf240 corresponds to 240 subframes, value sf480 corresponds to 480 subframes and so on.

pur-RSRP-ChangeThreshold Indicates the threshold(s) of change in serving cell RSRP in dB for TA validation. Value dB4 corresponds to 4 dB, value dB6 corresponds to 6 dB and so on. When pur-RSRP-ChangeThreshold is set to setup, if decreaseThresh is absent the value of increaseThresh is also used for decreaseThresh.

pur-TimeAlignmentTimer Indicates the idle mode TA timer in seconds for TA validation. Actual value = indicated value * PUR periodicity.

| Conditional presence | Explanation |
| --- | --- |
| SubPRB | This field is optionally present, need ON, if subPRB-Allocation is set to TRUE; otherwise the field is not present and UE shall delete any existing value for this field. |

#### –	PUR-ConfigID

The IE PUR-ConfigID is used to indicate the PUR configuration identity.

PUR-ConfigID information element

-- ASN1START

PUR-ConfigID-r16 ::= BIT STRING (SIZE(20))

-- ASN1STOP

#### –	PUR-PeriodicityAndOffset

The IE PUR-PeriodicityAndOffset is used to indicate H-SFN of the first PUR occasion and periodicity of the subsequent PUR occasions. The value of periodicity is in the unit of H-SFN duration (i.e., 10.24s). Value periodicity8 corresponds to periodicity of 8 H-SFN, value periodicity16 corresponds to periodicity of 16 H-SFN and so on. The value of offset is in the unit of H-SFN duration (i.e., 10.24s).

PUR-PeriodicityAndOffset information element

-- ASN1START

PUR-PeriodicityAndOffset-r16 ::= CHOICE {

periodicity8		INTEGER (1..7),

periodicity16		INTEGER (1..15),

periodicity32		INTEGER (1..31),

periodicity64		INTEGER (1..63),

periodicity128		INTEGER (1..127),

periodicity256		INTEGER (1..255),

periodicity512		INTEGER (1..511),

periodicity1024		INTEGER (1..1023),

periodicity2048		INTEGER (1..2047),

periodicity4096		INTEGER (1..4095),

periodicity8192		INTEGER (1..8191)

}

-- ASN1STOP

#### –	PUSCH-Config

The IE PUSCH-ConfigCommon is used to specify the common PUSCH configuration and the reference signal configuration for PUSCH and PUCCH. The IE PUSCH-ConfigDedicated is used to specify the UE specific PUSCH configuration.

PUSCH-Config information element

-- ASN1START

PUSCH-ConfigCommon ::=				SEQUENCE {

pusch-ConfigBasic					SEQUENCE {

n-SB								INTEGER (1..4),

hoppingMode							ENUMERATED {interSubFrame, intraAndInterSubFrame},

pusch-HoppingOffset					INTEGER (0..98),

enable64QAM							BOOLEAN

},

ul-ReferenceSignalsPUSCH			UL-ReferenceSignalsPUSCH

}

PUSCH-ConfigCommon-v1270 ::=		SEQUENCE {

enable64QAM-v1270						ENUMERATED {true}

}

PUSCH-ConfigCommon-v1310 ::=	SEQUENCE {

pusch-maxNumRepetitionCEmodeA-r13	ENUMERATED {

r8, r16, r32 }					OPTIONAL,	-- Need OR

pusch-maxNumRepetitionCEmodeB-r13	ENUMERATED {

r192, r256, r384, r512, r768, r1024,

r1536, r2048}					OPTIONAL,	-- Need OR

pusch-HoppingOffset-v1310

INTEGER (1..maxAvailNarrowBands-r13)	OPTIONAL	-- Need OR

}

PUSCH-ConfigCommon-v17xx ::=	SEQUENCE {

ce-PUSCH-TxDuration-r17			ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256}

OPTIONAL	-- Cond NTN

}

PUSCH-ConfigDedicated ::=			SEQUENCE {

betaOffset-ACK-Index				INTEGER (0..15),

betaOffset-RI-Index					INTEGER (0..15),

betaOffset-CQI-Index				INTEGER (0..15)

}

PUSCH-ConfigDedicated-v1020 ::=		SEQUENCE {

betaOffsetMC-r10					SEQUENCE {

betaOffset-ACK-Index-MC-r10			INTEGER (0..15),

betaOffset-RI-Index-MC-r10			INTEGER (0..15),

betaOffset-CQI-Index-MC-r10			INTEGER (0..15)

}																		OPTIONAL,	-- Need OR

groupHoppingDisabled-r10			ENUMERATED {true}					OPTIONAL,	-- Need OR

dmrs-WithOCC-Activated-r10			ENUMERATED {true}					OPTIONAL	-- Need OR

}

PUSCH-ConfigDedicated-v1130 ::=		SEQUENCE {

pusch-DMRS-r11						CHOICE {

release								NULL,

setup								SEQUENCE {

nPUSCH-Identity-r11					INTEGER (0..509),

nDMRS-CSH-Identity-r11				INTEGER (0..509)

}

}

}

PUSCH-ConfigDedicated-v1250::=		SEQUENCE {

uciOnPUSCH							CHOICE {

release								NULL,

setup									SEQUENCE {

betaOffset-ACK-Index-SubframeSet2-r12			INTEGER (0..15),

betaOffset-RI-Index-SubframeSet2-r12			INTEGER (0..15),

betaOffset-CQI-Index-SubframeSet2-r12			INTEGER (0..15),

betaOffsetMC-r12						SEQUENCE {

betaOffset-ACK-Index-MC-SubframeSet2-r12	INTEGER (0..15),

betaOffset-RI-Index-MC-SubframeSet2-r12		INTEGER (0..15),

betaOffset-CQI-Index-MC-SubframeSet2-r12	INTEGER (0..15)

}																OPTIONAL	-- Need OR

}

}

}

PUSCH-ConfigDedicated-r13 ::=			SEQUENCE {

betaOffset-ACK-Index-r13				INTEGER (0..15),

betaOffset2-ACK-Index-r13				INTEGER (0..15)					OPTIONAL,	-- Need OR

betaOffset-RI-Index-r13					INTEGER (0..15),

betaOffset-CQI-Index-r13				INTEGER (0..15),

betaOffsetMC-r13						SEQUENCE {

betaOffset-ACK-Index-MC-r13				INTEGER (0..15),

betaOffset2-ACK-Index-MC-r13			INTEGER (0..15)				OPTIONAL,	-- Need OR

betaOffset-RI-Index-MC-r13				INTEGER (0..15),

betaOffset-CQI-Index-MC-r13				INTEGER (0..15)

}																		OPTIONAL,	-- Need OR

groupHoppingDisabled-r13				ENUMERATED {true}				OPTIONAL,	-- Need OR

dmrs-WithOCC-Activated-r13				ENUMERATED {true}				OPTIONAL,	-- Need OR

pusch-DMRS-r11							CHOICE {

release									NULL,

setup									SEQUENCE {

nPUSCH-Identity-r13						INTEGER (0..509),

nDMRS-CSH-Identity-r13					INTEGER (0..509)

}

}																		OPTIONAL,	-- Need ON

uciOnPUSCH								CHOICE {

release									NULL,

setup									SEQUENCE {

betaOffset-ACK-Index-SubframeSet2-r13			INTEGER (0..15),

betaOffset2-ACK-Index-SubframeSet2-r13			INTEGER (0..15)	OPTIONAL,	-- Need OR

betaOffset-RI-Index-SubframeSet2-r13			INTEGER (0..15),

betaOffset-CQI-Index-SubframeSet2-r13			INTEGER (0..15),

betaOffsetMC-r12						SEQUENCE {

betaOffset-ACK-Index-MC-SubframeSet2-r13	INTEGER (0..15),

betaOffset2-ACK-Index-MC-SubframeSet2-r13	INTEGER (0..15)	OPTIONAL,	-- Need OR

betaOffset-RI-Index-MC-SubframeSet2-r13		INTEGER (0..15),

betaOffset-CQI-Index-MC-SubframeSet2-r13	INTEGER (0..15)

}																OPTIONAL	-- Need OR

}

}																		OPTIONAL,	-- Need ON

pusch-HoppingConfig-r13					ENUMERATED {on}					OPTIONAL	-- Need OR

}

PUSCH-ConfigDedicated-v1430 ::=			SEQUENCE {

ce-PUSCH-NB-MaxTBS-r14					ENUMERATED {on}					OPTIONAL,	-- Need OR

ce-PUSCH-MaxBandwidth-r14				ENUMERATED {bw5}				OPTIONAL,	-- Need OR

tdd-PUSCH-UpPTS-r14						TDD-PUSCH-UpPTS-r14				OPTIONAL,	-- Need ON

ul-DMRS-IFDMA-r14						BOOLEAN,

enable256QAM-r14						Enable256QAM-r14				OPTIONAL	-- Need ON

}

PUSCH-ConfigDedicated-v1530 ::=			SEQUENCE {

ce-PUSCH-FlexibleStartPRB-AllocConfig-r15	CHOICE {

release				NULL,

setup				SEQUENCE {

offsetCE-ModeB-r15			INTEGER (-1..3)	OPTIONAL		-- Cond CE-ModeB

}

},

ce-PUSCH-SubPRB-Config-r15	CHOICE {

release				NULL,

setup				SEQUENCE {

locationCE-ModeB-r15			INTEGER (0..5)		OPTIONAL,	-- Cond CE-ModeB

sixToneCyclicShift-r15		INTEGER (0..3),

threeToneCyclicShift-r15		INTEGER (0..2)

}

}		OPTIONAL -- Need ON

}

PUSCH-ConfigDedicated-v1610 ::=		SEQUENCE {

ce-PUSCH-MultiTB-Config-r16		SetupRelease {CE-PUSCH-MultiTB-Config-r16}

}

PUSCH-ConfigDedicated-v17xx ::=		SEQUENCE {

ce-PUSCH-TxDurationConfig-r17		SetupRelease {CE-PUSCH-TxDurationConfig-r17} OPTIONAL	-- Need ON

}

PUSCH-ConfigDedicatedSCell-r10 ::=		SEQUENCE {

groupHoppingDisabled-r10				ENUMERATED {true}				OPTIONAL,	-- Need OR

dmrs-WithOCC-Activated-r10				ENUMERATED {true}				OPTIONAL	-- Need OR

}

PUSCH-ConfigDedicatedSCell-v1430 ::=			SEQUENCE {

enable256QAM-r14						Enable256QAM-r14				OPTIONAL	-- Need OR

}

PUSCH-ConfigDedicatedScell-v1530 ::=			SEQUENCE {

uci-OnPUSCH-r15						CHOICE {

release									NULL,

setup									SEQUENCE {

betaOffsetAUL-r15								INTEGER (0..15)

}

}

}

TDD-PUSCH-UpPTS-r14 ::=					CHOICE {

release									NULL,

setup									SEQUENCE {

symPUSCH-UpPTS-r14						ENUMERATED {sym1, sym2, sym3, sym4, sym5, sym6}																					OPTIONAL,	-- Need ON

dmrs-LessUpPTS-Config-r14				ENUMERATED {true}			OPTIONAL	-- Need OR

}

}

CE-PUSCH-MultiTB-Config-r16	 ::=	SEQUENCE {

interleaving-r16						ENUMERATED {on}			OPTIONAL	-- Need OR

}

CE-PUSCH-TxDurationConfig-r17 ::=	SEQUENCE {

ce-PUSCH-TxDuration-r17			CHOICE {

fullPRB							ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256},

subPRB							ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128}

}

}

Enable256QAM-r14 ::=					CHOICE {

release								NULL,

setup								CHOICE {

tpc-SubframeSet-Configured-r14		SEQUENCE {

subframeSet1-DCI-Format0-r14										BOOLEAN,

subframeSet1-DCI-Format4-r14										BOOLEAN,

subframeSet2-DCI-Format0-r14										BOOLEAN,

subframeSet2-DCI-Format4-r14										BOOLEAN

},

tpc-SubframeSet-NotConfigured-r14	SEQUENCE {

dci-Format0-r14		BOOLEAN,

dci-Format4-r14		BOOLEAN

}

}

}

PUSCH-EnhancementsConfig-r14 ::=		CHOICE {

release							NULL,

setup							SEQUENCE {

pusch-HoppingOffsetPUSCH-Enh-r14			INTEGER (1..100)		OPTIONAL,	-- Need ON

interval-ULHoppingPUSCH-Enh-r14			CHOICE {

interval-FDD-PUSCH-Enh-r14				ENUMERATED {int1, int2, int4, int8},

interval-TDD-PUSCH-Enh-r14				ENUMERATED {int1, int5, int10, int20}

}																	OPTIONAL	-- Need ON

}

}

UL-ReferenceSignalsPUSCH ::=		SEQUENCE {

groupHoppingEnabled					BOOLEAN,

groupAssignmentPUSCH				INTEGER (0..29),

sequenceHoppingEnabled				BOOLEAN,

cyclicShift							INTEGER (0..7)

}

-- ASN1STOP

| PUSCH-Config field descriptions |
| --- |
| betaOffset-ACK-Index, betaOffset2-ACK-Index, betaOffset-ACK-Index-MC, betaOffset2-ACK-Index-MC Parameter: ,, and , for single- and multiple-codeword respectively, see TS 36.213 [23], Table 8.6.3-1. betaOffset-ACK-Index and betaOffset2-ACK-Index are used for single-codeword and betaOffset-ACK-Index-MC and betaOffset2-ACK-Index-MC are used for multiple-codeword. If betaOffset2-ACK-Index is configured; betaOffset-ACK-Index is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset2-ACK-Index is used. If betaOffset-ACK2-Index-MC is configured; betaOffset-ACK-Index-MC is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset2-ACK-Index-MC is used. One value applies for all serving cells with an uplink in a cell group (MCG or SCG or the group of cells configured to send PUCCH on the same cell in case PUCCH SCell is configured) and not configured with uplink power control subframe sets. The same value also applies for subframe set 1 of all serving cells with an uplink in that cell group and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell). |
| betaOffset-ACK-Index-SubframeSet2, betaOffset2-ACK-Index-SubframeSet2, betaOffset-ACK-Index-MC-SubframeSet2, betaOffset2-ACK-Index-MC-SubframeSet2 Parameter: ,,and respectively, see TS 36.213 [23], Table 8.6.3-1. betaOffset-ACK-Index-SubframeSet2 and betaOffset2-ACK-Index-SubframeSet2 are used for single-codeword, betaOffset-ACK-Index-MC-SubframeSet2, betaOffset2-ACK-Index-MC-SubframeSet2 are used for multiple-codeword. If betaOffset2-ACK-Index-SubframeSet2 is configured; betaOffset-ACK-Index-SubframeSet2 is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset2-ACK-Index-SubframeSet2 is used. If betaOffset2-ACK-Index-MC-SubframeSet2 is configured; betaOffset-ACK-Index-MC-SubframeSet2 is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset2-ACK-Index-MC-SubframeSet2 is used. One value applies for subframe set 2 of all serving cells with an uplink in a cell group (MCG or SCG or the group of cells configured to send PUCCH on the same cell in case PUCCH SCell is configured) and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell configured with uplink power control subframe sets). |
| betaOffsetAUL Parameter: see TS 36.213 [23], clause 8.6.3. |
| betaOffset-CQI-Index, betaOffset-CQI-Index-MC Parameter: , for single- and multiple-codeword respectively, see TS 36.213 [23], Table 8.6.3-3. One value applies for all serving cells with an uplink in a cell group (MCG or SCG or the group of cells configured to send PUCCH on the same cell in case PUCCH SCell is configured) and not configured with uplink power control subframe sets. The same value also applies for subframe set 1 of all serving cells with an uplink in that cell group and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell). |
| betaOffset-CQI-Index-SubframeSet2, betaOffset-CQI-Index-MC-SubframeSet2 Parameter: , for single- and multiple-codeword respectively, see TS 36.213 [23], Table 8.6.3-3. One value applies for subframe set 2 of all serving cells with an uplink in a cell group (MCG or SCG or the group of cells configured to send PUCCH on the same cell in case PUCCH SCell is configured) and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell configured with uplink power control subframe sets). |
| betaOffset-RI-Index, betaOffset-RI-Index-MC Parameter: , for single- and multiple-codeword respectively, see TS 36.213 [23], Table 8.6.3-2. One value applies for all serving cells with an uplink in a cell group (MCG or SCG or the group of cells configured to send PUCCH on the same cell in case PUCCH SCell is configured) and not configured with uplink power control subframe sets. The same value also applies for subframe set 1 of all serving cells with an uplink in that cell group and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell). |
| betaOffset-RI-Index-SubframeSet2, betaOffset-RI-Index-MC-SubframeSet2 Parameter: , for single- and multiple-codeword respectively, see TS 36.213 [23], Table 8.6.3-2. One value applies for subframe set 2 of all serving cells with an uplink in a cell group (MCG or SCG or the group of cells configured to send PUCCH on the same cell in case PUCCH SCell is configured) and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell configured with uplink power control subframe sets). |
| ce-PUSCH-FlexibleStartPRB-AllocConfig Activation of flexible starting PRB for PUSCH resource allocation in CE mode A or B. offsetCE-ModeB indicates starting PRB offset when flexible starting PRB for PUSCH resource allocation in CE mode B is enabled. See TS 36.212 [22] and TS 36.213 [23]. E-UTRAN does not configure this field when E-UTRA system bandwidth is 1.4 MHz. |
| ce-PUSCH-MaxBandwidth Maximum PUSCH channel bandwidth in CE mode A, see TS 36.212 [22] and TS 36.213 [23]. Value bw5 corresponds to 5 MHz. If this field is not configured, the maximum PUSCH channel bandwidth in CE mode A set to 1.4 MHz. The maximum PUSCH channel bandwidth in CE mode B is 1.4 MHz regardless of the setting of this parameter. Parameter: transmission bandwidth configuration, see TS 36.101 [42], table 5.6-1. |
| ce-PUSCH-MultiTB-Config Indicates whether UL multi-TB scheduling is enabled, i.e., a single DCI can schedule up to 8 PUSCH transport blocks in CE mode A and up to 4 PUSCH transport blocks in CE mode B. See TS 36.213 [23], clause 8.0. |  |
| ce-PUSCH-NB-MaxTBS Activation of 2984 bits maximum PUSCH TBS in 1.4 MHz in CE mode A, see TS 36.212 [22] and TS 36.213 [23]. |
| ce-PUSCH-SubPRB-Config Activation of PUSCH sub-PRB allocation in CE mode A or B, see TS 36.211 [21], TS 36.212 [22] and TS 36.213 [23]. |
| ce-PUSCH-TxDuration Duration of PUSCH segment transmission in NTN transmission, see TS 36.213 [23]. For full PRB allocation, value sf2 corresponds to 2 subframes, value sf4 corresponds to 4 subframes and so on. For sub-PRB allocation, value n1 corresponds to 1 resource unit, value n2 corresponds to 2 resource units and so on. |
| cyclicShift Parameters: cyclicShift, see TS 36.211 [21], Table 5.5.2.1.1-2. |
| dmrs-LessUpPTS-Config Indicates the UE not to transmit DMRS for PUSCH in UpPTS, see TS36.211 [21], clause 5.5.2.1.2. |
| dmrs-WithOCC-Activated Parameter: Activate-DMRS-with OCC, see TS 36.211 [21], clause 5.5.2.1. |
| enable256QAM See TS 36.213 [23], clause 8.6.1. If enable256QAM is included and if uplink power control subframe sets are configured by tpc-SubframeSet, the field indicates (if set to TRUE) per uplink power control subframe set and DCI format 0/0A/0B and 4/4A/4B that 256QAM is allowed for UE UL categories as indicated in TS 36.306 [5], Table 4.1A-2, while FALSE indicates that 256 QAM is not allowed. If enable256QAM is included and if uplink power control subframe sets are not configured by tpc-SubframeSet, the field indicates (if set to TRUE) per DCI format 0/0A/0B and 4/4A/4B that 256QAM is allowed for UE UL categories as indicated in TS 36.306 [5], Table 4.1A-2, while FALSE indicates that 256 QAM is not allowed. |
| enable64QAM See TS 36.213 [23], clause 8.6.1. If enable64QAM (without suffix) is set to TRUE, it indicates that 64QAM is allowed for UE categories 5 and 8 indicated in ue-Category and UL categories indicated in ue-CategoryUL which support UL 64QAM and can fallback to category 5 or 8, see TS 36.306 [5], Table 4.1A-2 and Table 4.1A-6, while FALSE indicates that 64QAM is not allowed. If enable64QAM-v1270 is set to TRUE, it indicates that 64QAM is allowed for UL categories indicated in ue-CategoryUL which support UL 64QAM but cannot fallback category 5 or 8, see TS 36.306 [5], Table 4.1A-2 and Table 4.1A-6. E-UTRAN configures enable64QAM-v1270 only when enable64QAM (without suffix) is set to TRUE. |
| interleaving Indicates whether interleaving for UL multi-TB scheduling is enabled, see TS 36.213 [23], clause 8.0. |
| interval-ULHoppingPUSCH-Enh Number of consecutive absolute subframes over which PUSCH stays at the same PRBs before hopping to other PRBs. For interval-FDD-PUSCH-Enh, int1 corresponds to 1 subframe, int2 corresponds to 2 subframes, and so on. For interval-TDD-PUSCH-Enh, int1 corresponds to 1 subframe, int5 corresponds to 5 subframes, and so on. See TS 36.211 [21], clause 5.3.4. |
| groupAssignmentPUSCH Parameter: SS See TS 36.211 [21], clause 5.5.1.3. |
| groupHoppingDisabled Parameter: Disable-sequence-group-hopping, see TS 36.211 [21], clause 5.5.1.3. |
| groupHoppingEnabled Parameter: Group-hopping-enabled, see TS 36.211 [21], clause 5.5.1.3. |
| hoppingMode Parameter: Hopping-mode, see TS 36.211 [21], clause 5.3.4. |
| locationCE-ModeB PRB location within the narrowband when PUSCH sub-PRB allocation is enabled in CE mode B. |
| nDMRS-CSH-Identity Parameter: , see TS 36.211 [21], clause 5.5.2.1.1. |
| nPUSCH-Identity Parameter: , see TS 36.211 [21], clause 5.5.1.5. |
| n-SB Parameter: Nsb see TS 36.211 [21], clause 5.3.4. |
| pusch-HoppingConfig For BL UEs and UEs in CE, frequency hopping activation/deactivation for unicast PUSCH, see TS 36.211 [21] |
| pusch-hoppingOffset Except for BL UEs and UEs in CE, parameter: , see TS 36.211 [21], clause 5.3.4. For BL UEs and UEs in CE, the pusch-hoppingOffset-v1310 indicates the parameter, see TS 36.211 [21], clause 5.3.4. . In case pusch-hoppingOffset-v1310 is signalled, the BL UEs and UEs in CE shall ignore pusch-hoppingOffset (i.e. without suffix). |
| pusch-HoppingOffsetPUSCH-Enh Indicates the freqeuncy domain hopping offset between PRBs for PUSCH in frequency hopping, see TS 36.211 [21], clause 5.3.4. Value 1 corresponds to 1 PRB, value 2 corresponds to 2 PRBs, and so on. |
| pusch-maxNumRepetitionCEmodeA Maximum value to indicate the set of PUSCH repetition numbers for CE mode A, see TS 36.211 [21] and TS 36.213 [23]. E-UTRAN does not configure value r8. If the field is not configured, the UE shall apply the default value as defined in TS 36.213 [23], clause 8.0. |
| pusch-maxNumRepetitionCEmodeB Maximum value to indicate the set of PUSCH repetition numbers for CE mode B, see TS 36.211 [21] and TS 36.213 [23]. |
| sequenceHoppingEnabled Parameter: Sequence-hopping-enabled, see TS 36.211 [21], clause 5.5.1.4. |
| sixToneCyclicShift, threeToneCyclicShift Cyclic shift for PUSCH reference signal sequence of six/three subcarriers in CE mode A or B. |
| symPUSCH-UpPTS Indicates the number of data symbols that configured for PUSCH transmission in UpPTS. Values sym2, sym3, sym4, sym5 and sym6 can be used for normal cyclic prefix, if dmrsLess-UpPTS is set to true, otherwise, values sym2, sym3, sym4, sym5 can be used for normal cyclic prefix and values sym1, sym2, sym3 and sym4 can be used for extended cyclic prefix, see TS 36.213 [23], clause 8.6.2 and TS 36.211 [21], clause 5.3.4. |
| ul-DMRS-IFDMA Value TRUE indicates that the UE is configured with enhanced UL DMRS. |
| ul-ReferenceSignalsPUSCH Used to specify parameters needed for the transmission on PUSCH (or PUCCH). |

| Conditional presence | Explanation |
| --- | --- |
| CE-ModeB | The field is optionally present, need ON, for CE Mode B. Otherwise, the field is not present. |
| NTN | The field is optionally present, need OR, for NTN. Otherwise, the field is not present and the UE shall delete any existing value for this field. |

#### –	RACH-ConfigCommon

The IE RACH-ConfigCommon is used to specify the generic random access parameters.

RACH-ConfigCommon information element

-- ASN1START

RACH-ConfigCommon ::=		SEQUENCE {

preambleInfo						SEQUENCE {

numberOfRA-Preambles				ENUMERATED {

n4, n8, n12, n16, n20, n24, n28,

n32, n36, n40, n44, n48, n52, n56,

n60, n64},

preamblesGroupAConfig				SEQUENCE {

sizeOfRA-PreamblesGroupA			ENUMERATED {

n4, n8, n12, n16, n20, n24, n28,

n32, n36, n40, n44, n48, n52, n56,

n60},

messageSizeGroupA						ENUMERATED {b56, b144, b208, b256},

messagePowerOffsetGroupB			ENUMERATED {

minusinfinity, dB0, dB5, dB8, dB10, dB12,

dB15, dB18},

...

}			OPTIONAL													-- Need OP

},

powerRampingParameters				PowerRampingParameters,

ra-SupervisionInfo					SEQUENCE {

preambleTransMax					PreambleTransMax,

ra-ResponseWindowSize				ENUMERATED {

sf2, sf3, sf4, sf5, sf6, sf7,

sf8, sf10},

mac-ContentionResolutionTimer		ENUMERATED {

sf8, sf16, sf24, sf32, sf40, sf48,

sf56, sf64}

},

maxHARQ-Msg3Tx						INTEGER (1..8),

...,

[[	preambleTransMax-CE-r13			PreambleTransMax					OPTIONAL,	-- Need OR

rach-CE-LevelInfoList-r13		RACH-CE-LevelInfoList-r13			OPTIONAL	-- Need OR

]],

[[	edt-SmallTBS-Subset-r15			ENUMERATED {true}					OPTIONAL		-- Cond EDT-OR

]]

}

RACH-ConfigCommon-v1250 ::=		SEQUENCE {

txFailParams-r12				SEQUENCE {

connEstFailCount-r12					ENUMERATED {n1, n2, n3, n4},

connEstFailOffsetValidity-r12			ENUMERATED {s30, s60, s120, s240,

s300, s420, s600, s900},

connEstFailOffset-r12					INTEGER (0..15)		OPTIONAL	-- Need OP

}

}

RACH-ConfigCommonSCell-r11 ::=		SEQUENCE {

powerRampingParameters-r11				PowerRampingParameters,

ra-SupervisionInfo-r11					SEQUENCE {

preambleTransMax-r11					PreambleTransMax

},

...

}

RACH-CE-LevelInfoList-r13 ::=	SEQUENCE (SIZE (1..maxCE-Level-r13)) OF RACH-CE-LevelInfo-r13

RACH-CE-LevelInfo-r13 ::=		SEQUENCE {

preambleMappingInfo-r13				SEQUENCE {

firstPreamble-r13					INTEGER(0..63),

lastPreamble-r13					INTEGER(0..63)

},

ra-ResponseWindowSize-r13			ENUMERATED {sf20, sf50, sf80, sf120, sf180,

sf240, sf320, sf400},

mac-ContentionResolutionTimer-r13	ENUMERATED {sf80, sf100, sf120,

sf160, sf200, sf240, sf480, sf960},

rar-HoppingConfig-r13				ENUMERATED {on,off},

...,

[[	edt-Parameters-r15			SEQUENCE {

edt-LastPreamble-r15		INTEGER(0..63),

edt-SmallTBS-Enabled-r15	BOOLEAN,

edt-TBS-r15				ENUMERATED {b328, b408, b504, b600, b712,

b808, b936, b1000or456},

mac-ContentionResolutionTimer-r15	ENUMERATED {sf240, sf480, sf960,

sf1920, sf3840, sf5760, sf7680, sf10240}		OPTIONAL -- Need OP

}	OPTIONAL		-- Cond EDT

]]

}

PowerRampingParameters ::=			SEQUENCE {

powerRampingStep					ENUMERATED {dB0, dB2,dB4, dB6},

preambleInitialReceivedTargetPower	ENUMERATED {

dBm-120, dBm-118, dBm-116, dBm-114, dBm-112,

dBm-110, dBm-108, dBm-106, dBm-104, dBm-102,

dBm-100, dBm-98, dBm-96, dBm-94,

dBm-92, dBm-90}

}

PreambleTransMax ::=				ENUMERATED {

n3, n4, n5, n6, n7,	n8, n10, n20, n50,

n100, n200}

-- ASN1STOP

Editor’s Note: Agreement: If the start of the RA Response window is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the ra-ResponseWindowSize for IoT NTN.

Editor’s Note: Agreement: If the start of mac-ContentionResolutionTimer is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the mac-ContentionResolutionTimer for IoT NTN.

RACH-ConfigCommon field descriptions

connEstFailCount Number of times that the UE detects T300 expiry on the same cell before applying connEstFailOffset.

connEstFailOffset Parameter "Qoffsettemp" in TS 36.304 [4]. If the field is not present the value of infinity shall be used for "Qoffsettemp".

connEstFailOffsetValidity Amount of time that the UE applies connEstFailOffset before removing the offset from evaluation of the cell. Value s30 corresponds to 30 seconds, s60 corresponds to 60 seconds, and so on.

edt-LastPreamble Provides the mapping of preambles to groups for each CE level for EDT, as specified in TS 36.321 [6]. For the concerned CE level, if PRACH resources configured by edt-PRACH-ParametersCE-r15 are different from the PRACH resources configured by PRACH-ParametersCE-r13 for all CE levels and edt-PRACH-ParametersCE-r15 for all other CE levels, the preambles for EDT are the preambles firstPreamble-r13 to edt-LastPreamble-r15, otherwise the preambles for EDT are the preambles lastPreamble-r13 +1 to edt-LastPreamble-r15.

edt-SmallTBS-Enabled Value TRUE indicates UE performing EDT is allowed to select TBS smaller than edt-TBS for Msg3 in the corresponding CE level, as specified in TS 36.213 [23].

edt-SmallTBS-Subset Presence indicates only two of the TBS values can be used according to edt-TBS corresponding to the CE level, as specified in TS 36.213 [23]. When the field is not present, any of the TBS values according to edt-TBS corresponding to the CE level can be used. This field is applicable for a CE level only when edt-SmallTBS-Enabled is included for the corresponding CE level.

edt-TBS Largest TBS for Msg3 for a CE level applicable to a UE performing EDT. Value in bits. Value b328 corresponds to 328 bits, b408 corresponds to 408 bits and so on. Additionally, value b1000or456 corresponds to 1000 bits for CE levels 0 and 1, and 456 bits for CE levels 2 and 3. See TS 36.213 [23].

mac-ContentionResolutionTimer Timer for contention resolution in TS 36.321 [6]. Value in subframes. Value sf8 corresponds to 8 subframes, sf16 corresponds to 16 subframes and so on. mac-ContentionResolutionTimer-r15 is only applicable for EDT. UE performing EDT shall use mac-ContentionResolutionTimer-r15, if present.

maxHARQ-Msg3Tx Maximum number of Msg3 HARQ transmissions in TS 36.321 [6], used for contention based random access. Value is an integer.

messagePowerOffsetGroupB Threshold for preamble selection in TS 36.321 [6]. Value in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on.

messageSizeGroupA Threshold for preamble selection in TS 36.321 [6]. Value in bits. Value b56 corresponds to 56 bits, b144 corresponds to 144 bits and so on.

numberOfRA-Preambles Number of non-dedicated random access preambles in TS 36.321 [6]. Value is an integer. Value n4 corresponds to 4, n8 corresponds to 8 and so on.

powerRampingStep Power ramping factor in TS 36.321 [6]. Value in dB. Value dB0 corresponds to 0 dB, dB2 corresponds to 2 dB and so on.

preambleInitialReceivedTargetPower Initial preamble power in TS 36.321 [6]. Value in dBm. Value dBm-120 corresponds to -120 dBm, dBm-118 corresponds to -118 dBm and so on.

preambleMappingInfo Provides the mapping of preambles to groups for each CE level, except for EDT, as specified in TS 36.321 [6]. When random access preambles group B is used, firstPreamble-r13 is set to 0 and lastPreamble-r13 is set to numberOfRA-Preambles-1.

preamblesGroupAConfig Provides the configuration for preamble grouping in TS 36.321 [6]. If the field is not signalled, the size of the random access preambles group A, as specified in TS 36.321 [6], is equal to numberOfRA-Preambles.

preambleTransMax, preambleTransMax-CE Maximum number of preamble transmission in TS 36.321 [6]. Value is an integer. Value n3 corresponds to 3, n4 corresponds to 4 and so on.

rach-CE-LevelInfoList Provides RACH information for each coverage level. The first entry in the list contains RACH information of CE level 0, the second entry in the list contains RACH information of CE level 1, and so on. If E-UTRAN includes rach-CE-LevelInfoList, it includes the same number of entries as in prach-ParametersListCE.

ra-ResponseWindowSize Duration of the RA response window in TS 36.321 [6]. Value in subframes. Value sf2 corresponds to 2 subframes, sf3 corresponds to 3 subframes and so on. The same value applies for each serving cell (although the associated functionality is performed independently for each cell).

rar-HoppingConfig Frequency hopping activation/deactivation for RAR/Msg3/Msg4 for a CE level, see TS 36.211 [21].

sizeOfRA-PreamblesGroupA Size of the random access preambles group A in TS 36.321 [6]. Value is an integer. Value n4 corresponds to 4, n8 corresponds to 8 and so on.

| Conditional presence | Explanation |
| --- | --- |
| EDT | The field is mandatory present if cp-EDT or up-EDT in SystemInformationBlockType2 is present; otherwise the field is not present and the UE shall delete any existing value for this field. |
| EDT-OR | The field is optionally present, Need OR, if cp-EDT or up-EDT in SystemInformationBlockType2 is present; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

RACH-ConfigDedicated ::=		SEQUENCE {

ra-PreambleIndex					INTEGER (0..63),

ra-PRACH-MaskIndex					INTEGER (0..15)

}

-- ASN1STOP

RACH-ConfigDedicated field descriptions

ra-PRACH-MaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 36.321 [6].

ra-PreambleIndex Explicitly signalled Random Access Preamble for RA Resource selection in TS 36.321 [6].

#### –	RadioResourceConfigCommon

The IE RadioResourceConfigCommonSIB and IE RadioResourceConfigCommon are used to specify common radio resource configurations in the system information and in the mobility control information, respectively, e.g., the random access parameters and the static physical layer parameters.

RadioResourceConfigCommon information element

-- ASN1START

RadioResourceConfigCommonSIB ::=	SEQUENCE {

rach-ConfigCommon					RACH-ConfigCommon,

bcch-Config						BCCH-Config,

pcch-Config						PCCH-Config,

prach-Config						PRACH-ConfigSIB,

pdsch-ConfigCommon					PDSCH-ConfigCommon,

pusch-ConfigCommon					PUSCH-ConfigCommon,

pucch-ConfigCommon					PUCCH-ConfigCommon,

soundingRS-UL-ConfigCommon			SoundingRS-UL-ConfigCommon,

uplinkPowerControlCommon			UplinkPowerControlCommon,

ul-CyclicPrefixLength				UL-CyclicPrefixLength,

...,

[[	uplinkPowerControlCommon-v1020	UplinkPowerControlCommon-v1020		OPTIONAL	-- Need OR

]],

[[	rach-ConfigCommon-v1250			RACH-ConfigCommon-v1250				OPTIONAL	-- Need OR

]],

[[	pusch-ConfigCommon-v1270		PUSCH-ConfigCommon-v1270			OPTIONAL	-- Need OR

]],

[[	bcch-Config-v1310				BCCH-Config-v1310					OPTIONAL,	-- Need OR

pcch-Config-v1310				PCCH-Config-v1310					OPTIONAL,	-- Need OR

freqHoppingParameters-r13		FreqHoppingParameters-r13			OPTIONAL,	-- Need OR

pdsch-ConfigCommon-v1310		PDSCH-ConfigCommon-v1310			OPTIONAL,	-- Need OR

pusch-ConfigCommon-v1310		PUSCH-ConfigCommon-v1310			OPTIONAL,	-- Need OR

prach-ConfigCommon-v1310		PRACH-ConfigSIB-v1310				OPTIONAL,	-- Need OR

pucch-ConfigCommon-v1310		PUCCH-ConfigCommon-v1310			OPTIONAL	-- Need OR

]],

[[	highSpeedConfig-r14				HighSpeedConfig-r14					OPTIONAL,	-- Need OR

prach-Config-v1430				PRACH-Config-v1430					OPTIONAL,	-- Need OR

pucch-ConfigCommon-v1430		PUCCH-ConfigCommon-v1430			OPTIONAL	-- Need OR

]],

[[	prach-Config-v1530				PRACH-ConfigSIB-v1530				OPTIONAL,	-- Cond EDT

ce-RSS-Config-r15				RSS-Config-r15						OPTIONAL,	-- Need OR

wus-Config-r15					WUS-Config-r15						OPTIONAL,	-- Need OR

highSpeedConfig-v1530			HighSpeedConfig-v1530				OPTIONAL	-- Need OR

]],

[[	uplinkPowerControlCommon-v1540	UplinkPowerControlCommon-v1530		OPTIONAL	-- Need OR

]],

[[	wus-Config-v1560				WUS-Config-v1560					OPTIONAL	-- Need OR

]],

[[

wus-Config-v1610				WUS-Config-v1610					OPTIONAL,	-- Need OR

highSpeedConfig-v1610		HighSpeedConfig-v1610			OPTIONAL,	-- Need OR

crs-ChEstMPDCCH-ConfigCommon-r16	CRS-ChEstMPDCCH-ConfigCommon-r16	OPTIONAL, -- Need OR

gwus-Config-r16					GWUS-Config-r16						OPTIONAL,	-- Need OR

uplinkPowerControlCommon-v1610	UplinkPowerControlCommon-v1610		OPTIONAL,	-- Need OR

rss-MeasConfig-r16				ENUMERATED {enabled}				OPTIONAL,	-- Need OR

rss-MeasNonNCL-r16				ENUMERATED {enabled}				OPTIONAL,	-- Need OR

puncturedSubcarriersDL-r16		BIT STRING (SIZE (2))				OPTIONAL,	-- Need OR

highSpeedInterRAT-NR-r16		BOOLEAN								OPTIONAL	-- Need OR

]],

[[	prach-Config-v17xx				PRACH-ConfigSIB-v17xx 				OPTIONAL,	-- Need OR

pusch-Config-v17xx				PUSCH-ConfigCommon-v17xx 			OPTIONAL,	-- Need OR

pucch-Config-v17xx				PUCCH-ConfigCommon-v17xx 			OPTIONAL	-- Need OR

]]

}

RadioResourceConfigCommon ::=		SEQUENCE {

rach-ConfigCommon					RACH-ConfigCommon					OPTIONAL,	-- Need ON

prach-Config						PRACH-Config,

pdsch-ConfigCommon					PDSCH-ConfigCommon					OPTIONAL,	-- Need ON

pusch-ConfigCommon					PUSCH-ConfigCommon,

phich-Config						PHICH-Config						OPTIONAL,	-- Need ON

pucch-ConfigCommon					PUCCH-ConfigCommon					OPTIONAL,	-- Need ON

soundingRS-UL-ConfigCommon			SoundingRS-UL-ConfigCommon			OPTIONAL,	-- Need ON

uplinkPowerControlCommon			UplinkPowerControlCommon			OPTIONAL,	-- Need ON

antennaInfoCommon					AntennaInfoCommon					OPTIONAL,	-- Need ON

p-Max								P-Max								OPTIONAL,	-- Need OP

tdd-Config							TDD-Config							OPTIONAL,	-- Cond TDD

ul-CyclicPrefixLength				UL-CyclicPrefixLength,

...,

[[	uplinkPowerControlCommon-v1020	UplinkPowerControlCommon-v1020		OPTIONAL	-- Need ON

]],

[[	tdd-Config-v1130				TDD-Config-v1130					OPTIONAL	-- Cond TDD3

]],

[[	pusch-ConfigCommon-v1270		PUSCH-ConfigCommon-v1270			OPTIONAL	-- Need OR

]],

[[

prach-Config-v1310				PRACH-Config-v1310					OPTIONAL,	-- Need ON

freqHoppingParameters-r13		FreqHoppingParameters-r13			OPTIONAL,	-- Need ON

pdsch-ConfigCommon-v1310		PDSCH-ConfigCommon-v1310			OPTIONAL,	-- Need ON

pucch-ConfigCommon-v1310		PUCCH-ConfigCommon-v1310			OPTIONAL,	-- Need ON

pusch-ConfigCommon-v1310		PUSCH-ConfigCommon-v1310			OPTIONAL,	-- Need ON

uplinkPowerControlCommon-v1310	UplinkPowerControlCommon-v1310		OPTIONAL	-- Need ON

]],

[[	highSpeedConfig-r14				HighSpeedConfig-r14					OPTIONAL,	-- Need OR

prach-Config-v1430				PRACH-Config-v1430					OPTIONAL,	-- Need OR

pucch-ConfigCommon-v1430		PUCCH-ConfigCommon-v1430			OPTIONAL,	-- Need OR

tdd-Config-v1430				TDD-Config-v1430					OPTIONAL	-- Cond TDD3

]],

[[

tdd-Config-v1450				TDD-Config-v1450					OPTIONAL	-- Cond TDD3

]],

[[	uplinkPowerControlCommon-v1530	UplinkPowerControlCommon-v1530		OPTIONAL,	-- Need ON

highSpeedConfig-v1530			HighSpeedConfig-v1530				OPTIONAL	-- Need OR

]],

[[

highSpeedConfig-v1610		HighSpeedConfig-v1610			OPTIONAL,	-- Need OR

uplinkPowerControlCommon-v1610	UplinkPowerControlCommon-v1610		OPTIONAL,	-- Need OR

highSpeedInterRAT-NR-r16		BOOLEAN								OPTIONAL	-- Need ON

]],

[[	prach-Config-v17xx				PRACH-Config-v17xx 					OPTIONAL,	-- Need OR

pusch-Config-v17xx				PUSCH-ConfigCommon-v17xx 			OPTIONAL,	-- Need OR

pucch-Config-v17xx				PUCCH-ConfigCommon-v17xx 			OPTIONAL	-- Need OR

]]

}

RadioResourceConfigCommonPSCell-r12 ::=	SEQUENCE {

basicFields-r12						RadioResourceConfigCommonSCell-r10,

pucch-ConfigCommon-r12				PUCCH-ConfigCommon,

rach-ConfigCommon-r12				RACH-ConfigCommon,

uplinkPowerControlCommonPSCell-r12	UplinkPowerControlCommonPSCell-r12,

...,

[[	uplinkPowerControlCommonPSCell-v1310

UplinkPowerControlCommon-v1310		OPTIONAL	-- Need ON

]],

[[	uplinkPowerControlCommonPSCell-v1530

UplinkPowerControlCommon-v1530		OPTIONAL	-- Need ON

]]

}

RadioResourceConfigCommonPSCell-v12f0 ::=	SEQUENCE {

basicFields-v12f0					RadioResourceConfigCommonSCell-v10l0

}

RadioResourceConfigCommonPSCell-v1440 ::=	SEQUENCE {

basicFields-v1440					RadioResourceConfigCommonSCell-v1440

}

RadioResourceConfigCommonSCell-r10 ::=	SEQUENCE {

-- DL configuration as well as configuration applicable for DL and UL

nonUL-Configuration-r10					SEQUENCE {

-- 1: Cell characteristics

dl-Bandwidth-r10						ENUMERATED {n6, n15, n25, n50, n75, n100},

-- 2: Physical configuration, general

antennaInfoCommon-r10					AntennaInfoCommon,

mbsfn-SubframeConfigList-r10			MBSFN-SubframeConfigList	OPTIONAL,	-- Need OR

-- 3: Physical configuration, control

phich-Config-r10						PHICH-Config,

-- 4: Physical configuration, physical channels

pdsch-ConfigCommon-r10					PDSCH-ConfigCommon,

tdd-Config-r10							TDD-Config					OPTIONAL	-- Cond TDDSCell

},

-- UL configuration

ul-Configuration-r10				SEQUENCE {

ul-FreqInfo-r10						SEQUENCE {

ul-CarrierFreq-r10					ARFCN-ValueEUTRA			OPTIONAL,	-- Need OP

ul-Bandwidth-r10					ENUMERATED {n6, n15,

n25, n50, n75, n100}	OPTIONAL,	-- Need OP

additionalSpectrumEmissionSCell-r10		AdditionalSpectrumEmission

},

p-Max-r10							P-Max							OPTIONAL,	-- Need OP

uplinkPowerControlCommonSCell-r10		UplinkPowerControlCommonSCell-r10,

-- A special version of IE UplinkPowerControlCommon may be introduced

-- 3: Physical configuration, control

soundingRS-UL-ConfigCommon-r10		SoundingRS-UL-ConfigCommon,

ul-CyclicPrefixLength-r10			UL-CyclicPrefixLength,

-- 4: Physical configuration, physical channels

prach-ConfigSCell-r10					PRACH-ConfigSCell-r10		OPTIONAL,	-- Cond TDD-OR-NoR11

pusch-ConfigCommon-r10				PUSCH-ConfigCommon

}																		OPTIONAL,	-- Need OR

...,

[[	ul-CarrierFreq-v1090				ARFCN-ValueEUTRA-v9e0			OPTIONAL	-- Need OP

]],

[[	rach-ConfigCommonSCell-r11			RACH-ConfigCommonSCell-r11		OPTIONAL,	-- Cond ULSCell

prach-ConfigSCell-r11				PRACH-Config					OPTIONAL,	-- Cond UL

tdd-Config-v1130					TDD-Config-v1130				OPTIONAL,	-- Cond TDD2

uplinkPowerControlCommonSCell-v1130

UplinkPowerControlCommonSCell-v1130			OPTIONAL	-- Cond UL

]],

[[	pusch-ConfigCommon-v1270		PUSCH-ConfigCommon-v1270			OPTIONAL	-- Need OR

]],

[[	pucch-ConfigCommon-r13				PUCCH-ConfigCommon		OPTIONAL,	-- Cond UL

uplinkPowerControlCommonSCell-v1310

UplinkPowerControlCommonSCell-v1310	OPTIONAL	-- Cond UL

]],

[[	highSpeedConfigSCell-r14		HighSpeedConfigSCell-r14			OPTIONAL,	-- Need OR

prach-Config-v1430				PRACH-Config-v1430					OPTIONAL,	-- Cond UL

ul-Configuration-r14				SEQUENCE {

ul-FreqInfo-r14						SEQUENCE {

ul-CarrierFreq-r14					ARFCN-ValueEUTRA-r9			OPTIONAL,	-- Need OP

ul-Bandwidth-r14					ENUMERATED {n6, n15,

n25, n50, n75, n100}	OPTIONAL,	-- Need OP

additionalSpectrumEmissionSCell-r14		AdditionalSpectrumEmission

},

p-Max-r14							P-Max							OPTIONAL,	-- Need OP

soundingRS-UL-ConfigCommon-r14		SoundingRS-UL-ConfigCommon,

ul-CyclicPrefixLength-r14			UL-CyclicPrefixLength,

prach-ConfigSCell-r14					PRACH-ConfigSCell-r10		OPTIONAL,	-- Cond TDD-OR-NoR11

uplinkPowerControlCommonPUSCH-LessCell-v1430

UplinkPowerControlCommonPUSCH-LessCell-v1430	OPTIONAL	-- Need OR

}																	OPTIONAL,	-- Cond ULSRS

harq-ReferenceConfig-r14					ENUMERATED {sa2,sa4,sa5}	OPTIONAL,		-- Need OR

soundingRS-FlexibleTiming-r14			ENUMERATED {true}			OPTIONAL		-- Need OR

]],

[[	mbsfn-SubframeConfigList-v1430		MBSFN-SubframeConfigList-v1430		OPTIONAL -- Need ON

]],

[[	uplinkPowerControlCommonSCell-v1530	UplinkPowerControlCommon-v1530		OPTIONAL -- Need ON

]],

[[	highSpeedEnhMeasFlagSCell-r16			BOOLEAN						OPTIONAL -- Need ON

]]

}

RadioResourceConfigCommonSCell-v10l0 ::=	SEQUENCE {

-- UL configuration

ul-Configuration-v10l0				SEQUENCE {

additionalSpectrumEmissionSCell-v10l0		AdditionalSpectrumEmission-v10l0

}

}

RadioResourceConfigCommonSCell-v1440 ::=	SEQUENCE {

ul-Configuration-v1440				SEQUENCE {

ul-FreqInfo-v1440						SEQUENCE {

additionalSpectrumEmissionSCell-v1440		AdditionalSpectrumEmission-v10l0

}

}

}

BCCH-Config ::=						SEQUENCE {

modificationPeriodCoeff				ENUMERATED {n2, n4, n8, n16}

}

BCCH-Config-v1310 ::=				SEQUENCE {

modificationPeriodCoeff-v1310		ENUMERATED {n64}

}

FreqHoppingParameters-r13 ::=		SEQUENCE {

dummy			ENUMERATED {nb2, nb4}				OPTIONAL,

dummy2			CHOICE {

interval-FDD-r13						ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13						ENUMERATED {int1, int5, int10, int20}

}																		OPTIONAL,

dummy3			CHOICE {

interval-FDD-r13						ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13						ENUMERATED { int5, int10, int20, int40}

}																								OPTIONAL,

interval-ULHoppingConfigCommonModeA-r13	CHOICE {

interval-FDD-r13						ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13						ENUMERATED {int1, int5, int10, int20}

}																		OPTIONAL,	-- Cond MP-A

interval-ULHoppingConfigCommonModeB-r13	CHOICE {

interval-FDD-r13						ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13						ENUMERATED { int5, int10, int20, int40}

}																		OPTIONAL,	-- Cond MP-B

dummy4				INTEGER (1..maxAvailNarrowBands-r13)			OPTIONAL

}

PCCH-Config ::=						SEQUENCE {

defaultPagingCycle					ENUMERATED {

rf32, rf64, rf128, rf256},

nB									ENUMERATED {

fourT, twoT, oneT, halfT, quarterT, oneEighthT,

oneSixteenthT, oneThirtySecondT}

}

PCCH-Config-v1310 ::=				SEQUENCE {

paging-narrowBands-r13				INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-NumRepetition-Paging-r13		ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

nB-v1310							ENUMERATED {one64thT, one128thT, one256thT}

OPTIONAL	-- Need OR

}

UL-CyclicPrefixLength ::=			ENUMERATED {len1, len2}

HighSpeedConfig-r14 ::=			SEQUENCE {

highSpeedEnhancedMeasFlag-r14			ENUMERATED {true}				OPTIONAL,	-- Need OR

highSpeedEnhancedDemodulationFlag-r14	ENUMERATED {true}				OPTIONAL	-- Need OR

}

HighSpeedConfig-v1530 ::=		SEQUENCE {

highSpeedMeasGapCE-ModeA-r15			ENUMERATED {true}

}

HighSpeedConfigSCell-r14 ::=		SEQUENCE {

highSpeedEnhancedDemodulationFlag-r14	ENUMERATED {true}				OPTIONAL	-- Need OR

}

HighSpeedConfig-v1610 ::=		SEQUENCE {

highSpeedEnhMeasFlag2-r16			ENUMERATED {true}			OPTIONAL,	-- Need OR

highSpeedEnhDemodFlag2-r16			ENUMERATED {true}				OPTIONAL	-- Need OR

}

-- ASN1STOP

RadioResourceConfigCommon field descriptions

additionalSpectrumEmissionSCell The UE requirements related to additionalSpectrumEmissionSCell are defined in TS 36.101 [42]. E-UTRAN configures the same value in additionalSpectrumEmissionSCell for all SCell(s) of the same band with UL configured. The additionalSpectrumEmissionSCell is applicable for all serving cells (including PCell) of the same band with UL configured.

crs-ChEstMPDCCH-ConfigCommon Presence of this field indicates use of CRS for improving channel estimation on MPDCCH is enabled in RRC_IDLE and RRC_CONNECTED.

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 36.304 [4]. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

harq-ReferenceConfig Indicates UL/ DL configuration used as the DL HARQ reference configuration for this serving cell. Value sa2 corresponds to Configuration2, sa4 to Configuration4 etc, as specified in TS 36.211 [21], table 4.2-2. E-UTRAN configures the same value for all serving cells residing on same frequency band.

highSpeedEnhancedMeasFlag If the field is present, the UE shall apply the high speed (350 km/h) measurement enhancements as specified in TS 36.133 [16]. If highSpeedEnhMeasFlag2 is present, the UE indicating measurementEnhancements2 shall ignore this field.

highSpeedEnhancedDemodulationFlag If the field is present, the UE shall apply the advanced receiver in SFN scenario (350 km/h) as specified in TS 36.101 [42]. If this field is included in HighSpeedConfig and highSpeedEnhDemodFlag2 is present, the UE indicating demodulationEnhancements2 shall ignore this field in HighSpeedConfig.

highSpeedEnhDemodFlag2 If the field is present, the UE shall apply the further enhanced receiver in HST-SFN scenario (500 km/h) as specified in TS 36.101 [42].

highSpeedEnhMeasFlag2 If the field is present, the UE shall apply the high speed (500 km/h) measurement enhancements as specified in TS 36.133 [16].

highSpeedEnhMeasFlagSCell If configured with value TRUE, the UE shall apply the high speed (350 km/h) SCell measurement enhancements as specified in TS 36.133 [16].

highSpeedInterRAT-NR If the field is present, the UE shall apply the enhanced inter-RAT NR measurement requirements to support high speed up to 500 km/h as specified in TS 36.133 [16].

highSpeedMeasGapCE-ModeA If the field is present, the UE in CE mode A shall apply the measurement gap sharing table associated with high-velocity scenario for measurements, as specified in TS 36.133 [16].

interval-DLHoppingConfigCommonModeX Number of consecutive absolute subframes over which MPDCCH or PDSCH for CE mode X stays at the same narrowband before hopping to another narrowband. For interval-FDD, int1 corresponds to 1 subframe, int2 corresponds to 2 subframes, and so on. For interval-TDD, int1 corresponds to 1 subframe, int5 corresponds to 5 subframes, and so on.

interval-ULHoppingConfigCommonModeX Number of consecutive absolute subframes over which PUCCH or PUSCH for CE mode X stays at the same narrowband before hopping to another narrowband. For interval-FDD, int1 corresponds to 1 subframe, int2 corresponds to 2 subframes, and so on. For interval-TDD, int1 corresponds to 1 subframe, int5 corresponds to 5 subframes, and so on.

modificationPeriodCoeff Actual modification period, expressed in number of radio frames= modificationPeriodCoeff * defaultPagingCycle. n2 corresponds to value 2, n4 corresponds to value 4, n8 corresponds to value 8, n16 corresponds to value 16, and n64 corresponds to value 64.

mpdcch-NumRepetition-Paging Maximum number of repetitions for MPDCCH common search space (CSS) for paging, see TS 36.211 [21].

mpdcch-pdsch-HoppingOffset Parameter: , see TS 36.211 [21], clause 6.4.1.

mpdcch-pdsch-HoppingNB The number of narrowbands for MPDCCH/PDSCH frequency hopping. Value nb2 corresponds to 2 narrowbands and value nb4 corresponds to 4 narrowbands.

nB Parameter: nB is used as one of parameters to derive the Paging Frame and Paging Occasion according to TS 36.304 [4]. Value in multiples of 'T' as defined in TS 36.304 [4]. A value of fourT corresponds to 4 * T, a value of twoT corresponds to 2 * T and so on. In case nB-v1310 is signalled, the UE shall ignore nB (i.e. without suffix). EUTRAN configures nB-v1310 only in the BR version of SI message.

paging-narrowBands Number of narrowbands used for paging, see TS 36.304 [4], TS 36.212 [22] and TS 36.213 [23].

p-Max Pmax to be used in the target cell. If absent, for the band used in the target cell, the UE applies the maximum power according to its capability as specified in 36.101 [42], clause 6.2.2. In case the UE is configured with uplink intra-band contiguous CA and the UE indicates ue-CA-PowerClass-N in that band combination, then the p-Max in RadioResourceConfigCommonSCell for that SCell, if present, also applies for that band combination whenever that SCell is activated. This field is ignored by IAB-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [107]

prach-ConfigSCell Indicates a PRACH configuration for an SCell. The field is not applicable for an LAA SCell in this release.

puncturedSubcarriersDL Indicates number of punctured DL subcarriers and their locations, see TS 36.211 [31].

rach-ConfigCommonSCell Indicates a RACH configuration for an SCell. The field is not applicable for an LAA SCell in this release.

rss-MeasConfig Indicates whether RSS-based measurement is enabled.

rss-MeasNonNCL Indicates RSS of neighbour cells not in the Neighbour Cell List may be used for measurements. When this field is included, the UE assumes for all neighbour cells not in the Neighbour Cell List the RSS power bias is same as used for the serving cell or the camped cell.

soundingRS-FlexibleTiming Indicates the SRS flexible timing (if configured) for aperiodic SRS triggered by DL grant. If the SRS transmission is collided with ACK/NACK, postpone once to the next configured SRS transmission opportunity.

ul-Bandwidth Parameter: transmission bandwidth configuration, NRB, in uplink, see TS 36.101 [42], table 5.6-1. Value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on. If for FDD this parameter is absent, the uplink bandwidth is equal to the downlink bandwidth. For TDD this parameter is absent and it is equal to the downlink bandwidth.

ul-CarrierFreq For FDD: If absent, the (default) value determined from the default TX-RX frequency separation defined in TS 36.101 [42], table 5.7.3-1, applies. For TDD: This parameter is absent and it is equal to the downlink frequency.

ul-CyclicPrefixLength Parameter: Uplink cyclic prefix length see TS 36.211 [21], clause 5.2.1, where len1 corresponds to normal cyclic prefix and len2 corresponds to extended cyclic prefix.

| Conditional presence | Explanation |
| --- | --- |
| EDT | The field is optionally present, Need OR, if edt-Parameters is present; otherwise the field is not present and the UE shall delete any existing value for this field. |
| MP-A | The field is mandatory present for CE mode A. Otherwise the field is optional, Need OR. |
| MP-B | The field is mandatory present for CE mode B. Otherwise the field is optional, Need OR. |
| TDD | The field is optional for TDD, Need ON; it is not present for FDD and the UE shall delete any existing value for this field. |
| TDD2 | If tdd-Config-r10 is present, the field is optional, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD3 | If tdd-Config is present, the field is optional, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD-OR-NoR11 | If prach-ConfigSCell-r11 is absent, the field is optional for TDD, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDDSCell | This field is mandatory present for TDD; it is not present for FDD and LAA SCell, and the UE shall delete any existing value for this field. |
| UL | If the SCell is part of the STAG or concerns the PSCell or PUCCH SCell and if ul-Configuration is included, the field is optional, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| ULSCell | For the PSCell (IE is included in RadioResourceConfigCommonPSCell) the field is absent. Otherwise, if the SCell is part of the STAG and if ul-Configuration is included, the field is optional, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| ULSRS | If ul-Configuration-r10 is absent, the field is optional, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	RadioResourceConfigDedicated

The IE RadioResourceConfigDedicated is used to setup/modify/release RBs, to modify the MAC main configuration, to modify the SPS configuration and to modify dedicated physical configuration.

RadioResourceConfigDedicated information element

-- ASN1START

RadioResourceConfigDedicated ::=		SEQUENCE {

srb-ToAddModList					SRB-ToAddModList			OPTIONAL,		-- Cond HO-Conn

drb-ToAddModList					DRB-ToAddModList			OPTIONAL,		-- Cond HO-toEUTRA

drb-ToReleaseList					DRB-ToReleaseList			OPTIONAL,		-- Need ON

mac-MainConfig						CHOICE {

explicitValue					MAC-MainConfig,

defaultValue					NULL

}		OPTIONAL,																-- Cond HO-toEUTRA2

sps-Config							SPS-Config					OPTIONAL,		-- Need ON

physicalConfigDedicated				PhysicalConfigDedicated		OPTIONAL,		-- Need ON

...,

[[	rlf-TimersAndConstants-r9		RLF-TimersAndConstants-r9			OPTIONAL	-- Need ON

]],

[[	measSubframePatternPCell-r10	MeasSubframePatternPCell-r10		OPTIONAL	-- Need ON

]],

[[	neighCellsCRS-Info-r11			NeighCellsCRS-Info-r11				OPTIONAL	-- Need ON

]],

[[	naics-Info-r12				NAICS-AssistanceInfo-r12			OPTIONAL	-- Need ON

]],

[[	neighCellsCRS-Info-r13			NeighCellsCRS-Info-r13				OPTIONAL,	-- Cond CRSIM

rlf-TimersAndConstants-r13		RLF-TimersAndConstants-r13			OPTIONAL	-- Need ON

]],

[[	sps-Config-v1430				SPS-Config-v1430					OPTIONAL	-- Cond SPS

]],

[[	srb-ToAddModListExt-r15			SRB-ToAddModListExt-r15				OPTIONAL,	-- Need ON

srb-ToReleaseListExt-r15		INTEGER (4)							OPTIONAL,	-- Need ON

sps-Config-v1530				SPS-Config-v1530					OPTIONAL,	-- Need ON

crs-IntfMitigConfig-r15	CHOICE {

release					NULL,

setup					CHOICE {

crs-IntfMitigEnabled			NULL,

crs-IntfMitigNumPRBs	ENUMERATED {n6, n24}

}

}											OPTIONAL,		-- Need ON

neighCellsCRS-Info-r15			NeighCellsCRS-Info-r15		OPTIONAL,	-- Need ON

drb-ToAddModList-r15			DRB-ToAddModList-r15		OPTIONAL,		-- Need ON

drb-ToReleaseList-r15			DRB-ToReleaseList-r15		OPTIONAL,		-- Need ON

dummy							SEQUENCE (SIZE (1..2)) OF INTEGER (1..2)	OPTIONAL	-- Need ON

]],

[[	sps-Config-v1540				SPS-Config-v1540					OPTIONAL	-- Need ON

]],

[[

rlf-TimersAndConstantsMCG-Failure-r16	RLF-TimersAndConstantsMCG-Failure-r16

OPTIONAL,	-- Cond Split-SRB1-SRB3

crs-ChEstMPDCCH-ConfigDedicated-r16	SetupRelease{CRS-ChEstMPDCCH-ConfigDedicated-r16}	OPTIONAL,	-- Need ON

newUE-Identity-r16				C-RNTI						OPTIONAL		-- Need OP

]]

}

RadioResourceConfigDedicated-v1370 ::=		SEQUENCE {

physicalConfigDedicated-v1370		PhysicalConfigDedicated-v1370		OPTIONAL	-- Need ON

}

RadioResourceConfigDedicated-v13c0 ::=		SEQUENCE {

physicalConfigDedicated-v13c0		PhysicalConfigDedicated-v13c0

}

RadioResourceConfigDedicatedPSCell-r12 ::=		SEQUENCE {

-- UE specific configuration extensions applicable for an PSCell

physicalConfigDedicatedPSCell-r12		PhysicalConfigDedicated		OPTIONAL,	-- Need ON

sps-Config-r12							SPS-Config					OPTIONAL,	-- Need ON

naics-Info-r12							NAICS-AssistanceInfo-r12	OPTIONAL,	-- Need ON

...,

[[	neighCellsCRS-InfoPSCell-r13		NeighCellsCRS-Info-r13		OPTIONAL	-- Need ON

]],

[[	sps-Config-v1430				SPS-Config-v1430				OPTIONAL	-- Cond SPS2

]],

[[	sps-Config-v1530				SPS-Config-v1530				OPTIONAL,	-- Need ON

crs-IntfMitigEnabled-r15			BOOLEAN						OPTIONAL,	-- Need ON

neighCellsCRS-Info-r15				NeighCellsCRS-Info-r15		OPTIONAL	-- Need ON

]],

[[	sps-Config-v1540				SPS-Config-v1540				OPTIONAL	-- Need ON

]]

}

RadioResourceConfigDedicatedPSCell-v1370 ::=		SEQUENCE {

physicalConfigDedicatedPSCell-v1370		PhysicalConfigDedicated-v1370	OPTIONAL	-- Need ON

}

RadioResourceConfigDedicatedPSCell-v13c0 ::=		SEQUENCE {

physicalConfigDedicatedPSCell-v13c0		PhysicalConfigDedicated-v13c0

}

RadioResourceConfigDedicatedSCG-r12 ::=		SEQUENCE {

drb-ToAddModListSCG-r12				DRB-ToAddModListSCG-r12			OPTIONAL,	-- Need ON

mac-MainConfigSCG-r12				MAC-MainConfig					OPTIONAL,	-- Need ON

rlf-TimersAndConstantsSCG-r12		RLF-TimersAndConstantsSCG-r12	OPTIONAL,	-- Need ON

...,

[[	drb-ToAddModListSCG-r15			DRB-ToAddModListSCG-r15			OPTIONAL	-- Need ON

]],

[[	srb-ToAddModListSCG-r15			SRB-ToAddModList					OPTIONAL,	-- Need ON

srb-ToReleaseListSCG-r15			SRB-ToReleaseList-r15				OPTIONAL	-- Need ON

]],

[[	-- NE-DC additions for release of RLC bearer config for DRBs

drb-ToReleaseListSCG-r15		DRB-ToReleaseList-r15		OPTIONAL		-- Need ON

]]

}

RadioResourceConfigDedicatedSCell-r10 ::=	SEQUENCE {

-- UE specific configuration extensions applicable for an SCell

physicalConfigDedicatedSCell-r10		PhysicalConfigDedicatedSCell-r10	OPTIONAL,	-- Need ON

...,

[[	mac-MainConfigSCell-r11			MAC-MainConfigSCell-r11			OPTIONAL	-- Cond SCellAdd

]],

[[	naics-Info-r12				NAICS-AssistanceInfo-r12		OPTIONAL	-- Need ON

]],

[[	neighCellsCRS-InfoSCell-r13			NeighCellsCRS-Info-r13		OPTIONAL	-- Need ON

]],

[[	physicalConfigDedicatedSCell-v1370	PhysicalConfigDedicatedSCell-v1370	OPTIONAL	-- Need ON

]],

[[	crs-IntfMitigEnabled-r15			BOOLEAN						OPTIONAL,	-- Need ON

neighCellsCRS-Info-r15				NeighCellsCRS-Info-r15		OPTIONAL,	-- Need ON

sps-Config-v1530					SPS-Config-v1530			OPTIONAL	-- Need ON

]]

}

RadioResourceConfigDedicatedSCell-v13c0 ::=	SEQUENCE {

physicalConfigDedicatedSCell-v13c0	PhysicalConfigDedicatedSCell-v13c0

}

SRB-ToAddModList ::=				SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddModListExt-r15 ::=				SEQUENCE (SIZE (1)) OF SRB-ToAddMod

SRB-ToAddMod ::=	SEQUENCE {

srb-Identity						INTEGER (1..2),

rlc-Config							CHOICE {

explicitValue						RLC-Config,

defaultValue						NULL

}		OPTIONAL,																-- Cond Setup

logicalChannelConfig				CHOICE {

explicitValue						LogicalChannelConfig,

defaultValue						NULL

}		OPTIONAL,																-- Cond Setup

...,

[[	pdcp-verChange-r15				ENUMERATED {true}		OPTIONAL,			-- Cond NR-PDCP

rlc-Config-v1530				RLC-Config-v1530		OPTIONAL,			-- Need ON

rlc-BearerConfigSecondary-r15	RLC-BearerConfig-r15	OPTIONAL,			-- Need ON

srb-Identity-v1530				INTEGER (4)				OPTIONAL			-- Need ON

]],

[[	rlc-Config-v1560					RLC-Config-v1510		OPTIONAL		-- Need ON

]],

[[	rlc-Config-v17xx					RLC-Config-v17xx		OPTIONAL		-- Need ON

]]

}

DRB-ToAddModList ::=				SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddModList-r15 ::=			SEQUENCE (SIZE (1..maxDRB-r15)) OF DRB-ToAddMod

DRB-ToAddModListSCG-r12 ::=		SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddModSCG-r12

DRB-ToAddModListSCG-r15 ::=		SEQUENCE (SIZE (1..maxDRB-r15)) OF DRB-ToAddModSCG-r12

DRB-ToAddMod ::=	SEQUENCE {

eps-BearerIdentity					INTEGER (0..15)			OPTIONAL,		-- Cond DRB-Setup

drb-Identity						DRB-Identity,

pdcp-Config							PDCP-Config				OPTIONAL,		-- Cond PDCP

rlc-Config							RLC-Config				OPTIONAL,		-- Cond SetupM

logicalChannelIdentity				INTEGER (3..10)			OPTIONAL,		-- Cond DRB-SetupM

logicalChannelConfig				LogicalChannelConfig	OPTIONAL,		-- Cond SetupM

...,

[[	drb-TypeChange-r12					ENUMERATED {toMCG}		OPTIONAL,		-- Need OP

rlc-Config-v1250					RLC-Config-v1250		OPTIONAL		-- Need ON

]],

[[	rlc-Config-v1310					RLC-Config-v1310		OPTIONAL,		-- Need ON

drb-TypeLWA-r13						BOOLEAN					OPTIONAL,		-- Need ON

drb-TypeLWIP-r13					ENUMERATED {lwip, lwip-DL-only,

lwip-UL-only, eutran}		OPTIONAL		-- Need ON

]],

[[	rlc-Config-v1430					RLC-Config-v1430		OPTIONAL,		-- Need ON

lwip-UL-Aggregation-r14				BOOLEAN					OPTIONAL,		-- Cond LWIP

lwip-DL-Aggregation-r14				BOOLEAN					OPTIONAL,		-- Cond LWIP

lwa-WLAN-AC-r14			ENUMERATED {ac-bk, ac-be, ac-vi, ac-vo}	OPTIONAL	-- Cond UL-LWA

]],

[[	rlc-Config-v1510					RLC-Config-v1510		OPTIONAL		-- Need ON

]],

[[	rlc-Config-v1530					RLC-Config-v1530		OPTIONAL,		-- Need ON

rlc-BearerConfigSecondary-r15		RLC-BearerConfig-r15	OPTIONAL,		-- Need ON

logicalChannelIdentity-r15			INTEGER (32..38)		OPTIONAL		-- Need ON

]],

[[	daps-HO-r16							ENUMERATED {true}		OPTIONAL		-- Cond DAPS

]],

[[	rlc-Config-v17xx					RLC-Config-v17xx		OPTIONAL		-- Need ON

]]

}

DRB-ToAddModSCG-r12 ::=	SEQUENCE {

drb-Identity-r12					DRB-Identity,

drb-Type-r12						CHOICE {

split-r12							NULL,

scg-r12								SEQUENCE {

eps-BearerIdentity-r12				INTEGER (0..15)	OPTIONAL,	-- Cond DRB-Setup

pdcp-Config-r12						PDCP-Config		OPTIONAL	-- Cond PDCP-S

}

}															OPTIONAL,	-- Cond SetupS2

rlc-ConfigSCG-r12					RLC-Config				OPTIONAL,	-- Cond SetupS

rlc-Config-v1250						RLC-Config-v1250			OPTIONAL,	-- Need ON

logicalChannelIdentitySCG-r12		INTEGER (3..10)			OPTIONAL,	-- Cond DRB-SetupS

logicalChannelConfigSCG-r12			LogicalChannelConfig	OPTIONAL,	-- Cond SetupS

...,

[[	rlc-Config-v1430					RLC-Config-v1430		OPTIONAL		-- Need ON

]],

[[	logicalChannelIdentitySCG-r15		INTEGER (32..38)	OPTIONAL,			-- Need ON

rlc-Config-v1530					RLC-Config-v1530		OPTIONAL,		-- Need ON

rlc-BearerConfigSecondary-r15		RLC-BearerConfig-r15	OPTIONAL		-- Need ON

]],

[[	rlc-Config-v1560					RLC-Config-v1510		OPTIONAL		-- Need ON

]]

}

DRB-ToReleaseList ::=				SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

DRB-ToReleaseList-r15 ::=			SEQUENCE (SIZE (1..maxDRB-r15)) OF DRB-Identity

SRB-ToReleaseList-r15 ::=			SEQUENCE (SIZE (1..2)) OF INTEGER (1..2)

MeasSubframePatternPCell-r10 ::=		CHOICE {

release								NULL,

setup							MeasSubframePattern-r10

}

NeighCellsCRS-Info-r11 ::=		CHOICE {

release							NULL,

setup							CRS-AssistanceInfoList-r11

}

CRS-AssistanceInfoList-r11 ::=	SEQUENCE (SIZE (1..maxCellReport)) OF CRS-AssistanceInfo-r11

CRS-AssistanceInfo-r11 ::= SEQUENCE {

physCellId-r11						PhysCellId,

antennaPortsCount-r11				ENUMERATED {an1, an2, an4, spare1},

mbsfn-SubframeConfigList-r11		MBSFN-SubframeConfigList,

...,

[[	mbsfn-SubframeConfigList-v1430	MBSFN-SubframeConfigList-v1430		OPTIONAL	-- Need ON

]]

}

NeighCellsCRS-Info-r13 ::=		CHOICE {

release							NULL,

setup							CRS-AssistanceInfoList-r13

}

CRS-AssistanceInfoList-r13 ::=	SEQUENCE (SIZE (1..maxCellReport)) OF CRS-AssistanceInfo-r13

CRS-AssistanceInfo-r13 ::= SEQUENCE {

physCellId-r13						PhysCellId,

antennaPortsCount-r13				ENUMERATED {an1, an2, an4, spare1},

mbsfn-SubframeConfigList-r13		MBSFN-SubframeConfigList			OPTIONAL,	-- Need ON

...,

[[	mbsfn-SubframeConfigList-v1430	MBSFN-SubframeConfigList-v1430		OPTIONAL	-- Need ON

]]

}

NeighCellsCRS-Info-r15 ::= CHOICE {

release								NULL,

setup								CRS-AssistanceInfoList-r15

}

CRS-AssistanceInfoList-r15 ::= SEQUENCE (SIZE (1..maxCellReport)) OF CRS-AssistanceInfo-r15

CRS-AssistanceInfo-r15 ::= SEQUENCE {

physCellId-r15						PhysCellId,

crs-IntfMitigEnabled-r15			ENUMERATED {enabled}				OPTIONAL	-- Need ON

}

NAICS-AssistanceInfo-r12 ::=		CHOICE {

release							NULL,

setup							SEQUENCE {

neighCellsToReleaseList-r12		NeighCellsToReleaseList-r12			OPTIONAL	,	-- Need ON

neighCellsToAddModList-r12		NeighCellsToAddModList-r12			OPTIONAL,	-- Need ON

servCellp-a-r12					P-a								OPTIONAL	-- Need ON

}

}

NeighCellsToReleaseList-r12 ::=	SEQUENCE (SIZE (1..maxNeighCell-r12)) OF PhysCellId

NeighCellsToAddModList-r12 ::=	SEQUENCE (SIZE (1..maxNeighCell-r12)) OF NeighCellsInfo-r12

NeighCellsInfo-r12	::=		SEQUENCE {

physCellId-r12					PhysCellId,

p-b-r12						INTEGER (0..3),

crs-PortsCount-r12				ENUMERATED {n1, n2, n4, spare},

mbsfn-SubframeConfig-r12		MBSFN-SubframeConfigList				OPTIONAL,	-- Need ON

p-aList-r12					SEQUENCE (SIZE (1..maxP-a-PerNeighCell-r12)) OF P-a,

transmissionModeList-r12		BIT STRING (SIZE(8)),

resAllocGranularity-r12			INTEGER (1..4),

...

}

P-a ::= ENUMERATED {	dB-6, dB-4dot77, dB-3, dB-1dot77,

dB0, dB1, dB2, dB3}

RLC-BearerConfig-r15 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

rlc-Config-r15						RLC-Config-r15				OPTIONAL,	-- Need ON

logicalChannelIdentityConfig-r15	CHOICE {

logicalChannelIdentity-r15			INTEGER (1..10),

logicalChannelIdentityExt-r15		INTEGER (32..38)

},

logicalChannelConfig-r15			LogicalChannelConfig		OPTIONAL	-- Need ON

}

}

-- ASN1STOP

RadioResourceConfigDedicated field descriptions

crs-ChEstMPDCCH-ConfigDedicated Indicates whether use of CRS for improving channel estimation on MPDCCH is enabled in RRC_CONNECTED. If this field is not configured, the field crs-ChEstMPDCCH-ConfigCommon in SystemInformationBlockType2 applies, if present.

crs-IntfMitigConfig crs-IntfMitigEnabled-r15 indicates CRS interference mitigation is enabled for the cell, as specified in TS 36.133 [16], clause 3.6.1.1. For BL UEs supporting ce-CRS-IntfMitig, presence of this field indicates CRS interference mitigation is enabled in the cell, as specified in TS 36.133 [16], clauses 3.6.1.2 and 3.6.1.3, and the value crs-IntfMitigNumPRBs indicates number of PRBs, i.e. 6 or 24 PRBs, for CRS transmission in the central cell BW when CRS interference mitigation is enabled. For UEs not supporting this feature, the behaviour is undefined if this field is configured and the field cellBarred in SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs or UEs in CE) is set to notbarred.

crs-PortsCount Parameter represents the number of antenna ports for cell-specific reference signal used by the signaled neighboring cell where n1 corresponds to 1 antenna port, n2 to 2 antenna ports etc. see TS 36.211 [21], clause 6.10.1.

daps-HO This field indicates that the handover, triggered in the same RRCConnectionReconfiguration message, shall be performed as a DAPS HO for the DRB. daps-HO is not configured if sidelink is configured.

drb-Identity In case of DC, the DRB identity is unique within the scope of the UE i.e. an SCG DRB can not use the same value as used for an MCG or split DRB. For a split DRB the same identity is used for the MCG- and SCG parts of the configuration.

drb-ToAddModList When drb-ToAddModList-r15 is configured, UE shall ignore the drb-ToAddModList (without suffix).

drb-ToAddModListSCG When an SCG is configured, E-UTRAN configures at least one SCG or split DRB. When drb-ToAddModListSCG-r15 is configured, UE shall ignore the drb-ToAddModListSCG (without suffix). When NE-DC is configured, this field indicates the SCG RLC bearers to be (re-)configured.

drb-ToReleaseList When drb-ToReleaseList-r15 is configured, UE shall ignore the drb-ToReleaseList (without suffix).

drb-ToReleaseListSCG When NE-DC is configured, this field indicates the SCG RLC bearers to be released.

drb-Type This field indicates whether the DRB is split or SCG DRB. E-UTRAN does not configure split and SCG DRBs simultaneously for the UE.

drb-TypeChange Indicates that a split/SCG DRB is reconfigured to an MCG DRB (i.e. E-UTRAN only signals the field in case the DRB type changes).

drb-TypeLWA Indicates whether a DRB is (re)configured as an LWA DRB or an LWA DRB is reconfigured not to use WLAN resources. NOTE 1

drb-TypeLWIP Indicates whether a DRB is (re)configured to use LWIP Tunnel in UL and DL (value lwip), DL only (value lwip-DL-only), UL only (value lwip-UL-only) or not to use LWIP Tunnel (value eutran).

dummy This field is not used in the specification. If received it shall be ignored by the UE.

logicalChannelConfig For SRBs a choice is used to indicate whether the logical channel configuration is signalled explicitly or set to the default logical channel configuration for SRB1 as specified in 9.2.1.1 or for SRB2 as specified in 9.2.1.2.

logicalChannelIdentity, LogicalChannelIdentityExt The logical channel identity for both UL and DL. Value 4 is not configured for DRBs if SRB4 is configured. When logicalChannelIdentity-r15 is signalled, UE shall ignore contents of logicalChannelIdentity (without suffix).

logicalChannelIdentitySCG The logical channel identity for both UL and DL. When logicalChannelIdentitySCG-r15 is signalled, UE shall ignore contents of logicalChannelIdentitySCG (without suffix).

lwa-WLAN-AC For LWA bearers, indicates the corresponding WLAN access category for uplink. AC-BK (value ac-bk) corresponds to Background access category, AC-BE (value ac-be) corresponds to Best Effort access category, AC-VI (value ac-vi) corresponds to Video access category and AC-VO (value ac-vo) corresponds to Voice access category as defined by IEEE 802.11-2012 [67]. If lwa-WLAN-AC is not configured, it is left up to UE to decide which IEEE 802.11 AC value to use when performing transmissions of packets for this DRB over WLAN in the uplink.

lwip-DL-Aggregation, lwip-UL-Aggregation Indicates whether LWIP is configured to utilize LWIP aggregation in DL or UL.

mac-MainConfig Although the ASN.1 includes a choice that is used to indicate whether the mac-MainConfig is signalled explicitly or set to the default MAC main configuration as specified in 9.2.2, EUTRAN does not apply "defaultValue".

mbsfn-SubframeConfig Defines the MBSFN subframe configuration used by the signaled neighboring cell. If absent, UE assumes no MBSFN configuration for the neighboring cell.

measSubframePatternPCell Time domain measurement resource restriction pattern for the PCell measurements (RSRP, RSRQ and the radio link monitoring).

neighCellsCRS-Info, neighCellsCRS-InfoSCell, neighCellsCRS-InfoPSCell This field contains assistance information used by the UE to mitigate interference from CRS while performing RRM/RLM/CSI measurement or data demodulation or DL control channel demodulation. When the received CRS assistance information is for a cell with CRS non-colliding with that of the CRS of the cell to measure, the UE may use the CRS assistance information to mitigate CRS interference. When the received CRS assistance information is for a cell with CRS colliding with that of the CRS of the cell to measure, the UE may use the CRS assistance information to mitigate CRS interference RRM/RLM (as specified in TS 36.133 [16]) and for CSI (as specified in TS 36.101 [42]) on the subframes indicated by measSubframePatternPCell, measSubframePatternConfigNeigh, csi-MeasSubframeSet1 if configured, and the CSI subframe set 1 if csi-MeasSubframeSets-r12 is configured. The UE may use CRS assistance information to mitigate CRS interference from the cells in the CRS-AssistanceInfoList for the demodulation purpose or DL control channel demodulation as specified in TS 36.101 [42]. EUTRAN does not configure neighCellsCRS-Info-r11 or neighCellsCRS-Info-r13 if eimta-MainConfigPCell-r12 is configured.

neighCellsToAddModList This field contains assistance information used by the UE to cancel and suppress interference of a neighbouring cell. If this field is present for a neighbouring cell, the UE assumes that the transmission parameters listed in the sub-fields are used by the neighbouring cell. If this field is present for a neighbouring cell, the UE assumes the neighbour cell is subframe and SFN synchronized to the serving cell, has the same system bandwidth, UL/DL and special subframe configuration, and cyclic prefix length as the serving cell.

newUE-Identity C-RNTI used after moving to RRC_CONNECTED in response to transmission using PUR.

p-aList Indicates the restricted subset of power offset for QPSK, 16QAM, and 64QAM PDSCH transmissions for the neighbouring cell by using the parameter, see TS 36.213 [23], clause 5.2. Value dB-6 corresponds to -6 dB, dB-4dot77 corresponds to -4.77 dB etc.

p-b Parameter: , indicates the cell-specific ratio used by the signaled neighboring cell, see TS 36.213 [23], Table 5.2-1.

pdcp-verChange Indicates that the PDCP version of the SRB is changed from NR PDCP to E-UTRA PDCP. Network only configures this version change for during handover, resume and first reconfiguration after re-establishment. E-UTRAN does not include this field when SRB-ToAddMod is included in srb-ToAddModListSCG.

physicalConfigDedicated The default dedicated physical configuration is specified in 9.2.4.

resAllocGranularity Indicates the resource allocation and precoding granularity in PRB pair level of the signaled neighboring cell, see TS 36.213 [23], clause 7.1.6.

rlc-BearerConfigSecondary The configuration of a secondary RLC bearer within the same Cell Group as may e.g. be used in case of PDCP duplication using CA. The configuration comprises a (secondary) RLC entity, a logical channel identity and a logical channel configuration. E-UTRAN may configure this for SRB1, SRB2 and DRBs. For SRBs, E-UTRAN only configures the field for MCG (i.e. if included in radioResourceConfigDedicated. E-UTRAN configures the same RLC mode (AM/ UM) as used for the original RLC entity. The primary RLC entity is configured by RLC-Config.

rlc-Config For SRBs a choice is used to indicate whether the RLC configuration is signalled explicitly or set to the values defined in the default RLC configuration for SRB1 in 9.2.1.1 or for SRB2 in 9.2.1.2. RLC AM is the only applicable RLC mode for SRB1 and SRB2. E-UTRAN does not reconfigure the RLC mode of DRBs except when a full configuration option is used, and may reconfigure the RLC SN field size and the AM RLC LI field size only upon handover within E-UTRA or upon the first reconfiguration after RRC connection re-establishment or upon SCG Change for SCG and split DRBs.

servCellp-a Indicates the power offset for QPSK C-RNTI based PDSCH transmissions used by the serving cell, see TS 36.213 [23], clause 5.2. Value dB-6 corresponds to -6 dB, dB-4dot77 corresponds to -4.77 dB etc.

sps-Config The default SPS configuration is specified in 9.2.3. Except for handover or releasing SPS for MCG, E-UTRAN does not reconfigure sps-Config for MCG when there is a configured downlink assignment or a configured uplink grant for MCG (see TS 36.321 [6]). Except for SCG change or releasing SPS for SCG, E-UTRAN does not reconfigure sps-Config for SCG when there is a configured downlink assignment or a configured uplink grant for SCG (see TS 36.321 [6]). In one serving cell, sps-Config-v1530 is not present simultaneously with either sps-Config (without suffix) or sps-Config-r12.

srb-Identity Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 4 is applicable for SRB4 only, if configured. For a split SRB the same identity is used for the MCG and NR SCG RLC bearer configurations. If srb-Identity-v1530 is received, the UE shall ignore srb-Identity (i.e. without suffix).

srb-Identity-v1530 E-UTRAN does not include this field when SRB-ToAddMod is included in srb-ToAddModListSCG.

srb-ToAddModListExt The field is to configure SRB4.

srb-ToAddModList E-UTRAN configures the same RAT type (i.e. EUTRA or NR) for PDCP configuration of SRB1 and SRB2.

transmissionModeList Indicates a subset of transmission mode 1, 2, 3, 4, 6, 8, 9, 10, for the signaled neighboring cell for which NeighCellsInfo applies. When TM10 is signaled, other signaled transmission parameters in NeighCellsInfo are not applicable to up to 8 layer transmission scheme of TM10. E-UTRAN may indicate TM9 when TM10 with QCL type A and DMRS scrambling with in TS 36.211 [21], clause 6.10.3.1, is used in the signalled neighbour cell and TM9 or TM10 with QCL type A and DMRS scrambling with in TS 36.211 [21], clause 6.10.3.1, is used in the serving cell. UE behaviour with NAICS when TM10 is used is only defined when QCL type A and DMRS scrambling with in TS 36.211 [21], clause 6.10.3.1, is used for the serving cell and all signalled neighbour cells. The first/ leftmost bit is for transmission mode 1, the second bit is for transmission mode 2, and so on.

NOTE 1:	It is up to eNB to ensure that the field indicating LWA bearer type is set to FALSE when LWA bearer is no longer used (e.g. during handover or re-establishment where LWA configuration is released).

| Conditional presence | Explanation |
| --- | --- |
| CRSIM | The field is optionally present, need ON, if neighCellsCRS-Info-r11 is not present; otherwise it is not present. |
| DRB-Setup | The field is mandatory present if the corresponding DRB is being set up and the UE is connected to EPC; otherwise it is not present. |
| DRB-SetupM | The field is: - mandatory present: - for the UE without SCG: upon setup of MCG DRB; - for E-UTRA DC, upon setup of MCG or split DRB; - for (NG)EN-DC: - upon setup of MCG RLC bearer; - optionally present, Need ON: - for E-UTRA DC, upon change from SCG to MCG DRB; - for (NG)EN-DC: - upon change of keyToUse, as defined in TS 38.331 [82], for a DRB configured with an MCG RLC bearer; - when configured with MCG RLC bearer, upon change of S-KgNB without handover; - not present otherwise. |
| DRB-SetupS | The field is: - mandatory present: - for E-UTRA DC: - upon setup of SCG or split DRB; - upon change from MCG to split DRB; - for NE-DC: - upon setup of SCG RLC bearer; - optionally present, Need ON: - for E-UTRA DC, upon change from MCG to SCG DRB; - for NE-DC, upon change of keyToUse, as defined in TS 38.331 [82], for a DRB configured with an SCG RLC bearer; - not present otherwise. |
| HO-Conn | The field is mandatory present in case of handover to E-UTRA or when the fullConfig is included in the RRCConnectionReconfiguration message or in case of RRC connection establishment (excluding RRConnectionResume); otherwise the field is optionally present, need ON. Upon connection establishment/ re-establishment only SRB1 is applicable (excluding RRConnectionResume). |
| HO-toEUTRA | The field is mandatory present - in case of handover to E-UTRA with the configuration for at least one MCG RLC bearer; or - when the fullConfig is included in the RRCConnectionReconfiguration message with the configuration for at least one MCG bearer or split data bearer; In case of RRC connection establishment (excluding RRConnectionResume); and RRC connection re-establishment the field is not present; otherwise the field is optionally present, need ON. |
| HO-toEUTRA2 | The field is mandatory present in case of handover to E-UTRA or when the fullConfig is included in the RRCConnectionReconfiguration message; otherwise the field is optionally present, need ON. |
| LWIP | The field is optionally present, Need ON, if drb-TypeLWIP-r13 is configured and not set to eutran; otherwise it is not present and the UE shall delete any existing value for this field. |
| DAPS | This field is optionally present, Need ON, - in case of handover within E-UTRA when the fullConfig and the rach-Skip are not included in the RRCConnectionReconfiguration message; and - when the uplinkDataCompression and the ethernetHeaderCompression are not configured for the DRB; and - when SCell(s) and SCG are not configured; and - when the conditionalReconfiguration for CHO is not configured; and - when the RRCConnectionReconfiguration message is not included in a conditionalReconfiguration. Otherwise the field is not present. |
| NR-PDCP | The field is optional present, Need ON, when the SRB is configured with NR-PDCP prior to reception of this reconfiguration message. Otherwise it is not present. |
| PDCP | The field is mandatory present: - when connected to E-UTRA/EPC: - for the bearers configured with E-UTRA PDCP, if the corresponding DRB is being setup; the field is optionally present, need ON: : - when connected to E-UTRA/EPC: - for the bearers configured with E-UTRA PDCP, upon reconfiguration of the corresponding split DRB or LWA DRB, upon the corresponding DRB type change from split to MCG bearer, upon the corresponding DRB type change from MCG to split bearer or LWA bearer, upon the corresponding DRB type change from LWA to LTE only bearer, upon handover within E-UTRA and upon the first reconfiguration after re-establishment but in all these cases only when fullConfig is not included in the RRCConnectionReconfiguration message; otherwise it is not present. |
| PDCP-S | The field is mandatory present if the corresponding DRB is being setup; the field is optionally present, need ON, upon SCG change; otherwise it is not present. |
| RLC-Setup | This field is optionally present if the corresponding DRB is being setup, need ON; otherwise it is not present. |
| SCellAdd | The field is optionally present, need ON, upon SCell addition; otherwise it is not present. |
| Setup | The field is mandatory present if the corresponding SRB/DRB is being setup; otherwise the field is optionally present, need ON. |
| SetupM | The field is mandatory present upon setup of an MCG or split DRB, or upon setup of MCG RLC bearer; otherwise the field is optionally present, need ON. |
| SetupS | The field is mandatory present: - for E-UTRA DC: - upon setup of an SCG or split DRB, - upon change from MCG to split DRB; - for NE-DC, upon setup of SCG RLC bearer; otherwise the field is optionally present, need ON. |
| SetupS2 | The field is: - mandatory present: - for E-UTRA DC: - upon setup of an SCG or split DRB, as well as upon change from MCG to split or SCG DRB. - optionally present, need ON: - for E-UTRA DC: - for an SCG DRB otherwise the field is not present. |
| Split-SRB1-SRB3 | This field is optionally present, Need ON, if the UE is configured with split SRB1 or SRB3. It is absent otherwise. |
| SPS | The field is optionally present, need ON, if sps-Config (without suffix) is not configured; otherwise it is not present. |
| SPS2 | The field is optionally present, need ON, if sps-Config-r12 is not configured; otherwise it is not present. |
| UL-LWA | The field is optionally present, need ON if ul-LWA-Config-r14 is present. Otherwise the field is not present. |

#### –	RCLWI-Configuration

The IE RCLWI-Configuration is used to add, modify or release the RCLWI configuration.

-- ASN1START

RCLWI-Configuration-r13 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

rclwi-Config-r13					RCLWI-Config-r13

}

}

RCLWI-Config-r13 ::=				SEQUENCE {

command								CHOICE {

steerToWLAN-r13						SEQUENCE {

mobilityConfig-r13					WLAN-Id-List-r12

},

steerToLTE-r13						NULL

},

...

}

-- ASN1STOP

#### –	ResourceReservationConfig

The IE ResourceReservationConfig is used to specify the resource reservation, e.g. for coexistence with NR.

ResourceReservationConfig information element

-- ASN1START

ResourceReservationConfigDL-r16 ::= SEQUENCE {

periodicityStartPos-r16		PeriodicityStartPos-r16,

resourceReservationFreq-r16	CHOICE {

rbg-Bitmap1dot4				BIT STRING (SIZE (6)),

rbg-Bitmap3					BIT STRING (SIZE (8)),

rbg-Bitmap5					BIT STRING (SIZE (13)),

rbg-Bitmap10				BIT STRING (SIZE (17)),

rbg-Bitmap15				BIT STRING (SIZE (19)),

rbg-Bitmap20				BIT STRING (SIZE (25))

}	OPTIONAL, -- Need OP

slotBitmap-r16				CHOICE {

slotPattern10ms				BIT STRING (SIZE (20)),

slotPattern40ms				BIT STRING (SIZE (80))

},

symbolBitmap1-r16			BIT STRING (SIZE (7))	OPTIONAL,	-- Cond Bitmap1

symbolBitmap2-r16			BIT STRING (SIZE (7))	OPTIONAL,	-- Cond Bitmap2

...

}

ResourceReservationConfigUL-r16 ::= SEQUENCE {

periodicityStartPos-r16		PeriodicityStartPos-r16,

slotBitmap-r16				CHOICE {

slotPattern10ms				BIT STRING (SIZE (20)),

slotPattern40ms				BIT STRING (SIZE (80))

} OPTIONAL,	-- Cond FDDandTDDnoDL

symbolBitmap1-r16			BIT STRING (SIZE (7))	OPTIONAL,	-- Cond Bitmap1

symbolBitmap2-r16			BIT STRING (SIZE (7))	OPTIONAL,	-- Cond Bitmap2

...

}

PeriodicityStartPos-r16 ::=		CHOICE {

periodicity10ms					NULL,

periodicity20ms					INTEGER(0..1),

periodicity40ms					INTEGER(0..3),

periodicity80ms					INTEGER(0..7),

periodicity160ms				INTEGER(0..15),

spare3 NULL, spare2 NULL, spare1 NULL

}

-- ASN1STOP

ResourceReservationConfig field descriptions

periodicityStartPos Indicates periodicity and start offset of the reserved resources. Value set to periodicity10ms corresponds to periodicity 10 milliseconds and corresponding start position is 0, value set to periodicity20ms corresponds to periodicity 20 milliseconds and corresponding start position in milliseconds = indicated value * 10ms, and so on.

resourceReservationFreq Downlink frequency domain resource reservation bitmap where each bit corresponds to a resource block group (RBG), see TS 36.213 [23]. Value rbg-Bitmap1dot4 corresponds to 1.4 MHz system bandwidth, value rbg-Bitmap3 corresponds to 3 MHz system bandwidth, and so on. If the field is absent, all RBGs in the system bandwidth are reserved.

slotBitmap Slot-level resource reservation configuration. Value slotPattern10ms corresponds to 10ms slot pattern and slotPattern40ms corresponds to 40ms slot pattern, see TS 36.213 [23] for DL and TS 36.211 [21] for UL. The first/leftmost 2-bits corresponds to the subframe #0 of the radio frame satisfying SFN mod periodicity = start position, as indicated by periopdicityStartPos. Two bits for each subframe coded as: 00: both slots are not reserved 01: the first slot is not reserved, the second slot is reserved 10: the first slot is reserved, the second slot is not reserved 11: both slots are reserved. For a UE that supports subframe-level resource reservation but does not support slot/symbol-level resource reservation, two bits for each subframe are interpreted as: 00: subframe is not reserved 01: subframe is reserved. E-UTRAN does not set the field to this value when included in dedicated signalling. 10: subframe is reserved. E-UTRAN does not set the field to this value when included in dedicated signalling. 11: subframe is reserved. If the field is not included in UL configuration, the value of the field from DL configuration applies.

symbolBitmap1, symbolBitmap2 Provides the symbol-level resource reservation for one subframe. If symbolBitmap1 is absent, value '01' in the slotBitmap corresponds to the whole 2nd slot being reserved. If symbolBitmap2 is absent, value '10' in the slotBitmap corresponds to the whole 1st slot being reserved. A UE that supports subframe-level resource reservation but does not support slot/symbol-level resource reservation shall ignore symbolBitmap1 and symbolBitmap2, if present.

| Conditional presence | Explanation |  |
| --- | --- | --- |
| Bitmap1 | The field is optionally present, need OR, if value of slotBitmap corresponding to at least one subframe is '01'; otherwise the field is not present. |
| Bitmap2 | The field is optionally present, need OR, if value of slotBitmap corresponding to at least one subframe is '10'; otherwise the field is not present. |
| FDDandTDDnoDL | The field is mandatory present for TDD when resource reservation for DL is not configured, and for FDD; otherwise the field is optionally present, need OP. |

#### –	RLC-Config

The IE RLC-Config is used to specify the RLC configuration of SRBs and DRBs.

RLC-Config information element

-- ASN1START

RLC-Config ::=				CHOICE {

am									SEQUENCE {

ul-AM-RLC							UL-AM-RLC,

dl-AM-RLC							DL-AM-RLC

},

um-Bi-Directional					SEQUENCE {

ul-UM-RLC							UL-UM-RLC,

dl-UM-RLC							DL-UM-RLC

},

um-Uni-Directional-UL				SEQUENCE {

ul-UM-RLC							UL-UM-RLC

},

um-Uni-Directional-DL				SEQUENCE {

dl-UM-RLC							DL-UM-RLC

},

...

}

RLC-Config-v1250 ::=				SEQUENCE {

ul-extended-RLC-LI-Field-r12			BOOLEAN,

dl-extended-RLC-LI-Field-r12			BOOLEAN

}

RLC-Config-v1310 ::=				SEQUENCE {

ul-extended-RLC-AM-SN-r13					BOOLEAN,

dl-extended-RLC-AM-SN-r13					BOOLEAN,

pollPDU-v1310								PollPDU-v1310		OPTIONAL	-- Need OR

}

RLC-Config-v1430 ::=				CHOICE {

release								NULL,

setup								SEQUENCE {

pollByte-r14						PollByte-r14

}

}

RLC-Config-v1510 ::=				SEQUENCE {

reestablishRLC-r15				ENUMERATED {true}

}

RLC-Config-v1530 ::=				CHOICE {

release								NULL,

setup								SEQUENCE {

rlc-OutOfOrderDelivery-r15			ENUMERATED {true}

}

}

RLC-Config-v17xx ::=				SEQUENCE {

t-ReorderingExt-r17					SetupRelease {T-ReorderingExt-r17}

}

RLC-Config-r15 ::=				SEQUENCE {

mode-r15								CHOICE {

am-r15								SEQUENCE {

ul-AM-RLC-r15						UL-AM-RLC-r15,

dl-AM-RLC-r15						DL-AM-RLC-r15

},

um-Bi-Directional-r15				SEQUENCE {

ul-UM-RLC-r15						UL-UM-RLC,

dl-UM-RLC-r15						DL-UM-RLC-r15

},

um-Uni-Directional-UL-r15			SEQUENCE {

ul-UM-RLC-r15						UL-UM-RLC

},

um-Uni-Directional-DL-r15			SEQUENCE {

dl-UM-RLC-r15						DL-UM-RLC-r15

}

},

reestablishRLC-r15					ENUMERATED {true}				OPTIONAL,	-- Need ON

rlc-OutOfOrderDelivery-r15			ENUMERATED {true}				OPTIONAL,	-- Need ON

...

}

UL-AM-RLC ::=						SEQUENCE {

t-PollRetransmit					T-PollRetransmit,

pollPDU								PollPDU,

pollByte							PollByte,

maxRetxThreshold					ENUMERATED {

t1, t2, t3, t4, t6, t8, t16, t32}

}

UL-AM-RLC-r15 ::=					SEQUENCE {

t-PollRetransmit-r15				T-PollRetransmit,

pollPDU-r15							PollPDU-r15,

pollByte-r15						PollByte-r14,

maxRetxThreshold-r15				ENUMERATED {

t1, t2, t3, t4, t6, t8, t16, t32},

extended-RLC-LI-Field-r15			BOOLEAN

}

DL-AM-RLC ::=						SEQUENCE {

t-Reordering						T-Reordering,

t-StatusProhibit					T-StatusProhibit

}

DL-AM-RLC-r15 ::=					SEQUENCE {

t-Reordering-r15					T-Reordering,

t-StatusProhibit-r15				T-StatusProhibit,

extended-RLC-LI-Field-r15			BOOLEAN

}

UL-UM-RLC ::=						SEQUENCE {

sn-FieldLength						SN-FieldLength

}

DL-UM-RLC ::=						SEQUENCE {

sn-FieldLength						SN-FieldLength,

t-Reordering						T-Reordering

}

DL-UM-RLC-r15 ::=					SEQUENCE {

sn-FieldLength-r15					SN-FieldLength-r15,

t-Reordering-r15					T-Reordering

}

SN-FieldLength ::=					ENUMERATED {size5, size10}

SN-FieldLength-r15 ::=				ENUMERATED {size5, size10, size16-r15}

T-PollRetransmit ::=				ENUMERATED {

ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800-v1310,

ms1000-v1310, ms2000-v1310, ms4000-v1310,

spare5, spare4, spare3, spare2, spare1}

PollPDU ::=							ENUMERATED {

p4, p8, p16, p32, p64, p128, p256, pInfinity}

PollPDU-v1310 ::=					ENUMERATED {

p512, p1024, p2048, p4096, p6144, p8192, p12288, p16384}

PollPDU-r15 ::=						ENUMERATED {

p4, p8, p16, p32, p64, p128, p256, p512, p1024,

p2048-r15, p4096-r15, p6144-r15, p8192-r15,

p12288-r15, p16384-r15, pInfinity}

PollByte ::=						ENUMERATED {

kB25, kB50, kB75, kB100, kB125, kB250, kB375,

kB500, kB750, kB1000, kB1250, kB1500, kB2000,

kB3000, kBinfinity, spare1}

PollByte-r14 ::=					ENUMERATED {

kB1, kB2, kB5, kB8, kB10, kB15, kB3500,

kB4000, kB4500, kB5000, kB5500, kB6000, kB6500,

kB7000, kB7500, kB8000, kB9000, kB10000, kB11000, kB12000,

kB13000, kB14000, kB15000, kB16000, kB17000, kB18000,

kB19000, kB20000, kB25000, kB30000, kB35000, kB40000}

T-Reordering ::=					ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms110,

ms120, ms130, ms140, ms150, ms160, ms170,

ms180, ms190, ms200, ms1600-v1310}

T-ReorderingExt-r17 ::=				ENUMERATED {ffs}

T-StatusProhibit ::=				ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800-v1310,

ms1000-v1310, ms1200-v1310, ms1600-v1310, ms2000-v1310, ms2400-v1310, spare2,

spare1}

-- ASN1STOP

Editor’s Note: Agreement: RAN2 confirm the SI agreement that the value range of the RLC t-Reordering timer will be extended to support IoT NTN. Agreement: On how to extend RLC t-Reordering in IoT NTN, wait for NR NTN agreements and see if they can be reused.

RLC-Config field descriptions

dl-extended-RLC-LI-Field, ul-extended-RLC-LI-Field Indicates the RLC LI field size. Value TRUE means that 15 bit LI length shall be used, otherwise 11 bit LI length shall be used; see TS 36.322 [7]. E-UTRAN enables this field only when RLC-Config (without suffix) is set to am.

maxRetxThreshold Parameter for RLC AM in TS 36.322 [7]. Value t1 corresponds to 1 retransmission, t2 to 2 retransmissions and so on.

reestablishRLC Indicates that RLC shall be re-established. For a UE configured with (NG)EN-DC, E-UTRAN may include this field for the (primary) RLC entity of an MCG RLC bearer of a DRB (used upon change from SN terminated split to MN terminated MCG RLC bearer). For a UE configured with NE-DC, E-UTRAN may include this field for the (primary) RLC entity of an SCG RLC bearer of a DRB or of an SRB (used upon key refresh for MN terminated split RB).

pollByte Parameter for RLC AM in TS 36.322 [7]. Value kB25 corresponds to 25 kBytes, kB50 to 50 kBytes and so on. kBInfinity corresponds to an infinite amount of kBytes. In case pollByte-r14 is signalled, the UE shall ignore pollByte (i.e. without suffix).

pollPDU Parameter for RLC AM in TS 36.322 [7]. Value p4 corresponds to 4 PDUs, p8 to 8 PDUs and so on. pInfinity corresponds to an infinite number of PDUs. In case pollPDU-r13 is signalled, the UE shall ignore pollPDU (i.e. without suffix). E-UTRAN enables pollPDU-v1310 field only when RLC-Config (without suffix) is set to am.

rlc-OutOfOrderDelivery Indicates that out-of-order delivery from RLC to PDCP is configured for this RLC entity as specified in TS 36.322 [7].

sn-FieldLength Indicates the UM RLC SN field size, see TS 36.322 [7], in bits. Value size5 means 5 bits, size10 means 10 bits.

t-PollRetransmit Timer for RLC AM in TS 36.322 [7], in milliseconds. Value ms5 means 5ms, ms10 means 10ms and so on. EUTRAN configures values msX-v1310 (with suffix) only if UE supports CE.

t-Reordering Timer for reordering in TS 36.322 [7], in milliseconds. Value ms0 means 0ms and behaviour as specified in 7.3.2 applies, ms5 means 5ms and so on.

t-ReorderingExt Timer for reordering in TS 36.322 [7], in milliseconds. Value ffs means FFS ms and so on. The UE shall use the extended value t-ReorderingExt-r17, if present, and ignore the value signaled by t-Reordering or t-Reordering-r15.

t-StatusProhibit Timer for status reporting in TS 36.322 [7], in milliseconds. Value ms0 means 0ms and behaviour as specified in 7.3.2 applies, ms5 means 5ms and so on. EUTRAN configures values msX-v1310 (with suffix) only if UE supports operation in CE.

ul-extended-RLC-AM-SN, dl-extended-RLC-AM-SN Indicates whether or not the UE shall use the extended SN and SO length for AM bearer. Value TRUE means that 16 bit SN length and 16 bit SO length shall be used, otherwise 10 bit SN length and 15 bit SO length shall be used; see TS 36.322 [7].

#### –	RLF-TimersAndConstants

The IE RLF-TimersAndConstants contains UE specific timers and constants applicable for UEs in RRC_CONNECTED.

RLF-TimersAndConstants information element

-- ASN1START

RLF-TimersAndConstants-r9 ::=			CHOICE {

release									NULL,

setup									SEQUENCE {

t301-r9								ENUMERATED {

ms100, ms200, ms300, ms400, ms600, ms1000, ms1500,

ms2000},

t310-r9								ENUMERATED {

ms0, ms50, ms100, ms200, ms500, ms1000, ms2000},

n310-r9								ENUMERATED {

n1, n2, n3, n4, n6, n8, n10, n20},

t311-r9								ENUMERATED {

ms1000, ms3000, ms5000, ms10000, ms15000,

ms20000, ms30000},

n311-r9								ENUMERATED {

n1, n2, n3, n4, n5, n6, n8, n10},

...

}

}

RLF-TimersAndConstants-r13 ::=			CHOICE {

release									NULL,

setup									SEQUENCE {

t301-v1310								ENUMERATED {

ms2500, ms3000, ms3500, ms4000, ms5000,

ms6000, ms8000, ms10000},

...,

[[	t310-v1330							ENUMERATED {ms4000, ms6000}	OPTIONAL	-- Need ON

]]

}

}

RLF-TimersAndConstantsSCG-r12 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

t313-r12							ENUMERATED {

ms0, ms50, ms100, ms200, ms500, ms1000, ms2000},

n313-r12							ENUMERATED {

n1, n2, n3, n4, n6, n8, n10, n20},

n314-r12							ENUMERATED {

n1, n2, n3, n4, n5, n6, n8, n10},

...

}

}

RLF-TimersAndConstantsMCG-Failure-r16 ::=	CHOICE {

release									NULL,

setup									SEQUENCE {

t316-r16								ENUMERATED {ms50, ms100, ms200, ms300, ms400,

ms500, ms600, ms1000, ms1500, ms2000},

...

}

}

-- ASN1STOP

RLF-TimersAndConstants field descriptions

n3xy Constants are described in clause 7.4. n1 corresponds with 1, n2 corresponds with 2 and so on.

t3xy Timers are described in clause 7.3. Value ms0 corresponds with 0 ms, ms50 corresponds with 50 ms and so on. E-UTRAN configures RLF-TimersAndConstants-r13 only if UE supports ce-ModeB. UE shall use the extended values t3xy-v1310 and t3xy-v1330, if present, and ignore the values signaled by t3xy-r9.

#### –	RN-SubframeConfig

The IE RN-SubframeConfig is used to specify the subframe configuration for an RN.

RN-SubframeConfig information element

-- ASN1START

RN-SubframeConfig-r10 ::=		SEQUENCE {

subframeConfigPattern-r10			CHOICE {

subframeConfigPatternFDD-r10	BIT STRING (SIZE(8)),

subframeConfigPatternTDD-r10	INTEGER (0..31)

}																	OPTIONAL,	-- Need ON

rpdcch-Config-r10				SEQUENCE {

resourceAllocationType-r10		ENUMERATED {type0, type1, type2Localized, type2Distributed,

spare4, spare3, spare2, spare1},

resourceBlockAssignment-r10			CHOICE {

type01-r10							CHOICE {

nrb6-r10							BIT STRING (SIZE(6)),

nrb15-r10							BIT STRING (SIZE(8)),

nrb25-r10							BIT STRING (SIZE(13)),

nrb50-r10							BIT STRING (SIZE(17)),

nrb75-r10							BIT STRING (SIZE(19)),

nrb100-r10							BIT STRING (SIZE(25))

},

type2-r10							CHOICE {

nrb6-r10							BIT STRING (SIZE(5)),

nrb15-r10							BIT STRING (SIZE(7)),

nrb25-r10							BIT STRING (SIZE(9)),

nrb50-r10							BIT STRING (SIZE(11)),

nrb75-r10							BIT STRING (SIZE(12)),

nrb100-r10							BIT STRING (SIZE(13))

},

...

},

demodulationRS-r10				CHOICE {

interleaving-r10				ENUMERATED {crs},

noInterleaving-r10				ENUMERATED {crs, dmrs}

},

pdsch-Start-r10					INTEGER (1..3),

pucch-Config-r10				CHOICE {

tdd								CHOICE {

channelSelectionMultiplexingBundling	SEQUENCE {

n1PUCCH-AN-List-r10			SEQUENCE (SIZE (1..4)) OF INTEGER (0..2047)

},

fallbackForFormat3				SEQUENCE {

n1PUCCH-AN-P0-r10				INTEGER (0..2047),

n1PUCCH-AN-P1-r10				INTEGER (0..2047)		OPTIONAL	-- Need OR

}

},

fdd								SEQUENCE {

n1PUCCH-AN-P0-r10				INTEGER (0..2047),

n1PUCCH-AN-P1-r10				INTEGER (0..2047)			OPTIONAL	-- Need OR

}

},

...

}																	OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

RN-SubframeConfig field descriptions

demodulationRS Indicates which reference signals are used for R-PDCCH demodulation according to TS 36.216 [55], clause 7.4.1. Value interleaving corresponds to cross-interleaving and value noInterleaving corresponds to no cross-interleaving according to TS 36.216 [55], clauses 7.4.2 and 7.4.3.

n1PUCCH-AN-List Parameter: , see TS 36.216, [55], clause 7.5.1. This parameter is only applicable for TDD. Configures PUCCH HARQ-ACK resources if the RN is configured to use HARQ-ACK channel selection, HARQ-ACK multiplexing or HARQ-ACK bundling.

n1PUCCH-AN-P0, n1PUCCH-AN-P1 Parameter: , for antenna port P0 and for antenna port P1 respectively, see TS 36.216, [55], clause 7.5.1, for FDD and [55], clause 7.5.2 for TDD.

pdsch-Start Parameter: DL-StartSymbol, see TS 36.216 [55], Table 5.4-1.

resourceAllocationType Represents the resource allocation used: type 0, type 1 or type 2 according to TS 36.213 [23], clause 7.1.6. Value type0 corresponds to type 0, value type1 corresponds to type 1, value type2Localized corresponds to type 2 with localized virtual resource blocks and type2Distributed corresponds to type 2 with distributed virtual resource blocks.

resourceBlockAssignment Indicates the resource block assignment bits according to TS 36.213 [23], clause 7.1.6. Value type01 corresponds to type 0 and type 1, and the value type2 corresponds to type 2. Value nrb6 corresponds to a downlink system bandwidth of 6 resource blocks, value nrb15 corresponds to a downlink system bandwidth of 15 resource blocks, and so on.

subframeConfigPatternFDD Parameter: SubframeConfigurationFDD, see TS 36.216 [55], Table 5.2-1. Defines the DL subframe configuration for eNB-to-RN transmission, i.e. those subframes in which the eNB may indicate downlink assignments for the RN. The radio frame in which the pattern starts (i.e. the radio frame in which the first bit of the subframeConfigPatternFDD corresponds to subframe #0) occurs when SFN mod 4 = 0.

subframeConfigPatternTDD Parameter: SubframeConfigurationTDD, see TS 36.216 [55], Table 5.2-2. Defines the DL and UL subframe configuration for eNB-RN transmission.

#### –	RSS-Config

The IE RSS-Config is used to specify the RSS configuration, see TS 36.211 [21].

RSS-Config information element

-- ASN1START

RSS-Config-r15 ::=				SEQUENCE {

duration-r15					ENUMERATED {sf8, sf16, sf32, sf40},

freqLocation-r15					INTEGER (0..98),

periodicity-r15					ENUMERATED {ms160, ms320, ms640, ms1280},

powerBoost-r15					ENUMERATED {dB0, dB3, dB4dot8, dB6},

timeOffset-r15					INTEGER (0..31)

}

-- ASN1STOP

RSS-Config field descriptions

duration Duration of RSS in subframes. Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes and so on.

freqLocation Frequency location (lowest PRB number) of RSS.

periodicity Periodicity of RSS. Value ms160 corresponds to 160 ms, value ms320 corresponds to 320 ms and so on.

powerBoost Power offset of RSS relative to CRS in dB. Value dB0 corresponds to 0 dB, value dB3 corresponds to 3 dB, value dB4dot8 corresponds to 4.8 dB and so on.

timeOffset Time offset of RSS in frames. The actual value of time offset is based on the value of periodicity, as follows: For periodicity 160 ms, only value range 0 to 15 are applicable. Actual value = timeOffset * 1 frame. For periodicity 320 ms, actual value = timeOffset * 1 frame. For periodicity 640 ms, actual value = timeOffset * 2 frames. For periodicity 1280 ms, actual value = timeOffset * 4 frames.

#### –	SchedulingRequestConfig

The IE SchedulingRequestConfig is used to specify the Scheduling Request related parameters

SchedulingRequestConfig information element

-- ASN1START

SchedulingRequestConfig ::=		CHOICE {

release								NULL,

setup								SEQUENCE {

sr-PUCCH-ResourceIndex				INTEGER (0..2047),

sr-ConfigIndex						INTEGER (0..157),

dsr-TransMax						ENUMERATED {

n4, n8, n16, n32, n64, spare3, spare2, spare1}

}

}

SchedulingRequestConfig-v1020 ::=	SEQUENCE {

sr-PUCCH-ResourceIndexP1-r10		INTEGER (0..2047)			OPTIONAL		-- Need OR

}

SchedulingRequestConfigSCell-r13 ::=		CHOICE {

release								NULL,

setup								SEQUENCE {

sr-PUCCH-ResourceIndex-r13			INTEGER (0..2047),

sr-PUCCH-ResourceIndexP1-r13		INTEGER (0..2047)			OPTIONAL,		-- Need OR

sr-ConfigIndex-r13					INTEGER (0..157),

dsr-TransMax-r13					ENUMERATED {

n4, n8, n16, n32, n64, spare3, spare2, spare1}

}

}

SchedulingRequestConfig-v1530 ::=	CHOICE {

release								NULL,

setup								SEQUENCE {

sr-SlotSPUCCH-IndexFH-r15			INTEGER (0..1319)		OPTIONAL, -- Need OR

sr-SlotSPUCCH-IndexNoFH-r15		INTEGER (0..3959)		OPTIONAL, -- Need OR

sr-SubslotSPUCCH-ResourceList-r15	SR-SubslotSPUCCH-ResourceList-r15 OPTIONAL, -- Need OR

sr-ConfigIndexSlot-r15				INTEGER (0..36)			OPTIONAL,		-- Need OR

sr-ConfigIndexSubslot-r15			INTEGER (0..122)		OPTIONAL,		-- Need OR

dssr-TransMax-r15					ENUMERATED {

n4, n8, n16, n32, n64, spare3, spare2, spare1}

}

}

SR-SubslotSPUCCH-ResourceList-r15 ::=	SEQUENCE (SIZE(1..4)) OF INTEGER (0..1319)

-- ASN1STOP

SchedulingRequestConfig field descriptions

dsr-TransMax Parameter for SR transmission in TS 36.321 [6], clause 5.4.4. The value n4 corresponds to 4 transmissions, n8 corresponds to 8 transmissions and so on. EUTRAN configures the same value for all serving cells for which this field is configured.

dssr-TransMax Parameter for SPUCCH SR transmission in TS 36.321 [6], clause 5.4.4. EUTRAN configures the same value for all serving cells for which this field is configured.

sr-ConfigIndex, sr-ConfigIndexSlot, sr-ConfigIndexSubslot Parameter. See TS 36.213 [23], clause 10.1. The values 156 and 157 are not applicable for Release 8.

sr-PUCCH-ResourceIndex, sr-PUCCH-ResourceIndexP1 Parameter: for antenna port P0 and for antenna port P1 respectively, see TS 36.213 [23], clause 10.1. E-UTRAN configures sr-PUCCH-ResourceIndexP1 only if sr-PUCCHResourceIndex is configured.

sr-SlotSPUCCH-IndexFH Resource configuration for SR using slot-SPUCCH when frequency hopping is enabled, see TS 36.213 [23], clause 10.1.5.

sr-SlotSPUCCH-IndexNoFH Resource configuration for SR using slot-SPUCCH when frequency hopping is disabled, see TS 36.213 [23], clause 10.1.5.

sr-SubslotSPUCCH-ResourceList Resource configuration for SR using subslot-SPUCCH, see TS 36.213 [23], clause 10.1.5.

#### –	SlotOrSubslotPDSCH-Config

The IE SlotOrSubslotPDSCH-Config is used to specify the UE specific PDSCH configuration when sTTI is used.

SlotOrSubslotPDSCH-Config information element

-- ASN1START

SlotOrSubslotPDSCH-Config-r15 ::= CHOICE {

release							NULL,

setup							SEQUENCE {

altCQI-TableSTTI-r15			ENUMERATED {

allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}			OPTIONAL, -- Need OR

altCQI-Table1024QAM-STTI-r15	ENUMERATED {

allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}			OPTIONAL, -- Need OR

resourceAllocation-r15			ENUMERATED {

resourceAllocationType0,resourceAllocationType2}	OPTIONAL, -- Need OR

tbsIndexAlt-STTI-r15			ENUMERATED {a33}			OPTIONAL, -- Need OR

tbsIndexAlt2-STTI-r15			ENUMERATED {b33}			OPTIONAL, -- Need OR

tbsIndexAlt3-STTI-r15			ENUMERATED {a37}			OPTIONAL, -- Need OR

...

}

}

-- ASN1STOP

SlotOrSubslotPDSCH-Config field descriptions

altCQI-TableSTTI, altCQI-Table1024QAM-STTI Indicates the applicability of the alternative CQI table (i.e. Table 7.2.3-2 and Table 7.2.3-4 in TS 36.213 [23]) for aperiodic CSI reporting for slot or subslot PDSCH for the concerned serving cell. Value allSubframes means the alternative CQI table applies to all the subframes and CSI processes, if configured, and value csi-SubframeSet1 means the alternative CQI table applies to CSI subframe set1, and value csi-SubframeSet2 means the alternative CQI table applies to CSI subframe set2. EUTRAN sets the value to csi-SubframeSet1 or csi-SubframeSet2 only if transmissionMode is set in range tm1 to tm9 and csi-SubframePatternConfig-r10 is configured for the concerned serving cell and different CQI tables apply to the two CSI subframe sets; otherwise EUTRAN sets the value to allSubframes. EUTRAN does not configure the same value for altCQI-TableSTTI-r15 and altCQI-Table-1024QAM-STTI-r15 in SlotOrSubslotPDSCH-Config-r15. EUTRAN does not configure altCQI-Table-1024QAM-STTI-r15 if the value of altCQI-TableSTTI-r15 is set to allSubframes. EUTRAN does not configure altCQI-TableSTTI-r15 if the value of altCQI-Table-1024QAM-STTI-r15 is set to allSubframes. If both altCQI-TableSTTI-r15 and altCQI-Table-1024QAM-STTI-r15 are absent, the UE shall use Table 7.2.3-1 in TS 36.213 [23] for all subframes and CSI processes, if configured.

resourceAllocation Parameter indicates resource allocation type for slot-PDSCH or subslot-PDSCH.

tbsIndexAlt-STTI Indicates the applicability of the alternative TBS index for the ITBS 33 (see TS 36.213 [23], Table 7.1.7.2.1-1) to all slots/subslots scheduled by DCI format 7-1F and 7-1G. Value a33 refers to the alternative TBS index ITBS 33A. If neither this field nor tbsIndexAlt2-STTI configures an alternative TBS index for ITBS 33, the UE shall use ITBS 33 specified in Table 7.1.7.2.1-1 in TS 36.213 [23] for all slots/subslots instead.

tbsIndexAlt2-STTI Indicates the applicability of the alternative TBS index for the ITBS 33 (see TS 36.213 [23], Table 7.1.7.2.1-1) to all slots/subslots scheduled by DCI format 7-1B/7-1C/7-1D. Value b33 refers to the alternative TBS index ITBS 33B. If neither this field nor tbsIndexAlt-STTI configures an alternative TBS index for ITBS 33, the UE shall use ITBS 33 specified in Table 7.1.7.2.1-1 in TS 36.213 [23] for all slots/subslots instead.

tbsIndexAlt3-STTI Indicates the applicability of the alternative TBS index for the ITBS 37 (see TS 36.213 [23], Table 7.1.7.2.1-1) to all slots/subslots scheduled by DCI format 7-1F/7-1G. Value a37 refers to the alternative TBS index ITBS 37A. If this field does not configure an alternative TBS index for ITBS 37, the UE shall use ITBS 37 specified in TS 36.213 [23], Table 7.1.7.2.1-1 for all slots/subslots instead.

#### –	SlotOrSubslotPUSCH-Config

The IE SlotOrSubslotPUSCH-Config is used to specify the UE specific PUSCH configuration when sTTI is used.

SlotOrSubslotPUSCH-Config information element

-- ASN1START

SlotOrSubslotPUSCH-Config-r15 ::= CHOICE {

release							NULL,

setup							SEQUENCE {

betaOffsetSlot-ACK-Index-r15		INTEGER(0..15)		OPTIONAL, -- Need OR

betaOffset2Slot-ACK-Index-r15		INTEGER(0..15)		OPTIONAL, -- Need OR

betaOffsetSubslot-ACK-Index-r15		SEQUENCE (SIZE(1..2)) OF INTEGER(0..15)	OPTIONAL, -- Need OR

betaOffset2Subslot-ACK-Index-r15	SEQUENCE (SIZE(1..2)) OF INTEGER(0..15) OPTIONAL, -- Need OR

betaOffsetSlot-RI-Index-r15			INTEGER(0..15)		OPTIONAL, -- Need OR

betaOffsetSubslot-RI-Index-r15		SEQUENCE (SIZE(1..2)) OF INTEGER(0..15)	OPTIONAL, -- Need OR

betaOffsetSlot-CQI-Index-r15		INTEGER(0..15)		OPTIONAL, -- Need OR

betaOffsetSubslot-CQI-Index-r15		INTEGER(0..15)		OPTIONAL, -- Need OR

enable256QAM-SlotOrSubslot-r15		Enable256QAM-r14	OPTIONAL, -- Need ON

resourceAllocationOffset-r15		INTEGER (1..2)		OPTIONAL, -- Need OR

ul-DMRS-IFDMA-SlotOrSubslot-r15		BOOLEAN,

...

}

}

-- ASN1STOP

SlotOrSubslotPUSCH-Config field descriptions

betaOffsetSlot-ACK-Index, betaOffsetSubslot-ACK-Index, betaOffset2Slot-ACK-Index, betaOffset2Subslot-ACK-Index Parameter: and for single-codeword, see TS 36.213 [23], Table 8.6.3-1. If betaOffset2Slot-ACK-Index/betaOffset2Subslot-ACK-Index is configured; betaOffsetSlot-ACK-Index/betaOffsetSubslot-ACK-Index is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset2Slot-ACK-Index/betaOffset2Subslot-ACK-Index is used. The values apply for all serving cells with an uplink in a cell group (MCG or SCG or the group of cells configured to send SPUCCH on the same cell in case SPUCCH SCell is configured) and not configured with uplink power control subframe sets. It is indicated in DCI format 7-0A/7-0B which of the two values taken by betaOffsetSubslot-ACK-Index-r15/betaOffset2Subslot-ACK-Index-r15/ betaOffsetSubslot-RI-Index-r15 to use. The same value also applies for subframe set 1 of all serving cells with an uplink in that cell group and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell).

betaOffsetSlot-RI-Index, betaOffsetSubslot-RI-Index Parameter: , for single codeword, see TS 36.213 [23], Table 8.6.3-2. One value applies for subframe set 2 of all serving cells with an uplink in a cell group (MCG or SCG or the group of cells configured to send SPUCCH on the same cell in case SPUCCH SCell is configured) and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell configured with uplink power control subframe sets).

betaOffsetSlot-CQI-Index, betaOffsetSubslot-CQI-Index Parameter: , for single codeword, see TS 36.213 [23], Table 8.6.3-3. One value applies for all serving cells with an uplink in a cell group (MCG or SCG or the group of cells configured to send SPUCCH on the same cell in case SPUCCH SCell is configured) and not configured with uplink power control subframe sets. The same value also applies for subframe set 1 of all serving cells with an uplink in that cell group and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell).

enable256QAM-SlotOrSubslot Indicates that 256QAM for slot or subslot is enabled, see TS 36.213 [23], clause 8.6.1.

resourceAllocationOffset Indicates an RB resource allocation offset of 1 or 2 PRBs for slot-PUSCH or subslot-PUSCH. When the field is absent, the UE assumes no offset is used (i.e. offset = 0).

ul-DMRS-IFDMA-SlotOrSubslot Value TRUE indicates that the UE is configured with enhanced UL DMRS.

#### –	SoundingRS-UL-Config

The IE SoundingRS-UL-Config is used to specify the uplink Sounding RS configuration for periodic and aperiodic sounding.

SoundingRS-UL-Config information element

-- ASN1START

SoundingRS-UL-ConfigCommon ::=		CHOICE {

release								NULL,

setup								SEQUENCE {

srs-BandwidthConfig					ENUMERATED {bw0, bw1, bw2, bw3, bw4, bw5, bw6, bw7},

srs-SubframeConfig					ENUMERATED {

sc0, sc1, sc2, sc3, sc4, sc5, sc6, sc7,

sc8, sc9, sc10, sc11, sc12, sc13, sc14, sc15},

ackNackSRS-SimultaneousTransmission	BOOLEAN,

srs-MaxUpPts						ENUMERATED {true}			OPTIONAL	-- Cond TDD

}

}

SoundingRS-UL-ConfigDedicated ::=	CHOICE{

release								NULL,

setup								SEQUENCE {

srs-Bandwidth						ENUMERATED {bw0, bw1, bw2, bw3},

srs-HoppingBandwidth				ENUMERATED {hbw0, hbw1, hbw2, hbw3},

freqDomainPosition					INTEGER (0..23),

duration							BOOLEAN,

srs-ConfigIndex						INTEGER (0..1023),

transmissionComb					INTEGER (0..1),

cyclicShift							ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7}

}

}

SoundingRS-UL-ConfigDedicated-v1020 ::=	SEQUENCE {

srs-AntennaPort-r10					SRS-AntennaPort

}

SoundingRS-UL-ConfigDedicated-v1310 ::=	CHOICE{

release								NULL,

setup								SEQUENCE {

transmissionComb-v1310				INTEGER (2..3)				OPTIONAL,	-- Need OR

cyclicShift-v1310					ENUMERATED {cs8, cs9, cs10, cs11}	OPTIONAL,	-- Need OR

transmissionCombNum-r13				ENUMERATED {n2, n4}		OPTIONAL	-- Need OR

}

}

SoundingRS-UL-ConfigDedicatedUpPTsExt-r13 ::=	CHOICE{

release								NULL,

setup								SEQUENCE {

srs-UpPtsAdd-r13						ENUMERATED {sym2, sym4},

srs-Bandwidth-r13					ENUMERATED {bw0, bw1, bw2, bw3},

srs-HoppingBandwidth-r13			ENUMERATED {hbw0, hbw1, hbw2, hbw3},

freqDomainPosition-r13				INTEGER (0..23),

duration-r13						BOOLEAN,

srs-ConfigIndex-r13					INTEGER (0..1023),

transmissionComb-r13				INTEGER (0..3),

cyclicShift-r13						ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7,

cs8, cs9, cs10, cs11},

srs-AntennaPort-r13					SRS-AntennaPort,

transmissionCombNum-r13				ENUMERATED {n2, n4}

}

}

SoundingRS-UL-ConfigDedicatedAperiodic-r10 ::=	CHOICE{

release								NULL,

setup								SEQUENCE {

srs-ConfigIndexAp-r10				INTEGER (0..31),

srs-ConfigApDCI-Format4-r10			SEQUENCE (SIZE (1..3)) OF SRS-ConfigAp-r10	OPTIONAL,--Need ON

srs-ActivateAp-r10					CHOICE {

release							NULL,

setup							SEQUENCE {

srs-ConfigApDCI-Format0-r10			SRS-ConfigAp-r10,

srs-ConfigApDCI-Format1a2b2c-r10		SRS-ConfigAp-r10,

...

}

}																	OPTIONAL	-- Need ON

}

}

SoundingRS-UL-ConfigDedicatedAperiodic-v1310 ::=	CHOICE{

release								NULL,

setup								SEQUENCE {

srs-ConfigApDCI-Format4-v1310		SEQUENCE (SIZE (1..3)) OF SRS-ConfigAp-v1310	OPTIONAL,--Need ON

srs-ActivateAp-v1310				CHOICE {

release							NULL,

setup							SEQUENCE {

srs-ConfigApDCI-Format0-v1310		SRS-ConfigAp-v1310	OPTIONAL,	-- Need ON

srs-ConfigApDCI-Format1a2b2c-v1310	SRS-ConfigAp-v1310	OPTIONAL	-- Need ON

}

}																	OPTIONAL	-- Need ON

}

}

SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13 ::=	CHOICE{

release								NULL,

setup								SEQUENCE {

srs-UpPtsAdd-r13					ENUMERATED {sym2, sym4},

srs-ConfigIndexAp-r13				INTEGER (0..31),

srs-ConfigApDCI-Format4-r13			SEQUENCE (SIZE (1..3)) OF SRS-ConfigAp-r13	OPTIONAL,--Need ON

srs-ActivateAp-r13					CHOICE {

release							NULL,

setup							SEQUENCE {

srs-ConfigApDCI-Format0-r13			SRS-ConfigAp-r13,

srs-ConfigApDCI-Format1a2b2c-r13		SRS-ConfigAp-r13

}

}																	OPTIONAL	-- Need ON

}

}

SoundingRS-UL-ConfigDedicatedAperiodic-v1430 ::=	CHOICE{

release								NULL,

setup								SEQUENCE {

srs-SubframeIndication-r14			INTEGER (1..4)	OPTIONAL		-- Need ON

}

}

SoundingRS-UL-ConfigDedicatedAdd-r16 ::= SEQUENCE {

srs-ConfigIndexAp-r16				INTEGER (0..31),

srs-ConfigApDCI-Format4-r16			SEQUENCE (SIZE (1..3)) OF SRS-ConfigAdd-r16

OPTIONAL,	--Need ON

srs-ActivateAp-r13					CHOICE {

release							NULL,

setup							SEQUENCE {

srs-ConfigApDCI-Format0-r16			SRS-ConfigAdd-r16,

srs-ConfigApDCI-Format1a2b2c-r16	SRS-ConfigAdd-r16

}

}																OPTIONAL	--Need ON

}

SRS-ConfigAp-r10 ::= SEQUENCE {

srs-AntennaPortAp-r10				SRS-AntennaPort,

srs-BandwidthAp-r10					ENUMERATED {bw0, bw1, bw2, bw3},

freqDomainPositionAp-r10			INTEGER (0..23),

transmissionCombAp-r10				INTEGER (0..1),

cyclicShiftAp-r10					ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7}

}

SRS-ConfigAp-v1310 ::= SEQUENCE {

transmissionCombAp-v1310			INTEGER (2..3)						OPTIONAL,	-- Need OR

cyclicShiftAp-v1310					ENUMERATED {cs8, cs9, cs10, cs11}	OPTIONAL,	-- Need OR

transmissionCombNum-r13				ENUMERATED {n2, n4}			OPTIONAL	-- Need OR

}

SRS-ConfigAp-r13 ::= SEQUENCE {

srs-AntennaPortAp-r13				SRS-AntennaPort,

srs-BandwidthAp-r13					ENUMERATED {bw0, bw1, bw2, bw3},

freqDomainPositionAp-r13			INTEGER (0..23),

transmissionCombAp-r13				INTEGER (0..3),

cyclicShiftAp-r13					ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7,

cs8, cs9, cs10, cs11},

transmissionCombNum-r13				ENUMERATED {n2, n4}

}

SRS-AntennaPort ::=					ENUMERATED {an1, an2, an4, spare1}

SRS-ConfigAdd-r16 ::=		SEQUENCE {

srs-RepNumAdd-r16				ENUMERATED {n1, n2, n3, n4, n6, n7, n8, n9, n12, n13},

srs-BandwidthAdd-r16			ENUMERATED {bw0, bw1, bw2, bw3},

srs-HoppingBandwidthAdd-r16		ENUMERATED {hbw0, hbw1, hbw2, hbw3},

srs-FreqDomainPosAdd-r16		INTEGER (0..23),

srs-AntennaPortAdd-r16			SRS-AntennaPort,

srs-CyclicShiftAdd-r16			ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7,

cs8, cs9, cs10, cs11},

srs-TransmissionCombNumAdd-r16	ENUMERATED {n2, n4},

srs-TransmissionCombAdd-r16		INTEGER (0..3),

srs-StartPosAdd-r16				INTEGER (1..13),

srs-DurationAdd-r16				INTEGER (1..13),

srs-GuardSymbolAS-Add-r16		ENUMERATED {enabled}			OPTIONAL,	-- Need ON

srs-GuardSymbolFH-Add-r16		ENUMERATED {enabled}			OPTIONAL	-- Need ON

}

-- ASN1STOP

SoundingRS-UL-Config field descriptions

ackNackSRS-SimultaneousTransmission Parameter: Simultaneous-AN-and-SRS, see TS 36.213 [23], clause 8.2. For SCells without PUCCH configured, this field is not applicable and the UE shall ignore the value.

cyclicShift, cyclicShiftAp, srs-CyclicShiftAdd Parameter: n_SRS for periodic, aperiodic and additional sounding reference signal transmission respectively except for an LAA SCell. See TS 36.211 [21], clause 5.5.3.1, where cs0 corresponds to 0 etc.

duration Parameter: Duration for periodic sounding reference signal transmission except for an LAA SCell. See TS 36.213 [21], clause 8.2. FALSE corresponds to "single" and value TRUE to "indefinite".

freqDomainPosition, freqDomainPositionAp, srs-FreqDomainPosAdd Parameter: for periodic, aperiodic and additional sounding reference signal transmission respectively, see TS 36.211 [21], clause 5.5.3.2.

srs-AntennaPort, srs-AntennaPortAp, srs-AntennaPortAdd Indicates the number of antenna ports used for periodic, aperiodic and additional sounding reference signal transmission respectively, see TS 36.211 [21], clause 5.5.3. UE shall release srs-AntennaPort if SoundingRS-UL-ConfigDedicated is released.

srs-Bandwidth, srs-BandwidthAp, srs-BandwidthAdd Parameter: for periodic, aperiodic and additional sounding reference signal transmission respectively, see TS 36.211 [21], tables 5.5.3.2-1, 5.5.3.2-2, 5.5.3.2-3 and 5.5.3.2-4. For LAA SCell only bw0 is applied.

srs-BandwidthConfig Parameter: SRS Bandwidth Configuration. See TS 36.211, [21], tables 5.5.3.2-1, 5.5.3.2-2, 5.5.3.2-3 and 5.5.3.2-4. Actual configuration depends on UL bandwidth. bw0 corresponds to value 0, bw1 to value 1 and so on.

srs-ConfigApDCI-Format0 / srs-ConfigApDCI-Format1a2b2c / srs-ConfigApDCI-Format4 Parameters indicate the resource configurations for aperiodic sounding reference signal transmissions triggered by DCI formats 0, 1A, 2B, 2C, 4. See TS 36.213 [23], clause 8.2.

srs-ConfigIndex, srs-ConfigIndexAp Parameter: ISRS for periodic and aperiodic sounding reference signal transmission respectively except for an LAA SCell. See TS 36.213 [23], tables 8.2-1 and 8.2-2, for periodic and TS 36.213 [23], tables 8.2-4 an8.2-5, for aperiodic and additional SRS transmission. If both srs-ConfigIndexAp-r10 and srs-ConfigIndexAp-r16 are included, E-UTRAN configures the same value for both fields.

srs-DurationAdd Indicates the duration of the additional SRS including guard symbols within a UL subframe, see TS 36.211 [21], clause 5.5.3. E-UTRAN configures addSRS-StartPos and this field such that all the configured additional SRS occur within the same subframe.

srs-GuardSymbolAS-Add If enabled, there is a guard period of one symbol after antenna switching, see TS 36.211 [21], clause 5.5.3 and TS 36.213 [23] clause 8.2.

srs-GuardSymbolFH-Add If enabled, there is a guard period of one symbol after frequency hopping, see TS 36.211 [21], clause 5.5.3 and TS 36.213 [23] clause 8.2.

srs-HoppingBandwidth, srs-HoppingBandwidthAdd Parameter: SRS hopping bandwidth for periodic and additional sounding reference signal transmission respectively except for an LAA SCell, see TS 36.211 [21], clause 5.5.3.2, where hbw0 corresponds to value 0, hbw1 to value 1 and so on.

srs-MaxUpPts Parameter: srsMaxUpPts, see TS 36.211 [21], clause 5.5.3.2. If this field is present, reconfiguration of applies for UpPts, otherwise reconfiguration does not apply.

srs-RepNumAdd Parameter: R which indicates the number of the additional SRS repetitions, see TS 36.211 [21], clause 5.5.3.2 and TS 36.213 [23] clause 8.3.

srs-StartPosAdd Indicates the starting position of the additional SRS within a UL subframe excluding UpPTS, see TS 36.211 [21], clause 5.5.3.

srs-SubframeConfig Parameter: SRS SubframeConfiguration except for an LAA SCell. See TS 36.211, [21], table 5.5.3.3-1, applies for FDD whereas TS 36.211 [21], table 5.5.3.3-2, applies for TDD. sc0 corresponds to value 0, sc1 corresponds to value 1 and so on.

srs-SubframeIndication Parameter: SRS subframe indication in SRS parameter set configuration for aperiodic sounding reference signal transmission on an LAA SCell configured with uplink, see TS 36.213 [23].

srs-UpPtsAdd The field only applies for TDD and frame structure type 3, see TS 36.211 [21]. If E-UTRAN configures both soundingRS-UL-ConfigDedicatedUpPTsExt and soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt, srs-UpPtsAdd in both fields is set to the same value. If E-UTRAN configures soundingRS-UL-PeriodicConfigDedicatedUpPTsExtList-r14 with a number of soundingRS-UL-ConfigDedicatedUpPTsExt and/or soundingRS-UL-AperiodicConfigDedicatedList-r14 with a number of soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt, srs-UpPtsAdd in all fields are set to the same value.

transmissionComb, transmissionCombAp, srs-TransmissionCombAdd Parameter: for periodic, aperiodic and additional sounding reference signal transmission respectively, see TS 36.211 [21], clause 5.5.3.2.

| Conditional presence | Explanation |
| --- | --- |
| TDD | This field is optional present for TDD, need OR; it is not present for FDD and the UE shall delete any existing value for this field. |

#### –	SPDCCH-Config

The IE SPDCCH-Config is used to specify the UE specific SPDCCH configuration.

SPDCCH-Config information element

-- ASN1START

SPDCCH-Config-r15 ::=		CHOICE {

release						NULL,

setup						SEQUENCE {

spdcch-L1-ReuseIndication-r15		ENUMERATED {n0,n1,n2}	OPTIONAL, -- Need OR

spdcch-SetConfig-r15				SPDCCH-Set-r15			OPTIONAL -- Need OR

}

}

SPDCCH-Set-r15 ::= SEQUENCE (SIZE (1..4)) OF SPDCCH-Elements-r15

SPDCCH-Elements-r15 ::= CHOICE {

release							NULL,

setup							SEQUENCE {

spdcch-SetConfigId-r15				INTEGER (0..3)			OPTIONAL, -- Need OR

spdcch-SetReferenceSig-r15			ENUMERATED {crs, dmrs}	OPTIONAL, -- Need OR

transmissionType-r15				ENUMERATED {localised, distributed} OPTIONAL, -- Need OR

spdcch-NoOfSymbols-r15				INTEGER (1..2)			OPTIONAL, -- Need OR

dmrs-ScramblingSequenceInt-r15		INTEGER (0..503)		OPTIONAL, -- Need OR

dci7-CandidatesPerAL-PDCCH-r15		SEQUENCE (SIZE(1..4)) OF

DCI7-Candidates-r15 OPTIONAL, -- Need OR

dci7-CandidateSetsPerAL-SPDCCH-r15	SEQUENCE (SIZE(1..2)) OF

DCI7-CandidatesPerAL-SPDCCH-r15 OPTIONAL, -- Need OR

resourceBlockAssignment-r15			SEQUENCE{

numberRB-InFreq-domain-r15			INTEGER (2..100),

resourceBlockAssignment-r15			BIT STRING (SIZE(98))

}															OPTIONAL, -- Need OR

subslotApplicability-r15			BIT STRING (SIZE(5))	OPTIONAL, -- Need OR

al-StartingPointSPDCCH-r15			SEQUENCE (SIZE(1..4)) OF

INTEGER(0..49)		OPTIONAL, -- Need OR

subframeType-r15					ENUMERATED {mbsfn, nonmbsfn, all}	OPTIONAL, -- Need OR

rateMatchingMode-r15				ENUMERATED {m1, m2, m3, m4}		OPTIONAL, -- Need OR

...

}

}

DCI7-Candidates-r15 ::=						INTEGER (0..6)

DCI7-CandidatesPerAL-SPDCCH-r15 ::=				SEQUENCE (SIZE(1..4)) OF DCI7-Candidates-r15

-- ASN1STOP

SPDCCH-Config field descriptions

al-StartingPointSPDCCH Indicates the starting SCCE index for an aggregation level, see TS 36.213 [23], clause 9.1.6.

dci7-Candidates Number of candidates in each aggregation level for DCI format 7. The number of PDCCH/SPDCCH candidate(s) M_DCI format 7^((L)) at aggregation level L for monitoring DCI format 7 in PDCCH and SPDCCH region shall conform to the following restriction: • less than or equal to 2 for aggregation level 4 and 8, • less than or equal to 6 for aggregation level 1 and 2

dci7-CandidatesPerAL-SPDCCH SPDCCH candidates configured per aggregation level in SPDCCH region

dmrs-ScramblingSeqSPDCCH The DMRS scrambling sequence initialization parameter defined in TS 36.211 [21], clause 6.10.3A.1.

numberRB-InFreq-domain Indicates the number of resource-blocks in the frequency domain used for the SPDCCH set. There is no restriction on the number of RBs in the frequency domain that can be configured to an SPDCCH resouce set (up to 100), but at least two need to be configured to contain at least one SCCE. The granularity of resource block allocation in frequency domain for configuring an SPDCCH resource set is one in case spdcch-SetReferenceSig-r15 is set to crs. The granularity of resource block allocation for configuring an SPDCCH resource set is two in case sPDCCH-SetReferenceSig-r15 is set to dmrs.

rateMatchingMode Indicates, per resource-set, the mode of SPDCCH rate-matching operation • Mode 1: UE rate-matches only around the DCI format 7 scheduling the slot or subslot PDSCH (if transmitted in theSPDCCH resouce-set), otherwise no rate-matching is performed for the RB set. • Mode 2: UE rate-matches around the whole SPDCCH resource set • Mode 3: UE rate-matches around the whole SPDCCH resource set if DCI format 7 scheduling the slot or subslot PDSCH is found in the resource-set, otherwise no rate-matching is performed for the RB set. • Mode 4: UE rate-matches around the whole SPDCCH resource set if DCI format 7 scheduling the slot or subslot PDSCH is not found in the resource-set, otherwise UE rate-matches only around the DCI format 7 scheduling the slot or subslot PDSCH (if transmitted in the SPDCCH resource-set) If the DCI format 7 scheduling the slot or subslot PDSCH is found on a candidate belonging to two SPDCCH resource sets, the DCI format 7 is assumed to be found in both resource sets.

resourceBlockAssignment Indicates the index to a specific combination of physical resource block in frequency for SPDCCH set, see TS 36.213 [23], clause 9.1.4.4. The value range is dependent on the combinatorial number defined in 36.213 [23], clause 9.1.4.4 with the assumption of no limitation in the number of RBs in frequency domain configured by the set.

spdcch-NoOfSymbols Indicates the number of OFDM symbols that the CRS based SPDCCH is mapped over.

spdcch-L1-ReuseIndication For the up to two resource sets configured with the same subframeType applicability, the SPDCCH-L1-ReuseIndication defines the allowed combinations for the two resource sets: {1,1}, {2,0} or {0,2} corresponding to the values n0, n1 and n2 repsectively. In case one resource set is configured, the allowed combinations are {2, 0} or {0,2} corresponding to n1 or n2. EUTRAN does not configure n0 in case one resource set is configured.

spdcch-SetConfigId Indicates the ID of the SPDCCH set configured in SPDCCH-Elements. Maximum two sets can be configured for MBSFN and two for non-MBSFN.

spdcch-SetReferenceSig Indicates CRS or DMRS based SPDCCH set.

subframeType Indicates applicable subframe type(s) for the SPDCCH set. CRS-based SPDCCH is only applied to non-MBSFN subframe.

subslotApplicability Indicates the set of subslots within the subframe where SPDCCH candidate set per aggregation levels applies, see DCI7-CandidateSetsPerAL-SPDCCH. The bitmap applies to the 5 DL subslot indices in a DL subframe. The first element in the sequence DCI7-CandidateSetsPerAL-SPDCCH applies to the indicated subslotApplicability. The second element in the sequence (if present) applies to the complement of the subslotApplicability.

transmissionType Indicates whether distributed or localized SPDCCH transmission mode is used as defined in TS 36.211 [21], clause 6.8A.1.

#### –	SPS-Config

The IE SPS-Config is used to specify the semi-persistent scheduling configuration.

SPS-Config information element

-- ASN1START

SPS-Config ::=	SEQUENCE {

semiPersistSchedC-RNTI			C-RNTI					OPTIONAL,			-- Need OR

sps-ConfigDL					SPS-ConfigDL			OPTIONAL,			-- Need ON

sps-ConfigUL					SPS-ConfigUL			OPTIONAL			-- Need ON

}

SPS-Config-v1430 ::=	SEQUENCE {

ul-SPS-V-RNTI-r14					C-RNTI					OPTIONAL,			-- Need OR

sl-SPS-V-RNTI-r14					C-RNTI					OPTIONAL,			-- Need OR

sps-ConfigUL-ToAddModList-r14		SPS-ConfigUL-ToAddModList-r14	OPTIONAL,	-- Need ON

sps-ConfigUL-ToReleaseList-r14		SPS-ConfigUL-ToReleaseList-r14	OPTIONAL,	-- Need ON

sps-ConfigSL-ToAddModList-r14		SPS-ConfigSL-ToAddModList-r14	OPTIONAL,	-- Need ON

sps-ConfigSL-ToReleaseList-r14		SPS-ConfigSL-ToReleaseList-r14	OPTIONAL	-- Need ON

}

SPS-ConfigUL-ToAddModList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigUL

SPS-ConfigUL-ToReleaseList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigIndex-r14

SPS-ConfigSL-ToAddModList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigSL-r14

SPS-ConfigSL-ToReleaseList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigIndex-r14

SPS-Config-v1530 ::=	SEQUENCE {

semiPersistSchedC-RNTI-r15		C-RNTI						OPTIONAL,			-- Need OR

sps-ConfigDL-r15					SPS-ConfigDL				OPTIONAL,			-- Need ON

sps-ConfigUL-STTI-ToAddModList-r15	SPS-ConfigUL-STTI-ToAddModList-r15	OPTIONAL,	-- Need ON

sps-ConfigUL-STTI-ToReleaseList-r15	SPS-ConfigUL-STTI-ToReleaseList-r15	OPTIONAL,	-- Need ON

sps-ConfigUL-ToAddModList-r15		SPS-ConfigUL-ToAddModList-r15		OPTIONAL,	-- Need ON

sps-ConfigUL-ToReleaseList-r15		SPS-ConfigUL-ToReleaseList-r15		OPTIONAL	-- Need ON

}

SPS-Config-v1540 ::=	SEQUENCE {

sps-ConfigDL-STTI-r15				SPS-ConfigDL-STTI-r15				OPTIONAL	-- Need OR

}

SPS-ConfigUL-STTI-ToAddModList-r15 ::= SEQUENCE (SIZE (1..maxConfigSPS-r15)) OF SPS-ConfigUL-STTI-r15

SPS-ConfigUL-STTI-ToReleaseList-r15 ::= SEQUENCE (SIZE (1..maxConfigSPS-r15)) OF SPS-ConfigIndex-r15

SPS-ConfigUL-ToAddModList-r15 ::= SEQUENCE (SIZE (1..maxConfigSPS-r15)) OF SPS-ConfigUL

SPS-ConfigUL-ToReleaseList-r15 ::= SEQUENCE (SIZE (1..maxConfigSPS-r15)) OF SPS-ConfigIndex-r15

SPS-ConfigDL ::=	CHOICE{

release							NULL,

setup							SEQUENCE {

semiPersistSchedIntervalDL			ENUMERATED {

sf10, sf20, sf32, sf40, sf64, sf80,

sf128, sf160, sf320, sf640, spare6,

spare5, spare4, spare3, spare2,

spare1},

numberOfConfSPS-Processes			INTEGER (1..8),

n1PUCCH-AN-PersistentList			N1PUCCH-AN-PersistentList,

...,

[[	twoAntennaPortActivated-r10		CHOICE {

release							NULL,

setup							SEQUENCE {

n1PUCCH-AN-PersistentListP1-r10	N1PUCCH-AN-PersistentList

}

}																OPTIONAL	-- Need ON

]]

}

}

SPS-ConfigUL ::=	CHOICE {

release							NULL,

setup							SEQUENCE {

semiPersistSchedIntervalUL			ENUMERATED {

sf10, sf20, sf32, sf40, sf64, sf80,

sf128, sf160, sf320, sf640, sf1-v1430,

sf2-v1430, sf3-v1430, sf4-v1430, sf5-v1430,

spare1},

implicitReleaseAfter				ENUMERATED {e2, e3, e4, e8},

p0-Persistent						SEQUENCE {

p0-NominalPUSCH-Persistent			INTEGER (-126..24),

p0-UE-PUSCH-Persistent				INTEGER (-8..7)

}		OPTIONAL,												-- Need OP

twoIntervalsConfig					ENUMERATED {true}			OPTIONAL,	-- Cond TDD

...,

[[	p0-PersistentSubframeSet2-r12		CHOICE {

release								NULL,

setup								SEQUENCE {

p0-NominalPUSCH-PersistentSubframeSet2-r12			INTEGER (-126..24),

p0-UE-PUSCH-PersistentSubframeSet2-r12				INTEGER (-8..7)

}

}															OPTIONAL	-- Need ON

]],

[[	numberOfConfUlSPS-Processes-r13			INTEGER (1..8)		OPTIONAL	-- Need OR

]],

[[	fixedRV-NonAdaptive-r14					ENUMERATED {true}		OPTIONAL,	-- Need OR

sps-ConfigIndex-r14						SPS-ConfigIndex-r14		OPTIONAL,	-- Need OR

semiPersistSchedIntervalUL-v1430		ENUMERATED {

sf50, sf100, sf200, sf300, sf400, sf500,

sf600, sf700, sf800, sf900, sf1000, spare5,

spare4, spare3, spare2, spare1}	OPTIONAL	-- Need OR

]],

[[ cyclicShiftSPS-r15				ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7}

OPTIONAL,	-- Need ON

harq-ProcID-Offset-r15				INTEGER (0..7)			OPTIONAL,	-- Need ON

rv-SPS-UL-Repetitions-r15			ENUMERATED {ulrvseq1, ulrvseq2, ulrvseq3} OPTIONAL, -- Need ON

tpc-PDCCH-ConfigPUSCH-SPS-r15		TPC-PDCCH-Config		OPTIONAL,	-- Need ON

totalNumberPUSCH-SPS-UL-Repetitions-r15	ENUMERATED {n2,n3,n4,n6}	OPTIONAL, -- Need ON

sps-ConfigIndex-r15					SPS-ConfigIndex-r15		OPTIONAL	-- Cond SPS

]]

}

}

SPS-ConfigSL-r14 ::=	SEQUENCE {

sps-ConfigIndex-r14				SPS-ConfigIndex-r14,

semiPersistSchedIntervalSL-r14	ENUMERATED {

sf20, sf50, sf100, sf200, sf300, sf400,

sf500, sf600, sf700, sf800, sf900, sf1000,

spare4, spare3, spare2, spare1}

}

SPS-ConfigIndex-r14 ::=			INTEGER (1..maxConfigSPS-r14)

SPS-ConfigIndex-r15 ::=			INTEGER (1..maxConfigSPS-r15)

N1PUCCH-AN-PersistentList ::=		SEQUENCE (SIZE (1..4)) OF INTEGER (0..2047)

N1SPUCCH-AN-PersistentList-r15 ::=		SEQUENCE (SIZE (1..4)) OF INTEGER (0..2047)

SPS-ConfigDL-STTI-r15 ::=	CHOICE{

release							NULL,

setup							SEQUENCE {

semiPersistSchedIntervalDL-STTI-r15		ENUMERATED {

sTTI1, sTTI2, sTTI3, sTTI4, sTTI6, sTTI8, sTTI12, sTTI16,

sTTI20, sTTI40, sTTI60, sTTI80, sTTI120, sTTI240,

spare2, spare1},

numberOfConfSPS-Processes-STTI-r15		INTEGER (1..12),

twoAntennaPortActivated-r15			CHOICE {

release							NULL,

setup							SEQUENCE {

n1SPUCCH-AN-PersistentListP1-r15		N1SPUCCH-AN-PersistentList-r15

}

}															OPTIONAL,	-- Need ON

sTTI-StartTimeDL-r15					INTEGER (0..5),

tpc-PDCCH-ConfigPUCCH-SPS-r15		TPC-PDCCH-Config			OPTIONAL,	-- Need ON

...

}

}

SPS-ConfigUL-STTI-r15 ::=	CHOICE {

release							NULL,

setup							SEQUENCE {

semiPersistSchedIntervalUL-STTI-r15		ENUMERATED {

sTTI1, sTTI2, sTTI3, sTTI4, sTTI6, sTTI8, sTTI12, sTTI16,

sTTI20, sTTI40, sTTI60, sTTI80, sTTI120, sTTI240,

spare2, spare1},

implicitReleaseAfter					ENUMERATED {e2, e3, e4, e8},

p0-Persistent-r15					SEQUENCE {

p0-NominalSPUSCH-Persistent-r15		INTEGER (-126..24),

p0-UE-SPUSCH-Persistent-r15			INTEGER (-8..7)

}																OPTIONAL,	-- Need OP

twoIntervalsConfig-r15					ENUMERATED {true}		OPTIONAL,	-- Cond TDD

p0-PersistentSubframeSet2-r15		CHOICE {

release							NULL,

setup							SEQUENCE {

p0-NominalSPUSCH-PersistentSubframeSet2-r15			INTEGER (-126..24),

p0-UE-SPUSCH-PersistentSubframeSet2-r15				INTEGER (-8..7)

}

}															OPTIONAL,	-- Need ON

numberOfConfUL-SPS-Processes-STTI-r15	INTEGER (1..12)			OPTIONAL,	-- Need OR

sTTI-StartTimeUL-r15				INTEGER (0..5),

tpc-PDCCH-ConfigPUSCH-SPS-r15		TPC-PDCCH-Config			OPTIONAL,	-- Need ON

cyclicShiftSPS-sTTI-r15				ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7}																				OPTIONAL,	-- Need ON

ifdma-Config-SPS-r15				BOOLEAN						OPTIONAL,	-- Need ON

harq-ProcID-offset-r15				INTEGER (0..15)			OPTIONAL,	-- Need ON

rv-SPS-STTI-UL-Repetitions-r15		ENUMERATED {ulrvseq1, ulrvseq2, ulrvseq3}	OPTIONAL, -- Need ON

sps-ConfigIndex-r15		SPS-ConfigIndex-r15		OPTIONAL,	-- Need OR

tbs-scalingFactorSubslotSPS-UL-Repetitions-r15	ENUMERATED {n6, n12}		OPTIONAL, -- Need ON

totalNumberPUSCH-SPS-STTI-UL-Repetitions-r15	ENUMERATED {n2,n3,n4,n6}	OPTIONAL, -- Need ON

...

}

}

-- ASN1STOP

SPS-Config field descriptions

cyclicShiftSPS, cyclicShiftSPS-sTTI, Indicates the cyclic shift to be used for the UE-specific reference signal in case of UL SPS, see TS 36.211 [5] clause 5.2.1.1.

fixedRV-NonAdaptive If this field is present and skipUplinkTxSPS is configured, non-adaptive retransmissions on configured uplink grant uses redundancy version 0, otherwise the redundancy version for each retransmission is updated based on the sequence of redundancy versions as described in TS 36.321 [6].

harq-ProcID-offset If configured, this field indicates the offset used in deriving the HARQ process IDs, see TS 36.321 [6], clause 5.4.1.

Ifdma-Config-SPS Indicated to be used for the UE-specific reference signal in case of UL SPS see TS 36.211 [5], clause 5.2,1.1.

implicitReleaseAfter Number of empty transmissions before implicit release, see TS 36.321 [6], clause 5.10.2. Value e2 corresponds to 2 transmissions, e3 corresponds to 3 transmissions and so on. If skipUplinkTxSPS is configured, the UE shall ignore this field.

n1PUCCH-AN-PersistentList, n1PUCCH-AN-PersistentListP1 List of parameter: for antenna port P0 and for antenna port P1 respectively, see TS 36.213 [23], clause 10.1. Field n1-PUCCH-AN-PersistentListP1 is applicable only if the twoAntennaPortActivatedPUCCH-Format1a1b in PUCCH-ConfigDedicated-v1020 is set to true. Otherwise the field is not configured.

numberOfConfSPS-Processes The number of configured HARQ processes for downlink Semi-Persistent Scheduling, see TS 36.321 [6].

numberOfConfSPS-Processes-STTI The number of configured HARQ processes for downlink Semi-Persistent Scheduling for sTTI in DL, see TS 36.321 [6].

numberOfConfUlSPS-Processes The number of configured HARQ processes for uplink Semi-Persistent Scheduling, see TS 36.321 [6]. E-UTRAN always configures this field for asynchronous UL HARQ. Otherwise it does not configure this field.

numberOfConfUL-SPS-Processes-STTI The number of configured HARQ processes for uplink Semi-Persistent Scheduling for sTTI in UL, see TS 36.321 [6]. E-UTRAN always configures this field for asynchronous UL HARQ. Otherwise it does not configure this field.

p0-NominalPUSCH-Persistent, p0-NominalSPUSCH-Persistent Parameter: . See TS 36.213 [23], clause 5.1.1.1, unit dBm step 1. This field is applicable for persistent scheduling, only. If choice setup is used and p0-Persistent is absent, apply the value of p0-NominalPUSCH for p0-NominalPUSCH-Persistent. If uplink power control subframe sets are configured by tpc-SubframeSet, this field applies for uplink power control subframe set 1.

p0-NominalPUSCH-PersistentSubframeSet2, p0-NominalSPUSCH-PersistentSubframeSet2 Parameter: . See TS 36.213 [23], clause 5.1.1.1, unit dBm step 1. This field is applicable for persistent scheduling, only. If p0-PersistentSubframeSet2-r12 is not configured, apply the value of p0-NominalPUSCH-SubframeSet2-r12 for p0-NominalPUSCH-PersistentSubframeSet2. E-UTRAN configures this field only if uplink power control subframe sets are configured by tpc-SubframeSet, in which case this field applies for uplink power control subframe set 2.

p0-UE-PUSCH-Persistent Parameter: . See TS 36.213 [23], clause 5.1.1.1, unit dB. This field is applicable for persistent scheduling, only. If choice setup is used and p0-Persistent is absent, apply the value of p0-UE-PUSCH for p0-UE-PUSCH-Persistent. If uplink power control subframe sets are configured by tpc-SubframeSet, this field applies for uplink power control subframe set 1.

p0-UE-PUSCH-PersistentSubframeSet2 Parameter: . See TS 36.213 [23], clause 5.1.1.1, unit dB. This field is applicable for persistent scheduling, only. If p0-PersistentSubframeSet2-r12 is not configured, apply the value of p0-UE-PUSCH-SubframeSet2 for p0-UE-PUSCH-PersistentSubframeSet2. E-UTRAN configures this field only if uplink power control subframe sets are configured by tpc-SubframeSet, in which case this field applies for uplink power control subframe set 2.

rv-SPS-STTI-UL-Repetitions Indicates the RV sequence of slot or subslot PUSCH for slot or subslot UL SPS repetitions. Value ulrvseq1= {0, 0, 0, 0, 0, 0} , value ulrvseq2={0, 2, 3, 1, 0, 2} and value ulrvseq3={0, 3, 0, 3, 0, 3}.

rv-SPS-UL-Repetitions Indicates the RV sequence of PUSCH for subframe UL SPS repetitions. Value ulrvseq1= {0, 0, 0, 0, 0, 0} , value ulrvseq2={0, 2, 3, 1, 0, 2} and value ulrvseq3={0, 3, 0, 3, 0, 3}.

semiPersistSchedC-RNTI Semi-persistent Scheduling C-RNTI, see TS 36.321 [6]. If sps-Config is present for more than one cells in the same cell group, semiPersistSchedC-RNTI is present in only one sps-Config.

semiPersistSchedIntervalDL Semi-persistent scheduling interval in downlink, see TS 36.321 [6]. Value in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. For TDD, the UE shall round this parameter down to the nearest integer (of 10 sub-frames), e.g. sf10 corresponds to 10 sub-frames, sf32 corresponds to 30 sub-frames, sf128 corresponds to 120 sub-frames.

semiPersistSchedIntervalDL-STTI Semi-persistent scheduling interval for sTTI in downlink, see TS 36.321 [6]. Value in number of sTTI. Value sTTI1 corresponds to a spacing of 1 sTTI interval, sTTI2 corresponds to 2 spacings of sTTI intervals and so on, e.g. sTTI1 equal to sub-slot of 2 symbols or 3 symbols when the type of 2OS sTTI is configured, or e.g. sTTI1 equal to slot of 7 symbols when type of 7OS sTTI is configured. SPS for sTTI is not supported for TDD.

semiPersistSchedIntervalSL Semi-persistent scheduling interval in sidelink, see TS 36.321 [6]. Value in number of sub-frames. Value sf20 corresponds to 20 sub-frames, sf50 corresponds to 50 sub-frames and so on.

semiPersistSchedIntervalUL Semi-persistent scheduling interval in uplink, see TS 36.321 [6]. Value in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. For TDD, when the configured Semi-persistent scheduling interval is greater than or equal to 10 sub-frames, the UE shall round this parameter down to the nearest integer (of 10 sub-frames), e.g. sf10 corresponds to 10 sub-frames, sf32 corresponds to 30 sub-frames, sf128 corresponds to 120 sub-frames. If semiPersistSchedIntervalUL-v1430 is configured, the UE only considers this extension (and ignores semiPersistSchedIntervalUL i.e. without suffix).

semiPersistSchedIntervalUL-STTI Semi-persistent scheduling interval for sTTI in uplink, see TS 36.321 [6]. Value in number of sTTI. Value sTTI1 corresponds to a spacing of 1 sTTI interval, sTTI2 corresponds to 2 spacings of sTTI intervals and so on, e.g. sTTI1 equal to sub-slot of 2 symbols or 3 symbols when the type of 2OS sTTI is configured, or e.g. sTTI1 equal to slot of 7 symbols when type of 7OS sTTI is configured. SPS for sTTI is not supported for TDD.

sl-SPS-V-RNTI SL Semi-Persistent Scheduling V-RNTI for V2X sidelink communication, see TS 36.321 [6].

sps-ConfigIndex Indicates the index of one of multiple SL/UL SPS configurations.

sps-ConfigDL-STTI If sps-ConfigDL-sTTI-r15 is signalled, the UE ignores sps-ConfigDL.

sps-ConfigSL-ToAddModList Indicates the SL SPS configurations to be added or modified, identified by SPS-ConfigIndex.

sps-ConfigSL-ToReleaseList Indicates the SL SPS configurations to be released, identified by SPS-ConfigIndex.

sps-ConfigUL-STTI-ToAddModList Indicates the UL sTTI SPS configurations to be added or modified, identified by SPS-ConfigIndex. If this list includes more than one entry, E-UTRAN includes totalNumberPUSCH-SPS-STTI-UL-Repetitions in each entry.

sps-ConfigUL-STTI-ToReleaseList Indicates the UL sTTI SPS configurations to be released, identified by SPS-ConfigIndex.

sps-ConfigUL-ToAddModList Indicates the UL SPS configurations to be added or modified, identified by SPS-ConfigIndex. If this list includes more than one entry, E-UTRAN includes totalNumberPUSCH-SPS-UL-Repetitions in each entry.

sps-ConfigUL-ToReleaseList Indicates the UL SPS configurations to be released, identified by SPS-ConfigIndex.

sTTI-StartTimeDL Indicates the DL sTTI index start offset for SPS (re-)initialization, see TS 36.321 [6].

sTTI-StartTimeUL Indicates the UL sTTI index start offset for SPS (re-)initialization, see TS 36.321 [6].

tbs-scalingFactorSubslotSPS-UL-Repetitions Indicates the TBS scaling factor of subslot PUSCH for UL SPS repetitions. Value n6 corresponds to 1/6 and value n12 corresponds to 1/12.

totalNumberPUSCH-SPS-STTI-UL-Repetitions Indicates the total number of UL transmissions for slot or subslot UL SPS repetitions. If the UE is configured with UL SPS and the configured number of SPS PUSCH transmissions k>1, simultaneous transmission of PUSCH and PUCCH is not configured.

totalNumberPUSCH-SPS-UL-Repetitions Indicates the total number of UL transmissions for subframe UL SPS repetitions. If the UE is configured with UL SPS and the configured number of SPS PUSCH transmissions k>1, simultaneous transmission of PUSCH and PUCCH is not configured.

tpc-PDCCH-ConfigPUCCH-SPS PDCCH configuration for power control of slot/subslot-PUCCH using format 3/3A, see TS 36.212 [22], when SPS-ConfigDL-STTI is configured.

tpc-PDCCH-ConfigPUSCH-SPS PDCCH configuration for power control of slot/subslot-PUSCH using format 3/3A, see TS 36.212 [22], when SPS-ConfigUL-STTI is configured. If a UE is configured with multiple UL SPS configurations in a serving cell, the same TPC index for DCI format 3/3A applies to all the UL SPS configurations in the serving cell.

twoIntervalsConfig Trigger of two-intervals-Semi-Persistent Scheduling in uplink. See TS 36.321 [6], clause 5.10. If this field is present and the configured Semi-persistent scheduling interval greater than or equal to 10 sub-frames, two-intervals-SPS is enabled for uplink. Otherwise, two-intervals-SPS is disabled.

ul-SPS-V-RNTI UL Semi-Persistent Scheduling V-RNTI for UEs capable of multiple uplink SPS configurations and which support V2X communication, see TS 36.321 [6].

| Conditional presence | Explanation |
| --- | --- |
| TDD | This field is optional present for TDD, need OR; it is not present for FDD and the UE shall delete any existing value for this field. |
| SPS | This field is optional present if sps-ConfigIndex-r14 is not configured, need OR; otherwise it is not present. |

#### –	SPUCCH-Config

The IE SPUCCH-Config is used to specify the UE specific SPUCCH configuration.

SPUCCH-Config information element

-- ASN1START

SPUCCH-Config-r15 ::= CHOICE {

release							NULL,

setup							SEQUENCE {

spucch-Set-r15					SPUCCH-Set-r15		OPTIONAL, -- Need ON

twoAntennaPortActivatedSPUCCH-Format1a1b-r15	ENUMERATED {true}	OPTIONAL,	-- Need OR

dummy							SEQUENCE {

n3SPUCCH-AN-List-r15			SEQUENCE (SIZE (1..4)) OF INTEGER (0..549)

}

}

}

SPUCCH-Config-v1550 ::= CHOICE {

release							NULL,

setup							SEQUENCE {

twoAntennaPortActivatedSPUCCH-Format3-v1550		SEQUENCE {

n3SPUCCH-AN-List-v1550		SEQUENCE (SIZE (1..4)) OF INTEGER (0..549)

}

}

}

SPUCCH-Set-r15 ::= SEQUENCE (SIZE (1..4)) OF SPUCCH-Elements-r15

SPUCCH-Elements-r15 ::= CHOICE {

release							NULL,

setup							SEQUENCE {

n1SubslotSPUCCH-AN-List-r15		SEQUENCE (SIZE(1..4)) OF INTEGER (0..1319) OPTIONAL, -- Need OR

n1SlotSPUCCH-FH-AN-List-r15		INTEGER (0..1319)		OPTIONAL, -- Need OR

n1SlotSPUCCH-NoFH-AN-List-r15	INTEGER (0..3959)		OPTIONAL, -- Need OR

n3SPUCCH-AN-List-r15			INTEGER (0..549)		OPTIONAL, -- Need OR

n4SPUCCHSlot-Resource-r15		SEQUENCE (SIZE(1..2)) OF N4SPUCCH-Resource-r15	OPTIONAL, -- Need OR

n4SPUCCHSubslot-Resource-r15	SEQUENCE (SIZE(1..2)) OF N4SPUCCH-Resource-r15	OPTIONAL, -- Need OR

n4maxCoderateSlotPUCCH-r15		INTEGER (0..7)			OPTIONAL, -- Need OR

n4maxCoderateSubslotPUCCH-r15	INTEGER (0..7)			OPTIONAL, -- Need OR

n4maxCoderateMultiResourceSlotPUCCH-r15	INTEGER (0..7)		OPTIONAL,	-- Need OR

n4maxCoderateMultiResourceSubslotPUCCH-r15	INTEGER (0..7)		OPTIONAL	-- Need OR

}

}

N4SPUCCH-Resource-r15 ::= SEQUENCE {

n4startingPRB-r15				INTEGER (0..109),

n4numberOfPRB-r15				INTEGER (0..7)

}

-- ASN1STOP

SPUCCH-Config field descriptions

dummy This field is not used in the specification. If received it shall be ignored by the UE.

n1SlotSPUCCH-FH-AN-List Resource configuration for slot-SPUCCH format 1 when frequency hopping is enabled. Parameter: for antenna port P0 and for antenna port P1 respectively, see TS 36.213 [23], clause 10.1.

n1SlotSPUCCH-NoFH-AN-List Resource configuration for slot-SPUCCH format 1 when frequency hopping is disabled. Parameter: for antenna port P0 and for antenna port P1 respectively, see TS 36.213 [23], clause 10.1.

n1SubslotSPUCCH-AN-List Resource configuration for subslot-SPUCCH format 1. Parameter: for antenna port P0 and for antenna port P1 respectively, see TS 36.213 [23], clause 10.1.

n3SPUCCH-AN-List Resource index for slot-SPUCCH format 3: , see TS 36.213 [23], clause 10.1.

n4maxCoderateSlotPUCCH, n4maxCoderateSubslotPUCCH Indicates the maximum coding rate for slot-PUCCH and subslot-PUCCH format 4 transmission.

n4maxCoderateMultiResourceSlotPUCCH, n4maxCoderateMultiResourceSubslotPUCCH Indicates the maximum coding rate for slot-PUCCH and subslot-PUCCH format 4 transmission in case of multiple resource configuration.

n4numberOfPRB, n4numberOfPRBSubslot Parameter see TS 36.213 [23], Table 10.1.1-2 for determining SPUCCH resource(s) of SPUCCH format 4.

n4startingPRB Parameter see TS 36.211 [21], clause 5.4A.3 for determining SPUCCH resource(s) of SPUCCH format 4.

twoAntennaPortActivatedSPUCCH-Format1a1b Indicates whether two antenna ports are configured for SPUCCH format 1a/1b for HARQ-ACK, see TS 36.213 [23], clause 10.1. The field also applies for SPUCCH format 1a/1b transmission when format3 is configured, see TS 36.213 [23], clauses 10.1.2.2.2 and 10.1.3.2.2.

twoAntennaPortActivatedSPUCCH-Format3 Indicates whether two antenna ports are configured for SPUCCH format 3 for HARQ-ACK, see TS 36.213 [23], clause 10.1.

#### –	SRS-TPC-PDCCH-Config

The IE SRS-TPC-PDCCH-Config is used to specify the RNTIs and indexes for A-SRS trigger and TPC according to TS 36.212 [22].

SRS-TPC-PDCCH-Config information element

-- ASN1START

SRS-TPC-PDCCH-Config-r14 ::=					CHOICE {

release								NULL,

setup								SEQUENCE {

srs-TPC-RNTI-r14									BIT STRING (SIZE (16)),

startingBitOfFormat3B-r14							INTEGER (0..31),

fieldTypeFormat3B-r14								INTEGER (1..4),

srs-CC-SetIndexlist-r14							SEQUENCE (SIZE(1..4)) OF SRS-CC-SetIndex-r14	OPTIONAL	-- Cond SRS-Trigger-TypeA

}

}

SRS-CC-SetIndex-r14 ::=			SEQUENCE {

cc-SetIndex-r14				INTEGER (0..3),

cc-IndexInOneCC-Set-r14		INTEGER (0..7)

}

-- ASN1STOP

SRS-TPC-PDCCH-Config field descriptions

cc-IndexInOneCC-Set Indicates the CC index in one CC set for Type A associated with the group DCI with SRS request field (optional) and TPC commands for a PUSCH-less SCell

cc-SetIndex Indicates the CC set index for Type A associated with the group DCI with SRS request field (optional) and TPC commands for a PUSCH-less SCell.

fieldTypeFormat3B The type of a field within the group DCI with SRS request fields (optional) and TPC commands for a PUSCH-less SCell, which indicates how many bits in the field are for SRS request (0 or 1/2) and how many bits in the field are for TPC (1 or 2). Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its own TPC command bits. See TS 36.212 [22], clause 5.3.3.1.7A. EUTRAN configures this field with the same value for all PUSCH-less SCells.

srs-CC-SetIndexlist Indicates the index of the SRS-TPC-PDCCH-Config for Type A trigger by the group DCI with SRS request field (optional) and TPC commands for a PUSCH-less SCell. Each set may contain at most 8 CCs.

srs-TPC-RNTI RNTI for SRS trigger and power control using DCI format 3B, see TS 36.212 [22], clause 5.1.3.1.

startingBitOfFormat3B The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands for a PUSCH-less SCell.

| Conditional presence | Explanation |
| --- | --- |
| SRS-Trigger-TypeA | The field is mandatory present if typeA-SRS-TPC-PDCCH-Group-r14 is present. Otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	TDD-Config

The IE TDD-Config is used to specify the TDD specific physical channel configuration.

TDD-Config information element

-- ASN1START

TDD-Config ::=						SEQUENCE {

subframeAssignment					ENUMERATED {

sa0, sa1, sa2, sa3, sa4, sa5, sa6},

specialSubframePatterns				ENUMERATED {

ssp0, ssp1, ssp2, ssp3, ssp4,ssp5, ssp6, ssp7,

ssp8}

}

TDD-Config-v1130 ::=				SEQUENCE {

specialSubframePatterns-v1130		ENUMERATED {ssp7,ssp9}

}

TDD-Config-v1430 ::=				SEQUENCE {

specialSubframePatterns-v1430		ENUMERATED {ssp10}

}

TDD-Config-v1450 ::=				SEQUENCE {

specialSubframePatterns-v1450		ENUMERATED {ssp10-CRS-LessDwPTS}

}

TDD-ConfigSL-r12 ::=		SEQUENCE {

subframeAssignmentSL-r12				ENUMERATED {

none, sa0, sa1, sa2, sa3, sa4, sa5, sa6}

}

-- ASN1STOP

TDD-Config field descriptions

specialSubframePatterns Indicates Configuration as in TS 36.211 [21], table 4.2-1, where ssp0 points to Configuration 0, ssp1 to Configuration 1 etc. Value ssp7 points to Configuration 7 for extended cyclic prefix, value ssp9 points to Configuration 9 for normal cyclic prefix and value ssp10 points to Configration 10 for normal cyclic prefix. Value ssp10-CRS-LessDwPTS corresponds to ssp10 without CRS transmission on the 5th symbol of DwPTS. E-UTRAN signals ssp7 only when setting specialSubframePatterns (without suffix i.e. the version defined in REL-8) to ssp4. E-UTRAN signals value ssp9 only when setting specialSubframePatterns (without suffix) to ssp5. E-UTRAN signals value ssp10 or ssp10-CRS-LessDwPTS only when setting specialSubframePatterns (without suffix) to ssp0 or ssp5. If specialSubframePatterns-v1130, specialSubframePatterns-v1430, or specialSubframePatterns-v1450 is present, the UE shall ignore specialSubframePatterns (without suffix). If specialSubframePatterns-v1430 or specialSubframePatterns-v1450 is present, the UE shall ignore specialSubframePatterns-v1130. E-UTRAN does not simultanuosly configure TDD-Config-v1430 and TDD-Config-v1450.

subframeAssignment Indicates DL/UL subframe configuration where sa0 points to Configuration 0, sa1 to Configuration 1 etc. as specified in TS 36.211 [21], table 4.2-2. E-UTRAN configures the same value for serving cells residing on same frequency band.

subframeAssignmentSL Indicates UL/ DL subframe configuration where sa0 points to Configuration 0, sa1 to Configuration 1 etc. as specified in TS 36.211 [21], table 4.2-2. The value none means that no TDD specific physical channel configuration is applicable (i.e. the carrier on which MasterInformationBlock-SL is transmitted is an FDD UL carrier or the carrier on which MasterInformationBlock-SL-V2X is transmitted is a carrier for V2X sidelink communication).

#### –	TDM-PatternConfig

The IE TDM-PatternConfig is used to specify the UL/DL reference configuration indicating the time during which a UE configured with (NG)EN-DC or NE-DC is allowed to transmit, as specified in TS 38.101-3 [101] and TS 38.213 [88].

TDM-PatternConfig information element

-- ASN1START

TDM-PatternConfig-r15 ::=		CHOICE {

release							NULL,

setup							SEQUENCE {

subframeAssignment-r15		SubframeAssignment-r15,

harq-Offset-r15				INTEGER (0..9)

}

}

SubframeAssignment-r15 ::=		ENUMERATED {sa0, sa1, sa2, sa3, sa4, sa5, sa6}

-- ASN1STOP

TDM-PatternConfig field descriptions

subframeAssignment Indicates DL/UL subframe configuration where sa0 points to Configuration 0, sa1 to Configuration 1 etc. as specified in TS 36.211 [21], table 4.2-2. When configured in EN-DC with LTE TDD PCell, the value range of this field is {sa2, sa4, sa5}.

harq-Offset Indicates a HARQ subframe offset that is applied to the subframes designated as UL in the associated subrame assignment, see TS 36.213 [23]. When configured in EN-DC with LTE TDD PCell, the network ensures it does not violate the TDD configuration in SIB1, and the value range of this field is {0, 1, 2, 5, 6}.

#### –	TimeAlignmentTimer

The IE TimeAlignmentTimer is used to control how long the UE considers the serving cells belonging to the associated TAG to be uplink time aligned. Corresponds to the Timer for time alignment in TS 36.321 [6]. Value in number of sub-frames. Value sf500 corresponds to 500 sub-frames, sf750 corresponds to 750 sub-frames and so on.

TimeAlignmentTimer information element

-- ASN1START

TimeAlignmentTimer ::=					ENUMERATED {

sf500, sf750, sf1280, sf1920, sf2560, sf5120,

sf10240, infinity}

-- ASN1STOP

#### –	TimeReferenceInfo

TimeReferenceInfo information elements

-- ASN1START

TimeReferenceInfo-r15 ::=		SEQUENCE {

time-r15							ReferenceTime-r15,

uncertainty-r15						INTEGER (0..12)				OPTIONAL,	-- Need OR

timeInfoType-r15					ENUMERATED {localClock}		OPTIONAL,	-- Need OR

referenceSFN-r15					INTEGER (0..1023)			OPTIONAL	-- Cond TimeRef

}

ReferenceTime-r15 ::=			SEQUENCE {

refDays-r15							INTEGER (0..72999),

refSeconds-r15						INTEGER (0..86399),

refMilliSeconds-r15					INTEGER (0..999),

refQuarterMicroSeconds-r15			INTEGER (0..3999)

}

-- ASN1STOP

TimeReferenceInfo field descriptions

referenceSFN This field indicates the reference SFN for time reference information. The time field indicates the time at the ending boundary of the SFN indicated by referenceSFN. The UE considers the frame indicated by the referenceSFN nearest to the frame where the field is received. If the time field is included in SystemInformationBlockType16 and the referenceSFN field is not included, the time field indicates the time at the SFN boundary at or immediately after the ending boundary of the SI-window in which SystemInformationBlockType16 is transmitted.

time, timeInfoType This field indicates time reference with 0.25 us granularity. The indicated time is referenced at the network, i.e., without compensating for RF propagation delay. The indicated time in 0.25 us unit from the origin is refDays*86400*1000*4000 + refSeconds*1000*4000 + refMilliSeconds*4000 + refQuarterMicroSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from the origin of the time field. If timeInfoType is not included, the origin of the time field is 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If timeInfoType is set to localClock, the interpretation of the origin of the time is unspecified and left up to upper layers. If time field is included in SystemInformationBlockType16, this field is excluded when estimating changes in system information, i.e. changes of time should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1.

uncertainty This field indicates the number of LSBs which may be inaccurate in the refQuarterMicroSeconds field. If uncertainty is absent, the uncertainty of refQuarterMicroSeconds is not specified.

| Conditional presence | Explanation |
| --- | --- |
| TimeRef | The field is mandatory present if TimeReferenceInfo is included in DLInformationTransfer message; otherwise the field is not present. |

#### –	TPC-PDCCH-Config

The IE TPC-PDCCH-Config is used to specify the RNTIs and indexes for PUCCH and PUSCH power control according to TS 36.212 [22]. The power control function can either be setup or released with the IE.

TPC-PDCCH-Config information element

-- ASN1START

TPC-PDCCH-Config ::=					CHOICE {

release								NULL,

setup								SEQUENCE {

tpc-RNTI							BIT STRING (SIZE (16)),

tpc-Index							TPC-Index

}

}

TPC-PDCCH-ConfigSCell-r13 ::=					CHOICE {

release								NULL,

setup								SEQUENCE {

tpc-Index-PUCCH-SCell-r13		TPC-Index

}

}

TPC-Index ::=							CHOICE {

indexOfFormat3							INTEGER (1..15),

indexOfFormat3A							INTEGER (1..31)

}

-- ASN1STOP

TPC-PDCCH-Config field descriptions

indexOfFormat3 Index of N when DCI format 3 is used. See TS 36.212 [22], clause 5.3.3.1.6.

IndexOfFormat3A Index of M when DCI format 3A is used. See TS 36.212 [22], clause 5.3.3.1.7.

tpc-Index Index of N or M, see TS 36.212 [22], clauses 5.3.3.1.6 and 5.3.3.1.7, where N or M is dependent on the used DCI format (i.e. format 3 or 3a).

tpc-Index-PUCCH-SCell Index of N or M, see TS 36.212 [22], clauses 5.3.3.1.6 and 5.3.3.1.7, where N or M is dependent on the used DCI format (i.e. format 3 or 3a).

tpc-RNTI RNTI for power control using DCI format 3/3A, see TS 36.212 [22].

#### –	TunnelConfigLWIP

The IE TunnelConfigLWIP is used to setup/release LWIP Tunnel.

-- ASN1START

TunnelConfigLWIP-r13 ::= SEQUENCE {

ip-Address-r13			IP-Address-r13,

ike-Identity-r13			IKE-Identity-r13,

...,

[[	lwip-Counter-r13	INTEGER (0..65535)		OPTIONAL	-- Cond LWIP-Setup

]]

}

IKE-Identity-r13 ::= SEQUENCE {

idI-r13					OCTET STRING

}

IP-Address-r13 ::= CHOICE {

ipv4-r13					BIT STRING (SIZE (32)),

ipv6-r13					BIT STRING (SIZE (128))

}

-- ASN1STOP

TunnelConfigLWIP field descriptions

ip-Address Parameter indicates the LWIP-SeGW IP Address to be used by the UE for initiating LWIP Tunnel establishment [32].

ike-Identity Parameter indicates the IKE Identity elements (IDi) to be used in IKE Authentication Procedures [32].

lwip-Counter Indicates the parameter used by UE for computing the security keys used in LWIP tunnel establishment, as specified in TS 33.401 [32].

| Conditional presence | Explanation |
| --- | --- |
| LWIP-Setup | The field is mandatory present upon setup of LWIP tunnel. Otherwise the field is optional, Need ON. |

#### –	UplinkPowerControl

The IE UplinkPowerControlCommon and IE UplinkPowerControlDedicated are used to specify parameters for uplink power control in the system information and in the dedicated signalling, respectively.

UplinkPowerControl information elements

-- ASN1START

UplinkPowerControlCommon ::=		SEQUENCE {

p0-NominalPUSCH						INTEGER (-126..24),

alpha								Alpha-r12,

p0-NominalPUCCH						INTEGER (-127..-96),

deltaFList-PUCCH					DeltaFList-PUCCH,

deltaPreambleMsg3					INTEGER (-1..6)

}

UplinkPowerControlCommon-v1020 ::=	SEQUENCE {

deltaF-PUCCH-Format3-r10				ENUMERATED {deltaF-1, deltaF0, deltaF1, deltaF2,

deltaF3, deltaF4, deltaF5, deltaF6},

deltaF-PUCCH-Format1bCS-r10				ENUMERATED {deltaF1, deltaF2, spare2, spare1}

}

UplinkPowerControlCommon-v1310 ::=	SEQUENCE {

deltaF-PUCCH-Format4-r13			ENUMERATED {deltaF16, deltaF15, deltaF14,deltaF13, deltaF12,

deltaF11, deltaF10, spare1}			OPTIONAL,	-- Need OR

deltaF-PUCCH-Format5-13				ENUMERATED { deltaF13, deltaF12, deltaF11, deltaF10, deltaF9,

deltaF8, deltaF7, spare1}				OPTIONAL	-- Need OR

}

UplinkPowerControlCommon-v1530 ::=	SEQUENCE {

deltaFList-SPUCCH-r15		DeltaFList-SPUCCH-r15

}

UplinkPowerControlCommon-v1610 ::=	SEQUENCE {

alphaSRS-Add-r16						Alpha-r12,

p0-NominalSRS-Add-r16					INTEGER (-126..24)

}

UplinkPowerControlCommonPSCell-r12 ::=		SEQUENCE {

-- For uplink power control the additional/ missing fields are signalled (compared to SCell)

deltaF-PUCCH-Format3-r12				ENUMERATED {deltaF-1, deltaF0, deltaF1, deltaF2,

deltaF3, deltaF4, deltaF5, deltaF6},

deltaF-PUCCH-Format1bCS-r12				ENUMERATED {deltaF1, deltaF2, spare2, spare1},

p0-NominalPUCCH-r12						INTEGER (-127..-96),

deltaFList-PUCCH-r12					DeltaFList-PUCCH

}

UplinkPowerControlCommonSCell-r10 ::=	SEQUENCE {

p0-NominalPUSCH-r10					INTEGER (-126..24),

alpha-r10							Alpha-r12

}

UplinkPowerControlCommonSCell-v1130 ::=	SEQUENCE {

deltaPreambleMsg3-r11				INTEGER (-1..6)

}

UplinkPowerControlCommonSCell-v1310 ::=	SEQUENCE {

-- For uplink power control the additional/ missing fields are signalled (compared to SCell)

p0-NominalPUCCH							INTEGER (-127..-96),

deltaFList-PUCCH						DeltaFList-PUCCH,

deltaF-PUCCH-Format3-r12				ENUMERATED {deltaF-1, deltaF0, deltaF1,

deltaF2, deltaF3, deltaF4, deltaF5,

deltaF6}						OPTIONAL,	-- Need OR

deltaF-PUCCH-Format1bCS-r12				ENUMERATED {deltaF1, deltaF2,

spare2, spare1}					OPTIONAL,	-- Need OR

deltaF-PUCCH-Format4-r13					ENUMERATED {deltaF16, deltaF15, deltaF14,

deltaF13, deltaF12, deltaF11, deltaF10,

spare1}							OPTIONAL,	-- Need OR

deltaF-PUCCH-Format5-13						ENUMERATED { deltaF13, deltaF12, deltaF11,

deltaF10, deltaF9, deltaF8, deltaF7,

spare1}							OPTIONAL	-- Need OR

}

UplinkPowerControlCommonPUSCH-LessCell-v1430 ::=	SEQUENCE {

p0-Nominal-PeriodicSRS-r14						INTEGER (-126..24)		OPTIONAL,	-- Need OR

p0-Nominal-AperiodicSRS-r14						INTEGER (-126..24)		OPTIONAL,	-- Need OR

alpha-SRS-r14								Alpha-r12					OPTIONAL	-- Need OR

}

UplinkPowerControlDedicated ::=		SEQUENCE {

p0-UE-PUSCH							INTEGER (-8..7),

deltaMCS-Enabled					ENUMERATED {en0, en1},

accumulationEnabled					BOOLEAN,

p0-UE-PUCCH							INTEGER (-8..7),

pSRS-Offset							INTEGER (0..15),

filterCoefficient					FilterCoefficient					DEFAULT fc4

}

UplinkPowerControlDedicated-v1020 ::= SEQUENCE {

deltaTxD-OffsetListPUCCH-r10		DeltaTxD-OffsetListPUCCH-r10		OPTIONAL,	-- Need OR

pSRS-OffsetAp-r10					INTEGER (0..15)						OPTIONAL	-- Need OR

}

UplinkPowerControlDedicated-v1130 ::=		SEQUENCE {

pSRS-Offset-v1130						INTEGER (16..31)				OPTIONAL,	-- Need OR

pSRS-OffsetAp-v1130						INTEGER (16..31)				OPTIONAL,	-- Need OR

deltaTxD-OffsetListPUCCH-v1130			DeltaTxD-OffsetListPUCCH-v1130	OPTIONAL	-- Need OR

}

UplinkPowerControlDedicated-v1250 ::=	SEQUENCE {

set2PowerControlParameter		CHOICE {

release							NULL,

setup							SEQUENCE {

tpc-SubframeSet-r12					BIT STRING (SIZE(10)),

p0-NominalPUSCH-SubframeSet2-r12		INTEGER (-126..24),

alpha-SubframeSet2-r12				Alpha-r12,

p0-UE-PUSCH-SubframeSet2-r12			INTEGER (-8..7)

}

}

}

UplinkPowerControlDedicated-v1530 ::= SEQUENCE {

alpha-UE-r15				Alpha-r12							OPTIONAL,	-- Need OR

p0-UE-PUSCH-r15				INTEGER (-16..15)					OPTIONAL	-- Need OR

}

UplinkPowerControlDedicatedSTTI-r15 ::= SEQUENCE {

accumulationEnabledSTTI-r15		BOOLEAN,

deltaTxD-OffsetListSPUCCH-r15	DeltaTxD-OffsetListSPUCCH-r15	OPTIONAL,	-- Need OR

uplinkPower-CSIPayload			BOOLEAN

}

UplinkPUSCH-LessPowerControlDedicated-v1430 ::=		SEQUENCE {

p0-UE-PeriodicSRS-r14						INTEGER (-8..7)			OPTIONAL,	-- Need OR

p0-UE-AperiodicSRS-r14						INTEGER (-8..7)			OPTIONAL,	-- Need OR

accumulationEnabled-r14						BOOLEAN

}

UplinkPowerControlAddSRS-r16 ::= SEQUENCE {

tpc-IndexSRS-Add-r16				TPC-Index					OPTIONAL,	-- Need ON

startingBitOfFormat3B-SRS-Add-r16	INTEGER (0..31)				OPTIONAL,	-- Need ON

fieldTypeFormat3B-SRS-Add-r16		INTEGER (1..2)				OPTIONAL,	-- Need ON

p0-UE-SRS-Add-r16					INTEGER (-16..15)			OPTIONAL,	-- Need ON

accumulationEnabledSRS-Add-r16		BOOLEAN

}

UplinkPowerControlDedicatedSCell-r10 ::=		SEQUENCE {

p0-UE-PUSCH-r10						INTEGER (-8..7),

deltaMCS-Enabled-r10					ENUMERATED {en0, en1},

accumulationEnabled-r10				BOOLEAN,

pSRS-Offset-r10						INTEGER (0..15),

pSRS-OffsetAp-r10					INTEGER (0..15)					OPTIONAL,	-- Need OR

filterCoefficient-r10				FilterCoefficient					DEFAULT fc4,

pathlossReferenceLinking-r10		ENUMERATED {pCell, sCell}

}

UplinkPowerControlDedicatedSCell-v1310 ::=	SEQUENCE {

--Release 8

p0-UE-PUCCH							INTEGER (-8..7),

--Release 10

deltaTxD-OffsetListPUCCH-r10		DeltaTxD-OffsetListPUCCH-r10		OPTIONAL	-- Need OR

}

DeltaFList-PUCCH ::=				SEQUENCE {

deltaF-PUCCH-Format1				ENUMERATED {deltaF-2, deltaF0, deltaF2},

deltaF-PUCCH-Format1b				ENUMERATED {deltaF1, deltaF3, deltaF5},

deltaF-PUCCH-Format2				ENUMERATED {deltaF-2, deltaF0, deltaF1, deltaF2},

deltaF-PUCCH-Format2a				ENUMERATED {deltaF-2, deltaF0, deltaF2},

deltaF-PUCCH-Format2b				ENUMERATED {deltaF-2, deltaF0, deltaF2}

}

DeltaFList-SPUCCH-r15 ::= CHOICE {

release					NULL,

setup					SEQUENCE {

deltaF-slotSPUCCH-Format1-r15	ENUMERATED {deltaF-1, deltaF0, deltaF1, deltaF2,

deltaF3, deltaF4, deltaF5, deltaF6}	OPTIONAL, --Need OR

deltaF-slotSPUCCH-Format1a-r15	ENUMERATED {deltaF1, deltaF2, deltaF3, deltaF4,

deltaF5, deltaF6, deltaF7, deltaF8}	OPTIONAL, --Need OR

deltaF-slotSPUCCH-Format1b-r15	ENUMERATED {deltaF3, deltaF4, deltaF5, deltaF6,

deltaF7, deltaF8, deltaF9, deltaF10}	OPTIONAL,--Need OR

deltaF-slotSPUCCH-Format3-r15	ENUMERATED {deltaF4, deltaF5, deltaF6, deltaF7,

deltaF8, deltaF9, deltaF10, deltaF11}	OPTIONAL,--Need OR

deltaF-slotSPUCCH-RM-Format4-r15	ENUMERATED {deltaF13, deltaF14, deltaF15, deltaF16,

deltaF17, deltaF18, deltaF19, deltaF20}	OPTIONAL,

--Need OR

deltaF-slotSPUCCH-TBCC-Format4-r15	ENUMERATED {deltaF10, deltaF11, deltaF12, deltaF13,

deltaF14, deltaF15, deltaF16, deltaF17}	OPTIONAL,

--Need OR

deltaF-subslotSPUCCH-Format1and1a-r15	ENUMERATED {deltaF5, deltaF6, deltaF7, deltaF8,

deltaF9, deltaF10, deltaF11, deltaF12}	OPTIONAL,

--Need OR

deltaF-subslotSPUCCH-Format1b-r15	ENUMERATED {deltaF6, deltaF7, deltaF8, deltaF9,

deltaF10, deltaF11, deltaF12, deltaF13}	OPTIONAL,

--Need OR

deltaF-subslotSPUCCH-RM-Format4-r15	ENUMERATED {deltaF15, deltaF16, deltaF17, deltaF18,

deltaF19, deltaF20, deltaF21, deltaF22}	OPTIONAL,

--Need OR

deltaF-subslotSPUCCH-TBCC-Format4-r15	ENUMERATED {deltaF10, deltaF11, deltaF12, deltaF13,

deltaF14, deltaF15, deltaF16, deltaF17}	OPTIONAL,

--Need OR

...

}

}

DeltaTxD-OffsetListPUCCH-r10 ::=	SEQUENCE {

deltaTxD-OffsetPUCCH-Format1-r10		ENUMERATED {dB0, dB-2},

deltaTxD-OffsetPUCCH-Format1a1b-r10		ENUMERATED {dB0, dB-2},

deltaTxD-OffsetPUCCH-Format22a2b-r10	ENUMERATED {dB0, dB-2},

deltaTxD-OffsetPUCCH-Format3-r10		ENUMERATED {dB0, dB-2},

...

}

DeltaTxD-OffsetListPUCCH-v1130 ::=	SEQUENCE {

deltaTxD-OffsetPUCCH-Format1bCS-r11		ENUMERATED {dB0, dB-1}

}

DeltaTxD-OffsetListSPUCCH-r15 ::=	SEQUENCE {

deltaTxD-OffsetSPUCCH-Format1-r15		ENUMERATED {dB0, dB-2},

deltaTxD-OffsetSPUCCH-Format1a-r15		ENUMERATED {dB0, dB-2},

deltaTxD-OffsetSPUCCH-Format1b-r15		ENUMERATED {dB0, dB-2},

deltaTxD-OffsetSPUCCH-Format3-r15		ENUMERATED {dB0, dB-2},

...

}

-- ASN1STOP

UplinkPowerControl field descriptions

accumulationEnabled, accumulationEnabledSTTI Parameter: Accumulation-enabled, see TS 36.213 [23], clauses 5.1.1.1 and 5.1.3.1. TRUE corresponds to "enabled" whereas FALSE corresponds to "disabled".

accumulationEnabledSRS-Add Parameter: accumulationEnabled-additionalSRS, see TS 36.213 [23], clauses 5.1.3.1. TRUE corresponds to "enabled" whereas FALSE corresponds to "disabled".

alpha Parameter: α See TS 36.213 [23], clause 5.1.1.1. This field applies for uplink power control subframe set 1 if uplink power control subframe sets are configured by tpc-SubframeSet.

alpha-SRS, alphaSRS-Add Parameter: αSRS. See TS 36.213 [23], clause 5.1.3.1. alpha-SRS applies for SRS power control on a PUSCH-less SCell, alphaSRS-Add applies for SRS power control on the additional SRS symbols.

alpha-SubframeSet2 Parameter: α. See TS 36.213 [23], clause 5.1.1.1. This field applies for uplink power control subframe set 2 if uplink power control subframe sets are configured by tpc-SubframeSet.

alpha-UE Parameter: αUE See TS 36.213 [23], clause 5.1.1.1.

deltaF-PUCCH-FormatX Parameter: for the PUCCH formats 1, 1b, 2, 2a, 2b, 3, 4, 5 and 1b with channel selection. See TS 36.213 [23], clause 5.1.2, where deltaF-2 corresponds to -2 dB, deltaF0 corresponds to 0 dB and so on.

deltaF-PUCCH-FormatX, deltaF-slotSPUCCH-FormatX, deltaF-subslotSPUCCH-FormatX Parameter: for the SPUCCH formats 1, 1a, 1b, 3 and 4. See TS 36.213 [23], clause 5.1.2 where deltaF-2 corresponds to -2 dB, deltaF0 corresponds to 0 dB and so on. In case both an A and a B configuration exist, configuration A is used in case SPUCCH carries ≤ 22 HARQ-ACK bits, and B otherwise.

deltaMCS-Enabled Parameter: Ks See TS 36.213 [23], clause 5.1.1.1. en0 corresponds to value 0 corresponding to state "disabled". en1 corresponds to value 1.25 corresponding to "enabled".

deltaPreambleMsg3 Parameter: see TS 36.213 [23], clause 5.1.1.1. Actual value = field value * 2 [dB].

deltaTxD-OffsetPUCCH-FormatX Parameter: for the PUCCH formats 1, 1a/1b, 1b with channel selection, 2/2a/2b and 3 when two antenna ports are configured for PUCCH transmission. See TS 36.213 [23], clause 5.1.2.1, where dB0 corresponds to 0 dB, dB-1 corresponds to -1 dB, dB-2 corresponds to -2 dB. EUTRAN configures the field deltaTxD-OffsetPUCCH-Format1bCS-r11 for the PCell and/or the PSCell only.

deltaTxD-OffsetSPUCCH-FormatX Parameter: for the SPUCCH formats 1, 1a/1b, 1b with channel selection and 3 when two antenna ports are configured for SPUCCH transmission. See TS 36.213 [23], clause 5.1.2.1 where dB0 corresponds to 0 dB, dB-1 corresponds to -1 dB, dB-2 corresponds to -2 dB.

fieldTypeFormat3B-SRS-Add Indicates the field width of power control field in DCI format 3B for additional SRS. See TS 36.212 [22], clause 5.3.3.1.7A.

filterCoefficient Specifies the filtering coefficient for RSRP measurements used to calculate path loss, as specified in TS 36.213 [23], clause 5.1.1.1. The same filtering mechanism applies as for quantityConfig described in 5.5.3.2.

p0-Nominal-AperiodicSRS Parameter: where m=1. See TS 36.213 [23], clause 5.1.3.1, unit dBm.

p0-Nominal-PeriodicSRS Parameter: where m=0. See TS 36.213 [23], clause5.1.3.1, unit dBm.

p0-NominalPUCCH Parameter: See TS 36.213 [23], clause 5.1.2.1, unit dBm.

p0-NominalPUSCH Parameter: See TS 36.213 [23], clause 5.1.1.1, unit dBm. This field is applicable for non-persistent scheduling only. This field applies for uplink power control subframe set 1 if uplink power control subframe sets are configured by tpc-SubframeSet.

p0-NominalPUSCH-SubframeSet2 Parameter: . See TS 36.213 [23], clause 5.1.1.1, unit dBm. This field is applicable for non-persistent scheduling only. This field applies for uplink power control subframe set 2 if uplink power control subframe sets are configured by tpc-SubframeSet.

p0-NominalSRS-Add Parameter: where m=2. See TS 36.213 [23], clause 5.1.3.1, unit dBm.

p0-UE-SRS-Add Parameter: where m=2. See TS 36.213 [23], clause 5.1.3.1, unit dB.

p0-UE-AperiodicSRS Parameter: where m=1. See TS 36.213 [23], clause 5.1.3.1, unit dB.

p0-UE-PeriodicSRS Parameter: where m=0. See TS 36.213 [23], clause 5.1.3.1, unit dB.

p0-UE-PUCCH Parameter: See TS 36.213 [23], clause 5.1.2.1. Unit dB

p0-UE-PUSCH Parameter: See TS 36.213 [23], clause 5.1.1.1, unit dB. This field is applicable for non-persistent scheduling, only. This field applies for uplink power control subframe set 1 if uplink power control subframe sets are configured by tpc-SubframeSet. If p0-UE-PUSCH-r15 is included, the UE ignores p0-UE-PUSCH (i.e., without suffix).

p0-UE-PUSCH-SubframeSet2 Parameter: See TS 36.213 [23], clause 5.1.1.1, unit dB. This field is applicable for non-persistent scheduling, only. This field applies for uplink power control subframe set 2 if uplink power control subframe sets are configured by tpc-SubframeSet.

pathlossReferenceLinking Indicates whether the UE shall apply as pathloss reference either the downlink of the PCell or of the SCell that corresponds with this uplink (i.e. according to the cellIdentification within the field sCellToAddMod). For SCells part of an STAG E-UTRAN sets the value to sCell.

pSRS-Offset, pSRS-OffsetAp Parameter: PSRS_OFFSET for periodic and aperiodic sounding reference signal transmission repectively. See TS 36.213 [23], clause 5.1.3.1. For Ks=1.25, the actual parameter value is pSRS-Offset value – 3. For Ks=0, the actual parameter value is -10.5 + 1.5*pSRS-Offset value. If pSRS-Offset-v1130 is included, the UE ignores pSRS-Offset (i.e., without suffix). Likewise, if pSRS-OffsetAp-v1130 is included, the UE ignores pSRS-OffsetAp-r10. For Ks=0, E-UTRAN does not set values larger than 26.

startingBitOfFormat3B-SRS-Add Indicates the starting position of a block to trigger and TPC commands for the additional SRS symbols. See TS 36.212 [22], clause 5.3.3.1.7A.

tpc-IndexSRS-Add Indicates the index to the TPC command for the SRS in additional symbols. See TS 36.212 [22], clause 5.3.3.1.6 and 5.3.3.1.7.

tpc-SubframeSet Indicates the uplink subframes (including UpPTS in special subframes) of the uplink power control subframe sets. Value 0 means the subframe belongs to uplink power control subframe set 1, and value 1 means the subframe belongs to uplink power control subframe set 2.

uplinkPower-CSIPayload TRUE indicates that the UE shall derive BPRE based on the actual value of O_CQI for slot/subslot-PUSCH, whereas FALSE indicates that the largest value of O_CQI across all RI values shall be used for the derivation of BPRE for slot/subslot-PUSCH.

#### –	WLAN-Id-List

The IE WLAN-Id-List is used to list WLAN(s) for configuration of WLAN measurements and WLAN mobility set.

-- ASN1START

WLAN-Id-List-r13 ::=				SEQUENCE (SIZE (1..maxWLAN-Id-r13)) OF WLAN-Identifiers-r12

-- ASN1STOP

#### –	WLAN-MobilityConfig

The IE WLAN-MobilityConfig is used for configuration of WLAN mobility set and WLAN Status Reporting. E-UTRAN configures at least one WLAN identifier in the WLAN-MobilityConfig.

-- ASN1START

WLAN-MobilityConfig-r13 ::=		SEQUENCE {

wlan-ToReleaseList-r13				WLAN-Id-List-r13			OPTIONAL,	-- Need ON

wlan-ToAddList-r13					WLAN-Id-List-r13			OPTIONAL,	-- Need ON

associationTimer-r13				ENUMERATED {s10, s30,

s60, s120, s240}			OPTIONAL,	-- Need OR

successReportRequested-r13			ENUMERATED {true}			OPTIONAL,	-- Need OR

...,

[[	wlan-SuspendConfig-r14			WLAN-SuspendConfig-r14		OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

WLAN-MobilityConfig field descriptions

associationTimer Indicates the maximum time for connection to WLAN before connection failure reporting is initiated. Value s10 means 10 seconds, value s30 means 30 seconds and so on. E-UTRAN includes associationTimer only upon change in WLAN mobility set, lwa-WT-Counter or lwip-Counter.

successReportRequested Indicates whether the UE shall report successful connection to WLAN. Applicable to LWA and LWIP.

wlan-ToAddList Indicates the WLAN identifiers to be added to the WLAN mobility set.

wlan-ToReleaseList Indicates the WLAN identifiers to be removed from the WLAN mobility set.

#### –	WUS-Config

The IE WUS-Config is used to specify the WUS configuration. For the UEs supporting WUS, E-UTRAN uses WUS to indicate that the UE shall attempt to receive paging in that cell, see TS 36.304 [4].

WUS-Config information element

-- ASN1START

WUS-Config-r15 ::=				SEQUENCE {

maxDurationFactor-r15					ENUMERATED {one32th, one16th, one8th, one4th},

numPOs-r15						ENUMERATED {n1, n2, n4, spare1}		DEFAULT n1,

freqLocation-r15					ENUMERATED {n0, n2, n4, spare1},

timeOffsetDRX-r15				ENUMERATED {ms40, ms80, ms160, ms240},

timeOffset-eDRX-Short-r15		ENUMERATED {ms40, ms80, ms160, ms240},

timeOffset-eDRX-Long-r15		ENUMERATED {ms1000, ms2000}		OPTIONAL	-- Need OP

}

WUS-Config-v1560 ::=			SEQUENCE {

powerBoost-r15					ENUMERATED {dB0, dB1dot8, dB3, dB4dot8}

}

WUS-Config-v1610 ::=			SEQUENCE {

numDRX-CyclesRelaxed-r16		ENUMERATED {n1, n2, n4, n8}

}

-- ASN1STOP

WUS-Config field descriptions

freqLocation Frequency location of WUS within paging narrowband for BL UEs and UEs in CE. Value n0 corresponds to WUS in the 1st and 2nd PRB, value n2 represents the 3rd and 4th PRB, and value n4 represents the 5th and 6th PRB.

maxDurationFactor Maximum WUS duration, expressed as a ratio of Rmax associated with Type 1-CSS, see TS 36.211 [21]. Value one32th corresponds to Rmax * 1/32, value one16th corresponds to Rmax * 1/16 and so on. The value LMWUSmax in TS 36.213 [23] considered by the UE is : maxDuration = Max (signalled value * Rmax, 1) where Rmax is the value of mpdcch-NumRepetitionPaging for the carrier.

numDRX-CyclesRelaxed Maximum number of consecutive DRX cycles during which the UE can use WUS for synchronisation and skip serving cell measurements, see TS 36.133 [16]. Value n1 corresponds to 1 DRX cycle, value n2 corresponds to 2 DRX cycles and so on.

numPOs Number of consecutive Paging Occasions (PO) mapped to one WUS, applicable to UEs configured to use extended DRX, see TS 36.304 [4]. Value n1 corresponds to 1 PO, value n2 corresponds to 2 POs and so on.

powerBoost Power offset of WUS relative to CRS in dB, see TS 36.213 [23] clause 5.2. Value db0 corresponds to 0dB, value db1dot8 corresponds to 1.8dB, and so on.

timeOffsetDRX Minimum time gap in milliseconds from the end of the configured maximum WUS duration to the first associated PO, see TS 36.211 [21]. Value ms40 corresponds to 40 ms, value ms80 corresponds to 80 ms and so on.

timeOffset-eDRX-Short When eDRX is used, the short non-zero gap in milliseconds from the end of the configured maximum WUS duration to the associated PO, see TS 36.211 [21]. Value ms40 corresponds to 40 ms, value ms80 corresponds to 80 ms and so on. E-UTRAN configures timeOffset-eDRX-Short to a value longer than or equal to timeOffsetDRX.

timeOffset-eDRX-Long When eDRX is used, the long non-zero gap in milliseconds from the end of the configured maximum WUS duration to the associated PO, see TS 36.211 [21]. Value ms1000 corresponds to 1000 ms and value ms2000 corresponds to 2000 ms. If the field is absent, UE uses timeOffset-eDRX-Short for monitoring WUS.

### 6.3.3	Security control information elements

#### –	NextHopChainingCount

The IE NextHopChainingCount is used to update the KeNB key and corresponds to parameter NCC: See TS 33.401 [32], clause 7.2.8.4.

NextHopChainingCount information element

-- ASN1START

NextHopChainingCount ::=					INTEGER (0..7)

-- ASN1STOP

#### –	SecurityAlgorithmConfig

The IE SecurityAlgorithmConfig is used to configure AS integrity protection algorithm (SRBs) and AS ciphering algorithm (SRBs and DRBs). For RNs, the IE SecurityAlgorithmConfig is also used to configure AS integrity protection algorithm for DRBs between the RN and the E-UTRAN.

SecurityAlgorithmConfig information element

-- ASN1START

SecurityAlgorithmConfig ::=			SEQUENCE {

cipheringAlgorithm					CipheringAlgorithm-r12,

integrityProtAlgorithm				ENUMERATED {

eia0-v920, eia1, eia2, eia3-v1130, spare4, spare3,

spare2, spare1, ...}

}

CipheringAlgorithm-r12 ::=				ENUMERATED {

eea0, eea1, eea2, eea3-v1130, spare4, spare3,

spare2, spare1, ...}

-- ASN1STOP

SecurityAlgorithmConfig field descriptions

cipheringAlgorithm Indicates the ciphering algorithm to be used for SRBs and DRBs, as specified in TS 33.401 [32], clause 5.1.3.2.

integrityProtAlgorithm Indicates the integrity protection algorithm to be used for SRBs, as specified in TS 33.401 [32], clause 5.1.4.2. For RNs, also indicates the integrity protection algorithm to be used for integrity protection-enabled DRB(s).

#### –	ShortMAC-I

The IE ShortMAC-I is used to identify and verify the UE at RRC connection re-establishment. The 16 least significant bits of the MAC-I calculated using the security configuration of the source PCell, as specified in 5.3.7.4.

ShortMAC-I information element

-- ASN1START

ShortMAC-I ::=						BIT STRING (SIZE (16))

-- ASN1STOP

### 6.3.4	Mobility control information elements

#### –	AdditionalSpectrumEmission

If an extension is signalled using the extended value range (as defined by IE AdditionalSpectrumEmission-v10l0), the corresponding original field, using the value range as defined by IE AdditionalSpectrumEmission i.e. without suffix) shall be set to value 32, if signalled. UE supporting an LTE band assigned NS values larger than 32 as defined in TS 36.101 [42], clause 6.2.4, needs to support extension signaling (as defined by IE AdditionalSpectrumEmission-v10l0).

AdditionalSpectrumEmission information element

-- ASN1START

AdditionalSpectrumEmission ::=		INTEGER (1..32)

AdditionalSpectrumEmission-v10l0 ::=	INTEGER (33..288)

-- ASN1STOP

#### –	AdditionalSpectrumEmissionNR

The IE AdditionalSpectrumEmissionNR is used to indicate NR emission requirements to be fulfilled by the UE (see TS 38.101-1 [85], clause 6.5.3.3, and TS 38.101-2 [100], clause 6.5.3.2 and TS 38.101-3 [101], clause 6.5B.2)

AdditionalSpectrumEmissionNR information element

-- ASN1START

AdditionalSpectrumEmissionNR-r15 ::=		INTEGER (0..7)

-- ASN1STOP

#### –	ARFCN-ValueCDMA2000

The IE ARFCN-ValueCDMA2000 used to indicate the CDMA2000 carrier frequency within a CDMA2000 band, see C.S0002 [12].

ARFCN-ValueCDMA2000 information element

-- ASN1START

ARFCN-ValueCDMA2000 ::=			INTEGER (0..2047)

-- ASN1STOP

#### –	ARFCN-ValueEUTRA

The IE ARFCN-ValueEUTRA is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) E-UTRA carrier frequency, as defined in TS 36.101 [42]. If an extension is signalled using the extended value range (as defined by IE ARFCN-ValueEUTRA-v9e0), the UE shall only consider this extension (and hence ignore the corresponding original field, using the value range as defined by IE ARFCN-ValueEUTRA i.e. without suffix, if signalled). In dedicated signalling, E-UTRAN only provides an EARFCN corresponding to an E-UTRA band supported by the UE.

ARFCN-ValueEUTRA information element

-- ASN1START

ARFCN-ValueEUTRA ::=				INTEGER (0..maxEARFCN)

ARFCN-ValueEUTRA-v9e0 ::=			INTEGER (maxEARFCN-Plus1..maxEARFCN2)

ARFCN-ValueEUTRA-r9 ::=				INTEGER (0..maxEARFCN2)

-- ASN1STOP

NOTE:	For fields using the original value range, as defined by IE ARFCN-ValueEUTRA i.e. without suffix, value maxEARFCN indicates that the E-UTRA carrier frequency is indicated by means of an extension. In such a case, UEs not supporting the extension consider the field to be set to a not supported value.

#### –	ARFCN-ValueGERAN

The IE ARFCN-ValueGERAN is used to specify the ARFCN value applicable for a GERAN BCCH carrier frequency, see TS 45.005 [20].

ARFCN-ValueGERAN information element

-- ASN1START

ARFCN-ValueGERAN ::=			INTEGER (0..1023)

-- ASN1STOP

#### –	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR carrier frequency, as defined in TS 38.101 [85].

ARFCN-ValueNR information element

-- ASN1START

ARFCN-ValueNR-r15 ::=				INTEGER (0.. 3279165)

-- ASN1STOP

#### –	ARFCN-ValueUTRA

The IE ARFCN-ValueUTRA is used to indicate the ARFCN applicable for a downlink (Nd, FDD) or bi-directional (Nt, TDD) UTRA carrier frequency, as defined in TS 25.331 [19].

ARFCN-ValueUTRA information element

-- ASN1START

ARFCN-ValueUTRA ::=					INTEGER (0..16383)

-- ASN1STOP

#### –	BandclassCDMA2000

The IE BandclassCDMA2000 is used to define the CDMA2000 band in which the CDMA2000 carrier frequency can be found, as defined in C.S0057 [24], table 1.5-1.

BandclassCDMA2000 information element

-- ASN1START

BandclassCDMA2000 ::=					ENUMERATED {

bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8,

bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16,

bc17, bc18-v9a0, bc19-v9a0, bc20-v9a0, bc21-v9a0,

spare10, spare9, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1, ...}

-- ASN1STOP

#### –	BandIndicatorGERAN

The IE BandIndicatorGERAN indicates how to interpret an associated GERAN carrier ARFCN, see TS 45.005 [20]. More specifically, the IE indicates the GERAN frequency band in case the ARFCN value can concern either a DCS 1800 or a PCS 1900 carrier frequency. For ARFCN values not associated with one of these bands, the indicator has no meaning.

BandIndicatorGERAN information element

-- ASN1START

BandIndicatorGERAN ::=			ENUMERATED {dcs1800, pcs1900}

-- ASN1STOP

#### –	CarrierFreqCDMA2000

The IE CarrierFreqCDMA2000 used to provide the CDMA2000 carrier information.

CarrierFreqCDMA2000 information element

-- ASN1START

CarrierFreqCDMA2000 ::=			SEQUENCE {

bandClass							BandclassCDMA2000,

arfcn							ARFCN-ValueCDMA2000

}

-- ASN1STOP

#### –	CarrierFreqGERAN

The IE CarrierFreqGERAN is used to provide an unambiguous carrier frequency description of a GERAN cell.

CarrierFreqGERAN information element

-- ASN1START

CarrierFreqGERAN ::=			SEQUENCE {

arfcn							ARFCN-ValueGERAN,

bandIndicator					BandIndicatorGERAN

}

-- ASN1STOP

CarrierFreqGERAN field descriptions

arfcn GERAN ARFCN of BCCH carrier.

bandIndicator Indicates how to interpret the ARFCN of the BCCH carrier.

#### –	CarrierFreqsGERAN

The IE CarrierFreqListGERAN is used to provide one or more GERAN ARFCN values, as defined in TS 45.005 [43], which represents a list of GERAN BCCH carrier frequencies.

CarrierFreqsGERAN information element

-- ASN1START

CarrierFreqsGERAN ::=			SEQUENCE {

startingARFCN						ARFCN-ValueGERAN,

bandIndicator						BandIndicatorGERAN,

followingARFCNs						CHOICE {

explicitListOfARFCNs				ExplicitListOfARFCNs,

equallySpacedARFCNs					SEQUENCE {

arfcn-Spacing						INTEGER (1..8),

numberOfFollowingARFCNs				INTEGER (0..31)

},

variableBitMapOfARFCNs				OCTET STRING (SIZE (1..16))

}

}

ExplicitListOfARFCNs ::=			SEQUENCE (SIZE (0..31)) OF ARFCN-ValueGERAN

-- ASN1STOP

CarrierFreqsGERAN field descriptions

arfcn-Spacing Space, d, between a set of equally spaced ARFCN values.

bandIndicator Indicates how to interpret the ARFCN of the BCCH carrier.

explicitListOfARFCNs The remaining ARFCN values in the set are explicitly listed one by one.

followingARFCNs Field containing a representation of the remaining ARFCN values in the set.

numberOfFollowingARFCNs The number, n, of the remaining equally spaced ARFCN values in the set. The complete set of (n+1) ARFCN values is defined as: {s, ((s + d) mod 1024), ((s + 2*d) mod 1024) ... ((s + n*d) mod 1024)}.

startingARFCN The first ARFCN value, s, in the set.

variableBitMapOfARFCNs Bitmap field representing the remaining ARFCN values in the set. The leading bit of the first octet in the bitmap corresponds to the ARFCN = ((s + 1) mod 1024), the next bit to the ARFCN = ((s + 2) mod 1024), and so on. If the bitmap consists of N octets, the trailing bit of octet N corresponds to ARFCN = ((s + 8*N) mod 1024). The complete set of ARFCN values consists of ARFCN = s and the ARFCN values, where the corresponding bit in the bitmap is set to "1".

#### –	CarrierFreqListMBMS

The IE CarrierFreqListMBMS is used to indicate the E-UTRA ARFCN values of the one or more MBMS frequencies the UE is interested to receive.

CarrierFreqListMBMS information element

-- ASN1START

CarrierFreqListMBMS-r11 ::=		SEQUENCE (SIZE (1..maxFreqMBMS-r11)) OF ARFCN-ValueEUTRA-r9

-- ASN1STOP

#### –	CDMA2000-Type

The IE CDMA2000-Type is used to describe the type of CDMA2000 network.

CDMA2000-Type information element

-- ASN1START

CDMA2000-Type ::=					ENUMERATED {type1XRTT, typeHRPD}

-- ASN1STOP

#### –	CellGlobalIdNR

The IE CellGlobalIdNR specifies the Cell Global Identifier (CGI), the globally unique identity and the tracking area code (TAC) of a cell in NR.

CellGlobalIdNR information element

-- ASN1START

CellGlobalIdNR-r16 ::=					SEQUENCE {

plmn-Identity-r16							PLMN-Identity,

cellIdentity-r16							CellIdentityNR-r15,

trackingAreaCode-r16						TrackingAreaCodeNR-r15			OPTIONAL

}

-- ASN1STOP

CellGlobalIdNR field descriptions

cellIdentity Identity of the cell within the context of the PLMN.

plmn-Identity Identifies the PLMN of the cell as given by the first PLMN entry in the plmn-IdentityInfoList in SIB1.

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs.

#### –	CellIdentity

The IE CellIdentity is used to unambiguously identify a cell within a PLMN.

CellIdentity information element

-- ASN1START

CellIdentity ::=					BIT STRING (SIZE (28))

-- ASN1STOP

#### –	CellIndexList

The IE CellIndexList concerns a list of cell indices, which may be used for different purposes.

CellIndexList information element

-- ASN1START

CellIndexList ::=						SEQUENCE (SIZE (1..maxCellMeas)) OF CellIndex

CellIndex ::=							INTEGER (1..maxCellMeas)

-- ASN1STOP

#### –	CellReselectionPriority

The IE CellReselectionPriority concerns the absolute priority of the concerned carrier frequency/ set of frequencies (GERAN)/ bandclass (CDMA2000), as used by the cell reselection procedure. Corresponds with parameter "priority" in TS 36.304 [4]. Value 0 means: lowest priority. The UE behaviour for the case the field is absent, if applicable, is specified in TS 36.304 [4].

CellReselectionPriority information element

-- ASN1START

CellReselectionPriority ::=				INTEGER (0..7)

-- ASN1STOP

#### –	CellSelectionInfoCE

The IE CellSelectionInfoCE contains cell selection information for CE. The q-RxLevMinCE corresponds to parameter Qrxlevmin_CE in TS 36.304 [4]. The q-QualMinRSRQ-CE corresponds to parameter Qqualmin_CE in TS 36.304 [4]. If q-QualMinRSRQ-CE is not present, the UE applies the (default) value of negative infinity for Qqualmin.

CellSelectionInfoCE information element

-- ASN1START

CellSelectionInfoCE-r13 ::=		SEQUENCE {

q-RxLevMinCE-r13				Q-RxLevMin,

q-QualMinRSRQ-CE-r13			Q-QualMin-r9						OPTIONAL	-- Need OR

}

CellSelectionInfoCE-v1530 ::=		SEQUENCE {

powerClass14dBm-Offset-r15			ENUMERATED {dB-6, dB-3, dB3, dB6, dB9, dB12}

}

-- ASN1STOP

CellSelectionInfoCE field descriptions

powerClass14dBm-Offset Parameter "Poffset" in TS 36.304 [4], only applicable for UE supporting powerClass-14dBm. Value in dB. Value dB-6 corresponds to -6 dB, dB-3 corresponds to -3 dB and so on. E-UTRAN configures this field only if cellSelectionInfoCE-r13 is configured. If the field is absent, the UE applies the (default) value of 0 dB for "Poffset" in TS 36.304 [4].

#### –	CellSelectionInfoCE1

The IE CellSelectionInfoCE1 contains cell selection information for BL UEs or UEs in CE supporting CE Mode B. The q-RxLevMinCE1 corresponds to parameter Qrxlevmin_CE1 in TS 36.304 [4]. If delta-RxLevMinCE1 is not included, actual value Qrxlevmin_CE1 = q-RxLevMinCE1 * 2 [dBm]. If delta-RxLevMinCE1 is included, the actual value Qrxlevmin_CE1 = (q-RxLevMinCE1 + delta-RxLevMinCE1) * 2 [dBm]. The q-QualMinRSRQ-CE1 corresponds to parameter Qqualmin_CE1 in TS 36.304 [4]. If q-QualMinRSRQ-CE1 is not present, the UE applies the (default) value of negative infinity for Qqualmin.

CellSelectionInfoCE1 information element

-- ASN1START

CellSelectionInfoCE1-r13 ::=		SEQUENCE {

q-RxLevMinCE1-r13				Q-RxLevMin,

q-QualMinRSRQ-CE1-r13			Q-QualMin-r9						OPTIONAL	-- Need OR

}

CellSelectionInfoCE1-v1360 ::=		SEQUENCE {

delta-RxLevMinCE1-v1360					INTEGER (-8..-1)

}

-- ASN1STOP

#### –	CellReselectionSubPriority

The IE CellReselectionSubPriority indicates a fractional value to be added to the value of cellReselectionPriority to obtain the absolute priority of the concerned carrier frequency for E-UTRA and NR. Value oDot2 corresponds to 0.2, oDot4 corresponds to 0.4 and so on.

CellReselectionSubPriority information element

-- ASN1START

CellReselectionSubPriority-r13 ::=			ENUMERATED {oDot2, oDot4, oDot6, oDot8}

-- ASN1STOP

#### –	CSFB-RegistrationParam1XRTT

The IE CSFB-RegistrationParam1XRTT is used to indicate whether or not the UE shall perform a CDMA2000 1xRTT pre-registration if the UE does not have a valid / current pre-registration.

-- ASN1START

CSFB-RegistrationParam1XRTT ::=		SEQUENCE {

sid									BIT STRING (SIZE (15)),

nid									BIT STRING (SIZE (16)),

multipleSID							BOOLEAN,

multipleNID							BOOLEAN,

homeReg								BOOLEAN,

foreignSIDReg						BOOLEAN,

foreignNIDReg						BOOLEAN,

parameterReg						BOOLEAN,

powerUpReg							BOOLEAN,

registrationPeriod					BIT STRING (SIZE (7)),

registrationZone					BIT STRING (SIZE (12)),

totalZone							BIT STRING (SIZE (3)),

zoneTimer							BIT STRING (SIZE (3))

}

CSFB-RegistrationParam1XRTT-v920 ::=	SEQUENCE {

powerDownReg-r9						ENUMERATED {true}

}

-- ASN1STOP

CSFB-RegistrationParam1XRTT field descriptions

foreignNIDReg The CDMA2000 1xRTT NID roamer registration indicator.

foreignSIDReg The CDMA2000 1xRTT SID roamer registration indicator.

homeReg The CDMA2000 1xRTT Home registration indicator.

multipleNID The CDMA2000 1xRTT Multiple NID storage indicator.

multipleSID The CDMA2000 1xRTT Multiple SID storage indicator.

nid Used along with the sid as a pair to control when the UE should Register or Re-Register with the CDMA2000 1xRTT network.

parameterReg The CDMA2000 1xRTT Parameter-change registration indicator.

powerDownReg The CDMA2000 1xRTT Power-down registration indicator. If set to TRUE, the UE that has a valid / current CDMA2000 1xRTT pre-registration will perform a CDMA2000 1xRTT power down registration when it is switched off.

powerUpReg The CDMA2000 1xRTT Power-up registration indicator.

registrationPeriod The CDMA2000 1xRTT Registration period.

registrationZone The CDMA2000 1xRTT Registration zone.

sid Used along with the nid as a pair to control when the UE should Register or Re-Register with the CDMA2000 1xRTT network.

totalZone The CDMA2000 1xRTT Number of registration zones to be retained.

zoneTimer The CDMA2000 1xRTT Zone timer length.

#### –	CellGlobalIdEUTRA

The IE CellGlobalIdEUTRA specifies the Evolved Cell Global Identifier (ECGI), the globally unique identity of a cell in E-UTRA.

CellGlobalIdEUTRA information element

-- ASN1START

CellGlobalIdEUTRA ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellIdentity							CellIdentity

}

-- ASN1STOP

CellGlobalIdEUTRA field descriptions

cellIdentity Identity of the cell within the context of the PLMN.

plmn-Identity Identifies the PLMN of the cell as given by the first PLMN entry in the plmn-IdentityList in SystemInformationBlockType1.

#### –	CellGlobalIdUTRA

The IE CellGlobalIdUTRA specifies the global UTRAN Cell Identifier, the globally unique identity of a cell in UTRA.

CellGlobalIdUTRA information element

-- ASN1START

CellGlobalIdUTRA ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellIdentity							BIT STRING (SIZE (28))

}

-- ASN1STOP

CellGlobalIdUTRA field descriptions

cellIdentity UTRA Cell Identifier which is unique within the context of the identified PLMN as defined in TS 25.331 [19].

plmn-Identity Identifies the PLMN of the cell as given by the common PLMN broadcast in the MIB, as defined in TS 25.331 [19].

#### –	CellGlobalIdGERAN

The IE CellGlobalIdGERAN specifies the Cell Global Identification (CGI), the globally unique identity of a cell in GERAN.

CellGlobalIdGERAN information element

-- ASN1START

CellGlobalIdGERAN ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

locationAreaCode						BIT STRING (SIZE (16)),

cellIdentity						BIT STRING (SIZE (16))

}

-- ASN1STOP

CellGlobalIdGERAN field descriptions

cellIdentity Cell Identifier which is unique within the context of the GERAN location area as defined in TS 23.003 [27].

locationAreaCode A fixed length code identifying the location area within a PLMN as defined in TS 23.003 [27].

plmn-Identity Identifies the PLMN of the cell, as defined in TS 23.003 [27]..

#### –	CellGlobalIdCDMA2000

The IE CellGlobalIdCDMA2000 specifies the Cell Global Identification (CGI), the globally unique identity of a cell in CDMA2000.

CellGlobalIdCDMA2000 information element

-- ASN1START

CellGlobalIdCDMA2000 ::=				CHOICE {

cellGlobalId1XRTT						BIT STRING (SIZE (47)),

cellGlobalIdHRPD						BIT STRING (SIZE (128))

}

-- ASN1STOP

CellGlobalIdCDMA2000 field descriptions

cellGlobalId1XRTT Unique identifier for a CDMA2000 1xRTT cell, corresponds to BASEID, SID and NID parameters (in that order) defined in C.S0005 [25].

cellGlobalIdHRPD Unique identifier for a CDMA2000 HRPD cell, corresponds to SECTOR ID parameter defined in C.S0024 [26], clause 14.9.

#### –	CellSelectionInfoNFreq

The IE CellSelectionInfoNFreq includes the parameters used for cell selection on a neighbouring frequency, see TS 36.304 [4].

CellSelectionInfoNFreq information element

-- ASN1START

CellSelectionInfoNFreq-r13 ::=	SEQUENCE {

-- Cell selection information as in SIB1

q-RxLevMin-r13					Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL,	-- Need OP

-- Cell re-selection information as in SIB3

q-Hyst-r13							ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

q-RxLevMinReselection-r13			Q-RxLevMin,

t-ReselectionEUTRA-r13				T-Reselection

}

-- ASN1STOP

#### –	ConditionalReconfiguration

The IE ConditionalReconfiguration is used to add, modify or release the configuration of a conditional handover per target candidate cell.

ConditionalReconfiguration information element

-- ASN1START

ConditionalReconfiguration-r16 ::= SEQUENCE {

condReconfigurationToAddModList-r16	CondReconfigurationToAddModList-r16		OPTIONAL, -- Need ON

condReconfigurationToRemoveList-r16	CondReconfigurationToRemoveList-r16		OPTIONAL, -- Need ON

attemptCondReconf-r16				ENUMERATED {true}						OPTIONAL, -- Cond CHO

...

}

CondReconfigurationToRemoveList-r16 ::= SEQUENCE (SIZE (1..maxCondConfig-r16)) OF CondReconfigurationId-r16

-- ASN1STOP

ConditionalReconfiguration field descriptions

attemptCondReconf If present, the UE shall perform conditional reconfiguration if selected cell is a target candidate cell and it is the first cell selection after failure as described in 5.3.7.3.

condReconfigurationToAddModList List of conditional reconfigurations (i.e. conditional handover) to add and/or modify.

condReconfigurationToRemoveList List of conditional reconfigurations (i.e. conditional handover) to remove.

| Conditional presence | Explanation |
| --- | --- |
| CHO | The field is optional present, Need OR, if the UE is configured with at least a candidate cell for CHO. Otherwise the field is not present. |

#### –	ConditionalReconfigurationId

The IE ConditionalReconfigurationId is used to identify a conditional reconfiguration (e.g. CHO).

ConditionalReconfigurationId information element

-- ASN1START

CondReconfigurationId-r16 ::= INTEGER (1.. maxCondConfig-r16)

-- ASN1STOP

#### –	CondReconfigurationToAddModList

The IE CondReconfigurationToAddModList concerns a list of conditional reconfigurations (i.e. conditional handover) to add or modify, for each entry the measId (associated to the triggering condition configuration) and the associated RRCConnectionReconfiguration.

CondReconfigurationToAddModList information element

-- ASN1START

CondReconfigurationToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxCondConfig-r16)) OF CondReconfigurationAddMod-r16

CondReconfigurationAddMod-r16 ::= SEQUENCE {

condReconfigurationId-r16			CondReconfigurationId-r16,

triggerCondition-r16				SEQUENCE (SIZE (1..2)) OF MeasId

OPTIONAL,  -- Cond CondReconfigurationAdd

condReconfigurationToApply-r16	OCTET STRING (CONTAINING RRCConnectionReconfiguration)

OPTIONAL,-- Cond CondReconfigurationAdd

...

}

-- ASN1STOP

CondReconfigurationToAddMod field descriptions

condReconfigurationToApply The RRCConnectionReconfiguration message to be applied when the condition(s) are fulfilled. The RRCConnectionReconfiguration in condReconfigurationToApply cannot contain a target node SCG configuration for CHO.

triggerCondition The condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration. When configuring two triggering events (MeasIds) for a candidate cell, the network ensures that both refer to the same measObject.

| Conditional presence | Explanation |
| --- | --- |
| CondReconfigurationAdd | The field is mandatory present if a condReconfigurationId is being added. Otherwise it is optional, need ON. |

#### –	CSG-Identity

The IE CSG-Identity is used to identify a Closed Subscriber Group.

CSG-Identity information element

-- ASN1START

CSG-Identity ::=					BIT STRING (SIZE (27))

-- ASN1STOP

#### –	FreqBandIndicator

The IE FreqBandIndicator indicates the E-UTRA operating band as defined in TS 36.101 [42], table 5.5-1. If an extension is signalled using the extended value range (as defined by IE FreqBandIndicator-v9e0), the UE shall only consider this extension (and hence ignore the corresponding original field, using the value range as defined by IE FreqBandIndicator i.e. without suffix, if signalled).

FreqBandIndicator information element

-- ASN1START

FreqBandIndicator ::=					INTEGER (1..maxFBI)

FreqBandIndicator-v9e0 ::=				INTEGER (maxFBI-Plus1..maxFBI2)

FreqBandIndicator-r11 ::=				INTEGER (1..maxFBI2)

-- ASN1STOP

NOTE:	For fields using the original value range, as defined by IE FreqBandIndicator i.e. without suffix, value maxFBI indicates that the frequency band is indicated by means of an extension. In such a case, UEs not supporting the extension consider the field to be set to a not supported value.

#### –	FreqBandIndicatorNR

The IE FreqBandIndicatorNR indicates the NR operating band as defined in TS 38.101 [85].

FreqBandIndicatorNR information element

-- ASN1START

FreqBandIndicatorNR-r15 ::=			INTEGER (1.. maxFBI-NR-r15)

-- ASN1STOP

#### –	MobilityControlInfo

The IE MobilityControlInfo includes parameters relevant for network controlled mobility to/within E-UTRA.

MobilityControlInfo information element

-- ASN1START

MobilityControlInfo ::=		SEQUENCE {

targetPhysCellId					PhysCellId,

carrierFreq							CarrierFreqEUTRA					OPTIONAL,	-- Cond HO-toEUTRA2

carrierBandwidth					CarrierBandwidthEUTRA				OPTIONAL,	-- Cond HO-toEUTRA

additionalSpectrumEmission			AdditionalSpectrumEmission			OPTIONAL,	-- Cond HO-toEUTRA

t304								ENUMERATED {

ms50, ms100, ms150, ms200, ms500, ms1000,

ms2000, ms10000-v1310},

newUE-Identity						C-RNTI,

radioResourceConfigCommon			RadioResourceConfigCommon,

rach-ConfigDedicated				RACH-ConfigDedicated				OPTIONAL,	-- Need OP

...,

[[	carrierFreq-v9e0				CarrierFreqEUTRA-v9e0				OPTIONAL	-- Need ON

]],

[[	drb-ContinueROHC-r11			ENUMERATED {true}					OPTIONAL	-- Cond HO

]],

[[	mobilityControlInfoV2X-r14	MobilityControlInfoV2X-r14				OPTIONAL,	-- Need ON

handoverWithoutWT-Change-r14	ENUMERATED {keepLWA-Config, sendEndMarker}	OPTIONAL,	-- Cond HO

makeBeforeBreak-r14				ENUMERATED {true}					OPTIONAL,	-- Need OR

rach-Skip-r14					RACH-Skip-r14						OPTIONAL,	-- Need OR

sameSFN-Indication-r14			ENUMERATED {true}					OPTIONAL	-- Cond HO-SFNsynced

]],

[[

mib-RepetitionStatus-r14		BOOLEAN								OPTIONAL,	-- Need OR

schedulingInfoSIB1-BR-r14		INTEGER (0..31)						OPTIONAL	-- Cond HO-SFNsynced

]],

[[	daps-Config-r16					DAPS-Config-r16						OPTIONAL	-- Cond NotFullConfigHO

]]

}

MobilityControlInfo-v10l0 ::=		SEQUENCE {

additionalSpectrumEmission-v10l0	AdditionalSpectrumEmission-v10l0	OPTIONAL	-- Need ON

}

MobilityControlInfoSCG-r12 ::=		SEQUENCE {

t307-r12							ENUMERATED {

ms50, ms100, ms150, ms200, ms500, ms1000,

ms2000, spare1},

ue-IdentitySCG-r12					C-RNTI							OPTIONAL,	-- Cond SCGEst,

rach-ConfigDedicated-r12			RACH-ConfigDedicated			OPTIONAL,	-- Need OP

cipheringAlgorithmSCG-r12		CipheringAlgorithm-r12		OPTIONAL,	-- Need ON

...,

[[	makeBeforeBreakSCG-r14			ENUMERATED {true}				OPTIONAL,	-- Need OR

rach-SkipSCG-r14				RACH-Skip-r14					OPTIONAL	-- Need OR

]]

}

MobilityControlInfoV2X-r14 ::=	SEQUENCE {

v2x-CommTxPoolExceptional-r14		SL-CommResourcePoolV2X-r14		OPTIONAL,		-- Need OR

v2x-CommRxPool-r14					SL-CommRxPoolListV2X-r14		OPTIONAL,		-- Need OR

v2x-CommSyncConfig-r14				SL-SyncConfigListV2X-r14		OPTIONAL,		-- Need OR

cbr-MobilityTxConfigList-r14		SL-CBR-CommonTxConfigList-r14	OPTIONAL		-- Need OR

}

CarrierBandwidthEUTRA ::=			SEQUENCE {

dl-Bandwidth						ENUMERATED {

n6, n15, n25, n50, n75, n100, spare10,

spare9, spare8, spare7, spare6, spare5,

spare4, spare3, spare2, spare1},

ul-Bandwidth						ENUMERATED {

n6, n15, n25, n50, n75, n100, spare10,

spare9, spare8, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}	OPTIONAL -- Need OP

}

CarrierFreqEUTRA ::=				SEQUENCE {

dl-CarrierFreq						ARFCN-ValueEUTRA,

ul-CarrierFreq						ARFCN-ValueEUTRA				OPTIONAL	-- Cond FDD

}

CarrierFreqEUTRA-v9e0 ::=			SEQUENCE {

dl-CarrierFreq-v9e0					ARFCN-ValueEUTRA-r9,

ul-CarrierFreq-v9e0					ARFCN-ValueEUTRA-r9			OPTIONAL	-- Cond FDD

}

DAPS-Config-r16 ::=					SEQUENCE {

daps-PowerCoordinationInfo-r16		DAPS-PowerCoordinationInfo-r16	OPTIONAL,	-- Need ON

...

}

DAPS-PowerCoordinationInfo-r16 ::=	SEQUENCE {

p-DAPS-Source-r16					INTEGER (1..16),

p-DAPS-Target-r16					INTEGER (1..16),

powerControlMode-r16				INTEGER (1..2)

}

RACH-Skip-r14 ::=					SEQUENCE {

targetTA-r14					CHOICE {

ta0-r14							NULL,

mcg-PTAG-r14						NULL,

scg-PTAG-r14						NULL,

mcg-STAG-r14					STAG-Id-r11,

scg-STAG-r14					STAG-Id-r11

},

ul-ConfigInfo-r14				SEQUENCE {

numberOfConfUL-Processes-r14			INTEGER (1..8),

ul-SchedInterval-r14			ENUMERATED {sf2, sf5, sf10},

ul-StartSubframe-r14			INTEGER (0..9),

ul-Grant-r14					BIT STRING (SIZE (16))

}																OPTIONAL	-- Need OR

}

-- ASN1STOP

MobilityControlInfo field descriptions

additionalSpectrumEmission For a UE with no SCells configured for UL in the same band as the PCell, the UE shall apply the value for the PCell instead of the corresponding value from SystemInformationBlockType2 or SystemInformationBlockType1. For a UE with SCell(s) configured for UL in the same band as the PCell, the UE shall, in case all SCells configured for UL in that band are released after handover completion, apply the value for the PCell instead of the corresponding value from SystemInformationBlockType2 or SystemInformationBlockType1. The UE requirements related to IE AdditionalSpectrumEmission are defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs and TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs.

carrierBandwidth Provides the parameters Downlink bandwidth, and Uplink bandwidth, see TS 36.101 [42].

carrierFreq Provides the EARFCN to be used by the UE in the target cell.

cbr-MobilityTxConfigList Indicates the list of CBR ranges and the list of PSSCH transmission parameter configurations available to configure congestion control to the UE for V2X sidelink communication during handover.

cipheringAlgorithmSCG Indicates the ciphering algorithm to be used for SCG DRBs. E-UTRAN includes the field upon SCG change when one or more SCG DRBs are configured. Otherwise E-UTRAN does not include the field.

dl-Bandwidth Parameter: Downlink bandwidth, see TS 36.101 [42].

drb-ContinueROHC This field indicates whether to continue or reset, for this handover, the header compression protocol context for the RLC UM bearers configured with the header compression protocol. Presence of the field indicates that the header compression protocol context continues while absence indicates that the header compression protocol context is reset. E-UTRAN includes the field only in case of a handover within the same eNB. This field does not apply to any configured DAPS bearers.

handoverWithoutWT-Change Indicates whether UE performs handover where LWA configuration is retained with the same WT If sendEndMarker is configured, the LWA end-marker for PDCP key change indication is used as defined in [8]. If value keepLWA-Config is configured, LWA end marker is not used and UE shall only retain the LWA configuration.

makeBeforeBreak Indicates that the UE shall continue uplink transmission/ downlink reception with the source cell(s) before performing the first transmission through PRACH to the target intra-frequency PCell, or performing initial PUSCH transmission to the target intra-frequency PCell while rach-Skip is configured.

makeBeforeBreakSCG Indicates that the UE shall continue uplink transmission/ downlink reception with the source cell(s) before performing the first transmission through PRACH to the target intra-frequency PSCell, or performing initial PUSCH transmission to the target intra-frequency PSCell while rach-SkipSCG is configured.

mib-RepetitionStatus Indicates whether additional MIB repetition is enabled in the target cell or not. Value TRUE indicates additional MIB repetition is enabled in the target cell. Value FALSE indicates additional MIB repetition is not enabled in the target cell. The absence of this field indicates additional MIB repetition may or may not be enabled in the target cell. See 5.2.1.2 and TS 36.211 [21], clause 6.4.1. This field is applicable to BL UE or UE in CE.

mobilityControlInfoV2X Indicates the sidelink configurations of the target cell for V2X sidelink communication during handover.

numberOfConfUL-Processes The number of configured HARQ processes for preallocated uplink grant, see TS 36.321 [6], clause 5.20. This field is applicable if a UE is configured with asynchronous HARQ, otherwise it shall be ignored.

p-DAPS-Source Indicates the guaranteed power for the source PCell during a DAPS handover, as specified in TS 36.213 [23]. The value N corresponds to N-1 in TS 36.213 [23].

p-DAPS-Target Indicates the guaranteed power for the target PCell during a DAPS handover as specified in TS 36.213 [23], Table 5.1.4.2-1. The value N corresponds to N-1 in TS 36.213 [23].

powerControlMode Indicates the power control mode used in during a DAPS handover. Value 1 corresponds to DC power control mode 1 and value 2 indicates DC power control mode 2, as specified in TS 36.213 [23].

rach-ConfigDedicated The dedicated random access parameters. If absent the UE applies contention based random access as specified in TS 36.321 [6].

rach-Skip This field indicates whether random access procedure for the target PCell is skipped.

rach-SkipSCG This field indicates whether random access procedure for the target PSCell is skipped.

sameSFN-Indication This field indicates that the target cell has the same SFN as the source cell and that the BL UE or UE in CE is not required to acquire MasterInformationBlock in the target PCell during handover to obtain the SFN of the target cell, as specified in clause 5.3.5.4.

schedulingInfoSIB1-BR Indicates the index to the tables that define SystemInformationBlockType1-BR scheduling information. The tables are specified in TS 36.213 [23], Table 7.1.6-1 and Table 7.1.7.2.7-1. Value 0 means SystemInformationBlockType1-BR is not scheduled. If absent when sameSFN-Indication is present, UE assumes that SystemInformationBlockType1-BR scheduling information in target cell may be different from source cell.

t304 Timer T304 as described in clause 7.3. ms50 corresponds with 50 ms, ms100 corresponds with 100 ms and so on. EUTRAN includes extended value ms10000-v1310 only when UE supports CE.

t307 Timer T307 as described in clause 7.3. ms50 corresponds with 50 ms, ms100 corresponds with 100 ms and so on.

targetTA This field refers to the timing adjustment indication, see TS 36.213 [23], indicating the NTA value which the UE shall use for the target PTAG of handover or the target PSTAG of SCG change. ta0 corresponds to NTA=0. mcg-PTAG corresponds to the latest NTA value of the PTAG associated with MCG. scg-PTAG corresponds to the latest NTA value of the PTAG associated with SCG. mcg-STAG corresponds to the latest NTA value of a MCG STAG indicated by the STAG-Id. scg-STAG corresponds to the latest NTA value of a SCG STAG indicated by the STAG-Id.

ul-Bandwidth Parameter: Uplink bandwidth, see TS 36.101 [42], table 5.6-1. For TDD, the parameter is absent and it is equal to downlink bandwidth. If absent for FDD, apply the same value as applies for the downlink bandwidth.

ul-Grant Indicates the resources of the target PCell/PSCell to be used for the uplink transmission of PUSCH [23], clause 8.8.

ul-SchedInterval Indicates the scheduling interval in uplink, see TS 36.321 [6], clause 5.20. Value in number of sub-frames. Value sf2 corresponds to 2 subframes, sf5 corresponds to 5 subframes and so on.

ul-StartSubframe Indicates the subframe in which the UE may initiate the uplink transmission, see TS 36.321 [6], clause 5.20. Value 0 corresponds to subframe number 0, 1 correponds to subframe number 1 and so on. The subframe indicating a valid uplink grant according to the calculation of UL grant configured by ul-StartSubframe and ul-SchedInterval, see TS 36.321 [6], clause 5.20, is the same across all radio frames.

v2x-CommRxPool Indicates reception pools for receiving V2X sidelink communication during handover.

v2x-CommSyncConfig Indicates synchronization configurations for performing V2X sidelink communication during handover.

v2x-CommTxPoolExceptional Indicates the transmission resources by which the UE is allowed to transmit V2X sidelink communication during handover.

| Conditional presence | Explanation |
| --- | --- |
| FDD | The field is mandatory with default value (the default duplex distance defined for the concerned band, as specified in TS 36.101 [42]) in case of "FDD"; otherwise the field is not present. |
| HO | This field is optionally present, need OP, in case of handover within E-UTRA when the fullConfig is not included; otherwise the field is not present. |
| HO-SFNsynced | This field is optionally present, need OP, in case of source E-UTRA and target E-UTRA cells are SFN synchronised. |
| HO-toEUTRA | The field is mandatory present in case of inter-RAT handover to E-UTRA; otherwise the field is optionally present, need ON. |
| HO-toEUTRA2 | The field is absent if carrierFreq-v9e0 is present. Otherwise it is mandatory present in case of inter-RAT handover to E-UTRA and optionally present, need ON, in all other cases. |
| NotFullConfigHO | This field is optionally present, Need OR, in case of handover within E-UTRA when the fullConfig is not included in the RRCConnectionReconfiguration message. Otherwise the field is not present. |
| SCGEst | This field is mandatory present in case of SCG establishment; otherwise the field is optionally present, need ON. |

#### –	MobilityParametersCDMA2000 (1xRTT)

The MobilityParametersCDMA2000 contains the parameters provided to the UE for handover and (enhanced) CSFB to 1xRTT support, as defined in C.S0097 [53].

MobilityParametersCDMA2000 information element

-- ASN1START

MobilityParametersCDMA2000 ::=			OCTET STRING

-- ASN1STOP

#### –	MobilityStateParameters

The IE MobilityStateParameters contains parameters to determine UE mobility state.

MobilityStateParameters information element

-- ASN1START

MobilityStateParameters ::=			SEQUENCE {

t-Evaluation						ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

t-HystNormal						ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

n-CellChangeMedium					INTEGER (1..16),

n-CellChangeHigh					INTEGER (1..16)

}

-- ASN1STOP

MobilityStateParameters field descriptions

n-CellChangeHigh The number of cell changes to enter high mobility state. Corresponds to NCR_H in TS 36.304 [4].

n-CellChangeMedium The number of cell changes to enter medium mobility state. Corresponds to NCR_M in TS 36.304 [4].

t-Evaluation The duration for evaluating criteria to enter mobility states. Corresponds to TCRmax in TS 36.304 [4]. Value in seconds, s30 corresponds to 30 s and so on.

t-HystNormal The additional duration for evaluating criteria to enter normal mobility state. Corresponds to TCRmaxHyst in TS 36.304 [4]. Value in seconds, s30 corresponds to 30 s and so on.

#### –	MultiBandInfoList

MultiBandInfoList information element

-- ASN1START

MultiBandInfoList ::=	SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicator

MultiBandInfoList-v9e0 ::=	SEQUENCE (SIZE (1..maxMultiBands)) OF MultiBandInfo-v9e0

MultiBandInfoList-v10j0 ::=	SEQUENCE (SIZE (1..maxMultiBands)) OF NS-PmaxList-r10

MultiBandInfoList-v10l0 ::=	SEQUENCE (SIZE (1..maxMultiBands)) OF NS-PmaxList-v10l0

MultiBandInfoList-r11 ::=	SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicator-r11

MultiBandInfo-v9e0 ::=		SEQUENCE {

freqBandIndicator-v9e0				FreqBandIndicator-v9e0		OPTIONAL	-- Need OP

}

-- ASN1STOP

#### –	MultiFrequencyBandListNR

The IE MultiFrequencyBandListNR is used to configure a list of one or multiple NR frequency bands.

MultiFrequencyBandListNR information element

-- ASN1START

MultiFrequencyBandListNR-r15 ::=		SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15

-- ASN1STOP

#### –	NS-PmaxList

The IE NS-PmaxList concerns a list of additionalPmax and additionalSpectrumEmission, as defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs and TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs, for a given frequency band. E-UTRAN does not include the same value of additionalSpectrumEmission in SystemInformationBlockType2 within this list.

NS-PmaxList information element

-- ASN1START

NS-PmaxList-r10 ::=				SEQUENCE (SIZE (1..maxNS-Pmax-r10)) OF NS-PmaxValue-r10

NS-PmaxList-v10l0 ::=			SEQUENCE (SIZE (1..maxNS-Pmax-r10)) OF NS-PmaxValue-v10l0

NS-PmaxValue-r10 ::=			SEQUENCE {

additionalPmax-r10					P-Max							OPTIONAL,	-- Need OP

additionalSpectrumEmission			AdditionalSpectrumEmission

}

NS-PmaxValue-v10l0 ::=			SEQUENCE {

additionalSpectrumEmission-v10l0	AdditionalSpectrumEmission-v10l0	OPTIONAL	-- Need OP

}

-- ASN1STOP

#### –	NS-PmaxListNR

The IE NS-PmaxListNR concerns a list of additionalPmax and additionalSpectrumEmission, as defined in TS 38.101 [85], table 6.2.3-1 for a given frequency band.

NS-PmaxListNR information element

-- ASN1START

NS-PmaxListNR-r15 ::=				SEQUENCE (SIZE (1..8)) OF NS-PmaxValueNR-r15

NS-PmaxValueNR-r15 ::=			SEQUENCE {

additionalPmaxNR-r15					P-MaxNR-r15					OPTIONAL,	-- Need ON

additionalSpectrumEmissionNR-r15		AdditionalSpectrumEmissionNR-r15

}

-- ASN1STOP

#### –	PhysCellId

The IE PhysCellId is used to indicate the physical layer identity of the cell, as defined in TS 36.211 [21].

PhysCellId information element

-- ASN1START

PhysCellId ::=						INTEGER (0..503)

-- ASN1STOP

#### –	PhysCellIdCDMA2000

The IE PhysCellIdCDMA2000 identifies the PNOffset that represents the "Physical cell identity" in CDMA2000.

PhysCellIdCDMA2000 information element

-- ASN1START

PhysCellIdCDMA2000 ::=			INTEGER (0..maxPNOffset)

-- ASN1STOP

#### –	PhysCellIdGERAN

The IE PhysCellIdGERAN contains the Base Station Identity Code (BSIC).

PhysCellIdGERAN information element

-- ASN1START

PhysCellIdGERAN ::=				SEQUENCE {

networkColourCode					BIT STRING (SIZE (3)),

baseStationColourCode				BIT STRING (SIZE (3))

}

-- ASN1STOP

PhysCellIdGERAN field descriptions

baseStationColourCode Base station Colour Code as defined in TS 23.003 [27].

networkColourCode Network Colour Code as defined in TS 23.003 [27].

#### –	PhysCellIdNR

The IE PhysCellIdNR indicates the physical layer identity (PCI) of an NR cell.

PhysCellIdNR information element

-- ASN1START

PhysCellIdNR-r15 ::=			INTEGER (0.. 1007)

-- ASN1STOP

#### –	PhysCellIdRange

The IE PhysCellIdRange is used to encode either a single or a range of physical cell identities. The range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range. For fields comprising multiple occurrences of PhysCellIdRange, E-UTRAN may configure overlapping ranges of physical cell identities.

PhysCellIdRange information element

-- ASN1START

PhysCellIdRange ::=				SEQUENCE {

start							PhysCellId,

range							ENUMERATED {

n4, n8, n12, n16, n24, n32, n48, n64, n84,

n96, n128, n168, n252, n504, spare2,

spare1}					OPTIONAL	-- Need OP

}

-- ASN1STOP

PhysCellIdRange field descriptions

range Indicates the number of physical cell identities in the range (including start). Value n4 corresponds with 4, n8 corresponds with 8 and so on. The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies.

start Indicates the lowest physical cell identity in the range.

#### –	PhysCellIdRangeNR

The IE PhysCellIdRangeNR is used to encode either a single or a range of physical layer identities of NR cells. The range is encoded by using a start value and by indicating the number of consecutive physical layer identities (including start) in the range. For fields comprising multiple occurrences of PhysCellIdRangeNR, E-UTRAN may configure overlapping ranges of physical layer identities.

PhysCellIdRangeNR information element

-- ASN1START

PhysCellIdRangeNR-r16 ::=		SEQUENCE {

start							PhysCellIdNR-r15,

range							ENUMERATED {

n4, n8, n12, n16, n24, n32, n48, n64, n84,

n96, n128, n168, n252, n504, n1008,

spare1}					OPTIONAL	-- Need OP

}

-- ASN1STOP

PhysCellIdRangeNR field descriptions

range Indicates the number of physical layer identities in the range (including start). Value n4 corresponds with 4, n8 corresponds with 8 and so on. The UE shall apply value 1 in case the field is absent, in which case only the physical layer identity value indicated by start applies.

start Indicates the lowest physical layer identity in the range.

#### –	PhysCellIdRangeUTRA-FDDList

The IE PhysCellIdRangeUTRA-FDDList is used to encode one or more of PhysCellIdRangeUTRA-FDD. While the IE PhysCellIdRangeUTRA-FDD is used to encode either a single physical layer identity or a range of physical layer identities, i.e. primary scrambling codes. Each range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range.

PhysCellIdRangeUTRA-FDDList information element

-- ASN1START

PhysCellIdRangeUTRA-FDDList-r9::=		SEQUENCE (SIZE (1..maxPhysCellIdRange-r9)) OF PhysCellIdRangeUTRA-FDD-r9

PhysCellIdRangeUTRA-FDD-r9 ::=			SEQUENCE {

start-r9							PhysCellIdUTRA-FDD,

range-r9							INTEGER (2..512)				OPTIONAL	-- Need OP

}

-- ASN1STOP

PhysCellIdRangeUTRA-FDDList field descriptions

range Indicates the number of primary scrambling codes in the range (including start). The UE shall apply value 1 in case the field is absent, in which case only the primary scrambling code value indicated by start applies.

start Indicates the lowest primary scrambling code in the range.

#### –	PhysCellIdUTRA-FDD

The IE PhysCellIdUTRA-FDD is used to indicate the physical layer identity of the cell, i.e. the primary scrambling code, as defined in TS 25.331 [19].

PhysCellIdUTRA-FDD information element

-- ASN1START

PhysCellIdUTRA-FDD ::=				INTEGER (0..511)

-- ASN1STOP

#### –	PhysCellIdUTRA-TDD

The IE PhysCellIdUTRA-TDD is used to indicate the physical layer identity of the cell, i.e. the cell parameters ID (TDD), as specified in TS 25.331 [19]. Also corresponds to the Initial Cell Parameter Assignment in TS 25.223 [46].

PhysCellIdUTRA-TDD information element

-- ASN1START

PhysCellIdUTRA-TDD ::=				INTEGER (0..127)

-- ASN1STOP

#### –	PLMN-Identity

The IE PLMN-Identity identifies a Public Land Mobile Network. Further information regarding how to set the IE are specified in TS 23.003 [27].

PLMN-Identity information element

-- ASN1START

PLMN-Identity ::=					SEQUENCE {

mcc									MCC					OPTIONAL,					-- Cond MCC

mnc									MNC

}

MCC ::=								SEQUENCE (SIZE (3)) OF

MCC-MNC-Digit

MNC ::=								SEQUENCE (SIZE (2..3)) OF

MCC-MNC-Digit

MCC-MNC-Digit ::=					INTEGER (0..9)

-- ASN1STOP

PLMN-Identity field descriptions

mcc The first element contains the first MCC digit, the second element the second MCC digit and so on. If the field is absent, it takes the same value as the mcc of the immediately preceding IE PLMN-Identity. See TS 23.003 [27].

mnc The first element contains the first MNC digit, the second element the second MNC digit and so on. See TS 23.003 [27].

| Conditional presence | Explanation |
| --- | --- |
| MCC | This IE is mandatory when PLMN-Identity is included in CellGlobalIdEUTRA, in CellGlobalIdUTRA, in CellGlobalIdGERAN or in RegisteredMME. This IE is also mandatory in the first occurrence of the IE PLMN-Identity within the IE PLMN-IdentityList. Otherwise it is optional, need OP. |

#### –	PLMN-IdentityList3

Includes a list of PLMN identities.

PLMN-IdentityList3 information element

-- ASN1START

PLMN-IdentityList3-r11 ::=				SEQUENCE (SIZE (1..16)) OF PLMN-Identity

-- ASN1STOP

#### –	PmaxNR

The IE PmaxNR concerns a list of additionalPmax and additionalSpectrumEmission, as defined in TS 38.101 [85], table 6.2.3-1 for a given frequency band.

PmaxNR information element

-- ASN1START

P-MaxNR-r15 ::=				INTEGER (-30..33)

-- ASN1STOP

#### –	PreRegistrationInfoHRPD

-- ASN1START

PreRegistrationInfoHRPD ::=			SEQUENCE {

preRegistrationAllowed				BOOLEAN,

preRegistrationZoneId				PreRegistrationZoneIdHRPD	OPTIONAL, -- cond PreRegAllowed

secondaryPreRegistrationZoneIdList	SecondaryPreRegistrationZoneIdListHRPD	OPTIONAL -- Need OR

}

SecondaryPreRegistrationZoneIdListHRPD ::=	SEQUENCE (SIZE (1..2)) OF PreRegistrationZoneIdHRPD

PreRegistrationZoneIdHRPD ::=			INTEGER (0..255)

-- ASN1STOP

PreRegistrationInfoHRPD field descriptions

preRegistrationAllowed TRUE indicates that a UE shall perform a CDMA2000 HRPD pre-registration if the UE does not have a valid / current pre-registration. FALSE indicates that the UE is not allowed to perform CDMA2000 HRPD pre-registration in the current cell.

preRegistrationZoneID ColorCode (see C.S0024 [26], C.S0087 [44]) of the CDMA2000 Reference Cell corresponding to the HRPD sector under the HRPD AN that is configured for this LTE cell. It is used to control when the UE should register or re-register.

secondaryPreRegistrationZoneIdList List of SecondaryColorCodes (see C.S0024 [26], C.S0087 [44]) of the CDMA2000 Reference Cell corresponding to the HRPD sector under the HRPD AN that is configured for this LTE cell. They are used to control when the UE should re-register.

| Conditional presence | Explanation |
| --- | --- |
| PreRegAllowed | The field is mandatory in case the preRegistrationAllowed is set to true. Otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	Q-QualMin

The IE Q-QualMin is used to indicate for cell selection/ re-selection the required minimum received RSRQ level in the (E-UTRA) cell. Corresponds to parameter Qqualmin in TS 36.304 [4]. Actual value Qqualmin = field value [dB].

Q-QualMin information element

-- ASN1START

Q-QualMin-r9 ::=					INTEGER (-34..-3)

-- ASN1STOP

#### –	Q-RxLevMin

The IE Q-RxLevMin is used to indicate for cell selection/ re-selection the required minimum received RSRP level in the (E-UTRA) cell. Corresponds to parameter Qrxlevmin in TS 36.304 [4]. Actual value Qrxlevmin = field value * 2 [dBm].

Q-RxLevMin information element

-- ASN1START

Q-RxLevMin ::=						INTEGER (-70..-22)

-- ASN1STOP

#### –	Q-OffsetRange

The IE Q-OffsetRange is used to indicate a cell, CSI-RS resource or frequency specific offset to be applied when evaluating candidates for cell re-selection or when evaluating triggering conditions for measurement reporting. The value in dB. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

Q-OffsetRange information element

-- ASN1START

Q-OffsetRange ::=						ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- ASN1STOP

#### –	Q-OffsetRangeInterRAT

The IE Q-OffsetRangeInterRAT is used to indicate a frequency specific offset to be applied when evaluating triggering conditions for measurement reporting. The value in dB.

Q-OffsetRangeInterRAT information element

-- ASN1START

Q-OffsetRangeInterRAT ::=					INTEGER (-15..15)

-- ASN1STOP

#### –	ReselectionThreshold

The IE ReselectionThreshold is used to indicate an Rx level threshold for cell reselection. Actual value of threshold = field value * 2 [dB].

ReselectionThreshold information element

-- ASN1START

ReselectionThreshold ::=				INTEGER (0..31)

-- ASN1STOP

#### –	ReselectionThresholdQ

The IE ReselectionThresholdQ is used to indicate a quality level threshold for cell reselection. Actual value of threshold = field value [dB].

ReselectionThresholdQ information element

-- ASN1START

ReselectionThresholdQ-r9 ::=			INTEGER (0..31)

-- ASN1STOP

#### –	RSS-ConfigCarrierInfo

The IE RSS-ConfigCarrierInfo contains RSS configuration for a carrier.

RSS-ConfigCarrierInfo information element

-- ASN1START

RSS-ConfigCarrierInfo-r16 ::=	SEQUENCE {

narrowbandIndex-r16				BIT STRING (SIZE (1..maxAvailNarrowBands-1-r16)),

timeOffsetGranularity-r16		ENUMERATED {g1, g2, g4, g8, g16, g32, g64, g128}

}

-- ASN1STOP

RSS-ConfigCarrierInfo field descriptions

narrowbandIndex Bitmap containing narrowbands used for RSS deployment in the carrier. Narrowbands including central 6 PRBs are excluded from the bitmap. The RSS Cell Frequency Location of a specific cell is determined according to IRSS = PCID MOD (3NNB) where IRSS is the index of possible RSS frequency locations starting with the lowest location and NNB is the number of narrowbands, determined from narrowbandIndex, such that there are three non-overlapping RSS locations in each narrowband.

timeOffsetGranularity RSS Time Offset granularity (GRSS). Value g1 corresponds to 1 frame, value g2 corresponds to 2 frames, and so on. Only the following values of GRSS are applicable depending on the serving cell RSS periodicity (PRSS) given by parameter periodicity in ce-RSS-Config-r15: GRSS = {1, 2, 4, 8, 16} frames for PRSS = 160 ms GRSS = {1, 2, 4, 8, 16, 32} frames for PRSS = 320 ms GRSS = {2, 4, 8, 16, 32, 64} frames for PRSS = 640 ms GRSS = {4, 8, 16, 32, 64, 128} frames for PRSS = 1280 ms. The actual RSS time offset of a specific cell (ORSS, see TS 36.211 [21] subclause 6.11.3.2) in SFN radio frames is given by (XRSS × GRSS) + ΔRSS where: - RSS Time Offset of a specific cell (XRSS) is determined based on its PCID using XRSS = FLOOR (PCID/(3 NNB)) modulo MRSS, and distributed across MRSS time locations per PRSS such that MRSS = PRSS /(10 × GRSS); and - ΔRSS is calculated by using the serving cell XRSS (i.e., based on serving cell PCID and parameters given in ce-RSS-Config-r15) such that serving cell ORSS = (XRSS × GRSS) + ΔRSS.

#### –	RSS-MeasPowerBias

The IE RSS-MeasPowerBias indicates power bias in dB relative to Qoffset of neighbour cell CRS. Value dB-6 corresponds to -6 dB, value dB-3 corresponds to -3 dB and so on. Value rssNotUsed indicates measurement based on RSS is not applicable for the corresponding neighbour cell.

RSS-MeasPowerBias information element

-- ASN1START

RSS-MeasPowerBias-r16 ::=	ENUMERATED {dB-6, dB-3, dB0, dB3, dB6, dB9, dB12, rssNotUsed}

-- ASN1STOP

#### –	SCellIndex

The IE SCellIndex concerns a short identity, used to identify an SCell.

SCellIndex information element

-- ASN1START

SCellIndex-r10 ::=						INTEGER (1..7)

SCellIndex-r13 ::=						INTEGER (1..31)

-- ASN1STOP

#### –	ServCellIndex

The IE ServCellIndex concerns a short identity, used to identify a serving cell (i.e. the PCell or an SCell). Value 0 applies for the PCell, while the SCellIndex that has previously been assigned applies for SCells.

ServCellIndex information element

-- ASN1START

ServCellIndex-r10 ::=					INTEGER (0..7)

ServCellIndex-r13 ::=				INTEGER (0..31)

-- ASN1STOP

#### –	SpeedStateScaleFactors

The IE SpeedStateScaleFactors concerns factors, to be applied when the UE is in medium or high speed state, used for scaling a mobility control related parameter.

SpeedStateScaleFactors information element

-- ASN1START

SpeedStateScaleFactors ::=			SEQUENCE {

sf-Medium							ENUMERATED {oDot25, oDot5, oDot75, lDot0},

sf-High								ENUMERATED {oDot25, oDot5, oDot75, lDot0}

}

-- ASN1STOP

SpeedStateScaleFactors field descriptions

sf-High The concerned mobility control related parameter is multiplied with this factor if the UE is in High Mobility state as defined in TS 36.304 [4]. Value oDot25 corresponds to 0.25, oDot5 corresponds to 0.5, oDot75 corresponds to 0.75 and so on.

sf-Medium The concerned mobility control related parameter is multiplied with this factor if the UE is in Medium Mobility state as defined in TS 36.304 [4]. Value oDot25 corresponds to 0.25, oDot5 corresponds to 0.5, oDot75 corresponds to 0.75 and so on.

#### –	SystemInfoListGERAN

The IE SystemInfoListGERAN contains system information of a GERAN cell.

SystemInfoListGERAN information element

-- ASN1START

SystemInfoListGERAN ::=				SEQUENCE (SIZE (1..maxGERAN-SI)) OF

OCTET STRING (SIZE (1..23))

-- ASN1STOP

SystemInfoListGERAN field descriptions

SystemInfoListGERAN Each OCTET STRING contains one System Information (SI) message as defined in TS 44.018 [45], table 9.1.1, excluding the L2 Pseudo Length, the RR management Protocol Discriminator and the Skip Indicator or a complete Packet System Information (PSI) message as defined in TS 44.060 [36], table 11.2.1.

#### –	SystemTimeInfoCDMA2000

The IE SystemTimeInfoCDMA2000 informs the UE about the absolute time in the current cell. The UE uses this absolute time knowledge to derive the CDMA2000 Physical cell identity, expressed as PNOffset, of neighbour CDMA2000 cells.

NOTE:	The UE needs the CDMA2000 system time with a certain level of accuracy for performing measurements as well as for communicating with the CDMA2000 network (HRPD or 1xRTT).

SystemTimeInfoCDMA2000 information element

-- ASN1START

SystemTimeInfoCDMA2000 ::=			SEQUENCE {

cdma-EUTRA-Synchronisation			BOOLEAN,

cdma-SystemTime						CHOICE {

synchronousSystemTime				BIT STRING (SIZE (39)),

asynchronousSystemTime				BIT STRING (SIZE (49))

}

}

-- ASN1STOP

SystemTimeInfoCDMA2000 field descriptions

asynchronousSystemTime The CDMA2000 system time corresponding to the SFN boundary at or after the ending boundary of the SI-Window in which SystemInformationBlockType8 is transmitted. E-UTRAN includes this field if the E-UTRA frame boundary is not aligned to the start of CDMA2000 system time. This field size is 49 bits and the unit is 8 CDMA chips based on 1.2288 Mcps.

cdma-EUTRA-Synchronisation TRUE indicates that there is no drift in the timing between E-UTRA and CDMA2000. FALSE indicates that the timing between E-UTRA and CDMA2000 can drift. NOTE 1

synchronousSystemTime CDMA2000 system time corresponding to the SFN boundary at or after the ending boundary of the SI-window in which SystemInformationBlockType8 is transmitted. E-UTRAN includes this field if the E-UTRA frame boundary is aligned to the start of CDMA2000 system time. This field size is 39 bits and the unit is 10 ms based on a 1.2288 Mcps chip rate.

NOTE 1:	The following table shows the recommended combinations of the cdma-EUTRA-Synchronisation field and the choice of cdma-SystemTime included by E-UTRAN for FDD and TDD:

| FDD/TDD | cdma-EUTRA-Synchronisation | synchronousSystemTime | asynchronousSystemTime |
| --- | --- | --- | --- |
| FDD | FALSE | Not Recommended | Recommended |
| FDD | TRUE | Recommended | Recommended |
| TDD | FALSE | Not Recommended | Recommended |
| TDD | TRUE | Recommended | Recommended |

#### –	ThresholdNR

The IE ThresholdNR and IE ThresholdListNR contain thresholds for NR related inter-RAT measurements.

ThresholdNR information element

-- ASN1START

ThresholdNR-r15 ::=				CHOICE{

nr-RSRP-r15							RSRP-RangeNR-r15,

nr-RSRQ-r15							RSRQ-RangeNR-r15,

nr-SINR-r15							RS-SINR-RangeNR-r15

}

ThresholdListNR-r15 ::=				SEQUENCE{

nr-RSRP-r15							RSRP-RangeNR-r15				OPTIONAL,	-- Need OR

nr-RSRQ-r15							RSRQ-RangeNR-r15				OPTIONAL,	-- Need OR

nr-SINR-r15							RS-SINR-RangeNR-r15			OPTIONAL	-- Need OR

}

-- ASN1STOP

#### –	TrackingAreaCode

The IE TrackingAreaCode is used to identify a tracking area within the scope of a PLMN, see TS 24.301 [35].

TrackingAreaCode information element

-- ASN1START

TrackingAreaCode ::=				BIT STRING (SIZE (16))

TrackingAreaCode-5GC-r15 ::=		BIT STRING (SIZE (24))

-- ASN1STOP

#### –	T-Reselection

The IE T-Reselection concerns the cell reselection timer TreselectionRAT for E-UTRA, UTRA, GERAN or CDMA2000. Value in seconds. For value 0, behaviour as specified in 7.3.2 applies.

T-Reselection information element

-- ASN1START

T-Reselection ::=					INTEGER (0..7)

-- ASN1STOP

#### –	T-ReselectionEUTRA-CE

The IE T-ReselectionEUTRA-CE concerns the cell reselection timer TreselectionEUTRA_CE as specified in TS 36.304 [4]. Value in seconds. For value 0, behaviour as specified in 7.3.2 applies.

T-ReselectionEUTRA-CE information element

-- ASN1START

T-ReselectionEUTRA-CE-r13 ::=				INTEGER (0..15)

-- ASN1STOP

### 6.3.5	Measurement information elements

#### –	AllowedMeasBandwidth

The IE AllowedMeasBandwidth is used to indicate the maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 [47]. The values mbw6, mbw15, mbw25, mbw50, mbw75, mbw100 indicate 6, 15, 25, 50, 75 and 100 resource blocks respectively.

AllowedMeasBandwidth information element

-- ASN1START

AllowedMeasBandwidth ::=				ENUMERATED {mbw6, mbw15, mbw25, mbw50, mbw75, mbw100}

-- ASN1STOP

#### –	BT-NameList

The IE BT-NameList is used to indicate the names of the Bluetooth beacon which the UE is configured to measure.

BT-NameList information element

-- ASN1START

BT-NameListConfig-r15 ::=		CHOICE{

release						NULL,

setup						BT-NameList-r15

}

BT-NameList-r15 ::=			SEQUENCE (SIZE (1..maxBT-Name-r15)) OF BT-Name-r15

BT-Name-r15 ::=		OCTET STRING (SIZE (1..248))

-- ASN1STOP

BT-NameList field descriptions

bt-Name If configured, the UE only performs Bluetooth measurements according to the names identified. For each name, it refers to LOCAL NAME defined in Bluetooth specification [93].

#### –	CSI-RSRP-Range

The IE CSI-RSRP-Range specifies the value range used in CSI-RSRP measurements and thresholds. Integer value for CSI-RSRP measurements according to mapping table in TS 36.133 [16].

CSI-RSRP-Range information element

-- ASN1START

CSI-RSRP-Range-r12 ::=						INTEGER(0..97)

-- ASN1STOP

#### –	Hysteresis

The IE Hysteresis is a parameter used within the entry and leave condition of an event triggered reporting condition. The actual value is field value * 0.5 dB, except if included in reportConfigEUTRA and associated to eventV1 or eventV2. If included in reportConfigEUTRA and associated to eventV1 or eventV2, the actual value is field value divided by 100.

Hysteresis information element

-- ASN1START

Hysteresis ::=							INTEGER (0..30)

-- ASN1STOP

#### –	LocationInfo

The IE LocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

LocationInfo information element

-- ASN1START

LocationInfo-r10 ::=	SEQUENCE {

locationCoordinates-r10					CHOICE {

ellipsoid-Point-r10						OCTET STRING,

ellipsoidPointWithAltitude-r10			OCTET STRING,

...,

ellipsoidPointWithUncertaintyCircle-r11					OCTET STRING,

ellipsoidPointWithUncertaintyEllipse-r11				OCTET STRING,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid-r11	OCTET STRING,

ellipsoidArc-r11										OCTET STRING,

polygon-r11												OCTET STRING

},

horizontalVelocity-r10					OCTET STRING				OPTIONAL,

gnss-TOD-msec-r10						OCTET STRING				OPTIONAL,

...,

[[	verticalVelocityInfo-r15		CHOICE {

verticalVelocity-r15				OCTET STRING,

verticalVelocityAndUncertainty-r15	OCTET STRING

}			OPTIONAL

]]

}

-- ASN1STOP

Editor’s Note: Agreement: RAN2 assumes that TA information (FFS what) reporting by the UE on network enabling will be needed in IoT NTN. Expect RAN1 need to progress on this, and can maybe reuse NR NTN progress. FFS in which message this is provided.

LocationInfo field descriptions

ellipsoidArc Parameter EllipsoidArc defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

ellipsoid-Point Parameter Ellipsoid-Point defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

ellipsoidPointWithAltitude Parameter EllipsoidPointWithAltitude defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid Parameter EllipsoidPointWithAltitudeAndUncertaintyEllipsoid defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

ellipsoidPointWithUncertaintyCircle Parameter Ellipsoid-PointWithUncertaintyCircle defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

ellipsoidPointWithUncertaintyEllipse Parameter EllipsoidPointWithUncertaintyEllipse defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

gnss-TOD-msec Parameter Gnss-TOD-msec defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

horizontalVelocity Parameter HorizontalVelocity defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

polygon Parameter Polygon defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

verticalVelocityAndUncertainty Parameter verticalVelocityAndUncertainty corresponds to horizontalWithVerticalVelocityAndUncertainty defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

verticalVelocity Parameter verticalVelocity corresponds to horizontalWithVerticalVelocity defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

#### –	LogMeasResultListBT

The IE LogMeasResultListBT covers measured results for Bluetooth.

LogMeasResultListBT information element

-- ASN1START

LogMeasResultListBT-r15 ::=		SEQUENCE (SIZE (1..maxBT-IdReport-r15)) OF LogMeasResultBT-r15

LogMeasResultBT-r15 ::= SEQUENCE {

bt-Addr-r15						BIT STRING (SIZE (48)),

rssi-BT-r15						INTEGER (-128..127)			OPTIONAL,

...

}

-- ASN1STOP

LogMeasResultListBT field descriptions

bt-Addr This field indicates the Bluetooth public address of the Bluetooth beacon as defined in TS 36.355 [54].

rssi-BT This field provides the beacon received signal strength indicator (RSSI) in dBm as defined in TS 36.355 [54].

#### –	LogMeasResultListWLAN

The IE LogMeasResultListWLAN covers measured results for WLAN.

LogMeasResultListWLAN information element

-- ASN1START

LogMeasResultListWLAN-r15 ::=		SEQUENCE (SIZE (1..maxWLAN-Id-Report-r14)) OF LogMeasResultWLAN-r15

LogMeasResultWLAN-r15 ::=	SEQUENCE {

wlan-Identifiers-r15			WLAN-Identifiers-r12,

rssiWLAN-r15					WLAN-RSSI-Range-r13				OPTIONAL,

rtt-WLAN-r15					WLAN-RTT-r15					OPTIONAL,

...

}

-- ASN1STOP

LogMeasResultListWLAN field descriptions

rssiWLAN Measured WLAN RSSI result in dBm.

rtt-WLAN This field provides the measured round trip time between the target device and WLAN AP and optionally the accuracy expressed as the standard deviation of the delay. Units for each of these are 1000ns, 100ns, 10ns, 1ns, and 0.1ns as defined in TS 36.355 [54].

wlan-Identifiers Indicates the WLAN parameters used for identification of the WLAN for which the measurement results are applicable.

#### –	MaxRS-IndexCellQualNR

The IE MaxRS-IndexCellQualNR indicates the maximum number of RS indices to be considered/ averaged to derive the cell quality for RRM.

MaxRS-IndexCellQualNR information element

-- ASN1START

MaxRS-IndexCellQualNR-r15::=				INTEGER (1..maxRS-IndexCellQual-r15)

-- ASN1STOP

#### –	MBSFN-RSRQ-Range

The IE MBSFN-RSRQ-Range specifies the value range used in MBSFN RSRQ measurements. Integer value for MBSFN RSRQ measurements according to mapping table in TS 36.133 [16].

MBSFN-RSRQ-Range information element

-- ASN1START

MBSFN-RSRQ-Range-r12 ::=				INTEGER(0..31)

-- ASN1STOP

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

MeasConfig ::=						SEQUENCE {

-- Measurement objects

measObjectToRemoveList				MeasObjectToRemoveList				OPTIONAL,	-- Need ON

measObjectToAddModList				MeasObjectToAddModList				OPTIONAL,	-- Need ON

-- Reporting configurations

reportConfigToRemoveList			ReportConfigToRemoveList			OPTIONAL,	-- Need ON

reportConfigToAddModList			ReportConfigToAddModList			OPTIONAL,	-- Need ON

-- Measurement identities

measIdToRemoveList					MeasIdToRemoveList					OPTIONAL,	-- Need ON

measIdToAddModList					MeasIdToAddModList					OPTIONAL,	-- Need ON

-- Other parameters

quantityConfig						QuantityConfig						OPTIONAL,	-- Need ON

measGapConfig						MeasGapConfig						OPTIONAL,	-- Need ON

s-Measure							RSRP-Range							OPTIONAL,	-- Need ON

preRegistrationInfoHRPD				PreRegistrationInfoHRPD				OPTIONAL,	-- Need OP

speedStatePars			CHOICE {

release								NULL,

setup								SEQUENCE {

mobilityStateParameters				MobilityStateParameters,

timeToTrigger-SF					SpeedStateScaleFactors

}

}																		OPTIONAL,	-- Need ON

...,

[[	measObjectToAddModList-v9e0			MeasObjectToAddModList-v9e0		OPTIONAL	-- Need ON

]],

[[	allowInterruptions-r11				BOOLEAN							OPTIONAL	-- Need ON

]],

[[	measScaleFactor-r12			CHOICE {

release						NULL,

setup						MeasScaleFactor-r12

}																OPTIONAL,	-- Need ON

measIdToRemoveListExt-r12			MeasIdToRemoveListExt-r12		OPTIONAL,	-- Need ON

measIdToAddModListExt-r12			MeasIdToAddModListExt-r12		OPTIONAL,	-- Need ON

measRSRQ-OnAllSymbols-r12		BOOLEAN							OPTIONAL	-- Need ON

]],

[[

measObjectToRemoveListExt-r13		MeasObjectToRemoveListExt-r13	OPTIONAL,	-- Need ON

measObjectToAddModListExt-r13		MeasObjectToAddModListExt-r13	OPTIONAL,	-- Need ON

measIdToAddModList-v1310			MeasIdToAddModList-v1310		OPTIONAL,	-- Need ON

measIdToAddModListExt-v1310			MeasIdToAddModListExt-v1310		OPTIONAL	-- Need ON

]],

[[	measGapConfigPerCC-List-r14			MeasGapConfigPerCC-List-r14		OPTIONAL,	-- Need ON

measGapSharingConfig-r14			MeasGapSharingConfig-r14		OPTIONAL	-- Need ON

]],

[[	fr1-Gap-r15								BOOLEAN		OPTIONAL,	-- Need ON

mgta-r15								BOOLEAN						OPTIONAL	-- Need ON

]],

[[	measGapConfigDensePRS-r15			MeasGapConfigDensePRS-r15	OPTIONAL,	 -- Need ON

heightThreshRef-r15	CHOICE {

release					NULL,

setup					INTEGER (0..31)

}										OPTIONAL	--Need ON

]]

}

MeasIdToRemoveList ::=				SEQUENCE (SIZE (1..maxMeasId)) OF MeasId

MeasIdToRemoveListExt-r12 ::=		SEQUENCE (SIZE (1..maxMeasId)) OF MeasId-v1250

MeasObjectToRemoveList ::=			SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectId

MeasObjectToRemoveListExt-r13 ::=	SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectId-v1310

ReportConfigToRemoveList ::=		SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- ASN1STOP

MeasConfig field descriptions

allowInterruptions Value TRUE indicates that the UE is allowed to cause interruptions to serving cells when performing measurements of deactivated SCell carriers for measCycleSCell of less than 640ms, as specified in TS 36.133 [16]. E-UTRAN enables this field only when an SCell is configured.

fr1-Gap Indicates whether the gap is only applicable for measurements on FR1. E-UTRAN sets this field to TRUE only when the UE is configured with (NG)EN-DC.

heightThreshRef Reference height threshold for eventH1 and eventH2 in reportConfig. Value 0 refers to -420m, value 1 refers to –120m, and so on until value 30 refers to 8880m. The actual value is height in meters relative to sea level. Value 31 is reserved.

measGapConfig Used to setup and release measurement gaps. E-UTRAN includes either measGapConfig or measGapConfigPerCC-List, if any.

measGapConfigDensePRS Used to setup and release additional measurement gap pattern with dense PRS configuration as specified in TS 36.133 [16], Table 8.1.2.1-3. E-UTRAN configures this field only when UE indicates the preference of measurement gap configuration for dense PRS, i.e., measPRS-Offset-r15.

measGapConfigPerCC-List Used to setup and release serving cell sepecific measurement gaps. E-UTRAN includes either measGapConfig or measGapConfigPerCC-List, if any.

measGapSharingConfig Used to setup and release measurement gap sharing for intra- and inter-frequency measurement as specified in TS 36.133 [16].

measIdToAddModList List of measurement identities. Field measIdToAddModListExt includes additional measurement identities i.e. extends the size of the measurement identity list using the general principles specified in 5.1.2. If E-UTRAN includes measIdToAddModList-v1310 it includes the same number of entries, and listed in the same order, as in measIdToAddModList (i.e. without suffix). If E-UTRAN includes measIdToAddModListExt-v1310, it includes the same number of entries, and listed in the same order, as in measIdToAddModListExt-r12.

measIdToRemoveList List of measurement identities to remove. Field measIdToRemoveListExt includes additional measurement identities i.e. extends the size of the measurement identity list using the general principles specified in 5.1.2.

measObjectToAddModList If E-UTRAN includes measObjectToAddModList-v9e0 it includes the same number of entries, and listed in the same order, as in measObjectToAddModList (i.e. without suffix). Field measObjectToAddModListExt includes additional measurement object identities i.e. extends the size of the measurement object identity list using the general principles specified in 5.1.2.

measObjectToRemoveList List of measurement objects to remove. Field measObjectToRemoveListExt includes additional measurement object identities i.e. extends the size of the measurement object identity list using the general principles specified in 5.1.2.

measRSRQ-OnAllSymbols Value TRUE indicates that the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. If widebandRSRQ-Meas is enabled for the frequency in MeasObjectEUTRA, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols with wider bandwidth for concerned frequency in accordance with TS 36.214 [48].

measScaleFactor Even if reducedMeasPerformance is not included in any measObjectEUTRA or measObjectUTRA, E-UTRAN may configure this field. The UE behavior is specified in TS 36.133 [16].

mgta Indicates whether a timing advance value of 0.5 ms is applicable to the measurement gap configuration provided by E-UTRAN according to TS 38.133 [84]. E-UTRAN sets mgta to TRUE only when the UE is configured to perform NR measurements.

preRegistrationInfoHRPD The CDMA2000 HRPD Pre-Registration Information tells the UE if it should pre-register with the CDMA2000 HRPD network and identifies the Pre-registration zone to the UE.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-Measure PCell (or PSCell, if the UE is in NE-DC) quality threshold controlling whether or not the UE is required to perform measurements of intra-frequency, inter-frequency and inter-RAT neighbouring cells. Value "0" indicates to disable s-Measure.

timeToTrigger-SF The timeToTrigger in ReportConfigEUTRA and in ReportConfigInterRAT are multiplied with the scaling factor applicable for the UE's speed state.

#### –	MeasDS-Config

The IE MeasDS-Config specifies information applicable for discovery signals measurement.

MeasDS-Config information elements

-- ASN1START

MeasDS-Config-r12 ::=			CHOICE {

release							NULL,

setup							SEQUENCE {

dmtc-PeriodOffset-r12			CHOICE {

ms40-r12						INTEGER(0..39),

ms80-r12						INTEGER(0..79),

ms160-r12						INTEGER(0..159),

...

},

ds-OccasionDuration-r12		CHOICE {

durationFDD-r12				INTEGER(1..maxDS-Duration-r12),

durationTDD-r12				INTEGER(2..maxDS-Duration-r12)

},

measCSI-RS-ToRemoveList-r12	MeasCSI-RS-ToRemoveList-r12	OPTIONAL,	-- Need ON

measCSI-RS-ToAddModList-r12	MeasCSI-RS-ToAddModList-r12	OPTIONAL,	-- Need ON

...

}

}

MeasCSI-RS-ToRemoveList-r12 ::=	SEQUENCE (SIZE (1..maxCSI-RS-Meas-r12)) OF MeasCSI-RS-Id-r12

MeasCSI-RS-ToAddModList-r12 ::=	SEQUENCE (SIZE (1..maxCSI-RS-Meas-r12)) OF MeasCSI-RS-Config-r12

MeasCSI-RS-Id-r12 ::=			INTEGER (1..maxCSI-RS-Meas-r12)

MeasCSI-RS-Config-r12 ::=		SEQUENCE {

measCSI-RS-Id-r12				MeasCSI-RS-Id-r12,

physCellId-r12					INTEGER (0..503),

scramblingIdentity-r12			INTEGER (0..503),

resourceConfig-r12				INTEGER (0..31),

subframeOffset-r12				INTEGER (0..4),

csi-RS-IndividualOffset-r12		Q-OffsetRange,

...

}

-- ASN1STOP

MeasDS-Config field descriptions

csi-RS-IndividualOffset CSI-RS individual offset applicable to a specific CSI-RS resource. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

dmtc-PeriodOffset Indicates the discovery signals measurement timing configuration (DMTC) periodicity (dmtc-Periodicity) and offset (dmtc-Offset) for this frequency. For DMTC periodicity, value ms40 corresponds to 40ms, ms80 corresponds to 80ms and so on. The value of DMTC offset is in number of subframe(s). The duration of a DMTC occasion is 6ms.

ds-OccasionDuration Indicates the duration of discovery signal occasion for this frequency. Discovery signal occasion duration is common for all cells transmitting discovery signals on one frequency. If the carrierFreq in the measurement object is on an unlicensed band as specified in [42], the UE shall ignore the field ds-OccasionDuration for the carrier frequency and apply a value 1 instead.

measCSI-RS-ToAddModList List of CSI-RS resources to add/ modify in the CSI-RS resource list for discovery signals measurement.

measCSI-RS-ToRemoveList List of CSI-RS resources to remove from the CSI-RS resource list for discovery signals measurement.

physCellId Indicates the physical cell identity where UE may assume that the CSI-RS and the PSS/SSS/CRS corresponding to the indicated physical cell identity are quasi co-located with respect to average delay and doppler shift.

resourceConfig Parameter: CSI reference signal configuration, see TS 36.211 [21], tables 6.10.5.2-1 and 6.10.5.2-2. If the carrierFreq in the measurement object is on an unlicensed band as specified in TS 36.101 [42], E-UTRAN does not configure the values {0, 4, 5, 9, 10, 11, 18, 19}.

scramblingIdentity Parameter: Pseudo-random sequence generator parameter, , see TS 36.213 [23], clause 7.2.5.

subframeOffset Indicates the subframe offset between SSS of the cell indicated by physCellId and the CSI-RS resource in a discovery signal occasion. The field subframeOffset is set to values 0 if the carrierFreq in the measurement object is on an unlicensed band as specified in TS 36.101 [42].

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/ release of measurement gaps.

MeasGapConfig information element

-- ASN1START

MeasGapConfig ::=					CHOICE {

release								NULL,

setup								SEQUENCE {

gapOffset							CHOICE {

gp0									INTEGER (0..39),

gp1									INTEGER (0..79),

...,

gp2-r14								INTEGER (0..39),

gp3-r14								INTEGER (0..79),

gp-ncsg0-r14						INTEGER (0..39),

gp-ncsg1-r14						INTEGER (0..79),

gp-ncsg2-r14						INTEGER (0..39),

gp-ncsg3-r14						INTEGER (0..79),

gp-nonUniform1-r14					INTEGER (0..1279),

gp-nonUniform2-r14					INTEGER (0..2559),

gp-nonUniform3-r14					INTEGER (0..5119),

gp-nonUniform4-r14					INTEGER (0..10239),

gp4-r15								INTEGER (0..19),

gp5-r15								INTEGER (0..159),

gp6-r15								INTEGER (0..19),

gp7-r15								INTEGER (0..39),

gp8-r15								INTEGER (0..79),

gp9-r15								INTEGER (0..159),

gp10-r15							INTEGER (0..19),

gp11-r15							INTEGER (0..159)

}

}

}

-- ASN1STOP

MeasGapConfig field descriptions

gapOffset Value gapOffset of gp0 corresponds to gap offset of Gap Pattern Id "0" with MGRP = 40ms, gapOffset of gp1 corresponds to gap offset of Gap Pattern Id "1" with MGRP = 80ms, gapOffset of gp2 corresponds to gap offset of Gap Pattern Id "2" with MGRP = 40ms and MGL = 3ms, gapOffset of gp3 Gap Pattern Id "3" with MGRP = 80ms and MGL = 3ms, gapOffset of gp-ncsg0 corresponds to gap offset of NCSG Pattern Id "0" with VIRP = 40ms and ML = 4ms, gapOffset of gp-ncsg1 corresponds to gap offset of of NCSG Pattern Id "1" with VIRP = 80ms and ML = 4ms, gapOffset of gp-ncsg2 corresponds to gap offset of NCSG Pattern Id "2" with VIRP = 40ms and ML = 3ms, gapOffset of gp-ncsg3 corresponds to gap offset of of NCSG Pattern Id "3" with VIRP = 80ms and ML =3ms. gapOffset of gp-nonUniform1 corresponds to gap offset of non uniform gap pattern Id "1" with LMGRP = 1280ms, gapOffset of gp-nonUniform2 corresponds to gap offset of non uniform gap pattern Id "2" with LMGRP = 2560ms, gapOffset of gp-nonUniform3 corresponds to gap offset of non uniform gap pattern Id "3" with LMGRP = 5120ms, gapOffset of gp-nonUniform4 corresponds to gap offset of non uniform gap pattern Id "4" with LMGRP = 10240ms. Also used to specify the measurement gap pattern to be applied, as defined in TS 36.133 [16]. For Gap Patterns (including non-uniform gap patterns, but excluding NCSG patterns), E-UTRAN includes the same gapOffset value (gap pattern id and gap offset) for all serving cells that are configured with a Gap Pattern. For NCSG Patterns, E-UTRAN includes gapOffset value indicating VIRP and gap offset consistent with the Gap Pattern configuration (MGRP and gap offset). Value gapOffset of gp4, gp5,…,gp11 are corresponding to gap pattern with Gap Pattern ID 4, 5,..11 respectively, see TS 38.133 [84], Table 9.1.2-1. Value gp4, gp5, …, gp11 can be applied for (NG)EN-DC, see TS 38.133 [84], Table 9.1.2-2.

servCellId Identifies the serving cell for which measurement gap configuration is provided (setup) or deleted (release).

#### –	MeasGapConfigDensePRS

The IE MeasGapConfigDensePRS specifies the additional measurement gap pattern configuration for RSTD measurements with dense PRS configuration, see TS 36.133 [16], Table 8.1.2.1-3. Measurement gaps are configured according to applicability rules specified in 36.133 [16], Table 8.1.2.1-3.

MeasGapConfigDensePRS information element

-- ASN1START

MeasGapConfigDensePRS-r15 ::=	CHOICE {

release							NULL,

setup							SEQUENCE {

gapOffsetDensePRS-r15			CHOICE {

rstd0-r15						INTEGER (0..79),

rstd1-r15						INTEGER (0..159),

rstd2-r15						INTEGER (0..319),

rstd3-r15						INTEGER (0..639),

rstd4-r15						INTEGER (0..1279),

rstd5-r15						INTEGER (0..159),

rstd6-r15						INTEGER (0..319),

rstd7-r15						INTEGER (0..639),

rstd8-r15						INTEGER (0..1279),

rstd9-r15						INTEGER (0..319),

rstd10-r15						INTEGER (0..639),

rstd11-r15						INTEGER (0..1279),

rstd12-r15						INTEGER (0..319),

rstd13-r15						INTEGER (0..639),

rstd14-r15						INTEGER (0..1279),

rstd15-r15						INTEGER (0..639),

rstd16-r15						INTEGER (0..1279),

rstd17-r15						INTEGER (0..639),

rstd18-r15						INTEGER (0..1279),

rstd19-r15						INTEGER (0..639),

rstd20-r15						INTEGER (0..1279),

...

}

}

}

-- ASN1STOP

MeasGapConfigDensePRS field descriptions

gapOffsetDensePRS Indicates the gap offset for performing RSTD measurements with dense PRS configurations as specified in 5.5.2.9a corresponding to measurement gap pattern ID specified in TS 36.133 [16].

#### –	MeasGapConfigPerCC-List

The IE MeasGapConfigPerCC-List specifies the measurement gap configuration and controls setup/ release of measurement gaps.

MeasGapConfigPerCC-List information element

-- ASN1START

MeasGapConfigPerCC-List-r14 ::=	CHOICE {

release						NULL,

setup						SEQUENCE {

measGapConfigToRemoveList-r14	MeasGapConfigToRemoveList-r14	OPTIONAL,	-- Need ON

measGapConfigToAddModList-r14	MeasGapConfigToAddModList-r14	OPTIONAL	-- Need ON

}

}

MeasGapConfigToRemoveList-r14 ::=	SEQUENCE (SIZE (1..maxServCell-r13)) OF ServCellIndex-r13

MeasGapConfigToAddModList-r14 ::=	SEQUENCE (SIZE (1..maxServCell-r13)) OF MeasGapConfigPerCC-r14

MeasGapConfigPerCC-r14 ::=	SEQUENCE {

servCellId-r14				ServCellIndex-r13,

measGapConfigCC-r14			MeasGapConfig

}

-- ASN1STOP

MeasGapConfigPerCC-List field descriptions

measGapConfigToAddModList List of serving cells and corresponding serving cell specific measurement gap configuration to add /modify.

measGapConfigToRemoveList List of serving cells for which measurement gap configuration is removed.

#### –	MeasGapSharingConfig

The IE MeasGapSharingConfig specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

MeasGapSharingConfig information element

-- ASN1START

MeasGapSharingConfig-r14 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

measGapSharingScheme-r14				ENUMERATED {scheme00, scheme01, scheme10, scheme11}

}

}

-- ASN1STOP

MeasGapSharingConfig field descriptions

measGapSharingScheme Indicates the measurement gaps sharing scheme for BL UEs in CE mode A and CE mode B and for (NG)EN-DC (for the measurement gap configured by E-UTRAN). For BL UEs, see TS 36.133 [16], Table 8.13.2.1.1.1-2 and Table 8.13.3.1.1.1-3. For (NG)EN-DC, see TS 36.133 [16], Table 8.17.1.1-1. Value scheme00 corresponds to "00", value scheme01 corresponds to "01", and so on.

#### –	MeasId

The IE MeasId is used to identify a measurement configuration, i.e., linking of a measurement object and a reporting configuration.

MeasId information element

-- ASN1START

MeasId ::=							INTEGER (1..maxMeasId)

MeasId-v1250 ::=					INTEGER (maxMeasId-Plus1..maxMeasId-r12)

-- ASN1STOP

#### –	MeasIdleConfig

The IE MeasIdleConfig is used to convey information to UE about measurements requested to be done while in RRC_IDLE or RRC_INACTIVE.

MeasIdleConfig information element

-- ASN1START

MeasIdleConfigSIB-r15 ::= SEQUENCE {

measIdleCarrierListEUTRA-r15	EUTRA-CarrierList-r15,

...

}

MeasIdleConfigSIB-NR-r16 ::= SEQUENCE {

measIdleCarrierListNR-r16		NR-CarrierList-r16,

...

}

MeasIdleConfigDedicated-r15 ::= SEQUENCE {

measIdleCarrierListEUTRA-r15	EUTRA-CarrierList-r15				OPTIONAL,	-- Need OR

measIdleDuration-r15		ENUMERATED {sec10, sec30, sec60, sec120,

sec180, sec240, sec300, spare},

...,

[[

measIdleCarrierListNR-r16		NR-CarrierList-r16					OPTIONAL,  -- Need OR

validityAreaList-r16			ValidityAreaList-r16				OPTIONAL   -- Need OR

]]

}

EUTRA-CarrierList-r15 ::= SEQUENCE (SIZE (1..maxFreqIdle-r15)) OF MeasIdleCarrierEUTRA-r15

NR-CarrierList-r16 ::= SEQUENCE (SIZE (1..maxFreqIdle-r15)) OF MeasIdleCarrierNR-r16

MeasIdleCarrierEUTRA-r15::=			SEQUENCE {

carrierFreq-r15						ARFCN-ValueEUTRA-r9,

allowedMeasBandwidth-r15			AllowedMeasBandwidth,

validityArea-r15					CellList-r15					OPTIONAL,	-- Need OR

measCellList-r15					CellList-r15					OPTIONAL,	-- Need OR

reportQuantities					ENUMERATED {rsrp, rsrq, both},

qualityThreshold-r15				SEQUENCE {

idleRSRP-Threshold-r15				RSRP-Range					OPTIONAL,	-- Need OR

idleRSRQ-Threshold-r15				RSRQ-Range-r13				OPTIONAL	-- Need OR

}																	OPTIONAL,	-- Need OP

...

}

ValidityAreaList-r16 ::= SEQUENCE (SIZE (1..maxFreqIdle-r15)) OF ValidityArea-r16

ValidityArea-r16 ::= SEQUENCE {

carrierFreq-r16			ARFCN-ValueEUTRA-r9,

validityCellList-r16		ValidityCellList-r16					OPTIONAL  -- Need ON

}

ValidityCellList-r16 ::= SEQUENCE (SIZE (1.. maxCellMeasIdle-r15)) OF PhysCellIdRange

MeasIdleCarrierNR-r16 ::=		SEQUENCE {

carrierFreqNR-r16				ARFCN-ValueNR-r15,

subcarrierSpacingSSB-r16		ENUMERATED {kHz15, kHz30, kHz120, kHz240},

frequencyBandList				MultiFrequencyBandListNR-r15		OPTIONAL,  -- Need OR

measCellListNR-r16				CellListNR-r16						OPTIONAL,  -- Need OR

reportQuantitiesNR-r16			ENUMERATED {rsrp, rsrq, both},

qualityThresholdNR-r16			SEQUENCE {

idleRSRP-ThresholdNR-r16		RSRP-RangeNR-r15				OPTIONAL,  -- Need OR

idleRSRQ-ThresholdNR-r16		RSRQ-RangeNR-r15				OPTIONAL   -- Need OR

}																	OPTIONAL,  -- Need OR

ssb-MeasConfig-r16				SEQUENCE {

maxRS-IndexCellQual-r16			MaxRS-IndexCellQualNR-r15		OPTIONAL,  -- Need OR

threshRS-Index-r16				ThresholdListNR-r15				OPTIONAL,  -- Need OR

measTimingConfig-r16			MTC-SSB-NR-r15					OPTIONAL,  -- Need OR

ssb-ToMeasure-r16				SSB-ToMeasure-r15				OPTIONAL,  -- Need OR

deriveSSB-IndexFromCell-r16		BOOLEAN,

ss-RSSI-Measurement-r16			SS-RSSI-Measurement-r15			OPTIONAL   -- Need OP

}																	OPTIONAL,   -- Need OP

beamMeasConfigIdle-r16			BeamMeasConfigIdleNR-r16			OPTIONAL,  -- Need OR

...

}

CellList-r15 ::=			SEQUENCE (SIZE (1..maxCellMeasIdle-r15)) OF PhysCellIdRange

CellListNR-r16 ::=			SEQUENCE (SIZE (1..maxCellMeasIdle-r15)) OF PhysCellIdRangeNR-r16

BeamMeasConfigIdleNR-r16 ::=		SEQUENCE {

reportQuantityRS-IndexNR-r16		ENUMERATED {rsrp, rsrq, both},

maxReportRS-Index-r16				INTEGER (0..maxRS-IndexReport-r15),

reportRS-IndexResultsNR-r16			BOOLEAN

}

-- ASN1STOP

MeasIdleConfig field descriptions

allowedMeasBandwidth If absent, the value corresponding to the downlink bandwidth indicated by the dl-Bandwidth included in MasterInformationBlock of serving cell applies.

beamMeasConfigIdle Indicates the beam level measurement configuration.

carrierFreq Indicates the E-UTRA carrier frequency to be used for measurements during RRC_IDLE or RRC_INACTIVE.

carrierFreqNR Indicates the NR carrier frequency to be used for measurements during RRC_IDLE or RRC_INACTIVE.

frequencyBandList Indicates the list of frequency bands for which the NR idle/inactive measurement parameters apply. The UE shall select the first listed band which it supports in the frequencyBandList field to represent the NR neighbour carrier frequency.

deriveSSB-IndexFromCell The field indicates whether the UE may use, to derive the SSB index of a cell on the indicated SSB frequency and subcarrier spacing, the timing of any detected cell with the same SSB frequency and subcarrier spacing. If this field is set to TRUE, the UE assumes SFN and frame boundary alignment across cells on the same NR carrier frequency as specified in TS 36.133 [16].

maxReportRS-Index Max number of beam indices to include in the idle/inactive measurement result.

maxRS-IndexCellQual Number of SS blocks to average for cell measurement derivation. Corresponds to the parameter nrofSS-BlocksToAverage in TS 38.304 [92].

measCellList Indicates the list of E-UTRA cells which the UE is requested to measure and report for idle/inactive measurements.

measCellListNR Indicates the list of NR cells which the UE is requested to measure and report for idle/inactive measurements.

measIdleCarrierListEUTRA Indicates the E-UTRA carriers to be measured during RRC_IDLE or RRC_INACTIVE.

measIdleCarrierListNR Indicates the NR carriers to be measured during RRC_IDLE or RRC_INACTIVE.

measIdleDuration Indicates the duration for performing measurements during RRC_IDLE or RRC_INACTIVE for measurements assigned via RRCConnectionRelease. Value sec10 correspond to 10 seconds, value sec30 to 30 seconds and so on.

measTimingConfig Used to configure the NR measurement timing configurations, i.e., timing occasions at which the UE measures SSBs. If the field is absent in VarMeasConfig, the UE assumes that SSB periodicity is 5ms in this frequency.

qualityThreshold Indicates the quality thresholds for reporting the measured cells for idle/inactive E-UTRA measurements.

qualityThresholdNR Indicates the quality thresholds for reporting the measured cells for idle/inactive NR measurements.

reportQuantities Indicates which E-UTRA measurement quantities the UE is requested to report in the idle/inactive measurement report. In this version of the specification, E-UTRAN always configures the value 'both'.

reportQuantitiesNR Indicates which NR measurement quantities the UE is requested to report in the idle/inactive measurement report.

reportQuantityRS-IndexNR Indicates which measurement information per beam index the UE shall include in the NR idle/inactive measurement results.

reportRS-IndexResultsNR Indicates whether or not the UE shall include beam measurements in the NR idle/inactive measurement results.

ss-RSSI-Measurement Indicates the SSB-based RSSI measurement configuration. If the field is absent in VarMeasConfig, the UE behaviour is defined in TS 38.215 [89], clause 5.1.3.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [89]). When the field is absent in VarMeasConfig, the UE measures on all SS-blocks.

subcarrierSpacingSSB Indicates subcarrier spacing of SSB of NR frequency.

threshRS-Index List of thresholds for consolidation of L1 measurements per RS index. Corresponds to the parameter absThreshSS-BlocksConsolidation in TS 38.304 [92].

validityArea Indicates the list of cells within which UE is requested to do measurements during RRC_IDLE or RRC_INACTIVE. If the UE reselects to a cell whose physical cell identity does not match any entry in validityArea for the corresponding carrier frequency, the measurements are no longer required. E-UTRAN configures this field only in RRCConnectionRelease.

validityAreaList Indicates the list of frequencies and optionally, for each frequency, a list of cells within which the UE is required to perform measurements during RRC_IDLE or RRC_INACTIVE. E-UTRAN configures this field only in RRCConnectionRelease. A UE can be configured either with validityArea or validityAreaList, but not both.

#### –	MeasIdToAddModList

The IE MeasIdToAddModList concerns a list of measurement identities to add or modify, with for each entry the measId, the associated measObjectId and the associated reportConfigId. Field measIdToAddModListExt includes additional measurement identities i.e. extends the size of the measurement identity list using the general principles specified in 5.1.2.

MeasIdToAddModList information element

-- ASN1START

MeasIdToAddModList ::=				SEQUENCE (SIZE (1..maxMeasId)) OF MeasIdToAddMod

MeasIdToAddModList-v1310 ::=		SEQUENCE (SIZE (1..maxMeasId)) OF MeasIdToAddMod-v1310

MeasIdToAddModListExt-r12 ::=		SEQUENCE (SIZE (1..maxMeasId)) OF MeasIdToAddModExt-r12

MeasIdToAddModListExt-v1310 ::=		SEQUENCE (SIZE (1..maxMeasId)) OF MeasIdToAddMod-v1310

MeasIdToAddMod ::=	SEQUENCE {

measId								MeasId,

measObjectId						MeasObjectId,

reportConfigId						ReportConfigId

}

MeasIdToAddModExt-r12 ::=	SEQUENCE {

measId-v1250						MeasId-v1250,

measObjectId-r12					MeasObjectId,

reportConfigId-r12					ReportConfigId

}

MeasIdToAddMod-v1310 ::=	SEQUENCE {

measObjectId-v1310			MeasObjectId-v1310		OPTIONAL

}

-- ASN1STOP

MeasIdToAddModList field descriptions

measObjectId If the measObjectId-v1310 is included, the measObjectId or measObjectId-r12 is ignored by the UE.

#### –	MeasObjectCDMA2000

The IE MeasObjectCDMA2000 specifies information applicable for inter-RAT CDMA2000 neighbouring cells.

MeasObjectCDMA2000 information element

-- ASN1START

MeasObjectCDMA2000 ::=				SEQUENCE {

cdma2000-Type						CDMA2000-Type,

carrierFreq							CarrierFreqCDMA2000,

searchWindowSize					INTEGER (0..15)						OPTIONAL,	-- Need ON

offsetFreq							Q-OffsetRangeInterRAT				DEFAULT 0,

cellsToRemoveList					CellIndexList						OPTIONAL,	-- Need ON

cellsToAddModList					CellsToAddModListCDMA2000			OPTIONAL,	-- Need ON

cellForWhichToReportCGI				PhysCellIdCDMA2000					OPTIONAL,	-- Need ON

...

}

CellsToAddModListCDMA2000 ::=		SEQUENCE (SIZE (1..maxCellMeas)) OF CellsToAddModCDMA2000

CellsToAddModCDMA2000 ::=	SEQUENCE {

cellIndex							INTEGER (1..maxCellMeas),

physCellId							PhysCellIdCDMA2000

}

-- ASN1STOP

MeasObjectCDMA2000 field descriptions

carrierInfo Identifies CDMA2000 carrier frequency for which this configuration is valid.

cdma2000-Type The type of CDMA2000 network: CDMA2000 1xRTT or CDMA2000 HRPD.

cellIndex Entry index in the neighbouring cell list.

cellsToAddModList List of cells to add/ modify in the neighbouring cell list.

cellsToRemoveList List of cells to remove from the neighbouring cell list.

physCellId CDMA2000 Physical cell identity of a cell in neighbouring cell list expressed as PNOffset.

searchWindowSize Provides the search window size to be used by the UE for the neighbouring pilot, see C.S0005 [25].

#### –	MeasObjectEUTRA

The IE MeasObjectEUTRA specifies information applicable for intra-frequency or inter-frequency E-UTRA cells.

MeasObjectEUTRA information element

-- ASN1START

MeasObjectEUTRA ::=					SEQUENCE {

carrierFreq							ARFCN-ValueEUTRA,

allowedMeasBandwidth				AllowedMeasBandwidth,

presenceAntennaPort1				PresenceAntennaPort1,

neighCellConfig						NeighCellConfig,

offsetFreq							Q-OffsetRange				DEFAULT dB0,

-- Cell list

cellsToRemoveList					CellIndexList				OPTIONAL,		-- Need ON

cellsToAddModList					CellsToAddModList			OPTIONAL,		-- Need ON

-- Black list

blackCellsToRemoveList				CellIndexList				OPTIONAL,		-- Need ON

blackCellsToAddModList				BlackCellsToAddModList		OPTIONAL,		-- Need ON

cellForWhichToReportCGI				PhysCellId					OPTIONAL,		-- Need ON

...,

[[measCycleSCell-r10				MeasCycleSCell-r10		OPTIONAL,		-- Need ON

measSubframePatternConfigNeigh-r10	MeasSubframePatternConfigNeigh-r10	OPTIONAL							-- Need ON

]],

[[widebandRSRQ-Meas-r11				BOOLEAN	OPTIONAL		-- Cond WB-RSRQ

]],

[[	altTTT-CellsToRemoveList-r12	CellIndexList				OPTIONAL,		-- Need ON

altTTT-CellsToAddModList-r12	AltTTT-CellsToAddModList-r12	OPTIONAL,		-- Need ON

t312-r12						CHOICE {

release							NULL,

setup							ENUMERATED {ms0, ms50, ms100, ms200,

ms300, ms400, ms500, ms1000}

}														OPTIONAL,		-- Need ON

reducedMeasPerformance-r12		BOOLEAN					OPTIONAL,		-- Need ON

measDS-Config-r12				MeasDS-Config-r12			OPTIONAL		-- Need ON

]],

[[

whiteCellsToRemoveList-r13		CellIndexList				OPTIONAL,		-- Need ON

whiteCellsToAddModList-r13		WhiteCellsToAddModList-r13	OPTIONAL,		-- Need ON

rmtc-Config-r13				RMTC-Config-r13			OPTIONAL,		-- Need ON

carrierFreq-r13					ARFCN-ValueEUTRA-v9e0		OPTIONAL			-- Need ON

]],

[[

tx-ResourcePoolToRemoveList-r14	Tx-ResourcePoolMeasList-r14		OPTIONAL,	-- Need ON

tx-ResourcePoolToAddList-r14	Tx-ResourcePoolMeasList-r14		OPTIONAL,	-- Need ON

fembms-MixedCarrier-r14				BOOLEAN					OPTIONAL			-- Need ON

]],

[[

measSensing-Config-r15			MeasSensing-Config-r15		OPTIONAL		-- Need ON

]],

[[

measRSS-DedicatedConfig-r16		SetupRelease {MeasRSS-DedicatedConfig-r16}		OPTIONAL	-- Need ON

]]

}

MeasObjectEUTRA-v9e0 ::=			SEQUENCE {

carrierFreq-v9e0					ARFCN-ValueEUTRA-v9e0

}

MeasRSS-DedicatedConfig-r16 ::= SEQUENCE {

rss-ConfigCarrierInfo-r16		RSS-ConfigCarrierInfo-r16	OPTIONAL,	-- Need OP

cellsToAddModList-v1610			CellsToAddModList-v1610		OPTIONAL	-- Need ON

}

CellsToAddModList ::=				SEQUENCE (SIZE (1..maxCellMeas)) OF CellsToAddMod

CellsToAddModList-v1610 ::=			SEQUENCE (SIZE (1..maxCellMeas)) OF CellsToAddMod-v1610

CellsToAddMod ::=	SEQUENCE {

cellIndex							INTEGER (1..maxCellMeas),

physCellId							PhysCellId,

cellIndividualOffset				Q-OffsetRange

}

CellsToAddMod-v1610 ::= 		SEQUENCE {

rss-MeasPowerBias-r16			RSS-MeasPowerBias-r16

}

BlackCellsToAddModList ::=			SEQUENCE (SIZE (1..maxCellMeas)) OF BlackCellsToAddMod

BlackCellsToAddMod ::=	SEQUENCE {

cellIndex							INTEGER (1..maxCellMeas),

physCellIdRange						PhysCellIdRange

}

MeasCycleSCell-r10 ::=				ENUMERATED {sf160, sf256, sf320, sf512,

sf640, sf1024, sf1280, spare1}

MeasSubframePatternConfigNeigh-r10 ::=	CHOICE {

release									NULL,

setup									SEQUENCE {

measSubframePatternNeigh-r10			MeasSubframePattern-r10,

measSubframeCellList-r10				MeasSubframeCellList-r10	OPTIONAL	-- Cond always

}

}

MeasSubframeCellList-r10 ::=	SEQUENCE (SIZE (1..maxCellMeas)) OF PhysCellIdRange

AltTTT-CellsToAddModList-r12 ::=	SEQUENCE (SIZE (1..maxCellMeas)) OF AltTTT-CellsToAddMod-r12

AltTTT-CellsToAddMod-r12 ::=	SEQUENCE {

cellIndex-r12							INTEGER (1..maxCellMeas),

physCellIdRange-r12						PhysCellIdRange

}

WhiteCellsToAddModList-r13 ::=			SEQUENCE (SIZE (1..maxCellMeas)) OF WhiteCellsToAddMod-r13

WhiteCellsToAddMod-r13 ::=	SEQUENCE {

cellIndex-r13							INTEGER (1..maxCellMeas),

physCellIdRange-r13						PhysCellIdRange

}

RMTC-Config-r13 ::=	CHOICE {

release								NULL,

setup								SEQUENCE {

rmtc-Period-r13					ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r13			INTEGER(0..639)					OPTIONAL,		-- Need ON

measDuration-r13				ENUMERATED {sym1, sym14, sym28, sym42, sym70},

...

}

}

Tx-ResourcePoolMeasList-r14 ::=	SEQUENCE (SIZE (1..maxSL-PoolToMeasure-r14)) OF SL-V2X-TxPoolReportIdentity-r14

-- ASN1STOP

MeasObjectEUTRA field descriptions

altTTT-CellsToAddModList List of cells to add/ modify in the cell list for which the alternative time to trigger specified by alternativeTimeToTrigger in reportConfigEUTRA, if configured, applies.

altTTT-CellsToRemoveList List of cells to remove from the list of cells for alternative time to trigger.

blackCellsToAddModList List of cells to add/ modify in the black list of cells.

blackCellsToRemoveList List of cells to remove from the black list of cells.

carrierFreq Identifies E-UTRA carrier frequency for which this configuration is valid. E-UTRAN does not configure more than one measurement object for the same physical frequency regardless of the E-ARFCN used to indicate this. CarrierFreq-r13 is included only when the extension list measObjectToAddModListExt-r13 is used. If carrierFreq-r13 is present, carrierFreq (i.e., without suffix) shall be set to value maxEARFCN.

cellIndex Entry index in the cell list. An entry may concern a range of cells, in which case this value applies to the entire range.

cellIndividualOffset Cell individual offset applicable to a specific cell. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

cellsToAddModList List of cells to add/ modify in the cell list. cellsToAddModList-v1610 indicates list of RSS assistance information which is used for the corresponding physCellId. If E-UTRAN includes cellsToAddModList-v1610, it includes the same number of entries, and listed in the same order, as in cellsToAddModList (i.e. without suffix).

cellsToRemoveList List of cells to remove from the cell list.

fembms-MixedCarrier If this field is set to TRUE, the cells on the carrier frequency indicated by the measObject are FeMBMS/Unicast-mixed cells.

measCycleSCell The parameter is used only when an SCell is configured on the frequency indicated by the measObject and is in deactivated state, see TS 36.133 [16], clause 8.3.3. E-UTRAN configures the parameter whenever an SCell is configured on the frequency indicated by the measObject, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, sf256 corresponds to 256 sub-frames and so on.

measDS-Config Parameters applicable to discovery signals measurement on the carrier frequency indicated by carrierFreq.

measDuration Number of consecutive symbols for which the Physical Layer reports samples of RSSI, see TS 36.214 [48]. Value sym1 corresponds to one symbol, sym14 corresponds to 14 symbols, and so on.

measRSS-DedicatedConfig The field indicates whether measurements based on RSS in RRC_CONNECTED is enabled and provides neighbour cell RSS information.

measSubframeCellList List of cells for which measSubframePatternNeigh is applied.

measSubframePatternNeigh Time domain measurement resource restriction pattern applicable to neighbour cell RSRP and RSRQ measurements on the carrier frequency indicated by carrierFreq. For cells in measSubframeCellList the UE shall assume that the subframes indicated by measSubframePatternNeigh are non-MBSFN subframes, and have the same special subframe configuration as PCell.

offsetFreq Offset value applicable to the carrier frequency. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

physCellId Physical cell identity of a cell in the cell list.

physCellIdRange Physical cell identity or a range of physical cell identities.

reducedMeasPerformance If set to TRUE, the EUTRA carrier frequency is configured for reduced measurement performance, otherwise it is configured for normal measurement performance, see TS 36.133 [16].

rmtc-Config Parameters applicable to RSSI and channel occupancy measurement on the carrier frequency indicated by carrierFreq.

rmtc-Period Indicates the RSSI measurement timing configuration (RMTC) periodicity for this frequency. Value ms40 corresponds to 40 ms periodicity, ms80 corresponds to 80 ms periodicity and so on, see TS 36.214 [48].

rmtc-SubframeOffset Indicates the RSSI measurement timing configuration (RMTC) subframe offset for this frequency. The value of rmtc-SubframeOffset should be smaller than the value of rmtc-Period, see TS 36.214 [48]. For inter-frequency measurements, this field is optional present and if it is not configured, the UE chooses a random value as rmtc-SubframeOffset for measDuration which shall be selected to be between 0 and the configured rmtc-Period with equal probability.

rss-ConfigCarrierInfo RSS configurations for this carrier frequency. If absent, RSS is collocated (time and frequency domain) in all cells.

t312 The value of timer T312. Value ms0 represents 0 ms, ms50 represents 50 ms and so on.

tx-ResourcePoolToAddList List of transmission pools identities to be added to the list of pools configured for CBR measurements and for which poolReportId is included in SL-V2X-ConfigDedicated, SystemInformationBlockType21 or SystemInformationBlockType26.

tx-ResourcePoolToRemoveList List of transmission resource pools identities to be removed from the list of pools configured for CBR measurements and for which poolReportId is included in SL-V2X-ConfigDedicated, SystemInformationBlockType21 or SystemInformationBlockType26.

widebandRSRQ-Meas If this field is set to TRUE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16].

whiteCellsToAddModList List of cells to add/modify in the white list of cells.

whiteCellsToRemoveList List of cells to remove from the white list of cells.

| Conditional presence | Explanation |
| --- | --- |
| always | The field is mandatory present. |
| WB-RSRQ | The field is optionally present, need ON, if the measurement bandwidth indicated by allowedMeasBandwidth is 50 resource blocks or larger; otherwise it is not present and the UE shall delete any existing value for this field, if configured. |

#### –	MeasObjectGERAN

The IE MeasObjectGERAN specifies information applicable for inter-RAT GERAN neighbouring frequencies.

MeasObjectGERAN information element

-- ASN1START

MeasObjectGERAN ::=					SEQUENCE {

carrierFreqs						CarrierFreqsGERAN,

offsetFreq							Q-OffsetRangeInterRAT		DEFAULT 0,

ncc-Permitted						BIT STRING(SIZE (8))		DEFAULT '11111111'B,

cellForWhichToReportCGI				PhysCellIdGERAN				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

MeasObjectGERAN field descriptions

ncc-Permitted Field encoded as a bit map, where bit N is set to "0" if a BCCH carrier with NCC = N-1 is not permitted for monitoring and set to "1" if a BCCH carrier with NCC = N-1 is permitted for monitoring; N = 1 to 8; bit 1 of the bitmap is the leading bit of the bit string.

carrierFreqs If E-UTRAN includes cellForWhichToReportCGI, it includes only one GERAN ARFCN value in carrierFreqs.

#### –	MeasObjectId

The IE MeasObjectId used to identify a measurement object configuration.

MeasObjectId information element

-- ASN1START

MeasObjectId ::=					INTEGER (1..maxObjectId)

MeasObjectId-v1310 ::=			INTEGER (maxObjectId-Plus1-r13..maxObjectId-r13)

MeasObjectId-r13 ::=				INTEGER (1..maxObjectId-r13)

-- ASN1STOP

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for inter-RAT NR neighbouring cells.

MeasObjectNR information element

-- ASN1START

MeasObjectNR-r15 ::=				SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

rs-ConfigSSB-r15					RS-ConfigSSB-NR-r15,

threshRS-Index-r15					ThresholdListNR-r15				OPTIONAL,		-- Need OR

maxRS-IndexCellQual-r15				MaxRS-IndexCellQualNR-r15		OPTIONAL,		-- Need OR

offsetFreq-r15						Q-OffsetRangeInterRAT			DEFAULT 0,

blackCellsToRemoveList-r15			CellIndexList					OPTIONAL,		-- Need ON

blackCellsToAddModList-r15			CellsToAddModListNR-r15			OPTIONAL,		-- Need ON

quantityConfigSet-r15				INTEGER (1.. maxQuantSetsNR-r15),

cellsForWhichToReportSFTD-r15		SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellIdNR-r15	OPTIONAL,	-- Need OR

...,

[[	cellForWhichToReportCGI-r15			PhysCellIdNR-r15				OPTIONAL,	-- Need ON

deriveSSB-IndexFromCell-r15			BOOLEAN							OPTIONAL,	-- Need ON

ss-RSSI-Measurement-r15				SS-RSSI-Measurement-r15			OPTIONAL,	-- Need ON

bandNR-r15					CHOICE {

release						NULL,

setup						FreqBandIndicatorNR-r15

}																OPTIONAL	-- Need ON

]],

[[

rmtc-ConfigNR-r16						SetupRelease {RMTC-ConfigNR-r16}			OPTIONAL		-- Cond SharedSpectrum

]]

}

RS-ConfigSSB-NR-r15 ::=			SEQUENCE {

measTimingConfig-r15			MTC-SSB-NR-r15,

subcarrierSpacingSSB-r15	ENUMERATED {kHz15, kHz30, kHz120, kHz240},

...,

[[	ssb-ToMeasure-r15			CHOICE {

release						NULL,

setup						SSB-ToMeasure-r15

}											OPTIONAL	-- Need ON

]],

[[

ssb-PositionQCL-CommonNR-r16	SSB-PositionQCL-RelationNR-r16	OPTIONAL,	-- Cond SharedSpectrum2

ssb-PositionQCL-CellsToAddModListNR-r16	SSB-PositionQCL-CellsToAddModListNR-r16	OPTIONAL,	-- Cond SharedSpectrum

ssb-PositionQCL-CellsToRemoveListNR-r16	SEQUENCE (SIZE (1..maxCellMeas)) OF PhysCellIdNR-r15	OPTIONAL	-- Cond SharedSpectrum

]]

}

CellsToAddModListNR-r15 ::=			SEQUENCE (SIZE (1..maxCellMeas)) OF CellsToAddModNR-r15

CellsToAddModNR-r15 ::=			SEQUENCE {

cellIndex-r15					INTEGER (1..maxCellMeas),

physCellId-r15					PhysCellIdNR-r15

}

SSB-PositionQCL-CellsToAddModListNR-r16 ::=	SEQUENCE (SIZE (1..maxCellMeas)) OF SSB-PositionQCL-CellsToAddNR-r16

SSB-PositionQCL-CellsToAddNR-r16 ::=	SEQUENCE {

physCellId-r16							PhysCellIdNR-r15,

ssb-PositionQCL-NR-r16					SSB-PositionQCL-RelationNR-r16

}

RMTC-ConfigNR-r16 ::=	SEQUENCE {

rmtc-PeriodicityNR-r16				ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffsetNR-r16			INTEGER(0..639)			OPTIONAL,	-- Need ON

measDurationNR-r16					ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60},

rmtc-FrequencyNR-r16				ARFCN-ValueNR-r15,

refSCS-CP-NR-r16					ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP},

...

}

-- ASN1STOP

MeasObjectNR field descriptions

bandNR Indicates the frequency band of the NR carrier frequency configured in this MeasObjectNR. This field is always set to setup when the network configures measurements with this MeasObjectNR.

carrierFreq Identifies the SSB frequency to be measured. E-UTRAN does not configure more than one measurement object for the same SSB frequency.

deriveSSB-IndexFromCell The field indicates whether the UE may use, to derive the SSB index of a cell on the indicated SSB frequency and subcarrier spacing, the timing of the NR serving cell with the same SSB frequency and subcarrier spacing if configured. Otherwise, the field indicates whether the UE may use the timing of any detected cell with the same SSB frequency and subcarrier spacing.

measDurationNR Number of consecutive symbols for which the Physical Layer reports samples of RSSI (see TS 38.215 [89]). Value sym1 corresponds to one symbol, sym14or12 corresponds to 14 symbols of the reference numerology for NCP and 12 symbols for ECP, and so on.

quantityConfigSet Indicates the n-th element of quantityConfigNRList provided in MeasConfig.

refSCS-CP-NR Indicates a reference subcarrier spacing and cyclic prefix to be used for RSSI measurements (see TS 38.215 [89]).

rmtc-FrequencyNR Indicates the center frequency of the measured bandwidth (see TS 38.215 [89]).

rmtc-PeriodicityNR Indicates the RSSI measurement timing configuration (RMTC) periodicity (see TS 38.215 [89]). Value ms40 corresponds to 40 ms periodicity, ms80 corresponds to 80 ms periodicity, and so on.

rmtc-SubframeOffsetNR Indicates the RSSI measurement timing configuration (RMTC) subframe offset (see TS 38.215 [89)). If not configured, the UE chooses a random value as rmtc-SubframeOffsetNR for measDurationNR which shall be selected to be between 0 and the configured rmtc-PeriodicityNR with equal probability.

rs-ConfigSSB Indicates the SSB configuration for measuring the set of SS blocks within the SMTC measurement duration.

ssb-PositionQCL-NR Indicates the QCL relationship between SS/PBCH blocks for a specific neighbor cell as specified in TS 38.213 [88], clause 4.1. If provided, the cell specific value overwrites the common value signalled by ssb-PositionQCL-CommonNR in MeasObjectNR for the indicated cell.

ssb-PositionQCL-CommonNR Indicates the QCL relationship between SS/PBCH blocks for NR neighbor cells as specified in TS 38.213 [88], clause 4.1.

threshRS-Index List of thresholds for consolidation of L1 measurements per RS index.

| Conditional presence | Explanation |
| --- | --- |
| SharedSpectrum | The field is optional Need ON if NR operates with shared spectrum channel access; otherwise, it is not present. |
| SharedSpectrum2 | The field is mandatory present if NR operates with shared spectrum channel access; otherwise, it is not present. |

#### –	MeasObjectToAddModList

The IE MeasObjectToAddModList concerns a list of measurement objects to add or modify

MeasObjectToAddModList information element

-- ASN1START

MeasObjectToAddModList ::=			SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddMod

MeasObjectToAddModListExt-r13 ::=	SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddModExt-r13

MeasObjectToAddModList-v9e0 ::=		SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddMod-v9e0

MeasObjectToAddMod ::=	SEQUENCE {

measObjectId						MeasObjectId,

measObject							CHOICE {

measObjectEUTRA						MeasObjectEUTRA,

measObjectUTRA						MeasObjectUTRA,

measObjectGERAN						MeasObjectGERAN,

measObjectCDMA2000					MeasObjectCDMA2000,

...,

measObjectWLAN-r13					MeasObjectWLAN-r13,

measObjectNR-r15					MeasObjectNR-r15

}

}

MeasObjectToAddModExt-r13 ::=	SEQUENCE {

measObjectId-r13					MeasObjectId-v1310,

measObject-r13							CHOICE {

measObjectEUTRA-r13						MeasObjectEUTRA,

measObjectUTRA-r13						MeasObjectUTRA,

measObjectGERAN-r13						MeasObjectGERAN,

measObjectCDMA2000-r13					MeasObjectCDMA2000,

...,

measObjectWLAN-v1320					MeasObjectWLAN-r13,

measObjectNR-r15						MeasObjectNR-r15

}

}

MeasObjectToAddMod-v9e0 ::=	SEQUENCE {

measObjectEUTRA-v9e0				MeasObjectEUTRA-v9e0		OPTIONAL	-- Cond eutra

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| eutra | The field is optional present, need OR, if for the corresponding entry in MeasObjectToAddModList or MeasObjectToAddModListExt-r13 field measObject is set to measObjectEUTRA and its sub-field carrierFreq is set to maxEARFCN. Otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	MeasObjectUTRA

The IE MeasObjectUTRA specifies information applicable for inter-RAT UTRA neighbouring cells.

MeasObjectUTRA information element

-- ASN1START

MeasObjectUTRA ::=					SEQUENCE {

carrierFreq							ARFCN-ValueUTRA,

offsetFreq							Q-OffsetRangeInterRAT		DEFAULT 0,

cellsToRemoveList					CellIndexList				OPTIONAL,			-- Need ON

cellsToAddModList					CHOICE {

cellsToAddModListUTRA-FDD			CellsToAddModListUTRA-FDD,

cellsToAddModListUTRA-TDD			CellsToAddModListUTRA-TDD

}																OPTIONAL,			-- Need ON

cellForWhichToReportCGI				CHOICE {

utra-FDD							PhysCellIdUTRA-FDD,

utra-TDD							PhysCellIdUTRA-TDD

}																OPTIONAL,	-- Need ON

...,

[[	csg-allowedReportingCells-v930			CSG-AllowedReportingCells-r9	OPTIONAL		-- Need ON

]],

[[	reducedMeasPerformance-r12				BOOLEAN			OPTIONAL		-- Need ON

]]

}

CellsToAddModListUTRA-FDD ::=		SEQUENCE (SIZE (1..maxCellMeas)) OF CellsToAddModUTRA-FDD

CellsToAddModUTRA-FDD ::=	SEQUENCE {

cellIndex							INTEGER (1..maxCellMeas),

physCellId							PhysCellIdUTRA-FDD

}

CellsToAddModListUTRA-TDD ::=		SEQUENCE (SIZE (1..maxCellMeas)) OF CellsToAddModUTRA-TDD

CellsToAddModUTRA-TDD ::=	SEQUENCE {

cellIndex							INTEGER (1..maxCellMeas),

physCellId							PhysCellIdUTRA-TDD

}

CSG-AllowedReportingCells-r9 ::=		SEQUENCE {

physCellIdRangeUTRA-FDDList-r9			PhysCellIdRangeUTRA-FDDList-r9	OPTIONAL	-- Need OR

}

-- ASN1STOP

MeasObjectUTRA field descriptions

carrierFreq Identifies UTRA carrier frequency for which this configuration is valid. E-UTRAN does not configure more than one measurement object for the same physical frequency regardless of the ARFCN used to indicate this.

cellIndex Entry index in the neighbouring cell list.

cellsToAddModListUTRA-FDD List of UTRA FDD cells to add/ modify in the neighbouring cell list.

cellsToAddModListUTRA-TDD List of UTRA TDD cells to add/modify in the neighbouring cell list.

cellsToRemoveList List of cells to remove from the neighbouring cell list.

csg-allowedReportingCells One or more ranges of physical cell identities for which UTRA-FDD reporting is allowed.

reducedMeasPerformance If set to TRUE the UTRA carrier frequency is configured for reduced measurement performance, otherwise it is configured for normal measurement performance, see TS 36.133 [16].

#### –	MeasObjectWLAN

The IE MeasObjectWLAN specifies information applicable for inter-RAT WLAN measurements. E-UTRAN configures at least one WLAN identifier in the MeasObjectWLAN.

-- ASN1START

MeasObjectWLAN-r13 ::=	SEQUENCE {

carrierFreq-r13					CHOICE {

bandIndicatorListWLAN-r13		SEQUENCE (SIZE (1..maxWLAN-Bands-r13)) OF WLAN-BandIndicator-r13,

carrierInfoListWLAN-r13			SEQUENCE (SIZE (1..maxWLAN-CarrierInfo-r13)) OF WLAN-CarrierInfo-r13

}		OPTIONAL,	-- Need ON

wlan-ToAddModList-r13				WLAN-Id-List-r13			OPTIONAL,	-- Need ON

wlan-ToRemoveList-r13				WLAN-Id-List-r13			OPTIONAL,	-- Need ON

...

}

WLAN-BandIndicator-r13 ::=	ENUMERATED {band2dot4, band5, band60-v1430, spare5, spare4, spare3, spare2, spare1, ...}

-- ASN1STOP

MeasObjectWLAN field descriptions

bandIndicatorListWLAN Includes the list of WLAN bands. Value band2dot4 indicates the 2.4GHz band, value band5 indicates the 5GHz band and value band60 indicates the 60GHz band.

carrierInfoListWLAN Includes the list of WLAN carrier information for the measurement object.

wlan-ToAddModList Includes the list of WLAN identifiers to be added to the measurement configuration.

wlan-ToRemoveList Includes the list of WLAN identifiers to be removed from the measurement configuration.

#### –	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency and inter- RAT mobility and for idle/inactive measurements.

MeasResults information element

-- ASN1START

MeasResults ::=						SEQUENCE {

measId								MeasId,

measResultPCell						SEQUENCE {

rsrpResult							RSRP-Range,

rsrqResult							RSRQ-Range

},

measResultNeighCells				CHOICE {

measResultListEUTRA					MeasResultListEUTRA,

measResultListUTRA					MeasResultListUTRA,

measResultListGERAN					MeasResultListGERAN,

measResultsCDMA2000					MeasResultsCDMA2000,

...,

measResultNeighCellListNR-r15			MeasResultCellListNR-r15

}																		OPTIONAL,

...,

[[	measResultForECID-r9				MeasResultForECID-r9			OPTIONAL

]],

[[	locationInfo-r10					LocationInfo-r10				OPTIONAL,

measResultServFreqList-r10			MeasResultServFreqList-r10		OPTIONAL

]],

[[	measId-v1250						MeasId-v1250					OPTIONAL,

measResultPCell-v1250				RSRQ-Range-v1250				OPTIONAL,

measResultCSI-RS-List-r12			MeasResultCSI-RS-List-r12		OPTIONAL

]],

[[	measResultForRSSI-r13				MeasResultForRSSI-r13			OPTIONAL,

measResultServFreqListExt-r13		MeasResultServFreqListExt-r13	OPTIONAL,

measResultSSTD-r13					MeasResultSSTD-r13				OPTIONAL,

measResultPCell-v1310				SEQUENCE {

rs-sinr-Result-r13					RS-SINR-Range-r13

}																	OPTIONAL,

ul-PDCP-DelayResultList-r13			UL-PDCP-DelayResultList-r13		OPTIONAL,

measResultListWLAN-r13				MeasResultListWLAN-r13			OPTIONAL

]],

[[	measResultPCell-v1360				RSRP-Range-v1360				OPTIONAL

]],

[[	measResultListCBR-r14				MeasResultListCBR-r14			OPTIONAL,

measResultListWLAN-r14				MeasResultListWLAN-r14			OPTIONAL

]],

[[	measResultServFreqListNR-r15		MeasResultServFreqListNR-r15	OPTIONAL,

measResultCellListSFTD-r15		MeasResultCellListSFTD-r15			OPTIONAL

]],

[[	logMeasResultListBT-r15				LogMeasResultListBT-r15			OPTIONAL,

logMeasResultListWLAN-r15			LogMeasResultListWLAN-r15		OPTIONAL,

measResultSensing-r15			MeasResultSensing-r15				OPTIONAL,

heightUE-r15						INTEGER (-400..8880)		OPTIONAL

]],

[[	ul-PDCP-DelayValueResultList-r16		UL-PDCP-DelayValueResultList-r16	OPTIONAL,

measResultForRSSI-NR-r16				MeasResultForRSSI-NR-r16		OPTIONAL

]]

}

MeasResultListEUTRA ::=				SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=	SEQUENCE {

physCellId							PhysCellId,

cgi-Info							SEQUENCE {

cellGlobalId						CellGlobalIdEUTRA,

trackingAreaCode					TrackingAreaCode,

plmn-IdentityList					PLMN-IdentityList2				OPTIONAL

}															OPTIONAL,

measResult							SEQUENCE {

rsrpResult							RSRP-Range						OPTIONAL,

rsrqResult							RSRQ-Range						OPTIONAL,

...,

[[	additionalSI-Info-r9				AdditionalSI-Info-r9		OPTIONAL

]],

[[	primaryPLMN-Suitable-r12			ENUMERATED {true}			OPTIONAL,

measResult-v1250					RSRQ-Range-v1250			OPTIONAL

]],

[[	rs-sinr-Result-r13					RS-SINR-Range-r13			OPTIONAL,

cgi-Info-v1310						SEQUENCE {

freqBandIndicator-r13				FreqBandIndicator-r11		OPTIONAL,

multiBandInfoList-r13				MultiBandInfoList-r11		OPTIONAL,

freqBandIndicatorPriority-r13		ENUMERATED {true}			OPTIONAL

}																OPTIONAL

]],

[[

measResult-v1360					RSRP-Range-v1360					OPTIONAL

]],

[[

cgi-Info-5GC-r15		SEQUENCE (SIZE (1..maxPLMN-r11)) OF CellAccessRelatedInfo-5GC-r15		OPTIONAL

]]

}

}

MeasResultListIdle-r15	::= SEQUENCE (SIZE (1..maxIdleMeasCarriers-r15)) OF MeasResultIdle-r15

MeasResultIdle-r15	::= SEQUENCE {

measResultServingCell-r15					SEQUENCE {

rsrpResult-r15					RSRP-Range,

rsrqResult-r15					RSRQ-Range-r13

},

measResultNeighCells-r15		CHOICE {

measResultIdleListEUTRA-r15		MeasResultIdleListEUTRA-r15,

...

}																	OPTIONAL,

...

}

MeasResultIdleListEUTRA-r15 ::=	SEQUENCE (SIZE (1..maxCellMeasIdle-r15)) OF MeasResultIdleEUTRA-r15

MeasResultIdleEUTRA-r15 ::=	SEQUENCE {

carrierFreq-r15						ARFCN-ValueEUTRA-r9,

physCellId-r15						PhysCellId,

measResult-r15						SEQUENCE {

rsrpResult-r15						RSRP-Range,

rsrqResult-r15						RSRQ-Range-r13

},

...

}

MeasResultListExtIdle-r16 ::= SEQUENCE(SIZE (1..maxIdleMeasCarriersExt-r16)) OF MeasResultIdleListEUTRA-r15

MeasResultListIdleNR-r16	::= SEQUENCE(SIZE (1..maxIdleMeasCarriers-r16)) OF MeasResultIdleNR-r16

MeasResultIdleNR-r16 ::=		SEQUENCE {

carrierFreqNR-r16						ARFCN-ValueNR-r15,

measResultsPerCellListIdleNR-r16	SEQUENCE (SIZE (1..maxCellMeasIdle-r15)) OF MeasResultsPerCellIdleNR-r16,

...

}

MeasResultsPerCellIdleNR-r16 ::=	SEQUENCE {

physCellIdNR-r16					PhysCellIdNR-r15,

measIdleResultNR-r16					SEQUENCE {

rsrpResultNR-r16						RSRP-RangeNR-r15				OPTIONAL,

rsrqResultNR-r16						RSRQ-RangeNR-r15				OPTIONAL,

resultRS-IndexList-r16				ResultsPerSSB-IndexList-r16		OPTIONAL

},

...

}

ResultsPerSSB-IndexList-r16 ::=	SEQUENCE (SIZE (1..maxRS-IndexReport-r15)) OF ResultsPerSSB-IndexIdle-r16

ResultsPerSSB-IndexIdle-r16 ::=		SEQUENCE {

ssb-Index-r16							RS-IndexNR-r15,

ssb-Results-r16							SEQUENCE {

ssb-RSRP-Result-r16						RSRP-RangeNR-r15			OPTIONAL,

ssb-RSRQ-Result-r16						RSRQ-RangeNR-r15			OPTIONAL

}																		OPTIONAL

}

MeasResultServFreqListNR-r15 ::=	SEQUENCE (SIZE (1..maxServCell-r13)) OF MeasResultServFreqNR-r15

MeasResultServFreqNR-r15 ::=		SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

measResultSCell-r15					MeasResultCellNR-r15				OPTIONAL,

measResultBestNeighCell-r15			MeasResultCellNR-r15				OPTIONAL,

...

}

MeasResultCellListNR-r15::=		SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultCellNR-r15

MeasResultCellNR-r15 ::=			SEQUENCE {

pci-r15								PhysCellIdNR-r15,

measResultCell-r15					MeasResultNR-r15,

measResultRS-IndexList-r15			MeasResultSSB-IndexList-r15				OPTIONAL,

...,

[[	cgi-Info-r15						CGI-InfoNR-r15				OPTIONAL

]]

}

MeasResultNR-r15 ::=				SEQUENCE {

rsrpResult-r15						RSRP-RangeNR-r15						OPTIONAL,

rsrqResult-r15						RSRQ-RangeNR-r15						OPTIONAL,

rs-sinr-Result-r15					RS-SINR-RangeNR-r15						OPTIONAL,

...

}

MeasResultSSB-IndexList-r15::=		SEQUENCE (SIZE (1..maxRS-IndexReport-r15)) OF MeasResultSSB-Index-r15

MeasResultSSB-Index-r15 ::=		SEQUENCE {

ssb-Index-r15						RS-IndexNR-r15,

measResultSSB-Index-r15				MeasResultNR-r15					OPTIONAL,

...

}

MeasResultServFreqList-r10 ::=	SEQUENCE (SIZE (1..maxServCell-r10)) OF MeasResultServFreq-r10

MeasResultServFreqListExt-r13 ::=	SEQUENCE (SIZE (1..maxServCell-r13)) OF MeasResultServFreq-r13

MeasResultServFreq-r10 ::=			SEQUENCE {

servFreqId-r10						ServCellIndex-r10,

measResultSCell-r10					SEQUENCE {

rsrpResultSCell-r10					RSRP-Range,

rsrqResultSCell-r10					RSRQ-Range

}															OPTIONAL,

measResultBestNeighCell-r10			SEQUENCE {

physCellId-r10						PhysCellId,

rsrpResultNCell-r10					RSRP-Range,

rsrqResultNCell-r10					RSRQ-Range

}															OPTIONAL,

...,

[[	measResultSCell-v1250				RSRQ-Range-v1250	OPTIONAL,

measResultBestNeighCell-v1250		RSRQ-Range-v1250	OPTIONAL

]],

[[	measResultSCell-v1310				SEQUENCE {

rs-sinr-Result-r13					RS-SINR-Range-r13

}		OPTIONAL,

measResultBestNeighCell-v1310		SEQUENCE {

rs-sinr-Result-r13					RS-SINR-Range-r13

}		OPTIONAL

]]

}

MeasResultServFreq-r13 ::=			SEQUENCE {

servFreqId-r13						ServCellIndex-r13,

measResultSCell-r13					SEQUENCE {

rsrpResultSCell-r13					RSRP-Range,

rsrqResultSCell-r13					RSRQ-Range-r13,

rs-sinr-Result-r13					RS-SINR-Range-r13	OPTIONAL

}															OPTIONAL,

measResultBestNeighCell-r13			SEQUENCE {

physCellId-r13						PhysCellId,

rsrpResultNCell-r13					RSRP-Range,

rsrqResultNCell-r13					RSRQ-Range-r13,

rs-sinr-Result-r13					RS-SINR-Range-r13	OPTIONAL

}															OPTIONAL,

...,

[[	measResultBestNeighCell-v1360		SEQUENCE {

rsrpResultNCell-v1360				RSRP-Range-v1360

}														OPTIONAL

]]

}

MeasResultCSI-RS-List-r12 ::=	SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultCSI-RS-r12

MeasResultCSI-RS-r12 ::=		SEQUENCE {

measCSI-RS-Id-r12				MeasCSI-RS-Id-r12,

csi-RSRP-Result-r12				CSI-RSRP-Range-r12,

...

}

MeasResultListUTRA ::=				SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultUTRA

MeasResultUTRA ::=	SEQUENCE {

physCellId							CHOICE {

fdd									PhysCellIdUTRA-FDD,

tdd									PhysCellIdUTRA-TDD

},

cgi-Info							SEQUENCE {

cellGlobalId						CellGlobalIdUTRA,

locationAreaCode					BIT STRING (SIZE (16))			OPTIONAL,

routingAreaCode						BIT STRING (SIZE (8))			OPTIONAL,

plmn-IdentityList					PLMN-IdentityList2				OPTIONAL

}															OPTIONAL,

measResult							SEQUENCE {

utra-RSCP							INTEGER (-5..91)				OPTIONAL,

utra-EcN0							INTEGER (0..49)					OPTIONAL,

...,

[[	additionalSI-Info-r9				AdditionalSI-Info-r9				OPTIONAL

]],

[[	primaryPLMN-Suitable-r12			ENUMERATED {true}			OPTIONAL

]]

}

}

MeasResultListGERAN ::=				SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultGERAN

MeasResultGERAN ::=	SEQUENCE {

carrierFreq							CarrierFreqGERAN,

physCellId							PhysCellIdGERAN,

cgi-Info							SEQUENCE {

cellGlobalId						CellGlobalIdGERAN,

routingAreaCode						BIT STRING (SIZE (8))			OPTIONAL

}																		OPTIONAL,

measResult							SEQUENCE {

rssi								INTEGER (0..63),

...

}

}

MeasResultsCDMA2000 ::=				SEQUENCE {

preRegistrationStatusHRPD			BOOLEAN,

measResultListCDMA2000				MeasResultListCDMA2000

}

MeasResultListCDMA2000 ::=			SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultCDMA2000

MeasResultCDMA2000 ::=	SEQUENCE {

physCellId							PhysCellIdCDMA2000,

cgi-Info							CellGlobalIdCDMA2000				OPTIONAL,

measResult							SEQUENCE {

pilotPnPhase						INTEGER	(0..32767)				OPTIONAL,

pilotStrength						INTEGER (0..63),

...

}

}

MeasResultListWLAN-r13 ::=		SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultWLAN-r13

MeasResultListWLAN-r14 ::=		SEQUENCE (SIZE (1..maxWLAN-Id-Report-r14)) OF MeasResultWLAN-r13

MeasResultWLAN-r13 ::=	SEQUENCE {

wlan-Identifiers-r13					WLAN-Identifiers-r12,

carrierInfoWLAN-r13						WLAN-CarrierInfo-r13	OPTIONAL,

bandWLAN-r13							WLAN-BandIndicator-r13	OPTIONAL,

rssiWLAN-r13							WLAN-RSSI-Range-r13,

availableAdmissionCapacityWLAN-r13		INTEGER (0..31250)		OPTIONAL,

backhaulDL-BandwidthWLAN-r13			WLAN-backhaulRate-r12	OPTIONAL,

backhaulUL-BandwidthWLAN-r13			WLAN-backhaulRate-r12	OPTIONAL,

channelUtilizationWLAN-r13				INTEGER (0..255)		OPTIONAL,

stationCountWLAN-r13					INTEGER (0..65535)		OPTIONAL,

connectedWLAN-r13						ENUMERATED {true}		OPTIONAL,

...

}

MeasResultListCBR-r14 ::=			SEQUENCE (SIZE (1..maxCBR-Report-r14)) OF MeasResultCBR-r14

MeasResultCBR-r14 ::=	SEQUENCE {

poolIdentity-r14		SL-V2X-TxPoolReportIdentity-r14,

cbr-PSSCH-r14			SL-CBR-r14,

cbr-PSCCH-r14			SL-CBR-r14				OPTIONAL

}

MeasResultSensing-r15 ::=	SEQUENCE {

sl-SubframeRef-r15			INTEGER (0..10239),

sensingResult-r15			SEQUENCE (SIZE (0..400)) OF SensingResult-r15

}

SensingResult-r15 ::=	SEQUENCE {

resourceIndex-r15			INTEGER (1..2000)

}

MeasResultForECID-r9 ::=		SEQUENCE {

ue-RxTxTimeDiffResult-r9				INTEGER (0..4095),

currentSFN-r9							BIT STRING (SIZE (10))

}

PLMN-IdentityList2 ::=				SEQUENCE (SIZE (1..5)) OF PLMN-Identity

AdditionalSI-Info-r9 ::=			SEQUENCE {

csg-MemberStatus-r9				ENUMERATED {member}				OPTIONAL,

csg-Identity-r9						CSG-Identity						OPTIONAL

}

MeasResultForRSSI-r13 ::=			SEQUENCE {

rssi-Result-r13							RSSI-Range-r13,

channelOccupancy-r13					INTEGER (0..100),

...

}

MeasResultForRSSI-NR-r16 ::=		SEQUENCE {

rssi-ResultNR-r16					RSSI-Range-r13,

channelOccupancyNR-r16			INTEGER (0..100),

...

}

UL-PDCP-DelayResultList-r13 ::=		SEQUENCE (SIZE (1..maxQCI-r13)) OF UL-PDCP-DelayResult-r13

UL-PDCP-DelayResult-r13 ::=			SEQUENCE {

qci-Id-r13							ENUMERATED {qci1, qci2, qci3, qci4, spare4, spare3, spare2, spare1},

excessDelay-r13						INTEGER (0..31),

...

}

UL-PDCP-DelayValueResultList-r16 ::=		SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-DelayValueResult-r16

UL-PDCP-DelayValueResult-r16 ::=		SEQUENCE {

drb-Id-r16								DRB-Identity,

averageDelay-r16						INTEGER (0..10000),

...

}

CGI-InfoNR-r15 ::=					SEQUENCE {

plmn-IdentityInfoList-r15			PLMN-IdentityInfoListNR-r15			OPTIONAL,

frequencyBandList-r15				MultiFrequencyBandListNR-r15		OPTIONAL,

noSIB1-r15							SEQUENCE {

ssb-SubcarrierOffset-r15				INTEGER (0..15),

pdcch-ConfigSIB1-r15					INTEGER (0..255)

}																		OPTIONAL,

...

}

CellIdentityNR-r15 ::=				BIT STRING (SIZE (36))

PLMN-IdentityListNR-r15 ::=			SEQUENCE (SIZE (1.. maxPLMN-NR-r15)) OF PLMN-Identity

PLMN-IdentityInfoListNR-r15 ::=		SEQUENCE (SIZE (1..maxPLMN-NR-r15)) OF PLMN-IdentityInfoNR-r15

PLMN-IdentityInfoNR-r15 ::=			SEQUENCE {

plmn-IdentityList-r15				PLMN-IdentityListNR-r15,

trackingAreaCode-r15				TrackingAreaCodeNR-r15			OPTIONAL,

ran-AreaCode-r15					RAN-AreaCode-r15				OPTIONAL,

cellIdentity-r15					CellIdentityNR-r15

}

TrackingAreaCodeNR-r15 ::=			BIT STRING (SIZE (24))

-- ASN1STOP

MeasResults field descriptions

availableAdmissionCapacityWLAN Indicates the available admission capacity of WLAN as defined in IEEE 802.11-2012 [67].

averageDelay Indicates average delay for the packets during the reporting period, as specified in TS 38.314 [103]. Value 0 corresponds to 0 millisecond, value 1 corresponds to 0.1 millisecond, value 2 corresponds to 0.2 millisecond, and so on.

backhaulDL-BandwidthWLAN Indicates the backhaul available downlink bandwidth of WLAN, equal to Downlink Speed times Downlink Load defined in Wi-Fi Alliance Hotspot 2.0 [76].

backhaulUL-BandwidthWLAN Indicates the backhaul available uplink bandwidth of WLAN, equal to Uplink Speed times Uplink Load defined in Wi-Fi Alliance Hotspot 2.0 [76].

bandWLAN Indicates the WLAN band.

carrierFreq Indicates the E-UTRA carrier frequency. Within MeasResultIdleListEUTRA-r15, UE only includes measurements with the same carrier frequency.

carrierFreqNR Indicates the NR carrier frequency.

carrierInfoWLAN Indicates the WLAN channel information.

cbr-PSSCH Indicates the CBR measurement results on the PSSCH of the pool indicated by poolIdentity. If adjacencyPSCCH-PSSCH is set to TRUE for the pool indicated by pooIIdentity, this field indicates the CBR measurement of both the PSSCH and PSCCH resources which are measured together.

cbr-PSCCH Indicates the CBR measurement results on the PSCCH of the pool indicated by poolIdentity. This field is only included if adjacencyPSCCH-PSSCH is set to FALSE for the pool indicated by pooIIdentity.

channelOccupancy Indicates the percentage of samples when the RSSI was above the configured channelOccupancyThreshold for the associated reportConfig.

channelUtilizationWLAN Indicates WLAN channel utilization as defined in IEEE 802.11-2012 [67].

connectedWLAN Indicates whether the UE is connected to the WLAN for which the measurement results are applicable.

csg-MemberStatus Indicates whether or not the UE is a member of the CSG of the neighbour cell.

currentSFN Indicates the current system frame number when receiving the UE Rx-Tx time difference measurement results from lower layer.

drb-Id Indicates the identity of DRB for which UL PDCP Packet Delay value is provided, according to TS 38.314 [103].

excessDelay Indicates excess queueing delay ratio in UL, according to excess delay ratio measurement report mapping table, as defined in TS 36.314 [71], Table 4.2.1.1.1-1.

heightUE Indicates height of the UE in meters relative to the sea level. Value 0 corresponds to sea level (i.e., negative value indicates depth of the UE below sea level). Value -400 corresponds to -400 m, value -399 corresponds to -399 m and so on.

locationAreaCode A fixed length code identifying the location area within a PLMN, as defined in TS 23.003 [27].

measId Identifies the measurement identity for which the reporting is being performed. If the measId-v1250 is included, the measId (i.e. without a suffix) is ignored by eNB.

measIdleResultNR Idle/inactive measurement results for an NR cell (optionally including beam level measurements).

measResult Measured result of an E-UTRA cell; Measured result of a UTRA cell; Measured result of a GERAN cell or frequency; Measured result of a CDMA2000 cell; Measured result of a WLAN; Measured result of UE Rx–Tx time difference; Measured result of UE SFN, radio frame and subframe timing difference; or Measured result of RSSI and channel occupancy.

measResultCSI-RS-List Measured results of the CSI-RS resources in discovery signals measurement.

measResultListCDMA2000 List of measured results for the maximum number of reported best cells for a CDMA2000 measurement identity.

measResultListEUTRA List of measured results for the maximum number of reported best cells for an E-UTRA measurement identity. For UE supporting CE Mode B, when CE mode B is not restricted by upper layers, measResult-v1360 is reported if the measured RSRP is less than -140 dBm.

measResultListGERAN List of measured results for the maximum number of reported best cells or frequencies for a GERAN measurement identity.

measResultListIdle List of measured results for E-UTRA idle/inactive measurements.

measResultListIdleNR List of measured results for NR idle/inactive measurements.

measResultListSFTD List of measured SFTD results for the reported cells for a NR measurement identity.

measResultListUTRA List of measured results for the maximum number of reported best cells for a UTRA measurement identity.

measResultListWLAN List of measured results for the maximum number of reported best WLAN outside the WLAN mobility set and connected WLAN, if any, for a WLAN measurement identity.

measResultPCell Measured result of the PCell. For BL UEs or UEs in CE, when operating in CE Mode B, measResultPCell-v1360 is reported if the measured RSRP is less than -140 dBm. If sending of the MeasurementReport message is triggered by a measurement configured by the field sl-ConfigDedicatedEUTRA that was received within an NR RRCReconfiguration message (i.e. CBR measurements), measResultPCell is not applicable, its contents is invalid and ignored by the network.

measResultsCDMA2000 Contains the CDMA2000 HRPD pre-registration status and the list of CDMA2000 measurements.

measResultServFreqList Measured results of the serving frequencies: the measurement result of each SCell, if any, and of the best neighbouring cell on each serving frequency. For UE supporting CE Mode B, when CE mode B is not restricted by upper layers, measResultBestNeighCell-v1360 is reported if the measured RSRP is less than -140 dBm.

measResultServingCell Measured results of the serving cell (i.e., PCell) from idle/inactive measurements.

measResultsPerCellListIdleNR List of idle/inactive measured results for the maximum number of reported best cells for a given NR carrier.

noSIB1 Contains ssb-SubcarrierOffset and pdcch-ConfigSIB1 fields acquired by the UE from MIB of the cell for which report CGI procedure was requested by the network in case SIB1 was not broadcast by the cell.

pilotPnPhase Indicates the arrival time of a CDMA2000 pilot, measured relative to the UE's time reference in units of PN chips, see C.S0005 [25]. This information is used in either SRVCC handover or enhanced 1xRTT CS fallback procedure to CDMA2000 1xRTT.

pilotStrength CDMA2000 Pilot Strength, the ratio of pilot power to total power in the signal bandwidth of a CDMA2000 Forward Channel. See C.S0005 [25] for CDMA2000 1xRTT and C.S0024 [26] for CDMA2000 HRPD.

poolIdentity The identity of the transmission resource pool which is corresponding to the poolReportId configured in a resource pool for V2X sidelink communication.

plmn-IdentityList The list of PLMN Identity read from broadcast information when the multiple PLMN Identities are broadcast.

preRegistrationStatusHRPD Set to TRUE if the UE is currently pre-registered with CDMA2000 HRPD. Otherwise set to FALSE. This can be ignored by the eNB for CDMA2000 1xRTT.

qci-Id Indicates QCI value for which excessDelay is provided, according to TS 36.314 [71].

resourceIndex Indicates the available resource candidates within the [T1, T2] window as specified in TS 36.213 [23]. clause 14.1.1.6. Value 1 indicates the resource candidate on the subframe indicated by sl-SubframeRef, from subchannel 0 to sensingSubchannelNumber-1. Value 2 indicates the resource candidate on the first subframe following the subframe indicated by sl-SubframeRef, from subchannel 0 to sensingSubchannelNumber-1 (Value 101 indicates the resource candidate on the subframe indicated by sl-SubframeRef, from subchannel 1 to sensingSubchannelNumber, if the numSubchannel of the resource pool is larger than sensingSubchannelNumber) and so on.

resultRS-IndexList Beam level measurement results (indexes and optionally, beam measurements).

routingAreaCode The RAC identity read from broadcast information, as defined in TS 23.003 [27].

rsrpResult Measured RSRP result of an E-UTRA cell. The rsrpResult is only reported if configured by the eNB.

rsrpResultNR Measured RSRP result of an NR cell. The rsrpResultNR is only reported if configured by the eNB.

rsrqResult Measured RSRQ result of an E-UTRA cell. The rsrqResult is only reported if configured by the eNB. If the measurement is performed in RRC_CONNECTED and measurements based on RSS is enabled in the cell in measRSS-DedicatedConfig-r16, E-UTRAN ignores rsrqResult.

rsrqResultNR Measured RSRQ result of an NR cell. The rsrqResultNR is only reported if configured by the eNB.

rssi GERAN Carrier RSSI. RXLEV is mapped to a value between 0 and 63, TS 45.008 [28]. When mapping the RXLEV value to the RSSI bit string, the first/leftmost bit of the bit string contains the most significant bit.

rssi-Result Measured RSSI result in dBm.

rs-sinr-Result Measured RS-SINR result of an E-UTRA or NR cell. The rs-sinr-Result is only reported if configured by the eNB.

rssiWLAN Measured WLAN RSSI result in dBm.

sl-SubframeRef Indicates the subframe corresponding to n+T1 used to obtain the sensing measurement results (see TS 36.213 [23]). Specifically, the value indicates the timing offset with respect to subframe#0 of DFN#0 in milliseconds.

stationCountWLAN Indicates the total number stations currently associated with this WLAN as defined in IEEE 802.11-2012 [67].

ue-RxTxTimeDiffResult UE Rx-Tx time difference measurement result of the PCell, provided by lower layers. If ue-RxTxTimeDiffPeriodicalTDD-r13 is set to TRUE, the measurement mapping is according to EUTRAN TDD UE Rx-Tx time difference report mapping in TS 36.133 [16] and measurement result includes NTAoffset, else the measurement mapping is according to EUTRAN FDD UE Rx-Tx time difference report mapping in TS 36.133 [16].

utra-EcN0 According to CPICH_Ec/No in TS 25.133 [29] for FDD. Fourteen spare values. The field is not present for TDD.

utra-RSCP According to CPICH_RSCP in TS 25.133 [29] for FDD and P-CCPCH_RSCP in TS 25.123 [30] for TDD. Thirty-one spare values.

wlan-Identifiers Indicates the WLAN parameters used for identification of the WLAN for which the measurement results are applicable.

#### –	MeasResultCellSFTD

The IE MeasResultCellSFTD consists of SFN and radio frame boundary difference between the PCell and an NR cell as specified in TS 38.215 [89] and TS 38.133 [84].

MeasResultCellSFTD information element

-- ASN1START

MeasResultCellListSFTD-r15 ::=			SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultCellSFTD-r15

MeasResultCellSFTD-r15 ::=				SEQUENCE {

physCellId-r15							PhysCellIdNR-r15,

sfn-OffsetResult-r15					INTEGER (0..1023),

frameBoundaryOffsetResult-r15			INTEGER (-30720..30719),

rsrpResult-r15							RSRP-RangeNR-r15						OPTIONAL

}

-- ASN1STOP

| MeasResultCellSFTD field descriptions |
| --- |
| physCellId Indicates the physical layer identity (PCI) of an NR cell. |  |
| sfn-OffsetResult Indicates the SFN difference between the PCell and the NR cell as an integer value according to TS 38.215 [89]. |
| frameBoundaryOffsetResult Indicates the frame boundary difference between the PCell and the NR cell as an integer value according to TS 38.215 [89]. |
| rsrpResult Measured RSRP result of an NR cell. |

#### –	MeasResultSCG-FailureMRDC

The IE MeasResultSCG-FailureMRDC is used to provide measurement information concerning E-UTRA measurements upon SCG failure detected by a UE configured with NE-DC.

MeasResultSCG-FailureMRDC information element

-- ASN1START

MeasResultSCG-FailureMRDC-r15 ::=	SEQUENCE {

measResultFreqListEUTRA-r15		MeasResultList3EUTRA-r15,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL

]]

}

MeasResultList3EUTRA-r15 ::=		SEQUENCE (SIZE (1..maxFreq)) OF MeasResult3EUTRA-r15

MeasResult3EUTRA-r15 ::=			SEQUENCE {

carrierFreq-r15						ARFCN-ValueEUTRA-r9,

measResultServingCell-r15			MeasResultEUTRA					OPTIONAL,

measResultNeighCellList-r15		MeasResultListEUTRA				OPTIONAL,

...

}

-- ASN1STOP

#### –	MeasResultSSTD

The IE MeasResultSSTD consists of SFN, radio frame and subframe boundary difference between the PCell and the PSCell as specified in TS 36.214 [48] and TS 36.133 [16].

MeasResultSSTD information element

-- ASN1START

MeasResultSSTD-r13 ::=						SEQUENCE {

sfn-OffsetResult-r13						INTEGER (0..1023),

frameBoundaryOffsetResult-r13				INTEGER (-5..4),

subframeBoundaryOffsetResult-r13			INTEGER (0..127)

}

-- ASN1STOP

MeasResultSSTD field descriptions

sfn-OffsetResult Indicates the SFN difference between the PCell and the PSCell as an integer value according to TS 36.214 [48].

frameBoundaryOffsetResult Indicates the frame boundary difference between the PCell and the PSCell as an integer value according to TS 36.214 [48].

subframeBoundaryOffsetResult Indicates the subframe boundary difference between the PCell and the PSCell as an integer value according to the mapping table in TS 36.133 [16].

#### –	MeasScaleFactor

The IE MeasScaleFactor specifies the factor for scaling the measurement performance requirements in TS 36.133 [16].

MeasScaleFactor information element

-- ASN1START

MeasScaleFactor-r12 ::=			ENUMERATED {sf-EUTRA-cf1, sf-EUTRA-cf2}

-- ASN1STOP

NOTE:	If the reducedMeasPerformance is not included in any measObjectEUTRA or measObjectUTRA and the measScaleFactor is included in the measConfig, E-UTRAN can configure any of the values for the measScaleFactor as specified in TS 36.133 [16].

#### –	MeasSensing-Config

The IE MeasSensing-Config specifies the input factors for sensing measurement as specified in TS 36.213 [23].

MeasSensing-Config information element

-- ASN1START

MeasSensing-Config-r15 ::=			SEQUENCE {

sensingSubchannelNumber-r15		INTEGER (1..20),

sensingPeriodicity-r15			ENUMERATED {ms20, ms50, ms100, ms200,

ms300, ms400, ms500, ms600,

ms700, ms800, ms900, ms1000},

sensingReselectionCounter-r15	INTEGER (5..75),

sensingPriority-r15				INTEGER (1..8)

}

-- ASN1STOP

MeasSensing-Config field descriptions

sensingReselectionCounter Indicate the value of SL_RESOURCE_RESELECTION_COUNTER, which is used to derive, as specified in TS 36.213 [23], clause 14.1.1.6.

sensingSubchannelNumber Indicate the number of sub-channels, i.e., parameter, as specified in TS 36.213 [23], clause 14.1.1.6.

sensingPeriodicity Indicate the resource reservation interval, i.e., parameter, as specified in TS 36.213 [23], clause 14.1.1.6.

sensingPriority Indicate the priority, i.e., parameter as specified in TS 36.213 [23], clause 14.1.1.6.

#### –	MTC-SSB-NR

The IE MTC-SSB-NR specifies the SS/PBCH block measurement timing configuration (SMTC) applicable for SSB based NR measurements i.e. the time occasions for performing these measurements, see 5.5.2.13.

MTC-SSB-NR information elements

-- ASN1START

MTC-SSB-NR-r15 ::=	SEQUENCE {

periodicityAndOffset-r15		CHOICE {

sf5-r15						INTEGER (0..4),

sf10-r15						INTEGER (0..9),

sf20-r15						INTEGER (0..19),

sf40-r15						INTEGER (0..39),

sf80-r15						INTEGER (0..79),

sf160-r15					INTEGER (0..159)

},

ssb-Duration-r15					ENUMERATED {sf1, sf2, sf3, sf4, sf5 }

}

MTC-SSB2-LP-NR-r16::= SEQUENCE {

pci-List-r16			SEQUENCE (SIZE (1..maxNrofPCI-PerSMTC-r16)) OF PhysCellIdNR-r15

OPTIONAL,   -- Need OR

periodicity-r16		ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

-- ASN1STOP

MTC-SSB-NR field descriptions

pci-List PCIs that are known to follow this SMTC.

#### –	QuantityConfig

The IE QuantityConfig specifies the measurement quantities and layer 3 filtering coefficients for E-UTRA and inter-RAT measurements.

QuantityConfig information element

-- ASN1START

QuantityConfig ::=					SEQUENCE {

quantityConfigEUTRA					QuantityConfigEUTRA					OPTIONAL,	-- Need ON

quantityConfigUTRA					QuantityConfigUTRA					OPTIONAL,	-- Need ON

quantityConfigGERAN					QuantityConfigGERAN					OPTIONAL,	-- Need ON

quantityConfigCDMA2000				QuantityConfigCDMA2000				OPTIONAL,	-- Need ON

...,

[[	quantityConfigUTRA-v1020		QuantityConfigUTRA-v1020			OPTIONAL	-- Need ON

]],

[[	quantityConfigEUTRA-v1250		QuantityConfigEUTRA-v1250			OPTIONAL	-- Need ON

]],

[[	quantityConfigEUTRA-v1310		QuantityConfigEUTRA-v1310			OPTIONAL,	-- Need ON

quantityConfigWLAN-r13			QuantityConfigWLAN-r13				OPTIONAL	-- Need ON

]],

[[	quantityConfigNRList-r15		QuantityConfigNRList-r15			OPTIONAL	-- Need ON

]]

}

QuantityConfigEUTRA ::=				SEQUENCE {

filterCoefficientRSRP				FilterCoefficient					DEFAULT fc4,

filterCoefficientRSRQ				FilterCoefficient					DEFAULT fc4

}

QuantityConfigEUTRA-v1250 ::=		SEQUENCE {

filterCoefficientCSI-RSRP-r12		FilterCoefficient					OPTIONAL		-- Need OR

}

QuantityConfigEUTRA-v1310 ::=		SEQUENCE {

filterCoefficientRS-SINR-r13		FilterCoefficient					DEFAULT fc4

}

QuantityConfigUTRA ::=				SEQUENCE {

measQuantityUTRA-FDD				ENUMERATED {cpich-RSCP, cpich-EcN0},

measQuantityUTRA-TDD				ENUMERATED {pccpch-RSCP},

filterCoefficient					FilterCoefficient					DEFAULT fc4

}

QuantityConfigUTRA-v1020 ::=		SEQUENCE {

filterCoefficient2-FDD-r10			FilterCoefficient					DEFAULT fc4

}

QuantityConfigGERAN ::=				SEQUENCE {

measQuantityGERAN					ENUMERATED {rssi},

filterCoefficient					FilterCoefficient					DEFAULT fc2

}

QuantityConfigCDMA2000 ::=			SEQUENCE {

measQuantityCDMA2000				ENUMERATED {pilotStrength, pilotPnPhaseAndPilotStrength}

}

QuantityConfigNRList-r15 ::=		SEQUENCE (SIZE (1..maxQuantSetsNR-r15)) OF QuantityConfigNR-r15

QuantityConfigNR-r15 ::=			SEQUENCE {

measQuantityCellNR-r15				QuantityConfigRS-NR-r15,

measQuantityRS-IndexNR-r15			QuantityConfigRS-NR-r15				OPTIONAL

}

QuantityConfigRS-NR-r15 ::=				SEQUENCE {

filterCoeff-RSRP-r15				FilterCoefficient					DEFAULT fc4,

filterCoeff-RSRQ-r15				FilterCoefficient					DEFAULT fc4,

filterCoefficient-SINR-r13			FilterCoefficient					DEFAULT fc4

}

QuantityConfigWLAN-r13 ::=			SEQUENCE {

measQuantityWLAN-r13				ENUMERATED {rssiWLAN},

filterCoefficient-r13				FilterCoefficient					DEFAULT fc4

}

-- ASN1STOP

QuantityConfig field descriptions

filterCoefficient2-FDD Specifies the filtering coefficient used for the UTRAN FDD measurement quantity, which is not included in measQuantityUTRA-FDD, when reportQuantityUTRA-FDD is present in ReportConfigInterRAT.

filterCoefficientCSI-RSRP Specifies the filtering coefficient used for CSI-RSRP.

filterCoefficientRSRP Specifies the filtering coefficient used for RSRP.

filterCoefficientRSRQ Specifies the filtering coefficient used for RSRQ.

filterCoefficientRS-SINR Specifies the filtering coefficient used for RS-SINR.

measQuantityCDMA2000 Measurement quantity used for CDMA2000 measurements. pilotPnPhaseAndPilotStrength is only applicable for MeasObjectCDMA2000 of cdma2000-Type = type1XRTT.

measQuantityRS-IndexNR Specifies L3 filter configurations for measurement results of an NR RS index for a particular RS Type (e.g. SS/PBCH block) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

measQuantityGERAN Measurement quantity used for GERAN measurements.

measQuantityCellINR Specifies L3 filter configurations for measurement results of an NR cell for a particular RS Type (e.g. SS/PBCH block) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

measQuantityUTRA Measurement quantity used for UTRA measurements.

measQuantityWLAN Measurement quantity used for WLAN measurements.

quantityConfigCDMA2000 Specifies quantity configurations for CDMA2000 measurements.

quantityConfigEUTRA Specifies filter configurations for E-UTRA measurements.

quantityConfigGERAN Specifies quantity and filter configurations for GERAN measurements.

quantityConfigUTRA Specifies quantity and filter configurations for UTRA measurements. Field quantityConfigUTRA-v1020 is applicable only when reportQuantityUTRA-FDD is configured.

quantityConfigWLAN Specifies quantity and filter configurations for WLAN measurements.

#### –	ReportConfigEUTRA

The IE ReportConfigEUTRA specifies criteria for triggering of an E-UTRA measurement reporting or conditional reconfiguration (i.e. conditional handover) event. The E-UTRA measurement reporting events concerning CRS are labelled AN with N equal to 1, 2 and so on.

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/ PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/ PSCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2;

Event A6:	Neighbour becomes amount of offset better than SCell.

The E-UTRA measurement reporting events concerning CRS for conditional reconfigurations are labelled AN with N equal to 3 or 5.

CondEvent A3:	Conditional reconfiguration candidate becomes amount of offset better than PCell;

CondEvent A5:	PCell becomes worse than absolute threshold1 AND conditional reconfiguration candidate becomes better than another absolute threshold2;

The E-UTRA measurement reporting events concerning CSI-RS are labelled CN with N equal to 1 and 2.

Event C1:	CSI-RS resource becomes better than absolute threshold;

Event C2:	CSI-RS resource becomes amount of offset better than reference CSI-RS resource.

The E-UTRA measurement reporting events concerning CBR are labelled VN with N equal to 1 and 2.

Event V1:	CBR becomes larger than absolute threshold;

Event V2:	CBR becomes smaller than absolute threshold.

The E-UTRA reporting events concerning Aerial UE height are labelled HN with N equal to 1 and 2.

Event H1:	Aerial UE height becomes higher than absolute threshold;

Event H2:	Aerial UE height becomes lower than absolute threshold.

ReportConfigEUTRA information element

-- ASN1START

ReportConfigEUTRA ::=				SEQUENCE {

triggerType							CHOICE {

event								SEQUENCE {

eventId								CHOICE {

eventA1								SEQUENCE {

a1-Threshold						ThresholdEUTRA

},

eventA2								SEQUENCE {

a2-Threshold						ThresholdEUTRA

},

eventA3								SEQUENCE {

a3-Offset							INTEGER (-30..30),

reportOnLeave						BOOLEAN

},

eventA4								SEQUENCE {

a4-Threshold						ThresholdEUTRA

},

eventA5								SEQUENCE {

a5-Threshold1						ThresholdEUTRA,

a5-Threshold2						ThresholdEUTRA

},

...,

eventA6-r10							SEQUENCE {

a6-Offset-r10						INTEGER (-30..30),

a6-ReportOnLeave-r10				BOOLEAN

},

eventC1-r12							SEQUENCE {

c1-Threshold-r12					ThresholdEUTRA-v1250,

c1-ReportOnLeave-r12				BOOLEAN

},

eventC2-r12							SEQUENCE {

c2-RefCSI-RS-r12					MeasCSI-RS-Id-r12,

c2-Offset-r12						INTEGER (-30..30),

c2-ReportOnLeave-r12				BOOLEAN

},

eventV1-r14							SEQUENCE {

v1-Threshold-r14					SL-CBR-r14

},

eventV2-r14							SEQUENCE {

v2-Threshold-r14					SL-CBR-r14

},

eventH1-r15							SEQUENCE {

h1-ThresholdOffset-r15				INTEGER (0..300),

h1-Hysteresis-r15					INTEGER (1..16)

},

eventH2-r15							SEQUENCE {

h2-ThresholdOffset-r15				INTEGER (0..300),

h2-Hysteresis-r15					INTEGER (1..16)

}

},

hysteresis							Hysteresis,

timeToTrigger						TimeToTrigger

},

periodical								SEQUENCE {

purpose									ENUMERATED {

reportStrongestCells, reportCGI}

}

},

triggerQuantity						ENUMERATED {rsrp, rsrq},

reportQuantity						ENUMERATED {sameAsTriggerQuantity, both},

maxReportCells						INTEGER (1..maxCellReport),

reportInterval						ReportInterval,

reportAmount						ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

...,

[[	si-RequestForHO-r9					ENUMERATED {setup}		OPTIONAL,	-- Cond reportCGI

ue-RxTxTimeDiffPeriodical-r9		ENUMERATED {setup}		OPTIONAL	-- Need OR

]],

[[	includeLocationInfo-r10				ENUMERATED {true}		OPTIONAL,	-- Need OR

reportAddNeighMeas-r10				ENUMERATED {setup}		OPTIONAL	-- Need OR

]],

[[	alternativeTimeToTrigger-r12		CHOICE {

release								NULL,

setup								TimeToTrigger

}													OPTIONAL,	-- Need ON

useT312-r12							BOOLEAN			OPTIONAL,	-- Need ON

usePSCell-r12						BOOLEAN			OPTIONAL,	-- Need ON

aN-Threshold1-v1250					RSRQ-RangeConfig-r12		OPTIONAL,	-- Need ON

a5-Threshold2-v1250					RSRQ-RangeConfig-r12		OPTIONAL,	-- Need ON

reportStrongestCSI-RSs-r12			BOOLEAN			OPTIONAL,	-- Need ON

reportCRS-Meas-r12					BOOLEAN			OPTIONAL,	-- Need ON

triggerQuantityCSI-RS-r12			BOOLEAN			OPTIONAL		-- Need ON

]],

[[	reportSSTD-Meas-r13					BOOLEAN			OPTIONAL,		-- Need ON

rs-sinr-Config-r13					CHOICE {

release								NULL,

setup								SEQUENCE {

triggerQuantity-v1310				ENUMERATED {sinr}		OPTIONAL,	-- Need ON

aN-Threshold1-r13					RS-SINR-Range-r13		OPTIONAL,	-- Need ON

a5-Threshold2-r13					RS-SINR-Range-r13		OPTIONAL,	-- Need ON

reportQuantity-v1310				ENUMERATED {rsrpANDsinr, rsrqANDsinr, all}

}

}																OPTIONAL,	-- Need ON

useWhiteCellList-r13				BOOLEAN						OPTIONAL,	-- Need ON

measRSSI-ReportConfig-r13			MeasRSSI-ReportConfig-r13	OPTIONAL,	-- Need ON

includeMultiBandInfo-r13			ENUMERATED {true}			OPTIONAL,	-- Cond reportCGI

ul-DelayConfig-r13					UL-DelayConfig-r13			OPTIONAL	-- Need ON

]],

[[	ue-RxTxTimeDiffPeriodicalTDD-r13	BOOLEAN						OPTIONAL	-- Need ON

]],

[[

purpose-v1430			ENUMERATED {reportLocation, sidelink, spare2, spare1}

OPTIONAL	-- Need ON

]],

[[

maxReportRS-Index-r15		INTEGER (0..maxRS-IndexReport-r15)	OPTIONAL	-- Need ON

]],

[[	includeBT-Meas-r15				BT-NameListConfig-r15			OPTIONAL,	-- Need ON

includeWLAN-Meas-r15				WLAN-NameListConfig-r15			OPTIONAL,		-- Need ON

purpose-r15				ENUMERATED {sensing}					OPTIONAL,	-- Need ON

numberOfTriggeringCells-r15			INTEGER	(2..maxCellReport)	OPTIONAL,	-- Cond a3a4a5

a4-a5-ReportOnLeave-r15				BOOLEAN						OPTIONAL	-- Cond a4a5

]],

[[ condReconfigurationTriggerEUTRA-r16	CondReconfigurationTriggerEUTRA-r16	OPTIONAL,

-- Need ON

ul-DelayValueConfig-r16				UL-DelayValueConfig-r16		OPTIONAL	-- Need ON

]]

}

CondReconfigurationTriggerEUTRA-r16 ::= SEQUENCE {

condEventId-r16							CHOICE {

condEventA3-r16							SEQUENCE {

a3-Offset-r16								INTEGER (-30..30),

hysteresis-r16								Hysteresis,

timeToTrigger-r16							TimeToTrigger

},

condEventA5-r16							SEQUENCE {

a5-Threshold1-r16							ThresholdEUTRA,

a5-Threshold2-r16							ThresholdEUTRA,

hysteresis-r16								Hysteresis,

timeToTrigger-r16							TimeToTrigger

},

...

}

}

RSRQ-RangeConfig-r12 ::=			CHOICE {

release								NULL,

setup								RSRQ-Range-v1250

}

ThresholdEUTRA ::=					CHOICE{

threshold-RSRP						RSRP-Range,

threshold-RSRQ						RSRQ-Range

}

ThresholdEUTRA-v1250 ::=			CSI-RSRP-Range-r12

MeasRSSI-ReportConfig-r13 ::=	SEQUENCE {

channelOccupancyThreshold-r13			RSSI-Range-r13				OPTIONAL	-- Need OR

}

-- ASN1STOP

| ReportConfigEUTRA field descriptions |
| --- |
| a3-Offset/ a6-Offset/ c2-Offset Offset value to be used in EUTRA measurement report triggering condition for event a3/ a6/ c2, or to be used in conditional reconfiguration trigger condition for cond event a3. The actual value is field value * 0.5 dB. |
| a5-Threshold1/ a5-Threshold2 Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) to be used in conditional reconfiguration trigger condition for cond event a5. In the same condeventA5, the network configures the same quantity for the TriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2. |
| alternativeTimeToTrigger Indicates the time to trigger applicable for cells specified in altTTT-CellsToAddModList of the associated measurement object, if configured |
| aN-ThresholdM/ cN-ThresholdM Threshold to be used in EUTRA measurement report triggering condition for event number aN/ cN. If multiple thresholds are defined for event number aN/ cN, the thresholds are differentiated by M. E-UTRAN configures aN-Threshold1 only for events A1, A2, A4, A5 and a5-Threshold2 only for event A5. |
| c1-ReportOnLeave/ c2-ReportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a CSI-RS resource in csi-RS-TriggeredList, as specified in 5.5.4.1. |
| c2-RefCSI-RS Identity of the CSI-RS resource from the measCSI-RS-ToAddModList of the associated measObject, to be used as the reference CSI-RS resource in EUTRA measurement report triggering condition for event c2. |
| channelOccupancyThreshold RSSI threshold which is used for channel occupancy evaluation. |
| condEventId Choice of conditional reconfiguration event triggered criteria. |
| condReconfigurationTriggerEUTRA Event configured for conditional reconfiguration. If this field is configured, the UE shall ignore the configuration of triggerType, reportQuantity, maxReportCells, reportInterval, and reportAmount. |
| eventId Choice of E-UTRA event triggered reporting criteria. EUTRAN may set this field to eventC1 or eventC2 only if measDS-Config is configured in the associated measObject with one or more CSI-RS resources. The eventC1 and eventC2 are not applicable for the eventId if RS-SINR is configured as triggerQuantity or reportQuantity. |
| h1-Hysteresis, h2-Hysteresis This parameter is used within the entry and leave condition of an event triggered reporting condition for event H1 and event H2. The actual value is field value. If this field is configured UE shall ignore parameter hysteresis. |
| h1-ThresholdOffset, h2-ThresholdOffset An offset value to heightThreshRef to obtain the threshold to be used in EUTRA height report triggering condition for event H1 and event H2. The value for h1-ThresholdOffset and h2-ThresholdOffset is expressed in meters such that granularity is 2meters. Value 0 corresponds to offset value 0m, value 1 corresponds to offset value 2m, value 2 correspond to offset value 4m, and so on. |
| includeMultiBandInfo If this field is present, the UE shall acquire and include multi band information in the measurement report. |  |
| maxReportCells Max number of cells, excluding the serving cell, to include in the measurement report concerning CRS, and max number of CSI-RS resources to include in the measurement report concerning CSI-RS. |
| measRSSI-ReportConfig If this field is present, the UE shall perform measurement reporting for RSSI and channel occupancy and ignore the triggerQuantity, reportQuantity and maxReportCells fields. E-UTRAN sets this field to true only when setting triggerType to periodical and purpose to reportStrongestCells. |
| numberOfTriggeringCells Indicates the number of cells detected that are required to fulfill an event for a measurement report to be triggered. This field is set only for the events concerning neighbor cells, i.e. eventA3, eventA4, eventA5. |
| reportAmount Number of measurement reports applicable for triggerType event as well as for triggerType periodical. In case purpose is set to reportCGI or reportSSTD-Meas is set to true, only value 1 applies. |
| reportCRS-Meas If this field is set to TRUE the UE shall include rsrp, rsrq together with csi-rsrp in the measurement report, if possible. |
| reportOnLeave/ a6-ReportOnLeave/ a4-a5-ReportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1. |
| reportQuantity The quantities to be included in the measurement report. The value both means that both the rsrp and rsrq quantities are to be included in the measurement report. The value rsrpANDsinr and rsrqANDsinr mean that both rsrp and rs-sinr quantities, and both rsrq and rs-sinr quantities are to be included respectively in the measurement report. The value all means that rsrp, rsrq and rs-sinr are to be included in the measurement report. In case triggerQuantityCSI-RS is set to TRUE, only value sameAsTriggerQuantity applies. If reportQuantity-v1310 is configured, the UE only considers this extension (and ignores reportQuantity i.e. without suffix). |
| reportSSTD-Meas If this field is set to true, the UE shall measure SSTD between the PCell and the PSCell as specified in TS 36.214 [48] and ignore the triggerQuantity, reportQuantity and maxReportCells fields. E-UTRAN sets this field to true only when setting triggerType to periodical and purpose to reportStrongestCells. |
| reportStrongestCSI-RSs Indicates that periodical CSI-RS measurement report is performed. EUTRAN configures value TRUE only if measDS-Config is configured in the associated measObject with one or more CSI-RS resources. |
| si-RequestForHO The field applies to the reportCGI functionality, and when the field is included, the UE is allowed to use autonomous gaps in acquiring system information from the neighbour cell, applies a different value for T321, and includes different fields in the measurement report. |
| ThresholdEUTRA For RSRP: RSRP based threshold for event evaluation. The actual value is field value – 140 dBm. For RSRQ: RSRQ based threshold for event evaluation. The actual value is (field value – 40)/2 dB. For RS-SINR: RS-SINR based threshold for event evaluation. The actual value is (field value -46)/2 dB. For CSI-RSRP: CSI-RSRP based threshold for event evaluation. The actual value is field value – 140 dBm. EUTRAN configures the same threshold quantity for all the thresholds of an event. |
| timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report, or to execute the conditional reconfiguration evaluation. |
| triggerQuantity The quantity used to evaluate the triggering condition for the event concerning CRS. EUTRAN sets the value according to the quantity of the ThresholdEUTRA for this event. The values rsrp, rsrq and sinr correspond to Reference Signal Received Power (RSRP), Reference Signal Received Quality (RSRQ) and Reference Signal Signal to Noise and Interference Ratio (RS-SINR), see TS 36.214 [48]. If triggerQuantity-v1310 is configured, the UE only considers this extension (and ignores triggerQuantity i.e. without suffix). |
| triggerQuantityCSI-RS The quantity used to evaluate the triggering condition for the event concerning CSI-RS. The value TRUE corresponds to CSI Reference Signal Received Power (CSI-RSRP), see TS 36.214 [48]. E-UTRAN configures value TRUE if and only if the measurement reporting event concerns CSI-RS. |
| ue-RxTxTimeDiffPeriodical If this field is present, the UE shall perform UE Rx-Tx time difference measurement reporting and ignore the fields triggerQuantity, reportQuantity and maxReportCells. If the field is present, the only applicable values for the corresponding triggerType and purpose are periodical and reportStrongestCells respectively. |
| ue-RxTxTimeDiffPeriodicalTDD If this field is set to TRUE, the UE shall perform UE Rx-Tx time difference measurement reporting according to EUTRAN TDD UE Rx-Tx time difference report mapping in TS 36.133 [16]. If the field is configured, the ue-RxTxTimeDiffPeriodical shall be configured. The field is applicable for TDD only. |
| usePSCell If this field is set to TRUE the UE shall use the PSCell instead of the PCell. E-UTRAN configures value TRUE only for events A3 and A5, see 5.5.4.4 and 5.5.4.6. |
| useT312 If value TRUE is configured, the UE shall use the timer T312 with the value t312 as specified in the corresponding measObject. If the corresponding measObject does not include the timer T312 then the timer T312 is considered as not configured. E-UTRAN configures value TRUE only if triggerType is set to event. |
| useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1. E-UTRAN does not configure the field for events A1, A2, C1 and C2. |
| ul-DelayConfig If the field is present, E-UTRAN configures UL PDCP Packet Delay per QCI measurement and the UE shall ignore the fields triggerQuantity and maxReportCells. The applicable values for the corresponding triggerType and reportInterval are periodical and (one of the) ms1024, ms2048, ms5120 or ms10240 respectively.The reportInterval indicates the periodicity for performing and reporting of UL PDCP Delay per QCI measurement as specified in TS 36.314 [71]. |
| ul-DelayValueConfig If the field is present, the UE shall perform the UL PDCP Packet Delay measurement per DRB as specified in TS 38.314 [103] and the UE shall ignore the fields reportQuantityCell and maxReportCells. The applicable values for the corresponding reportInterval are (one of the) { ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60}. The reportInterval indicates the periodicity for performing and reporting of UL PDCP Packet Delay per DRB measurement as specified in TS 38.314 [103]. |  |

| Conditional presence | Explanation |
| --- | --- |
| reportCGI | The field is optional, need OR, in case purpose is included and set to reportCGI; otherwise the field is not present and the UE shall delete any existing value for this field. |
| a3a4a5 | This field is optional, need OR, in case eventId is set to eventA3 or eventA4 or eventA5; otherwise, this field is not present and the UE shall delete any existing value of this field. |
| a4a5 | This field is optional, need OR, in case eventId is set to eventA4 or eventA5; otherwise, this field is not present and the UE shall delete any existing value of this field. |

#### –	ReportConfigId

The IE ReportConfigId is used to identify a measurement reporting configuration.

ReportConfigId information element

-- ASN1START

ReportConfigId ::=					INTEGER (1..maxReportConfigId)

-- ASN1STOP

#### –	ReportConfigInterRAT

The IE ReportConfigInterRAT specifies criteria for triggering of an inter-RAT measurement reporting event. The inter-RAT measurement reporting events for NR, UTRAN, GERAN and CDMA2000 are labelled BN with N equal to 1, 2 and so on. The inter-RAT measurement reporting events for WLAN are labelled WN with N equal to 1, 2 and so on.

Event B1:	Neighbour becomes better than absolute threshold;

Event B2:	PCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2.

Event W1:	WLAN becomes better than a threshold;

Event W2:	All WLAN inside WLAN mobility set become worse than a threshold1 and a WLAN outside WLAN mobility set becomes better than a threshold2;

Event W3:	All WLAN inside WLAN mobility set become worse than a threshold.

The b1 and b2 event thresholds for CDMA2000 are the CDMA2000 pilot detection thresholds are expressed as an unsigned binary number equal to [-2 x 10 log 10 Ec/Io] in units of 0.5dB, see C.S0005 [25] for details.

ReportConfigInterRAT information element

-- ASN1START

ReportConfigInterRAT ::=			SEQUENCE {

triggerType							CHOICE {

event								SEQUENCE {

eventId								CHOICE {

eventB1								SEQUENCE {

b1-Threshold						CHOICE {

b1-ThresholdUTRA					ThresholdUTRA,

b1-ThresholdGERAN					ThresholdGERAN,

b1-ThresholdCDMA2000				ThresholdCDMA2000

}

},

eventB2								SEQUENCE {

b2-Threshold1						ThresholdEUTRA,

b2-Threshold2						CHOICE {

b2-Threshold2UTRA					ThresholdUTRA,

b2-Threshold2GERAN					ThresholdGERAN,

b2-Threshold2CDMA2000				ThresholdCDMA2000

}

},

...,

eventW1-r13						SEQUENCE {

w1-Threshold-r13			WLAN-RSSI-Range-r13

},

eventW2-r13						SEQUENCE {

w2-Threshold1-r13			WLAN-RSSI-Range-r13,

w2-Threshold2-r13			WLAN-RSSI-Range-r13

},

eventW3-r13						SEQUENCE {

w3-Threshold-r13			WLAN-RSSI-Range-r13

},

eventB1-NR-r15							SEQUENCE {

b1-ThresholdNR-r15					ThresholdNR-r15,

reportOnLeave-r15					BOOLEAN

},

eventB2-NR-r15							SEQUENCE {

b2-Threshold1-r15					ThresholdEUTRA,

b2-Threshold2NR-r15					ThresholdNR-r15,

reportOnLeave-r15					BOOLEAN

}

},

hysteresis						Hysteresis,

timeToTrigger					TimeToTrigger

},

periodical								SEQUENCE {

purpose									ENUMERATED {

reportStrongestCells,

reportStrongestCellsForSON,

reportCGI}

}

},

maxReportCells					INTEGER (1..maxCellReport),

reportInterval					ReportInterval,

reportAmount					ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

...,

[[	si-RequestForHO-r9				ENUMERATED {setup}		OPTIONAL	-- Cond reportCGI

]],

[[	reportQuantityUTRA-FDD-r10		ENUMERATED {both}		OPTIONAL	-- Need OR

]],

[[	includeLocationInfo-r11			BOOLEAN					OPTIONAL	-- Need ON

]],

[[	b2-Threshold1-v1250				CHOICE {

release							NULL,

setup							RSRQ-Range-v1250

}														OPTIONAL	-- Need ON

]],

[[	reportQuantityWLAN-r13			ReportQuantityWLAN-r13	OPTIONAL	-- Need ON

]],

[[	reportAnyWLAN-r14				BOOLEAN					OPTIONAL	-- Need ON

]],

[[	reportQuantityCellNR-r15		ReportQuantityNR-r15	OPTIONAL,	-- Need ON

maxReportRS-Index-r15			INTEGER (0..maxRS-IndexReport-r15)	OPTIONAL,	-- Need ON

reportQuantityRS-IndexNR-r15	ReportQuantityNR-r15	OPTIONAL,	-- Need ON

reportRS-IndexResultsNR			BOOLEAN					OPTIONAL,	-- Need ON

reportSFTD-Meas-r15				ENUMERATED {pSCell, neighborCells }	OPTIONAL	-- Need ON

]],

[[

useAutonomousGapsNR-r16			ENUMERATED {setup}		OPTIONAL,	-- Cond reportCGI-NR

measRSSI-ReportConfigNR-r16		MeasRSSI-ReportConfig-r13	OPTIONAL	-- Need ON

]]

}

ThresholdUTRA ::=					CHOICE{

utra-RSCP							INTEGER (-5..91),

utra-EcN0							INTEGER (0..49)

}

ThresholdGERAN ::=				INTEGER (0..63)

ThresholdCDMA2000 ::=			INTEGER (0..63)

ReportQuantityNR-r15::=						SEQUENCE {

ss-rsrp										BOOLEAN,

ss-rsrq										BOOLEAN,

ss-sinr										BOOLEAN

}

ReportQuantityWLAN-r13 ::=		SEQUENCE {

bandRequestWLAN-r13							ENUMERATED {true}	OPTIONAL,	-- Need OR

carrierInfoRequestWLAN-r13					ENUMERATED {true}	OPTIONAL,	-- Need OR

availableAdmissionCapacityRequestWLAN-r13	ENUMERATED {true}	OPTIONAL,	-- Need OR

backhaulDL-BandwidthRequestWLAN-r13			ENUMERATED {true}	OPTIONAL,	-- Need OR

backhaulUL-BandwidthRequestWLAN-r13			ENUMERATED {true}	OPTIONAL,	-- Need OR

channelUtilizationRequestWLAN-r13			ENUMERATED {true}	OPTIONAL,	-- Need OR

stationCountRequestWLAN-r13					ENUMERATED {true}	OPTIONAL,	-- Need OR

...

}

-- ASN1STOP

| ReportConfigInterRAT field descriptions |
| --- |
| availableAdmissionCapacityRequestWLAN The value true indicates that the UE shall include, if available, WLAN Available Admission Capacity in measurement reports. |
| backhaulDL-BandwidthRequestWLAN The value true indicates that the UE shall include, if available, WLAN Backhaul Downlink Bandwidth in measurement reports. |
| backhaulUL-BandwidthRequestWLAN The value true indicates that the UE shall include, if available, WLAN Backhaul Uplink Bandwidth in measurement reports. |
| bandRequestWLAN The value true indicates that the UE shall include WLAN band in measurement reports. |
| bN-ThresholdM Threshold to be used in inter RAT measurement report triggering condition for event number bN. If multiple thresholds are defined for event number bN, the thresholds are differentiated by M. |
| carrierInfoRequestWLAN The value true indicates that the UE shall include, if available, WLAN Carrier Information in measurement reports. |
| channelUtilizationRequest-WLAN The value true indicates that the UE shall include, if available, WLAN Channel Utilization in measurement reports. |
| eventId Choice of inter-RAT event triggered reporting criteria. |
| maxReportCells Max number of cells, excluding the serving cell, to include in the measurement report. In case purpose is set to reportStrongestCellsForSON only value 1 applies. For inter-RAT WLAN, it is the maximum number of WLANs to include in the measurement report. |
| maxReportRS-Index Max number of RS indices to include in the measurement report. E-UTRAN configures value 0 only if it sets reportRS-IndexResultsNR to FALSE. |
| measRSSI-ReportConfigNR If this field is present, the UE shall perform measurement reporting for RSSI and channel occupancy and ignore the triggerQuantity, reportQuantity and maxReportCells fields. E-UTRAN sets this field to true only when setting triggerType to periodical and purpose to reportStrongestCells. |
| Purpose reportStrongestCellsForSON applies only in case reportConfig is linked to a measObject set to measObjectUTRA or measObjectCDMA2000. |
| reportAmount Number of measurement reports applicable for triggerType event as well as for triggerType periodical. In case purpose is set to reportCGI or reportStrongestCellsForSON only value 1 applies. In case reportSFTD-Meas is configured, only value 1 applies. |
| reportAnyWLAN Indicates UE to report any WLAN AP meeting the triggering requirements, even if it is not included in the corresponding MeasObjectWLAN. |
| reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1. |
| reportQuantityUTRA-FDD The quantities to be included in the UTRA measurement report. The value both means that both the cpich RSCP and cpich EcN0 quantities are to be included in the measurement report. |
| reportRS-IndexResultsNR Indicates whether or not the UE shall report beam measurement result of NR in the measurement report. |
| reportSFTD-Meas If this field is set to pSCell, the UE shall measure SFTD between the PCell and the PSCell as specified in TS 38.215 [89], in this case, the frequency of PSCell is configured in the corresponding measObjectNR. If the field is set to neighborCells, the UE shall measure SFTD between the PCell and the NR cells included in cellsForWhichToReportSFTD (if configured in the corresponding measObjectNR) or between the PCell and up to 3 strongest detected NR cells (if cellsForWhichToReportSFTD is not configured in the corresponding measObjectNR), as specified in TS 38.215 [89]. E-UTRAN only includes this field when setting triggerType to periodical and purpose to reportStrongestCells. If included, the UE shall ignore the maxReportCells field. |
| si-RequestForHO The field applies to the reportCGI functionality, and when the field is included, the UE is allowed to use autonomous gaps in acquiring system information from the neighbour cell, applies a different value for T321, and includes different fields in the measurement report. EUTRAN does not configure the field if reportConfig is linked to a measObject set to measObjectNR. |
| ss-rsrp Indicates whether or not the UE shall report SS-RSRP quantity of NR. |
| ss-rsrq Indicates whether or not the UE shall report SS-RSRQ quantity of NR. |
| ss-sinr Indicates whether or not the UE shall report SS-SINR quantity of NR. |
| stationCountRequestWLAN The value true indicates that the UE shall include, if available, WLAN Station Count in measurement reports. |
| b1-ThresholdGERAN, b2-Threshold2GERAN The actual value is field value – 110 dBm. |
| b1-ThresholdUTRA, b2-Threshold2UTRA utra-RSCP corresponds to CPICH_RSCP in TS 25.133 [29] for FDD and P-CCPCH_RSCP in TS 25.123 [30] for TDD. utra-EcN0 corresponds to CPICH_Ec/No in TS 25.133 [29] for FDD, and is not applicable for TDD. For utra-RSCP: The actual value is field value – 115 dBm. For utra-EcN0: The actual value is (field value – 49)/2 dB. |
| timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report. |
| triggerType E-UTRAN does not configure the value periodical in case reportConfig is linked to a measObject set to measObjectWLAN. |
| useAutonomousGapsNR The field applies to the reportCGI functionality, and when the field is included, the UE is allowed to use autonomous gaps in acquiring system information from the NR neighbour cell, applies the corresponding value for T321, EUTRAN can configure the field only if reportConfig is linked to a measObject set to measObjectNR. |  |

| Conditional presence | Explanation |
| --- | --- |
| reportCGI | The field is optional, need OR, in case purpose is included and set to reportCGI; otherwise the field is not present and the UE shall delete any existing value for this field. |
| reportCGI-NR | The field is optional, need OR, in case purpose is included and set to reportCGI, and reportConfig is linked to a measObject set to measObjectNR, otherwise the field is not present and the UE shall delete any existing value for this field. |  |

#### –	ReportConfigToAddModList

The IE ReportConfigToAddModList concerns a list of reporting configurations to add or modify

ReportConfigToAddModList information element

-- ASN1START

ReportConfigToAddModList ::=		SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod

ReportConfigToAddMod ::=	SEQUENCE {

reportConfigId						ReportConfigId,

reportConfig						CHOICE {

reportConfigEUTRA					ReportConfigEUTRA,

reportConfigInterRAT				ReportConfigInterRAT

}

}

-- ASN1STOP

#### –	ReportInterval

The ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e. when reportAmount exceeds 1), for triggerType event as well as for triggerType periodical. Value ms120 corresponds with 120 ms, ms240 corresponds with 240 ms and so on, while value min1 corresponds with 1 min, min6 corresponds with 6 min and so on.

ReportInterval information element

-- ASN1START

ReportInterval ::=					ENUMERATED {

ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240,

min1, min6, min12, min30, min60, spare3, spare2, spare1}

-- ASN1STOP

#### –	RS-IndexNR

The IE RS-IndexNR is used to identify an NR Reference Signal.

RS-IndexNR information element

-- ASN1START

RS-IndexNR-r15 ::=			INTEGER (0.. maxRS-Index-1-r15)

-- ASN1STOP

#### –	RSRP-Range

The IE RSRP-Range specifies the value range used in RSRP measurements and thresholds. Integer value for RSRP measurements according to mapping table in TS 36.133 [16]. A given field using RSRP-Range-v1360 shall only be signalled if the corresponding original field (using RSRP-Range i.e. without suffix) is set to value 0.

RSRP-Range information element

-- ASN1START

RSRP-Range ::=						INTEGER(0..97)

RSRP-Range-v1360 ::=				INTEGER(-17..-1)

RSRP-RangeSL-r12 ::=				INTEGER(0..13)

RSRP-RangeSL2-r12 ::=				INTEGER(0..7)

RSRP-RangeSL3-r12 ::=				INTEGER(0..11)

RSRP-RangeSL4-r13 ::=				INTEGER(0..49)

-- ASN1STOP

RSRP-Range field descriptions

RSRP-Range For UE supporting CE Mode B, when CE mode B is not restricted by upper layers, RSRP-Range-v1360 (i.e., with suffix) is reported if the measured RSRP is less than -140 dBm.

RSRP-RangeSL Value 0 corresponds to -infinity, value 1 to -115dBm, value 2 to -110dBm, and so on (i.e. in steps of 5dBm) until value 12, which corresponds to -60dBm, while value 13 corresponds to +infinity.

RSRP-RangeSL2 Value 0 corresponds to -infinity, value 1 to -110dBm, value 2 to -100dBm, and so on (i.e. in steps of 10dBm) until value 6, which corresponds to -60dBm, while value 7 corresponds to +infinity.

RSRP-RangeSL3 Value 0 corresponds to -110dBm, value 1 to -105dBm, value 2 to -100dBm, and so on (i.e. in steps of 5dBm) until value 10, which corresponds to -60dBm, while value 11 corresponds to +infinity.

RSRP-RangeSL4 Indicates the range for SD-RSRP. Value 0 corresponds to -130dBm, value 1 to -128dBm, value 2 to -126dBm, and so on (i.e. in steps of 2dBm) until value 48, which corresponds to -34dBm, while value 49 corresponds to +infinity.

#### –	RSRP-RangeNR

The IE RSRP-RangeNR specifies the value range used in RSRP measurements and thresholds. For RSRP measurements, integer value is according to mapping table in TS 38.133 [84]. For thresholds, the actual value is (field value – 156) dBm, except for field value 127, in which case the actual value is infinity.

RSRP-RangeNR information element

-- ASN1START

RSRP-RangeNR-r15 ::=						INTEGER (0..127)

-- ASN1STOP

#### –	RSRQ-Range

The IE RSRQ-Range specifies the value range used in RSRQ measurements and thresholds. Integer value for RSRQ measurements is according to mapping table in TS 36.133 [16]. A given field using RSRQ-Range-v1250 shall only be signalled if the corresponding original field (using RSRQ-Range i.e. without suffix) is set to value 0 or 34. Only a UE indicating support of extendedRSRQ-LowerRange-r12 or rsrq-OnAllSymbols-r12 may report RSRQ-Range-v1250, and this may be done without explicit configuration from the E-UTRAN. If received, the UE shall use the value indicated by the RSRQ-Range-v1250 and ignore the value signalled by RSRQ-Range (without the suffix). RSRQ-Range-r13 covers the original range and extended RSRQ-Range-v1250. RSRQ-Range-r13 may be signalled without the corresponding original field and without any requirements for indicated support of extendedRSRQ-LowerRange-r12 or rsrq-OnAllSymbols-r12.

RSRQ-Range information element

-- ASN1START

RSRQ-Range ::=					INTEGER(0..34)

RSRQ-Range-v1250 ::=			INTEGER(-30..46)

RSRQ-Range-r13 ::=				INTEGER(-30..46)

-- ASN1STOP

#### –	RSRQ-RangeNR

The IE RSRQ-RangeNR specifies the value range used in RSRQ measurements and thresholds. For RSRQ measurements, integer value is according to mapping table in TS 38.133 [84]. For thresholds, the actual value is (field value – 87) / 2 dB.

RSRQ-RangeNR information element

-- ASN1START

RSRQ-RangeNR-r15 ::=			INTEGER (0..127)

-- ASN1STOP

#### –	RSRQ-Type

The IE RSRQ-Type specifies the RSRQ value type used in RSRQ measurements, see TS 36.214 [48].

RSRQ-Type information element

-- ASN1START

RSRQ-Type-r12 ::=					SEQUENCE {

allSymbols-r12							BOOLEAN,

wideBand-r12							BOOLEAN

}

-- ASN1STOP

RSRQ-Type field descriptions

allSymbols Value TRUE indicates use of all OFDM symbols when performing RSRQ measurements.

wideBand Value TRUE indicates use of a wider bandwidth when performing RSRQ measurements.

#### –	RS-SINR-Range

The IE RS-SINR-Range specifies the value range used in RS-SINR measurements and thresholds. Integer value for RS-SINR measurements is according to mapping table in TS 36.133 [16].

RS-SINR-Range information element

-- ASN1START

RS-SINR-Range-r13 ::=						INTEGER(0..127)

-- ASN1STOP

#### –	RS-SINR-RangeNR

The IE RS-SINR-RangeNR specifies the value range used in RS-SINR measurements and thresholds. For RS-SINR measurements, integer value is according to mapping table in TS 38.133 [84]. For thresholds, the actual value is (field value – 46) / 2 dB.

RS-SINR-RangeNR information element

-- ASN1START

RS-SINR-RangeNR-r15 ::=					INTEGER (0..127)

-- ASN1STOP

#### –	RSSI-Range-r13

The IE RSSI-Range specifies the value range used in RSSI measurements and thresholds. Integer value for RSSI measurements is according to mapping table in TS 36.133 [16].

RSSI-Range information element

-- ASN1START

RSSI-Range-r13 ::=						INTEGER(0..76)

-- ASN1STOP

#### –	SS-RSSI-Measurement

The IE SS-RSSI-Measurement specifies the configuration of NR SSB based RSSI measurements.

SS-RSSI-Measurement information element

-- ASN1START

SS-RSSI-Measurement-r15 ::=			SEQUENCE {

measurementSlots-r15				BIT STRING (SIZE(1..80)),

endSymbol-r15						INTEGER(0..3)

}

-- ASN1STOP

SS-RSSI-Measurement field descriptions

endSymbol Within a slot that is configured for RSSI measurements (see measurementSlots) the UE measures the RSSI from symbol 0 to symbol endSymbol. This field identifies the entry in Table 5.1.33-1 in TS 36.214 which determines the actual end symbol.

measurementSlots Indicates the slots in which the UE can perform NR RSSI measurements. The length of the BIT STRING is equal to the number of slots in the configured SMTC window (determined by the ssb-duration and by the subcarrierSpacingSSB). The first (left-most / most significant) bit in the bitmap corresponds to the first slot in the SMTC window, the second bit in the bitmap corresponds to the second slot in the SMTC window, and so on. The UE measures in slots for which the corresponding bit in the bitmap is set to 1.

#### –	SSB-PositionQCL-RelationNR

The IE SSB-PositionQCL-RelationNR is used to indicate the QCL relationship between SSB positions on the indicated frequency or cell (see TS 38.213 [88], clause 4.1) for NR operation with shared spectrum channel access. Value n1 corresponds to 1, value n2 corresponds to 2 and so on.

SSB-PositionQCL-RelationNR information element

-- ASN1START

SSB-PositionQCL-RelationNR-r16 ::=		ENUMERATED {n1, n2, n4, n8}

-- ASN1STOP

#### –	SSB-ToMeasure

The IE SSB-ToMeasure is used to configure a pattern of SSBs. For operation with shared spectrum channel access, only mediumBitmap is used.

SSB-ToMeasure information element

-- ASN1START

SSB-ToMeasure-r15 ::=		CHOICE {

shortBitmap-r15				BIT STRING (SIZE (4)),

mediumBitmap-r15				BIT STRING (SIZE (8)),

longBitmap-r15					BIT STRING (SIZE (64))

}

-- ASN1STOP

SSB-ToMeasure field descriptions

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [88], clause 4.1.

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [88], clause 4.1. For operation with shared spectrum channel access, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k - 1 may be transmitted; if the k-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. The k-th bit is set to 0, where k > ssb-PositionQCL-CommonNR and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1's in the bitmap. If ssb-PositionQCL-NR is configured with a value smaller than ssb-PositionQCL-CommonNR, only the leftmost K bits (K = ssb-PositionQCL-NR) are applicable for the corresponding cell.

shortBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [88], clause 4.1.

#### –	TimeToTrigger

The IE TimeToTrigger specifies the value range used for time to trigger parameter, which concerns the time during which specific criteria for the event needs to be met in order to trigger a measurement report. Value ms0 corresponds to 0 ms and behaviour as specified in 7.3.2 applies, ms40 corresponds to 40 ms, and so on.

TimeToTrigger information element

-- ASN1START

TimeToTrigger ::=					ENUMERATED {

ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256,

ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560,

ms5120}

-- ASN1STOP

#### –	UL-DelayConfig

The IE UL-DelayConfig IE specifies the configuration of the UL PDCP Packet Delay per QCI measurement specified in TS 36.314 [71].

UL-DelayConfig information element

-- ASN1START

UL-DelayConfig-r13 ::=					CHOICE {

release								NULL,

setup								SEQUENCE {

delayThreshold-r13							ENUMERATED {

ms30, ms40, ms50, ms60, ms70, ms80,

ms90,ms100, ms150, ms300, ms500, ms750, spare4,

spare3, spare2, spare1}

}

}

-- ASN1STOP

UL-DelayConfig field descriptions

delayThreshold Indicates the delay threshold value used by UE to provide results of UL PDCP Packet Delay per QCI measurement as specified in TS 36.314 [71]. Value in milliseconds. Value ms30 means 30 ms and so on.

#### –	UL-DelayValueConfig

The IE UL-DelayValueConfig specifies the configuration of the UL PDCP Packet Delay value per DRB measurements specified in TS 38.314 [103].

UL-DelayValueConfig information element

-- ASN1START

UL-DelayValueConfig-r16 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

delay-DRBlist-r16						SEQUENCE (SIZE(1..maxDRB)) OF DRB-Identity

}

}

-- ASN1STOP

UL-DelayValueConfig field descriptions

delay-DRBlist Indicates the DRB IDs used by UE to provide results of UL PDCP Packet Delay value per DRB measurement as specified in TS 38.314 [103].

#### –	WLAN-CarrierInfo

The IE WLAN-CarrierInfo is used to identify the WLAN frequency band information, as specified in Annex E in [67].

WLAN-CarrierInfo information element

-- ASN1START

WLAN-CarrierInfo-r13 ::=	SEQUENCE {

operatingClass-r13			INTEGER (0..255)			OPTIONAL,	-- Need ON

countryCode-r13				ENUMERATED {unitedStates, europe, japan, global, ...}

OPTIONAL,	-- Need ON

channelNumbers-r13			WLAN-ChannelList-r13		OPTIONAL,	-- Need ON

...

}

WLAN-ChannelList-r13 ::=	SEQUENCE (SIZE (1..maxWLAN-Channels-r13)) OF WLAN-Channel-r13

WLAN-Channel-r13 ::=	INTEGER(0..255)

-- ASN1STOP

WLAN-CarrierInfo field descriptions

channelNumbers Indicates the WLAN channels as defined in IEEE 802.11-2012 [67]. Value 0 is not used.

countryCode Indicates the country code of WLAN as defined in IEEE 802.11-2012 [67].

operatingClass Indicates the Operating Class of WLAN as defined in IEEE 802.11-2012 [67].

#### –	WLAN-NameList

The IE WLAN-NameList is used to indicate the names of the WLAN AP for which the UE is configured to measure.

WLAN-NameList information element

-- ASN1START

WLAN-NameListConfig-r15 ::=		CHOICE{

release						NULL,

setup						WLAN-NameList-r15

}

WLAN-NameList-r15 ::=			SEQUENCE (SIZE (1..maxWLAN-Name-r15)) OF WLAN-Name-r15

WLAN-Name-r15 ::=		OCTET STRING (SIZE (1..32))

-- ASN1STOP

WLAN-NameList field descriptions

WLAN-Name If configured, the UE only performs WLAN measurements according to the names identified. For each name, it refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 [67].

#### –	WLAN-RSSI-Range

The IE WLAN-RSSI-Range specifies the value range used in WLAN RSSI measurements and thresholds. Integer value for WLAN RSSI measurements is according to mapping table in TS 36.133 [16]. Value 0 corresponds to -infinity, value 1 to -100dBm, value 2 to -99dBm, and so on (i.e. in steps of 1dBm) until value 140, which corresponds to 39dBm, while value 141 corresponds to +infinity.

WLAN-RSSI-Range information element

-- ASN1START

WLAN-RSSI-Range-r13 ::=						INTEGER(0..141)

-- ASN1STOP

#### –	WLAN-RTT

The IE WLAN-RTT covers the measured round trip time between the target device and WLAN AP and optionally the accuracy expressed as the standard deviation of the delay.

WLAN-RTT information element

-- ASN1START

WLAN-RTT-r15 ::= SEQUENCE {

rttValue-r15					INTEGER (0..16777215),

rttUnits-r15					ENUMERATED {	microseconds,

hundredsofnanoseconds,

tensofnanoseconds,

nanoseconds,

tenthsofnanoseconds,

... },

rttAccuracy-r15				INTEGER (0..255)							OPTIONAL,

...

}

-- ASN1STOP

WLAN-RTT field descriptions

rttValue This field specifies the Round Trip Time (RTT) measurement between the target device and WLAN AP in units given by the field rttUnits as defined in TS 36.355 [54].

rttUnits This field specifies the Units for the fields rttValue and rttAccuracy. The available Units are 1000ns, 100ns, 10ns, 1ns, and 0.1ns as defined in TS 36.355 [54].

rttAccuracy This field provides the estimated accuracy of the provided rttValue expressed as the standard deviation in units given by the field rttUnits as defined in TS 36.355 [54].

#### –	WLAN-Status

The IE WLAN-Status indicates the current status of WLAN connection. The values are set as described in clause 5.6.15.2 and 5.6.15.4.

WLAN-Status information element

-- ASN1START

WLAN-Status-r13 ::=		ENUMERATED {successfulAssociation, failureWlanRadioLink, failureWlanUnavailable, failureTimeout}

WLAN-Status-v1430 ::=	ENUMERATED {suspended, resumed}

-- ASN1STOP

#### –	WLAN-SuspendConfig

The IE WLAN-SuspendConfig is used for configuration of WLAN suspend/resume functionality.

-- ASN1START

WLAN-SuspendConfig-r14 ::=	SEQUENCE {

wlan-SuspendResumeAllowed-r14				BOOLEAN		OPTIONAL,	-- Need ON

wlan-SuspendTriggersStatusReport-r14		BOOLEAN		OPTIONAL	-- Need ON

}

-- ASN1STOP

WLAN-SuspendConfig field descriptions

wlan-SuspendResumeAllowed Indicates whether the UE is allowed to use suspend-resume mechanism, i.e., to indicate WLAN being temporarily unavailable and WLAN being available again after temporary unavailability.

wlan-SuspendTriggersStatusReport Indicates whether the UE shall trigger PDCP status report as defined in TS 36.323 [8] when WLAN is temporarily unavailable and UE reports this status.

### 6.3.6	Other information elements

#### –	AbsoluteTimeInfo

The IE AbsoluteTimeInfo indicates an absolute time in a format YY-MM-DD HH:MM:SS and using BCD encoding. The first/ leftmost bit of the bit string contains the most significant bit of the most significant digit of the year and so on.

AbsoluteTimeInfo information element

-- ASN1START

AbsoluteTimeInfo-r10 ::=				BIT STRING (SIZE (48))

-- ASN1STOP

#### –	AMF-Identifier

The IE AMF-Identifier (AMFI) comprises of an AMF Region ID, an AMF Set ID and an AMF Pointer as specified in 23.003 [27], clause 2.10.1.

AMF-Identifier information element

-- ASN1START

AMF-Identifier-r15 ::=						BIT STRING (SIZE (24))

-- ASN1STOP

#### –	AreaConfiguration

The AreaConfiguration indicates area for which UE is requested to perform measurement logging. If not configured, measurement logging is not restricted to specific cells or tracking areas but applies as long as the RPLMN is contained in plmn-IdentityList stored in VarLogMeasReport.

AreaConfiguration information element

-- ASN1START

AreaConfiguration-r10 ::=	CHOICE {

cellGlobalIdList-r10			CellGlobalIdList-r10,

trackingAreaCodeList-r10		TrackingAreaCodeList-r10

}

AreaConfiguration-v1130 ::=		SEQUENCE {

trackingAreaCodeList-v1130		TrackingAreaCodeList-v1130

}

CellGlobalIdList-r10 ::=				SEQUENCE (SIZE (1..32)) OF CellGlobalIdEUTRA

TrackingAreaCodeList-r10 ::=			SEQUENCE (SIZE (1..8)) OF TrackingAreaCode

TrackingAreaCodeList-v1130 ::=	SEQUENCE {

plmn-Identity-perTAC-List-r11			SEQUENCE (SIZE (1..8)) OF PLMN-Identity

}

-- ASN1STOP

AreaConfiguration field descriptions

plmn-Identity-perTAC-List Includes the PLMN identity for each of the TA codes included in trackingAreaCodeList. The PLMN identity listed first in plmn-Identity-perTAC-List corresponds with the TA code listed first in trackingAreaCodeList and so on.

#### –	BandCombinationList

The IE BandCombinationList contains a list of CA band combinations.

BandCombinationList information element

-- ASN1START

BandCombinationList-r14 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombination-r14

BandCombination-r14 ::= SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF BandIndication-r14

BandIndication-r14 ::=	SEQUENCE {

bandEUTRA-r14					FreqBandIndicator-r11,

ca-BandwidthClassDL-r14			CA-BandwidthClass-r10,

ca-BandwidthClassUL-r14			CA-BandwidthClass-r10			OPTIONAL

}

-- ASN1STOP

#### –	C-RNTI

The IE C-RNTI identifies a UE having a RRC connection within a cell.

C-RNTI information element

-- ASN1START

C-RNTI ::=							BIT STRING (SIZE (16))

-- ASN1STOP

#### –	DedicatedInfoCDMA2000

The DedicatedInfoCDMA2000 is used to transfer UE specific CDMA2000 information between the network and the UE. The RRC layer is transparent for this information.

DedicatedInfoCDMA2000 information element

-- ASN1START

DedicatedInfoCDMA2000 ::=				OCTET STRING

-- ASN1STOP

#### –	DedicatedInfoF1c

The IE DedicatedInfoF1c is used to transfer IAB-DU specific F1-C related information between the network and the IAB-node. The carried information consists of F1AP message encapsulated in SCTP/IP or F1-C related IP packet with or without SCTP encapsulation, see TS 38.472 [105] and TS 36.423 [108]. The RRC layer is transparent for this information.

DedicatedInfoF1c information element

-- ASN1START

DedicatedInfoF1c-r16 ::=		OCTET STRING

-- ASN1STOP

#### –	DedicatedInfoNAS

The IE DedicatedInfoNAS is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this information.

DedicatedInfoNAS information element

-- ASN1START

DedicatedInfoNAS ::=		OCTET STRING

-- ASN1STOP

#### –	FilterCoefficient

The IE FilterCoefficient specifies the measurement filtering coefficient. Value fc0 corresponds to k = 0, fc1 corresponds to k = 1, and so on.

FilterCoefficient information element

-- ASN1START

FilterCoefficient ::=					ENUMERATED {

fc0, fc1, fc2, fc3, fc4, fc5,

fc6, fc7, fc8, fc9, fc11, fc13,

fc15, fc17, fc19, spare1, ...}

-- ASN1STOP

#### –	FlightPathInfoReportConfig

The IE FlightPathInfoReportConfig specifies flight path information report configuration.

FlightPathInfoReportConfig information element

-- ASN1START

FlightPathInfoReportConfig-r15 ::= SEQUENCE {

maxWayPointNumber-r15				INTEGER (1..maxWayPoint-r15),

includeTimeStamp-r15				ENUMERATED {true}					OPTIONAL

}

-- ASN1STOP

FlightPathInfoReportConfig field descriptions

maxWayPointNumber Indicates the maximum number of way points UE can include in the flight path information report if this information is available at the UE.

includeTimeStamp Indicates whether time stamp of each way point can be reported in the flight path information report if time stamp information is available at the UE.

#### –	GNSS-ID

The IE GNSS-ID is used to indicate a specific GNSS (see also TS 36.355 [54]).

GNSS-ID information element

-- ASN1START

GNSS-ID-r15 ::= SEQUENCE {

gnss-id-r15				ENUMERATED{gps, sbas, qzss, galileo, glonass, bds, ..., navic-v1610},

...

}

-- ASN1STOP

#### –	I-RNTI

The I-RNTI IE is used to identify the suspended UE context of a UE in RRC_INACTIVE and for User plane CIoT 5GS optimisation.

I-RNTI information element

-- ASN1START

I-RNTI-r15 ::=						BIT STRING (SIZE(40))

-- ASN1STOP

#### –	LoggingDuration

The LoggingDuration indicates the duration for which UE is requested to perform measurement logging. Value min10 corresponds to 10 minutes, value min20 corresponds to 20 minutes and so on.

LoggingDuration information element

-- ASN1START

LoggingDuration-r10 ::=			ENUMERATED {

min10, min20, min40, min60, min90, min120, spare2, spare1}

-- ASN1STOP

#### –	LoggingInterval

The LoggingInterval indicates the periodicity for logging measurement results. Value ms1280 corresponds to 1.28s, value ms2560 corresponds to 2.56s and so on.

LoggingInterval information element

-- ASN1START

LoggingInterval-r10 ::=			ENUMERATED {

ms1280, ms2560, ms5120, ms10240, ms20480,

ms30720, ms40960, ms61440}

-- ASN1STOP

#### –	MeasSubframePattern

The IE MeasSubframePattern is used to specify a subframe pattern. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where SFN is that of PCell and x is the size of the bit string divided by 10. "1" denotes that the corresponding subframe is used.

MeasSubframePattern information element

-- ASN1START

MeasSubframePattern-r10 ::= CHOICE {

subframePatternFDD-r10				BIT STRING (SIZE (40)),

subframePatternTDD-r10				CHOICE {

subframeConfig1-5-r10					BIT STRING (SIZE (20)),

subframeConfig0-r10						BIT STRING (SIZE (70)),

subframeConfig6-r10						BIT STRING (SIZE (60)),

...

},

...

}

-- ASN1STOP

#### –	MMEC

The IE MMEC identifies an MME within the scope of an MME Group within a PLMN, see TS 23.003 [27].

MMEC information element

-- ASN1START

MMEC ::=							BIT STRING (SIZE (8))

-- ASN1STOP

#### –	NeighCellConfig

The IE NeighCellConfig is used to provide the information related to MBSFN and TDD UL/DL configuration of neighbour cells.

NeighCellConfig information element

-- ASN1START

NeighCellConfig ::=			BIT STRING (SIZE (2))

-- ASN1STOP

NeighCellConfig field descriptions

neighCellConfig Provides information related to MBSFN and TDD UL/DL configuration of neighbour cells of this frequency 00: Not all neighbour cells have the same MBSFN subframe allocation as the serving cell on this frequency, if configured, and as the PCell otherwise 10: The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell on this frequency, if configured, and of that in the PCell otherwise 01: No MBSFN subframes are present in all neighbour cells 11: Different UL/DL allocation in neighbouring cells for TDD compared to the serving cell on this frequency, if configured, and compared to the PCell otherwise For TDD, 00, 10 and 01 are only used for same UL/DL allocation in neighbouring cells compared to the serving cell on this frequency, if configured, and compared to the PCell otherwise.

#### –	NG-5G-S-TMSI

The IE NG-5G-S-TMSI contains a 5G S-Temporary Mobile Subscriber Identity, a temporary UE identity provided by the AMF which uniquely identifies the UE within the tracking area, see TS 23.003 [27].

NG-5G-S-TMSI information element

-- ASN1START

NG-5G-S-TMSI-r15::=							BIT STRING (SIZE (48))

-- ASN1STOP

#### –	OtherConfig

The IE OtherConfig contains configuration related to other configuration.

OtherConfig information element

-- ASN1START

OtherConfig-r9 ::= SEQUENCE	{

reportProximityConfig-r9			ReportProximityConfig-r9		OPTIONAL,	-- Need ON

...,

[[	idc-Config-r11					IDC-Config-r11					OPTIONAL,	-- Need ON

powerPrefIndicationConfig-r11	PowerPrefIndicationConfig-r11	OPTIONAL,	-- Need ON

obtainLocationConfig-r11		ObtainLocationConfig-r11		OPTIONAL	-- Need ON

]],

[[	bw-PreferenceIndicationTimer-r14	ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20,

s30, s60, s90, s120, s300, s600, spare3,

spare2, spare1}			OPTIONAL,	-- Need OR

sps-AssistanceInfoReport-r14		BOOLEAN			OPTIONAL,	-- Need ON

delayBudgetReportingConfig-r14	CHOICE{

release					NULL,

setup					SEQUENCE{

delayBudgetReportingProhibitTimer-r14	ENUMERATED {

s0, s0dot4, s0dot8,

s1dot6, s3, s6, s12, s30}

}

}																OPTIONAL,	-- Need ON

rlm-ReportConfig-r14			CHOICE {

release					NULL,

setup					SEQUENCE{

rlmReportTimer-r14				ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, spare3, spare2, spare1},

rlmReportRep-MPDCCH-r14			ENUMERATED {setup}		OPTIONAL	-- Need OR

}

}	OPTIONAL	-- Need ON

]],

[[	overheatingAssistanceConfig-r14	CHOICE{

release					NULL,

setup					SEQUENCE{

overheatingIndicationProhibitTimer-r14	ENUMERATED {s0, s0dot5, s1, s2, s5, s10,

s20, s30, s60, s90, s120, s300, s600,

spare3, spare2, spare1}

}

}	OPTIONAL		-- Need ON

]],

[[	measConfigAppLayer-r15		CHOICE{

release					NULL,

setup					SEQUENCE{

measConfigAppLayerContainer-r15		OCTET STRING (SIZE(1..1000)),

serviceType-r15						ENUMERATED {qoe, qoemtsi, spare6, spare5, spare4, spare3, spare2, spare1}

}

}	OPTIONAL,	-- Need ON

ailc-BitConfig-r15				BOOLEAN							OPTIONAL,	-- Need ON

bt-NameListConfig-r15		BT-NameListConfig-r15					OPTIONAL,	--Need ON

wlan-NameListConfig-r15		WLAN-NameListConfig-r15					OPTIONAL		--Need ON

]],

[[	overheatingAssistanceConfigForSCG-r16	BOOLEAN		OPTIONAL	-- Cond overheating

]]

}

IDC-Config-r11 ::=				SEQUENCE {

idc-Indication-r11					ENUMERATED {setup}				OPTIONAL,	-- Need OR

autonomousDenialParameters-r11		SEQUENCE {

autonomousDenialSubframes-r11			ENUMERATED {n2, n5, n10, n15,

n20, n30, spare2, spare1},

autonomousDenialValidity-r11			ENUMERATED {

sf200, sf500, sf1000, sf2000,

spare4, spare3, spare2, spare1}

}		OPTIONAL,		-- Need OR

...,

[[	idc-Indication-UL-CA-r11			ENUMERATED {setup}		OPTIONAL	-- Cond idc-Ind

]],

[[	idc-HardwareSharingIndication-r13	ENUMERATED {setup}		OPTIONAL	-- Need OR

]],

[[	idc-Indication-MRDC-r15		CHOICE{

release					NULL,

setup					CandidateServingFreqListNR-r15

}			OPTIONAL	-- Cond idc-Ind

]]

}

ObtainLocationConfig-r11 ::= SEQUENCE {

obtainLocation-r11				ENUMERATED {setup}					OPTIONAL	-- Need OR

}

PowerPrefIndicationConfig-r11 ::= CHOICE{

release					NULL,

setup					SEQUENCE{

powerPrefIndicationTimer-r11		ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20,

s30, s60, s90, s120, s300, s600, spare3,

spare2, spare1}

}

}

ReportProximityConfig-r9 ::= SEQUENCE {

proximityIndicationEUTRA-r9		ENUMERATED {enabled}			OPTIONAL,	-- Need OR

proximityIndicationUTRA-r9		ENUMERATED {enabled}			OPTIONAL	-- Need OR

}

CandidateServingFreqListNR-r15 ::= SEQUENCE (SIZE (1..maxFreqIDC-r11)) OF ARFCN-ValueNR-r15

-- ASN1STOP

OtherConfig field descriptions

ailc-BitConfig Indicates whether the UE is allowed to provide assistance information bit for local cache. If configured, the UE shall only apply to a DRB configured with 12-bit PDCP SN format as specified in TS 36.323 [8].

autonomousDenialSubframes Indicates the maximum number of the UL subframes for which the UE is allowed to deny any UL transmission. Value n2 corresponds to 2 subframes, n5 to 5 subframes and so on. E-UTRAN does not configure autonomous denial for frequencies on which SCG cells are configured.

autonomousDenialValidity Indicates the validity period over which the UL autonomous denial subframes shall be counted. Value sf200 corresponds to 200 subframes, sf500 corresponds to 500 subframes and so on.

bw-PreferenceIndicationTimer Prohibit timer for bandwidth preference indication reporting. Value in seconds. Value s0 means prohibit timer is set to 0 second, value s0dot5 means prohibit timer is set to 0.5 second, value s1 means prohibit timer is set to 1 second and so on.

CandidateServingFreqListNR Indicates for each candidate NR serving cells, the center frequency around which UE is requested to report IDC issues for MR-DC.

delayBudgetReportingProhibitTimer Prohibit timer for delay budget reporting. Value in seconds. Value s0 means prohibit timer is set to 0 second, value s0dot4 means prohibit timer is set to 0.4 second, and so on.

idc-HardwareSharingIndication The field is used to indicate whether the UE is allowed indicate in InDeviceCoexIndication that the cause of the problems are due to hardware sharing, and whether the UE is allowed to omit the TDM assistance information.

idc-Indication The field is used to indicate whether the UE is configured to initiate transmission of the InDeviceCoexIndication message to the network.

idc-Indication-MRDC The field is used to indicate whether the UE is configured to provide IDC indications for MR-DC using the InDeviceCoexIndication message.

idc-Indication-UL-CA The field is used to indicate whether the UE is configured to provide IDC indications for UL CA using the InDeviceCoexIndication message.

measConfigAppLayerContainer The field contains configuration of application layer measurements, see Annex L (normative) in TS 26.247 [90] and clause 16.5 in TS 26.114 [99].

serviceType Indicates the type of application layer measurement. Value qoe indicates Quality of Experience Measurement Collection for streaming services, value qoemtsi indicates Enhanced Quality of Experience Measurement Collection for MTSI.

obtainLocation Requests the UE to attempt to have detailed location information available using GNSS. E-UTRAN configures the field only if includeLocationInfo is configured for one or more measurements.

overheatingAssistanceConfig Configuration for the UE to report assistance information to inform the eNB about UE detected internal overheating.

overheatingAssistanceConfigForSCG The field is used to indicate whether the UE is configured to provide overheating assistance information for NR SCG. E-UTRAN configures value TRUE only when the UE is configured with an NR SCG.

overheatingIndicationProhibitTimer Prohibit timer for overheating assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 second, value s1 means prohibit timer is set to 1 second and so on.

powerPrefIndicationTimer Prohibit timer for Power Preference Indication reporting. Value in seconds. Value s0 means prohibit timer is set to 0 second, value s0dot5 means prohibit timer is set to 0.5 second, value s1 means prohibit timer is set to 1 second and so on.

reportProximityConfig Indicates, for each of the applicable RATs (EUTRA, UTRA), whether or not proximity indication is enabled for CSG member cell(s) of the concerned RAT. Note.

rlmReportTimer Prohibit timer for RLM event reporting, i.e. "early-out-of-sync" and "early-in-sync" event reporting, as specified in clause 5.6.10. Value in seconds. Value s0 means prohibit timer is set to 0 second, value s0dot5 means prohibit timer is set to 0.5 second, value s1 means prohibit timer is set to 1 second and so on.

rlmReportRep-MPDCCH The field is used to indicate whether the UE is configured to report excess repetitions on MPDCCH.

sps-AssistanceInfoReport Value TRUE indicates that the UE is allowed to report SPS-AssistanceInformation. If the sl-V2X-SPS-Config is provided by an E-UTRA RRCConnectionReconfiguration message embedded within an NR RRCReconfiguration for V2X sidelink communication (i.e. sl-ConfigDedicatedEUTRA) as in TS 38.331 [82], the network should configure the otherConfig and set this field to TRUE.

NOTE:	Enabling/ disabling of proximity indication includes enabling/ disabling of the related functionality e.g. autonomous search in connected mode.

| Conditional presence | Explanation |
| --- | --- |
| idc-Ind | The field is optionally present if idc-Indication is present, need OR. Otherwise the field is not present. |
| overheating | The field is optionally present, need ON, if overheatingAssistanceConfig is included and set to setup; otherwise, the field is not present and the UE shall delete any existing value for this field. |

#### –	RAN-AreaCode

The RAN-AreaCode IE indicates RAN area code of the cell.

RAN-AreaCode information element

-- ASN1START

RAN-AreaCode-r15 ::=					INTEGER (0..255)

-- ASN1STOP

#### –	RAND-CDMA2000 (1xRTT)

The RAND-CDMA2000 concerns a random value, generated by the eNB, to be passed to the CDMA2000 upper layers.

RAND-CDMA2000 information element

-- ASN1START

RAND-CDMA2000 ::=						BIT STRING (SIZE (32))

-- ASN1STOP

#### –	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including E-UTRA, of the requested/ transferred UE capabilities. A separate value applies for some EUTRA-NR capabilities that are transferred by a separate UE capability container, used in case of MR-DC.

RAT-Type information element

-- ASN1START

RAT-Type ::=						ENUMERATED {

eutra, utra, geran-cs, geran-ps, cdma2000-1XRTT,

nr, eutra-nr, spare1, ...}

-- ASN1STOP

#### –	ResumeIdentity

The IE ResumeIdentity is used to identify the suspended UE context

ResumeIdentity information element

-- ASN1START

ResumeIdentity-r13 ::=						BIT STRING (SIZE(40))

-- ASN1STOP

#### –	RRC-TransactionIdentifier

The IE RRC-TransactionIdentifier is used, together with the message type, for the identification of an RRC procedure (transaction).

RRC-TransactionIdentifier information element

-- ASN1START

RRC-TransactionIdentifier ::=		INTEGER (0..3)

-- ASN1STOP

#### –	SBAS-ID

The IE SBAS-ID is used to indicate a specific SBAS (see also TS 36.355 [54]).

SBAS-ID information element

-- ASN1START

SBAS-ID-r15 ::= SEQUENCE {

sbas-id-r15				ENUMERATED {waas, egnos, msas, gagan, ...},

...

}

-- ASN1STOP

#### –	ShortI-RNTI

The ShortI-RNTI IE is used to identify the suspended UE context of a UE in RRC_INACTIVE using fewer bits compared to I-RNTI.

ShortI-RNTI information element

-- ASN1START

ShortI-RNTI-r15 ::=						BIT STRING (SIZE(24))

-- ASN1STOP

#### –	S-NSSAI

The IE S-NSSAI identifies a Network Slice end to end and comprises a slice/service type and a slice differentiator, see TS 23.003 [27].

S-NSSAI information element

-- ASN1START

S-NSSAI-r15 ::=			CHOICE{

sst							BIT STRING (SIZE (8)),

sst-SD						BIT STRING (SIZE (32))

}

-- ASN1STOP

S-NSSAI field descriptions

sst Indicates the S-NSSAI consists of Slice/Service Type, see TS 23.003 [27].

sst-SD Indicates the S-NSSAI consists of Slice/Service Type and Slice Differentiator, see TS 23.003 [27].

#### –	S-TMSI

The IE S-TMSI contains an S-Temporary Mobile Subscriber Identity, a temporary UE identity provided by the EPC which uniquely identifies the UE within the tracking area, see TS 23.003 [27].

S-TMSI information element

-- ASN1START

S-TMSI ::=							SEQUENCE {

mmec								MMEC,

m-TMSI								BIT STRING (SIZE (32))

}

-- ASN1STOP

S-TMSI field descriptions

m-TMSI The first/leftmost bit of the bit string contains the most significant bit of the M-TMSI.

#### –	TimeUTC-r17

The IE TimeUTC provides the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

TimeUTC-r17 information element

-- ASN1START

TimeUTC-r17 ::=		INTEGER (0..549755813887)

-- ASN1STOP

#### –	TraceReference

The TraceReference contains parameter Trace Reference as defined in TS 32.422 [58].

TraceReference information element

-- ASN1START

TraceReference-r10 ::=			SEQUENCE {

plmn-Identity-r10				PLMN-Identity,

traceId-r10						OCTET STRING (SIZE (3))

}

-- ASN1STOP

#### –	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains list of containers, one for each RAT for which UE capabilities are transferred, if any.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

UE-CapabilityRAT-ContainerList ::=SEQUENCE (SIZE (0..maxRAT-Capabilities)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::= SEQUENCE {

rat-Type							RAT-Type,

ueCapabilityRAT-Container			OCTET STRING

}

-- ASN1STOP

UECapabilityRAT-ContainerList field descriptions

ueCapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For E-UTRA: the encoding of UE capabilities is defined in IE UE-EUTRA-Capability. For UTRA: the octet string contains the INTER RAT HANDOVER INFO message defined in TS 25.331 [19]. For GERAN CS: the octet string contains the concatenated string of the Mobile Station Classmark 2 and Mobile Station Classmark 3. The first 5 octets correspond to Mobile Station Classmark 2 and the following octets correspond to Mobile Station Classmark 3. The Mobile Station Classmark 2 is formatted as 'TLV' and is coded in the same way as the Mobile Station Classmark 2 information element in TS 24.008 [49]. The first octet is the Mobile station classmark 2 IEI and its value shall be set to 33H. The second octet is the Length of mobile station classmark 2 and its value shall be set to 3. The octet 3 contains the first octet of the value part of the Mobile Station Classmark 2 information element, the octet 4 contains the second octet of the value part of the Mobile Station Classmark 2 information element and so on. For each of these octets, the first/ leftmost/ most significant bit of the octet contains b8 of the corresponding octet of the Mobile Station Classmark 2. The Mobile Station Classmark 3 is formatted as 'V' and is coded in the same way as the value part in the Mobile station classmark 3 information element in TS 24.008 [49]. The sixth octet of this octet string contains octet 1 of the value part of Mobile station classmark 3, the seventh of octet of this octet string contains octet 2 of the value part of Mobile station classmark 3 and so on. Note. For GERAN PS: the encoding of UE capabilities is formatted as 'V' and is coded in the same way as the value part in the MS Radio Access Capability information element in TS 24.008 [49]. For CDMA2000-1XRTT: the octet string contains the A21 Mobile Subscription Information and the encoding of this is defined in A.S0008 [33]. The A21 Mobile Subscription Information contains the supported CDMA2000 1xRTT band class and band sub-class information. For NR: The octet string contains the IE UE-NR-Capability as defined in TS 38.331 [82]. For EUTRA-NR: The octet string contains the IE UE-MRDC-Capability as defined in TS 38.331 [82]

NOTE:	The value part is specified by means of CSN.1, which encoding results in a bit string, to which final padding may be appended up to the next octet boundary TS 24.008 [49]. The first/ leftmost bit of the CSN.1 bit string is placed in the first/ leftmost/ most significant bit of the first octet. This continues until the last bit of the CSN.1 bit string, which is placed in the last/ rightmost/ least significant bit of the last octet.

#### –	UE-EUTRA-Capability

The IE UE-EUTRA-Capability is used to convey the E-UTRA UE Radio Access Capability Parameters, see TS 36.306 [5], and the Feature Group Indicators for mandatory features (defined in Annexes B.1 and C.1) to the network. The IE UE-EUTRA-Capability is transferred in E-UTRA or in another RAT.

NOTE 0:	For (UE capability specific) guidelines on the use of keyword OPTIONAL, see Annex A.3.5.

UE-EUTRA-Capability information element

-- ASN1START

UE-EUTRA-Capability ::=			SEQUENCE {

accessStratumRelease			AccessStratumRelease,

ue-Category						INTEGER (1..5),

pdcp-Parameters					PDCP-Parameters,

phyLayerParameters				PhyLayerParameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters,

featureGroupIndicators			BIT STRING (SIZE (32))					OPTIONAL,

interRAT-Parameters				SEQUENCE {

utraFDD							IRAT-ParametersUTRA-FDD				OPTIONAL,

utraTDD128						IRAT-ParametersUTRA-TDD128			OPTIONAL,

utraTDD384						IRAT-ParametersUTRA-TDD384			OPTIONAL,

utraTDD768						IRAT-ParametersUTRA-TDD768			OPTIONAL,

geran							IRAT-ParametersGERAN				OPTIONAL,

cdma2000-HRPD					IRAT-ParametersCDMA2000-HRPD		OPTIONAL,

cdma2000-1xRTT					IRAT-ParametersCDMA2000-1XRTT		OPTIONAL

},

nonCriticalExtension			UE-EUTRA-Capability-v920-IEs			OPTIONAL

}

-- Late non critical extensions

UE-EUTRA-Capability-v9a0-IEs ::=	SEQUENCE {

featureGroupIndRel9Add-r9			BIT STRING (SIZE (32))				OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-r9	UE-EUTRA-CapabilityAddXDD-Mode-r9	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-r9	UE-EUTRA-CapabilityAddXDD-Mode-r9	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v9c0-IEs		OPTIONAL

}

UE-EUTRA-Capability-v9c0-IEs ::=	SEQUENCE {

interRAT-ParametersUTRA-v9c0		IRAT-ParametersUTRA-v9c0		OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v9d0-IEs	OPTIONAL

}

UE-EUTRA-Capability-v9d0-IEs ::=	SEQUENCE {

phyLayerParameters-v9d0				PhyLayerParameters-v9d0			OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v9e0-IEs	OPTIONAL

}

UE-EUTRA-Capability-v9e0-IEs ::=	SEQUENCE {

rf-Parameters-v9e0					RF-Parameters-v9e0						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v9h0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v9h0-IEs ::=	SEQUENCE {

interRAT-ParametersUTRA-v9h0		IRAT-ParametersUTRA-v9h0				OPTIONAL,

-- Following field is only to be used for late REL-9 extensions

lateNonCriticalExtension			OCTET STRING							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v10c0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v10c0-IEs ::=	SEQUENCE {

otdoa-PositioningCapabilities-r10	OTDOA-PositioningCapabilities-r10		OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v10f0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v10f0-IEs ::=	SEQUENCE {

rf-Parameters-v10f0					RF-Parameters-v10f0						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v10i0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v10i0-IEs ::=	SEQUENCE {

rf-Parameters-v10i0					RF-Parameters-v10i0						OPTIONAL,

-- Following field is only to be used for late REL-10 extensions

lateNonCriticalExtension			OCTET STRING (CONTAINING UE-EUTRA-Capability-v10j0-IEs)	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v11d0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v10j0-IEs ::=	SEQUENCE {

rf-Parameters-v10j0					RF-Parameters-v10j0						OPTIONAL,

nonCriticalExtension				SEQUENCE {}								OPTIONAL

}

UE-EUTRA-Capability-v11d0-IEs ::=	SEQUENCE {

rf-Parameters-v11d0					RF-Parameters-v11d0						OPTIONAL,

otherParameters-v11d0				Other-Parameters-v11d0					OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v11x0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v11x0-IEs ::=	SEQUENCE {

-- Following field is only to be used for late REL-11 extensions

lateNonCriticalExtension			OCTET STRING								OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v12b0-IEs				OPTIONAL

}

UE-EUTRA-Capability-v12b0-IEs ::= SEQUENCE {

rf-Parameters-v12b0					RF-Parameters-v12b0						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v12x0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v12x0-IEs ::= SEQUENCE {

-- Following field is only to be used for late REL-12 extensions

lateNonCriticalExtension			OCTET STRING							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1370-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1370-IEs ::= SEQUENCE {

ce-Parameters-v1370					CE-Parameters-v1370						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1370	UE-EUTRA-CapabilityAddXDD-Mode-v1370	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1370	UE-EUTRA-CapabilityAddXDD-Mode-v1370	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1380-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1380-IEs ::= SEQUENCE {

rf-Parameters-v1380					RF-Parameters-v1380						OPTIONAL,

ce-Parameters-v1380					CE-Parameters-v1380,

fdd-Add-UE-EUTRA-Capabilities-v1380	UE-EUTRA-CapabilityAddXDD-Mode-v1380,

tdd-Add-UE-EUTRA-Capabilities-v1380	UE-EUTRA-CapabilityAddXDD-Mode-v1380,

nonCriticalExtension				UE-EUTRA-Capability-v1390-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1390-IEs ::= SEQUENCE {

rf-Parameters-v1390					RF-Parameters-v1390						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v13e0a-IEs			OPTIONAL

}

UE-EUTRA-Capability-v13e0a-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING UE-EUTRA-Capability-v13e0b-IEs)							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1470-IEs			OPTIONAL

}

UE-EUTRA-Capability-v13e0b-IEs ::= SEQUENCE {

phyLayerParameters-v13e0			PhyLayerParameters-v13e0,

-- Following field is only to be used for late REL-13 extensions

nonCriticalExtension				SEQUENCE {}								OPTIONAL

}

UE-EUTRA-Capability-v1470-IEs ::= SEQUENCE {

mbms-Parameters-v1470				MBMS-Parameters-v1470					OPTIONAL,

phyLayerParameters-v1470			PhyLayerParameters-v1470				OPTIONAL,

rf-Parameters-v1470					RF-Parameters-v1470						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v14a0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v14a0-IEs ::= SEQUENCE {

phyLayerParameters-v14a0				PhyLayerParameters-v14a0,

-- Following field is only to be used for late REL-14 extensions

nonCriticalExtension					UE-EUTRA-Capability-v14b0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v14b0-IEs ::= SEQUENCE {

rf-Parameters-v14b0				RF-Parameters-v14b0				OPTIONAL,

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- Regular non critical extensions

UE-EUTRA-Capability-v920-IEs ::=		SEQUENCE {

phyLayerParameters-v920					PhyLayerParameters-v920,

interRAT-ParametersGERAN-v920			IRAT-ParametersGERAN-v920,

interRAT-ParametersUTRA-v920			IRAT-ParametersUTRA-v920			OPTIONAL,

interRAT-ParametersCDMA2000-v920		IRAT-ParametersCDMA2000-1XRTT-v920	OPTIONAL,

deviceType-r9							ENUMERATED {noBenFromBatConsumpOpt}	OPTIONAL,

csg-ProximityIndicationParameters-r9	CSG-ProximityIndicationParameters-r9,

neighCellSI-AcquisitionParameters-r9	NeighCellSI-AcquisitionParameters-r9,

son-Parameters-r9						SON-Parameters-r9,

nonCriticalExtension					UE-EUTRA-Capability-v940-IEs		OPTIONAL

}

UE-EUTRA-Capability-v940-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING UE-EUTRA-Capability-v9a0-IEs)			OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1020-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1020-IEs ::=	SEQUENCE {

ue-Category-v1020					INTEGER (6..8)							OPTIONAL,

phyLayerParameters-v1020			PhyLayerParameters-v1020				OPTIONAL,

rf-Parameters-v1020					RF-Parameters-v1020						OPTIONAL,

measParameters-v1020				MeasParameters-v1020					OPTIONAL,

featureGroupIndRel10-r10			BIT STRING (SIZE (32))					OPTIONAL,

interRAT-ParametersCDMA2000-v1020	IRAT-ParametersCDMA2000-1XRTT-v1020		OPTIONAL,

ue-BasedNetwPerfMeasParameters-r10	UE-BasedNetwPerfMeasParameters-r10		OPTIONAL,

interRAT-ParametersUTRA-TDD-v1020	IRAT-ParametersUTRA-TDD-v1020			OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1060-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1060-IEs ::=	SEQUENCE {

fdd-Add-UE-EUTRA-Capabilities-v1060	UE-EUTRA-CapabilityAddXDD-Mode-v1060	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1060	UE-EUTRA-CapabilityAddXDD-Mode-v1060	OPTIONAL,

rf-Parameters-v1060					RF-Parameters-v1060						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1090-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1090-IEs ::=	SEQUENCE {

rf-Parameters-v1090					RF-Parameters-v1090						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1130-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1130-IEs ::=	SEQUENCE {

pdcp-Parameters-v1130				PDCP-Parameters-v1130,

phyLayerParameters-v1130			PhyLayerParameters-v1130				OPTIONAL,

rf-Parameters-v1130					RF-Parameters-v1130,

measParameters-v1130				MeasParameters-v1130,

interRAT-ParametersCDMA2000-v1130	IRAT-ParametersCDMA2000-v1130,

otherParameters-r11					Other-Parameters-r11,

fdd-Add-UE-EUTRA-Capabilities-v1130	UE-EUTRA-CapabilityAddXDD-Mode-v1130	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1130	UE-EUTRA-CapabilityAddXDD-Mode-v1130	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1170-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1170-IEs ::=	SEQUENCE {

phyLayerParameters-v1170			PhyLayerParameters-v1170				OPTIONAL,

ue-Category-v1170					INTEGER (9..10)							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1180-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1180-IEs ::=	SEQUENCE {

rf-Parameters-v1180					RF-Parameters-v1180						OPTIONAL,

mbms-Parameters-r11					MBMS-Parameters-r11						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1180	UE-EUTRA-CapabilityAddXDD-Mode-v1180	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1180	UE-EUTRA-CapabilityAddXDD-Mode-v1180	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v11a0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v11a0-IEs ::=	SEQUENCE {

ue-Category-v11a0					INTEGER (11..12)						OPTIONAL,

measParameters-v11a0				MeasParameters-v11a0					OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1250-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1250-IEs ::=	SEQUENCE {

phyLayerParameters-v1250				PhyLayerParameters-v1250				OPTIONAL,

rf-Parameters-v1250						RF-Parameters-v1250						OPTIONAL,

rlc-Parameters-r12						RLC-Parameters-r12						OPTIONAL,

ue-BasedNetwPerfMeasParameters-v1250	UE-BasedNetwPerfMeasParameters-v1250	OPTIONAL,

ue-CategoryDL-r12						INTEGER (0..14)							OPTIONAL,

ue-CategoryUL-r12						INTEGER (0..13)							OPTIONAL,

wlan-IW-Parameters-r12					WLAN-IW-Parameters-r12					OPTIONAL,

measParameters-v1250					MeasParameters-v1250					OPTIONAL,

dc-Parameters-r12						DC-Parameters-r12						OPTIONAL,

mbms-Parameters-v1250					MBMS-Parameters-v1250					OPTIONAL,

mac-Parameters-r12						MAC-Parameters-r12						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1250		UE-EUTRA-CapabilityAddXDD-Mode-v1250	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1250		UE-EUTRA-CapabilityAddXDD-Mode-v1250	OPTIONAL,

sl-Parameters-r12						SL-Parameters-r12						OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1260-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1260-IEs ::=	SEQUENCE {

ue-CategoryDL-v1260					INTEGER (15..16)						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1270-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1270-IEs ::= SEQUENCE {

rf-Parameters-v1270					RF-Parameters-v1270						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1280-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1280-IEs ::= SEQUENCE {

phyLayerParameters-v1280			PhyLayerParameters-v1280				OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1310-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1310-IEs ::= SEQUENCE {

ue-CategoryDL-v1310					ENUMERATED {n17, m1}					OPTIONAL,

ue-CategoryUL-v1310					ENUMERATED {n14, m1}					OPTIONAL,

pdcp-Parameters-v1310				PDCP-Parameters-v1310,

rlc-Parameters-v1310				RLC-Parameters-v1310,

mac-Parameters-v1310				MAC-Parameters-v1310					OPTIONAL,

phyLayerParameters-v1310			PhyLayerParameters-v1310				OPTIONAL,

rf-Parameters-v1310					RF-Parameters-v1310						OPTIONAL,

measParameters-v1310				MeasParameters-v1310					OPTIONAL,

dc-Parameters-v1310					DC-Parameters-v1310						OPTIONAL,

sl-Parameters-v1310					SL-Parameters-v1310						OPTIONAL,

scptm-Parameters-r13				SCPTM-Parameters-r13					OPTIONAL,

ce-Parameters-r13					CE-Parameters-r13						OPTIONAL,

interRAT-ParametersWLAN-r13			IRAT-ParametersWLAN-r13,

laa-Parameters-r13					LAA-Parameters-r13						OPTIONAL,

lwa-Parameters-r13					LWA-Parameters-r13						OPTIONAL,

wlan-IW-Parameters-v1310			WLAN-IW-Parameters-v1310,

lwip-Parameters-r13					LWIP-Parameters-r13,

fdd-Add-UE-EUTRA-Capabilities-v1310	UE-EUTRA-CapabilityAddXDD-Mode-v1310	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1310	UE-EUTRA-CapabilityAddXDD-Mode-v1310	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1320-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1320-IEs ::= SEQUENCE {

ce-Parameters-v1320					CE-Parameters-v1320						OPTIONAL,

phyLayerParameters-v1320			PhyLayerParameters-v1320				OPTIONAL,

rf-Parameters-v1320					RF-Parameters-v1320						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1320	UE-EUTRA-CapabilityAddXDD-Mode-v1320	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1320	UE-EUTRA-CapabilityAddXDD-Mode-v1320	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1330-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1330-IEs ::= SEQUENCE {

ue-CategoryDL-v1330					INTEGER (18..19)						OPTIONAL,

phyLayerParameters-v1330			PhyLayerParameters-v1330				OPTIONAL,

ue-CE-NeedULGaps-r13				ENUMERATED {true}						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1340-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1340-IEs ::= SEQUENCE {

ue-CategoryUL-v1340					INTEGER (15)							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1350-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1350-IEs ::= SEQUENCE {

ue-CategoryDL-v1350					ENUMERATED {oneBis}						OPTIONAL,

ue-CategoryUL-v1350					ENUMERATED {oneBis}						OPTIONAL,

ce-Parameters-v1350					CE-Parameters-v1350,

nonCriticalExtension				UE-EUTRA-Capability-v1360-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1360-IEs ::= SEQUENCE {

other-Parameters-v1360				Other-Parameters-v1360					OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1430-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1430-IEs ::= SEQUENCE {

phyLayerParameters-v1430			PhyLayerParameters-v1430,

ue-CategoryDL-v1430					ENUMERATED {m2}								OPTIONAL,

ue-CategoryUL-v1430					ENUMERATED {n16, n17, n18, n19, n20, m2}	OPTIONAL,

ue-CategoryUL-v1430b				ENUMERATED {n21}							OPTIONAL,

mac-Parameters-v1430				MAC-Parameters-v1430						OPTIONAL,

measParameters-v1430				MeasParameters-v1430						OPTIONAL,

pdcp-Parameters-v1430				PDCP-Parameters-v1430						OPTIONAL,

rlc-Parameters-v1430				RLC-Parameters-v1430,

rf-Parameters-v1430					RF-Parameters-v1430							OPTIONAL,

laa-Parameters-v1430				LAA-Parameters-v1430						OPTIONAL,

lwa-Parameters-v1430				LWA-Parameters-v1430						OPTIONAL,

lwip-Parameters-v1430				LWIP-Parameters-v1430						OPTIONAL,

otherParameters-v1430				Other-Parameters-v1430,

mmtel-Parameters-r14				MMTEL-Parameters-r14						OPTIONAL,

mobilityParameters-r14				MobilityParameters-r14						OPTIONAL,

ce-Parameters-v1430					CE-Parameters-v1430,

fdd-Add-UE-EUTRA-Capabilities-v1430	UE-EUTRA-CapabilityAddXDD-Mode-v1430		OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1430	UE-EUTRA-CapabilityAddXDD-Mode-v1430		OPTIONAL,

mbms-Parameters-v1430				MBMS-Parameters-v1430						OPTIONAL,

sl-Parameters-v1430					SL-Parameters-v1430							OPTIONAL,

ue-BasedNetwPerfMeasParameters-v1430	UE-BasedNetwPerfMeasParameters-v1430	OPTIONAL,

highSpeedEnhParameters-r14			HighSpeedEnhParameters-r14					OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1440-IEs				OPTIONAL

}

UE-EUTRA-Capability-v1440-IEs ::= SEQUENCE {

lwa-Parameters-v1440				LWA-Parameters-v1440,

mac-Parameters-v1440				MAC-Parameters-v1440,

nonCriticalExtension				UE-EUTRA-Capability-v1450-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1450-IEs ::= SEQUENCE {

phyLayerParameters-v1450			PhyLayerParameters-v1450		OPTIONAL,

rf-Parameters-v1450					RF-Parameters-v1450			OPTIONAL,

otherParameters-v1450				OtherParameters-v1450,

ue-CategoryDL-v1450					INTEGER (20)					OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1460-IEs	OPTIONAL

}

UE-EUTRA-Capability-v1460-IEs ::= SEQUENCE {

ue-CategoryDL-v1460				INTEGER (21)							OPTIONAL,

otherParameters-v1460				Other-Parameters-v1460,

nonCriticalExtension				UE-EUTRA-Capability-v1510-IEs		OPTIONAL

}

UE-EUTRA-Capability-v1510-IEs ::= SEQUENCE {

irat-ParametersNR-r15					IRAT-ParametersNR-r15					OPTIONAL,

featureSetsEUTRA-r15					FeatureSetsEUTRA-r15					OPTIONAL,

pdcp-ParametersNR-r15					PDCP-ParametersNR-r15					OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1510		UE-EUTRA-CapabilityAddXDD-Mode-v1510	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1510		UE-EUTRA-CapabilityAddXDD-Mode-v1510	OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1520-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1520-IEs ::= SEQUENCE {

measParameters-v1520					MeasParameters-v1520,

nonCriticalExtension					UE-EUTRA-Capability-v1530-IEs	OPTIONAL

}

UE-EUTRA-Capability-v1530-IEs ::= SEQUENCE {

measParameters-v1530					MeasParameters-v1530					OPTIONAL,

otherParameters-v1530					Other-Parameters-v1530					OPTIONAL,

neighCellSI-AcquisitionParameters-v1530	NeighCellSI-AcquisitionParameters-v1530	OPTIONAL,

mac-Parameters-v1530					MAC-Parameters-v1530					OPTIONAL,

phyLayerParameters-v1530				PhyLayerParameters-v1530				OPTIONAL,

rf-Parameters-v1530						RF-Parameters-v1530						OPTIONAL,

pdcp-Parameters-v1530					PDCP-Parameters-v1530					OPTIONAL,

ue-CategoryDL-v1530						INTEGER (22..26)						OPTIONAL,

ue-BasedNetwPerfMeasParameters-v1530	UE-BasedNetwPerfMeasParameters-v1530	OPTIONAL,

rlc-Parameters-v1530					RLC-Parameters-v1530					OPTIONAL,

sl-Parameters-v1530						SL-Parameters-v1530						OPTIONAL,

extendedNumberOfDRBs-r15				ENUMERATED {supported}					OPTIONAL,

reducedCP-Latency-r15					ENUMERATED {supported}					OPTIONAL,

laa-Parameters-v1530					LAA-Parameters-v1530					OPTIONAL,

ue-CategoryUL-v1530						INTEGER (22..26)						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1530		UE-EUTRA-CapabilityAddXDD-Mode-v1530	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1530		UE-EUTRA-CapabilityAddXDD-Mode-v1530	OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1540-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1540-IEs ::= SEQUENCE {

phyLayerParameters-v1540				PhyLayerParameters-v1540				OPTIONAL,

otherParameters-v1540					Other-Parameters-v1540,

fdd-Add-UE-EUTRA-Capabilities-v1540		UE-EUTRA-CapabilityAddXDD-Mode-v1540	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1540		UE-EUTRA-CapabilityAddXDD-Mode-v1540	OPTIONAL,

sl-Parameters-v1540						SL-Parameters-v1540						OPTIONAL,

irat-ParametersNR-v1540					IRAT-ParametersNR-v1540					OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1550-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1550-IEs ::= SEQUENCE {

neighCellSI-AcquisitionParameters-v1550	NeighCellSI-AcquisitionParameters-v1550	OPTIONAL,

phyLayerParameters-v1550				PhyLayerParameters-v1550,

mac-Parameters-v1550					MAC-Parameters-v1550,

fdd-Add-UE-EUTRA-Capabilities-v1550		UE-EUTRA-CapabilityAddXDD-Mode-v1550,

tdd-Add-UE-EUTRA-Capabilities-v1550		UE-EUTRA-CapabilityAddXDD-Mode-v1550,

nonCriticalExtension					UE-EUTRA-Capability-v1560-IEs	OPTIONAL

}

UE-EUTRA-Capability-v1560-IEs ::= SEQUENCE {

pdcp-ParametersNR-v1560				PDCP-ParametersNR-v1560,

irat-ParametersNR-v1560				IRAT-ParametersNR-v1560,

appliedCapabilityFilterCommon-r15		OCTET STRING							OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1560	UE-EUTRA-CapabilityAddXDD-Mode-v1560,

tdd-Add-UE-EUTRA-Capabilities-v1560	UE-EUTRA-CapabilityAddXDD-Mode-v1560,

nonCriticalExtension					UE-EUTRA-Capability-v1570-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1570-IEs ::= SEQUENCE {

rf-Parameters-v1570				RF-Parameters-v1570					OPTIONAL,

irat-ParametersNR-v1570			IRAT-ParametersNR-v1570				OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v15a0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v15a0-IEs ::= SEQUENCE {

neighCellSI-AcquisitionParameters-v15a0	NeighCellSI-AcquisitionParameters-v15a0,

eutra-5GC-Parameters-r15				EUTRA-5GC-Parameters-r15				OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v15a0	UE-EUTRA-CapabilityAddXDD-Mode-v15a0	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v15a0	UE-EUTRA-CapabilityAddXDD-Mode-v15a0	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1610-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1610-IEs ::= SEQUENCE {

highSpeedEnhParameters-v1610			HighSpeedEnhParameters-v1610				OPTIONAL,

neighCellSI-AcquisitionParameters-v1610	NeighCellSI-AcquisitionParameters-v1610		OPTIONAL,

mbms-Parameters-v1610					MBMS-Parameters-v1610						OPTIONAL,

pdcp-Parameters-v1610					PDCP-Parameters-v1610						OPTIONAL,

mac-Parameters-v1610					MAC-Parameters-v1610						OPTIONAL,

phyLayerParameters-v1610				PhyLayerParameters-v1610					OPTIONAL,

measParameters-v1610 					MeasParameters-v1610 						OPTIONAL,

pur-Parameters-r16						PUR-Parameters-r16							OPTIONAL,

eutra-5GC-Parameters-v1610				EUTRA-5GC-Parameters-v1610					OPTIONAL,

otherParameters-v1610					Other-Parameters-v1610						OPTIONAL,

dl-DedicatedMessageSegmentation-r16		ENUMERATED {supported}						OPTIONAL,

mmtel-Parameters-v1610					MMTEL-Parameters-v1610,

irat-ParametersNR-v1610					IRAT-ParametersNR-v1610						OPTIONAL,

rf-Parameters-v1610						RF-Parameters-v1610							OPTIONAL,

mobilityParameters-v1610				MobilityParameters-v1610					OPTIONAL,

ue-BasedNetwPerfMeasParameters-v1610	UE-BasedNetwPerfMeasParameters-v1610,

sl-Parameters-v1610						SL-Parameters-v1610							OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1610		UE-EUTRA-CapabilityAddXDD-Mode-v1610		OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1610		UE-EUTRA-CapabilityAddXDD-Mode-v1610		OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1630-IEs				OPTIONAL

}

UE-EUTRA-Capability-v1630-IEs ::= SEQUENCE {

rf-Parameters-v1630						RF-Parameters-v1630							OPTIONAL,

sl-Parameters-v1630						SL-Parameters-v1630							OPTIONAL,

earlySecurityReactivation-r16			ENUMERATED {supported}					OPTIONAL,

mac-Parameters-v1630					MAC-Parameters-v1630,

measParameters-v1630					MeasParameters-v1630						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1630		UE-EUTRA-CapabilityAddXDD-Mode-v1630,

tdd-Add-UE-EUTRA-Capabilities-v1630		UE-EUTRA-CapabilityAddXDD-Mode-v1630,

nonCriticalExtension					UE-EUTRA-Capability-v1650-IEs		OPTIONAL

}

UE-EUTRA-Capability-v1650-IEs ::= SEQUENCE {

otherParameters-v1650				Other-Parameters-v1650			OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1660-IEs		OPTIONAL

}

UE-EUTRA-Capability-v1660-IEs ::= SEQUENCE {

irat-ParametersNR-v1660			IRAT-ParametersNR-v1660,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-r9 ::=	SEQUENCE {

phyLayerParameters-r9					PhyLayerParameters						OPTIONAL,

featureGroupIndicators-r9				BIT STRING (SIZE (32))					OPTIONAL,

featureGroupIndRel9Add-r9				BIT STRING (SIZE (32))					OPTIONAL,

interRAT-ParametersGERAN-r9				IRAT-ParametersGERAN					OPTIONAL,

interRAT-ParametersUTRA-r9				IRAT-ParametersUTRA-v920				OPTIONAL,

interRAT-ParametersCDMA2000-r9			IRAT-ParametersCDMA2000-1XRTT-v920		OPTIONAL,

neighCellSI-AcquisitionParameters-r9	NeighCellSI-AcquisitionParameters-r9	OPTIONAL,

...

}

UE-EUTRA-CapabilityAddXDD-Mode-v1060 ::=	SEQUENCE {

phyLayerParameters-v1060				PhyLayerParameters-v1020				OPTIONAL,

featureGroupIndRel10-v1060				BIT STRING (SIZE (32))					OPTIONAL,

interRAT-ParametersCDMA2000-v1060		IRAT-ParametersCDMA2000-1XRTT-v1020		OPTIONAL,

interRAT-ParametersUTRA-TDD-v1060		IRAT-ParametersUTRA-TDD-v1020			OPTIONAL,

...,

[[	otdoa-PositioningCapabilities-r10	OTDOA-PositioningCapabilities-r10		OPTIONAL

]]

}

UE-EUTRA-CapabilityAddXDD-Mode-v1130 ::=	SEQUENCE {

phyLayerParameters-v1130					PhyLayerParameters-v1130			OPTIONAL,

measParameters-v1130						MeasParameters-v1130				OPTIONAL,

otherParameters-r11							Other-Parameters-r11				OPTIONAL,

...

}

UE-EUTRA-CapabilityAddXDD-Mode-v1180 ::=	SEQUENCE {

mbms-Parameters-r11					MBMS-Parameters-r11

}

UE-EUTRA-CapabilityAddXDD-Mode-v1250 ::=	SEQUENCE {

phyLayerParameters-v1250			PhyLayerParameters-v1250			OPTIONAL,

measParameters-v1250				MeasParameters-v1250				OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1310 ::=	SEQUENCE {

phyLayerParameters-v1310			PhyLayerParameters-v1310			OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1320 ::=	SEQUENCE {

phyLayerParameters-v1320			PhyLayerParameters-v1320			OPTIONAL,

scptm-Parameters-r13				SCPTM-Parameters-r13				OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1370 ::=	SEQUENCE {

ce-Parameters-v1370					CE-Parameters-v1370					OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1380 ::=	SEQUENCE {

ce-Parameters-v1380					CE-Parameters-v1380

}

UE-EUTRA-CapabilityAddXDD-Mode-v1430 ::=	SEQUENCE {

phyLayerParameters-v1430			PhyLayerParameters-v1430			OPTIONAL,

mmtel-Parameters-r14				MMTEL-Parameters-r14				OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1510 ::=	SEQUENCE {

pdcp-ParametersNR-r15						PDCP-ParametersNR-r15		OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1530 ::=	SEQUENCE {

neighCellSI-AcquisitionParameters-v1530	NeighCellSI-AcquisitionParameters-v1530	OPTIONAL,

reducedCP-Latency-r15			ENUMERATED {supported}					OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1540 ::=	SEQUENCE {

eutra-5GC-Parameters-r15					EUTRA-5GC-Parameters-r15		OPTIONAL,

irat-ParametersNR-v1540						IRAT-ParametersNR-v1540			OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1550 ::=	SEQUENCE {

neighCellSI-AcquisitionParameters-v1550	NeighCellSI-AcquisitionParameters-v1550	OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1560 ::=	SEQUENCE {

pdcp-ParametersNR-v1560					PDCP-ParametersNR-v1560

}

UE-EUTRA-CapabilityAddXDD-Mode-v15a0 ::=	SEQUENCE {

phyLayerParameters-v1530				PhyLayerParameters-v1530				OPTIONAL,

phyLayerParameters-v1540				PhyLayerParameters-v1540				OPTIONAL,

phyLayerParameters-v1550				PhyLayerParameters-v1550				OPTIONAL,

neighCellSI-AcquisitionParameters-v15a0	NeighCellSI-AcquisitionParameters-v15a0

}

UE-EUTRA-CapabilityAddXDD-Mode-v1610 ::= SEQUENCE {

phyLayerParameters-v1610					PhyLayerParameters-v1610				OPTIONAL,

pur-Parameters-r16							PUR-Parameters-r16						OPTIONAL,

measParameters-v1610						MeasParameters-v1610					OPTIONAL,

eutra-5GC-Parameters-v1610					EUTRA-5GC-Parameters-v1610				OPTIONAL,

irat-ParametersNR-v1610						IRAT-ParametersNR-v1610					OPTIONAL,

neighCellSI-AcquisitionParameters-v1610		NeighCellSI-AcquisitionParameters-v1610	OPTIONAL,

mobilityParameters-v1610					MobilityParameters-v1610				OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1630 ::= SEQUENCE {

measParameters-v1630						MeasParameters-v1630

}

AccessStratumRelease ::=			ENUMERATED {

rel8, rel9, rel10, rel11, rel12, rel13,

rel14, rel15, ..., rel16}

FeatureSetsEUTRA-r15 ::=	SEQUENCE {

featureSetsDL-r15			SEQUENCE (SIZE (1..maxFeatureSets-r15)) OF FeatureSetDL-r15		OPTIONAL,

featureSetsDL-PerCC-r15		SEQUENCE (SIZE (1..maxPerCC-FeatureSets-r15)) OF FeatureSetDL-PerCC-r15		OPTIONAL,

featureSetsUL-r15			SEQUENCE (SIZE (1..maxFeatureSets-r15)) OF FeatureSetUL-r15		OPTIONAL,

featureSetsUL-PerCC-r15		SEQUENCE (SIZE (1..maxPerCC-FeatureSets-r15)) OF FeatureSetUL-PerCC-r15		OPTIONAL,

...,

[[	featureSetsDL-v1550		SEQUENCE (SIZE (1..maxFeatureSets-r15)) OF FeatureSetDL-v1550	OPTIONAL

]]

}

MobilityParameters-r14 ::=			SEQUENCE {

makeBeforeBreak-r14					ENUMERATED {supported}					OPTIONAL,

rach-Less-r14						ENUMERATED {supported}					OPTIONAL

}

MobilityParameters-v1610 ::=		SEQUENCE {

cho-r16								ENUMERATED {supported}					OPTIONAL,

cho-FDD-TDD-r16						ENUMERATED {supported}					OPTIONAL,

cho-Failure-r16						ENUMERATED {supported}					OPTIONAL,

cho-TwoTriggerEvents-r16			ENUMERATED {supported}					OPTIONAL

}

DC-Parameters-r12 ::=			SEQUENCE {

drb-TypeSplit-r12						ENUMERATED {supported}			OPTIONAL,

drb-TypeSCG-r12							ENUMERATED {supported}			OPTIONAL

}

DC-Parameters-v1310 ::=			SEQUENCE {

pdcp-TransferSplitUL-r13				ENUMERATED {supported}			OPTIONAL,

ue-SSTD-Meas-r13						ENUMERATED {supported}			OPTIONAL

}

MAC-Parameters-r12 ::=				SEQUENCE {

logicalChannelSR-ProhibitTimer-r12	ENUMERATED {supported}					OPTIONAL,

longDRX-Command-r12					ENUMERATED {supported}					OPTIONAL

}

MAC-Parameters-v1310 ::=				SEQUENCE {

extendedMAC-LengthField-r13		ENUMERATED {supported}				OPTIONAL,

extendedLongDRX-r13				ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-v1430 ::=				SEQUENCE {

shortSPS-IntervalFDD-r14			ENUMERATED {supported}				OPTIONAL,

shortSPS-IntervalTDD-r14			ENUMERATED {supported}				OPTIONAL,

skipUplinkDynamic-r14				ENUMERATED {supported}				OPTIONAL,

skipUplinkSPS-r14					ENUMERATED {supported}				OPTIONAL,

multipleUplinkSPS-r14				ENUMERATED {supported}				OPTIONAL,

dataInactMon-r14					ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-v1440 ::=				SEQUENCE {

rai-Support-r14					ENUMERATED {supported}			OPTIONAL

}

MAC-Parameters-v1530 ::=		SEQUENCE {

min-Proc-TimelineSubslot-r15	SEQUENCE (SIZE(1..3)) OF ProcessingTimelineSet-r15	OPTIONAL,

skipSubframeProcessing-r15			SkipSubframeProcessing-r15						OPTIONAL,

earlyData-UP-r15					ENUMERATED {supported}							OPTIONAL,

dormantSCellState-r15				ENUMERATED {supported}							OPTIONAL,

directSCellActivation-r15			ENUMERATED {supported}							OPTIONAL,

directSCellHibernation-r15			ENUMERATED {supported}							OPTIONAL,

extendedLCID-Duplication-r15		ENUMERATED {supported}							OPTIONAL,

sps-ServingCell-r15					ENUMERATED {supported}							OPTIONAL

}

MAC-Parameters-v1550 ::=				SEQUENCE {

eLCID-Support-r15					ENUMERATED {supported}			OPTIONAL

}

MAC-Parameters-v1610 ::=		SEQUENCE {

directMCG-SCellActivationResume-r16	ENUMERATED {supported}			OPTIONAL,

directSCG-SCellActivationResume-r16	ENUMERATED {supported}			OPTIONAL,

earlyData-UP-5GC-r16				ENUMERATED {supported}			OPTIONAL,

rai-SupportEnh-r16					ENUMERATED {supported}			OPTIONAL

}

MAC-Parameters-v1630 ::=		SEQUENCE {

directSCG-SCellActivationNEDC-r16	ENUMERATED {supported}			OPTIONAL

}

ProcessingTimelineSet-r15 ::=		ENUMERATED {set1, set2}

RLC-Parameters-r12 ::=				SEQUENCE {

extended-RLC-LI-Field-r12			ENUMERATED {supported}

}

RLC-Parameters-v1310 ::=				SEQUENCE {

extendedRLC-SN-SO-Field-r13				ENUMERATED {supported}				OPTIONAL

}

RLC-Parameters-v1430 ::=				SEQUENCE {

extendedPollByte-r14						ENUMERATED {supported}			OPTIONAL

}

RLC-Parameters-v1530 ::=				SEQUENCE {

flexibleUM-AM-Combinations-r15			ENUMERATED {supported}			OPTIONAL,

rlc-AM-Ooo-Delivery-r15					ENUMERATED {supported}			OPTIONAL,

rlc-UM-Ooo-Delivery-r15					ENUMERATED {supported}			OPTIONAL

}

PDCP-Parameters ::=				SEQUENCE {

supportedROHC-Profiles				ROHC-ProfileSupportList-r15,

maxNumberROHC-ContextSessions		ENUMERATED {

cs2, cs4, cs8, cs12, cs16, cs24, cs32,

cs48, cs64, cs128, cs256, cs512, cs1024,

cs16384, spare2, spare1}				DEFAULT cs16,

...

}

PDCP-Parameters-v1130 ::=		SEQUENCE {

pdcp-SN-Extension-r11					ENUMERATED {supported}			OPTIONAL,

supportRohcContextContinue-r11			ENUMERATED {supported}			OPTIONAL

}

PDCP-Parameters-v1310 ::=				SEQUENCE {

pdcp-SN-Extension-18bits-r13			ENUMERATED {supported}	OPTIONAL

}

PDCP-Parameters-v1430 ::=				SEQUENCE {

supportedUplinkOnlyROHC-Profiles-r14		SEQUENCE {

profile0x0006-r14						BOOLEAN

},

maxNumberROHC-ContextSessions-r14		ENUMERATED {

cs2, cs4, cs8, cs12, cs16, cs24, cs32,

cs48, cs64, cs128, cs256, cs512, cs1024,

cs16384, spare2, spare1}				DEFAULT cs16

}

PDCP-Parameters-v1530 ::=			SEQUENCE {

supportedUDC-r15					SupportedUDC-r15				OPTIONAL,

pdcp-Duplication-r15				ENUMERATED {supported}		OPTIONAL

}

PDCP-Parameters-v1610 ::=			SEQUENCE {

pdcp-VersionChangeWithoutHO-r16		ENUMERATED {supported}		OPTIONAL,

ehc-r16								ENUMERATED {supported}		OPTIONAL,

continueEHC-Context-r16				ENUMERATED {supported}		OPTIONAL,

maxNumberEHC-Contexts-r16 			ENUMERATED {cs2, cs4, cs8, cs16, cs32, cs64, cs128, cs256,

cs512, cs1024, cs2048, cs4096, cs8192, cs16384,

cs32768, cs65536}	OPTIONAL,

jointEHC-ROHC-Config-r16			ENUMERATED {supported}		OPTIONAL

}

SupportedUDC-r15 ::=				SEQUENCE {

supportedStandardDic-r15			ENUMERATED {supported}		OPTIONAL,

supportedOperatorDic-r15			SupportedOperatorDic-r15	OPTIONAL

}

SupportedOperatorDic-r15 ::=		SEQUENCE {

versionOfDictionary-r15				INTEGER (0..15),

associatedPLMN-ID-r15				PLMN-Identity

}

PhyLayerParameters ::=				SEQUENCE {

ue-TxAntennaSelectionSupported		BOOLEAN,

ue-SpecificRefSigsSupported		BOOLEAN

}

PhyLayerParameters-v920 ::=		SEQUENCE {

enhancedDualLayerFDD-r9			ENUMERATED {supported}			OPTIONAL,

enhancedDualLayerTDD-r9			ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v9d0 ::=			SEQUENCE {

tm5-FDD-r9						ENUMERATED {supported}			OPTIONAL,

tm5-TDD-r9						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1020 ::=			SEQUENCE {

twoAntennaPortsForPUCCH-r10				ENUMERATED {supported}					OPTIONAL,

tm9-With-8Tx-FDD-r10					ENUMERATED {supported}					OPTIONAL,

pmi-Disabling-r10						ENUMERATED {supported}					OPTIONAL,

crossCarrierScheduling-r10				ENUMERATED {supported}					OPTIONAL,

simultaneousPUCCH-PUSCH-r10				ENUMERATED {supported}					OPTIONAL,

multiClusterPUSCH-WithinCC-r10			ENUMERATED {supported}					OPTIONAL,

nonContiguousUL-RA-WithinCC-List-r10	NonContiguousUL-RA-WithinCC-List-r10	OPTIONAL

}

PhyLayerParameters-v1130 ::=			SEQUENCE {

crs-InterfHandl-r11						ENUMERATED {supported}					OPTIONAL,

ePDCCH-r11								ENUMERATED {supported}					OPTIONAL,

multiACK-CSI-Reporting-r11				ENUMERATED {supported}					OPTIONAL,

ss-CCH-InterfHandl-r11					ENUMERATED {supported}					OPTIONAL,

tdd-SpecialSubframe-r11					ENUMERATED {supported}					OPTIONAL,

txDiv-PUCCH1b-ChSelect-r11				ENUMERATED {supported}					OPTIONAL,

ul-CoMP-r11								ENUMERATED {supported}					OPTIONAL

}

PhyLayerParameters-v1170 ::=			SEQUENCE {

interBandTDD-CA-WithDifferentConfig-r11	BIT STRING (SIZE (2))			OPTIONAL

}

PhyLayerParameters-v1250 ::=			SEQUENCE {

e-HARQ-Pattern-FDD-r12					ENUMERATED {supported}			OPTIONAL,

enhanced-4TxCodebook-r12				ENUMERATED {supported}			OPTIONAL,

tdd-FDD-CA-PCellDuplex-r12				BIT STRING (SIZE (2))			OPTIONAL,

phy-TDD-ReConfig-TDD-PCell-r12			ENUMERATED {supported}			OPTIONAL,

phy-TDD-ReConfig-FDD-PCell-r12			ENUMERATED {supported}			OPTIONAL,

pusch-FeedbackMode-r12					ENUMERATED {supported}			OPTIONAL,

pusch-SRS-PowerControl-SubframeSet-r12	ENUMERATED {supported}			OPTIONAL,

csi-SubframeSet-r12						ENUMERATED {supported}			OPTIONAL,

noResourceRestrictionForTTIBundling-r12	ENUMERATED {supported}			OPTIONAL,

discoverySignalsInDeactSCell-r12		ENUMERATED {supported}			OPTIONAL,

naics-Capability-List-r12				NAICS-Capability-List-r12		OPTIONAL

}

PhyLayerParameters-v1280 ::=			SEQUENCE {

alternativeTBS-Indices-r12				ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1310 ::=			SEQUENCE {

aperiodicCSI-Reporting-r13				BIT STRING (SIZE (2))			OPTIONAL,

codebook-HARQ-ACK-r13					BIT STRING (SIZE (2))			OPTIONAL,

crossCarrierScheduling-B5C-r13			ENUMERATED {supported}			OPTIONAL,

fdd-HARQ-TimingTDD-r13					ENUMERATED {supported}			OPTIONAL,

maxNumberUpdatedCSI-Proc-r13			INTEGER(5..32)					OPTIONAL,

pucch-Format4-r13						ENUMERATED {supported}			OPTIONAL,

pucch-Format5-r13						ENUMERATED {supported}			OPTIONAL,

pucch-SCell-r13							ENUMERATED {supported}			OPTIONAL,

spatialBundling-HARQ-ACK-r13			ENUMERATED {supported}			OPTIONAL,

supportedBlindDecoding-r13				SEQUENCE {

maxNumberDecoding-r13					INTEGER(1..32)				OPTIONAL,

pdcch-CandidateReductions-r13			ENUMERATED {supported}		OPTIONAL,

skipMonitoringDCI-Format0-1A-r13		ENUMERATED {supported}		OPTIONAL

}																		OPTIONAL,

uci-PUSCH-Ext-r13						ENUMERATED {supported}			OPTIONAL,

crs-InterfMitigationTM10-r13			ENUMERATED {supported}			OPTIONAL,

pdsch-CollisionHandling-r13				ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1320 ::=			SEQUENCE {

mimo-UE-Parameters-r13					MIMO-UE-Parameters-r13			OPTIONAL

}

PhyLayerParameters-v1330 ::=			SEQUENCE {

cch-InterfMitigation-RefRecTypeA-r13	ENUMERATED {supported}			OPTIONAL,

cch-InterfMitigation-RefRecTypeB-r13	ENUMERATED {supported}			OPTIONAL,

cch-InterfMitigation-MaxNumCCs-r13		INTEGER (1.. maxServCell-r13)	OPTIONAL,

crs-InterfMitigationTM1toTM9-r13		INTEGER (1.. maxServCell-r13)	OPTIONAL

}

PhyLayerParameters-v13e0 ::=			SEQUENCE {

mimo-UE-Parameters-v13e0				MIMO-UE-Parameters-v13e0

}

PhyLayerParameters-v1430 ::=			SEQUENCE {

ce-PUSCH-NB-MaxTBS-r14					ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-PUSCH-MaxBandwidth-r14			ENUMERATED {bw5, bw20}			OPTIONAL,

ce-HARQ-AckBundling-r14					ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-TenProcesses-r14				ENUMERATED {supported}			OPTIONAL,

ce-RetuningSymbols-r14					ENUMERATED {n0, n1}				OPTIONAL,

ce-PDSCH-PUSCH-Enhancement-r14			ENUMERATED {supported}			OPTIONAL,

ce-SchedulingEnhancement-r14			ENUMERATED {supported}			OPTIONAL,

ce-SRS-Enhancement-r14					ENUMERATED {supported}			OPTIONAL,

ce-PUCCH-Enhancement-r14				ENUMERATED {supported}			OPTIONAL,

ce-ClosedLoopTxAntennaSelection-r14		ENUMERATED {supported}			OPTIONAL,

tdd-SpecialSubframe-r14					ENUMERATED {supported}			OPTIONAL,

tdd-TTI-Bundling-r14					ENUMERATED {supported}			OPTIONAL,

dmrs-LessUpPTS-r14						ENUMERATED {supported}			OPTIONAL,

mimo-UE-Parameters-v1430				MIMO-UE-Parameters-v1430		OPTIONAL,

alternativeTBS-Index-r14				ENUMERATED {supported}			OPTIONAL,

feMBMS-Unicast-Parameters-r14			FeMBMS-Unicast-Parameters-r14	OPTIONAL

}

PhyLayerParameters-v1450 ::=			SEQUENCE {

ce-SRS-EnhancementWithoutComb4-r14		ENUMERATED {supported}			OPTIONAL,

crs-LessDwPTS-r14						ENUMERATED {supported}			OPTIONAL}

PhyLayerParameters-v1470 ::=			SEQUENCE {

mimo-UE-Parameters-v1470				MIMO-UE-Parameters-v1470		OPTIONAL,

srs-UpPTS-6sym-r14						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v14a0 ::=			SEQUENCE {

ssp10-TDD-Only-r14						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1530 ::=			SEQUENCE {

stti-SPT-Capabilities-r15				SEQUENCE {

aperiodicCsi-ReportingSTTI-r15			ENUMERATED {supported}			OPTIONAL,

dmrs-BasedSPDCCH-MBSFN-r15				ENUMERATED {supported}			OPTIONAL,

dmrs-BasedSPDCCH-nonMBSFN-r15			ENUMERATED {supported}			OPTIONAL,

dmrs-PositionPattern-r15				ENUMERATED {supported}			OPTIONAL,

dmrs-SharingSubslotPDSCH-r15			ENUMERATED {supported}			OPTIONAL,

dmrs-RepetitionSubslotPDSCH-r15			ENUMERATED {supported}			OPTIONAL,

epdcch-SPT-differentCells-r15			ENUMERATED {supported}			OPTIONAL,

epdcch-STTI-differentCells-r15			ENUMERATED {supported}			OPTIONAL,

maxLayersSlotOrSubslotPUSCH-r15			ENUMERATED {oneLayer,twoLayers,fourLayers}

OPTIONAL,

maxNumberUpdatedCSI-Proc-SPT-r15		INTEGER(5..32)					OPTIONAL,

maxNumberUpdatedCSI-Proc-STTI-Comb77-r15		INTEGER(1..32)			OPTIONAL,

maxNumberUpdatedCSI-Proc-STTI-Comb27-r15		INTEGER(1..32)			OPTIONAL,

maxNumberUpdatedCSI-Proc-STTI-Comb22-Set1-r15	INTEGER(1..32)			OPTIONAL,

maxNumberUpdatedCSI-Proc-STTI-Comb22-Set2-r15	INTEGER(1..32)			OPTIONAL,

mimo-UE-ParametersSTTI-r15				MIMO-UE-Parameters-r13			OPTIONAL,

mimo-UE-ParametersSTTI-v1530			MIMO-UE-Parameters-v1430		OPTIONAL,

numberOfBlindDecodesUSS-r15				INTEGER(4..32)					OPTIONAL,

pdsch-SlotSubslotPDSCH-Decoding-r15		ENUMERATED {supported}			OPTIONAL,

powerUCI-SlotPUSCH						ENUMERATED {supported}			OPTIONAL,

powerUCI-SubslotPUSCH					ENUMERATED {supported}			OPTIONAL,

slotPDSCH-TxDiv-TM9and10				ENUMERATED {supported}			OPTIONAL,

subslotPDSCH-TxDiv-TM9and10				ENUMERATED {supported}			OPTIONAL,

spdcch-differentRS-types-r15			ENUMERATED {supported}			OPTIONAL,

srs-DCI7-TriggeringFS2-r15				ENUMERATED {supported}			OPTIONAL,

sps-cyclicShift-r15						ENUMERATED {supported}			OPTIONAL,

spdcch-Reuse-r15						ENUMERATED {supported}			OPTIONAL,

sps-STTI-r15							ENUMERATED {slot, subslot, slotAndSubslot}

OPTIONAL,

tm8-slotPDSCH-r15						ENUMERATED {supported}			OPTIONAL,

tm9-slotSubslot-r15						ENUMERATED {supported}			OPTIONAL,

tm9-slotSubslotMBSFN-r15				ENUMERATED {supported}			OPTIONAL,

tm10-slotSubslot-r15					ENUMERATED {supported}			OPTIONAL,

tm10-slotSubslotMBSFN-r15				ENUMERATED {supported}			OPTIONAL,

txDiv-SPUCCH-r15						ENUMERATED {supported}			OPTIONAL,

ul-AsyncHarqSharingDiff-TTI-Lengths-r15	ENUMERATED {supported}			OPTIONAL

}																			OPTIONAL,

ce-Capabilities-r15					SEQUENCE {

ce-CRS-IntfMitig-r15					ENUMERATED {supported}			OPTIONAL,

ce-CQI-AlternativeTable-r15				ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-FlexibleStartPRB-CE-ModeA-r15	ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-FlexibleStartPRB-CE-ModeB-r15	ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-64QAM-r15						ENUMERATED {supported}			OPTIONAL,

ce-PUSCH-FlexibleStartPRB-CE-ModeA-r15	ENUMERATED {supported}			OPTIONAL,

ce-PUSCH-FlexibleStartPRB-CE-ModeB-r15	ENUMERATED {supported}			OPTIONAL,

ce-PUSCH-SubPRB-Allocation-r15			ENUMERATED {supported}			OPTIONAL,

ce-UL-HARQ-ACK-Feedback-r15				ENUMERATED {supported}			OPTIONAL

}	OPTIONAL,

shortCQI-ForSCellActivation-r15			ENUMERATED {supported}			OPTIONAL,

mimo-CBSR-AdvancedCSI-r15				ENUMERATED {supported}			OPTIONAL,

crs-IntfMitig-r15						ENUMERATED {supported}			OPTIONAL,

ul-PowerControlEnhancements-r15			ENUMERATED {supported}			OPTIONAL,

urllc-Capabilities-r15					SEQUENCE {

pdsch-RepSubframe-r15					ENUMERATED {supported}		OPTIONAL,

pdsch-RepSlot-r15						ENUMERATED {supported}		OPTIONAL,

pdsch-RepSubslot-r15					ENUMERATED {supported}		OPTIONAL,

pusch-SPS-MultiConfigSubframe-r15		INTEGER (0..6)				OPTIONAL,

pusch-SPS-MaxConfigSubframe-r15			INTEGER (0..31)				OPTIONAL,

pusch-SPS-MultiConfigSlot-r15			INTEGER (0..6)				OPTIONAL,

pusch-SPS-MaxConfigSlot-r15				INTEGER (0..31)				OPTIONAL,

pusch-SPS-MultiConfigSubslot-r15		INTEGER (0..6)				OPTIONAL,

pusch-SPS-MaxConfigSubslot-r15			INTEGER (0..31)				OPTIONAL,

pusch-SPS-SlotRepPCell-r15				ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SlotRepPSCell-r15				ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SlotRepSCell-r15				ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubframeRepPCell-r15			ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubframeRepPSCell-r15			ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubframeRepSCell-r15			ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubslotRepPCell-r15			ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubslotRepPSCell-r15			ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubslotRepSCell-r15			ENUMERATED {supported}		OPTIONAL,

semiStaticCFI-r15						ENUMERATED {supported}		OPTIONAL,

semiStaticCFI-Pattern-r15				ENUMERATED {supported}		OPTIONAL

}	OPTIONAL,

altMCS-Table-r15						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1540 ::=			SEQUENCE {

stti-SPT-Capabilities-v1540			SEQUENCE {

slotPDSCH-TxDiv-TM8-r15					ENUMERATED {supported}

}												OPTIONAL,

crs-IM-TM1-toTM9-OneRX-Port-v1540		ENUMERATED {supported}			OPTIONAL,

cch-IM-RefRecTypeA-OneRX-Port-v1540		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1550 ::=			SEQUENCE {

dmrs-OverheadReduction-r15				ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1610 ::=			SEQUENCE {

ce-Capabilities-v1610	SEQUENCE {

ce-CSI-RS-Feedback-r16						ENUMERATED {supported}			OPTIONAL,

ce-CSI-RS-FeedbackCodebookRestriction-r16	ENUMERATED {supported}			OPTIONAL,

crs-ChEstMPDCCH-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

crs-ChEstMPDCCH-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

crs-ChEstMPDCCH-CSI-r16						ENUMERATED {supported}			OPTIONAL,

crs-ChEstMPDCCH-ReciprocityTDD-r16			ENUMERATED {supported}			OPTIONAL,

etws-CMAS-RxInConnCE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

etws-CMAS-RxInConnCE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

mpdcch-InLteControlRegionCE-ModeA-r16		ENUMERATED {supported}			OPTIONAL,

mpdcch-InLteControlRegionCE-ModeB-r16		ENUMERATED {supported}			OPTIONAL,

pdsch-InLteControlRegionCE-ModeA-r16		ENUMERATED {supported}			OPTIONAL,

pdsch-InLteControlRegionCE-ModeB-r16		ENUMERATED {supported}			OPTIONAL,

multiTB-Parameters-r16						CE-MultiTB-Parameters-r16 		OPTIONAL,

resourceResvParameters-r16					CE-ResourceResvParameters-r16	OPTIONAL

}	OPTIONAL,

widebandPRG-Slot-r16				ENUMERATED {supported}			OPTIONAL,

widebandPRG-Subslot-r16				ENUMERATED {supported}			OPTIONAL,

widebandPRG-Subframe-r16			ENUMERATED {supported}			OPTIONAL,

addSRS-r16		SEQUENCE {

addSRS-FrequencyHopping-r16		ENUMERATED {supported}			OPTIONAL,

addSRS-AntennaSwitching-r16		ENUMERATED {useBasic}			OPTIONAL,

addSRS-CarrierSwitching-r16		ENUMERATED {supported}			OPTIONAL

} OPTIONAL,

virtualCellID-BasicSRS-r16			ENUMERATED {supported}			OPTIONAL,

virtualCellID-AddSRS-r16		ENUMERATED {supported}			OPTIONAL

}

MIMO-UE-Parameters-r13 ::=				SEQUENCE {

parametersTM9-r13						MIMO-UE-ParametersPerTM-r13		OPTIONAL,

parametersTM10-r13						MIMO-UE-ParametersPerTM-r13		OPTIONAL,

srs-EnhancementsTDD-r13					ENUMERATED {supported}			OPTIONAL,

srs-Enhancements-r13					ENUMERATED {supported}			OPTIONAL,

interferenceMeasRestriction-r13			ENUMERATED {supported}			OPTIONAL

}

MIMO-UE-Parameters-v13e0 ::=			SEQUENCE {

mimo-WeightedLayersCapabilities-r13		MIMO-WeightedLayersCapabilities-r13	OPTIONAL

}

MIMO-UE-Parameters-v1430 ::=			SEQUENCE {

parametersTM9-v1430						MIMO-UE-ParametersPerTM-v1430	OPTIONAL,

parametersTM10-v1430					MIMO-UE-ParametersPerTM-v1430	OPTIONAL

}

MIMO-UE-Parameters-v1470 ::=			SEQUENCE {

parametersTM9-v1470					MIMO-UE-ParametersPerTM-v1470,

parametersTM10-v1470					MIMO-UE-ParametersPerTM-v1470

}

MIMO-UE-ParametersPerTM-r13 ::=			SEQUENCE {

nonPrecoded-r13							MIMO-NonPrecodedCapabilities-r13	OPTIONAL,

beamformed-r13							MIMO-UE-BeamformedCapabilities-r13	OPTIONAL,

channelMeasRestriction-r13				ENUMERATED {supported}				OPTIONAL,

dmrs-Enhancements-r13					ENUMERATED {supported}				OPTIONAL,

csi-RS-EnhancementsTDD-r13				ENUMERATED {supported}				OPTIONAL

}

MIMO-UE-ParametersPerTM-v1430 ::=		SEQUENCE {

nzp-CSI-RS-AperiodicInfo-r14			SEQUENCE {

nMaxProc-r14							INTEGER(5..32),

nMaxResource-r14						ENUMERATED {n1, n2, n4, n8}

}																			OPTIONAL,

nzp-CSI-RS-PeriodicInfo-r14				SEQUENCE {

nMaxResource-r14						ENUMERATED {n1, n2, n4, n8}

}																			OPTIONAL,

zp-CSI-RS-AperiodicInfo-r14					ENUMERATED {supported}			OPTIONAL,

ul-dmrs-Enhancements-r14				ENUMERATED {supported}				OPTIONAL,

densityReductionNP-r14					ENUMERATED {supported}				OPTIONAL,

densityReductionBF-r14					ENUMERATED {supported}				OPTIONAL,

hybridCSI-r14							ENUMERATED {supported}				OPTIONAL,

semiOL-r14								ENUMERATED {supported}				OPTIONAL,

csi-ReportingNP-r14						ENUMERATED {supported}				OPTIONAL,

csi-ReportingAdvanced-r14				ENUMERATED {supported}				OPTIONAL

}

MIMO-UE-ParametersPerTM-v1470 ::=		SEQUENCE {

csi-ReportingAdvancedMaxPorts-r14		ENUMERATED {n8, n12, n16, n20, n24, n28}	OPTIONAL

}

MIMO-CA-ParametersPerBoBC-r13 ::=		SEQUENCE {

parametersTM9-r13						MIMO-CA-ParametersPerBoBCPerTM-r13		OPTIONAL,

parametersTM10-r13						MIMO-CA-ParametersPerBoBCPerTM-r13		OPTIONAL

}

MIMO-CA-ParametersPerBoBC-r15 ::=		SEQUENCE {

parametersTM9-r15						MIMO-CA-ParametersPerBoBCPerTM-r15	OPTIONAL,

parametersTM10-r15						MIMO-CA-ParametersPerBoBCPerTM-r15	OPTIONAL

}

MIMO-CA-ParametersPerBoBC-v1430 ::=		SEQUENCE {

parametersTM9-v1430						MIMO-CA-ParametersPerBoBCPerTM-v1430	OPTIONAL,

parametersTM10-v1430					MIMO-CA-ParametersPerBoBCPerTM-v1430	OPTIONAL

}

MIMO-CA-ParametersPerBoBC-v1470 ::=		SEQUENCE {

parametersTM9-v1470						MIMO-CA-ParametersPerBoBCPerTM-v1470,

parametersTM10-v1470						MIMO-CA-ParametersPerBoBCPerTM-v1470

}

MIMO-CA-ParametersPerBoBCPerTM-r13 ::=	SEQUENCE {

nonPrecoded-r13							MIMO-NonPrecodedCapabilities-r13	OPTIONAL,

beamformed-r13							MIMO-BeamformedCapabilityList-r13	OPTIONAL,

dmrs-Enhancements-r13					ENUMERATED {different}				OPTIONAL

}

MIMO-CA-ParametersPerBoBCPerTM-v1430 ::=	SEQUENCE {

csi-ReportingNP-r14						ENUMERATED {different}				OPTIONAL,

csi-ReportingAdvanced-r14				ENUMERATED {different}				OPTIONAL

}

MIMO-CA-ParametersPerBoBCPerTM-v1470 ::=	SEQUENCE {

csi-ReportingAdvancedMaxPorts-r14		ENUMERATED {n8, n12, n16, n20, n24, n28}	OPTIONAL

}

MIMO-CA-ParametersPerBoBCPerTM-r15 ::=	SEQUENCE {

nonPrecoded-r13							MIMO-NonPrecodedCapabilities-r13	OPTIONAL,

beamformed-r13							MIMO-BeamformedCapabilityList-r13	OPTIONAL,

dmrs-Enhancements-r13					ENUMERATED {different}				OPTIONAL,

csi-ReportingNP-r14						ENUMERATED {different}				OPTIONAL,

csi-ReportingAdvanced-r14				ENUMERATED {different}				OPTIONAL

}

MIMO-NonPrecodedCapabilities-r13 ::=	SEQUENCE {

config1-r13								ENUMERATED {supported}			OPTIONAL,

config2-r13								ENUMERATED {supported}			OPTIONAL,

config3-r13								ENUMERATED {supported}			OPTIONAL,

config4-r13								ENUMERATED {supported}			OPTIONAL

}

MIMO-UE-BeamformedCapabilities-r13 ::=		SEQUENCE {

altCodebook-r13							ENUMERATED {supported}			OPTIONAL,

mimo-BeamformedCapabilities-r13			MIMO-BeamformedCapabilityList-r13

}

MIMO-BeamformedCapabilityList-r13 ::=		SEQUENCE (SIZE (1..maxCSI-Proc-r11)) OF MIMO-BeamformedCapabilities-r13

MIMO-BeamformedCapabilities-r13 ::=		SEQUENCE {

k-Max-r13								INTEGER (1..8),

n-MaxList-r13							BIT STRING (SIZE (1..7))		OPTIONAL

}

MIMO-WeightedLayersCapabilities-r13 ::=		SEQUENCE {

relWeightTwoLayers-r13	ENUMERATED {v1, v1dot25, v1dot5, v1dot75, v2, v2dot5, v3, v4},

relWeightFourLayers-r13	ENUMERATED {v1, v1dot25, v1dot5, v1dot75, v2, v2dot5, v3, v4}	OPTIONAL,

relWeightEightLayers-r13	ENUMERATED {v1, v1dot25, v1dot5, v1dot75, v2, v2dot5, v3, v4}	OPTIONAL,

totalWeightedLayers-r13	INTEGER (2..128)

}

NonContiguousUL-RA-WithinCC-List-r10 ::= SEQUENCE (SIZE (1..maxBands)) OF NonContiguousUL-RA-WithinCC-r10

NonContiguousUL-RA-WithinCC-r10 ::=		SEQUENCE {

nonContiguousUL-RA-WithinCC-Info-r10	ENUMERATED {supported}					OPTIONAL

}

RF-Parameters ::=					SEQUENCE {

supportedBandListEUTRA				SupportedBandListEUTRA

}

RF-Parameters-v9e0 ::=					SEQUENCE {

supportedBandListEUTRA-v9e0				SupportedBandListEUTRA-v9e0				OPTIONAL

}

RF-Parameters-v1020 ::=				SEQUENCE {

supportedBandCombination-r10			SupportedBandCombination-r10

}

RF-Parameters-v1060 ::=				SEQUENCE {

supportedBandCombinationExt-r10			SupportedBandCombinationExt-r10

}

RF-Parameters-v1090 ::=					SEQUENCE {

supportedBandCombination-v1090			SupportedBandCombination-v1090			OPTIONAL

}

RF-Parameters-v10f0 ::=					SEQUENCE {

modifiedMPR-Behavior-r10					BIT STRING (SIZE (32))				OPTIONAL

}

RF-Parameters-v10i0 ::=					SEQUENCE {

supportedBandCombination-v10i0			SupportedBandCombination-v10i0			OPTIONAL

}

RF-Parameters-v10j0 ::=					SEQUENCE {

multiNS-Pmax-r10						ENUMERATED {supported}					OPTIONAL

}

RF-Parameters-v1130 ::=				SEQUENCE {

supportedBandCombination-v1130			SupportedBandCombination-v1130			OPTIONAL

}

RF-Parameters-v1180 ::=				SEQUENCE {

freqBandRetrieval-r11					ENUMERATED {supported}			OPTIONAL,

requestedBands-r11						SEQUENCE (SIZE (1.. maxBands)) OF FreqBandIndicator-r11						OPTIONAL,

supportedBandCombinationAdd-r11			SupportedBandCombinationAdd-r11		OPTIONAL

}

RF-Parameters-v11d0 ::=					SEQUENCE {

supportedBandCombinationAdd-v11d0		SupportedBandCombinationAdd-v11d0		OPTIONAL

}

RF-Parameters-v1250 ::=				SEQUENCE {

supportedBandListEUTRA-v1250				SupportedBandListEUTRA-v1250			OPTIONAL,

supportedBandCombination-v1250			SupportedBandCombination-v1250			OPTIONAL,

supportedBandCombinationAdd-v1250		SupportedBandCombinationAdd-v1250		OPTIONAL,

freqBandPriorityAdjustment-r12			ENUMERATED {supported}					OPTIONAL

}

RF-Parameters-v1270 ::=				SEQUENCE {

supportedBandCombination-v1270			SupportedBandCombination-v1270			OPTIONAL,

supportedBandCombinationAdd-v1270		SupportedBandCombinationAdd-v1270		OPTIONAL

}

RF-Parameters-v1310 ::=				SEQUENCE {

eNB-RequestedParameters-r13			SEQUENCE {

reducedIntNonContCombRequested-r13	ENUMERATED {true}						OPTIONAL,

requestedCCsDL-r13					INTEGER (2..32)							OPTIONAL,

requestedCCsUL-r13					INTEGER (2..32)							OPTIONAL,

skipFallbackCombRequested-r13		ENUMERATED {true}						OPTIONAL

}																				OPTIONAL,

maximumCCsRetrieval-r13					ENUMERATED {supported}					OPTIONAL,

skipFallbackCombinations-r13			ENUMERATED {supported}					OPTIONAL,

reducedIntNonContComb-r13				ENUMERATED {supported}					OPTIONAL,

supportedBandListEUTRA-v1310			SupportedBandListEUTRA-v1310			OPTIONAL,

supportedBandCombinationReduced-r13		SupportedBandCombinationReduced-r13		OPTIONAL

}

RF-Parameters-v1320 ::=				SEQUENCE {

supportedBandListEUTRA-v1320			SupportedBandListEUTRA-v1320			OPTIONAL,

supportedBandCombination-v1320			SupportedBandCombination-v1320			OPTIONAL,

supportedBandCombinationAdd-v1320		SupportedBandCombinationAdd-v1320		OPTIONAL,

supportedBandCombinationReduced-v1320	SupportedBandCombinationReduced-v1320	OPTIONAL

}

RF-Parameters-v1380 ::=				SEQUENCE {

supportedBandCombination-v1380			SupportedBandCombination-v1380			OPTIONAL,

supportedBandCombinationAdd-v1380		SupportedBandCombinationAdd-v1380		OPTIONAL,

supportedBandCombinationReduced-v1380	SupportedBandCombinationReduced-v1380	OPTIONAL

}

RF-Parameters-v1390 ::=				SEQUENCE {

supportedBandCombination-v1390			SupportedBandCombination-v1390			OPTIONAL,

supportedBandCombinationAdd-v1390		SupportedBandCombinationAdd-v1390		OPTIONAL,

supportedBandCombinationReduced-v1390	SupportedBandCombinationReduced-v1390	OPTIONAL

}

RF-Parameters-v12b0 ::=				SEQUENCE {

maxLayersMIMO-Indication-r12			ENUMERATED {supported}					OPTIONAL

}

RF-Parameters-v1430 ::=				SEQUENCE {

supportedBandCombination-v1430			SupportedBandCombination-v1430			OPTIONAL,

supportedBandCombinationAdd-v1430		SupportedBandCombinationAdd-v1430		OPTIONAL,

supportedBandCombinationReduced-v1430	SupportedBandCombinationReduced-v1430	OPTIONAL,

eNB-RequestedParameters-v1430			SEQUENCE {

requestedDiffFallbackCombList-r14		BandCombinationList-r14

}																				OPTIONAL,

diffFallbackCombReport-r14				ENUMERATED {supported}					OPTIONAL

}

RF-Parameters-v1450 ::=				SEQUENCE {

supportedBandCombination-v1450			SupportedBandCombination-v1450			OPTIONAL,

supportedBandCombinationAdd-v1450		SupportedBandCombinationAdd-v1450		OPTIONAL,

supportedBandCombinationReduced-v1450	SupportedBandCombinationReduced-v1450	OPTIONAL

}

RF-Parameters-v1470 ::=				SEQUENCE {

supportedBandCombination-v1470			SupportedBandCombination-v1470			OPTIONAL,

supportedBandCombinationAdd-v1470		SupportedBandCombinationAdd-v1470		OPTIONAL,

supportedBandCombinationReduced-v1470	SupportedBandCombinationReduced-v1470	OPTIONAL

}

RF-Parameters-v14b0 ::=				SEQUENCE {

supportedBandCombination-v14b0			SupportedBandCombination-v14b0			OPTIONAL,

supportedBandCombinationAdd-v14b0		SupportedBandCombinationAdd-v14b0		OPTIONAL,

supportedBandCombinationReduced-v14b0	SupportedBandCombinationReduced-v14b0	OPTIONAL

}

RF-Parameters-v1530 ::=				SEQUENCE {

sTTI-SPT-Supported-r15					ENUMERATED {supported}					OPTIONAL,

supportedBandCombination-v1530			SupportedBandCombination-v1530			OPTIONAL,

supportedBandCombinationAdd-v1530		SupportedBandCombinationAdd-v1530		OPTIONAL,

supportedBandCombinationReduced-v1530	SupportedBandCombinationReduced-v1530	OPTIONAL,

powerClass-14dBm-r15					ENUMERATED {supported}					OPTIONAL

}

RF-Parameters-v1570 ::=			SEQUENCE {

dl-1024QAM-ScalingFactor-r15			ENUMERATED {v1, v1dot2, v1dot25},

dl-1024QAM-TotalWeightedLayers-r15		INTEGER (0..10)

}

RF-Parameters-v1610 ::=				SEQUENCE {

supportedBandCombination-v1610			SupportedBandCombination-v1610			OPTIONAL,

supportedBandCombinationAdd-v1610		SupportedBandCombinationAdd-v1610		OPTIONAL,

supportedBandCombinationReduced-v1610	SupportedBandCombinationReduced-v1610	OPTIONAL

}

RF-Parameters-v1630 ::=				SEQUENCE {

supportedBandCombination-v1630			SupportedBandCombination-v1630			OPTIONAL,

supportedBandCombinationAdd-v1630		SupportedBandCombinationAdd-v1630		OPTIONAL,

supportedBandCombinationReduced-v1630	SupportedBandCombinationReduced-v1630	OPTIONAL

}

SkipSubframeProcessing-r15 ::=		SEQUENCE {

skipProcessingDL-Slot-r15			INTEGER (0..3)					OPTIONAL,

skipProcessingDL-SubSlot-r15		INTEGER (0..3)					OPTIONAL,

skipProcessingUL-Slot-r15			INTEGER (0..3)					OPTIONAL,

skipProcessingUL-SubSlot-r15		INTEGER (0..3)					OPTIONAL

}

SPT-Parameters-r15 ::=				SEQUENCE {

frameStructureType-SPT-r15			BIT STRING (SIZE (3))			OPTIONAL,

maxNumberCCs-SPT-r15				INTEGER (1..32)					OPTIONAL

}

STTI-SPT-BandParameters-r15 ::= SEQUENCE {

dl-1024QAM-Slot-r15						ENUMERATED {supported}			OPTIONAL,

dl-1024QAM-SubslotTA-1-r15				ENUMERATED {supported}			OPTIONAL,

dl-1024QAM-SubslotTA-2-r15				ENUMERATED {supported}			OPTIONAL,

simultaneousTx-differentTx-duration-r15	ENUMERATED {supported}			OPTIONAL,

sTTI-CA-MIMO-ParametersDL-r15			CA-MIMO-ParametersDL-r15		OPTIONAL,

sTTI-CA-MIMO-ParametersUL-r15			CA-MIMO-ParametersUL-r15,

sTTI-FD-MIMO-Coexistence				ENUMERATED {supported}			OPTIONAL,

sTTI-MIMO-CA-ParametersPerBoBCs-r15		MIMO-CA-ParametersPerBoBC-r13	OPTIONAL,

sTTI-MIMO-CA-ParametersPerBoBCs-v1530	MIMO-CA-ParametersPerBoBC-v1430	OPTIONAL,

sTTI-SupportedCombinations-r15			STTI-SupportedCombinations-r15	OPTIONAL,

sTTI-SupportedCSI-Proc-r15				ENUMERATED {n1, n3, n4}			OPTIONAL,

ul-256QAM-Slot-r15						ENUMERATED {supported}			OPTIONAL,

ul-256QAM-Subslot-r15					ENUMERATED {supported}			OPTIONAL,

...

}

STTI-SupportedCombinations-r15 ::=	SEQUENCE {

combination-22-r15					DL-UL-CCs-r15					OPTIONAL,

combination-77-r15					DL-UL-CCs-r15					OPTIONAL,

combination-27-r15					DL-UL-CCs-r15					OPTIONAL,

combination-22-27-r15				SEQUENCE (SIZE (1..2)) OF DL-UL-CCs-r15		OPTIONAL,

combination-77-22-r15				SEQUENCE (SIZE (1..2)) OF DL-UL-CCs-r15		OPTIONAL,

combination-77-27-r15				SEQUENCE (SIZE (1..2)) OF DL-UL-CCs-r15		OPTIONAL

}

DL-UL-CCs-r15 ::= SEQUENCE {

maxNumberDL-CCs-r15				INTEGER (1..32)						OPTIONAL,

maxNumberUL-CCs-r15				INTEGER (1..32)						OPTIONAL

}

SupportedBandCombination-r10 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-r10

SupportedBandCombinationExt-r10 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParametersExt-r10

SupportedBandCombination-v1090 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1090

SupportedBandCombination-v10i0 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v10i0

SupportedBandCombination-v1130 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1130

SupportedBandCombination-v1250 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1250

SupportedBandCombination-v1270 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1270

SupportedBandCombination-v1320 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1320

SupportedBandCombination-v1380 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1380

SupportedBandCombination-v1390 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1390

SupportedBandCombination-v1430 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1430

SupportedBandCombination-v1450 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1450

SupportedBandCombination-v1470 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1470

SupportedBandCombination-v14b0 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v14b0

SupportedBandCombination-v1530 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1530

SupportedBandCombination-v1610 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1610

SupportedBandCombination-v1630 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1630

SupportedBandCombinationAdd-r11 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-r11

SupportedBandCombinationAdd-v11d0 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v10i0

SupportedBandCombinationAdd-v1250 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1250

SupportedBandCombinationAdd-v1270 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1270

SupportedBandCombinationAdd-v1320 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1320

SupportedBandCombinationAdd-v1380 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1380

SupportedBandCombinationAdd-v1390 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1390

SupportedBandCombinationAdd-v1430 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1430

SupportedBandCombinationAdd-v1450 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1450

SupportedBandCombinationAdd-v1470 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1470

SupportedBandCombinationAdd-v14b0 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v14b0

SupportedBandCombinationAdd-v1530 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1530

SupportedBandCombinationAdd-v1610 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1610

SupportedBandCombinationAdd-v1630 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1630

SupportedBandCombinationReduced-r13 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-r13

SupportedBandCombinationReduced-v1320 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1320

SupportedBandCombinationReduced-v1380 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1380

SupportedBandCombinationReduced-v1390 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1390

SupportedBandCombinationReduced-v1430 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1430

SupportedBandCombinationReduced-v1450 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1450

SupportedBandCombinationReduced-v1470 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1470

SupportedBandCombinationReduced-v14b0 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v14b0

SupportedBandCombinationReduced-v1530 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1530

SupportedBandCombinationReduced-v1610 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1610

SupportedBandCombinationReduced-v1630 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1630

BandCombinationParameters-r10 ::= SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF BandParameters-r10

BandCombinationParametersExt-r10 ::= SEQUENCE {

supportedBandwidthCombinationSet-r10	SupportedBandwidthCombinationSet-r10	OPTIONAL

}

BandCombinationParameters-v1090 ::= SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF BandParameters-v1090

BandCombinationParameters-v10i0::= SEQUENCE {

bandParameterList-v10i0			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v10i0	OPTIONAL

}

BandCombinationParameters-v1130 ::=	SEQUENCE {

multipleTimingAdvance-r11		ENUMERATED {supported}					OPTIONAL,

simultaneousRx-Tx-r11			ENUMERATED {supported}					OPTIONAL,

bandParameterList-r11			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF BandParameters-v1130	OPTIONAL,

...

}

BandCombinationParameters-r11 ::=	SEQUENCE {

bandParameterList-r11			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-r11,

supportedBandwidthCombinationSet-r11	SupportedBandwidthCombinationSet-r10	OPTIONAL,

multipleTimingAdvance-r11		ENUMERATED {supported}					OPTIONAL,

simultaneousRx-Tx-r11			ENUMERATED {supported}					OPTIONAL,

bandInfoEUTRA-r11				BandInfoEUTRA,

...

}

BandCombinationParameters-v1250::= SEQUENCE {

dc-Support-r12					SEQUENCE {

asynchronous-r12				ENUMERATED {supported}			OPTIONAL,

supportedCellGrouping-r12		CHOICE {

threeEntries-r12				BIT STRING (SIZE(3)),

fourEntries-r12					BIT STRING (SIZE(7)),

fiveEntries-r12					BIT STRING (SIZE(15))

}																OPTIONAL

}																	OPTIONAL,

supportedNAICS-2CRS-AP-r12		BIT STRING (SIZE (1..maxNAICS-Entries-r12))		OPTIONAL,

commSupportedBandsPerBC-r12				BIT STRING (SIZE (1.. maxBands))		OPTIONAL,

...

}

BandCombinationParameters-v1270 ::= SEQUENCE {

bandParameterList-v1270			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1270		OPTIONAL

}

BandCombinationParameters-r13 ::=	SEQUENCE {

differentFallbackSupported-r13	ENUMERATED {true}				OPTIONAL,

bandParameterList-r13			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF BandParameters-r13,

supportedBandwidthCombinationSet-r13	SupportedBandwidthCombinationSet-r10	OPTIONAL,

multipleTimingAdvance-r13		ENUMERATED {supported}				OPTIONAL,

simultaneousRx-Tx-r13			ENUMERATED {supported}				OPTIONAL,

bandInfoEUTRA-r13				BandInfoEUTRA,

dc-Support-r13					SEQUENCE {

asynchronous-r13			ENUMERATED {supported}				OPTIONAL,

supportedCellGrouping-r13		CHOICE {

threeEntries-r13				BIT STRING (SIZE(3)),

fourEntries-r13					BIT STRING (SIZE(7)),

fiveEntries-r13					BIT STRING (SIZE(15))

}																OPTIONAL

}																	OPTIONAL,

supportedNAICS-2CRS-AP-r13		BIT STRING (SIZE (1..maxNAICS-Entries-r12))	OPTIONAL,

commSupportedBandsPerBC-r13		BIT STRING (SIZE (1.. maxBands))		OPTIONAL

}

BandCombinationParameters-v1320 ::= SEQUENCE {

bandParameterList-v1320			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1320		OPTIONAL,

additionalRx-Tx-PerformanceReq-r13		ENUMERATED {supported}					OPTIONAL

}

BandCombinationParameters-v1380 ::= SEQUENCE {

bandParameterList-v1380		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1380		OPTIONAL

}

BandCombinationParameters-v1390 ::= SEQUENCE {

ue-CA-PowerClass-N-r13			ENUMERATED {class2}				OPTIONAL

}

BandCombinationParameters-v1430 ::= SEQUENCE {

bandParameterList-v1430			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1430		OPTIONAL,

v2x-SupportedTxBandCombListPerBC-r14			BIT STRING (SIZE (1.. maxBandComb-r13))		OPTIONAL,

v2x-SupportedRxBandCombListPerBC-r14			BIT STRING (SIZE (1.. maxBandComb-r13))		OPTIONAL

}

BandCombinationParameters-v1450 ::= SEQUENCE {

bandParameterList-v1450			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1450		OPTIONAL

}

BandCombinationParameters-v1470 ::= SEQUENCE {

bandParameterList-v1470			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1470		OPTIONAL,

srs-MaxSimultaneousCCs-r14	INTEGER (1..31)				OPTIONAL

}

BandCombinationParameters-v14b0 ::= SEQUENCE {

bandParameterList-v14b0			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v14b0		OPTIONAL

}

BandCombinationParameters-v1530 ::= SEQUENCE {

bandParameterList-v1530		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF							BandParameters-v1530		OPTIONAL,

spt-Parameters-r15				SPT-Parameters-r15				OPTIONAL

}

-- If an additional band combination parameter is defined, which is supported for MR-DC,

--  it shall be defined in the IE CA-ParametersEUTRA in TS 38.331 [82].

BandCombinationParameters-v1610 ::= SEQUENCE {

measGapInfoNR					MeasGapInfoNR					OPTIONAL,

bandParameterList-v1610 		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF 							BandParameters-v1610		OPTIONAL,

interFreqDAPS-r16						SEQUENCE {

interFreqAsyncDAPS-r16					ENUMERATED {supported}		OPTIONAL,

interFreqMultiUL-TransmissionDAPS-r16	ENUMERATED {supported}		OPTIONAL

}																		OPTIONAL

}

BandCombinationParameters-v1630 ::= SEQUENCE {

v2x-SupportedTxBandCombListPerBC-v1630		BIT STRING (SIZE (1..maxBandCombSidelinkNR-r16))		OPTIONAL,

v2x-SupportedRxBandCombListPerBC-v1630		BIT STRING (SIZE (1..maxBandCombSidelinkNR-r16))		OPTIONAL,

scalingFactorTxSidelink-r16					SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF ScalingFactorSidelink-r16		OPTIONAL,

scalingFactorRxSidelink-r16					SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF ScalingFactorSidelink-r16		OPTIONAL,

interBandPowerSharingSyncDAPS-r16			ENUMERATED {supported}	OPTIONAL,

interBandPowerSharingAsyncDAPS-r16			ENUMERATED {supported}	OPTIONAL

}

ScalingFactorSidelink-r16 ::=						ENUMERATED {f0p4, f0p75, f0p8, f1}

SupportedBandwidthCombinationSet-r10 ::=	BIT STRING (SIZE (1..maxBandwidthCombSet-r10))

BandParameters-r10 ::= SEQUENCE {

bandEUTRA-r10					FreqBandIndicator,

bandParametersUL-r10			BandParametersUL-r10					OPTIONAL,

bandParametersDL-r10			BandParametersDL-r10					OPTIONAL

}

BandParameters-v1090 ::= SEQUENCE {

bandEUTRA-v1090					FreqBandIndicator-v9e0					OPTIONAL,

...

}

BandParameters-v10i0::= SEQUENCE {

bandParametersDL-v10i0		SEQUENCE (SIZE (1..maxBandwidthClass-r10)) OF CA-MIMO-ParametersDL-v10i0

}

BandParameters-v1130 ::= SEQUENCE {

supportedCSI-Proc-r11			ENUMERATED {n1, n3, n4}

}

BandParameters-r11 ::= SEQUENCE {

bandEUTRA-r11					FreqBandIndicator-r11,

bandParametersUL-r11			BandParametersUL-r10					OPTIONAL,

bandParametersDL-r11			BandParametersDL-r10					OPTIONAL,

supportedCSI-Proc-r11			ENUMERATED {n1, n3, n4}					OPTIONAL

}

BandParameters-v1270 ::= SEQUENCE {

bandParametersDL-v1270			SEQUENCE (SIZE (1..maxBandwidthClass-r10)) OF CA-MIMO-ParametersDL-v1270

}

BandParameters-r13 ::= SEQUENCE {

bandEUTRA-r13					FreqBandIndicator-r11,

bandParametersUL-r13				BandParametersUL-r13				OPTIONAL,

bandParametersDL-r13				BandParametersDL-r13				OPTIONAL,

supportedCSI-Proc-r13			ENUMERATED {n1, n3, n4}			OPTIONAL

}

BandParameters-v1320 ::= SEQUENCE {

bandParametersDL-v1320			MIMO-CA-ParametersPerBoBC-r13

}

BandParameters-v1380 ::=	SEQUENCE {

txAntennaSwitchDL-r13			INTEGER (1..32)					OPTIONAL,

txAntennaSwitchUL-r13			INTEGER (1..32)					OPTIONAL

}

BandParameters-v1430 ::= SEQUENCE {

bandParametersDL-v1430			MIMO-CA-ParametersPerBoBC-v1430	OPTIONAL,

ul-256QAM-r14						ENUMERATED {supported}		OPTIONAL,

ul-256QAM-perCC-InfoList-r14		SEQUENCE (SIZE (2..maxServCell-r13)) OF UL-256QAM-perCC-Info-r14		OPTIONAL,

srs-CapabilityPerBandPairList-r14		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

SRS-CapabilityPerBandPair-r14	OPTIONAL

}

BandParameters-v1450 ::= SEQUENCE {

must-CapabilityPerBand-r14		MUST-Parameters-r14		OPTIONAL

}

BandParameters-v1470 ::= SEQUENCE {

bandParametersDL-v1470			MIMO-CA-ParametersPerBoBC-v1470	OPTIONAL

}

BandParameters-v14b0 ::= SEQUENCE {

srs-CapabilityPerBandPairList-v14b0		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF		SRS-CapabilityPerBandPair-v14b0		OPTIONAL

}

BandParameters-v1530 ::=	SEQUENCE {

ue-TxAntennaSelection-SRS-1T4R-r15				ENUMERATED {supported}	OPTIONAL,

ue-TxAntennaSelection-SRS-2T4R-2Pairs-r15		ENUMERATED {supported}	OPTIONAL,

ue-TxAntennaSelection-SRS-2T4R-3Pairs-r15		ENUMERATED {supported}	OPTIONAL,

dl-1024QAM-r15									ENUMERATED {supported}	OPTIONAL,

qcl-TypeC-Operation-r15							ENUMERATED {supported}	OPTIONAL,

qcl-CRI-BasedCSI-Reporting-r15					ENUMERATED {supported}	OPTIONAL,

stti-SPT-BandParameters-r15					STTI-SPT-BandParameters-r15	OPTIONAL

}

BandParameters-v1610 ::= 	SEQUENCE {

intraFreqDAPS-r16		SEQUENCE {

intraFreqAsyncDAPS-r16					ENUMERATED {supported}		OPTIONAL,

dummy									ENUMERATED {supported}		OPTIONAL,

intraFreqTwoTAGs-DAPS-r16				ENUMERATED {supported}		OPTIONAL

}																	OPTIONAL,

addSRS-FrequencyHopping-r16 ENUMERATED {supported}			OPTIONAL,

addSRS-AntennaSwitching-r16	SEQUENCE {

addSRS-1T2R-r16			ENUMERATED {supported}			OPTIONAL,

addSRS-1T4R-r16			ENUMERATED {supported}			OPTIONAL,

addSRS-2T4R-2pairs-r16	ENUMERATED {supported}			OPTIONAL,

addSRS-2T4R-3pairs-r16	ENUMERATED {supported}			OPTIONAL

}				OPTIONAL,

srs-CapabilityPerBandPairList-v1610		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

SRS-CapabilityPerBandPair-v1610	OPTIONAL

}

V2X-BandParameters-r14 ::= SEQUENCE {

v2x-FreqBandEUTRA-r14			FreqBandIndicator-r11,

bandParametersTxSL-r14			BandParametersTxSL-r14				OPTIONAL,

bandParametersRxSL-r14			BandParametersRxSL-r14				OPTIONAL

}

V2X-BandParameters-v1530 ::= SEQUENCE {

v2x-EnhancedHighReception-r15			ENUMERATED {supported}		OPTIONAL

}

BandParametersTxSL-r14 ::= SEQUENCE {

v2x-BandwidthClassTxSL-r14		V2X-BandwidthClassSL-r14,

v2x-eNB-Scheduled-r14			ENUMERATED {supported}				OPTIONAL,

v2x-HighPower-r14				ENUMERATED {supported}				OPTIONAL

}

BandParametersRxSL-r14 ::= SEQUENCE {

v2x-BandwidthClassRxSL-r14		V2X-BandwidthClassSL-r14,

v2x-HighReception-r14			ENUMERATED {supported}				OPTIONAL

}

V2X-BandwidthClassSL-r14 ::= SEQUENCE (SIZE (1..maxBandwidthClass-r10)) OF V2X-BandwidthClass-r14

UL-256QAM-perCC-Info-r14 ::= SEQUENCE {

ul-256QAM-perCC-r14			ENUMERATED {supported}				OPTIONAL

}

FeatureSetDL-r15 ::=	SEQUENCE {

mimo-CA-ParametersPerBoBC-r15	MIMO-CA-ParametersPerBoBC-r15			OPTIONAL,

featureSetPerCC-ListDL-r15	SEQUENCE (SIZE (1..maxServCell-r13)) OF FeatureSetDL-PerCC-Id-r15

}

FeatureSetDL-v1550 ::=	SEQUENCE {

dl-1024QAM-r15				ENUMERATED {supported}			OPTIONAL

}

FeatureSetDL-PerCC-r15 ::=	SEQUENCE {

fourLayerTM3-TM4-r15						ENUMERATED {supported}				OPTIONAL,

supportedMIMO-CapabilityDL-MRDC-r15		MIMO-CapabilityDL-r10					OPTIONAL,

supportedCSI-Proc-r15						ENUMERATED {n1, n3, n4}				OPTIONAL

}

FeatureSetUL-r15 ::=	SEQUENCE {

featureSetPerCC-ListUL-r15	SEQUENCE (SIZE(1..maxServCell-r13)) OF FeatureSetUL-PerCC-Id-r15

}

FeatureSetUL-PerCC-r15 ::=	SEQUENCE {

supportedMIMO-CapabilityUL-r15		MIMO-CapabilityUL-r10				OPTIONAL,

ul-256QAM-r15						ENUMERATED {supported}				OPTIONAL

}

FeatureSetDL-PerCC-Id-r15 ::=	INTEGER (0..maxPerCC-FeatureSets-r15)

FeatureSetUL-PerCC-Id-r15 ::=	INTEGER (0..maxPerCC-FeatureSets-r15)

BandParametersUL-r10 ::= SEQUENCE (SIZE (1..maxBandwidthClass-r10)) OF CA-MIMO-ParametersUL-r10

BandParametersUL-r13 ::= CA-MIMO-ParametersUL-r10

CA-MIMO-ParametersUL-r10 ::= SEQUENCE {

ca-BandwidthClassUL-r10				CA-BandwidthClass-r10,

supportedMIMO-CapabilityUL-r10		MIMO-CapabilityUL-r10				OPTIONAL

}

CA-MIMO-ParametersUL-r15 ::= SEQUENCE {

supportedMIMO-CapabilityUL-r15		MIMO-CapabilityUL-r10				OPTIONAL

}

BandParametersDL-r10 ::= SEQUENCE (SIZE (1..maxBandwidthClass-r10)) OF CA-MIMO-ParametersDL-r10

BandParametersDL-r13 ::= CA-MIMO-ParametersDL-r13

CA-MIMO-ParametersDL-r10 ::= SEQUENCE {

ca-BandwidthClassDL-r10				CA-BandwidthClass-r10,

supportedMIMO-CapabilityDL-r10		MIMO-CapabilityDL-r10				OPTIONAL

}

CA-MIMO-ParametersDL-v10i0 ::= SEQUENCE {

fourLayerTM3-TM4-r10				ENUMERATED {supported}				OPTIONAL

}

CA-MIMO-ParametersDL-v1270 ::= SEQUENCE {

intraBandContiguousCC-InfoList-r12			SEQUENCE (SIZE (1..maxServCell-r10)) OF IntraBandContiguousCC-Info-r12

}

CA-MIMO-ParametersDL-r13 ::= SEQUENCE {

ca-BandwidthClassDL-r13					CA-BandwidthClass-r10,

supportedMIMO-CapabilityDL-r13			MIMO-CapabilityDL-r10				OPTIONAL,

fourLayerTM3-TM4-r13						ENUMERATED {supported}				OPTIONAL,

intraBandContiguousCC-InfoList-r13		SEQUENCE (SIZE (1..maxServCell-r13)) OF IntraBandContiguousCC-Info-r12

}

CA-MIMO-ParametersDL-r15 ::= SEQUENCE {

supportedMIMO-CapabilityDL-r15			MIMO-CapabilityDL-r10				OPTIONAL,

fourLayerTM3-TM4-r15					ENUMERATED {supported}				OPTIONAL,

intraBandContiguousCC-InfoList-r15		SEQUENCE (SIZE (1..maxServCell-r13)) OF

IntraBandContiguousCC-Info-r12				OPTIONAL

}

IntraBandContiguousCC-Info-r12 ::= SEQUENCE {

fourLayerTM3-TM4-perCC-r12			ENUMERATED {supported}				OPTIONAL,

supportedMIMO-CapabilityDL-r12		MIMO-CapabilityDL-r10				OPTIONAL,

supportedCSI-Proc-r12				ENUMERATED {n1, n3, n4}				OPTIONAL

}

CA-BandwidthClass-r10 ::= ENUMERATED {a, b, c, d, e, f, ...}

V2X-BandwidthClass-r14 ::= ENUMERATED {a, b, c, d, e, f, ..., c1-v1530}

MIMO-CapabilityUL-r10 ::= ENUMERATED {twoLayers, fourLayers}

MIMO-CapabilityDL-r10 ::= ENUMERATED {twoLayers, fourLayers, eightLayers}

MUST-Parameters-r14 ::= SEQUENCE {

must-TM234-UpTo2Tx-r14						ENUMERATED {supported}		OPTIONAL,

must-TM89-UpToOneInterferingLayer-r14		ENUMERATED {supported}		OPTIONAL,

must-TM10-UpToOneInterferingLayer-r14		ENUMERATED {supported}		OPTIONAL,

must-TM89-UpToThreeInterferingLayers-r14	ENUMERATED {supported}		OPTIONAL,

must-TM10-UpToThreeInterferingLayers-r14	ENUMERATED {supported}		OPTIONAL

}

SupportedBandListEUTRA ::=			SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA

SupportedBandListEUTRA-v9e0::=			SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v9e0

SupportedBandListEUTRA-v1250 ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v1250

SupportedBandListEUTRA-v1310 ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v1310

SupportedBandListEUTRA-v1320 ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v1320

SupportedBandEUTRA ::=				SEQUENCE {

bandEUTRA							FreqBandIndicator,

halfDuplex							BOOLEAN

}

SupportedBandEUTRA-v9e0 ::=		SEQUENCE {

bandEUTRA-v9e0						FreqBandIndicator-v9e0		OPTIONAL

}

SupportedBandEUTRA-v1250 ::=		SEQUENCE {

dl-256QAM-r12						ENUMERATED {supported}		OPTIONAL,

ul-64QAM-r12						ENUMERATED {supported}		OPTIONAL

}

SupportedBandEUTRA-v1310 ::=		SEQUENCE {

ue-PowerClass-5-r13			ENUMERATED {supported}		OPTIONAL

}

SupportedBandEUTRA-v1320 ::=		SEQUENCE {

intraFreq-CE-NeedForGaps-r13				ENUMERATED {supported}				OPTIONAL,

ue-PowerClass-N-r13			ENUMERATED {class1, class2, class4}		OPTIONAL

}

MeasParameters ::=					SEQUENCE {

bandListEUTRA						BandListEUTRA

}

MeasParameters-v1020 ::=			SEQUENCE {

bandCombinationListEUTRA-r10			BandCombinationListEUTRA-r10

}

MeasParameters-v1130 ::=			SEQUENCE {

rsrqMeasWideband-r11			ENUMERATED {supported}					OPTIONAL

}

MeasParameters-v11a0 ::=			SEQUENCE {

benefitsFromInterruption-r11			ENUMERATED {true}				OPTIONAL

}

MeasParameters-v1250 ::=			SEQUENCE {

timerT312-r12						ENUMERATED {supported}		OPTIONAL,

alternativeTimeToTrigger-r12		ENUMERATED {supported}		OPTIONAL,

incMonEUTRA-r12						ENUMERATED {supported}		OPTIONAL,

incMonUTRA-r12						ENUMERATED {supported}		OPTIONAL,

extendedMaxMeasId-r12				ENUMERATED {supported}		OPTIONAL,

extendedRSRQ-LowerRange-r12			ENUMERATED {supported}		OPTIONAL,

rsrq-OnAllSymbols-r12				ENUMERATED {supported}		OPTIONAL,

crs-DiscoverySignalsMeas-r12		ENUMERATED {supported}		OPTIONAL,

csi-RS-DiscoverySignalsMeas-r12		ENUMERATED {supported}		OPTIONAL

}

MeasParameters-v1310 ::=			SEQUENCE {

rs-SINR-Meas-r13						ENUMERATED {supported}		OPTIONAL,

whiteCellList-r13						ENUMERATED {supported}		OPTIONAL,

extendedMaxObjectId-r13					ENUMERATED {supported}		OPTIONAL,

ul-PDCP-Delay-r13						ENUMERATED {supported}		OPTIONAL,

extendedFreqPriorities-r13				ENUMERATED {supported}		OPTIONAL,

multiBandInfoReport-r13					ENUMERATED {supported}		OPTIONAL,

rssi-AndChannelOccupancyReporting-r13	ENUMERATED {supported}		OPTIONAL

}

MeasParameters-v1430 ::=			SEQUENCE {

ceMeasurements-r14						ENUMERATED {supported}		OPTIONAL,

ncsg-r14								ENUMERATED {supported}				OPTIONAL,

shortMeasurementGap-r14					ENUMERATED {supported}				OPTIONAL,

perServingCellMeasurementGap-r14		ENUMERATED {supported}				OPTIONAL,

nonUniformGap-r14						ENUMERATED {supported}				OPTIONAL

}

MeasParameters-v1520 ::=			SEQUENCE {

measGapPatterns-r15					BIT STRING (SIZE (8))		OPTIONAL

}

MeasParameters-v1530 ::=			SEQUENCE {

qoe-MeasReport-r15					ENUMERATED {supported}		OPTIONAL,

qoe-MTSI-MeasReport-r15				ENUMERATED {supported}		OPTIONAL,

ca-IdleModeMeasurements-r15				ENUMERATED {supported}		OPTIONAL,

ca-IdleModeValidityArea-r15				ENUMERATED {supported}		OPTIONAL,

heightMeas-r15							ENUMERATED {supported}			OPTIONAL,

multipleCellsMeasExtension-r15			ENUMERATED {supported}			OPTIONAL

}

MeasParameters-v1610 ::=	SEQUENCE {

bandInfoNR-v1610					SEQUENCE (SIZE (1..maxBands)) OF MeasGapInfoNR	OPTIONAL,

altFreqPriority-r16					ENUMERATED {supported}							OPTIONAL,

ce-DL-ChannelQualityReporting-r16	ENUMERATED {supported}							OPTIONAL,

ce-MeasRSS-Dedicated-r16			ENUMERATED {supported}							OPTIONAL,

eutra-IdleInactiveMeasurements-r16			ENUMERATED {supported}		OPTIONAL,

nr-IdleInactiveMeasFR1-r16			ENUMERATED {supported}		OPTIONAL,

nr-IdleInactiveMeasFR2-r16			ENUMERATED {supported}		OPTIONAL,

idleInactiveValidityAreaList-r16		ENUMERATED {supported}		OPTIONAL,

measGapPatterns-NRonly-r16			ENUMERATED {supported}		OPTIONAL,

measGapPatterns-NRonly-ENDC-r16		ENUMERATED {supported}		OPTIONAL

}

MeasParameters-v1630 ::=	SEQUENCE {

nr-IdleInactiveBeamMeasFR1-r16		ENUMERATED {supported}		OPTIONAL,

nr-IdleInactiveBeamMeasFR2-r16		ENUMERATED {supported}		OPTIONAL,

ce-MeasRSS-DedicatedSameRBs-r16		ENUMERATED {supported}		OPTIONAL

}

MeasGapInfoNR ::= SEQUENCE {

interRAT-BandListNR-EN-DC		InterRAT-BandListNR					OPTIONAL,

interRAT-BandListNR-SA		InterRAT-BandListNR					OPTIONAL

}

BandListEUTRA ::=					SEQUENCE (SIZE (1..maxBands)) OF BandInfoEUTRA

BandCombinationListEUTRA-r10 ::=	SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandInfoEUTRA

BandInfoEUTRA ::=					SEQUENCE {

interFreqBandList					InterFreqBandList,

interRAT-BandList					InterRAT-BandList		OPTIONAL

}

InterFreqBandList ::=				SEQUENCE (SIZE (1..maxBands)) OF InterFreqBandInfo

InterFreqBandInfo ::=				SEQUENCE {

interFreqNeedForGaps				BOOLEAN

}

InterRAT-BandList ::=				SEQUENCE (SIZE (1..maxBands)) OF InterRAT-BandInfo

InterRAT-BandListNR ::=				SEQUENCE (SIZE (1..maxBandsNR-r15)) OF InterRAT-BandInfoNR

InterRAT-BandInfo ::=				SEQUENCE {

interRAT-NeedForGaps				BOOLEAN

}

InterRAT-BandInfoNR ::=			SEQUENCE {

interRAT-NeedForGapsNR				BOOLEAN

}

IRAT-ParametersNR-r15 ::=		SEQUENCE {

en-DC-r15							ENUMERATED {supported}						OPTIONAL,

eventB2-r15						ENUMERATED {supported}						OPTIONAL,

supportedBandListEN-DC-r15		SupportedBandListNR-r15						OPTIONAL

}

IRAT-ParametersNR-v1540 ::=		SEQUENCE {

eutra-5GC-HO-ToNR-FDD-FR1-r15		ENUMERATED {supported}				OPTIONAL,

eutra-5GC-HO-ToNR-TDD-FR1-r15		ENUMERATED {supported}				OPTIONAL,

eutra-5GC-HO-ToNR-FDD-FR2-r15		ENUMERATED {supported}				OPTIONAL,

eutra-5GC-HO-ToNR-TDD-FR2-r15		ENUMERATED {supported}				OPTIONAL,

eutra-EPC-HO-ToNR-FDD-FR1-r15		ENUMERATED {supported}				OPTIONAL,

eutra-EPC-HO-ToNR-TDD-FR1-r15		ENUMERATED {supported}				OPTIONAL,

eutra-EPC-HO-ToNR-FDD-FR2-r15		ENUMERATED {supported}				OPTIONAL,

eutra-EPC-HO-ToNR-TDD-FR2-r15		ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-FR1-r15				ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-FR2-r15				ENUMERATED {supported}				OPTIONAL,

sa-NR-r15								ENUMERATED {supported}				OPTIONAL,

supportedBandListNR-SA-r15			SupportedBandListNR-r15				OPTIONAL

}

IRAT-ParametersNR-v1560 ::=		SEQUENCE {

ng-EN-DC-r15							ENUMERATED {supported}				OPTIONAL

}

IRAT-ParametersNR-v1570 ::=		SEQUENCE {

ss-SINR-Meas-NR-FR1-r15				ENUMERATED {supported}				OPTIONAL,

ss-SINR-Meas-NR-FR2-r15				ENUMERATED {supported}				OPTIONAL

}

IRAT-ParametersNR-v1610 ::=		SEQUENCE {

nr-HO-ToEN-DC-r16					ENUMERATED {supported}				OPTIONAL,

ce-EUTRA-5GC-HO-ToNR-FDD-FR1-r16	ENUMERATED {supported}				OPTIONAL,

ce-EUTRA-5GC-HO-ToNR-TDD-FR1-r16	ENUMERATED {supported}				OPTIONAL,

ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16	ENUMERATED {supported}				OPTIONAL,

ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16	ENUMERATED {supported}				OPTIONAL

}

IRAT-ParametersNR-v1660 ::=		SEQUENCE {

extendedBand-n77-r16				ENUMERATED {supported}				OPTIONAL

}

EUTRA-5GC-Parameters-r15 ::=		SEQUENCE {

eutra-5GC-r15								ENUMERATED {supported}			OPTIONAL,

eutra-EPC-HO-EUTRA-5GC-r15				ENUMERATED {supported}			OPTIONAL,

ho-EUTRA-5GC-FDD-TDD-r15					ENUMERATED {supported}			OPTIONAL,

ho-InterfreqEUTRA-5GC-r15					ENUMERATED {supported}			OPTIONAL,

ims-VoiceOverMCG-BearerEUTRA-5GC-r15	ENUMERATED {supported}			OPTIONAL,

inactiveState-r15							ENUMERATED {supported}			OPTIONAL,

reflectiveQoS-r15							ENUMERATED {supported}			OPTIONAL

}

EUTRA-5GC-Parameters-v1610 ::=	SEQUENCE {

ce-InactiveState-r16			ENUMERATED {supported}			OPTIONAL,

ce-EUTRA-5GC-r16				ENUMERATED {supported}			OPTIONAL

}

PDCP-ParametersNR-r15 ::=		SEQUENCE {

rohc-Profiles-r15					ROHC-ProfileSupportList-r15,

rohc-ContextMaxSessions-r15			ENUMERATED {

cs2, cs4, cs8, cs12, cs16, cs24, cs32,

cs48, cs64, cs128, cs256, cs512, cs1024,

cs16384, spare2, spare1}			DEFAULT cs16,

rohc-ProfilesUL-Only-r15				SEQUENCE {

profile0x0006-r15						BOOLEAN

},

rohc-ContextContinue-r15			ENUMERATED {supported}				OPTIONAL,

outOfOrderDelivery-r15				ENUMERATED {supported}				OPTIONAL,

sn-SizeLo-r15						ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-PDCP-MCG-Bearer-r15	ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-PDCP-SCG-Bearer-r15	ENUMERATED {supported}				OPTIONAL

}

PDCP-ParametersNR-v1560 ::=		SEQUENCE {

ims-VoNR-PDCP-SCG-NGENDC-r15			ENUMERATED {supported}				OPTIONAL

}

ROHC-ProfileSupportList-r15 ::=	SEQUENCE {

profile0x0001-r15					BOOLEAN,

profile0x0002-r15					BOOLEAN,

profile0x0003-r15					BOOLEAN,

profile0x0004-r15					BOOLEAN,

profile0x0006-r15					BOOLEAN,

profile0x0101-r15					BOOLEAN,

profile0x0102-r15					BOOLEAN,

profile0x0103-r15					BOOLEAN,

profile0x0104-r15					BOOLEAN

}

SupportedBandListNR-r15 ::=		SEQUENCE (SIZE (1..maxBandsNR-r15)) OF SupportedBandNR-r15

SupportedBandNR-r15 ::=			SEQUENCE {

bandNR-r15							FreqBandIndicatorNR-r15

}

IRAT-ParametersUTRA-FDD ::=		SEQUENCE {

supportedBandListUTRA-FDD			SupportedBandListUTRA-FDD

}

IRAT-ParametersUTRA-v920 ::=		SEQUENCE {

e-RedirectionUTRA-r9				ENUMERATED {supported}

}

IRAT-ParametersUTRA-v9c0 ::=		SEQUENCE {

voiceOverPS-HS-UTRA-FDD-r9						ENUMERATED {supported}		OPTIONAL,

voiceOverPS-HS-UTRA-TDD128-r9					ENUMERATED {supported}		OPTIONAL,

srvcc-FromUTRA-FDD-ToUTRA-FDD-r9				ENUMERATED {supported}		OPTIONAL,

srvcc-FromUTRA-FDD-ToGERAN-r9					ENUMERATED {supported}		OPTIONAL,

srvcc-FromUTRA-TDD128-ToUTRA-TDD128-r9			ENUMERATED {supported}		OPTIONAL,

srvcc-FromUTRA-TDD128-ToGERAN-r9				ENUMERATED {supported}		OPTIONAL

}

IRAT-ParametersUTRA-v9h0 ::=		SEQUENCE {

mfbi-UTRA-r9						ENUMERATED {supported}

}

SupportedBandListUTRA-FDD ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandUTRA-FDD

SupportedBandUTRA-FDD ::=			ENUMERATED {

bandI, bandII, bandIII, bandIV, bandV, bandVI,

bandVII, bandVIII, bandIX, bandX, bandXI,

bandXII, bandXIII, bandXIV, bandXV, bandXVI, ...,

bandXVII-8a0, bandXVIII-8a0, bandXIX-8a0, bandXX-8a0,

bandXXI-8a0, bandXXII-8a0, bandXXIII-8a0, bandXXIV-8a0,

bandXXV-8a0, bandXXVI-8a0, bandXXVII-8a0, bandXXVIII-8a0,

bandXXIX-8a0, bandXXX-8a0, bandXXXI-8a0, bandXXXII-8a0}

IRAT-ParametersUTRA-TDD128 ::=		SEQUENCE {

supportedBandListUTRA-TDD128		SupportedBandListUTRA-TDD128

}

SupportedBandListUTRA-TDD128 ::=	SEQUENCE (SIZE (1..maxBands)) OF SupportedBandUTRA-TDD128

SupportedBandUTRA-TDD128 ::=		ENUMERATED {

a, b, c, d, e, f, g, h, i, j, k, l, m, n,

o, p, ...}

IRAT-ParametersUTRA-TDD384 ::=		SEQUENCE {

supportedBandListUTRA-TDD384		SupportedBandListUTRA-TDD384

}

SupportedBandListUTRA-TDD384 ::=	SEQUENCE (SIZE (1..maxBands)) OF SupportedBandUTRA-TDD384

SupportedBandUTRA-TDD384 ::=		ENUMERATED {

a, b, c, d, e, f, g, h, i, j, k, l, m, n,

o, p, ...}

IRAT-ParametersUTRA-TDD768 ::=		SEQUENCE {

supportedBandListUTRA-TDD768		SupportedBandListUTRA-TDD768

}

SupportedBandListUTRA-TDD768 ::=	SEQUENCE (SIZE (1..maxBands)) OF SupportedBandUTRA-TDD768

SupportedBandUTRA-TDD768 ::=		ENUMERATED {

a, b, c, d, e, f, g, h, i, j, k, l, m, n,

o, p, ...}

IRAT-ParametersUTRA-TDD-v1020 ::=		SEQUENCE {

e-RedirectionUTRA-TDD-r10				ENUMERATED {supported}

}

IRAT-ParametersGERAN ::=			SEQUENCE {

supportedBandListGERAN				SupportedBandListGERAN,

interRAT-PS-HO-ToGERAN				BOOLEAN

}

IRAT-ParametersGERAN-v920 ::=		SEQUENCE {

dtm-r9								ENUMERATED {supported}			OPTIONAL,

e-RedirectionGERAN-r9				ENUMERATED {supported}			OPTIONAL

}

SupportedBandListGERAN ::=			SEQUENCE (SIZE (1..maxBands)) OF SupportedBandGERAN

SupportedBandGERAN ::=				ENUMERATED {

gsm450, gsm480, gsm710, gsm750, gsm810, gsm850,

gsm900P, gsm900E, gsm900R, gsm1800, gsm1900,

spare5, spare4, spare3, spare2, spare1, ...}

IRAT-ParametersCDMA2000-HRPD ::=	SEQUENCE {

supportedBandListHRPD				SupportedBandListHRPD,

tx-ConfigHRPD						ENUMERATED {single, dual},

rx-ConfigHRPD						ENUMERATED {single, dual}

}

SupportedBandListHRPD ::=			SEQUENCE (SIZE (1..maxCDMA-BandClass)) OF BandclassCDMA2000

IRAT-ParametersCDMA2000-1XRTT ::=	SEQUENCE {

supportedBandList1XRTT				SupportedBandList1XRTT,

tx-Config1XRTT						ENUMERATED {single, dual},

rx-Config1XRTT						ENUMERATED {single, dual}

}

IRAT-ParametersCDMA2000-1XRTT-v920 ::=	SEQUENCE {

e-CSFB-1XRTT-r9						ENUMERATED {supported},

e-CSFB-ConcPS-Mob1XRTT-r9			ENUMERATED {supported}			OPTIONAL

}

IRAT-ParametersCDMA2000-1XRTT-v1020 ::=	SEQUENCE {

e-CSFB-dual-1XRTT-r10				ENUMERATED {supported}

}

IRAT-ParametersCDMA2000-v1130 ::=		SEQUENCE {

cdma2000-NW-Sharing-r11					ENUMERATED {supported}		OPTIONAL

}

SupportedBandList1XRTT ::=			SEQUENCE (SIZE (1..maxCDMA-BandClass)) OF BandclassCDMA2000

IRAT-ParametersWLAN-r13 ::=		SEQUENCE {

supportedBandListWLAN-r13		SEQUENCE (SIZE (1..maxWLAN-Bands-r13)) OF WLAN-BandIndicator-r13					OPTIONAL

}

CSG-ProximityIndicationParameters-r9 ::=	SEQUENCE {

intraFreqProximityIndication-r9		ENUMERATED {supported}			OPTIONAL,

interFreqProximityIndication-r9		ENUMERATED {supported}			OPTIONAL,

utran-ProximityIndication-r9		ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-r9 ::=	SEQUENCE {

intraFreqSI-AcquisitionForHO-r9		ENUMERATED {supported}			OPTIONAL,

interFreqSI-AcquisitionForHO-r9		ENUMERATED {supported}			OPTIONAL,

utran-SI-AcquisitionForHO-r9		ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-v1530 ::=	SEQUENCE {

reportCGI-NR-EN-DC-r15					ENUMERATED {supported}			OPTIONAL,

reportCGI-NR-NoEN-DC-r15				ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-v1550 ::=	SEQUENCE {

eutra-CGI-Reporting-ENDC-r15				ENUMERATED {supported}			OPTIONAL,

utra-GERAN-CGI-Reporting-ENDC-r15			ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-v15a0 ::=	SEQUENCE {

eutra-CGI-Reporting-NEDC-r15				ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-v1610 ::=	SEQUENCE {

eutra-SI-AcquisitionForHO-ENDC-r16			ENUMERATED {supported}			OPTIONAL,

nr-AutonomousGaps-ENDC-FR1-r16				ENUMERATED {supported}			OPTIONAL,

nr-AutonomousGaps-ENDC-FR2-r16				ENUMERATED {supported}			OPTIONAL,

nr-AutonomousGaps-FR1-r16					ENUMERATED {supported}			OPTIONAL,

nr-AutonomousGaps-FR2-r16					ENUMERATED {supported}			OPTIONAL

}

SON-Parameters-r9 ::=				SEQUENCE {

rach-Report-r9						ENUMERATED {supported}			OPTIONAL

}

PUR-Parameters-r16 ::=				SEQUENCE {

pur-CP-5GC-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

pur-CP-5GC-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

pur-UP-5GC-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

pur-UP-5GC-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

pur-CP-EPC-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

pur-CP-EPC-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

pur-UP-EPC-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

pur-UP-EPC-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

pur-CP-L1Ack-r16					ENUMERATED {supported}			OPTIONAL,

pur-FrequencyHopping-r16			ENUMERATED {supported}			OPTIONAL,

pur-PUSCH-NB-MaxTBS-r16				ENUMERATED {supported}			OPTIONAL,

pur-RSRP-Validation-r16				ENUMERATED {supported}			OPTIONAL,

pur-SubPRB-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

pur-SubPRB-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL

}

UE-BasedNetwPerfMeasParameters-r10 ::=	SEQUENCE {

loggedMeasurementsIdle-r10				ENUMERATED {supported}		OPTIONAL,

standaloneGNSS-Location-r10				ENUMERATED {supported}		OPTIONAL

}

UE-BasedNetwPerfMeasParameters-v1250 ::=	SEQUENCE {

loggedMBSFNMeasurements-r12				ENUMERATED {supported}

}

UE-BasedNetwPerfMeasParameters-v1430 ::=	SEQUENCE {

locationReport-r14						ENUMERATED {supported}		OPTIONAL

}

UE-BasedNetwPerfMeasParameters-v1530 ::=	SEQUENCE {

loggedMeasBT-r15						ENUMERATED {supported}		OPTIONAL,

loggedMeasWLAN-r15						ENUMERATED {supported}		OPTIONAL,

immMeasBT-r15							ENUMERATED {supported}		OPTIONAL,

immMeasWLAN-r15							ENUMERATED {supported}		OPTIONAL

}

UE-BasedNetwPerfMeasParameters-v1610 ::=	SEQUENCE {

ul-PDCP-AvgDelay-r16						ENUMERATED {supported}		OPTIONAL

}

OTDOA-PositioningCapabilities-r10 ::=	SEQUENCE {

otdoa-UE-Assisted-r10					ENUMERATED {supported},

interFreqRSTD-Measurement-r10			ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-r11 ::=				SEQUENCE {

inDeviceCoexInd-r11						ENUMERATED {supported}		OPTIONAL,

powerPrefInd-r11						ENUMERATED {supported}		OPTIONAL,

ue-Rx-TxTimeDiffMeasurements-r11		ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v11d0 ::=				SEQUENCE {

inDeviceCoexInd-UL-CA-r11				ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1360 ::=	SEQUENCE {

inDeviceCoexInd-HardwareSharingInd-r13		ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1430 ::=			SEQUENCE {

bwPrefInd-r14					ENUMERATED {supported}		OPTIONAL,

rlm-ReportSupport-r14			ENUMERATED {supported}		OPTIONAL

}

OtherParameters-v1450 ::=	SEQUENCE {

overheatingInd-r14				ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1460 ::=	SEQUENCE {

nonCSG-SI-Reporting-r14			ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1530 ::=			SEQUENCE {

assistInfoBitForLC-r15			ENUMERATED {supported}		OPTIONAL,

timeReferenceProvision-r15		ENUMERATED {supported}		OPTIONAL,

flightPathPlan-r15				ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1540 ::=			SEQUENCE {

inDeviceCoexInd-ENDC-r15		ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1610 ::=		SEQUENCE {

resumeWithStoredMCG-SCells-r16	ENUMERATED {supported}		OPTIONAL,

resumeWithMCG-SCellConfig-r16	ENUMERATED {supported}		OPTIONAL,

resumeWithStoredSCG-r16			ENUMERATED {supported}		OPTIONAL,

resumeWithSCG-Config-r16		ENUMERATED {supported}		OPTIONAL,

mcgRLF-RecoveryViaSCG-r16		ENUMERATED {supported}		OPTIONAL,

overheatingIndForSCG-r16		ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1650 ::=		SEQUENCE {

mpsPriorityIndication-r16			ENUMERATED {supported}		OPTIONAL

}

MBMS-Parameters-r11 ::=				SEQUENCE {

mbms-SCell-r11							ENUMERATED {supported}		OPTIONAL,

mbms-NonServingCell-r11					ENUMERATED {supported}		OPTIONAL

}

MBMS-Parameters-v1250 ::=				SEQUENCE {

mbms-AsyncDC-r12						ENUMERATED {supported}		OPTIONAL

}

MBMS-Parameters-v1430 ::=				SEQUENCE {

fembmsDedicatedCell-r14				ENUMERATED {supported}		OPTIONAL,

fembmsMixedCell-r14					ENUMERATED {supported}		OPTIONAL,

subcarrierSpacingMBMS-khz7dot5-r14	ENUMERATED {supported}		OPTIONAL,

subcarrierSpacingMBMS-khz1dot25-r14	ENUMERATED {supported}		OPTIONAL

}

MBMS-Parameters-v1470 ::=		SEQUENCE {

mbms-MaxBW-r14					CHOICE {

implicitValue					NULL,

explicitValue					INTEGER(2..20)

},

mbms-ScalingFactor1dot25-r14		ENUMERATED {n3, n6, n9, n12}	OPTIONAL,

mbms-ScalingFactor7dot5-r14		ENUMERATED {n1, n2, n3, n4}		OPTIONAL

}

MBMS-Parameters-v1610 ::=		SEQUENCE {

mbms-ScalingFactor2dot5-r16		ENUMERATED {n2, n4, n6, n8}			OPTIONAL,

mbms-ScalingFactor0dot37-r16	ENUMERATED {n12, n16, n20, n24}		OPTIONAL,

mbms-SupportedBandInfoList-r16	SEQUENCE (SIZE (1..maxBands)) OF MBMS-SupportedBandInfo-r16

}

MBMS-SupportedBandInfo-r16 ::=		SEQUENCE {

subcarrierSpacingMBMS-khz2dot5-r16	ENUMERATED {supported}		OPTIONAL,

subcarrierSpacingMBMS-khz0dot37-r16	SEQUENCE {

timeSeparationSlot2-r16			ENUMERATED {supported}			OPTIONAL,

timeSeparationSlot4-r16			ENUMERATED {supported}			OPTIONAL

}	OPTIONAL

}

FeMBMS-Unicast-Parameters-r14 ::=		SEQUENCE {

unicast-fembmsMixedSCell-r14			ENUMERATED {supported}		OPTIONAL,

emptyUnicastRegion-r14					ENUMERATED {supported}		OPTIONAL

}

SCPTM-Parameters-r13 ::=				SEQUENCE {

scptm-ParallelReception-r13					ENUMERATED {supported}		OPTIONAL,

scptm-SCell-r13								ENUMERATED {supported}		OPTIONAL,

scptm-NonServingCell-r13					ENUMERATED {supported}		OPTIONAL,

scptm-AsyncDC-r13							ENUMERATED {supported}		OPTIONAL

}

CE-Parameters-r13 ::=		SEQUENCE {

ce-ModeA-r13						ENUMERATED {supported}				OPTIONAL,

ce-ModeB-r13						ENUMERATED {supported}				OPTIONAL

}

CE-Parameters-v1320 ::=		SEQUENCE {

intraFreqA3-CE-ModeA-r13				ENUMERATED {supported}				OPTIONAL,

intraFreqA3-CE-ModeB-r13				ENUMERATED {supported}				OPTIONAL,

intraFreqHO-CE-ModeA-r13				ENUMERATED {supported}				OPTIONAL,

intraFreqHO-CE-ModeB-r13				ENUMERATED {supported}				OPTIONAL

}

CE-Parameters-v1350 ::=		SEQUENCE {

unicastFrequencyHopping-r13				ENUMERATED {supported}				OPTIONAL

}

CE-Parameters-v1370 ::=		SEQUENCE {

tm9-CE-ModeA-r13						ENUMERATED {supported}			OPTIONAL,

tm9-CE-ModeB-r13						ENUMERATED {supported}			OPTIONAL

}

CE-Parameters-v1380 ::=		SEQUENCE {

tm6-CE-ModeA-r13						ENUMERATED {supported}			OPTIONAL

}

CE-Parameters-v1430 ::=		SEQUENCE {

ce-SwitchWithoutHO-r14					ENUMERATED {supported}				OPTIONAL

}

CE-MultiTB-Parameters-r16 ::=	SEQUENCE {

pdsch-MultiTB-CE-ModeA-r16			ENUMERATED {supported}			OPTIONAL,

pdsch-MultiTB-CE-ModeB-r16			ENUMERATED {supported}			OPTIONAL,

pusch-MultiTB-CE-ModeA-r16			ENUMERATED {supported}			OPTIONAL,

pusch-MultiTB-CE-ModeB-r16			ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-64QAM-r16 				ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-EarlyTermination-r16 	ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-FrequencyHopping-r16		ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-HARQ-AckBundling-r16		ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-Interleaving-r16			ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-SubPRB-r16 				ENUMERATED {supported}			OPTIONAL

}

CE-ResourceResvParameters-r16 ::=	SEQUENCE {

subframeResourceResvDL-CE-ModeA-r16 	ENUMERATED {supported}			OPTIONAL,

subframeResourceResvDL-CE-ModeB-r16 	ENUMERATED {supported}			OPTIONAL,

subframeResourceResvUL-CE-ModeA-r16 	ENUMERATED {supported}			OPTIONAL,

subframeResourceResvUL-CE-ModeB-r16 	ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvDL-CE-ModeA-r16 	ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvDL-CE-ModeB-r16 	ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvUL-CE-ModeA-r16 	ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvUL-CE-ModeB-r16 	ENUMERATED {supported}			OPTIONAL,

subcarrierPuncturingCE-ModeA-r16 		ENUMERATED {supported}			OPTIONAL,

subcarrierPuncturingCE-ModeB-r16 		ENUMERATED {supported}			OPTIONAL

}

LAA-Parameters-r13 ::=				SEQUENCE {

crossCarrierSchedulingLAA-DL-r13			ENUMERATED {supported}		OPTIONAL,

csi-RS-DRS-RRM-MeasurementsLAA-r13			ENUMERATED {supported}		OPTIONAL,

downlinkLAA-r13								ENUMERATED {supported}		OPTIONAL,

endingDwPTS-r13								ENUMERATED {supported}		OPTIONAL,

secondSlotStartingPosition-r13				ENUMERATED {supported}		OPTIONAL,

tm9-LAA-r13									ENUMERATED {supported}		OPTIONAL,

tm10-LAA-r13								ENUMERATED {supported}		OPTIONAL

}

LAA-Parameters-v1430 ::=				SEQUENCE {

crossCarrierSchedulingLAA-UL-r14			ENUMERATED {supported}		OPTIONAL,

uplinkLAA-r14								ENUMERATED {supported}		OPTIONAL,

twoStepSchedulingTimingInfo-r14				ENUMERATED {nPlus1, nPlus2, nPlus3}	OPTIONAL,

uss-BlindDecodingAdjustment-r14				ENUMERATED {supported}		OPTIONAL,

uss-BlindDecodingReduction-r14				ENUMERATED {supported}		OPTIONAL,

outOfSequenceGrantHandling-r14				ENUMERATED {supported}		OPTIONAL

}

LAA-Parameters-v1530 ::=				SEQUENCE {

aul-r15										ENUMERATED {supported}		OPTIONAL,

laa-PUSCH-Mode1-r15							ENUMERATED {supported}		OPTIONAL,

laa-PUSCH-Mode2-r15							ENUMERATED {supported}		OPTIONAL,

laa-PUSCH-Mode3-r15							ENUMERATED {supported}		OPTIONAL

}

WLAN-IW-Parameters-r12 ::=	SEQUENCE {

wlan-IW-RAN-Rules-r12					ENUMERATED {supported}		OPTIONAL,

wlan-IW-ANDSF-Policies-r12						ENUMERATED {supported}		OPTIONAL

}

LWA-Parameters-r13 ::=		SEQUENCE {

lwa-r13						ENUMERATED {supported}		OPTIONAL,

lwa-SplitBearer-r13			ENUMERATED {supported}		OPTIONAL,

wlan-MAC-Address-r13		OCTET STRING (SIZE (6))		OPTIONAL,

lwa-BufferSize-r13			ENUMERATED {supported}		OPTIONAL

}

LWA-Parameters-v1430 ::=		SEQUENCE {

lwa-HO-WithoutWT-Change-r14			ENUMERATED {supported}		OPTIONAL,

lwa-UL-r14							ENUMERATED {supported}		OPTIONAL,

wlan-PeriodicMeas-r14				ENUMERATED {supported}		OPTIONAL,

wlan-ReportAnyWLAN-r14				ENUMERATED {supported}		OPTIONAL,

wlan-SupportedDataRate-r14			INTEGER (1..2048)			OPTIONAL

}

LWA-Parameters-v1440 ::=		SEQUENCE {

lwa-RLC-UM-r14						ENUMERATED {supported}		OPTIONAL

}

WLAN-IW-Parameters-v1310 ::=	SEQUENCE {

rclwi-r13										ENUMERATED {supported}		OPTIONAL

}

LWIP-Parameters-r13 ::=		SEQUENCE {

lwip-r13					ENUMERATED {supported}				OPTIONAL

}

LWIP-Parameters-v1430 ::=		SEQUENCE {

lwip-Aggregation-DL-r14					ENUMERATED {supported}				OPTIONAL,

lwip-Aggregation-UL-r14					ENUMERATED {supported}				OPTIONAL

}

NAICS-Capability-List-r12 ::= SEQUENCE (SIZE (1..maxNAICS-Entries-r12)) OF NAICS-Capability-Entry-r12

NAICS-Capability-Entry-r12	::=	SEQUENCE {

numberOfNAICS-CapableCC-r12				INTEGER(1..5),

numberOfAggregatedPRB-r12				ENUMERATED {

n50, n75, n100, n125, n150, n175,

n200, n225, n250, n275, n300, n350,

n400, n450, n500, spare},

...

}

SL-Parameters-r12 ::=				SEQUENCE {

commSimultaneousTx-r12					ENUMERATED {supported}		OPTIONAL,

commSupportedBands-r12					FreqBandIndicatorListEUTRA-r12	OPTIONAL,

discSupportedBands-r12					SupportedBandInfoList-r12	OPTIONAL,

discScheduledResourceAlloc-r12			ENUMERATED {supported}		OPTIONAL,

disc-UE-SelectedResourceAlloc-r12		ENUMERATED {supported}		OPTIONAL,

disc-SLSS-r12							ENUMERATED {supported}		OPTIONAL,

discSupportedProc-r12					ENUMERATED {n50, n400}		OPTIONAL

}

SL-Parameters-v1310 ::=				SEQUENCE {

discSysInfoReporting-r13					ENUMERATED {supported}		OPTIONAL,

commMultipleTx-r13							ENUMERATED {supported}		OPTIONAL,

discInterFreqTx-r13							ENUMERATED {supported}		OPTIONAL,

discPeriodicSLSS-r13						ENUMERATED {supported}		OPTIONAL

}

SL-Parameters-v1430 ::=				SEQUENCE {

zoneBasedPoolSelection-r14				ENUMERATED {supported}				OPTIONAL,

ue-AutonomousWithFullSensing-r14		ENUMERATED {supported}				OPTIONAL,

ue-AutonomousWithPartialSensing-r14		ENUMERATED {supported}				OPTIONAL,

sl-CongestionControl-r14				ENUMERATED {supported}				OPTIONAL,

v2x-TxWithShortResvInterval-r14			ENUMERATED {supported}				OPTIONAL,

v2x-numberTxRxTiming-r14				INTEGER(1..16)						OPTIONAL,

v2x-nonAdjacentPSCCH-PSSCH-r14			ENUMERATED {supported}				OPTIONAL,

slss-TxRx-r14							ENUMERATED {supported}				OPTIONAL,

v2x-SupportedBandCombinationList-r14	V2X-SupportedBandCombination-r14	OPTIONAL

}

SL-Parameters-v1530 ::=				SEQUENCE {

slss-SupportedTxFreq-r15				ENUMERATED {single, multiple}		OPTIONAL,

sl-64QAM-Tx-r15						ENUMERATED {supported}				OPTIONAL,

sl-TxDiversity-r15						ENUMERATED {supported}				OPTIONAL,

ue-CategorySL-r15						UE-CategorySL-r15					OPTIONAL,

v2x-SupportedBandCombinationList-v1530	V2X-SupportedBandCombination-v1530	OPTIONAL

}

SL-Parameters-v1540 ::=				SEQUENCE {

sl-64QAM-Rx-r15						ENUMERATED {supported}				OPTIONAL,

sl-RateMatchingTBSScaling-r15			ENUMERATED {supported}				OPTIONAL,

sl-LowT2min-r15							ENUMERATED {supported}				OPTIONAL,

v2x-SensingReportingMode3-r15			ENUMERATED {supported}				OPTIONAL

}

SL-Parameters-v1610 ::=		SEQUENCE {

sl-ParameterNR-r16			OCTET STRING								OPTIONAL,

dummy						V2X-SupportedBandCombinationEUTRA-NR-r16	OPTIONAL

}

SL-Parameters-v1630 ::=					SEQUENCE {

v2x-SupportedBandCombinationListEUTRA-NR-r16	V2X-SupportedBandCombinationEUTRA-NR-v1630	OPTIONAL

}

UE-CategorySL-r15 ::=			SEQUENCE {

ue-CategorySL-C-TX-r15				INTEGER(1..5),

ue-CategorySL-C-RX-r15				INTEGER(1..4)

}

V2X-SupportedBandCombination-r14 ::=		SEQUENCE (SIZE (1..maxBandComb-r13)) OF V2X-BandCombinationParameters-r14

V2X-SupportedBandCombination-v1530	::=		SEQUENCE (SIZE (1..maxBandComb-r13)) OF V2X-BandCombinationParameters-v1530

V2X-BandCombinationParameters-r14 ::=	SEQUENCE (SIZE (1.. maxSimultaneousBands-r10)) OF V2X-BandParameters-r14

V2X-BandCombinationParameters-v1530 ::=	SEQUENCE (SIZE (1.. maxSimultaneousBands-r10)) OF V2X-BandParameters-v1530

V2X-SupportedBandCombinationEUTRA-NR-r16	::=	SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF V2X-BandParametersEUTRA-NR-r16

V2X-SupportedBandCombinationEUTRA-NR-v1630	::=	SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF V2X-BandCombinationParametersEUTRA-NR-v1630

V2X-BandCombinationParametersEUTRA-NR-v1630 ::=	SEQUENCE {

bandListSidelinkEUTRA-NR-r16					SEQUENCE (SIZE (1.. maxSimultaneousBands-r10)) OF V2X-BandParametersEUTRA-NR-r16,

bandListSidelinkEUTRA-NR-v1630					SEQUENCE (SIZE (1.. maxSimultaneousBands-r10)) OF V2X-BandParametersEUTRA-NR-v1630

}

V2X-BandParametersEUTRA-NR-r16 ::=	CHOICE {

eutra									SEQUENCE {

v2x-BandParameters1-r16				V2X-BandParameters-r14		OPTIONAL,

v2x-BandParameters2-r16				V2X-BandParameters-v1530		OPTIONAL

},

nr										SEQUENCE {

v2x-BandParametersNR-r16					OCTET STRING				OPTIONAL

}

}

V2X-BandParametersEUTRA-NR-v1630 ::=	CHOICE {

eutra									NULL,

nr										SEQUENCE {

tx-Sidelink-r16							ENUMERATED {supported}	OPTIONAL,

rx-Sidelink-r16							ENUMERATED {supported}	OPTIONAL

}

}

SupportedBandInfoList-r12 ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandInfo-r12

SupportedBandInfo-r12 ::=			SEQUENCE {

support-r12								ENUMERATED {supported}	OPTIONAL

}

FreqBandIndicatorListEUTRA-r12 ::=		SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicator-r11

MMTEL-Parameters-r14 ::=			SEQUENCE {

delayBudgetReporting-r14					ENUMERATED {supported}		OPTIONAL,

pusch-Enhancements-r14						ENUMERATED {supported}		OPTIONAL,

recommendedBitRate-r14						ENUMERATED {supported}		OPTIONAL,

recommendedBitRateQuery-r14					ENUMERATED {supported}		OPTIONAL

}

MMTEL-Parameters-v1610 ::=				SEQUENCE {

recommendedBitRateMultiplier-r16			ENUMERATED {supported}			OPTIONAL

}

SRS-CapabilityPerBandPair-r14 ::= SEQUENCE {

retuningInfo				SEQUENCE {

rf-RetuningTimeDL-r14			ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3,

n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5,

n7, spare1}		OPTIONAL,

rf-RetuningTimeUL-r14			ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3,

n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5,

n7, spare1}		OPTIONAL

}

}

SRS-CapabilityPerBandPair-v14b0 ::= SEQUENCE {

srs-FlexibleTiming-r14				ENUMERATED {supported}		OPTIONAL,

srs-HARQ-ReferenceConfig-r14			ENUMERATED {supported}		OPTIONAL

}

SRS-CapabilityPerBandPair-v1610::= SEQUENCE {

addSRS-CarrierSwitching-r16				ENUMERATED {supported}		OPTIONAL

}

HighSpeedEnhParameters-r14 ::= SEQUENCE {

measurementEnhancements-r14		ENUMERATED {supported}		OPTIONAL,

demodulationEnhancements-r14	ENUMERATED {supported}		OPTIONAL,

prach-Enhancements-r14			ENUMERATED {supported}		OPTIONAL

}

HighSpeedEnhParameters-v1610 ::= SEQUENCE {

measurementEnhancementsSCell-r16	ENUMERATED {supported}		OPTIONAL,

measurementEnhancements2-r16		ENUMERATED {supported}		OPTIONAL,

demodulationEnhancements2-r16	ENUMERATED {supported}		OPTIONAL,

interRAT-enhancementNR-r16		ENUMERATED {supported}		OPTIONAL

}

-- ASN1STOP

Editor’s Note: Agreement: Rel-16 LTE CHO mechanism is supported for LTE-M devices in IoT NTN. FFS which CE Mode(s) to apply

| UE-EUTRA-Capability field descriptions |  | FDD/ TDD diff |  |
| --- | --- | --- | --- |
| accessStratumRelease Set to rel16 in this version of the specification. NOTE 7. |  | - |  |
| additionalRx-Tx-PerformanceReq Indicates whether the UE supports the additional Rx and Tx performance requirement for a given band combination as specified in TS 36.101 [42]. |  | - |  |
| addSRS Presence of this field indicates the UE supports the additional SRS symbol(s) within the normal UL subframes in TDD as described in TS 36.213 [23]. |  | - |  |
| addSRS-1T2R Indicates whether the UE supports selecting one antenna among two antennas to transmit additional SRS symbol(s) for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |  |
| addSRS-1T4R Indicates whether the UE supports selecting one antenna among four antennas to transmit additional SRS symbol(s) for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |  |
| addSRS-2T4R-2Pairs Indicates whether the UE supports selecting one antenna pair between two antenna pairs to transmit additional SRS symbol(s) simultaneously for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |  |
| addSRS-2T4R-3Pairs Indicates whether the UE supports selecting one antenna pair among three antenna pairs to transmit additional SRS symbol(s) simultaneously for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |  |
| addSRS-AntennaSwitching (in addSRS) Value useBasic indicates the antenna switching capabilities for additional SRS symbol(s) for a band of band combination for which the capability is not signalled in bandParameterList-v1610 is the same as indicated by bandParameterList-v1380 and/or bandParameterList-v1530 for the concerned band of band combination. |  | - |  |
| addSRS-AntennaSwitching (in bandParameterList-v1610) If signalled, the field indicates the antenna switching capabilities for additional SRS symbol(s) for the concerned band of band combination. |  | - |  |
| addSRS-CarrierSwitching (in addSRS) Indicates whether carrier switching is supported for additional SRS symbol(s) for all band pairs of band combinations for which UE supports SRS carrier switching. This field is included only if srs-CapabilityPerBandPairList-r14 is included. If this field is included, addSRS-CarrierSwitching (in bandParameterList-v1610) is not included. |  | - |  |
| addSRS-CarrierSwitching (in bandParameterList-v1610) Indicates whether carrier switching is supported for additional SRS symbol(s) for the concerned band pair of band combination. This field is included only if srs-CapabilityPerBandPairList-r14 is included.If this field is included, addSRS-CarrierSwitching (in addSRS) is not included. |  | - |  |
| addSRS-FrequencyHopping (in addSRS) Indicates whether frequency hopping is supported for additional SRS symbol(s) for all bands of band combinations for which the capability is not signalled in bandParameterList-v1610. |  | - |  |
| addSRS-FrequencyHopping (in bandParameterList-v1610) If signalled, the field indicates whether frequency hopping is supported for additional SRS symbol(s) for the concerned band of band combination. |  | - |  |
| alternativeTBS-Indices Indicates whether the UE supports alternative TBS indices ITBS 26A and 33A as specified in TS 36.213 [23]. |  | - |  |
| alternativeTBS-Index Indicates whether the UE supports alternative TBS index ITBS 33B as specified in TS 36.213 [23]. |  | No |  |
| alternativeTimeToTrigger Indicates whether the UE supports alternativeTimeToTrigger. |  | No |  |
| altFreqPriority Indicates whether the UE supports alternative cell reselection priority. |  | No |  |
| altMCS-Table Indicates whether the UE supports the 6-bit MCS table as specified in TS 36.212 [22] and TS 36.213 [23]. |  | Yes |  |
| aperiodicCSI-Reporting Indicates whether the UE supports aperiodic CSI reporting with 3 bits of the CSI request field size as specified in TS 36.213 [23], clause 7.2.1 and/or aperiodic CSI reporting mode 1-0 and mode 1-1 as specified in TS 36.213 [23], clause 7.2.1. The first bit is set to "1" if the UE supports the aperiodic CSI reporting with 3 bits of the CSI request field size. The second bit is set to "1" if the UE supports the aperiodic CSI reporting mode 1-0 and mode 1-1. |  | No |  |
| aperiodicCsi-ReportingSTTI Indicates whether the UE supports aperiodic CSI reporting for short TTI as specified in TS 36.213 [23], clause 7.2.1. |  | Yes |  |
| appliedCapabilityFilterCommon Contains the filter, applied by the UE, common for all MR-DC related capability containers that are requested and as defined by UE-CapabilityRequestFilterCommon IE in TS 38.331 [82]. |  | - |  |
| assistInfoBitForLC Indicates whether the UE supports assistance information bit for local cache. |  | - |  |
| aul Indicates whether the UE supports AUL as specified n TS 36.321 [6]. |  | - |  |
| bandCombinationListEUTRA One entry corresponding to each supported band combination listed in the same order as in supportedBandCombination. |  | - |  |
| BandCombinationParameters-v1090, BandCombinationParameters-v10i0, BandCombinationParameters-v1270 If included, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationParameters-r10. |  | - |  |
| BandCombinationParameters-v1130 The field is applicable to each supported CA bandwidth class combination (i.e. CA configuration in TS 36.101 [42], clause 5.6A.1) indicated in the corresponding band combination. If included, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationParameters-r10. |  | - |  |
| bandEUTRA E-UTRA band as defined in TS 36.101 [42]. In case the UE includes bandEUTRA-v9e0 or bandEUTRA-v1090, the UE shall set the corresponding entry of bandEUTRA (i.e. without suffix) or bandEUTRA-r10 respectively to maxFBI. |  | - |  |
| bandInfoNR-v1610 One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. If absent, network assumes gap is required when measurement is performed on any NR bands while UE is served by cell(s) belongs to a E-UTRA band listed in supportedBandListEUTRA except for the FR2 inter-RAT measurement which depends on the support of independentGapConfig. |  | - |  |
| bandListEUTRA One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. |  | - |  |
| bandParameterList-v1380 If included, the UE shall include the same number of entries listed in the same order as the band entries in the corresponding band combination. |  | - |  |
| bandParametersUL, bandParametersDL Indicates the supported parameters for the band. Each of CA-MIMO-ParametersUL and CA-MIMO-ParametersDL can be included only once for one band in a single band combination entry. |  | - |  |
| beamformed (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode, the UE capabilities concerning beamformed EBF/ FD-MIMO operation (class B) applicable for the concerned band combination. |  | - |  |
| beamformed (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode, the UE capabilities concerning beamformed EBF/ FD-MIMO operation (class B) applicable for band combinations for which the concerned capabilities are not signalled. |  | Yes |  |
| benefitsFromInterruption Indicates whether the UE power consumption would benefit from being allowed to cause interruptions to serving cells when performing measurements of deactivated SCell carriers for measCycleSCell of less than 640ms, as specified in TS 36.133 [16]. |  | No |  |
| bwPrefInd Indicates whether the UE supports maximum PDSCH/PUSCH bandwidth preference indication. |  | - |  |
| ca-BandwidthClass The CA bandwidth class supported by the UE as defined in TS 36.101 [42], Table 5.6A-1. The UE explicitly includes all the supported CA bandwidth class combinations in the band combination signalling. Support for one CA bandwidth class does not implicitly indicate support for another CA bandwidth class. |  | - |  |
| ca-IdleModeMeasurements Indicates whether UE supports reporting measurements performed during RRC_IDLE. |  |  | - |
| ca-IdleModeValidityArea Indicates whether UE supports validity area for IDLE measurements during RRC_IDLE. |  |  | - |
| cch-IM-RefRecTypeA-OneRX-Port This field defines whether the DL Category 1bis or the DL Category M2 UE supports Type A downlink control channel interference mitigation (CCH-IM) receiver "LMMSE-IRC + CRS-IC" for PDCCH/PCFICH/PHICH/EPDCCH receive processing (Enhanced downlink control channel performance requirements Type A in TS 36.101 [6]). |  | No |  |
| cch-InterfMitigation-RefRecTypeA, cch-InterfMitigation-RefRecTypeB, cch-InterfMitigation-MaxNumCCs The field cch-InterfMitigation-RefRecTypeA defines whether the UE supports Type A downlink control channel interference mitigation (CCH-IM) receiver "LMMSE-IRC + CRS-IC" for PDCCH/PCFICH/PHICH/EPDCCH receive processing (Enhanced downlink control channel performance requirements Type A in the TS 36.101 [6]). The field cch-InterfMitigation-RefRecTypeB defines whether the UE supports Type B downlink CCH-IM receiver "E-LMMSE-IRC + CRS-IC" for PDCCH/PCFICH/PHICH receive processing in synchronous networks (Enhanced downlink control channel performance requirements Type B in the TS 36.101 [6]). The UE supporting the capability defined by cch-InterfMitigation-RefRecTypeB-r13 shall also support the capability defined by cch-InterfMitigation-RefRecTypeA-r13. If the UE sets one or more of the fields cch-InterfMitigation-RefRecTypeA and cch-InterfMitigation-RefRecTypeB to "supported", the UE shall include the parameter cch-InterfMitigation-MaxNumCCs to indicate that the UE supports CCH-IM on at least one arbitrary downlink CC for up to cch-InterfMitigation-MaxNumCCs downlink CC CA configuration. The UE shall not include the parameter cch-InterfMitigation-MaxNumCCs if neither cch-InterfMitigation-RefRecTypeA nor cch-InterfMitigation-RefRecTypeB is present. The UE may not perform CCH-IM on more than 1 DL CCs. For example, the UE sets "cch-InterfMitigation-MaxNumCCs = 3" to indicate that UE supports CCH-IM on at least one DL CC for supported non-CA, 2DL CA and 3DL CA configurations. For CA scenarios, the CCH-IM is guaranteed to be supported on at least one arbitrary component carrier. |  | - |  |
| cdma2000-NW-Sharing Indicates whether the UE supports network sharing for CDMA2000. |  | - |  |
| ce-ClosedLoopTxAntennaSelection Indicates whether the UE supports UL closed-loop Tx antenna selection in CE mode A, as specified in TS 36.212 [22]. |  | Yes |  |
| ce-CQI-AlternativeTable Indicates whether the UE supports alternative CQI table in CE mode A. See TS 36.213 [22]. |  | Yes |  |
| ce-CRS-IntfMitig Indicates whether UE supports CRS interference mitigation, i.e., value supported indicates UE does not rely on the CRS outside certain PRBs and subframes as defined in TS 36.133 [16], clauses 3.6.1.2 and 3.6.1.3, and TS 36.213 [23] when operating in coverage enhancement mode. |  | Yes |  |
| ce-CSI-RS-Feedback Indicates whether the UE supports CSI-RS based feedback when the UE is operating in CE mode A, as specified in TS 36.213 [23]. |  | Yes |  |
| ce-CSI-RS-FeedbackCodebookRestriction Indicates whether the UE supports CSI-RS based feedback with codebook subset restriction when the UE in CE is operating in CE mode A, as specified in TS 36.213 [23]. |  | Yes |  |
| ce-DL-ChannelQualityReporting Indicates whether UE operating in CE mode supports aperiodic DL channel quality reporting in RRC_CONNECTED. |  | Yes |  |
| ce-EUTRA-5GC Indicates whether the UE operating in CE mode A or B supports E-UTRA/5GC. |  | Yes |  |
| ce-EUTRA-5GC-HO-ToNR-FDD-FR1 Indicates whether the UE operating in CE mode A or B supports handover from E-UTRA/5GC to NR FDD FR1. |  | Yes |  |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR1 Indicates whether the UE operating in CE mode A or B supports handover from E-UTRA/5GC to NR TDD FR1. |  | Yes |  |
| ce-EUTRA-5GC-HO-ToNR-FDD-FR2 Indicates whether the UE operating in CE mode A or B supports handover from E-UTRA/5GC to NR FDD FR2. |  | Yes |  |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR2 Indicates whether the UE operating in CE mode A or B supports handover from E-UTRA/5GC to NR TDD FR2. |  | Yes |  |
| ce-HARQ-AckBundling Indicates whether the UE supports HARQ-ACK bundling in half duplex FDD in CE mode A, as specified in TS 36.212 [22] and TS 36.213 [23]. |  | - |  |
| ce-InactiveState Indicates whether UE operating in CE mode supports RRC_INACTIVE when connected to 5GC. A UE including this field also supports short eDRX cycles in RRC_INACTIVE when connected to 5GC. |  | No |  |
| ce-MeasRSS-Dedicated, ce-MeasRSS-DedicatedSameRBs Indicates whether the UE operating in CE mode A/B supports receiving neighbour cell RSS information in dedicated signalling and performing serving cell and neighbour cell measurements based on RSS in RRC_CONNECTED as specified in TS 36.306 [5] and TS 36.133 [16]. |  | Yes |  |
| ce-ModeA, ce-ModeB Indicates whether the UE supports operation in CE mode A and/or B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | - |  |
| crs-ChEstMPDCCH-CE-ModeA, crs-ChEstMPDCCH-CE-ModeB Indicates whether UE operating in CE mode A/B supports using CRS for improving MPDCCH channel estimation. |  | Yes |  |
| crs-ChEstMPDCCH-CSI Indicates whether UE operating in CE mode A supports CSI-based mapping for improving MPDCCH channel estimation. |  | Yes |  |
| crs-ChEstMPDCCH-ReciprocityTDD Indicates whether UE operating in CE mode A supports using CRS for improving MPDCCH channel estimation with reciprocity-based candidates in TDD. |  | No |  |
| ceMeasurements Indicates whether the UE supports intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED, as specified in TS 36.133 [16] and TS 36.304 [4]. |  | - |  |
| ce-MultiTB-64QAM Indicates whether the UE supports downlink 64QAM for multiple TB scheduling in connected mode for PDSCH when operating in CE mode A, as specified in TS 36.211 [21] and TS 36.213 [23]. This field can be included only if ce-PUSCH-SubPRB-Allocation is included. |  | Yes |  |
| ce-MultiTB-EarlyTermination Indicates whether the UE supports early termination of PUSCH transmission for multiple TB scheduling in connected mode, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |  |
| ce-MultiTB-FrequencyHopping Indicates whether the UE supports frequency hopping for multiple TB scheduling for PDSCH/PUSCH in connected mode, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |  |
| ce-MultiTB-HARQ-AckBundling Indicates whether the UE supports downlink HARQ-ACK bundling for multiple TB scheduling in connected mode when operating in CE mode A, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |  |
| ce-MultiTB-Interleaving Indicates whether the UE supports TB interleaving for multiple TB scheduling in connected mode for PDSCH/PUSCH when operating in CE mode A or B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |  |
| ce-MultiTB-SubPRB Indicates whether the UE supports sub-PRB allocation for multiple TB scheduling for PUSCH in connected mode, as specified in TS 36.211 [21] and TS 36.213 [23]. This field can be included only if ce-PUSCH-SubPRB-Allocation is included. |  | Yes |  |
| ce-PDSCH-64QAM Indicates whether the UE supports 64QAM for non-repeated unicast PDSCH in CE mode A. |  |  | Yes |
| ce-PDSCH-FlexibleStartPRB-CE-ModeA, ce-PDSCH-FlexibleStartPRB-CE-ModeB, ce-PUSCH-FlexibleStartPRB-CE-ModeA, ce-PUSCH-FlexibleStartPRB-CE-ModeB This field indicates whether UE supports flexible starting PRB for PDSCH/PUSCH when operating in coverage enhancement mode A/B, as specified in TS 36.211 [21] and TS 36.213 [22]. |  | Yes |  |
| ce-PDSCH-PUSCH-Enhancement Indicates whether the UE supports new numbers of repetitions for PUSCH and modulation restrictions for PDSCH/PUSCH in CE mode A as specified in TS 36.212 [22] and TS 36.213 [23]. |  | No |  |
| ce-PDSCH-PUSCH-MaxBandwidth Indicates the maximum supported PDSCH/PUSCH channel bandwidth in CE mode A and B, as specified in TS 36.212 [22] and TS 36.213 [23]. Value bw5 corresponds to 5 MHz and value bw20 corresponds to 20 MHz. If the field is absent the maximum PDSCH/PUSCH channel bandwidth in CE mode A and B is 1.4 MHz. If the setting of this parameter is 20 MHz, the max supported PUSCH channel bandwidth in CE mode A is 5 MHz. The maximum PUSCH channel bandwidth in CE mode B is 1.4 MHz regardless of the setting of this parameter. Parameter: transmission bandwidth configuration, see TS 36.101 [42], table 5.6-1. |  | Yes |  |
| ce-PDSCH-TenProcesses Indicates whether the UE supports 10 DL HARQ processes in FDD in CE mode A. |  | Yes |  |
| ce-PUCCH-Enhancement Indicates whether the UE supports repetition levels 64 and 128 for PUCCH in CE Mode B, as specified in TS 36.211 [21] and in TS 36.213 [23]. |  | No |  |
| ce-PUSCH-NB-MaxTBS Indicates whether the UE supports 2984 bits max UL TBS in 1.4 MHz in CE mode A operation, as specified in TS 36.212 [22] and TS 36.213 [23]. |  | Yes |  |
| ce-PUSCH-SubPRB-Allocation Indicates whether the UE supports sub-PRB resource allocation for PUSCH in CE mode A or B, as specified in TS 36.211 [21], TS 36.212 [22] and TS 36.213 [23]. |  | Yes |  |
| ce-RetuningSymbols Indicates the number of retuning symbols in CE mode A and B as specified in TS 36.211 [21]. Value n0 corresponds to 0 retuning symbols and value n1 corresponds to 1 retuning symbol. If the field is absent the number of retuning symbols in CE mode A and B is 2. |  | No |  |
| ce-SchedulingEnhancement Indicates whether the UE supports dynamic HARQ-ACK delay for HD-FDD in CE mode A as specified in TS 36.212 [22] and TS 36.213 [23]. |  | No |  |
| ce-SRS-Enhancement Indicates whether the UE supports SRS coverage enhancement in TDD with support of SRS combs 2 and 4 as specified in TS 36.213 [23]. This field can be included only if ce-SRS-EnhancementWithoutComb4 is not included. |  | Yes |  |
| ce-SRS-EnhancementWithoutComb4 Indicates whether the UE supports SRS coverage enhancement in TDD with support of SRS comb 2 but without support of SRS comb 4 as specified in TS 36.213 [23]. This field can be included only if ce-SRS-Enhancement is not included. |  | - |  |
| ce-SwitchWithoutHO Indicates whether the UE supports switching between normal mode and enhanced coverage mode without handover. |  | - |  |
| ce-UL-HARQ-ACK-Feedback This field indicates whether UE supports uplink HARQ ACK feedback when operating in coverage enhancement, as specified in TS36.213 [22]. |  | Yes |  |
| channelMeasRestriction Indicates for a particular transmission mode whether the UE supports channel measurement restriction. |  | Yes |  |
| cho Indicates whether the UE supports conditional handover including execution condition, candidate cell configuration and maximum 8 candidate cells. |  | Yes |  |
| cho-Failure Indicates whether the UE supports conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover. |  | Yes |  |
| cho-FDD-TDD Indicates whether the UE supports conditional handover between FDD and TDD cells. |  | No |  |
| cho-TwoTriggerEvents Indicates whether the UE supports 2 trigger events for same execution condition. It is mandatory supported if the UE suppors cho. |  | Yes |  |
| codebook-HARQ-ACK Indicates whether the UE supports determining HARQ ACK codebook size based on the DAI-ased solution and/or the number of configured CCs. The first bit is set to "1" if the UE supports the DAI-based codebook size determination. The second bit is set to "1" if the UE supports the codebook determination based on the number of configured CCs. |  | No |  |
| commMultipleTx Indicates whether the UE supports multiple transmissions of sidelink communication to different destinations in one SC period. If commMultipleTx-r13 is set to supported then the UE support 8 transmitting sidelink processes. |  | - |  |
| commSimultaneousTx Indicates whether the UE supports simultaneous transmission of EUTRA and sidelink communication (on different carriers) in all bands for which the UE indicated sidelink support in a band combination (using commSupportedBandsPerBC). |  | - |  |
| commSupportedBands Indicates the bands on which the UE supports sidelink communication, by an independent list of bands i.e. separate from the list of supported E-UTRA band, as indicated in supportedBandListEUTRA. |  | - |  |
| commSupportedBandsPerBC Indicates, for a particular band combination, the bands on which the UE supports simultaneous reception of EUTRA and sidelink communication. If the UE indicates support simultaneous transmission (using commSimultaneousTx), it also indicates, for a particular band combination, the bands on which the UE supports simultaneous transmission of EUTRA and sidelink communication. The first bit refers to the first band included in commSupportedBands, with value 1 indicating sidelink is supported. |  | - |  |
| configN (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode whether the UE supports non-precoded EBF/ FD-MIMO (class A) related configuration N for the concerned band combination. |  | - |  |
| configN (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode whether the UE supports non-precoded EBF/ FD-MIMO (class A) related configuration N for band combinations for which the concerned capabilities are not signalled. |  | Yes |  |
| continueEHC-Context Indicates that the UE supports EHC context continuation operation where the UE keeps the established EHC context(s) upon PDCP re-establishment, as specified in TS 36.323 [8]. |  | No |  |
| crossCarrierScheduling |  | Yes |  |
| crossCarrierScheduling-B5C Indicates whether the UE supports cross carrier scheduling beyond 5 DL CCs. |  | No |  |
| crossCarrierSchedulingLAA-DL Indicates whether the UE supports cross-carrier scheduling from a licensed carrier for LAA cell(s) for downlink. This field can be included only if downlinkLAA is included. |  | - |  |
| crossCarrierSchedulingLAA-UL Indicates whether the UE supports cross-carrier scheduling from a licensed carrier for LAA cell(s) for uplink. This field can be included only if uplinkLAA is included. |  | - |  |
| crs-DiscoverySignalsMeas Indicates whether the UE supports CRS based discovery signals measurement, and PDSCH/EPDCCH RE mapping with zero power CSI-RS configured for discovery signals. |  | Yes |  |
| crs-IM-TM1-toTM9-OneRX-Port Indicates whether the DL Cateogry 1bis UE ot the DL Category M2 UE supports CRS interference mitigation (IM) while operating in the following transmission modes (TM): TM 1, TM 2, …, TM 8 and TM 9. |  |  | No |
| crs-InterfHandl Indicates whether the UE supports CRS interference handling. |  | Yes |  |
| crs-InterfMitigationTM10 The field defines whether the UE supports CRS interference mitigation in transmission mode 10. The UE supporting the crs-InterfMitigationTM10 capability shall also support the crs-InterfHandl capability. |  | No |  |
| crs-InterfMitigationTM1toTM9 Indicates whether the UE supports CRS interference mitigation (IM) while operating in the following transmission modes (TM): TM 1, TM 2, …, TM 8 and TM 9. The UE shall not include the field if it does not support CRS IM in TMs 1-9. If the field is present, the UE supports CRS-IM on at least one arbitrary downlink CC for up to crs-InterfMitigationTM1toTM9-r13 downlink CC CA configuration. The UE signals crs-InterfMitigationTM1toTM9-r13 value to indicate the maximum crs-InterfMitigationTM1toTM9-r13 downlink CC CA configuration where UE may apply CRS IM. For example, the UE sets "crs-InterfMitigationTM1toTM9-r13 = 3" to indicate that the UE supports CRS-IM on at least one DL CC for supported non-CA, 2DL CA and 3DL CA configurations. The UE supporting the crs-InterfMitigationTM1toTM9-r13 capability shall also support the crs-InterfHandl-r11 capability. |  | - |  |
| crs-IntfMitig Indicate whether the UE supports CRS interference mitigation as specified in TS 36.133 [16], clause 3.6.1.1. |  |  | Yes |
| crs-LessDwPTS Indicates whether the UE supports TDD special subframe configuration 10 without CRS transmission on the 5th symbol of DwPTS, i.e. ssp10-CRS-LessDwPTS, as specified in TS 36.211 [17]. |  | - |  |
| csi-ReportingAdvanced, csi-ReportingAdvancedMaxPorts (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates that for a particular transmission mode, the maximum number of CSI-RS ports supported by the UE for advanced CSI reporting is different in the concerned band of band combination than the value indicated by the field csi-ReportingAdvanced or csi-ReportingAdvancedMaxPorts in MIMO-UE-ParametersPerTM. The UE shall not include both csi-ReportingAdvanced and csi-ReportingAdvancedMaxPorts for a particular transmission mode in the concerned band of band combination. |  | - |  |
| csi-ReportingAdvanced (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of CSI-RS ports supported by the UE for advanced CSI reporting. The field csi-ReportingAdvanced indicates 32 CSI-RS ports. The UE shall not include both csi-ReportingAdvanced and csi-ReportingAdvancedMaxPorts for a particular transmission mode. | Yes |  |  |
| csi-ReportingAdvancedMaxPorts (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of CSI-RS ports supported by the UE for advanced CSI reporting. The field csi-ReportingAdvancedMaxPorts indicates 8, 12, 16, 20, 24 or 28 CSI-RS ports. The UE shall not include both csi-ReportingAdvanced and csi-ReportingAdvancedMaxPorts for a particular transmission mode. | - |  |  |
| csi-ReportingNP (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, value different indicates that for a particular transmission mode, the CSI reporting on non-precoded CSI-RS with 20, 24, 28 or 32 antenna ports for the concerned band of band combination is different than the value indicated by field csi-ReportingNP in MIMO-UE-ParametersPerTM. | - |  |  |
| csi-ReportingNP (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode whether the UE supports CSI reporting on non-precoded CSI-RS with 20, 24, 28, or 32 antenna ports for band combinations for which the concerned capabilities are not signalled in MIMO-CA-ParametersPerBoBCPerTM, and the FD-MIMO processing capability condition as described in NOTE 8 is satisfied. | Yes |  |  |
| csi-RS-DiscoverySignalsMeas Indicates whether the UE supports CSI-RS based discovery signals measurement. If this field is included, the UE shall also include crs-DiscoverySignalsMeas. |  | Yes |  |
| csi-RS-DRS-RRM-MeasurementsLAA Indicates whether the UE supports performing RRM measurements on LAA cell(s) based on CSI-RS-based DRS. This field can be included only if downlinkLAA is included. |  | - |  |
| csi-RS-EnhancementsTDD Indicates for a particular transmission mode whether the UE supports CSI-RS enhancements applicable for TDD. |  | Yes |  |
| csi-SubframeSet Indicates whether the UE supports REL-12 DL CSI subframe set configuration, REL-12 DL CSI subframe set dependent CSI measurement/feedback, configuration of up to 2 CSI-IM resources for a CSI process with no more than 4 CSI-IM resources for all CSI processes of one frequency if the UE supports tm10, configuration of two ZP-CSI-RS for tm1 to tm9, PDSCH RE mapping with two ZP-CSI-RS configurations, and EPDCCH RE mapping with two ZP-CSI-RS configurations if the UE supports EPDCCH. This field is only applicable for UEs supporting TDD. |  | Yes |  |
| dataInactMon Indicates whether the UE supports the data inactivity monitoring as specified in TS 36.321 [6]. |  | - |  |
| dc-Support Including this field indicates that the UE supports synchronous DC and power control mode 1. Including this field for a band combination entry comprising of single band entry indicates that the UE supports intra-band contiguous DC. Including this field for a band combination entry comprising of two or more band entries, indicates that the UE supports DC for these bands and that the serving cells corresponding to a band entry shall belong to one cell group (i.e. MCG or SCG). Including field asynchronous indicates that the UE supports asynchronous DC and power control mode 2. Including this field for a TDD/FDD band combination indicates that the UE supports TDD/FDD DC for this band combination. |  | - |  |
| delayBudgetReporting Indicates whether the UE supports delay budget reporting. |  | No |  |
| demodulationEnhancements This field defines whether the UE supports advanced receiver in SFN scenario (350 km/h) as specified in TS 36.101 [42]. |  | - |  |
| demodulationEnhancements2 This field defines whether the UE supports further enhanced receiver in HST-SFN scenario (up to 500 km/h velocity) as specified in TS 36.101 [42]. |  | - |  |
| densityReductionNP, densityReductionBF Indicates whether the UE supports CSI-RS density reduction with values 1, 1/2 and 1/3 for non-precoded CSI-RS and beamformed CSI-RS respectively. |  | Yes |  |
| deviceType UE may set the value to "noBenFromBatConsumpOpt" when it does not foresee to particularly benefit from NW-based battery consumption optimisation. Absence of this value means that the device does benefit from NW-based battery consumption optimisation. |  | - |  |
| diffFallbackCombReport Indicates that the UE supports reporting of UE radio access capabilities for the CA band combinations asked by the eNB as well as, if any, reporting of different UE radio access capabilities for their fallback band combination as specified in TS 36.306 [5]. The UE does not report fallback combinations if their UE radio access capabilities are the same as the ones for the CA band combination asked by the eNB. |  | - |  |
| differentFallbackSupported Indicates that the UE supports different capabilities for at least one fallback case of this band combination. |  | - |  |
| directMCG-SCellActivationResume Indicates whether the UE supports having an E-UTRA MCG SCell configured in activated SCell state. |  | - |  |
| directSCellActivation Indicates whether the UE supports having an E-UTRA SCell configured in activated SCell state in the RRCConnectionReconfiguration message. This field is applicable to both LTE standalone and LTE-DC. |  |  | - |
| directSCellHibernation Indicates whether the UE supports having an SCell configured in dormant SCell state. |  |  | - |
| directSCG-SCellActivationNEDC Indicates whether the UE supports having an E-UTRA SCG SCell configured in activated SCell state in the RRCConnectionReconfiguration message contained in the NR RRCReconfiguration message, as defined in TS 36.321 [6] and TS 38.331 [82]. If the UE indicates support of directSCG-SCellActivationNEDC-r16, the UE shall also indicate support of ne-dc as specified in TS 38.331 [82]. |  |  | - |
| directSCG-SCellActivationResume Indicates whether the UE supports having an E-UTRA SCG SCell configured in activated SCell state. |  |  | - |
| discInterFreqTx Indicates whether the UE support sidelink discovery announcements either a) on the primary frequency only or b) on other frequencies also, regardless of the UE configuration (e.g. CA, DC). The UE may set discInterFreqTx to supported when having a separate transmitter or if it can request sidelink discovery transmission gaps. |  | - |  |
| discoverySignalsInDeactSCell Indicates whether the UE supports the behaviour on DL signals and physical channels when SCell is deactivated and discovery signals measurement is configured as specified in TS 36.211 [21], clause 6.11A. This field is included only if UE supports carrier aggregation and includes crs-DiscoverySignalsMeas. |  | Yes |  |
| discPeriodicSLSS Indicates whether the UE supports periodic (i.e. not just one time before sidelink discovery announcement) Sidelink Synchronization Signal (SLSS) transmission and reception for sidelink discovery. |  | - |  |
| discScheduledResourceAlloc Indicates whether the UE supports transmission of discovery announcements based on network scheduled resource allocation. |  | - |  |
| disc-UE-SelectedResourceAlloc Indicates whether the UE supports transmission of discovery announcements based on UE autonomous resource selection. |  | - |  |
| disc-SLSS Indicates whether the UE supports Sidelink Synchronization Signal (SLSS) transmission and reception for sidelink discovery. |  | - |  |
| discSupportedBands Indicates the bands on which the UE supports sidelink discovery. One entry corresponding to each supported E-UTRA band, listed in the same order as in supportedBandListEUTRA. |  | - |  |
| discSupportedProc Indicates the number of processes supported by the UE for sidelink discovery. |  | - |  |
| discSysInfoReporting Indicates whether the UE supports reporting of system information for inter-frequency/PLMN sidelink discovery. |  | - |  |
| dl-256QAM Indicates whether the UE supports 256QAM in DL on the band. |  | - |  |
| dl-1024QAM Indicates whether the UE supports 1024QAM in DL on the band or on the band within the band combination. When dl-1024QAM-ScalingFactor and dl-1024QAM-TotalWeightedLayers are included, the UE supports 1024QAM in a set of CCs in a band combination if the CCs belong to bands indicated to support 1024QAM in that band combination and the 1024QAM processing capability condition as specified in equation 4.3.5.31-1 in TS 36.306 [5] is satisfied. |  | - |  |
| dl-1024QAM-ScalingFactor Indicates scaling factor for processing a CC configured with 1024QAM with respect to a CC not configured with 1024QAM as described in 4.3.5.31 in TS 36.306 [5]. Value v1 indicates 1, value v1dot2 indicates 1.2 and value v1dot25 indicates 1.25. |  | - |  |
| dl-1024QAM-TotalWeightedLayers Indicates total number of weighted layers the UE can process for 1024QAM as described in 4.3.5.31 in TS 36.306 [5]. Actual value = (10 + indicated value x 2), i.e., value 0 indicates 10 layers, value 1 indicates 12 layers and so on. |  | - |  |
| dl-1024QAM-Slot Indicates whether the UE supports 1024QAM in DL on the band for slot TTI operation. |  | - |  |
| dl-1024QAM-SubslotTA-1 Indicates whether the UE supports 1024QAM in DL on the band for subslot TTI operation with TA set 1. |  | - |  |
| dl-1024QAM-SubslotTA-2 Indicates whether the UE supports 1024QAM in DL on the band for subslot TTI operation with TA set 2, dmrsBasedSPDCCH-nonMBSFN |  | - |  |
| dl-DedicatedMessageSegmentation Indicates whether the UE supports reception of segmented DL RRC messages. |  | - |  |
| dmrs-BasedSPDCCH-MBSFN Indicates whether the UE supports sDCI monitoring in DMRS based SPDCCH for MBSFN subframe. If UE supports this, it also provides the corresponding DMRS based SPDCCH capability in min-Proc-TimelineSubslot. |  | Yes |  |
| dmrs-BasedSPDCCH-nonMBSFN Indicates whether the UE supports sDCI monitoring in DMRS based SPDCCH for non-MBSFN subframe. If UE supports this, it also provides the corresponding DMRS based SPDCCH capability in min-Proc-TimelineSubslot. |  | Yes |  |
| dmrs-Enhancements (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode, that for the concerned band combination the DMRS enhancements are different than the value indicated by field dmrs-Enhancements in MIMO-UE-ParametersPerTM. |  | - |  |
| dmrs-Enhancements (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode whether the UE supports DMRS enhancements for the indicated transmission mode. |  | Yes |  |
| dmrs-LessUpPTS Indicates whether the UE supports not to transmit DMRS for PUSCH in UpPTS. |  | No |  |
| dmrs-OverheadReduction Indicates whether the UE supports OCC4 for rank 3 and 4 transmission as specified in clause 5.3.3.1.5C of TS 36.212 [22]. |  | Yes |  |
| dmrs-PositionPattern Indicates whether the UE supports uplink DMRS position pattern 'D D D' in subslot #5 with application of the 1/6 as the TBS scaling factor. |  | Yes |  |
| dmrs-RepetitionSubslotPDSCH Indicates whether the UE supports back-to-back 3/4-layer DMRS reception in two consecutive subslots across subframe boundary for subslot-PDSCH. |  | Yes |  |
| dmrs-SharingSubslotPDSCH Indicates whether the UE supports DMRS sharing in two consecutive subslots across subframe boundary for subslot-PDSCH. |  | Yes |  |
| dormantSCellState Indicates whether UE supports Dormant SCell state (i.e. SCell state with CQI and RRM measurement reporting but no PDCCH monitoring). |  |  | - |
| downlinkLAA Presence of the field indicates that the UE supports downlink LAA operation including identification of downlink transmissions on LAA cell(s) for full downlink subframes, decoding of common downlink control signalling on LAA cell(s), CSI feedback for LAA cell(s), RRM measurements on LAA cell(s) based on CRS-based DRS. |  | - |  |
| drb-TypeSCG Indicates whether the UE supports SCG bearer. |  | - |  |
| drb-TypeSplit Indicates whether the UE supports split bearer except for PDCP data transfer in UL. |  | - |  |
| dtm Indicates whether the UE supports DTM in GERAN. |  | - |  |
| dummy This field is not used in the specification. It shall not be sent by the UE. |  | - |  |
| earlyData-UP Indicates whether the UE supports UP-EDT when connected to EPC. |  |  | - |
| earlyData-UP-5GC Indicates whether the UE supports UP-EDT when connected to 5GC. |  |  | - |
| earlySecurityReactivation Indicates whether the UE supports early security reactivation when resuming a suspended RRC connection. |  |  | - |
| e-CSFB-1XRTT Indicates whether the UE supports enhanced CS fallback to CDMA2000 1xRTT or not. |  | Yes |  |
| e-CSFB-ConcPS-Mob1XRTT Indicates whether the UE supports concurrent enhanced CS fallback to CDMA2000 1xRTT and PS handover/ redirection to CDMA2000 HRPD. |  | Yes |  |
| e-CSFB-dual-1XRTT Indicates whether the UE supports enhanced CS fallback to CDMA2000 1xRTT for dual Rx/Tx configuration. This bit can only be set to supported if tx-Config1XRTT and rx-Config1XRTT are both set to dual. |  | Yes |  |
| e-HARQ-Pattern-FDD Indicates whether the UE supports enhanced HARQ pattern for TTI bundling operation for FDD. |  | Yes |  |
| ehc Indicates that the UE supports Ethernet header compression and decompression using EHC protocol, as specified in TS 36.323 [8] and in Annex A of TS 38.323 [83]. The UE indicating this capability and indicating support for at least one ROHC profile, shall support simultaneous configuration of EHC and ROHC on different DRBs. |  | No |  |
| eLCID-Support Indicates whether the UE supports LCID "10000" and MAC PDU subheader containing the eLCID field as described in TS 36.321 [6]. |  | - |  |
| emptyUnicastRegion Indicates whether the UE supports unicast reception in subframes with empty unicast control region as described in TS 36.213 [23] clause 12. This field can be included only if unicast-fembmsMixedSCell and crossCarrierScheduling are included. |  | No |  |
| en-DC Indicates whether the UE supports EN-DC. |  | - |  |
| endingDwPTS Indicates whether the UE supports reception ending with a subframe occupied for a DwPTS-duration as described in TS 36.211 [21] and TS 36.213 [23]. This field can be included only if downlinkLAA is included. |  | - |  |
| Enhanced-4TxCodebook Indicates whether the UE supports enhanced 4Tx codebook. |  | No |  |
| enhancedDualLayerTDD Indicates whether the UE supports enhanced dual layer (PDSCH transmission mode 8) for TDD or not. |  | - |  |
| ePDCCH Indicates whether the UE can receive DCI on UE specific search space on Enhanced PDCCH. |  | Yes |  |
| epdcch-SPT-differentCells Indicates whether the UE supports EPDCCH and short processing time on different serving cells. |  | Yes |  |
| epdcch-STTI-differentCells Indicates whether the UE supports EPDCCH and sTTI on different serving cells. |  | Yes |  |
| e-RedirectionUTRA |  | Yes |  |
| e-RedirectionUTRA-TDD Indicates whether the UE supports enhanced redirection to UTRA TDD to multiple carrier frequencies both with and without using related SIB provided by RRCConnectionRelease or not. |  | Yes |  |
| etws-CMAS-RxInConnCE-ModeA, etws-CMAS-RxInConn Indicates whether the UE operating in CE mode A/B supports reception of ETWS/CMAS indication in RRC_CONNECTED mode as specified in TS 36.212 [22]. |  | - |  |
| eutra-5GC Indicates whether the UE supports E-UTRA/5GC. |  | Yes |  |
| eutra-5GC-HO-ToNR-FDD-FR1 Indicates whether the UE supports handover from E-UTRA/5GC to NR FDD FR1. |  | Yes |  |
| eutra-5GC-HO-ToNR-TDD-FR1 Indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR1. |  | Yes |  |
| eutra-5GC-HO-ToNR-FDD-FR2 Indicates whether the UE supports handover from E-UTRA/5GC to NR FDD FR2. |  | Yes |  |
| eutra-5GC-HO-ToNR-TDD-FR2 Indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR2. |  | Yes |  |
| eutra-CGI-Reporting-ENDC Indicates whether the UE supports Intra-RAT report CGI procedure when it is configured with (NG) EN-DC wherein either MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if their DRX cycles are same. |  |  | Yes |
| eutra-CGI-Reporting-NEDC Indicates whether the UE supports acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network when the NE-DC is configured. |  |  | Yes |
| eutra-EPC-HO-ToNR-FDD-FR1 Indicates whether the UE supports handover from E-UTRA/EPC to NR FDD FR1. |  | Yes |  |
| eutra-EPC-HO-ToNR-TDD-FR1 Indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR1. |  | Yes |  |
| eutra-EPC-HO-ToNR-FDD-FR2 Indicates whether the UE supports handover from E-UTRA/EPC to NR FDD FR2. |  | Yes |  |
| eutra-EPC-HO-ToNR-TDD-FR2 Indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR2. |  | Yes |  |
| eutra-EPC-HO-EUTRA-5GC Indicates whether the UE supports handover between E-UTRA/EPC and E-UTRA/5GC. |  | Yes |  |
| eutra-IdleInactiveMeasurements Indicates whether UE supports reporting measurements performed during RRC_IDLE or RRC_INACTIVE. |  |  | No |
| eutra-SI-AcquisitionForHO-ENDC Indicates whether the UE supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell using autonomous gaps and reporting the acquired information to the network. |  | Yes |  |
| eventB2 Indicates whether the UE supports event B2. A UE supporting NR SA operation shall set this bit to supported. |  | - |  |
| extendedBand-n77 This field is only applicable for UEs that indicate support for band n77. If present, the UE supports the restriction to 3450 - 3550 MHz and 3700 - 3980 MHz ranges of band n77 in the USA as specified in Note 12 of Table 5.2-1 in TS 38.101-1 [85]. If absent, the UE supports only restriction to the 3700 - 3980 MHz range of band n77 in the USA. A UE that indicates this field shall support NS value 55 as specified in TS 38.101-1 [85]. |  | - |  |
| extendedFreqPriorities Indicates whether the UE supports extended E-UTRA frequency priorities indicated by cellReselectionSubPriority field. A UE supporting NR SA operation shall set this bit to supported. |  | - |  |
| extendedLCID-Duplication Indicates whether the UE supports use of extended LCIDs 32-38 for PDCP duplication. |  | - |  |
| extendedLongDRX Indicates whether the UE supports extended long DRX cycle values of 5.12s and 10.24s in RRC_CONNECTED. |  | - |  |
| extendedMAC-LengthField Indicates whether the UE supports the MAC header with L field of size 16 bits as specified in TS 36.321 [6], clause 6.2.1. |  | - |  |
| extendedMaxMeasId Indicates whether the UE supports extended number of measurement identies as defined by maxMeasId-r12. |  | No |  |
| extendedMaxObjectId Indicates whether the UE supports extended number of measurement object identies as defined by maxObjectId-r13. |  | No |  |
| extendedNumberOfDRBs Indicates whether the UE supports up to 15 DRBs. The UE shall support any combination of RLC AM and RLC UM entities for the configured DRBs. |  |  | - |
| extendedPollByte Indicates whether the UE supports extended pollByte values as defined by pollByte-r14. |  | - |  |
| extended-RLC-LI-Field Indicates whether the UE supports 15 bit RLC length indicator. |  | - |  |
| extendedRLC-SN-SO-Field Indicates whether the UE supports 16 bits of RLC sequence number and segmentation offset. |  | - |  |
| extendedRSRQ-LowerRange Indicates whether the UE supports the extended RSRQ lower value range from -34dB to -19.5dB in measurement configuration and reporting as specified in TS 36.133 [16]. |  | No |  |
| fdd-HARQ-TimingTDD Indicates whether UE supports FDD HARQ timing for TDD SCell when configured with TDD PCell. |  | Yes |  |
| featureGroupIndicators, featureGroupIndRel9Add, featureGroupIndRel10 The definitions of the bits in the bit string are described in Annex B.1 (for featureGroupIndicators and featureGroupIndRel9Add) and in Annex C.1 (for featureGroupIndRel10). |  | Yes |  |
| featureSetsDL-PerCC In MR-DC, indicates a set of features that the UE supports on one component carrier in a bandwidth class for a band in a given band combination. The UE shall hence include at least as many FeatureSetDL-PerCC-Id in this list as the number of carriers it supports according to the ca-bandwidthClassDL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set. The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDL-PerCC-Id in this list. |  | - |  |
| FeatureSetDL-PerCC-Id In MR-DC, indicates the index position of the FeatureSetDL-PerCC-r15 in the featureSetsDL-PerCC-r15 list. Value 0 corresponds to the first element in the list, value 1 corresponds to the second element in the list, and so on. Value 32 is not used. |  | - |  |
| featureSetsUL-PerCC In MR-DC, indicates a set of features that the UE supports on one component carrier in a bandwidth class for a band in a given band combination. The UE shall hence include at least as many FeatureSetUL-PerCC-Id in this list as the number of carriers it supports according to the ca-bandwidthClassUL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set. The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetUL-PerCC-Id in this list. |  | - |  |
| FeatureSetUL-PerCC-Id In MR-DC, indicates the index position of the FeatureSetUL-PerCC-r15 in the featureSetsUL-PerCC-r15 list. Value 0 corresponds to the first element in the list, value 1 corresponds to the second element in the list, and so on. Value 32 is not used. |  | - |  |
| fembmsMixedCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception with 15 kHz subcarrier spacings via MBSFN from FeMBMS/Unicast mixed cells on a frequency indicated in an MBMSInterestIndication message. |  |  |  |
| fembmsDedicatedCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception with 15 kHz subcarrier spacings via MBSFN from MBMS-dedicated cells on a frequency indicated in an MBMSInterestIndication message. |  |  |  |
| flexibleUM-AM-Combinations Indicates whether the UE supports any combination of RLC UM and RLC AM bearers as long as the total number of bearers is at most 8, regardless of what FGI20 indicates. |  |  | - |
| flightPathPlan Indicates whether UE supports reporting of flight path plan information. |  |  | - |
| fourLayerTM3-TM4 Indicates whether the UE supports 4-layer spatial multiplexing for TM3 and TM4. |  | - |  |
| fourLayerTM3-TM4 (in FeatureSetDL-PerCC) Indicates whether the UE supports 4-layer spatial multiplexing for TM3 and TM4 for MR-DC within the indicated feature set. If this field is absent, UE supports two layer MIMO for TM3/TM4. |  | - |  |
| fourLayerTM3-TM4-perCC Indicates whether the UE supports 4-layer spatial multiplexing for TM3 and TM4 for the component carrier. |  | - |  |
| frameStructureType-SPT This field indicates the supported FS-type(s) for short processing time. The UE capability is reported per band combination. The reported FS-type(s) apply to the reported maxNumberCCs-SPT-r15 for the given band combination. |  | - |  |
| freqBandPriorityAdjustment Indicates whether the UE supports the prioritization of frequency bands in multiBandInfoList over the band in freqBandIndicator as defined by freqBandIndicatorPriority-r12. |  | - |  |
| freqBandRetrieval Indicates whether the UE supports reception of requestedFrequencyBands. |  | - |  |
| halfDuplex If halfDuplex is set to true, only half duplex operation is supported for the band, otherwise full duplex operation is supported. |  | - |  |
| heightMeas Indicates whether UE supports the measurement events H1/H2. |  | - |  |
| ho-EUTRA-5GC-FDD-TDD Indicates whether the UE supports handover between E-UTRA/5GC FDD and E-UTRA/5GC TDD. |  | No |  |
| ho-InterfreqEUTRA-5GC Indicates whether the UE supports inter frequency handover within E-UTRA/5GC. |  | Yes |  |
| hybridCSI Indicates whether the UE supports hybrid CSI transmission as described in TS 36.213 [23]. |  | Yes |  |
| idleInactiveValidityAreaList Indicates whether the UE supports list of validity areas for measurements during RRC_IDLE and RRC_INACTIVE. |  | No |  |
| immMeasBT Indicates whether the UE supports Bluetooth measurements in RRC connected mode. |  | - |  |
| immMeasWLAN Indicates whether the UE supports WLAN measurements in RRC connected mode. |  | - |  |
| ims-VoiceOverMCG-BearerEUTRA-5GC Indicates whether the UE supports IMS voice over NR PDCP for MCG bearer for E-UTRA/5GC. |  | No |  |
| ims-VoiceOverNR-FR1 Indicates whether the UE supports IMS voice over NR FR1. |  | No |  |
| ims-VoiceOverNR-FR2 Indicates whether the UE supports IMS voice over NR FR2. |  | No |  |
| ims-VoiceOverNR-PDCP-MCG-Bearer Indicates whether the UE supports IMS voice over NR PDCP with only MCG RLC bearer. |  | Yes |  |
| ims-VoiceOverNR-PDCP-SCG-Bearer Indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer when configured with EN-DC. |  | Yes |  |
| ims-VoNR-PDCP-SCG-NGENDC Indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer when configured with NGEN-DC. |  | Yes |  |
| inactiveState Indicates whether the UE supports RRC_INACTIVE. |  | No |  |
| incMonEUTRA Indicates whether the UE supports increased number of E-UTRA carrier monitoring in RRC_IDLE and RRC_CONNECTED, as specified in TS 36.133 [16]. |  | No |  |
| incMonUTRA Indicates whether the UE supports increased number of UTRA carrier monitoring in RRC_IDLE and RRC_CONNECTED, as specified in TS 36.133 [16]. |  | No |  |
| inDeviceCoexInd Indicates whether the UE supports in-device coexistence indication as well as autonomous denial functionality. |  | Yes |  |
| inDeviceCoexInd-ENDC Indicates whether the UE supports in-device coexistence indication for (NG)EN-DC operation. This field can be included only if inDeviceCoexInd is included. The UE supports inDeviceCoexInd-ENDC in the same duplexing modes as it supports inDeviceCoexInd. |  | - |  |
| inDeviceCoexInd-HardwareSharingInd Indicates whether the UE supports indicating hardware sharing problems when sending the InDeviceCoexIndication, as well as omitting the TDM assistance information. A UE that supports hardware sharing indication shall also indicate support of LAA operation. |  | - |  |
| inDeviceCoexInd-UL-CA Indicates whether the UE supports UL CA related in-device coexistence indication. This field can be included only if inDeviceCoexInd is included. The UE supports inDeviceCoexInd-UL-CA in the same duplexing modes as it supports inDeviceCoexInd. |  | - |  |
| interBandTDD-CA-WithDifferentConfig Indicates whether the UE supports inter-band TDD carrier aggregation with different UL/DL configuration combinations. The first bit indicates UE supports the configuration combination of SCell DL subframes are a subset of PCell and PSCell by SIB1 configuration and the configuration combination of SCell DL subframes are a superset of PCell and PSCell by SIB1 configuration; the second bit indicates UE supports the configuration combination of SCell DL subframes are neither superset nor subset of PCell and PSCell by SIB1 configuration. This field is included only if UE supports inter-band TDD carrier aggregation. |  | - |  |
| interBandPowerSharingAsyncDAPS Indicates whether the UE supports power sharing for asynchronous inter-band DAPS handovers. |  | - |  |
| interBandPowerSharingSyncDAPS Indicates whether the UE supports power sharing for synchronous inter-band DAPS handovers. |  | - |  |
| interferenceMeasRestriction Indicates whether the UE supports interference measurement restriction. |  | Yes |  |
| interFreqAsyncDAPS Indicates whether the UE supports asynchronous DAPS handover in source PCell and inter-frequency target PCell. |  | - |  |
| interFreqBandList One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. |  | - |  |
| interFreqDAPS Indicates whether the UE supports DAPS handover in source PCell and inter-frequency target PCell, i.e. support of simultaneous DL reception of PDCCH and PDSCH from source and target cell. For a BC, the capability applies to every carrier pair for source and target. A UE indicating this capability shall also support synchronous DAPS handover, and single UL transmission for inter-frequency DAPS handover. |  | - |  |
| interFreqMultiUL-TransmissionDAPS Indicates that the UE supports simultaneous UL transmission in source PCell and inter-frequency target PCell. |  | - |  |
| interFreqNeedForGaps Indicates need for measurement gaps when operating on the E-UTRA band given by the entry in bandListEUTRA or on the E-UTRA band combination given by the entry in bandCombinationListEUTRA and measuring on the E-UTRA band given by the entry in interFreqBandList. |  | - |  |
| interFreqProximityIndication Indicates whether the UE supports proximity indication for inter-frequency E-UTRAN CSG member cells. |  | - |  |
| interFreqRSTD-Measurement Indicates whether the UE supports inter-frequency RSTD measurements for OTDOA positioning, as specified in TS 36.355 [54]. |  | Yes |  |
| interFreqSI-AcquisitionForHO Indicates whether the UE supports, upon configuration of si-RequestForHO by the network, acquisition and reporting of relevant information using autonomous gaps by reading the SI from a neighbouring inter-frequency cell. |  | Yes |  |
| interRAT-BandList One entry corresponding to each supported band of another RAT listed in the same order as in the interRAT-Parameters. The NR bands reported in SupportedBandListNR are excluded from this list. |  | - |  |
| interRAT-BandListNR-EN-DC One entry corresponding to each supported NR band listed in the same order as in the supportedBandListEN-DC-r15. If both interRAT-BandListNR-EN-DC and interRAT-BandListNR-SA are included, the UE shall set the same interRAT-NeedForGapsNR value for the same NR band. |  | - |  |
| interRAT-BandListNR-SA One entry corresponding to each supported NR band listed in the same order as in the supportedBandListNR-SA. If both interRAT-BandListNR-EN-DC and interRAT-BandListNR-SA are included, the UE shall set the same interRAT-NeedForGapsNR value for the same NR band. |  | - |  |
| interRAT-enhancementNR Indicates whether the UE supports enhanced inter-RAT NR measurement requirements to support high speed up to 500 km/h as specified in TS 36.133 [16], when EN-DC is not configured and when EN-DC is configured. |  | - |  |
| interRAT-NeedForGaps Indicates need for DL measurement gaps when operating on the E-UTRA band given by the entry in bandListEUTRA or on the E-UTRA band combination given by the entry in bandCombinationListEUTRA and measuring on the inter-RAT band given by the entry in the interRAT-BandList. |  | - |  |
| interRAT-NeedForGapsNR Indicates need for measurement gaps when operating on the E-UTRA band given by the entry in supportedBandListEUTRA or on the E-UTRA band combination given by the entry in supportedBandCombination-r10 or supportedBandCombinationAdd-r11 or supportedBandCombinationReduced-r13 and measuring on the NR band given by the entry in the InterRAT-BandListNR. |  | - |  |
| interRAT-ParametersWLAN Indicates whether the UE supports WLAN measurements configured by MeasObjectWLAN with corresponding quantity and report configuration in the supported WLAN bands. |  | - |  |
| interRAT-PS-HO-ToGERAN Indicates whether the UE supports inter-RAT PS handover to GERAN or not. |  | Yes |  |
| intraBandContiguousCC-InfoList Indicates, per serving carrier of which the corresponding bandwidth class includes multiple serving carriers (i.e. bandwidth class B, C, D and so on), the maximum number of supported layers for spatial multiplexing in DL and the maximum number of CSI processes supported. The number of entries is equal to the number of component carriers in the corresponding bandwidth class. The UE shall support the setting indicated in each entry of the list regardless of the order of entries in the list.The UE shall include the field only if it supports 4-layer spatial multiplexing in transmission mode3/4 for a subset of component carriers in the corresponding bandwidth class, or if the maximum number of supported layers for at least one component carrier is higher than supportedMIMO-CapabilityDL-r10 in the corresponding bandwidth class, or if the number of CSI processes for at least one component carrier is higher than supportedCSI-Proc-r11 in the corresponding band. This field may also be included for bandwidth class A but in such a case without including any sub-fields in IntraBandContiguousCC-Info-r12 (see NOTE 6). |  | - |  |
| intraFreqA3-CE-ModeA Indicates whether the UE when operating in CE Mode A supports eventA3 for intra-frequency neighbouring cells. |  | - |  |
| intraFreqA3-CE-ModeB Indicates whether the UE when operating in CE Mode B supports eventA3 for intra-frequency neighbouring cells. |  | - |  |
| intraFreq-CE-NeedForGaps Indicates need for measurement gaps when operating in CE on the E-UTRA band given by the entry in supportedBandListEUTRA. |  |  |  |
| intraFreqAsyncDAPS Indicates whether the UE supports asynchronous DAPS handover in source PCell and intra-frequency target PCell. |  | - |  |
| intraFreqDAPS Indicates whether UE supports DAPS handover in source PCell and intra-frequency target PCell, i.e. support of simultaneous DL reception of PDCCH and PDSCH from source and target cell. A UE indicating this capability shall also support synchronous DAPS handover, and single UL transmission for intra-frequency DAPS handover. |  | - |  |
| intraFreqHO-CE-ModeA Indicates whether the UE when operating in CE Mode A supports intra-frequency handover. |  | - |  |
| intraFreqHO-CE-ModeB Indicates whether the UE when operating in CE Mode B supports intra-frequency handover. |  | - |  |
| intraFreqProximityIndication Indicates whether the UE supports proximity indication for intra-frequency E-UTRAN CSG member cells. |  | - |  |
| intraFreqSI-AcquisitionForHO Indicates whether the UE supports, upon configuration of si-RequestForHO by the network, acquisition and reporting of relevant information using autonomous gaps by reading the SI from a neighbouring intra-frequency cell. |  | Yes |  |
| intraFreqTwoTAGs-DAPS Indicates whether the UE supports different timing advance groups in source PCell and intra-frequency target PCell. It is mandatory for intraFreqDAPS capable UE. |  | - |  |
| jointEHC-ROHC-Config Indicates whether the UE supports simultaneous configuration of EHC and ROHC protocols for the same DRB. |  | No |  |
| k-Max (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode the maximum number of NZP CSI RS resource configurations supported within a CSI process applicable for the concerned band combination. |  | No |  |
| k-Max (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of NZP CSI RS resource configurations supported within a CSI process applicable for band combinations for which the concerned capabilities are not signalled. |  | Yes |  |
| laa-PUSCH-Mode1 Indicates whether the UE supports LAA PUSCH mode 1 as defined in TS 36.213 [23]. |  | - |  |
| laa-PUSCH-Mode2 Indicates whether the UE supports LAA PUSCH mode 2 as defined in TS 36.213 [23]. |  | - |  |
| laa-PUSCH-Mode3 Indicates whether the UE supports LAA PUSCH mode 3 as defined in TS 36.213 [23]. |  | - |  |
| locationReport Indicates whether the UE supports reporting of its geographical location information to eNB. |  | - |  |
| loggedMBSFNMeasurements Indicates whether the UE supports logged measurements for MBSFN. A UE indicating support for logged measurements for MBSFN shall also indicate support for logged measurements in Idle mode. |  | - |  |
| loggedMeasBT Indicates whether the UE supports Bluetooth measurements in RRC idle mode. |  | - |  |
| loggedMeasurementsIdle Indicates whether the UE supports logged measurements in Idle mode. |  | - |  |
| loggedMeasWLAN Indicates whether the UE supports WLAN measurements in RRC idle mode. |  | - |  |
| logicalChannelSR-ProhibitTimer Indicates whether the UE supports the logicalChannelSR-ProhibitTimer as defined in TS 36.321 [6]. |  | - |  |
| longDRX-Command Indicates whether the UE supports Long DRX Command MAC Control Element. |  | - |  |
| lwa Indicates whether the UE supports LTE-WLAN Aggregation (LWA). The UE which supports LWA shall also indicate support of interRAT-ParametersWLAN-r13. |  | - |  |
| lwa-BufferSize Indicates whether the UE supports the layer 2 buffer sizes for "with support for split bearers" as defined in Table 4.1-3 and 4.1A-3 of TS 36.306 [5] for LWA. |  | - |  |
| lwa-HO-WithoutWT-Change Indicates whether the UE supports handover where LWA configuration is retained without WT change and using LWA end-marker for PDCP key change indication for LWA operation. |  | - |  |
| lwa-RLC-UM Indicates whether the UE supports RLC UM for LWA bearer. |  | - |  |
| lwa-SplitBearer Indicates whether the UE supports the split LWA bearer (as defined in TS 36.300 [9]). |  | - |  |
| lwa-UL Indicates whether the UE supports UL transmission over WLAN for LWA bearer. |  | - |  |
| lwip Indicates whether the UE supports LTE/WLAN Radio Level Integration with IPsec Tunnel (LWIP). The UE which supports LWIP shall also indicate support of interRAT-ParametersWLAN-r13. |  | - |  |
| lwip-Aggregation-DL, lwip-Aggregation-UL Indicates whether the UE supports aggregation of LTE and WLAN over DL/UL LWIP. The UE that indicates support of LWIP aggregation over DL or UL shall also indicate support of lwip. |  | - |  |
| makeBeforeBreak Indicates whether the UE supports intra-frequency Make-Before-Break handover, and whether the UE which indicates dc-Parameters supports intra-frequency Make-Before-Break SeNB change, as defined in TS 36.300 [9]. |  | - |  |
| measGapPatterns-NRonly Indicates whether the UE supports gap patterns 2, 3 and 11 in LTE standalone when the frequencies to be measured within this measurement gap are all NR frequencies. |  | No |  |
| measGapPatterns-NRonly-ENDC Indicates whether the UE supports gap patterns 2, 3 and 11 in (NG)EN-DC when the frequencies to be measured within this measurement gap are all NR frequencies. |  | No |  |
| maximumCCsRetrieval Indicates whether UE supports reception of requestedMaxCCsDL and requestedMaxCCsUL. |  | - |  |
| maxLayersMIMO-Indication Indicates whether the UE supports the network configuration of maxLayersMIMO. If the UE supports fourLayerTM3-TM4 or intraBandContiguousCC-InfoList or FeatureSetDL-PerCC for MR-DC, UE supports the configuration of maxLayersMIMO for these cases regardless of indicating maxLayersMIMO-Indication. |  | - |  |
| maxLayersSlotOrSubslotPUSCH Indicates the maxiumum number of layers for slot-PUSCH or subslot-PUSCH transmission. |  | Yes |  |
| maxNumberCCs-SPT Indicates the maximum number of supported CCs for short processing time. The UE capability is reported per band combination. The reported number of carriers applies to all the FS-type(s) frameStructureType-SPT-r15 supported in a given band combination. Absence of the field indicates that 0 number of CCs are supported for short processing time. |  | - |  |
| maxNumberDL-CCs, maxNumberUL-CCs Indicates for each TTI combination "sTTI-SupportedCombinations", the maximum number of supported DL CCs/UL CCs for short TTI. Absence of the field indicates that 0 number of CCs are supported for short TTI. |  | - |  |
| maxNumberDecoding Indicates the maximum number of blind decodes in UE-specific search space per UE in one subframe for CA with more than 5 CCs as defined in TS 36.213 [23] which is supported by the UE. The number of blind decodes supported by the UE is the field value * 32. Only values 5 to 32 can be used in this version of the specification. |  | No |  |
| maxNumberEHC-Contexts Defines the maximum number of Ethernet header compression contexts supported by the UE across all DRBs and across UE's EHC compressor and EHC decompressor. The indicated number defines the number of contexts in addition to CID = "all zeros" as specified in Annex A of TS 38.323 [83]. |  | No |  |
| maxNumberROHC-ContextSessions Set to the maximum number of concurrently active ROHC contexts supported by the UE, excluding context sessions that leave all headers uncompressed. cs2 corresponds with 2 (context sessions), cs4 corresponds with 4 and so on. The network ignores this field if the UE supports none of the ROHC profiles in supportedROHC-Profiles. If the UE indicates both maxNumberROHC-ContextSessions and maxNumberROHC-ContextSessions-r14, same value shall be indicated. |  | - |  |
| maxNumberUpdatedCSI-Proc, maxNumberUpdatedCSI-Proc-SPT Indicates the maximum number of CSI processes to be updated across CCs. |  | No |  |
| maxNumberUpdatedCSI-Proc-STTI-Comb77, maxNumberUpdatedCSI-Proc-STTI-Comb27, maxNumberUpdatedCSI-Proc-STTI-Comb22-Set1, maxNumberUpdatedCSI-Proc-STTI-Comb22-Set2 Indicates the maximum number of CSI processes to be updated across CCs. Comb77 is applicable for {slot, slot}, Comb27 for {subslot, slot}, Comb22-Set1 for {subslot, subslot} processing timeline set 1 and the Comb22-Set2 for {subslot, subslot} processing timeline set 2. |  |  |  |
| mbms-AsyncDC Indicates whether the UE in RRC_CONNECTED supports MBMS reception via MRB on a frequency indicated in an MBMSInterestIndication message, where (according to supportedBandCombination) the carriers that are or can be configured as serving cells in the MCG and the SCG are not synchronized. If this field is included, the UE shall also include mbms-SCell and mbms-NonServingCell. The field indicates that the UE supports the feature for xDD if mbms-SCell and mbms-NonServingCell are supported for xDD. |  | - |  |
| mbms-MaxBW Indicates maximum supported bandwidth (T) for MBMS reception, see TS 36.213 [23]. clause 11.1. If the value is set to implicitValue, the corresponding value of T is calculated as specified in TS 36.213 [23], clause 11.1. If the value is set to explicitValue, the actual value of T = explicitValue * 40 MHz. |  | - |  |
| mbms-NonServingCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception via MRB on a frequency indicated in an MBMSInterestIndication message, where (according to supportedBandCombination and to network synchronization properties) a serving cell may be additionally configured. If this field is included, the UE shall also include the mbms-SCell field. |  | Yes |  |
| mbms-ScalingFactor1dot25, mbms-ScalingFactor7dot5 Indicates parameter A(1.25 / A(7.5, i.e., scaling factor for processing one unit of bandwidth corresponding to subcarrier spacing of 1.25 kHz / 7.5 kHz, with respect to one unit of bandwidth corresponding to subcarrier spacing of 15 kHz. See TS 36.213 [23], clause 11.1. This field is included only if subcarrierSpacingMBMS-khz1dot25 / subcarrierSpacingMBMS-khz7dot5 is included. This field shall be included if mbms-MaxBW and subcarrierSpacingMBMS-khz1dot25 / subcarrierSpacingMBMS-khz7dot5 are included. |  | - |  |
| mbms-ScalingFactor0dot37, mbms-ScalingFactor2dot5 Indicates parameter A(0.37 / A(2..5, i.e., scaling factor for processing one unit of bandwidth corresponding to subcarrier spacing of 0.37 kHz / 2.5 kHz, with respect to one unit of bandwidth corresponding to subcarrier spacing of 15 kHz. See TS 36.213 [23], clause 11.1. This field is included only if fembmsMixedCell or fembmsDedicatedCell is included. This field shall be included if subcarrierSpacingMBMS-khz0dot37 / subcarrierSpacingMBMS-khz2dot5 is included for at least one E-UTRA band in mbms-SupportedBandInfoList. |  | - |  |
| mbms-SCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception via MRB on a frequency indicated in an MBMSInterestIndication message, when an SCell is configured on that frequency (regardless of whether the SCell is activated or deactivated). |  | Yes |  |
| mbms-SupportedBandInfoList One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. This list is included only if fembmsMixedCell or fembmsDedicatedCell is included. |  | - |  |
| mcgRLF-RecoveryViaSCG Indicates whether the UE supports recovery from MCG RLF via split SRB1 (if supported) and via SRB3 (if supported). |  | - |  |
| measGapPatterns-NRonly Indicates whether the UE supports gap patterns 2, 3 and 11 in LTE standalone when the frequencies to be measured within this measurement gap are all NR frequencies. |  | No |  |
| measGapPatterns-NRonly-ENDC Indicates whether the UE supports gap patterns 2, 3 and 11 in (NG)EN-DC when the frequencies to be measured within this measurement gap are all NR frequencies. |  | No |  |
| measurementEnhancements This field defines whether UE supports measurement enhancements in high speed scenario (350 km/h) as specified in TS 36.133 [16]. |  | - |  |
| measurementEnhancements2 This field defines whether UE supports measurement enhancements in high speed scenario (up to 500 km/h velocity) as specified in TS 36.133 [16]. |  | - |  |
| measurementEnhancementsSCell This field defines whether UE supports SCell measurement enhancements in high speed scenario (350 km/h) as specified in TS 36.133 [16]. |  | - |  |
| measGapPatterns Indicates whether the UE that supports NR supports gap patterns 4 to 11 in LTE standalone as specified in TS 36.133 [16], and for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC as specified in TS 38.133 [84]. The first/ leftmost bit covers pattern 4, and so on. Value 1 indicates that the UE supports the concerned gap pattern. |  | - |  |
| mfbi-UTRA It indicates if the UE supports the signalling requirements of multiple radio frequency bands in a UTRA FDD cell, as defined in TS 25.307 [65]. |  | - |  |
| MIMO-BeamformedCapabilityList A list of pairs of {k-Max, n-MaxList} values with the nth entry indicating the values that the UE supports for each CSI process in case n CSI processes would be configured. |  | No |  |
| MIMO-CapabilityDL The number of supported layers for spatial multiplexing in DL. The field may be absent for category 0 and category 1 UE in which case the number of supported layers is 1. |  | - |  |
| MIMO-CapabilityUL The number of supported layers for spatial multiplexing in UL. Absence of the field means that the number of supported layers is 1. |  | - |  |
| MIMO-CA-ParametersPerBoBC A set of MIMO parameters provided per band of a band combination. In case a subfield is absent, the concerned capabilities are the same as indicated at the per UE level (i.e. by MIMO-UE-ParametersPerTM). |  | - |  |
| mimo-CBSR-AdvancedCSI Indicates whether UE supports CBSR for advanced CSI reporting with and without amplitude restriction as defined in TS 36.213 [23], clause 7.2. |  |  | Yes |
| min-Proc-TimelineSubslot Minimum processing timeline for subslot operation. The minimum processing timeline can belong to one of two sets of associated processing and maximum TA operation. The sets supported can be different for 1os CRS-based SPDCCH, 2os CRS-based SPDCCH and DMRS-based SPDCCH. The sequence applies to: 1. 1os CRS based SPDCCH 2. 2os CRS based SPDCCH 3. DMRS based SPDCCH |  | - |  |
| modifiedMPR-Behavior Field encoded as a bit map, where at least one bit N is set to "1" if UE supports modified MPR/A-MPR behaviour N, see TS 36.101 [42]. All remaining bits of the field are set to "0". The leading / leftmost bit (bit 0) corresponds to modified MPR/A-MPR behaviour 0, the next bit corresponds to modified MPR/A-MPR behaviour 1 and so on. Absence of this field means that UE does not support any modified MPR/A-MPR behaviour. |  | - |  |
| mpdcch-InLteControlRegionCE-ModeA, mpdcch-InLteControlRegionCE-ModeB Indicates whether UE operating in CE mode A/B supports MPDCCH reception in LTE control channel region as specified in TS 36.211 [21]. |  | Yes |  |
| mpsPriorityIndication Indicates whether the UE supports mpsPriorityIndication on release with redirect. |  | - |  |
| multiACK-CSI-reporting Indicates whether the UE supports multi-cell HARQ ACK and periodic CSI reporting and SR on PUCCH format 3. |  | Yes |  |
| multiBandInfoReport Indicates whether the UE supports the acquisition and reporting of multi band information for reportCGI. |  | - |  |
| multiClusterPUSCH-WithinCC |  | Yes |  |
| multiNS-Pmax Indicates whether the UE supports the mechanisms defined for cells broadcasting NS-PmaxList. |  | - |  |
| multipleCellsMeasExtension Indicates whether the UE supports numberOfTriggeringCells in the report configuration. |  |  | - |
| multipleTimingAdvance Indicates whether the UE supports multiple timing advances for each band combination listed in supportedBandCombination. If the band combination comprised of more than one band entry (i.e., inter-band or intra-band non-contiguous band combination), the field indicates that the same or different timing advances on different band entries are supported. If the band combination comprised of one band entry (i.e., intra-band contiguous band combination), the field indicates that the same or different timing advances across component carriers of the band entry are supported. It is mandatory for UEs to support 2 TAGs for inter frequency DAPS handover. |  | - |  |
| multipleUplinkSPS Indicates whether the UE supports multiple uplink SPS and reporting SPS assistance information. A UE indicating multipleUplinkSPS shall also support V2X communication via Uu, as defined in TS 36.300 [9]. |  | - |  |
| must-CapabilityPerBand Indicates that UE supports MUST, as specified in 36.212 [22], clause 5.3.3.1, on the band in the band combination. |  | - |  |
| must-TM234-UpTo2Tx-r14 Indicates that the UE supports MUST operation for TM2/3/4 using up to 2Tx. |  | - |  |
| must-TM89-UpToOneInterferingLayer-r14 Indicates that the UE supports MUST operation for TM8/9 with assistance information for up to 1 interfering layer. |  | - |  |
| must-TM89-UpToThreeInterferingLayers-r14 Indicates that the UE supports MUST operation for TM8/9 with assistance information for up to 3 interfering layers. |  | - |  |
| must-TM10-UpToOneInterferingLayer-r14 Indicates that the UE supports MUST operation for TM10 with assistance information for up to 1 interfering layer. |  | - |  |
| must-TM10-UpToThreeInterferingLayers-r14 Indicates that the UE supports MUST operation for TM10 with assistance information for up to 3 interfering layers. |  | - |  |
| naics-Capability-List Indicates that UE supports NAICS, i.e. receiving assistance information from serving cell and using it to cancel or suppress interference of neighbouring cell(s) for at least one band combination. If not present, UE does not support NAICS for any band combination. The field numberOfNAICS-CapableCC indicates the number of component carriers where the NAICS processing is supported and the field numberOfAggregatedPRB indicates the maximum aggregated bandwidth across these of component carriers (expressed as a number of PRBs) with the restriction that NAICS is only supported over the full carrier bandwidth. The UE shall indicate the combination of {numberOfNAICS-CapableCC, numberOfNAICS-CapableCC} for every supported numberOfNAICS-CapableCC, e.g. if a UE supports {x CC, y PRBs} and {x-n CC, y-m PRBs} where n>=1 and m>=0, the UE shall indicate both. - For numberOfNAICS-CapableCC = 1, UE signals one value for numberOfAggregatedPRB from the range {50, 75, 100}; - For numberOfNAICS-CapableCC = 2, UE signals one value for numberOfAggregatedPRB from the range {50, 75, 100, 125, 150, 175, 200}; - For numberOfNAICS-CapableCC = 3, UE signals one value for numberOfAggregatedPRB from the range {50, 75, 100, 125, 150, 175, 200, 225, 250, 275, 300}; - For numberOfNAICS-CapableCC = 4, UE signals one value for numberOfAggregatedPRB from the range {50, 100, 150, 200, 250, 300, 350, 400}; - For numberOfNAICS-CapableCC = 5, UE signals one value for numberOfAggregatedPRB from the range {50, 100, 150, 200, 250, 300, 350, 400, 450, 500}. |  | No |  |
| ncsg Indicates whether the UE supports measurement NCSG Pattern Id 0, 1, 2 and 3, as specified in TS 36.133 [16]. If this field is included and the UE supports asynchronous DC, the UE shall support NCSG Pattern Id 0, 1, 2 and 3. If this field is included but the UE does not support asynchronous DC, only NCSG Pattern Id 0 and 1 shall be supported |  | No |  |
| ng-EN-DC Indicates whether the UE supports NGEN-DC. |  | - |  |
| n-MaxList (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of NZP CSI RS ports supported within a CSI process applicable for band combinations for which the concerned capabilities are not signalled. For k-Max values exceeding 1, the UE shall include the field and signal k-Max minus 1 bits. The first bit indicates n-Max2, with value 0 indicating 8 and value 1 indicating 16. The second bit indicates n-Max3, with value 0 indicating 8 and value 1 indicating 16. The third bit indicates n-Max4, with value 0 indicating 8 and value 1 indicating 32. The fourth bit indicates n-Max5, with value 0 indicating 16 and value 1 indicating 32. The fifth bit indicates n-Max6, with value 0 indicating 16 and value 1 indicating 32. The sixt bit indicates n-Max7, with value 0 indicating 16 and value 1 indicating 32. The seventh bit indicates n-Max8, with value 0 indicating 16 and value 1 indicating 64. |  | Yes |  |
| n-MaxList (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode the maximum number of NZP CSI RS ports supported within a CSI process applicable for band the concerned combination. Further details are as indicated for n-MaxList in MIMO-UE-ParametersPerTM. |  | No |  |
| NonContiguousUL-RA-WithinCC-List One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. |  | No |  |
| nonPrecoded (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the UE capabilities concerning non-precoded EBF/ FD-MIMO operation (class A) for band combinations for which the concerned capabilities are not signalled in MIMO-CA-ParametersPerBoBCPerTM, and the FD-MIMO processing capability condition as described in NOTE 8 is satisfied. |  | Yes |  |
| nonPrecoded (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode, the UE capabilities concerning non-precoded EBF/ FD-MIMO operation (class A) applicable for the concerned band combination. |  | - |  |
| nonUniformGap Indicates whether the UE supports measurement non uniform Pattern Id 1, 2, 3 and 4 in LTE standalone as specified in TS 36.133 [16]. |  | No |  |
| noResourceRestrictionForTTIBundling Indicate whether the UE supports TTI bundling operation without resource allocation restriction. |  | No |  |
| nonCSG-SI-Reporting Indicates whether UE will report PLMN list from non-CSG cells. |  | - |  |
| nr-AutonomousGaps-ENDC-FR1 Indicates whether the UE supports, upon configuration of useAutonomousGapsNR by the network, acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell on FR1 using autonomous gaps and reporting the acquired information to the network when it is configured with (NG)EN-DC. |  | Yes |  |
| nr-AutonomousGaps-ENDC-FR2 Indicates whether the UE supports, upon configuration of useAutonomousGapsNR by the network, acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell on FR2 using autonomous gaps and reporting the acquired information to the network when it is configured with (NG)EN-DC. |  | Yes |  |
| nr-AutonomousGaps-FR1 Indicates whether the UE supports, upon configuration of useAutonomousGapsNR by the network, acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell on FR1 using autonomous gaps and reporting the acquired information to the network when it is not configured with (NG)EN-DC. |  | Yes |  |
| nr-AutonomousGaps-FR2 Indicates whether the UE supports, upon configuration of useAutonomousGapsNR by the network, acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell on FR2 using autonomous gaps and reporting the acquired information to the network when it is not configured with (NG)EN-DC. |  | Yes |  |
| nr-HO-ToEN-DC Indicates whether the UE supports inter-RAT handover from NR to EN-DC while NR-DC or NE-DC is not configured. This field is mandatory present if EN-DC is supported. |  | - |  |
| nr-IdleInactiveBeamMeasFR1 Indicates whether the UE supports performing eNB-configured SSB-based beam level RRM measurements for configured NR FR1 carrier(s) in RRC_IDLE and in RRC_INACTIVE as specified in TS 36.306 [5], clause 4.3.6.46. |  | No |  |
| nr-IdleInactiveBeamMeasFR2 Indicates whether the UE supports performing eNB-configured SSB-based beam level RRM measurements for configured NR FR2 carrier(s) in RRC_IDLE and in RRC_INACTIVE as specified in TS 36.306 [5], clause 4.3.6.47. |  | No |  |
| nr-IdleInactiveMeasFR1 Indicates whether UE supports reporting measurements performed on NR FR1 carrier(s) during RRC_IDLE and RRC_INACTIVE. |  | No |  |
| nr-IdleInactiveMeasFR2 Indicates whether UE supports reporting measurements performed on NR FR2 carrier(s) during RRC_IDLE and RRC_INACTIVE. |  | No |  |
| numberOfBlindDecodesUSS Indicates the maximum number of blind decodes in UE specific search space in one subframe for CCs configured with sTTI operation supported by the UE. The number of blind decodes supported by the UE is the field value X*68. Field value ranges from 4 to 32. |  | Yes |  |
| nzp-CSI-RS-AperiodicInfo Indicates whether the UE supports aperiodic NZP CSI-RS transmission for the indicated transmission mode. |  | Yes |  |
| nzp-CSI-RS-PeriodicInfo Indicates whether the UE supports periodic NZP CSI-RS transmission for the indicated transmission mode. |  | Yes |  |
| otdoa-UE-Assisted Indicates whether the UE supports UE-assisted OTDOA positioning, as specified in TS 36.355 [54]. |  | Yes |  |
| outOfOrderDelivery Same as "outOfOrderDelivery" defined in TS 38.306 [87]. |  | No |  |
| outOfSequenceGrantHandling Indicates whether the UE supports PUSCH transmissions with out of sequence UL grants as defined in TS 36.213 [23]. This field can be included only if uplinkLAA is included. |  | - |  |
| overheatingInd Indicates whether the UE supports overheating assistance information. |  | No |  |
| overheatingIndForSCG Indicates whether the UE supports the inclusion of NR SCG reduced configuration in the overheating assistance information. The UE which indicates support of overheatingIndForSCG shall also indicate support of overheatingInd. |  | - |  |
| pdcch-CandidateReductions Indicates whether the UE supports PDCCH candidate reduction on UE specific search space as specified in TS 36.213 [23], clause 9.1.1. |  | No |  |
| pdcp-Duplication Indicates whether the UE supports PDCP duplication. |  | - |  |
| pdcp-SN-Extension Indicates whether the UE supports 15 bit length of PDCP sequence number. |  | - |  |
| pdcp-SN-Extension-18bits Indicates whether the UE supports 18 bit length of PDCP sequence number. |  | - |  |
| pdcp-TransferSplitUL Indicates whether the UE supports PDCP data transfer split in UL for the drb-TypeSplit as specified in TS 36.323 [8]. |  | - |  |
| pdcp-VersionChangeWithoutHO Indicates whether, the UE supports changing the PDCP version of DRBs, from LTE PDCP to NR PDCP and vice versa, with and without handover. A UE supporting PDCP version change shall signal field pdcp-Parameters-v1610. When the field pdcp-VersionChangeWithoutHO is not included and pdcp-Parameters-v1610 is included, it implies the UE supports PDCP version change only with handover. |  | - |  |
| pdsch-CollisionHandling Indicates whether the UE supports PDSCH collision handling as specified in TS 36.213 [23]. |  | No |  |
| pdsch-InLteControlRegionCE-ModeA, pdsch-InLteControlRegionCE-ModeB Indicates whether UE operating in CE mode A/B supports PDSCH reception in LTE control channel region as specified in TS 36.211 [21]. |  | Yes |  |
| pdsch-MultiTB-CE-ModeA, pdsch-MultiTB-CE-ModeB Indicates whether the UE supports multiple TB scheduling in connected mode for PDSCH when operating in CE mode A/B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |  |
| pdsch-RepSubframe Indicates whether the UE supports subframe PDSCH repetition. |  | Yes |  |
| pdsch-RepSlot Indicates whether the UE supports slot PDSCH repetition. |  | Yes |  |
| pdsch-RepSubslot Indicates whether the UE supports subslot PDSCH repetition. This field is only applicable for UEs supporting FDD. |  | - |  |
| pdsch-SlotSubslotPDSCH-Decoding Indicates whether the UE supports decoding of PDSCH and slot-PDSCH/subslot-PDSCH assigned with C-RNTI/SPS C-RNTI in the same subframe for a given carrier. |  | Yes |  |
| perServingCellMeasurementGap Indicates whether the UE supports per serving cell measurement gap indication, as specified in TS 36.133 [16]. |  | - |  |
| phy-TDD-ReConfig-FDD-PCell Indicates whether the UE supports TDD UL/DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI on a FDD PCell, and HARQ feedback according to UL and DL HARQ reference configurations. This bit can only be set to supported only if the UE supports FDD PCell and phy-TDD-ReConfig-TDD-PCell is set to supported. |  | No |  |
| phy-TDD-ReConfig-TDD-PCell Indicates whether the UE supports TDD UL/DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI on a TDD PCell, and HARQ feedback according to UL and DL HARQ reference configurations, and PUCCH format 3. |  | Yes |  |
| pmi-Disabling |  | Yes |  |
| powerClass-14dBm Indicates whether the UE supports power class 14 dBm when operating in CE mode A or B for all the bands that are supported by the UE, as specified in TS 36.101 [42]. |  |  | - |
| powerPrefInd Indicates whether the UE supports power preference indication. |  | No |  |
| powerUCI-SlotPUSCH, powerUCI-SubslotPUSCH Indicates whether the UE supports BPRE derivation based on the actual derived O_CQI. The parameter uplinkPower-CSIPayload configures the UE to derive BPRE based on either the actual value of O_CQI or the largest value of O_CQI across all RI values. If the UE does not support the capability, the UE will derive BPRE based on the largest value of O_CQI across all RI values. |  | Yes |  |
| prach-Enhancements This field defines whether the UE supports random access preambles generated from restricted set type B in high speed scenoario as specified in TS 36.211 [21]. |  | - |  |
| processingTimelineSet Indicates, for each SPDCCH configuration, support for a set of TA values. Each set consists of two different processing timelines and associated maximum TA. Set 1 indicates support for n+4 and n+6 and set 2 indicates support for n+6 and n+8, see TS 36.211 [21], clause 8.1, The minimum processing timeline to use, out of the two options for a given set is configured by parameter proc-Timeline. Support of Set 1 implicitly means support of Set 2. |  | - |  |
| pucch-Format4 Indicates whether the UE supports PUCCH format 4. |  | Yes |  |
| pucch-Format5 Indicates whether the UE supports PUCCH format 5. |  | Yes |  |
| pucch-SCell Indicates whether the UE supports PUCCH on SCell. |  | No |  |
| pur-CP-EPC-CE-ModeA, pur-CP-EPC-CE-ModeB, pur-CP-5GC-CE-ModeA, pur-CP-5GC-CE-ModeB Indicates whether UE operating in CE mode A/B supports CP transmission using PUR when connected to EPC/ 5GC. |  | Yes |  |
| pur-CP-L1Ack Indicates whether UE supports L1 acknowledgement in response to CP transmission using PUR when connected to EPC/ 5GC. |  | Yes |  |
| pur-FrequencyHopping Indicates whether UE supports frequency hopping for transmission using PUR. |  | Yes |  |
| pur-PUSCH-NB-MaxTBS Indicates whether the UE supports 2984 bits max UL TBS in 1.4 MHz for transmission using PUR when operating in CE mode A, as specified in TS 36.212 [22] and TS 36.213 [23]. |  | Yes |  |
| pur-RSRP-Validation Indicates whether UE supports serving cell RSRP for TA validation for transmission using PUR when connected to EPC/ 5GC. |  | Yes |  |
| pur-SubPRB-CE-ModeA, pur-SubPRB-CE-ModeB Indicates whether UE supports subPRB resource allocation for PUSCH for transmission using PUR when operating in CE mode A/B. |  | Yes |  |
| pur-UP-EPC-CE-ModeA, pur-UP-EPC-CE-ModeB, pur-UP-5GC-CE-ModeA, pur-UP-5GC-CE-ModeB Indicates whether UE operating in CE mode A/B supports UP transmission using PUR when connected to EPC/ 5GC. |  | Yes |  |
| pusch-Enhancements Indicates whether the UE supports the PUSCH enhancement mode as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |  |
| pusch-FeedbackMode Indicates whether the UE supports PUSCH feedback mode 3-2. |  | No |  |
| pusch-MultiTB-CE-ModeA, pusch-MultiTB-CE-ModeB Indicates whether the UE supports multiple TB scheduling in connected mode for PUSCH when operating in CE mode A/B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |  |
| pusch-SPS-MaxConfigSlot Indicates the max number of SPS configurations across all cells for slot PUSCH. |  | Yes |  |
| pusch-SPS-MultiConfigSlot Indicates the number of multiple SPS configurations of slot PUSCH for each serving cell. |  | Yes |  |
| pusch-SPS-MaxConfigSubframe Indicates the max number of SPS configurations across all cells for subframe PUSCH. |  | Yes |  |
| pusch-SPS-MultiConfigSubframe Indicates the number of multiple SPS configurations of subframe PUSCH for each serving cell. |  | Yes |  |
| pusch-SPS-MaxConfigSubslot Indicates the max number of SPS configurations across all cells for subslot PUSCH. |  | - |  |
| pusch-SPS-MultiConfigSubslot Indicates the number of multiple SPS configurations of subslot PUSCH for each serving cell. This field is only applicable for UEs supporting FDD. |  | - |  |
| pusch-SPS-SlotRepPCell Indicates whether the UE supports SPS repetition for slot PUSCH for PCell. |  | Yes |  |
| pusch-SPS-SlotRepPSCell Indicates whether the UE supports SPS repetition for slot PUSCH for PSCell. |  | Yes |  |
| pusch-SPS-SlotRepSCell Indicates whether the UE supports SPS repetition for slot PUSCH for serving cells other than SpCell. |  | Yes |  |
| pusch-SPS-SubframeRepPCell Indicates whether the UE supports SPS repetition for subframe PUSCH for PCell. |  | Yes |  |
| pusch-SPS-SubframeRepPSCell Indicates whether the UE supports SPS repetition for subframe PUSCH for PSCell. |  | Yes |  |
| pusch-SPS-SubframeRepSCell Indicates whether the UE supports SPS repetition for subframe PUSCH for serving cells other than SpCell. |  | Yes |  |
| pusch-SPS-SubslotRepPCell Indicates whether the UE supports SPS repetition for subslot PUSCH for PCell. This field is only applicable for UEs supporting FDD. |  | - |  |
| pusch-SPS-SubslotRepPSCell Indicates whether the UE supports SPS repetition for subslot PUSCH for PSCell. This field is only applicable for UEs supporting FDD. |  | - |  |
| pusch-SPS-SubslotRepSCell Indicates whether the UE supports SPS repetition for subslot PUSCH for serving cells other than SpCell. This field is only applicable for UEs supporting FDD. |  | - |  |
| pusch-SRS-PowerControl-SubframeSet Indicates whether the UE supports subframe set dependent UL power control for PUSCH and SRS. This field is only applicable for UEs supporting TDD. |  | Yes |  |
| qcl-CRI-BasedCSI-Reporting Indicates whether the UE supports CRI based CSI feedback for the FeCoMP feature as specified in TS 36.213 [23], clause 7.1.10. |  | - |  |
| qcl-TypeC-Operation The UE uses this field to indicate the support of all of the following three features: QCL Type-C operation for FeCoMP, the capability to support separate PDSCH RE mapping for different PDSCH CWs in non-coherent joint transmission and the capability to support handling new DMRS port to MIMO layer mapping for the CWs, as specified in TS 36.213 [23], clause 7.1.10. |  | - |  |
| qoe-MeasReport Indicates whether the UE supports QoE Measurement Collection for streaming services. |  | - |  |
| qoe-MTSI-MeasReport Indicates whether the UE supports QoE Measurement Collection for MTSI services. |  |  |  |
| rach-Less Indicates whether the UE supports RACH-less handover, and whether the UE which indicates dc-Parameters supports RACH-less SeNB change, as defined in TS 36.300 [9]. |  | - |  |
| rach-Report Indicates whether the UE supports delivery of rach-Report. |  | - |  |
| rai-Support Defines whether the UE supports release assistance indication (RAI) as specified in TS 36.321 [6] for BL UEs. |  | No |  |
| rai-SupportEnh Indicates whether the UE supports 2-bit RAI when connected to EPC as specified in TS 36.321 [6]. |  | - |  |
| rclwi Indicates whether the UE supports RCLWI, i.e. reception of rclwi-Configuration. The UE which supports RLCWI shall also indicate support of interRAT-ParametersWLAN-r13. The UE which supports RCLWI and wlan-IW-RAN-Rules shall also support applying WLAN identifiers received in rclwi-Configuration for the access network selection and traffic steering rules when in RRC_IDLE. |  | - |  |
| recommendedBitRate Indicates whether the UE supports the bit rate recommendation message from the eNB to the UE as specified in TS 36.321 [6], clause 6.1.3.13. |  | No |  |
| recommendedBitRateMultiplier Indicates whether the UE supports the bit rate multiplier for recommended bit rate MAC CE as specified in TS 36.321 [6], clause 6.1.3.13. If this field is included, the UE shall also include the recommendedBitRate field. |  | - |  |
| recommendedBitRateQuery Indicates whether the UE supports the bit rate recommendation query message from the UE to the eNB as specified in TS 36.321 [6], clause 6.1.3.13. If this field is included, the UE shall also include the recommendedBitRate field. |  | No |  |
| reducedCP-Latency Indicates whether the UE supports reduced CP latency. |  | Yes |  |
| reducedIntNonContComb Indicates whether the UE supports receiving requestReducedIntNonContComb that requests the UE to exclude supported intra-band non-contiguous CA band combinations other than included in capability signalling as specified in TS 36.306 [5], clause 4.3.5.21. |  | - |  |
| reducedIntNonContCombRequested Indicates that the UE excluded supported intra-band non-contiguous CA band combinations other than included in capability signalling as specified in TS 36.306 [5,] clause 4.3.5.21. |  | - |  |
| reflectiveQoS Indicates whether the UE supports AS reflective QoS. |  | No |  |
| relWeightTwoLayers/ relWeightFourLayers/ relWeightEightLayers Indicates relative weight of processing FD-MIMO with 2/ 4/ 8 layers with respect to non-FD-MIMO with the same number of layers, see NOTE 8. Value v1 corresponds to relative weight of 1, value v1dot25 corresponds to relative weight of 1.25 and so on. This field can be included only if the UE supports the corresponding number of layers (i.e., 2/ 4/ 8 layers). |  | - |  |
| reportCGI-NR-EN-DC Indicates whether the UE supports Inter-RAT report CGI procedure towards NR cell when it is configured with (NG)EN-DC. |  |  | Yes |
| reportCGI-NR-NoEN-DC Indicates whether the UE supports Inter-RAT report CGI procedure towards NR cell when it is not configured with (NG)EN-DC. |  |  | Yes |
| resumeWithMCG-SCellConfig Indicates whether the UE supports (re-)configuration of E-UTRA MCG SCells. |  |  | - |
| resumeWithSCG-Config Indicates whether the UE supports (re-)configuration of an NR SCG. |  |  | - |
| resumeWithStoredMCG-SCells Indicates whether the UE supports not deleting the stored E-UTRA MCG SCell configuration when initiating the resume procedure. |  |  | - |
| resumeWithStoredSCG Indicates whether the UE supports not deleting the stored NR SCG configuration when initiating the resume procedure. |  |  | - |
| srs-CapabilityPerBandPairList Indicates, for a particular pair of bands, the SRS carrier switching parameters when switching between the band pair to transmit SRS on a PUSCH-less SCell as specified in TS 36.212 [22] and TS 36.213 [23]. If included, the UE shall include a number of entries as indicated in the following, and listed in the same order, as in bandParameterList for the concerned band combination: - For the first band, the UE shall include the same number of entries as in bandParameterList i.e. first entry corresponds to first band in bandParameterList and so on, - For the second band, the UE shall include one entry less i.e. first entry corresponds to the second band in bandParameterList and so on - And so on. |  | - |  |
| requestedBands Indicates the frequency bands requested by E-UTRAN. |  | - |  |
| requestedCCsDL, requestedCCsUL Indicates the maximum number of CCs requested by E-UTRAN. |  | - |  |
| requestedDiffFallbackCombList Indicates the CA band combinations for which report of different UE capabilities is requested by E-UTRAN. |  | - |  |
| rf-RetuningTimeDL Indicates the interruption time on DL reception within a band pair during the RF retuning for switching between the band pair to transmit SRS on a PUSCH-less SCell. n0 represents 0 OFDM symbols, n0dot5 represents 0.5 OFDM symbols, n1 represents 1 OFDM symbol and so on. This field is mandatory present if switching between the band pair is supported. |  | - |  |
| rf-RetuningTimeUL Indicates the interruption time on UL transmission within a band pair during the RF retuning for switching between the band pair to transmit SRS on a PUSCH-less SCell. n0 represents 0 OFDM symbols, n0dot5 represents 0.5 OFDM symbols, n1 represents 1 OFDM symbol and so on. This field is mandatory present if switching between the band pair is supported. |  | - |  |
| rlc-AM-Ooo-Delivery Indicates whether the UE supports out-of-order delivery from RLC to PDCP for RLC AM. |  | - |  |
| rlc-UM-Ooo-Delivery Indicates whether the UE supports out-of-order delivery from RLC to PDCP for RLC UM. |  | - |  |
| rlm-ReportSupport Indicates whether the UE supports RLM event and information reporting. |  | - |  |
| rohc-ContextContinue Same as "continueROHC-Context" defined in TS 38.306 [87]. |  | No |  |
| rohc-ContextMaxSessions Same as "maxNumberROHC-ContextSessions" defined in TS 38.306 [87]. |  | No |  |
| rohc-Profiles Same as "supportedROHC-Profiles" defined in TS 38.306 [87]. |  | No |  |
| rohc-ProfilesUL-Only Same as "uplinkOnlyROHC-Profiles" defined in TS 38.306 [87]. |  | No |  |
| rsrqMeasWideband Indicates whether the UE can perform RSRQ measurements with wider bandwidth. |  | Yes |  |
| rsrq-OnAllSymbols Indicates whether the UE can perform RSRQ measurement on all OFDM symbols and also support the extended RSRQ upper value range from -3dB to 2.5dB in measurement configuration and reporting as specified in TS 36.133 [16]. |  | No |  |
| rs-SINR-Meas Indicates whether the UE can perform RS-SINR measurements in RRC_CONNECTED as specified in TS 36.214 [48]. |  | - |  |
| rssi-AndChannelOccupancyReporting Indicates whether the UE supports performing measurements and reporting of RSSI and channel occupancy. This field can be included only if downlinkLAA is included. |  | - |  |
| sa-NR Indicates whether the UE supports standalone NR as specified in TS 38.331 [82]. |  | No |  |
| scalingFactorTxSidelink, scalingFactorRxSidelink Indicates, for a particular band combination of EUTRA, the scaling facor, as defined in TS 38.306 [87], for the PC5 band combination(s) v2x-SupportedBandCombinationListEUTRA-NR on which the UE supports simultaneous transmission/reception of EUTRA and NR sidelink communication respectively, or simultaneous transmission or reception of EUTRA and joint V2X sidelink communication and NR sidelink communication respectively (as indicated by v2x-SupportedTxBandCombListPerBC-v1630 / v2x-SupportedRxBandCombListPerBC-v1630). The leading / leftmost value corresponds to the first band combination included in v2x-SupportedBandCombinationListEUTRA-NR which is indicated with value 1 by v2x-SupportedTxBandCombListPerBC-v1630 / v2x-SupportedRxBandCombListPerBC-v1630, the next value corresponds to the second band combination included in v2x-SupportedBandCombinationListEUTRA-NR which is indicated with value 1 by v2x-SupportedTxBandCombListPerBC-v1630 / v2x-SupportedRxBandCombListPerBC-v1630 and so on. For each value of ScalingFactorSidelink-r16, value f0p4 indicates the scaling factor 0.4, f0p75 indicates 0.75, and so on. |  | - |  |
| scptm-AsyncDC Indicates whether the UE in RRC_CONNECTED supports MBMS reception via SC-MRB on a frequency indicated in an MBMSInterestIndication message, where (according to supportedBandCombination) the carriers that are or can be configured as serving cells in the MCG and the SCG are not synchronized. If this field is included, the UE shall also include scptm-SCell and scptm-NonServingCell. |  | Yes |  |
| scptm-NonServingCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception via SC-MRB on a frequency indicated in an MBMSInterestIndication message, where (according to supportedBandCombination and to network synchronization properties) a serving cell may be additionally configured. If this field is included, the UE shall also include the scptm-SCell field. |  | Yes |  |
| scptm-Parameters Presence of the field indicates that the UE supports SC-PTM reception as specified in TS 36.306 [5]. |  | Yes |  |
| scptm-SCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception via SC-MRB on a frequency indicated in an MBMSInterestIndication message, when an SCell is configured on that frequency (regardless of whether the SCell is activated or deactivated). |  | Yes |  |
| scptm-ParallelReception Indicates whether the UE in RRC_CONNECTED supports parallel reception in the same subframe of DL-SCH transport blocks transmitted using C-RNTI/Semi-Persistent Scheduling C-RNTI and using SC-RNTI/G-RNTI as specified in TS 36.306 [5]. |  | Yes |  |
| secondSlotStartingPosition Indicates whether the UE supports reception of subframes with second slot starting position as described in TS 36.211 [21] and TS 36.213 [23]. This field can be included only if downlinkLAA is included. |  | - |  |
| semiOL Indicates whether the UE supports semi-open-loop transmission for the indicated transmission mode. |  | Yes |  |
| semiStaticCFI Indicates whether the UE supports the semi-static configuration of CFI for subframe/slot/sub-slot operation. |  | Yes |  |
| semiStaticCFI-Pattern Indicates whether the UE supports the semi-static configuration of CFI pattern for subframe/slot/sub-slot operation. This field is only applicable for UEs supporting TDD. |  | - |  |
| shortCQI-ForSCellActivation Indicates whether the UE supports additional CQI reporting periodicity after SCell activation. |  | Yes |  |
| shortMeasurementGap Indicates whether the UE supports shorter measurement gap length (i.e. gp2 and gp3) in LTE standalone as specified in TS 36.133 [16], and for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC as specified in TS38.133 [84]. |  | No |  |
| shortSPS-IntervalFDD Indicates whether the UE supports uplink SPS intervals shorter than 10 subframes in FDD mode. |  | - |  |
| shortSPS-IntervalTDD Indicates whether the UE supports uplink SPS intervals shorter than 10 subframes in TDD mode. |  | - |  |
| simultaneousPUCCH-PUSCH Indicates whether the UE supports simultaneous transmission of PUSCH/PUCCH and SlotOrSubslotPUSCH/SPUCCH (if supported). |  | Yes |  |
| simultaneousRx-Tx Indicates whether the UE supports simultaneous reception and transmission on different bands for each band combination listed in supportedBandCombination. This field is only applicable for inter-band TDD band combinations. A UE indicating support of simultaneousRx-Tx and dc-Support-r12 shall support different UL/DL configurations between PCell and PSCell. |  | - |  |
| simultaneousTx-DifferentTx-Duration Indicates whether the UE supports simultaneous transmission of different transmission durations over different carriers. The different transmission durations can be of subframe, slot or subslot duration. |  | - |  |
| skipFallbackCombinations Indicates whether UE supports receiving requestSkipFallbackComb that requests UE to exclude fallback band combinations from capability signalling. |  | - |  |
| skipFallbackCombRequested Indicates whether requestSkipFallbackComb is requested by E-UTRAN. |  | - |  |
| skipMonitoringDCI-Format0-1A Indicates whether UE supports blind decoding reduction on UE specific search space by not monitoring DCI Format 0 and 1A as specified in TS 36.213 [23], clause 9.1.1. |  | No |  |
| skipSubframeProcessing This fields defines whether the UE supports aborting reception of PDSCH if the UE receives slot-PDSCH/subslot-PDSCH during an ongoing PDSCH reception and instead starts receiving the slot-PDSCH/subslot-PDSCH, as well as whether the UE supports aborting a PUSCH transmission if the UE gets a grant for a slot-PUSCH/ subslot-PUSCH transmission that overlaps with a grant received for a PUSCH transmission. The capability indicates the number of subframes that the UE may drop prior to the subframe in which it prioritizes the processing of slot/subslot PDSCH/PUSCH as described in TS 36.213 [23], clauses 7.1 and 8.0. Separate capability for UL and DL and per sTTI length in each direction: skipProcessingDL-Slot, skipProcessingDL-Subslot, skipProcessingUL-Slot and skipProcessingUL-Subslot. |  | - |  |
| skipUplinkDynamic Indicates whether the UE supports skipping of UL transmission for an uplink grant indicated on PDCCH if no data is available for transmission as described in TS 36.321 [6]. |  | - |  |
| skipUplinkSPS Indicates whether the UE supports skipping of UL transmission for a configured uplink grant if no data is available for transmission as described in TS 36.321 [6]. |  | - |  |
| sl-64QAM-Rx Indicates whether the UE supports 64QAM for the reception of V2X sidelink communication. |  |  | - |
| sl-64QAM-Tx Indicates whether the UE supports 64QAM for the transmission of V2X sidelink communication. |  |  | - |
| sl-CongestionControl Indicates whether the UE supports Channel Busy Ratio measurement and reporting of Channel Busy Ratio measurement results to eNB for V2X sidelink communication. |  | - |  |
| sl-LowT2min Indicates whether the UE supports 10ms as minimum value of T2 for resource selection procedure of V2X sidelink communication. |  | - |  |
| sl-ParameterNR Includes the SidelinkParametersNR IE as specified in TS 38.331 [82]. The field includes the sidelink capability for NR-PC5, where multipleSR-ConfigurationsSidelink and logicalChannelSR-DelayTimerSidelink is not applicable. |  | - |  |
| sl-RateMatchingTBSScaling Indicates whether the UE supports rate matching and TBS scalling for V2X sidelink communication. |  | - |  |
| slotPDSCH-TxDiv-TM8 Indicates whether the UE supports TX diversity transmission using ports 7 and 8 for TM8 for slot PDSCH. |  | - |  |
| slotPDSCH-TxDiv-TM9and10 Indicates whether the UE supports TX diversity transmission using ports 7 and 8 for TM9/10 for slot PDSCH. |  | Yes |  |
| slotSymbolResourceResvDL-CE-ModeA, slotSymbolResourceResvDL-CE-ModeB, slotSymbolResourceResvUL-CE-ModeA, slotSymbolResourceResvUL-CE-ModeB Indicates whether the UE supports slot/symbol-level time-domain resource reservation in downlink/uplink when operating in CE mode A/B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |  |
| slss-SupportedTxFreq Indicates whether the UE supports the SLSS transmission on single carrier or on multiple carriers in the case of sidelink carrier aggregation. |  |  | - |
| slss-TxRx Indicates whether the UE supports SLSS/PSBCH transmission and reception in UE autonomous resource selection mode and eNB scheduled mode in a band for V2X sidelink communication. |  | - |  |
| sl-TxDiversity Indicates whether the UE supports transmit diversity for V2X sidelink communication. See TS 36.101 [42]. |  |  | - |
| sn-SizeLo Same as "shortSN" defined in TS 38.306 [87]. |  | No |  |
| spatialBundling-HARQ-ACK Indicates whether UE supports HARQ-ACK spatial bundling on PUCCH or PUSCH as specified in TS 36.213 [23], clauses 7.3.1 and 7.3.2. |  | No |  |
| spdcch-differentRS-types Indicates whether the UE supports monitoring of sPDCCH on RB sets with different RS types within a TTI. |  | Yes |  |
| spdcch-Reuse Indicates whether the UE supports L1 based SPDCCH reuse. |  | Yes |  |
| sps-CyclicShift Indicates whether the UE supports RRC configuration of cyclic shift for DMRS for UL SPS using 1ms TTI. |  | Yes |  |
| sps-ServingCell Indicates whether the UE supports multiple UL/DL SPS configurations simultaneously active on different serving cells as specified in TS 36.321 [6]. |  | - |  |
| sps-STTI Indicates whether the UE supports SPS in DL and/or UL for slot or subslot based PDSCH and PUSCH, respectively. |  | Yes |  |
| srs-DCI7-TriggeringFS2 Indicates whether the UE supports SRS triggerring via DCI format 7 for FS2. |  | - |  |
| srs-Enhancements Indicates whether the UE supports SRS enhancements. |  | Yes |  |
| srs-EnhancementsTDD Indicates whether the UE supports TDD specific SRS enhancements. |  | Yes |  |
| srs-FlexibleTiming Indicates whether the UE supports configuration of soundingRS-FlexibleTiming-r14 for the corresponding band pair. For a TDD-TDD band pair, UE shall include at least one of srs-FlexibleTiming and/or srs-HARQ-ReferenceConfig when rf-RetuningTimeDL or rf-RetuningTimeUL corresponding to the band pair is larger than 1 OFDM symbol. |  | - |  |
| srs-HARQ-ReferenceConfig Indicates whether the UE supports configuration of harq-ReferenceConfig-r14 for the corresponding band pair. For a TDD-TDD band pair, UE shall include at least one of srs-FlexibleTiming and/or srs-HARQ-ReferenceConfig when rf-RetuningTimeDL or rf-RetuningTimeUL corresponding to the band pair is larger than 1 OFDM symbol. |  | - |  |
| srs-MaxSimultaneousCCs Indicates the maximum number of simultaneously configurable target CCs for SRS switching (i.e., CCs for which srs-SwitchFromServCellIndex is configured) supported by the UE. |  | - |  |
| srs-UpPTS-6sym Indicates whether the UE supports up to 6-symbol SRS in UpPTS. |  | - |  |
| srvcc-FromUTRA-FDD-ToGERAN Indicates whether UE supports SRVCC handover from UTRA FDD PS HS to GERAN CS. |  | - |  |
| srvcc-FromUTRA-FDD-ToUTRA-FDD Indicates whether UE supports SRVCC handover from UTRA FDD PS HS to UTRA FDD CS. |  | - |  |
| srvcc-FromUTRA-TDD128-ToGERAN Indicates whether UE supports SRVCC handover from UTRA TDD 1.28Mcps PS HS to GERAN CS. |  | - |  |
| srvcc-FromUTRA-TDD128-ToUTRA-TDD128 Indicates whether UE supports SRVCC handover from UTRA TDD 1.28Mcps PS HS to UTRA TDD 1.28Mcps CS. |  | - |  |
| ss-CCH-InterfHandl Indicates whether the UE supports synchronisation signal and common channel interference handling. |  | Yes |  |
| ss-SINR-Meas-NR-FR1, ss-SINR-Meas-NR-FR2 Indicates whether the UE can perform NR SS-SINR measurement for a frequency range (i.e. FR1 or FR2) as specified in TS 38.215 [89]. |  | - |  |
| ssp10-TDD-Only Indicates the UE supports special subframe configuration 10 when operating only in TDD carriers (i.e., not in TDD/FDD CA or TDD/FS3 CA). A UE including this field shall not include tdd-SpecialSubframe-r14. |  | - |  |
| standaloneGNSS-Location Indicates whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements. |  | - |  |
| sTTI-SPT-Supported Indicates whether the UE supports the features STTI and/or SPT. If the UE supports STTI and/or SPT features, the UE shall report the field sTTI-SPT-Supported set to supported in capability signalling, irrespective of whether requestSTTI-SPT-Capability field is present or not. |  | - |  |
| sTTI-FD-MIMO-Coexistence Indicates whether the UE supports CSI feedback for more than 8 NZP CSI-RS ports on subframe based PUSCH in any serving cell and supporting STTI in any serving cell. |  | - |  |
| sTTI-SupportedCombinations Indicates the different combinations of short TTI lengths, see field description for dl-STTI-Length and ul-STTI-Length, that the UE supports in a single PUCCH group or in two PUCCH groups. A short TTI length combination is reported for DL first followed by UL. In case of two PUCCH groups the support for the primary PUCCH group is indicated first. |  | - |  |
| subcarrierPuncturingCE-ModeA, subcarrierPuncturingCE-ModeB Indicates whether the UE supports subcarrier puncturing in downlink when operating in CE mode A/B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |  |
| subcarrierSpacingMBMS-khz7dot5, subcarrierSpacingMBMS-khz1dot25 Indicates the supported subcarrier spacings for MBSFN subframes in addition to 15 kHz subcarrier spacing. subcarrierSpacingMBMS-khz1dot25 and subcarrierSpacingMBMS-khz7dot5 indicates that the UE supports 1.25 and 7.5 kHz respectively for MBSFN subframes as described in TS 36.211 [21], clause 6.12. This field is included only if fembmsMixedCell or fembmsDedicatedCell is included. |  | - |  |
| subcarrierSpacingMBMS-khz2dot5, subcarrierSpacingMBMS-khz0dot37 Presence of this field indicates the supported subcarrier spacings of 2.5kHz / 0.37kHz for MBSFN subframes in addition to 15 kHz subcarrier spacing when operating on the E-UTRA band given by the entry in mbms-SupportedBandInfoList as described in TS 36.211 [21], clause 6.12. |  | - |  |
| subframeResourceResvDL-CE-ModeA, subframeResourceResvDL-CE-ModeB, subframeResourceResvUL-CE-ModeA, subframeResourceResvUL-CE-ModeB Indicates whether the UE supports Subframe-level time-domain resource reservation in downlink/uplink when operating in CE mode A/B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |  |
| subslotPDSCH-TxDiv-TM9and10 Indicates whether the UE supports TX diversity transmission using ports 7 and 8 for TM9/10 for subslot PDSCH. |  | Yes |  |
| supportedBandCombination Includes the supported CA band combinations, if any, and may include all the supported non-CA bands. |  | - |  |
| supportedBandCombinationAdd-r11 Includes additional supported CA band combinations in case maximum number of CA band combinations of supportedBandCombination is exceeded. |  | - |  |
| SupportedBandCombinationAdd-v11d0, SupportedBandCombinationAdd-v1250, SupportedBandCombinationAdd-v1270, SupportedBandCombinationAdd-v1320, SupportedBandCombinationAdd-v1380, SupportedBandCombinationAdd-v1390, SupportedBandCombinationAdd-v1430, SupportedBandCombinationAdd-v1450, SupportedBandCombinationAdd-v1470, SupportedBandCombinationAdd-v14b0, SupportedBandCombinationAdd-v1530, SupportedBandCombinationAdd-v1630 If included, the UE shall include the same number of entries, and listed in the same order, as in SupportedBandCombinationAdd-r11. |  | - |  |
| SupportedBandCombinationAdd-v1610 If included, the UE shall include the same number of entries, and listed in the same order, as in SupportedBandCombinationAdd-r11. If absent, network assumes gap is required when measurement is performed on any NR bands while UE is served by cell(s) belongs to an E-UTRA CA band combinations listed in SupportedBandCombinationAdd-r11 except for the FR2 inter-RAT measurement which depends on the support of independentGapConfig. |  | - |  |
| SupportedBandCombinationExt, SupportedBandCombination-v1090, SupportedBandCombination-v10i0, SupportedBandCombination-v1130, SupportedBandCombination-v1250, SupportedBandCombination-v1270, SupportedBandCombination-v1320, SupportedBandCombination-v1380, SupportedBandCombination-v1390, SupportedBandCombination-v1430, SupportedBandCombination-v1450, SupportedBandCombination-v1470, SupportedBandCombination-v14b0, SupportedBandCombination-v1530, SupportedBandCombination-v1630 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandCombination-r10. |  | - |  |
| SupportedBandCombination-v1610 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandCombination-r10. If absent, network assumes gap is required when measurement is performed on any NR bands while UE is served by cell(s) belongs to an E-UTRA CA band combinations listed in supportedBandCombination-r10 except for the FR2 inter-RAT measurement which depends on the support of independentGapConfig. |  | - |  |
| supportedBandCombinationReduced Includes the supported CA band combinations, and may include the fallback CA combinations specified in TS 36.101 [42], clause 4.3A. This field also indicates whether the UE supports reception of requestReducedFormat. |  | - |  |
| SupportedBandCombinationReduced-v1320, SupportedBandCombinationReduced-v1380, SupportedBandCombinationReduced-v1390, SupportedBandCombinationReduced-v1430, SupportedBandCombinationReduced-v1450, SupportedBandCombinationReduced-v1470, SupportedBandCombinationReduced-v14b0, SupportedBandCombinationReduced-v1530, SupportedBandCombinationReduced-v1630 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandCombinationReduced-r13. |  | - |  |
| SupportedBandCombinationReduced-v1610 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandCombinationReduced-r13. If absent, network assumes gap is required when measurement is performed on any NR bands while UE is served by cell(s) belongs to an E-UTRA CA band combinations listed in supportedBandCombinationReduced-r13 except for the FR2 inter-RAT measurement which depends on the support of independentGapConfig. |  | - |  |
| SupportedBandGERAN GERAN band as defined in TS 45.005 [20]. |  | No |  |
| SupportedBandList1XRTT One entry corresponding to each supported CDMA2000 1xRTT band class. |  | - |  |
| SupportedBandListEUTRA Includes the supported E-UTRA bands. This field shall include all bands which are indicated in BandCombinationParameters. |  | - |  |
| SupportedBandListEUTRA-v9e0, SupportedBandListEUTRA-v1250, SupportedBandListEUTRA-v1310, SupportedBandListEUTRA-v1320 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandListEUTRA (i.e. without suffix). |  | - |  |
| SupportedBandListGERAN |  | No |  |
| SupportedBandListHRPD One entry corresponding to each supported CDMA2000 HRPD band class. |  | - |  |
| SupportedBandListNR-SA Includes the NR bands supported by the UE in NR-SA (for handover and redirection). The field is included in case the UE supports NR SA as specified in TS 38.331 [32] and not otherwise. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [89]. |  | No |  |
| supportedBandListEN-DC Includes the NR bands supported by the UE in (NG)EN-DC. The field is included in case the parameter en-DC or ng-EN-DC is present and set to supported and not otherwise. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [89]. |  | - |  |
| supportedBandListWLAN Indicates the supported WLAN bands by the UE. |  | - |  |
| SupportedBandUTRA-FDD UTRA band as defined in TS 25.101 [17]. |  | - |  |
| SupportedBandUTRA-TDD128 UTRA band as defined in TS 25.102 [18]. |  | - |  |
| SupportedBandUTRA-TDD384 UTRA band as defined in TS 25.102 [18]. |  | - |  |
| SupportedBandUTRA-TDD768 UTRA band as defined in TS 25.102 [18]. |  | - |  |
| supportedBandwidthCombinationSet The supportedBandwidthCombinationSet indicated for a band combination is applicable to all bandwidth classes indicated by the UE in this band combination. Field encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination, see 36.101 [42]. The leading / leftmost bit (bit 0) corresponds to the Bandwidth Combination Set 0, the next bit corresponds to the Bandwidth Combination Set 1 and so on. The UE shall neither include the field for a non-CA band combination, nor for a CA band combination for which the UE only supports Bandwidth Combination Set 0. |  | - |  |
| supportedCellGrouping This field indicates for which mapping of serving cells to cell groups (i.e. MCG or SCG) the UE supports asynchronous DC. This field is only present for a band combination with more than two but less than six band entries where the UE supports asynchronous DC. If this field is not present but asynchronous operation is supported, the UE supports all possible mappings of serving cells to cell groups for the band combination. The bitmap size is selected based on the number of entries in the combinations, i.e., in case of three entries, the bitmap corresponding to threeEntries is selected and so on. A bit in the bit string set to 1 indicates that the UE supports asynchronous DC for the cell grouping option represented by the concerned bit position. Each bit position represents a different cell grouping option, as illustrated by a table, see NOTE 5. A cell grouping option is represented by a number of bits, each representing a particular band entry in the band combination with the left-most bit referring to the band listed first in the band combination, etc. Value 0 indicates that the carriers of the corresponding band entry are mapped to a first cell group, while value 1 indicates that the carriers of the corresponding band entry are mapped to a second cell group. It is noted that the mapping table does not include entries with all bits set to the same value (0 or 1) as this does not represent a DC scenario (i.e. indicating that the UE supports that all carriers of the corresponding band entry are in one cell group). |  | - |  |
| supportedCSI-Proc, sTTI-SupportedCSI-Proc Indicates the maximum number of CSI processes supported on a component carrier within a band. Value n1 corresponds to 1 CSI process, value n3 corresponds to 3 CSI processes, and value n4 corresponds to 4 CSI processes. If this field is included, the UE shall include the same number of entries listed in the same order as in BandParameters/STTI-SPT-BandParameters. If the UE supports at least 1 CSI process on any component carrier, then the UE shall include this field in all bands in all band combinations. |  | - |  |
| supportedCSI-Proc (in FeatureSetDL-PerCC) In MR-DC, indicates the number of CSI processes for the component carrier in the corresponding bandwidth class. If the UE supports at least 1 CSI process, then the UE shall include this field. |  | - |  |
| supportedMIMO-CapabilityDL-MRDC (in FeatureSetDL-PerCC) In MR-DC, indicates the maximum number of supported layers in TM9/10 for the component carrier in the corresponding bandwidth class. |  | - |  |
| supportedNAICS-2CRS-AP If included, the UE supports NAICS for the band combination. The UE shall include a bitmap of the same length, and in the same order, as in naics-Capability-List, to indicate 2 CRS AP NAICS capability of the band combination. The first/ leftmost bit points to the first entry of naics-Capability-List, the second bit points to the second entry of naics-Capability-List, and so on. For band combinations with a single component carrier, UE is only allowed to indicate {numberOfNAICS-CapableCC, numberOfAggregatedPRB} = {1, 100} if NAICS is supported. |  | - |  |
| supportedOperatorDic Indicates whether the UE supports operator defined dictionary. If UE supports operator defined dictionary, the UE shall report versionOfDictionary and associatedPLMN-ID of the stored operator defined dictionary. This parameter is not required to be present if the UE is in VPLMN. In this release of the specification, UE can only support one operator defined dictionary. The associatedPLMN-ID is only associated to the operator defined dictionary which has no relationship with UE's HPLMN ID. |  | - |  |
| supportRohcContextContinue Indicates whether the UE supports ROHC context continuation operation where the UE does not reset the current ROHC context upon handover. |  | - |  |
| supportedROHC-Profiles Indicates the ROHC profiles that UE supports in both uplink and downlink. |  | - |  |
| supportedUplinkOnlyROHC-Profiles Indicates the ROHC profiles that UE supports in uplink and not in downlink, see TS 36.323 [8] |  | - |  |
| supportedStandardDic Indicates whether the UE supports standard dictionary for SIP and SDP as specified in TS 36.323 [8]. |  | - |  |
| supportedUDC Indicates whether the UE supports UL data compression, see TS 36.323 [8]. |  | - |  |
| tdd-SpecialSubframe Indicates whether the UE supports TDD special subframe defined in TS 36.211 [21]. A UE shall indicate tdd-SpecialSubframe-r11 if it supports the TDD special subframes ssp7 and ssp9. A UE shall indicate tdd-SpecialSubframe-r14 if it supports the TDD special subframe ssp10, except when ssp10-TDD-Only-r14 is included. |  | Yes |  |
| tdd-FDD-CA-PCellDuplex The presence of this field indicates that the UE supports TDD/FDD CA in any supported band combination including at least one FDD band with bandParametersUL and at least one TDD band with bandParametersUL. The first bit is set to "1" if UE supports the TDD PCell. The second bit is set to "1" if UE supports FDD PCell. This field is included only if the UE supports band combination including at least one FDD band with bandParametersUL and at least one TDD band with bandParametersUL. If this field is included, the UE shall set at least one of the bits as "1". If this field is included with DC, then it is applicable within a CG, and the presence of this field indicates the capability of the UE to support TDD/FDD CA with at least one FDD band and at least one TDD band in the same CG, with the value indicating the support for TDD/FDD PCell (PSCell). |  | No |  |
| tdd-TTI-Bundling The presence of this field indicates whether the UE supporting TDD special subframe configuration 10 also supports TTI bundling for TDD configuration 2 and 3 when PUSCH transimission in UpPTS is configured, see TS 36.213 [23], clause 8.0. If this field is present, the tdd-SpecialSubframe-r14 or ssp10-TDD-Only-r14 shall be present. |  | Yes |  |
| timeReferenceProvision Indicates whether the UE supports provision of time reference in DLInformationTransfer message. |  | - |  |
| timeSeparationSlot2, timeSeparationSlot4 Indicates whether the UE supports time staggering length of 2 slots (MBSFN reference signal pattern type 2) / 4 slots (MBSFN reference signal pattern type 1) for MBSFN-RS associated with PMCH with subcarrier spacing of 0.37 kHz for MBSFN subframes when operating on the E-UTRA band given by the entry in mbms-SupportedBandInfoList as described in TS 36.211 [21], clause 6.10.2.2.4. |  | - |  |
| timerT312 Indicates whether the UE supports T312. |  | No |  |
| tm5-FDD Indicates whether the UE supports the PDSCH transmission mode 5 in FDD. | - |  |  |
| tm5-TDD Indicates whether the UE supports the PDSCH transmission mode 5 in TDD. | - |  |  |
| tm6-CE-ModeA Indicates whether the UE supports tm6 operation in CE mode A, see TS 36.213 [23], clause 7.2.3. This field can be included only if ce-ModeA is included. |  | Yes |  |
| tm8-slotPDSCH Indicates whether the UE supports configuration and decoding of TM8 for slot PDSCH in TDD. |  | - |  |
| tm9-CE-ModeA Indicates whether the UE supports tm9 operation in CE mode A, see TS 36.213 [23], clause 7.2.3. This field can be included only if ce-ModeA is included. |  | Yes |  |
| tm9-CE-ModeB Indicates whether the UE supports tm9 operation in CE mode B, see TS 36.213 [23], clause 7.2.3. This field can be included only if ce-ModeB is included. |  | Yes |  |
| tm9-LAA Indicates whether the UE supports tm9 operation on LAA cell(s). This field can be included only if downlinkLAA is included. |  | - |  |
| tm9-slotSubslot Indicates whether the UE supports configuration and decoding of TM9 for slot and/or subslot PDSCH for non-MBSFN. |  | Yes |  |
| tm9-slotSubslotMBSFN Indicates whether the UE supports configuration and decoding of TM9 for slot and/or subslot PDSCH for MBSFN. |  | Yes |  |
| tm9-With-8Tx-FDD Indicates whether the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports for FDD when not operating in CE mode. |  | Yes |  |
| tm10-LAA Indicates whether the UE supports tm10 operation on LAA cell(s). This field can be included only if downlinkLAA is included. |  | - |  |
| tm10-slotSubslot Indicates whether the UE supports configuration and decoding of TM10 for slot and/or subslot PDSCH for non-MBSFN. |  | Yes |  |
| tm10-slotSubslotMBSFN Indicates whether the UE supports configuration and decoding of TM10 for slot and/or subslot PDSCH for MBSFN. |  | Yes |  |
| totalWeightedLayers Indicates total number of weighted layers the UE can process for FD-MIMO. See NOTE 8. |  | - |  |
| twoAntennaPortsForPUCCH |  | No |  |
| twoStepSchedulingTimingInfo Presence of this field indicates that the UE supports uplink scheduling using PUSCH trigger A and PUSCH trigger B (as defined in TS 36.213 [23]). This field also indicates the timing between the PUSCH trigger B and the earliest time the UE supports performing the associated UL transmission. For reception of PUSCH trigger B in subframe N, value nPlus1 indicates that the UE supports performing the UL transmission in subframe N+1, value nPlus2 indicates that the UE supports performing the UL transmission in subframe N+2, and so on. This field can be included only if uplinkLAA is included. |  | - |  |
| txAntennaSwitchDL, txAntennaSwitchUL The presence of txAntennaSwitchUL indicates the UE supports transmit antenna selection for this UL band in the band combination as described in TS 36.213 [23], clauses 8.2 and 8.7. The field txAntennaSwitchDL indicates the entry number of the first-listed band with UL in the band combination that affects this DL. The field txAntennaSwitchUL indicates the entry number of the first-listed band with UL in the band combination that switches together with this UL. Value 1 means first entry, value 2 means second entry and so on. All DL and UL that switch together indicate the same entry number. For the case of carrier switching, the antenna switching capability for the target carrier configuration is indicated as follows: For UE configured with a set of component carriers belonging to a band combination Cbaseline = {b1(1),…,bx(1),…,by(0),…}, where "1/0" denotes whether the corresponding band has an uplink, if a component carrier in bx is to be switched to a component carrier in by (according to srs-SwitchFromServCellIndex), the antenna switching capability is derived based on band combination Ctarget = {b1(1),…,bx(0),…,by(1),…}. |  | - |  |
| txDiv-PUCCH1b-ChSelect Indicates whether the UE supports transmit diversity for PUCCH format 1b with channel selection. |  | Yes |  |
| txDiv-SPUCCH Indicates whether the UE supports Tx diversity on SPUCCH format 1/1a/1b/3. |  | Yes |  |
| tx-Sidelink, rx-Sidelink Indicates that the UE supports sidelink transmission/reception on the band in the band combination. For NR sidelink transmission, tx-Sidelink is only applicable if the UE supports at least one of sl-TransmissionMode1-r16 and sl-TransmissionMode2-r16 on the band as specified in TS 38.331 [82]. For NR sidelink reception, rx-Sidelink is only applicable if the UE supports sl-Reception-r16 on the band as specified in TS 38.331 [82]. |  | - |  |
| uci-PUSCH-Ext Indicates whether the UE supports an extension of UCI delivering more than 22 HARQ-ACK bits on PUSCH as specified in TS 36.212 [22], clause 5.2.2.6 and TS 36.213 [23], clause 8.6.3. |  | No |  |
| ue-AutonomousWithFullSensing Indicates whether the UE supports transmitting PSCCH/PSSCH using UE autonomous resource selection mode with full sensing (i.e., continuous channel monitoring) for V2X sidelink communication and the UE supports maximum transmit power associated with Power class 3 V2X UE, see TS 36.101 [42]. |  | - |  |
| ue-AutonomousWithPartialSensing Indicates whether the UE supports transmitting PSCCH/PSSCH using UE autonomous resource selection mode with partial sensing (i.e., channel monitoring in a limited set of subframes) for V2X sidelink communication and the UE supports maximum transmit power associated with Power class 3 V2X UE, see TS 36.101 [42]. |  | - |  |
| ue-Category UE category as defined in TS 36.306 [5]. Set to values 1 to 12 in this version of the specification. |  | - |  |
| ue-CategoryDL UE DL category as defined in TS 36.306 [5]. Value n17 corresponds to UE category 17, value m1 corresponds to UE category M1, value oneBis corresponds to UE category 1bis, value m2 corresponds to UE category M2. For ASN.1 compatibility, a UE indicating DL category 0, m1 or m2 shall also indicate any of the categories (1..5) in ue-Category (without suffix), which is ignored by the eNB, a UE indicating UE category oneBis shall also indicate UE category 1 in ue-Category (without suffix), and a UE indicating UE category m2 shall also indicate UE category m1. The field ue-CategoryDL is set to values 0, m1, oneBis, m2, 4, 6, 7, 9 to 16, n17, 18, 19, 20, 21, 22, 23, 24, 25, 26 in this version of the specification. |  | - |  |
| ue-CategorySL-C-TX UE SL category for V2X transmission as defined in TS 36.306 [5]. Set to values 1 to 5 in this version of the specification. |  |  | - |
| ue-CategorySL-C-RX UE SL category for V2X reception as defined in TS 36.306 [5]. Set to values 1 to 4 in this version of the specification. |  |  | - |
| ue-CategoryUL UE UL category as defined in TS 36.306 [5]. Value n14 corresponds to UE category 14, value n16 corresponds to UE category 16 and so on. Value m1 corresponds to UE category M1, value m2 corresponds to UE category M2, value oneBis corresponds to UE category 1bis. The field ue-CategoryUL is set to values m1, m2, 0, oneBis, 3, 5, 7, 8, 13, n14, 15, n16 to n21 or 22 to 26 in this version of the specification. |  | - |  |
| ue-CA-PowerClass-N Indicates whether the UE supports UE power class N in the E-UTRA band combination, see TS 36.101 [42] and TS 36.307 [78]. If ue-CA-PowerClass-N is not included, UE supports the default UE power class in the E-UTRA band combination, see TS 36.101 [42]. |  | - |  |
| ue-CE-NeedULGaps Indicates whether the UE needs uplink gaps during continuous uplink transmission in FDD as specified in TS 36.211 [21] and TS 36.306 [5]. |  | - |  |
| ue-PowerClass-N, ue-PowerClass-5 Indicates whether the UE supports UE power class 1, 2, 4 or 5 in the E-UTRA band, see TS 36.101 [42] and TS 36.307 [79]. UE includes either ue-PowerClass-N or ue-PowerClass-5. If neither ue-PowerClass-N nor ue-PowerClass-5 is included, UE supports the default UE power class in the E-UTRA band, see TS 36.101 [42]. |  | - |  |
| ue-Rx-TxTimeDiffMeasurements Indicates whether the UE supports Rx - Tx time difference measurements. |  | No |  |
| ue-SpecificRefSigsSupported |  | No |  |
| ue-SSTD-Meas Indicates whether the UE supports SSTD measurements between the PCell and the PSCell as specified in TS 36.214 [48] and TS 36.133 [16]. |  | - |  |
| ue-TxAntennaSelectionSupported Except for the supported band combinations for which bandParameterList-v1380 is included, TRUE indicates that the UE is capable of supporting UE transmit antenna selection such that all the supported bands in the band combination are affected by transmit antenna switching, as described in TS 36.213 [23], clause 8.7. E-UTRAN ignores this field for band combinations for which bandParameterList-v1380 is included. |  | Yes |  |
| ue-TxAntennaSelection-SRS-1T4R Indicates whether the UE supports selecting one antenna among four antennas to transmit SRS for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |  |
| ue-TxAntennaSelection-SRS-2T4R-2Pairs Indicates whether the UE supports selecting one antenna pair between two antenna pairs to transmit SRS simultaneously for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |  |
| ue-TxAntennaSelection-SRS-2T4R-3Pairs Indicates whether the UE supports selecting one antenna pair among three antenna pairs to transmit SRS simultaneously for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |  |
| ul-64QAM Indicates whether the UE supports 64QAM in UL on the band. This field is only present when the field ue-CategoryUL indicates UL UE category that supports UL 64QAM, see TS 36.306 [5], Table 4.1A-2. If the field is present for one band, the field shall be present for all bands including downlink only bands. |  | - |  |
| ul-256QAM Indicates whether the UE supports 256QAM in UL on the band in the band combination. This field is only present when the field ue-CategoryUL indicates UL UE category that supports 256QAM in UL, see TS 36.306 [5], Table 4.1A-2. The UE includes this field only if the field ul-256QAM-perCC-InfoList is not included. |  | - |  |
| ul-256QAM (in FeatureSetUL-PerCC) Indicates whether the UE supports 256QAM in UL for MR-DC within the indicated feature set. This field is only present when the field ue-CategoryUL indicates UL UE category that supports 256QAM in UL, see TS 36.306 [5], Table 4.1A-2. |  | - |  |
| ul-256QAM-perCC-InfoList Indicates, per serving carrier of which the corresponding bandwidth class includes multiple serving carriers (i.e. bandwidth class B, C, D and so on), whether the UE supports 256QAM in the band combination. The number of entries is equal to the number of component carriers in the corresponding bandwidth class. The UE shall support the setting indicated in each entry of the list regardless of the order of entries in the list. This field is only present when the field ue-CategoryUL indicates UL UE category that supports 256QAM in UL, see TS 36.306 [5], Table 4.1A-2. The UE includes this field only if the field ul-256QAM is not included. |  | - |  |
| ul-256QAM-Slot Indicates whether the UE supports 256QAM in UL for slot TTI operation on the band. |  | - |  |
| ul-256QAM-Subslot Indicates whether the UE supports 256QAM in UL for subslot TTI operation on the band. |  | - |  |
| ul-AsyncHarqSharingDiff-TTI-Lengths Indicates whether the UE supports UL asynchronous HARQ sharing between different TTI lengths for an UL serving cell. |  | Yes |  |
| ul-CoMP Indicates whether the UE supports UL Coordinated Multi-Point operation. |  | No |  |
| ul-dmrs-Enhancements Indicates whether the UE supports UL DMRS enhancements as defined in TS 36.211 [21], clause 6.10.3A. |  | Yes |  |
| ul-PDCP-AvgDelay Indicates whether the UE supports UL PDCP Packet Average Delay measurement (as specified in TS 38.314 [103]) and reporting in RRC_CONNECTED. |  | - |  |
| ul-PDCP-Delay Indicates whether the UE supports UL PDCP Packet Delay per QCI measurement as specified in TS 36.314 [71]. |  | - |  |
| ul-powerControlEnhancements Indicates whether UE supports UplinkPowerControlDedicated. |  | Yes |  |
| uplinkLAA Presence of the field indicates that the UE supports uplink LAA operation. |  | - |  |
| uss-BlindDecodingAdjustment Indicates whether the UE supports blind decoding adjustment on UE specific search space as defined in TS 36.213 [22]. This field can be included only if uplinkLAA is included. |  | - |  |
| uss-BlindDecodingReduction Indicates whether the UE supports blind decoding reduction on UE specific search space by not monitoring DCI format 0A/0B/4A/4B as defined in TS 36.213 [22]. This field can be included only if uplinkLAA is included. |  | - |  |
| unicastFrequencyHopping Indicates whether the UE supports frequency hopping for unicast MPDCCH/PDSCH (configured by mpdcch-pdsch-HoppingConfig) and unicast PUSCH (configured by pusch-HoppingConfig). |  | - |  |
| unicast-fembmsMixedSCell Indicates whether the UE supports unicast reception from FeMBMS/Unicast mixed cell. This field is included only if UE supports carrier aggregation. |  | No |  |
| utra-GERAN-CGI-Reporting-ENDC Indicates whether the UE supports Inter-RAT report CGI procedure towards GERAN/UTRA cell when it is configured with (NG)EN-DC wherein either MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if their DRX cycles are same. |  |  | Yes |
| utran-ProximityIndication Indicates whether the UE supports proximity indication for UTRAN CSG member cells. |  | - |  |
| utran-SI-AcquisitionForHO Indicates whether the UE supports, upon configuration of si-RequestForHO by the network, acquisition and reporting of relevant information using autonomous gaps by reading the SI from a neighbouring UMTS cell. |  | Yes |  |
| v2x-BandParametersNR Includes the NR BandParametersSidelink-r16 IE as specified in TS 38.331 [82]. The field includes the per-band sidelink capability for NR-PC5. |  | - |  |
| v2x-BandwidthClassTxSL, v2x-BandwidthClassRxSL The bandwidth class for V2X sidelink transmission and reception supported by the UE as defined in TS 36.101 [42], Table 5.6G.1-3. The UE explicitly includes all the supported bandwidth class combinations for V2X sidelink transmission or reception in the band combination signalling. Support for one bandwidth class does not implicitly indicate support for another bandwidth class. |  | - |  |
| v2x-eNB-Scheduled Indicates whether the UE supports transmitting PSCCH/PSSCH using dynamic scheduling, SPS in eNB scheduled mode for V2X sidelink communication, reporting SPS assistance information and the UE supports maximum transmit power associated with Power class 3 V2X UE, see TS 36.101 [42] in a band. |  | - |  |
| v2x-EnhancedHighReception Indicates whether the UE supports reception of 30 PSCCH in a subframe and decoding of 204 RBs per subframe counting both PSCCH and PSSCH in a band for V2X sidelink communication. |  |  | - |
| v2x-HighPower Indicates whether the UE supports maximum transmit power associated with Power class 2 V2X UE for V2X sidelink transmission in a band, see TS 36.101 [42]. |  | - |  |
| v2x-HighReception Indicates whether the UE supports reception of 20 PSCCH in a subframe and decoding of 136 RBs per subframe counting both PSCCH and PSSCH in a band for V2X sidelink communication. |  | - |  |
| v2x-nonAdjacentPSCCH-PSSCH Indicates whether the UE supports transmission and reception in the configuration of non-adjacent PSCCH and PSSCH for V2X sidelink communication. |  | - |  |
| v2x-numberTxRxTiming Indicates the number of multiple reference TX/RX timings counted over all the configured sidelink carriers for V2X sidelink communication. |  | - |  |
| v2x-SensingReportingMode3 Indicates whether the UE supports sensing measurements and reporting of measurement results in eNB scheduled mode for V2X sidelink communication. |  | - |  |
| v2x-SupportedBandCombinationList Indicates the supported band combination list on which the UE supports simultaneous transmission and/or reception of V2X sidelink communication. |  |  |  |
| v2x-SupportedBandCombinationListEUTRA-NR Indicates the supported band combination list on which the UE supports simultaneous transmission and/or reception of NR sidelink communication only, or joint V2X sidelink communication and NR sidelink communication. |  | - |  |
| v2x-SupportedTxBandCombListPerBC, v2x-SupportedRxBandCombListPerBC Indicates, for a particular band combination of EUTRA, the supported band combination list among v2x-SupportedBandCombinationList on which the UE supports simultaneous transmission or reception of EUTRA and V2X sidelink communication respectively. The first bit refers to the first entry of v2x-SupportedBandCombinationList, with value 1 indicating V2X sidelink transmission/reception is supported. |  | - |  |
| v2x-SupportedTxBandCombListPerBC-v1630, v2x-SupportedRxBandCombListPerBC-v1630 Indicates, for a particular band combination of EUTRA, the supported band combination list among v2x-SupportedBandCombinationListEUTRA-NR on which the UE supports simultaneous transmission or reception of EUTRA and NR sidelink communication respectively, or simultaneous transmission or reception of EUTRA and joint V2X sidelink communication and NR sidelink communication respectively. The first bit refers to the first entry of v2x-SupportedBandCombinationListEUTRA-NR, with value 1 indicating V2X sidelink transmission/reception is supported. |  | - |  |
| v2x-TxWithShortResvInterval Indicates whether the UE supports 20 ms and 50 ms resource reservation periods for UE autonomous resource selection and eNB scheduled resource allocation for V2X sidelink communication. |  | - |  |
| virtualCellID-BasicSRS Indicates whether the UE supports virtual cell ID for basic SRS symbol(s). |  | - |  |
| virtualCellID-AddSRS This field indicates whether the UE supports virtual cell ID for additional SRS symbol(s). |  | - |  |
| voiceOverPS-HS-UTRA-FDD Indicates whether UE supports IMS voice according to GSMA IR.58 profile in UTRA FDD. |  | - |  |
| voiceOverPS-HS-UTRA-TDD128 Indicates whether UE supports IMS voice in UTRA TDD 1.28Mcps. |  | - |  |
| whiteCellList Indicates whether the UE supports EUTRA white cell listing to limit the set of cells applicable for measurements. |  | - |  |
| widebandPRG-Slot, widebandPRG-Subslot, widebandPRG-Subframe Indicates whether the UE supports wideband precoding resource block group size for slot/subslot/subframe operation as specified in TS 36.213 [23]. |  | - |  |
| wlan-IW-RAN-Rules Indicates whether the UE supports RAN-assisted WLAN interworking based on access network selection and traffic steering rules. |  | - |  |
| wlan-IW-ANDSF-Policies Indicates whether the UE supports RAN-assisted WLAN interworking based on ANDSF policies. |  | - |  |
| wlan-MAC-Address Indicates the WLAN MAC address of this UE. |  | - |  |
| wlan-PeriodicMeas Indicates whether the UE supports periodic reporting of WLAN measurements. |  | - |  |
| wlan-ReportAnyWLAN Indicates whether the UE supports reporting of WLANs not listed in the measObjectWLAN. |  | - |  |
| wlan-SupportedDataRate Indicates the maximum WLAN data rate supported by the UE over all LWA bearers. Actual value of supported data rate is field value * 10 Mbps (i.e., value 1 corresponds to 10 Mbps, value 2 corresponds to 20 Mbps and so on). |  | - |  |
| zp-CSI-RS-AperiodicInfo Indicates whether the UE supports aperiodic ZP-CSI-RS transmission for the indicated transmission mode. |  | Yes |  |

NOTE 1:	The IE UE-EUTRA-Capability does not include AS security capability information, since these are the same as the security capabilities that are signalled by NAS. Consequently, AS need not provide "man-in-the-middle" protection for the security capabilities.

NOTE 2:	The column FDD/ TDD diff indicates if the UE is allowed to signal, as part of the additional capabilities for an XDD mode i.e. within UE-EUTRA-CapabilityAddXDD-Mode-xNM, a different value compared to the value signalled elsewhere within UE-EUTRA-Capability (i.e. the common value, supported for both XDD modes). A '-' is used to indicate that it is not possible to signal different values (used for fields for which the field description is provided for other reasons). Annex E specifies for which TDD and FDD serving cells a UE supporting TDD/FDD CA shall support a capability for which it indicates support within the capability signalling.

NOTE 2a:	From REL-15 onwards, the UE is not allowed to signal different values for FDD and TDD unless yes is indicated in column FDD/ TDD diff (i.e. no need to introduce field description solely for the purpose of indicate no).

NOTE 3:	The BandCombinationParameters for the same band combination can be included more than once.

NOTE 4:	UE CA and measurement capabilities indicate the combinations of frequencies that can be configured as serving frequencies.

NOTE 5:	The grouping of the cells to the first and second cell group, as indicated by supportedCellGrouping, is shown in the table below. The leading / leftmost bit of supportedCellGrouping corresponds to the Bit String Position 1.

| Nr of Band Entries: | 5 | 4 | 3 |
| --- | --- | --- | --- |
| Length of Bit-String: | 15 | 7 | 3 |
| Bit String Position | Cell grouping option (0= first cell group, 1= second cell group) |  |  |
| 1 | 00001 | 0001 | 001 |
| 2 | 00010 | 0010 | 010 |
| 3 | 00011 | 0011 | 011 |
| 4 | 00100 | 0100 |  |
| 5 | 00101 | 0101 |  |
| 6 | 00110 | 0110 |  |
| 7 | 00111 | 0111 |  |
| 8 | 01000 |  |  |
| 9 | 01001 |  |  |
| 10 | 01010 |  |  |
| 11 | 01011 |  |  |
| 12 | 01100 |  |  |
| 13 | 01101 |  |  |
| 14 | 01110 |  |  |
| 15 | 01111 |  |  |

NOTE 6:	UE includes the intraBandContiguousCC-InfoList-r12 also for bandwidth class A because of the presence conditions in BandCombinationParameters-v1270. For example, if UE supports CA_1A_41D band combination, if UE includes the field intraBandContiguousCC-InfoList-r12 for band 41, the UE includes intraBandContiguousCC-InfoList-r12 also for band 1.

NOTE 6a:	For multiple BandParameters entries with the same bandEUTRA and same ca-BandwidthClassDL in a supported band combination, the UE capabilities indicated by BandParameters are agnostic to the order in which they are indicated in the bandParameterList, under the condition that the set of the capabilities indicated for the concerned bandEUTRA (e.g. bandParametersDL and bandParametersUL) are used together, and the concerned BandParameters correspond to the supportedBandwithCombinationSet for which set of channel bandwidths for carrier(s) is the same among sub-blocks, as defined in TS 36.101 [42], Table 5.6A.1-3, Table 5.6A.1-4, Table 5.6A.1-5.

NOTE 7:	For a UE that indicates release X in field accessStratumRelease but supports a feature specified in release X+ N (i.e. early UE implementation), the ASN.1 comprehension requirement are specified in Annex F.

NOTE 8:	For a UE that does not include mimo-WeightedLayersCapabilities-r13, or for the case with no CC configured with FD-MIMO, the FD-MIMO processing capability condition is not applicable (i.e. considered as satisfied). For a UE that includes mimo-WeightedLayersCapabilities-r13, the FD-MIMO processing capability condition is satisfied if the equation 4.3.28.13-1 in TS 36.306 [5] is satisfied.

#### –	UE-RadioPagingInfo

The UE-RadioPagingInfo IE contains UE capability information needed for paging.

UE-RadioPagingInfo information element

-- ASN1START

UE-RadioPagingInfo-r12 ::=				SEQUENCE {

ue-Category-v1250						INTEGER (0)			OPTIONAL,

...,

[[	ue-CategoryDL-v1310					ENUMERATED {m1}		OPTIONAL,

ce-ModeA-r13						ENUMERATED {true}	OPTIONAL,

ce-ModeB-r13						ENUMERATED {true}	OPTIONAL

]],

[[	wakeUpSignal-r15					ENUMERATED {true}	OPTIONAL,

wakeUpSignal-TDD-r15				ENUMERATED {true}	OPTIONAL,

wakeUpSignalMinGap-eDRX-r15			ENUMERATED {ms40, ms240, ms1000, ms2000}		OPTIONAL,

wakeUpSignalMinGap-eDRX-TDD-r15		ENUMERATED {ms40, ms240, ms1000, ms2000}		OPTIONAL

]],

[[	ue-CategoryDL-v1610					ENUMERATED {m2}		OPTIONAL,

groupWakeUpSignal-r16				ENUMERATED {true}	OPTIONAL,

groupWakeUpSignalTDD-r16			ENUMERATED {true}	OPTIONAL,

groupWakeUpSignalAlternation-r16	ENUMERATED {true}	OPTIONAL,

groupWakeUpSignalAlternationTDD-r16	ENUMERATED {true}	OPTIONAL

]]

}

-- ASN1STOP

UE-RadioPagingInfo field descriptions

ce-ModeA, ce-ModeB Indicates whether the UE supports operation in CE mode A and/or B, as specified in TS 36.211 [21] and TS 36.213 [23].

groupWakeUpSignal, groupWakeUpSignalTDD Indicates whether the UE supports GWUS for paging in RRC_IDLE as specified in TS 36.211 [21], TS 36.213 [23] and TS 36.304 [4]. If this field is included, the minimum gap between GWUS and associated PO for DRX is fixed as 40 ms.

groupWakeUpSignalAlternation, groupWakeUpSignalAlternationTDD Indicates whether the UE supports GWUS with group resource alternation for paging in RRC_IDLE as specified in TS 36.211 [21], TS 36.213 [23] and TS 36.304 [4]. If this field is included, the minimum gap between GWUS and associated PO for DRX is fixed as 40 ms.

ue-Category, ue-CategoryDL UE category as defined in TS 36.306 [5]. A category M2 UE shall also include the field ue-CategoryDL-v1310 in this version of the specification.

wakeUpSignal, wakeUpSignal-TDD Indicates whether the UE supports WUS for paging in RRC_IDLE as specified in TS 36.213 [22] and TS 36.304 [4]. If this field is included, the minimum gap between WUS and associated PO for DRX is fixed as 40 ms.

wakeUpSignalMinGap-eDRX, wakeUpSignalMinGap-eDRX-TDD Indicates the minimum gap the UE supports between WUS and associated PO for eDRX as specified in TS 36.213 [22] and TS 36.304 [4]. Value ms40 corresponds to 40 ms, ms240 corresponds to 240 ms and so on. If this field is included, the UE shall also indicate support of WUS or GWUS for paging.

#### –	UE-TimersAndConstants

The IE UE-TimersAndConstants contains timers and constants used by the UE in either RRC_CONNECTED or RRC_IDLE.

UE-TimersAndConstants information element

-- ASN1START

UE-TimersAndConstants ::=			SEQUENCE {

t300								ENUMERATED {

ms100, ms200, ms300, ms400, ms600, ms1000, ms1500,

ms2000},

t301								ENUMERATED {

ms100, ms200, ms300, ms400, ms600, ms1000, ms1500,

ms2000},

t310								ENUMERATED {

ms0, ms50, ms100, ms200, ms500, ms1000, ms2000},

n310								ENUMERATED {

n1, n2, n3, n4, n6, n8, n10, n20},

t311								ENUMERATED {

ms1000, ms3000, ms5000, ms10000, ms15000,

ms20000, ms30000},

n311								ENUMERATED {

n1, n2, n3, n4, n5, n6, n8, n10},

...,

[[	t300-v1310						ENUMERATED {

ms2500, ms3000, ms3500, ms4000, ms5000, ms6000, ms8000,

ms10000}		OPTIONAL,	-- Need OR

t301-v1310						ENUMERATED {

ms2500, ms3000, ms3500, ms4000, ms5000, ms6000, ms8000,

ms10000}		OPTIONAL	-- Need OR

]],

[[	t310-v1330							ENUMERATED {ms4000, ms6000}

OPTIONAL	-- Need OR

]],

[[	t300-r15						ENUMERATED {ms4000, ms6000, ms8000, ms10000, ms15000,

ms25000, ms40000, ms60000}	OPTIONAL		-- Cond EDTorPUR

]]

}

-- ASN1STOP

UE-TimersAndConstants field descriptions

n3xy Constants are described in clause 7.4. n1 corresponds with 1, n2 corresponds with 2 and so on.

t3xy Timers are described in clause 7.3. Value ms0 corresponds with 0 ms, ms50 corresponds with 50 ms and so on. EUTRAN includes an extended value t3xy-v1310 and t3xy-v1330 only in the Bandwidth Reduced (BR) version of the SIB. UEs that support Coverage Enhancement (CE) mode B shall use the extended values t3xy-v1310 and t3xy-v1330, if present, and ignore the value signaled by t3xy (without the suffix). t300-r15 is only applicable for EDT for mobile originating calls and for UL data transmission using PUR. UE performing EDT for mobile originating calls or UL data transmission using PUR shall use t300-r15, if present.

| Conditional presence | Explanation |
| --- | --- |
| EDTorPUR | The field is optionally present, Need OR, if edt-Parameters is present in SIB2 or the UE is configured with pur-Config; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	VisitedCellInfoList

The IE VisitedCellInfoList includes the mobility history information of maximum of 16 most recently visited cells or time spent outside E-UTRA. The most recently visited cell is stored first in the list. The list includes cells visited in RRC_IDLE and RRC_CONNECTED states.

VisitedCellInfoList information element

-- ASN1START

VisitedCellInfoList-r12 ::=	SEQUENCE (SIZE (1..maxCellHistory-r12)) OF VisitedCellInfo-r12

VisitedCellInfo-r12 ::=				SEQUENCE {

visitedCellId-r12					CHOICE {

cellGlobalId-r12						CellGlobalIdEUTRA,

pci-arfcn-r12							SEQUENCE {

physCellId-r12							PhysCellId,

carrierFreq-r12							ARFCN-ValueEUTRA-r9

}

}																OPTIONAL,

timeSpent-r12						INTEGER (0..4095),

...

}

-- ASN1STOP

VisitedCellInfoList field descriptions

timeSpent This field indicates the duration of stay in the cell or outside E-UTRA approximated to the closest second. If the duration of stay exceeds 4095s, the UE shall set it to 4095s.

#### –	WLAN-OffloadConfig

The IE WLAN-OffloadConfig includes information for traffic steering between E-UTRAN and WLAN. The fields are applicable to both RAN-assisted WLAN interworking based on access network selection and traffic steering rules and RAN-assisted WLAN interworking based on ANDSF policies unless stated otherwise in the field description.

WLAN-OffloadConfig information element

-- ASN1START

WLAN-OffloadConfig-r12 ::=				SEQUENCE {

thresholdRSRP-r12						SEQUENCE {

thresholdRSRP-Low-r12					RSRP-Range,

thresholdRSRP-High-r12					RSRP-Range

}																	OPTIONAL, -- Need OR

thresholdRSRQ-r12						SEQUENCE {

thresholdRSRQ-Low-r12					RSRQ-Range,

thresholdRSRQ-High-r12					RSRQ-Range

}																	OPTIONAL, -- Need OR

thresholdRSRQ-OnAllSymbolsWithWB-r12	SEQUENCE {

thresholdRSRQ-OnAllSymbolsWithWB-Low-r12			RSRQ-Range,

thresholdRSRQ-OnAllSymbolsWithWB-High-r12			RSRQ-Range

}																	OPTIONAL, -- Need OP

thresholdRSRQ-OnAllSymbols-r12			SEQUENCE {

thresholdRSRQ-OnAllSymbolsLow-r12					RSRQ-Range,

thresholdRSRQ-OnAllSymbolsHigh-r12					RSRQ-Range

}																	OPTIONAL, -- Need OP

thresholdRSRQ-WB-r12					SEQUENCE {

thresholdRSRQ-WB-Low-r12							RSRQ-Range,

thresholdRSRQ-WB-High-r12							RSRQ-Range

}																	OPTIONAL, -- Need OP

thresholdChannelUtilization-r12			SEQUENCE {

thresholdChannelUtilizationLow-r12		INTEGER (0..255),

thresholdChannelUtilizationHigh-r12		INTEGER (0..255)

}																	OPTIONAL, -- Need OR

thresholdBackhaul-Bandwidth-r12			SEQUENCE {

thresholdBackhaulDL-BandwidthLow-r12	WLAN-backhaulRate-r12,

thresholdBackhaulDL-BandwidthHigh-r12	WLAN-backhaulRate-r12,

thresholdBackhaulUL-BandwidthLow-r12	WLAN-backhaulRate-r12,

thresholdBackhaulUL-BandwidthHigh-r12	WLAN-backhaulRate-r12

}																	OPTIONAL, -- Need OR

thresholdWLAN-RSSI-r12						SEQUENCE {

thresholdWLAN-RSSI-Low-r12					INTEGER (0..255),

thresholdWLAN-RSSI-High-r12					INTEGER (0..255)

}																	OPTIONAL, -- Need OR

offloadPreferenceIndicator-r12			BIT STRING (SIZE (16))		OPTIONAL, -- Need OR

t-SteeringWLAN-r12						T-Reselection				OPTIONAL, -- Need OR

...

}

WLAN-backhaulRate-r12 ::=					ENUMERATED

{r0, r4, r8, r16, r32, r64, r128, r256, r512,

r1024, r2048, r4096, r8192, r16384, r32768, r65536, r131072,

r262144, r524288, r1048576, r2097152, r4194304, r8388608,

r16777216, r33554432, r67108864, r134217728, r268435456,

r536870912, r1073741824, r2147483648, r4294967296}

-- ASN1STOP

WLAN-OffloadConfig field descriptions

offloadPreferenceIndicator Indicates the offload preference indicator. Parameter: OPI in TS 24.312 [66]. Only applicable to RAN-assisted WLAN interworking based on ANDSF policies.

thresholdBackhaulDLBandwidth-High Indicates the backhaul available downlink bandwidth threshold used by the UE for traffic steering to WLAN. Parameter: ThreshBackhRateDLWLAN, High in TS 36.304 [4]. Value in kilobits/second. Value rN corresponds to N kbps.

thresholdBackhaulDLBandwidth-Low Indicates the backhaul available downlink bandwidth threshold used by the UE for traffic steering to E-UTRAN. Parameter: ThreshBackhRateDLWLAN, Low in TS 36.304 [4]. Value in kilobits/second. Value rN corresponds to N kbps.

thresholdBackhaulULBandwidth-High Indicates the backhaul available uplink bandwidth threshold used by the UE for traffic steering to WLAN. Parameter: ThreshBackhRateULWLAN, High in TS 36.304 [4]. Value in kilobits/second. Value rN corresponds to N kbps.

thresholdBackhaulULBandwidth-Low Indicates the backhaul available uplink bandwidth threshold used by the UE for traffic steering to E-UTRAN. Parameter: ThreshBackhRateULWLAN, Low in TS 36.304 [4]. Value in kilobits/second. Value rN corresponds to N kbps.

thresholdChannelUtilization-High Indicates the WLAN channel utilization (BSS load) threshold used by the UE for traffic steering to E-UTRAN. Parameter: ThreshChUtilWLAN, High in TS 36.304 [4].

thresholdChannelUtilization-Low Indicates the WLAN channel utilization (BSS load) threshold used by the UE for traffic steering to WLAN. Parameter: ThreshChUtilWLAN, Low in TS 36.304 [4].

thresholdRSRP-High Indicates the RSRP threshold (in dBm) used by the UE for traffic steering to E-UTRAN. Parameter: ThreshServingOffloadWLAN, HighP in TS 36.304 [4].

thresholdRSRP-Low Indicates the RSRP threshold (in dBm) used by the UE for traffic steering to WLAN. Parameter: ThreshServingOffloadWLAN, LowP in TS 36.304 [4].

thresholdRSRQ-High,thresholdRSRQ-OnAllSymbolsHigh, thresholdRSRQ-WB-High, thresholdRSRQ-OnAllSymbolsWithWB-High Indicates the RSRQ threshold (in dB) used by the UE for traffic steering to E-UTRAN. Parameter: ThreshServingOffloadWLAN, HighQ in TS 36.304 [4]. The UE shall only apply one of threshold values of thresholdRSRQ-OnAllSymbolsWithWB-High, thresholdRSRQ-OnAllSymbolsHigh, thresholdRSRQ-WB-High and thresholdRSRQ-High as present in wlan-OffloadConfigCommon and forward this to upper layer. NOTE 1.

thresholdRSRQ-Low,thresholdRSRQ-OnAllSymbolsLow, thresholdRSRQ-WB-Low, thresholdRSRQ-OnAllSymbolsWithWB-Low Indicates the RSRQ threshold (in dB) used by the UE for traffic steering to WLAN. Parameter: ThreshServingOffloadWLAN, LowQ in TS 36.304 [4]. The UE shall only apply one of threshold values of thresholdRSRQ-OnAllSymbolsWithWB-Low, thresholdRSRQ-OnAllSymbolsLow, thresholdRSRQ-WB-Low and thresholdRSRQ-Low as present in wlan-OffloadConfigCommon and forward this to upper layer. NOTE 1.

thresholdWLAN-RSSI-High Indicates the WLAN RSSI threshold used by the UE for traffic steering to WLAN. Parameter: ThreshWLANRSSI, High in TS 36.304 [4]. Value 0 corresponds to -128dBm, 1 corresponds to -127dBm and so on.

thresholdWLAN-RSSI-Low Indicates the WLAN RSSI threshold used by the UE for traffic steering to E-UTRAN. Parameter: ThreshWLANRSSI, Low in TS 36.304 [4]. Value 0 corresponds to -128dBm, 1 corresponds to -127dBm and so on.

t-SteeringWLAN Indicates the timer value during which the rules should be fulfilled before starting traffic steering between E-UTRAN and WLAN. Parameter: TsteeringWLAN in TS 36.304 [4]. Only applicable to RAN-assisted WLAN interworking based on access network selection and traffic steering rules.

NOTE 1:	Within SIB17, E-UTRAN includes the fields corresponding to same RSRQ types as included in SIB1. E.g. if E-UTRAN includes q-QualMinRSRQ-OnAllSymbols in SIB1 it also includes thresholdRSRQ-OnAllSymbols in SIB17. Within the RRCConnectionReconfiguration message E-UTRAN only includes thresholdRSRQ, setting the value according to the RSRQ type used for E-UTRAN. The UE shall apply the RSRQ fields (RSRQ threshold, high and low) corresponding to one RSRQ type i.e. the same as it applies for E-UTRAN.

### 6.3.7	MBMS information elements

#### –	MBMS-NotificationConfig

The IE MBMS-NotificationConfig specifies the MBMS notification related configuration parameters, that are applicable for all MBSFN areas.

MBMS-NotificationConfig information element

-- ASN1START

MBMS-NotificationConfig-r9 ::=				SEQUENCE {

notificationRepetitionCoeff-r9		ENUMERATED {n2, n4},

notificationOffset-r9				INTEGER (0..10),

notificationSF-Index-r9				INTEGER (1..6)

}

MBMS-NotificationConfig-v1430 ::=				SEQUENCE {

notificationSF-Index-v1430				INTEGER (7..10)

}

-- ASN1STOP

MBMS-NotificationConfig field descriptions

notificationOffset Indicates, together with the notificationRepetitionCoeff, the radio frames in which the MCCH information change notification is scheduled i.e. the MCCH information change notification is scheduled in radio frames for which: SFN mod notification repetition period = notificationOffset.

notificationRepetitionCoeff Actual change notification repetition period common for all MCCHs that are configured= shortest modification period/ notificationRepetitionCoeff. The 'shortest modificaton period' corresponds with the lowest value of mcch-ModificationPeriod of all MCCHs that are configured. Value n2 corresponds to coefficient 2, and so on.

notificationSF-Index Indicates the subframe used to transmit MCCH change notifications on PDCCH. FDD: Value 1, 2, 3, 4, 5 and 6 correspond with subframe #1, #2, #3 #6, #7, and #8 respectively. Value 7, 8, 9 and 10 correspond with subframe #0, #4, #5 and #9 respectively. If notificationSF-Index-v1430 is included, UE ignores notificationSF-Index-r9. TDD: Value 1, 2, 3, 4, and 5 correspond with subframe #3, #4, #7, #8, and #9 respectively.

#### –	MBMS-ServiceList

The IE MBMS-ServiceList provides the list of MBMS services which the UE is receiving or interested to receive.

MBMS-ServiceList information element

-- ASN1START

MBMS-ServiceList-r13 ::=			SEQUENCE (SIZE (0..maxMBMS-ServiceListPerUE-r13)) OF MBMS-ServiceInfo-r13

MBMS-ServiceInfo-r13 ::=				SEQUENCE	{

tmgi-r13							TMGI-r9

}

-- ASN1STOP

#### –	MBSFN-AreaId

The IE MBSFN-AreaId identifies an MBSFN area by means of a locally unique value at lower layers i.e. it concerns parameter NIDMBSFN in TS 36.211 [21], clause 6.10.2.1.

MBSFN-AreaId information element

-- ASN1START

MBSFN-AreaId-r12 ::=					INTEGER (0..255)

-- ASN1STOP

#### –	MBSFN-AreaInfoList

The IE MBSFN-AreaInfoList contains the information required to acquire the MBMS control information associated with one or more MBSFN areas.

MBSFN-AreaInfoList information element

-- ASN1START

MBSFN-AreaInfoList-r9 ::=			SEQUENCE (SIZE(1..maxMBSFN-Area)) OF MBSFN-AreaInfo-r9

MBSFN-AreaInfo-r9 ::=				SEQUENCE {

mbsfn-AreaId-r9						MBSFN-AreaId-r12,

non-MBSFNregionLength				ENUMERATED {s1, s2},

notificationIndicator-r9			INTEGER (0..7),

mcch-Config-r9						SEQUENCE {

mcch-RepetitionPeriod-r9		ENUMERATED {rf32, rf64, rf128, rf256},

mcch-Offset-r9					INTEGER (0..10),

mcch-ModificationPeriod-r9		ENUMERATED {rf512, rf1024},

sf-AllocInfo-r9					BIT STRING (SIZE(6)),

signallingMCS-r9				ENUMERATED {n2, n7, n13, n19}

},

...,

[[	mcch-Config-r14				SEQUENCE {

mcch-RepetitionPeriod-v1430		ENUMERATED {rf1, rf2, rf4, rf8,

rf16 }		OPTIONAL,	-- Need OR

mcch-ModificationPeriod-v1430	ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128,

rf256, spare7}					OPTIONAL	-- Need OR

}																	OPTIONAL,	-- Need OR

subcarrierSpacingMBMS-r14		ENUMERATED {kHz7dot5, kHz1dot25}	OPTIONAL	-- Need OR

]]

}

MBSFN-AreaInfoList-r16 ::=		SEQUENCE (SIZE(1..maxMBSFN-Area)) OF MBSFN-AreaInfo-r16

MBSFN-AreaInfo-r16 ::=				SEQUENCE {

mbsfn-AreaId-r16					MBSFN-AreaId-r12,

notificationIndicator-r16			INTEGER (0..7),

mcch-Config-r16						SEQUENCE {

mcch-RepetitionPeriod-r16			ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64,

rf128, rf256, spare7, spare6, spare5,

spare4, spare3, spare2, spare1},

mcch-ModificationPeriod-r16			ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128,

rf256, rf512, rf1024, spare5, spare4,

spare3,spare2, spare1},

mcch-Offset-r16					INTEGER (0..10),

sf-AllocInfo-r16				BIT STRING (SIZE(10)),

signallingMCS-r16				ENUMERATED {n2, n7, n13, n19}

},

subcarrierSpacingMBMS-r16		ENUMERATED {kHz7dot5, kHz2dot5, kHz1dot25, kHz0dot37,

spare4, spare3, spare2, spare1},

timeSeparation-r16				ENUMERATED {sl2, sl4} OPTIONAL,	-- Need OR

...

}

-- ASN1STOP

MBSFN-AreaInfoList field descriptions

mcch-ModificationPeriod Defines periodically appearing boundaries, i.e. radio frames for which SFN mod mcch-ModificationPeriod = 0. The contents of different transmissions of MCCH information can only be different if there is at least one such boundary in-between them. In case mcch-ModificationPeriod-v1430 is configured, the UE shall ignore the mcch-ModificationPeriod-r9.

mcch-Offset Indicates, together with the mcch-RepetitionPeriod, the radio frames in which MCCH is scheduled i.e. MCCH is scheduled in radio frames for which: SFN mod mcch-RepetitionPeriod = mcch-Offset.

mcch-RepetitionPeriod Defines the interval between transmissions of MCCH information, in radio frames, Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on. In case mcch-RepetitionPeriod-v1430 is configured, the UE shall ignore the mcch-RepetitionPeriod-r9.

non-MBSFNregionLength Indicates how many symbols from the beginning of the subframe constitute the non-MBSFN region. This value applies in all subframes of the MBSFN area used for PMCH transmissions as indicated in the MSI. The values s1 and s2 correspond with 1 and 2 symbols, respectively: see TS 36.211 [21], Table 6.7-1.

notificationIndicator Indicates which PDCCH bit is used to notify the UE about change of the MCCH applicable for this MBSFN area. Value 0 corresponds with the least significant bit as defined in TS 36.212 [22], clause 5.3.3.1 and so on.

sf-AllocInfo-r9 Indicates the subframes of the radio frames indicated by the mcch-RepetitionPeriod and the mcch-Offset, that may carry MCCH. Value "1" indicates that the corresponding subframe is allocated. If the bitmap is set to all zeros, the corresponding MBSFN area is considered as not configured. The following mapping applies: FDD: The first/ leftmost bit defines the allocation for subframe #1 of the radio frame indicated by mcch-RepetitionPeriod and mcch-Offset, the second bit for #2, the third bit for #3, the fourth bit for #6, the fifth bit for #7 and the sixth bit for #8. TDD: The first/leftmost bit defines the allocation for subframe #3 of the radio frame indicated by mcch-RepetitionPeriod and mcch-Offset, the second bit for #4, third bit for #7, fourth bit for #8, fifth bit for #9. Uplink subframes are not allocated. The last bit is not used.

sf-AllocInfo-r16 Indicates the subframes of the radio frames indicated by the mcch-RepetitionPeriod and the mcch-Offset, that may carry MCCH. Value "1" indicates that the corresponding subframe is allocated. The first/ leftmost bit defines the allocation for subframe #0 of the radio frame indicated by mcch-RepetitionPeriod and mcch-Offset, the second bit for #1 and so on. When subcarrierSpacingMBMS indicates 0.37 kHz subcarrier spacing, a valid MBMS slot can carry MCCH if any subframe corresponding to the slot is configured to carry MCCH.

signallingMCS Indicates the MCS applicable for the subframes indicated by the field sf-AllocInfo and for each (P)MCH that is configured for this MBSFN area, for the first subframe allocated to the (P)MCH within each MCH scheduling period (which may contain the MCH scheduling information provided by MAC). Value n2 corresponds with the value 2 for parameter in TS 36.213 [23], Table 7.1.7.1-1, and so on.

subcarrierSpacingMBMS The value indicates subcarrier spacing for MBSFN subframes, kHz7dot5 refers to 7.5 kHz subcarrier spacing, kHz2dot5 refers to 2.5 kHz subcarrier spacing and so on as defined in TS 36.211 [21], clause 6.12. These subframes do not have non-MBSFN region. If subcarrierSpacingMBMS-r14 is present, then non-MBSFNregionLength shall be ignored. EUTRAN configures parameter subcarrierSpacingMBMS only when the MBSFN subframes have subcarrier spacing other than 15 kHz. If subcarrierSpacingMBMS indicates 0.37 kHz subcarrier spacing, the slot as defined in TS 36.211 [21], clause 4.1 is valid only when all the corresponding subframes are configured as MBSFN subframes in this slot.

timeSeparation Indicates the staggering length for MBSFN-RS associated with PMCH as defined in TS 36.211 [21], clause 6.10.2.2.4. Value sl2 refers to staggering length of 2 slots (MBSFN reference signal pattern type 2) and sl4 refers to staggering length of 4 slots (MBSFN reference signal pattern type 1). E-UTRAN always configures this field when subcarrierSpacingMBMS indicates 0.37 kHz subcarrier spacing. Othewise the field is not configured.

#### –	MBSFN-SubframeConfig

The IE MBSFN-SubframeConfig defines subframes that are reserved for MBSFN in downlink.

MBSFN-SubframeConfig information element

-- ASN1START

MBSFN-SubframeConfig ::=			SEQUENCE {

radioframeAllocationPeriod			ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset			INTEGER (0..7),

subframeAllocation					CHOICE {

oneFrame							BIT STRING (SIZE(6)),

fourFrames							BIT STRING (SIZE(24))

}

}

MBSFN-SubframeConfig-v1430 ::=		SEQUENCE {

subframeAllocation-v1430				CHOICE {

oneFrame-v1430						BIT STRING (SIZE(2)),

fourFrames-v1430					BIT STRING (SIZE(8))

}

}

MBSFN-SubframeConfig-v1610 ::=		SEQUENCE {

subframeAllocation-v1610				CHOICE {

oneFrame-v1610						BIT STRING (SIZE(2)),

fourFrames-v1610					BIT STRING (SIZE(8))

}

}

-- ASN1STOP

MBSFN-SubframeConfig field descriptions

fourFrames A bit-map indicating MBSFN subframe allocation in four consecutive radio frames, "1" denotes that the corresponding subframe is allocated for MBSFN. The bitmap is interpreted as follows: FDD: Starting from the first radioframe and from the first/leftmost bit in the bitmap, the allocation applies to subframes #1, #2, #3, #6, #7, and #8 in the sequence of the four radio-frames. TDD: Starting from the first radioframe and from the first/leftmost bit in the bitmap, the allocation applies to subframes #3, #4, #7, #8, and #9 in the sequence of the four radio-frames. The last four bits are not used. E-UTRAN allocates uplink subframes only if eimta-MainConfig is configured.

fourFrames-v1430, fourFrames-v1610 A bit-map indicating MBSFN subframe allocation in four consecutive radio frames, "1" denotes that the corresponding subframe is allocated for MBSFN. The bitmap is interpreted as follows: FDD: Starting from the first radioframe and from the first/leftmost bit in the bitmap, the allocation indicated by fourFrames-v1430 applies to subframes #4 and #9 in the sequence of the four radio-frames. Starting from the first radioframe and from the first/leftmost bit in the bitmap, the allocation indicated by fourFrames-v1610, if present, applies to subframes #0 and #5 in the sequence of the four radio-frames.

oneFrame "1" denotes that the corresponding subframe is allocated for MBSFN. The following mapping applies: FDD: The first/leftmost bit defines the MBSFN allocation for subframe #1, the second bit for #2, third bit for #3, fourth bit for #6, fifth bit for #7, sixth bit for #8. TDD: The first/leftmost bit defines the allocation for subframe #3, the second bit for #4, third bit for #7, fourth bit for #8, fifth bit for #9. E-UTRAN allocates uplink subframes only if eimta-MainConfig is configured. The last bit is not used.

oneFrame-v1430, oneFrame-v1610 "1" denotes that the corresponding subframe is allocated for MBSFN. The following mapping applies: FDD: The first/leftmost bit indicated by oneFrame-v1430 defines the MBSFN allocation for subframe #4 and the second bit for #9. The first/leftmost bit indicated by oneFrame-v1610, if present, defines the MBSFN allocation for subframe #0 and the second bit for #5.

radioFrameAllocationPeriod, radioFrameAllocationOffset Radio-frames that contain MBSFN subframes occur when equation SFN mod radioFrameAllocationPeriod = radioFrameAllocationOffset is satisfied. Value n1 for radioframeAllocationPeriod denotes value 1, n2 denotes value 2, and so on. When fourFrames is used for subframeAllocation, the equation defines the first radio frame referred to in the description below. Values n1 and n2 are not applicable when fourFrames is used.

subframeAllocation Defines the subframes that are allocated for MBSFN within the radio frame allocation period defined by the radioFrameAllocationPeriod and the radioFrameAllocationOffset.

#### –	PMCH-InfoList

The IE PMCH-InfoList specifies configuration of all PMCHs of an MBSFN area, while IE PMCH-InfoListExt includes additional PMCHs, i.e. extends the PMCH list using the general principles specified in 5.1.2. The information provided for an individual PMCH includes the configuration parameters of the sessions that are carried by the concerned PMCH. For all PMCH that E-UTRAN includes in PMCH-InfoList, the list of ongoing sessions has at least one entry.

PMCH-InfoList information element

-- ASN1START

PMCH-InfoList-r9 ::=				SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF PMCH-Info-r9

PMCH-InfoListExt-r12 ::=			SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF PMCH-InfoExt-r12

PMCH-Info-r9 ::=					SEQUENCE {

pmch-Config-r9						PMCH-Config-r9,

mbms-SessionInfoList-r9			MBMS-SessionInfoList-r9,

...

}

PMCH-InfoExt-r12 ::=				SEQUENCE {

pmch-Config-r12						PMCH-Config-r12,

mbms-SessionInfoList-r12			MBMS-SessionInfoList-r9,

...

}

MBMS-SessionInfoList-r9 ::=		SEQUENCE (SIZE (0..maxSessionPerPMCH)) OF MBMS-SessionInfo-r9

MBMS-SessionInfo-r9 ::=			SEQUENCE {

tmgi-r9								TMGI-r9,

sessionId-r9						OCTET STRING (SIZE (1))		OPTIONAL,	-- Need OR

logicalChannelIdentity-r9			INTEGER (0..maxSessionPerPMCH-1),

...

}

PMCH-Config-r9 ::=					SEQUENCE {

sf-AllocEnd-r9						INTEGER (0..1535),

dataMCS-r9							INTEGER (0..28),

mch-SchedulingPeriod-r9			ENUMERATED {

rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024},

...

}

PMCH-Config-r12 ::=					SEQUENCE {

sf-AllocEnd-r12						INTEGER (0..1535),

dataMCS-r12							CHOICE {

normal-r12							INTEGER (0..28),

higerOrder-r12						INTEGER (0..27)

},

mch-SchedulingPeriod-r12		ENUMERATED {

rf4, rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024},

...,

[[	mch-SchedulingPeriod-v1430		ENUMERATED {rf1, rf2}			OPTIONAL	-- Need OR

]]

}

TMGI-r9 ::=						SEQUENCE {

plmn-Id-r9							CHOICE {

plmn-Index-r9						INTEGER (1..maxPLMN-r11),

explicitValue-r9					PLMN-Identity

},

serviceId-r9						OCTET STRING (SIZE (3))

}

-- ASN1STOP

PMCH-InfoList field descriptions

dataMCS Indicates the value for parameter in TS 36.213 [23], which defines the MCS applicable for the subframes of this (P)MCH as indicated by the field commonSF-Alloc. Value normal corresponds to Table 7.1.7.1-1 and value higherOrder corresponds to Table 7.1.7.1-1A. The MCS does however neither apply to the subframes that may carry MCCH i.e. the subframes indicated by the field sf-AllocInfo within SystemInformationBlockType13 nor for the first subframe allocated to this (P)MCH within each MCH scheduling period (which may contain the MCH scheduling information provided by MAC).

mch-SchedulingPeriod Indicates the MCH scheduling period i.e. the periodicity used for providing MCH scheduling information at lower layers (MAC) applicable for an MCH. Value rf8 corresponds to 8 radio frames, rf16 corresponds to 16 radio frames and so on. The mch-SchedulingPeriod starts in the radio frames for which: SFN mod mch-SchedulingPeriod = 0. E-UTRAN configures mch-SchedulingPeriod of the (P)MCH listed first in PMCH-InfoList to be smaller than or equal to mcch-RepetitionPeriod. In case mch-SchedulingPeriod-v1430 is configured, the UE shall ignore mch-SchedulingPeriod-r12.

plmn-Index Index of the entry across the plmn-IdentityList fields within SystemInformationBlockType1.

sessionId Indicates the optional MBMS Session Identity, which together with TMGI identifies a transmission or a possible retransmission of a specific MBMS session: see TS 29.061 [51], clauses 20.5, 17.7.11, and 17.7.15. The field is included whenever upper layers have assigned a session identity i.e. one is available for the MBMS session in E-UTRAN.

serviceId Uniquely identifies the identity of an MBMS service within a PLMN. The field contains octet 3- 5 of the IE Temporary Mobile Group Identity (TMGI) as defined in TS 24.008 [49]. The first octet contains the third octet of the TMGI, the second octet contains the fourth octet of the TMGI and so on.

sf-AllocEnd Indicates the last subframe allocated to this (P)MCH within a period identified by field commonSF-AllocPeriod. The subframes allocated to (P)MCH corresponding with the nth entry in pmch-InfoList are the subsequent subframes starting from either the next subframe after the subframe identified by sf-AllocEnd of the (n-1)th listed (P)MCH or, for n=1, the first subframe defined by field commonSF-Alloc, through the subframe identified by sf-AllocEnd of the nth listed (P)MCH. Value 0 corresponds with the first subframe defined by field commonSF-Alloc.

### 6.3.7a	SC-PTM information elements

#### –	SC-MTCH-InfoList

The IE SC-MTCH-InfoList provides the list of ongoing MBMS sessions transmitted via SC-MRB and for each MBMS session, the associated G-RNTI and scheduling information.

SC-MTCH-InfoList information element

-- ASN1START

SC-MTCH-InfoList-r13 ::=			SEQUENCE (SIZE (0..maxSC-MTCH-r13)) OF SC-MTCH-Info-r13

SC-MTCH-Info-r13 ::=				SEQUENCE	{

mbmsSessionInfo-r13						MBMSSessionInfo-r13,

g-RNTI-r13								BIT STRING(SIZE(16)),

sc-mtch-schedulingInfo-r13				SC-MTCH-SchedulingInfo-r13			OPTIONAL,	-- Need OP

sc-mtch-neighbourCell-r13				BIT STRING (SIZE(maxNeighCell-SCPTM-r13))	OPTIONAL,	-- Need OP

...,

[[	p-a-r13								ENUMERATED {

dB-6, dB-4dot77, dB-3, dB-1dot77,

dB0, dB1, dB2, dB3}		OPTIONAL	-- Need ON

]]

}

MBMSSessionInfo-r13 ::=				SEQUENCE	{

tmgi-r13								TMGI-r9,

sessionId-r13							OCTET STRING (SIZE (1))		OPTIONAL	-- Need OR

}

SC-MTCH-SchedulingInfo-r13::=		SEQUENCE	{

onDurationTimerSCPTM-r13				ENUMERATED {

psf1, psf2, psf3, psf4, psf5, psf6,

psf8, psf10, psf20, psf30, psf40,

psf50, psf60, psf80, psf100,

psf200},

drx-InactivityTimerSCPTM-r13			ENUMERATED {

psf0, psf1, psf2, psf4, psf8,

psf10, psf20, psf40,

psf80, psf160, ps320,

psf640, psf960,

psf1280, psf1920, psf2560},

schedulingPeriodStartOffsetSCPTM-r13	CHOICE {

sf10									INTEGER(0..9),

sf20									INTEGER(0..19),

sf32									INTEGER(0..31),

sf40									INTEGER(0..39),

sf64									INTEGER(0..63),

sf80									INTEGER(0..79),

sf128									INTEGER(0..127),

sf160									INTEGER(0..159),

sf256									INTEGER(0..255),

sf320									INTEGER(0..319),

sf512									INTEGER(0..511),

sf640									INTEGER(0..639),

sf1024									INTEGER(0..1023),

sf2048									INTEGER(0..2048),

sf4096									INTEGER(0..4096),

sf8192									INTEGER(0..8192)

},

...

}

-- ASN1STOP

SC-MTCH-InfoList field descriptions

drx-InactivityTimerSCPTM Timer for SC-MTCH in TS 36.321 [6]. Value in number of PDCCH sub-frames. Value psf0 corresponds to 0 PDCCH sub-frame and behaviour as specified in 7.3.2 applies, psf1 corresponds to 1 PDCCH sub-frame, psf2 corresponds to 2 PDCCH sub-frames and so on.

g-RNTI G-RNTI used to scramble the scheduling and transmission of a SC-MTCH.

mbmsSessionInfo Indicates the ongoing MBMS session in a SC-MTCH.

onDurationTimerSCPTM Timer for SC-MTCH reception in TS 36.321 [6]. Value in number of PDCCH sub-frames. Value psf1 corresponds to 1 PDCCH sub-frame, psf2 corresponds to 2 PDCCH sub-frames and so on.

p-a Parameter: , for the SC-MTCH per G-RNTI, see TS 36.213 [23], clause 5.2. Value dB-6 corresponds to -6 dB, dB-4dot77 corresponds to -4.77 dB etc.

schedulingPeriodStartOffsetSCPTM SCPTM-SchedulingCycle and SCPTM-SchedulingOffset in TS 36.321 [6]. The value of SCPTM-SchedulingCycle is in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. The value of SCPTM-SchedulingOffset is in number of sub-frames. The E-UTRAN does not configure a maximum value 2048 for sf2048, 4096 for sf4096 or 8192 for sf8192.

sc-mtch-neighbourCell Indicates neighbour cells which also provide this service on SC-MTCH. The first bit is set to 1 if the service is provided on SC-MTCH in the first cell in scptmNeighbourCellList, otherwise it is set to 0. The second bit is set to 1 if the service is provided on SC-MTCH in the second cell in scptmNeighbourCellList, and so on. If this field is absent, the UE shall assume that this service is not available on SC-MTCH in any neighbour cell.

sc-mtch-schedulingInfo DRX information for the SC-MTCH. If this field is absent, the SC-MTCH may be scheduled in any subframe.

#### –	SC-MTCH-InfoList-BR

The IE SC-MTCH-InfoList-BR provides the list of ongoing MBMS sessions transmitted via SC-MRB and for each MBMS session, the associated G-RNTI and scheduling information.

SC-MTCH-InfoList-BR information element

-- ASN1START

SC-MTCH-InfoList-BR-r14 ::=		SEQUENCE (SIZE (0..maxSC-MTCH-BR-r14)) OF SC-MTCH-Info-BR-r14

SC-MTCH-Info-BR-r14 ::=			SEQUENCE	{

sc-mtch-CarrierFreq-r14					ARFCN-ValueEUTRA-r9,

mbmsSessionInfo-r14						MBMSSessionInfo-r13,

g-RNTI-r14								BIT STRING(SIZE(16)),

sc-mtch-schedulingInfo-r14			SC-MTCH-SchedulingInfo-BR-r14				OPTIONAL,	-- Need OP

sc-mtch-neighbourCell-r14				BIT STRING (SIZE(maxNeighCell-SCPTM-r13))	OPTIONAL,	-- Need OP

mpdcch-Narrowband-SC-MTCH-r14				INTEGER (1.. maxAvailNarrowBands-r13),

mpdcch-NumRepetition-SC-MTCH-r14			ENUMERATED {r1, r2, r4, r8, r16,

r32, r64, r128, r256},

mpdcch-StartSF-SC-MTCH-r14		CHOICE {

fdd-r14								ENUMERATED {v1, v1dot5, v2, v2dot5, v4,

v5, v8, v10},

tdd-r14								ENUMERATED {v1, v2, v4, v5, v8, v10,

v20}

},

mpdcch-PDSCH-HoppingConfig-SC-MTCH-r14		ENUMERATED {on, off},

mpdcch-PDSCH-CEmodeConfig-SC-MTCH-r14		ENUMERATED {ce-ModeA, ce-ModeB},

mpdcch-PDSCH-MaxBandwidth-SC-MTCH-r14		ENUMERATED {bw1dot4, bw5},

mpdcch-Offset-SC-MTCH-r14					ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth},

p-a-r14										ENUMERATED { dB-6, dB-4dot77, dB-3,

dB-1dot77, dB0, dB1, dB2,

dB3}				OPTIONAL,--	Need OR

...

}

SC-MTCH-SchedulingInfo-BR-r14::=	SEQUENCE	{

onDurationTimerSCPTM-r14				ENUMERATED {

psf300, psf400, psf500, psf600,

psf800, psf1000, psf1200, psf1600},

drx-InactivityTimerSCPTM-r14			ENUMERATED {

psf0, psf1, psf2, psf4, psf8, psf16,

psf32, psf64, psf128, psf256, ps512,

psf1024, psf2048, psf4096, psf8192, psf16384},

schedulingPeriodStartOffsetSCPTM-r14	CHOICE {

sf10									INTEGER(0..9),

sf20									INTEGER(0..19),

sf32									INTEGER(0..31),

sf40									INTEGER(0..39),

sf64									INTEGER(0..63),

sf80									INTEGER(0..79),

sf128									INTEGER(0..127),

sf160									INTEGER(0..159),

sf256									INTEGER(0..255),

sf320									INTEGER(0..319),

sf512									INTEGER(0..511),

sf640									INTEGER(0..639),

sf1024									INTEGER(0..1023),

sf2048									INTEGER(0..2047),

sf4096									INTEGER(0..4095),

sf8192									INTEGER(0..8191)

},

...

}

-- ASN1STOP

SC-MTCH-InfoList-BR field descriptions

drx-InactivityTimerSCPTM Timer for SC-MTCH in TS 36.321 [6]. Value in number of MPDCCH sub-frames. Value psf0 corresponds to 0 MPDCCH sub-frame and behaviour as specified in 7.3.2 applies, psf1 corresponds to 1 MPDCCH sub-frame, psf2 corresponds to 2 MPDCCH sub-frames and so on.

g-RNTI G-RNTI used to scramble the scheduling and transmission of a SC-MTCH

mbmsSessionInfo Indicates the ongoing MBMS session in a SC-MTCH.

mpdcch-Narrowband-SC-MTCH Narrowband for MPDCCH for SC-MTCH, see TS 36.213 [23].

mpdcch-NumRepetitions-SC-MTCH The maximum number of MPDCCH repetitions the UE needs to monitor for SC-MTCH, see TS 36.213 [23].

mpdcch-Offset-SC-MTCH Fractional period offset of starting subframes for MPDCCH search space for SC-MTCH, see TS 36.213 [23].

mpdcch-PDSCH-CEmodeConfig-SC-MTCH Coverage enhancement mode configuration for MPDCCH/PDSCH for SC-MTCH, see TS 36.213 [23].

mpdcch-PDSCH-HoppingConfig-SC-MTCH Frequency hopping configuration for MPDCCH/PDSCH for SC-MTCH, see TS 36.213 [23].

mpdcch-PDSCH-MaxBandwidth-SC-MTCH Maximum PDSCH channel bandwidth for SC-MTCH, see TS 36.213 [23]. Value bw1dot4 corresponds to 1.4 MHz channel bandwidth and value bw5 corresponds to 5 MHz channel bandwidth. Corresponding maximum TBS are specified in TS 36.213 [23], clause 7.1.7.2.

mpdcch-StartSF-SC-MTCH Starting subframes configuration of the MPDCCH search space for SC-MTCH, see TS 36.213 [23].

onDurationTimerSCPTM Timer for SC-MTCH reception in TS 36.321 [6]. Value in number of MPDCCH sub-frames. Value psf300 corresponds to 300 MPDCCH sub-frames, psf400 corresponds to 400 MPDCCH sub-frames and so on.

schedulingPeriodStartOffsetSCPTM SCPTM-SchedulingCycle and SCPTM-SchedulingOffset in TS 36.321 [6]. The value of SCPTM-SchedulingCycle is in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. The value of SCPTM-SchedulingOffset is in number of sub-frames.

sc-mtch-CarrierFreq Downlink carrier used for multicast SC-MTCH transmissions.

sc-mtch-neighbourCell Indicates neighbour cells which also provide this service on SC-MTCH. The first bit is set to 1 if the service is provided on SC-MTCH in the first cell in scptmNeighbourCellList, otherwise it is set to 0. The second bit is set to 1 if the service is provided on SC-MTCH in the second cell in scptmNeighbourCellList, and so on. If this field is absent, the UE shall assume that this service is not available on SC-MTCH in any neighbour cell.

sc-mtch-schedulingInfo DRX information for the SC-MTCH. If this field is absent, DRX is not used for SC-MTCH reception.

p-a Parameter: for the SC-MTCH per G-RNTI, see TS 36.213 [23], clause 5.2. Value dB-6 corresponds to -6 dB, dB-4dot77 corresponds to -4.77 dB etc.

#### –	SCPTM-NeighbourCellList

The IE SCPTM-NeighbourCellList indicates a list of neighbour cells where ongoing MBMS sessions provided via SC-MRB in the current cells are also provided.

-- ASN1START

SCPTM-NeighbourCellList-r13 ::=		SEQUENCE (SIZE (1..maxNeighCell-SCPTM-r13)) OF PCI-ARFCN-r13

PCI-ARFCN-r13 ::=					SEQUENCE {

physCellId-r13						PhysCellId,

carrierFreq-r13						ARFCN-ValueEUTRA-r9		OPTIONAL

}

-- ASN1STOP

SCPTM-NeighbourCellList field description

carrierFreq Indicates the frequency of the neighbour cell indicated by physCellId. Absence of the IE means that the neighbour cell is on the same frequency as the current cell.

### 6.3.8	Sidelink information elements

#### –	SL-AnchorCarrierFreqList-V2X

The IE SL-AnchorCarrierFreqList-V2X specifies the SL V2X anchor frequencies i.e. frequencies that include inter-carrier resource configuration for V2X sidelink communication.

SL-AnchorCarrierFreqList-V2X information element

-- ASN1START

SL-AnchorCarrierFreqList-V2X-r14 ::= SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF ARFCN-ValueEUTRA-r9

-- ASN1STOP

#### –	SL-CBR-CommonTxConfigList

The IE SL-CBR-CommonTxConfigList indicates the list of PSSCH transmission parameters (such as MCS, sub-channel number, retransmission number, CR limit) in sl-CBR-PSSCH-TxConfigList, and the list of CBR ranges in cbr-RangeCommonConfigList, to configure congestion control to the UE for V2X sidelink communication.

SL-CBR-CommonTxConfigList information element

-- ASN1START

SL-CBR-CommonTxConfigList-r14 ::=	SEQUENCE {

cbr-RangeCommonConfigList-r14	SEQUENCE (SIZE (1..maxSL-V2X-CBRConfig-r14)) OF SL-CBR-Levels-Config-r14,

sl-CBR-PSSCH-TxConfigList-r14	SEQUENCE (SIZE (1..maxSL-V2X-TxConfig-r14)) OF SL-CBR-PSSCH-TxConfig-r14

}

SL-CBR-Levels-Config-r14 ::=		SEQUENCE (SIZE (1..maxCBR-Level-r14)) OF SL-CBR-r14

SL-CBR-PSSCH-TxConfig-r14 ::=		SEQUENCE {

cr-Limit-r14					INTEGER(0..10000),

tx-Parameters-r14				SL-PSSCH-TxParameters-r14

}

SL-CBR-r14 ::=						INTEGER(0..100)

-- ASN1STOP

SL-CBR-CommonTxConfigList field descriptions

cbr-RangeCommonConfigList Indicates the list of CBR ranges. Each entry of the list indicates in SL-CBR-Levels-Config the upper bound of the CBR range for the respective entry. The upper bounds of the CBR ranges are configured in ascending order for consecutive entries of cbr-RangeCommonConfigList. For the first entry of cbr-RangeCommonConfigList the lower bound of the CBR range is 0.

cr-Limit Indicates the maximum limit on the occupancy ratio. Value 0 corresponds to 0, value 1 to 0.0001, value 2 to 0.0002, and so on (i.e. in steps of 0.0001) until value 10000, which corresponds to 1.

sl-CBR-PSSCH-TxConfigList Indicates the list of available PSSCH transmission parameters (such as MCS, sub-channel number, retransmission number and CR limit) configurations.

SL-CBR Value 0 corresponds to 0, value 1 to 0.01, value 2 to 0.02, and so on.

tx-Parameters Indicates PSSCH transmission parameters.

#### –	SL-CBR-PPPP-TxConfigList

The IE SL-CBR-PPPP-TxConfigList indicates the mapping between PSSCH transmission parameter (such as MCS, PRB number, retransmission number, CR limit) sets by using the indexes of the configurations provided in sl-CBR-PSSCH-TxConfigList, CBR ranges by an index to the entry of the CBR range configuration in cbr-RangeCommonConfigList, and PPPP ranges. It also indicates the default PSSCH transmission parameters to be used when CBR measurement results are not available.

SL-CBR-PPPP-TxConfigList information element

-- ASN1START

SL-CBR-PPPP-TxConfigList-r14 ::=	SEQUENCE (SIZE (1..8)) OF SL-PPPP-TxConfigIndex-r14

SL-PPPP-TxConfigIndex-r14 ::=		SEQUENCE {

priorityThreshold-r14			SL-Priority-r13,

defaultTxConfigIndex-r14		INTEGER(0..maxCBR-Level-1-r14),

cbr-ConfigIndex-r14				INTEGER(0..maxSL-V2X-CBRConfig-1-r14),

tx-ConfigIndexList-r14			SEQUENCE (SIZE (1..maxCBR-Level-r14)) OF Tx-ConfigIndex-r14

}

Tx-ConfigIndex-r14 ::=				INTEGER(0..maxSL-V2X-TxConfig-1-r14)

SL-CBR-PPPP-TxConfigList-v1530 ::=	SEQUENCE (SIZE (1..8)) OF SL-PPPP-TxConfigIndex-v1530

SL-PPPP-TxConfigIndex-v1530 ::=		SEQUENCE {

mcs-PSSCH-RangeList-r15				SEQUENCE (SIZE (1..maxCBR-Level-r14)) OF MCS-PSSCH-Range-r15						OPTIONAL		--Need OR

}

MCS-PSSCH-Range-r15 ::=		SEQUENCE{

minMCS-PSSCH-r15			INTEGER (0..31),

maxMCS-PSSCH-r15			INTEGER (0..31)

}

SL-CBR-PPPP-TxConfigList-r15 ::=	SEQUENCE (SIZE (1..8)) OF SL-PPPP-TxConfigIndex-r15

SL-PPPP-TxConfigIndex-r15 ::=		SEQUENCE {

priorityThreshold-r15			SL-Priority-r13,

defaultTxConfigIndex-r15		INTEGER(0..maxCBR-Level-1-r14),

cbr-ConfigIndex-r15				INTEGER(0..maxSL-V2X-CBRConfig-1-r14),

tx-ConfigIndexList-r15			SEQUENCE (SIZE (1..maxCBR-Level-r14)) OF Tx-ConfigIndex-r14,

mcs-PSSCH-RangeList-r15				SEQUENCE (SIZE (1..maxCBR-Level-r14)) OF MCS-PSSCH-Range-r15

}

-- ASN1STOP

SL-CBR-PPPP-TxConfigList field descriptions

cbr-ConfigIndex Indicates the CBR ranges to be used by an index to the entry of the CBR range configuration in cbr-RangeCommonConfigList.

defaultTxConfigIndex Indicates the PSSCH transmission parameters to be used by the UEs which do not have available CBR measurement results, by means of an index to the corresponding entry in tx-ConfigIndexList. Value 0 indicates the first entry in tx-ConfigIndexList. The field is ignored if the UE has available CBR measurement results.

mcs-PSSCH-RangeList If included, this field applies to the PPPP(s) indicated by the priorityThreshold and each entry in this field sequentially corresponds to each CBR range indicated by cbr-ConfigIndex.

minMCS-PSSCH, maxMCS-PSSCH Indicates the minimum and maximum MCS values which correspond to both the MCS table in Table 8.6.1-1 and Table 14.1.1-2 in TS 36.213 [23] used for transmission on PSSCH.

priorityThreshold Indicates the upper bound of PPPP range which is associated with the configurations in cbr-ConfigIndex and in tx-ConfigIndexList. The upper bounds of the PPPP ranges are configured in ascending order for consecutive entries of SL-PPPP-TxConfigIndex in SL-CBR-PPPP-TxConfigList. For the first entry of SL-PPPP-TxConfigIndex, the lower bound of the PPPP range is 1.

SL-CBR-PPPP-TxConfigList-v1530 If included, E-UTRAN shall include the same number of entries, and listed in the same order, as in SL-CBR-PPPP-TxConfigList-r14.

tx-ConfigIndexList Indicates the list of the PSSCH transmission parameters and CR limit by the indexes to the entries of the configurations in sl-CBR-PSSCH-TxConfigList. Each index in tx-ConfigIndexList sequentially maps to each CBR range indicated by cbr-ConfigIndex.

#### –	SL-CommConfig

The IE SL-CommConfig specifies the dedicated configuration information for sidelink communication. In particular it concerns the transmission resource configuration for sidelink communication on the primary frequency.

SL-CommConfig information element

-- ASN1START

SL-CommConfig-r12 ::=				SEQUENCE	{

commTxResources-r12					CHOICE {

release								NULL,

setup								CHOICE {

scheduled-r12					SEQUENCE {

sl-RNTI-r12						C-RNTI,

mac-MainConfig-r12				MAC-MainConfigSL-r12,

sc-CommTxConfig-r12				SL-CommResourcePool-r12,

mcs-r12							INTEGER (0..28)				OPTIONAL	-- Need OP

},

ue-Selected-r12					SEQUENCE {

-- Pool for normal usage

commTxPoolNormalDedicated-r12	SEQUENCE {

poolToReleaseList-r12			SL-TxPoolToReleaseList-r12 OPTIONAL,	-- Need ON

poolToAddModList-r12			SL-CommTxPoolToAddModList-r12 OPTIONAL	-- Need ON

}

}

}

}																		OPTIONAL,	-- Need ON

...,

[[	commTxResources-v1310				CHOICE {

release								NULL,

setup								CHOICE {

scheduled-v1310						SEQUENCE {

logicalChGroupInfoList-r13			LogicalChGroupInfoList-r13,

multipleTx-r13						BOOLEAN

},

ue-Selected-v1310					SEQUENCE {

commTxPoolNormalDedicatedExt-r13	SEQUENCE {

poolToReleaseListExt-r13			SL-TxPoolToReleaseListExt-r13 OPTIONAL,	-- Need ON

poolToAddModListExt-r13				SL-CommTxPoolToAddModListExt-r13	OPTIONAL	-- Need ON

}

}

}

}																OPTIONAL,	-- Need ON

commTxAllowRelayDedicated-r13		BOOLEAN			OPTIONAL	-- Need ON

]]

}

LogicalChGroupInfoList-r13 ::=		SEQUENCE (SIZE (1..maxLCG-r13)) OF SL-PriorityList-r13

SL-CommTxPoolToAddModList-r12 ::=		SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-CommTxPoolToAddMod-r12

SL-CommTxPoolToAddModListExt-r13 ::=	SEQUENCE (SIZE (1..maxSL-TxPool-v1310)) OF SL-CommTxPoolToAddModExt-r13

SL-CommTxPoolToAddMod-r12 ::=		SEQUENCE	{

poolIdentity-r12					SL-TxPoolIdentity-r12,

pool-r12							SL-CommResourcePool-r12

}

SL-CommTxPoolToAddModExt-r13 ::=		SEQUENCE	{

poolIdentity-v1310					SL-TxPoolIdentity-v1310,

pool-r13							SL-CommResourcePool-r12

}

MAC-MainConfigSL-r12 ::=		SEQUENCE	{

periodic-BSR-TimerSL					PeriodicBSR-Timer-r12		OPTIONAL,	-- Need ON

retx-BSR-TimerSL						RetxBSR-Timer-r12

}

-- ASN1STOP

SL-CommConfig field descriptions

commTxAllowRelayDedicated Indicates whether the UE is allowed to transmit relay related sidelink communication using the configured dedicated transmission resources i.e. either via scheduled or via UE selected resources.

commTxPoolNormalDedicated Indicates a pool of transmission resources the UE is allowed to use while in RRC_CONNECTED.

logicalChGroupInfoList Indicates for each logical channel group the list of associated priorities, used as specified in TS 36.321 [6], in order of increasing logical channel group identity.

mcs Indicates the MCS as defined in TS 36.212 [23], clause 14.2.1. If not configured, the selection of MCS is up to UE implementation.

multipleTx Indicates whether the UE should perform multiple transmissions to different destinations in one SC period in accordance with TS 36.321 [6], clause 5.14.1.1. Value TRUE indicates that multiple transmissions should be performed.

sc-CommTxConfig Indicates a pool of resources for SC when E-UTRAN schedules Tx resources (i.e. when indices included in DCI format 5 indicate the actual data resources to be used as specified in TS 36.212 [22], clause 5.3.3.1.9).

scheduled Indicates the configuration for the case E-UTRAN schedules the transmission resources based on sidelink specific BSR from the UE.

ue-Selected Indicates the configuration for the case the UE selects the transmission resources from a pool of resources configured by E-UTRAN.

#### –	SL-CommResourcePool

The IE SL-CommResourcePool and SL-CommResourcePoolV2X specifies the configuration information for an individual pool of resources for sidelink communication and V2X sidelink communication respectively. The IE covers the configuration of both the sidelink control information and the data.

SL-CommResourcePool information element

-- ASN1START

SL-CommTxPoolList-r12 ::=		SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-CommResourcePool-r12

SL-CommTxPoolListExt-r13 ::=	SEQUENCE (SIZE (1..maxSL-TxPool-v1310)) OF SL-CommResourcePool-r12

SL-CommTxPoolListV2X-r14 ::=		SEQUENCE (SIZE (1..maxSL-V2X-TxPool-r14)) OF SL-CommResourcePoolV2X-r14

SL-CommRxPoolList-r12 ::=		SEQUENCE (SIZE (1..maxSL-RxPool-r12)) OF SL-CommResourcePool-r12

SL-CommRxPoolListV2X-r14 ::=		SEQUENCE (SIZE (1..maxSL-V2X-RxPool-r14)) OF SL-CommResourcePoolV2X-r14

SL-CommResourcePool-r12 ::=		SEQUENCE {

sc-CP-Len-r12						SL-CP-Len-r12,

sc-Period-r12						SL-PeriodComm-r12,

sc-TF-ResourceConfig-r12			SL-TF-ResourceConfig-r12,

data-CP-Len-r12						SL-CP-Len-r12,

dataHoppingConfig-r12				SL-HoppingConfigComm-r12,

ue-SelectedResourceConfig-r12			SEQUENCE {

data-TF-ResourceConfig-r12				SL-TF-ResourceConfig-r12,

trpt-Subset-r12						SL-TRPT-Subset-r12	OPTIONAL	-- Need OP

}																OPTIONAL,	-- Need OR

rxParametersNCell-r12				SEQUENCE {

tdd-Config-r12					TDD-Config					OPTIONAL,	-- Need OP

syncConfigIndex-r12			INTEGER (0..15)

}																OPTIONAL,	-- Need OR

txParameters-r12					SEQUENCE {

sc-TxParameters-r12				SL-TxParameters-r12,

dataTxParameters-r12			SL-TxParameters-r12

}																OPTIONAL,	-- Cond Tx

...,

[[	priorityList-r13				SL-PriorityList-r13			OPTIONAL	-- Cond Tx

]]

}

SL-CommResourcePoolV2X-r14 ::=		SEQUENCE {

sl-OffsetIndicator-r14				SL-OffsetIndicator-r12		OPTIONAL,	-- Need OR

sl-Subframe-r14						SubframeBitmapSL-r14,

adjacencyPSCCH-PSSCH-r14			BOOLEAN,

sizeSubchannel-r14					ENUMERATED {

n4, n5, n6, n8, n9, n10, n12, n15, n16, n18, n20, n25, n30,

n48, n50, n72, n75, n96, n100, spare13, spare12, spare11,

spare10, spare9, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1},

numSubchannel-r14					ENUMERATED {n1, n3, n5, n8, n10, n15, n20, spare1},

startRB-Subchannel-r14				INTEGER (0..99),

startRB-PSCCH-Pool-r14				INTEGER (0..99)				OPTIONAL,	-- Need OR

rxParametersNCell-r14				SEQUENCE {

tdd-Config-r14					TDD-Config					OPTIONAL,	-- Need OP

syncConfigIndex-r14				INTEGER (0..15)

}																OPTIONAL,	-- Need OR

dataTxParameters-r14				SL-TxParameters-r12			OPTIONAL,	-- Cond Tx

zoneID-r14							INTEGER (0..7)				OPTIONAL,	-- Need OR

threshS-RSSI-CBR-r14					INTEGER (0..45)				OPTIONAL,	-- Need OR

poolReportId-r14					SL-V2X-TxPoolReportIdentity-r14		OPTIONAL,	-- Need OR

cbr-pssch-TxConfigList-r14			SL-CBR-PPPP-TxConfigList-r14	OPTIONAL,	-- Need OR

resourceSelectionConfigP2X-r14		SL-P2X-ResourceSelectionConfig-r14	OPTIONAL,	-- Cond P2X

syncAllowed-r14						SL-SyncAllowed-r14				OPTIONAL,	-- Need OR

restrictResourceReservationPeriod-r14	SL-RestrictResourceReservationPeriodList-r14	OPTIONAL,	-- Need OR

...,

[[	sl-MinT2ValueList-r15		SL-MinT2ValueList-r15		OPTIONAL,	-- Need OR

cbr-pssch-TxConfigList-v1530	SL-CBR-PPPP-TxConfigList-v1530	OPTIONAL	-- Need OR

]]

}

SL-TRPT-Subset-r12 ::=			BIT STRING (SIZE (3..5))

SL-V2X-TxPoolReportIdentity-r14::=		INTEGER (1..maxSL-PoolToMeasure-r14)

SL-MinT2ValueList-r15 ::=	SEQUENCE (SIZE (1..maxSL-Prio-r13)) OF SL-MinT2Value-r15

SL-MinT2Value-r15 ::=			SEQUENCE {

priorityList-r15					SL-PriorityList-r13,

minT2Value-r15						INTEGER (10..20)

}

-- ASN1STOP

SL-CommResourcePool field descriptions

adjacencyPSCCH-PSSCH Indicates whether a UE shall always transmit PSCCH and PSSCH in adjacent RBs (indicated by TRUE) or in non-adjacent RBs (indicated by FALSE) (see TS 36.213 [23]).

cbr-pssch-TxConfigList Indicates the mapping between PPPPs, CBR ranges by using indexes of the entry in cbr-RangeCommonConfigList, and PSSCH transmission parameters and CR limit by using indexes of the entry in sl-CBR-PSSCH-TxConfigList. If SL-CommResourcePoolV2X is included in MobilityControlInfoV2X, it refers to cbr-MobilityTxConfigList for cbr-RangeCommonConfigList and sl-CBR-PSSCH-TxConfigList. If SL-CommResourcePoolV2X is included in SL-V2X-ConfigDedicated, it refers to cbr-DedicatedTxConfigList for cbr-RangeCommonConfigList and sl-CBR-PSSCH-TxConfigList. Otherwise, it refers to cbr-CommonTxConfigList included in the SystemInformationBlockType21 of the serving cell / PCell for cbr-RangeCommonConfigList and sl-CBR-PSSCH-TxConfigList.

minT2Value Indicates the minimum value of T2 that applies to the PPPP(s), as specified in TS 36.300 [9], included in priorityList.

numSubchannel indicates the number of subchannels in the corresponding resource pool (see TS 36.213 [23]).

poolReportId The identity of the transmission resource pool used for CBR measurement reporting, which is corresponding to the poolIdentity reported in measResultListCBR. This field is only present in the transmission pools configured in RRCConnectionReconfiguration and v2x-CommTxPoolExceptional, p2x-CommTxPoolNormalCommon, v2x-CommTxPoolNormalCommon, v2x-CommTxPoolNormal in SystemInformationBlockType21 or SystemInformationBlockType26. Otherwise, the field is absent.

resourceSelectionConfigP2X Indicates the allowed resource selection mechanism(s), i.e. partial sensing and/or random selection, for P2X related V2X sidelink communication.

restrictResourceReservationPeriod If configured, the field restrictResourceReservationPeriod configured in v2x-ResourceSelectionConfig shall be ignored for transmission on this pool.

sc-Period Indicates the period over which resources are allocated in a cell for SC and over which scheduled and UE selected data transmissions occur, see PSCCH period in TS 36.213 [23]. Value in number of subframes. Value sf40 corresponds to 40 subframes, sf80 corresponds to 80 subframes and so on. E-UTRAN configures values sf40, sf80, sf160 and sf320 for FDD and for TDD config 1 to 5, values sf70, sf140 and sf280 for TDD config 0, and finally values sf60, sf120 and sf240 for TDD config 6.

sizeSubchannel Indicates the number of PRBs of each subchannel in the corresponding resource pool (see TS 36.213 [23]). The value n5 denotes 5 PRBs; n6 denotes 6 PRBs and so on. E-UTRAN configures values n5, n6, n10, n15, n20, n25, n50, n75 and n100 in the case of adjacencyPSCCH-PSSCH set to TRUE; otherwise, E-UTRAN configures values n4, n5, n6, n8, n9, n10, n12, n15, n16, n18, n20, n30, n48, n72 and n96 in the case of adjacencyPSCCH-PSSCH set to FALSE,

sl-minT2ValueList Indicates a list of minimum value sets for the parameter T2 which is used for UE autonomous resource selection in this resource pool (see TS 36.213 [23]).

sl-OffsetIndicator Indicates the offset of the first subframe of a resource pool, i.e., the starting subframe of the repeating bitmap sl-Subframe, within a SFN cycle. If absent, the resource pool starts from first subframe of SFN=0. This field is not applicable to V2X sidelink communication.

sl-Subframe Indicates the bitmap of the resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 36.213 [23]).

startRB-PSCCH-Pool Indicates the lowest RB index of the PSCCH pool (see TS 36.213 [23]). This field is absent when a pool is (pre)configured such that a UE always transmits SC and data in adjacent RBs in the same subframe.

startRB-Subchannel Indicates the lowest RB index of the subchannel with the lowest index (see TS 36.213 [23]).

syncAllowed Indicates the allowed synchronization reference(s) which is (are) allowed to use the configured resource pool.

syncConfigIndex Indicates the synchronisation configuration that is associated with a reception pool, by means of an index to the corresponding entry of commSyncConfig in SystemInformationBlockType18 for sidelink communication, or by means of an index to the corresponding entry of v2x-SyncConfig in SystemInformationBlockType21 or SystemInformationBlockType26 for V2X sidelink communication.

tdd-Config TDD configuration associated with the reception pool of the cell indicated by syncConfigIndex. Absence of the field indicates that the duplex mode is FDD and no TDD specific physical channel configuration is applicable.

threshS-RSSI-CBR Indicates the S-RSSI threshold for determining the contribution of a sub-channel to the CBR measurement, as specified in TS 36.214 [48]. Value 0 corresponds to -112 dBm, value 1 to -110 dBm, value n to (-112 + n*2) dBm, and so on.

trpt-Subset Indicates the subset of T-RPT available (see TS 36.213 [23], clause 14.1.1.1.1). Consists of a bitmap which is used to indicate the set of available 'k' values to be used for sidelink communication (see TS 36.213 [23], clause 14.1.1.3). If T-RPT subset configuration is not signaled/ preconfigured then UE assumes the whole T-RPT set is available.

zoneID Indicates the zone ID for which the UE shall use this resource pool as described in 5.10.13.2. The field is absent in v2x-CommTxPoolExceptional, p2x-CommTxPoolNormalCommon, p2x- CommTxPoolNormal and v2x-CommRxPool in SIB21, in SIB26 or in mobilityControlInfoV2X.

| Conditional presence | Explanation |
| --- | --- |
| Tx | The field is mandatory present when included in commTxPoolNormalDedicated, commTxPoolNormalDedicatedExt, commTxPoolNormalCommon, commTxPoolNormalCommonExt, commTxPoolExceptional, sc-CommTxConfig, v2x-CommTxPoolNormalCommon, v2x-CommTxPoolExceptional, v2x-CommTxPoolNormalDedicated, p2x-CommTxPoolNormalCommon or v2x-CommTxPoolNormal and p2x-CommTxPoolNormal in v2x-InterFreqInfoList. Otherwise the field is not present. |
| P2X | The field is mandatory present when included in p2x-CommTxPoolNormalCommon, v2x-CommTxPoolNormalDedicated in sl-V2X-ConfigDedicated for P2X related V2X sidelink communication or p2x-CommTxPoolNormal in v2x-InterFreqInfoList. Otherwise the field is not present. |

#### –	SL-CommTxPoolSensingConfig

The IE SL-CommTxPoolSensingConfig specifies V2X sidelink communication configurations used for UE autonomous resource selection.

SL-CommTxPoolSensingConfig information element

-- ASN1START

SL-CommTxPoolSensingConfig-r14 ::=		SEQUENCE {

pssch-TxConfigList-r14					SL-PSSCH-TxConfigList-r14,

thresPSSCH-RSRP-List-r14				SL-ThresPSSCH-RSRP-List-r14,

restrictResourceReservationPeriod-r14	SL-RestrictResourceReservationPeriodList-r14	OPTIONAL,	-- Need OR

probResourceKeep-r14				ENUMERATED {v0, v0dot2, v0dot4, v0dot6, v0dot8,

spare3,spare2, spare1},

p2x-SensingConfig-r14					SEQUENCE {

minNumCandidateSF-r14				INTEGER (1..13),

gapCandidateSensing-r14				BIT STRING (SIZE (10))

}		OPTIONAL,	-- Need OR

sl-ReselectAfter-r14				ENUMERATED {n1, n2, n3, n4, n5, n6, n7, n8, n9,

spare7, spare6, spare5, spare4, spare3, spare2,

spare1}				OPTIONAL		-- Need OR

}

-- ASN1STOP

SL-CommTxPoolSensingConfig field descriptions

gapCandidateSensing Indicates which subframe should be sensed when a certain subframe is considered as a candidate resource (see TS 36.213 [23]).

minNumCandidateSF Indicates the minimum number of subframes that are included in the possible candidate resources.

p2x-SensingConfig Indicates the sensing configuration for P2X related V2X sidelink communication transmission only.

probResourceKeep Indicates the probability with which the UE keeps the current resource when the resource reselection counter reaches zero for sensing based UE autonomous resource selection (see TS 36.321 [6]).

pssch-TxConfigList Indicates PSSCH TX parameters such as MCS, PRB number, retransmission number, associated to different UE absolute speeds and different synchronization reference types for UE autonomous resource selection (see TS 36.213 [23]).

restrictResourceReservationPeriod Indicates which values are allowed for the signaling of the resource reservation period in PSCCH.

sl-ReselectAfter Indicates the number of consecutive skipped transmissions before triggering resource reselection for V2X sidelink communication (see TS 36.321 [6]).

thresPSSCH-RSRP-List Indicates a list of 64 thresholds, and the threshold should be selected based on the priority in the decoded SCI and the priority in the SCI to be transmitted (see TS 36.213 [23]). A resource is excluded if it is indicated or reserved by a decoded SCI and PSSCH RSRP in the associated data resource is above a threshold.

#### –	SL-CP-Len

The IE SL-CP-Len indicates the cyclic prefix length, see TS 36.211 [21].

SL-CP-Len information element

-- ASN1START

SL-CP-Len-r12 ::=				ENUMERATED {normal, extended}

-- ASN1STOP

#### –	SL-DiscConfig

The IE SL-DiscConfig specifies the dedicated configuration information for sidelink discovery.

SL-DiscConfig information element

-- ASN1START

SL-DiscConfig-r12 ::=					SEQUENCE {

discTxResources-r12						CHOICE {

release								NULL,

setup								CHOICE {

scheduled-r12					SEQUENCE {

discTxConfig-r12					SL-DiscResourcePool-r12	OPTIONAL, -- Need ON

discTF-IndexList-r12				SL-TF-IndexPairList-r12	OPTIONAL, -- Need ON

discHoppingConfig-r12				SL-HoppingConfigDisc-r12

OPTIONAL	-- Need ON

},

ue-Selected-r12					SEQUENCE {

discTxPoolDedicated-r12			SEQUENCE {

poolToReleaseList-r12			SL-TxPoolToReleaseList-r12 OPTIONAL,	-- Need ON

poolToAddModList-r12			SL-DiscTxPoolToAddModList-r12 OPTIONAL	-- Need ON

}													OPTIONAL	-- Need ON

}

}

}																OPTIONAL,	-- Need ON

...,

[[	discTF-IndexList-v1260				CHOICE {

release								NULL,

setup								SEQUENCE {

discTF-IndexList-r12b				SL-TF-IndexPairList-r12b

}

}															OPTIONAL	-- Need ON

]],

[[	discTxResourcesPS-r13			CHOICE {

release								NULL,

setup								CHOICE {

scheduled-r13						SL-DiscTxConfigScheduled-r13,

ue-Selected-r13						SEQUENCE {

discTxPoolPS-Dedicated-r13			SL-DiscTxPoolDedicated-r13

}

}

}															OPTIONAL,	-- Need ON

discTxInterFreqInfo-r13			CHOICE {

release								NULL,

setup								SEQUENCE {

discTxCarrierFreq-r13				ARFCN-ValueEUTRA-r9			OPTIONAL,	-- Need OR

discTxRefCarrierDedicated-r13		SL-DiscTxRefCarrierDedicated-r13	OPTIONAL,	-- Need OR

discTxInfoInterFreqListAdd-r13			SL-DiscTxInfoInterFreqListAdd-r13	OPTIONAL	-- Need ON

}

}															OPTIONAL,	-- Need ON

gapRequestsAllowedDedicated-r13		BOOLEAN			OPTIONAL,	-- Need ON

discRxGapConfig-r13					CHOICE {

release								NULL,

setup								SL-GapConfig-r13

}															OPTIONAL,	-- Need ON

discTxGapConfig-r13					CHOICE {

release								NULL,

setup								SL-GapConfig-r13

}															OPTIONAL,	-- Need ON

discSysInfoToReportConfig-r13		CHOICE {

release								NULL,

setup								SL-DiscSysInfoToReportFreqList-r13

}															OPTIONAL	-- Need ON

]]

}

SL-DiscSysInfoToReportFreqList-r13 ::= SEQUENCE (SIZE (1..maxFreq)) OF ARFCN-ValueEUTRA-r9

SL-DiscTxInfoInterFreqListAdd-r13 ::=	SEQUENCE {

discTxFreqToAddModList-r13				SEQUENCE (SIZE (1..maxFreq)) OF SL-DiscTxResourceInfoPerFreq-r13	OPTIONAL,	-- Need ON

discTxFreqToReleaseList-r13				SEQUENCE (SIZE (1..maxFreq)) OF ARFCN-ValueEUTRA-r9	OPTIONAL,	-- Need ON

...

}

SL-DiscTxResourceInfoPerFreq-r13 ::=	SEQUENCE	{

discTxCarrierFreq-r13					ARFCN-ValueEUTRA-r9,

discTxResources-r13						SL-DiscTxResource-r13	OPTIONAL,	-- Need OR

discTxResourcesPS-r13					SL-DiscTxResource-r13	OPTIONAL,	-- Need OR

discTxRefCarrierDedicated-r13			SL-DiscTxRefCarrierDedicated-r13	OPTIONAL,	-- Need OR

discCellSelectionInfo-r13					CellSelectionInfoNFreq-r13			OPTIONAL,	-- Need OR

...

}

SL-DiscTxResource-r13 ::=				CHOICE {

release								NULL,

setup								CHOICE {

scheduled-r13					SL-DiscTxConfigScheduled-r13,

ue-Selected-r13					SL-DiscTxPoolDedicated-r13

}

}

SL-DiscTxPoolToAddModList-r12 ::=		SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-DiscTxPoolToAddMod-r12

SL-DiscTxPoolToAddMod-r12 ::=		SEQUENCE	{

poolIdentity-r12					SL-TxPoolIdentity-r12,

pool-r12							SL-DiscResourcePool-r12

}

SL-DiscTxConfigScheduled-r13 ::=			SEQUENCE {

discTxConfig-r13					SL-DiscResourcePool-r12	OPTIONAL, -- Need ON

discTF-IndexList-r13				SL-TF-IndexPairList-r12b	OPTIONAL, -- Need ON

discHoppingConfig-r13				SL-HoppingConfigDisc-r12	OPTIONAL,-- Need ON

...

}

SL-DiscTxPoolDedicated-r13 ::=			SEQUENCE {

poolToReleaseList-r13			SL-TxPoolToReleaseList-r12 OPTIONAL,	-- Need ON

poolToAddModList-r13			SL-DiscTxPoolToAddModList-r12 OPTIONAL	-- Need ON

}

SL-TF-IndexPairList-r12 ::=		SEQUENCE (SIZE (1..maxSL-TF-IndexPair-r12)) OF SL-TF-IndexPair-r12

SL-TF-IndexPair-r12 ::=		SEQUENCE	{

discSF-Index-r12					INTEGER (1.. 200)		OPTIONAL,	-- Need ON

discPRB-Index-r12					INTEGER (1.. 50)		OPTIONAL	-- Need ON

}

SL-TF-IndexPairList-r12b ::=		SEQUENCE (SIZE (1..maxSL-TF-IndexPair-r12)) OF SL-TF-IndexPair-r12b

SL-TF-IndexPair-r12b ::=		SEQUENCE	{

discSF-Index-r12b					INTEGER (0..209)		OPTIONAL,	-- Need ON

discPRB-Index-r12b					INTEGER (0..49)			OPTIONAL	-- Need ON

}

SL-DiscTxRefCarrierDedicated-r13 ::=	CHOICE {

pCell								NULL,

sCell								SCellIndex-r10

}

-- ASN1STOP

SL-DiscConfig field descriptions

discCellSelectionInfo Parameters that may be used by the UE to select/ reselect a cell on the concerned non serving frequency. If absent, the UE acquires the information from the target cell on the concerned frequency. See TS 36.304 [4], clause 11.4.

discSysInfoToReportConfig Indicates the request to start a SidelinkUEInformation procedure for reporting system information acquired during an inter-frequency discovery procedure.

discTF-IndexList Indicates a list of time-frequency resource indices pair where each pair of indices corresponds to one discovery message. E-UTRAN only configures discTF-IndexList-r12b when configuring the UE with scheduled SL discovery Tx resources. When receiving discTF-IndexList-r12b, the UE shall only consider this field (and hence ignore discTF-IndexList-r12, if included or previously configured).

discTxConfig Indicates the resources configuration used when E-UTRAN schedules Tx resources (i.e. the fields discSF-Index and discPRB-Index indicate the actual resources to be used).

discTxInterFreqInfo Indicates frequency applicable for the resources indicated by discTxResources-r12 (i.e. original resource field may cover first inter-frequency), and possibly resource allocations on additional frequencies as may be indicated by field discTxInfoInterFreqListAdd.

discTxRefCarrierDedicated Indicates if the PCell or an SCell is to be used as reference for DL measurements and synchronization, instead of the DL frequency paired with the one used to transmit sidelink discovery announcements on, see TS 36.213 [23], clause 14.3.1.

discTxResources Indicates the resources assigned to the UE for discovery announcements, which can either be a pool from which the UE may select or a set of resources specifically assigned for use by the UE.

discTxResourcesPS Indicates the resources assigned to the UE for PS discovery announcements, which can either be a pool from which the UE may select or a set of resources specifically assigned for use by the UE.

SL-TF-IndexPair A pair of indices, one for the time domain and one for the frequency domain, indicating the start of resources within the pool covered by discTxConfig, see TS 36.211 [21], clause 9.5.6 for one discovery message. The upper limits of discSF-Index and discPRB-Index are defined in TS 36.213 [23], clause 14.3.1.

#### –	SL-DiscResourcePool

The IE SL-DiscResourcePool specifies the configuration information for an individual pool of resources for sidelink discovery.

SL-DiscResourcePool information element

-- ASN1START

SL-DiscTxPoolList-r12 ::=		SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-DiscResourcePool-r12

SL-DiscRxPoolList-r12 ::=		SEQUENCE (SIZE (1..maxSL-RxPool-r12)) OF SL-DiscResourcePool-r12

SL-DiscResourcePool-r12 ::=		SEQUENCE	{

cp-Len-r12						SL-CP-Len-r12,

discPeriod-r12				ENUMERATED {rf32, rf64, rf128,

rf256, rf512, rf1024, rf16-v1310, spare},

numRetx-r12					INTEGER (0..3),

numRepetition-r12				INTEGER (1..50),

tf-ResourceConfig-r12			SL-TF-ResourceConfig-r12,

txParameters-r12				SEQUENCE {

txParametersGeneral-r12		SL-TxParameters-r12,

ue-SelectedResourceConfig-r12	SEQUENCE {

poolSelection-r12				CHOICE {

rsrpBased-r12					SL-PoolSelectionConfig-r12,

random-r12						NULL

},

txProbability-r12			ENUMERATED {p25, p50, p75, p100}

}															OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond Tx

rxParameters-r12				SEQUENCE {

tdd-Config-r12					TDD-Config					OPTIONAL,	-- Need OR

syncConfigIndex-r12			INTEGER (0..15)

}																OPTIONAL,	-- Need OR

...,

[[	discPeriod-v1310				CHOICE {

release								NULL,

setup								ENUMERATED {rf4, rf6, rf7, rf8,

rf12, rf14, rf24, rf28}

}														OPTIONAL,	-- Need ON

rxParamsAddNeighFreq-r13		CHOICE {

release							NULL,

setup							SEQUENCE {

physCellId-r13					PhysCellIdList-r13

}

}															OPTIONAL,	-- Need ON

txParamsAddNeighFreq-r13		CHOICE {

release							NULL,

setup							SEQUENCE {

physCellId-r13					PhysCellIdList-r13,

p-Max							P-Max					OPTIONAL,	-- Need OP

tdd-Config-r13					TDD-Config				OPTIONAL,	-- Cond TDD-OR

tdd-Config-v1130				TDD-Config-v1130		OPTIONAL,	-- Cond TDD-OR

freqInfo							SEQUENCE {

ul-CarrierFreq						ARFCN-ValueEUTRA	OPTIONAL,	-- Need OP

ul-Bandwidth						ENUMERATED {n6, n15, n25, n50, n75, n100}

OPTIONAL,	-- Need OP

additionalSpectrumEmission			AdditionalSpectrumEmission

},

referenceSignalPower				INTEGER (-60..50),

syncConfigIndex-r13				INTEGER (0..15)			OPTIONAL	-- Need OR

}

}															OPTIONAL	-- Need ON

]],

[[	txParamsAddNeighFreq-v1370		CHOICE {

release							NULL,

setup							SEQUENCE {

freqInfo-v1370					SEQUENCE {

additionalSpectrumEmission-v1370		AdditionalSpectrumEmission-v10l0

}

}

}															OPTIONAL	-- Need ON

]]

}

PhysCellIdList-r13	::=		SEQUENCE (SIZE (1.. maxSL-DiscCells-r13)) OF PhysCellId

SL-PoolSelectionConfig-r12 ::=		SEQUENCE {

threshLow-r12							RSRP-RangeSL2-r12,

threshHigh-r12							RSRP-RangeSL2-r12

}

-- ASN1STOP

SL-DiscResourcePool field descriptions

discPeriod Indicates the period over which resources are allocated in a cell for discovery message transmission/reception, see PSDCH period in TS 36.213 [23]. Value in number of radio frames. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on. The extended values apply for PS discovery (not only for sidelink relaying). When broadcasting an extended value, E-UTRAN sets the original field to spare to ensure legacy UEs ignore the concerned pool entry.

numRepetition Indicates the number of times subframeBitmap is repeated for mapping to subframes that occurs within a discPeriod. The highest value E-UTRAN uses is value 5 for FDD and TDD configuration 0, value 13 for TDD configuration 1, value 25 for TDD configuration 2, value 17 for TDD configuration 3, value 25 for TDD configuration 4, value 50 for TDD configuration 5 and value 7 for TDD configuration 6. E-UTRAN configures numRepetition and subframeBitmap such that the mapped subframes do not exceed the discPeriod.

poolSelection Indicates the mechanism for selecting a (transmission) pool when multiple candidates are provided. E-UTRAN configures the same value (i.e. a pool selection method) for all candidate pools within one pool list (discTxPoolCommon or discTxPoolDedicated) but the pool selection method in different pool lists may or may not be the same.

syncConfigIndex Indicates the synchronisation configuration that is associated with a reception or transmission pool, by means of an index to the corresponding entry of discSyncConfig in SystemInformationBlockType19.

threshLow, threshHigh Specifies the thresholds used to select a resource pool in RSRP based pool selection. The E-UTRAN should configure threshLow and threshHigh such that the UE selects only one resource pool upon RSRP based pool selection.

txProbability Indicates the probability of transmitting announcement in a discovery period when configured with a pool of resources, see TS 36.321 [6].

| Conditional presence | Explanation |
| --- | --- |
| TDD-OR | The field is optional present for TDD, need OR; it is not present for FDD. |
| Tx | The field is mandatory present when included in discTxPoolDedicated or discTxPoolCommon. Otherwise the field is not present. |

#### –	SL-DiscSysInfoReport

The IE SL-DiscSysInfoReport contains the parameters related to sidelink discovery acquired from system information of inter-frequency cells (including inter-PLMN).

SL-DiscSysInfoReport information element

-- ASN1START

SL-DiscSysInfoReport-r13 ::=	SEQUENCE {

plmn-IdentityList-r13			PLMN-IdentityList			OPTIONAL,

cellIdentity-13					CellIdentity				OPTIONAL,

carrierFreqInfo-13				ARFCN-ValueEUTRA-r9			OPTIONAL,

discRxResources-r13				SL-DiscRxPoolList-r12		OPTIONAL,

discTxPoolCommon-r13			SL-DiscTxPoolList-r12		OPTIONAL,

discTxPowerInfo-r13				SL-DiscTxPowerInfoList-r12	OPTIONAL,

discSyncConfig-r13				SL-SyncConfigNFreq-r13		OPTIONAL,

discCellSelectionInfo-r13		SEQUENCE {

q-RxLevMin-r13					Q-RxLevMin,

q-RxLevMinOffset-r13			INTEGER (1..8)			OPTIONAL

}															OPTIONAL,

cellReselectionInfo-r13			SEQUENCE {

q-Hyst-r13						ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

q-RxLevMin-r13					Q-RxLevMin,

t-ReselectionEUTRA-r13			T-Reselection

}															OPTIONAL,

tdd-Config-r13					TDD-Config					OPTIONAL,

freqInfo-r13					SEQUENCE {

ul-CarrierFreq-r13				ARFCN-ValueEUTRA				OPTIONAL,

ul-Bandwidth-r13				ENUMERATED {n6, n15, n25, n50, n75, n100}

OPTIONAL,

additionalSpectrumEmission-r13	AdditionalSpectrumEmission		OPTIONAL

}																OPTIONAL,

p-Max-r13						P-Max	OPTIONAL,

referenceSignalPower-r13		INTEGER (-60..50)	OPTIONAL,

...,

[[

freqInfo-v1370					SEQUENCE {

additionalSpectrumEmission-v1370	AdditionalSpectrumEmission-v10l0

}																OPTIONAL

]]

}

-- ASN1STOP

SL-DiscSysInfoReport field descriptions

carrierFreqInfo Indicates the frequency of the cell from which the UE acquired the system information relevant for discovery

cellIdentity Indicated the identity of the cell from which the UE acquired the system information relevant for discovery

plmn-IdentityList Indicates the list of PLMN identity of the cell from which the UE acquired the system information relevant for discovery

#### –	SL-DiscTxPowerInfo

The IE SL-DiscTxPowerInfo specifies power control parameters for one or more power classes.

SL-DiscTxPowerInfo information element

-- ASN1START

SL-DiscTxPowerInfoList-r12 ::=	SEQUENCE (SIZE (maxSL-DiscPowerClass-r12)) OF SL-DiscTxPowerInfo-r12

SL-DiscTxPowerInfo-r12 ::=				SEQUENCE	{

discMaxTxPower-r12							P-Max,

...

}

-- ASN1STOP

SL-DiscTxPowerInfo field descriptions

discMaxTxPower Indicates the P-Max parameter used to calculate the maximum transmit power a UE configured with the concerned range class, see TS 24.333 [70], clause 4.2.11. The first entry in SL-DiscTxPowerInfoList corresponds to UE range class 'short', the second entry corresponds to 'medium' and the third entry corresponds to 'long'.

#### –	SL-GapConfig

The IE SL-GapConfig indicates the gaps, requested or assigned, to enable the UE to receive or transmit sidelink discovery, intra or inter frequency (includings inter-PLMN).

SL-GapConfig information element

-- ASN1START

SL-GapConfig-r13 ::=				SEQUENCE {

gapPatternList-r13				SL-GapPatternList-r13

}

SL-GapPatternList-r13 ::=	SEQUENCE (SIZE (1..maxSL-GP-r13)) OF SL-GapPattern-r13

SL-GapPattern-r13 ::=				SEQUENCE {

gapPeriod-r13						ENUMERATED {sf40, sf60, sf70, sf80, sf120, sf140, sf160,

sf240, sf280, sf320, sf640, sf1280, sf2560, sf5120,

sf10240},

gapOffset-r12						SL-OffsetIndicator-r12,

gapSubframeBitmap-r13				BIT STRING (SIZE (1..10240)),

...

}

-- ASN1STOP

| SL-GapConfig field descriptions |
| --- |
| gapOffset Indicates the offset from the start of SFN 0 to the start of the first gapPeriod. If the SFN period is not an integer multiple of gapPeriod, no subframes within this period (i.e. from SFN 0 to offset) are considered part of the gap. |  |
| gapPeriod Indicates the period by which gapSubframeBitmap is repeated. |  |
| gapSubframeBitmap Indicates the subframes of one or more individual gaps, not only covering the subframes of the associated discovery resources but also including e.g. re-tuning and synchronisation delays. The UE and E-UTRAN signal bit strings of valid sizes only i.e. sizes equal to or less than gapPeriod. Value 1 indicates that the UE is allowed to use the subframe for sidelink discovery. |

#### –	SL-GapRequest

The IE SL-GapRequest indicates the gaps requested by the UE to receive or transmit sidelink discovery, intra or inter frequency (includings inter-PLMN).

SL-GapRequest information element

-- ASN1START

SL-GapRequest-r13 ::=				SEQUENCE (SIZE (1..maxFreq)) OF SL-GapFreqInfo-r13

SL-GapFreqInfo-r13 ::=				SEQUENCE {

carrierFreq-r13					ARFCN-ValueEUTRA-r9					OPTIONAL,

gapPatternList-r13				SL-GapPatternList-r13

}

-- ASN1STOP

#### –	SL-HoppingConfig

The IE SL-HoppingConfig indicates the hopping configuration used for sidelink.

SL-HoppingConfig information element

-- ASN1START

SL-HoppingConfigComm-r12 ::=		SEQUENCE	{

hoppingParameter-r12				INTEGER (0..504),

numSubbands-r12						ENUMERATED {ns1, ns2, ns4},

rb-Offset-r12						INTEGER (0..110)

}

SL-HoppingConfigDisc-r12 ::=	SEQUENCE	{

a-r12									INTEGER (1..200),

b-r12									INTEGER (1..10),

c-r12									ENUMERATED {n1, n5}

}

-- ASN1STOP

SL-HoppingConfig field descriptions

a Per cell parameter: see TS 36.213 [23], clause 14.3.1.

b Per UE parameter: see TS 36.213 [23], clause 14.3.1.

c Per cell parameter: see TS 36.213 [23], clause 14.3.1.

hoppingParameter Affects the hopping performed as specificed in TS 36.213 [23], clauses 14.1.1.2 and 14.1.1.4. In case value 504 is received, the value used by the UE is 510.

numSubbands Parameter: Nsb see TS 36.211 [21], clause 9.3.6.

rb-Offset Parameter: , see TS 36.211 [21], clause 9.3.6.

#### –	SL-InterFreqInfoListV2X

The IE SL-InterFreqInfoListV2X indicates synchronization and resource allocation configurations of the neighboring frequency for V2X sidelink communication.

SL-InterFreqInfoListV2X information element

-- ASN1START

SL-InterFreqInfoListV2X-r14 ::=	SEQUENCE (SIZE (0..maxFreqV2X-1-r14)) OF SL-InterFreqInfoV2X-r14

SL-InterFreqInfoV2X-r14 ::=		SEQUENCE {

plmn-IdentityList-r14				PLMN-IdentityList			OPTIONAL,		-- Need OP

v2x-CommCarrierFreq-r14			ARFCN-ValueEUTRA-r9,

sl-MaxTxPower-r14					P-Max				OPTIONAL,		-- Need OR

sl-Bandwidth-r14					ENUMERATED {n6, n15, n25, n50, n75, n100}	OPTIONAL,	-- Need OR

v2x-SchedulingPool-r14				SL-CommResourcePoolV2X-r14				OPTIONAL,	-- Need OR

v2x-UE-ConfigList-r14		SL-V2X-UE-ConfigList-r14	OPTIONAL,	-- Need OR

...,

[[	additionalSpectrumEmissionV2X-r14		CHOICE {

additionalSpectrumEmission-r14			AdditionalSpectrumEmission,

additionalSpectrumEmission-v1440		AdditionalSpectrumEmission-v10l0

}				OPTIONAL		-- Need ON

]],

[[	v2x-FreqSelectionConfigList-r15	SL-V2X-FreqSelectionConfigList-r15	OPTIONAL	--Need OR

]]

}

-- ASN1STOP

SL-InterFreqInfoListV2X field descriptions

plmn-IdentityList Indicates PLMN identities of this frequency for reception of V2X sidelink communication. If this field is not present, the UE considers this frequency for reception of V2X sidelink communication concerns the first PLMN entry in the plmn-IdentityList in SystemInformationBlockType1.

sl-MaxTxPower Indicates the maximum transmission power for transmitting V2X sidelink communication on the corresponding frequency.

additionalSpectrumEmissionV2X Indicates the additionalSpectrumEmission value defined in TS 36.101 [42], clause 6.2.4, for V2X sidelink communication.

v2x-FreqSelectionConfigList Indicates the configuration information for the carrier selection for V2X sidelink communication transmission. The configuration applies to the carrier frequency identified by v2x-CommCarrierFreq (i.e. carrier specific configuration).

v2x-SchedulingPool Indicates the resource pool for inter-carrier scheduled resource allocation. This field is configured in RRC dedicated signalling only when scheduled is configured in IE SL-V2X-ConfigDedicated.

v2x-UE-ConfigList Indicates the inter-carrier resource configuration. If there is only one entry in the list without physCellId configured, the configuration is applied to the frequency identified by v2x-CommCarrierFreq (i.e. carrier specific configuration); if the entry of this field includes physCellIdList, the configuration is applied to the cell(s) identified by physCellIdList (i.e. cell specific configuration).

#### –	SL-NR-AnchorCarrierFreqList

The IE SL-NR-AnchorCarrierFreqList specifies the NR anchor frequencies i.e. frequencies that include inter-carrier resource configuration for V2X sidelink communication.

SL-NR-AnchorCarrierFreqList information element

-- ASN1START

SL-NR-AnchorCarrierFreqList-r16 ::= SEQUENCE (SIZE (1..maxFreqSL-NR-r16)) OF ARFCN-ValueNR-r15

-- ASN1STOP

#### –	SL-V2X-UE-ConfigList

The IE SL-V2X-UE-ConfigList indicates inter-frequency resource configuration per-carrier or per-cell.

SL-V2X-UE-ConfigList information element

-- ASN1START

SL-V2X-UE-ConfigList-r14 ::=	SEQUENCE (SIZE (1.. maxCellIntra)) OF SL-V2X-InterFreqUE-Config-r14

SL-V2X-InterFreqUE-Config-r14 ::=		SEQUENCE {

physCellIdList-r14					PhysCellIdList-r13					OPTIONAL,	-- Need OR

typeTxSync-r14						SL-TypeTxSync-r14					OPTIONAL,	-- Need OR

v2x-SyncConfig-r14					SL-SyncConfigListNFreqV2X-r14		OPTIONAL,	-- Need OR

v2x-CommRxPool-r14					SL-CommRxPoolListV2X-r14				OPTIONAL,	-- Need OR

v2x-CommTxPoolNormal-r14				SL-CommTxPoolListV2X-r14				OPTIONAL,	-- Need OR

p2x-CommTxPoolNormal-r14				SL-CommTxPoolListV2X-r14				OPTIONAL,	-- Need OR

v2x-CommTxPoolExceptional-r14		SL-CommResourcePoolV2X-r14			OPTIONAL,	-- Need OR

v2x-ResourceSelectionConfig-r14		SL-CommTxPoolSensingConfig-r14		OPTIONAL,	-- Need OR

zoneConfig-r14						SL-ZoneConfig-r14					OPTIONAL,	-- Need OR

offsetDFN-r14						INTEGER (0..1000)					OPTIONAL,	-- Need OR

...

}

-- ASN1STOP

SL-V2X-UE-ConfigList field descriptions

offsetDFN Indicates the timing offset for the UE to determine DFN timing when GNSS is used for timing reference. Value 0 corresponds to 0 milliseconds, value 1 corresponds to 0.001 milliseconds, value 2 corresponds to 0.002 milliseconds, and so on.

p2x-CommTxPoolNormal Indicates the resources on a carrier frequency by which the UE may transmit P2X related V2X sidelink communication.

physCellIdList If configured, the resource configuration is applicable for the cell(s) identified by this field. Otherwise, the resource configuration is for a given carrier frequency.

typeTxSync Indicates the prioritized synchronization type (i.e. eNB or GNSS) for performing V2X sidelink communication on a carrier frequency.

v2x-CommRxPool Indicates the resources on a carrier frequency by which the UE may receive V2X sidelink communication. This field is absent within v2x-InterFreqInfoList included in RRCConnectionReconfiguration except if received with MobilityControlInfo or MobilityControlInfoV2X.

v2x-CommTxPoolExceptional Indicates the resources on a carrier frequency by which the UE may transmit V2X sidelink communication in exceptional conditions, as specified in 5.10.13.

v2x-CommTxPoolNormal Indicates the resources on a carrier frequency by which the UE may transmit V2X sidelink communication.

v2x-SyncConfig Indicates the synchronization configuration used for transmission/reception of SLSS on the given frequency.

#### –	SL-OffsetIndicator

The IE SL-OffsetIndicator indicates the offset of the pool of resources relative to SFN 0 of the cell from which it was obtained or, when out of coverage, relative to DFN 0.

SL-OffsetIndicator information element

-- ASN1START

SL-OffsetIndicator-r12 ::=			CHOICE {

small-r12								INTEGER (0..319),

large-r12								INTEGER (0..10239)

}

SL-OffsetIndicatorSync-r12 ::=			INTEGER (0..39)

SL-OffsetIndicatorSync-v1430 ::=		INTEGER (40..159)

SL-OffsetIndicatorSync-r14 ::=			INTEGER (0..159)

-- ASN1STOP

SL-OffsetIndicator field descriptions

SL-OffsetIndicator In sc-TF-ResourceConfig, it indicates the offset of the first period of pool of resources within a SFN cycle. For data-TF-ResourceConfig, it corresponds to the offsetIndicator as defined in TS 36.213 [23], clause 14.1.3.

SL-OffsetIndicatorSync For sidelink discovery and sidelink communication, synchronisation resources are present in those SFN and subframes which satisfy the relation: (SFN*10+ Subframe Number) mod 40 = SL-OffsetIndicatorSync. For V2X sidelink communication, synchronisation resources are present in those SFN and subframes which satisfy the relation: (SFN*10+ Subframe Number) mod 160 = SL-OffsetIndicatorSync.

#### –	SL-P2X-ResourceSelectionConfig

The IE SL-P2X-ResourceSelectionConfig includes the configuration of resource selection for P2X related V2X sidelink communication. E-UTRAN configures at least one resource selection mechanism.

SL-P2X-ResourceSelectionConfig information element

-- ASN1START

SL-P2X-ResourceSelectionConfig-r14 ::=			SEQUENCE {

partialSensing-r14				ENUMERATED {true}				OPTIONAL,	-- Need OR

randomSelection-r14				ENUMERATED {true}				OPTIONAL	-- Need OR

}

-- ASN1STOP

SL-P2X-ResourceSelectionConfig field descriptions

partialSensing Indicates that partial sensing is allowed for UE autonomous resource selection in a resource pool.

randomSelection Indicates that random selection is allowed for UE autonomous resource selection in a resource pool.

#### –	SL-PeriodComm

The IE SL-PeriodComm indicates the period over which resources allocated in a cell for sidelink communication.

SL-PeriodComm information element

-- ASN1START

SL-PeriodComm-r12 ::=					ENUMERATED {sf40, sf60, sf70, sf80, sf120, sf140,

sf160, sf240, sf280, sf320, spare6, spare5,

spare4, spare3, spare2, spare}

-- ASN1STOP

#### –	SL-Priority

The IE SL-Priority indicates the one or more priorities of resource pool used for sidelink communication, or of a logical channel group used in case of scheduled sidelink communication resources, see TS 36.321 [6].

SL-Priority information element

-- ASN1START

SL-PriorityList-r13 ::=		SEQUENCE (SIZE (1..maxSL-Prio-r13)) OF SL-Priority-r13

SL-Priority-r13 ::=			INTEGER (1..8)

-- ASN1STOP

#### –	SL-PSSCH-TxConfigList

The IE SL-PSSCH-TxConfigList indicates PSSCH transmission parameters. When lower layers select parameters from the range indicated in IE SL-PSSCH-TxConfigList, the UE considers both configurations in IE SL-PSSCH-TxConfigList and the CBR-dependent configurations represented in IE SL-CBR-PPPP-TxConfigList. Only one IE SL-PSSCH-TxConfig is provided per typeTxSync.

SL-PSSCH-TxConfigList information element

-- ASN1START

SL-PSSCH-TxConfigList-r14 ::=	SEQUENCE (SIZE (1..maxPSSCH-TxConfig-r14)) OF SL-PSSCH-TxConfig-r14

SL-PSSCH-TxConfig-r14 ::=		SEQUENCE {

typeTxSync-r14				SL-TypeTxSync-r14		OPTIONAL,	-- Need OR

thresUE-Speed-r14			ENUMERATED {kmph60, kmph80, kmph100, kmph120,

kmph140, kmph160, kmph180, kmph200},

parametersAboveThres-r14	SL-PSSCH-TxParameters-r14,

parametersBelowThres-r14	SL-PSSCH-TxParameters-r14,

...,

[[	parametersAboveThres-v1530	SL-PSSCH-TxParameters-v1530		OPTIONAL,	-- Need OR

parametersBelowThres-v1530	SL-PSSCH-TxParameters-v1530		OPTIONAL	-- Need OR

]]

}

SL-PSSCH-TxParameters-r14 ::=		SEQUENCE {

minMCS-PSSCH-r14			INTEGER (0..31),

maxMCS-PSSCH-r14			INTEGER (0..31),

minSubChannel-NumberPSSCH-r14		INTEGER (1..20),

maxSubchannel-NumberPSSCH-r14		INTEGER (1..20),

allowedRetxNumberPSSCH-r14	ENUMERATED {n0, n1, both, spare1},

maxTxPower-r14				SL-TxPower-r14				OPTIONAL			-- Cond CBR

}

SL-PSSCH-TxParameters-v1530 ::=		SEQUENCE {

minMCS-PSSCH-r15			INTEGER (0..31),

maxMCS-PSSCH-r15			INTEGER (0..31)

}

-- ASN1STOP

SL-PSSCH-TxConfigList field descriptions

allowedRetxNumberPSSCH Indicates the allowed retransmission number for transmissions on PSSCH (see TS 36.213 [23]). The value n0 indicates no retransmission for a transport block allowed; the value n1 indicates that the UE shall perform one retransmission for a transport block; and the value both indicates that the UE may autonomously select no retransmission or one retransmission for a transport block.

maxTxPower Indicates the maximum transmission power for transmission on PSSCH and PSCCH (see TS 36.213 [23]).

minMCS-PSSCH, maxMCS-PSSCH Indicates the minimum and maximum MCS values used for transmissions on PSSCH (see TS 36.213 [23]). If included, minMCS-PSSCH-r14 and maxMCS-PSSCH-r14 correspond to the MCS table in Table 8.6.1-1 with 64QAM indices overridden by 16QAM used for transmission on PSSCH. If included, minMCS-PSSCH-r15 and maxMCS-PSSCH-r15 correspond to both the MCS table in Table 8.6.1-1 and Table 14.1.1-2 in TS 36.213 [23] used for transmission on PSSCH.

minSubchannel-NumberPSSCH, maxSubchannel-NumberPSSCH Indicates the minimum and maximum number of sub-channels which may be used for transmissions on PSSCH (see TS 36.213 [23]).

thresUE-Speed Indicates a UE speed threshold.

typeTxSync Indicates the synchronization reference type (see TS 36.213 [23]). For configurations by the eNB, only gnss and enb can be configured; and for pre-configuration, only gnss and ue can be configured. If the field is absent, the configuration is applicable for all synchronization reference types.

parametersAboveThres Indicates TX parameters for the UE speed above thresUE-Speed.

parametersBelowThres Indicates TX parameters for the UE speed below thresUE-Speed.

| Conditional presence | Explanation |
| --- | --- |
| CBR | The field is optionally present, need OR, in IE SL-CBR-CommonTxConfigList-r14, or in IE SL-CBR-PreconfigTxConfigList-r14. Otherwise the field is not present. Need OR. |

#### –	SL-Reliability

The IE SL-Reliability indicates one or more reliabilities of a logical channel group used in case of scheduled sidelink communication resources or traffic reliability(ies) associated with the reported traffic pattern for V2X sidelink communication; see TS 36.321 [6].

SL-Reliability information element

-- ASN1START

SL-ReliabilityList-r15 ::=		SEQUENCE (SIZE (1..maxSL-Reliability-r15)) OF SL-Reliability-r15

SL-Reliability-r15 ::=			INTEGER (1..8)

-- ASN1STOP

#### –	SL-RestrictResourceReservationPeriodList

The IE SL-RestrictResourceReservationPeriodList indicates which values are allowed for the signaling of the resource reservation period in PSCCH for V2X sidelink communication, see TS 36.321 [6].

SL-RestrictResourceReservationPeriodList information element

-- ASN1START

SL-RestrictResourceReservationPeriodList-r14 ::=	SEQUENCE (SIZE (1..maxReservationPeriod-r14)) OF SL-RestrictResourceReservationPeriod-r14

SL-RestrictResourceReservationPeriod-r14 ::=		ENUMERATED {v0dot2, v0dot5, v1, v2, v3, v4, v5, v6, v7, v8, v9, v10, spare4,spare3, spare2, spare1}

-- ASN1STOP

SL-RestrictResourceReservationPeriodList field descriptions

SL-RestrictResourceReservationPeriod Value v0dot2 means SL-RestrictResourceReservationPeriod is set to 0.2, value v0dot5 means SL-RestrictResourceReservationPeriod is set to 0.5, value v1 means SL-RestrictResourceReservationPeriod is set to 1, and so on. Value v0dot2 and value v0dot5 are configured in a pool-specific manner only. E-UTRAN should not set value v0dot2 and v0dot5 for transmission pool for P2X related V2X sidelink communication.

#### –	SLSSID

The IE SLSSID identifies a cell and is used by the receiving UE to detect asynchronous neighbouring cells, and by transmitting UEs to extend the synchronisation signals beyond the cell's coverage area.

SLSSID information element

-- ASN1START

SLSSID-r12 ::=					INTEGER (0..167)

-- ASN1STOP

#### –	SL-SyncAllowed

The IE SL-SyncAllowed indicates the allowed the synchronization references for a transmission resource pool for V2X sidelink communication.

SL-SyncAllowed information element

-- ASN1START

SL-SyncAllowed-r14 ::=		SEQUENCE {

gnss-Sync-r14						ENUMERATED {true}				OPTIONAL,	-- Need OR

enb-Sync-r14						ENUMERATED {true}				OPTIONAL,	-- Need OR

ue-Sync-r14							ENUMERATED {true}				OPTIONAL	-- Need OR

}

-- ASN1STOP

SL-SyncAllowed field descriptions

enb-Sync If configured, the (pre-) configured resources can be used if the UE is directly or indirectly synchronized to eNB (i.e., synchronized to a reference UE which is directly synchronized to eNB).

gnss-Sync If configured, the (pre-) configured resources can be used if the UE is directly or indirectly synchronized to GNSS (i.e. synchronized to a reference UE which is directly synchronized to GNSS).

ue-Sync If configured, the (pre-) configured resources can be used if the UE is synchronized to a reference UE which is synchronized to neither GNSS nor eNB directly or indirectly.

#### –	SL-SyncConfig

The IE SL-SyncConfig specifies the configuration information concerning reception of synchronisation signals from neighbouring cells as well as concerning the transmission of synchronisation signals for sidelink communication and sidelink discovery.

SL-SyncConfig information element

-- ASN1START

SL-SyncConfigList-r12 ::=		SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfig-r12

SL-SyncConfigListV2X-r14 ::=	SEQUENCE (SIZE (1.. maxSL-V2X-SyncConfig-r14)) OF SL-SyncConfig-r12

SL-SyncConfig-r12 ::=					SEQUENCE {

syncCP-Len-r12							SL-CP-Len-r12,

syncOffsetIndicator-r12				SL-OffsetIndicatorSync-r12,

slssid-r12								SLSSID-r12,

txParameters-r12							SEQUENCE {

syncTxParameters-r12					SL-TxParameters-r12,

syncTxThreshIC-r12						RSRP-RangeSL-r12,

syncInfoReserved-r12						BIT STRING (SIZE (19))	OPTIONAL	-- Need OR

}																	OPTIONAL,	-- Need OR

rxParamsNCell-r12						SEQUENCE {

physCellId-r12							PhysCellId,

discSyncWindow-r12				ENUMERATED {w1, w2}

}																	OPTIONAL,	-- Need OR

...,

[[	syncTxPeriodic-r13					ENUMERATED {true}			OPTIONAL	-- Need OR

]],

[[	syncOffsetIndicator-v1430		SL-OffsetIndicatorSync-v1430	OPTIONAL,	-- Need OR

gnss-Sync-r14					ENUMERATED {true}				OPTIONAL	-- Need OR

]],

[[	syncOffsetIndicator2-r14		SL-OffsetIndicatorSync-r14	OPTIONAL,	-- Need OR

syncOffsetIndicator3-r14		SL-OffsetIndicatorSync-r14	OPTIONAL	-- Need OR

]],

[[	slss-TxDisabled-r15				ENUMERATED {true}				OPTIONAL	-- Need OR

]]

}

SL-SyncConfigListNFreq-r13 ::=		SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfigNFreq-r13

SL-SyncConfigListNFreqV2X-r14 ::=		SEQUENCE (SIZE (1..maxSL-V2X-SyncConfig-r14)) OF SL-SyncConfigNFreq-r13

SL-SyncConfigNFreq-r13 ::=			SEQUENCE {

asyncParameters-r13					SEQUENCE {

syncCP-Len-r13						SL-CP-Len-r12,

syncOffsetIndicator-r13				SL-OffsetIndicatorSync-r12,

slssid-r13							SLSSID-r12

}																OPTIONAL,	-- Need OR

txParameters-r13					SEQUENCE {

syncTxParameters-r13				SL-TxParameters-r12,

syncTxThreshIC-r13					RSRP-RangeSL-r12,

syncInfoReserved-r13					BIT STRING (SIZE (19))	OPTIONAL,	-- Need OR

syncTxPeriodic-r13					ENUMERATED {true}		OPTIONAL	-- Need OR

}																OPTIONAL,	-- Need OR

rxParameters-r13					SEQUENCE {

discSyncWindow-r13					ENUMERATED {w1, w2}

}																	OPTIONAL,	-- Need OR

...,

[[	syncOffsetIndicator-v1430		SL-OffsetIndicatorSync-v1430	OPTIONAL,	-- Need OR

gnss-Sync-r14					ENUMERATED {true}				OPTIONAL	-- Need OR

]],

[[	syncOffsetIndicator2-r14		SL-OffsetIndicatorSync-r14	OPTIONAL,	-- Need OR

syncOffsetIndicator3-r14		SL-OffsetIndicatorSync-r14	OPTIONAL	-- Need OR

]],

[[	slss-TxDisabled-r15				ENUMERATED {true}				OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

SL-SyncConfig field descriptions

discSyncWindow Indicates the synchronization window over which the UE expects that SLSS or discovery resources indicated by the pool configuration (see TS 36.213 [23], clause 14.4). The value w1 denotes 5 milliseconds. The value w2 denotes the length corresponding to normal cyclic prefix divided by 2.

gnss-Sync if configured, the synchronization configuration is used for SLSS transmission/reception when the UE is synchronized to GNSS, by using slssid=0 and ignoring slssid-r12 configured. If not configured, the synchronization configuration is used for SLSS transmission/reception when the UE is synchronized to eNB, by using the configured slssid-r12.

slss-TxDisabled Value TRUE indicates that the carrier, even though equipped with synchronisation resources, cannot be used as a synchronisation carrier frequency to transmit SLSS or PSBCH. This parameter cannot be included in SystemInformationBlockType21 or SystemInformationBlockType26.

syncCP-Len In case of V2X sidelink communications this field is always configured to normal.

syncInfoReserved Reserved for future use.

syncOffsetIndicator E-UTRAN should ensure syncOffsetIndicator is set to the same value as syncOffsetIndicator1 or syncOffsetIndicator2 in preconfigSync within SL-Preconfiguration, if configured. If syncOffsetIndicator-v1430 is configured, the UE shall ignore the field syncOffsetIndicator-r12. E-UTRAN should ensure syncOffsetIndicator is set to the same value as syncOffsetIndicator1 in v2x-CommPreconfigSync within SL-V2X-Preconfiguration, if configured. E-UTRAN should ensure syncOffsetIndicator2 is set to the same value as syncOffsetIndicator2 in v2x-CommPreconfigSync within SL-V2X-Preconfiguration, if configured. E-UTRAN should ensure syncOffsetIndicator3 is set to the same value as syncOffsetIndicator3 in v2x-CommPreconfigSync within SL-V2X-Preconfiguration, if configured. E-UTRAN should ensure all values in syncOffsetIndicator are same across all carrier frequencies configured for UEs performing V2X sidelink communication on multiple carrier frequencies. For SL-V2X-Preconfiguration, all values in syncOffsetIndicator should be same across all carrier frequencies configured for UEs performing V2X sidelink communication on multiple carrier frequencies.

syncTxPeriodic Indicates whether in each discovery period in which UE transmits discovery, the UE transmits SLSS once or periodically (i.e. every 40ms). In the latter case (periodic) the UE also transmits the MasterInformationBlock-SL message alongside. E-UTRAN configures this field only for synchronisation configurations applicable for PS discovery.

syncTxThreshIC Indicates the threshold used while in coverage. In case the RSRP measurement of the cell chosen for transmission of sidelink communication/ discovery announcements/ V2X sidelink communication, or of the cell used as reference for DL measurements and synchronization, is below the level indicated by this field, the UE may transmit SLSS (i.e. become synchronisation reference) when performing the corresponding sidelink transmission..

txParameters Includes parameters relevant only for transmission. E-UTRAN includes the field in one entry per list, as included in commSyncConfig or discSyncConfig.

#### –	SL-TF-ResourceConfig

The IE SL-TF-ResourceConfig specifies a set of time/ frequency resources used for sidelink.

SL-TF-ResourceConfig information element

-- ASN1START

SL-TF-ResourceConfig-r12 ::=		SEQUENCE	{

prb-Num-r12							INTEGER (1..100),

prb-Start-r12						INTEGER (0..99),

prb-End-r12							INTEGER (0..99),

offsetIndicator-r12					SL-OffsetIndicator-r12,

subframeBitmap-r12					SubframeBitmapSL-r12

}

SubframeBitmapSL-r12 ::=		CHOICE {

bs4-r12									BIT STRING (SIZE (4)),

bs8-r12									BIT STRING (SIZE (8)),

bs12-r12								BIT STRING (SIZE (12)),

bs16-r12								BIT STRING (SIZE (16)),

bs30-r12								BIT STRING (SIZE (30)),

bs40-r12								BIT STRING (SIZE (40)),

bs42-r12								BIT STRING (SIZE (42))

}

SubframeBitmapSL-r14 ::=		CHOICE {

bs10-r14								BIT STRING (SIZE (10)),

bs16-r14								BIT STRING (SIZE (16)),

bs20-r14								BIT STRING (SIZE (20)),

bs30-r14								BIT STRING (SIZE (30)),

bs40-r14								BIT STRING (SIZE (40)),

bs50-r14								BIT STRING (SIZE (50)),

bs60-r14								BIT STRING (SIZE (60)),

bs100-r14								BIT STRING (SIZE (100))

}

-- ASN1STOP

SL-TF-ResourceConfig field descriptions

prb-Start, prb-End, prb-Num Sidelink transmissions on a sub-frame can occur on PRB with index greater than or equal to prb-Start and less than prb-Start + prb-Num, and on PRB with index greater than prb-End - prb-Num and less than or equal to prb-End. Even for neighbouring cells, prb-Start and prb-End are relative to PRB #0 of the cell from which it was obtained. See TS 36.213 [23], clauses 14.1.3, 14.2.3 and 14.3.3.

subframeBitmap Indicates the subframe bitmap indicating resources used for sidelink. For sidelink communication, E-UTRAN configures value bs40 for FDD and the following values for TDD: value bs42 for configuration0, value bs16 for configuration1, value bs8 for configuration2, value bs12 for configuration3, value bs8 for configuration4, value bs4 for configuration5 and value bs30 for configuration6. For V2X sidelink communication, E-UTRAN configures value bs16, bs20 or bs100 for FDD or Frame Structure Type 1 as defined in TS 36.211 [21],and the following values for TDD or Frame Structure Type 2 as defined in TS 36.211 [21]: value bs60 for configuration0, value bs40 for configuration1, value bs20 for configuration2, value bs30 for configuration3, value bs20 for configuration4, value bs10 for configuration5 and value bs50 for configuration6.

#### –	SL-TxPower

The IE SL-TxPower is used to limit the UE's sidelink transmission power on a carrier frequency. The unit is dBm. Value minusinfinity corresponds to –infinity.

SL-TxPower information element

-- ASN1START

SL-TxPower-r14 ::=		CHOICE {

minusinfinity-r14				NULL,

txPower-r14						INTEGER (-41..31)

}

-- ASN1STOP

#### –	SL-TypeTxSync

The IE SL-TypeTxSync indicates the synchronization reference type.

SL-TypeTxSync information element

-- ASN1START

SL-TypeTxSync-r14 ::=		ENUMERATED {gnss, enb, ue}

-- ASN1STOP

#### –	SL-ThresPSSCH-RSRP-List

IE SL-ThresPSSCH-RSRP-List indicates a threshold used for sensing based UE autonomous resource selection (see TS 36.213 [23]). A resource is excluded if it is indicated or reserved by a decoded SCI and PSSCH RSRP in the associated data resource is above the threshold defined by IE SL-ThresPSSCH-RSRP-List.

SL-ThresPSSCH-RSRP-List information element

-- ASN1START

SL-ThresPSSCH-RSRP-List-r14 ::=	SEQUENCE (SIZE (64)) OF SL-ThresPSSCH-RSRP-r14

SL-ThresPSSCH-RSRP-r14 ::=		INTEGER (0..66)

-- ASN1STOP

SL-ThresPSSCH-RSRP-List field descriptions

SL-ThresPSSCH-RSRP Value 0 corresponds to minus infinity dBm, value 1 corresponds to -128dBm, value 2 corresponds to -126dBm, value n corresponds to (-128 + (n-1)*2) dBm and so on, value 66 corresponds to infinity dBm.

#### –	SL-TxParameters

The IE SL-TxParameters identifies a set of parameters configured for sidelink transmission, used for communication, discovery and synchronisation.

SL-TxParameters information element

-- ASN1START

SL-TxParameters-r12 ::=				SEQUENCE	{

alpha-r12								Alpha-r12,

p0-r12									P0-SL-r12

}

P0-SL-r12 ::=							INTEGER (-126..31)

-- ASN1STOP

SL-TxParameters field descriptions

alpha Parameter(s): ,,,,,,, See TS 36.213 [23], clauses 14.1.1.5, 14.2.1.3, 14.3.1 and 14.4, where al0 corresponds to 0, al04 corresponds to value 0.4, al05 to 0.5, al06 to 0.6, al07 to 0.7, al08 to 0.8, al09 to 0.9 and al1 corresponds to 1. This field applies for sidelink power control.

p0 Parameter: ,,,,,,, see TS 36.213 [23], clauses 14.1.1.5, 14.2.1.3, 14.3.1 and 14.4, unit dBm.

#### –	SL-TxPoolIdentity

The IE SL-TxPoolIdentity identifies an individual pool entry configured for sidelink transmission, used for communication and discovery.

SL-TxPoolIdentity information element

-- ASN1START

SL-TxPoolIdentity-r12 ::=			INTEGER (1.. maxSL-TxPool-r12)

SL-TxPoolIdentity-v1310 ::=		INTEGER (maxSL-TxPool-r12Plus1-r13.. maxSL-TxPool-r13)

SL-V2X-TxPoolIdentity-r14 ::=		INTEGER (1.. maxSL-V2X-TxPool-r14)

-- ASN1STOP

#### –	SL-TxPoolToReleaseList

The IE SL-TxPoolToReleaseList is used to release one or more individual pool entries used for sidelink transmission, for communication and discovery.

SL-TxPoolToReleaseList information element

-- ASN1START

SL-TxPoolToReleaseList-r12 ::=	SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-TxPoolIdentity-r12

SL-TxPoolToReleaseListExt-r13 ::=	SEQUENCE (SIZE (1..maxSL-TxPool-v1310)) OF SL-TxPoolIdentity-v1310

-- ASN1STOP

#### –	SL-V2X-ConfigDedicated

The IE SL-V2X-ConfigDedicated specifies the dedicated configuration information for V2X sidelink communication.

SL-V2X-ConfigDedicated information element

-- ASN1START

SL-V2X-ConfigDedicated-r14 ::=				SEQUENCE	{

commTxResources-r14					CHOICE {

release								NULL,

setup								CHOICE {

scheduled-r14					SEQUENCE {

sl-V-RNTI-r14			C-RNTI,

mac-MainConfig-r14				MAC-MainConfigSL-r12,

v2x-SchedulingPool-r14			SL-CommResourcePoolV2X-r14	OPTIONAL,	-- Need ON

mcs-r14							INTEGER (0..31)				OPTIONAL,	-- Need OR

logicalChGroupInfoList-r14		LogicalChGroupInfoList-r13

},

ue-Selected-r14					SEQUENCE {

-- Pool for normal usage

v2x-CommTxPoolNormalDedicated-r14	SEQUENCE {

poolToReleaseList-r14	SL-TxPoolToReleaseListV2X-r14	OPTIONAL,	-- Need ON

poolToAddModList-r14		SL-TxPoolToAddModListV2X-r14		OPTIONAL,	-- Need ON

v2x-CommTxPoolSensingConfig-r14		SL-CommTxPoolSensingConfig-r14

OPTIONAL	-- Need ON

}

}

}

}																		OPTIONAL,	-- Need ON

v2x-InterFreqInfoList-r14			SL-InterFreqInfoListV2X-r14			OPTIONAL,	-- Need ON

thresSL-TxPrioritization-r14			SL-Priority-r13						OPTIONAL,	-- Need OR

typeTxSync-r14						SL-TypeTxSync-r14					OPTIONAL,	-- Need OR

cbr-DedicatedTxConfigList-r14		SL-CBR-CommonTxConfigList-r14	OPTIONAL,	-- Need OR

...,

[[	commTxResources-v1530					CHOICE {

release								NULL,

setup								CHOICE {

scheduled-v1530					SEQUENCE {

logicalChGroupInfoList-v1530	LogicalChGroupInfoList-v1530	OPTIONAL,	-- Need OR

mcs-r15							INTEGER (0..31)			OPTIONAL	-- Need OR

},

ue-Selected-v1530				SEQUENCE {

v2x-FreqSelectionConfigList-r15	SL-V2X-FreqSelectionConfigList-r15	OPTIONAL	--Need OR

}

}

}																OPTIONAL,		-- Need ON

v2x-PacketDuplicationConfig-r15	SL-V2X-PacketDuplicationConfig-r15	OPTIONAL,	-- Need OR

syncFreqList-r15				SL-V2X-SyncFreqList-r15				OPTIONAL,	-- Need OR

slss-TxMultiFreq-r15			ENUMERATED {true}					OPTIONAL	-- Need OR

]],

[[

slss-TxDisabled-r15			ENUMERATED {true}					OPTIONAL	-- Need OR

]]

}

LogicalChGroupInfoList-v1530 ::=		SEQUENCE (SIZE (1..maxLCG-r13)) OF SL-ReliabilityList-r15

SL-TxPoolToAddModListV2X-r14 ::=		SEQUENCE (SIZE (1.. maxSL-V2X-TxPool-r14)) OF SL-TxPoolToAddMod-r14

SL-TxPoolToAddMod-r14 ::=		SEQUENCE	{

poolIdentity-r14					SL-V2X-TxPoolIdentity-r14,

pool-r14							SL-CommResourcePoolV2X-r14

}

SL-TxPoolToReleaseListV2X-r14 ::=	SEQUENCE (SIZE (1.. maxSL-V2X-TxPool-r14)) OF SL-V2X-TxPoolIdentity-r14

-- ASN1STOP

SL-V2X-ConfigDedicated field descriptions

cbr-DedicatedTxConfigList Indicates the dedicated list of CBR range division and the list of PSCCH TX configurations available to configure congestion control to the UE for V2X sidelink communication.

logicalChGroupInfoList Indicates for each logical channel group the list of associated priorities and reliabilities, used as specified in TS 36.321 [6], in order of increasing logical channel group identity. If E-UTRAN includes logicalChGroupInfoList-v1530, it includes the same number of entries, and listed in the same order, as in logicalChGroupInfoList–r14, and a logical channel group identity of the same entry in logicalChGroupInfoList-r14 and in logicalChGroupInfo-v1530 is associated with both the priorties (as in logicalChGroupInfoList-r14) and reliablities (as in logicalChGroupInfoList-v1530) of that entry. If logicalChGroupInfoList-v1530 is not included, this field indicates for each logical channel group the list of associated priorities.

mcs Indicates the MCS as defined in TS 36.213 [23], clause 14.2.1. If not configured, the selection of MCS is up to UE implementation. If included, mcs-r14 corresponds to the MCS table in Table 8.6.1-1 with 64QAM indices overridden by 16QAM used for transmission on PSSCH. If included, mcs-r15 corresponds to both the MCS table in Table 8.6.1-1 in TS 36.213 [23] and the MCS table supporting 64QAM in Table 14.1.1-2 in TS 36.213 [23] used for transmission on PSSCH. If this field is present, E-UTRAN shall configure both mcs-r14 and mcs-r15.

scheduled Indicates the configuration for the case E-UTRAN schedules the transmission resources based on sidelink specific BSR from the UE.

sl-V-RNTI Indicates the RNTI used for DCI dynamically scheduling sidelink resources for V2X sidelink communication.

slss-TxDisabled Value TRUE indicates that the primary carrier, even though equipped with synchronisation resources, cannot be used as a synchronisation carrier frequency to transmit SLSS or PSBCH.

thresSL-TxPrioritization Indicates the threshold used to determine whether SL V2X transmission is prioritized over uplink transmission if they overlap in time (see TS 36.321 [6]). This value shall overwrite thresSL-TxPrioritization configured in SIB21 or SL-V2X-Preconfiguration if any.

typeTxSync Indicates the prioritized synchronization type (i.e. eNB or GNSS) for performing V2X sidelink communication on PCell.

ue-Selected Indicates the configuration for the case the UE selects the transmission resources from a pool of resources configured by E-UTRAN.

v2x-InterFreqInfoList Indicates synchronization and resource allocation configurations of other carrier frequencies than the serving carrier frequency for V2X sidelink communication. For inter-carrier scheduled resource allocation, CIF=1 in DCI-5A corresponds to the first entry in this frequency list, CIF=2 corresponds to the second entry, and so on (see TS 36.213 [23]). CIF=0 in DCI-5A corresponds to the frequency where the DCI is received.

v2x-SchedulingPool Indicates a pool of resources when E-UTRAN schedules Tx resources for V2X sidelink communications.

#### –	SL-V2X-FreqSelectionConfigList

The IE SL-V2X-FreqSelectionConfigList specifies the configuration information for carrier selection for V2X sidelink communication transmission using UE autonomous resource selection.

SL-V2X-FreqSelectionConfigList information element

-- ASN1START

SL-V2X-FreqSelectionConfigList-r15 ::=	SEQUENCE (SIZE (1..8)) OF SL-V2X-FreqSelectionConfig-r15

SL-V2X-FreqSelectionConfig-r15 ::=				SEQUENCE {

priorityList-r15					SL-PriorityList-r13,

threshCBR-FreqReselection-r15		SL-CBR-r14			OPTIONAL,	-- Need OR

threshCBR-FreqKeeping-r15			SL-CBR-r14			OPTIONAL	-- Need OR

}

-- ASN1STOP

SL-V2X-FreqSelectionConfig field descriptions

priorityList Indicates the list of PPPP(s) which is associated with the configurations in threshCBR-FreqReselection and in threshCBR-FreqKeeping.

threshCBR-FreqReselection Indicates the CBR threshold to determine whether the carrier frequency can be (re)selected for the transmission of V2X sidelink communication. See TS 36.321 [6].

threshCBR-FreqKeeping Indicates the CBR threshold to determine whether the UE can keep using the carrier which was selected for the transmission of V2X sidelink communication. See TS 36.321 [6].

#### –	SL-V2X-PacketDuplicationConfig

The IE SL-V2X-PacketDuplicationConfig specifies the configuration information for sidelink packet duplication for V2X sidelink communication transmission.

SL-V2X-PacketDuplicationConfig information element

-- ASN1START

SL-V2X-PacketDuplicationConfig-r15 ::=	SEQUENCE {

threshSL-Reliability-r15		SL-Reliability-r15,

allowedCarrierFreqConfig-r15	SL-PPPR-Dest-CarrierFreqList-r15		OPTIONAL,	-- Need OR

...

}

SL-PPPR-Dest-CarrierFreqList-r15 ::=	SEQUENCE (SIZE (1..maxSL-Dest-r12)) OF SL-PPPR-Dest-CarrierFreq

SL-PPPR-Dest-CarrierFreq ::=	SEQUENCE {

destinationInfoList-r15			SL-DestinationInfoList-r12		OPTIONAL,	-- Need OR

allowedCarrierFreqList-r15		SL-AllowedCarrierFreqList-r15			OPTIONAL	-- Need OR

}

SL-AllowedCarrierFreqList-r15 ::=	SEQUENCE {

allowedCarrierFreqSet1		SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF ARFCN-ValueEUTRA-r9,

allowedCarrierFreqSet2		SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF ARFCN-ValueEUTRA-r9

}

-- ASN1STOP

SL-V2X-PacketDuplicationConfig field descriptions

allowedCarrierFreqList, allowedCarrierFreqSet1, allowedCarrierFreqSet2 Indicates, for V2X sidelink communication, the set of carrier frequencies applicable for the transmission of the MAC SDUs from the sidelink logical channels whose associated destination are included in destinationInfoList (see TS 36.321 [6]). If present, E-UTRAN shall ensure allowedCarrierFreqSet1 and allowedCarrierFreqSet2 do not include the same carrier frequency.

threshSL-Reliability Indicates the reliability threshold used to determine whether sidelinik packet duplication is configured and activated for V2X sidelink communication transmission. See TS 36.323 [8] and TS 36.321 [6].

#### –	SL-V2X-SyncFreqList

The IE SL-V2X-SyncFreqList specifies the list of candidate synchronisation carrier frequencies used for V2X sidelink communication.

SL-V2X-SyncFreqList information element

-- ASN1START

SL-V2X-SyncFreqList-r15 ::= SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF ARFCN-ValueEUTRA-r9

-- ASN1STOP

#### –	SL-ZoneConfig

The IE SL-ZoneConfig indicates zone configurations used for V2X sidelink communication.

SL-ZoneConfig information element

-- ASN1START

SL-ZoneConfig-r14 ::=		SEQUENCE {

zoneLength-r14	ENUMERATED { m5, m10, m20, m50, m100, m200, m500, spare1},

zoneWidth-r14	ENUMERATED { m5, m10, m20, m50, m100, m200, m500, spare1},

zoneIdLongiMod-r14	INTEGER (1..4),

zoneIdLatiMod-r14	INTEGER (1..4)

}

-- ASN1STOP

SL-ZoneConfig field descriptions

zoneLength Indicates the length of each geographic zone. Value m5 corresponds to 5 meters, m10 corresponds to 10 meters and so on.

zoneWidth Indicates the width of each geographic zone. Value m5 corresponds to 5 meters, m10 corresponds to 10 meters and so on.

zoneIdLongiMod Indicates the total number of zones that is configured with respect to longitude.

zoneIdLatiMod Indicates the total number of zones that is configured with respect to latitude.

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

ffsValue					INTEGER ::= 65536 -- Placeholder for all FFS value

maxAccessCat-1-r15			INTEGER ::=	63	-- Maximum number of Access Categories - 1

maxACDC-Cat-r13				INTEGER ::=	16	-- Maximum number of ACDC categories (per PLMN)

maxAvailNarrowBands-r13		INTEGER ::=	16	-- Maximum number of narrowbands

maxAvailNarrowBands-1-r16	INTEGER ::= 15	-- Maximum number of narrowbands minus one

maxBandComb-r10				INTEGER ::=	128	-- Maximum number of band combinations.

maxBandComb-r11				INTEGER ::=	256	-- Maximum number of additional band combinations.

maxBandComb-r13				INTEGER ::=	384 -- Maximum number of band combinations in Rel-13

maxBandCombSidelinkNR-r16	INTEGER ::=	512	-- Maximum number of NR sidelink band combinations

maxBands					INTEGER ::= 64	-- Maximum number of bands listed in EUTRA UE caps

maxBandsNR-r15				INTEGER ::= 1024	-- Maximum number of NR bands listed in EUTRA UE caps

maxBandsENDC-r16			INTEGER ::= 10	-- Maximum number of NR bands from across all the PLMNs

-- sharing the serving cell in EN-DC for the forwarding

-- of upperLayerIndication.

maxBandwidthClass-r10		INTEGER ::=	16	-- Maximum number of supported CA BW classes per band

maxBandwidthCombSet-r10		INTEGER ::=	32	-- Maximum number of bandwidth combination sets per

-- supported band combination

maxBarringInfoSet-r15		INTEGER ::= 8	-- Maximum number of UAC barring information sets

maxBT-IdReport-r15			INTEGER ::= 32	-- Maximum number of Bluetooth IDs to report

maxBT-Name-r15				INTEGER ::= 4	-- Maximum number of Bluetooth name

maxCBR-Level-r14			INTEGER ::= 16	-- Maximum number of CBR levels

maxCBR-Level-1-r14			INTEGER ::= 15

maxCBR-Report-r14			INTEGER ::= 72	-- Maximum number of CBR results in a report

maxCDMA-BandClass			INTEGER ::= 32	-- Maximum value of the CDMA band classes

maxCE-Level-r13				INTEGER ::=	4	-- Maximum number of CE levels

maxCellBlack				INTEGER ::= 16	-- Maximum number of blacklisted physical cell identity

-- ranges listed in SIB type 4 and 5

maxCellHistory-r12			INTEGER ::= 16	-- Maximum number of visited EUTRA cells reported

maxCellInfoGERAN-r9		INTEGER ::=	32	-- Maximum number of GERAN cells for which system in-

-- formation can be provided as redirection assistance

maxCellInfoUTRA-r9			INTEGER ::=	16	-- Maximum number of UTRA cells for which system

-- information can be provided as redirection

-- assistance

maxCellMeasIdle-r15			INTEGER ::= 8	-- Maximum number of neighbouring inter-frequency

-- cells per carrier measured in RRC_IDLE and RRC_INACTIVE

maxCombIDC-r11				INTEGER ::= 128	-- Maximum number of reported UL CA or

-- MR-DC combinations

maxCSI-IM-r11				INTEGER ::= 3	-- Maximum number of CSI-IM configurations

-- (per carrier frequency)

maxCSI-IM-r12				INTEGER ::= 4	-- Maximum number of CSI-IM configurations

-- (per carrier frequency)

minCSI-IM-r13				INTEGER ::= 5	-- Minimum number of CSI IM configurations from which

-- REL-13 extension is used

maxCSI-IM-r13				INTEGER ::= 24	-- Maximum number of CSI-IM configurations

-- (per carrier frequency)

maxCSI-IM-v1310				INTEGER ::= 20	-- Maximum number of additional CSI-IM configurations

-- (per carrier frequency)

maxCSI-Proc-r11				INTEGER ::= 4	-- Maximum number of CSI processes (per carrier

-- frequency)

maxCSI-RS-NZP-r11			INTEGER ::= 3	-- Maximum number of CSI RS resource

-- configurations using non-zero Tx power

-- (per carrier frequency)

minCSI-RS-NZP-r13			INTEGER ::= 4	-- Minimum number of CSI RS resource from which

-- REL-13 extension is used

maxCSI-RS-NZP-r13			INTEGER ::= 24	-- Maximum number of CSI RS resource

-- configurations using non-zero Tx power

-- (per carrier frequency)

maxCSI-RS-NZP-v1310			INTEGER ::= 21	-- Maximum number of additional CSI RS resource

-- configurations using non-zero Tx power

-- (per carrier frequency)

maxCSI-RS-ZP-r11			INTEGER ::= 4	-- Maximum number of CSI RS resource

-- configurations using zero Tx power(per carrier

-- frequency)

maxCQI-ProcExt-r11			INTEGER ::= 3	-- Maximum number of additional periodic CQI

-- configurations (per carrier frequency)

maxFreqUTRA-TDD-r10			INTEGER ::=	6	-- Maximum number of UTRA TDD carrier frequencies for

-- which system information can be provided as

-- redirection assistance

maxCellInter				INTEGER ::= 16	-- Maximum number of neighbouring inter-frequency

-- cells listed in SIB type 5

maxCellIntra				INTEGER ::= 16	-- Maximum number of neighbouring intra-frequency

-- cells listed in SIB type 4

maxCellListGERAN			INTEGER ::= 3	-- Maximum number of lists of GERAN cells

maxCellMeas					INTEGER ::= 32	-- Maximum number of entries in each of the

-- cell lists in a measurement object

maxCellReport				INTEGER ::= 8	-- Maximum number of reported cells/CSI-RS resources

maxCellSFTD				INTEGER ::= 3	-- Maximum number of cells for SFTD reporting

maxCellWhiteNR-r16			INTEGER ::= 16	-- Maximum number of whitelisted NR cells in SIB24

maxCondConfig-r16			INTEGER ::= 8	-- Maximum number of conditional configurations

maxConfigSPS-r14			INTEGER ::= 8	-- Maximum number of simultaneous SPS configurations

maxConfigSPS-r15			INTEGER ::= 6	-- Maximum number of simultaneous SPS configurations

-- configured with SPS C-RNTI

maxCSI-RS-Meas-r12			INTEGER ::= 96	-- Maximum number of entries in the CSI-RS list

-- in a measurement object

maxDRB						INTEGER ::= 11	-- Maximum number of Data Radio Bearers

maxDRBExt-r15				INTEGER ::= 4	-- Maximum number of additional DRBs

maxDRB-r15					INTEGER ::= 15	-- Highest value of extended maximum number of DRBs

maxDS-Duration-r12			INTEGER ::= 5	-- Maximum number of subframes in a discovery signals

-- occasion

maxDS-ZTP-CSI-RS-r12		INTEGER ::= 5	-- Maximum number of zero transmission power CSI-RS for

-- a serving cell concerning discovery signals

maxEARFCN					INTEGER ::= 65535	-- Maximum value of EUTRA carrier frequency

maxEARFCN-Plus1				INTEGER ::= 65536	-- Lowest value extended EARFCN range

maxEARFCN2					INTEGER ::= 262143	-- Highest value extended EARFCN range

maxEPDCCH-Set-r11			INTEGER ::= 2	-- Maximum number of EPDCCH sets

maxFBI						INTEGER ::= 64	-- Maximum value of fequency band indicator

maxFBI-NR-r15				INTEGER ::= 1024	-- Highest value FBI range for NR.

maxFBI-Plus1				INTEGER ::= 65	-- Lowest value extended FBI range

maxFBI2						INTEGER ::= 256	-- Highest value extended FBI range

maxFeatureSets-r15			INTEGER ::= 256	-- Total number of feature sets (size of pool)

maxPerCC-FeatureSets-r15	INTEGER ::= 32	-- Total number of CC-specific feature sets

-- (size of the pool)

maxFreq						INTEGER ::= 8	-- Maximum number of carrier frequencies

maxFreq-1-r16				INTEGER ::= 7	-- Maximum number of carrier frequencies

maxFreqIDC-r11				INTEGER ::= 32	-- Maximum number of carrier frequencies that are

-- affected by the IDC problems

maxFreqIdle-r15				INTEGER ::= 8	-- Maximum number of carrier frequencies for

-- IDLE mode measurements configured by eNB

maxFreqMBMS-r11				INTEGER ::= 5	-- Maximum number of carrier frequencies for which an

-- MBMS capable UE may indicate an interest

maxFreqNBIOT-r16			INTEGER ::= 8	-- Maximum number of NB-IoT carrier frequencies that can

-- be provided as assistance information for inter-RAT

-- cell selection

maxFreqNR-r15				INTEGER ::= 5	-- Maximum number of NR carrier frequencies for

-- which a UE may provide measurement results upon

-- NR SCG failure

maxFreqSL-NR-r16			INTEGER ::= 8	-- Maximum number of NR anchor carrier frequencies on

-- which configurations for V2X sidelink communication

-- are provided

maxFreqV2X-r14				INTEGER ::= 8	-- Maximum number of carrier frequencies for which V2X

-- sidelink communication can be configured

maxFreqV2X-1-r14			INTEGER ::= 7	-- Highest index of frequencies

maxGERAN-SI					INTEGER ::= 10	-- Maximum number of GERAN SI blocks that can be

-- provided as part of NACC information

maxGNFG						INTEGER ::= 16	-- Maximum number of GERAN neighbour freq groups

maxGWUS-Groups-1-r16		INTEGER ::= 31	-- Maximum number of groups minus one for each

-- probability group

maxGWUS-Resources-r16		INTEGER	::= 4	-- Maximum number of GWUS resources for each group

maxGWUS-ProbThresholds-r16	INTEGER	::= 3	-- Maximum number of paging probability thresholds

maxIdleMeasCarriers-r15		INTEGER ::= 3	-- Maximum number of neighbouring inter-

-- frequency carriers measured in RRC_IDLE and RRC_INACTIVE

maxIdleMeasCarriersExt-r16		INTEGER ::= 5	--Additional number of neighbouring inter-

-- frequency carriers measured in RRC_IDLE and RRC_INACTIVE

maxIdleMeasCarriers-r16		INTEGER ::= 8	-- Maximum number of neighbouring inter-

-- frequency/inter-RAT carriers measured in RRC_IDLE and RRC_INACTIVE

maxLCG-r13					INTEGER ::= 4	-- Maximum number of logical channel groups

maxLogMeasReport-r10		INTEGER ::= 520	-- Maximum number of logged measurement entries

-- that can be reported by the UE in one message

maxMBSFN-Allocations		INTEGER ::= 8	-- Maximum number of MBSFN frame allocations with

-- different offset

maxMBSFN-Area				INTEGER ::= 8

maxMBSFN-Area-1				INTEGER ::= 7

maxMBMS-ServiceListPerUE-r13	INTEGER ::= 15	-- Maximum number of services which the UE can

-- include in the MBMS interest indication

maxMeasId					INTEGER ::= 32

maxMeasId-Plus1				INTEGER ::= 33

maxMeasId-r12				INTEGER ::= 64

maxMultiBands				INTEGER ::= 8	-- Maximum number of additional frequency bands

-- that a cell belongs to

maxMultiBandsNR-r15			INTEGER ::= 32	-- Maximum number of additional NR frequency bands

-- that a cell belongs to

maxMultiBandsNR-1-r15		INTEGER ::= 31

maxNS-Pmax-r10				INTEGER ::= 8	-- Maximum number of NS and P-Max values per band

maxNAICS-Entries-r12		INTEGER ::= 8	-- Maximum number of supported NAICS combination(s)

maxNeighCell-r12			INTEGER ::= 8	-- Maximum number of neighbouring cells in NAICS

-- configuration (per carrier frequency)

maxNeighCell-SCPTM-r13		INTEGER ::= 8	-- Maximum number of SCPTM neighbour cells

maxNrofPCI-PerSMTC-r16		INTEGER ::= 64  -- Maximum number of PCIs per SMTC

maxNrofS-NSSAI-r15			INTEGER ::= 8	-- Maximum number of S-NSSAI

maxObjectId					INTEGER ::= 32

maxObjectId-Plus1-r13		INTEGER ::= 33

maxObjectId-r13				INTEGER ::= 64

maxP-a-PerNeighCell-r12		INTEGER ::= 3	-- Maximum number of power offsets for a neighbour cell

-- in NAICS configuration

maxPageRec					INTEGER ::= 16	--

maxPhysCellIdRange-r9		INTEGER ::= 4	-- Maximum number of physical cell identity ranges

maxPLMN-r11					INTEGER ::=	6	-- Maximum number of PLMNs

maxPLMN-1-r14				INTEGER ::=	5	-- Maximum number of PLMNs minus one

maxPLMN-r15					INTEGER ::= 8	-- Maximum number of PLMNs for RNA configuration

maxPLMN-NR-r15				INTEGER ::= 12	-- Maximum number of NR PLMNs

maxPNOffset					INTEGER ::=	511	-- Maximum number of CDMA2000 PNOffsets

maxPMCH-PerMBSFN			INTEGER ::= 15

maxPSSCH-TxConfig-r14		INTEGER ::= 16	-- Maximum number of PSSCH TX configurations

maxQuantSetsNR-r15			INTEGER ::= 2	-- Maximum number of NR quantity configuration sets

maxQCI-r13					INTEGER ::= 6	-- Maximum number of QCIs

maxRAT-Capabilities			INTEGER ::= 8	-- Maximum number of interworking RATs (incl EUTRA)

maxRE-MapQCL-r11			INTEGER ::= 4	-- Maximum number of PDSCH RE Mapping configurations

-- (per carrier frequency)

maxReportConfigId			INTEGER ::= 32

maxReservationPeriod-r14	INTEGER ::= 16	-- Maximum number of resource reservation periodicities

-- for sidelink V2X communication

maxRS-Index-r15				INTEGER ::= 64	-- Maximum number of RS indices

maxRS-Index-1-r15			INTEGER ::= 63	-- Highest value of RS index as used to identify

-- RS index in RRM reports.

maxRS-IndexCellQual-r15		INTEGER ::= 16	-- Maximum number of RS indices averaged to derive

-- cell quality for RRM.

maxRS-IndexReport-r15		INTEGER ::= 32	-- Maximum number of RS indices for RRM.

maxRSTD-Freq-r10			INTEGER ::= 3	-- Maximum number of frequency layers for RSTD

-- measurement

maxSAI-MBMS-r11				INTEGER ::= 64	-- Maximum number of MBMS service area identities

-- broadcast per carrier frequency

maxSCell-r10				INTEGER ::= 4	-- Maximum number of SCells

maxSCell-r13				INTEGER ::= 31	-- Highest value of extended number range of SCells

maxSCellGroups-r15			INTEGER ::= 4	-- Maximum number of SCell common parameter groups

maxSC-MTCH-r13				INTEGER ::= 1023	-- Maximum number of SC-MTCHs in one cell

maxSC-MTCH-BR-r14			INTEGER ::= 128	-- Maximum number of SC-MTCHs in one cell for feMTC

maxSL-CommRxPoolNFreq-r13	INTEGER ::= 32	-- Maximum number of individual sidelink communication

-- Rx resource pools on neighbouring freq

maxSL-CommRxPoolPreconf-v1310	INTEGER ::= 12	-- Maximum number of additional preconfigured

-- sidelink communication Rx resource pool entries

maxSL-TxPool-r12Plus1-r13	INTEGER ::= 5	-- First additional individual sidelink

-- Tx resource pool

maxSL-TxPool-v1310			INTEGER ::= 4	-- Maximum number of additional sidelink

-- Tx resource pool entries

maxSL-TxPool-r13			INTEGER ::= 8	-- Maximum number of individual sidelink

-- Tx resource pools

maxSL-CommTxPoolPreconf-v1310	INTEGER ::= 7	-- Maximum number of additional preconfigured

-- sidelink Tx resource pool entries

maxSL-Dest-r12			INTEGER ::= 16			-- Maximum number of sidelink destinations

maxSL-DiscCells-r13		INTEGER ::= 16			-- Maximum number of cells with similar sidelink

-- configurations

maxSL-DiscPowerClass-r12	INTEGER ::= 3		-- Maximum number of sidelink power classes

maxSL-DiscRxPoolPreconf-r13		INTEGER ::= 16	-- Maximum number of preconfigured sidelink

-- discovery Rx resource pool entries

maxSL-DiscSysInfoReportFreq-r13	INTEGER ::= 8	-- Maximum number of frequencies to include in a

-- SidelinkUEInformation for SI reporting

maxSL-DiscTxPoolPreconf-r13		INTEGER ::= 4	-- Maximum number of preconfigured sidelink

-- discovery Tx resource pool entries

maxSL-GP-r13			INTEGER ::= 8	-- Maximum number of gap patterns that can be requested

-- for a frequency or assigned

maxSL-PoolToMeasure-r14	INTEGER ::= 72	-- Maximum number of TX resource pools for CBR

-- measurement and report

maxSL-Prio-r13			INTEGER ::= 8	-- Maximum number of entries in sidelink priority list

maxSL-RxPool-r12			INTEGER ::= 16	-- Maximum number of individual sidelink Rx resource pools

maxSL-Reliability-r15	INTEGER ::= 8	-- Maximum number of entries in sidelink reliability list

maxSL-SyncConfig-r12		INTEGER ::= 16	-- Maximum number of sidelink Sync configurations

maxSL-TF-IndexPair-r12	INTEGER ::= 64	-- Maximum number of sidelink Time Freq resource index

-- pairs

maxSL-TxPool-r12			INTEGER ::= 4	-- Maximum number of individual sidelink Tx resource pools

maxSL-V2X-RxPool-r14		INTEGER ::= 16	-- Maximum number of RX resource pools for

-- V2X sidelink communication

maxSL-V2X-RxPoolPreconf-r14	INTEGER ::= 16		-- Maximum number of RX resource pools for

-- V2X sidelink communication

maxSL-V2X-TxPool-r14		INTEGER ::= 8	-- Maximum number of TX resource pools for

-- V2X sidelink communication

maxSL-V2X-TxPoolPreconf-r14	INTEGER ::= 8		-- Maximum number of TX resource pools for

-- V2X sidelink communication

maxSL-V2X-SyncConfig-r14	INTEGER ::= 16	-- Maximum number of sidelink Sync configurations

-- for V2X sidelink communication

maxSL-V2X-CBRConfig-r14		INTEGER ::= 4	-- Maximum number of CBR range configurations

-- for V2X sidelink communication congestion

-- control

maxSL-V2X-CBRConfig-1-r14	INTEGER ::= 3

maxSL-V2X-TxConfig-r14		INTEGER ::= 64	-- Maximum number of TX parameter configurations

-- for V2X sidelink communication congestion

-- control

maxSL-V2X-TxConfig-1-r14	INTEGER ::= 63

maxSL-V2X-CBRConfig2-r14		INTEGER ::= 8	-- Maximum number of CBR range configurations in

-- pre-configuration for V2X sidelink

-- communication congestion control

maxSL-V2X-CBRConfig2-1-r14	INTEGER ::= 7

maxSL-V2X-TxConfig2-r14		INTEGER ::= 128	-- Maximum number of TX parameter

-- configurations in pre-configuration for V2X

-- sidelink communication congestion control

maxSL-V2X-TxConfig2-1-r14	INTEGER ::= 127

maxSTAG-r11					INTEGER ::= 3	-- Maximum number of STAGs

maxServCell-r10				INTEGER ::= 5	-- Maximum number of Serving cells

maxServCell-r13				INTEGER ::= 32	-- Highest value of extended number range of Serving cells

maxServCellNR-r15			INTEGER ::= 16	-- Maximum number of NR serving cells

maxServiceCount			INTEGER ::= 16	-- Maximum number of MBMS services that can be included

-- in an MBMS counting request and response

maxServiceCount-1			INTEGER ::= 15

maxSessionPerPMCH			INTEGER ::= 29

maxSessionPerPMCH-1			INTEGER ::= 28

maxSIB						INTEGER ::= 32	-- Maximum number of SIBs

maxSIB-1					INTEGER ::= 31

maxSI-Message				INTEGER ::= 32	-- Maximum number of SI messages

maxSimultaneousBands-r10	INTEGER ::= 64	-- Maximum number of simultaneously aggregated bands

maxSubframePatternIDC-r11	INTEGER ::= 8	-- Maximum number of subframe reservation patterns

-- that the UE can simultaneously recommend to the

-- E-UTRAN for use

maxTAC-r17					INTEGER	::=	ffsValue	-- Maximum number of Tracking Area Codes

-- broadcast in a cell

maxTrafficPattern-r14		INTEGER ::= 8	-- Maximum number of periodical traffic patterns

-- that the UE can simultaneously report to the

-- E-UTRAN.

maxUTRA-FDD-Carrier			INTEGER ::= 16	-- Maximum number of UTRA FDD carrier frequencies

maxUTRA-TDD-Carrier			INTEGER ::= 16	-- Maximum number of UTRA TDD carrier frequencies

maxWayPoint-r15				INTEGER ::= 20	-- Maximum number of flight path information waypoints

maxWLAN-Id-r12				INTEGER ::=	16	-- Maximum number of WLAN identifiers

maxWLAN-Bands-r13			INTEGER ::= 8	-- Maximum number of WLAN bands

maxWLAN-Id-r13				INTEGER ::= 32	-- Maximum number of WLAN identifiers

maxWLAN-Channels-r13		INTEGER ::= 16	-- maximum number of WLAN channels used in

-- WLAN-CarrierInfo

maxWLAN-CarrierInfo-r13	INTEGER ::= 8	-- Maximum number of WLAN Carrier Information

maxWLAN-Id-Report-r14		INTEGER ::= 32	-- Maximum number of WLAN IDs to report

maxWLAN-Name-r15			INTEGER ::= 4	-- Maximum number of WLAN name

-- ASN1STOP

NOTE: The value of maxDRB aligns with SA2.

### –	End of EUTRA-RRC-Definitions

-- ASN1START

END

-- ASN1STOP

Next change

## 6.6	Direct Indication Information

Direct Indication information is transmitted on MPDCCH using P-RNTI but without associated Paging message or using SI-RNTI. Table 6.6-1 defines the Direct Indication information on MPDCCH using P-RNTI, see TS 36.212 [22], clause 5.3.3.1.14. Table 6.6-2 defines the Direct Indication on MPDCCH using SI-RNTI in RRC_CONNECTED, see TS 36.212 [22], clauses 5.3.3.1.12 and 5.3.3.1.13.

When bit n is set to 1, UE shall behave as if the corresponding field is set in the Paging message, see 5.3.2.3. Bit 1 is the least significant bit.

Table 6.6-1: Direct Indication information using P-RNTI

| Bit | Direct Indication information |
| --- | --- |
| 1 | systemInfoModification |
| 2 | etws-Indication |
| 3 | cmas-Indication |
| 4 | eab-ParamModification |
| 5 | systemInfoModification-eDRX |
| 6 | uac-ParamModification |
| 6, 7, 8 | Not used, and shall be ignored by UE if received. |

Table 6.6-2: Direct Indication information using SI-RNTI

| Bit | Direct Indication information |
| --- | --- |
| 1 | etws-Indication |
| 2 | cmas-Indication |
| 3, 4, 5, 6, 7, 8 | Not used, and shall be ignored by UE if received. |

Editor’s Note: Agreement: System information update notification procedure is not used to inform TAC updates, at least for TAC additions (FFS removals).

Next change

## 6.7	NB-IoT RRC messages

### 6.7.1	General NB-IoT message structure

-- ASN1START

NBIOT-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

TypeFFS,		-- to be removed

ffsValue,		-- to be removed

RRCConnectionReestablishmentReject,

SecurityModeCommand,

SecurityModeComplete,

SecurityModeFailure,

AdditionalSpectrumEmission,

ARFCN-ValueEUTRA-r9,

CarrierFreqsGERAN,

CellGlobalIdEUTRA,

CellIdentity,

C-RNTI,

DedicatedInfoNAS,

DRB-Identity,

InitialUE-Identity,

IntraFreqBlackCellList,

IntraFreqNeighCellList,

I-RNTI-r15,

LocationInfo-r10,

maxAccessCat-1-r15,

maxBands,

maxCellBlack,

maxCellInter,

maxCellIntra,

maxFBI2,

maxFreq,

maxMultiBands,

maxNrofS-NSSAI-r15,

maxPageRec,

maxPLMN-r11,

maxSAI-MBMS-r11,

maxSIB,

maxSIB-1,

MBMS-SAI-r11,

MBMS-SAI-List-r11,

MBMSSessionInfo-r13,

NextHopChainingCount,

NG-5G-S-TMSI-r15,

PagingUE-Identity,

PLMN-Identity,

PLMN-IdentityList2,

P-Max,

PowerRampingParameters,

PreambleTransMax,

PhysCellId,

Q-OffsetRange,

Q-QualMin-r9,

Q-RxLevMin,

ReestabUE-Identity,

RegisteredAMF-r15,

RegisteredMME,

ReselectionThreshold,

ResumeIdentity-r13,

RRC-TransactionIdentifier,

RSRP-Range,

SetupRelease,

ShortMAC-I,

S-NSSAI-r15,

S-TMSI,

ServingSatelliteInfo-r17,

SystemInformationBlockType16-r11,

SystemInfoValueTagSI-r13,

T-Reordering,

T-ReorderingExt-r17,

TimeAlignmentTimer,

TimeSinceFailure-r11,

TMGI-r9,

TMGI-r9,

TrackingAreaCode,

TrackingAreaCode-5GC-r15,

UAC-AC1-SelectAssistInfo-r15,

DataInactivityTimer-r14

FROM EUTRA-RRC-Definitions;

-- ASN1STOP

#### –	BCCH-BCH-Message-NB

The BCCH-BCH-Message-NB class is the set of RRC messages that may be sent from the E-UTRAN to the UE via BCH on the BCCH logical channel in FDD.

-- ASN1START

BCCH-BCH-Message-NB ::= SEQUENCE {

message					BCCH-BCH-MessageType-NB

}

BCCH-BCH-MessageType-NB::=	MasterInformationBlock-NB

-- ASN1STOP

#### –	BCCH-BCH-Message-TDD-NB

The BCCH-BCH-Message-TDD-NB class is the set of RRC messages that may be sent from the E-UTRAN to the UE via BCH on the BCCH logical channel in TDD.

-- ASN1START

BCCH-BCH-Message-TDD-NB ::= SEQUENCE {

message					BCCH-BCH-MessageType-TDD-NB-r15

}

BCCH-BCH-MessageType-TDD-NB-r15 ::=	MasterInformationBlock-TDD-NB-r15

-- ASN1STOP

#### –	BCCH-DL-SCH-Message-NB

The BCCH-DL-SCH-Message-NB class is the set of RRC messages that may be sent from the E-UTRAN to the UE via DL-SCH on the BCCH logical channel.

-- ASN1START

BCCH-DL-SCH-Message-NB ::= SEQUENCE {

message					BCCH-DL-SCH-MessageType-NB

}

BCCH-DL-SCH-MessageType-NB ::= CHOICE {

c1						CHOICE {

systemInformation-r13				SystemInformation-NB,

systemInformationBlockType1-r13		SystemInformationBlockType1-NB

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	PCCH-Message-NB

The PCCH-Message-NB class is the set of RRC messages that may be sent from the E-UTRAN to the UE on the PCCH logical channel.

-- ASN1START

PCCH-Message-NB ::= SEQUENCE {

message					PCCH-MessageType-NB

}

PCCH-MessageType-NB ::= CHOICE {

c1						CHOICE {

paging-r13							Paging-NB

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	DL-CCCH-Message-NB

The DL-CCCH-Message-NB class is the set of RRC messages that may be sent from the E-UTRAN to the UE on the downlink CCCH logical channel.

-- ASN1START

DL-CCCH-Message-NB ::= SEQUENCE {

message					DL-CCCH-MessageType-NB

}

DL-CCCH-MessageType-NB ::= CHOICE {

c1						CHOICE {

rrcConnectionReestablishment-r13		RRCConnectionReestablishment-NB,

rrcConnectionReestablishmentReject-r13	RRCConnectionReestablishmentReject,

rrcConnectionReject-r13					RRCConnectionReject-NB,

rrcConnectionSetup-r13					RRCConnectionSetup-NB,

rrcEarlyDataComplete-r15				RRCEarlyDataComplete-NB-r15,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	DL-DCCH-Message-NB

The DL-DCCH-Message-NB class is the set of RRC messages that may be sent from the E-UTRAN to the UE on the downlink DCCH logical channel.

-- ASN1START

DL-DCCH-Message-NB ::= SEQUENCE {

message					DL-DCCH-MessageType-NB

}

DL-DCCH-MessageType-NB ::= CHOICE {

c1						CHOICE {

dlInformationTransfer-r13				DLInformationTransfer-NB,

rrcConnectionReconfiguration-r13		RRCConnectionReconfiguration-NB,

rrcConnectionRelease-r13				RRCConnectionRelease-NB,

securityModeCommand-r13					SecurityModeCommand,

ueCapabilityEnquiry-r13					UECapabilityEnquiry-NB,

rrcConnectionResume-r13					RRCConnectionResume-NB,

ueInformationRequest-r16				UEInformationRequest-NB-r16,

spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	UL-CCCH-Message-NB

The UL-CCCH-Message-NB class is the set of RRC messages that may be sent from the UE to the E-UTRAN on the uplink CCCH logical channel.

-- ASN1START

UL-CCCH-Message-NB ::= SEQUENCE {

message					UL-CCCH-MessageType-NB

}

UL-CCCH-MessageType-NB ::= CHOICE {

c1						CHOICE {

rrcConnectionReestablishmentRequest-r13	RRCConnectionReestablishmentRequest-NB,

rrcConnectionRequest-r13				RRCConnectionRequest-NB,

rrcConnectionResumeRequest-r13			RRCConnectionResumeRequest-NB,

rrcEarlyDataRequest-r15				RRCEarlyDataRequest-NB-r15

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	SC-MCCH-Message-NB

The SC-MCCH-Message-NB class is the set of RRC messages that may be sent from the E-UTRAN to the NB-IoT UE on the SC-MCCH logical channel.

-- ASN1START

SC-MCCH-Message-NB ::= SEQUENCE {

message					SC-MCCH-MessageType-NB

}

SC-MCCH-MessageType-NB ::= CHOICE {

c1						CHOICE {

scptmConfiguration-r14						SCPTMConfiguration-NB-r14

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	UL-DCCH-Message-NB

The UL-DCCH-Message-NB class is the set of RRC messages that may be sent from the UE to the E-UTRAN on the uplink DCCH logical channel.

-- ASN1START

UL-DCCH-Message-NB ::= SEQUENCE {

message					UL-DCCH-MessageType-NB

}

UL-DCCH-MessageType-NB ::= CHOICE {

c1						CHOICE {

rrcConnectionReconfigurationComplete-r13	RRCConnectionReconfigurationComplete-NB,

rrcConnectionReestablishmentComplete-r13	RRCConnectionReestablishmentComplete-NB,

rrcConnectionSetupComplete-r13				RRCConnectionSetupComplete-NB,

securityModeComplete-r13					SecurityModeComplete,

securityModeFailure-r13						SecurityModeFailure,

ueCapabilityInformation-r13					UECapabilityInformation-NB,

ulInformationTransfer-r13					ULInformationTransfer-NB,

rrcConnectionResumeComplete-r13				RRCConnectionResumeComplete-NB,

ueInformationResponse-r16					UEInformationResponse-NB-r16,

purConfigurationRequest-r16					PURConfigurationRequest-NB-r16,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

### 6.7.2	NB-IoT Message definitions

#### –	DLInformationTransfer-NB

The DLInformationTransfer-NB message is used for the downlink transfer of NAS dedicated information.

Signalling radio bearer: SRB1or SRB1bis

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

DLInformationTransfer-NB message

-- ASN1START

DLInformationTransfer-NB ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

dlInformationTransfer-r13		DLInformationTransfer-NB-r13-IEs,

spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

DLInformationTransfer-NB-r13-IEs ::=	SEQUENCE {

dedicatedInfoNAS-r13					DedicatedInfoNAS,

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

nonCriticalExtension					SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

#### –	MasterInformationBlock-NB

The MasterInformationBlock-NB includes the system information transmitted on BCH in FDD.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

MasterInformationBlock-NB

-- ASN1START

MasterInformationBlock-NB ::=	SEQUENCE {

systemFrameNumber-MSB-r13		BIT STRING (SIZE (4)),

hyperSFN-LSB-r13				BIT STRING (SIZE (2)),

schedulingInfoSIB1-r13			INTEGER (0..15),

systemInfoValueTag-r13			INTEGER (0..31),

ab-Enabled-r13					BOOLEAN,

operationModeInfo-r13			CHOICE {

inband-SamePCI-r13				Inband-SamePCI-NB-r13,

inband-DifferentPCI-r13			Inband-DifferentPCI-NB-r13,

guardband-r13					Guardband-NB-r13,

standalone-r13					Standalone-NB-r13

},

additionalTransmissionSIB1-r15	BOOLEAN,

ab-Enabled-5GC-r16				BOOLEAN,

spare							BIT STRING (SIZE (9))

}

Guardband-NB-r13 ::=			SEQUENCE {

rasterOffset-r13				ChannelRasterOffset-NB-r13,

spare							BIT STRING (SIZE (3))

}

Inband-SamePCI-NB-r13 ::=		SEQUENCE {

eutra-CRS-SequenceInfo-r13		INTEGER (0..31)

}

Inband-DifferentPCI-NB-r13 ::=	SEQUENCE {

eutra-NumCRS-Ports-r13			ENUMERATED {same, four},

rasterOffset-r13				ChannelRasterOffset-NB-r13,

spare							BIT STRING (SIZE (2))

}

Standalone-NB-r13 ::=			SEQUENCE {

spare							BIT STRING (SIZE (5))

}

-- ASN1STOP

MasterInformationBlock-NB field descriptions

ab-Enabled Value TRUE indicates that access barring is enabled for UEs connected to EPC.

ab-Enabled-5GC Value TRUE indicates that access barring is enabled for UEs connected to 5GC.

additionalTransmissionSIB1 Value TRUE indicates that additional SIB1-NB transmissions are present. See TS 36.211 [21] and TS 36.213 [23]. E-UTRAN only configures additionalTransmissionSIB1 to TRUE if schedulingInfoSIB1 indicates that the number of NPDSCH repetitions is 16, see TS 36.213 [23], Table 16.4.1.3-3.

eutra-CRS-SequenceInfo Information of the carrier containing NPSS/NSSS/NPBCH. Each value is associated with an E-UTRA PRB index as an offset from the middle of the LTE system sorted out by channel raster offset. See TS 36.211[21] and TS 36.213 [23].

eutra-NumCRS-Ports Number of E-UTRA CRS antenna ports, either the same number of ports as NRS or 4 antenna ports. See TS 36.211 [21], TS 36.212 [22], and TS 36.213 [23].

hyperSFN-LSB Indicates the 2 least significant bits of hyper SFN. The remaining bits are present in SystemInformationBlockType1-NB.

operationModeInfo Deployment scenario (in-band/guard-band/standalone) and related information. See TS 36.211 [21] and TS 36.213 [23]. Inband-SamePCI indicates an in-band deployment and that the NB-IoT and LTE cell share the same physical cell id and have the same number of NRS and CRS ports. Inband-DifferentPCI indicates an in-band deployment and that the NB-IoT and LTE cell have different physical cell id. guardband indicates a guard-band deployment. standalone indicates a standalone deployment.

schedulingInfoSIB1 This field contains an index to a table specified in TS 36.213 [23], Table 16.4.1.3-3, that defines SystemInformationBlockType1-NB scheduling information.

systemFrameNumber-MSB Defines the 4 most significant bits of the SFN. As indicated in TS 36.211 [21], the 6 least significant bits of the SFN are acquired implicitly by decoding the NPBCH.

systemInfoValueTag Common for all SIBs other than MIB-NB, SIB14-NB and SIB16-NB.

#### –	MasterInformationBlock-TDD-NB

The MasterInformationBlock-TDD-NB includes the system information transmitted on BCH in TDD.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

MasterInformationBlock-TDD-NB

-- ASN1START

MasterInformationBlock-TDD-NB-r15 ::=	SEQUENCE {

systemFrameNumber-MSB-r15				BIT STRING (SIZE (4)),

hyperSFN-LSB-r15						BIT STRING (SIZE (2)),

schedulingInfoSIB1-r15					INTEGER (0..15),

systemInfoValueTag-r15					INTEGER (0..31),

ab-Enabled-r15							BOOLEAN,

operationModeInfo-r15				CHOICE {

inband-SamePCI-r15					Inband-SamePCI-TDD-NB-r15,

inband-DifferentPCI-r15				Inband-DifferentPCI-TDD-NB-r15,

guardband-r15						GuardbandTDD-NB-r15,

standalone-r15						StandaloneTDD-NB-r15

},

sib1-CarrierInfo-r15					ENUMERATED {anchor, non-anchor},

ab-Enabled-5GC-r16						BOOLEAN,

spare									BIT STRING (SIZE (8))

}

GuardbandTDD-NB-r15 ::=				SEQUENCE {

rasterOffset-r15					ChannelRasterOffset-NB-r13,

sib-GuardbandInfo-r15				CHOICE {

sib-GuardbandAnchor-r15				SIB-GuardbandAnchorTDD-NB-r15,

sib-GuardbandGuardband-r15			SIB-GuardbandGuardbandTDD-NB-r15,

sib-GuardbandInbandSamePCI-r15		SIB-GuardbandInbandSamePCI-TDD-NB-r15,

sib-GuardbandinbandDiffPCI-r15		SIB-GuardbandInbandDiffPCI-TDD-NB-r15

},

eutra-Bandwitdh-r15					ENUMERATED {bw5or10, bw15or20}

}

Inband-SamePCI-TDD-NB-r15 ::=		SEQUENCE {

eutra-CRS-SequenceInfo-r15			INTEGER (0..31),

sib-InbandLocation-r15				ENUMERATED {lower, higher}

}

Inband-DifferentPCI-TDD-NB-r15 ::=		SEQUENCE {

eutra-NumCRS-Ports-r15					ENUMERATED {same, four},

rasterOffset-r15						ChannelRasterOffset-NB-r13,

sib-InbandLocation-r15					ENUMERATED {lower, higher},

spare									BIT STRING (SIZE (2))

}

StandaloneTDD-NB-r15 ::=				SEQUENCE {

sib-StandaloneLocation-r15				ENUMERATED {lower, higher},

spare									BIT STRING (SIZE (5))

}

SIB-GuardbandAnchorTDD-NB-r15 ::=		SEQUENCE {

spare									BIT STRING (SIZE (1))

}

SIB-GuardbandGuardbandTDD-NB-r15 ::=	SEQUENCE {

sib-GuardbandGuardbandLocation-r15		ENUMERATED {same, opposite}

}

SIB-GuardbandInbandSamePCI-TDD-NB-r15 ::= SEQUENCE {

spare									BIT STRING (SIZE (1))

}

SIB-GuardbandInbandDiffPCI-TDD-NB-r15 ::= SEQUENCE {

sib-EUTRA-NumCRS-Ports-r15				ENUMERATED {same, four}

}

-- ASN1STOP

MasterInformationBlock-TDD-NB field descriptions

ab-Enabled Value TRUE indicates that access barring is enabled for UEs connected to EPC.

ab-Enabled-5GC Value TRUE indicates that access barring is enabled for UEs connected to 5GC.

eutra-Bandwidth EUTRA system bandwidth. Value bw5or10 corresponds to bandwidth 5 or 10 MHz, value bw15or20 corresponds to bandwidth 15 or 20 MHz. If the value of eutra-Bandwidth is bw5or10 and rasterOffset is set to khz7dot5 or khz-7dot5, the E-UTRA system bandwidth is 5 MHz. If the value of eutra-Bandwidth is bw5or10 and rasterOffset is set to khz2dot5 or khz-2dot5, the E-UTRA system bandwidth is 10 MHz. If the value of eutra-Bandwidth is bw15or20 and rasterOffset is set to khz7dot5 or khz-7dot5, the E-UTRA system bandwidth is 15 MHz. If the value of eutra-Bandwidth is bw15or20 and rasterOffset is set to khz2dot5 or khz-2dot5, the E-UTRA system bandwidth is 20 MHz. When the E-UTRA system bandwidth is 5 MHz or 15 MHz, if the value of sib-GuardbandInfo is sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, the offset between the anchor carrier and the non-anchor carrier used for SIB1 and/or SI transmission is 45 kHz.

eutra-CRS-SequenceInfo Information of the carrier containing NPSS/NSSS/NPBCH. Each value is associated with an E-UTRA PRB index as an offset from the middle of the LTE system sorted out by channel raster offset. See TS 36.211 [21] and TS 36.213 [23].

eutra-NumCRS-Ports, sib-eutra-NumCRS-Ports Number of E-UTRA CRS antenna ports, either the same number of ports as NRS or 4 antenna ports. See TS 36.211 [21], TS 36.212 [22], and TS 36.213 [23].

hyperSFN-LSB Indicates the 2 least significant bits of hyper SFN. The remaining bits are present in SystemInformationBlockType1-NB.

operationModeInfo Deployment scenario (in-band/guard-band/standalone) and related information. See TS 36.211 [21] and TS 36.213 [23]. Inband-SamePCI indicates an in-band deployment and that the NB-IoT and LTE cell share the same physical cell id and have the same number of NRS and CRS ports. Inband-DifferentPCI indicates an in-band deployment and that the NB-IoT and LTE cell have different physical cell id. guardband indicates a guard-band deployment. standalone indicates a standalone deployment. When operationmodeInfo is set to guardband, if rasterOffset is set to khz-7dot5 or khz-2dot5, the guardband anchor carrier is at the higher edge of the LTE carrier. If rasterOffset is set to khz7dot5 or khz2dot5, the guardband anchor carrier is at the lower edge of the LTE carrier

schedulingInfoSIB1 This field contains an index to a table specified in TS 36.213 [23], Table 16.4.1.3-5 or Table 16.4.1.3-7 when sib1-CarrierInfo is set to anchor or to non-anchor respectively, that defines SystemInformationBlockType1-NB scheduling information. If sib1-CarrierInfo is set to non-anchor, E-UTRAN configures a value between 0 and 7.

sib-GuardbandGuardbandLocation Location of the non-anchor carrier used for SIB1 and/or SI transmission when operationmodeInfo is set to guardband and the non-anchor carrier is in guardband. See TS 36.213 [23]. Value same corresponds to the carrier adjacent to the anchor carrier on the outer side of the guardband, value opposite corresponds to the carrier closest to the edge of the LTE carrier in the opposite guardband.

sib-GuardbandInfo Information of the carrier used for SIB1 and/or SI transmission when operationmodeInfo is set to guardband. See TS 36.213 [23]. sib-GuardbandAnchor indicates the anchor carrier. sib-GuardbandGuardband indicates a non-anchor carrier in guardband mode. sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI indicates a non-anchor carrier in inband mode, and at the edge of the LTE carrier and on the same side as the anchor carrier.

sib-InbandLocation Location of the non-anchor carrier used for SIB1 and/or SI transmission when operationmodeInfo is set to inband-SamePCI or inband-DifferentPCI, and sib1-CarrierInfo value and/or tdd-SI-CarrierInfo in SIB1-NB is set to non-anchor. See TS 36.213 [23]. Value lower corresponds to the lower adjacent carrier relative to the anchor carrier and value higher corresponds to the higher adjacent carrier relative to the anchor carrier. If both sib1-CarrierInfo value and tdd-SI-CarrierInfo value in SIB1-NB are set to anchor, the UE ignores sib-InbandLocation.

sib-StandaloneLocation Location of the non-anchor carrier used for SIB1 and/or SI transmission when operationmodeInfo is set to standalone, and sib1-CarrierInfo value and/or tdd-SI-CarrierInfo in SIB1-NB is set to non-anchor. See TS 36.213 [23]. Value lower corresponds to the lower adjacent carrier relative to the anchor carrier and value higher corresponds to the higher adjacent carrier relative to the anchor carrier. If both sib1-CarrierInfo value and tdd-SI-CarrierInfo value in SIB1-NB are set to anchor, the UE ignores sib-StandaloneLocation.

sib1-CarrierInfo Carrier used for SIB1 transmission. See TS 36.213 [23], clause 16.4.1.3. Value anchor corresponds to anchor carrier, value non-anchor corresponds to non-anchor carrier.

systemFrameNumber-MSB Defines the 4 most significant bits of the SFN. As indicated in TS 36.211 [21], the 6 least significant bits of the SFN are acquired implicitly by decoding the NPBCH.

systemInfoValueTag Common for all SIBs other than MIB-NB, SIB14-NB and SIB16-NB.

#### –	Paging-NB

The Paging-NB message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: E-UTRAN to UE

Paging-NB message

-- ASN1START

Paging-NB ::=						SEQUENCE {

pagingRecordList-r13				PagingRecordList-NB-r13		OPTIONAL,	-- Need ON

systemInfoModification-r13			ENUMERATED {true}				OPTIONAL,	-- Need ON

systemInfoModification-eDRX-r13		ENUMERATED {true}				OPTIONAL,	-- Need ON

nonCriticalExtension				Paging-NB-v1610-IEs				OPTIONAL

}

Paging-NB-v1610-IEs	::=				SEQUENCE {

pagingRecordList-v1610				PagingRecordList-NB-v1610		OPTIONAL,	-- Need ON

nonCriticalExtension 				SEQUENCE {}						OPTIONAL

}

PagingRecordList-NB-r13 ::=			SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-NB-r13

PagingRecordList-NB-v1610 ::=		SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-NB-v1610

PagingRecord-NB-r13 ::=				SEQUENCE {

ue-Identity-r13						PagingUE-Identity,

...

}

PagingRecord-NB-v1610 ::=			SEQUENCE {

mt-EDT-r16							ENUMERATED {true}			OPTIONAL	-- Need ON

}

-- ASN1STOP

Editor’s Note: Agreement: System information update notification procedure is not used to inform TAC updates, at least for TAC additions (FFS removals).

Paging-NB field descriptions

mt-EDT Indication of mobile-terminated EDT.

pagingRecordList If E-UTRAN includes pagingRecordList-v1610, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix).

systemInfoModification If present: indication of a BCCH modification other than for SystemInformationBlockType14-NB (SIB14-NB), SystemInformationBlockType16-NB (SIB16-NB) and SystemInformationBlockTypeXX-NB (SIBXX-NB). This indication does not apply to UEs using eDRX cycle longer than the BCCH modification period.

systemInfoModification-eDRX If present: indication of a BCCH modification other than for SystemInformationBlockType14-NB (SIB14-NB), SystemInformationBlockType16-NB (SIB16-NB) and SystemInformationBlockTypeXX-NB (SIBXX-NB). This indication applies only to UEs using eDRX cycle longer than the BCCH modification period.

ue-Identity Provides the NAS identity of the UE that is being paged.

#### –	PURConfigurationRequest-NB

The PURConfigurationRequest-NB message is used by the UE to transfer PUR related information to the E-UTRAN.

Signalling radio bearer: SRB1 or SRB1bis

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

PURConfigurationRequest-NB message

-- ASN1START

PURConfigurationRequest-NB-r16 ::=	SEQUENCE {

criticalExtensions					CHOICE {

purConfigurationRequest-r16			PURConfigurationRequest-NB-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

PURConfigurationRequest-NB-r16-IEs ::=	SEQUENCE {

pur-ConfigRequest-r16					PUR-ConfigRequest-NB-r16			OPTIONAL,

lateNonCriticalExtension				OCTET STRING						OPTIONAL,

nonCriticalExtension					SEQUENCE {}							OPTIONAL

}

PUR-ConfigRequest-NB-r16 ::=		CHOICE{

pur-ReleaseRequest					NULL,

pur-SetupRequest					SEQUENCE {

requestedNumOccasions-r16			ENUMERATED {one, infinite},

requestedPeriodicityAndOffset-r16	PUR-PeriodicityAndOffset-NB-r16,

requestedTBS-r16					ENUMERATED {b328, b376, b424, b472, b504, b552, b584,

b616, b680, b744, b776, b808, b872, b904,

b936, b968, b1000, b1032, b1096, b1128,

b1192, b1224, b1256, b1352, b1384, b1544,

b1608, b1736, b1800, b2024, b2280, b2536},

rrc-ACK-r16							ENUMERATED {true}					OPTIONAL

}

}

-- ASN1STOP

PURConfigurationRequest-NB field descriptions

requestedNumOccasions Indicates the requested number of PUR occasions. Value one corresponds to one occasion and value infinite corresponds to infinite occasions.

requestedPeriodicityAndOffset Indicates the requested periodicity of the PUR occasions and time offset until the first PUR occasion.

requestedTBS Indicates the requested TBS. Value b328 corresponds to 328 bits, value b376 corresponds to 376 bits, and so on.

rrc-ACK Indicates RRC response message is prefered by the UE for acknowledging the reception of a transmission using PUR.

#### –	RRCConnectionReconfiguration-NB

The RRCConnectionReconfiguration-NB message is the command to modify an RRC connection. It may convey information for resource configuration (including RBs, MAC main configuration and physical channel configuration) including any associated dedicated NAS information.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionReconfiguration-NB message

-- ASN1START

RRCConnectionReconfiguration-NB ::=	SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

c1										CHOICE{

rrcConnectionReconfiguration-r13		RRCConnectionReconfiguration-NB-r13-IEs,

spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfiguration-NB-r13-IEs ::= SEQUENCE {

dedicatedInfoNASList-r13			SEQUENCE (SIZE(1..maxDRB-NB-r13)) OF

DedicatedInfoNAS		OPTIONAL,	-- Need ON

radioResourceConfigDedicated-r13	RadioResourceConfigDedicated-NB-r13	OPTIONAL,	-- Need ON

fullConfig-r13						ENUMERATED {true}					OPTIONAL,	-- Cond Reestab

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}	OPTIONAL

}

-- ASN1STOP

RRCConnectionReconfiguration-NB field descriptions

dedicatedInfoNASList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

fullConfig Indicates the full configuration option is applicable for the RRC Connection Reconfiguration message.

| Conditional presence | Explanation |
| --- | --- |
| Reestab | This field is optionally present, need ON upon the first reconfiguration after RRC connection re-establishment; otherwise the field is not present. |

#### –	RRCConnectionReconfigurationComplete-NB

The RRCConnectionReconfigurationComplete-NB message is used to confirm the successful completion of an RRC connection reconfiguration.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

RRCConnectionReconfigurationComplete-NB message

-- ASN1START

RRCConnectionReconfigurationComplete-NB ::= SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

rrcConnectionReconfigurationComplete-r13	RRCConnectionReconfigurationComplete-NB-r13-IEs,

criticalExtensionsFuture					SEQUENCE {}

}

}

RRCConnectionReconfigurationComplete-NB-r13-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

#### –	RRCConnectionReestablishment-NB

The RRCConnectionReestablishment-NB message is used to re-establish SRB1.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: E-UTRAN to UE

RRCConnectionReestablishment-NB message

-- ASN1START

RRCConnectionReestablishment-NB ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcConnectionReestablishment-r13	RRCConnectionReestablishment-NB-r13-IEs,

spare1	NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReestablishment-NB-r13-IEs ::= SEQUENCE {

radioResourceConfigDedicated-r13			RadioResourceConfigDedicated-NB-r13,

nextHopChainingCount-r13					NextHopChainingCount,

lateNonCriticalExtension					OCTET STRING						OPTIONAL,

nonCriticalExtension						RRCConnectionReestablishment-NB-v1430-IEs	OPTIONAL

}

RRCConnectionReestablishment-NB-v1430-IEs ::=	SEQUENCE {

dl-NAS-MAC							BIT STRING (SIZE (16))	OPTIONAL,	-- Cond Reestablish-CP

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

-- ASN1STOP

RRCConnectionReestablishment-NB field descriptions

dl-NAS-MAC Downlink authentication token, see TS 33.401 [32]. If this field is present, the UE shall ignore the field nextHopChainingCount.

| Conditional presence | Explanation |
| --- | --- |
| Reestablish-CP | This field is mandatory present for NB-IoT UE using the Control Plane CIoT EPS/5GS optimisation; otherwise the field is not present. |

#### –	RRCConnectionReestablishmentComplete-NB

The RRCConnectionReestablishmentComplete-NB message is used to confirm the successful completion of an RRC connection re-establishment.

Signalling radio bearer: SRB1 or SRB1bis

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

RRCConnectionReestablishmentComplete-NB message

-- ASN1START

RRCConnectionReestablishmentComplete-NB ::= SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

rrcConnectionReestablishmentComplete-r13	RRCConnectionReestablishmentComplete-NB-r13-IEs,

criticalExtensionsFuture					SEQUENCE {}

}

}

RRCConnectionReestablishmentComplete-NB-r13-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				RRCConnectionReestablishmentComplete-NB-v1470-IEs	OPTIONAL

}

RRCConnectionReestablishmentComplete-NB-v1470-IEs ::= SEQUENCE {

measResultServCell-r14			MeasResultServCell-NB-r14		OPTIONAL,

nonCriticalExtension			RRCConnectionReestablishmentComplete-NB-v1610-IEs	OPTIONAL

}

RRCConnectionReestablishmentComplete-NB-v1610-IEs ::= SEQUENCE {

rlf-InfoAvailable-r16				ENUMERATED {true}				OPTIONAL,

anr-InfoAvailable-r16				ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

RRCConnectionReestablishmentComplete-NB field descriptions

anr-InfoAvailable Indicates the availability of ANR measurement information.

measResultServCell This field refers to the last idle mode measurement results taken of the serving cell.

rlf-InfoAvailable Indicates the availability of radio link failure related information.

#### –	RRCConnectionReestablishmentRequest-NB

The RRCConnectionReestablishmentRequest-NB message is used to request the reestablishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to E-UTRAN

RRCConnectionReestablishmentRequest-NB message

-- ASN1START

RRCConnectionReestablishmentRequest-NB ::= SEQUENCE {

criticalExtensions					CHOICE {

rrcConnectionReestablishmentRequest-r13

RRCConnectionReestablishmentRequest-NB-r13-IEs,

later								CHOICE {

rrcConnectionReestablishmentRequest-r14

RRCConnectionReestablishmentRequest-NB-r14-IEs,

later							CHOICE {

rrcConnectionReestablishmentRequest-r16

RRCConnectionReestablishmentRequest-5GC-NB-r16-IEs,

criticalExtensionsFuture	SEQUENCE {}

}

}

}

}

RRCConnectionReestablishmentRequest-NB-r13-IEs ::= SEQUENCE {

ue-Identity-r13						ReestabUE-Identity,

reestablishmentCause-r13			ReestablishmentCause-NB-r13,

cqi-NPDCCH-r14						CQI-NPDCCH-NB-r14,

earlyContentionResolution-r14		BOOLEAN,

spare								BIT STRING (SIZE (20))

}

RRCConnectionReestablishmentRequest-NB-r14-IEs ::= SEQUENCE {

ue-Identity-r14						ReestabUE-Identity-CP-NB-r14,

reestablishmentCause-r14			ReestablishmentCause-NB-r13,

cqi-NPDCCH-r14						CQI-NPDCCH-Short-NB-r14,

earlyContentionResolution-r14		BOOLEAN,

spare								BIT STRING (SIZE (1))

}

RRCConnectionReestablishmentRequest-5GC-NB-r16-IEs ::= SEQUENCE {

ue-Identity-r16						ReestabUE-Identity-CP-5GC-NB-r16,

reestablishmentCause-r16			ReestablishmentCause-NB-r13,

cqi-NPDCCH-r16						CQI-NPDCCH-Short-NB-r14,

spare								BIT STRING (SIZE (1))

}

ReestablishmentCause-NB-r13 ::=			ENUMERATED {

reconfigurationFailure, otherFailure,

spare2, spare1}

ReestabUE-Identity-CP-NB-r14 ::=		SEQUENCE {

s-TMSI-r14								S-TMSI,

ul-NAS-MAC-r14							BIT STRING (SIZE (16)),

ul-NAS-Count-r14						BIT STRING (SIZE (5))

}

ReestabUE-Identity-CP-5GC-NB-r16 ::=	SEQUENCE {

truncated5G-S-TMSI-r16					BIT STRING (SIZE (40)),

ul-NAS-MAC-r16							BIT STRING (SIZE (16)),

ul-NAS-Count-r16						BIT STRING (SIZE (5))

}

-- ASN1STOP

RRCConnectionReestablishmentRequest-NB field descriptions

earlyContentionResolution Value TRUE indicates UE supports MAC PDU containing the UE contention resolution identity MAC control element without RRC response message. This field is always set to TRUE in this version of the specification.

reestablishmentCause Indicates the failure cause that triggered the re-establishment procedure. eNB is not expected to reject a RRCConnectionReestablishmentRequest due to unknown cause value being used by the UE.

truncated5G-S-TMSI For description of this field see TS 23.003 [27].

ue-Identity UE identity included to retrieve UE context and to facilitate contention resolution by lower layers.

ul-NAS-Count For description of this field see TS 33.401 [32] for EPC, and TS 33.501 [86] for 5GC.

ul-NAS-MAC For description of this field see TS 33.401 [32] for EPC, and TS 33.501 [86] for 5GC.

#### –	RRCConnectionReject-NB

The RRCConnectionReject-NB message is used to reject the RRC connection establishment or RRC connection resume or to reject the EDT procedure.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: E-UTRAN to UE

RRCConnectionReject-NB message

-- ASN1START

RRCConnectionReject-NB ::=				SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionReject-r13				RRCConnectionReject-NB-r13-IEs,

spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReject-NB-r13-IEs ::=		SEQUENCE {

extendedWaitTime-r13					INTEGER (1..1800),

rrc-SuspendIndication-r13				ENUMERATED {true}			OPTIONAL,	-- Need ON

lateNonCriticalExtension				OCTET STRING				OPTIONAL,

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

-- ASN1STOP

RRCConnectionReject-NB field descriptions

extendedWaitTime Value in seconds.

rrc-SuspendIndication If present, this field indicates that the UE should remain suspended and not release its stored context.

#### –	RRCConnectionRelease-NB

The RRCConnectionRelease-NB message is used to command the release of an RRC connection, or to complete an UP-EDT procedure.

Signalling radio bearer: SRB1 or SRB1bis

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionRelease-NB message

-- ASN1START

RRCConnectionRelease-NB ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionRelease-r13			RRCConnectionRelease-NB-r13-IEs,

spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionRelease-NB-r13-IEs ::=	SEQUENCE {

releaseCause-r13					ReleaseCause-NB-r13,

resumeIdentity-r13					ResumeIdentity-r13				OPTIONAL,	-- Need OR

extendedWaitTime-r13				INTEGER (1..1800)				OPTIONAL,	-- Need ON

redirectedCarrierInfo-r13			RedirectedCarrierInfo-NB-r13	OPTIONAL,	-- Need ON

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				RRCConnectionRelease-NB-v1430-IEs		OPTIONAL

}

RRCConnectionRelease-NB-v1430-IEs ::=	SEQUENCE {

redirectedCarrierInfo-v1430			RedirectedCarrierInfo-NB-v1430	OPTIONAL,	-- Cond Redirection

extendedWaitTime-CPdata-r14		INTEGER (1..1800)	OPTIONAL,	-- Cond NoExtendedWaitTime

nonCriticalExtension				RRCConnectionRelease-NB-v1530-IEs	OPTIONAL

}

RRCConnectionRelease-NB-v1530-IEs ::=	SEQUENCE {

drb-ContinueROHC-r15					ENUMERATED {true}			OPTIONAL,	-- Cond UP-EDT

nextHopChainingCount-r15				NextHopChainingCount		OPTIONAL,	-- Cond EarlySec

nonCriticalExtension				RRCConnectionRelease-NB-v1550-IEs	OPTIONAL

}

RRCConnectionRelease-NB-v1550-IEs ::=	SEQUENCE {

redirectedCarrierInfo-v1550			RedirectedCarrierInfo-NB-v1550	OPTIONAL,	-- Cond Redirection-TDD

nonCriticalExtension				RRCConnectionRelease-NB-v15b0-IEs	OPTIONAL

}

RRCConnectionRelease-NB-v15b0-IEs ::=	SEQUENCE {

noLastCellUpdate-r15					ENUMERATED {true} 		OPTIONAL,	-- Need OP

nonCriticalExtension					RRCConnectionRelease-NB-v1610-IEs		OPTIONAL

}

RRCConnectionRelease-NB-v1610-IEs ::=	SEQUENCE {

resumeIdentity-r16						I-RNTI-r15					OPTIONAL,	-- Need OR

anr-MeasConfig-r16						ANR-MeasConfig-NB-r16		OPTIONAL,	-- Need OP

pur-Config-r16							SetupRelease {PUR-Config-NB-r16}

OPTIONAL,	-- Need ON

nonCriticalExtension					SEQUENCE {}		OPTIONAL

}

ReleaseCause-NB-r13 ::=					ENUMERATED {loadBalancingTAUrequired, other,

rrc-Suspend, spare1}

RedirectedCarrierInfo-NB-r13::=			CarrierFreq-NB-r13

RedirectedCarrierInfo-NB-v1430	::=		SEQUENCE {

redirectedCarrierOffsetDedicated-r14	ENUMERATED{

dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24, dB26},

t322-r14								ENUMERATED{

min5, min10, min20, min30, min60, min120, min180,

spare1}

}

RedirectedCarrierInfo-NB-v1550::=		CarrierFreq-NB-v1550

-- ASN1STOP

RRCConnectionRelease-NB field descriptions

drb-ContinueROHC This field indicates whether to continue or reset the header compression protocol context for the DRBs configured with the header compression protocol. Presence of the field indicates that the header compression protocol context continues when UE initiates UP-EDT in the same cell, while absence indicates that the header compression protocol context is reset.

extendedWaitTime Value in seconds.

extendedWaitTime-CPdata Wait time for data transfer using the Control Plane CIoT EPS optimisation. Value in seconds. See TS 24.301 [35].

noLastCellUpdate Presence of the field indicates that the last used cell for (G)WUS shall not be updated.

redirectedCarrierInfo The redirectedCarrierInfo indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to a NB-IoT carrier frequency, by means of the cell selection upon leaving RRC_CONNECTED as specified in TS 36.304 [4].

redirectedCarrierOffsetDedicated Parameter "Qoffsetdedicatedfrequency" in TS 36.304 [4]. For NB-IoT carrier frequencies, a UE that supports multi-band cells considers the redirectedCarrierOffsetDedicated to be common for all overlapping bands (i.e. regardless of the EARFCN that is used).

releaseCause The releaseCause is used to indicate the reason for releasing the RRC Connection. E-UTRAN should not set the releaseCause to loadBalancingTAURequired if the extendedWaitTime is present and/or if the UE is connected to 5GC.

resumeIdentity UE identity to facilitate UE context retrieval at eNB. E-UTRAN configures resumeIdentity-r13 only when the UE is connected to EPC and configures resumeIdentity-r16 only when the UE is connected to 5GC.

t322 Timer T322 as described in clause 7.3. Value minN corresponds to N minutes.

| Conditional presence | Explanation |
| --- | --- |
| NoExtendedWaitTime | The field is optionally present, Need ON, if the extendedWaitTime is not included; otherwise the field is not present. |
| Redirection | The field is optionally present, Need ON, if redirectedCarrierInfo is included; otherwise the field is not present. |
| Redirection-TDD | The field is optionally present, Need ON, if redirectedCarrierInfo is included in TDD mode. Otherwise, the field is not present. |
| UP-EDT | The field is optionally present, Need ON, if the UE supports UP-EDT or UP transmission using PUR and releaseCause is set to rrc-Suspend; otherwise the field is not present. |
| EarlySec | For EPC, the field is optionally present, Need ON, if the UE supports early security reactivation or UP-EDT or UP transmission using PUR and releaseCause is set to rrc-Suspend; otherwise the field is not present. For 5GC, the field is mandatory present if releaseCause is set to rrc-Suspend; otherwise the field is not present. |

#### –	RRCConnectionRequest-NB

The RRCConnectionRequest-NB message is used to request the establishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to E-UTRAN

RRCConnectionRequest-NB message

-- ASN1START

RRCConnectionRequest-NB ::=		SEQUENCE {

criticalExtensions					CHOICE {

rrcConnectionRequest-r13			RRCConnectionRequest-NB-r13-IEs,

later								CHOICE {

rrcConnectionRequest-r16			RRCConnectionRequest-5GC-NB-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

}

RRCConnectionRequest-NB-r13-IEs ::=		SEQUENCE {

ue-Identity-r13							InitialUE-Identity,

establishmentCause-r13					EstablishmentCause-NB-r13,

multiToneSupport-r13					ENUMERATED {true}				OPTIONAL,

multiCarrierSupport-r13					ENUMERATED {true}				OPTIONAL,

earlyContentionResolution-r14			BOOLEAN,

cqi-NPDCCH-r14							CQI-NPDCCH-NB-r14,

spare									BIT STRING (SIZE (17))

}

RRCConnectionRequest-5GC-NB-r16-IEs ::=	SEQUENCE {

ue-Identity-r16							InitialUE-Identity-5GC-NB-r16,

establishmentCause-r16					ENUMERATED {

mt-Access, mo-Signalling, mo-Data, mo-ExceptionData,

spare4, spare3, spare2, spare1},

cqi-NPDCCH-r16							CQI-NPDCCH-NB-r14,

spare									BIT STRING (SIZE (11))

}

InitialUE-Identity-5GC-NB-r16 ::=		CHOICE {

ng-5G-S-TMSI-r16						NG-5G-S-TMSI-r15,

randomValue								BIT STRING (SIZE (48))

}

-- ASN1STOP

RRCConnectionRequest-NB field descriptions

earlyContentionResolution Value TRUE indicates UE supports MAC PDU containing the UE contention resolution identity MAC control element without RRC response message. This field is always set to TRUE in this version of the specification.

establishmentCause Provides the establishment cause for the RRC connection request as provided by the upper layers. eNB is not expected to reject a RRCConnectionRequest due to unknown cause value being used by the UE.

multiCarrierSupport If present, this field indicates that the UE supports multi-carrier operation in the mode, FDD or TDD, used for access.

multiToneSupport If present, this field indicates that the UE supports UL multi-tone transmissions on NPUSCH in the mode, FDD or TDD, used for access.

randomValue Integer value in the range 0 to 248 1.

ue-Identity UE identity included to facilitate contention resolution by lower layers.

#### –	RRCConnectionResume-NB

The RRCConnectionResume-NB message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionResume-NB message

-- ASN1START

RRCConnectionResume-NB ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionResume-r13				RRCConnectionResume-NB-r13-IEs,

spare1								NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionResume-NB-r13-IEs ::=		SEQUENCE {

radioResourceConfigDedicated-r13		RadioResourceConfigDedicated-NB-r13	OPTIONAL,		-- Need ON

nextHopChainingCount-r13				NextHopChainingCount,

drb-ContinueROHC-r13					ENUMERATED {true}				OPTIONAL,	-- Need OP

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

nonCriticalExtension					RRCConnectionResume-NB-v1610-IEs	OPTIONAL

}

RRCConnectionResume-NB-v1610-IEs ::=	SEQUENCE {

fullConfig-r16							ENUMERATED {true}		OPTIONAL,	-- Cond 5GC

nonCriticalExtension					SEQUENCE {}				OPTIONAL

}

-- ASN1STOP

RRCConnectionResume-NB field descriptions

drb-ContinueROHC This field indicates whether to continue or reset the header compression protocol context for the DRBs configured with the header compression protocol. Presence of the field indicates that the header compression protocol context continues while absence indicates that the header compression protocol context is reset.

fullConfig Indicates that the full configuration option is applicable for the RRCConnectionResume-NB message.

| Conditional presence | Explanation |
| --- | --- |
| 5GC | The field is optionally present, Need ON, if the UE is connected to 5GC; otherwise the field is not present. |

#### –	RRCConnectionResumeComplete-NB

The RRCConnectionResumeComplete-NB message is used to confirm the successful completion of an RRC connection resumption

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

RRCConnectionResumeComplete-NB message

-- ASN1START

RRCConnectionResumeComplete-NB ::= SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions							CHOICE {

rrcConnectionResumeComplete-r13				RRCConnectionResumeComplete-NB-r13-IEs,

criticalExtensionsFuture					SEQUENCE {}

}

}

RRCConnectionResumeComplete-NB-r13-IEs ::= SEQUENCE {

selectedPLMN-Identity-r13					INTEGER (1..maxPLMN-r11)	OPTIONAL,

dedicatedInfoNAS-r13						DedicatedInfoNAS	OPTIONAL,

lateNonCriticalExtension					OCTET STRING					OPTIONAL,

nonCriticalExtension						RRCConnectionResumeComplete-NB-v1470-IEs	OPTIONAL

}

RRCConnectionResumeComplete-NB-v1470-IEs ::= SEQUENCE {

measResultServCell-r14						MeasResultServCell-NB-r14	OPTIONAL,

nonCriticalExtension						RRCConnectionResumeComplete-NB-v1610-IEs	OPTIONAL

}

RRCConnectionResumeComplete-NB-v1610-IEs ::= SEQUENCE {

rlf-InfoAvailable-r16				ENUMERATED {true}				OPTIONAL,

anr-InfoAvailable-r16				ENUMERATED {true}				OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

RRCConnectionResumeComplete-NB field descriptions

anr-InfoAvailable Indicates the availability of ANR measurement information.

measResultServCell This field refers to the last idle mode measurement results taken of the serving cell.

rlf-InfoAvailable Indicates the availability of radio link failure related information.

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityList included in SystemInformationBlockType1-NB. 1 if the 1st PLMN is selected from the plmn-IdentityList included in SIB1-NB, 2 if the 2nd PLMN is selected from the plmn-IdentityList included in SIB1-NB and so on.

#### –	RRCConnectionResumeRequest-NB

The RRCConnectionResumeRequest-NB message is used to request the resumption of a suspended RRC connection or to perform UP-EDT.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to E-UTRAN

RRCConnectionResumeRequest-NB message

-- ASN1START

RRCConnectionResumeRequest-NB ::=	SEQUENCE {

criticalExtensions						CHOICE {

rrcConnectionResumeRequest-r13			RRCConnectionResumeRequest-NB-r13-IEs,

later									CHOICE {

rrcConnectionResumeRequest-r16			RRCConnectionResumeRequest-5GC-NB-r16-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

}

RRCConnectionResumeRequest-NB-r13-IEs ::=	SEQUENCE {

resumeID-r13								ResumeIdentity-r13,

shortResumeMAC-I-r13						ShortMAC-I,

resumeCause-r13								EstablishmentCause-NB-r13,

earlyContentionResolution-r14				BOOLEAN,

cqi-NPDCCH-r14								CQI-NPDCCH-NB-r14,

anr-InfoAvailable-r16						BOOLEAN,

spare										BIT STRING (SIZE (3))

}

RRCConnectionResumeRequest-5GC-NB-r16-IEs ::=	SEQUENCE {

resumeID-r16								I-RNTI-r15,

shortResumeMAC-I-r16						ShortMAC-I,

resumeCause-r16								EstablishmentCause-NB-r13,

cqi-NPDCCH-r16								CQI-NPDCCH-NB-r14,

spare										BIT STRING (SIZE (4))

}

-- ASN1STOP

RRCConnectionResumeRequest-NB field descriptions

anr-InfoAvailable Indicates the availability of ANR measurement information when the UE is perfoming UP-EDT.

earlyContentionResolution Value TRUE indicates UE supports MAC PDU containing the UE contention resolution identity MAC control element without RRC response message. This field is always set to TRUE in this version of the specification.

resumeCause Provides the resume cause for the RRC connection resume request as provided by the upper layers. eNB is not expected to reject a RRCConnectionResumeRequest due to unknown cause value being used by the UE.

resumeID UE identity to facilitate UE context retrieval at eNB.

shortResumeMAC-I Authentication token to facilitate UE authentication at eNB.

#### –	RRCConnectionSetup-NB

The RRCConnectionSetup-NB message is used to establish SRB1 and SRB1bis.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: E-UTRAN to UE

RRCConnectionSetup-NB message

-- ASN1START

RRCConnectionSetup-NB ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionSetup-r13				RRCConnectionSetup-NB-r13-IEs,

spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionSetup-NB-r13-IEs ::=		SEQUENCE {

radioResourceConfigDedicated-r13		RadioResourceConfigDedicated-NB-r13,

lateNonCriticalExtension				OCTET STRING						OPTIONAL,

nonCriticalExtension					RRCConnectionSetup-NB-v1610-IEs	OPTIONAL

}

RRCConnectionSetup-NB-v1610-IEs ::=		SEQUENCE {

dedicatedInfoNAS-r16					DedicatedInfoNAS			OPTIONAL,	-- Need ON

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

-- ASN1STOP

RRCConnectionSetup-NB field descriptions

dedicatedInfoNAS Downlink NAS PDU in case of mobile terminated CP-EDT. E-UTRAN may include this field only if the RRCConnectionSetup is in response to RRCEarlyDataRequest with establishment cause mt-Access.

#### –	RRCConnectionSetupComplete-NB

The RRCConnectionSetupComplete-NB message is used to confirm the successful completion of an RRC connection establishment.

Signalling radio bearer: SRB1bis

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

RRCConnectionSetupComplete-NB message

-- ASN1START

RRCConnectionSetupComplete-NB ::=	SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE{

rrcConnectionSetupComplete-r13		RRCConnectionSetupComplete-NB-r13-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionSetupComplete-NB-r13-IEs ::= SEQUENCE {

selectedPLMN-Identity-r13				INTEGER (1..maxPLMN-r11),

s-TMSI-r13								S-TMSI							OPTIONAL,

registeredMME-r13						RegisteredMME					OPTIONAL,

dedicatedInfoNAS-r13					DedicatedInfoNAS,

attachWithoutPDN-Connectivity-r13		ENUMERATED {true}				OPTIONAL,

up-CIoT-EPS-Optimisation-r13			ENUMERATED {true}				OPTIONAL,

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

nonCriticalExtension					RRCConnectionSetupComplete-NB-v1430-IEs	OPTIONAL

}

RRCConnectionSetupComplete-NB-v1430-IEs ::= SEQUENCE {

gummei-Type-r14							ENUMERATED { mapped}	OPTIONAL,

dcn-ID-r14								INTEGER (0..65535)			OPTIONAL,

nonCriticalExtension					RRCConnectionSetupComplete-NB-v1470-IEs	OPTIONAL

}

RRCConnectionSetupComplete-NB-v1470-IEs ::= SEQUENCE {

measResultServCell-r14						MeasResultServCell-NB-r14	OPTIONAL,

nonCriticalExtension						RRCConnectionSetupComplete-NB-v1610-IEs	OPTIONAL

}

RRCConnectionSetupComplete-NB-v1610-IEs ::= SEQUENCE {

ng-5G-S-TMSI-r16							NG-5G-S-TMSI-r15			OPTIONAL,

registeredAMF-r16							RegisteredAMF-r15			OPTIONAL,

gummei-Type-v1610							ENUMERATED {mappedFrom5G}	OPTIONAL,

guami-Type-r16								ENUMERATED {native, mapped}	OPTIONAL,

s-NSSAI-list-r16							SEQUENCE(SIZE (1..maxNrofS-NSSAI-r15)) OF

S-NSSAI-r15		OPTIONAL,

ng-U-DataTransfer-r16						ENUMERATED {true}			OPTIONAL,

up-CIoT-5GS-Optimisation-r16				ENUMERATED {true}			OPTIONAL,

rlf-InfoAvailable-r16						ENUMERATED {true}			OPTIONAL,

anr-InfoAvailable-r16						ENUMERATED {true}			OPTIONAL,

pur-ConfigID-r16							PUR-ConfigID-NB-r16			OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

-- ASN1STOP

RRCConnectionSetupComplete-NB field descriptions

anr-InfoAvailable This field is used to indicate the availability of ANR measurement information.

attachWithoutPDN-Connectivity This field is used to indicate that the UE performs an Attach without PDN connectivity procedure, as indicated by the upper layers, TS 24.301 [35].

dcn-ID The Dedicated Core Network Identity, see TS 23.401 [41].

guami-Type This field is used to indicate whether the GUAMI included is native (derived from native 5G-GUTI) or mapped (from EPS, derived from EPS GUTI) as specified in TS 24.501 [95].

gummei-Type This field is used to indicate that the GUMMEI included is mapped (from 2G/3G identifiers or 5G identifiers) as indicated by the upper layers, TS 24.301 [35] and TS 24.501 [95]. The value mapped indicates the GUMMEI is mapped from 2G/3G identifiers, and mappedFrom5G indicates the GUMMEI is mapped from 5G identifiers. A UE shall not include both gummei-Type-r14 and gummei-Type-v1610.

measResultServCell This field refers to the last idle mode measurement results taken of the serving cell.

ng-U-DataTransfer This field is included when the UE supports NG-U data transfer, as indicated by the upper layers, see TS 24.501 [95].

registeredAMF This field is used to transfer the GUAMI of the AMF where the UE is registered, as provided by upper layers, see TS 23.003 [27].

registeredMME This field is used to transfer the GUMMEI of the MME where the UE is registered, as provided by upper layers.

rlf-InfoAvailable This field is used to indicate the availability of radio link failure related information.

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityList included in SystemInformationBlockType1-NB. 1 if the 1st PLMN is selected from the plmn-IdentityList included in SIB1, 2 if the 2nd PLMN is selected from the plmn-IdentityList included in SIB1 and so on.

s-NSSAI-List This field is a list of S-NSSAI as indicated by the upper layers. The UE can report up to eight S-NSSAI per NSSAI, see TS 23.003 [27].

up-CIoT-5GS-Optimisation This field is included when the UE supports User plane CIoT 5GS Optimisation, as indicated by the upper layers, see TS 24.501 [95].

up-CIoT-EPS-Optimisation This field is included when the UE supports S1-U data transfer or the User plane CIoT EPS Optimisation, as indicated by the upper layers, see TS 24.301 [35].

#### –	RRCEarlyDataComplete-NB

The RRCEarlyDataComplete-NB message is used to confirm the successful completion of the CP-EDT procedure.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: E-UTRAN to UE

RRCEarlyDataComplete-NB message

-- ASN1START

RRCEarlyDataComplete-NB-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

rrcEarlyDataComplete-r15			RRCEarlyDataComplete-NB-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCEarlyDataComplete-NB-r15-IEs ::=	SEQUENCE {

dedicatedInfoNAS-r15				DedicatedInfoNAS				OPTIONAL,	-- Need ON

extendedWaitTime-r15				INTEGER (1..1800)				OPTIONAL,	-- Need ON

redirectedCarrierInfo-r15			RedirectedCarrierInfo-NB-r13	OPTIONAL,	-- Need ON

redirectedCarrierInfoExt-r15		RedirectedCarrierInfo-NB-v1430	OPTIONAL,	-- Cond Redirection

nonCriticalExtension				RRCEarlyDataComplete-NB-v1590-IEs	OPTIONAL

}

RRCEarlyDataComplete-NB-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension					OCTET STRING					OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

-- ASN1STOP

RRCEarlyDataComplete-NB field descriptions

extendedWaitTime Value in seconds.

| Conditional presence | Explanation |
| --- | --- |
| Redirection | The field is optionally present, Need ON, if redirectedCarrierInfo is included; otherwise the field is not present. |

#### –	RRCEarlyDataRequest-NB

The RRCEarlyDataRequest-NB message is used to initiate CP-EDT.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to E-UTRAN

RRCEarlyDataRequest-NB message

-- ASN1START

RRCEarlyDataRequest-NB-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

rrcEarlyDataRequest-r15				RRCEarlyDataRequest-NB-r15-IEs,

later								CHOICE {

rrcEarlyDataRequest-r16				RRCEarlyDataRequest-5GC-NB-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

}

RRCEarlyDataRequest-NB-r15-IEs ::=	SEQUENCE {

s-TMSI-r15							S-TMSI,

establishmentCause-r15				ENUMERATED {mo-Data, mo-ExceptionData, delayTolerantAccess, mt-Access-v1610},

cqi-NPDCCH-r15						CQI-NPDCCH-NB-r14						OPTIONAL,

dedicatedInfoNAS-r15				DedicatedInfoNAS,

nonCriticalExtension				RRCEarlyDataRequest-NB-v1590-IEs		OPTIONAL

}

RRCEarlyDataRequest-NB-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension				OCTET STRING				OPTIONAL,

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

RRCEarlyDataRequest-5GC-NB-r16-IEs ::=	SEQUENCE {

ng-5G-S-TMSI-r16					NG-5G-S-TMSI-r15,

establishmentCause-r16				ENUMERATED {mo-Data, mo-ExceptionData, mt-Access, spare1},

cqi-NPDCCH-r16						CQI-NPDCCH-NB-r14			OPTIONAL,

dedicatedInfoNAS-r16				DedicatedInfoNAS,

lateNonCriticalExtension			OCTET STRING				OPTIONAL,

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- ASN1STOP

RRCEarlyDataRequest-NB field descriptions

establishmentCause Provides the establishment cause for the RRC early data request as provided by the upper layers. eNB is not expected to reject a RRCEarlyDataRequest due to unknown cause value being used by the UE.

#### –	SCPTMConfiguration-NB

The SCPTMConfiguration-NB message contains the control information applicable for MBMS services transmitted via SC-MRB.

Signalling radio bearer: N/A

RLC-SAP: UM

Logical channel: SC-MCCH

Direction: E-UTRAN to UE

SCPTMConfiguration-NB message

-- ASN1START

SCPTMConfiguration-NB-r14 ::=	SEQUENCE {

sc-mtch-InfoList-r14			SC-MTCH-InfoList-NB-r14,

scptm-NeighbourCellList-r14		SCPTM-NeighbourCellList-NB-r14		OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SCPTMConfiguration-NB-v1610	OPTIONAL

}

SCPTMConfiguration-NB-v1610 ::=	SEQUENCE {

sc-mtch-InfoListMultiTB-r16		SC-MTCH-InfoList-NB-r14,

multiTB-Gap-r16					ENUMERATED {sf16, sf32, sf64, sf128}	OPTIONAL,	-- Need OR

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

SCPTMConfiguration-NB field descriptions

multiTB-Gap Indicates the scheduling gap for SC-MTCH using multiple TB scheduling, see TS 36.211 [21] and TS 36.213 [23]. Value sf16 corresponds to 16 subframes, sf32 corresponds to 32 subframes, and so on. If the field is absent, there is no scheduling gap.

sc-mtch-InfoList Provides the configuration of each SC-MTCH not using multiple TB scheduling in the current cell.

sc-mtch-InfoListMultiTB Provides the configuration of each SC-MTCH using multiple TB scheduling in the current cell. The total number of signalled SC-MTCH configuration in sc-mtch-InfoList and sc-mtch-InfoListMultiTB cannot be more than maxSC-MTCH-NB-r14.

scptm-NeighbourCellList List of neighbour cells providing MBMS services via SC-MRB. When absent, the UE shall assume that MBMS services listed in the SCPTMConfiguration-NB message are not provided via SC-MRB in any neighbour cell.

#### –	SystemInformation-NB

The SystemInformation-NB message is used to convey one or more System Information Blocks. All the SIBs included are transmitted with the same periodicity.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

SystemInformation-NB message

-- ASN1START

SystemInformation-NB ::=		SEQUENCE {

criticalExtensions					CHOICE {

systemInformation-r13				SystemInformation-NB-r13-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

SystemInformation-NB-r13-IEs ::=	SEQUENCE {

sib-TypeAndInfo-r13					SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

sib2-r13							SystemInformationBlockType2-NB-r13,

sib3-r13							SystemInformationBlockType3-NB-r13,

sib4-r13							SystemInformationBlockType4-NB-r13,

sib5-r13							SystemInformationBlockType5-NB-r13,

sib14-r13							SystemInformationBlockType14-NB-r13,

sib16-r13							SystemInformationBlockType16-NB-r13,

...,

sib15-v1430							SystemInformationBlockType15-NB-r14,

sib20-v1430							SystemInformationBlockType20-NB-r14,

sib22-v1430							SystemInformationBlockType22-NB-r14,

sib23-v1530							SystemInformationBlockType23-NB-r15,

sib27-v1610							SystemInformationBlockType27-NB-r16,

sibXX-v17xx							SystemInformationBlockTypeXX-NB-r17

},

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	SystemInformationBlockType1-NB

The SystemInformationBlockType1-NB message contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

SystemInformationBlockType1-NB message

-- ASN1START

SystemInformationBlockType1-NB ::=	SEQUENCE {

hyperSFN-MSB-r13					BIT STRING (SIZE (8)),

cellAccessRelatedInfo-r13			SEQUENCE {

plmn-IdentityList-r13				PLMN-IdentityList-NB-r13,

trackingAreaCode-r13				TrackingAreaCode,

cellIdentity-r13					CellIdentity,

cellBarred-r13						ENUMERATED {barred, notBarred},

intraFreqReselection-r13			ENUMERATED {allowed, notAllowed}

},

cellSelectionInfo-r13				SEQUENCE {

q-RxLevMin-r13						Q-RxLevMin,

q-QualMin-r13						Q-QualMin-r9

},

p-Max-r13							P-Max					OPTIONAL,	-- Need OP

freqBandIndicator-r13				FreqBandIndicator-NB-r13,

freqBandInfo-r13					NS-PmaxList-NB-r13				OPTIONAL,	-- Need OR

multiBandInfoList-r13				MultiBandInfoList-NB-r13		OPTIONAL,	-- Need OR

downlinkBitmap-r13					DL-Bitmap-NB-r13				OPTIONAL,	-- Cond SIB1

eutraControlRegionSize-r13			ENUMERATED {n1, n2, n3}			OPTIONAL,	-- Cond inband

nrs-CRS-PowerOffset-r13				ENUMERATED {dB-6,      dB-4dot77, dB-3,

dB-1dot77, dB0,       dB1,

dB1dot23,  dB2,       dB3,

dB4,       dB4dot23,  dB5,

dB6,       dB7,       dB8,

dB9}		OPTIONAL,	-- Cond inband-SamePCI

schedulingInfoList-r13				SchedulingInfoList-NB-r13,

si-WindowLength-r13					ENUMERATED {ms160, ms320, ms480, ms640,

ms960, ms1280, ms1600, spare1},

si-RadioFrameOffset-r13				INTEGER (1..15)		OPTIONAL,	-- Need OP

systemInfoValueTagList-r13			SystemInfoValueTagList-NB-r13	OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-NB-v1350	OPTIONAL

}

SystemInformationBlockType1-NB-v1350 ::=	SEQUENCE {

cellSelectionInfo-v1350				CellSelectionInfo-NB-v1350	OPTIONAL,	-- Cond Qrxlevmin

nonCriticalExtension				SystemInformationBlockType1-NB-v1430	OPTIONAL

}

SystemInformationBlockType1-NB-v1430 ::=	SEQUENCE {

cellSelectionInfo-v1430				CellSelectionInfo-NB-v1430		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-NB-v1450					OPTIONAL

}

SystemInformationBlockType1-NB-v1450 ::= SEQUENCE {

nrs-CRS-PowerOffset-v1450				ENUMERATED {dB-6,  dB-4dot77, dB-3,

dB-1dot77, dB0,       dB1,

dB1dot23,  dB2,       dB3,

dB4,       dB4dot23,  dB5,

dB6,       dB7,       dB8,

dB9}		OPTIONAL,	-- Cond inband-SamePCI-ExceptAnchor

nonCriticalExtension				SystemInformationBlockType1-NB-v1530					OPTIONAL

}

SystemInformationBlockType1-NB-v1530 ::= SEQUENCE {

tdd-Parameters-r15						SEQUENCE {

tdd-Config-r15							TDD-Config-NB-r15,

tdd-SI-CarrierInfo-r15					ENUMERATED {anchor, non-anchor},

tdd-SI-SubframesBitmap-r15				DL-Bitmap-NB-r13		OPTIONAL	-- Cond TDD-SI-NonAnchor

}	OPTIONAL,	-- Cond TDD

schedulingInfoList-v1530			SchedulingInfoList-NB-v1530		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-NB-v1610	OPTIONAL

}

SystemInformationBlockType1-NB-v1610 ::= SEQUENCE {

cellAccessRelatedInfo-5GC-r16			SEQUENCE {

plmn-IdentityList-r16				PLMN-IdentityList-5GC-NB-r16,

trackingAreaCode-5GC-r16			TrackingAreaCode-5GC-r15,

cellIdentity-r16					CellIdentity	OPTIONAL,	-- Need OP

cellBarred-5GC-r16					ENUMERATED {barred, notBarred}

}	OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-NB-v17xx	OPTIONAL

}

SystemInformationBlockType1-NB-v17xx ::= SEQUENCE {

cellAccessRelatedInfo-v17xx				SEQUENCE {

trackingAreaList-r17					TrackingAreaList-NB-r17

} OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

PLMN-IdentityList-NB-r13 ::=		SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-NB-r13

PLMN-IdentityList-5GC-NB-r16 ::=	SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-5GC-NB-r16

PLMN-IdentityInfo-NB-r13 ::=		SEQUENCE {

plmn-Identity-r13						PLMN-Identity,

cellReservedForOperatorUse-r13			ENUMERATED {reserved, notReserved},

attachWithoutPDN-Connectivity-r13		ENUMERATED {true}	OPTIONAL	-- Need OP

}

PLMN-IdentityInfo-5GC-NB-r16 ::=	SEQUENCE {

plmn-Identity-5GC-r16					CHOICE {

plmn-Identity-r16						PLMN-Identity,

plmn-Index-r16							INTEGER (1..maxPLMN-r11)

},

cellReservedForOperatorUse-r16			ENUMERATED {reserved, notReserved},

ng-U-DataTransfer-r16					ENUMERATED {true}	OPTIONAL,	-- Need OR

up-CIoT-5GS-Optimisation-r16			ENUMERATED {true}	OPTIONAL	-- Need OR

}

TrackingAreaList-NB-r17 ::= SEQUENCE (SIZE (1..maxTAC-NB-r17)) OF  TrackingAreaCode

SchedulingInfoList-NB-r13 ::= SEQUENCE (SIZE (1..maxSI-Message-NB-r13)) OF SchedulingInfo-NB-r13

SchedulingInfoList-NB-v1530 ::= SEQUENCE (SIZE (1..maxSI-Message-NB-r13)) OF SchedulingInfo-NB-v1530

SchedulingInfo-NB-r13::=		SEQUENCE {

si-Periodicity-r13				ENUMERATED {rf64, rf128, rf256, rf512,

rf1024, rf2048, rf4096, spare},

si-RepetitionPattern-r13		ENUMERATED {every2ndRF, every4thRF, every8thRF, every16thRF},

sib-MappingInfo-r13				SIB-MappingInfo-NB-r13,

si-TB-r13						ENUMERATED {b56, b120, b208, b256, b328, b440, b552, b680}

}

SchedulingInfo-NB-v1530::=		SEQUENCE {

sib-MappingInfo-v1530				SIB-MappingInfo-NB-v1530	OPTIONAL	-- Need OR

}

SystemInfoValueTagList-NB-r13 ::=	SEQUENCE (SIZE (1.. maxSI-Message-NB-r13)) OF

SystemInfoValueTagSI-r13

SIB-MappingInfo-NB-r13 ::=			SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type-NB-r13

SIB-MappingInfo-NB-v1530 ::=		SEQUENCE (SIZE (1..8)) OF SIB-Type-NB-v1530

SIB-Type-NB-r13 ::=					ENUMERATED {

sibType3-NB-r13, sibType4-NB-r13, sibType5-NB-r13,

sibType14-NB-r13, sibType16-NB-r13, sibType15-NB-r14,

sibType20-NB-r14, sibType22-NB-r14}

SIB-Type-NB-v1530 ::=				ENUMERATED {

sibType23-NB-r15, sibType27-NB-r16, sibTypeXX-NB-r17, spare5,

spare4, spare3, spare2, spare1}

CellSelectionInfo-NB-v1350 ::=		SEQUENCE {

delta-RxLevMin-v1350				INTEGER (-8..-1)

}

CellSelectionInfo-NB-v1430 ::=		SEQUENCE {

powerClass14dBm-Offset-r14			ENUMERATED {dB-6, dB-3, dB3, dB6, dB9, dB12}	OPTIONAL,	--	Need OP

ce-authorisationOffset-r14			ENUMERATED {dB5, dB10, dB15, dB20, dB25, dB30, dB35}	OPTIONAL	--	Need OP

}

-- ASN1STOP

Editor’s Note: Agreement: Satellite assistance information will be used by the UE for predicting coverage discontinuity. The details of the assistance information is FFS. FFS whether any applicable agreements made in NR-NTN can be reused. Editor: this agreement is captured here until it is decided in which SIB the assistance information is located.

Editor’s Note: Agreement: RAN2 assumes that Satellite assistance information, e.g. for cell selection reselection, for serving cell is provided to UE. Editor: this agreement is captured here until it is decided in which SIB the assistance information is located.

Editor’s Note: Agreement: FFS if Satellite assistance information for neighbour cell(s) is provided to UE for cell selection/reselection (justification would be needed). Editor: this agreement is captured here until it is decided in which SIB the assistance information is located.

Editor’s Note: Satellite Ephemeris Parameters (not same as for L1 pre-compensation, for the constellation, not just single satellite) is needed for the UE for predicting coverage discontinuity. Other info, e.g. beam info, elevation angle, reference location or corresponding is FFS. Editor: this agreement is captured here until it is decided in which SIB the assistance information is located.

Editor’s Note: Providing the start-time of (incoming) satellite’s coverage and end time of serving satellite’s coverage is needed for Quasi-Earth Fixed satellites. Editor: this agreement is captured here until it is decided in which SIB the incoming satellite assistance information is located

Editor’s Note: FFS whether the maximum number of TACs that can be broadcast is the same for NB-IoT and eMTC

Editor’s Note: Agreement: The network may broadcast more than one TAC per PLMN in a cell, which is up to network implementation. Editor: Enhanced RAN sharing is not supported in NB-IoT, should that be introduced for NTN?

Editor’s Note: Agreement: When the network stops broadcasting a TAC, the UE needs to know it. FFS how this is done. Editor: this agreement is captured here until the FFS is resolved.

Editor’s Note: Agreement: It is feasible to use the legacy barring bit to block legacy UEs, and it is possible to have a new bit that assumes the functionality of the old bit. It is FFS if it is needed to use the barring bit or whether other mechanism can be assumed (new band etc).

Editor’s Note: Agreement: K_mac value is broadcasted by network. Editor: this agreement is captured here until it is decided in which SIB the information is located.

SystemInformationBlockType1-NB field descriptions

attachWithoutPDN-Connectivity If present, the field indicates that attach without PDN connectivity as specified in TS 24.301 [35] is supported for this PLMN.

ce-authorisationOffset Parameter "Qoffsetauthorization" in TS 36.304 [4]. Value in dB. Value dB5 corresponds to 5 dB, dB10 corresponds to 10 dB and so on. If the field is absent, the value of 0 dB shall be used for "Qoffsetauthorization".

cellBarred Barred means the cell is barred for connectivity to EPC, as defined in TS 36.304 [4].

cellBarred-5GC Barred means the cell is barred for connectivity to 5GC, as defined in TS 36.304 [4].

cellIdentity Indicates the cell identity. If the field is absent in cellAccessRelatedInfo-5GC, the cell identity indicated by the cellIdentity field included in cellAccessRelatedInfo for EPC is used when connected to 5GC.

cellReservedForOperatorUse As defined in TS 36.304 [4].

cellSelectionInfo Cell selection information as specified in TS 36.304 [4].

downlinkBitmap For FDD, NB-IoT downlink subframe configuration for downlink transmission as specified in TS 36.213 [23], clause 16.4. For TDD, NB-IoT downlink, uplink and special subframes configuration for transmission on the anchor carrier as specified in TS 36.213 [23], clause 16.4. If the bitmap is not present, the UE shall assume that all subframes are valid (except for subframes carrying NPSS/NSSS/NPBCH/SIB1-NB) as specified in TS 36.213 [23], clause 16.4.

eutraControlRegionSize Indicates the control region size of the E-UTRA cell for the in-band operation mode, see TS 36.213 [23]. Unit is in number of OFDM symbols.

freqBandInfo A list of additionalPmax and additionalSpectrumEmission values as defined in TS 36.101 [42], clause 6.2.4F for the frequency band in freqBandIndicator.

hyperSFN-MSB Indicates the 8 most significant bits of hyper-SFN. Together with hyperSFN-LSB in MIB-NB, the complete hyper-SFN is built up. hyper-SFN is incremented by one when the SFN wraps around.

intraFreqReselection Used to control cell reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 36.304 [4].

multiBandInfoList A list of additional frequency band indicators, additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42], table 5.5-1. If the UE supports the frequency band in the freqBandIndicator IE it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList IE.

ng-U-DataTransfer Indicates whether the NG-U data transfer as specified in TS 24.501 [95] is supported.

nrs-CRS-PowerOffset NRS power offset between NRS and E-UTRA CRS, see TS 36.213 [23], clause 16.2.2. Unit in dB. Default value of 0.

plmn-IdentityList List of PLMN identities. The first listed PLMN-Identity is the primary PLMN.

plmn-Index Index of the PLMN in the plmn-IdentityList field included in cellAccessRelatedInfo for EPC, indicating the same PLMN ID is used when connected to 5GC.

powerClass14dBm-Offset Parameter "Poffset" in TS 36.304 [4]. Only applicable for UE supporting powerClassNB-14dBm. Value in dB. Value dB-6 corresponds to -6 dB, dB-3 corresponds to -3 dB and so on. If the fied is absent, the UE applies the (default) value of 0 dB for "Poffset" in TS 36.304 [4].

p-Max Value applicable for the cell. If absent the UE applies the maximum power according to the UE capability.

q-QualMin Parameter "Qqualmin" in TS 36.304 [4].

q-RxLevMin, delta-RxLevMin Parameter Qrxlevmin in TS 36.304 [4]. If delta-RxLevMin is not included, actual value Qrxlevmin = q-RxLevMin * 2 [dBm]. If delta-RxLevMin is included, actual value Qrxlevmin = (q-RxLevMin + delta-RxLevMin) * 2 [dBm].

schedulingInfoList Indicates additional scheduling information of SI messages.

si-Periodicity Periodicity of the SI-message in radio frames, such that rf256 denotes 256 radio frames, rf512 denotes 512 radio frames, and so on.

si-RadioFrameOffset Offset in number of radio frames to calculate the start of the SI window. If the field is absent, no offset is applied.

si-RepetitionPattern Indicates the starting radio frames within the SI window used for SI message transmission. Value every2ndRF corresponds to every 2 radio frames, value every4thRF corresponds to every 4 radio frames and so on. The first transmission of the SI message is transmitted from the first radio frame of the SI window.

si-TB This field indicates the transport block size in number of bits and the corresponding number of consecutive NB-IoT downlink subframes that are used to broadcast the SI message. Value b56 corresponds to 56 bits, b120 corresponds to 120 bits and so on. TBS of 56 bits and 120 bits are transmitted over 2 sub-frames, other TBS are transmitted over 8 sub-frames, see TS 36.213 [23], Table 16.4.1.5.1-1.

si-WindowLength Common SI scheduling window for all SIs. Unit in milliseconds, where ms160 denotes 160 milliseconds, ms320 denotes 320 milliseconds and so on.

sib-MappingInfo List of the SIBs mapped to this SystemInformation message. There is no mapping information of SIB2-NB; it is always present in the first SystemInformation message listed in the schedulingInfoList list.

systemInfoValueTagList Indicates SI message specific value tags. It includes the same number of entries, and listed in the same order, as in SchedulingInfoList.

systemInfoValueTagSI SI message specific value tag as specified in Clause 5.2.1.3. Common for all SIBs within the SI message other than SIB14-NB.

tdd-Config Indicates the the TDD specific physical channel configuration.

tdd-SI-CarrierInfo Carrier used for SI message transmission. Value anchor corresponds to anchor carrier, value non-anchor corresponds to non-anchor carrier. See TS 36.213 [23]. When tdd-SI-CarrierInfo set to value non-anchor then sib-GuardbandInfo in MIB-TDD-NB (in case of operationmodeInfo is set to guardband) or sib-InbandLocation in MIB-TDD-NB (in case of operationmodeInfo is set to inband-SamePCI or inband-DifferentPCI) or sib-StandaloneLocation in MIB-TDD-NB (in case of operationmodeInfo is set to standalone) defines which non-anchor carrier is used (see MIB-NB-TDD).

tdd-SI-SubframesBitmap NB-IoT downlink, uplink and special subframes configuration for transmission on the carrier carrying the SI message as specified in TS 36.213 [23], clause 16.4.

trackingAreaCode, trackingAreaCode-5GC A trackingAreaCode that is common for all the PLMNs listed.

trackingAreaList A list of additional tracking area codes that is common for all the PLMNs listed.

up-CIoT-5GS-Optimisation Indicates whether the UE is allowed to resume the connection with User plane CIoT 5GS Optimisation, see TS24.501 [95].

| Conditional presence | Explanation |
| --- | --- |
| inband | In FDD: The field is mandatory present if IE operationModeInfo in MIB-NB is set to inband-SamePCI or inband-DifferentPCI. Otherwise the field is not present. In TDD: The field is mandatory present if: - IE operationModeInfo in MIB-TDD-NB is set to inband-SamePCI or inband-DifferentPCI or - IE operationModeInfo in MIB-TDD-NB is set to guardband and IE sib-GuardbandInfo in MIB-TDD-NB is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI and IE tdd-SI-CarrierInfo is set to non-anchor |
| inband-SamePCI | The field is mandatory present, if IE operationModeInfo in MIB-NB is set to inband-SamePCI. Otherwise the field is not present. |
| inband-SamePCI-ExceptAnchor | The field is optionally present if IE operationModeInfo in MIB-NB is set to a value other than inband-SamePCI, and at least one non-anchor carrier is inband carrier and uses the same PCI as the E-UTRA carrier. Otherwise the field is not present. |
| Qrxlevmin | This field is optionally present, Need OR, if q-RxLevMin is set to the minimum value. Otherwise the field is not present. |
| SIB1 | The field is mandatory present if IE additionalTransmissionSIB1 in MIB-NB is set to TRUE. Otherwise the field is optionally present, Need OP. |
| TDD | The field is mandatory present for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD-SI-NonAnchor | The field is mandatory present for TDD if si-CarrierInfo is set to non-anchor; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	UECapabilityEnquiry-NB

The UECapabilityEnquiry-NB message is used to request the transfer of UE radio access capabilities for NB-IoT.

Signalling radio bearer: SRB1 or SRB1bis

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

UECapabilityEnquiry-NB message

-- ASN1START

UECapabilityEnquiry-NB ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

ueCapabilityEnquiry-r13				UECapabilityEnquiry-NB-r13-IEs,

spare1								NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

UECapabilityEnquiry-NB-r13-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	UECapabilityInformation-NB

The UECapabilityInformation-NB message is used to transfer of UE radio access capabilities requested by the E-UTRAN.

Signalling radio bearer: SRB1 or SRB1bis

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UECapabilityInformation-NB message

-- ASN1START

UECapabilityInformation-NB ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE{

ueCapabilityInformation-r13		UECapabilityInformation-NB-r13-IEs,

criticalExtensionsFuture		SEQUENCE {}

}

}

UECapabilityInformation-NB-r13-IEs ::=	SEQUENCE {

ue-Capability-r13						UE-Capability-NB-r13,

ue-RadioPagingInfo-r13					UE-RadioPagingInfo-NB-r13,

lateNonCriticalExtension				OCTET STRING						OPTIONAL,

nonCriticalExtension					UECapabilityInformation-NB-Ext-r14-IEs							OPTIONAL

}

UECapabilityInformation-NB-Ext-r14-IEs ::=	SEQUENCE {

ue-Capability-ContainerExt-r14			OCTET STRING (CONTAINING UE-Capability-NB-Ext-r14-IEs),

nonCriticalExtension					SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

UECapabilityInformation-NB field descriptions

ue-RadioPagingInfo This field contains UE capability information used for paging.

#### –	UEInformationRequest-NB

The UEInformationRequest-NB is the command used by E-UTRAN to retrieve information from the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

UEInformationRequest-NB message

-- ASN1START

UEInformationRequest-NB-r16	::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

ueInformationRequest-r16			UEInformationRequest-NB-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

UEInformationRequest-NB-r16-IEs ::=		SEQUENCE {

rach-ReportReq-r16					BOOLEAN,

rlf-ReportReq-r16					BOOLEAN,

anr-ReportReq-r16					BOOLEAN,

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

UEInformationRequest-NB field descriptions

anr-ReportReq Indicates whether the UE shall report, if available, ANR measurement information.

rach-ReportReq Indicates whether the UE shall report, if available, information about the random access procedure.

rlf-ReportReq Indicates whether the UE shall report, if available, information about radio link failure.

#### –	UEInformationResponse-NB

The UEInformationResponse-NB message is used by the UE to transfer the information requested by the E-UTRAN.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UEInformationResponse-NB message

-- ASN1START

UEInformationResponse-NB-r16		::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

ueInformationResponse-r16			UEInformationResponse-NB-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

UEInformationResponse-NB-r16-IEs ::=	SEQUENCE {

rach-Report-r16							RACH-Report-NB-r16					OPTIONAL,

rlf-Report-r16							RLF-Report-NB-r16					OPTIONAL,

anr-MeasReport-r16						ANR-MeasReport-NB-r16				OPTIONAL,

lateNonCriticalExtension				OCTET STRING						OPTIONAL,

nonCriticalExtension					SEQUENCE {}							OPTIONAL

}

RACH-Report-NB-r16 ::=					SEQUENCE {

numberOfPreamblesSent-r16				INTEGER (1..64),

contentionDetected-r16					BOOLEAN,

initialNRSRP-Level-r16					INTEGER (0..2),

edt-Fallback-r16						BOOLEAN

}

RLF-Report-NB-r16 ::=					SEQUENCE {

failedPCellId-r16						CellGlobalIdEUTRA,

reestablishmentCellId-r16				CellGlobalIdEUTRA					OPTIONAL,

locationInfo-r16						LocationInfo-r10					OPTIONAL,

measResultLastServCell-r16				SEQUENCE {

nrsrpResult-r16							NRSRP-Range-NB-r14,

nrsrqResult-r16							NRSRQ-Range-NB-r14				OPTIONAL

},

timeSinceFailure-r16					TimeSinceFailure-r11				OPTIONAL

}

-- ASN1STOP

UEInformationResponse-NB field descriptions

anr-MeasReport Indicates the ANR measurement information.

contentionDetected Value TRUE indicates that contention was detected for at least one of the transmitted preambles, see TS 36.321 [6].

edt-Fallback Value TRUE indicates that EDT fallback indication was received from the lower layers, see TS 36.321 [6].

failedPCellId Indicates the PCell in which RLF is detected.

initialNRSRP-Level Indicates the NRSRP level of the NPRACH resource selected for the first preamble transmission.

measResultLastServCell Refers to the last measurement results taken in the PCell, where radio link failure happened.

numberOfPreamblesSent Indicates the number of RACH preambles that were transmitted. Corresponds to parameter PREAMBLE_TRANSMISSION_COUNTER in TS 36.321 [6].

reestablishmentCellId Indicates the cell in which the re-establishment attempt was made after connection failure.

timeSinceFailure Indicates the time that elapsed since the connection failure. Value in seconds. The maximum value 172800 means 172800s or longer.

#### –	ULInformationTransfer-NB

The ULInformationTransfer-NB message is used for the uplink transfer of NAS information.

Signalling radio bearer: SRB1 or SRB1bis

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ULInformationTransfer-NB message

-- ASN1START

ULInformationTransfer-NB ::=	SEQUENCE {

criticalExtensions					CHOICE {

ulInformationTransfer-r13		ULInformationTransfer-NB-r13-IEs,

criticalExtensionsFuture		SEQUENCE {}

}

}

ULInformationTransfer-NB-r13-IEs ::=	SEQUENCE {

dedicatedInfoNAS-r13					DedicatedInfoNAS,

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

nonCriticalExtension					SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

### 6.7.3	NB-IoT information elements

#### 6.7.3.1	NB-IoT System information blocks

#### –	SystemInformationBlockType2-NB

The IE SystemInformationBlockType2-NB contains radio resource configuration information that is common for all UEs.

NOTE:	UE timers and constants related to functionality for which parameters are provided in another SIB are included in the corresponding SIB.

SystemInformationBlockType2-NB information element

-- ASN1START

SystemInformationBlockType2-NB-r13 ::=	SEQUENCE {

radioResourceConfigCommon-r13			RadioResourceConfigCommonSIB-NB-r13,

ue-TimersAndConstants-r13				UE-TimersAndConstants-NB-r13,

freqInfo-r13							SEQUENCE {

ul-CarrierFreq-r13						CarrierFreq-NB-r13			OPTIONAL,	-- Need OP

additionalSpectrumEmission-r13			AdditionalSpectrumEmission

},

timeAlignmentTimerCommon-r13			TimeAlignmentTimer,

multiBandInfoList-r13	SEQUENCE (SIZE (1..maxMultiBands)) OF AdditionalSpectrumEmission		OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...,

[[	cp-Reestablishment-r14				ENUMERATED {true}				OPTIONAL		-- Need OP

]],

[[	servingCellMeasInfo-r14				ENUMERATED {true}				OPTIONAL,		-- Need OR

cqi-Reporting-r14					ENUMERATED {true}				OPTIONAL		-- Need OR

]],

[[	enhancedPHR-r15						ENUMERATED {true}		OPTIONAL,	-- Need OR

freqInfo-v1530						SEQUENCE {

tdd-UL-DL-AlignmentOffset-r15		TDD-UL-DL-AlignmentOffset-NB-r15

}	OPTIONAL,		-- Cond TDD

cp-EDT-r15							ENUMERATED {true}		OPTIONAL,	-- Need OR

up-EDT-r15							ENUMERATED {true}		OPTIONAL	-- Need OR

]],

[[	earlySecurityReactivation-r16		ENUMERATED {true}		OPTIONAL,	-- Need OR

cp-EDT-5GC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

up-EDT-5GC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

cp-PUR-EPC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

up-PUR-EPC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

cp-PUR-5GC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

up-PUR-5GC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

rai-ActivationEnh-r16				ENUMERATED {true}		OPTIONAL	-- Need OR

]],

[[	ta-Report-r17						ENUMERATED {true}		OPTIONAL	-- Need OR

-- Editor’s Note: FFS on where this parameter is placed

]]

}

-- ASN1STOP

SystemInformationBlockType2-NB field descriptions

additionalSpectrumEmission The UE requirements related to IE AdditionalSpectrumEmission are defined in TS 36.101 [42], clause 6.2.4F.

cp-EDT For FDD: This field indicates whether the UE is allowed to initiate CP-EDT when connected to EPC, see 5.3.3.1b.

cp-EDT-5GC For FDD: This field indicates whether the UE is allowed to initiate CP-EDT when connected to 5GC, see 5.3.3.1b.

cp-PUR-5GC For FDD: Indicates whether CP transmission using PUR is allowed in the cell when connected to 5GC, see 5.3.3.1c.

cp-PUR-EPC For FDD: Indicates whether CP transmission using PUR is allowed in the cell when connected to EPC, see 5.3.3.1c.

cp-Reestablishment This field indicates if the NB-IoT UE is allowed to trigger RRC connection re-establishment when AS security has not been activated.

cqi-Reporting For FDD: This field indicates if downlink channel quality reporting in RRCConnectionReestablishmentRequest-NB, RRCConnectionRequest-NB and RRCConnectionResumeRequest-NB message is allowed.

earlySecurityReactivation Indicates that early security reactivation when resuming a suspended RRC connection as specified in 5.3.3.18 is supported.

enhancedPHR For FDD: This field indicates if the NB-IoT UE is allowed to report enhanced PHR in MSG3 as specified in TS 36.321 [6].

multiBandInfoList A list of additionalSpectrumEmission i.e. one for each additional frequency band included in multiBandInfoList in SystemInformationBlockType1-NB, listed in the same order.

rai-ActivationEnh Indicates whether the UE is allowed to report the AS Release Assistance Indication using the DCQR and AS RAI MAC CE as specified in TS 36.321 [6] when connected to EPC.

servingCellMeasInfo This field indicates if serving cell idle mode measurement reporting in RRCConnectionReestablishmentComplete-NB, RRCConnectionResumeComplete-NB and RRCConnectionSetupComplete-NB is allowed.

ta-Report Indicates whether UE specific TA reporting in Msg3 or Msg5 at initial access is enabled as specified in TS 36.321 [6].

tdd-UL-DL-AlignmentOffset Indicates the offset between the UL carrier frequency center with respect to DL carrier frequency center for the anchor carrier.

ul-CarrierFreq For FDD: Uplink carrier frequency as defined in TS 36.101 [42], clause 5.7.3F. If operationModeInfo in the MIB-NB is set to standalone and the field is absent, the value of the carrier frequency is determined by the TX-RX frequency separation defined in TS 36.101 [42], table 5.7.4-1, and the value of the carrier frequency offset is 0. If operationModeInfo in the MIB-NB is not set to standalone, the field is mandatory present. For TDD: This field is absent and the uplink carrier frequency is same as the downlink frequency.

up-EDT For FDD: This field indicates whether the UE is allowed to initiate UP-EDT when connected to EPC, see 5.3.3.1b.

up-EDT-5GC For FDD: This field indicates whether the UE is allowed to initiate UP-EDT when connected to 5GC, see 5.3.3.1b.

up-PUR-5GC For FDD: Indicates whether UP transmission using PUR is allowed in the cell when connected to 5GC, see 5.3.3.1c.

up-PUR-EPC For FDD: Indicates whether UP transmission using PUR is allowed in the cell when connected to EPC, see 5.3.3.1c.

| Conditional presence | Explanation |
| --- | --- |
| TDD | The field is mandatory present for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	SystemInformationBlockType3-NB

The IE SystemInformationBlockType3-NB contains cell re-selection information common for intra-frequency, and inter-frequency cell re-selection as well as intra-frequency cell re-selection information other than neighbouring cell related.

SystemInformationBlockType3-NB information element

-- ASN1START

SystemInformationBlockType3-NB-r13 ::=	SEQUENCE {

cellReselectionInfoCommon-r13			SEQUENCE {

q-Hyst-r13								ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24

}

},

cellReselectionServingFreqInfo-r13		SEQUENCE {

s-NonIntraSearch-r13					ReselectionThreshold

},

intraFreqCellReselectionInfo-r13		SEQUENCE {

q-RxLevMin-r13							Q-RxLevMin,

q-QualMin-r13							Q-QualMin-r9			OPTIONAL,	-- Need OP

p-Max-r13								P-Max					OPTIONAL,	-- Need OP

s-IntraSearchP-r13						ReselectionThreshold,

t-Reselection-r13						T-Reselection-NB-r13

},

freqBandInfo-r13						NS-PmaxList-NB-r13				OPTIONAL,	-- Need OR

multiBandInfoList-r13					SEQUENCE (SIZE (1..maxMultiBands)) OF

NS-PmaxList-NB-r13			OPTIONAL,	-- Need OR

lateNonCriticalExtension					OCTET STRING				OPTIONAL,

...,

[[	intraFreqCellReselectionInfo-v1350	IntraFreqCellReselectionInfo-NB-v1350 OPTIONAL	-- Cond Qrxlevmin

]],

[[	intraFreqCellReselectionInfo-v1360	IntraFreqCellReselectionInfo-NB-v1360 OPTIONAL	-- Need OR

]],

[[	intraFreqCellReselectionInfo-v1430	IntraFreqCellReselectionInfo-NB-v1430 OPTIONAL	-- Need OR

]],

[[	cellReselectionInfoCommon-v1450		CellReselectionInfoCommon-NB-v1450	OPTIONAL	-- Need OR

]],

[[	nsss-RRM-Config-r15					NSSS-RRM-Config-NB-r15	OPTIONAL,	-- Need OR

npbch-RRM-Config-r15				ENUMERATED {enabled}	OPTIONAL	-- Need OR

]]

}

IntraFreqCellReselectionInfo-NB-v1350 ::=	SEQUENCE {

delta-RxLevMin-v1350						INTEGER (-8..-1)

}

IntraFreqCellReselectionInfo-NB-v1360 ::=	SEQUENCE {

s-IntraSearchP-v1360							ReselectionThreshold-NB-v1360

}

IntraFreqCellReselectionInfo-NB-v1430 ::=	SEQUENCE {

powerClass14dBm-Offset-r14		ENUMERATED {dB-6, dB-3, dB3, dB6, dB9, dB12}	OPTIONAL,	-- Need OP

ce-AuthorisationOffset-r14		ENUMERATED {dB5, dB10, dB15, dB20, dB25, dB30, dB35}	OPTIONAL	-- Need OP

}

CellReselectionInfoCommon-NB-v1450 ::=	SEQUENCE {

s-SearchDeltaP-r14					ENUMERATED {dB6, dB9, dB12, dB15}

}

-- ASN1STOP

SystemInformationBlockType3-NB field descriptions

ce-AuthorisationOffset Parameter "Qoffsetauthorization" in TS 36.304 [4]. Value in dB. Value dB5 corresponds to 5 dB, dB10 corresponds to 10 dB and so on. If the field is absent, the UE applies the value of ce-authorisationOffset in SystemInformationBlockType1-NB.

multiBandInfoList A list of additionalPmax and additionalSpectrumEmission values as defined in TS 36.101 [42], clause 6.2.4F, applicable for the intra-frequency neighbouring NB-IoT cells if the UE selects the frequency band from freqBandIndicator in SystemInformationBlockType1-NB.

npbch-RRM-Config For FDD: Configuration for NPBCH-based RRM measurements. See TS 36.214 [24]. If enabled, NPBCH can be used in addition to NRS for RRM measurements for serving cell.

nsss-RRM-Config For FDD: Configuration for NSSS-based RRM measurements for the serving cell.

powerClass14dBm-Offset Parameter "Poffset" in TS 36.304 [4], only applicable for UE supporting powerClassNB-14dBm. Value in dB. Value dB-6 corresponds to -6 dB, dB-3 corresponds to -3 dB and so on. If the field is absent, the UE applies the (default) value of 0 dB for "Poffset" in TS 36.304 [4].

p-Max Value applicable for the intra-frequency neighbouring E-UTRA cells. If absent the UE applies the maximum power according to the UE capability.

q-Hyst Parameter Qhyst in TS 36.304 [4], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on.

q-QualMin Parameter "Qqualmin" in TS 36.304 [4], applicable for intra-frequency neighbour cells. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin, delta-RxLevMin Parameter "Qrxlevmin" in TS 36.304 [4], applicable for intra-frequency neighbour cells. If delta-RxLevMin is not included, actual value Qrxlevmin = q-RxLevMin * 2 [dBm]. If delta-RxLevMin is included, actual value Qrxlevmin = (q-RxLevMin + delta-RxLevMin) * 2 [dBm].

s-IntraSearchP Parameter "SIntraSearchP" in TS 36.304 [4]. In case s-IntraSearchP-v1360 is included, the UE shall ignore s-IntraSearchP (i.e. without suffix).

s-NonIntraSearch Parameter "SnonIntraSearchP" in TS 36.304 [4].

s-SearchDeltaP Parameter "SSearchDeltaP" in TS 36.304 [4]. This parameter is only applicable for UEs supporting relaxed monitoring as specified in TS 36.306 [5]. Value dB6 corresponds to 6 dB, dB9 corresponds to 9 dB and so on.

t-Reselection Parameter "TreselectionNB-IoT_Intra" in TS 36.304 [4].

| Conditional presence | Explanation |
| --- | --- |
| Qrxlevmin | This field is optionally present, Need OR, if q-RxLevMin is set to the minimum value. Otherwise the field is not present. |

#### –	SystemInformationBlockType4-NB

The IE SystemInformationBlockType4-NB contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters.

SystemInformationBlockType4-NB information element

-- ASN1START

SystemInformationBlockType4-NB-r13 ::=		SEQUENCE {

intraFreqNeighCellList-r13			IntraFreqNeighCellList	OPTIONAL,	-- Need OR

intraFreqBlackCellList-r13			IntraFreqBlackCellList	OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING			OPTIONAL,

...,

[[	nsss-RRM-Config-r15				NSSS-RRM-Config-NB-r15	OPTIONAL,	-- Need OR

intraFreqNeighCellList-v1530	IntraFreqNeighCellList-NB-v1530	OPTIONAL	-- Need OR

]]

}

IntraFreqNeighCellList-NB-v1530 ::=		SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo-NB-v1530

IntraFreqNeighCellInfo-NB-v1530 ::=		SEQUENCE {

nsss-RRM-Config-r15						NSSS-RRM-Config-NB-r15	OPTIONAL	-- Cond NSSS-RRM

}

-- ASN1STOP

SystemInformationBlockType4-NB field descriptions

intraFreqBlackCellList List of blacklisted intra-frequency neighbouring cells.

intraFreqNeighCellList List of intra-frequency neighbouring cells with specific cell re-selection parameters.

nsss-RRM-Config For FDD: Configuration for NSSS-based RRM measurements. If intraFreqNeighCellList-NB-v1530 is present then for a cell which is included in intraFreqNeighCellList, the UE applies the nsss-RRM-Config configured in the corresponding entry of IntraFreqNeighCellList-NB-v1530. Otherwise, the UE applies the nsss-RRM-Config configured in SystemInformationBlockType4-NB-r13.

| Conditional presence | Explanation |
| --- | --- |
| NSSS-RRM | This field is optionally present, Need OR, when nsss-RRM-Config is present in SystemInformationBlockType4-NB. Otherwise, the field is not present, and the UE shall delete any existing value for this field. |

#### –	SystemInformationBlockType5-NB

The IE SystemInformationBlockType5-NB contains information relevant only for inter-frequency cell re-selection i.e. information about other NB-IoT frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SystemInformationBlockType5-NB information element

-- ASN1START

SystemInformationBlockType5-NB-r13 ::=	SEQUENCE {

interFreqCarrierFreqList-r13			InterFreqCarrierFreqList-NB-r13,

t-Reselection-r13						T-Reselection-NB-r13,

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...,

[[	scptm-FreqOffset-r14				INTEGER (1..8)					OPTIONAL	-- Need OP

]]

}

InterFreqCarrierFreqList-NB-r13 ::=		SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-NB-r13

InterFreqCarrierFreqInfo-NB-r13 ::=	SEQUENCE {

dl-CarrierFreq-r13					CarrierFreq-NB-r13,

q-RxLevMin-r13						Q-RxLevMin,

q-QualMin-r13						Q-QualMin-r9					OPTIONAL,		-- Need OP

p-Max-r13							P-Max							OPTIONAL,		-- Need OP

q-OffsetFreq-r13					Q-OffsetRange					DEFAULT dB0,

interFreqNeighCellList-r13			InterFreqNeighCellList-NB-r13	OPTIONAL,		-- Need OR

interFreqBlackCellList-r13			InterFreqBlackCellList-NB-r13	OPTIONAL,		-- Need OR

multiBandInfoList-r13				MultiBandInfoList-NB-r13		OPTIONAL,		-- Need OR

...,

[[	delta-RxLevMin-v1350			INTEGER (-8..-1)		OPTIONAL	-- Cond Qrxlevmin

]],

[[	powerClass14dBm-Offset-r14		ENUMERATED {dB-6, dB-3, dB3, dB6, dB9, dB12}

OPTIONAL,	--	Need OP

ce-AuthorisationOffset-r14		ENUMERATED {dB5, dB10, dB15, dB20, dB25, dB30, dB35}	OPTIONAL	-- Need OP

]],

[[	nsss-RRM-Config-r15				NSSS-RRM-Config-NB-r15	OPTIONAL,	-- Need OR

interFreqNeighCellList-v1530	InterFreqNeighCellList-NB-v1530	OPTIONAL -- Need OR

]],

[[	dl-CarrierFreq-v1550			CarrierFreq-NB-v1550	OPTIONAL -- Cond TDD

]]

}

InterFreqNeighCellList-NB-r13 ::=		SEQUENCE (SIZE (1..maxCellInter)) OF PhysCellId

InterFreqNeighCellList-NB-v1530 ::=		SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-NB-v1530

InterFreqNeighCellInfo-NB-v1530 ::=		SEQUENCE {

nsss-RRM-Config-r15						NSSS-RRM-Config-NB-r15	OPTIONAL	-- Cond NSSS-RRM

}

InterFreqBlackCellList-NB-r13 ::=		SEQUENCE (SIZE (1..maxCellBlack)) OF PhysCellId

-- ASN1STOP

SystemInformationBlockType5-NB field descriptions

ce-AuthorisationOffset Parameter "Qoffsetauthorization" in TS 36.304 [4]. Value in dB. Value dB5 corresponds to 5 dB, dB10 corresponds to 10 dB and so on. If the field is absent, the UE applies the value of ce-authorisationOffset in SystemInformationBlockType1-NB.

interFreqBlackCellList List of blacklisted inter-frequency neighbouring cells.

interFreqCarrierFreqList List of neighbouring inter-frequencies. E-UTRAN does not configure more than one entry for the same physical frequency regardless of the E-ARFCN used to indicate this.

interFreqNeighCellList List of inter-frequency neighbouring cells. E-UTRAN may include interFreqNeighCellList when including InterFreqNeighCellList-NB-v1530 to provide cell specific NSSS-based measurement configuration. The UE that does not support NSSS-based RRM measurements shall ignore this field in this version of the specification.

multiBandInfoList Indicates the list of frequency bands, with the associated additionalPmax and additionalSpectrumEmission values as defined in TS 36.101 [42], clause 6.2.4, in addition to the band represented by dl-CarrierFreq for which cell reselection parameters are common.

nsss-RRM-Config For FDD: Configuration for NSSS-based RRM measurements. If InterFreqNeighCellList-NB-v1530 is present then for a cell which is included in interFreqNeighCellList, the UE applies the nsss-RRM-Config configured in the corresponding entry of InterFreqNeighCellList-NB-v1530. Otherwise, the UE applies the nsss-RRM-Config configured in InterFreqCarrierFreqInfo.

p-Max Value applicable for the neighbouring NB-IoT cells on this carrier frequency. If absent the UE applies the maximum power according to the UE capability.

powerClass14dBm-Offset Parameter "Poffset" in TS 36.304 [4], only applicable for UE supporting powerClassNB-14dBm. Value in dB. Value dB-6 corresponds to -6 dB, dB-3 corresponds to -3 dB and so on. If the field is absent, the UE applies the (default) value of 0 dB for "Poffset" in TS 36.304 [4]

q-OffsetFreq Parameter "Qoffsetfrequency" in TS 36.304 [4].

q-QualMin Parameter "Qqualmin" in TS 36.304 [4]. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxlevMin, delta-RxLevMin Parameter "QRxLevmin" in TS 36.304 [4]. If delta-RxLevMin is not included, actual value Qrxlevmin = q-RxLevMin * 2 [dBm]. If delta-RxLevMin is included, actual value Qrxlevmin = (q-RxLevMin + delta-RxLevMin) * 2 [dBm].

scptm-FreqOffset Parameter QoffsetSCPTM in TS 36.304 [4]. Actual value QoffsetSCPTM = field value * 2 [dB]. If the field is absent, the UE uses infinite dBs for the SC-PTM frequency offset with cell ranking as specified in TS 36.304 [4].

t-Reselection Parameter "TreselectionNB-IoT_Inter" in TS 36.304 [4].

| Conditional presence | Explanation |
| --- | --- |
| NSSS-RRM | This field is optionally present, Need OR, when nsss-RRM-Config is present in InterFreqCarrierFreqInfo. Otherwise, the field is not present, and the UE shall delete any existing value for this field. |
| Qrxlevmin | This field is optionally present, Need OR, if q-RxLevMin is set to the minimum value. Otherwise the field is not present. |
| TDD | The field is optionally present, Need OR, in TDD. Otherwise, the field is not present. |

#### –	SystemInformationBlockType14-NB

The IE SystemInformationBlockType14-NB contains the AB parameters for EPC and 5GC.

SystemInformationBlockType14-NB information element

-- ASN1START

SystemInformationBlockType14-NB-r13 ::=	SEQUENCE {

ab-Param-r13					CHOICE {

ab-Common-r13					AB-Config-NB-r13,

ab-PerPLMN-List-r13				SEQUENCE (SIZE (1..maxPLMN-r11)) OF AB-ConfigPLMN-NB-r13

}															OPTIONAL, -- Need OR

lateNonCriticalExtension		OCTET STRING				OPTIONAL,

...,

[[	ab-PerNRSRP-r15				ENUMERATED {thresh1, thresh2}	OPTIONAL	--	Need OR

]],

[[	uac-Param-r16				UAC-Param-NB-r16				OPTIONAL	--	Need OR

]]

}

AB-ConfigPLMN-NB-r13 ::=	SEQUENCE {

ab-Config-r13					AB-Config-NB-r13			OPTIONAL -- Need OR

}

AB-Config-NB-r13 ::=		SEQUENCE {

ab-Category-r13					ENUMERATED {a, b, c},

ab-BarringBitmap-r13			BIT STRING (SIZE(10)),

ab-BarringForExceptionData-r13	ENUMERATED {true}			OPTIONAL,	-- Need OP

ab-BarringForSpecialAC-r13		BIT STRING (SIZE(5))

}

UAC-Param-NB-r16	::=		CHOICE {

uac-BarringCommon			UAC-Barring-NB-r16,

uac-BarringPerPLMN-List		SEQUENCE (SIZE (1..maxPLMN-r11)) OF UAC-Barring-NB-r16

}

UAC-Barring-NB-r16	::=		SEQUENCE {

uac-BarringPerCatList-r16			UAC-BarringPerCatList-NB-r16	OPTIONAL,	-- Need OR

uac-AC1-SelectAssistInfo-r16		UAC-AC1-SelectAssistInfo-r15	OPTIONAL,	-- Need OR

uac-BarringForAccessIdentity-r16	BIT STRING (SIZE(7))

}

UAC-BarringPerCatList-NB-r16 ::= SEQUENCE (SIZE (1..maxAccessCat-1-r15)) OF UAC-BarringPerCat-NB-r16

UAC-BarringPerCat-NB-r16 ::=	SEQUENCE {

uac-accessCategory-r16			INTEGER (1..maxAccessCat-1-r15),

uac-BarringFactor-r16			ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

uac-BarringTime-r16				ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512}

}

-- ASN1STOP

SystemInformationBlockType14-NB field descriptions

ab-BarringBitmap Access class barring for AC 0-9. The first/ leftmost bit is for AC 0, the second bit is for AC 1, and so on.

ab-BarringForExceptionData Indicates whether ExceptionData is subject to access barring.

ab-BarringForSpecialAC Access class barring for AC 11-15. The first/ leftmost bit is for AC 11, the second bit is for AC 12, and so on.

ab-Category Indicates the category of UEs for which AB applies. Value a corresponds to all UEs, value b corresponds to the UEs that are neither in their HPLMN nor in a PLMN that is equivalent to it, and value c corresponds to the UEs that are neither in the PLMN listed as most preferred PLMN of the country where the UEs are roaming in the operator-defined PLMN selector list on the USIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN, see TS 22.011 [10].

ab-Common The AB parameters applicable for all PLMN(s).

ab-Param The AB parameters for connectivity to EPC

ab-PerNRSRP Access barring per NRSRP. Value thresh1 corresponds to the first entry configured in rsrp-ThresholdsPrachInfoList, value thresh2 corresponds to the second entry configured in rsrp-ThresholdsPrachInfoList.

ab-PerPLMN-List The AB parameters per PLMN, listed in the same order as the PLMN(s) occur in plmn-IdentityList in SystemInformationBlockType1-NB.

uac-AC1-SelectAssistInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [96]. The field is forwarded to upper layers, if present.

uac-accessCategory The Access Category according to TS 22.261 [96].

uac-BarringCommon The UAC parameters applicable for all PLMN(s).

uac-BarringFactor Represents the probability that access attempt would be allowed during access barring check.

uac-BarringForAccessIdentity Indicates whether access attempt is allowed for each Access Identity. The leftmost bit, bit 0 in the bit string corresponds to Access Identity 1, bit 1 in the bit string corresponds to Access Identity 2, bit 2 in the bit string corresponds to Access Identity 11, bit 3 in the bit string corresponds to Access Identity 12, and so on. Value 0 means that access attempt is allowed for the corresponding access identity.

uac-BarringPerCatList Access control parameters for each access category for the specific PLMN.

uac-BarringPerPLMN-List The UAC parameters per PLMN, listed in the same order as the PLMN(s) occur in plmn-IdentityList in SystemInformationBlockType1-NB.

uac-BarringTime The average time in seconds before a new access attempt is to be performed after an access attempt was barred at access barring check for the same access category, see 5.3.16.5.

uac-Param The UAC parameters for connectivity to 5GC.

#### –	SystemInformationBlockType15-NB

The IE SystemInformationBlockType15-NB contains the MBMS Service Area Identities (SAI) of the current and/ or neighbouring carrier frequencies.

SystemInformationBlockType15-NB information element

-- ASN1START

SystemInformationBlockType15-NB-r14 ::=	SEQUENCE {

mbms-SAI-IntraFreq-r14					MBMS-SAI-List-r11				OPTIONAL,	-- Need OR

mbms-SAI-InterFreqList-r14				MBMS-SAI-InterFreqList-NB-r14	OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...

}

MBMS-SAI-InterFreqList-NB-r14 ::=		SEQUENCE (SIZE (1..maxFreq)) OF MBMS-SAI-InterFreq-NB-r14

MBMS-SAI-InterFreq-NB-r14 ::=			SEQUENCE {

dl-CarrierFreq-r14						CarrierFreq-NB-r13,

mbms-SAI-List-r14						MBMS-SAI-List-r11,

multiBandInfoList-r14					AdditionalBandInfoList-NB-r14	OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType15-NB field descriptions

mbms-SAI-InterFreqList Contains a list of neighboring frequencies including additional frequency bands, if any, that provide MBMS services and the corresponding MBMS SAIs.

mbms-SAI-IntraFreq Contains the list of MBMS SAIs for the current frequency. A duplicate MBMS SAI indicates that this and all following SAIs are not offered by this cell but only by neighbour cells on the current frequency. For MBMS service continuity, the UE shall use all MBMS SAIs listed in mbms-SAI-IntraFreq to derive the MBMS frequencies of interest.

mbms-SAI-List Contains a list of MBMS SAIs for a specific frequency.

multiBandInfoList A list of additional frequency bands applicable for the cells participating in the SC-PTM transmission.

#### –	SystemInformationBlockType16-NB

The IE SystemInformationBlockType16-NB contains information related to GPS time and Coordinated Universal Time (UTC). The UE may use the parameters provided in this system information block to obtain the UTC, the GPS and the local time.

-- ASN1START

SystemInformationBlockType16-NB-r13 ::= SystemInformationBlockType16-r11

-- ASN1STOP

#### –	SystemInformationBlockType20-NB

For FDD, the IE SystemInformationBlockType20-NB contains the information required to acquire the control information associated with transmission of MBMS using SC-PTM.

SystemInformationBlockType20-NB information element

-- ASN1START

SystemInformationBlockType20-NB-r14 ::=	SEQUENCE {

npdcch-SC-MCCH-Config-r14				NPDCCH-SC-MCCH-Config-NB-r14,

sc-mcch-CarrierConfig-r14				CHOICE {

dl-CarrierConfig-r14					DL-CarrierConfigCommon-NB-r14,

dl-CarrierIndex-r14						INTEGER (0.. maxNonAnchorCarriers-NB-r14)

},

sc-mcch-RepetitionPeriod-r14			ENUMERATED {rf32, rf128, rf512, rf1024,

rf2048, rf4096, rf8192, rf16384},

sc-mcch-Offset-r14						INTEGER (0..10),

sc-mcch-ModificationPeriod-r14			ENUMERATED { rf32, rf128, rf256, rf512, rf1024,

rf2048, rf4096, rf8192, rf16384, rf32768,

rf65536, rf131072, rf262144, rf524288,

rf1048576, spare1},

sc-mcch-SchedulingInfo-r14				SC-MCCH-SchedulingInfo-NB-r14		OPTIONAL,	-- Need OP

lateNonCriticalExtension				OCTET STRING						OPTIONAL,

...

}

NPDCCH-SC-MCCH-Config-NB-r14 ::=	SEQUENCE {

npdcch-NumRepetitions-SC-MCCH-r14		ENUMERATED {r1, r2, r4, r8, r16,

r32, r64, r128, r256,

r512, r1024, r2048},

npdcch-StartSF-SC-MCCH-r14				ENUMERATED {v1dot5, v2, v4, v8,

v16, v32, v48, v64},

npdcch-Offset-SC-MCCH-r14				ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth}

}

SC-MCCH-SchedulingInfo-NB-r14::=	SEQUENCE	{

onDurationTimerSCPTM-r14					ENUMERATED {

pp1, pp2, pp3, pp4,

pp8, pp16, pp32, spare},

drx-InactivityTimerSCPTM-r14				ENUMERATED {

pp0, pp1, pp2, pp3,

pp4, pp8, pp16, pp32},

schedulingPeriodStartOffsetSCPTM-r14		CHOICE {

sf10										INTEGER(0..9),

sf20										INTEGER(0..19),

sf32										INTEGER(0..31),

sf40										INTEGER(0..39),

sf64										INTEGER(0..63),

sf80										INTEGER(0..79),

sf128										INTEGER(0..127),

sf160										INTEGER(0..159),

sf256										INTEGER(0..255),

sf320										INTEGER(0..319),

sf512										INTEGER(0..511),

sf640										INTEGER(0..639),

sf1024										INTEGER(0..1023),

sf2048										INTEGER(0..2047),

sf4096										INTEGER(0..4095),

sf8192										INTEGER(0..8191)

},

...

}

-- ASN1STOP

SystemInformationBlockType20-NB field descriptions

dl-CarrierConfig Downlink carrier used for SC-MCCH. E-UTRAN cannot configure a downlink carrier operating in mixed operation mode.

dl-CarrierIndex Index to a downlink carrier signalled in system information. Value '0' corresponds to the anchor carrier, value '1' corresponds to the first entry in dl-ConfigList in SystemInformationBlockType22-NB, value '2' corresponds to the second entry in dl-ConfigList and so on.

drx-InactivityTimerSCPTM Timer for SC-MCCH reception in TS 36.321 [6]. Value in number of NPDCCH periods. Value pp1 corresponds to 1 NPDCCH period, pp2 corresponds to 2 NPDCCH periods and so on.

npdcch-NumRepetitions-SC-MCCH The maximum number of NPDCCH repetitions the UE needs to monitor for SC-MCCH multicast search space, see TS 36.213 [23].

npdcch-Offset-SC-MCCH Fractional period offset of starting subframe for NPDCCH multicast search space for SC-MCCH, see TS 36.213 [23].

npdcch-StartSF-SC-MCCH Starting subframes configuration of the NPDCCH multicast search space for SC-MCCH, see TS 36.213 [23].

onDurationTimerSCPTM Timer for SC-MCCH reception in TS 36.321 [6]. Value in number of NPDCCH periods. Value pp1 corresponds to 1 NPDCCH period, pp2 corresponds to 2 NPDCCH periods and so on.

schedulingPeriodStartOffsetSCPTM SCPTM-SchedulingCycle and SCPTM-SchedulingOffset in TS 36.321 [6]. The value of SCPTM-SchedulingCycle is in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. The value of SCPTM-SchedulingOffset is in number of sub-frames.

sc-mcch-CarrierConfig Downlink carrier that is used for SC-MCCH.

sc-mcch-ModificationPeriod Defines periodically appearing boundaries, i.e. radio frames for which (H-SFN * 1024 +SFN) mod sc-mcch-ModificationPeriod = 0. The contents of different transmissions of SC-MCCH information can only be different if there is at least one such boundary in-between them. Value rf32 corresponds to 32 radio frames, value rf128 corresponds to 128 radio frames and so on.

sc-mcch-Offset Indicates, together with the sc-mcch-RepetitionPeriod, the boundary of the repetition period: (H-SFN * 1024 +SFN) mod sc-mcch-RepetitionPeriod = sc-mcch-Offset.

sc-mcch-RepetitionPeriod Defines the interval between transmissions of SC-MCCH information, in radio frames. Value rf32 corresponds to 32 radio frames, rf128 corresponds to 128 radio frames and so on.

sc-mcch-SchedulingInfo DRX information for the SC-MCCH. If the field is absent, DRX is not used for SC-MCCH reception.

#### –	SystemInformationBlockType22-NB

The IE SystemInformationBlockType22-NB contains radio resource configuration for paging and random access procedure on non-anchor carriers.

SystemInformationBlockType22-NB information element

-- ASN1START

SystemInformationBlockType22-NB-r14 ::=	SEQUENCE {

dl-ConfigList-r14					DL-ConfigCommonList-NB-r14	OPTIONAL,	-- Need OR

ul-ConfigList-r14					UL-ConfigCommonList-NB-r14	OPTIONAL,	-- Need OR

pagingWeightAnchor-r14				PagingWeight-NB-r14			OPTIONAL,	-- Cond pcch-config

nprach-ProbabilityAnchorList-r14	NPRACH-ProbabilityAnchorList-NB-r14	OPTIONAL,	-- Cond nprach-config

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	mixedOperationModeConfig-r15	SEQUENCE {

dl-ConfigListMixed-r15			DL-ConfigCommonList-NB-r14	OPTIONAL,	-- Cond dl-ConfigList

ul-ConfigListMixed-r15			UL-ConfigCommonList-NB-r14	OPTIONAL,	-- Cond ul-ConfigList

pagingDistribution-r15			ENUMERATED {true}			OPTIONAL,	-- Need OR

nprach-Distribution-r15			ENUMERATED {true}			OPTIONAL	-- Need OR

}																OPTIONAL,	-- Need OR

ul-ConfigList-r15				UL-ConfigCommonListTDD-NB-r15	OPTIONAL	-- Cond TDD

]]

}

DL-ConfigCommonList-NB-r14 ::=		SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF

DL-ConfigCommon-NB-r14

UL-ConfigCommonList-NB-r14 ::=		SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF

UL-ConfigCommon-NB-r14

UL-ConfigCommonListTDD-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF

UL-ConfigCommonTDD-NB-r15

DL-ConfigCommon-NB-r14 ::=			SEQUENCE {

dl-CarrierConfig-r14				DL-CarrierConfigCommon-NB-r14,

pcch-Config-r14					PCCH-Config-NB-r14			OPTIONAL, -- Need OR

...,

[[	wus-Config-r15					WUS-ConfigPerCarrier-NB-r15		OPTIONAL	-- Cond WUS

]],

[[	gwus-Config-r16					WUS-ConfigPerCarrier-NB-r15		OPTIONAL	-- Cond GWUS

]]

}

PCCH-Config-NB-r14 ::=				SEQUENCE {

npdcch-NumRepetitionPaging-r14		ENUMERATED {

r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1} OPTIONAL, -- Need OP

pagingWeight-r14						PagingWeight-NB-r14	DEFAULT w1,

...

}

PagingWeight-NB-r14	::=			ENUMERATED {w1, w2, w3, w4, w5, w6, w7, w8,

w9, w10, w11, w12, w13, w14, w15, w16}

UL-ConfigCommon-NB-r14 ::=			SEQUENCE {

ul-CarrierFreq-r14					CarrierFreq-NB-r13,

nprach-ParametersList-r14			NPRACH-ParametersList-NB-r14	OPTIONAL, -- Need OR

...,

[[	nprach-ParametersListEDT-r15	NPRACH-ParametersList-NB-r14	OPTIONAL -- Cond EDT

]]

}

UL-ConfigCommonTDD-NB-r15 ::=		SEQUENCE {

tdd-UL-DL-AlignmentOffset-r15		TDD-UL-DL-AlignmentOffset-NB-r15,

nprach-ParametersListTDD-r15		NPRACH-ParametersListTDD-NB-r15	OPTIONAL, -- Need OR

...

}

NPRACH-ProbabilityAnchorList-NB-r14 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-ProbabilityAnchor-NB-r14

NPRACH-ProbabilityAnchor-NB-r14 ::=		SEQUENCE {

nprach-ProbabilityAnchor-r14			ENUMERATED {

zero, oneSixteenth, oneFifteenth, oneFourteenth,

oneThirteenth, oneTwelfth, oneEleventh, oneTenth,

oneNinth, oneEighth, oneSeventh, oneSixth,

oneFifth, oneFourth, oneThird, oneHalf}

OPTIONAL	-- Need OP

}

-- ASN1STOP

SystemInformationBlockType22-NB field descriptions

dl-CarrierConfig For FDD: Provides the configuration of the DL non-anchor carrier. For TDD: Provides the configuration of the non-anchor carrier.

dl-ConfigList, dl-ConfigListMixed For FDD: List of DL non-anchor carriers and associated configuration that can be used for paging and/or random access. E-UTRAN configures DL non-anchor carriers operating in mixed operation mode only in dl-ConfigListMixed and only a UE that supports mixed operation mode uses the carriers in dl-ConfigListMixed. A given carrier is either signalled in the dl-ConfigList or in dl-ConfigListMixed. If dl-ConfigListMixed is present and at least one of the carriers in dl-ConfigListMixed is configured for paging: - If pagingDistribution is present, the UE supporting mixed operation mode creates a combined list of DL carriers for paging by appending dl-ConfigListMixed to the dl-ConfigList while maintaining the order among dl-ConfigList and dl-ConfigListMixed; the total number of signalled DL non-anchor carriers cannot be more than maxNonAnchorCarriers-NB-r14. - If pagingDistribution is absent, the UE supporting mixed operation mode uses the list of DL carriers for paging provided in dl-ConfigListMixed and considers pagingWeightAnchor being set to w0, i.e. the anchor carrier is not used. Otherwise, the pagingDistribution field is not applicable and the UE shall ignore the value. For TDD: List of non-anchor carriers and associated configuration that can be used for paging and/or random access.

gwus-Config For FDD: Carrier specific GWUS Configuration. If both gwus-Config and wus-Config are present for the carrier, E-UTRAN configures the same value for both fields.

mixedOperationModeConfig For FDD: Provides the configuration of DL and UL non-anchor carriers that can be used for paging and random access by a UE that supports mixed operation mode. For TDD: This parameter is absent.

npdcch-NumRepetitionPaging Maximum number of repetitions for NPDCCH common search space (CSS) for paging, see TS 36.213 [23], clause 16.6. If the field is absent, the value of npdcch-NumRepetitionPaging configured in SystemInformationBlockType2-NB in IE pcch-Config applies.

nprach-Distribution Indicates which UL carriers a UE supporting mixed operation mode uses for random access as defined in description of ul-ConfigList, ul-ConfigListMixed.

nprach-ParametersList, nprach-ParametersList-EDT Configure NPRACH parameters for each NPRACH resource on one non-anchor UL carrier. Up to three NPRACH resources can be configured on one non-anchor UL carrier. Each NPRACH resource is associated with a different number of NPRACH repetitions. NPRACH resources in nprach-ParametersListEDT are used to initiate EDT. Each NPRACH resource is associated with a maximum TBS signalled in the corresponding entry of edt-TBS-InfoList in SystemInformationBlockType2-NB. E-UTRAN includes the same number of entries, and listed in the same order, as in nprach-ParametersList in SystemInformationBlockType2-NB.

nprach-ParametersListTDD For TDD: Configure NPRACH parameters for each NPRACH resource on one non-anchor UL carrier. Up to three NPRACH resources can be configured on one non-anchor UL carrier. Each NPRACH resource is associated with a different number of NPRACH repetitions. E-UTRAN includes the same number of entries in nprach-ParametersListTDD, and listed in the same order, as in nprach-ParametersListTDD in SystemInformationBlockType2-NB..

nprach-ProbabilityAnchor Configure the selection probability for the anchor carrier NPRACH resource, see TS 36.321 [6]. Value zero corresponds to a probability of 0, oneSixteenth corresponds to the probability of 1/16, oneFifteenth corresponds to the probability of 1/15, and so on. If the field is absent, the selection probability of the anchor carrier NPRACH resource is 1. All non-anchor carriers NPRACH resources have equal probability between them. If there is no NPRACH resource defined on the anchor carrier for one repetition level in nprach-ParametersList-EDT, (respectively nprach-ParametersListFmt2, nprach-ParametersListFmt2-EDT), the UE shall use the value 'zero' and ignore the signalled value of nprach-ProbabilityAnchor for this repetition level for the NPRACH resources defined by nprach-ParametersList-EDT (respectively nprach-ParametersListFmt2, nprach-ParametersListFmt2-EDT).

nprach-ProbabilityAnchorList Configures the selection probability for each NPRACH resource on the anchor carrier. E-UTRAN includes the same number of entries, and listed in the same order, as in nprach-ParametersList in SystemInformationBlockType2-NB.

pagingDistribution Indicates which DL carriers a UE supporting mixed operation mode monitors for paging as defined in description of dl-ConfigList, dl-ConfigListMixed.

pagingWeight Weight of the non-anchor paging carrier for uneven paging load distribution across the carriers. Value w1 corresponds to a relative weight of 1, w2 corresponds to a relative weight of 2, and so on. The paging load for a carrier 'i' is equal to w(i)/W where i is equal to 0 for the anchor carrier and equal to the index of the carrier in the dl-ConfigList / dl-ConfigListMixed for a non-anchor carrier, W is the sum of the weights of all paging carriers. To avoid correlation between paging carrier and paging occasion, the weights should be assigned such that: nB * W <= 16384.

pagingWeightAnchor Weight of the anchor carrier for uneven paging load distribution across the carriers. Value w1 corresponds to a relative weight of 1, w2 corresponds to a relative weight of 2, and so on. If the field is absent, the (default) value of w0 is applied, i.e. the anchor carrier is not used for paging.

pcch-Config Configure the PCCH parameters for the non-anchor DL carrier.

tdd-UL-DL-AlignmentOffset Indicates the offset between the UL carrier frequency center with respect to DL carrier frequency center for the non-anchor carrier.

ul-CarrierFreq For FDD: UL carrier frequency of the non-anchor carrier as defined in TS 36.101 [42], clause 5.7.3F. For TDD: This field is absent and the uplink carrier frequency is same as the downlink frequency.

ul-ConfigList, ul-ConfigListMixed For FDD: List of UL non-anchor carriers and associated configuration that can be used for random access. E-UTRAN configures UL non-anchor carriers operating in mixed operation mode only in ul-ConfigListMixed and only a UE that supports mixed operation mode uses the carriers in ul-ConfigListMixed. A given carrier is either signalled in the ul-ConfigList or in ul-ConfigListMixed. If ul-ConfigListMixed is present and at least one of the carriers in ul-ConfigListMixed is configured for random access: - If nprach-Distribution is present, the UE supporting mixed operation mode creates a combined list of UL carriers for random access by appending ul-ConfigListMixed to the ul-ConfigList while maintaining the order among both ul-ConfigList and ul-ConfigListMixed; the total number of signalled UL non-anchor carriers cannot be more than maxNonAnchorCarriers-NB-r14. - If nprach-Distribution is absent, the UE supporting mixed operation mode uses the list of UL carriers for random access provided in ul-ConfigListMixed and considers nprach-ProbabiliyAnchor being set to zero for each NPRACH resource, i.e. the anchor carrier is not used for random access. Otherwise, the nprach-Distribution field is not applicable and the UE shall ignore the value. For TDD: E-UTRAN configures ul-ConfigList-r15 and includes the same number of entries as in dl-ConfigList. The UL carrier frequency of the non-anchor carrier is same as the DL carrier frequency.

wus-Config For FDD: Carrier specific WUS Configuration.

| Conditional presence | Explanation |
| --- | --- |
| dl-ConfigList | This field is optionally present, Need OR, if the field dl-ConfigList is present. Otherwise the field is not present. |
| EDT | The field is optionally present, Need OR, if edt-Parameters in SystemInformationBlockType2-NB is present; otherwise the field is not present and the UE shall delete any existing value for this field. |
| GWUS | This field is optionally present, Need OR, if gwus-Config-r16 is present in SystemInformationBlockType2-NB. Otherwise the field is not present. |
| pcch-config | This field is optionally present, Need OP, if the field dl-ConfigList is present and at least one of the carriers in dl-ConfigList is configured for paging. Otherwise the field is not present and only the anchor carrier is used for paging. |
| nprach-config | This field is mandatory present, if the field ul-ConfigList is present and at least one of the carriers in ul-ConfigList is configured for random access. Otherwise the field is not present and only the anchor carrier is used for random access. |
| TDD | This field is optionally present, Need OR, for TDD. Otherwise the field is not present. |
| ul-ConfigList | This field is optionally present, Need OR, if the field ul-ConfigList is present. Otherwise the field is not present. |
| WUS | This field is mandatory present, if the field wus-Config is present in SystemInformationBlockType2-NB. Otherwise the field is not present, Need OR. |

#### –	SystemInformationBlockType23-NB

For FDD, the IE SystemInformationBlockType23-NB contains radio resource configuration for NPRACH resources using preamble format 2 on non-anchor carriers.

SystemInformationBlockType23-NB information element

-- ASN1START

SystemInformationBlockType23-NB-r15 ::=	SEQUENCE {

ul-ConfigList-v1530					UL-ConfigCommonList-NB-v1530	OPTIONAL,	-- Need OR

ul-ConfigListMixed-v1530				UL-ConfigCommonList-NB-v1530	OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...

}

UL-ConfigCommonList-NB-v1530 ::=		SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF

UL-ConfigCommon-NB-v1530

UL-ConfigCommon-NB-v1530 ::=			SEQUENCE {

nprach-ParametersListFmt2-r15			NPRACH-ParametersListFmt2-NB-r15	OPTIONAL, -- Need OR

nprach-ParametersListFmt2EDT-r15		NPRACH-ParametersListFmt2-NB-r15	OPTIONAL, -- Cond EDT

...

}

-- ASN1STOP

SystemInformationBlockType23-NB field descriptions

nprach-ParametersListFmt2, nprach-ParametersListFmt2EDT Configures NPRACH parameters for each NPRACH resource format 2 on one UL carrier. Up to three NPRACH resources can be configured on one carrier. Each NPRACH resource is associated with a different number of NPRACH repetitions. E-UTRAN includes the same number of entries, and listed in the same order, as in nprach-ParametersList in SystemInformationBlockType2-NB. The NPRACH resources in nprach-ParametersListFmt2EDT are used to initiate EDT. Each NPRACH resource is associated with a TBS signalled in the corresponding entry of edt-TBS-InfoList. E-UTRAN configures the NPRACH resources format 2 so that they do not overlap in time domain with the NPRACH resources configured in nprach-ParametersList and nprach-ParametersListEDT on the same UL carrier. If there is no NPRACH resource in nprach-ParametersListFmt2 (respectively nprach-ParametersListFmt2EDT) on any UL carrier, including the anchor carrier, for one NPRACH repetition level, the UE uses the NPRACH resources in nprach-ParametersList (respectively nprach-ParametersListEDT) for this NPRACH repetition level. Otherwise, the UE uses only NPRACH resources in nprach-ParametersListFmt2 (respectively nprach-ParametersListFmt2EDT). If E-UTRAN configures NPRACH resources format 2 in one NPRACH repetition level, the E-UTRAN configures NPRACH resources format 2 in all NPRACH repetition levels upwards.

ul-ConfigList, ul-ConfigListMixed ul-ConfigList (respectively ul-ConfigListMixed) is parallel to ul-ConfigList (respectively ul-ConfigListMixed) in SystemInformationBlockType22-NB. E-UTRAN includes the same number of entries and in the same order in ul-ConfigList (respectively ul-ConfigListMixed) in SystemInformationBlockType23-NB as in ul-ConfigList (respectively ul-ConfigListMixed) in SystemInformationBlockType22-NB. The UE combines each entry in ul-ConfigList (respectively ul-ConfigListMixed) in SystemInformationBlockType23-NB with the corresponding entry in ul-ConfigList (respectively ul-ConfigListMixed) in SystemInformationBlockType22-NB.

| Conditional presence | Explanation |
| --- | --- |
| EDT | The field is optionally present, Need OR, if edt-Parameters in SystemInformationBlockType2-NB is present; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	SystemInformationBlockType27-NB

The IE SystemInformationBlockType27-NB contains information relevant only for inter-RAT cell selection i.e. assistance information about E-UTRA frequencies and/ or GERAN frequencies for cell selection.

SystemInformationBlockType27-NB information element

-- ASN1START

SystemInformationBlockType27-NB-r16 ::=	SEQUENCE {

carrierFreqListEUTRA-r16				CarrierFreqListEUTRA-NB-r16		OPTIONAL,	-- Need OR

carrierFreqsListGERAN-r16				CarrierFreqsListGERAN-NB-r16	OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...

}

CarrierFreqListEUTRA-NB-r16 ::=			SEQUENCE (SIZE (1..maxFreqEUTRA-NB-r16)) OF

CarrierFreqEUTRA-NB-r16

CarrierFreqsListGERAN-NB-r16 ::=		SEQUENCE (SIZE (1..maxFreqsGERAN-NB-r16)) OF

CarrierFreqsGERAN-NB-r16

CarrierFreqEUTRA-NB-r16 ::=				SEQUENCE {

carrierFreq-r16							ARFCN-ValueEUTRA-r9,

sib1-r16								ENUMERATED {supported}		OPTIONAL,	-- Need OR

sib1-BR-r16								ENUMERATED {supported}		OPTIONAL,	-- Need OR

...

}

CarrierFreqsGERAN-NB-r16 ::=			SEQUENCE {

carrierFreqs-r16						CarrierFreqsGERAN,

ec-GSM-IOT-r16							ENUMERATED {supported}		OPTIONAL,	-- Need OR

peo-r16									ENUMERATED {supported}		OPTIONAL,	-- Need OR

...

}

-- ASN1STOP

SystemInformationBlockType27-NB field descriptions

carrierFreq E-UTRAN carrier frequency.

carrierFreqListEUTRA Provides a list of neighbouring E-UTRA carrier frequencies, which may be searched for neighbouring E-UTRAN cells.

carrierFreqs The list of GERAN carrier frequencies organised into one group of GERAN carrier frequencies.

carrierFreqsListGERAN Provides a list of neighbouring GERAN carrier frequencies, which may be searched for neighbouring GERAN cells. The GERAN carrier frequencies are organised in groups and the parameters are indicated per group of GERAN carrier frequencies.

ec-GSM-IOT Indicates that the GERAN carrier frequencies support EC-GSM-IOT.

peo Indicates that the GERAN carrier frequencies support Power Efficient Operation (PEO).

sib1 Indicates that SIB1 is scheduled in the E-UTRAN cells.

sib1-BR Indicates that SIB1-BR is scheduled in the E-UTRAN cells.

#### –	SystemInformationBlockTypeXX-NB

The IE SystemInformationBlockTypeXX-NB contains satellite assistance information.

SystemInformationBlockTypeXX-NB information element

-- ASN1START

SystemInformationBlockTypeXX-NB-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...

}

-- ASN1STOP

#### 6.7.3.2	NB-IoT Radio resource control information elements

#### –	CarrierConfigDedicated-NB

The IE CarrierConfigDedicated-NB is used to specify a carrier in NB-IoT.

CarrierConfigDedicated-NB information elements

-- ASN1START

CarrierConfigDedicated-NB-r13 ::=		SEQUENCE {

dl-CarrierConfig-r13		DL-CarrierConfigDedicated-NB-r13,

ul-CarrierConfig-r13		UL-CarrierConfigDedicated-NB-r13

}

DL-CarrierConfigDedicated-NB-r13 ::=	SEQUENCE {

dl-CarrierFreq-r13						CarrierFreq-NB-r13,

downlinkBitmapNonAnchor-r13				CHOICE {

useNoBitmap-r13							NULL,

useAnchorBitmap-r13						NULL,

explicitBitmapConfiguration-r13			DL-Bitmap-NB-r13,

spare									NULL

}		OPTIONAL,	-- Need ON

dl-GapNonAnchor-r13						CHOICE {

useNoGap-r13							NULL,

useAnchorGapConfig-r13					NULL,

explicitGapConfiguration-r13			DL-GapConfig-NB-r13,

spare									NULL

}		OPTIONAL,	-- Need ON

inbandCarrierInfo-r13					SEQUENCE {

samePCI-Indicator-r13					CHOICE	{

samePCI-r13								SEQUENCE {

indexToMidPRB-r13						INTEGER (-55..54)

},

differentPCI-r13						SEQUENCE {

eutra-NumCRS-Ports-r13					ENUMERATED {same, four}

}

}							OPTIONAL,		-- Cond anchor-guardband-or-standalone

eutraControlRegionSize-r13				ENUMERATED {n1, n2, n3}

}								OPTIONAL,		-- Cond non-anchor-inband

...,

[[	nrs-PowerOffsetNonAnchor-v1330		ENUMERATED {dB-12, dB-10, dB-8, dB-6,

dB-4, dB-2, dB0, dB3}

OPTIONAL	-- Need ON

]],

[[	dl-GapNonAnchor-v1530				DL-GapConfig-NB-v1530	OPTIONAL	-- Cond TDD1

]],

[[	dl-CarrierFreq-v1550				CarrierFreq-NB-v1550	OPTIONAL	-- Cond TDD1

]]

}

UL-CarrierConfigDedicated-NB-r13 ::=	SEQUENCE {

ul-CarrierFreq-r13			CarrierFreq-NB-r13		OPTIONAL,	-- Need OP

...,

[[	tdd-UL-DL-AlignmentOffset-r15		TDD-UL-DL-AlignmentOffset-NB-r15		OPTIONAL		-- Cond TDD

]]

}

-- ASN1STOP

CarrierConfigDedicated-NB field descriptions

dl-CarrierConfig Downlink carrier used for all unicast transmissions.

dl-CarrierFreq DL carrier frequency. The downlink carrier is not in a E-UTRA PRB which contains E-UTRA PSS/SSS/PBCH.

dl-GapNonAnchor Downlink transmission gap configuration for the anchor/ non-anchor carrier, see TS 36.211 [21], clause 10.2.3.4. E-UTRAN may configure dl-GapNonAnchor-v1530 only if dl-GapNonAnchor-r13 is set to explicitGapConfiguration.

downlinkBitmapNonAnchor For FDD: NB-IoT downlink subframe configuration for downlink transmission on the anchor/ non-anchor carrier. See TS 36.213 [23], clause 16.4. For TDD: NB-IoT downlink, uplink and special subframes configuration for transmission on the anchor/ non-anchor carrier. See TS 36.213 [23], clause 16.4.

eutraControlRegionSize Indicates the control region size of the E-UTRA cell for the in-band operation mode, see TS 36.213 [23]. Unit is in number of OFDM symbols. If operationModeInfo in MIB-NB is set to inband-SamePCI or inband-DifferentPCI, it should be set to the value broadcast in SIB1-NB.

eutra-NumCRS-Ports Number of E-UTRA CRS antenna ports, either the same number of ports as NRS or 4 antenna ports. See TS 36.211 [21], TS 36.212 [22], and TS 36.213 [23].

inbandCarrierInfo Provides the configuration of the anchor/ non-anchor inband carrier. If operationModeInfo is set to standalone in the MIB-NB, E-UTRAN only configures this field if the UE supports mixed operation mode.

indexToMidPRB The PRB index is signaled by offset from the middle of the EUTRA system.

nrs-PowerOffsetNonAnchor Provides the power offset of the downlink narrowband reference-signal EPRE of the anchor/ non-anchor carrier relative to the anchor carrier, unit in dB. Value dB-12 corresponds to -12 dB, dB-10 corresponds to -10 dB and so on. See TS 36.213 [23], clause16.2.2.

samePCI-Indicator This parameter specifies whether the anchor/ non-anchor carrier reuses the same PCI as the EUTRA carrier.

ul-CarrierConfig Uplink anchor/ non-anchor carrier used for all unicast transmissions.

ul-CarrierFreq For FDD: UL carrier frequency as defined in TS 36.101 [42], clause 5.7.3F. If absent, the same TX-RX frequency separation and carrier frequency offset as for the anchor carrier applies. For TDD: This field is absent and the uplink carrier frequency is equal to the downlink frequency.

| Conditional presence | Explanation |
| --- | --- |
| non-anchor-inband | The field is mandatory present if the anchor/ non-anchor carrier is an inband carrier; otherwise it is not present. |
| anchor-guardband-or-standalone | The field is mandatory present if operationModeInfo is set to guardband or standalone in the MIB; otherwise it is not present. |
| TDD | The field is mandatory present for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD1 | The field is optionally present, Need OR, for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	CarrierFreq-NB

The IE CarrierFreq-NB is used to provide the NB-IoT carrier frequency, as defined in TS 36.101 [42].

CarrierFreq-NB information elements

-- ASN1START

CarrierFreq-NB-r13 ::=		SEQUENCE {

carrierFreq-r13				ARFCN-ValueEUTRA-r9,

carrierFreqOffset-r13		ENUMERATED {

v-10, v-9, v-8,	v-7, v-6, v-5, v-4, v-3, v-2, v-1, v-0dot5,

v0, v1, v2, v3, v4, v5, v6, v7, v8, v9

}	OPTIONAL	-- Need ON

}

CarrierFreq-NB-v1550	::=		SEQUENCE {

carrierFreqOffset-v1550		ENUMERATED {v-8dot5, v-4dot5, v3dot5, v7dot5}

}

-- ASN1STOP

CarrierFreq-NB field descriptions

carrierFreq Provides the ARFCN applicable for the NB-IoT carrier frequency as defined in TS 36.101 [42], Table 5.7.3-1.

carrierFreqOffset Offset of the NB-IoT channel number to EARFCN as defined in TS 36.101 [42], clause 5.7.3F. Value v-10 means -10, v-9 means -9, and so on. E-UTRAN may configure the values v-8dot5, v-4dot5, v3dot5 and v7dot5 only for a carrier in a TDD band. For TDD, the UE shall use the value signalled in carrierFreqOffset-v1550, if present, and ignore the value signaled in carrierFreqOffset-r13.

#### –	ChannelRasterOffset-NB

The IE ChannelRasterOffset-NB is used to specify the NB-IoT offset from LTE channel raster. Unit in kHz in set { -7.5, -2.5, 2.5, 7.5} See TS 36.211[21] and TS 36.213 [23].

ChannelRasterOffset-NB information element

-- ASN1START

ChannelRasterOffset-NB-r13 ::= ENUMERATED {khz-7dot5, khz-2dot5, khz2dot5, khz7dot5}

-- ASN1STOP

#### –	DL-Bitmap-NB

The IE DL-Bitmap-NB is used to specify the set of NB-IoT downlink subframes for downlink transmission.

DL-Bitmap-NB information element

-- ASN1START

DL-Bitmap-NB-r13 ::=			CHOICE {

subframePattern10-r13			BIT STRING (SIZE (10)),

subframePattern40-r13			BIT STRING (SIZE (40))

}

-- ASN1STOP

DL-Bitmap-NB field descriptions

subframePattern10, subframePattern40 For FDD: NB-IoT downlink subframe configuration over 10ms or 40ms for inband and 10ms for standalone/guardband. For TDD: NB-IoT downlink, uplink and special subframes configuration over 10ms or 40ms for inband and 10ms for standalone/guardband. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission.

#### –	DL-CarrierConfigCommon-NB

The IE DL-CarrierConfigCommon-NB is used to specify the common configuration of a DL non-anchor carrier in NB-IoT.

DL-CarrierConfigCommon-NB information elements

-- ASN1START

DL-CarrierConfigCommon-NB-r14 ::=	SEQUENCE {

dl-CarrierFreq-r14					CarrierFreq-NB-r13,

downlinkBitmapNonAnchor-r14			CHOICE {

useNoBitmap-r14						NULL,

useAnchorBitmap-r14					NULL,

explicitBitmapConfiguration-r14		DL-Bitmap-NB-r13

},

dl-GapNonAnchor-r14					CHOICE {

useNoGap-r14						NULL,

useAnchorGapConfig-r14				NULL,

explicitGapConfiguration-r14		DL-GapConfig-NB-r13

},

inbandCarrierInfo-r14				SEQUENCE {

samePCI-Indicator-r14				CHOICE	{

samePCI-r14							SEQUENCE {

indexToMidPRB-r14					INTEGER (-55..54)

},

differentPCI-r14					SEQUENCE {

eutra-NumCRS-Ports-r14				ENUMERATED {same, four}

}

}	OPTIONAL,		-- Cond anchor-guardband-or-standalone

eutraControlRegionSize-r14			ENUMERATED {n1, n2, n3}

}	OPTIONAL,		-- Cond non-anchor-inband

nrs-PowerOffsetNonAnchor-r14		ENUMERATED {dB-12, dB-10, dB-8, dB-6,

dB-4, dB-2, dB0, dB3}	DEFAULT dB0,

...,

[[	dl-GapNonAnchor-v1530			DL-GapConfig-NB-v1530	OPTIONAL	-- Cond TDD

]],

[[	dl-CarrierFreq-v1550			CarrierFreq-NB-v1550	OPTIONAL	-- Cond TDD

]]

}

-- ASN1STOP

DL-CarrierConfigCommon-NB field descriptions

dl-CarrierFreq DL carrier frequency. The downlink carrier is not in a E-UTRA PRB which contains E-UTRA PSS/SSS/PBCH.

dl-GapNonAnchor Downlink transmission gap configuration for the non-anchor carrier, see TS 36.211 [21], clause 10.2.3.4. E-UTRAN may configure dl-GapNonAnchor-v1530 only if dl-GapNonAnchor-r14 is set to explicitGapConfiguration.

downlinkBitmapNonAnchor For FDD: NB-IoT downlink subframe configuration for downlink transmission on the non-anchor carrier. See TS 36.213 [23], clause 16.4. For TDD: NB-IoT downlink, uplink and special subframes configuration for transmission on the anchor/ non-anchor carrier. See TS 36.213 [23], clause 16.4.

eutraControlRegionSize Indicates the control region size of the E-UTRA cell for the in-band operation mode, see TS 36.213 [23]. Unit is in number of OFDM symbols. If operationModeInfo in MIB-NB is set to inband-SamePCI or inband-DifferentPCI, it should be set to the value broadcast in SIB1-NB.

eutra-NumCRS-Ports Number of E-UTRA CRS antenna ports, either the same number of ports as NRS or 4 antenna ports. See TS 36.211 [21], TS 36.212 [22], and TS 36.213 [23].

inbandCarrierInfo Provides the configuration of a non-anchor inband carrier.

indexToMidPRB The PRB index is signaled by offset from the middle of the EUTRA system.

nrs-PowerOffsetNonAnchor Provides the downlink narrowband reference-signal EPRE offset of the non-anchor carrier relative to the downlink narrowband reference-signal EPRE of the anchor carrier, unit in dB. Value dB-12 corresponds to -12 dB, dB-10 corresponds to -10 dB and so on. See TS 36.213 [23], clause 16.2.2.

samePCI-Indicator This parameter specifies whether the non-anchor carrier reuses the same PCI as the EUTRA carrier.

| Conditional presence | Explanation |
| --- | --- |
| non-anchor-inband | The field is mandatory present if the non-anchor carrier is an inband carrier; otherwise it is not present. |
| anchor-guardband-or-standalone | The field is mandatory present, if operationModeInfo is set to guardband or standalone in the MIB; otherwise it is not present. |
| TDD | The field is optionally present, Need OR, for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	DL-GapConfig-NB

The IE DL-GapConfig-NB is used to specify the downlink gap configuration for NPDCCH and NPDSCH. Downlink gaps apply to all NPDCCH/NPDSCH transmissions except for BCCH.

DL-GapConfig-NB information element

-- ASN1START

DL-GapConfig-NB-r13	::=		SEQUENCE {

dl-GapThreshold-r13			ENUMERATED {n32, n64, n128, n256},

dl-GapPeriodicity-r13		ENUMERATED {sf64, sf128, sf256, sf512},

dl-GapDurationCoeff-r13		ENUMERATED {oneEighth, oneFourth, threeEighth, oneHalf}

}

DL-GapConfig-NB-v1530	::=	SEQUENCE {

dl-GapPeriodicity-v1530		ENUMERATED {sf1024}

}

-- ASN1STOP

DL-GapConfig-NB field descriptions

dl-GapDurationCoeff Coefficient to calculate the gap duration of a DL transmission: dl-GapDurationCoeff * dl-GapPeriodicity, Duration in number of subframes. See TS 36.211 [21], clause 10.2.3.4.

dl-GapPeriodicity Periodicity of a DL transmission gap in number of subframes. See TS 36.211 [21], clause 10.2.3.4. Value sf64 corresponds to 64 subframes, value sf128 corresponds to 128 subframes, value sf256 corresponds to 256 subframes and so on. E-UTRAN may configure the value sf64 only in FDD mode and the value sf1024 only in TDD mode. The UE shall use the value signalled in dl-GapPeriodicity-v1530, if present, and ignore the value signaled in dl-GapPeriodicity-r13.

dl-GapThreshold Threshold on the maximum number of repetitions configured for NPDCCH before application of DL transmission gap configuration. See TS 36.211 [21], clause 10.2.3.4.

#### –	GWUS-Config-NB

The IE GWUS-Config-NB is used to specify the GWUS configuration. For UEs supporting GWUS, E-UTRAN uses GWUS to indicate that the UE shall attempt to receive paging in that cell, see TS 36.304 [4].

GWUS-Config-NB information element

-- ASN1START

GWUS-Config-NB-r16 ::= 			SEQUENCE {

groupAlternation-r16			ENUMERATED {true}			OPTIONAL, -- Need OR

commonSequence-r16				ENUMERATED {g0, g126}		OPTIONAL, -- Need OR

timeParameters-r16				WUS-Config-NB-r15			OPTIONAL, -- Cond noWUSr15

resourceConfigDRX-r16			GWUS-ResourceConfig-NB-r16,

resourceConfig-eDRX-Short-r16	GWUS-ResourceConfig-NB-r16	OPTIONAL, -- Need OP

resourceConfig-eDRX-Long-r16	GWUS-ResourceConfig-NB-r16	OPTIONAL, -- Cond timeOffset

probThreshList-r16				GWUS-ProbThreshList-NB-r16	OPTIONAL, -- Cond probabilityBased

...

}

GWUS-ResourceConfig-NB-r16 ::= 	SEQUENCE {

resourcePosition-r16			ENUMERATED {primary, secondary},

numGroupsList-r16				GWUS-NumGroupsList-NB-r16			OPTIONAL, 	-- Need OP

groupsForServiceList-r16		GWUS-GroupsForServiceList-NB-r16

OPTIONAL 	-- Cond probabilityBased

}

GWUS-ProbThreshList-NB-r16 ::= 			SEQUENCE (SIZE (1..maxGWUS-ProbThresholds-NB-r16)) OF

GWUS-Paging-ProbThresh-NB-r16

GWUS-Paging-ProbThresh-NB-r16 ::= 		ENUMERATED {p20, p30, p40, p50, p60, p70, p80, p90}

GWUS-NumGroupsList-NB-r16 ::= 			SEQUENCE (SIZE (1..maxGWUS-Resources-NB-r16)) OF

GWUS-NumGroups-NB-r16

GWUS-NumGroups-NB-r16 ::= 				ENUMERATED {n1, n2, n4, n8}

GWUS-GroupsForServiceList-NB-r16 ::=	SEQUENCE (SIZE (1..maxGWUS-ProbThresholds-NB-r16)) OF

INTEGER (1..maxGWUS-Groups-1-NB-r16)

-- ASN1STOP

GWUS-Config-NB field descriptions

commonSequence Presence of the field indicates common WUS sequence is configured. Value g0 indicates common WUS sequence for the shared WUS resource is g=0, value g126 indicates common WUS sequence for the shared WUS resource is g=126, see TS 36.211 [21].

groupAlternation Presence of the field enables WUS group alternation between the two WUS resources for the gap type, see TS 36.304 [4].

groupsForServiceList Number of WUS groups for each paging probability group, see TS 36.304 [4]. The first entry corresponds to the first paging probability group, second entry corresponds to the second paging probability group, and so on. E-UTRAN includes the same number of entries and in the same order in groupsForServiceList and probThreshList. Total number of WUS groups in this list cannot be more than total number of WUS groups in numGroupsList.

numGroupsList List of WUS groups for each WUS resource, see TS 36.304 [4]. First entry corresponds to the first resource, the second entry corresponds to the second resource. numGroupsList shall be present in resourceConfigDRX. If numGroupsList is not present in resourceconfig-eDRX-Short, parameters for DRX WUS resource applies for short eDRX WUS resource. If numGroupsList is not present in resourceConfig-eDRX-Long, parameters for short eDRX WUS resource applies for long eDRX WUS resource.

probThreshList Paging probability thresholds corresponding to the paging probability groups, see TS 36.304 [4]. Value p20 corresponds to 20%, value p30 corresponds to 30%, and so on.

resourceConfigDRX, resourceConfig-eDRX-Short, resourceConfig-eDRX-Long WUS resource configured for each gap type, see TS 36.304 [4]. If resourceConfig-eDRX-Short is not present, DRX WUS parameters apply for short eDRX WUS resource. If resourceConfig-eDRX-Long is not present, short eDRX WUS parameters apply for long eDRX WUS resource.

resourcePosition Indicates the position of the WUS resource corresponding to the first entry in numGroupsList. Value primary indicates that the end of the WUS resource is defined by the timeoffset value for the corresponding gap type, value secondary indicates that the end of the WUS resource is immediately before the WUS resource configured by wus-Config. E-UTRAN may only configure secondary when only one entry exists in numGroupsList and wus-Config is present in SystemInformationBlockType2-NB. If two entries exist in numGroupsList, the position for the second WUS resource corresponds to value secondary.

timeParameters Time domain WUS configuration information. For individual field descriptions, see WUS-Config-NB. If the field is absent, the parameters in wus-Config apply.

| Conditional presence | Explanation |
| --- | --- |
| noWUSr15 | The field is mandatory present if wus-Config-r15 is not present in SystemInformationBlockType2-NB; otherwise the field is not present. |
| probabilityBased | The field is mandatory present if paging probability based WUS group selection is configured; otherwise the field is not present, and the UE shall delete any existing value for this field. |
| timeOffset | The field is optionally present, Need OP, if timeOffset-eDRX-Long is present in timeParameters; otherwise the field is not present, and the UE shall delete any existing value for this field. |

#### –	LogicalChannelConfig-NB

The IE LogicalChannelConfig-NB is used to configure the logical channel parameters.

LogicalChannelConfig-NB information element

-- ASN1START

LogicalChannelConfig-NB-r13 ::=		SEQUENCE {

priority-r13						INTEGER (1..16)			OPTIONAL,		-- Cond UL

logicalChannelSR-Prohibit-r13		BOOLEAN					OPTIONAL,		-- Need ON

...

}

-- ASN1STOP

LogicalChannelConfig-NB field descriptions

logicalChannelSR-Prohibit Value TRUE indicates that the logicalChannelSR-ProhibitTimer is enabled for the logical channel. If logicalChannelSR-Prohibit is configured (i.e. indicates value TRUE), E-UTRAN also configures logicalChannelSR-ProhibitTimer. See TS 36.321 [6].

priority Logical channel priority in TS 36.321 [6]. Value is an integer.

| Conditional presence | Explanation |
| --- | --- |
| UL | The field is mandatory present for UL logical channels; otherwise it is not present. |

#### –	MAC-MainConfig-NB

The IE MAC-MainConfig-NB is used to specify the MAC main configuration for signalling and data radio bearers.

MAC-MainConfig-NB information element

-- ASN1START

MAC-MainConfig-NB-r13 ::=			SEQUENCE {

ul-SCH-Config-r13					SEQUENCE {

periodicBSR-Timer-r13				PeriodicBSR-Timer-NB-r13		OPTIONAL,	-- Need ON

retxBSR-Timer-r13					RetxBSR-Timer-NB-r13

}																	OPTIONAL,	-- Need ON

drx-Config-r13						DRX-Config-NB-r13				OPTIONAL,	-- Need ON

timeAlignmentTimerDedicated-r13		TimeAlignmentTimer,

logicalChannelSR-Config-r13			CHOICE {

release								NULL,

setup								SEQUENCE {

logicalChannelSR-ProhibitTimer-r13	ENUMERATED {

pp2, pp8, pp32, pp128, pp512,

pp1024, pp2048, spare}

}

}																	OPTIONAL,	-- Need ON

...,

[[	rai-Activation-r14						ENUMERATED {true}			OPTIONAL,	-- Need OR

dataInactivityTimerConfig-r14	CHOICE {

release								NULL,

setup								SEQUENCE {

dataInactivityTimer-r14				DataInactivityTimer-r14

}

}																OPTIONAL	-- Need ON

]],

[[	drx-Cycle-v1430					ENUMERATED {

sf1280, sf2560, sf5120, sf10240}	OPTIONAL	-- Need ON

]],

[[	ra-CFRA-Config-r14				ENUMERATED {true}				OPTIONAL	-- Need ON

]],

[[ ta-ReportConfig-r17				SetupRelease {TA-ReportConfig-NB-r17}OPTIONAL	-- Need ON

]]

}

PeriodicBSR-Timer-NB-r13 ::=		ENUMERATED {

pp2, pp4, pp8, pp16, pp64, pp128, infinity, spare}

RetxBSR-Timer-NB-r13 ::=			ENUMERATED {

pp4, pp16, pp64, pp128, pp256, pp512, infinity, spare}

DRX-Config-NB-r13 ::=				CHOICE {

release								NULL,

setup								SEQUENCE {

onDurationTimer-r13					ENUMERATED {

pp1, pp2, pp3, pp4, pp8, pp16, pp32, spare},

drx-InactivityTimer-r13				ENUMERATED {

pp0, pp1, pp2, pp3, pp4, pp8, pp16, pp32},

drx-RetransmissionTimer-r13			ENUMERATED {

pp0, pp1, pp2, pp4, pp6, pp8, pp16, pp24,

pp33, spare7, spare6, spare5,

spare4, spare3, spare2, spare1},

drx-Cycle-r13						ENUMERATED {

sf256, sf512, sf1024, sf1536, sf2048, sf3072,

sf4096, sf4608, sf6144, sf7680, sf8192, sf9216,

spare4, spare3, spare2, spare1},

drx-StartOffset-r13					INTEGER (0..255),

drx-ULRetransmissionTimer-r13		ENUMERATED {

pp0, pp1, pp2, pp4, pp6, pp8, pp16, pp24,

pp33, pp40, pp64, pp80, pp96,

pp112, pp128, pp160, pp320}

}

}

TA-ReportConfig-NB-r17 ::=		TypeFFS

-- ASN1STOP

Editor’s Note: Agreement: Support TA reporting in RRC connected mode in IoT NTN. Agreement: Support event-triggered for TA reporting in connected mode. Wait for NR NTN agreements for other triggers

MAC-MainConfig-NB field descriptions

drx-Config Used to configure DRX as specified in TS 36.321 [6].

drx-Cycle longDRX-Cycle in TS 36.321 [6]. The value of longDRX-Cycle is in number of sub-frames. Value sf256 corresponds to 256 sub-frames, sf512 corresponds to 512 sub-frames and so on. In case drx-Cycle-v1430 is signalled, the UE shall ignore drx-Cycle-r13.

drx-StartOffset drxStartOffset in TS 36.321 [6]. Value is in number of sub-frames by step of (drx-cycle / 256).

drx-InactivityTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH periods. Value pp0 corresponds to 0 PDCCH period and behaviour as specified in 7.3.2 applies, pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on.

drx-RetransmissionTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH periods. Value pp0 corresponds to 0 PDCCH period and behaviour as specified in 7.3.2 applies, pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on.

drx-ULRetransmissionTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH periods. Value pp0 corresponds to 0 PDCCH period and behaviour as specified in 7.3.2 applies, value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on.

logicalChannelSR-ProhibitTimer Timer used to delay the transmission of an SR. See TS 36.321 [6]. Value in number of PDCCH periods. Value pp2 corresponds to 2 PDCCH periods, pp8 corresponds to 8 PDCCH periods and so on.

periodicBSR-Timer Timer for BSR reporting in TS 36.321 [6]. Value in number of PDCCH periods. Value pp2 corresponds to 2 PDCCH periods, pp4 corresponds to 4 PDCCH periods and so on.

ra-CFRA-Config Activation of contention free random access (CFRA), see TS 36.321 [6].

rai-Activation Activation of release assistance indication (RAI) in TS 36.321 [6].

retxBSR-Timer Timer for BSR reporting in TS 36.321 [6]. Value in number of PDCCH periods. Value pp4 corresponds to 4 PDCCH periods, pp16 corresponds to 16 PDCCH periods and so on.

onDurationTimer Timer for DRX in TS 36.321 [6]. Value in number of PDCCH periods. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on.

ta-ReportConfig Configuration of UE specific TA reporting, see TS 36.321 [6].

timeAlignmentTimer Indicates the value of the time alignment timer, see TS 36.321 [6].

#### –	NPDCCH-ConfigDedicated-NB

The IE NPDCCH-ConfigDedicated-NB specifies the subframes and resource blocks for NPDCCH monitoring.

NPDCCH-ConfigDedicated-NB information element

-- ASN1START

NPDCCH-ConfigDedicated-NB-r13 ::=	SEQUENCE {

npdcch-NumRepetitions-r13			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

npdcch-StartSF-USS-r13				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64},

npdcch-Offset-USS-r13				ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

}

NPDCCH-ConfigDedicated-NB-v1530 ::=	SEQUENCE {

npdcch-StartSF-USS-v1530			ENUMERATED {v96, v128}

}

-- ASN1STOP

NPDCCH-ConfigDedicated-NB field descriptions

npdcch-NumRepetitions Maximum number of repetitions for NPDCCH UE specific search space (USS), see TS 36.213 [23], clause 16.6. UE monitors one set of values (consisting of aggregation level, number of repetitions and number of blind decodes) according to the configured maximum number of repetitions.

npdcch-Offset-USS Fractional period offset of starting subframe for NPDCCH UE specific search space (USS), see TS 36.213 [23], clause 16.6.

npdcch-StartSF-USS Starting subframe configuration for an NPDCCH UE-specific search space, see TS 36.213 [23], clause 16.6. Value v1dot5 corresponds to 1.5, value 2 corresponds to 2 and so on. E-UTRAN may configure values v1dot5 and v2 only in FDD mode and values v96 and v128 only in TDD mode. The UE shall use the value signalled in npdcch-StartSF-USS-v1530, if present, and ignore the value signalled in npdcch-StartSF-USS-r13.

#### –	NPDSCH-Config-NB

The IE NPDSCH-ConfigCommon-NB is used to specify the common NPDSCH configuration. The IE NPDSCH-ConfigDedicated-NB is used to specify the UE specific NPDSCH configuration.

NPDSCH-Config-NB information element

-- ASN1START

NPDSCH-ConfigCommon-NB-r13 ::=	SEQUENCE {

nrs-Power-r13					INTEGER (-60..50)

}

NPDSCH-ConfigDedicated-NB-r16 ::=	SEQUENCE {

npdsch-MultiTB-Config-r16			NPDSCH-MultiTB-Config-NB-r16	 OPTIONAL	-- Cond twoHARQ

}

NPDSCH-MultiTB-Config-NB-r16 ::=	SEQUENCE {

multiTB-Config-r16					ENUMERATED {interleaved, nonInterleaved},

harq-AckBundling-r16				ENUMERATED {true}		OPTIONAL	-- Cond interleaved

}

-- ASN1STOP

NPDSCH-Config-NB field descriptions

multiTB-Config For FDD: Activation of multiple TBs scheduling in DL, see TS 36.213 [23]. Value interleaved indicates that multiple TBs scheduling with interleaved transmission is enabled, value nonInterleaved indicates that multiple TBs scheduling without interleaved transmission is enabled.

harq-AckBundling For FDD: Activation of HARQ ACK bundling for DL multiple TBs scheduling with interleaved transmission, see TS 36.213 [23].

nrs-Power Provides the downlink narrowband reference-signal EPRE, see TS 36.213 [23], clause 16.2. The actual value in dBm.

| Conditional presence | Explanation |
| --- | --- |
| interleaved | The field is optionally present, Need OR, if multiTB-Config is set to interleaved; otherwise the field is not present and the UE shall delete any existing value for this field. |
| twoHARQ | The field is optionally present, Need OR, if twoHARQ-ProcessesConfig is configured; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	NPRACH-ConfigSIB-NB

The IE NPRACH-ConfigSIB-NB is used to specify the NPRACH configuration for the anchor and non-anchor carriers.

NPRACH-ConfigSIB-NB information elements

-- ASN1START

NPRACH-ConfigSIB-NB-r13 ::=			SEQUENCE {

nprach-CP-Length-r13				ENUMERATED {us66dot7, us266dot7},

rsrp-ThresholdsPrachInfoList-r13	RSRP-ThresholdsNPRACH-InfoList-NB-r13	OPTIONAL,	-- Need OR

nprach-ParametersList-r13		NPRACH-ParametersList-NB-r13

}

NPRACH-ConfigSIB-NB-v1330 ::=		SEQUENCE {

nprach-ParametersList-v1330			NPRACH-ParametersList-NB-v1330

}

NPRACH-ConfigSIB-NB-v1450 ::=		SEQUENCE {

maxNumPreambleAttemptCE-r14			ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1}

}

NPRACH-ConfigSIB-NB-v1530 ::=		SEQUENCE {

tdd-Parameters-r15					SEQUENCE {

nprach-PreambleFormat-r15			ENUMERATED {

fmt0, fmt1, fmt2, fmt0-a, fmt1-a},

dummy								ENUMERATED {

n1, n2, n4, n8, n16, n32, n64, n128,

n256, n512, n1024},

nprach-ParametersListTDD-r15		NPRACH-ParametersListTDD-NB-r15

}	OPTIONAL,		-- Cond TDD

fmt2-Parameters-r15					SEQUENCE {

nprach-ParametersListFmt2-r15		NPRACH-ParametersListFmt2-NB-r15 OPTIONAL,	-- Need OR

nprach-ParametersListFmt2EDT-r15	NPRACH-ParametersListFmt2-NB-r15 OPTIONAL	-- Cond EDT2

}	OPTIONAL,		-- Need OR

edt-Parameters-r15					SEQUENCE {

edt-SmallTBS-Subset-r15				ENUMERATED {true}				OPTIONAL,	-- Need OR

edt-TBS-InfoList-r15				EDT-TBS-InfoList-NB-r15,

nprach-ParametersListEDT-r15		NPRACH-ParametersList-NB-r14	OPTIONAL	-- Need OR

}	OPTIONAL		-- Cond EDT1

}

NPRACH-ConfigSIB-NB-v1550 ::=		SEQUENCE {

tdd-Parameters-v1550				SEQUENCE {

nprach-ParametersListTDD-v1550		NPRACH-ParametersListTDD-NB-v1550

}

}

NPRACH-ConfigSIB-NB-v17xx ::=		SEQUENCE {

nprach-TxDurationFmt01-r17		ENUMERATED {v2dot4, v4dot4, v8dot4, v16dot4, v32dot4, v64dot4}

OPTIONAL,	-- Cond NTN

nprach-TxDuration-Fmt2-r17		ENUMERATED {v1dot6, v2dot6, v4dot6, v8dot6, v16dot6}

OPTIONAL	-- Cond NTN

}

NPRACH-ParametersList-NB-r13 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r13

NPRACH-ParametersList-NB-v1330 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-v1330

NPRACH-Parameters-NB-r13::=			SEQUENCE {

nprach-Periodicity-r13					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560},

nprach-StartTime-r13					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024},

nprach-SubcarrierOffset-r13				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1},

nprach-NumSubcarriers-r13				ENUMERATED {n12, n24, n36, n48},

nprach-SubcarrierMSG3-RangeStart-r13	ENUMERATED {zero, oneThird, twoThird, one},

maxNumPreambleAttemptCE-r13				ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1},

numRepetitionsPerPreambleAttempt-r13	ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128},

npdcch-NumRepetitions-RA-r13			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

npdcch-StartSF-CSS-RA-r13				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64},

npdcch-Offset-RA-r13					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

}

NPRACH-Parameters-NB-v1330 ::=		SEQUENCE {

nprach-NumCBRA-StartSubcarriers-r13		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

}

NPRACH-ParametersList-NB-r14 ::=		SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-Parameters-NB-r14

NPRACH-Parameters-NB-r14 ::=			SEQUENCE {

nprach-Parameters-r14					SEQUENCE {

nprach-Periodicity-r14					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560}

OPTIONAL,	-- NEED OP

nprach-StartTime-r14					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r14				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r14				ENUMERATED {n12, n24, n36, n48}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r14	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r14			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r14				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r14					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r14		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

OPTIONAL,	-- NEED OP

npdcch-CarrierIndex-r14					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

...

}	OPTIONAL	-- Need OR

}

NPRACH-ParametersListTDD-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-ParametersTDD-NB-r15

NPRACH-ParametersTDD-NB-r15 ::=		SEQUENCE {

nprach-Parameters-r15					SEQUENCE {

nprach-Periodicity-r15					ENUMERATED {ms80, ms160, ms320, ms640,

ms1280, ms2560, ms5120, ms10240}

OPTIONAL,	-- NEED OP

nprach-StartTime-r15					ENUMERATED {ms10, ms20, ms40, ms80,

ms160, ms320, ms640, ms1280,

ms2560, ms5120, spare6, spare5,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r15				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r15				ENUMERATED {n12, n24, n36, n48}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r15	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r15			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r15				ENUMERATED {v4, v8, v16, v32, v48, v64, v96, v128}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r15					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r15		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

OPTIONAL,	-- NEED OP

...

}	OPTIONAL	-- Need OR

}

NPRACH-ParametersListTDD-NB-v1550 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-ParametersTDD-NB-v1550

NPRACH-ParametersTDD-NB-v1550 ::=	SEQUENCE {

maxNumPreambleAttemptCE-v1550			ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1},

numRepetitionsPerPreambleAttempt-v1550	ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128,

n256, n512, n1024}

}

NPRACH-ParametersListFmt2-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ParametersFmt2-NB-r15

NPRACH-ParametersFmt2-NB-r15 ::=		SEQUENCE {

nprach-Parameters-r15					SEQUENCE {

nprach-Periodicity-r15					ENUMERATED {ms40, ms80, ms160, ms320,

ms640, ms1280, ms2560, ms5120}

OPTIONAL,	-- NEED OP

nprach-StartTime-r15					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r15				ENUMERATED {n0, n36, n72, n108, n6, n54, n102, n42,

n78, n90, n12, n24, n48, n84, n60, n18}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r15				ENUMERATED {n36, n72, n108, n144}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r15	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r15			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r15				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r15					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r15		ENUMERATED {

n24, n30, n33, n36, n60, n66, n69, n72,

n96, n102, n105, n108, n120, n132, n138, n144}

OPTIONAL,	-- NEED OP

npdcch-CarrierIndex-r15					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

...

}	OPTIONAL	-- Need OR

}

RSRP-ThresholdsNPRACH-InfoList-NB-r13 ::= SEQUENCE (SIZE(1..2)) OF RSRP-Range

EDT-TBS-InfoList-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF EDT-TBS-NB-r15

EDT-TBS-NB-r15 ::=	SEQUENCE {

edt-SmallTBS-Enabled-r15		BOOLEAN,

edt-TBS-r15						ENUMERATED {b328, b408, b504, b584, b680, b808, b936, b1000}

}

-- ASN1STOP

NPRACH-ConfigSIB-NB field descriptions

dummy This field is not used in the specification. If received it shall be ignored by the UE.

edt-SmallTBS-Enabled Value TRUE indicates UE performing EDT is allowed to select TBS smaller than edt-TBS for Msg3 according to the corresponding NPRACH resource, as specified in TS 36.213 [23].

edt-SmallTBS-Subset Presence indicates only two of the TBS values can be used according to edt-TBS corresponding to the NPRACH resource, as specified in TS 36.213 [23]. When the field is not present, any of the TBS values according to edt-TBS corresponding to the NPRACH resource can be used. This field is applicable for a NPRACH resource only when edt-SmallTBS-Enabled is included for the corresponding NPRACH resource.

edt-TBS Largest TBS for Msg3 for a NPRACH resource applicable to a UE performing EDT. Value in bits. Value b328 corresponds to 328 bits, value b408 corresponds to 408 bits and so on. See TS 36.213 [23].

maxNumPreambleAttemptCE Maximum number of preamble transmission attempts per NPRACH resource. See TS 36.321 [6]. If the UE supports enhanced random access power control and maxNumPreambleAttemptCE-r14 is included, the UE shall use maxNumPreambleAttemptCE-r14 instead of maxNumPreambleAttemptCE-r13 for the first entry in nprach-ParametersList. maxNumPreambleAttemptCE-r13 applies to FDD and maxNumPreambleAttemptCE-v1550 applies to TDD.

npdcch-CarrierIndex For FDD: Index of the carrier in the list of DL non anchor carriers. The first entry in the list has index '1', the second entry has index '2' and so on. If the UE supports mixed operation mode and dl-ConfigListMixed is present in systemInformationBlockType22-NB, the UE creates a combined list of DL carriers for random access by appending dl-ConfigListMixed to the dl-ConfigList while maintaining the order among both dl-ConfigList and dl-ConfigListMixed; only the first maxNonAnchorCarriers-NB-r14 DL non-anchor carriers in the concatenated list can be used for random access. If the field is absent in the entry in nprach-ParametersListEDT in SystemInformationBlockType22-NB, the value of npdcch-CarrierIndex in the corresponding entry of nprach-ParametersList applies, if present. If the field is absent in an entry in nprach-ParametersListFmt2EDT in SystemInformationBlockType23-NB, the value of npdcch-CarrierIndex in the corresponding entry of nprach-ParametersListFmt2 applies, if present. Otherwise, the DL anchor carrier is used. For TDD: This parameter is absent and the same carrier is used in uplink and downlink.

npdcch-NumRepetitions-RA Maximum number of repetitions for NPDCCH common search space (CSS) for RAR, Msg3 retransmission and Msg4, see TS 36.213 [23], clause 16.6. See NOTE.

npdcch-Offset-RA Fractional period offset of starting subframe for NPDCCH common search space (CSS Type 2), see TS 36.213 [23], clause 16.6. See NOTE.

npdcch-StartSF-CSS-RA Starting subframe configuration for NPDCCH common search space (CSS), including RAR, Msg3 retransmission, and Msg4, see TS 36.213 [23], clause 16.6. See NOTE.

nprach-CP-Length Cyclic prefix length for NPRACH transmission (TCP), see TS 36.211 [21], clause 10.1.6. Value us66dot7 corresponds to 66.7 microseconds and value us266dot7 corresponds to 266.7 microseconds. If the UE uses a NPRACH resource for preamble format 2, the UE ignores the value signalled in nprach-CP-Length and considers the value to be 800 microseconds.

nprach-NumCBRA-StartSubcarriers The number of start subcarriers from which a UE can randomly select a start subcarrier as specified in TS 36.321 [6]. If nprach-Config-v1330 is not included in SystemInformationBlockType2-NB, the UE sets the value of nprach-NumCBRA-StartSubcarriers-r13 to the value signalled by nprach-NumSubcarriers-r13 for the corresponding NPRACH resource. The start subcarrier indices that the UE is allowed to randomly select from, are given by: nprach-SubcarrierOffset + [0, nprach-NumCBRA-StartSubcarriers - 1]. See NOTE.

nprach-NumSubcarriers Number of sub-carriers in a NPRACH resource, see TS 36.211 [21], clause 10.1.6. In number of subcarriers. See NOTE.

nprach-ParametersList, nprach-ParametersListEDT Configures NPRACH parameters for each NPRACH resource. Up to three PRACH resources can be configured in nprach-ParametersList in a cell. Each NPRACH resource is associated with a different number of NPRACH repetitions. E-UTRAN includes the same number of entries, and listed in the same order for nprach-ParametersListEDT, as in nprach-ParametersList in SystemInformationBlockType2-NB. The NPRACH resources in nprach-ParametersListEDT are used to initiate EDT. Each NPRACH resource is associated with a TBS signalled in the corresponding entry of edt-TBS-InfoList. For TDD: The UE shall use nprach-ParametersListTDD and ignore nprach-ParametersList.

nprach-ParametersListTDD For TDD: Configure NPRACH parameters for each NPRACH. Up to three NPRACH resources can be configured in a cell. Each NPRACH resource is associated with a different number of NPRACH repetitions.

nprach-ParametersListFmt2, nprach-ParametersListFmt2EDT Configures NPRACH parameters for each NPRACH resource format 2. Up to three NPRACH resources can be configured on one carrier. Each NPRACH resource is associated with a different number of NPRACH repetitions. E-UTRAN includes the same number of entries, and listed in the same order, as in nprach-ParametersList in SystemInformationBlockType2-NB. The NPRACH resources in nprach-ParametersListFmt2EDT are used to initiate EDT. Each NPRACH resource is associated with a TBS signalled in the corresponding entry of edt-TBS-InfoList. E-UTRAN configures the NPRACH resources format 2 so that they do not overlap in time domain with the NPRACH resources configured in nprach-ParametersList and nprach-ParametersListEDT. If there is no NPRACH resource in nprach-ParametersListFmt2 (respectively nprach-ParametersListFmt2EDT) on any UL carrier for one NPRACH repetition level, the UE uses the NPRACH resources in nprach-ParametersList (respectively nprach-ParametersListEDT) for this NPRACH repetition level. Otherwise, the UE uses only NPRACH resources in nprach-ParametersListFmt2 (respectively nprach-ParametersListFmt2EDT).

nprach-Periodicity Periodicity of a NPRACH resource, see TS 36.211 [21], clause10.1.6. Unit in millisecond. See NOTE.

nprach-PreambleFormat TDD: TDD preamble format, see TS 36.211 [21]. clause 10.1.6, Value fmt0 corresponds to preamble format 0, value fmt1 corresponds to preamble format 1 and so on.

nprach-StartTime Start time of the NPRACH resource in one period, see TS 36.211 [21], clause 10.1.6. Unit in millisecond. See NOTE.

nprach-SubcarrierOffset Frequency location of the NPRACH resource, see TS 36.211 [21], clause 10.1.6. In number of subcarriers, offset from sub-carrier 0. See NOTE.

nprach-SubcarrierMSG3-RangeStart Fraction for calculating the starting subcarrier index of the range reserved for indication of UE support for multi-tone Msg3 transmission, within the NPRACH resource, see TS 36.211 [21], clause 10.1.6. Multi-tone Msg3 transmission is not supported for {32, 64, 128} repetitions of NPRACH. For at least one of the NPRACH resources with the number of NPRACH repetitions other than {32, 64, 128}, the value of nprach-SubcarrierMSG3-RangeStart should not be 0. If nprach-SubcarrierMSG3-RangeStart is equal to zero, no start subcarrier index for the single-tone Msg3 NPRACH is allocated and the start subcarrier indexes for the multi-tone Msg3 NPRACH partition are given by nprach-SubcarrierOffset + [0, nprach-NumCBRA-StartSubcarriers - 1]. If nprach-SubcarrierMSG3-RangeStart is equal to oneThird or twoThird, the start subcarrier indexes for the two partitions are given by: nprach-SubcarrierOffset + [0, FLOOR (nprach-NumCBRA-StartSubcarriers * nprach-SubcarrierMSG3-RangeStart) -1] for the single-tone Msg3 NPRACH partition; nprach-SubcarrierOffset + [FLOOR (nprach-NumCBRA-StartSubcarriers * nprach-SubcarrierMSG3-RangeStart), nprach-NumCBRA-StartSubcarriers - 1] for the multi-tone Msg3 NPRACH partition; If nprach-SubcarrierMSG3-RangeStart is equal to one, the start subcarrier indexes for the single-tone Msg3 NPRACH are given by nprach-SubcarrierOffset + [0, nprach-NumCBRA-StartSubcarriers - 1] and no start subcarrier index for the multi-tone Msg3 NPRACH partition is allocated. See NOTE.

nprach-TxDurationFmt01 Duration of PRACH segment transmission for PRACH resource format 0 and format 1 in NTN transmission, see TS 36.213 [23]. Unit in duration of one preamble transmission (TCP+TSEQ). Value v2dot4 corresponds to the duration of 2.4 preamble transmission, value v4dot4 corresponds to the duration of 4.4 preambles transmission and so on.

nprach-TxDurationFmt2 Duration of PRACH segment transmission for PRACH resource format 2 in NTN transmission, see TS 36.213 [23]. Unit in duration of one preamble transmission (TCP+TSEQ). Value v1dot6 corresponds to the duration of 1.6 preamble transmission, value v2dot6 corresponds to the duration of 2.6 preambles transmission and so on.

numRepetitionsPerPreambleAttempt Number of NPRACH repetitions per attempt for each NPRACH resource, See TS 36.211 [21], clause 10.1.6. numRepetitionsPerPreambleAttempt-r13 applies to FDD and numRepetitionsPerPreambleAttempt-v1550 applies to TDD.

rsrp-ThresholdsPrachInfoList The criterion for UEs to select a NPRACH resource. Up to 2 RSRP threshold values can be signalled. The first element corresponds to RSRP threshold 1, the second element corresponds to RSRP threshold 2. See TS 36.321 [6]. If absent, there is only one NPRACH resource. A UE that supports powerClassNB-14dBm-r14 shall correct the RSRP threshold values before applying them as follows: RSRP threshold = Signalled RSRP threshold - min{0, (14-min(23, P-Max))} where P-Max: is the value of p-Max field in SystemInformationBlockType1-NB.

NOTE:

-	If the field is absent in an entry of nprach-ParametersList in SystemInformationBlockType22-NB, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

-	If the field is absent in the entry in nprach-ParametersListEDT, the value of the same field in the corresponding entry of nprach-ParametersList on the same UL carrier applies, if present. Otherwise, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

-	If the field is absent in an entry of nprach-ParametersListTDD in SystemInformationBlockType22-NB, the value of the same field in the corresponding entry of nprach-ParametersListTDD in SystemInformationBlockType2-NB applies. The field is mandatory present in nprach-ParametersListTDD in SystemInformationBlockType2-NB.

-	If the field is absent in an entry of nprach-ParametersListFmt2 in SystemInformationBlockType23-NB, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB applies. Otherwise the value of the same field, if present, in the corresponding entry of the first occurence of nprach-ParametersListFmt2 in the non anchor carrier list applies. Otherwise, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

-	If the field is absent in an entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

-	If the field is absent in an entry of nprach-ParametersListFmt2EDT in SystemInformationBlockType23-NB, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 on the same UL carrier applies. Otherwise, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB applies. Otherwise the value of the same field, if present, in the corresponding entry of the first occurence of nprach-ParametersListFmt2 in the non anchor carrier list applies. Otherwise, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

-	If the field is absent in an entry of nprach-ParametersListFmt2EDT in SystemInformationBlockType2-NB, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB applies. Otherwise the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

| Conditional presence | Explanation |
| --- | --- |
| EDT1 | The field is mandatory present if cp-EDT, cp-EDT-5GC, up-EDT or up-EDT-5GC in SystemInformationBlockType2-NB is present; otherwise the field is not present and the UE shall delete any existing value for this field. |
| EDT2 | The field is optionally present, Need OR, if edt-Parameters is present; otherwise the field is not present and the UE shall delete any existing value for this field. |
| NTN | The field is optionally present, need OR, for NTN. Otherwise, the field is not present and the UE shall delete any existing value for this field. |
| TDD | This field is mandatory present for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	NPUSCH-Config-NB

The IE NPUSCH-ConfigCommon-NB is used to specify the common NPUSCH configuration. The IE NPUSCH-ConfigDedicated-NB is used to specify the UE specific NPUSCH configuration.

NPUSCH-Config-NB information element

-- ASN1START

NPUSCH-ConfigCommon-NB-r13 ::=		SEQUENCE {

ack-NACK-NumRepetitions-Msg4-r13	SEQUENCE (SIZE(1.. maxNPRACH-Resources-NB-r13)) OF

ACK-NACK-NumRepetitions-NB-r13,

srs-SubframeConfig-r13				ENUMERATED {

sc0, sc1, sc2, sc3, sc4, sc5, sc6, sc7,

sc8, sc9, sc10, sc11, sc12, sc13, sc14, sc15

}							OPTIONAL,	-- Need OR

dmrs-Config-r13						SEQUENCE {

threeTone-BaseSequence-r13			INTEGER (0..12)			OPTIONAL,	-- Need OP

threeTone-CyclicShift-r13			INTEGER (0..2),

sixTone-BaseSequence-r13			INTEGER (0..14)			OPTIONAL,	-- Need OP

sixTone-CyclicShift-r13				INTEGER (0..3),

twelveTone-BaseSequence-r13			INTEGER (0..30)			OPTIONAL	-- Need OP

}		OPTIONAL,	-- Need OR

ul-ReferenceSignalsNPUSCH-r13		UL-ReferenceSignalsNPUSCH-NB-r13

}

NPUSCH-ConfigCommon-NB-v17xx ::=	SEQUENCE {

npusch-TxDuration-r17		ENUMERATED {ms2, ms4, ms8, ms16, ms32, ms64, ms128, ms256}

OPTIONAL	-- Cond NTN

}

UL-ReferenceSignalsNPUSCH-NB-r13 ::=	SEQUENCE {

groupHoppingEnabled-r13					BOOLEAN,

groupAssignmentNPUSCH-r13				INTEGER (0..29)

}

NPUSCH-ConfigDedicated-NB-r13 ::=	SEQUENCE {

ack-NACK-NumRepetitions-r13			ACK-NACK-NumRepetitions-NB-r13	OPTIONAL,	-- Need ON

npusch-AllSymbols-r13				BOOLEAN							OPTIONAL,	-- Cond SRS

groupHoppingDisabled-r13			ENUMERATED {true}				OPTIONAL	-- Need OR

}

NPUSCH-ConfigDedicated-NB-v1610 ::=	SEQUENCE {

npusch-MultiTB-Config-r16			ENUMERATED {interleaved, nonInterleaved}

}

NPUSCH-ConfigDedicated-NB-v17xx ::=		SEQUENCE {

npusch-TxDurationConfig-r17		SetupRelease {NPUSCH-TxDurationConfig-NB-r17} OPTIONAL	-- Need ON

}

NPUSCH-TxDurationConfig-NB-r17 ::=	SEQUENCE {

npusch-TxDuration-r17		ENUMERATED {ms2, ms4, ms8, ms16, ms32, ms64, ms128, ms256}

}

ACK-NACK-NumRepetitions-NB-r13	::=	ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128}

-- ASN1STOP

NPUSCH-Config-NB field descriptions

ack-NACK-NumRepetitions Number of repetitions for the ACK NACK resource unit carrying HARQ response to NPDSCH, see TS 36.213 [23], clause 16.4.2. If this field is absent and no value was configured via dedicated signalling, the value used for reception of Msg4 is used.

ack-NACK-NumRepetitions-Msg4 Number of repetitions for ACK/NACK HARQ response to NPDSCH containing Msg4 per NPRACH resource, see TS 36.213 [23], clause 16.4.2.

groupAssignmentNPUSCH See TS 36.211 [21], clause 10.1.4.1.3.

groupHoppingDisabled See TS 36.211 [21], clause 10.1.4.1.3.

groupHoppingEnabled See TS 36.211 [21], clause 10.1.4.1.3.

npusch-AllSymbols If set to TRUE, the UE shall use all NB-IoT symbols for NPUSCH transmission. If set to FALSE, the UE punctures the NPUSCH transmissions in the symbols that collides with SRS. If the field is not present, the UE uses all NB-IoT symbols for NPUSCH transmission. See TS 36.211 [21], clause 10.1.3.6.

npusch-MultiTB-Config For FDD: Activation of multiple TBs scheduling in UL, see TS 36.213 [23]. Value interleaved indicates that multiple TBs scheduling with interleaved transmission is enabled, value nonInterleaved indicates that multiple TBs scheduling without interleaved transmission is enabled.

npusch-TxDuration Duration of NPUSCH segment transmission in NTN transmission, see TS 36.213 [23]. Unit in ms. Value ms2 corresponds to 2 ms, value ms4 corresponds to 4 ms and so on.

sixTone-BaseSequence The base sequence of DMRS sequence in a cell for 6 tones transmission; see TS 36.211 [21], clause 10.1.4.1.2. If absent, it is given by NB-IoT CellID mod 14. Value 14 is not used.

sixTone-CyclicShift Define 4 cyclic shifts for the 6-tone case, see TS 36.211 [21], clause 10.1.4.1.2.

srs-SubframeConfig SRS SubframeConfiguration. See TS 36.211 [21], table 5.5.3.3-1. Value sc0 corresponds to value 0, sc1 to value 1 and so on.

threeTone-BaseSequence The base sequence of DMRS sequence in a cell for 3 tones transmission; see TS 36.211 [21], clause 10.1.4.1.2. If absent, it is given by NB-IoT CellID mod 12. Value 12 is not used.

threeTone-CyclicShift Define 3 cyclic shifts for the 3-tone case, see TS 36.211 [21], clause 10.1.4.1.2.

twelveTone-BaseSequence The base sequence of DMRS sequence in a cell for 12 tones transmission; see TS 36.211 [21], clause 10.1.4.1.2. If absent, it is given by NB-IoT CellID mod 30. Value 30 is not used.

ul-ReferenceSignalsNPUSCH Used to specify parameters needed for the transmission on NPUSCH.

| Conditional presence | Explanation |
| --- | --- |
| NTN | The field is optionally present, need OR, for NTN. Otherwise, the field is not present and the UE shall delete any existing value for this field. |
| SRS | This field is optionally present, need OP, if srs-SubframeConfig is broadcasted. Otherwise, the IE is not present. |

#### –	PDCP-Config-NB

The IE PDCP-Config-NB is used to set the configurable PDCP parameters for data radio bearers.

PDCP-Config-NB information element

-- ASN1START

PDCP-Config-NB-r13 ::=		SEQUENCE {

discardTimer-r13			ENUMERATED {

ms5120, ms10240, ms20480, ms40960,

ms81920, infinity, spare2, spare1

}	OPTIONAL,			-- Cond Setup

headerCompression-r13		CHOICE {

notUsed						NULL,

rohc						SEQUENCE {

maxCID-r13					INTEGER (1..16383)				DEFAULT 15,

profiles-r13				SEQUENCE {

profile0x0002				BOOLEAN,

profile0x0003				BOOLEAN,

profile0x0004				BOOLEAN,

profile0x0006				BOOLEAN,

profile0x0102				BOOLEAN,

profile0x0103				BOOLEAN,

profile0x0104				BOOLEAN

},

...

}

},

...,

[[	cipheringDisabled-r16		ENUMERATED {true}		OPTIONAL    -- Cond ConnectedTo5GC

]]

}

-- ASN1STOP

PDCP-Config-NB field descriptions

cipheringDisabled If included, ciphering is disabled for this DRB regardless of which ciphering algorithm is configured for the SRB/DRBs. E-UTRAN may include this field only when the UE is connected to 5GC. The value for this field cannot be changed after the DRB is set up.

discardTimer Indicates the discard timer value specified in TS 36.323 [8]. Value in milliseconds. Value ms5120 means 5120 ms, ms10240 means 10240 ms and so on.

headerCompression E-UTRAN does not reconfigure header compression except optionally upon RRC Connection Resumption.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 36.323 [8]. The total value of MAX_CIDs across all bearers for the UE should be less than or equal to the value of maxNumberROHC-ContextSessions parameter as indicated by the UE.

profiles The profiles used by both compressor and decompressor in both UE and E-UTRAN. The field indicates which of the ROHC profiles specified in TS 36.323 [8] are supported, i.e. value true indicates that the profile is supported. Profile 0x0000 shall always be supported when the use of ROHC is configured. If support of two ROHC profile identifiers with the same 8 LSB's is signalled, only the profile corresponding to the highest value shall be applied.

| Conditional presence | Explanation |
| --- | --- |
| ConnectedTo5GC | The field is optionally present, need OR, if the UE is connected to 5GC. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| Setup | The field is mandatory present in case of radio bearer setup. Otherwise the field is optionally present, need ON. |

#### –	PhysicalConfigDedicated-NB

The IE PhysicalConfigDedicated-NB is used to specify the UE specific physical channel configuration.

PhysicalConfigDedicated-NB information element

-- ASN1START

PhysicalConfigDedicated-NB-r13 ::=	SEQUENCE {

carrierConfigDedicated-r13			CarrierConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

npdcch-ConfigDedicated-r13			NPDCCH-ConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

npusch-ConfigDedicated-r13			NPUSCH-ConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

uplinkPowerControlDedicated-r13		UplinkPowerControlDedicated-NB-r13	OPTIONAL,	-- Need ON

...,

[[	twoHARQ-ProcessesConfig-r14		ENUMERATED {true}	OPTIONAL	-- Need OR

]],

[[	interferenceRandomisationConfig-r14	ENUMERATED {true}	OPTIONAL	-- Need OR

]],

[[	npdcch-ConfigDedicated-v1530	NPDCCH-ConfigDedicated-NB-v1530		OPTIONAL	-- Cond TDD

]],

[[	additionalTxSIB1-Config-v1540	ENUMERATED {true}	OPTIONAL	-- Cond additionalSIB1

]],

[[	npusch-ConfigDedicated-v1610		NPUSCH-ConfigDedicated-NB-v1610

OPTIONAL,	-- Cond twoHARQ

npdsch-ConfigDedicated-r16			NPDSCH-ConfigDedicated-NB-r16

OPTIONAL,	-- Need ON

resourceReservationConfigDL-r16		SetupRelease {ResourceReservationConfig-NB-r16}

OPTIONAL,	-- Cond dl-NonAnchor

resourceReservationConfigUL-r16		SetupRelease {ResourceReservationConfig-NB-r16}

OPTIONAL	-- Cond ul-NonAnchor

]] ,

[[	npusch-ConfigDedicated-v17xx		NPUSCH-ConfigDedicated-NB-v17xx 	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

PhysicalConfigDedicated-NB field descriptions

additionalTxSIB1-Config Indicates if subframe #3 not containing additional SIB1 transmission is a NB-IoT DL subframe, as specified in TS 36.213 [23], clause 16.4.

carrierConfigDedicated Anchor/ non-anchor carrier used for all unicast transmissions.

interferenceRandomisationConfig For FDD: Interference randomisation enabled in connected mode, except for random access procedure in connected mode, see TS 36.211 [21]. For random access in connected mode interference randomisation on non-anchor is used and is not used on anchor carrier, see TS 36.211 [21]. For TDD: the parameter is not present.

npdcch-ConfigDedicated NPDCCH configuration.

npdsch-ConfigDedicated NPDSCH configuration.

npusch-ConfigDedicated UL unicast configuration.

resourceReservationConfigDL Configuration of downlink reserved resources, e.g. for NB-IoT co-existence with NR, see TS 36.211 [21], TS 36.212 [22], and TS 36.213 [23].

resourceReservationConfigUL Configuration of uplink reserved resources, e.g. for NB-IoT co-existence with NR, see TS 36.211 [21], TS 36.212 [22], and TS 36.213 [23].

twoHARQ-ProcessesConfig Activation of two HARQ processes, see TS 36.212 [22] and TS 36.213 [23].

uplink-PowerControlDedicated UL power control parameter.

| Conditional presence | Explanation |
| --- | --- |
| additionalSIB1 | This field is optionally present, Need OR, if additionalTransmissionSIB1 is set to TRUE in MasterInformationBlock-NB; otherwise it is not present. |
| dl-NonAnchor | The field is optionally present, Need ON, for a DL non-anchor carrier; otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD | The field is optionally present, Need OR, for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |
| twoHARQ | The field is optionally present, Need OR, if twoHARQ-ProcessesConfig is configured; otherwise the field is not present and the UE shall delete any existing value for this field. |
| ul-NonAnchor | The field is optionally present, Need ON, for an UL non-anchor carrier; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	PUR-Config-NB

The IE PUR-Config-NB is used to specify PUR configuration.

PUR-Config-NB information element

-- ASN1START

PUR-Config-NB-r16	::=				SEQUENCE {

pur-ConfigID-r16					PUR-ConfigID-NB-r16				OPTIONAL,	--Need OR

pur-TimeAlignmentTimer-r16			INTEGER (1..8)				OPTIONAL,	--Need OR

pur-NRSRP-ChangeThreshold-r16		SetupRelease {PUR-NRSRP-ChangeThreshold-r16}

OPTIONAL,	--Need ON

pur-ImplicitReleaseAfter-r16		ENUMERATED {n2, n4, n8, spare}	OPTIONAL,	--Need OR

pur-RNTI-r16						C-RNTI							OPTIONAL,	--Need ON

pur-ResponseWindowTimer-r16			ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

OPTIONAL,	--Need ON

pur-StartTimeParameters-r16			SEQUENCE {

periodicityAndOffset-r16			PUR-PeriodicityAndOffset-NB-r16,

startSFN-r16						INTEGER (0..1023),

startSubframe-r16					INTEGER (0..9),

hsfn-LSB-Info-r16					BIT STRING (SIZE(1))

}																	OPTIONAL,	--Need ON

pur-NumOccasions-r16				ENUMERATED {one, infinite},

pur-PhysicalConfig-r16				SEQUENCE {

carrierConfig-r16					CarrierConfigDedicated-NB-r13,

npusch-NumRUsIndex-r16				INTEGER (0..7),

npusch-NumRepetitionsIndex-r16		INTEGER (0..7),

npusch-SubCarrierSetIndex-r16		CHOICE {

khz15								INTEGER (0..18),

khz3dot75							INTEGER (0..47)

},

npusch-MCS-r16						CHOICE {

singleTone							INTEGER (0..10),

multiTone							INTEGER (0..13)

},

p0-UE-NPUSCH-r16					INTEGER (-8..7),

alpha-r16							ENUMERATED {al0, al04, al05, al06,

al07, al08, al09, al1},

npusch-CyclicShift-r16				ENUMERATED {n0, n6},

npdcch-Config-r16					NPDCCH-ConfigDedicated-NB-r13

}	OPTIONAL,	-- Need ON

...,

[[

pur-PhysicalConfig-v1650			SEQUENCE {

ack-NACK-NumRepetitions-r16		ACK-NACK-NumRepetitions-NB-r13

}														OPTIONAL	--Need ON

]]

}

PUR-NRSRP-ChangeThreshold-r16 ::=	SEQUENCE {

increaseThresh-r16						NRSRP-ChangeThresh-NB-r16,

decreaseThresh-r16						NRSRP-ChangeThresh-NB-r16	OPTIONAL	--Need OP

}

NRSRP-ChangeThresh-NB-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

-- ASN1STOP

PUR-Config-NB field descriptions

ack-NACK-NumRepetitions Number of repetitions for the ACK NACK resource unit carrying HARQ response to NPDSCH, see TS 36.213 [23], clause 16.4.2. If this field is absent and no value was configured via pur-Config, the value of ack-NACK-NumRepetitions used for HARQ response to NPDSCH containing this RRCConnectionRelease-NB message applies.

alpha Parameter: αc(3). See TS 36.213 [23], clause 16.2.1.1.1.

carrierConfig Carrier used for PUR.

hsfn-LSB-Info LSB of the H-SFN corresponding to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config.

npdcch-Config NPDCCH configuration for PUR.

npusch-CyclicShift Parameter: ncs. See TS 36.211 [21], clause 10.1.4.1.2. Value n0 corresponds to value 0 and value n6 corresponds to value 6.

npusch-MCS Index to tables specified in TS 36.213 [23], Table 16.5.1.2-1 and Table 16.5.1.2-2 for single tone and multi tone respectively, that defines modulation and TBS index for NPUSCH for PUR.

npusch-NumRepetitionsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-3, that defines number of repetitions for NPUSCH for PUR.

npusch-NumRUsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-2, that defines number of resource units for NPUSCH for PUR.

npusch-SubCarrierSetIndex For NPUSCH transmission with subcarrier spacing 3.75 kHz, indicates the subcarrier used for PUR specified in TS 36.213 [23]. For NPUSCH transmission with subcarrier spacing 15 kHz, index to a table specified in TS 36.213 [23], Table 16.5.1.1-1, that defines the set of subcarriers for NPUSCH for PUR.

p0-UE-NPUSCH Parameter: . See TS 36.213 [23], clause 16.2.1.1.1, unit dB.

pur-ImplicitReleaseAfter Number of consecutive PUR occasions that can be skipped before implicit release of PUR configuration. Value n2 corresponds to 2 PUR occasions, value n4 corresponds to 4 PUR occasions, and so on.

pur-NRSRP-ChangeThreshold Threshold(s) of change in serving cell NRSRP in dB for TA validation. Value dB4 corresponds to 4 dB, value dB6 corresponds to 6 dB, and so on. When pur-NRSRP-ChangeThreshold is set to setup, if decreaseThrsh is absent the value of increaseThresh is also used for decreaseThresh.

pur-NumOccasions Number of PUR occasions. Value one corresponds to 1 PUR occasion, and value infinite corresponds to an infinite number of PUR occasions.

pur-PeriodicityAndOffset Indicates the periodicity for the PUR occasions and time offset until the first PUR occasion.

pur-ResponseWindowTimer Duration of the PUR response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods, and so on. The value considered by the UE is: pur-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s).

pur-TimeAlignmentTimer Value of the time alignment timer for PUR. Value in number of periodicity of PUR.

#### –	PUR-ConfigID-NB

The IE PUR-ConfigID-NB is used to indicate the PUR configuration identity.

PUR-ConfigID-NB information element

-- ASN1START

PUR-ConfigID-NB-r16 ::= BIT STRING (SIZE(20))

-- ASN1STOP

#### –	PUR-PeriodicityAndOffset-NB

The IE PUR-PeriodicityAndOffset is used to indicate H-SFN of the first PUR occasion and periodicity of the subsequent PUR occasions. The value of periodicity is in the unit of H-SFN duration (i.e., 10.24s). Value periodicity8 corresponds to periodicity of 8 H-SFN, value periodicity16 corresponds to periodicity of 16 H-SFN and so on. The value of offset is in the unit of H-SFN duration (i.e., 10.24s).

PUR-PeriodicityAndOffset-NB information element

-- ASN1START

PUR-PeriodicityAndOffset-NB-r16 ::= 	CHOICE {

periodicity8		INTEGER (1..7),

periodicity16		INTEGER (1..15),

periodicity32		INTEGER (1..31),

periodicity64		INTEGER (1..63),

periodicity128		INTEGER (1..127),

periodicity256		INTEGER (1..257),

periodicity512		INTEGER (1..511),

periodicity1024		INTEGER (1..1023),

periodicity2048		INTEGER (1..2047),

periodicity4096		INTEGER (1..4095),

periodicity8192		INTEGER (1..8191)

}

-- ASN1STOP

#### –	RACH-ConfigCommon-NB

The IE RACH-ConfigCommon-NB is used to specify the generic random access parameters.

RACH-ConfigCommon-NB information element

-- ASN1START

RACH-ConfigCommon-NB-r13 ::=		SEQUENCE {

preambleTransMax-CE-r13				PreambleTransMax,

powerRampingParameters-r13			PowerRampingParameters,

rach-InfoList-r13					RACH-InfoList-NB-r13,

connEstFailOffset-r13				INTEGER (0..15)					OPTIONAL,	-- Need OP

...,

[[	powerRampingParameters-v1450	PowerRampingParameters-NB-v1450	OPTIONAL	-- Need OR

]],

[[ rach-InfoList-v1530				RACH-InfoList-NB-v1530	OPTIONAL -- Cond EDT

]]

}

RACH-InfoList-NB-r13 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF RACH-Info-NB-r13

RACH-InfoList-NB-v1530 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF RACH-Info-NB-v1530

RACH-Info-NB-r13	::=		SEQUENCE {

ra-ResponseWindowSize-r13			ENUMERATED {

pp2, pp3, pp4, pp5, pp6, pp7, pp8, pp10},

mac-ContentionResolutionTimer-r13	ENUMERATED {

pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

}

RACH-Info-NB-v1530 ::=		SEQUENCE {

mac-ContentionResolutionTimer-r15	ENUMERATED {

pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

}

PowerRampingParameters-NB-v1450 ::=		SEQUENCE {

preambleInitialReceivedTargetPower-v1450		ENUMERATED {

dBm-130, dBm-128, dBm-126, dBm-124, dBm-122,

dBm-88, dBm-86, dBm-84,dBm-82, dBm-80}

OPTIONAL,	-- Need OR

powerRampingParametersCE1-r14				SEQUENCE {

powerRampingStepCE1-r14						ENUMERATED {dB0, dB2, dB4, dB6},

preambleInitialReceivedTargetPowerCE1-r14	ENUMERATED {

dBm-130, dBm-128, dBm-126, dBm-124, dBm-122,

dBm-120, dBm-118, dBm-116, dBm-114, dBm-112,

dBm-110, dBm-108, dBm-106, dBm-104, dBm-102,

dBm-100, dBm-98, dBm-96, dBm-94, dBm-92,

dBm-90, dBm-88, dBm-86, dBm-84,	dBm-82, dBm-80}

} OPTIONAL	-- Need OR

}

-- ASN1STOP

RACH-ConfigCommon-NB field descriptions

connEstFailOffset Parameter "Qoffsettemp" in TS 36.304 [4]. If the field is not present the value of infinity shall be used for "Qoffsettemp".

mac-ContentionResolutionTimer Timer for contention resolution in TS 36.321 [6]. Value in PDCCH periods. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on. mac-ContentionResolutionTimer-r15 is only applicable for EDT. UE performing EDT shall use mac-ContentionResolutionTimer-r15, if present. For FDD: The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 20.48s).

powerRampingParameters, powerRampingParametersCE1 Power ramping step and preamble initial received target power – same as TS 36.213 [23] and TS 36.321 [6]. For FDD, if the UE does not support enhanced random access power control and more than one repetition level is configured in the cell, then the UE transmits NPRACH with max power except for the lowest repetition level. Otherwise, the UE uses NPRACH power ramping. For FDD, if the UE supports enhanced random access power control and powerRampingParameters-v1450 is signalled, or for TDD, the UE uses NPRACH power ramping across repetition levels as specified in TS 36.321 [6]. If preambleInitialReceivedTargetPower-v1450 is present, the UE shall use preambleInitialReceivedTargetPower-v1450 instead of preambleInitialReceivedTargetPower (i.e. without suffix). If powerRampingParametersCE1 is present, the UE shall use powerRampingParametersCE1 instead of powerRampingParameters for NPRACH power ramping in the second repetition level.

preambleTransMax-CE Maximum number of preamble transmission in TS 36.321 [6]. Value is an integer.

ra-ResponseWindowSize Duration of the RA response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods and so on. For FDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 20.48s).

Editor’s Note: Agreement: If the start of the RA Response window is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the ra-ResponseWindowSize for IoT NTN.

Editor’s Note: Agreement: If the start of mac-ContentionResolutionTimer is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the mac-ContentionResolutionTimer for IoT NTN.

| Conditional presence | Explanation |
| --- | --- |
| EDT | The field is optionally present, Need OR, if edt-Parameters is present; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	RadioResourceConfigCommonSIB-NB

The IE RadioResourceConfigCommonSIB-NB is used to specify common radio resource configurations in the system information, e.g., the random access parameters and the static physical layer parameters.

RadioResourceConfigCommonSIB-NB information element

-- ASN1START

RadioResourceConfigCommonSIB-NB-r13 ::=	SEQUENCE {

rach-ConfigCommon-r13					RACH-ConfigCommon-NB-r13,

bcch-Config-r13							BCCH-Config-NB-r13,

pcch-Config-r13							PCCH-Config-NB-r13,

nprach-Config-r13						NPRACH-ConfigSIB-NB-r13,

npdsch-ConfigCommon-r13					NPDSCH-ConfigCommon-NB-r13,

npusch-ConfigCommon-r13					NPUSCH-ConfigCommon-NB-r13,

dl-Gap-r13								DL-GapConfig-NB-r13			OPTIONAL,		-- Need OP

uplinkPowerControlCommon-r13			UplinkPowerControlCommon-NB-r13,

...,

[[	nprach-Config-v1330					NPRACH-ConfigSIB-NB-v1330	OPTIONAL		-- Need OR

]],

[[	nprach-Config-v1450					NPRACH-ConfigSIB-NB-v1450	OPTIONAL		-- Cond EnhPowerControl

]],

[[	nprach-Config-v1530					NPRACH-ConfigSIB-NB-v1530	OPTIONAL,	-- Need OR

dl-Gap-v1530						DL-GapConfig-NB-v1530		OPTIONAL,	-- Cond TDD

wus-Config-r15						WUS-Config-NB-r15			OPTIONAL	-- Need OR

]],

[[	nprach-Config-v1550					NPRACH-ConfigSIB-NB-v1550	OPTIONAL	-- Cond TDD1

]],

[[

gwus-Config-r16						GWUS-Config-NB-r16			OPTIONAL,	-- Need OR

nrs-NonAnchorConfig-r16				ENUMERATED {true}			OPTIONAL,	-- Need OR

ue-SpecificDRX-CycleMin-r16			ENUMERATED {rf32, rf64, rf128, rf256, rf512,

rf1024}		OPTIONAL	-- Need OR

]],

[[	nprach-Config-v17xx					NPRACH-ConfigSIB-NB-v17xx 		OPTIONAL,	-- Need OR

npusch-Config-v17xx					NPUSCH-ConfigCommon-NB-v17xx 	OPTIONAL	-- Need OR

]]

}

BCCH-Config-NB-r13 ::=					SEQUENCE {

modificationPeriodCoeff-r13				ENUMERATED {n16, n32, n64, n128}

}

PCCH-Config-NB-r13 ::=					SEQUENCE {

defaultPagingCycle-r13					ENUMERATED {rf128, rf256, rf512, rf1024},

nB-r13									ENUMERATED {

fourT, twoT, oneT, halfT, quarterT, one8thT,

one16thT, one32ndT, one64thT,

one128thT, one256thT, one512thT, one1024thT,

spare3, spare2, spare1},

npdcch-NumRepetitionPaging-r13			ENUMERATED {

r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

}

-- ASN1STOP

RadioResourceConfigCommonSIB-NB field descriptions

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 36.304 [4]. Value rf128 corresponds to 128 radio frames, rf256 corresponds to 256 radio frames and so on.

dl-Gap Downlink transmission gap configuration for the anchor carrier. See TS 36.211 [21], clause 10.2.3.4. If the field is absent, there is no gap.

gwus-Config For FDD: GWUS Configuration.

modificationPeriodCoeff Actual modification period, expressed in number of radio frames= modificationPeriodCoeff * defaultPagingCycle. n16 corresponds to value 16, n32 corresponds to value 32, and so on. The BCCH modification period should be larger or equal to 40.96s.

nB Parameter: nB is used as one of parameters to derive the Paging Frame and Paging Occasion according to TS 36.304 [4]. Value in multiples of 'T' as defined in TS 36.304 [4]. A value of fourT corresponds to 4 * T, a value of twoT corresponds to 2 * T and so on.

npdcch-NumRepetitionPaging Maximum number of repetitions for NPDCCH common search space (CSS) for paging, see TS 36.213 [23], clause 16.6.

nrs-NonAnchorConfig For FDD: Indicates if NRS are present on non-anchor paging carriers even when no paging NPDCCH is transmitted, see TS 36.211 [21], clause 10.2.6.

ue-SpecificDRX-CycleMin Minimum UE specific DRX cycle in the cell, see TS 36.304 [4], clause 7.1. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on. If present, E-UTRAN ensures PCCH configuration does not lead to CSS overlap for ue-SpecificDRX-CycleMin. If the field is not present, use of UE specific DRX cycle is not allowed in the cell.

wus-Config For FDD: WUS Configuration.

| Conditional presence | Explanation |
| --- | --- |
| EnhPowerControl | This field is optional present, Need OR, if PowerRampingParameters-NB-v1450 is included in SIB2-NB. Otherwise the field is not present. |
| TDD | The field is optionally present, Need OR, for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD1 | The field is mandatory present for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	RadioResourceConfigDedicated-NB

The IE RadioResourceConfigDedicated-NB is used to setup/modify/release RBs, to modify the MAC main configuration, and to modify dedicated physical configuration.

RadioResourceConfigDedicated-NB information element

-- ASN1START

RadioResourceConfigDedicated-NB-r13 ::=	SEQUENCE {

srb-ToAddModList-r13					SRB-ToAddModList-NB-r13			OPTIONAL,	-- Need ON

drb-ToAddModList-r13					DRB-ToAddModList-NB-r13			OPTIONAL,	-- Need ON

drb-ToReleaseList-r13					DRB-ToReleaseList-NB-r13		OPTIONAL,	-- Need ON

mac-MainConfig-r13						CHOICE {

explicitValue-r13						MAC-MainConfig-NB-r13,

defaultValue-r13						NULL

}																		OPTIONAL,	-- Need ON

physicalConfigDedicated-r13				PhysicalConfigDedicated-NB-r13	OPTIONAL,	-- Need ON

rlf-TimersAndConstants-r13				RLF-TimersAndConstants-NB-r13	OPTIONAL,	-- Need ON

...,

[[	schedulingRequestConfig-r15			SchedulingRequestConfig-NB-r15	OPTIONAL	-- Need ON

]],

[[	newUE-Identity-r16					C-RNTI							OPTIONAL	-- Need OP

]]

}

SRB-ToAddModList-NB-r13 ::=			SEQUENCE (SIZE (1)) OF SRB-ToAddMod-NB-r13

SRB-ToAddMod-NB-r13 ::=				SEQUENCE {

rlc-Config-r13						CHOICE {

explicitValue						RLC-Config-NB-r13,

defaultValue						NULL

}		OPTIONAL,															-- Cond Setup

logicalChannelConfig-r13			CHOICE {

explicitValue						LogicalChannelConfig-NB-r13,

defaultValue						NULL

}		OPTIONAL,															-- Cond Setup

...,

[[	rlc-Config-v1430				RLC-Config-NB-v1430			OPTIONAL	-- Need ON

]],

[[	rlc-Config-v17xx				RLC-Config-NB-v17xx		OPTIONAL	-- Need ON

]]

}

DRB-ToAddModList-NB-r13 ::=			SEQUENCE (SIZE (1..maxDRB-NB-r13)) OF DRB-ToAddMod-NB-r13

DRB-ToAddMod-NB-r13 ::=				SEQUENCE {

eps-BearerIdentity-r13				INTEGER (0..15)				OPTIONAL,	-- Cond DRB-Setup-EPC

drb-Identity-r13					DRB-Identity,

pdcp-Config-r13						PDCP-Config-NB-r13			OPTIONAL,	-- Cond Setup

rlc-Config-r13						RLC-Config-NB-r13			OPTIONAL,	-- Cond Setup

logicalChannelIdentity-r13			INTEGER (3..10)				OPTIONAL,	-- Cond DRB-Setup

logicalChannelConfig-r13			LogicalChannelConfig-NB-r13	OPTIONAL,	-- Cond Setup

...,

[[	rlc-Config-v1430				RLC-Config-NB-v1430			OPTIONAL	-- Need ON

]],

[[	pdu-Session-r16				PDU-SessionID-NB-r16		OPTIONAL	-- Cond DRB-Setup-5GC

]],

[[	rlc-Config-v17xx				RLC-Config-NB-v17xx		OPTIONAL	-- Need ON

]]

}

PDU-SessionID-NB-r16 ::=			INTEGER (0..255)

DRB-ToReleaseList-NB-r13 ::=		SEQUENCE (SIZE (1..maxDRB-NB-r13)) OF DRB-Identity

-- ASN1STOP

RadioResourceConfigDedicated-NB field descriptions

logicalChannelConfig For SRB a choice is used to indicate whether the logical channel configuration is signalled explicitly or set to the default logical channel configuration for SRB1 as specified in 9.2.1.1.

logicalChannelIdentity The logical channel identity for both UL and DL for a DRB. Value 3 is not used.

mac-MainConfig The default MAC MAIN configuration is specified in 9.2.2.

newUE-Identity C-RNTI used after moving to RRC_CONNECTED in response to transmission using PUR.

pdu-Session Identity of the PDU session whose QoS flow is mapped to the DRB.

physicalConfigDedicated The default dedicated physical configuration is specified in 9.2.4.

rlc-Config For SRBs a choice is used to indicate whether the RLC configuration is signalled explicitly or set to the values defined in the default RLC configuration for SRB1 in 9.2.1.1. RLC AM is the only applicable RLC mode for SRB1 and SRB1bis.

schedulingRequestConfig For FDD: Scheduling request configuration.

| Conditional presence | Explanation |
| --- | --- |
| DRB-Setup | The field is mandatory present if the corresponding DRB is being set up; otherwise it is not present. |
| DRB-Setup-5GC | The field is mandatory present if the corresponding DRB is being set up when connected to 5GC; otherwise it is not present. |
| DRB-Setup-EPC | The field is mandatory present if the corresponding DRB is being set up when connected to EPC; otherwise it is not present. |
| Setup | The field is mandatory present if the corresponding SRB/DRB is being setup; otherwise the field is optionally present, need ON. |

#### –	ResourceReservationConfig-NB

The IE ResourceReservationConfig-NB is used to specify the reserved downlink or uplink resources on a NB-IoT carrier, e.g. for deployment within a NR carrier.

ResourceReservationConfig-NB information element

-- ASN1START

ResourceReservationConfig-NB-r16::=	SEQUENCE {

periodicity-r16				ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1},

startPosition-r16			INTEGER (0..15),

resourceReservation-r16		CHOICE {

subframeBitmap-r16			CHOICE {

subframePattern10ms			BIT STRING (SIZE (10)),

subframePattern40ms			BIT STRING (SIZE (40))

},

slotConfig-r16				SEQUENCE {

slotBitmap-r16				CHOICE {

slotPattern10ms				BIT STRING (SIZE (20)),

slotPattern40ms				BIT STRING (SIZE (80))

},

symbolBitmap-r16			CHOICE {

symbolBitmapFddDl			SEQUENCE {

symbolBitmap1-r16			BIT STRING (SIZE (5))	OPTIONAL,	-- Cond Bitmap1

symbolBitmap2-r16			BIT STRING (SIZE (5))	OPTIONAL	-- Cond Bitmap2

},

symbolBitmapFddUlOrTdd		SEQUENCE {

symbolBitmap1-r16			BIT STRING (SIZE (7))	OPTIONAL,	-- Cond Bitmap1

symbolBitmap2-r16			BIT STRING (SIZE (7))	OPTIONAL	-- Cond Bitmap2

}

}

}

},

...

}

-- ASN1STOP

| ResourceReservationConfig field descriptions |
| --- |
| periodicity Periodicity of the reserved resource. Value ms10 corresponds to 10 milliseconds, value ms20 corresponds to 20 milliseconds, and so on. |
| slotPattern10ms, slotPattern40ms For FDD: Downlink slot-level resource reservation configuration over 10ms or 40ms. Parameter slot-reserved-resource-config-DL in TS 36.211 [21] and TS 36.213 [23] The first/leftmost 2-bits corresponds to the subframe #0 of the radio frame satisfying SFN mod x = startPosition, where x is the periodicity of the reserved resource divided by 10. Two bits for each subframe coded as: 00: both slots are not reserved 01: the first slot is not reserved, the second slot is reserved 10: the first slot is reserved, the second slot is not reserved 11: both slots are reserved |
| startPosition Start time of the resource reservation pattern in one period. Unit in multiple of 10 milliseconds. E-UTRAN configures the value of startPosition such as startPosition * 10 < periodicity. |
| subframePattern10ms, subframePattern40ms For FDD: Downlink subframe-level resource reservation configuration over 10ms or 40ms. Parameters valid-subframe-config-DL in TS 36.211 [21] and TS 36.213 [23]. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = startPosition, where x is the periodicity of the reserved resource divided by 10. Value 0 indicates that the corresponding subframe is not reserved, value 1 indicates that the corresponding subframe is reserved. |
| symbolBitmap Symbol-level resource reservation for one subframe. E-UTRAN configures symbolConfigFddDl for a DL FDD NB-IoT carrier. E-UTRAN configures symbolConfigFddULOrTdd for an UL FDD NB-IoT carrier or a TDD NB-IoT carrier. |
| symbolBitmap1, symbolBitmap2 Symbol-level resource reservation over the first or the second slot of one subframe, see TS 36.211 [21]. The first/leftmost bit corresponds to the symbol #0 in the slot. Value 0 indicates that the corresponding symbol is not reserved, value 1 indicates that the corresponding symbol is reserved. If symbolBitmap1 is absent, value '01' in the slotBitmap corresponds to the second slot being reserved. If symbolBitmap2 is absent, value '10' in the slotBitmap corresponds to the first slot being reserved. |  |
| symbolBitmapFddDl For FDD: Downlink symbol-level resource reservation over the first and the second slot of one subframe, see TS 36.211 [21]. Symbols that carry NRS are not reserved. |
| symbolBitmapFddUlOrTdd For FDD: Uplink symbol-level resource reservation over the first and the second slot of one subframe, see TS 36.211 [21]. For TDD: Uplink or downlink symbol-level resource reservation over the first and the second slot of one subframe, see TS 36.211 [21]. Symbols that carry NRS are not reserved. |

| Conditional presence | Explanation |  |
| --- | --- | --- |
| Bitmap1 | The field is optional present, need OR, if value of slotBitmap corresponditing to at least one subrame is '01'; otherwise the field is not present. |
| Bitmap2 | The field is optional present, need OR, if value of slotBitmap corresponditing to at least one subrame is '10'; otherwise the field is not present. |

#### –	RLC-Config-NB

The IE RLC-Config-NB is used to specify the RLC configuration of SRBs and DRBs.

RLC-Config-NB information element

-- ASN1START

RLC-Config-NB-r13 ::=	CHOICE	{

am						SEQUENCE {

ul-AM-RLC-r13				UL-AM-RLC-NB-r13,

dl-AM-RLC-r13				DL-AM-RLC-NB-r13

},

...,

um-Bi-Directional-r15		NULL,

um-Uni-Directional-UL-r15	NULL,

um-Uni-Directional-DL-r15	NULL

}

RLC-Config-NB-v1430 ::=	SEQUENCE {

t-Reordering-r14			T-Reordering		OPTIONAL		-- Cond twoHARQ

}

RLC-Config-NB-v17xx ::=	SEQUENCE {

t-ReorderingExt-r17					SetupRelease {T-ReorderingExt-r17}

}

UL-AM-RLC-NB-r13 ::=		SEQUENCE {

t-PollRetransmit-r13		T-PollRetransmit-NB-r13,

maxRetxThreshold-r13		ENUMERATED {t1, t2, t3, t4, t6, t8, t16, t32}

}

DL-AM-RLC-NB-r13 ::=		SEQUENCE {

enableStatusReportSN-Gap-r13	ENUMERATED {true}	OPTIONAL

}

T-PollRetransmit-NB-r13 ::=	ENUMERATED {

ms250,	ms500,	ms1000,	ms2000,	ms3000,	ms4000,

ms6000, ms10000, ms15000, ms25000, ms40000, ms60000,

ms90000, ms120000, ms180000, ms300000-v1530}

-- ASN1STOP

Editor’s Note: Agreement: RAN2 confirm the SI agreement that the value range of the RLC t-Reordering timer will be extended to support IoT NTN. Agreement: On how to extend RLC t-Reordering in IoT NTN, wait for NR NTN agreements and see if they can be reused.

RLC-Config-NB field descriptions

enableStatusReportSN-Gap Indicates that status reporting due to detection of reception failure is enabled, as specified in TS 36.322 [7].

maxRetxThreshold Parameter for RLC AM in TS 36.322 [7]. Value t1 corresponds to 1 retransmission, t2 to 2 retransmissions and so on.

t-PollRetransmit Timer for RLC AM in TS 36.322 [7], in milliseconds. Value msX means X ms, msY means Y ms and so on. E-UTRAN may configure the value msX-v1530 (with suffix) only in TDD mode.

t-Reordering Timer for reordering in TS 36.322 [7], in milliseconds.

t-ReorderingExt Timer for reordering in TS 36.322 [7], in milliseconds. The UE shall use the extended value t-ReorderingExt-r17, if present, and ignore the value signaled by t-Reordering-r14. E-UTRAN may configure t-ReorderingExt only if twoHARQ-ProcessesConfig is set to TRUE.

| Conditional presence | Explanation |
| --- | --- |
| twoHARQ | The field is mandatory present if twoHARQ-ProcessesConfig is set to TRUE. Otherwise, the field is not present and, if previously configured, the timer is released. |

#### –	RLF-TimersAndConstants-NB

The IE RLF-TimersAndConstants-NB contains UE specific timers and constants applicable for UEs in RRC_CONNECTED.

RLF-TimersAndConstants-NB information element

-- ASN1START

RLF-TimersAndConstants-NB-r13 ::=	CHOICE {

release								NULL,

setup								SEQUENCE {

t301-r13							ENUMERATED {

ms2500, ms4000, ms6000, ms10000,

ms15000, ms25000, ms40000, ms60000},

t310-r13							ENUMERATED {

ms0, ms200, ms500, ms1000, ms2000, ms4000, ms8000},

n310-r13							ENUMERATED {

n1, n2, n3, n4, n6, n8, n10, n20},

t311-r13							ENUMERATED {

ms1000, ms3000, ms5000, ms10000, ms15000,

ms20000, ms30000},

n311-r13							ENUMERATED {

n1, n2, n3, n4, n5, n6, n8, n10},

...,

[[ t311-v1350						ENUMERATED {

ms40000, ms60000, ms90000, ms120000}

OPTIONAL	-- Need OR

]],

[[	t301-v1530						ENUMERATED {

ms80000, ms100000, ms120000}

OPTIONAL,	-- Cond TDD

t311-v1530						ENUMERATED {

ms160000, ms200000}

OPTIONAL	-- Cond TDD

]]

}

}

-- ASN1STOP

RLF-TimersAndConstants-NB field descriptions

n3xy Constants are described in clause 7.4. n1 corresponds with 1, n2 corresponds with 2 and so on.

t3xy Timers are described in clause 7.3. Value ms0 corresponds with 0 ms, ms200 corresponds with 200 ms and so on. The UE shall use the extended values t311-v1350, t301-v1530 and t311-v1530, if present, and ignore the value signaled by t311-r13, t301-r13 and t311-r13 respectively.

| Conditional presence | Explanation |
| --- | --- |
| TDD | The field is optionally present, Need OR, in TDD mode. Otherwise, the field is not present. |

#### –	SchedulingRequestConfig-NB

The IE SchedulingRequestConfig-NB is used to specify the Scheduling Request related parameters.

SchedulingRequestConfig-NB information element

-- ASN1START

SchedulingRequestConfig-NB-r15 ::=	SEQUENCE {

sr-WithHARQ-ACK-Config-r15			ENUMERATED {true}	OPTIONAL,

sr-WithoutHARQ-ACK-Config-r15			SR-WithoutHARQ-ACK-Config-NB-r15	OPTIONAL,	-- Need ON

sr-SPS-BSR-Config-r15				SR-SPS-BSR-Config-NB-r15			OPTIONAL,	-- Need ON

...

}

SR-WithoutHARQ-ACK-Config-NB-r15 ::= CHOICE {

release								NULL,

setup								SEQUENCE {

sr-ProhibitTimer-r15				INTEGER (0..7)	OPTIONAL,	-- Need ON

sr-NPRACH-Resource-r15				SR-NPRACH-Resource-NB-r15	OPTIONAL -- Need ON

}

}

SR-NPRACH-Resource-NB-r15		::=	SEQUENCE {

nprach-CarrierIndex-r15				INTEGER (0..maxNonAnchorCarriers-NB-r14),

nprach-ResourceIndex-r15			INTEGER (1..maxNPRACH-Resources-NB-r13),

nprach-SubCarrierIndex-r15			CHOICE {

nprach-Fmt0Fmt1-r15					INTEGER (0..47),

nprach-Fmt2-r15						INTEGER (0..143)

},

p0-SR-r15							INTEGER (-126..24),

alpha-r15							ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}}

SR-SPS-BSR-Config-NB-r15	 ::= CHOICE {

release								NULL,

setup								SEQUENCE {

semiPersistSchedC-RNTI-r15			C-RNTI,

semiPersistSchedIntervalUL-r15		ENUMERATED {sf128, sf256, sf512, sf1024,

sf1280, sf2048, sf2560, sf5120}

}

}

-- ASN1STOP

Editor’s Note: Agreement: RAN2 assumes that sr-ProhibitTimer need to be extended. Postpone treatment of sr-ProhibitTimer values until the NR NTN details have been decided.

SchedulingRequestConfig-NB field descriptions

alpha Parameter: αc. Fractional power control parameter for SR without HARQ-ACK. See TS 36.213 [23], clause 16.2.1.2.1, where value al0 corresponds to 0, value al04 corresponds to 0.4, value al05 to 0.5, value al06 to 0.6, value al07 to 0.7, value al08 to 0.8, value al09 to 0.9 and value al1 corresponds to 1.

nprach-CarrierIndex Index of the carrier in the list of UL non anchor carriers in SystemInformationBlockType22-NB. The first entry in the list has index '1', the second entry has index '2' and so on. Value '0' indicates the anchor carrier.

nprach-ResourceIndex Index of the NPRACH resource in the list of NPRACH resources in NPRACH-ParametersList or NPRACH-ParametersList-Fmt2 for the UL carrier indicated by nprach-CarrierIndex. The first entry in the list has index '1', the second entry has index '2' and so on. E-UTRAN configures a NPRACH resource in NPRACH-ParametersList-Fmt2 only to UEs that have reported support of NPRACH resource Format2.

nprach-SubCarrierIndex Index of the subcarrier in the NPRACH resource in NPRACH-ParametersList or or NPRACH-ParametersList-Fmt2 for the indicated UL carrier. E-UTRAN does not configure nprach-SubcarrierIndex to a smaller value than nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers for the indicated NPRACH resource.

p0-SR Parameter:. Target power for SR without HARQ-ACK. See TS 36.213 [23], clause 16.2.1.2.1, unit dBm.

semiPersistSchedC-RNTI Semi-persistent Scheduling C-RNTI, see TS 36.321 [6].

semiPersistSchedIntervalUL Semi-persistent scheduling interval in uplink, see TS 36.321 [6]. Value in number of sub-frames. Value sf128 corresponds to 128 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

sr-SPS-BSR-Config Activation of SR with SPS BSR, see TS 36.321 [6]. E-UTRAN cannot configure sr-SPS-BSR together with sr-WithoutHARQ-ACK-Config.

sr-NPRACH-Resource NPRACH resource for physical layer SR without HARQ-ACK, see TS 36.211 [21] and TS 36.213 [23].

sr-ProhibitTimer Timer for SR transmission on the NPRACH resource for SR in TS 36.321 [6]. Value in number of SR period, where the SR period is equal to the field nprach-Periodicity of the NPRACH resource. Value 0 means that behaviour as specified in 7.3.2 applies. Value 1 corresponds to one SR period, Value 2 corresponds to 2*SR period and so on.

sr-WithHARQ-ACK-Config Activation of physical layer SR with HARQ ACK, see TS 36.213 [23].

sr-WithoutHARQ-ACK-Config Activation of physical layer SR without HARQ ACK, see TS 36.211 [21] and TS 36.213 [23]. E-UTRAN cannot configure sr-WithoutHARQ-ACK-Config together with sr-SPS-BSR.

#### –	TDD-Config-NB

The IE TDD-Config-NB is used to specify the TDD specific physical channel configuration.

TDD-Config information element

-- ASN1START

TDD-Config-NB-r15 ::=					SEQUENCE {

subframeAssignment-r15					ENUMERATED {

sa1, sa2, sa3, sa4, sa5},

specialSubframePatterns-r15				ENUMERATED {

ssp0, ssp1, ssp2, ssp3, ssp4, ssp5, ssp6, ssp7,

ssp8, ssp9, ssp10, ssp10-CRS-LessDwPTS}

}

-- ASN1STOP

TDD-Config field descriptions

specialSubframePatterns Indicates Configuration as in TS 36.211 [21], table 4.2-1 where ssp0 points to Configuration 0, ssp1 to Configuration 1 etc. Value ssp10-CRS-LessDwPTS corresponds to ssp10 without CRS transmission on the 5th symbol of DwPTS.

subframeAssignment Indicates DL/UL subframe configuration where sa1 points to Configuration1, sa2 to Configuration 2 and so on, as specified in TS 36.211 [21], table 4.2-2. E-UTRAN configures the same value for serving cells residing on same frequency band.

#### –	TDD-UL-DL-AlignmentOffset-NB

The IE TDD-UL-DL-AlignmentOffset-NB is used to specify the offset between the UL carrier frequency center with respect to DL carrier frequency center. This information should be used to calculate the Mul value, see TS 36.101 [42].

TDD-UL-DL-AlignmentOffset-NB information element

-- ASN1START

TDD-UL-DL-AlignmentOffset-NB-r15 ::=				ENUMERATED {	khz-7dot5, khz0, khz7dot5}

-- ASN1STOP

#### –	UplinkPowerControl-NB

The IE UplinkPowerControlCommon-NB and IE UplinkPowerControlDedicated-NB are used to specify parameters for uplink power control in the system information and in the dedicated signalling, respectively.

UplinkPowerControl-NB information elements

-- ASN1START

UplinkPowerControlCommon-NB-r13 ::=	SEQUENCE {

p0-NominalNPUSCH-r13				INTEGER (-126..24),

alpha-r13							ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1},

deltaPreambleMsg3-r13				INTEGER (-1..6)

}

UplinkPowerControlDedicated-NB-r13 ::=	SEQUENCE {

p0-UE-NPUSCH-r13						INTEGER (-8..7)

}

-- ASN1STOP

UplinkPowerControl-NB field descriptions

alpha Parameter: αc(1). See TS 36.213 [23], clause 16.2.1.1, where al0 corresponds to 0, al04 corresponds to value 0.4, al05 to 0.5, al06 to 0.6, al07 to 0.7, al08 to 0.8, al09 to 0.9 and al1 corresponds to 1.

deltaPreambleMsg3 Parameter: . See TS 36.213 [23], clause 16.2.1.1. Actual value = IE value * 2 [dB].

p0-NominalNPUSCH Parameter: . See TS 36.213 [23], clause 16.2.1.1, unit dBm.

p0-UE-NPUSCH Parameter: . See TS 36.213 [23], clause 16.2.1.1, unit dB.

#### –	WUS-Config-NB

The IE WUS-Config-NB is used to specify the WUS configuration. For UEs supporting WUS, E-UTRAN uses WUS to indicate that the UE shall attempt to receive paging in that cell, see TS 36.304 [4].

WUS-Config-NB information element

-- ASN1START

WUS-Config-NB-r15 ::=			SEQUENCE {

maxDurationFactor-r15			WUS-MaxDurationFactor-NB-r15,

numPOs-r15						ENUMERATED {n1, n2, n4}		DEFAULT n1,

numDRX-CyclesRelaxed-r15			ENUMERATED {n1, n2, n4, n8},

timeOffsetDRX-r15				ENUMERATED {ms40, ms80, ms160, ms240},

timeOffset-eDRX-Short-r15		ENUMERATED {ms40, ms80, ms160, ms240},

timeOffset-eDRX-Long-r15		ENUMERATED {ms1000, ms2000}	OPTIONAL,	-- Need OP

...

}

WUS-ConfigPerCarrier-NB-r15 ::=	SEQUENCE {

maxDurationFactor-r15			WUS-MaxDurationFactor-NB-r15

}

WUS-MaxDurationFactor-NB-r15 ::= ENUMERATED {one128th, one64th, one32th, one16th,

oneEighth, oneQuarter, oneHalf}

-- ASN1STOP

WUS-Config-NB field descriptions

maxDurationFactor Maximum WUS duration, expressed as a ratio of Rmax for Type 1-CSS. Value one128th means Rmax * 1/128, value one64th means Rmax * 1/64 and so on. The value LNWUS_max in TS 36.213 [23] considered by the UE is : maxDuration = Max (signalled value * Rmax, 1) where Rmax is the value of npdcch-NumRepetitionPaging for the carrier.

numDRX-CyclesRelaxed Maximum number of consecutive DRX cycles during which the UE may use WUS for synchronisation and skip serving cell measurements, see TS 36.133 [16]. Value n1 corresponds to 1 DRX cycle, value n2 corresponds to 2 DRX cycles and so on.

numPOs Number of consecutive Paging Occasions (PO) mapped to one Wake Up Signal (WUS), applicable to UEs configured to use extended DRX, see TS 36.304 [4]. Value n1 corresponds to 1 PO and value n2 corresponds to 2 POs and so on.

timeOffsetDRX When DRX is used, non-zero gap from the end of the configured maximum WUS duration to the associated PO, see TS 36.304 [4], clause 7.4 and TS 36.211 [21]. In milliseconds. Value ms40 corresponds to 40ms, value ms80 corresponds to 80 ms and so on.

timeOffset-eDRX-Short When eDRX is used, the short non-zero gap from the end of the configured maximum WUS duration to the associated PO, see TS 36.304 [4], clause 7.4 and TS 36.211 [21]. In milliseconds. Value ms40 corresponds to 40ms, value ms80 corresponds to 80 ms and so on. E-UTRAN configures timeOffset-eDRX-Short to a value longer than or equal to timeOffsetDRX.

timeOffset-eDRX-Long When eDRX is used, the long non-zero gap from the end of the configured maximum WUS duration to the associated PO, see TS 36.304 [4], clause 7.4 and TS 36.211 [21]. In milliseconds. Value ms1000 corresponds to 1000 ms, value ms2000 corresponds to 2000 ms.

#### 6.7.3.3	NB-IoT Security control information elements

Void

#### 6.7.3.4	NB-IoT Mobility control information elements

#### –	AdditionalBandInfoList-NB

AdditionalBandInfoList-NB information element

-- ASN1START

AdditionalBandInfoList-NB-r14 ::=	SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicator-NB-r13

-- ASN1STOP

#### –	FreqBandIndicator-NB

The IE FreqBandIndicator-NB indicates the E-UTRA operating band as defined in TS 36.101 [42], table 5.5-1.

FreqBandIndicator-NB information element

-- ASN1START

FreqBandIndicator-NB-r13 ::=			INTEGER (1.. maxFBI2)

-- ASN1STOP

#### –	MultiBandInfoList-NB

MultiBandInfoList-NB information element

-- ASN1START

MultiBandInfoList-NB-r13 ::=	SEQUENCE (SIZE (1..maxMultiBands)) OF MultiBandInfo-NB-r13

MultiBandInfo-NB-r13 ::=		SEQUENCE {

freqBandIndicator-r13			FreqBandIndicator-NB-r13		OPTIONAL,	-- Need OR

freqBandInfo-r13				NS-PmaxList-NB-r13				OPTIONAL	-- Need OR

}

-- ASN1STOP

#### –	NS-PmaxList-NB

The IE NS-PmaxList-NB concerns a list of additionalPmax and additionalSpectrumEmission as defined in TS 36.101 [42], clause 6.2.4F, for a given frequency band. E-UTRAN does not include the same value of additionalSpectrumEmission in SystemInformationBlockType2-NB within this list.

NS-PmaxList-NB information element

-- ASN1START

NS-PmaxList-NB-r13 ::=			SEQUENCE (SIZE (1..maxNS-Pmax-NB-r13)) OF NS-PmaxValue-NB-r13

NS-PmaxValue-NB-r13 ::=			SEQUENCE {

additionalPmax-r13				P-Max						OPTIONAL,	-- Need OR

additionalSpectrumEmission-r13	AdditionalSpectrumEmission

}

-- ASN1STOP

#### –	ReselectionThreshold-NB

The IE ReselectionThreshold-NB is used to indicate an Rx level threshold for cell reselection. Actual value of threshold = field value * 2 [dB].

ReselectionThreshold-NB information element

-- ASN1START

ReselectionThreshold-NB-v1360 ::=			INTEGER (32..63)

-- ASN1STOP

#### –	T-Reselection-NB

The IE T-Reselection-NB concerns the cell reselection timer TreselectionRAT for NB-IoT.

Value in seconds. s0 means 0 second and behaviour as specified in 7.3.2 applies, s3 means 3 seconds and so on.

T-Reselection-NB information element

-- ASN1START

T-Reselection-NB-r13 ::=		ENUMERATED {s0, s3, s6, s9, s12, s15, s18, s21}

-- ASN1STOP

#### 6.7.3.5	NB-IoT Measurement information elements

#### –	ANR-MeasConfig-NB

The IE ANR-MeasConfig-NB is used to convey the configuration of the measurements to be performed by the UE in RRC_IDLE for ANR.

ANR-MeasConfig-NB information element

-- ASN1START

ANR-MeasConfig-NB-r16 ::= SEQUENCE {

anr-QualityThreshold-r16		NRSRP-Range-NB-r14,

anr-CarrierList-r16				ANR-CarrierList-NB-r16,

...

}

ANR-CarrierList-NB-r16 ::=		SEQUENCE (SIZE (1..maxFreqANR-NB-r16)) OF ANR-Carrier-NB-r16

ANR-Carrier-NB-r16::=			SEQUENCE {

carrierFreqIndex-r16			INTEGER (1..maxFreq),

blackCellList-r16				ANR-BlackCellList-NB-r16	OPTIONAL,		-- Need OP

...

}

ANR-BlackCellList-NB-r16 ::=	SEQUENCE (SIZE (1..maxCellBlack)) OF PhysCellId

-- ASN1STOP

ANR-MeasConfig-NB field descriptions

anr-CarrierList List of NB-IoT carriers to be measured for ANR.

anr-QualityThreshold Indicates the quality threshold for reporting the CGI of the strongest cell.

BlackCellList List of blacklisted neighbouring cells for ANR reporting.

carrierFreqIndex Index of the carrier frequency in interFreqCarrierFreqList in SystemInformationBlockType5-NB.

#### –	ANR-MeasReport-NB

The IE ANR-MeasReport-NB includes the ANR measurements information.

ANR-MeasReport-NB information element

-- ASN1START

ANR-MeasReport-NB-r16 ::=	SEQUENCE {

servCellIdentity-r16			CellGlobalIdEUTRA			OPTIONAL,

measResultServCell-r16			MeasResultServCell-NB-r14,

relativeTimeStamp-r16			INTEGER (0..95),

measResultList-r16					SEQUENCE (SIZE (1..maxFreqANR-NB-r16)) OF ANR-MeasResult-NB-r16,

...

}

ANR-MeasResult-NB-r16 ::=	SEQUENCE {

carrierFreq-r16						CarrierFreq-NB-r13,

physCellId-r16						PhysCellId					OPTIONAL,

measResultLastServCell-r16			MeasResultServCell-NB-r14,

measResult-r16						NRSRP-Range-NB-r14		OPTIONAL,

cgi-Info-r16						SEQUENCE {

cellGlobalId-r16					CellGlobalIdEUTRA,

trackingAreaCode-r16				TrackingAreaCode,

plmn-IdentityList-r16				PLMN-IdentityList2		OPTIONAL

}	OPTIONAL

}

-- ASN1STOP

ANR-MeasReport-NB field descriptions

carrierFreq Indicates the carrier frequency of the reported cell.

cgi-info Broadcast information of the reported cell.

measResult Measured result of the reported cell.

measResultList List of measured results for the maximum number of reported carrier frequencies.

measResultLastServCell The last measurement results taken in the serving cell when the measured results of the reported cell is stored.

measResultServingCell Measurement results taken in the serving cell when the configuration of the measurements is received.

plmn-IdentityList The list of PLMN Identity read from the broadcast information of the reported cell.

relativeTimeStamp Indicates the time when the ANR measurements are complete, measured relative to the time when the configuration of the measurements was received. Value in hours.

servingCellIdentity Indicates the cell where the measurement configuration was received. If the field is absent, it is the same as the current serving cell.

#### –	CQI-NPDCCH-NB

The IE CQI-NPDCCH-NB represents the downlink channel quality measurement of the NB-IoT carrier where the random access response is received. The codepoints for the CQI-NPDCCH measurements are according to the mapping table in TS 36.133 [16]. The value noMeasurements indicates no measurement reporting.

CQI-NPDCCH-NB information element

-- ASN1START

CQI-NPDCCH-NB-r14 ::=	ENUMERATED {

noMeasurements, candidateRep-A, candidateRep-B, candidateRep-C,

candidateRep-D, candidateRep-E, candidateRep-F, candidateRep-G,

candidateRep-H, candidateRep-I, candidateRep-J, candidateRep-K,

candidateRep-L}

-- ASN1STOP

#### –	CQI-NPDCCH-Short-NB

The IE CQI-NPDCCH-Short-NB represents the short version of the downlink channel quality measurement of the NB-IoT carrier where the random access response is received. The codepoints for the CQI-NPDCCH-Short measurements are according to the mapping table in TS 36.133 [16]. The value noMeasurements indicates no measurement reporting.

CQI-NPDCCH-Short-NB information element

-- ASN1START

CQI-NPDCCH-Short-NB-r14 ::=	ENUMERATED {

noMeasurements, candidateRep-1, candidateRep-2, candidateRep-3}

-- ASN1STOP

#### –	MeasResultServCell-NB

The IE MeasResultServCell-NB covers the measured results for the serving cell.

MeasResultServCell-NB information element

-- ASN1START

MeasResultServCell-NB-r14 ::=	SEQUENCE {

nrsrpResult-r14					NRSRP-Range-NB-r14,

nrsrqResult-r14					NRSRQ-Range-NB-r14

}

-- ASN1STOP

#### –	NRSRP-Range-NB

The IE NRSRP-Range-NB specifies the value range used in NRSRP measurements and thresholds. Integer value for NRSRP measurements according to mapping table in TS 36.133 [16], Table 9.1.22.9-1.

NRSRP-Range-NB information element

-- ASN1START

NRSRP-Range-NB-r14 ::=				INTEGER(0..113)

-- ASN1STOP

#### –	NRSRQ-Range-NB

The IE NRSRQ-Range-NB specifies the value range used in NRSRQ measurements and thresholds. Integer value for RSRQ measurements is according to mapping table in TS 36.133 [16], Table 9.1.22.14-1. The UE shall not report values 0 and 34.

NRSRQ-Range-NB information element

-- ASN1START

NRSRQ-Range-NB-r14 ::=				INTEGER(-30..46)

-- ASN1STOP

#### –	NSSS-RRM-Config-NB

The IE NSSS-RRM-Config-NB provides the configuration for NSSS-based RRM measurements. See TS 36.133 [16], TS 36.211 [21] and TS 36.214 [48]. The UE only perfoms NSSS-based RRM measurement on cells for which the configuration has been provided.

NSSS-RRM-Config-NB information element

-- ASN1START

NSSS-RRM-Config-NB-r15	::=				SEQUENCE {

nsss-RRM-PowerOffset-r15			ENUMERATED {dB-3, db0, dB3},

nsss-NumOccDiffPrecoders-r15		ENUMERATED {n1, n2, n4, n8}	OPTIONAL	--	Need OP

}

-- ASN1STOP

NSSS-RRM-Config-NB field descriptions

nsss-RRM-PowerOffset NSSS to NRS ratio for the serving cell as specified in TS 36.214 [48]. Value in dB. Value dB-3 corresponds to -3 dB, dB0 corresponds to 0 dB and so on.

nsss-NumOccDiffPrecoders Number of consecutive NSSS occasions that use different precoders for NSSS transmission.See TS 36.211 [21]. Value n1 corresponds to 1 occasion, n2 corresponds to 2 occasions and so on. For value n2, n4, and n8, UE may assume for nsss-NumOccDiffPrecoders consecutive NSSS occasions, E-UTRAN uses different precoders for NSSS transmission. For value n1, UE may assume that E-UTRAN always uses the same precoder. If the field is absent, the UE makes no assumption on the antenna port(s) used for NSSS.

#### 6.7.3.6	NB-IoT Other information elements

#### –	EstablishmentCause-NB

The IE EstablishmentCause-NB provides the establishment cause for the RRC connection request or the RRC connection resume request as provided by the upper layers.

EstablishmentCause-NB information element

-- ASN1START

EstablishmentCause-NB-r13 ::=			ENUMERATED {

mt-Access, mo-Signalling, mo-Data, mo-ExceptionData,

delayTolerantAccess-v1330, mt-EDT-v1610, spare2, spare1}

-- ASN1STOP

#### –	UE-Capability-NB

The IE UE-Capability-NB is used to convey the NB-IoT UE Radio Access Capability Parameters, see TS 36.306 [5]. The IE UE-Capability-NB is transferred in NB-IoT only.

UE-Capability-NB information element

-- ASN1START

UE-Capability-NB-r13 ::=		SEQUENCE {

accessStratumRelease-r13		AccessStratumRelease-NB-r13,

ue-Category-NB-r13				ENUMERATED {nb1}					OPTIONAL,

multipleDRB-r13					ENUMERATED {supported}				OPTIONAL,

pdcp-Parameters-r13				PDCP-Parameters-NB-r13				OPTIONAL,

phyLayerParameters-r13			PhyLayerParameters-NB-r13,

rf-Parameters-r13				RF-Parameters-NB-r13,

dummy							SEQUENCE {}							OPTIONAL

}

UE-Capability-NB-Ext-r14-IEs ::=		SEQUENCE {

ue-Category-NB-r14					ENUMERATED {nb2}				OPTIONAL,

mac-Parameters-r14					MAC-Parameters-NB-r14			OPTIONAL,

phyLayerParameters-v1430			PhyLayerParameters-NB-v1430		OPTIONAL,

rf-Parameters-v1430					RF-Parameters-NB-v1430,

nonCriticalExtension				UE-Capability-NB-v1440-IEs		OPTIONAL

}

UE-Capability-NB-v1440-IEs ::=		SEQUENCE {

phyLayerParameters-v1440			PhyLayerParameters-NB-v1440		OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v14x0-IEs		OPTIONAL

}

UE-Capability-NB-v14x0-IEs ::=		SEQUENCE {

-- Following field is only to be used for late REL-14 extensions

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v1530-IEs		OPTIONAL

}

UE-Capability-NB-v1530-IEs ::=		SEQUENCE {

earlyData-UP-r15					ENUMERATED {supported}			OPTIONAL,

rlc-Parameters-r15					RLC-Parameters-NB-r15,

mac-Parameters-v1530				MAC-Parameters-NB-v1530,

phyLayerParameters-v1530			PhyLayerParameters-NB-v1530		OPTIONAL,

tdd-UE-Capability-r15				TDD-UE-Capability-NB-r15		OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v15x0-IEs		OPTIONAL

}

UE-Capability-NB-v15x0-IEs ::=		SEQUENCE {

-- Following field is only to be used for late REL-15 extensions

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v1610-IEs		OPTIONAL

}

UE-Capability-NB-v1610-IEs ::=		SEQUENCE {

earlySecurityReactivation-r16		ENUMERATED {supported}			OPTIONAL,

earlyData-UP-5GC-r16				ENUMERATED {supported}			OPTIONAL,

pur-Parameters-r16					PUR-Parameters-NB-r16			OPTIONAL,

mac-Parameters-v1610				MAC-Parameters-NB-v1610,

phyLayerParameters-v1610			PhyLayerParameters-NB-v1610		OPTIONAL,

son-Parameters-r16					SON-Parameters-NB-r16		OPTIONAL,

meas-Parameters-r16					Meas-Parameters-NB-r16,

tdd-UE-Capability-v1610				TDD-UE-Capability-NB-v1610		OPTIONAL,

nonCriticalExtension				SEQUENCE	{}					OPTIONAL

}

TDD-UE-Capability-NB-r15 ::=		SEQUENCE {

ue-Category-NB-r15					ENUMERATED {nb2}				OPTIONAL,

phyLayerParametersRel13-r15			PhyLayerParameters-NB-r13		OPTIONAL,

phyLayerParametersRel14-r15			PhyLayerParameters-NB-v1430		OPTIONAL,

phyLayerParameters-v1530			PhyLayerParameters-NB-v1530		OPTIONAL,

...

}

TDD-UE-Capability-NB-v1610 ::=		SEQUENCE {

slotSymbolResourceResvDL-r16			ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvUL-r16			ENUMERATED {supported}			OPTIONAL,

subframeResourceResvDL-r16				ENUMERATED {supported}			OPTIONAL,

subframeResourceResvUL-r16			ENUMERATED {supported}			OPTIONAL

}

AccessStratumRelease-NB-r13 ::=		ENUMERATED {rel13, rel14, rel15, rel16, spare4, spare3, spare2, spare1, ...}

PDCP-Parameters-NB-r13		::= SEQUENCE {

supportedROHC-Profiles-r13			SEQUENCE {

profile0x0002						BOOLEAN,

profile0x0003						BOOLEAN,

profile0x0004						BOOLEAN,

profile0x0006						BOOLEAN,

profile0x0102						BOOLEAN,

profile0x0103						BOOLEAN,

profile0x0104						BOOLEAN

},

maxNumberROHC-ContextSessions-r13	ENUMERATED {cs2, cs4, cs8, cs12}	DEFAULT cs2,

...

}

RLC-Parameters-NB-r15		::=		SEQUENCE {

rlc-UM-r15							ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-NB-r14		::=		SEQUENCE {

dataInactMon-r14					ENUMERATED {supported}					OPTIONAL,

rai-Support-r14						ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-NB-v1530		::=		SEQUENCE {

sr-SPS-BSR-r15						ENUMERATED {supported}			OPTIONAL

}

MAC-Parameters-NB-v1610		::=		SEQUENCE {

rai-SupportEnh-r16					ENUMERATED {supported}			OPTIONAL

}

Meas-Parameters-NB-r16		::=		SEQUENCE {

dl-ChannelQualityReporting-r16		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-r13	::=		SEQUENCE {

multiTone-r13						ENUMERATED {supported}			OPTIONAL,

multiCarrier-r13						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1430	::=		SEQUENCE {

multiCarrier-NPRACH-r14				ENUMERATED {supported}			OPTIONAL,

twoHARQ-Processes-r14				ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1440	::=		SEQUENCE {

interferenceRandomisation-r14		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1530	::=		SEQUENCE {

mixedOperationMode-r15				ENUMERATED {supported}			OPTIONAL,

sr-WithHARQ-ACK-r15					ENUMERATED {supported}			OPTIONAL,

sr-WithoutHARQ-ACK-r15				ENUMERATED {supported}			OPTIONAL,

nprach-Format2-r15					ENUMERATED {supported}			OPTIONAL,

additionalTransmissionSIB1-r15		ENUMERATED {supported}			OPTIONAL,

npusch-3dot75kHz-SCS-TDD-r15		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1610	::=		SEQUENCE {

npdsch-MultiTB-r16					ENUMERATED {supported}			OPTIONAL,

npdsch-MultiTB-Interleaving-r16		ENUMERATED {supported}			OPTIONAL,

npusch-MultiTB-r16					ENUMERATED {supported}			OPTIONAL,

npusch-MultiTB-Interleaving-r16		ENUMERATED {supported}			OPTIONAL,

multiTB-HARQ-AckBundling-r16		ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvDL-r16			ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvUL-r16			ENUMERATED {supported}			OPTIONAL,

subframeResourceResvDL-r16				ENUMERATED {supported}			OPTIONAL,

subframeResourceResvUL-r16			ENUMERATED {supported}			OPTIONAL

}

PUR-Parameters-NB-r16	::=			SEQUENCE {

pur-CP-EPC-r16						ENUMERATED {supported}			OPTIONAL,

pur-CP-5GC-r16						ENUMERATED {supported}			OPTIONAL,

pur-UP-EPC-r16						ENUMERATED {supported}			OPTIONAL,

pur-UP-5GC-r16						ENUMERATED {supported}			OPTIONAL,

pur-NRSRP-Validation-r16			ENUMERATED {supported}			OPTIONAL,

pur-CP-L1Ack-r16					ENUMERATED {supported}			OPTIONAL

}

RF-Parameters-NB-r13	::=			SEQUENCE {

supportedBandList-r13				SupportedBandList-NB-r13,

multiNS-Pmax-r13					ENUMERATED {supported}		OPTIONAL

}

RF-Parameters-NB-v1430 ::=			SEQUENCE {

powerClassNB-14dBm-r14				ENUMERATED {supported}		OPTIONAL

}

SupportedBandList-NB-r13 ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBand-NB-r13

SupportedBand-NB-r13	::=			SEQUENCE {

band-r13							FreqBandIndicator-NB-r13,

powerClassNB-20dBm-r13				ENUMERATED {supported}		OPTIONAL

}

SON-Parameters-NB-r16 ::=			SEQUENCE {

anr-Report-r16						ENUMERATED {supported}		OPTIONAL,

rach-Report-r16						ENUMERATED {supported}		OPTIONAL

}

-- ASN1STOP

| UE-Capability-NB field descriptions | FDD/TDD appl | FDD/TDD diff |
| --- | --- | --- |
| accessStratumRelease Set to rel16 in this version of the specification. | FDD/TDD | No |
| additionalTransmissionSIB1 Indicates whether the UE supports additional SIB1 transmission as specified in TS 36.213 [23]. | FDD | - |
| anr-Report Indicates whether the UE supports ANR measurements in RRC_IDLE. | FDD/TDD | No |
| dataInactMon Indicates whether the UE supports the data inactivity monitoring as specified in TS 36.321 [6]. | FDD/TDD | No |
| dl-ChannelQualityReporting-r16 Indicates whether the UE supports DL channel quality reporting in connected mode as specified in TS 36.321 [6]. | FDD | - |
| dummy This field is not used in the specification. It shall not be sent by the UE. | NA | NA |
| earlyData-UP, earlyData-UP-5GC Indicates whether the UE supports EDT for User plane CIoT EPS/5GS optimisations, as defined in TS 24.301 [35] and 24.501 [95] respectively. | FDD | - |
| earlySecurityReactivation Indicates whether the UE supports early security reactivation when resuming a suspended RRC connection. | FDD/TDD | No |
| interferenceRandomisation For FDD: Indicates whether the UE supports interference randomisation in connected mode as defined in TS.36.211 [21]. | FDD | - |
| maxNumberROHC-ContextSessions Set to the maximum number of concurrently active ROHC contexts supported by the UE, excluding context sessions that leave all headers uncompressed. cs2 corresponds with 2 (context sessions), cs4 corresponds with 4 and so on. The network ignores this field if the UE supports none of the ROHC profiles in supportedROHC-Profiles. | FDD/TDD | No |
| mixedOperationMode Defines whether the UE supports multi-carrier operation with mixed operation mode, standalone or inband/guardband, between the anchor carrier and the non-anchor carrier for unicast, paging, and random access as specified in TS 36.300 [9]. | FDD | - |
| multiCarrier Defines whether the UE supports multi -carrier operation. | FDD/TDD | Yes |
| multicarrier-NPRACH Defines whether the UE supports NPRACH on non-anchor carrier as specified in TS 36.321 [6]. | FDD/TDD | Yes |
| multipleDRB Defines whether the UE supports multiple DRBs. | FDD/TDD | No |
| multiNS-Pmax Defines whether the UE supports the mechanisms defined for NB-IoT cells broadcasting NS-PmaxList-NB. | FDD/TDD | No |
| multiTB-HARQ-AckBundling Indicates whether the UE supports HARQ ACK bundling for interleaved transmission for DL. If multiTB-HARQ-AckBundling is included, the UE shall also indicate support for npdsch-MultiTB-Interleaving. | FDD | - |
| multiTone Defines whether the UE supports UL multi-tone transmissions on NPUSCH. | FDD/TDD | Yes |
| npdsch-MultiTB Indicates whether the UE supports multiple TBs scheduling in RRC_CONNECTED for DL. If npdsch-MultiTB is included, the UE shall also indicate support for twoHARQ-Processes. | FDD | - |
| npdsch-MultiTB-Interleaving Indicates whether the UE supports interleaved transmission when multiple TBs is scheduled in RRC_CONNECTED for DL. | FDD | - |
| nprach-Format2 Defines whether the UE supports NPRACH resources using preamble format 2. | FDD | - |
| npusch-3dot75kHz-SCS-TDD Indicates whether the UE supports NPUSCH with 3.75kHz SCS for TDD. | TDD | - |
| npusch-MultiTB Indicates whether the UE supports multiple TBs scheduling in RRC_CONNECTED for UL. If npusch-MultiTB is included, the UE shall also indicate support for twoHARQ-Processes. | FDD | - |
| npusch-MultiTB-Interleaving Indicates whether the UE supports interleaved transmission when multiple TBs is scheduled in RRC_CONNECTED for UL. | FDD | - |
| powerClassNB-14dBm Defines whether the UE supports power class 14dBm in all the bands supported by the UE as specified in TS 36.101 [42]. If powerClassNB-20dBm is included, the UE shall not include the field powerClassNB-14dBm. | FDD/TDD | No |
| powerClassNB-20dBm Defines whether the UE supports power class 20dBm in NB-IoT for the band, as specified in TS 36.101 [42]. If neither powerClassNB-14dBm nor powerClassNB-20dBm is included, UE supports power class 23 dBm in the NB-IoT band. | FDD/TDD | No |
| pur-CP-EPC, pur-CP-5GC Indicates whether the UE supports transmission using PUR for Control plane CIoT EPS/5GS optimisations, as defined in TS 24.301 [35] and TS 24.501 [95] respectively. | FDD | - |
| pur-CP-L1Ack Indicates whether UE supports L1 acknowledgement in response to CP transmission using PUR. If pur-CP-L1Ack is included, the UE shall also indicate support for pur-CP-EPC or pur-CP-5GC. | FDD | - |
| pur-NRSRP-Validation Indicates whether UE supports serving cell NRSRP for TA validation for transmission using PUR. If pur-NRSRP-Validation is included, the UE shall also indicate support for pur-CP-EPC, pur-CP-5GC, pur-UP-EPC or pur-CP-5GC. | FDD | - |
| pur-UP-EPC, pur-UP-5GC Indicates whether the UE supports transmission using PUR for User plane CIoT EPS/5GS optimisations, as defined in TS 24.301 [35] and TS 24.501 [95] repectively. | FDD | - |
| rach-Report Indicates whether the UE supports delivery of rach-Report. | FDD/TDD | No |
| rai-Support Defines whether the UE supports release assistance indication (RAI) as specified in TS 36.321 [6]. | FDD/TDD | No |
| rai-SupportEnh Indicates whether the UE supports AS Release Assistance Indication via the DCQR and AS RAI MAC CE when connected to EPC as specified in TS 36.321 [6]. | FDD/TDD | No |
| rlc-UM Defines whether the UE supports RLC UM as specified in TS 36.322 [7]. | FDD/TDD | No |
| slotSymbolResourceResvDL Indicates whether the UE supports slot/symbol-level time-domain DL resource reservation, e.g. for NB-IoT coexistence with NR. If slotSymbolResourceResvDL is included, the UE shall also indicate support for subframeResourceResvDL. | FDD/TDD | Yes |
| slotSymbolResourceResvUL Indicates whether the UE supports slot/symbol-level time-domain UL resource reservation, e.g. for NB-IoT coexistence with NR. If slotSymbolResourceResvUL is included, the UE shall also indicate support for subframeResourceResvUL. | FDD/TDD | Yes |
| supportedBandList Includes the supported NB-IoT bands as defined in TS 36.101 [42]. | FDD/TDD | No |
| sr-SPS-BSR Defines whether the UE supports SR using SPS BSR as specified in TS 36.321 [6]. | FDD | - |
| sr-withHARQ-ACK Defines whether the UE supports physical layer SR with HARQ ACK as specified in TS 36.213 [23]. | FDD | - |
| sr-withoutHARQ-ACK Defines whether the UE supports physical layer SR without HARQ ACK as specified in TS 36.211 [21] and TS 36.213 [23]. | FDD | - |
| subframeResourceResvDL Indicates whether the UE supports subframe-level time-domain DL resource reservation, e.g. for NB-IoT coexistence with NR. | FDD/TDD | Yes |
| subframeResourceResvUL Indicates whether the UE supports subframe-level time-domain UL resource reservation, e.g. for NB-IoT coexistence with NR. | FDD/TDD | Yes |
| supportedROHC-Profiles List of supported ROHC profiles as defined in TS 36.323 [8]. | FDD/TDD | No |
| twoHARQ-Processes Defines whether the UE supports two HARQ processes operation in DL and UL as specified in TS 36.212 [22] and TS 36.213 [23]. | FDD/TDD | Yes |
| ue-Category-NB UE category as defined in TS 36.306 [5]. Value nb1 corresponds to UE category NB1, value nb2 corresponds to UE category NB2. A UE shall always include the field ue-Category-NB-r13 in this version of the specification. | FDD/TDD | Yes |

NOTE 1:	The IE UE-Capability-NB does not include AS security capability information, since these are the same as the security capabilities that are signalled by NAS. Consequently AS need not provide "man-in-the-middle" protection for the security capabilities.

NOTE 2:	The column 'FDD/TDD appl' indicates the applicability to the xDD mode: 'FDD' means applicable to FDD only, 'TDD' means applicable to TDD only and 'FDD/TDD' means applicable to FDD and TDD.

NOTE 3:	The column 'FDD/TDD diff' indicates if the UE is allowed to signal a different value for FDD and TDD when the capability applies to both FDD and TDD modes. '-' is used when the capability applies to one mode only, 'No' is used for dual mode capabilities where a common value is signalled for both modes, and 'Yes' is used for dual mode capabilities where a separate value is signalled for each mode. Common capabilities and FDD capabilities are reported in the fields of UE-Capability-NB except field tdd-UE-Capability. TDD capabilities are reported in tdd-UE-Capability.

#### –	UE-RadioPagingInfo-NB

The IE UE-RadioPagingInfo-NB contains UE NB-IoT capability information needed for paging.

UE-RadioPagingInfo-NB information element

-- ASN1START

UE-RadioPagingInfo-NB-r13 ::=		SEQUENCE {

ue-Category-NB-r13				ENUMERATED {nb1}			OPTIONAL,

...,

[[ multiCarrierPaging-r14		ENUMERATED {true}			OPTIONAL

]],

[[	mixedOperationMode-r15		ENUMERATED {supported}		OPTIONAL,

wakeUpSignal-r15			ENUMERATED {true}			OPTIONAL,

wakeUpSignalMinGap-eDRX-r15	ENUMERATED {ms40, ms240, ms1000, ms2000}	OPTIONAL,

multiCarrierPagingTDD-r15	ENUMERATED {true}			OPTIONAL

]],

[[	ue-Category-NB-r16					ENUMERATED {nb2}			OPTIONAL,

groupWakeUpSignal-r16				ENUMERATED {true}			OPTIONAL,

groupWakeUpSignalAlternation-r16	ENUMERATED {true}			OPTIONAL

]]

}

-- ASN1STOP

UE-RadioPagingInfo-NB field descriptions

groupWakeUpSignal Indicates whether the UE in RRC_IDLE supports GWUS without group resource alternation for paging in DRX in FDD as specified in TS 36.211 [21], TS 36.213 [23] and TS 36.304 [4]. If this field is included, the minimum gap between GWUS and associated PO for DRX is fixed as 40 ms.

groupWakeUpSignalAlternation Indicates whether the UE in RRC_IDLE supports GWUS with group resource alternation for paging in DRX in FDD as specified in TS 36.211 [21], TS 36.213 [23] and TS 36.304 [4]. If this field is included, the minimum gap between GWUS and associated PO for DRX is fixed as 40 ms.

mixedOperationMode Indicates whether the UE supports multi-carrier operation with mixed operation mode, standalone or inband/guardband, between the anchor carrier and non-anchor carrier for unicast, paging, and random access, as specified in TS 36.300 [9].

multiCarrierPaging Indicates whether the UE supports paging on non-anchor carriers as defined in TS 36.304 [4].

multiCarrierPagingTDD Indicates whether the UE supports paging on non-anchor carriers for TDD as defined in TS 36.304 [4].

ue-Category-NB UE NB-IoT category as defined in TS 36.306 [5]. Value nb1 corresponds to UE category NB1, value nb2 corresponds to UE category NB2. A UE shall always include the field ue-Category-NB-r13 in this version of the specification.

wakeUpSignal Indicates whether the UE supports WUS for paging in DRX in FDD as specified in TS 36.304 [4]. If this field is included, the minimum gap between WUS and associated PO for DRX is fixed as 40 ms.

wakeUpSignalMinGap-eDRX Indicates the minimum gap the UE supports between WUS or GWUS and associated PO in case of eDRX in FDD, as specified in TS 36.304 [4]. Value ms40 corresponds to 40 ms, value ms240 corresponds to 240 ms and so on. If this field is included, the UE shall also indicate support for WUS or GWUS for paging in DRX.

#### –	UE-TimersAndConstants-NB

The IE UE-TimersAndConstants-NB contains timers and constants used by the UE in either RRC_CONNECTED or RRC_IDLE.

UE-TimersAndConstants-NB information element

-- ASN1START

UE-TimersAndConstants-NB-r13 ::=	SEQUENCE {

t300-r13							ENUMERATED {

ms2500, ms4000, ms6000, ms10000,

ms15000, ms25000, ms40000, ms60000},

t301-r13							ENUMERATED {

ms2500, ms4000, ms6000, ms10000,

ms15000, ms25000, ms40000, ms60000},

t310-r13							ENUMERATED {

ms0, ms200, ms500, ms1000, ms2000, ms4000, ms8000},

n310-r13							ENUMERATED {

n1, n2, n3, n4, n6, n8, n10, n20},

t311-r13							ENUMERATED {

ms1000, ms3000, ms5000, ms10000, ms15000,

ms20000, ms30000},

n311-r13							ENUMERATED {

n1, n2, n3, n4, n5, n6, n8, n10},

...,

[[ t311-v1350						ENUMERATED {

ms40000, ms60000, ms90000, ms120000}

OPTIONAL	-- Need OR

]],

[[	t300-v1530						ENUMERATED {

ms80000, ms100000, ms120000}	OPTIONAL,	-- Cond TDD

t301-v1530						ENUMERATED {

ms80000, ms100000, ms120000}	OPTIONAL,	-- Cond TDD

t311-v1530						ENUMERATED {

ms160000, ms200000}				OPTIONAL,	-- Cond TDD

t300-r15						ENUMERATED {ms6000, ms10000, ms15000, ms25000, ms40000,

ms60000, ms80000, ms120000}	OPTIONAL		-- Cond EDTorPUR

]]

}

-- ASN1STOP

UE-TimersAndConstants-NB field descriptions

n3xy Constants are described in clause 7.4. n1 corresponds with 1, n2 corresponds with 2 and so on.

t3xy Timers are described in clause 7.3. Value ms0 corresponds with 0 ms, ms200 corresponds with 200 ms and so on. The UE shall use the extended values t311-v1350, t300-v1530, t301-v1530 and t311-v1530, if present, and ignore the value signaled by t311-r13, t300-r13, t301-r13 and t311-r13 respectively. t300-r15 is only applicable for EDT or transmission using PUR with uplink data. UE performing EDT or transmission using PUR with uplink data shall use t300-r15, if present.

| Conditional presence | Explanation |
| --- | --- |
| EDTorPUR | The field is optionally present, Need OR, if edt-Parameters or cp-PUR-5GC or cp-PUR-EPC or up-PUR-5GC or up-PUR-EPC is present in SIB2-NB; otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD | The field is optionally present, Need OR, in TDD mode. Otherwise, the field is not present. |

#### 6.7.3.7	NB-IoT MBMS information elements

Void

#### 6.7.3.7a	NB-IoT SC-PTM information elements

#### –	SC-MTCH-InfoList-NB

The IE SC-MTCH-InfoList-NB provides the list of ongoing MBMS sessions transmitted via SC-MRB and for each MBMS session, the associated G-RNTI and scheduling information.

SC-MTCH-InfoList-NB information element

-- ASN1START

SC-MTCH-InfoList-NB-r14 ::=			SEQUENCE (SIZE (0.. maxSC-MTCH-NB-r14)) OF SC-MTCH-Info-NB-r14

SC-MTCH-Info-NB-r14 ::=				SEQUENCE	{

sc-mtch-CarrierConfig-r14			CHOICE {

dl-CarrierConfig-r14					DL-CarrierConfigCommon-NB-r14,

dl-CarrierIndex-r14					INTEGER (0.. maxNonAnchorCarriers-NB-r14)

},

mbmsSessionInfo-r14					MBMSSessionInfo-r13,

g-RNTI-r14							BIT STRING(SIZE(16)),

sc-mtch-SchedulingInfo-r14			SC-MTCH-SchedulingInfo-NB-r14		OPTIONAL,	-- Need OP

sc-mtch-NeighbourCell-r14			BIT STRING (SIZE(maxNeighCell-SCPTM-NB-r14))	OPTIONAL,	-- Need OP

npdcch-NPDSCH-MaxTBS-SC-MTCH-r14		ENUMERATED {n680, n2536},

npdcch-NumRepetitions-SC-MTCH-r14	ENUMERATED {r1, r2, r4, r8, r16,

r32, r64, r128, r256,

r512, r1024, r2048, spare4,

spare3, spare2, spare1},

npdcch-StartSF-SC-MTCH-r14			ENUMERATED {v1dot5, v2, v4, v8,

v16, v32, v48, v64},

npdcch-Offset-SC-MTCH-r14			ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth},

...

}

SC-MTCH-SchedulingInfo-NB-r14 ::=		SEQUENCE	{

onDurationTimerSCPTM-r14				ENUMERATED {

pp1, pp2, pp3, pp4,

pp8, pp16, pp32, spare},

drx-InactivityTimerSCPTM-r14			ENUMERATED {

pp0, pp1, pp2, pp3,

pp4, pp8, pp16, pp32},

schedulingPeriodStartOffsetSCPTM-r14	CHOICE {

sf10									INTEGER(0..9),

sf20									INTEGER(0..19),

sf32									INTEGER(0..31),

sf40									INTEGER(0..39),

sf64									INTEGER(0..63),

sf80									INTEGER(0..79),

sf128									INTEGER(0..127),

sf160									INTEGER(0..159),

sf256									INTEGER(0..255),

sf320									INTEGER(0..319),

sf512									INTEGER(0..511),

sf640									INTEGER(0..639),

sf1024									INTEGER(0..1023),

sf2048									INTEGER(0..2047),

sf4096									INTEGER(0..4095),

sf8192									INTEGER(0..8191)

},

...

}

-- ASN1STOP

SC-MTCH-InfoList-NB field descriptions

dl-CarrierConfig Downlink carrier used for SC-MTCH. E-UTRAN cannot configure a downlink carrier operating in mixed operation mode.

dl-CarrierIndex Index to a downlink carrier signalled in system information. Value '0' corresponds to the anchor carrier, value '1' corresponds to the first entry in dl-ConfigList in SystemInformationBlockType22-NB, value '2' corresponds to the second entry in dl-ConfigList and so on.

drx-InactivityTimerSCPTM Timer for SC-MTCH reception in TS 36.321 [6]. Value in number of NPDCCH periods. Value pp1 corresponds to 1 NPDCCH period, pp2 corresponds to 2 NPDCCH periods and so on.

g-RNTI G-RNTI used to scramble the scheduling and transmission of a SC-MTCH.

mbmsSessionInfo Indicates the ongoing MBMS session in a SC-MTCH.

npdcch-NPDSCH-MaxTBS-SC-MTCH Maximum NPDSCH TBS for the SC-MTCH, see TS 36.213 [23]. Value n680 corresponds to 680 bits and value n2536 corresponds to 2536 bits.

npdcch-NumRepetition-SC-MTCH The maximum number of NPDCCH repetitions the UE needs to monitor for SC-MTCH multicast search space, see TS 36.213 [23].

npdcch-Offset-SC-MTCH Fractional period offset of starting subframe for NPDCCH multicast search space for SC-MTCH, see TS 36.213 [23].

npdcch-startSF-SC-MTCH Starting subframes configuration of the NPDCCH multicast search space for SC-MTCH, see TS 36.213 [23].

onDurationTimerSCPTM Timer for SC-MTCH reception in TS 36.321 [6]. Value in number of NPDCCH periods. Value pp1 corresponds to 1 NPDCCH period, pp2 corresponds to 2 NPDCCH periods and so on.

schedulingPeriodStartOffsetSCPTM SCPTM-SchedulingCycle and SCPTM-SchedulingOffset in TS 36.321 [6]. The value of SCPTM-SchedulingCycle is in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. The value of SCPTM-SchedulingOffset is in number of sub-frames.

sc-mtch-CarrierConfig Downlink carrier that is used for SC-MTCH.

sc-mtch-NeighbourCell Indicates neighbour cells which also provide this service on SC-MTCH. The first bit is set to 1 if the service is provided on SC-MTCH in the first cell in scptmNeighbourCellList, otherwise it is set to 0. The second bit is set to 1 if the service is provided on SC-MTCH in the second cell in scptmNeighbourCellList, and so on. If this field is absent, the UE shall assume that this service is not available on SC-MTCH in any neighbour cell.

sc-mtch-SchedulingInfo DRX information for the SC-MTCH. If this field is absent, DRX is not used for the SC-MTCH.

#### –	SCPTM-NeighbourCellList-NB

The IE SCPTM-NeighbourCellList-NB indicates a list of neighbour cells where ongoing MBMS sessions provided via SC-MRB in the current cells are also provided.

-- ASN1START

SCPTM-NeighbourCellList-NB-r14 ::=	SEQUENCE (SIZE (1..maxNeighCell-SCPTM-NB-r14)) OF PCI-ARFCN-NB-r14

PCI-ARFCN-NB-r14 ::=				SEQUENCE {

physCellId-r14						PhysCellId,

carrierFreq-r14						CarrierFreq-NB-r13		OPTIONAL	-- Need OP

}

-- ASN1STOP

SCPTM-NeighbourCellList-NB field descriptions

physCellId Physical Cell Identity of the neighbour cell.

carrierFreq Carrier frequency of the neighbour cell. Absence of the IE means that the neighbour cell is on the same frequency as the current cell.

### 6.7.4	NB-IoT RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

maxFreqANR-NB-r16			INTEGER ::= 2	-- Maximum number of NB-IOT carrier frequencies that can

-- be configured or reported for ANR measurement

maxFreqEUTRA-NB-r16			INTEGER ::= 8	-- Maximum number of EUTRAN carrier frequencies that can

-- be provided as assistance information for inter-RAT

-- cell selection

maxFreqsGERAN-NB-r16		INTEGER ::= 8	-- Maximum number of groups of GERAN carrier frequencies

-- that can be provided as assistance information for

-- inter-RAT cell selection

maxGWUS-Groups-1-NB-r16		INTEGER ::= 15	-- Maximum number of groups for each paging probability

-- group

maxGWUS-Resources-NB-r16	INTEGER ::= 2	-- Maximum number of GWUS resources for each gap

maxGWUS-ProbThresholds-NB-r16 INTEGER ::= 3	-- Maximum number of paging probability thresholds

maxNPRACH-Resources-NB-r13	INTEGER ::=	3	-- Maximum number of NPRACH resources for NB-IoT

maxNonAnchorCarriers-NB-r14	INTEGER ::= 15	-- Maximum number of non-anchor carriers for NB-IoT

maxDRB-NB-r13				INTEGER ::= 2	-- Maximum number of Data Radio Bearers for NB-IoT

maxNeighCell-SCPTM-NB-r14	INTEGER ::= 8	-- Maximum number of SCPTM neighbour cells

maxNS-Pmax-NB-r13			INTEGER ::= 4	-- Maximum number of NS and P-Max values per band

maxSC-MTCH-NB-r14			INTEGER ::= 64	-- Maximum number of SC-MTCHs in one cell for NB-IoT

maxSI-Message-NB-r13		INTEGER ::= 8	-- Maximum number of SI messages for NB-IoT

maxTAC-NB-r17				INTEGER	::=	ffsValue	-- Maximum number of Tracking Area Codes

-- broadcast in a cell

-- ASN1STOP

### –	End of NBIOT-RRC-Definitions

-- ASN1START

END

-- ASN1STOP

### 6.7.5	Direct Indication Information

Direct Indication information is transmitted on NPDCCH using P-RNTI but without associated Paging-NB message. Table 6.7.5-1 defines the Direct Indication information, see TS 36.212 [22], clause 6.4.3.3.

When bit n is set to 1, the UE shall behave as if the corresponding field is set in the Paging-NB message, see 5.3.2.3. Bit 1 is the least significant bit.

Table 6.7.5-1: Direct Indication information

| Bit | Field in Direct Indication information |
| --- | --- |
| 1 | systemInfoModification |
| 2 | systemInfoModification-eDRX |
| 3, 4, 5, 6, 7, 8 | Not used, and shall be ignored by UE if received |

Editor’s Note: Agreement: System information update notification procedure is not used to inform TAC updates, at least for TAC additions (FFS removals).

# 7	Variables and constants

## 7.1	UE variables

NOTE:	To facilitate the specification of the UE behavioural requirements, UE variables are represented using ASN.1. Unless explicitly specified otherwise, it is however up to UE implementation how to store the variables. The optionality of the IEs in ASN.1 is used only to indicate that the values may not always be available.

#### –	EUTRA-UE-Variables

This ASN.1 segment is the start of the E-UTRA UE variable definitions.

-- ASN1START

EUTRA-UE-Variables DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

AbsoluteTimeInfo-r10,

AreaConfiguration-r10,

AreaConfiguration-v1130,

ARFCN-ValueNR-r15,

BT-NameList-r15,

CarrierFreqGERAN,

CellIdentity,

CellList-r15,

CondReconfigurationToAddModList-r16,

ConnEstFailReport-r11,

EUTRA-CarrierList-r15,

SpeedStateScaleFactors,

C-RNTI,

LoggingDuration-r10,

LoggingInterval-r10,

LogMeasInfo-r10,

MeasCSI-RS-Id-r12,

MeasId,

MeasId-v1250,

MeasIdToAddModList,

MeasIdToAddModListExt-r12,

MeasIdToAddModList-v1310,

MeasIdToAddModListExt-v1310,

MeasObjectToAddModList,

MeasObjectToAddModList-v9e0,

MeasObjectToAddModListExt-r13,

MeasResultListExtIdle-r16,

MeasResultListIdle-r15,

MeasResultListIdleNR-r16,

MeasScaleFactor-r12,

MobilityStateParameters,

NeighCellConfig,

NR-CarrierList-r16,

PhysCellId,

PhysCellIdCDMA2000,

PhysCellIdGERAN,

PhysCellIdUTRA-FDD,

PhysCellIdUTRA-TDD,

PLMN-Identity,

PLMN-IdentityList3-r11,

QuantityConfig,

ReportConfigToAddModList,

RLF-Report-r9,

TargetMBSFN-AreaList-r12,

TraceReference-r10,

Tx-ResourcePoolMeasList-r14,

VisitedCellInfoList-r12,

maxCellMeas,

maxCSI-RS-Meas-r12,

maxMeasId,

maxMeasId-r12,

maxRS-Index-r15,

PhysCellIdNR-r15,

RS-IndexNR-r15,

UL-DelayConfig-r13,

ValidityAreaList-r16,

WLAN-CarrierInfo-r13,

WLAN-Identifiers-r12,

WLAN-Id-List-r13,

WLAN-NameList-r15,

WLAN-Status-r13,

WLAN-Status-v1430,

WLAN-SuspendConfig-r14

FROM EUTRA-RRC-Definitions;

-- ASN1STOP

#### –	VarConditionalReconfiguration

The UE variable VarConditionalReconfiguration includes the accumulated configuration of conditional reconfigurations (i.e. conditional handovers) including the configurations of triggering conditions to be monitored and the stored RRCConnectionReconfiguration per target candidate, to be applied upon the fulfilment of the associated triggering conditions.

VarConditionalReconfiguration UE variable

-- ASN1START

VarConditionalReconfiguration ::= SEQUENCE {

-- Conditional reconfigurations list

condReconfigurationList-r16			CondReconfigurationToAddModList-r16

OPTIONAL

}

-- ASN1STOP

#### –	VarConnEstFailReport

The UE variable VarConnEstFailReport includes the connection establishment failure information.

VarConnEstFailReport UE variable

-- ASN1START

VarConnEstFailReport-r11 ::=		SEQUENCE {

connEstFailReport-r11				ConnEstFailReport-r11,

plmn-Identity-r11					PLMN-Identity

}

-- ASN1STOP

#### –	VarLogMeasConfig

The UE variable VarLogMeasConfig includes the configuration of the logging of measurements to be performed by the UE while in RRC_IDLE, covering intra-frequency, inter-frequency, inter-RAT mobility and MBSFN related measurements. If MBSFN logging is configured, the UE performs logging of measurements while in both RRC_IDLE and RRC_CONNECTED. Otherwise, the UE performs logging of measurements only while in RRC_IDLE.

VarLogMeasConfig UE variable

-- ASN1START

VarLogMeasConfig-r10 ::=				SEQUENCE {

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10

}

VarLogMeasConfig-r11 ::=		SEQUENCE {

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,

areaConfiguration-v1130			AreaConfiguration-v1130		OPTIONAL,

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10

}

VarLogMeasConfig-r12 ::=		SEQUENCE {

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,

areaConfiguration-v1130			AreaConfiguration-v1130		OPTIONAL,

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

targetMBSFN-AreaList-r12		TargetMBSFN-AreaList-r12	OPTIONAL

}

VarLogMeasConfig-r15 ::=		SEQUENCE {

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,

areaConfiguration-v1130			AreaConfiguration-v1130		OPTIONAL,

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

targetMBSFN-AreaList-r12			TargetMBSFN-AreaList-r12		OPTIONAL,

bt-NameList-r15				BT-NameList-r15					OPTIONAL,

wlan-NameList-r15				WLAN-NameList-r15					OPTIONAL

}

-- ASN1STOP

#### –	VarLogMeasReport

The UE variable VarLogMeasReport includes the logged measurements information.

VarLogMeasReport UE variable

-- ASN1START

VarLogMeasReport-r10 ::=				SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10			OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-Identity-r10					PLMN-Identity,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

}

VarLogMeasReport-r11 ::=			SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10		OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-IdentityList-r11				PLMN-IdentityList3-r11,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

}

LogMeasInfoList2-r10 ::=				SEQUENCE (SIZE (1..maxLogMeas-r10)) OF LogMeasInfo-r10

-- ASN1STOP

#### –	VarMeasConfig

The UE variable VarMeasConfig includes the accumulated configuration of the measurements to be performed by the UE, covering intra-frequency, inter-frequency and inter-RAT mobility related measurements.

NOTE:	The amount of measurement configuration information, which a UE is required to store, is specified in clause 11.1. If the number of frequencies configured for a particular RAT exceeds the minimum performance requirements specified in TS 36.133 [16], it is up to UE implementation which frequencies of that RAT are measured. If the total number of frequencies for all RATs provided to the UE in the measurement configuration exceeds the minimum performance requirements specified in TS 36.133 [16], it is up to UE implementation which frequencies/RATs are measured.

VarMeasConfig UE variable

-- ASN1START

VarMeasConfig ::=					SEQUENCE {

-- Measurement identities

measIdList							MeasIdToAddModList					OPTIONAL,

measIdListExt-r12					MeasIdToAddModListExt-r12			OPTIONAL,

measIdList-v1310						MeasIdToAddModList-v1310				OPTIONAL,

measIdListExt-v1310					MeasIdToAddModListExt-v1310			OPTIONAL,

-- Measurement objects

measObjectList						MeasObjectToAddModList				OPTIONAL,

measObjectListExt-r13				MeasObjectToAddModListExt-r13		OPTIONAL,

measObjectList-v9i0					MeasObjectToAddModList-v9e0			OPTIONAL,

-- Reporting configurations

reportConfigList					ReportConfigToAddModList			OPTIONAL,

-- Other parameters

quantityConfig						QuantityConfig						OPTIONAL,

measScaleFactor-r12					MeasScaleFactor-r12					OPTIONAL,

s-Measure							INTEGER (-140..-44)					OPTIONAL,

speedStatePars						CHOICE {

release								NULL,

setup								SEQUENCE {

mobilityStateParameters				MobilityStateParameters,

timeToTrigger-SF					SpeedStateScaleFactors

}

}																		OPTIONAL,

allowInterruptions-r11			BOOLEAN								OPTIONAL

}

-- ASN1STOP

#### –	VarMeasIdleConfig

The UE variable VarMeasIdleConfig includes the configuration of the measurements to be performed by the UE while in RRC_IDLE or RRC_INACTIVE for E-UTRA inter-frequency and inter-RAT (i.e. NR) measurements.

VarMeasIdleConfig UE variable

-- ASN1START

VarMeasIdleConfig-r15 ::=	SEQUENCE {

measIdleCarrierListEUTRA-r15			EUTRA-CarrierList-r15			OPTIONAL,

measIdleDuration-r15					ENUMERATED {sec10, sec30, sec60, sec120,

sec180, sec240, sec300}

}

VarMeasIdleConfig-r16 ::=	SEQUENCE {

measIdleCarrierListNR-r16				NR-CarrierList-r16		OPTIONAL,

validityAreaList-r16					ValidityAreaList-r16	OPTIONAL

}

-- ASN1STOP

#### –	VarMeasIdleReport

The UE variable VarMeasIdleReport includes the logged measurements information.

VarMeasIdleReport UE variable

-- ASN1START

VarMeasIdleReport-r15 ::=	SEQUENCE {

measReportIdle-r15				MeasResultListIdle-r15

}

VarMeasIdleReport-r16 ::=	SEQUENCE {

measReportIdle-r16				MeasResultListExtIdle-r16					OPTIONAL,

measReportIdleNR-r16			MeasResultListIdleNR-r16						OPTIONAL

}

-- ASN1STOP

#### –	VarMeasReportList

The UE variable VarMeasReportList includes information about the measurements for which the triggering conditions have been met.

VarMeasReportList UE variable

-- ASN1START

VarMeasReportList ::=				SEQUENCE (SIZE (1..maxMeasId)) OF VarMeasReport

VarMeasReportList-r12 ::=			SEQUENCE (SIZE (1..maxMeasId-r12)) OF VarMeasReport

VarMeasReport ::=					SEQUENCE {

-- List of measurement that have been triggered

measId								MeasId,

measId-v1250						MeasId-v1250					OPTIONAL,

cellsTriggeredList					CellsTriggeredList				OPTIONAL,

csi-RS-TriggeredList-r12			CSI-RS-TriggeredList-r12		OPTIONAL,

poolsTriggeredList-r14				Tx-ResourcePoolMeasList-r14	OPTIONAL,

numberOfReportsSent					INTEGER

}

CellsTriggeredList ::=				SEQUENCE (SIZE (1..maxCellMeas)) OF CHOICE {

physCellIdEUTRA							PhysCellId,

physCellIdUTRA							CHOICE {

fdd										PhysCellIdUTRA-FDD,

tdd										PhysCellIdUTRA-TDD

},

physCellIdGERAN							SEQUENCE {

carrierFreq								CarrierFreqGERAN,

physCellId								PhysCellIdGERAN

},

physCellIdCDMA2000						PhysCellIdCDMA2000,

wlan-Identifiers-r13					WLAN-Identifiers-r12,

physCellIdNR-r15						SEQUENCE {

carrierFreq								ARFCN-ValueNR-r15,

physCellId								PhysCellIdNR-r15,

rs-IndexList-r15						SSB-IndexList-r15				OPTIONAL

}

}

CSI-RS-TriggeredList-r12 ::=		SEQUENCE (SIZE (1..maxCSI-RS-Meas-r12)) OF MeasCSI-RS-Id-r12

SSB-IndexList-r15::=			SEQUENCE (SIZE (1..maxRS-Index-r15)) OF RS-IndexNR-r15

-- ASN1STOP

#### –	VarMobilityHistoryReport

The UE variable VarMobilityHistoryReport includes the mobility history information.

-- ASN1START

VarMobilityHistoryReport-r12 ::=	VisitedCellInfoList-r12

-- ASN1STOP

#### –	VarPendingRnaUpdate

The UE variable VarPendingRnaUpdate indicates whether there is a pending RNAU procedure or not. The setting of this BOOLEAN variable to TRUE means that there is a pending RANU procedure.

VarPendingRnaUpdate UE variable

-- ASN1START

VarPendingRnaUpdate-r15 ::=					SEQUENCE {

pendingRnaUpdate					BOOLEAN								OPTIONAL

}

-- ASN1STOP

#### –	VarRLF-Report

The UE variable VarRLF-Report includes the radio link failure information or handover failure information.

VarRLF-Report UE variable

-- ASN1START

VarRLF-Report-r10 ::=				SEQUENCE {

rlf-Report-r10							RLF-Report-r9,

plmn-Identity-r10						PLMN-Identity

}

VarRLF-Report-r11 ::=				SEQUENCE {

rlf-Report-r10						RLF-Report-r9,

plmn-IdentityList-r11				PLMN-IdentityList3-r11

}

-- ASN1STOP

#### –	VarShortINACTIVE-MAC-Input

The UE variable VarShortINACTIVE-MAC-Input specifies the input used to generate the shortResume-MAC-I during RRC Connection Resume procedure for RRC_INACTIVE.

VarShortINACTIVE-MAC-Input UE variable

-- ASN1START

VarShortINACTIVE-MAC-Input-r15 ::=		SEQUENCE {

cellIdentity-r15							CellIdentity,

physCellId-r15							PhysCellId,

c-RNTI-r15								C-RNTI

}

-- ASN1STOP

VarShortINACTIVE-MAC-Input field descriptions

cellIdentity An input variable used to calculate the shortResume-MAC-I. Set to CellIdentity included in cellIdentity (without suffix) in SIB1 of the current cell.

c-RNTI Set to C-RNTI that the UE had in the PCell it was connected to prior to suspension of the RRC connection.

physCellId Set to the physical cell identity of the PCell the UE was connected to prior to suspension of the RRC connection.

#### –	VarShortMAC-Input

The UE variable VarShortMAC-Input specifies the input used to generate the shortMAC-I.

VarShortMAC-Input UE variable

-- ASN1START

VarShortMAC-Input ::=					SEQUENCE {

cellIdentity							CellIdentity,

physCellId								PhysCellId,

c-RNTI									C-RNTI

}

-- ASN1STOP

VarShortMAC-Input field descriptions

cellIdentity An input variable used to calculate the shortMAC-I. Set to CellIdentity included in cellIdentity (without suffix) in SIB1 of the current cell.

c-RNTI Set to C-RNTI that the UE had in the PCell it was connected to prior to the failure.

physCellId Set to the physical cell identity of the PCell the UE was connected to prior to the failure.

#### –	VarShortResumeMAC-Input

The UE variable VarShortResumeMAC-Input specifies the input used to generate the shortResumeMAC-I during RRC Connection Resume procedure.

VarShortResumeMAC-Input UE variable

-- ASN1START

VarShortResumeMAC-Input-r13 ::=		SEQUENCE {

cellIdentity-r13						CellIdentity,

physCellId-r13							PhysCellId,

c-RNTI-r13								C-RNTI,

resumeDiscriminator-r13					BIT STRING(SIZE(1))

}

-- ASN1STOP

VarShortResumeMAC-Input field descriptions

cellIdentity An input variable used to calculate the shortResumeMAC-I. Set to CellIdentity included in cellIdentity (without suffix) in SIB1 of the current cell.

c-RNTI Set to C-RNTI that the UE had in the PCell it was connected to prior to suspension of the RRC connection.

physCellId Set to the physical cell identity of the PCell the UE was connected to prior to suspension of the RRC connection.

resumeDiscriminator A constant that allows differentiation in the calculation of the MAC-I for shortResumeMAC-I The resumeDiscriminator is set to '1'

#### –	VarWLAN-MobilityConfig

The UE variable VarWLAN-MobilityConfig includes information about WLAN for access selection and mobility.

VarWLAN-MobilityConfig UE variable

-- ASN1START

VarWLAN-MobilityConfig ::=					SEQUENCE {

wlan-MobilitySet-r13					WLAN-Id-List-r13			OPTIONAL,

successReportRequested					ENUMERATED {true}			OPTIONAL,

wlan-SuspendConfig-r14					WLAN-SuspendConfig-r14		OPTIONAL

}

-- ASN1STOP

VarWLAN-MobilityConfig field descriptions

wlan-MobilitySet Indicates the WLAN mobility set configured.

successReportRequested Indicates whether the UE shall report successful connection to WLAN. Applicable to LWA and LWIP.

#### –	VarWLAN-Status

The UE variable VarWLAN-Status includes information about the status of WLAN connection for LWA, RCLWI or LWIP.

VarWLAN-Status UE variable

-- ASN1START

VarWLAN-Status-r13 ::=				SEQUENCE {

status-r13								WLAN-Status-r13,

status-r14								WLAN-Status-v1430	OPTIONAL

}

-- ASN1STOP

VarWLAN-Status field descriptions

status Indicates the connection status to WLAN and causes for connection failures.

#### –	Multiplicity and type constraint definitions

This clause includes multiplicity and type constraints applicable (only) for UE variables.

-- ASN1START

maxLogMeas-r10				INTEGER ::= 4060-- Maximum number of logged measurement entries

-- that can be stored by the UE

-- ASN1STOP

#### –	End of EUTRA-UE-Variables

-- ASN1START

END

-- ASN1STOP

## 7.1a	NB-IoT UE variables

NOTE:	To facilitate the specification of the UE behavioural requirements, UE variables are represented using ASN.1. Unless explicitly specified otherwise, it is however up to UE implementation how to store the variables. The optionality of the IEs in ASN.1 is used only to indicate that the values may not always be available.

#### –	NBIOT-UE-Variables

This ASN.1 segment is the start of the NB-IoT UE variable definitions.

-- ASN1START

NBIOT-UE-Variables DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

CellGlobalIdEUTRA,

maxFreq,

PLMN-IdentityList3-r11

FROM EUTRA-RRC-Definitions

VarShortMAC-Input,

VarShortResumeMAC-Input-r13

FROM EUTRA-UE-Variables

ANR-CarrierList-NB-r16,

ANR-MeasResult-NB-r16,

maxFreqANR-NB-r16,

MeasResultServCell-NB-r14,

NRSRP-Range-NB-r14,

RLF-Report-NB-r16

FROM NBIOT-RRC-Definitions;

-- ASN1STOP

#### –	VarANR-MeasConfig-NB

The UE variable VarANR-MeasConfig-NB includes the configuration of the measurements to be performed by the UE in RRC_IDLE for ANR. The UE performs these measurements once while in RRC_IDLE and only in the cell where it receives the measurement configuration.

VarANR-MeasConfig-NB

-- ASN1START

VarANR-MeasConfig-NB-r16::=	SEQUENCE {

anr-QualityThreshold-r16			NRSRP-Range-NB-r14,

anr-CarrierList-r16					ANR-CarrierList-NB-r16

}

-- ASN1STOP

#### –	VarANR-MeasReport-NB

The UE variable VarANR-MeasReport-NB includes the stored ANR measurements information.

VarANR-MeasReport-NB

-- ASN1START

VarANR-MeasReport-NB-r16::=	SEQUENCE {

plmn-IdentityList-r16			PLMN-IdentityList3-r11,

servCellIdentity-r16			CellGlobalIdEUTRA,

measResultServCell-r16			MeasResultServCell-NB-r14,

relativeTimeStamp-r16			INTEGER (0..95),

measResultList-r16				SEQUENCE (SIZE (1..maxFreqANR-NB-r16)) OF ANR-MeasResult-NB-r16

}

-- ASN1STOP

#### –	VarRLF-Report-NB

The UE variable VarRLF-Report-NB includes the radio link failure information.

VarRLF-Report-NB UE variable

-- ASN1START

VarRLF-Report-NB-r16 ::=		SEQUENCE {

rlf-Report-r16					RLF-Report-NB-r16,

plmn-IdentityList-r16			PLMN-IdentityList3-r11

}

-- ASN1STOP

#### –	VarShortMAC-Input-NB

The UE variable VarShortMAC-Input-NB specifies the input used to generate the shortMAC-I.

VarShortMAC-Input-NB UE variable

-- ASN1START

VarShortMAC-Input-NB-r13	::=		VarShortMAC-Input

-- ASN1STOP

#### –	VarShortResumeMAC-Input-NB

The UE variable VarShortResumeMAC-Input-NB specifies the input used to generate the shortResumeMAC-I during RRC Connection Resume procedure.

VarShortResumeMAC-Input-NB UE variable

-- ASN1START

VarShortResumeMAC-Input-NB-r13	::=		VarShortResumeMAC-Input-r13

-- ASN1STOP

#### –	End of NBIOT-UE-Variables

-- ASN1START

END

-- ASN1STOP

## 7.2	Counters

| Counter | Reset | Incremented | When reaching max value |
| --- | --- | --- | --- |

## 7.3	Timers

### 7.3.1	Timers (Informative)

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T300 NOTE1 | Transmission of RRCConnectionRequest or RRCConnectionResumeRequest or RRCEarlyDataRequest | Reception of RRCConnectionSetup, RRCConnectionReject or RRCConnectionResume or RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT, cell re-selection and upon abortion of connection establishment by upper layers | Perform the actions as specified in 5.3.3.6 |
| T301 NOTE1 | Transmission of RRCConnectionReestabilshmentRequest | Reception of RRCConnectionReestablishment or RRCConnectionReestablishmentReject message as well as when the selected cell becomes unsuitable | Go to RRC_IDLE |
| T302 | Reception of RRCConnectionReject while performing RRC connection establishment or reception of RRCConnectionRelease including waitTime | Upon entering RRC_CONNECTED and upon cell re-selection, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT or RRCConnectionRelease for UP transmission using PUR, or upon reception of RRCConnectionReject message for E-UTRA/5GC. | Inform upper layers about barring alleviation as specified in 5.3.3.7 |
| T303 | Access barred while performing RRC connection establishment for mobile originating calls | Upon entering RRC_CONNECTED and upon cell re-selection, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT or RRCConnectionRelease for UP transmission using PUR. | Inform upper layers about barring alleviation as specified in 5.3.3.7 |
| T304 | Reception of RRCConnectionReconfiguration message including the MobilityControl Info or reception of MobilityFromEUTRACommand message including CellChangeOrder or upon conditional reconfiguration execution i.e. when applying a stored RRCConnectionReconfiguration message including the MobilityControl Info. | Criterion for successful completion of handover within E-UTRA, handover to E-UTRA or cell change order is met (the criterion is specified in the target RAT in case of inter-RAT) | In case of cell change order from E-UTRA or intra E-UTRA handover, initiate the RRC connection re-establishment procedure; In case of handover to E-UTRA, perform the actions defined in the specifications applicable for the source RAT; If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. |
| T305 | Access barred while performing RRC connection establishment for mobile originating signalling | Upon entering RRC_CONNECTED and upon cell re-selection, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT or RRCConnectionRelease for UP transmission using PUR. | Inform upper layers about barring alleviation as specified in 5.3.3.7 |
| T306 | Access barred while performing RRC connection establishment for mobile originating CS fallback. | Upon entering RRC_CONNECTED and upon cell re-selection, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT or RRCConnectionRelease for UP transmission using PUR. | Inform upper layers about barring alleviation as specified in 5.3.3.7 |
| T307 | Reception of RRCConnectionReconfiguration message including MobilityControlInfoSCG | Successful completion of random access on the PSCell, upon initiating re-establishment and upon SCG release | Initiate the SCG failure information procedure as specified in 5.6.13. |
| T308 | Access barred due to ACDC while performing RRC connection establishment subject to ACDC | Upon entering RRC_CONNECTED and upon cell re-selection, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT or RRCConnectionRelease for UP transmission using PUR. | Inform upper layers about barring alleviation for ACDC as specified in 5.3.3.7 |
| T309 NOTE1 | When access attempt is barred at access barring check for an Access Category. The UE shall maintain one instance of this timer per Access Category. | Upon entering RRC_CONNECTED, upon cell (re)selection, upon reception of RRCConnectionRelease, upon change of PCell while in RRC_CONNECTED, or upon reception of MobilityFromEUTRACommand. | Perform the actions as specified in 5.3.16.4. |
| T310 NOTE1 NOTE2 | Upon detecting physical layer problems for the PCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers | Upon receiving N311 consecutive in-sync indications from lower layers for the PCell, upon triggering the handover procedure, upon initiating the connection re-establishment procedure, and upon initiating the MCG failure information procedure. | If security is not activated and the UE is not a NB-IoT UE that supports RRC connection re-establishment for the Control Plane CIoT EPS/5GS optimisation: go to RRC_IDLE else: initiate the MCG failure information procedure as specified in 5.6.26 or the connection re-establishment procedure as specified in 5.3.7. |
| T311 NOTE1 | Upon initiating the RRC connection re-establishment procedure | Selection of a suitable E-UTRA cell or a cell using another RAT. | Go to RRC_IDLE |
| T312 NOTE2 | Upon triggering a measurement report for a measurement identity for which T312 has been configured and useT312 has been set to true, while T310 is running | Upon receiving N311 consecutive in-sync indications from lower layers, upon triggering the handover procedure, upon initiating the connection re-establishment procedure, upon initiating the MCG failure information procedure, and upon the expiry of T310 | Initiate the MCG failure information procedure as specified in 5.6.26 or the connection re-establishment procedure as specified in 5.3.7. |
| T313 NOTE2 | Upon detecting physical layer problems for the PSCell i.e. upon receiving N313 consecutive out-of-sync indications from lower layers | Upon receiving N314 consecutive in-sync indications from lower layers for the PSCell, upon initiating the connection re-establishment procedure, upon SCG release and upon receiving RRCConnectionReconfiguration including MobilityControlInfoSCG | Inform E-UTRAN about the SCG radio link failure by initiating the SCG failure information procedure as specified in 5.6.13. |
| T316 | Upon transmission of the MCGFailureInformation message | Upon receiving RRCConnectionRelease, RRCConnectionReconfiguration with mobilityControlInfo, MobilityFromEUTRACommand, or upon initiaitng the re-establishment procedure, | Perform the actions as specified in 5.6.26.5. |
| T320 | Upon receiving t320 or upon cell (re)selection to E-UTRA from another RAT with validity time configured for dedicated priorities (in which case the remaining validity time is applied). | Upon entering RRC_CONNECTED, when PLMN selection is performed on request by NAS, when the UE enters RRC_IDLE from RRC_INACTIVE, or upon cell (re)selection to another RAT (in which case the timer is carried on to the other RAT) , or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT or RRCConnectionRelease for UP transmission using PUR. | Discard the cell reselection priority information provided by dedicated signalling. |
| T321 | Upon receiving measConfig including a reportConfig with the purpose set to reportCGI | Upon acquiring the information needed to set all fields of cellGlobalId for the requested cell, upon receiving measConfig that includes removal of the reportConfig with the purpose set to reportCGI and upon detecting that a cell is not broadcasting SIB1. | Initiate the measurement reporting procedure, stop performing the related measurements and remove the corresponding measId |
| T322 NOTE1 | Upon receiving redirectedCarrierOffsetDedicated included in RedirectedCarrierInfo | Upon entering RRC_CONNECTED, when PLMN selection is performed on request by NAS, or upon cell (re)selection to another frequency or RAT, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT or RRCConnectionRelease for UP transmission using PUR. | Release redirectedCarrierOffsetDedicated. |
| T323 | Upon receiving t323. | Upon entering RRC_CONNECTED, when PLMN selection is performed on request by NAS, when the UE enters RRC_IDLE from RRC_INACTIVE, or upon cell (re)selection to another RAT, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT or RRCConnectionRelease for UP transmission using PUR. | Discard the altFreqPriorities provided by dedicated signalling. UE shall apply the cell reselection priority information broadcast in the system information via cellReselectionPriority and cellReselectionSubPriority. |
| T325 | Timer (re)started upon receiving RRCConnectionReject message with deprioritisationTimer. |  | Stop deprioritisation of all frequencies or E-UTRA signalled by RRCConnectionReject. |
| T330 | Upon receiving LoggedMeasurementConfiguration message | Upon log volume exceeding the suitable UE memory, upon initiating the release of LoggedMeasurementConfiguration procedure | Perform the actions specified in 5.6.6.4 |
| T331 | Upon receiving RRCConnectionRelease message including measIdleConfig. | Upon receiving RRCConnectionSetup, RRCConnectionResume, RRCConnectionRelease with an idle/inactive measurement configuration or indication to release the configuration, if validityArea is configured, upon cell selection/reselection to a cell that does not belong to the validityArea (if configured), or upon reselecting to an inter-RAT cell. | Perform the actions specified in 5.6.20.3. |
| T340 NOTE2 | Upon transmitting UEAssistanceInformation message with powerPrefIndication set to normal | Upon releasing powerPrefIndication during the connection re-establishment procedure | No action. |
| T341 NOTE2 | Upon transmitting UEAssistanceInformation message with bw-Preference. | Upon resuming an RRC connection or upon releasing bw-Preference during the connection re-establishment procedure | No action. |
| T342 NOTE2 | Upon transmitting UEAssistanceInformation message with delayBudgetReport. | Upon releasing delayBudgetReportingConfig during the connection re-establishment and connection resume procedures | No action. |
| T350 | Upon entering RRC_IDLE if t350 has been received in wlan-OffloadInfo. | Upon entering RRC_CONNECTED, or upon cell reselection. | Perform the actions specified in 5.6.12.4. |
| T351 | Reception of RRCConnectionReconfiguration message including the associationTimer in WLAN-MobilityConfig. | Upon successful connection to WLAN, upon WLAN connection failure, upon leaving RRC_CONNECTED, upon triggering the handover procedure, or upon initiating the connection re-establishment procedure. | Perform WLAN Connection Status Reporting specified in 5.6.15.2. |
| T360 | Upon performing the redistribution target selection as specified in TS 36.304 [4]. | Upon entering RRC_CONNECTED, upon receiving a Paging message including redistributionIndication; upon reselecting a cell not belonging to the redistribution target. | Stop considering a frequency or cell to be redistribution target, and perform the redistribution target selection if the condition specified in TS 36.304 [4] is met. |
| T370 | Upon receiving SL-DiscConfig including a discSysInfoToReportConfig set to setup. | Upon initiating the transmission of SidelinkUEInformation including discSysInfoReportFreqList, upon receiving SL-DiscConfig including discSysInfoToReportConfig set to release, upon handover and re-establishment. | Release discSysInfoToReportConfig. |
| T314 NOTE2 | Upon early detecting physical layer problems for the PCell i.e. upon receiving N310 consecutive "early-out-of-sync" indications from lower layers. | Upon receiving N311 consecutive in-sync indications from lower layers for the PCell, upon triggering the handover procedure and upon initiating the connection re-establishment procedure | Initiate the UE Assistance Information procedure to report early detection of physical layer problems in accordance with 5.6.10. |
| T315 NOTE2 | Upon detecting physical layer improvements of the PCell i.e. upon receiving N311 consecutive "early-in-sync" indications from lower layers. | Upon receiving N310 consecutive "early-out-of-sync" indications from lower layers for the PCell. | Initiate the UE Assistance Information procedure to report detection of physical layer improvements in accordance with 5.6.10. |
| T343 NOTE2 | Upon transmitting UEAssistanceInformation message with RLM-Report including earlyOutOfSync. | Upon initiating the connection re-establishment procedure | No action. |
| T344 NOTE2 | Upon transmitting UEAssistanceInformation message with RLM-Report including earlyInSync. | Upon initiating the connection re-establishment procedure | No action. |
| T345 | Upon transmitting UEAssistanceInformation message with overheatingAssistance | Upon releasing overheatingAssistance during the connection re-establishment procedure, or connection resume procedure. | No action. |
| T380 | Upon reception of periodic-RNAU-timer in RRCConnectionRelease. | Upon reception of RRCConnectionResume, RRCConnectionRelease or RRCConnectionSetup. | Initiate the RAN notification area update procedure |
| NOTE1: Only the timers marked with "NOTE1" are applicable to NB-IoT. NOTE2: The behaviour as specified in 7.3.2 applies. |  |  |  |

### 7.3.2	Timer handling

When the UE applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise.

## 7.4	Constants

| Constant | Usage |
| --- | --- |
| N310 | Maximum number of consecutive "out-of-sync" or "early-out-of-sync" indications for the PCell received from lower layers |
| N311 | Maximum number of consecutive "in-sync" or "early-in-sync" indications for the PCell received from lower layers |
| N313 | Maximum number of consecutive "out-of-sync" indications for the PSCell received from lower layers |
| N314 | Maximum number of consecutive "in-sync" indications for the PSCell received from lower layers |

Appendix to be removed later on

# Annex X (informative): RAN2 agreements for IOT NTN

List of RAN2 agreements.

## RAN2#115-e

Discontinous coverage

RAN2 confirms that the following will be supported: discontinuous coverage without excessive UE power consumption and without excessive failures / recovery actions. It is expected that this need to be taken into account at least for Idle mode. The requirement is applicable for all reference scenarios (GEO, MEO and LEO).

Sattelite assistance information will be used by the UE for predicting coverage discontinuity. The details of the assistance information is FFS. FFS whether any applicable agreements made in NR-NTN can be reused.

The details of UEs actions when predicted to be out of coverage is FFS, e.g. stopping unnecessary cell search in the Idle mode, and FFS to what extent this need to be specified.

It is FFS to what extent it need to be specified the details of UE’s prediction of discontinuous coverage and its ability to detect when it is back in coverage.

RAN2 sends an LS to SA2 and CT1 (cc: RAN3) for the possible alignment work in their specification due to the support of discontinuous coverage.

User plane

Start of ra-ResponseWindow is delayed by an offset. Postpone discussion on the offset value until further agreements regarding RACH are made in RAN1.

If the start of the RA Response window is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the ra-ResponseWindowSize for IoT NTN.

Start of mac-ContentionResolutionTimer is delayed by an offset, (assumed equal to UE-eNB RTT). This can be revisited if RAN1 decides something that requires to change this.

If the start of mac-ContentionResolutionTimer is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the mac-ContentionResolutionTimer for IoT NTN.

From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to eNB implementation to ensure sufficient time on UE side for the Msg3 transmission for IoT NTN.

RAN2 assumes that TA information (FFS what) reporting by the UE on network enabling will be needed in IoT NTN. Expect RAN1 need to progress on this, and can maybe reuse NR NTN progress. FFS in which message this is provided.

UE-eNB RTT is taken into account when calculating the (UL) HARQ RTT timer.

RAN2 assumes that sr-ProhibitTimer need to be extended. Postpone treatment of sr-ProhibitTimer values until the NR NTN details have been decided.

From RAN2’s perspective, delayed start of pur-ResponseWindowTimer with UE-eNB RTT can be supported. This can be revised if RAN1 finds issues to support PUR that are not small.

pur-ResponseWindowSize is not extended for IoT NTN.

SPS is supported without modification for IoT NTN.

RAN2 confirm the SI agreement that the value range of the RLC t-Reordering timer will be extended to support IoT NTN.

Do not extend the PDCP discardTimer for NB-IoT over NTN.

FFS whether to extend the PDCP discardTimer for eMTC over NTN.

Do not extend PDCP t-Reordering for IoT NTN.

Control plane

Cell selection / reselection procedures for NB-IoT and LTE-M in TN is the baseline in NB-IoT/LTE-M NTN.

RAN2 assumes that Satellite assistance information, e.g. for cell selection reselection, for serving cell is provided to UE.

Wait for the progress in RAN1 before discussion on whether satellite assistance information is broadcast in a separate information block.

The timing information on when a cell is going to stop serving the area is broadcast at least for the quasi-earth fixed case. FFS details.

The network may broadcast more than one TAC per PLMN in a cell, which is up to network implementation.

The UE determines the Tracking Area based on the broadcast information (the use of other information is not excluded).

When the network stops broadcasting a TAC, the UE needs to know it. FFS how this is done.

UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area.

Rel-16 LTE CHO mechanism is supported for LTE-M devices in IoT NTN. FFS which CE Mode(s) to apply

No procedural update is required to support connected mode mobility for LTE-M.

Rel-16 RLF / connection re-establishment mechanisms are supported in IoT NTN assuming that minor adjustments to UE specific timers and constants would be sufficient.

FFS if Satellite assistance information for neighbour cell(s) is provided to UE for cell selection/reselection (justification would be needed).

The value range for parameter t304 is not extended with larger values.

Send an LS to RAN4 to inform that RRM impacts for supporting CHO should be taken into consideration.

Postpone the discussion on whether specific timers and constants for RLF and RRC connection re-establishment procedures require extended value range and/or new behaviour till next meeting.

System information update notification procedure is not used to inform TAC updates, at least for TAC additions (FFS)

## RAN2#116-e

Discontinous coverage

Satellite Ephemeris Parameters (not same as for L1 pre-compensation, for the constellation, not just single satellite) is needed for the UE for predicting coverage discontinuity. Other info, e.g. beam info, elevation angle, reference location or corresponding is FFS.

Providing the start-time of (incoming) satellite’s coverage and end-time of serving satellite’s coverage is needed for Quasi-Earth Fixed satellites.

From RAN2 point of view, the existing power saving mechanisms e.g. DRX, PSM, eDRX, relaxed monitoring, and WUS can be reused in IoT-NTN. Minor enhancements in existing power saving mechanisms to support discontinuous coverage is FFS.

User plane

The estimate of UE-eNB RTT is equal to the sum of UE’s TA and K_mac, where the UE’s TA is given by T_TA=(N_TA+N_(TA,UE-specific) 〖+N〗_(TA,common) 〖+N〗_(TA,offset) )×T_s, and K_mac value is broadcasted by network.

RAN2 confirm that the start of mac-ContentionResolutionTimer is delayed by UE-eNB RTT in IoT NTN.

Any enhancements on (N)PRACH resource selection in IoT NTN will not be pursued in Rel-17.

An offset equal to UE-eNB RTT is added to the formula used for calculating the (UL) HARQ RTT timer in IoT NTN.

Support UE-specific TA reporting using MAC CE in Msg3/Msg5 for IoT NTN.

For IoT NTN, UE specific TA reporting during RACH procedure (MSG3/MSG5) in RRC IDLE is enabled/disabled by SI, similar with NR NTN.

Support TA reporting in RRC connected mode in IoT NTN.

UE-specific TA report uses MAC CE.

Support event-triggered for TA reporting in connected mode. Wait for NR NTN agreements for other triggers.

On how to extend RLC t-Reordering in IoT NTN, wait for NR NTN agreements and see if they can be reused.

Don’t change the L2 buffer requirement for IoT NTN (assume the network may need to limit the bit rate in order to not exceed L2 buffer).

The PDCP discardTimer should be extended to support eMTC over NTN.

If PDCP discardTimer is agreed to be extended to support eMTC over NTN, how to extend the timer value can wait for the conclusion for RLC t-reordering timer.

The ra window start offset is defined as sum (current offset, UE-eNB RTT) and current offset is defined in TS36.321 (FFS if applicable to NB-IoT 41ms offset).

Control plane

The AS layer indicates to NAS layer all of the received TACs for the selected PLMN.

For quasi-earth fixed cell, UE should start measurements on neighbour cells before the broadcast stop time of the serving cell, i.e the time when the serving cell stops covering the current area, and the exact time to start measurements (inter and intra-frequency) is up to UE implementation. FFS to what extent this need to be covered in the TS.

Location-assisted cell reselection (e.g. as for NR NTN) is not supported for IoT NTN in rel 17.

The use of hard TAC or soft TAC is up to network implementation in earth-fixed and earth-moving cells.

Relaxed monitoring further enhancements are not considered for IoT NTN in rel-17.

The serving cell ephemeris information (used for L1 pre-compensation) is signalled in a new SIB, which is NTN specific.

Update to serving cell ephemeris information does not affect the system information value tag and does not trigger System information modification procedure. How to trigger re-read of this information is FFS. FFS if the UE shall reacquire the new SIB when SI update is triggered.

Updates to serving cell ephemeris information are not bound to the BCCH modification period.

The timing information on when a serving cell is going to stop serving the area is broadcast in the same SIB as the ephemeris information.

Broadcast of the timing information on when a serving cell is going to stop serving the area is only applicable to quasi earth fixed cell (not to moving cell).

No enhancement to R16 RLF and RRC connection Re-establishment procedures are introduced in R17.  (this does not include handling of UL synchronisation loss which is FFS and does not include non continuous coverage).

No extension to timers and constants is required for RLF and RRC connection Re-establishment.

No need to extend the 10 s delay for actions upon reception of RRCConnectionRelease in NB-IoT.

It is feasible to use the legacy barring bit to block legacy UEs, and it is possible to have a new bit that assumes the functionality of the old bit. It is FFS if it is needed to use the barring bit or whether other mechanism can be assumed (new band etc).

No enhancement to R16 CHO are introduced in R17.