3GPP TSG-RAN2 Meeting #116-bis-e	R2-2201494

Online, 17-25 January 2022

Agenda item:	8.12.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM relaxations in CONNECTED

WID/SID:	NR_redcap-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

RAN2 objectives for reduced capability (REDCAP) devices were revised in RP-211574:

In this contribution we discuss RRM relaxations as per the following revised work item objectives:

Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]: - Specify measurement (RSRP/RSRQ) based stationarity criterion and not-at-cell-edge criterion [RAN2] - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation. - Specify UE requirements for RRM measurement relaxation [RAN4] - No RRM measurement relaxations are specified for the serving cell.

# 2	Discussion

The following was agreed for RRM relaxations in RAN2#114 meeting:

Agreements:

- An RSRP/RSRQ based stationarity criterion (Working Assumption: the same as in idle/inactive) can be configured for UEs in RRC Connected. If the criterion is met, this is reported to the network (FFS how/when). It is FFS whether, based on this, besides possibly reconfiguring RRM measurements (up to network implementation), the network can enable RRM measurement relaxation (FFS whether same method as in Idle/Inactive)

The following was agreed for RRM relaxations for CONNECTED in RAN2#115 meeting:

Agreements:

- Do not introduce nor reuse not-at-cell-edge threshold for R17 RRC_CONNECTED UEs.

Agreements via email - from offline 110:

- Do not introduce beam change based criterion in Rel-17.

- The network provides the configuration of stationarity criterion to the UE via dedicated signalling (e.g. RRCReconfiguration message) in RRC_CONNECTED.

Agreements:

- Introduce separate Rel-17 not-at-cell-edge threshold, and the new threshold is only associated with Rel-17 stationary criterion (if configured). If configured with a not-at-cell-edge criterion, the R17 stationary criterion can only be configured together with the R17 not-at-cell-edge criterion, not with the R16 one

The following was agreed for RRM relaxations for CONNECTED in RAN2#116 meeting:

Agreements online:

- RAN2 assume that the existing RRM measurement framework can be used as baseline for enabling and disabling RRM relaxations for UEs in RRC Connected. Other methods can be considered too based on relaxation methods agreed by RAN4.

- RAN2 understands that no prohibit timer is needed, if legacy measurement reporting framework is reused by UE to report its relaxation status

- The granularity of RRM measurement relaxations (i.e. whether it should be specified per beam, per cell or per frequency) should be handled by RAN4

Agreements via email - from offline 111:

- UE is not allowed to relax its RRM measurements if both stationarity criterion and R17 not-at-cell-edge criterion are configured but UE meets only the R17 not-at-cell-edge criterion.

- UE reports to network when it no longer meets relaxation criteria.

- No additional signaling is introduced for network to tell UE whether and which criteria for RRM relaxation is considered satisfied when leaving RRC_CONNECTED state.

- No need for UE to send UE Assistance Information to request network configuring it with relaxation criteria.

- UE does not report its history/state of RRM relaxation when transitioning from RRC Idle/Inactive to RRC Connected.

- Relaxation criteria for UEs in RRC Connected are configured by only dedicated signaling.

RRM relaxation especially in CONNECTED needs to be strictly under the network control as also explained in the WID:

- Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation.

UE autonomous measurement relaxation may degrade mobility performance and therefore it should be avoided. Based on the UE reporting when stationarity / not-at-cell-edge criterion is met or not met anymore the network can enable or disable the RRM measurement relaxations. Alternatively, or additionally, network can monitor regular measurement reports (periodic or event based) from the UE and take those into account when allowing the RRM measurement relaxations.

Proposal 1: The UE is not allowed autonomously relax the RRM measurements in RRC CONNECTED

Proposal 2: In addition to measurement relaxation configuration the network can enable/disable RRM measurement relaxation in RRC CONNECTED with explicit allowed/not allowed indication

# 3	Conclusion

In this contribution RRM relaxations in CONNECTED for REDCAP UE was discussed, and the following is proposed:

Proposal 1: The UE is not allowed autonomously relax the RRM measurements in RRC CONNECTED

Proposal 2: In addition to measurement relaxation configuration the network can enable/disable RRM measurement relaxation in RRC CONNECTED with explicit allowed/not allowed indication