3GPP TSG-RAN2 Meeting #116bis-e	R2-2201555

eMeeting, 17 - 25 January 2022

Agenda Item:	8.9.2.1 Paging Sub-grouping and Paging Early Indication

Source: 	Ericsson

Title:  	PEI in last used cell

Document for:	Discussion and Decision

# Introduction

In LTE the UE only monitors the WUS in the last used cell, i.e. the cell where the UE was released from connected mode the last time. Only when the UE camps on the last used cell the UE monitors the WUS, otherwise the UE monitors the Paging Occasion (PO) directly. A similar use of PEI in NR is proposed.

When the PEI is only used in last used cell, this means that the NW only transmits the WUS when the UE is paged on the last used cell e.g. during the first paging attempt. In case the UE has moved and there is paging escalation, e.g. the UE does not respond to the first paging attempt and the NW pages the UE in all the cells of the TAI list, then the UE does not monitor PEI but the UE monitors the Paging Occasion (PO) directly.

In case the PEI would also be transmitted and used during paging escalation, e.g. when the NW cannot find the UE and pages the UE in all the cells of the TAI list, then this would cause of lot of (useless) WUS transmissions, because the UE only replies in a single cell.

In case PEI is only transmitted when UE is paged in its last used cell, this also implies that when a UE is stationary, i.e. camped on the last used cell, this stationary UE will not receive paging generated for mobile UEs. Even though the first paging attempts are typically successful (e.g. 10%) the paging generated for mobility UEs can be substantial, because paging may be needed in many cells. The latter is a big power saving advantage for the UE.

Finally, it is noted that a UE implementation, without any inter-operability problems, can decide not to monitor the PEI and monitor the Paging PDCCH directly. However the NW is required to transmit the PEI, whether the UE monitors the PEI or not. It is not beneficial from a NW perspective when a UE implementation decides to skip the PEI while the NW broadcasts it, i.e. NW resources are wasted.

# Background

RAN2

The use of PEI in last used cell was discussed during offline #046 [1] and in [2-6], but no agreements were reached.

SA2 and RAN3

SA2 noted in agreed CR 23.501 that PEI in last used cell would be further discussed in RAN2 [7].

RAN3 noted in summary of offline #9 whether PEI is only used in last used cell depends on RAN2 progress [8].

(G)WUS in last used cell LTE

In 36.300 it is specified that the UE monitors (G)WUS only in the last used cell, i.e. the cell where the UE was released from connected mode the last time:

-	The UE monitors (G)WUS only in the last used cell;

The last used cell is signalled from eNB to MME in UE CONTEXT RELEASE COMPLETE and UE CONTEXT SUSPEND REQUEST message via Information on Recommended Cells and eNBs for Paging (see section 9.2.1.105 in 36.413). And the MME provides the last used cell info via Recommended Cells for Paging in the PAGING message for a (G)WUS capable UEs (see section 4.3.21.4 in 23.401). Similar messages and information elements exist for the signalling between gNB and AMF in NR (see 38.413).

The UE capability (G)WUS is included in the UE-RadioPagingInfo information IE in the PAGING message. RAN2 did not decide on the UE capabilities for Rel-17 UE power saving, but a PEI capability needs to be added to the UERadioPagingInformation IE in the PAGING message between AMF and gNB.

# Discussion

False paging alarms

False paging alarms may happen when multiple stationary UEs are monitoring the same Paging Occasion (PO) and the UE receives a Paging message for another UE. Or it happens when the UE receives a Paging message for a UE that is mobile, i.e. during paging escalation when the mobile UE did not respond to the first paging attempt.

During the RAN1 UE power saving study a 10% probability that the UE receives paging in the PO was generally assumed. With such paging probability it is beneficial to use PEI. However in [3] a probability close to 100% was observed in the field. In the latter case it is not beneficial to use the PEI. The PEI (just like WUS) prior to a Paging Occasion (PO) is only effective when most of the time there is no paging, i.e. when most of the time the UE does not need to wake-up during the following PO. In case the PEI most of the times indicates that the UE has to wake up during the following PO, then monitoring PEI only increases the UE wake-frequency, i.e. increases the UE power consumption:

Observation 1: Monitoring PEI/WUS only saves power when the PEI/WUS most of the times indicates there is no Paging message in the following PO.

Paging escalation

Paging escalation can be a major contributor to high paging load. Consider a NW configuration of 1.28 sec DRX where each frame is configured as a Paging Frame (PF) and there is one PO configured per PF, i.e. 128 POs/DRX cycle. In case there are several hundreds of UEs camped on the cell, this implies that several UEs are monitoring the same PO. The false paging alarms scales with the number of UEs monitoring the same PO. But if there is paging escalation with 10% probability, then false alarms due to mobility quickly escalate this for example when there are 100 cells/TAI-list.

Observation 2: Paging escalation can contribute to a high paging load which makes monitoring PEI not power efficient.

For these reasons it was decided that WUS in LTE is only used in last used cell [4]:

RAN2 re-discussed “whether and how using WUS may affect MME paging strategies” in R15 and concluded that with some MME paging strategies, e.g. always paging a UE in the entire TA list, this may lead to increased power consumption for UEs using WUS.

The PEI is triggered frequently when:

- the CN uses a paging strategy to minimize the paging latency: every time the UE is paged in the complete Tracking Area List (TAI List) e.g. for voice calls.

- there is high mobility: when the CN does not page in the complete TA every time, the first paging attempt is more likely to fail when there is high mobility and the UE is not reached in the cell where it was the last time.

- the number of cells in the TAI List is large: a failed paging attempt triggers paging in many cells, e.g. x100 with 100 cells in the TAI List.

For the above reasons it is also proposed in NR:

Proposal 1: PEI in NR is only monitored in the last used cell when the UE is in idle or inactive mode, like for WUS in LTE

This means that PEI is only transmitted in the last used cell, where the UE may have benefit to monitor PEI and a UE implementation does not skip PEI transmissions. Furthermore PEI in last used cell only reduces the number of unnecessary PEI transmissions during paging escalation and avoids wasting NW resources. Limiting PEI to last used cell also has a big power saving benefit for the UE because the UE skips receiving Paging due to mobility of other UEs.

For RRC Inactive, the RAN paging, including paging escalation, is handled by the gNB(s). The RNA is configured by the gNB in the RRCRelease and an RNA can be as big as a Tracking Area. Without additional complexity the same PEI monitoring rules can apply for inactive mode, i.e. the UE only monitors the PEI in the last used cell, and, the gNB that initiates the RAN Paging (because there is DL data or signalling from the CN) uses PEI when the UE that is paged supports it. A gNB that receives a PAGING message over the Xn-interface during Paging escalation does not use PEI.

# Summary

RAN2 is kindly asked to discuss:

Proposal 1: PEI in NR is only monitored in the last used cell when the UE is in idle or inactive mode, like for WUS in LTE

# References

- R2-2111562, Summary of [AT116-e][046][ePowSav] Paging Early Indication, Report, Ericsson, RAN2#116-e

- R2-2110415, PEI monitoring in last used cell, Ericsson, Vodafone, DISC, RAN2#116-e

- R2-2109880, Further considerations for subgrouping, Intel, DISC, RAN2#116-e

- R2-2109453, PEI configuration and monitoring procedures, Qualcomm, DISC, RAN2#116-e

- R2-2109736, Architecture for paging enhancement by UE subgrouping, vivo, DISC, RAN2#116-e

- R2-2110481, Consideration on Idle/inactive-mode UE power saving, Lenovo, DISC, RAN2#116-e

- S2-2107857, Support for Paging Early Indication, CR 23.501, MediaTek et all, SA2#147-e

- R3-215812, Summary of Offline Discussion on CB: # 9_UEPowerSaving, Report, ZTE, RAN3#114-e