3GPP TSG RAN WG2 Meeting #116bis-e                                                        	R2-2201579

e-Meeting, January 17 - 25, 2022

Agenda item:	8.10.3.1

Source:	Samsung

Title:	UE location reporting in initial access

Document for:	Discussion and Decision

# 1. Introduction

RAN2 made an agreement that the location information is reported in MSG5. In this contribution, we would like to discuss further signalling details.

# 2. Discussion

RAN2 made the following agreements on UE location reporting at RAN2#115-e:

Agreements via email - via offline 102:

- If SA3 has no concern reporting coarse location during initial access, the coarse location information is reported in Msg5, i.e., via RRCSetupComplete/RRCResumeComplete message.

- For coarse UE location reporting during initial access, the location granularity is not indicated to UE via SIB.

- Enhancements to validate the UE ’s coarse location information is not needed from RAN2 perspective. Whether this is needed by the network is up to other WGs.

- After AS security is established, gNB can obtain a GNSS-based location information from the UE using existing signalling method, i.e., by configuring includeCommonLocationInfo in the corresponding reportConfig. It is up to SA3 to decide whether User Consent is required before NW acquires location information from the UE in NTN. RAN2 discuss whether to send LS to SA3.

- Aperiodic location reporting (e.g., via DCI) is not supported.

Working assumption:

- Event triggered-based UE location reporting are configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED

RAN2 also made the following agreements on UE TA reporting at RAN2#115-e:

Agreements via email - via offline 106:

- UE specific TA reporting during RACH procedure is enabled/disabled by SI (FFS for RACH in connected mode)

- The content of UE specific TA pre-compensation reported in RA procedure using MAC CE is UE specific TA (this can be revisited after receiving RAN1 response).

- Reporting on the information about UE specific TA in connected mode is supported, FFS via RRC signalling or MAC CE

- Event-triggers for reporting on the information about UE specific TA in connected mode is supported. FFS on the details. Confirmation by RAN1 is also needed

- If configured, the UE shall report information of the UE specific TA pre-compensation to the target cell during the random access. FFS if a new indication in RRC reconfiguration with sync is needed or not (besides the SIB indication carried in HO command on whether TA report is enabled/disabled in the target cell).

- Information about UE specific TA pre-compensation is not reported in RA procedures triggered due to “Request for Other SI”

In RRC running CR, a network is able to configure whether TA reporting is required or not in initial access:

[[

enableTA-Report-r17			ENUMERATED {enabled}				OPTIONAL   -- Need R

-- Editor’s note: FFS on where this parameter is placed.

]]

However, currently a network cannot configure whether UE location reporting is required or not in initial access. Considering i) UE location reporting for RRC connected UE is configurable by the network, ii) UE location reporting in initial access is not always required, iii) UE location reporting can be used for the purpose of TA reporting and iv) signalling overhead of UE location reporting, it is proposed a network is able to configure whether UE location reporting is required or not in initial access.

[Proposal]: Whether UE location reporting is required or not in initial access is configurable by the network.

# 3. Conclusion

RAN2 is asked to agree with the following proposal:

[Proposal]: Whether UE location reporting is required or not in initial access is configurable by the network.