3GPP TSG-RAN WG2 Meeting #116bis electronic	R2-2201596

e-Meeting, 17th - 25th January 2022

Source:	vivo

Title:          Discussion on RAN Visible QoE

Agenda Item:	8.14.2

Document for:	Discussion and Decision

- Introduction

At the RAN2#116e meeting, the following agreements were made regarding the RAN visible QoE.

RAN2 assumes that RAN2 is responsible to define the procedure to support RVQOE configuration and reporting, and leave the definition of RAN QoE metrics and what should be included in RVQOE configuration and report to other WGs, e.g. RAN3, SA4. RAN2 confirms the following is feasible from RAN2 point of view. - It is feasible to configure RVQOE using explicit RRC IEs - Multiple simultaneous QoE measurements can be supported for RVQOE. Each RVQOE measurement configuration is identified by the MeasConfigAppLayerId (or change to another generic term) corresponding to the regular QoE configuration. - UE RRC layer forwards the received RVQOE configuration to the upper (application) layer, indicating the service type. - RAN configures the required RVQOE metrics in the RVQOE configuration for UE to report.

At the RAN3#114e meeting, RAN3 discussed the RAN visible QoE and made certain agreements, which were captured in the LS R2-2200110 [1].

In this contribution, we further discuss the spec impact of RAN visible QoE from RAN2 perspective.

- Discussion

In [1], RAN3 provides the following agreements and asks RAN2 to consider the above agreements in their corresponding work on RAN visible QoE and to provide specification support for the same.

RVQoE metrics Interaction latency or comparable quality viewport switching latency metric is NOT considered as a RAN visible QoE metric in Rel-17 Buffer level is confirmed as a RAN visible QoE metric for DASH and VR service types Playout delay for media startup is confirmed as a RAN visible QoE metric for DASH and VR service types RVQoE configuration RAN Visible QoE and legacy QoE can be configured together or separately. In case RAN visible QoE is configured separately, it can be configured only after configuring legacy QoE. NG-RAN can release a list of RAN visible QoE configurations while not releasing the corresponding legacy QoE configurations If the legacy QoE configuration is released, the corresponding RAN visible QoE configuration is released as well RAN visible QoE configuration can include at least the RAN visible QoE metrics to be reported, service type and a measurement ID for the RAN visible QoE. Whether existing IEs can be reused for service type and measurement ID and the signaling design is up to RAN2 There is no need to consider Start Time, Duration and Sample Percentage in the RAN Visible QoE configuration in Rel-17 RVQoE reporting RAN3 should discuss whether the existing identified RAN visible QoE metrics (or values if agreed) justifies the need of a separate reporting periodicity for RAN visible QoE RAN3’s decision on whether to have a different reporting periodicity for RAN visible QoE is independent of RAN2’s decision on which SRB to use for RAN visible QoE Misc proposals NG-RAN can configure RAN visible QoE for only a subset of those metrics which are already configured as part of legacy QoE configuration. The OAM sends a list of the available RAN visible QoE metrics to the RAN node, outside the legacy QoE configuration container.

Based on the LS, we think the general procedure of RAN-visible QoE configuration and reporting are as follows:

Figure 1 RAN-visible QoE procedure

In step 3, RAN visible QoE and legacy QoE can be configured together or separately. In case RAN visible QoE is configured separately, it can be configured only after configuring legacy QoE.

For the RAN-visible QoE configuration, include the measConfigAppLayerRANVisible field in the MeasConfigAppLayer.

MeasConfigAppLayer-r17 ::= SEQUENCE { measConfigAppLayerId-r17 MeasConfigAppLayerId-r17, measConfigAppLayerContainer-r17 OCTET STRING OPTIONAL, -- Need N measConfigAppLayerRANVisible-r17 SEQUENCE { bufferLevel ENUMERATED {true} OPTIONAL, playoutDelay ENUMERATED {true} OPTIONAL, } serviceType-r17 ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1} OPTIONAL, -- Need N ... }

Based on the above analysis, we propose:

Proposal 1: Include the measConfigAppLayerRANVisible field in the MeasConfigAppLayer.

- Indications of RAN-visible metric associate with a measConfigAppLayerId

For step 6, whether RAN visible QoE and legacy QoE reports can be reported separately is for RAN3 further study. RAN2 may further decide how to trigger RVQOE reporting based on RAN3 conclusion.

In our understanding, one simple solution is reusing the reporting interval configured in the container and then the RAN visible QoE and legacy QoE reports can be reported together.

@reportinginterval Indicates the time(s) reports should be sent. If not present, then the client should send a report after the streaming session has ended. If present, @reportingInterval=n indicates that the client should send a report every n-th second provided that new metrics information has become available since the previous report. For each report sent, only the newly collected information since the previous report shall be reported.

For the RAN-visible QoE report, include the measurementReportAppLayerRANVisible field in the MeasurementReportAppLayer.

MeasurementReportAppLayer-r17-IEs ::= SEQUENCE { measurementReportAppLayerContainer-r17 OCTET STRING, measConfigAppLayerId-r17 MeasConfigAppLayerId-r17, measurementReportAppLayerRANVisible-r17 SEQUENCE { bufferLevelReport BufferLevelReport OPTIONAL, playoutDelayReport playoutDelayReport OPTIONAL, } lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE{} OPTIONAL }

Based on the above analysis, we propose:

Proposal 2: Include the measurementReportAppLayerRANVisible field in the MeasurementReportAppLayer.

- report of RAN-visible metric associated with a measConfigAppLayerId

Proposal 3: Agree on the following working assumption regarding the RVQoE report and revisit it when RAN3 made further agreements:

- the RAN visible QoE and legacy QoE reports are reported together

- reuse the reporting interval configured in the container to trigger the RAN visible QoE reporting

- Conclusion

Proposal 1: Include the measConfigAppLayerRANVisible field in the MeasConfigAppLayer.

- Indications of RAN-visible metric associate with a measConfigAppLayerId

Proposal 2: Include the measurementReportAppLayerRANVisible field in the MeasurementReportAppLayer.

- report of RAN-visible metric associated with a measConfigAppLayerId

Proposal 3: Agree on the following working assumption regarding the RVQoE report and revisit it when RAN3 made further agreements:

- the RAN visible QoE and legacy QoE reports are reported together

- reuse the reporting interval configured in the container to trigger the RAN visible QoE reporting

- Reference

- R2-2200110	RAN3 agreements on RAN visible QoE