3GPP TSG-RAN WG2 Meeting #116bis-e	R2-2201952

# E-meeting, 17th – 25th January 2022

Title:	LS on UE providing Location Information for NB-IoT

Release:	Rel-17

Work Item:	LTE_NBIOT_eMTC_NTN

Source:	RAN2

To:	RAN3, SA2, CT1

Cc:	SA3, SA3-Li

Contact person:	Jonas Sedin

Jonas.sedin@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

Following liaisons with SA2, SA3 and RAN3 (see R2-2109216, R2-2200145, R2-2200149, R2-2202542) on the reporting of UE location information in NR NTN, RAN2 has started to discuss location reporting aspects for IoT NTN.

For NB-IoT UEs AS security cannot be assumed to be supported. This has lead RAN2 to assume that sending a detailed location over AS cannot be supported and RAN2 has thus discussed whether to send coarse location. However, in latest received LS by SA3 in R2-2200149/S3-213360 regarding NR NTN location reporting, it is stated that even sending coarse location unprotected should be avoided, so RAN2 has assumed that not even coarse location may be reported over AS for NB-IoT. RAN2 also discussed the possibility UE providing location to the MME via NAS procedures as an alternative to deliver the location information via AS.

RAN2 have the following questions to RAN3/SA2 and CT1:

- To RAN3/SA2, we ask whether there are any issues from a RAN3 and SA2 point of view that a NB-IoT UE does not provide any location information over AS to eNB.

  - RAN2 assumes that not providing location over AS may lead to impact on E-UTRAN selecting correct core network, NNSF and constructing the mapped cell ID and any potential ULI.

- To RAN3/SA2/CT1, if location is not provided over AS and the location is needed for network operation is it feasible from a RAN3 and SA2 point of view that UE instead provides location to the MME via NAS, which is assumed to be protected.

  - If providing location over NAS is a viable solution that is acceptable for network operation, then is it feasible that this is introduced in Rel-17.

# 2	Actions

To RAN2

ACTION: 	RAN2 asks of RAN3/SA2/CT1 to consider the questions above and get back to RAN2 regarding issues or feasibility of the above.

# 3	Dates of next TSG RAN WG2 meetings

RAN2#117-e	21st February - 3rd March 2021	electronic meeting

RAN2#118-e	16th May – 27th May 	electronic meeting