3GPP TSG-RAN WG2 Meeting #116-bis-e                                                    R2-220xxxx

E-meeting, January 17 – 25, 2022

Agenda item:	8.4.2.3

Source: 		Qualcomm Incorporated (Rapporteur)

Title: 	 [AT116bis-e][049][eIAB] BAP Routing (Qualcomm)

Document for:	Discussion

# 1	Introduction

This document captures:

- [AT116bis-e][049][eIAB] BAP Routing (Qualcomm)

Scope: Continue progressing proposals from R2-2201690. Agree offline if possible

Intended outcome: Report, agreements

Deadline: For potential CB Monday W2

This document is based on the contribution summary R2-2201690 and R2#116bis-e online session.

# 2	Discussion

## 2.1	BAP address configuration of the boundary node

Issue: How does the boundary node know which of the two BAP addresses configured applies to what topology?

We agreed during online session:

- For each topology, the BAP address is configured to the boundary node by the CU of that topology via RRC (may need to check different scenarios).

## 2.2	Next-hop BAP address configuration of the boundary node

Issue: How does the boundary node know to what topology the configuration of a next-hop BAP address in upstream direction refers?

We agreed during online session:

- In the Routing configuration: A BH link and the corresponding next-hop BAP address belong to the topology of the CU that provided the configuration of that BH link and next-hop BAP address.

## 2.3	Routing configuration of the boundary node

Issue: How does the boundary node know to which topology the BAP routing ID and next-hop BAP address of a routing entry apply?

We agreed during online session:

- FFS if The routing entry is associated by configuration with the topology the entry applies to, e.g. by an explicit indicator.

We need to make more progress. Let us summarize the problem:

- If the boundary node does not know the topology a routing entry refers to, it may use a routing configuration for topology 1 to route a packet in topology 2 (or vv). This will most likely lead to an erroneous behaviour since each topology uses their BAP addresses and BAP path IDs independently.

- Since the routing configurations for both topologies are delivered by the same F1AP, the F1AP message needs to include explicit information that allows the boundary node to determine for which topology the routing configuration should be used.

- Since this is F1AP, the St3 details need to be hammered out by RAN3. Therefore, RAN2 only needs to capture the behaviour.

Proposal 3: The routing configuration to include information that allows the boundary node to determine the topology each routing entry applies to.

Q3: Do you agree with P3? If not, please propose rewording.

| Company | Yes/No | Comments, proposed rewording |
| --- | --- | --- |
| Samsung | Yes |  |
| Kyocera | Yes | We assume the association with topology is just one of options. As another option, we think the information could indicate where each routing entry is used in, i.e., IAB-DU part or IAB-MT part in BAP layer. In this case, IAB-DU handles downstream packets which are always forwarded to the 1st topology. On the other hand, IAB-MT handles upstream packets. We think the IAB-MT can identify whether an upstream packet is forwarded to the 1st topology or the 2nd topology, by whether the packet is performed the header rewriting. |
| LGE | Yes | We are ok with P3, but it does not mean that one routing configuration has mixed entries, i.e., one entry associated with topology 1 and other entry associated with topology 2 in one routing configuration. A simple approach is that one routing configuration only has entries for one topology which is indicated by the CU. If there are two topologies in the boundary node, the boundary node is configured with two routing configurations by the CU, one is for topology 1 and another is for topology 2. |
| Fujitsu | Yes |  |
| ZTE | Yes |  |

## 2.4	Rewriting configuration of the boundary node – Re-routing not considered

Issue: How is rewriting configuration provided, via F1AP or RRC?

We agreed during online session:

- The header rewriting configuration is provided via F1AP.

Next issue: How does the boundary node know if an inter-topology header rewriting entry refers to Top1Top2 or Top2Top1?

We agreed during online session:

- FFS if The header rewriting configuration to include an indicator, which identifies either the egress topology, or the ingress topology, or the traffic direction (RAN2 to select one of these three options).

We need to make more progress. Let us summarize the problem:

- The boundary node needs to know if a header rewriting configuration is top1top2 or top2top1. Otherwise, it may use a header rewriting configuration for top1top2 to rewrite a packet forwarded from top2top1. This will most likely lead to an erroneous behaviour.

- Since the routing configurations for both topologies are delivered by the same F1AP, the F1AP message needs to include explicit information that allows the boundary node to determine if the header rewriting is top1top2 or top2top1.

- Since this is F1AP, the St3 details need to be hammered out by RAN3. Therefore, RAN2 only needs to capture the behavior.

Proposal 5: For inter-topology routing, the header rewriting configuration to include information that allows the boundary node to determine either the egress topology, or the ingress topology, or the traffic direction of a header-rewriting entry (FFS further selection).

Q5: Do you agree with P5? If not, please propose rewording.

| Company | Yes/No | Comments, proposed rewording |
| --- | --- | --- |
| Samsung | Yes | For FFS, we would like to use the ingress topology. The reason is that the header writing operation at BAP layer is: - BAP RX part determines to send the packet to BAP TX part or upper layer, and at RX part, it can only know the ingress topology. - BAP TX part perform the rewriting based on the ingress topology indicated by the RX part. |
| Kyocera | Yes for including information | We support to include the information. For the detailed options, however, as commented in Q3 above, we think the information could be where each routing entry is used in, i.e., IAB-DU part or IAB-MT part in BAP layer. We assume it’s another interpretation of “traffic direction”. |
| LGE | Yes | We can determine details at the next meeting. |
| Fujitsu | Yes | We slightly prefer the configuration can specify the traffic direction of a rewriting entry. |
| ZTE | Yes with comments | According to the rapporteur’s analysis, does it mean that the FFS further selection will be resolved by RAN3? If yes, it is suggested to explicitly point it out in the proposal. |

## 2.5	Rewriting configuration of the boundary node – Re-routing included

We are differentiating between inter-topology routing vs. inter-donor-DU re-routing for a dual-connected boundary node.

- Inter-topology routing (main path): The packet is crossing from one topology to the other and the egress link is available.

- Inter-donor-DU re-routing (backup path): This includes two scenarios:

  - Intra-to-inter-topology re-routing: The packet is supposed to be routed within the same topology, but since the egress link is not available, it is re-routed to the other topology.

  - Inter-to-intra-topology re-routing: The packet is supposed to be routed inter-topology, but since the egress link is not available, it is re-routed to the initial topology.

Issue: For inter-topology routing and intra-to-inter-topology re-routing, there is exactly one header rewriting, and the corresponding header rewriting entry includes the packet’s BAP routing ID in the ingress topology and the packet’s BAP routing ID in the egress topology:

We agreed during online session:

- For the two scenario of inter-topology routing and intra-to-inter-topology re-routing, there is only one header rewriting for a packet, where the header rewriting entry includes the BAP routing ID of the packet’s ingress topology and the BAP routing ID of the packet’s egress topology.

Issue: For inter-to-intra-topology re-routing, three options were proposed in R2-2201690.

Based on online discussion, these options are reworded in the following manner:

Option 1: no header rewriting is applied for inter-to-intra-topology re-routing. This implies that in the ingress topology, the packet must carry a BAP address of a donor-DU in the same topology.

Option 2: header rewriting is applied for inter-to-intra-topology re-routing, where the header-rewriting entry contains the packet’s ingress BAP routing ID and the BAP routing ID of the packet’s egress topology after inter-to-intra re-routing.

Option 3: header rewriting is applied for inter-to-intra-topology re-routing, where the header-rewriting entry contains the packet’s intended egress topology after inter-topology routing and the BAP routing ID of the packet’s egress topology after inter-to-intra re-routing:

Questions raised during online discussion: Is header rewriting performed before routing?

Answer: The execution of header rewriting always occurs before the execution of routing as shown here:

Option 1:

- Lookup inter-topology rewriting entries for top1 top2: Match found

- Since top2 egress link is not available:

  - Apply routing in top1

Option 2:

- Lookup inter-topology rewriting entries for top1 top2: Match found

- Since top2 egress link is not available,

  - Lookup inter-to-intra rewriting entries based on ingress BAP routing ID: Match found

    - Rewrite header based on match

    - Route packet in top1

Option 3:

- Lookup inter-topology rewriting entries for top1 top2: Match found

- Determine intended egress BAP routing ID based on match

- Since top2 egress link is not available,

  - Lookup inter-to-intra rewriting entries based on intended egress BAP routing ID: Match found

    - Rewrite header based on match

    - Route packet in top1

Note that all options require lookup for header rewriting entries for inter-topology-routing. Further, options 2 and 3 require lookup of header rewriting entries for inter-to-intra topology rerouting.

Proposal 6: For the scenario of inter-to-intra-topology re-routing, select one of the following options:

- Option 1: No header rewriting is applied, and the upstream packet’s BAP routing ID in the ingress topology contains the BAP address of the IAB-donor-DU in the same topology.

- Option 2: Header rewriting is applied based on a header-rewriting entry, which contains the packet’s ingress BAP routing ID and the BAP routing ID of the packet’s egress topology after inter-to-intra re-routing.

- Option 3: Header rewriting is applied based on a header-rewriting entry, which contains the BAP routing ID of the packet’s intended egress topology after inter-topology routing and the BAP routing ID of the packet’s egress topology after inter-to-intra re-routing.

- Option 4: The boundary node is configured with a default BAP routing ID for each topology via RRC, and such default BAP routing ID can be used as the egress routing ID when applying inter-topology rerouting.

Q6: Do you agree with P6? If not, please propose rewording.

| Company | Yes/No | Comments, proposed rewording |
| --- | --- | --- |
| Samsung | See comment | We are not in favour of option 1 since inter-topology routing is triggered when the original routing path in CU1’s topology has a problem. So, there is no need to re-route the packet via the original path. Then, we think rerouting is aimed at some abnormal case that won’t happen frequently. So, it is unnecessary to spend much configuration effort on configuring the header rewriting table for re-routing. Please note that, in general, the entries for header rewriting of rerouting needs XnAP signalling from CU2. With this in mind, we think a simple solution can be applied, i.e., in each topology, one default BAP routing ID is configured as the egress BAP routing ID. In particular, if the egress link in CU1’s topology is unavailable, the boundary node use default BAP routing ID in CU2’s topology for rewriting, if the egress link in CU2’s topology is unavailable, the boundary node use default BAP routing ID in CU1’s topology. With this solution, we can harmonize intra- to-inter rerouting and inter-to-intra rerouting, and also harmonize option 2&3 for inter-to-intra rerouting. So, we propose a unified option for re-routing, which we add as option 4 above: The boundary node is configured with a default BAP routing ID for each topology via RRC, and such default BAP routing ID can be used as the egress routing ID when applying inter-topology rerouting. |
| Kyocera | No | We think it’s still not excluded that the header rewriting is performed twice for inter-to-intra-topology re-routing, i.e., one header rewriting for inter-topology routing and another header rewriting for inter-to-intra-topology re-routing. |
| LGE | Yes, but | For option 1, considering that pseudo BAP address is used for inter-topology traffic, we wonder whether inter-to-intra-topology re-routing can be achieved without header rewriting. For option 2 and 3, we think that option 2 is more aligned with the agreements for intra-to-inter-topology re-routing which made in Monday online session, i.e., the BAP routing ID of the packet’s ingress topology and the BAP routing ID of the packet’s egress topology. However, if option 3 is selected, two different header rewriting configuration/behaviour may be needed. It would be good to have option 2 for defining unified solution to cover all local re-routing. |
| Fujitsu | See comment | Option 1 implies that for upstream traffic, the packet’s pseudo BAP routing ID in the ingress topology must contain the BAP address of the donor-DU in the same topology. RAN2 should really discuss what the pseudo BAP routing ID is. What the BAP address in the pseudo BAP routing ID refers to. It may affect the choice of options. For option 2, the first step “Lookup inter-topology rewriting entries for top1-> top2: Match found” may be skipped, if the boundary node checks the egress link availability first. All options are feasible with some assumptions. We prefer option 1 or 2, since in option 3, BAP header rewriting is applied twice. |
| ZTE | See comments | Agree with Kyocera that the header rewriting is performed twice for this scenario. Only after the first header rewriting of inter-topology routing, the BAP routing ID of the packet’s intended egress topology can be determined, which can be further used to determine whether header rewriting for inter-to-intra topology re-routing is necessary or not. A simple routing and re-routing procedure description is preferred and should be captured in the spec. The potential optimization for reducing the times of header rewriting can be up to implementation. |

Issue: How does the boundary node know if a header rewriting entry applies for the inter-topology-routing scenario, the intra-to-inter-topology re-routing scenario, or for inter-to-intra topology re-routing scenario (the latter only for options 2 and 3)?

R2-2200195 – QC, R2-2200566 – Fujitsu, R2-2201243 – Kyocera, R2-2201299 – HW, R2-2201322 – SS, R2-2201351 – ZTE propose to explicitly include information whether a header-rewriting entry applies for a re-routing scenario.

R2-2201052 – Nokia proposes not to have mappings of routing IDs for re-routing but rather perform re-routing based on BAP address only. In this case, header-rewriting entries only refer to inter-topology transport. The rapporteur emphasizes that RAN2 has made the agreement to “have rewriting mapping configuration(s) Old routing ID to New routing ID” for re-routing. RAN2 can discuss if further restrictions should be applied for re-routing (e.g., path ID is not affected etc.)

Proposal 7: The header rewriting configuration to include information that allows the boundary node to determine if a header rewriting entry applies for re-routing.

Q7: Do you agree with P7? If not, please propose rewording.

| Company | Yes/No | Comments, proposed rewording |
| --- | --- | --- |
| Samsung |  | If the option we introduced in Q6 is applied (option 4), such proposal is not needed. |
| Kyocera | Yes |  |
| LGE | Yes, but | If header rewriting configuration is used for header rewriting for local re-routing, this information is needed, but if header rewriting for local re-routing is performed based on routing configuration, this information may not be needed. |
| Fujitsu | Yes | Because NW may change the parent of each topology for the boundary node, the inter-donor DU rerouting table can be configured for each topology. |
| ZTE |  | Actually, we prefer separate header-writing configurations for routing and re-routing purpose respectively. In this case, Since RLF happens rarely, it is not necessary for IAB node that only need to perform inter-topology routing to check the routing table with a lot of re-routing entries. which is time consuming and meaningless. |

Issue: P5 proposed that the header rewriting scenario to include information on either egress topology, or ingress topology, or the traffic direction. The discussion for P5 did not consider re-routing. If re-routing is included, the question arises if all any of these three can be used or if some of them are not sufficient.

If Option 1 of Proposal 6 is used:

- There is no header rewriting entry for inter-to-intra-topology re-routing. Therefore, any of the three candidate indicators considered for inter-topology routing can be used.

If Option 2 of Proposal 6 is used:

- If the header-rewriting configuration includes information on the egress topology:

  - Rewriting entries for UL inter-topology routing and intra-to-inter-topology re-routing to the non-F1-terminating CU’s topology can be differentiated from rewriting entries for DL inter-topology routing and re-routing to the F1-terminating CU’s topology.

  - Additional re-routing information is needed to differentiate between the rewriting entries for UL inter-topology routing and UL intra-to-inter-topology re-routing to non-F1-terminating CU’s topology so that the latter entries are only used when the parent link in the F1-terminating CU’s topology is not available. This is information has already been captured in Proposal 7.

  - The information on re-routing is also needed to differentiate between the rewriting entries for DL inter-topology routing and UL inter-to-intra-topology re-routing, which both have F1-terminating CU’s topology as egress topology.

- If the header-rewriting configuration includes information on the ingress topology:

  - Rewriting entries for DL inter-topology routing from the non-F1-terminating CU’s topology can be differentiated from rewriting entries for UL inter-topology routing, intra-to-inter-topology re-routing and inter-to-intra-topology re-routing from the F1-terminating CU’s topology.

  - Additional re-routing information is needed to differentiate between the rewriting entries for UL inter-topology routing and intra-to-inter-topology re-routing to the non-F1-terminating CU’s topology so that the latter entries are only used when the parent link in the F1-terminating CU’s topology is not available.

  - The rerouting information is also needed to differentiate between the rewriting for UL inter-topology routing to the non-F1-terminating CU’s topology and inter-to-intra-topology rerouting to the F1-terminating CU’s topology since both entries have the same ingress topology.

  - No further information is needed to differentiate between inter-to-intra-topology re-routing and intra-to-inter-topology re-routing since they use different ingress topologies.

- If the header-rewriting configuration includes information on the traffic direction:

  - Since the DL and UL rewriting have different ingress topologies, the analysis is the same as for the prior bullet.

If Option 3 of Proposal 6 is used:

- If the header-rewriting configuration includes information on the egress topology:

  - Rewriting entries for UL inter-topology routing and UL intra-to-inter topology rerouting to the non-F1-terminating CU’s topology can be differentiated from rewriting entries for DL inter-topology routing and inter-to-intra topology rerouting to the F1-terminating CU’s topology.

  - Additional re-routing information is needed to differentiate between the rewriting entries for UL inter-topology routing and intra-to-inter-topology re-routing to the non-F1-terminating CU’s topology so that the latter entries are only used when the parent link in the F1-terminating CU’s topology is not available. This is information has already been captured in Proposal 7.

  - No further information is needed to differentiate between the rewriting entries for DL inter-topology routing and the second rewriting for inter-to-intra topology re-routing to the F1-terminating CU’s topology since they can be differentiated by the re-routing information.

- If the header-rewriting configuration includes information on the ingress topology:

  - Rewriting entries for UL inter-topology routing and intra-to-inter topology re- from the F1-terminating CU’s topology can be differentiated from rewriting entries for DL inter-topology routing and the second rewriting for inter-to-intra-topology re-routing from the non-F1-terminating CU’s topology.

  - Additional re-routing information is needed to differentiate between the rewriting entries for UL inter-topology routing and intra-to-inter-topology re-routing to the non-F1-terminating CU’s topology so that the latter entries are only used when the parent link in the F1-terminating CU’s topology is not available. This is information has already been captured in Proposal 7.

  - No further information is needed to differentiate between the rewriting entries for DL inter-topology routing and the second rewriting for inter-to-intra topology re-routing to the F1-terminating CU’s topology since they can be differentiated by the re-routing indicator.

- If the header-rewriting configuration includes information on the traffic direction:

  - Rewriting entries for DL inter topology routing are differentiated from UL inter-topology routing and UL intra-to-inter topology re-routing.

  - Additional rerouting information is needed to differentiate between UL inter-topology routing and UL intra-to-inter topology re-routing so that the latter entries are only used when the parent link in the F1-terminating CU’s topology is not available. This is information has already been captured in Proposal 7.

  - The re-routing information is also needed to differentiate between the rewriting entries for UL inter-topology routing and the second rewriting for inter-to-intra-topology re-routing since they both refer to UL traffic but the previous routing IDs belong to different topologies.

  - Additional information is needed to differentiate between rewriting entries for intra-to-inter topology re-routing to the non-F1-terminating CU’s topology and for the second rewriting for inter-to-intra topology re-routing to the F1-terminating CU’s topology since they both refer to UL, both include the re-routing indicator, but they the previous routing IDs belong to different topologies.

Based on the above:

- For intra-to-inter-topology re-routing Options 1 and 2, the header rewriting configuration can include information related to the egress topology, or the ingress topology or the traffic direction.

- For intra-to-inter-topology re-routing Option 3, the rewriting entry can include information related to the egress topology or the ingress topology. If it includes information on the traffic direction, additional information would be required so that the boundary node can differentiate between intra-to-inter-topology re-routing and inter-to-intra-topology re-routing entries. For this reason, we may only want to consider the information related to one of ingress or egress topology for Option 3.

(If P5 has been agreed and inter-to-intra-topology re-routing Option 1 or Option 2 has been selected)

Proposal 8a: RAN2 to determine if the boundary node differentiates header rewriting entries based on egress topology, ingress topology or traffic direction.

(If P5 has been agreed and inter-to-intra-topology re-routing Option 3 has been selected)

Proposal 8b: RAN2 to determine if the boundary node differentiates header rewriting entries based on egress topology or ingress topology.

Q8: Do you agree to P8a/b? Which of the sub-options proposed in 8a and 8b do you prefer?

| Company | Yes/No | Which sub-options proposed in 8a/8b do you prefer? Other comments. |
| --- | --- | --- |
| Samsung |  | If option 4 in Q6 is selected, such discussion is not needed. |
| Kyocera | Yes | We prefer Proposal 8a. |
| LGE | See comment | In our understanding, as you addressed above, if P5 and P6 is agreed, one of two options above should be agreed. So, we wonder whether preference between two proposals should be expressed here. Anyway we prefer P8a. |
| Fujitsu | 8a | We agree with 8a, and prefer traffic direction as indicator. There are 3 types of rewriting info for both inter-donor DU rerouting and inter-CU routing: Top1=>top1 rewriting info (uplink), top1=>top2 rewriting info (uplink) and top2=>top1 rewriting info (downlink). |
| ZTE | Yes | We prefer the proposal 8b. Whether egress or ingress topology is used can be further discussed. |

## 2.6	BH RLC CH mapping configuration of the boundary node

Several contributions referred to (one or multiple) “BH RLC CH mapping tables”. The rapporteur would like to clarify that the “table” is an implementation-based concept. Instead, we should refer to “BH RLC CH mapping configurations” or “BH RLC CH mapping entries”.

Issue: How does the boundary node know if a BH RLC CH mapping entry applies to inter-topology DL vs. inter-topology UL vs. intra-topology DL vs. intra-topology UL transport?

R2-2200195 – QC proposes to add one indicator for inter-topology vs. intra-topology and a second indicator for egress topology to each BH RLC CH mapping entry.

R2-2200808 – vivo proposes to add one indicator for inter-topology vs. intra-topology and a second indicator for the traffic direction to each BH RLC CH mapping entry.

R2-2200842 – Canon, R2-2201299 – HW, R2-2201322 – Samsung propose to add one indicator for the ingress topology (that of the prior-hop node) and a second indicator for the egress topology (that of the next-hop node) to each BH RLC CH mapping entry.

R2-2200352 – Intel proposes to include no topology info for each BH RLC CH in BH RLC CH mapping entry. The rapporteur emphasizes that there will be ambiguity in the BH RLC CH mapping configuration since BAP addresses of the prior-hop node or next-hop node may be reused in the two topologies.

R2-2201351 – ZTE, R2-2201606 – E/// propose to add/consider a single indicator of inter-topology vs. intra-topology (also referred to as “concatenated traffic” vs. “non-concatenated traffic”) to the BH RLC CH mapping entry. The rapporteur emphasizes that there would be ambiguity between BH RLC CH mapping entries labelled as “inter-topology” e.g., an inter-topology UL entry and an inter-topology DL entry may share the prior-hop BAP address and/or the next-hop BAP address since they are configured by different CUs.

Based on these contributions, there are three options for BH RLC CH mapping at the boundary node:

- Option A: The BH RLC CH mapping configuration includes information for the boundary node to differentiate mappings based on inter-topology- vs. intra-topology and based on the egress topology.

- Option B: The BH RLC CH mapping configuration includes information for the boundary node to differentiate mappings based on inter-topology- vs intra-topology and based on the traffic direction.

- Option C: The BH RLC CH mapping configuration includes information for the boundary node to differentiate mappings based on ingress topology and egress topology.

Proposal 9: RAN2 to select one of the following options for BH RLC CH mapping configuration of the boundary node:

- Option A: The BH RLC CH mapping configuration includes information for the boundary node to differentiate mappings based on inter-topology- vs. intra-topology and based on the egress topology.

- Option B: The BH RLC CH mapping configuration includes information for the boundary node to differentiate mappings based on inter-topology- vs intra-topology and based on traffic direction.

- Option C: The BH RLC CH mapping configuration includes information for the boundary node to differentiate mappings based on ingress topology and egress topology.

Q9: Do you agree to P9? Can we even agree which of the Options A, B or C?

| Company | Yes/No | Comments, proposed rewording |
| --- | --- | --- |
| Samsung |  | Option C |
| Kyocera | Yes | We prefer Option B, whereby we think the traffic direction may be where the mapping configuration is used in, i.e., IAB-DU part or IAB-MT part, as commented in Q3 and Q5 above. |
| LGE | Yes | Prefer Option C |
| Fujitsu | Yes | Option C is most straightforward. |
| ZTE | Yes | Option A |

## 2.7	UL mapping configuration of the boundary node

Issue: How does the boundary node know what topology an UL mapping configuration refers to?

Again, the UL mapping configuration is provided via F1AP by the F1-terminating CU.

R2-2200195 – QC, R2-2200808 – vivo propose that an UL mapping entry includes information for the boundary node to determine the egress (UL) topology.

Proposal 10: The UL mapping configuration to include information for the boundary node to determine the egress topology of each UL mapping entry.

Q10: Do you agree to P10?

| Company | Yes/No | Comments, proposed rewording |
| --- | --- | --- |
| Samsung | Yes |  |
| Kyocera | Yes |  |
| LGE | Yes |  |
| Fujitsu | Yes |  |
| ZTE | Yes |  |

## 2.8	Identification of a topology

Issue: How to refer to a “topology” in a configuration?

R2-2200195 – QC proposes that a topology in a configuration is referred to as “F1-terminating CU’s topology” vs. “non-F1-terminating CU’s topology.”

R2-2201322 – Samsung proposes two methods to identify a topology in a configuration: either include an indicator of “source topology vs. target topology” in migration/recovery scenarios and an indicator of “MCG topology/SCG topology” in redundancy scenario, or include a topology ID.

The rapporteur believes a common indicator of the topology should be used for partial migration, partial RLF recovery and inter-donor redundancy scenarios. The rapporteur further believes that an abstract topology ID, e.g., “top1 vs. top2” still needs to be mapped to a physical link.

Proposal 11: In configurations, the topology is referred to as “F1-terminating CU’s topology” vs. “non-F1-terminating CU’s topology”.

Q11: Do you agree to P11?

| Company | Yes/No | Comments, proposed rewording |
| --- | --- | --- |
| Samsung | Yes | For simplicity, it can be “F1-terminating topology” vs. “non-F1-terminating topology” |
| Kyocera | Yes, but… | We just wonder how the F1-terminating CU (or non-F1-terminating CU) can be identified, from L2/L3 point of view. |
| LGE | Yes | From RAN2 perspective, identification of a topology would be mapped to an ingress or egress physical link to identify the required topology of the BAP PDU correctly. |
| Fujitsu | Yes |  |
| ZTE | Yes |  |

## 2.9 	BAP modelling

Issue: Which entity of the boundary node determines/executes header rewriting?

R2-2200195 – QC, R2-2201243 – Kyocera, R2-2201322 – SS, R2-2201429 – LG, R2-2201606 – E/// propose that the TX BAP entity performs header rewriting.

R2-2201299 – HW proposes that the TX BAP entity performs header rewriting in upstream and the RX BAP entity performs header rewriting in downstream.

In Rel-16, the BAP modelling did not differentiate between UL and DL direction. The rapporteur believes that we should keep this approach for Rel-17.

Proposal 12: Determination/execution of header rewriting is handled by the BAP TX entity.

Q12: Do you agree to P12?

| Company | Yes/No | Comments, proposed rewording |
| --- | --- | --- |
| Samsung | Yes |  |
| Kyocera | Yes |  |
| LGE | Yes |  |
| Fujitsu | Yes |  |
| ZTE | Yes |  |

Issue: Is egress link selection performed before or after header rewriting?

R2-2201052 – Nokia, R2-2200760 – Lenovo propose to have egress link selection before header rewriting.

R2-2201243 – Kyocera, R2-2201299 – HW, R2-2201351 – ZTE propose to have egress link selection after header rewriting.

It seems that “egress link selection” refers to routing, which should not occur before header rewriting. The rapporteur believes details can be handled in the running CR for TS 38.340.

## 2.10	Others

Several companies proposed to constrain the selection criteria for pseudo-BAP routing IDs. The rapporteur believes this can be left to implementation.

R2-2201322 – Samsung proposes to determine header rewriting based on both the ingress routing ID and the ingress BH RLC CH. The rapporteur recognizes that this proposal allows to split traffic of same BAP routing ID across MCG and SCG paths in upstream for a dual-connected node. The rapporteur believes that such optimizations can be discussed if time remains.

R2-2201606 – E/// proposes that “configuration for re-routing of certain BH traffics due to load balancing” is a trigger to apply BAP header rewriting for inter-donor routing. The rapporteur does not understand the proposal. RAN2 only discussed re-routing due to congestion but not for load-balancing. Further, re-routing due to congestion is handled in the same manner as re-routing due to RLF.

R2-2201606 – E/// proposes that type-2 RLF indication may trigger header rewriting for re-routing. The rapporteur believes that RAN2 has not agreed that re-routing due to type-2 RLF indication obtains special treatment. This matter should be handled as part of the RLF indication discussion.

# 3	Conclusion

…

# 4	References

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2200195 | Open issues on BAP routing | Qualcomm Incorporated |
| R2-2200325 | On BAP Routing and Rerouting | CATT |
| R2-2200352 | Open issues on BAP routing for inter-donor topology routing | Intel Corporation |
| R2-2200566 | Discussion on the routing issues | Fujitsu |
| R2-2200760 | Discussion on remaining issues for IAB rerouting | Lenovo, Motorola Mobility |
| R2-2200808 | Remaining Issues of Intra/Inter-Topology Routing | vivo |
| R2-2200842 | Discussion on the configuration of a boundary node | CANON Research Centre France |
| R2-2200907 | Introduce cost factor in local re-routing | Sony |
| R2-2200918 | BAP Header Rewriting Configuration | Sony |
| R2-2201052 | IAB inter-CU (re)routing issues | Nokia, Nokia Shanghai Bell |
| R2-2201053 | IAB CP-UP separation remaining issues | Nokia, Nokia Shanghai Bell |
| R2-2201243 | Details of routing and re-routing enhancements for eIAB | Kyocera |
| R2-2201299 | Leftover issues for BAP header rewriting based (re)routing | Huawei, HiSilicon |
| R2-2201322 | Discussion on the inter-CU routing | Samsung Electronics France SA |
| R2-2201351 | Discussion o BAP routing and rerouting | ZTE, Sanechips |
| R2-2201429 | Open issues for BAP routing operation | LG Electronics Inc. |
| R2-2201430 | Text Proposal of TS 38.340 for BAP routing operation | LG Electronics Inc. |
| R2-2201606 | Boundary IAB node behaviour for partial inter-donor migration | Ericsson |