3GPP TSG-RAN WG2 Meeting #116bis electronic										R2-22xxxxx

Online, January 17 – 25, 2022

Agenda Item:		8.13.1

Souce:	Samsung

Title:					[AT116bise][801][SON/MDT] Reply LS on NR-U (Samsung)

Document for:		Discussion

- 1. Introduction

This paper addresses the following email discussion:

- [AT116bise][801][SON/MDT] Reply LS on NR-U (Samsung)

Draft reply LS for R2-2200103. R2-2200664 can be used as baseline.

Intended outcome: LS ready for being approved.

Deadline: 22:22 UTC, Monday Week2

This offline discussion is progressed in two phases.

Phase 1:

Input from companies is provided for each issue.

Companies are invited to provide their comments by the deadline, 22:22 UTC, Friday Jan 21.

Phase 2:

Input from companies on the summary and the draft Reply LS is provided.

Companies are invited to provide their comments by the deadline, 22:22 UTC, Monday Jan 24.

To aid better communication between the respective delegates handling this topic from different companies, it is requested to fill-in the contact information:

Contact Information

| Company | Contact person | Email |
| --- | --- | --- |
| Samsung | Sangbum Kim | Sb07.kim@samsung.com |
| Apple | Sasha Sirotkin | ssirotkin@apple.com |

- 2.	Discussion

In the context of SON optimization for NR-U, RAN3 has agreed to support NR-U in Mobility Load Balancing and identified that it is beneficial to exchange load metrics on a per cell and per NR-U channel granularity between base stations. In order to define the load metrics in network interfaces and keep in line with the concept in RAN1 and RAN2, RAN3 has provided the following questions to RAN1 and RAN2 [1].

Companies are invited to provide the view on the following questions. For easy compromise, the draft answers in [2] can be used as baseline.

#### Issue 1: How should an NR-U channel be represented?

A possible description identified by RAN3 for the NR-U channel representation is as following. In this representation an NR-U channel can be recognized via its centre frequency and bandwidth.

| NR-U Channel List |  | 0..1 |  |
| --- | --- | --- | --- |
| >NR-U Channel Item |  | 1..<maxnoofNR-UChannels> |  |
| >>Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannels, …) |
| >>NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) |
| >>Bandwidth | M |  | ENUMERATED (10Mhz, 20Mhz. …) |

RAN3 would like to check with RAN1 and RAN2 if the above information is enough to identify a NR-U channel or if more details are needed.

In order to define the channel occupancy time of an NG-RAN node, RAN3 also discussed whether channel sensing is carried out even when no data needs to be transmitted or whether channel sensing is performed only when the NG-RAN node needs to exchange traffic over the NR-U channel.

The draft answer from [2] is as follows:

From RAN2 perspective, 'NR ARFCN' and 'Bandwidth' are sufficient, and 'Channel ID' which does not exist in RAN2 specifications is not needed.

Q1: Do companies agree the draft answer above? If not or needed to be updated, please provide view in the Comments below.

| Company | Agree/disagree | Comments |
| --- | --- | --- |
| Samsung | Agree | Proponent |
| Apple | Agree |  |

Rapporteur Summary:

TBD

#### Issue 2:  According to current specifications, is an NG-RAN node supposed to sense the NR-U channel even when no data needs to be transmitted or is channel sensing performed only when the NG-RAN node needs to exchange traffic over the NR-U channel?

The draft answer from [2] is as follows:

It is not specified in RAN2 specifications, but RAN2 understands that it is up to network implementation so both would be possible.

Q2: Do companies agree the draft answer above? If not or needed to be updated, please provide view in the Comments below.

| Company | Agree/disagree | Comments |
| --- | --- | --- |
| Samsung | Agree | Proponent |
| Apple | Agree | Maybe it would be better to simply state that “RAN2 does not specify…” |

Rapporteur Summary:

TBD

Furthermore, RAN3 discussed exchanging Energy Detection (ED) threshold between RAN nodes and have two more questions:

#### Issue 3: How is the ED threshold configured in RAN node?

The draft answer from [2] is as follows:

Since it’s about how to decide the ED threshold in RAN node, RAN2 understands that it is specified in RAN1 specification (i.e. TS 37.213, subclause 4.1.5). Hence, RAN1 confirmation would be required.

Q3: Do companies agree the draft answer above? If not or needed to be updated, please provide view in the Comments below.

| Company | Agree/disagree | Comments |
| --- | --- | --- |
| Samsung | Agree | Proponent |
| Apple | Agree | Perhaps it is better to delegate the answer to RAN1 instead of asking for confirmation. |

Rapporteur Summary:

TBD

#### Issue 4: What is the ED threshold granularity (per channel, per cell, per UE…)?

The draft answer from [2] is as follows:

Since it’s about how to decide the ED threshold in RAN node, RAN2 understands that it is specified in RAN1 specification (i.e. TS 37.213, subclause 4.1.5). Hence, RAN1 confirmation would be required.

Cf. ED threshold for the UE can be configured under ServingCellConfig, which usually contains the configuration per cell.

Q4: Do companies agree the draft answer above? If not or needed to be updated, please provide view in the Comments below.

| Company | Agree/disagree | Comments |
| --- | --- | --- |
| Samsung | Agree | Proponent |
| Apple | Agree | Perhaps it is better to delegate the answer to RAN1 instead of asking for confirmation. |

Rapporteur Summary:

TBD

- 3. Conclusion

TBD

# 4. Reference

[1] R2-2200103	LS on NR-U channel information and procedures (R3-216042; contact: Samsung)	RAN3	LS in	Rel-17	NR_ENDC_SON_MDT_enh-Core	To:RAN1, RAN2

[2] R2-2200664	[Draft] Reply LS on NR-U channel information and procedures		Samsung	LS out		Rel-17	NR_ENDC_SON_MDT_enh-Core	To:RAN3	Cc:RAN1