3GPP TSG-RAN WG2 Meeting #116bis electronic										R2-22xxxxx

Online, January 17 – 25, 2022

Agenda Item:		8.13.1

Souce:	Samsung

Title:					[AT116bise][801][SON/MDT] Reply LS on NR-U (Samsung)

Document for:		Discussion

- 1. Introduction

This paper addresses the following email discussion:

- [AT116bise][801][SON/MDT] Reply LS on NR-U (Samsung)

Draft reply LS for R2-2200103. R2-2200664 can be used as baseline.

Intended outcome: LS ready for being approved.

Deadline: 22:22 UTC, Monday Week2

This offline discussion is progressed in two phases.

Phase 1:

Input from companies is provided for each issue.

Companies are invited to provide their comments by the deadline, 22:22 UTC, Friday Jan 21.

Phase 2:

Input from companies on the summary and the draft Reply LS is provided.

Companies are invited to provide their comments by the deadline, 22:22 UTC, Monday Jan 24.

To aid better communication between the respective delegates handling this topic from different companies, it is requested to fill-in the contact information:

Contact Information

| Company | Contact person | Email |
| --- | --- | --- |
| Samsung | Sangbum Kim | Sb07.kim@samsung.com |
| Apple | Sasha Sirotkin | ssirotkin@apple.com |
| Ericsson | Marco Belleschi | Marco.belleschi@ericsson.com |
| Huawei, HiSilicon | Jun Chen | jun.chen@huawei.com |
| Qualcomm | Rajeev Kumar | rkum@qti.qualcomm.com |
| Intel | Candy Yiu | Candy.yiu@intel.com |
| Sharp | Ningjuan Chang | ningjuan.chang@cn.sharp-world.com |
| Lenovo | Le Yan | yanle1@lenovo.com |
| CATT | ShiJie | shijie@catt.cn |
| ITRI | Nai-Lun Huang | NellenHuang@itri.org.tw |

- 2.	Discussion

In the context of SON optimization for NR-U, RAN3 has agreed to support NR-U in Mobility Load Balancing and identified that it is beneficial to exchange load metrics on a per cell and per NR-U channel granularity between base stations. In order to define the load metrics in network interfaces and keep in line with the concept in RAN1 and RAN2, RAN3 has provided the following questions to RAN1 and RAN2 [1].

Companies are invited to provide the view on the following questions. For easy compromise, the draft answers in [2] can be used as baseline.

#### Issue 1: How should an NR-U channel be represented?

A possible description identified by RAN3 for the NR-U channel representation is as following. In this representation an NR-U channel can be recognized via its centre frequency and bandwidth.

| NR-U Channel List |  | 0..1 |  |
| --- | --- | --- | --- |
| >NR-U Channel Item |  | 1..<maxnoofNR-UChannels> |  |
| >>Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannels, …) |
| >>NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) |
| >>Bandwidth | M |  | ENUMERATED (10Mhz, 20Mhz. …) |

RAN3 would like to check with RAN1 and RAN2 if the above information is enough to identify a NR-U channel or if more details are needed.

In order to define the channel occupancy time of an NG-RAN node, RAN3 also discussed whether channel sensing is carried out even when no data needs to be transmitted or whether channel sensing is performed only when the NG-RAN node needs to exchange traffic over the NR-U channel.

The draft answer from [2] is as follows:

From RAN2 perspective, 'NR ARFCN' and 'Bandwidth' are sufficient, and 'Channel ID' which does not exist in RAN2 specifications is not needed.

Q1: Do companies agree the draft answer above? If not or needed to be updated, please provide view in the Comments below.

| Company | Agree/disagree | Comments |
| --- | --- | --- |
| Samsung | Agree | Proponent |
| Apple | Agree |  |
| Ericsson | Partially (the specific BPW should be indicated instead of channel ID) | The UE performs the LBT operation per its active BWP (as specified in TS 38.300, clause 5.6 for access to the shared spectrum). The current method of indicating the BWP’s frequency location is based on locationAndBandwidth and subcarrierSpacing fields in the BWP IE. In our understanding, this information is needed to know which channel related LBT is being performed based on this BWP. Thus, some indication of the BWP is required. Therefore, we should include the locationAndBandwidth + subcarrierSPacing |
| Nokia | Agree |  |
| Huawei, HiSilicon | Agree |  |
| Qualcomm | Agree |  |
| Intel | Agree |  |
| Sharp | Agree |  |
| Lenovo | Agree |  |
| CATT | Agree |  |
| ZTE | Agree |  |
| ITRI | Agree |  |

Rapporteur Summary:

Agree 11/12

Partially agree 1/12

All companies agreed the draft answer. One company wanted to add BWP-related information, but any other companies didn’t provide support on the additional information.

From the inputs, rapporteur would like to suggest the following RAN2 answer:

From RAN2 perspective, 'NR ARFCN' and 'Bandwidth' are sufficient, and 'Channel ID' which does not exist in RAN2 specifications is not needed.

#### Issue 2:  According to current specifications, is an NG-RAN node supposed to sense the NR-U channel even when no data needs to be transmitted or is channel sensing performed only when the NG-RAN node needs to exchange traffic over the NR-U channel?

The draft answer from [2] is as follows:

It is not specified in RAN2 specifications, but RAN2 understands that it is up to network implementation so both would be possible.

Q2: Do companies agree the draft answer above? If not or needed to be updated, please provide view in the Comments below.

| Company | Agree/disagree | Comments |
| --- | --- | --- |
| Samsung | Agree | Proponent |
| Apple | Agree | Maybe it would be better to simply state that “RAN2 does not specify…” |
| Ericsson | Agree with the intention but we should refer to TS 38.300 in the reply. | It is up to the implementation, but TS 38.300 it is quite clear on when the LBT may be applied by the NG-RAN node. As proposed in our paper R2-2201605 (see annex in that paper), we can indicate as follows: ‘According to TS 38.300 Section 5.6.1, the NG-RAN node may apply LBT in order to transmit packets to UEs over the air interface. It is not specified in 3GPP specifications whether the NG-RAN node can sense the NR-U channel even when no data are available for transmission.’ |
| Nokia | Agree |  |
| Huawei, HiSilicon | Agree | We think Ericsson’s wording is more clear. |
| Qualcomm | Agree |  |
| Intel | Agree | Prefer also to either use Ericsson reply or include in the reply. |
| Sharp | Agree | Ericsson’s suggestion is more clear. |
| Lenovo | Agree | E///’s wording seems better. |
| CATT | Agree |  |
| ZTE | Agree |  |
| ITRI | Agree | Ericsson’s wording is more clear. |

Rapporteur Summary:

Agree 11/12

Agree but suggest to update 1/12

All companies agreed the intention of the draft answer, and most companies supported to update with the suggestion from the R2-2201605.

From the inputs, rapporteur would like to suggest the following RAN2 answer:

According to TS 38.300 Section 5.6.1, the NG-RAN node may apply LBT in order to transmit packets to UEs over the air interface. It is not specified in 3GPP specifications whether the NG-RAN node can sense the NR-U channel even when no data are available for transmission.

Furthermore, RAN3 discussed exchanging Energy Detection (ED) threshold between RAN nodes and have two more questions:

#### Issue 3: How is the ED threshold configured in RAN node?

The draft answer from [2] is as follows:

Since it’s about how to decide the ED threshold in RAN node, RAN2 understands that it is specified in RAN1 specification (i.e. TS 37.213, subclause 4.1.5). Hence, RAN1 confirmation would be required.

Q3: Do companies agree the draft answer above? If not or needed to be updated, please provide view in the Comments below.

| Company | Agree/disagree | Comments |
| --- | --- | --- |
| Samsung | Agree | Proponent |
| Apple | Agree | Perhaps it is better to delegate the answer to RAN1 instead of asking for confirmation. |
| Ericsson | Disagree | The ED threshold is used in RAN1 spec, but the configuration is in RRC specification. Hence RAN2 can indicate to RAN3 how the ED threshold is configured in our specification without involving RAN1 (see below ASN.1): Based on the above ASN.1 and the related field description, we propose to reply as follows (see R2-2201605 and the annex therein): “The energy detection threshold configuration is configured as part of the NR-U channel access configuration, which in turn is included in the serving cell configuration, according to TS 38.331. The energy detection threshold can be configured as an offset to the default maximum energy detection threshold value, or as an absolute configurable maximum energy detection threshold value.” Please note that the RAN3 LS was also sent to RAN1 (RAN1 is in the To list), hence RAN1 is expected to provide their reply from their point of view (as pointed out by Huawei below). We can just reply for our RAN2 point of view indicating that the energy detection threshold is configured to the UE either as an offset to the default maximum energy detection threshold value, or as an absolute configurable maximum energy detection threshold value. |
| Nokia | Agree | This is part of Channel Access Config, referring to RAN1 specification. RRC spec does only refer to TS37.213 (also as stated in the field description), thus RAN1 confirmation would be required. |
| Huawei, HiSilicon | Disagree | The original LS was sent to RAN1 and RAN2. From RAN2 point of view, we should provide our feedbacks based on our specifications, so we tend to agree with Ericsson’s wording. RAN1 can check their specs and provide their feedbacks. |
| Qualcomm | Agree | Agree with Nokia. |
| Intel | Agree | May need to massage the wording but agree in general. |
| Sharp | Agree | This threshold is actually a RAN1 parameter, so need RAN1 confirmation. |
| Lenovo | Agree | Even though it is RAN2 configures ED threshold in TS38.331, RAN2 needs to check with RAN1 how to represent ED threshold. |
| CATT | Agree |  |
| ZTE | Agree | RAN1 confirmation is needed |
| ITRI | Agree | RAN1 confirmation would be required. |

Rapporteur Summary:

Agree 10/12

Disagree 2/12

10 companies agreed the intention of the draft answer, i.e. to need RAN1 confirmation, and 2 companies suggested to explain RRC signalling.

Actually, RAN3 asked the question while discussing “exchanging Energy Detection (ED) threshold between RAN nodes”. Thus, rapporteur have assumed to require RAN1 confirmation because the RAN1 specification, TS37.213 describes how to decide the ED threshold in RAN node.

As compromise, rapporteur would like to suggest the following RAN2 answer:

Regarding how to determine the ED threshold in RAN node, RAN2 understands that it is specified in RAN1 specification (i.e. TS 37.213 Section 4.1.5). Hence, RAN1 confirmation would be required.

In RAN2 point of view, the energy detection threshold is configured to the UE either as an offset to the default maximum energy detection threshold value, or as an absolute configurable maximum energy detection threshold value.

#### Issue 4: What is the ED threshold granularity (per channel, per cell, per UE…)?

The draft answer from [2] is as follows:

Since it’s about how to decide the ED threshold in RAN node, RAN2 understands that it is specified in RAN1 specification (i.e. TS 37.213, subclause 4.1.5). Hence, RAN1 confirmation would be required.

Cf. ED threshold for the UE can be configured under ServingCellConfig, which usually contains the configuration per cell.

Q4: Do companies agree the draft answer above? If not or needed to be updated, please provide view in the Comments below.

| Company | Agree/disagree | Comments |
| --- | --- | --- |
| Samsung | Agree | Proponent |
| Apple | Agree | Perhaps it is better to delegate the answer to RAN1 instead of asking for confirmation. |
| Ericsson | Disagree (it should be UE-specific per cell) | Similar to the previous question, the ED threshold granularity is specified in RAN2 specifications, hence RAN2 has to reply to this not RAN1. In particular, the ED threshold is configured within the servingCellConfig and whatever that is configured in servingCelllConfig is a UE specific configuration. Thus, two UEs connected to the same cell can be configured with different ED threshold values. CellGroupConfig  spCellConfig  spCellConfigDedicated  ServingCellConfig -> channelAccessConfig  energyDetectionConfig maxEnergyDetectionThreshold This is an important information to be captured in the reply LS and we can reply as follows: “As it is described in the reply to Q3, the ED threshold is configured in a dedicated way to a specific UE per serving cell.” |
| Nokia | Agree | Since RRC spec refers to TS 37.213, subclause 4.1.5, this configuration is set according to RAN1 specification. |
| Huawei, HiSilicon | See our comments | We think that above answer and Ericsson’s wording have the following common part: ED threshold for the UE can be configured under ServingCellConfig, which usually contains the configuration per cell. So this part can be used in the RAN2 response LS and we think Ericsson’s wording is more clear. Similar as our responses for Q3, RAN2 should reply based on RAN2 specifications. |
| Qualcomm | Agree |  |
| Intel | Agree |  |
| Sharp | Agree |  |
| Lenovo | Agree | As commentsed in Q3, RAN2 needs to check with RAN1 how to represent ED threshold. |
| CATT | Agree |  |
| ZTE | Agree | RAN1 confirmation is needed |
| ITRI | Agree | RAN1 confirmation would be required. |

Rapporteur Summary:

Agree 10/12

Disagree 2/12

10 companies agreed the intention of the draft answer, i.e. to need RAN1 confirmation, and 2 companies suggested to explain RRC signalling.

As compromise as in Q3, rapporteur would like to suggest the following RAN2 answer:

Regarding how to determine the ED threshold in RAN node, RAN2 understands that it is specified in RAN1 specification (i.e. TS 37.213 Section 4.1.5). Hence, RAN1 confirmation would be required.

In RAN2 point of view, ED threshold for the UE can be configured under ServingCellConfig, which usually contains the configuration per cell.

- 3. Conclusion

Rapporteur would like to suggest the following answers:

Regarding the question 1: How should an NR-U channel be represented?

Answer from RAN2:

From RAN2 perspective, 'NR ARFCN' and 'Bandwidth' are sufficient, and 'Channel ID' which does not exist in RAN2 specifications is not needed.

Regarding the question 2:  According to current specifications, is an NG-RAN node supposed to sense the NR-U channel even when no data needs to be transmitted or is channel sensing performed only when the NG-RAN node needs to exchange traffic over the NR-U channel?

Answer from RAN2:

According to TS 38.300 Section 5.6.1, the NG-RAN node may apply LBT in order to transmit packets to UEs over the air interface. It is not specified in 3GPP specifications whether the NG-RAN node can sense the NR-U channel even when no data are available for transmission.

Regarding the question 3: How is the ED threshold configured in RAN node?

Answer from RAN2:

Regarding how to determine the ED threshold in RAN node, RAN2 understands that it is specified in RAN1 specification (i.e. TS 37.213 Section 4.1.5). Hence, RAN1 confirmation would be required.

In RAN2 point of view, the energy detection threshold is configured to the UE either as an offset to the default maximum energy detection threshold value, or as an absolute configurable maximum energy detection threshold value.

Regarding the question 4: What is the ED threshold granularity (per channel, per cell, per UE…)?

Answer from RAN2:

Regarding how to determine the ED threshold in RAN node, RAN2 understands that it is specified in RAN1 specification (i.e. TS 37.213 Section 4.1.5). Hence, RAN1 confirmation would be required.

In RAN2 point of view, ED threshold for the UE can be configured under ServingCellConfig, which usually contains the configuration per cell.

# 4. Reference

[1] R2-2200103	LS on NR-U channel information and procedures (R3-216042; contact: Samsung)	RAN3	LS in	Rel-17	NR_ENDC_SON_MDT_enh-Core	To:RAN1, RAN2

[2] R2-2200664	[Draft] Reply LS on NR-U channel information and procedures		Samsung	LS out		Rel-17	NR_ENDC_SON_MDT_enh-Core	To:RAN3	Cc:RAN1