3GPP TSG-RAN WG2 #117-e	R2-2202529

Electronic meeting, Feb 21st – Mar 3rd, 2022

Agenda Item:	8.12.2.2.1

Source:	Apple Inc

Title:	NCD-SSB and handover related aspects

Document for:	Discussion, Decision

# Introduction

In the pre-RAN2#117-e discussion [1], RAN2 discussed on the configuration options of providing the UE with measurement objects and their applicability with respect to the serving cell measurement object.

While there was no clear consensus (companies views were diverse on whether the serving cell measurement object should be associated with the CD-SSB or not, depending on the different deployment/measurement requirement scenarios from the NW perspective), the topic of using the NCD-SSB as part of handover was also discussed albeit briefly.

In this short paper, we intend to discuss the other handover dynamics in the view of NCD_SSB and their implications on the UE.

# 2	Discussion

We would like start the discussion by noting that the CD-SSB is expected to provide the essential information (QCL/TCI assumptions, beam information, SFN etc) that the UE can then expect the NCD-SSB to also be transmitted with.

In the case where the UE camps or re-selects to a cell before going into CONNECTED mode later, the UE is expected to have synced with the CD-SSB and also be update on the SIB1 content.

But in a handover where the UE is expected to sync to NCD-SSB for a BWP in which this RedCap UE operates; the question arises on whether:

Alternative 1: The UE should sync to CD-SSB first, and then using that information, sync to the NCD-SSB

Or

Alternative 2: The UE directly sync to the NCD-SSB as part of handover. For this to work, the NCD-SSB is expected to carry all the ‘essential’ information the UE expects to derive from the CD-SSB ( SFN, QCL, beam indices etc..)

Observation 1: RedCap UE design have two choices during the handover in case the handover message has a separate NCD-SSB the UE would be operating in: whether the UE first sync to CD-SSB to derive the necessary information to be used in syncing to the NCD-SSB, or, the UE directly sync to the NCD-SSB assuming that the NCD-SSB can also be used by itself to derive the needed information for the UE for the handover.

But it can be seen that the UE directly syncing to the NCD-SSB is more efficient in terms of overall system operation, and so if we can ensure that the NCD-SSB (that is configured to the UE) can provide the necessary information, then we can confirm in RAN2 on the expectations from the NCD-SSB. While RAN2 did agree to NCD-SSB having most of the information that CD-SSB carrier, as the same, it is essential that RAN2 discuss and agree this from a handover perspective.

Observation 2: It is more efficient for the RedCap UE to directly sync to the NCD-SSB at handover instead of syncing to the CD-SSB first.

In addition, the NCD-SSB might be different with different BWPs for the same UE (this is also something that RAN2 has not concluded on), and/or the NCD-SSB configuration does not need to be the same for all UEs in that cell ( this would depend on the NW configuration, in that the NW is not restricted to do so). We note that NCD-SSB is primarily a CONNECTED mode construct for RedCap operation, and so a dedicated configuration.

Observation 3: There is no requirement that the NCD-SSB configuration given to one RedCap UE has to be the same as the NCD-SSB config given to other RedCap UE.

We also note that the UE does need to read SIB1/MIB during the course of operation in this newly handed over cell, unless the UE is handed over again to a new cell. We see that this is anyway needed from the UE once the UE goes into IDLE mode (in case there is an initial BWP specifically for the RedCap UEs, then the SIB1 would be from that BWP).

Observation 4: The RedCap UE still needs to read the SIB1/MIB of the cell (of the initial BWP which can be specific to RedCap UEs), but this can be done once the UE moves out of CONNECTED mode, in case the BWP the RedCap UE is in, cannot provide the means to read SIB1.

Based on all of the above, we can see that the RedCap specific BWP config provided to the UE at handover, should follow the below principles for an efficient operation of the RedCap feature:

- The common config given in the handover message should match the common config provided in the cell (as in legacy), but the additional implication being that there is now a redCap specific common config. For eg:, the RedCap specific initial DL/UL common BWP configuration.

- The NCD-SSB should also provide the same information that CD-SSB provides, so that the UE can directly sync to the NCD-SSB as part of handover: same SFN, same beam info, and same QCL configuration. While the SFN/beam index is agreed to be the same, a larger periodicity of NCD-SSB can result in longer time for the handover!

There we propose the below:

The RedCap specific common cell configuration provided in servingCellConfigCommon at the handover should be the same as broadcasted RedCap config in the cell.

The RedCap UE is not expected to acquire SIB1 while in CONENCTED mode in case the RedCap UE operating BWP does not have either the CD-SSB or the NCD-SSB.

The NW can optionally provide the SIB1 content to the UE in a dedicated message in case the RedCap UE operating BWP does not have either the CD-SSB or the NCD-SSB

Observation 5: If the RedCap UE directly sync to the NCD-SSB as part of the handover (instead of syncing to the CD-SSB) first, then if the periodicity of the NCD-SSB is larger than the periodicity of the CD-SSB, the handover performance would be affected.

Observation 6: In case the NCD-SSB is longer in periodicity, the UE might try to sync to CD-SSB first (as an implementation). In anyway, a longer NCD-SSB can only result in longer time the UE has to spend to try to sync.

Related to the handover dynamics, we propose the below:

RAN2 to discuss if the UE is expected to directly sync to the NCD-SSB as part of the handover (instead of syncing to the CD-SSB) first:

  - In case direct sync to NCD-SSB is agreed, then the periodicity of the NCD-SSB should be the same as the periodicity of the CD-SSB. Otherwise, the RRC processing delay needs to account for the delay in handover.

# 3	Conclusion

Observation 1: RedCap UE design have two choices during the handover in case the handover message has a separate NCD-SSB the UE would be operating in: whether the UE first sync to CD-SSB to derive the necessary information to be used in syncing to the NCD-SSB, or, the UE directly sync to the NCD-SSB assuming that the NCD-SSB can also be used by itself to derive the needed information for the UE for the handover.

Observation 2: It is more efficient for the RedCap UE to directly sync to the NCD-SSB at handover instead of syncing to the CD-SSB first.

Observation 3: There is no requirement that the NCD-SSB configuration given to one RedCap UE has to be the same as the NCD-SSB config given to other RedCap UE.

Observation 4: The RedCap UE still needs to read the SIB1/MIB of the cell (of the initial BWP which can be specific to RedCap UEs), but this can be done once the UE moves out of CONNECTED mode, in case the BWP the RedCap UE is in, cannot provide the means to read SIB1.

Observation 5: If the RedCap UE directly sync to the NCD-SSB as part of the handover (instead of syncing to the CD-SSB) first, then if the periodicity of the NCD-SSB is larger than the periodicity of the CD-SSB, the handover performance would be affected.

Observation 6: In case the NCD-SSB is longer in periodicity, the UE might try to sync to CD-SSB first (as an implementation). In anyway, a longer NCD-SSB can only result in longer time the UE has to spend to try to sync.

- The RedCap specific common cell configuration provided in servingCellConfigCommon at the handover should be the same as broadcasted RedCap config in the cell.

The RedCap UE is not expected to acquire SIB1 while in CONENCTED mode in case the RedCap UE operating BWP does not have either the CD-SSB or the NCD-SSB.

The NW can optionally provide the SIB1 content to the UE in a dedicated message in case the RedCap UE operating BWP does not have either the CD-SSB or the NCD-SSB

RAN2 to discuss if the UE is expected to directly sync to the NCD-SSB as part of the handover (instead of syncing to the CD-SSB) first:

    - In case direct sync to NCD-SSB is agreed, then the periodicity of the NCD-SSB should be the same as the periodicity of the CD-SSB. Otherwise, the RRC processing delay needs to account for the delay in handover.

# References

R2-220XXXX Email discussion report for [Pre117-e][105][RedCap] CP open issues	Ericsson