3GPP TSG-RAN2 #117-e	R2-2202715

# Online, 21 February – 3 March, 2022

Title:	[DRAFT] Reply LS on Pemax for NR-V2X

Response to:	R2-2200121_R4-2120047 LS on PEMAX for NR-V2X

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	Huawei, HiSilicon, CATT [To be RAN2]

To:	RAN4

Cc:	RAN1

Contact Person:

#### Name:	Tao Cai

#### mail Address:	tao.cai@huawei.com

#### Name:	Hao Xu

#### E-mail Address:	xuhao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks RAN4 for the LS R2-2200121_R4-2120047 and RAN2 has the following understanding regarding the two issues:

Issue 1: It is RAN4 understanding that the parameter to limit the transmitted power of PSSCH/PSCCH for V2X can be used in both in-coverage and out-of-coverage. RAN4 would like to check with RAN1 and RAN2 which parameter (sl-maxTxPower, sl-MaxTransPower, SL-TxPower) is the correct one to be used to fulfill the purpose?

Answer 1:

As specified in TS 38.331, sl-maxTxPower is a conditional parameter pending on CBR and it is associated with a priority level of the PSSCH transmission and a CBR range while sl-MaxTransPower is the parameter used to indicate the maximum value of the UE’s sidelink transmission power on this resource pool.

Further, it should be noted that SL-TxPower is the value for sl-maxTxPower and it is not suitable to be considered as a reference parameter here.

Therefore, RAN2 think sl-MaxTransPower is the correct parameter to be used to fulfil said purpose.

Issue 2: RAN4 also had some discussion on whether the parameter should be associated with or without a serving cell on the NR V2X carrier, and three options are proposed:

- Option 1: The parameter can be associated either with a serving cell or without a serving cell, and it can be configured separately with p-max for Uu

- Option 2: The parameter can be associated either with a serving cell or without a serving cell, when the parameter is associated with a serving cell, PEMAX,c is the smaller value given by this parameter for SL and p-max for Uu of that serving cell.

- Option 3: when UE is associated with a serving cell on the NR V2X carrier, p-max is used for serving cell c; when the UE is not associated with a serving cell on the NR V2X carrier, the parameter given for SL in RAN2 specification is used.

RAN4 would like to check which option is aligned with the RAN1 and RAN2 specification.

Answer 2:

In RAN2 specification, there is no restriction on the configuration of sl-MaxTransPower and p-max, therefore, Option 1 is considered as aligned with RAN2 specification.

2. Actions:

To RAN4:

ACTION: 	RAN2 kindly asks RAN4 to take the above agreement into account for further work.

3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #118-e	16 – 27 May 2022

TSG RAN WG2 Meeting #119	22 – 26 August 2022