3GPP TSG-RAN WG2 Meeting #117 electronic	R2-2202846

Online, February 21 – March 3, 2022	Revision of R2-2200739

Agenda Item:		8.6.3

Source:				ASUSTeK

Title:		Remaining issue on CS-RNTI configuration for CG-SDT

Document for: 	Discussion and Decision

- Introduction

In RAN2#115 meeting, the CS-RNTI based dynamic retransmission mechanism has been agreed and two options for CS-RNTI configuration are discussed in the e-mail discussion [Post115-e][509].

Agreement

- CS-RNTI based dynamic retransmission mechanism can be reused for CG-SDT.  FFS whether CS-RNTI is the same one as the one previously configured in RRC_CONNECTED or a new CS-RNTI one is provided to the UE

In RAN2#116bis meeting, one of the two options is included in the e-mail discussion [Post116-e][509] and is agreed.

Agreements:

9 	CS-RNTI for CG-SDT is provided to the UE in RRCRelease message.

In this contribution, we discuss the remaining issue on CS-RNTI configuration for CG-SDT.

- Discussion

In [Post115-e][509], the two options to configure the CS-RNTI for CG-SDT has been discussed (i.e., CS-RNTI is the same as the one previously configured in connected mode or CS-RNTI is provided via RRCRelease message). And 17 out of the 26 companies support to adopt both the options. The offline proposal is that the CS-RNTI for CG-SDT can be the same as the one configured in RRC_CONNECTED or provided via RRCRelease message [1]. However, there is no time to confirm this proposal online during RAN2#116 meeting.

Observation 1: Majority support that the CS-RNTI for CG-SDT could be the same as the one configured in connected state or could be provided via RRCRelease message.

In [Post116-e][509], the question to discuss is only about whether the CS-RNTI for CG-SDT is provided to the UE in RRCRelease message [2]. Whether the CS-RNTI for CG-SDT could be the same as the one configured in connected state was not discussed. In this e-mail discussion, companies comment that the provision of CS-RNTI in RRCRelease message should not be mandatory. That is, the CS-RNTI could be optionally provided in RRCRelease message if there is no CS-RNTI previously configured in connected state. Since the UE could be configured with Type 1 CG configuration in connected state, the CS-RNTI used in connected state can be reused for the CG-SDT. And the redundant configuration could be prevented. It should not restrict NW to always configure a new CS-RNTI in RRCRelease message.

Proposal 1: The CS-RNTI for CG-SDT is optionally configured to the UE in RRCRelease message.

If the UE receives a CS-RNTI in the RRCRelease message, the UE could use the received CS-RNTI for CG-SDT. If the UE doesn’t receive a new CS-RNTI in RRCRelease message, the UE could reuse the CS-RNTI (used for RRC_CONNECTED state) for CG-SDT. During the CG-SDT procedure, a UE could monitor a CS-RNTI used in RRC_CONNECTED state or obtained from the RRCRelease message. This also provides NW flexibility to configure CS-RNTI to the UE.

Proposal 2: The CS-RNTI for RRC_CONNECTED state is reused for CG-SDT if the UE doesn’t receive a CS-RNTI in RRCRelease message.

- Conclusion

In this contribution, we discuss the CS-RNTI configuration for CG-SDT, and have the following observation and proposals:

Observation 1: Majority support that the CS-RNTI for CG-SDT could be the same as the one configured in connected state or could be provided via RRCRelease message.

Proposal 1: The CS-RNTI for CG-SDT is optionally configured to the UE in RRCRelease message.

Proposal 2: The CS-RNTI for RRC_CONNECTED state is reused for CG-SDT if the UE doesn’t receive a CS-RNTI in RRCRelease message.

- Reference

- R2-2110670, “Summary of [Post115-e][509][SDT] CG open issues (Xiaomi)”

- R2-2200033, “[Post116-e][509][SDT]CG open issues (Huawei)”