3GPP TSG-RAN WG2 Meeting #117 electronic	R2-2203064

Online, Feb 21 – Mar 3, 2022

Title	Remaining issues on slice based RACH

Source	LG Electronics

Document for	Discussion and Decision

Agenda Item	8.8.3

WID/SID	RAN slicing

1 	Introduction

In [1], followings are remained as open issues regarding slice-specific RACH:

Q 1.5: Whether to support dedicated RACH resources and RACH prioritization parameters in dedicated signalling. OI 2.1: FFS whether RA prioritization and RA partitioning will work independently or RA prioritization and RA partitioning should configure/work simultaneously for a specific slice group.

This contribution shares our views on the remaining open issues on slice-specific RACH

2	Discussion

Aspects on dedicated signalling of slice-specific RACH configuration

Regarding the signalling method of slice-specific RACH configuration, RAN2 has agreed to followings:

scalingFactorBI and powerRampingStepHighPriority can be configured at least in SIB (FFS for dedicated RRC signalling).

The agreement already concludes that current SIB includes slice-specific RACH prioritization parameter (e.g., scalingFactorBI and powerRampingStepHighPriority for slice or slice group). On the other hand, the slice-specific RACH is discussed as a feature in common RACH session, and the RACH partition would be configured via SIB1 (i.e., broadcast signalling). Considering that the slice-specific RACH is performed when the UE is in RRC_INACTIVE or RRC_IDLE state, the duplicated configuration via dedicated RRC signalling is not needed.

Proposal 1. Slice-specific RACH is not configured in dedicated RRC signalling.

Aspects on dependency of slice-based RACH prioritization on slice-based RACH configuration

Regarding the slice-specific RACH configuration, followings have been agreed in previous meetings:

4 RAN2 will use the following assumptions on slice groups and slice-specific RACH configuration in the work on Stage 3 details: - 1. For slice-specific cell re-selection, cell reselection priorities for one or multiple slice group for the serving frequency are indicated in SIB of the serving cell. - 2. Slice to slice-group configuration is common to cell reselection and RACH. Configuration of whether to use slice-specific cell re-selection or slice-specific RACH is up to network configuration (i.e. some slice groups may use cell reselection but not RACH, some may use RACH but not cell reselection, some may use both). - 3. In a cell, there may be multiple slice-specific RACH configurations. - 4. One or more of the slice groups are linked to a slice-specific RACH configuration. - 5. There may be slice groups that are not linked to a slice-specific RACH configuration (they use the common RACH configuration). - 6. All slices of a slice group use the slice-specific RACH configuration of the slice group. 3: RACH prioritization parameters can be configured per slice group.

Meanwhile, there are two parts of slice-based RACH in RAN slicing [2]:

- Slice-based RACH resource isolation: to define the separated RACH preamble or RACH occasion in order to guarantee RA resources for specific slice group(s)

- Slice-based RACH prioritization: to prioritize the access with some slice group(s) during the RA procedure. (i.e., configuration of scalingFactorBI and powerRampingStepHighPriority for speicifc slice group)

In RAN2#113bis-e through RAN2#116e meetings, the two sub-categories of slice-based RACH operation have been discussed separatedly. There was no discussion regarding whether the two sub-features of slice-specific RACH (i.e., slice-based RACH isolation and slice-based RACH prioritization) can be configured independently.

Observation. It is not determined whether the slice-based RACH prioritization can be configured without slice-based RACH partition.

For this issue, there are two options regarding dependency of slice-based RACH prioritization and slice-based RACH isolation:

- Option 1. Slice-based RACH isolation and slice-based RACH prioritization can be independently. For example, even though the network cannot allocate the RACH resource for slice group#1, the network may configure slice-based RACH prioritization (i.e., ra-PrioritizationForSlicing) for slice group#1 in legacy RACH resource.

- Option 2. Slice-based RACH prioritization can only be configured in slice-based RACH configuration. That is, if there is no partition for specific slice group, slice-specific RACH prioritization parameter should not be configured.

Option 1 is to provide more flexibility on slice-based RACH prioritization. If Option 1 is taken, when a UE triggered an RA procedure with slice information(e.g., slice group#1), the UE first checks whether there is slice-specific RACH configuration (i.e, slice-specific RACH resource). If there is no resource for the slice information, the UE uses the common RACH resource (or another RACH partition if the corresponding feature is applied). Within the selected resource, the UE may use prioritized RACH parameters (i.e., scalingFactorBI and powerRampingStepHighPriority defined in RA-PrioritizationForSlicing) if it is configured in the common (or feature-specific) RACH configuration.

On the other hand, Option 2 is to simplify slice-based RACH operation. If Option 2 is taken, when a UE triggered an RA procedure with slice information (e.g., slice group#1), the UE checks whether there is slice-specific RACH configuration (i.e, slice-specific RACH resource). If there is no resource for the slice information, the UE uses the common RACH resource (or another RACH partition). Since there is no slice-specific RACH prioritization in other partitions, the UE does not apply slice-specific prioritization.

In our view, the independent configuration of slice-based RACH prioritization parameter as in Option 1 would be complicated without the significant improvement of entire RA procedure. Since there are multiple features which define feature-specific RACH operation in Rel-17, the RA procedure in each feature should be defined as simple as possible considering the coordinated cases of multiple features. In addition, if the slice-specific RACH prioritization parameter is defined in multiple RACH partition as in Option 1, the amount of signalling in SIB1 may be significantly increased, considering that there are multiple RACH configurations for features or feature combinations. Configuring slice-specific RACH parameter (including RACH parioritization) only in slice-specific RACH configuration, as in Option 2, is also aligned with the baseline agreement of common RACH session in RAN2#116bis-e meeting, which proposes the common RACH parameter in one RACH partition as follows [3]:

As baseline agreement: - RACH parameters (e.g. power ramping step, max RACH transmissions etc) are configured per RACH partition rather than per feature within the partition.

Therefore, for simplicity of RA procedure in Rel-17, the RACH prioritization should be configured only in slice-specific RACH configuration (i.e., slice-based RACH partition).

Proposal 2. Slice-specific RACH prioritization parameter should be configured only if slice-specific RA resource partition is configured for the slice group.

3	Conclusion

In this paper, we discussed the slice-specific RACH operation. The discussion includes the following observations:

Observation. It is not determined whether the slice-based RACH prioritization can be configured without slice-based RACH partition.

Based on the observation, followings are proposed:

Proposal 1. Slice-specific RACH is not configured in dedicated RRC signalling.

Proposal 2. Slice-specific RACH prioritization parameter should be configured only if slice-specific RA resource partition is configured for the slice group.

4	Reference

[1] R2-2201730 Open issues for slicing

[2] 3GPP TR 38.832 V1.0.0, Study on enhancement of Radio Access Network (RAN) slicing

[3] Report of 3GPP TSG RAN WG2 meeting #116bis-e, Online