3GPP TSG-RAN WG2 Meeting #117 electronic	R2-2203102

Online, Feb 21th – Mar 3nd, 2022

Source:	CATT

Title:	Discussions on the remaining RRC issues of feMIMO

Agenda Item:	8.17.4.1

Document for:	Discussion and Decision

Introduction

In the contribution, we would like to further discuss the remaining open issues on R17 feMIMO, based on the RRC running CR [1] and RAN1 parameters [2], including:

- FFS how to configure the mode of R17 unified TCI framework;

- FFS how to implement the association of the PCI index and the TCI state;

- FFS how to implement the association of the additional PCI and SSB;

- FFS on Scenarios on R17 unified TCI framework.

Discussion

  - FFS how to configure the mode of R17 unified TCI framework

According to RAN1 LS [3], there are two modes of the R17 unified TCI framework, i.e., one is joint beam indication mode with joint TCI state for both UL and DL; another is separate beam indication, which is separate DL and/or UL TCI state.  And the UE should be informed of the mode to be used for each TCI state codepoint, so that UE can determine whether the indicated TCI states apply to DL only, UL only, or for both DL and UL.

Further, RAN1 made the following agreements in RAN1#105e [4], which implies that the mode of the R17 unified TCI framework should be configured by RRC.

Conclusion On Rel-17 unified TCI framework, for a UE configured with both joint TCI and separate DL/UL TCI, configuration of joint TCI or separate DL/UL TCI is based on RRC signaling - There is no consensus in RAN1 on how to support dynamic switching (either MAC-CE or codepoint based)

Observation 1: The mode of R17 unified TCI framework is configured via RRC.

As for how to implement the the mode of R17 unified TCI framework via RRC, it can either be configured per-cell level within ServingCellConfig, or per-BWP level within the BWP-DownlinkDedicated, or in PDSCH-Config. Although it is more flexible configured in per-BWP or per-PDSCH level, extra overhead would be introduced.

Proposal 1: RAN2 to agree that a new IE indicating the mode of the R17 unified TCI framework is introduced within the ServingCellConfig.

Correspondingly, the following TP as highlighted in yellow is given on how to configure the mode of the R17 unified TCI framework based on proposal 1.

#### – ServingCellConfig The IE ServingCellConfig is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts). Reconfiguration between a PUCCH and PUCCHless SCell is only supported using an SCell release and add. ServingCellConfig information element -- ASN1START -- TAG-SERVINGCELLCONFIG-START ServingCellConfig ::= SEQUENCE { tdd-UL-DL-ConfigurationDedicated TDD-UL-DL-ConfigDedicated OPTIONAL, -- Cond TDD initialDownlinkBWP BWP-DownlinkDedicated OPTIONAL, -- Need M downlinkBWP-ToReleaseList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id OPTIONAL, -- Need N downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink OPTIONAL, -- Need N firstActiveDownlinkBWP-Id BWP-Id OPTIONAL, -- Cond SyncAndCellAdd bwp-InactivityTimer ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500, ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL, --Need R defaultDownlinkBWP-Id BWP-Id OPTIONAL, -- Need S uplinkConfig UplinkConfig OPTIONAL, -- Need M supplementaryUplink UplinkConfig OPTIONAL, -- Need M pdcch-ServingCellConfig SetupRelease { PDCCH-ServingCellConfig } OPTIONAL, -- Need M pdsch-ServingCellConfig SetupRelease { PDSCH-ServingCellConfig } OPTIONAL, -- Need M csi-MeasConfig SetupRelease { CSI-MeasConfig } OPTIONAL, -- Need M sCellDeactivationTimer ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240, ms320, ms400, ms480, ms520, ms640, ms720, ms840, ms1280, spare2,spare1} OPTIONAL, -- Cond ServingCellWithoutPUCCH crossCarrierSchedulingConfig CrossCarrierSchedulingConfig OPTIONAL, -- Need M tag-Id TAG-Id, dummy1 ENUMERATED {enabled} OPTIONAL, -- Need R pathlossReferenceLinking ENUMERATED {spCell, sCell} OPTIONAL, -- Cond SCellOnly servingCellMO MeasObjectId OPTIONAL, -- Cond MeasObject ..., [[ lte-CRS-ToMatchAround SetupRelease { RateMatchPatternLTE-CRS } OPTIONAL, -- Need M rateMatchPatternToAddModList SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern OPTIONAL, -- Need N rateMatchPatternToReleaseList SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL, -- Need N downlinkChannelBW-PerSCS-List SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier OPTIONAL -- Need S ]], [[ supplementaryUplinkRelease-r16 ENUMERATED {true} OPTIONAL, -- Need N tdd-UL-DL-ConfigurationDedicated-IAB-MT-r16 TDD-UL-DL-ConfigDedicated-IAB-MT-r16 OPTIONAL, -- Cond TDD_IAB dormantBWP-Config-r16 SetupRelease { DormantBWP-Config-r16 } OPTIONAL, -- Need M ca-SlotOffset-r16 CHOICE { refSCS15kHz INTEGER (-2..2), refSCS30KHz INTEGER (-5..5), refSCS60KHz INTEGER (-10..10), refSCS120KHz INTEGER (-20..20) } OPTIONAL, -- Cond AsyncCA dummy2 SetupRelease { DummyJ } OPTIONAL, -- Need M intraCellGuardBandsDL-List-r16 SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16 OPTIONAL, -- Need S intraCellGuardBandsUL-List-r16 SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16 OPTIONAL, -- Need S csi-RS-ValidationWithDCI-r16 ENUMERATED {enabled} OPTIONAL, -- Need R lte-CRS-PatternList1-r16 SetupRelease { LTE-CRS-PatternList-r16 } OPTIONAL, -- Need M lte-CRS-PatternList2-r16 SetupRelease { LTE-CRS-PatternList-r16 } OPTIONAL, -- Need M crs-RateMatch-PerCORESETPoolIndex-r16 ENUMERATED {enabled} OPTIONAL, -- Need R enableTwoDefaultTCI-States-r16 ENUMERATED {enabled} OPTIONAL, -- Need R enableDefaultTCI-StatePerCoresetPoolIndex-r16 ENUMERATED {enabled} OPTIONAL, -- Need R enableBeamSwitchTiming-r16 ENUMERATED {true} OPTIONAL, -- Need R cbg-TxDiffTBsProcessingType1-r16 ENUMERATED {enabled} OPTIONAL, -- Need R cbg-TxDiffTBsProcessingType2-r16 ENUMERATED {enabled} OPTIONAL -- Need R ]], [[ directionalCollisionHandling-r16 ENUMERATED {enabled} OPTIONAL, -- Need R channelAccessConfig-r16 SetupRelease { ChannelAccessConfig-r16 } OPTIONAL -- Need M ]] , [[ SSB-MTCAdditionalPCIList-r17::= SEQUENCE (SIZE(1..FFS)) OF SSB-MTCAddionalPCI-r16 OPTIONAL, -- Need R unifiedTCIMode-r17 ENUMERATED {Joint, seperate} OPTIONAL -- Need M ]] } UplinkConfig ::= SEQUENCE { initialUplinkBWP BWP-UplinkDedicated OPTIONAL, -- Need M uplinkBWP-ToReleaseList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id OPTIONAL, -- Need N uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink OPTIONAL, -- Need N firstActiveUplinkBWP-Id BWP-Id OPTIONAL, -- Cond SyncAndCellAdd pusch-ServingCellConfig SetupRelease { PUSCH-ServingCellConfig } OPTIONAL, -- Need M carrierSwitching SetupRelease { SRS-CarrierSwitching } OPTIONAL, -- Need M ..., [[ powerBoostPi2BPSK BOOLEAN OPTIONAL, -- Need M uplinkChannelBW-PerSCS-List SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier OPTIONAL -- Need S ]], [[ enablePL-RS-UpdateForPUSCH-SRS-r16 ENUMERATED {enabled} OPTIONAL, -- Need R enableDefaultBeamPL-ForPUSCH0-0-r16 ENUMERATED {enabled} OPTIONAL, -- Need R enableDefaultBeamPL-ForPUCCH-r16 ENUMERATED {enabled} OPTIONAL, -- Need R enableDefaultBeamPL-ForSRS-r16 ENUMERATED {enabled} OPTIONAL, -- Need R uplinkTxSwitching-r16 SetupRelease { UplinkTxSwitching-r16 } OPTIONAL, -- Need M mpr-PowerBoost-FR2-r16 ENUMERATED {true} OPTIONAL -- Need R ]] } DummyJ ::= SEQUENCE { maxEnergyDetectionThreshold-r16 INTEGER(-85..-52), energyDetectionThresholdOffset-r16 INTEGER (-20..-13), ul-toDL-COT-SharingED-Threshold-r16 INTEGER (-85..-52) OPTIONAL, -- Need R absenceOfAnyOtherTechnology-r16 ENUMERATED {true} OPTIONAL -- Need R } ChannelAccessConfig-r16 ::= SEQUENCE { energyDetectionConfig-r16 CHOICE { maxEnergyDetectionThreshold-r16 INTEGER (-85..-52), energyDetectionThresholdOffset-r16 INTEGER (-13..20) } OPTIONAL, -- Need R ul-toDL-COT-SharingED-Threshold-r16 INTEGER (-85..-52) OPTIONAL, -- Need R absenceOfAnyOtherTechnology-r16 ENUMERATED {true} OPTIONAL -- Need R } IntraCellGuardBandsPerSCS-r16 ::= SEQUENCE { guardBandSCS-r16 SubcarrierSpacing, intraCellGuardBands-r16 SEQUENCE (SIZE (1..4)) OF GuardBand-r16 } GuardBand-r16 ::= SEQUENCE { startCRB-r16 INTEGER (0..274), nrofCRBs-r16 INTEGER (0..15) } DormancyGroupID-r16 ::= INTEGER (0..4) DormantBWP-Config-r16::= SEQUENCE { dormantBWP-Id-r16 BWP-Id OPTIONAL, -- Need M withinActiveTimeConfig-r16 SetupRelease { WithinActiveTimeConfig-r16 } OPTIONAL, -- Need M outsideActiveTimeConfig-r16 SetupRelease { OutsideActiveTimeConfig-r16 } OPTIONAL -- Need M } WithinActiveTimeConfig-r16 ::= SEQUENCE { firstWithinActiveTimeBWP-Id-r16 BWP-Id OPTIONAL, -- Need M dormancyGroupWithinActiveTime-r16 DormancyGroupID-r16 OPTIONAL -- Need R } OutsideActiveTimeConfig-r16 ::= SEQUENCE { firstOutsideActiveTimeBWP-Id-r16 BWP-Id OPTIONAL, -- Need M dormancyGroupOutsideActiveTime-r16 DormancyGroupID-r16 OPTIONAL -- Need R } UplinkTxSwitching-r16 ::= SEQUENCE { uplinkTxSwitchingPeriodLocation-r16 BOOLEAN, uplinkTxSwitchingCarrier-r16 ENUMERATED {carrier1, carrier2} } -- TAG-SERVINGCELLCONFIG-STOP -- ASN1STOP

Proposal 2: RAN2 to agree the above TP on configuring the mode of R17 unified TCI framework.

  - FFS how to implement the association of the PCI index and the TCI state

The additional PCI index should be introduced within the R17 TCI-state configuration, so as to identify the TRP that the TCI state applies, or equally the TRP that the target RS is referred. However, according to the running CR [1], the additional PCI index (additionalPCI-r17) is configured within the QCL-Info. It seems that even within a single tci-StateId, the PCI can be different for different QCL-Type, which is indeed conflict with the RAN1’s intention.

Proposal 3: The PCI index associated with the TCI state shall be used to identify the TRP that the TCI state applies, i.e., the two QCL-Type if included within the TCI-state should share a single PCI index.

Correspondingly, the following TP as highlighted in yellow is given on how to implement the association of the PCI index and the TCI state based on proposal 3.

#### – TCI-State The IE TCI-State associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type. TCI-State information element -- ASN1START -- TAG-TCI-STATE-START TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ... } QCL-Info ::= SEQUENCE { cell ServCellIndex OPTIONAL, -- Need R bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb SSB-Index }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD}, ..., [[ --Editor’s note: Can be discussed if ASN1 overhead reasons should have another way to implement than using this extension. --Editor’s note: Needed in Rel-15/16 TCI state for mTRP intercell and in Rel-17 TCI state for BM intercell ]] } DLorJoint-TCIState-r17 ::= SEQUENCE { tci-StateUnifiedId-r17 DLorJoint-TCIState-Id-r17, tci-StateType-r17 ENUMERATED {DLOnly, JointULDL}, qcl-Type1-r17 QCL-Info, qcl-Type2-r17 QCL-Info OPTIONAL -- Need R additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL -- Need R } UL-TCIState-r17 ::= SEQUENCE { UL-TCIState-Id-r17 UL-TCIState-Id-r17, servingCellId ServCellIndex OPTIONAL, -- Need S referenceSignal CHOICE { ssb-Index SSB-Index, csi-RS-Index NZP-CSI-RS-ResourceId, srs PUCCH-SRS }, additionalPCI-r17 AdditionalPCIIndex OPTIONAL, -- Need R ul-powerControl-r17 UL-powerControl-r17 OPTIONAL -- Need R pathlossReferenceRS-Id PUSCH-PathlossReferenceRS-Id OPTIONAL -- Need S --Editor’s note: pending on LS response the power control and PL-reference signal part may change } -- TAG-TCI-STATE-STOP -- ASN1STOP

Proposal 4: RAN2 to agree the above TP on implementing the association of the PCI index and the TCI state.

  - FFS how to implement the association of the additional PCI and SSB;

In last meeting, RAN2 has sent a LS to RAN1 on the association of the additional PCI and SSB [5]:

------------------------------------------------------------------------------------------------------------------------------

CSI-SSB-ResourceSet

Question 1.13: Should it be possible for different SSB indexes in the same CSI-SSB-ResourceSet to be associated with different additionalPCI?

------------------------------------------------------------------------------------------------------------------------------

In RAN1#107e [6], RAN1 already agreed that within CSI-SSB-ResourceSet, the SSB can be associated with multiple PCIs.

RAN1#107e Agreement On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, a CSI-SSB-ResourceSet configured for L1-RSRP measurement/reporting includes at least a set of SSB indices where PCI indices are associated with the set of SSB indices, respectively. The PCI indices refer to PCIs within the set of PCIs configured for inter-cell beam management or inter-cell multi-TRP. - The additionalInfo associated with SSB(s) with PCI(s) different from the serving cell agreed in RAN1 Agenda Item 8.1.2.2 is also applicable to inter-cell BM - Detailed signaling design is up to RAN2 - FFS (to be concluded in RAN1#107-e): Whether the above L1-RSRP measurement/reporting also includes group-based beam report for inter-cell mTRP

Observation 2: The SSB resources within a SSB resource set can be associated with different PCIs.

As for how to make such association of the additional PCI and SSB resources, the following TP as highlighted in yellow can be considered.

#### – CSI-SSB-ResourceSet The IE CSI-SSB-ResourceSet is used to configure one SS/PBCH block resource set which refers to SS/PBCH as indicated in ServingCellConfigCommon. CSI-SSB-ResourceSet information element -- ASN1START -- TAG-CSI-SSB-RESOURCESET-START CSI-SSB-ResourceSet ::= SEQUENCE { csi-SSB-ResourceSetId CSI-SSB-ResourceSetId, csi-SSB-ResourceList SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index, ... , [[ additionalPCIList-r17 SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF AdditionalPCIIndex-r17 OPTIONAL -- Need S ]] } -- TAG-CSI-SSB-RESOURCESET-STOP -- ASN1STOP CSI-SSB-ResourceSet field descriptions. additionalPCIList If this field is present, each SSB resource within the csi-SSB-ResourceList is associated with PCI as indicated by the additionalPCIList as in the same position, e.g., the first PCI as indicated within the additionalPCIList apply to the first SSB within the csi-SSB-ResourceList. Otherwise, the csi-SSB-ResourceList associated with the serving cell..

Proposal 5: RAN2 to agree the above TP on implementing the association of the additional PCI and SSB.

  - FFS on Scenarios on R17 unified TCI framework

The CORESETPoolindex is introduced in R16 to support the Multi-DCI based intra-cell mTRP. According to RAN1 [7] [8], for DL TCI, the source reference signal (analogous to Rel.15, two, if qcl_Type2 is configured in addition to qcl_Type1) in the DL TCI provides QCL information at least for UE-dedicated reception on PDSCH and all of CORESETs in a CC. Thus, the R17 unified TCI framework will not apply to R16 intra-cell mTRP scenario.

RAN1#106e: On Rel-17 unified TCI, for Rel-17, there is no consensus in supporting additional (M,N) values other than (M,N)=(1,1) RAN1#104e: On Rel.17 unified TCI framework, based on the agreements in RAN1#102-e and 103-e, the following terms are defined as follows (at least for the purpose of discussion and reaching agreements). For M=1:  DL TCI: The source reference signal (analogous to Rel.15, two, if qcl_Type2 is configured in addition to qcl_Type1) in the DL TCI provides QCL information at least for UE-dedicated reception on PDSCH and all of CORESETs in a CC For N=1:  UL TCI: The source reference signal in the UL TCI provides a reference for determining UL TX spatial filter at least for dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources in a CC For M=N=1:  Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter.  Separate DL/UL TCI: The DL TCI and UL TCI are distinct (therefore, separate). For M>1:  DL TCI: Each of the M source reference signals (or 2M, if qcl_Type2 is configured in addition to qcl_Type1) in the M DL TCIs provides QCL information at least for one of the M beam pair links for UE-dedicated receptions on PDSCH and/or subset of CORESETs in a CC For N>1:  UL TCI: Each of the N source reference signals in the N UL TCIs provide a reference for determining UL TX spatial filter at least for one of the N beam pair links associated with dynamic-grant /configured-grant based PUSCH, and/or subset of dedicated PUCCH resources in a CC For M>1 and/or N>1:  Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. In this case, M=N.  Separate DL/UL TCI: The M DL TCIs and N UL TCIs are distinct (therefore, separate). Note: Other TCI types/terms such as “common TCI” are not used.

Observation 3: The R17 unified TCI framework does not apply to R16 intra-cell mTRP scenario.

Based on above analysis, we think that NW needs to ensure that the CORESETPoolindex within BWP should be the same when configured with the R17 unified TCI framework. Also the newly introduced MAC CE for R17 unified TCI framework is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP.

Proposal 6: NW to ensure that CORESETPoolindex within a BWP should be the same value when configured with R17 unified TCI framework.

Proposal 7: The newly introduced MAC CE for R17 unified TCI framework is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP.

Conclusion

This contribution discusses on R17 unified TCI framework. The main observations and proposals are summarized in the following.

Observation 1: The mode of R17 unified TCI framework is configured via RRC.

Observation 2: The SSB resources within a SSB resource set can be associated with different PCIs.

Observation 3: The R17 unified TCI framework does not apply to R16 intra-cell mTRP scenario.

Proposal 1: RAN2 to agree that a new IE indicating the mode of the R17 unified TCI framework is introduced within the ServingCellConfig.

Proposal 2: RAN2 to agree the above TP on configuring the mode of R17 unified TCI framework.

Proposal 3: The PCI index associated with the TCI state shall be used to identify the TRP that the TCI state applies, i.e., the two QCL-Type if included within the TCI-state should share a single PCI index.

Proposal 4: RAN2 to agree the above TP on implementing the association of the PCI index and the TCI state.

Proposal 5: RAN2 to agree the above TP on implementing the association of the additional PCI and SSB.

Proposal 6: NW to ensure that CORESETPoolindex within a BWP should be the same value when configured with R17 unified TCI framework.

Proposal 7: The newly introduced MAC CE for R17 unified TCI framework is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP.

Reference

[1] R2-2202000, Running RRC CR for FeMIMO Rel-17, Ericsson

[2] R2-2202055, FeMIMO L1 parameters with RAN2 notes Rel-17 NR Post 116bis

[3] R1-2112842, LS on Rel-17 MAC-CE impacts

[4] Draft Report of 3GPP TSG RAN WG1 #105e v0.1.0

[5] R2-2202002, LS on feMIMO RRC, Ericsson

[6] Draft Report of 3GPP TSG RAN WG1 #107-e v0.1.0

[7] Draft Report of 3GPP TSG RAN WG1 #106-e v0.1.0

[8] Draft Report of 3GPP TSG RAN WG1 #104-e