3GPP TSG-RAN WG2 Meeting #117 electronic	R2-2203288

Online, February 21th– March 3rd, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1213 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on HARQ attribute of SL SRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-2-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current MAC specification was written under the assumption that the IE sl-HARQ-FeedbackEnabled must be present or has a default value,which is not aligned with RRC spec in which the HARQ attribute is an optional IE. In RAN2 116 meeting, to eliminate such discrepancy between MAC and RRC spec, RAN2 agrees following change: sl-HARQ-FeedbackEnabled Network always includes this field. It indicates the HARQ feedback enabled/disabled restriction in LCP for this sidelink logical channel. If set to enabled, the sidelink logical channel will be multiplexed only with a logical channel which enabling the HARQ feedback. If set to disabled, the sidelink logical channel cannot be multiplexed with a logical channel which enabling the HARQ feedback. Corresponds to 'sl-HARQ-FeedbackEnabled' in TS 38.321 [3]. If this field of at least one sidelink logical channel for the UE is set to enabled, sl-PSFCH-Config should be mandatory present in at least one of the SL-ResourcePool.. However, since only RRC signaling is modified, the agreed change can only be applied on SL DRB. As we know, besides SL DRB, SL SRB are used for sidelink unicast communication. According to current spec, all SL SRBs use specified RB configuration, and it does not include the HARQ attribute. Therefore, based on current spec(i.e. only LCH with HARQ disabled can use resource pool without PSFCH), SL SRB still can not use the SL grant associated to resource pool without PSFCH resource pool. To provide more flexible, the HARQ attribute of SL SRB is selected by UE implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of how to handle the SL SRB without HARQ attribute during LCP. Impact analysis Impacted 5G architecture options: Standalone/Non-standalone Impacted functionality: NR sidelink communication Inter-operability: - If the UE is implemented according to this CR but the network is not, no inter-operability issue foreseen. - If the network is implemented according to this CR but the UE is not, no inter-operability issue foreseen. - If one UE is implemented according to this CR but the other UE is not, no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SL SRB still can not use the SL grant associated to resource pool without PSFCH resource pool. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

5.22.1.4.1.2	Selection of logical channels

The MAC entity shall for each SCI corresponding to a new transmission:

1>	select a Destination associated to one of unicast, groupcast and broadcast, having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

2>	SL data is available for transmission; and

2>	SBj > 0, in case there is any logical channel having SBj > 0; and

2>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

2>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

2>	sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

NOTE 1:	If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE, which Destination is selected among them is up to UE implementation.

1>	select the logical channels satisfying all the following conditions among the logical channels belonging to the selected Destination:

2>	SL data is available for transmission; and

2>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and.

2>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

3>	if PSFCH is configured for the sidelink grant associated to the SCI:

4>	sl-HARQ-FeedbackEnabled is set to enabled, if sl-HARQ-FeedbackEnabled is set to enabled for the highest priority logical channel satisfying the above conditions; or

4>	sl-HARQ-FeedbackEnabled is set to disabled, if sl-HARQ-FeedbackEnabled is set to disabled for the highest priority logical channel satisfying the above conditions.

3>	else:

4>	sl-HARQ-FeedbackEnabled is set to disabled.

NOTE 2:	sl-HARQ-FeedbackEnabled is set to disabled for the transmission of a MAC PDU only carrying CSI reporting MAC CE.

Note X: it’s up to UE implementation to select a HARQ attribute for logical channel not configured with sl-HARQ-FeedbackEnabled .

End of the change