3GPP TSG-RAN WG2 Meeting #117e	R2-2203348

Online, 21 February – 3 March, 2022

Agenda item:	8.14.3.2

Source:	Huawei, HiSilicon

Title:	RAN visible QoE during mobility

WID/SID:	NR_QoE-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In the last meetings, RAN2 made some agreements on the handling of legacy QoE measurement during handover. Also, RAN3 has agreed to support the RAN visible QoE. In this contribution, we will discuss whether solutions of legacy QoE mobility could be applied to RAN visible QoE and the specific aspects applied only for RAN visible QoE mobility, as per the following open issue:

- Whether solutions of legacy QoE mobility could be applied to RAN visible QoE and the specific aspects applied only for RAN visible QoE mobility.

# 2	Discussion

## 2.1	Latest progresses in RAN2

RAN2#115 agreements on legacy QoE measurement during handover RAN2 assumes that all QoE mobility related agreements made by RAN2 are applicable at least to signalling based QoE. Whether the same applies to management-based QoE is pending further input from SA5 and RAN3. Area scope parameter is not introduced in RRC procedures supporting QoE. When the UE resumes the connection in a gNB supporting QoE, the target gNB should explicitly indicate which QoE measurement configurations should be kept by the UE during RRC resume procedure, e.g. in RRCResume message. The UE shall release all QoE measurement configurations not indicated by the gNB for restoration. FFS how the indication looks like, e.g. granularity per QoE configuration or common for all QoE configurations. During the handover to target gNB which supports QoE, the target gNB decides which QoE configurations to keep and which to release during a handover, e.g. based on QoE configuration information received from the source gNB in Xn/Ng signalling (exact information is up to RAN3) including the RRC container. The UE discards the reports received from application layer in case it has no associated QoE configuration configured. FFS whether the gNB needs to know the QoE configurations for which there are ongoing QoE sessions, e.g. to enable QoE configuration handling upon mobility (pending SA4 reply on the ongoing QoE measurement session continuity requirement). In case the UE resumes the connection in a gNB not supporting QoE, the UE should release all QoE measurement configurations. RAN2#116bis agreements on legacy QoE measurement during handover Upper layers are informed of the release of the application layer measurements at RRCSetup (can be done if RRC setup is provided as a response to RRCresumerequest or RRC reestablishmentrequest). At Resume with delta configuration the network indicates possible differences to the QoE configurations. At mobility with fullConfig, upper layers are informed of the release of the application layer measurements if no measConfigAppLayerId is indicated by the network. RAN2#116 agreements on RAN Visible QoE RAN2 assumes that RAN2 is responsible to define the procedure to support RVQOE configuration and reporting, and leave the definition of RAN QoE metrics and what should be included in RVQOE configuration and report to other WGs, e.g. RAN3, SA4. RAN2 confirms the following is feasible from RAN2 point of view. - It is feasible to configure RVQOE using explicit RRC IEs - Multiple simultaneous QoE measurements can be supported for RVQOE. Each RVQOE measurement configuration is identified by the MeasConfigAppLayerId (or change to another generic term) corresponding to the regular QoE configuration. - UE RRC layer forwards the received RVQOE configuration to the upper (application) layer, indicating the service type. - RAN configures the required RVQOE metrics in the RVQOE configuration for UE to report. RAN2#116bis agreements on RAN Visible QoE [029] RVQoE configuration can share the same measConfigAppLayerId and service type RRC IEs with legacy QoE configuration. [029] Modification of RVQoE configuration can be supported from RRC layer point of view, it can be revisited if any problem according to further stage 3. [029] RAN2 confirm it is feasible that NG-RAN can release a list of RAN visible QoE configurations while not releasing the corresponding legacy QoE configuration and if the corresponding legacy QoE configuration is released, the RAN visible QoE configuration is released as well. [029] RVQoE measurements can be included into MeasurementReportAppLayer message. [029] MeasConfigAppLayerId can be used to identify both of associated legacy QoE report and RVQoE report, and it is irrespective whether RVQoE should be reported independently or together with legacy QoE. [029] Multiple RVQoE reports can be included in one MeasurementReportAppLayer message, and can be revisited according to legact QoE reporting progress.

## 2.2	Technical discussions on RAN visible QoE during mobility

First we think RAN2 can analyse whether the agreements of legacy QoE mobility could be applied to RAN visible QoE.

| Agreements on legacy QoE measurement | Analysis on whether the agreement can be used for RAN visible QoE |
| --- | --- |
| When the UE resumes the connection in a gNB supporting QoE, the target gNB should explicitly indicate which QoE measurement configurations should be kept by the UE during RRC resume procedure, e.g. in RRCResume message. The UE shall release all QoE measurement configurations not indicated by the gNB for restoration. | Different gNB may have different requirements on RAN visible QoE. Also RAN3 has the following agreement ” Target node may generate new RAN visible QoE configuration and send to UE during handover or RRC resume procedure.” Therefore we think the agreement could be applied to RAN visible QoE. |
| During the handover to target gNB which supports QoE, the target gNB decides which QoE configurations to keep and which to release during a handover, e.g. based on QoE configuration information received from the source gNB in Xn/Ng signalling (exact information is up to RAN3) including the RRC container | Same to the above analysis, we think the agreement could be applied to RAN visible QoE. |
| In case the UE resumes the connection in a gNB not supporting QoE, the UE should release all QoE measurement configurations | RAN2 has agreed that “it is feasible that NG-RAN can release a list of RAN visible QoE configurations while not releasing the corresponding legacy QoE configuration and if the corresponding legacy QoE configuration is released, the RAN visible QoE configuration is released as well.” Therefore the RAN visible QoE should be released if the new Gnb does not support the QoE. We think the agreement could be applied to RAN visible QoE. |
| Upper layers are informed of the release of the application layer measurements at RRCSetup (can be done if RRC setup is provided as a response to RRCresumerequest or RRC reestablishmentrequest) | For the RRCSetup provided as a response to RRCResumeRqeust or RRC reestablishment request, the UE also need to release the RAN visible QoE. Therefore we think the agreement could be applied to RAN visible QoE. |
| At Resume with delta configuration the network indicates possible differences to the QoE configurations | RAN3 has the following agreement ” Target node may generate new RAN visible QoE configuration and send to UE during handover or RRC resume procedure.” Therefore we think the agreement could be applied to RAN visible QoE. |
| At mobility with fullConfig, upper layers are informed of the release of the application layer measurements if no measConfigAppLayerId is indicated by the network | For the same reason as the legacy QoE measurement, we think the agreement could be applied to RAN visible QoE. |

Proposal 1: All the existing agreements of legacy QoE mobility can be applied to RAN visible QoE.

RAN2 has agreed to configure the QoE measurement container and RAN visible QoE measurement in the RRC Reconfiguration message. During the handover, the source node will transport all the configuration generated by the source node to the target node in the RRC container within the handover request message.  Therefore there is no RAN2 impacts.

Proposal 2: RAN visible QoE metrics configured at the UE are included in RRC container during the handover procedure. There are no further RAN2 impacts to support mobility for RAN visible QoE.

# 3	Conclusions

Based on the discussion in this paper, the following is observed and proposed:

Proposal 1: All the existing agreements of legacy QoE mobility can be applied to RAN visible QoE.

Proposal 2: RAN visible QoE metrics configured at the UE are included in RRC container during the handover procedure. There are no further RAN2 impacts to support mobility for RAN visible QoE.