3GPP TSG-RAN WG2 #116bis-e	R2-2203450

Electronic meeting, February 21 – March 3, 2022

Agenda Item:	8.22.4

Source:	Ericsson

Title:	UE capabilities for MGE

Document for:	Discussion

# 1	Introduction

The NR and MR-DC Measurement Gap Enhancements WI addresses three different objectives as seen in the WID RP-213312 [1]:

- Pre-configured measurement gap (also known as “fast MG configuration”),

- Concurrent/simultaneous measurement gaps, and

- Network Controlled Small Gap.

In this paper, we focus on the UE capabilities needed for each of the previously mentioned features.

# 2	Discussion

## 2.1	Pre-Configured Measurement Gap

RAN2 currently only supports the case in which pre-configured measurement gaps are activated/deactivated according to rules (i.e., autonomous procedure). To achieve this, the NW will signal a 1-bit indication in gapConfig to point out that the configured gap is a pre-configured one.

However, RAN4 has also made agreements regarding the case in which the gap “status” (i.e., whether the pre-configured gap is activated or deactivated) should be signalled per BWP by the network, so that the UE is aware when a gap is to be “ON” or “OFF” upon switching its active BWP.

RAN4’s intention is to specify both cases, i.e., rule-based and per BWP indication. As a matter of fact, the following agreements regarding activation/deactivation of Pre-MG are included in the following LS from RAN4 (R2-2200125 [2]):

- Define separate UE capabilities for different pre-MG activation/deactivation mechanisms (i.e. NW-Controlled activation/deactivation mechanism and UE autonomous activation/deactivation mechanism).

However, there is no clear motivation to also support the explicit per BWP indication approach and adopting a similar approach as for Rel-16’s ‘no inter-frequency gap’ is sufficient. Hence, the following.

There is no clear motivation for RAN2 to also support the case in which the NW needs to signal whether a pre-configured MG should activated upon switching to a new active BWP.

As done for Rel-16’s ‘inter-frequency no gap’, introduce a new (single) UE capability to indicate whether the UE can perform measurements according to Rel-17’s pre-configured MG.

## 2.2	Concurrent Measurement Gaps

The following was agreed during RAN2#116:

- RAN2 confirms the following understanding for concurrent gap operation: - 1. Concurrent gaps are multiple measurement gaps and each gap pattern could be associated with one or multiple frequency layers. - 2. Each frequency layer can be associated with only one of the concurrent gaps. - 3. Without considering pre-configured MG, concurrent gaps are always activated if it is setup by the network. - 4. No new gap pattern is introduced for concurrent gap, the existing R15/R16 gap pattern could be configured for the concurrent gaps.

While this has been agreed in RAN2#116bis-e:

- Introduce multiple gap configuration in IE MeasGapConfig (i.e. by configuring multiple GapConfig). - RAN2 don’t supports concurrent gap association to 3G/2G from signalling perspective, but the signalling shall be extendable if this need to be introduced. - For association between concurrent MG and measured frequencies: Indicate the associated gaps (via “gap ID”) in MO; (for PRS measurement, indicating in the association in MG configuration).

On this matter, we see no need to add more than a new single/general UE capability to indicate support of this feature.

Introduce a general UE capability to indicate support of Rel-17’s concurrent measurement gaps.

## 2.3	NCSG

The following has been included in RAN4’s LS (see R2-2200127 [3]):

Capability indication: Agreements: - How to indicate UE capability to support of NCSG feature before NW inquiring - Introduce a general UE capability for support of NCSG

It is thus straightforward that a general/single UE capability is needed to indicate support of the NCSG feature before NW inquiring, introduce a general UE capability for support of NCSG

Introduce a new general/single UE capability to indicate support of the Rel-17 NCSG feature.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	There is no clear motivation for RAN2 to also support the case in which the NW needs to signal whether a pre-configured MG should activated upon switching to a new active BWP.

Based on the discussion in the previous sections we propose the following:

Proposal 1	As done for Rel-16’s ‘inter-frequency no gap’, introduce a new (single) UE capability to indicate whether the UE can perform measurements according to Rel-17’s pre-configured MG.

Proposal 2	Introduce a general UE capability to indicate support of Rel-17’s concurrent measurement gaps.

Proposal 3	Introduce a new general/single UE capability to indicate support of the Rel-17 NCSG feature.

# 4	References

RP-213312, “Revised WID on NR and MR-DC measurement gap enhancements”, Intel Corporation, MediaTek Inc., RAN#94e, 2021-12

R2-2200125, “LS on R17 NR MG enhancements – Pre-configured MG”, RAN4, RAN4#101-e, 2021-11

R2-2200127, “LS on NCSG”, RAN4, RAN4#101-e, 2021-11

R4-2120304, “LS on multiple concurrent MGs”, RAN4, RAN4#101-e, 2021-11

R2-2111471, “Report of [AT116-e][041][MGE] Concurrent MG”, MediaTek, RAN2#116-e, 2021-11