3GPP TSG-RAN WGX #117 electronic	R2- 2203508

21st February – 3rd March 2022

Online

Source:                    	DENSO CORPORATION

Title:  	C-plane related open issues on NCD-SSB

Document for:        	Discussion and decision

Agenda Item:         	8.12.4	NCD-SSB aspects

- Introduction

The C-plane related open issues on NCD-SSB has been discussed under the email discussion of [Pre117-e][105][RedCap]. This paper discusses the other NCD-SSB related issues not covered by the email discussion.

- Discussion

There is a scenario that RedCap UE in RRC_IDLE\INACTIVE performs RACH on the separate initial DL/UL BWP where CD-SSB is not present. The RRC connection establishment procedure is invoked over the separate initial DL/UL BWP. According to the current RRC specification (sub-clause 5.3.3.3), the UE continues to perform cell reselection and measurements for it, even after submitting the RRCSetupRequest message to the lower layer, as excerpted below.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.6.

Likewise, the same UE behaviour has been defined for the RRC connection resume procedure (sub-clause 5.3.13.3). Given that SSB is not present on the separate initial DL BWP, the RedCap UE cannot perform cell re-selection related measurements. Therefore, an exception needs to be allowed for the RedCap UE. The following is proposed.

Proposal 1:	Upon submitting the RRCSetupRequest/RRCResumeRequest message to the lower layer, if the RedCap UE is in the separate DL BWP where CD-SSB is not present, the RedCap UE does not have to continue cell re-selection related measurements as well as cell re-selection evaluation.

Furthermore, there is a case that RRC setup/resume request is rejected by the network. There is also a failure case that T300 or T319 is expired, e.g. due to the random access problem. In this case, if the RedCap UE resides in the separate initial DL BWP, the RedCap UE cannot perform cell reselection even though the UE is still in RRC_IDLE/INACTIVE. Thus, the UE should move back to the default initial DL BWP where SSB is present to perform cell reselection and paging monitoring. The following is proposed.

Proposal 2:	Upon the failure of RRC connection setup/resume, if the RedCap UE is in the separate DL BWP where CD-SSB is not present, the RedCap UE moves back to the default initial DL BWP where CD-SSB is present.

- Summary and proposal

This paper discussed the other NCD-SSB related issues not covered by [1]. In summary, the followings were proposed.

Proposal 1:	Upon submitting the RRCSetupRequest/RRCResumeRequest message to the lower layer, if the RedCap UE is in the separate DL BWP where CD-SSB is not present, the RedCap UE does not have to continue cell re-selection related measurements as well as cell re-selection evaluation.

Proposal 2:	Upon the failure of RRC connection setup/resume, if the RedCap UE is in the separate DL BWP where CD-SSB is not present, the RedCap UE moves back to the default initial DL BWP where CD-SSB is present.

- References

[1] R2-220XXXX, “Email discussion report for [Pre117-e][105][RedCap] CP open issues,” Ericsson (Rapporteur)