3GPP TSG RAN WG2#117-e	R2-2203737

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG-RAN WG1 Meeting #108-e	R1-2202760

E-meeting, February 21st – March 3rd, 2022

# Title:	LS on updated Rel-17 LTE and NR higher-layers parameter list

# Response to:	-

# Release:	Rel-17

# Work Item:	NR_feMIMO, NR_ext_to_71GHz, NR_IIOT_URLLC_enh, NR_NTN_solutions, NR_pos_enh, NR_redcap, NR_UE_pow_sav_enh, NR_cov_enh, NR_IAB_enh, NR_SL_enh, NR_MBS, NR_DSS, LTE_NR_DC_enh2, NR_RF_FR1_enh, NR_SmallData_INACTIVE, NB_IOTenh4_LTE_eMTC6, LTE_NBIOT_eMTC_NTN, LTE_terr_bcast_bands_part1

Source:	RAN WG1

To:	RAN WG2, RAN WG3

Cc:	RAN WG4

Contact Person:

Name:	Sorour Falahati

E-mail Address:	sorour.falahati@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	R1-2202759 (Consolidated higher layers parameter list for Rel-17 NR)

R1-2202758 (Consolidated higher layers parameter list for Rel-17 LTE)

1. Overall Description:

RAN1 has further updated the list of new higher layers parameters and updates on the existing higher-layer parameters for the Rel-17 work items.

RAN1 would like to clarify the following regarding the attached lists:

- The updates in the list as compared to the lists attached with R1-2202542 are highlighted in blue.

- In addition, RAN1 may see the need for additional update of the higher layer parameters. RAN1 will continue working on consolidating them and will share the corresponding updates with RAN2 and RAN3 in the next LS if needed.

- RAN1 understands that RAN2 and RAN3 can modify the RAN1 higher layers parameter list for the purpose of proper implementation of the functionalities, when needed.

For Enh. IIoT and URLLC WI, Row 60 and Row 61 are removed but the indication of the removal is missing in R1-2202542 which is handled in this LS to track properly the changes throughout the LS exchanges.

Additionally, RAN1 would like to inform RAN2 and RAN3 about the following agreements made in RAN1#108-e meeting for NR MBS WI:

NR MBS WI: Agreement Send an LS to inform RAN2 that the following parameters are NOT needed for PDSCH-Config-Multicast: - zp-CSI-RS-ResourceToAddModList, zp-CSI-RS-ResourceToReleaseList Agreement For multicast RRC_CONNECTED UEs, p-ZP-CSI-RS-ResourceSet can be configured in PDSCH-Config-Multicast for GC-PDSCH rate matching, subject to UE capability. For PDSCH resource mapping with RE symbol level granularity, - the REs indicated by p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast are declared as not available for GC-PDSCH. - p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for unicast do not apply for GC-PDSCHs. - p-ZP-CSI-RS-ResourceSet in PDSCH-Config-Multicast for multicast do not apply for unicast PDSCHs. - The total number of p-ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 Also include this agreement in an LS to RAN2. Agreement For multicast RRC_CONNECTED UEs, sp-ZP-CSI-RS-ResourceSetsToAddModList can be configured in PDSCH-Config-Multicast for GC-PDSCH rate matching, subject to UE capability. For PDSCH resource mapping with RE symbol level granularity, - the REs indicated by sp-ZP-CSI-RS-ResourceSetsToAddModList configured in PDSCH-Config-Multicast are declared as not available for GC-PDSCH when their activation delivered by unicast PDSCH is applied. - sp-ZP-CSI-RS-ResourceSetsToAddModList configured in PDSCH-Config for unicast do not apply for GC-PDSCHs. - sp-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config-Multicast for multicast do not apply for unicast PDSCHs. - The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 Also include this agreement in an LS to RAN2. Agreement Update the previous agreement for p-ZP-CSI-RS-ResourceSet as below: For multicast RRC_CONNECTED UEs, p-ZP-CSI-RS-ResourceSet can be configured in PDSCH-Config-Multicast for GC-PDSCH rate matching, subject to UE capability. For PDSCH resource mapping with RE symbol level granularity, - the REs indicated by p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast are declared as not available for GC-PDSCH. - p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for unicast do not apply for GC-PDSCHs. - p-ZP-CSI-RS-ResourceSet in PDSCH-Config-Multicast for multicast do not apply for unicast PDSCHs. - The total number of periodic ZP-CSI-RS-Resources p-ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 - If p-ZP-CSI-RS-ResourceSet is configured in both PDSCH-Config and PDSCH-Config-Multicast, it is subject to UE capability whether the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast can be different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config. Also include this agreement in an LS to RAN2.

RAN1 would like to inform RAN2 and RAN3 that the recommendations in R1-2202913 are used for preparation of the list of higher layers parameters.

2. Actions:

To RAN WG2 and RAN WG3

ACTION: RAN1 would like to kindly ask RAN2 and RAN3 to consider the design of the corresponding higher-layer parameters in Rel-17.

3. Date of Next RAN WG1 Meetings:

TSG RAN WG1 Meeting #109-e	May 16th - 27th, 2022 	e-meeting

TSG RAN WG1 Meeting #110	August 22nd - 26th, 2022 	Toulouse, France