3GPP TSG RAN WG2#117-e	R2-2203891

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG RAN WG1 Meeting #108-e	R1-2202704

e-Meeting, February 21 – March 3, 2022

Title:	LS on Stage 2 description for fast SCell activation

Response to:

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2

Source:	RAN WG1

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Frank Long

E-mail Address:	frank.longyi {@ }huawei.com

Attachments:

1. Overall Description:

RAN1 has endorsed the following TP of stage 2 description for aperiodic CSI-RS for tracking for fast SCell activation.

TS 38.300:

----------------------------------------------- TP start------------------------------------------------ ## 10.6 Activation/Deactivation Mechanism ==== Unchanged parts ==== To enable fast SCell activation when CA is configured, one dormant BWP can be configured for an SCell. If the active BWP of the activated SCell is a dormant BWP, the UE stops monitoring PDCCH and transmitting SRS/PUSCH/PUCCH on the SCell but continues performing CSI measurements, AGC and beam management, if configured. A DCI is used to control entering/leaving the dormant BWP for one or more SCell(s) or one or more SCell group(s). The dormant BWP is one of the UE’s dedicated BWPs configured by network via dedicated RRC signalling. The SpCell and PUCCH Scell cannot be configured with a dormant BWP. To enable fast SCell activation when CA is configured, aperiodic CSI-RS for tracking for fast SCell activation can be configured for an SCell to assist AGC and time/frequency synchronization. A MAC CE is used to trigger activation of one or more SCell(s) and trigger the aperiodic CSI-RS for tracking for fast SCell activation for a (set of) deactivated SCell(s). ==== Unchanged parts ==== ----------------------------------------------- TP end ------------------------------------------------

2. Actions:

To: RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above endorsed TP into consideration.

3. Date of Next TSG-RAN1 Meetings:

3GPP RAN1#109-e	16 - 27, May 2022	Online

3GPP RAN1#110	22 - 26, August 2022	Toulouse, FR