3GPP TSG RAN WG2#117-e	R2-2204142

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG-RAN WG2 Meeting #102-e	R4-2207032

E-Meeting, 21 Feb. – 03 March, 2022

Title:	Reply LS on RAN2’s agreement for MUSIM gaps

Response to:	R2-2201717

Release:	Release 17

Work Item:	LTE_NR_MUSIM-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	xusheng wei

- E-mail Address:	xusheng.wei@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 thanks for RAN2 LS R2-2201717.

Regarding the MGL and MRGP for periodic gaps for MUSIM purpose, and MGL for aperiodic gap in R2-2201717, RAN4 specifies the following periodic and aperiodic gap patterns:

| MUSIM Gap Pattern Id | MUSIM Gap Length (MGL, ms) | MUSIM Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 10 | 80 |
| 13 | 20 | 160 |
| 14 | 6 | 320 |
| 15 | 6 | 640 |
| 16 | 6 | 1280 |
| 17 | 6 | 2560 |
| 18 | 10 | 320 |
| 19 | 10 | 640 |
| 20 | 10 | 1280 |
| 21 | 10 | 2560 |
| 22 | 20 | 320 |
| 23 | 20 | 640 |
| 24 | 20 | 1280 |
| 25 | 20 | 2560 |
| 26 | 20 | 5120 |
| 27 | 10 | NA |
| 28 | 20 | NA |
| Note 1: Measurement gap pattern #27,#28 are the aperiodic gap pattern without MGRP. |  |  |

Regarding Q3 in the LS R2-2201717 copied below,

3: RAN2 keep three gaps agreement (i.e., 2 periodic gaps and 1 aperiodic gap) for now. However, RAN2 also sees the low efficiency in some cases if only 2 periodic gaps are allowed.

RAN2 would like RAN4 to clarify if one additional periodic gap can be possible without sacrificing NW A performance?

RAN4 discusses the application scenario of question 3 and concludes that RAN4 will not take any work on Q3 of [R2-2201717] within Rel-17 time frame. From signalling perspective, RAN4 understand it is up to RAN2 to determine whether to introduce signalling to support configurations with more than two periodic gaps for MUSIM in Rel-17 or not.

2. Actions:

To RAN2

ACTION: 	RAN4 kindly asks RAN2 to take RAN4 above RAN4 answers into consideration in their future work.

3. Date of Next TSG-RAN WG4 Meetings:

3GPP RAN4 103e May 16 -27, 2022