3GPP TSG RAN WG2#117-e	R2-2204197

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG-RAN WG4 Meeting # 102-e	R4-2206602

Online Meeting, 21 February – 03 March 2022

Title:	Reply LS on DC location for >2CC

Response to:	R2-2201978, (R4-2203517) “Reply LS on DC location for >2CC”, RAN2

Release:	Rel-17

Work Item:	NR_RF_FR2_req_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Ville Vintola

vvintola@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 would like to thank you RAN2 for their Reply LS on DC location for >2CC.

RAN4 has discussed the presented questions and conclude the following:

For the Question 1 there are different understanding on the meaning of “Per band per band combination” and “Per intra-band UL CA component per band combination” in RAN4. And it is RAN4 understanding as below:

For an intra-band UL CA configuration, a UE may have one or two default DC locations. For all the CCs within this band, there can be one or two CC groups where each CC group is associated with one default DC location. CC group can consist of multiple CCs. The default DC location framework applies to each CC group.

For Question 2, RAN4 has concluded that UE should declare if it has two default DC locations via dualPA-Architecture capability. Frequency component type is same for both default DC locations. For the calculation of the default DC locations, the component carriers associated with each default DC location must be known. UE may need a non-zero offset for each DC location.

Additionally, RAN4 has discussed and would like to clarify previous LS that in some cases for intra-band non-contiguous CA with one LO, the DC frequency may be outside the configured spectrum. The exact location of the carrier leakage must be known.

# 2	Actions

To RAN2

ACTION: RAN4 respectfully asks RAN2 to take the above information into consideration in their work.

# 3	Dates of next TSG RAN WG 4 meetings

RAN4#103-e	2022-05-16 2022-05-27 		Electronic

RAN4#104	2022-08-22 2022-08-26 		Toulouse, FR