3GPP TSG RAN WG2#118-e	R2-2204454

e-Meeting, 9th - 20th May, 2022

3GPP TSG RAN WG1 Meeting #108-e 	R1-2202816

e-Meeting, Feb 21 – March 3, 2022

Title:	Reply LS to RAN4 on PEMAX for NR-V2X

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact Person:

#### Name:	Fan Yang

Tel. Number:

E-mail Address:	james.yangfan@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 thanks RAN4 for the LS on PEMAX for NR-V2X and would like to provide replies to the following issues:

Issue 1: It is RAN4 understanding that the parameter to limit the transmitted power of PSSCH/PSCCH for V2X can be used in both in-coverage and out-of-coverage. RAN4 would like to check with RAN1 and RAN2 which parameter (sl-maxTxPower, sl-MaxTransPower, SL-TxPower) is the correct one to be used to fulfill the purpose?

Answer: In RAN1 understanding, sl-MaxTransPower is the correct parameter configuring PEMAX,c which is to limit the transmitted power of PSSCH/PSCCH  in a resource pool for V2X and can be used in both in-coverage and out-of-coverage.

Issue 2: RAN4 also had some discussion on whether the parameter should be associated with or without a serving cell on the NR V2X carrier, and three options are proposed: - Option 1: The parameter can be associated either with a serving cell or without a serving cell, and it can be configured separately with p-max for Uu - Option 2: The parameter can be associated either with a serving cell or without a serving cell, when the parameter is associated with a serving cell, PEMAX,c is the smaller value given by this parameter for SL and p-max for Uu of that serving cell. - Option 3: when UE is associated with a serving cell on the NR V2X carrier, p-max is used for serving cell c; when the UE is not associated with a serving cell on the NR V2X carrier, the parameter given for SL in RAN2 specification is used. RAN4 would like to check which option is aligned with the RAN1 and RAN2 specification.

Answer: It is RAN1 understanding that the parameter can be associated either with a serving cell or without a serving cell, and it can be configured separately with p-max for Uu, i.e. Option 1.

2. Actions:

To RAN4

ACTION: 	RAN1 kindly asks RAN4 to take the above reply into account for further work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e	16 – 27 May 2022			E-Meeting