3GPP TSG RAN WG2#118-e	R2-2204474

e-Meeting, 9th - 20th May, 2022

3GPP TSG-RAN4 Meeting #102-e	R4-2206890

Online, 21 February – 3 March, 2022

Title:	LS on R17 MG enhancement - NCSG

Response to:

Release:	Rel-17

Work Item:	NR_MG_enh-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact person:

#### Name:	Qiming Li

#### E-mail Address:	li_qiming@apple.com

Attachments: None

# 1	Overall description

RAN4 further discussed NCSG design in RAN4#102-e and reached the following agreements which may have RAN2 impact:

- Scenarios and use cases - For different types of measurement with NCSG: - NCSG for CSI-RS based inter-frequency measurement with gap is NOT supported in R17. - NCSG patterns - Timing offset for NCSG - The offset of NCSG refers to the starting point of VIL1. - MGTA for NCSG - It is RAN4 understanding that mgta=0.25ms is useless for NCSG. RAN4 will not define requirement for per-UE or per-FR1 NCSG with mgta=0.25ms. It is up to RAN2 whether to consider it in RRC signaling design. - Measurement related requirements - Tolerance requirement for deriveSSB-IndexFromCell-inter - Tolerance requirement for deriveSSB-IndexFromCell-inter (△t) needs to be defined in RAN4: - When deriveSSB-IndexFromCell-inter is enabled, the UE assumes frame boundary alignment (including half frame, subframe and slot boundary alignment) across cells on the target carrier and reference carrier is within a tolerance not worse than △t and the SFNs of all cells on the target carrier and reference carrier are the same, where △t will be specified in TS38.133. - Requirement applicability of deriveSSB-IndexFromCell-inter - In RAN4#101-e-bis RAN4 agreed deriveSSB-IndexFromCell-inter can only be configured if the SCS of SSB is the same between target cell and the reference cell. - In RAN4#102e RAN4 further discussed and agreed that deriveSSB-IndexFromCell-inter can also be configured even if the SCS of SSB is different between target cell and the reference cell. - UE feature list discussion on NCSG support - On top of agreed UE capability on NCSG support 19-1 as captured in R4-2202400, RAN4 further agreed on the following capabilities: Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between Ues (V2X WI only)”.; Consequence if the feature is not configured by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 19-X; Network controlled small gap (NCSG); Support of per-FR NCSG; 19-1; yes; no; UE cannot be configured with per-FR NCSG; per-UE; No; No; Optional with capability signalling. 19-X+1; Network controlled small gap (NCSG); Supported NCSG patterns; 19-1; yes; no; Network does not know whether some NCSG patterns can be configured to UE; per-UE; No; No; Optional with capability signalling NCSG patterns #0, #1 are conditional mandatory if UE supports 19-1 NCSG patterns #13, #14 are conditional mandatory if UE supports 19-1 and 19-X. 19-X+2; Network controlled small gap (NCSG); Supported NR-only NCSG patterns; 19-1; yes; no; Network does not know whether some NR-only NCSG patterns can be configured to UE; per-UE; No; No; Optional with capability signalling NCSG patterns #2 and #3 are conditional mandatory if UE supports 19-1 NCSG patterns #17 and #18 are conditional mandatory if UE supports 19-1 and a FR2 band. - Note: It may be a bit redundant for UE who already reports 19-X+1 to report 19-1 additionally. It is up to RAN2 for signaling overhead reduction.

Besides, RAN4 also discussed the question from RAN2 in R2-2201935:

Q: Whether to support simultaneous configurations on the following combinations? - NCSG FR1 gap + NCSG FR2 gap - Legacy FR1 gap + NCSG FR2 gap - Legacy FR2 gap + NCSG FR1 gap - One legacy perUE gap + one NCSG perUE gap - One legacy perUE gap + NCSG FR1 gap - One legacy perUE gap + NCSG FR2 gap

RAN4 reached the following agreements:

- RAN4 confirms the following combinations are supported:

- NCSG FR1 gap + NCSG FR2 gap

- Legacy FR1 gap + NCSG FR2 gap

- Legacy FR2 gap + NCSG FR1 gap

- RAN4 confirms that the following combinations are not supported from RAN4 requirement perspective:

  - One legacy perUE gap + one NCSG perUE gap

  - One legacy perUE gap + NCSG FR1 gap

  - One legacy perUE gap + NCSG FR2 gap

# 2	Actions

To RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to take the above information into account and implement the configuration of NCSG.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #103-e	16 - 27 May 2022	Online

TSG RAN WG4 Meeting #104	22 - 26 August 2022	Toulouse, FR