3GPP TSG RAN WG2 Meeting #118-e	R2-2204552

e-Meeting, May 9th – 20th, 2022

Source:	Sharp

Title:	Clarification on resource re-selection for pre-empted resource with SL DRX

Agenda Item:	6.15.2.3

Document for:	Discussion and Decision

Introduction

In this document, we share our views on one remaining issue on resource (re-)selection for pre-emption in SL DRX.

Discussion

Resource re-selection for pre-emption in SL DRX

In RAN2#117e, it was agreed that the reselected resource should not be earlier than the pre-empted resource for resource re-selection due to pre-emption, shown as follows,

Agreements on SL DRX open issues: 22: For resource reselection due to pre-emption, the reselected resource should not be earlier than the pre-empted resource in time domain.

When the pre-emption occurs, PHY shall report the pre-empted resource and MAC layers accordingly perform resource re-selection for the pre-empted resource. It is noted that one prior SCI indicates the pre-empted resource to RX UE and the RX UE keeps monitoring SCI from the TX UE till the SL HARQ Retransmission timer expires which is initiated after RTT timer expires. If the re-selected resource comes after the SL Retransmission timer expires, the receiver UE could possibly miss the SCI. Therefore, besides the re-selected resource comes after the pre-empted resource, the time gap between the re-selected resource and pre-empted resource is not larger than the duration of SL HARQ Retransmission timer.

Proposal 1: For resource re-selection of the pre-emption check in SL DRX, the time gap between the re-selected resource and the reported pre-empted resource is not larger than the duration of SL HARQ Retransmission timer.

Conclusion

In this contribution, we discuss one remaining issue on resource (re-)selection for pre-emption in SL DRX, and make the following proposal:

Proposal 1: For resource re-selection of the pre-emption check in SL DRX, the time gap between the re-selected resource and the reported pre-empted resource is not larger than the duration of SL HARQ Retransmission timer.