3GPP TSG RAN WG2 Meeting #118-e	R2-2204555

Electronic meeting, May 9th – May 20th, 2022

Agenda item:	6.1.3.1

Source:	TD Tech, Chengdu TD Tech

Title:	MBS reception interruption problem in LTE and SFN in NR MBS

Document for:	Discussion and Decision

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e to provide MBS in a cell. According to the agreement on NR MBS in the past RAN1/RAN2 meetings, multicast mode and broadcast mode are used to provide multicast sessions and broadcast sessions respectively. The standard work for broadcast mode is based on SC-PTM in LTE.

SC-PTM in LTE is used to transmit broadcast sessions and multicast sessions. In other word, SC-PTM in LTE can’t distinguish the type of an MBS session over Uu. Furthermore, SC-PTM suffers a serious problem in the commercial network: a RRC_IDLE UE will undergo a long MBS reception interruption time during its mobility from the source cell to the target cell and a RRC_CONNECTED UE will undergo a long MBS reception interruption time during handover.

The analysis of the MBS reception interruption time during the UE mobility in LTE is given in section 2 and the solution to the MBS reception interruption problem in NR MBS is suggested in section 3.

In section 4, support of SFN in NR MBS is discussed. In section 5, the proposals for solving the MBS interruption problem in NR MBS and supporting SFN in NR MBS are suggested.

- MBS reception interruption problem during UE mobility in LTE

In LTE, SC-PTM introduces SC-MCCH and SIB20. SC-MCCH carries the SC-PTM configuration information which consists of the configuration information of all MBS sessions. SIB 20 carries the SC-MCCH configuration information.

When a RRC_IDLE UE moves from the source cell to the target cell which is transmitting the MBS session received by the UE, it stops receiving the MBS session in the source cell. It acquires the configuration information of the MBS session in the target cell and then continue to receive the MBS session. The specific procedure for a RRC_IDLE UE to restore the MBS session reception in the target cell is listed as below.

- Receive PBCH, SIB1 and SIB2 (SIB2 carries the cell barring information)

- Receive SIB20 and other SIBs according to the scheduling information on SIB1

- Acquire SC-MCCH according to the SC-MCCH configuration information on SIB20

- Receive the MBS session according to the configuration information of the MBS session on SC-MCCH

Therefore, a RRC_IDLE UE will undergo the MBS interruption in the target cell. Similarly a RRC_CONNECTED UE receiving an MBS session in the source cell will undergo the MBS interruption in the target cell during handover.

PBCH is transmitted one times every 10ms. SIB1 is transmitted one times every 20ms. The period of SIB2/SIB20 is denoted by T1/T2ms. The delay introduced by the PBCH/SIB1/SIB2/SIB20 reception is at least 10/20/T1/T2ms. Use Pms to denote the repetition period of SC-MCCH. The delay introduced by the SC-MCCH reception is at least Pms. Use Dms to denote the period of the DRX mode of the MBS. The delay introduced by the first TB of the MBS in the target cell is at least Dms. Therefore, the MBS interruption time is no less than Ams, where A=10+20+T1+T2+P+D=30+T1+T2+P+D.

In the commercial network, some important SIBs have the period of 160ms while the other SIBs have the period of 320ms or 640ms. In detail, SIB2/SIB20 has the period of 160/640ms. The radio resource for the SIBs is about 5%. If the period of each SIB is shortened, the radio resource for the SIBs will be increased. Therefore, although the minimum period of SIB20 is 80ms, T2 is generally no less than 640ms. Otherwise, SIB20 will consume much more radio resource. Due to the same reason, although the minimum period of SC-MCCH is 20ms, P=80ms is used in the commercial network. For a voice service, the minimum value of D is D=20ms. Therefore, the MBS interruption time of a voice service is about A=930ms, which is a worse scenario for acquiring the configuration information of the MBS session in the target cell. But it’s not the worst scenario because a RRC_IDLE UE may not decode SIB2/SIB20/SC-MCCH correctly for the first time. Certainly, there exists a better scenario where the MBS interruption time A=10+20+(T1+T2+P+D)/2=480ms.

A voice service has one packet every 20ms. During the 930ms interval, UE misses 47 packets, which leads to the bad service experience in the UE. During the 480ms interval, UE misses 24 packets, which also leads to the bad service experience in the UE.

If the period of SIB20 is shortened as 320ms, the MBS interruption time A can be A=30+T1+T2+P+D=610ms or A=30+(T1+T2+P+D)/2=320ms, which also leads to the bad experience in the UE.

For an MBS session, if the source cell and the target cell have the same configuration information, the UE has no need to receive SIB20 and SC-MCCH. It can directly receive the MBS session after it receives PBCH, SIB1 and SIB2. In detail, when the source cell and target cell have the same configuration information, the UE restores the MBS reception in the target cell as below.

- Receive PBCH, SIB1 and SIB2

- Receive the MBS session according to the configuration information acquired in the source cell

During the above procedure, the MBS interruption time is B=10+20+T1+D=10+20+160+20=210ms or B=10+20+(T1+D)/2=30+80+10=120ms. The service experience in the UE will be improved greatly.

- Solutions to MBS interruption problem in NR MBS

Due to the long MBS interruption time introduced by SC-PTM in LTE during cell reselection and handover, we hope the existing MBS interruption problem in LTE will be solved from the first NR MBS release. The following two options can be used to solve the MBS interruption problem in NR MBS.

Option 1: For an MBS session provided within a SA indicated by SAI, the PTM configuration information of the MBS session can be area specific. That is, an indicator is used to indicate the PTM configuration information of the MBS session is area specific.

Option 2: For an MBS session in the source cell and for each neighbour cell which provides the MBS session, one additional bit is used to indicate whether or not the neighbour cell has the same PTM configuration information as the source cell.

The above two options have different application scenarios. They are supplementary to each other. If an MBS session is transmitted within a predefined area, option 1 can be used to make the PTM configuration information of the MBS session area specific. With such configuration, a UE receiving the MBS session has no need to reacquire the PTM configuration information during its mobility within the area, which will reduce the MBS reception interruption time greatly during its mobility. If an MBS session is only transmitted in several cells within a predefined area, option 2 can be used to indicate whether or not the source cell and target cell have the same PTM configuration information for the MBS session. If the source cell and target cell have the same PTM configuration information, the UE has no need to reacquire the PTM configuration information in the target cell, which will reduce the MBS reception interruption time greatly.

- Support of SFN in NR MBS

SFN can be realized within the cells belonging to a gNB-CU/gNB-DU. In order to make UE know an SFN area and which MBS sessions are broadcast in SFN mode within the SFN area, it’s better to reuse the above two options: Option 1 and Option 2. If Option 1/Option 2 is reused, UE has no need to re-acquire the MBS configuration information within the SFN area. Furthermore, the QOE of UE can be improved.

- Conclusion

Based on the discussion in sections 2-4, in order to solve the MBS interruption problem in NR MBS and support SFN in NR MBS, the following proposal is suggested.

Proposal 1: In order to solve the MBS reception interruption problem by SC-PTM in LTE and support SFN in NR MBS. The following two options which are supplementary to each other are suggested.

Option 1: For an MBS session provided within a SA indicated by SAI, the PTM configuration information of the MBS session can be area specific.

Option 2: For an MBS session in the source cell and for each neighbour cell which provides the MBS session, one additional bit is used to indicate whether or not the neighbour cell has the same PTM configuration information as the source cell.

- References

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e.

- R2-2008929, Discussion on the protocol stack for NR MBS, RAN2 Meeting #112-e, Chengdu TD Tech, TD Tech

- R1-2007639, Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs, RAN1 Meeting#103-e, Chengdu TD Tech, TD Tech

- R2-2108034, Service continuity for delivery mode 2, RAN2#115-e, Chengdu TD Tech, TD Tech