3GPP TSG-RAN WG2 Meeting #118 Electronic 	R2-2204614

9th  May – 19th May 2022

Agenda item:	6.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Alternative ASN.1 proposal for MUSIM gap configuration

WID/SID:	LTE_NR_MUSIM-Core

Document for:	Discussion and Decision

# 1	Introduction

The ASN.1 for the MUSIM gap configuration for periodic and aperiodic gap could be improved as there is a lot of repetition in the design which could be streamlined.

In this contribution a text proposal is made as a text proposal for the same in the form of a draft CR.

# 2	Text proposal

The ASN.1 is restructured to clearly isolate the periodic and aperiodic gap configurations into separate IEs for clear readability. Changes are proposed in the draft CR format as follows. Also extension markers are put for the gap length (it may be further discussed if a few spare values should be added instead).

First Modified Subclause

-- ASN1START

-- TAG-MUSIM-GAPCONFIG-START

MUSIM-GapConfig-r17 ::=  SEQUENCE {

musim-GapToReleaseList-r17       		 SEQUENCE (SIZE (1..2)) OF MUSIM-GapID-r17               OPTIONAL, -- Need N

musim-PeriodicGapToAddModList-r17        SEQUENCE (SIZE (1..2)) OF MUSIM-PeriodicGapInfo-r17                  OPTIONAL, -- Need N

musim-AperiodicGap-r17           		 MUSIM-AperiodicGapInfo-r17              OPTIONAL, -- Need N

...

}

MUSIM-PeriodicGapInfo-r17 ::=          SEQUENCE {

musim-GapID-r17                        			MUSIM-GapID-r17,

musim-GapLength-r17                    			ENUMERATED {ms3, ms4, ms6, ms10, ms20, ...},

musim-GapRepetitionAndOffset-r17       			CHOICE {

ms20-r17                            		INTEGER (0..19),

ms40-r17                            		INTEGER (0..39),

ms80-r17                            		INTEGER (0..79),

ms160-r17                           		INTEGER (0..159),

ms320-r17                           		INTEGER (0..319),

ms640-r17                           		INTEGER (0..639),

ms1280-r17                          		INTEGER (0..1279),

ms2560-r17                          		INTEGER (0..2559),

ms5120-r17                          		INTEGER (0..5119),

...

}

}

MUSIM-AperiodicGapInfo-r17 ::=  SEQUENCE {

musim-GapID-r17                        MUSIM-GapID-r17,

starting-SFN-r17            		   INTEGER (0..1023) OPTIONAL,

startingSubframe-r17        		   INTEGER (0..9)

musim-GapLength-r17                    ENUMERATED {ms3, ms4, ms6, ms10, ms20, ...}

}

-- TAG-MUSIM-GAPCONFIG-STOP

-- ASN1STOP

Second Modified Subclause

1> if the RRCReconfiguration message includes the ul-GapFR2-Config: 2> perform the FR2 UL gap configuration procedure as specified in 5.3.5.13c; 1> if the RRCReconfiguration message includes the musim-GapConfig: 2> for each musim-GapID included in the received musim-GapToReleaseList: 3> release the MUSIM periodic gap associated to the musim-GapID; 2> for each musim-GapID included in the received musim-PeriodicGapToAddModList and musim-AperiodicGap that is part of the current UE configuration: 3> reconfigure the entry with the value received for this musim-GapID; 3> else: 4> add a new entry for this musim-GapID; 1> if the RRCReconfiguration message includes the appLayerMeasConfig: 2> perform the application layer measurement configuration procedure as specified in 5.3.5.13d;

End of changes

# 3	Conclusion

In this contribution a text proposal was made as a text proposal for the same in the form of a draft CR. The ASN.1 is restructured to clearly isolate the periodic and aperiodic gap configurations into separate IEs for clear readability.

Proposal 1: Discuss the text proposal for improving the ASN.1 for the MUSIM gap configuration.

# References

[1] RP-193263 – WID For Support of Multi-SIM devices -Rel-17

[2] TS38.331 V17.0.0 -RRC Specification Rel-17