3GPP TSG-RAN WG2 Meeting #118 Electronic 	R2-2204618

9th  May – 19th May 2022

Agenda item:	6.3.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarifications for MUSIM UAI handling

WID/SID:	LTE_NR_MUSIM-Core

Document for:	Discussion and Decision

# Introduction

In this discussion paper we analyse the UAI procedure for MUSIM operation as per current specification and propose some clarification based on the analysis.

# Discussion

Clarification on MUSIM UAI Triggering

When UAI is triggered with gap preference for MUSIM operation, network may not configure requested configuration if the current operation does not allow creation of scheduling gaps. In such case, the RRC Reconfiguration will not include the MUSIM gap configuration for the gap which was not accepted. In such case, the UE may try after sometimes if prohibit timer is not running, by sending UAI with the same MUSIM Gap preference. As per current specification, the UE is allowed to send the Gap-preference only if it is different from earlier gap preference it has sent. In the above scenario, the gap preference which the UE wants to try is same as the earlier one but it was not configured. So we propose that specification changes clarify that UE is allowed to send UAI with gap configuration which was not accepted by network earlier based on prohibit timer value.

Proposal 1: Specification for MUSIM UAI is modified to support  UE behaviour to retry sending the UAI not configured in response to earlier UAI transmission. The text proposal is given in Table 1.

The MUSIM assistance information for leaving connected state is meant to enable the UE to leave RRC_CONNECTED state for MUSIM operation. There could be other triggers possible for leaving RRC_CONNECTED state due to different applications. For example, the NR-NTN UE may need to leave RRC connected state for GNSS operation if GNSS information is updated. There could be other triggers also. To avoid UE misusing the leave-indication capability for other purpose, we propose to clarify that UE trigger UAI for leave indication only if UE is leaving RRC connection for MUSIM operation. As there is only one application can trigger the UAI for leave indication, there is no possibility for triggering another UAI when the timer for leave is running. So we also propose to remove the timer check in the current specification.

Proposal 2: UAI triggering for leave indication is clarified that it is applicable only for leaving RRC connected state for MUSIM operation. The text proposal is given in Table 1.

Table 1: Proposed Specification changes for MUSIM UAI clarification

#### 5.7.4.2 Initiation A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report and upon change of delay budget preference. A UE capable of providing overheating assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting internal overheating, or upon detecting that it is no longer experiencing an overheating condition. A UE capable of providing IDC assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting IDC problem if the UE did not transmit an IDC assistance information since it was configured to provide IDC indications, or upon change of IDC problem information. A UE capable of providing its preference on DRX parameters of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a preference on DRX parameters and upon change of its preference on DRX parameters. A UE capable of providing its preference on the maximum aggregated bandwidth of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum aggregated bandwidth preference and upon change of its maximum aggregated bandwidth preference. A UE capable of providing its preference on the maximum number of secondary component carriers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of secondary component carriers preference and upon change of its maximum number of secondary component carriers preference. A UE capable of providing its preference on the maximum number of MIMO layers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of MIMO layers preference and upon change of its maximum number of MIMO layers preference. A UE capable of providing its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a minimum scheduling offset preference and upon change of its minimum scheduling offset preference. A UE capable of providing assistance information to transition out of RRC_CONNECTED state may initiate the procedure if it was configured to do so, upon determining that it prefers to transition out of RRC_CONNECTED state, or upon change of its preferred RRC state. A UE capable of providing configured grant assistance information for NR sidelink communication in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide traffic pattern information and upon change of traffic patterns. A UE capable of providing an indication of its preference in being provisioned with reference time information may initiate the procedure upon being configured to provide this indication, or if it was configured to provide this indication and upon change of its preference. A UE capable of providing an indication of its preference in FR2 UL gap may initiate the procedure if it was configured to do so, upon detecting the need of FR2 UL gap activation/deactivation. A UE capable of providing MUSIM assistance information may initiate the procedure if it was configured to do so, upon determining that it needs to leave RRC_CONNECTED state, or upon determining it needs the gaps, or upon change of the gap information without leaving RRC_CONNECTED state. A UE capable of relaxing its RLM measurements of a cell group in RRC_CONNECTED state shall initiate the procedure for providing an indication of its relaxation state for RLM measurements upon being configured to do so, and upon change of its relaxation state for RLM measurements in RRC_CONNECTED state. A UE capable of relaxing its BFD measurements in serving cells of a cell group in RRC_CONNECTED shall initiate the procedure for providing an indication of its relaxation state for BFD measurements upon being configured to do so, and upon change of its relaxation state for BFD measurements in RRC_CONNECTED state. Editor's NOTE: How to / whether to limit or remove impact of prohibit timer w.r.t. consistency between UE relaxation state and the corresponding knowledge at gNB. A UE capable of SDT initiates this procedure when data and/or signalling mapped to radio bearers that are not configured for SDT becomes available during SDT (i.e. while T319a is running). A UE capable of providing its preference for SCG deactivation may initiate the procedure if it was configured to do so, upon determining that it prefers or does no more prefer the SCG to be deactivated. A UE capable of providing an indication of fulfilment of the RRM measurement relaxation criterion in connected mode may initiate the procedure upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode. Upon initiating the procedure, the UE shall: 1> if configured to provide delay budget report: 2> if the UE did not transmit a UEAssistanceInformation message with delayBudgetReport since it was configured to provide delay budget report; or 2> if the current delay budget is different from the one indicated in the last transmission of the UEAssistanceInformation message including delayBudgetReport and timer T342 is not running: 3> start or restart timer T342 with the timer value set to the delayBudgetReportingProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide a delay budget report; 1> if configured to provide overheating assistance information: 2> if the overheating condition has been detected and T345 is not running; or 2> if the current overheating assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message including overheatingAssistance and timer T345 is not running: 3> start timer T345 with the timer value set to the overheatingIndicationProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide overheating assistance information; 1> if configured to provide IDC assistance information: 2> if the UE did not transmit a UEAssistanceInformation message with idc-Assistance since it was configured to provide IDC assistance information: 3> if on one or more frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself; or 3> if on one or more supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself: 4> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information; 2> else if the current IDC assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information; NOTE 1: The term "IDC problems" refers to interference issues applicable across several subframes/slots where not necessarily all the subframes/slots are affected. NOTE 2: For the frequencies on which a serving cell or serving cells is configured that is activated, IDC problems consist of interference issues that the UE cannot solve by itself, during either active data exchange or upcoming data activity which is expected in up to a few hundred milliseconds. For frequencies on which a SCell or SCells is configured that is deactivated, reporting IDC problems indicates an anticipation that the activation of the SCell or SCells would result in interference issues that the UE would not be able to solve by itself. For a non-serving frequency, reporting IDC problems indicates an anticipation that if the non-serving frequency or frequencies became a serving frequency or serving frequencies then this would result in interference issues that the UE would not be able to solve by itself. 1> if configured to provide its preference on DRX parameters of a cell group for power saving: 2> if the UE has a preference on DRX parameters of the cell group and the UE did not transmit a UEAssistanceInformation message with drx-Preference for the cell group since it was configured to provide its preference on DRX parameters of the cell group for power saving; or 2> if the current drx-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including drx-Preference for the cell group and timer T346a associated with the cell group is not running: 3> start the timer T346a with the timer value set to the drx-PreferenceProhibitTimer of the cell group; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current drx-Preference; 1> if configured to provide its preference on the maximum aggregated bandwidth of a cell group for power saving: 2> if the UE has a preference on the maximum aggregated bandwidth of the cell group and the UE did not transmit a UEAssistanceInformation message with maxBW-Preference and/or maxBW-PreferenceFR2-2 for the cell group since it was configured to provide its preference on the maximum aggregated bandwidth of the cell group for power saving; or 2> if the current maxBW-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxBW-Preference and/or maxBW-PreferenceFR2-2for the cell group and timer T346b associated with the cell group is not running: 3> start the timer T346b with the timer value set to the maxBW-PreferenceProhibitTimer of the cell group; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxBW-Preference and/or maxBW-PreferenceFR2-2; 1> if configured to provide its preference on the maximum number of secondary component carriers of a cell group for power saving: 2> if the UE has a preference on the maximum number of secondary component carriers of the cell group and the UE did not transmit a UEAssistanceInformation message with maxCC-Preference for the cell group since it was configured to provide its preference on the maximum number of secondary component carriers of the cell group for power saving; or 2> if the current maxCC-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxCC-Preference for the cell group and timer T346c associated with the cell group is not running: 3> start the timer T346c with the timer value set to the maxCC-PreferenceProhibitTimer of the cell group; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxCC-Preference; 1> if configured to provide its preference on the maximum number of MIMO layers of a cell group for power saving: 2> if the UE has a preference on the maximum number of MIMO layers of the cell group and the UE did not transmit a UEAssistanceInformation message with maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2 for the cell group since it was configured to provide its preference on the maximum number of MIMO layers of the cell group for power saving; or 2> if the current maxMIMO-LayerPreference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2 for the cell group and timer T346d associated with the cell group is not running: 3> start the timer T346d with the timer value set to the maxMIMO-LayerPreferenceProhibitTimer of the cell group; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2; 1> if configured to provide its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving: 2> if the UE has a preference on the minimum scheduling offset for cross-slot scheduling of the cell group and the UE did not transmit a UEAssistanceInformation message with minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt for the cell group since it was configured to provide its preference on the minimum scheduling offset for cross-slot scheduling of the cell group for power saving; or 2> if the current minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt for the cell group and timer T346e associated with the cell group is not running: 3> start the timer T346e with the timer value set to the minSchedulingOffsetPreferenceProhibitTimer of the cell group; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt; 1> if configured to provide its release preference and timer T346f is not running: 2> if the UE determines that it would prefer to transition out of RRC_CONNECTED state; or 2> if the UE is configured with connectedReporting and the UE determines that it would prefer to revert an earlier indication to transition out of RRC_CONNECTED state: 3> start timer T346f with the timer value set to the releasePreferenceProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the release preference; 1> if configured to provide configured grant assistance information for NR sidelink communication: 2> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide configured grant assistance information for NR sidelink communication; 1> if configured to provide preference in being provisioned with reference time information: 2> if the UE did not transmit a UEAssistanceInformation message with referenceTimeInfoPreference since it was configured to provide preference; or 2> if the UE's preference changed from the last time UE initiated transmission of the UEAssistanceInformation message including referenceTimeInfoPreference: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide preference in being provisioned with reference time information. 1> if configured to provide its preference on FR2 UL gap: 2> if the UE did not transmit a UEAssistanceInformation message with ul-GapFR2-Preference since it was configured to provide its preference on FR2 UL gap information: 3> if the UE has a preference on FR2 UL gap activation/deactivation: 4> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide FR2 UL gap preference; 2> else if the current FR2 UL gap preference is different from the one indicated in the last transmission of the UEAssistanceInformation message: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide FR2 UL gap preference. 1> if configured to provide MUSIM assistance information for leaving RRC_CONNECTED: 2> if the UE needs to leave RRC_CONNECTED state for MUSIM operation: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information for leaving RRC_CONNECTED; 3> start the timer T346g with the timer value set to the musim-LeaveWithoutResponseTimer; 1> if configured to provide MUSIM assistance information without leaving RRC_CONNECTED: 2> if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList since it was configured to provide MUSIM assistance information without leaving RRC_CONNECTED; or 2> if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running or: 2> if the current musim-GapAssistanceConfig did not include the gap for which UE have preference and the timer T346h is not running 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList; 3> start the timer T346h with the timer value set to the musim-GapProhibitTimer. 1> if configured to provide the relaxation state of RLM measurements of a cell group: 2> if the UE did not transmit a UEAssistanceInformation message with rlm-MeasRelaxationState since it was configured to provide the relaxation state of RLM measurements for the cell group; or 2> if the relaxation state of RLM measurements for the cell group has changed since the last transmission of the UEAssistanceInformation message including rlm-MeasRelaxationState of the cell group and timer T346j associated with the cell group is not running: 3> start timer T346j with the timer value set to the rlm-RelaxtionReportingProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the relaxation state of RLM measurements of the cell group; 1> if configured to provide the relaxation state of BFD measurements of serving cells of a cell group: 2> if the UE did not transmit a UEAssistanceInformation message with bfd-MeasRelaxationState since it was configured to provide the relaxation state of BFD measurements for the cell group; or 2> if the relaxation state of BFD measurements in any serving cell of the cell group has changed since the last transmission of the UEAssistanceInformation message including bfd-MeasRelaxationState of the cell group and timer T346k associated with the cell group is not running: 3> start timer T346k with the timer value set to the bfd-RelaxtionReportingProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the relaxation state of BFD measurements of serving cells of the cell group. 1> if data and/or signalling mapped to radio bearers not configured for SDT becomes available during SDT (i.e. while T319a is running): 2> if the UE did not transmit a UEAssistanceInformation message with nonSDT-DataIndication since the initiation of the current resume procedure for SDT: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide nonSDT-DataIndication. 1> if configured to provide its preference for SCG deactivation and timer T346i is not running; 2> if the UE prefers the SCG to be deactivated and did not transmit a UEAssistanceInformation message with scg-DeactivationPreference since it was configured to provide its SCG deactivation preference; or 2> if the UE preference for SCG deactivation is different from the last indicated scg-DeactivationPreference since the UE was configured to provide its SCG deactivation preference: 3> start timer T346i with the timer value set to the scg-DeactivationPreferenceProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the UE preference for SCG deactivation; 1> if the SCG is deactivated, and, 1> the UE has uplink data to send for an SCG RLC entity while the UE previously did not have any uplink data to send for any SCG RLC entity: 2> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the UE has uplink data to send for a DRB whose DRB-Identity is not included in any RLC-BearerConfig in the CellGroupConfig associated with the MCG. 1> if configured to send indications of RRM measurement relaxation criterion fulfilment: 2> if the criterion in 5.7.4.4 is met for a period of TSearchDeltaP-StationaryConnected: 3> if the UE did not transmit a UEAssistanceInformation message with rrm-MeasRelaxationFulfilment as true since it was configured to provide indications of RRM measurement relaxation criterion fulfilment; or 3> the last UEAssistanceInformation message indicated the criterion in 5.7.4.4 is not fulfilled with rrm-MeasRelaxationFulfilment as false: 4> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is fulfilled; 2> else: 3> if the last UEAssistanceInformation message indicated fulfilment of the criterion in 5.7.4.4 with rrm-MeasRelaxationFulfilment as true: 4> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is not fulfilled.

Prohibit Timer Related Clarifications

The prohibit timer for MUSIM UAI is configured to avoid UE sending successive UAI for gap preference within very short interval. When the network disable the MUSIM assistance for gap preference UE is expected not to send any further UAI for gap preference. In this case the prohibit timer even if running need not be stopped.

Proposal 3: Prohibit timer operation when MUSIM Assistance configuration is disabled should be modified as given in the text proposal in Table 2.

Table 2: Proposed Specification changes for prohibit timer modification

#### 5.3.5.9 Other configuration The UE shall: 1> if the received otherConfig includes the delayBudgetReportingConfig: 2> if delayBudgetReportingConfig is set to setup: 3> consider itself to be configured to send delay budget reports in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to send delay budget reports and stop timer T342, if running. 1> if the received otherConfig includes the overheatingAssistanceConfig: 2> if overheatingAssistanceConfig is set to setup: 3> consider itself to be configured to provide overheating assistance information in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide overheating assistance information and stop timer T345, if running; 1> if the received otherConfig includes the idc-AssistanceConfig: 2> if idc-AssistanceConfig is set to setup: 3> consider itself to be configured to provide IDC assistance information in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide IDC assistance information; 1> if the received otherConfig includes the drx-PreferenceConfig: 2> if drx-PreferenceConfig is set to setup: 3> consider itself to be configured to provide its preference on DRX parameters for power saving for the cell group in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide its preference on DRX parameters for power saving for the cell group and stop timer T346a associated with the cell group, if running; 1> if the received otherConfig includes the maxBW-PreferenceConfig: 2> if maxBW-PreferenceConfig is set to setup: 3> consider itself to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group in accordance with 5.7.4; 3> if otherConfig includes maxBW-PreferenceConfigFR2-2: 4> consider itself to be configured to provide its preference on the maximum aggregated bandwidth for FR2-2 for power saving for the cell group in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group and stop timer T346b associated with the cell group, if running; 1> if the received otherConfig includes the maxCC-PreferenceConfig: 2> if maxCC-PreferenceConfig is set to setup: 3> consider itself to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group and stop timer T346c associated with the cell group, if running; 1> if the received otherConfig includes the maxMIMO-LayerPreferenceConfig: 2> if maxMIMO-LayerPreferenceConfig is set to setup: 3> consider itself to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group in accordance with 5.7.4; 3> if otherConfig includes maxMIMO-LayerPreferenceConfigFR2-2: 4> consider itself to be configured to provide its preference on the maximum number of MIMO layers for FR2-2 for power saving for the cell group in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group and stop timer T346d associated with the cell group, if running; 1> if the received otherConfig includes the minSchedulingOffsetPreferenceConfig: 2> if minSchedulingOffsetPreferenceConfig is set to setup: 3> consider itself to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4; 3> if otherConfig includes minSchedulingOffsetPreferenceConfigExt: 4> consider itself to be configured to provide its preference on the minimum scheduling offset for 480 kHz SCS and/or 960 kHz SCS for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group and stop timer T346e associated with the cell group, if running; 1> if the received otherConfig includes the releasePreferenceConfig: 2> if releasePreferenceConfig is set to setup: 3> consider itself to be configured to provide assistance information to transition out of RRC_CONNECTED in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide assistance information to transition out of RRC_CONNECTED and stop timer T346f, if running. 1> if the received otherConfig includes the obtainCommonLocation: 2> include available detailed location information for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation; NOTE 1: The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation. 1> if the received otherConfig includes the btNameList: 2> if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation; 1> if the received otherConfig includes the wlanNameList: 2> if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation; 1> if the received otherConfig includes the sensorNameList: 2> if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation; NOTE 2: The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation. 1> if the received otherConfig includes the sl-AssistanceConfigNR: 2> consider itself to be configured to provide configured grant assistance information for NR sidelink communication in accordance with 5.7.4; 1> if the received otherConfig includes the referenceTimePreferenceReporting: 2> consider itself to be configured to provide UE reference time assistance information in accordance with 5.7.4; 1> else: 2> consider itself not to be configured to provide UE reference time assistance information; 1> if the received otherConfig includes the successHO-Config: 2> consider itself to be configured to provide the successful handover information in accordance with 5.7.10.6; 1> else: 2> consider itself not to be configured to provide the successful handover information. 1> if the received otherConfig includes the ul-GapFR2-PreferenceConfig: 2> consider itself to be configured to provide its preference on FR2 UL gap in accordance with 5.7.4; 1> else: 2> consider itself not to be configured to provide its preference on FR2 UL gap; 1> if the received otherConfig includes the musim-GapAssistanceConfig: 2> if musim-GapAssistanceConfig is set to setup: 3> consider itself to be configured to provide MUSIM assistance information without leaving RRC_CONNECTED in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide MUSIM assistance information without leaving RRC_CONNECTED in accordance with 5.7.4; 1> if the received otherConfig includes the musim-LeaveAssistanceConfig: 2> if musim-LeaveAssistanceConfig is set to setup: 3> consider itself to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4 and stop timer T346g, if running. 1> if the received otherConfig includes the rlm-RelaxationReportingConfig: 2> if rlm-RelaxationReportingConfig is set to setup: 3> consider itself to be configured to report the relaxation state of RLM measurements with 5.7.4; 2> else: 3> consider itself not to be configured to report the relaxation state of RLM measurements; 1> if the received otherConfig includes the bfd-RelaxationReportingConfig: 2> if bfd-RelaxationReportingConfig is set to setup: 3> consider itself to be configured to report the relaxation state of BFD measurements with 5.7.4; 2> else: 3> consider itself not to be configured to report the relaxation state of BFD measurements; 1> if the received otherConfig includes the scg-DeactivationPreferenceConfig: 2> if the scg-DeactivationPreferenceConfig is set to setup: 3> consider itself to be configured to provide its SCG deactivation preference in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide its SCG deactivation preference and stop timer T346i, if running.

Preservation of MUSIM Gap Assistance Configuration in RRC-INACTIVE state

As per current specifications the UE releases the MUSIM Gap assistance configuration when the UE resumes for RRC connection from RRC-INACTIVE state. As the feature is disabled during the resume procedure UE cannot send UAI to request for MUSIM gaps until reception of RRC Reconfiguration enabling the MUSIM assistance configuration. This will lead to the idle MUSIM operation which was ongoing in other USIM when the UE was in RRC-INACTIVE state will have to be stopped until the UE starts new UAI after RRC Reconfiguration and reception of another RRC Reconfiguration. The delay in triggering UAI during resume can be avoided if the UE is allowed to preserve the MUSIM-assistance-configuration while resuming the RRC connection.

Proposal 4: RAN2 to consider preservation of MUSIM Assistance configuration for RRC-Resume to avoid delay in activation of MUSIM gaps. The text proposal is given in Table 3.

#### 5.3.13.2 Initiation The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/V2X sidelink communication as specified in clause 5.3.13.1a) requests the resume of a suspended RRC connection or for initiating SDT as specified in clause 5.3.13.1b. The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure. Upon initiation of the procedure, the UE shall: 1> if the resumption of the RRC connection is triggered by response to NG-RAN paging: 2> select '0' as the Access Category; 2> perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers; 3> if the access attempt is barred, the procedure ends; 1> else if the resumption of the RRC connection is triggered by upper layers: 2> if the upper layers provide an Access Category and one or more Access Identities: 3> perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers; 4> if the access attempt is barred, the procedure ends; 2> if the resumption occurs after release with redirect with mpsPriorityIndication: 3> set the resumeCause to mps-PriorityAccess; 2> else: 3> set the resumeCause in accordance with the information received from upper layers; 1> else if the resumption of the RRC connection is triggered due to an RNA update as specified in 5.3.13.8: 2> if an emergency service is ongoing: NOTE 1: How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation. 3> select '2' as the Access Category; 3> set the resumeCause to emergency; 2> else: 3> select '8' as the Access Category; 2> perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [23]; 3> if the access attempt is barred: 4> set the variable pendingRNA-Update to true; 4> the procedure ends; NOTE 2: In case the L2 U2N Relay UE initiates RRC connection resume according to conditions as specified in 5.3.13.1a, the L2 U2N Relay UE sets the resumeCause by implementation. If the cause value in the message received from the L2 U2N Remote UE via SL-RLC0 is emergency, mps-PriorityAccess, or mcs-PriorityAccess, the L2 U2N Relay UE can set the same value. Otherwise, the L2 U2N Relay UE does not set the value as emergency, mps-PriorityAccess, or mcs-PriorityAccess. 1> if the UE is in NE-DC or NR-DC: 2> if the UE does not support maintaining SCG configuration upon connection resumption: 3> release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored; 1> if the UE does not support maintaining the MCG SCell configurations upon connection resumption: 2> release the MCG SCell(s) from the UE Inactive AS context, if stored; 1> if the UE performs connection resumption in a different cell than the cell where srs-PosRRC-InactiveConfig was configured; 2> release srs-PosRRC-InactiveConfig, if configured; Editor's Note: This agreement/clause may not be needed because we have this agreement TA timer configuration is invalidated upon any cell reselection; see 5.7.15. 1> if the UE is connected with a L2 U2N Relay UE via PC5-RRC connection (i.e. the UE is a L2 U2N Remote UE): 2> apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1; 2> apply the default PDCP configuration defined in 9.2.1 for SRB1; 1> else: 2> apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1; 2> apply the default SRB1 configuration as specified in 9.2.1; 2> apply the default MAC Cell Group configuration as specified in 9.2.2; 1> release delayBudgetReportingConfig from the UE Inactive AS context, if stored; 1> stop timer T342, if running; 1> release overheatingAssistanceConfig from the UE Inactive AS context, if stored; 1> stop timer T345, if running; 1> release idc-AssistanceConfig from the UE Inactive AS context, if stored; 1> release drx-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored; 1> stop all instances of timer T346a, if running; 1> release maxBW-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored; 1> stop all instances of timer T346b, if running; 1> release maxCC-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored; 1> stop all instances of timer T346c, if running; 1> release maxMIMO-LayerPreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored; 1> stop all instances of timer T346d, if running; 1> release minSchedulingOffsetPreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored; 1> stop all instances of timer T346e, if running; 1> release rlm-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored; 1> stop all instances of timer T346j, if running; 1> release bfd-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored; 1> stop all instances of timer T346k, if running; 1> release releasePreferenceConfig from the UE Inactive AS context, if stored; 1> release wlanNameList from the UE Inactive AS context, if stored; 1> release btNameList from the UE Inactive AS context, if stored; 1> release sensorNameList from the UE Inactive AS context, if stored; 1> release obtainCommonLocation from the UE Inactive AS context, if stored; 1> stop timer T346f, if running; 1> stop timer T346i, if running; 1> release referenceTimePreferenceReporting from the UE Inactive AS context, if stored; 1> release sl-AssistanceConfigNR from the UE Inactive AS context, if stored; 1> if the UE is connected with a L2 U2N Relay UE via PC5-RRC connection (i.e. the UE is a L2 U2N Remote UE): 2> apply the specified configuration of SL-RLC0 used for the delivery of RRC message over SRB0 as specified in 9.1.1.4; 2> apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0; 1> else: 2> apply the CCCH configuration as specified in 9.1.1.2; 2> apply the timeAlignmentTimerCommon included in SIB1; 1> if sdt-MAC-PHY-CG-Config is configured: 2> if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored sdt-MAC-PHY-CG-Config: 3> release the stored sdt-MAC-PHY-CG-Config; 1> if conditions for initiating SDT in accordance with 5.3.13.1b are fulfilled: 2> consider the resume procedure is initiated for SDT; 2> start timer T319a; 1> else: 2> start timer T319; 2> instruct the MAC entity to consider the cg-SDT-TimeAlignmentTimer as expired, if it is running; 1> set the variable pendingRNA-Update to false; 1> release successHO-Config from the UE Inactive AS context, if stored; 1> initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

Gap Priority in UAI

According to current specification for MUSIM functionality, UAI for gap preference and gap configuration is supported for 2 periodic gaps and one aperiodic gap. Out of the two periodic gaps the UE may include its preference in terms of priorities, for example when one gap is needed for paging monitoring in other USIM and other gap is for idle mode measurements in other network. The gap for paging monitoring has high priority, and if not configured from NTWK-A, the UE may need to trigger NAS procedure for paging collision avoidance to change the gap location. As the paging collision avoidance will require RRC connection in other network, it may lead to release the RRC connection in NTWK-A. Otherwise, the UE may still continue paging monitoring in other network without gap configuration, i.e. without awareness at NTWK-A. This will lead to packet loss at NTWK-A as there is no scheduling gap created for this purpose.

Observation 1: If the Gap preference for paging monitoring is not accepted at NTWK-A, the impact to UE performance is higher in terms of throughput impact or release of RRC connection.

The situation is different for measurement gaps. Because the UE may still defer or identify natural gap such as connected mode DRX or gaps between search space to complete the idle mode measurements. Moreover, delay in idle mode measurements in NTWK-A if the gaps are not possible is not as severe as compared to the paging monitoring operation.

Observation 2: If Gap preference for idle mode measurements is not accepted at NTWK-A, the impact to NTWK-B operation is not severe. The UE can still manage to make these measurements without impacting NTKW-A scheduling.

Based on the above observations, it is evident that one of the periodic gap is more critical and essential for NTWK-B operation compared to other periodic gap.

As per current specification, the UAI does not provide any indication on which gap configuration is preferred in case if network cannot configure both the periodic gap configurations. So we propose that gap priority is introduced in UAI so that network can decide the gap preference to be considered for activation if it cannot configure all the periodic gaps.

Proposal 5: Gap priority is included in UAI for MUSIM Gap preference. The text proposal for this change is given in Table 4.

Table 4: Proposal for specification changes for Gap Priority in MUSIM UAI

MUSIM-GapInfo-r17 ::= SEQUENCE { musim-Starting-SFN-AndSubframe-r17 MUSIM-Starting-SFN-AndSubframe-r17 OPTIONAL, musim-GapLength-r17 ENUMERATED {ms4, ms5dot5, ms6, ms10, ms20}, musim-GapRepetitionAndOffset-r17 CHOICE { ms20-r17 INTEGER (0..19), ms40-r17 INTEGER (0..39), ms80-r17 INTEGER (0..79), ms160-r17 INTEGER (0..159), ms320-r17 INTEGER (0..319), ms640-r17 INTEGER (0..639), ms1280-r17 INTEGER (0..1279), ms2560-r17 INTEGER (0..2559), ... } OPTIONAL -- Cond periodic, Musim-GapPriority ENUM(high) -- OPTIONAL } MUSIM-Starting-SFN-AndSubframe-r17 ::= SEQUENCE { starting-SFN-r17 INTEGER (0..1023), startingSubframe-r17 INTEGER (0..9) } Musim-GapPriority: This field indicates that requested gap is essential for basic MUSIM operation.

# 3	Conclusion

In this discussion paper we analysed the UAI procedure for MUSIM operation in Rel-17 specification and make following proposals and observations.

Proposal 1: Specification for MUSIM UAI is modified for UE behaviour to retry sending the UAI for the gap preference not configured in response to earlier UAI transmission. The text proposal is given in Table 1.

Proposal 2: UAI triggering for leave indication is clarified that it is applicable only for leaving RRC connected state for MUSIM operation. The text proposal is given in Table 1.

Proposal 3: Prohibit timer operation when MUSIM Assistance configuration is disabled should be modified as given in the text proposal in Table 2.

Proposal 4: RAN2 to consider preservation of MUSIM Assistance configuration for RRC-Resume to avoid delay in activation of MUSIM gaps. The text proposal is given in Table 3.

Observation 1: If the Gap preference for paging monitoring is not accepted at NTWK-A, the impact to UE performance is higher in terms of throughput impact or release of RRC connection.

Observation 2: If Gap preference for idle mode measurements is not accepted at NTWK-A, the impact to NTWK-B operation is not severe. The UE can still manage to make these measurements without impacting NTKW-A scheduling.

Proposal 5: Gap priority is included in UAI for MUSIM Gap preference. The text proposal for this change is given in Table 4.

# References

[1] RP-193263 – WID For Support of Multi-SIM devices -Rel-17

[2] TS38.331 V17.0.0 -RRC Specification Rel-17