3GPP TSG-RAN2 Meeting #118	R2-2204632

# E-meeting, May 9th - May 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0001 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SRAP for L2 UE-to-Network Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In 3.1, the definition of in/egress link/RLC-channel did not touch the network node side. Yet the figure of 4.2.2-2/3 includes network node when indicating in/egress interface (PC5 and Uu). - In figure 4.2.2-2, for the transmitting side of relay UE, the operation of SRAP header removal is missing, which is limited to SRB0, and the operation of UE/BEARER ID determination and header addition is limited to bearers other than SRB0, but not applicable to SRB0. - In figure 4.2.2-2, upon receiving SL-RLC0 packets from relay UE, it is not needed for SRAP Rx part of remote UE to remove SRAP header. - In figure 4.2.2-3, different from figure 4.2.2-2, “SL-RLC0” is used instead of “SRB0”, which is unnecessary since the related judgement based on SL-RLC0 can be described in detail in the procedural text. - Both “SRAP PDU/SDU” and “SRAP Data PDU/SDU” are used in the whole spec, and data packet are also used somewhere - In 4.2.2, the description of “received from SL-RLC0 as specified in TS 38.331 [3]” is not applicable to the forwarding from Uu interface to PC5 interface. - In 4.2.2, the exceptional description for SRB0 is only for forwarding from PC5 interface to Uu interface, yet missing the forwarding from Uu interface to PC5 interface. The former one requires SRAP header addition while the latter requires SRAP header removal. - The “UE ID” and “BEARER ID” are described in the spec, sometimes with and sometimes without “field”. - In 5.2.2.2, BEARER ID = 0 is not sufficient to judge the case for SRB0, since 0 can be used for DRB as well. - In 5.2.2, before submitting the Data PDU to the egress RLC channel, there is a missing operation of SRAP header removal for the packet of SRB0. - In 5.2.3, the SRAP header removal, and submission to PDCP entity is only applicable to bearers other than SRB0 (for SRB0, there is no header to remove, and there is no PDCP entity). - In 5.3.1, the Data PDU construction is limited to the case other than SRB0, yet the Data PDU submission operation is applicable to all cases including SRB0, then the Data PDU construction / definition is missing for SRB0. - In 5.3.1.2, although it starts with “if the SRAP SDU is for SRB0”, this clause is called after PDU construction. - In 6.3.4, the SDU is limited to PDCP PDU, missing the case for SRB0 where the SDU is RRC PDU. - Other editorial change |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Include network node in the definition of in/egress link/RLC-channel. - Add the SRAP header removal operation for SRB0 only in figure 4.2.2-2, and clarify SRB0 does not need header addition when sent by remote UE. - In figure 4.2.2-2, add a new line for SRB0 for delivery the received SL-RLC0 packets to upper layers on remote UE without header removal. - Change “SL-RLC0” to “SRB0” in figure 4.2.2-3. - Align the term in the spec to “SRAP Data PDU” and “SRAP SDU” only. And limit the usage of “data packet” to the places where the data is transferred between the collocated Rx and Tx entities. - In 4.2.2, change the description of “received from SL-RLC0 as specified in TS 38.331 [3]” to “for SRB0”. And rely on detailed procedural text below to clarify how to judge which packet is for SRB0. - In 4.2.2, add the exceptional description for SRB0 for the forwarding from Uu interface to PC5 interface. - Align the description so that “UE ID” and “BEARER ID” are described in the spec always with “field” - Revise the condition so that not only BEARER ID = 0, but also the RB type is judged as SRB. - In 5.2.2, add the SRAP header removal operation for SRB0. - In 5.2.3, differentiate between SRB0 and other case, where for SRB0, there is no SRAP header removal and the upper layer is RRC layer. - In 6.2.2, define another SRAP Data PDU format which is without SRAP header and applicable only to SRB0. And in 5.3.1, refer to this SRAP Data PDU format (w/o header). And align the text in the spec to differentiate between Data PDU w/ and w/o header. - In 5.3.1.2, change SDU to PDU. - In 6.3.4, add RRC PDU into the SRAP SDU definition. - Correction for the editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Wrong definition of in/egress link/RLC-channel. - The operation of SRAP header removal operation is missing for SRB0, and the operation is SRAP header addition is applicable to SRB0 in a wrong way. - Misunderstanding that header removal is needed when remote UE receives SRB0 message. - The misalignment of 4.2.2-2/3 on “SL-RLC0”/”SRB0” may lead to unnecessary discussion in the future. - Misunderstanding on the possible difference between “SRAP PDU/SDU” and “SRAP Data PDU/SDU”, and also misunderstanding on the usage of “data packet”. - Misunderstanding that the “received from SL-RLC0 as specified in TS 38.331 [3]” is also applicable to the forwarding from Uu interface to PC5 interface. - The exceptional description for SRB0 for the forwarding from Uu interface to PC5 interface is missing. - Misalignment in the spec that the “UE ID” and “BEARER ID” are described in the spec, sometimes with and sometimes without “field”. - In 5.2.2.2, BEARER ID = 0 is not sufficient to judge the case for SRB0, since 0 can be used for DRB as well. - The SRAP Data PDU delivered from relay to remote UE for SRB0 is with SRAP header. - The header removal and submission to PDCP is applicable to SRB0 by mistake. - It is contradictory on how to handle SRB0 from SRAP entity perspective, i.e., how to define the SRAP SDU/PDU for SRB0. - Misunderstanding that this operation is done before PDU construction. - The SDU definition of SRB0 is missing. - The editorial errors remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Egress RLC channel: a RLC channel on which a packet is transmitted by a U2N Relay UE, a U2N Remote UE or a network node.

Egress link: a radio link on which a packet is transmitted by a U2N Relay UE, a U2N Remote UE, or a network node.

Ingress RLC channel: a RLC channel on which a packet is received from a U2N Relay UE, a U2N Remote UE or a network node.

Ingress link: a radio link on which a packet is received from a U2N Relay UE, a U2N Remote UE or a network node.

U2N Relay UE: a UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: a UE that communicates with the network via a U2N Relay UE.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SRAP	Sidelink Relay Adaptation Protocol

U2N 	UE-to-Network

# 4	General

## 4.1	Introduction

The objective is to describe the SRAP architecture and the SRAP entities from a functional point of view.

## 4.2	SRAP architecture

### 4.2.1	General

This clause describes a model of the SRAP, i.e., it does not specify or restrict implementations.

### 4.2.2	SRAP entities

Figure 4.2.2-1 represents one possible structure for the SRAP sublayer. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

Figure 4.2.2-1: SRAP structure overview

On the U2N Relay UE, the SRAP sublayer contains one SRAP entity at Uu interface and a separate collocated SRAP entity at the PC5 interface. On the U2N Remote UE, the SRAP sublayer contains only one SRAP entity at the PC5 interface.

Each SRAP entity has a transmitting part and a receiving part. Across the PC5 interface, the transmitting part of the SRAP entity at the U2N Remote UE has a corresponding receiving part of an SRAP entity at the U2N Relay UE, and vice versa. Across the Uu interface, the transmitting part of the SRAP entity at the U2N Relay UE has a corresponding receiving part of an SRAP entity at the gNB, and vice versa.

Figure 4.2.2-2 and Figure 4.2.2-3 represents the functional view of the SRAP entity for the SRAP sublayer at PC5 interface and at Uu interface respectively.

Figure 4.2.2-2: Example of functional view of SRAP sublayer at PC5 interface

Figure 4.2.2-3: Example of functional view of SRAP sublayer at Uu interface

In the example of Figure 4.2.2-2 and Figure 4.2.2-3, at relay UE,

-	The receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the receiving part on the SRAP entity of PC5 interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of Uu interface, except for data packet for SRB0. As an alternative mode, the receiving part may deliver SRAP SDUs to the transmitting part on the collocated SRAP entity. When passing SRAP SDUs, the receiving part removes the SRAP header and the transmitting part adds the SRAP header with the same SRAP header content as carried on the SRAP Data PDU header prior to removal. Passing SRAP SDUs in this manner is therefore functionally equivalent to passing SRAP Data PDUs, in implementation. The following specification therefore refers to the passing of SRAP data packets in supporting the alternative mode.

-	For data packet corresponding to SRB0, the receiving part on the SRAP entity of PC5 interface delivers SRAP SDUs to the transmitting part on the collocated SRAP entity of Uu interface, and the transmitting part adds the SRAP header in accordance with clause 5.3.3.

-	For data packet for SRB0, the receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the transmitting part removes the SRAP header in accordance with clause 5.2.2.

## 4.3	Services

### 4.3.1	Services provided to upper layers

The following services are provided by the SRAP sublayer to upper layers:

-	Data transfer.

### 4.3.2	Services expected from lower layers

An SRAP sublayer expects the following services from lower layers per RLC entity (for a detailed description see TS 38.322 [4]):

-	Acknowledged data transfer service;

-	Unacknowledged data transfer service.

## 4.4	Functions

The SRAP sublayer supports the following functions:

-	Data transfer;

-	Determination of UE ID field and BEARER ID field for data packets received from collocated SRAP entity;

-	Determination of egress link;

-	Determination of egress RLC channel;

## 4.5	Configurations

The configuration of the SRAP entity for U2N Remote UE includes:

-	Mapping from a radio bearer identified by BEARER ID field to egress PC5 RLC channel via RRC.

-	The local identity via RRC.

The configuration of the SRAP entity for U2N Relay UE includes:

-	The local identity for each U2N Remote UE via RRC.

-	Mapping from UE ID field and BEARER ID field to egress Uu RLC channel for each U2N Remote UE via RRC.

-	Mapping from UE ID field and BEARER ID field to egress PC5 RLC channel for each U2N Remote UE via RRC.

# 5	Procedures

## 5.1	SRAP entity handling

### 5.1.1	SRAP entity establishment

When upper layers request establishment of an SRAP entity, UE shall:

-	establish an SRAP entity;

-	follow the procedures in clause 5.

### 5.1.2	SRAP entity release

When upper layers request release of an SRAP entity, UE shall:

-	release the SRAP entity and the related SRAP configurations.

## 5.2	DL Data transfer

### 5.2.1	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the Uu interface of U2N Relay UE shall:

-	deliver the SRAP data packet to the transmitting part of the collocated SRAP entity on the PC5 interface.

### 5.2.2	Transmitting operation of U2N Relay UE

The transmitting part of the SRAP entity on the PC5 interface of U2N Relay UE receives SRAP data packets from the receiving part of the SRAP entity on the Uu interface of the same U2N Relay UE.

When the transmitting part of the SRAP entity on the PC5 interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the egress link in accordance with clause 5.2.2.1;

-	Determine the egress RLC channel in accordance with clause 5.2.2.2;

-	if the SRAP Data PDU is for SRB0 (the BEARER ID field is 0 and SRB and DRB are differentiated based on sl-Egress-RLC-Channel-Uu):

-	Removes the SRAP header from the SRAP Data PDU;

-	Submit this SRAP Data PDU to the determined egress RLC channel of the determined egress link.

#### 5.2.2.1	Egress link determination

For a SRAP Data PDU to be transmitted, SRAP entity shall:

-	if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU:

-	Determine the egress link on PC5 interface corresponding to sl-L2Identity-Remote configured for the concerned sl-LocalIdentity as specified in TS 38.331 [3];

#### 5.2.2.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if the SRAP Data PDU is for SRB0 (the BEARER ID field is 0 and SRB and DRB are differentiated based on sl-Egress-RLC-Channel-Uu):

-	Determine the egress PC5 RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3];

-	else if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, which includes an sl-RemoteUE-RB-Identity that matches the SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (SRB and DRB are differentiated based on sl-Egress-RLC-Channel-Uu),

-	Determine the egress PC5 RLC channel in the determined egress link corresponding to sl-Egress-RLC-Channel-PC5 configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3];

### 5.2.3	Receiving operation of U2N Remote UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity shall:

-	if the SRAP Data PDU is not for SRB0 (not received from SL-RLC0 as specified in TS 38.331 [3]):

-	remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer, i.e., PDCP layer entity (TS 38.323 [5]), corresponding to the BEARER ID field of this SRAP Data PDU (SRB and DRB are differentiated based on sl-Egress-RLC-Channel-PC5);

-	else:

-	deliver the SRAP SDU to upper layer, i.e., RRC layer entity (TS 38.331 [3]);

## 5.3	UL Data transfer

### 5.3.1	Transmitting operation of U2N Remote UE

The transmitting part of the SRAP entity on the PC5 interface of U2N Remote UE can receive SRAP SDU from upper layer, and constructs SRAP Data PDU as needed (see clause 4.2.2).

Upon receiving an SRAP SDU from upper layer, the transmitting part of the SRAP entity on the PC5 interface shall:

-	if the SRAP SDU is not for SRB0:

-	Determine the UE ID field and BEARER ID field in accordance with clause 5.3.1.1;

-	Construct an SRAP Data PDU with SRAP header, where the UE ID field and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	else:

-	Construct an SRAP Data PDU without SRAP header in accordance with clause 6.2.2.

-	Determine the egress RLC channel in accordance with clause 5.3.1.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

#### 5.3.1.1	UE ID field and BEARER ID field determination

For an SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID field corresponding to sl-LocalIdentity, configured as specified in TS 38.331 [3];

-	Determine the BEARER ID field corresponding to SRB identity for SRB (i.e., set the BEARER ID field to srb-Identity), or corresponding to DRB identity minus 1 for DRB (i.e., “set the BEARER ID field to drb-Identity minus 1), from which the SRAP SDU is received, configured as specified in TS 38.331 [3];

#### 5.3.1.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if the SRAP Data PDU is for SRB0:

-	Determine the egress PC5 RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3];

-	else if there is an entry in sl-SRAP-Config-Remote, whose sl-RemoteUE-RB-Identity matches the SRB identity or DRB identity of the SRAP Data PDU,

-	Determine the egress PC5 RLC channel of the link with U2N Relay UE corresponding to sl-Egress-RLC-Channel-PC5 configured for the concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3];

### 5.3.2	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the PC5 interface shall:

-	deliver the SRAP data packet to the transmitting part of the collocated SRAP entity on the Uu interface.

### 5.3.3	Transmitting operation of U2N Relay UE

The transmitting part of the SRAP entity on the Uu interface of U2N Relay UE can receive SRAP data packets from the receiving part of the SRAP entity on the PC5 interface of the same U2N Relay UE, and construct SRAP Data PDUs as needed (see clause 4.2.2).

Upon receiving SRAP data packet from the receiving part on the collocated SRAP entity on the PC5 interface, the transmitting part of the SRAP entity on the Uu interface shall:

-	if the SRAP Data PDU is received from SL-RLC0 as specified in TS 38.331 [3]:

-	Determine the UE ID field and BEARER ID field in accordance with clause 5.3.3.1;

-	Construct an SRAP Data PDU with SRAP header, where the UE ID field and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	Determine the egress RLC channel in accordance with clause 5.3.3.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

#### 5.3.3.1	UE ID field and BEARER ID field determination

For an SRAP Data PDU received from SL-RLC0 as specified in TS 38.331 [3], the SRAP entity shall:

-	if there is an entry in sl-RemoteUE-ToAddModList, whose sl-L2Identity-Remote matches the Layer-2 ID of the remote UE from which the SRAP Data PDU is received,

-	Determine the UE ID field corresponding to sl-LocalIdentity configured for the concerned sl-L2Identity-Remote as specified in TS 38.331 [3];

-	Determine the BEARER ID field as 0 (i.e., set BEARER ID field as 0), configured as specified in TS 38.331 [3];

#### 5.3.3.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, and which includes an sl-RemoteUE-RB-Identity matches SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (SRB and DRB are differentiated based on sl-Egress-RLC-Channel-PC5),

-	Determine the egress Uu RLC channel corresponding to sl-Egress-RLC-Channel-Uu configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3];

## 5.4	Handling of unknown, unforeseen, and erroneous protocol data

When a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which is not included in sl-SRAP-Config-Remote (for Remote UE) or sl-SRAP-Config-Relay (for Relay UE) is received, the SRAP entity shall:

-	discard the received SRAP Data PDU.

# 6	Protocol data units, formats, and parameters

## 6.1	Protocol data units

### 6.1.1	Data PDU

The SRAP Data PDU is used to convey the following with or without the PDU header:

-	upper layer data.

## 6.2	Formats

### 6.2.1	General

An SRAP Data PDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. The formats of SRAP Data PDUs are described in clause 6.2.2 and their parameters are described in clause 6.3.

### 6.2.2	Data PDU

Figure 6.2.2-1 shows the format of the SRAP Data PDU with SRAP header being configured. This SRAP Data PDU format is applicable to SRAP SDU except those for SRB0 delivered over PC5 interface.

Figure 6.2.2-1: SRAP Data PDU format with SRAP header

Figure 6.2.2-2 shows the format of the SRAP Data PDU consisting only of a data field without any SRAP header. This SRAP Data PDU format is applicable to SRAP SDU for SRB0 delivered over PC5 interface.

Figure 6.2.2-2: SRAP Data PDU format without SRAP header

## 6.3	Parameters

### 6.3.1	General

If not otherwise mentioned in the definition of each field the bits in the parameters shall be interpreted as follows: the left most bit is the first and most significant and the right most bit is the last and least significant bit.

Unless otherwise mentioned, integers are encoded in standard binary encoding for unsigned integers. In all cases the bits appear ordered from MSB to LSB when read in the PDU.

### 6.3.2	UE ID

Length: 8 bits.

This field carries local identity of U2N Remote UE.

### 6.3.3	BEARER ID

Length: 5 bits.

This field carries Uu radio bearer identity for U2N Remote UE.

### 6.3.4	Data

Length: Variable

This field carries the SRAP SDU (i.e. PDCP PDU or RRC PDU).

### 6.3.5	R

Length: 1 bit

Reserved. In this release, reserved bits shall be set to 0. Reserved bits shall be ignored by the receiver.

### 6.3.6	D/C

Length: 1 bit

This field indicates whether the corresponding SRAP PDU is an SRAP Data PDU or an SRAP Control PDU (not used in this release).

Table 6.3.6-1: D/C field

| Bit | Description |
| --- | --- |
| 0 | SRAP Data PDU |
| 1 | SRAP Control PDU (not used in this release) |

End of Change