3GPP TSG-RAN2 Meeting #118-e	R2-2204742

# Online, May 9th –20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1228 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the TS38.321 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The clause 5.24 specifies the UE behaviour when PPW is actived, but not how to active the PPW. - According to the LS (R2-2203739) from RAN1, the maximum number of activated PRS processing windows across all active DL BWPs is 4. However the current TS38.321 spcified that the maximum entries N of PPW MAC CE is 4, i.e., when the numEntry is 11, it means that there is at most 3 OCT containing the PPW ID, i.e., the current PPW activation/deactivation MAC CE at most include 3 actived PPW ID. - Further, the PPW acitivation/deactivation MAC CE is with variable size but not fixed size. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the UE behaviour when PPW is activeted should follow the clause 5.24; - Correct the maximum number of N within PPW MAC CE, so as to support active 4 PPW at the same time; - Delete the description of PPW MAC CE that “and consists of a single octet”. Impact analysis Impacted 5G architecture options: SA/NSA Impacted functionality: positioning latency enhancement Inter-operability: If the UE implement the CR and the NG-RAN does not, NW cannot activate/deactivate 4 PPW ID simultaneously. If the NG-RAN implement the CR and the UE does not, there is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current PPW activation/deactivation MAC CE cannot support to activate/deactivate 4 PPW ID simultaneously. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.21, 6.1.3.42 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 5.18.21	PPW Activation/Deactivation Command

If the UE is configured with pre-configured PPW, the network may send DL MAC CE for PPW Activation/Deactivation Command to the UE as in clause 6.1.3.42. For the activated PPW, the UE shall follow the specified UE behaviour in clause 5.24.

Upon the reception of the MAC CE for PPW Activation/Deactivation Command, the MAC entity shall:

1>	if the DL MAC CE for PPW Activation/Deactivation Command indicates the deactivation of a pre-configured PPW:

2>	deactivate the PPW.

1>	else if the DL MAC CE for PPW Activation/Deactivation Command indicates the activation of a pre-configured PPW:

2>	activate the PPW and perform  the procedure specified in clause 5.24.

NEXT CHANGE

#### 6.1.3.42	PPW Activation/Deactivation Command MAC CE

The PPW Activation/Deactivation Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b.

It has variable size with fields defined as follows (Figure 6.1.3.42-1):

-	numEntry: This field indicates the number of entries N in the MAC CE. 00 indicates that N equals to 2; 01 indicates that N equals to 3 and so on. The length of the field is 2 bits;

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	PPW ID: This field indicates the PPW configured on active DL BWP of the Serving Cell identified by the above Serving Cell ID. The length of the field is 2 bits;

-	A/D: This field indicates the activation or deactivation of the PPW. The field is set to 1 to indicate activation, otherwise it indicates deactivation. The length of the field is 1 bit.

-	R: Reserved bit, set to 0.

Figure 6.1.3.42-1: PPW Activation/Deactivation Command MAC CE

END OF CHANGE