3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2204873

# Elbonia, 09 – 20 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1239 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RA initialization for slicing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Slice-Core |  |  |  |  |  | Date: |  |  | 2022-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The RRC configuration parameter enableRA-PrioritizationForSlicing is defined to give priority to slice specific RACH configuration over MPS and MCS and conditioned only if both parameters ra-PrioritizationForAccessIdentity and ra-PrioritizationForSlicing are configured. The procedures in 5.1.1a ‘Initialization of variables specific to Random Access type’ require the MAC layer provision with Slice Group Identity and Access Identity (MPS and MCS) before determining the flag gives priority to slicing, which is unnecessary. Yet, one of the corresponding Access Identity provision is required to apply Slice specific parameters, which isnt’ correct. Furthermore, the provision of Slice Group Identity to MAC layer was not agreed and the step can be replaced by generic association of RA procedure to the Slice Group to ease feature implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explain the corresponding changes: - Move the check enableRA-PrioritizationForSlicing flag before checking Slice Group Identity and Access Identities provision to MAC - Replace Slice Group Id provision with generic association of the Slice Groups with RA procedure for which the procedure was initiated - Paremeters and variables names changed to italic - Impact analysis Impacted functionality: RAN Slicing, RA-prioritization Inter-operability: - If the network is implemented according to the CR and the UE is not the RA-prioritization configuration provided by the network may be ignored. - If the UE is implemented according to the CR and the network is not there are no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecesary requirements to check RA configuration parameters (that are not going to be used) and provide the Slice Group Id from NAS through RRC to MAC remain in the specification and lead to wrong assessment settings of the applicable RA configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Modified Subclause

### 5.1.1a	Initialization of variables specific to Random Access type

The MAC entity shall:

1>	if RA_TYPE is set to 2-stepRA:

2>	set PREAMBLE_POWER_RAMPING_STEP to msgA-PreamblePowerRampingStep;

2>	set SCALING_FACTOR_BI to 1;

2>	apply preambleTransMax included in the RACH-ConfigGenericTwoStepRA;

2>	if the Random Access procedure was initiated for reconfiguration with sync or for SCG activation; and

2>	if cfra-TwoStep is configured for the selected carrier:

3>	if msgA-TransMax is configured in the cfra-TwoStep:

4>	apply msgA-TransMax configured in the cfra-TwoStep.

2>	else if msgA-TransMax is included in the RACH-ConfigCommonTwoStepRA:

3>	apply msgA-TransMax included in the RACH-ConfigCommonTwoStepRA.

2>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17); and

2>	if beamFailureRecoveryConfig is configured for the active UL BWP of the selected carrier; and

2>	if ra-PrioritizationTwoStep is configured in the beamFailureRecoveryConfig:

3>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority included in the ra-PrioritizationTwoStep in beamFailureRecoveryConfig;

3>	if scalingFactorBI is configured in the ra-PrioritizationTwoStep in beamFailureRecoveryConfig:

4>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	else if the Random Access procedure was initiated for reconfiguration with sync or for SCG activation; and

2>	if rach-ConfigDedicated is configured for the selected carrier; and

2>	if ra-PrioritizationTwoStep is configured in the rach-ConfigDedicated:

3>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority included in the ra-PrioritizationTwoStep in rach-ConfigDedicated;

3>	if scalingFactorBI is configured in ra-PrioritizationTwoStep in the rach-ConfigDedicated:

4>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	else if both ra-PrioritizationForSlicingTwoStep for a slice group identity and ra-PrioritizationForAccessIdentityTwoStep are configured for the selected carrier:

3>	if enableRA-PrioritizationForSlicing in BWP-UplinkCommon is set to true:

4>	if powerRampingStepHighPriority is configured in the ra-PrioritizationForSlicingTwoStep for the slice group identity for which the procedure was initiated:

5>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

4>	if scalingFactorBI is configured in the ra-PrioritizationForSlicingTwoStep for the slice group identity for which the procedure was initiated:

5>	set SCALING_FACTOR_BI to the scalingFactorBI.

3>	else:

4>	if the MAC entity is provided by upper layers with Access Identity 1 or 2; and

4>	if for at least one of these Access Identities the corresponding bit in the ra-PrioritizationForAI is set to one:

5>	if powerRampingStepHighPriority is configured in the ra-PrioritizationForAccessIdentityTwoStep:

6>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

5>	if scalingFactorBI is configured in the ra-PrioritizationForAccessIdentityTwoStep:

6>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	else if ra-PrioritizationForSlicingTwoStep for a slice group identity is configured for the selected carrier:

3>	if powerRampingStepHighPriority is configured in the ra-PrioritizationForSlicingTwoStep for the slice group identity for which the procedure was initiated:

4>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

3>	if scalingFactorBI is configured in the ra-PrioritizationForSlicingTwoStep for the slice group identity for which the procedure was initiated:

4>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	else if ra-PrioritizationForAccessIdentityTwoStep is configured for the selected carrier; and

2>	if the MAC entity is provided by upper layers with Access Identity 1 or 2; and

2>	if for at least one of these Access Identities the corresponding bit in the ra-PrioritizationForAI is set to one:

3>	if powerRampingStepHighPriority is configured in the ra-PrioritizationForAccessIdentityTwoStep:

4>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

3>	if scalingFactorBI is configured in the ra-PrioritizationForAccessIdentityTwoStep:

4>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	set MSGA_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP.

1>	else (i.e. RA_TYPE is set to 4-stepRA):

2>	set PREAMBLE_POWER_RAMPING_STEP to powerRampingStep;

2>	set SCALING_FACTOR_BI to 1;

2>	set preambleTransMax to preambleTransMax included in the RACH-ConfigGeneric;

2>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17); and

2>	if beamFailureRecoveryConfig is configured for the active UL BWP of the selected carrier:

3>	start the beamFailureRecoveryTimer, if configured;

3>	apply the parameters powerRampingStep, preambleReceivedTargetPower, and preambleTransMax configured in the beamFailureRecoveryConfig.

2>	if the Random Access procedure was initiated for beam failure recovery (as specified in clause 5.17); and

2>	if beamFailureRecoveryConfig is configured for the active UL BWP of the selected carrier; and

2>	if ra-Prioritization is configured in the beamFailureRecoveryConfig:

3>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority included in the ra-Prioritization in beamFailureRecoveryConfig;

3>	if scalingFactorBI is configured in ra-Prioritization in the beamFailureRecoveryConfig:

4>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	else if the Random Access procedure was initiated for reconfiguration with sync or for SCG activation; and

2>	if rach-ConfigDedicated is configured for the selected carrier; and

2>	if ra-Prioritization is configured in the rach-ConfigDedicated:

3>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority included in the ra-Prioritization in rach-ConfigDedicated;

3>	if scalingFactorBI is configured in ra-Prioritization in the rach-ConfigDedicated:

4>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	else if both ra-PrioritizationForSlicing for a slice group identity and ra-PrioritizationForAccessIdentity are configured for the selected carrier:

3>	if enableRA-PrioritizationForSlicing in BWP-UplinkCommon is set to true:

4>	if powerRampingStepHighPriority is configured in the ra-PrioritizationForSlicing for the slice group identity for which the procedure was initiated:

5>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

4>	if scalingFactorBI is configured in the ra-PrioritizationForSlicing for the slice group identity for which the procedure was initiated:

5>	set SCALING_FACTOR_BI to the scalingFactorBI.

3>	else:

4>	if the MAC entity is provided by upper layers with Access Identity 1 or 2; and

4>	if for at least one of these Access Identities the corresponding bit in the ra-PrioritizationForAI is set to one:

5

>	if powerRampingStepHighPriority is configured in the ra-PrioritizationForAccessIdentity:

6>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

5>>	if scalingFactorBI is configured in the ra-PrioritizationForAccessIdentity:

6>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	else if ra-PrioritizationForSlicing for a slice group identity is configured for the selected carrier:

3>	if powerRampingStepHighPriority is configured in the ra-PrioritizationForSlicing for the slice group identity for which the procedure was initiated:

4>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

3>	if scalingFactorBI is configured in the ra-PrioritizationForSlicing for the slice group identity for which the procedure was initiated:

4>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	else if ra-PrioritizationForAccessIdentity is configured for the selected carrier; and

2>	if the MAC entity is provided by upper layers with Access Identity 1 or 2; and

2>	if for at least one of these Access Identities the corresponding bit in the ra-PrioritizationForAI is set to one:

3>	if powerRampingStepHighPriority is configured in the ra-PrioritizationForAccessIdentity:

4>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

3>	if scalingFactorBI is configured in the ra-PrioritizationForAccessIdentity:

4>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	if RA_TYPE is switched from 2-stepRA to 4-stepRA during this Random Access procedure:

3>	set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

NOTE:	If enableRA-PrioritizationForSlicing is not configured in BWP-UplinkCommon and if both the provided slice group identity and the provided Access Identity whose corresponding bit in the ra-PrioritizationForAI is set to one are configured with ra-Prioritization either in RACH-ConfigCommon or RACH-ConfigCommonTwoStepRA, it is up to UE implementation how to determine the values of PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI.

Editor's Note (RAN Slicing):	At least for 5.1.1 and 5.1.1a, leave RACH partitioning for Slicing to general MAC CR and capture RA prioritization for Slicing in Slicing MAC CR. Note that all these changes in Slicing MAC CR are subject to the final decisions in the common RACH session, which will reflect in the combined MAC CR.

End of Changes