3GPP TSG-RAN2 #118-e	R2-2204887

# Electronic meeting, May 9 – May 20, 2022

Agenda item:		6.0.4 (TEI17)

Source:	LG Electronics

Title: 	LCP priority of MAC CEs

Document for:	Discussion and Decision

1.	Introduction

In Rel-17, a larger number of new MAC CEs are introduced from various WIs. The new MAC CEs introduced for UL is summarized below.

| Index | LCID values | WI |
| --- | --- | --- |
| 35 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) for a RedCap UE | RedCap |
| 36 | CCCH1 of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) for a RedCap UE | RedCap |
| 44 | Timing Advance Report | NTN |
| 292 | Enhanced Multiple Entry PHR for multiple TRP (four octets Ci) | FeMIMO |
| 293 | Enhanced Multiple Entry PHR for multiple TRP (one octets Ci) | FeMIMO |
| 294 | Enhanced Single Entry PHR for multiple TRP | FeMIMO |
| 295 | Enhanced Multiple Entry PHR (four octets Ci) | FeMIMO |
| 296 | Enhanced Multiple Entry PHR (one octets Ci) | FeMIMO |
| 297 | Enhanced Single Entry PHR | FeMIMO |
| 298 | Enhanced BFR (one octet Ci) | FeMIMO |
| 299 | Truncated Enhanced BFR (one octet Ci) | FeMIMO |
| 300 | Enhanced BFR (four octet Ci) | FeMIMO |
| 301 | Truncated Enhanced BFR (four octet Ci) | FeMIMO |
| 302 | Positioning Measurement Gap Activation/Deactivation Request | Pos |
| 303 | IAB-MT Recommended Beam Indication | IAB |
| 304 | Desired IAB-MT PSD range | IAB |
| 305 | Desired DL Tx Power Adjustment | IAB |
| 306 | Case-6 Timing Request | IAB |
| 307 | Desired Guard Symbols for Case 6 timing | IAB |
| 308 | Desired Guard Symbols for Case 7 timing | IAB |
| 309 | Extended Short Truncated BSR | IAB |
| 310 | Extended Long Truncated BSR | IAB |
| 311 | Extended Short BSR | IAB |
| 312 | Extended Long BSR | IAB |
| 313 | Extended Pre-emptive BSR | IAB |

When a new transmission is performed, the MAC entity shall allocate UL resources to the logical channels and MAC CEs based on the priority. However, in the latest version of MAC specification (v17.0.0), the new MAC CEs (except Timing Advance Report MAC CE) are not listed in the logical channel prioritization procedure, as shown below.

5.4.3.1.3 Allocation of resources Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - C-RNTI MAC CE or data from UL-CCCH; - Configured Grant Confirmation MAC CE or MAC CEs for BFR or Multiple Entry Configured Grant Confirmation MAC CE; - Sidelink Configured Grant Confirmation MAC CE; - LBT failure MAC CE; - MAC CE for Timing Advance Report; - MAC CE for SL-BSR prioritized according to clause 5.22.1.6; - MAC CE for BSR, with exception of BSR included for padding; - Single Entry PHR MAC CE or Multiple Entry PHR MAC CE; - MAC CE for the number of Desired Guard Symbols; - MAC CE for Pre-emptive BSR; - MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding; - data from any Logical Channel, except data from UL-CCCH; - MAC CE for Recommended bit rate query; - MAC CE for BSR included for padding; - MAC CE for SL-BSR included for padding.

This document discusses the issues in logical channel prioritization procedure caused by introduction of bunch of new MAC CEs.

2.	Discussion

The LCP priority is used in LCP procedure to allocate UL resource for logical channels and MAC CEs. Though many items are listed in the LCP priority order, it can be simply modelled as follows.

- C-RNTI MAC CE or data from UL-CCCH

- High priority MAC CEs (e.g. CG confirmation, failure reporting)

- Middle priority MAC CEs (e.g. scheduling assistance information reporting)

- data from any Logical Channel, except data from UL-CCCH

- Low priority MAC CEs (e.g. RBR, padding BSR)

We think above LCP priority order is important for resource allocation, and should be kept in any case.

Proposal 1: RAN2 keeps the following LCP priority order for LCP procedure.

- C-RNTI MAC CE or data from UL-CCCH

- High priority MAC CEs

- Middle priority MAC CEs

- data from any Logical Channel, except data from UL-CCCH

- Low priority MAC CEs

If Proposal 1 is agreed, the next issue is how to set the LCP priority of each MAC CE within High/Middle/Low priority.

In current specification, the MAC CEs are listed in the priority order. If we follow this rule, each time a new MAC CE is introduced, RAN2 must discuss and decide the LCP priority of the new MAC CE considering existing MAC CEs and other new MAC CEs.

If only one or two MAC CEs are introduced, it may be easy to decide the LCP priority. However, if bunch of new MAC CEs are introduced as in Rel-17, it is easily expected that lots of time is required to decide the LCP priority of each new MAC CE. We think it is time-consuming process without much gain, and better to be avoided. Moreover, if more new MAC CEs are introduced in a future, RAN2 have to have same discussion again, which should be avoided.

Therefore, we propose that RAN2 only categorize MAC CEs into High/Middle/Low priority and do not specify LCP priority order within High/Middle/Low priority category.

Proposal 2: Only categorize MAC CEs into High/Middle/Low priority and do not specify LCP priority order within High/Middle/Low priority category.

Following table shows our initial analysis of priority category of each MAC CE. Definitely, RAN2 need further discussion, but this table can be used as a baseline. Note that new MAC CEs introduced in Rel-17 are highlighted in yellow. Also note that the CCCHs for RedCap are not listed in the table because they belong to “data from UL-CCCH” which has the highest priority.

| Priority category | Types | MAC CE |
| --- | --- | --- |
| High | CG confirmation | Configured Grant Confirmation |
|  |  | Multiple Entry Configured Grant Confirmation |
|  |  | Sidelink Configured Grant Confirmation |
|  | BFR | BFR |
|  |  | Enhanced BFR (one octet Ci) |
|  |  | Truncated Enhanced BFR (one octet Ci) |
|  |  | Enhanced BFR (four octet Ci) |
|  |  | Truncated Enhanced BFR (four octet Ci) |
|  | LBT failure | LBT failure |
| Middle | BSR | BSR except padding BSR |
|  |  | Extended Short BSR |
|  |  | Extended Short Truncated BSR |
|  |  | Extended Long BSR |
|  |  | Extended Long Truncated BSR |
|  |  | prioritized SL-BSR |
|  |  | non-prioritized SL-BSR except padding SL-BSR |
|  |  | Pre-emptive BSR |
|  |  | Extended Pre-emptive BSR |
|  | PHR | Single Entry PHR MAC CE |
|  |  | Enhanced Single Entry PHR |
|  |  | Enhanced Single Entry PHR for multiple TRP |
|  |  | Multiple Entry PHR MAC CE |
|  |  | Enhanced Multiple Entry PHR (one octets Ci) |
|  |  | Enhanced Multiple Entry PHR for multiple TRP (one octets Ci) |
|  |  | Enhanced Multiple Entry PHR (four octets Ci) |
|  |  | Enhanced Multiple Entry PHR for multiple TRP (four octets Ci) |
|  | ETC | Timing Advance Report |
|  |  | Positioning Measurement Gap Activation/Deactivation Request |
|  |  | IAB-MT Recommended Beam Indication |
|  |  | Desired IAB-MT PSD range |
|  |  | Desired DL Tx Power Adjustment |
|  |  | Case-6 Timing Request |
|  |  | Desired Guard Symbols |
|  |  | Desired Guard Symbols for Case 6 timing |
|  |  | Desired Guard Symbols for Case 7 timing |
| Low | RBR | Recommended bit rate query |
|  | Padding BSR | Padding BSR |
|  |  | Padding SL-BSR |

Proposal 3: Use the proposed MAC CE priority table as a baseline for further discussion.

For resource allocation, the MAC entity anyway needs to decide the LCP priority of each MAC CE even within each priority category. Two options can be considered.

Option 1: LCP priority by UE implementation

In this option, the UE autonomously determines LCP priority of each MAC CE within each priority category by implementation. The advantage of this option is that the spec maintenance becomes easy and more future proof. However, it may be difficult for the network to decide how much UL grant it has to provide to the UE because the network does not know which MAC CE the UE will transmit on the UL grant.

Option 2: Network configurable LCP priority

In this option, the network signals LCP priority of each MAC CE within each priority category. The advantage of this option is that the network can control the UE’s transmission of MAC CEs, and thus can receive necessary information from the UE. However, providing LCP priority for each MAC CE increases signaling overhead and requires changes in ASN.1 structure. In addition, signaling design for LCP priority requires further discussion.

We have no strong view between two options. But considering the limited time in Rel-17, Option 1 may be preferable. Thus, we propose that LCP priority within each priority category is determined by UE implementation.

Proposal 4: Within each priority category, the LCP priority of each MAC CE is determined by UE implementation.

3.	Conclusions

In this paper, we discuss the issues in logical channel prioritization procedure considering the newly introduced MAC CEs. We have following proposals:

Proposal 1: RAN2 keeps the following LCP priority order for LCP procedure.

- C-RNTI MAC CE or data from UL-CCCH

- High priority MAC CEs

- Middle priority MAC CEs

- data from any Logical Channel, except data from UL-CCCH

- Low priority MAC CEs

Proposal 2: Only categorize MAC CEs into High/Middle/Low priority and do not specify LCP priority order within High/Middle/Low priority category.

Proposal 3: Use the proposed MAC CE priority table as a baseline for further discussion.

Proposal 4: Within each priority category, the LCP priority of each MAC CE is determined by UE implementation.