3GPP TSG-RAN WG2 #118e	R2-2204902

Electronic meeting, 9 – 20 May, 2022

Agenda item:		5.1.4.1.4	Inter-Node RRC messages

Title:		Confirmation for inter-MN HO without SN change

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#117e, the issue on inter-MN handover without SN change was further discussed [1]. Finally, the Stage 2 CRs were agreed [2][3]. The need of Stage 3 CR was also discussed but postponed without consensus. In this contribution, we would like to confirm the Stage 3 aspects and propose to capture that in Chairman notes to close this discussions.

# 2. Discussion

As the origin of this discussion, the current RRC specification has some ambiguity in the field descriptions of sourceConfigSCG and scg-RB-Config in CG-ConfigInfo message [4]. For example, it was pointed out that it is not clear whether these fields can be sent during the inter-MN HO without SN change, even though there was consensus that it should be allowed.

scg-RB-Config Contains all of the fields in the IE RadioBearerConfig used in SN, used to allow the target SN to use delta configuration to the UE, e.g. during SN change. The field is signalled upon change of SN unless MN uses full configuration option. Otherwise, the field is absent.

sourceConfigSCG Includes all of the current SCG configurations used by the target SN to build delta configuration to be sent to UE, e.g. during SN change. The field contains the RRCReconfiguration message, i.e. including secondaryCellGroup and measConfig. The field is signalled upon change of SN, unless MN uses full configuration option. Otherwise, the field is absent.

By having LS exchanges with RAN3 [5], it was clarified that “If the target MN decides to keep the SN, the target MN sends SN Addition Request to the SN including the SN UE X2/XnAP ID as a reference to the UE context in the SN that was established by the source MN”. It was also clarified that “In case the SN UE X2/XnAP ID is provided alone, the SN is able to retrieve the stored UE context”. Based on these clarifications, RAN2 could confirm that the following cases were supported and update the Stage 2 (37.340) specification.

Case 0:

- Inter-MN HO without SN change (delta config is allowed in SN)

  - SN UE X2/XnAP ID	present

  - sourceConfigSCG	present

  - scg-RB-Config		present

Case 1:

- Inter-MN HO without SN change (delta config is allowed in SN)

  - SN UE X2AP ID		present

  - sourceConfigSCG	not present

  - scg-RB-Config		not present

Case 2:

- Inter-MN HO without SN node change (delta config is allowed in SN)

  - SN UE X2AP ID		not present

  - sourceConfigSCG	present

  - scg-RB-Config		present

We have thought that similarly to the Stage 2, the Stage 3 RRC spec can be improved to make it clear which scenarios are supported. However, after offline discussions and coordination, it has been identified that we should not touch the Stage 3, as a perfect wording to clarify everything could not be thought of and even if there is, it would be too much complicated and not suitable for Rel-15 frozen very long time ago. Thus, we would like to propose to capture what are confirmed in a Chairman notes explicitly to avoid repeating the same or similar discussion.

Proposal 1: RAN2 to agree to capture the following in a Chairman notes.

- RAN2 confirms that according to the current RRC spec, both fields sourceConfigSCG and scg-RB-Config in CG-ConfigInfo can be sent in the following cases:

  - SN change procedure

  - Inter-MN HO with SN change

  - Inter-MN HO without SN change (Case 0)

  - Inter-MN HO without SN node change (Case 2)

With this confirmation, we believe that the issue can be completely closed after long time discussions for almost one year.

# 3. Conclusion

In this contribution we discussed the postponed issue of Stage 3 clarifications on inter-MN handover without SN change and made the following proposal.

Proposal 1: RAN2 to agree to capture the following in a Chairman notes.

- RAN2 confirms that according to the current RRC spec, both fields sourceConfigSCG and scg-RB-Config in CG-ConfigInfo can be sent in the following cases:

  - SN change procedure

  - Inter-MN HO with SN change

  - Inter-MN HO without SN change (Case 0)

  - Inter-MN HO without SN node change (Case 2)

With this confirmation, we believe that the issue can be completely closed after long time discussions for almost one year.

# References

[1] RAN2#117e Chairman notes

[2] R2-2203936, Clarification on inter-MN handover without SN change (R15 CR), Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, Samsung, NEC

[3] R2-2203937, Clarification on inter-MN handover without SN change (R16 CR), Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, Samsung, NEC

[4] TS 38.331

[5] R2-2202121, Reply LS on inter-MN handover without SN change, RAN3

# Annex for reference [1]

Inter-MN HO without SN change

R2-2202121	Reply LS on inter-MN handover without SN change (R3-216165; contact: Huawei)	RAN3	LS in	Rel-15	To:RAN2

Moved from 5.1

[029] Noted

R2-2203500	Clarification on inter-MN handover without SN change (R15)	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, Samsung	CR	Rel-15	37.340	15.15.0	0299	-	F	NR_newRAT-Core

=> Revised in R2-2203936

R2-2203936	Clarification on inter-MN handover without SN change (R15)	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, Samsung, NEC	CR	Rel-15	37.340	15.15.0	0299	1	F	NR_newRAT-Core

[029] agreed

R2-2203501	Clarification on inter-MN handover without SN change (R16)	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, Samsung	CR	Rel-16	37.340	16.8.0	0300	-	A	NR_newRAT-Core

=> Revised in R2-2203937

R2-2203937	Clarification on inter-MN handover without SN change (R16)	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, Samsung, NEC	CR	Rel-16	37.340	16.8.0	0300	1	A	NR_newRAT-Core

[029] agreed

R2-2202806	Signalling in inter-MN HO without SN change	NEC	discussion	Rel-15	NR_newRAT-Core

[029] Noted

R2-2202807	Clarification on inter-MN handover without SN change	NEC	CR	Rel-15	38.331	15.16.0	2907	-	F	NR_newRAT-Core

R2-2202808	Clarification on inter-MN handover without SN change	NEC	CR	Rel-16	38.331	16.7.0	2908	-	A	NR_newRAT-Core

[029] Both Postponed