3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2204905

Online, May 9 – 20, 2022

Agenda Item:	6.1.3.2

Source:	MediaTek Inc.

Title:	Corrections on CSI-mask and DCP coexistence for multicast DRX

Document for:	Discussion, Decision

Introduction

In the last RAN2 meeting, the following issues are left over for multicast DRX:

- FFS CSI-mask for multicast DRX OnDuration

RAN2 has agreed that UE can report CSI/SRS transmission during the Active Time of multicast DRX and/or during the running of drx-onDurationTimerPTM, and the spec has changed for unicast DRX(section 5.7) as follows:

1> else: 2> in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured or, in current symbol n, if all multicast DRX would not be in Active Time considering multicast assignments and DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group; 3> not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

However, it is not clear if CSI-masking is also impact by multicast DRX. It is also not clear the unicast DCP monitoring/WUS configuration when multicast DRX is configured.

Assume that Unicast DCP monitoring/WUS may be configured when Multicast DRX is configured. CSI reporting, SRS impact, and whether some restriction need to be captured is FFS (can revert the assumption if the FFS resolution turns out to be complex)

In this paper, we further discuss the CSI-masking impact and WUS configuration for multicast DRX.

Discussion

In the last meeting, it was left over whether to capture similar text for CSI-mask (when CSI masking is setup) as follow in section 5.7(unicast DRX).

2> if CSI masking (csi-Mask) is setup by upper layers: 3> in current symbol n, if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 3> in current symbol n, if drx-onDurationTimerPTM would not be running considering grants/assignments when evaluating all DRX Active Time conditions and DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b: 4> not report CSI on PUCCH in this DRX group.

The reason is that if similar text is not captured, the CSI-masking is only applicable to unicast. We think CSI-mask should be aligned for both unicast and multicast DRX. So for the case which RRC indicate CSI-masking, and the multicast DRX is considered in active time but the drx-onDurationTimerPTM is not running, the CSI shall not be reported.

Proposal 1: The CSI shall not be reported when CSI-masking is setup and the multicast DRX is considered in active time but the drx-onDurationTimerPTM is not running.

Proposal 2: The above text should be added for CSI-masking in MAC running CR section 5.7.

For the DCP monitoring, it seems RAN2 agrees that Unicast DCP monitoring/WUS may be configured when Multicast DRX is configured. Therefore, it is also left over whether to add the similar text in section 5.7:

1> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and 1> if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause, 2> if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause, and the MAC entity would not be in Multicast DRX’s Active Time defined in clause 5.7b: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report semi-persistent CSI configured on PUSCH; 3> if ps-TransmitPeriodicL1-RSRP is not configured with value true: 4> not report periodic CSI that is L1-RSRP on PUCCH. 3> if ps-TransmitOtherPeriodicCSI is not configured with value true: 4> not report periodic CSI that is not L1-RSRP on PUCCH.

In our understanding, the DCP is designed for unicast DRX, which is independent with multicast DRX. The above text should be add to unicast DRX(section 5.7) to ensure both multicast DRX and unicast DRX are not activated, and UE reporting CSI/SRS for multicast DRX is not affected by the DCP configuration.

Proposal 3: The CSI/SRS reporting for multicast DRX is not affected by the DCP configuration.

Proposal 4: The above text should be added for DCP monitoring in MAC running CR section 5.7 to ensure both multicast DRX and unicast DRX are not activated.

Conclusion

In this contribution, the following proposals are made:

Proposal 1: The CSI shall not be reported when CSI-masking is setup and the multicast DRX is considered in active time but the drx-onDurationTimerPTM is not running.

Proposal 2: The following text should be added for CSI-masking in MAC running CR section 5.7.

2> if CSI masking (csi-Mask) is setup by upper layers: 3> in current symbol n, if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 3> in current symbol n, if drx-onDurationTimerPTM would not be running considering grants/assignments when evaluating all DRX Active Time conditions and DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b: 4> not report CSI on PUCCH in this DRX group.

Proposal 3: The CSI/SRS reporting for multicast DRX is not affected by the DCP configuration.

Proposal 4: The following text should be added for DCP monitoring in MAC running CR section 5.7 to ensure both multicast DRX and unicast DRX are not activated.

1> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and 1> if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause, 2> if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause, and the MAC entity would not be in Multicast DRX’s Active Time defined in clause 5.7b: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report semi-persistent CSI configured on PUSCH; 3> if ps-TransmitPeriodicL1-RSRP is not configured with value true: 4> not report periodic CSI that is L1-RSRP on PUCCH. 3> if ps-TransmitOtherPeriodicCSI is not configured with value true: 4> not report periodic CSI that is not L1-RSRP on PUCCH.

References

R2_117-e Chair Notes EOM

R2-2203761	Report of Offline 042: Invited Tdocs Open Issues UP

R2-2203818 Introduction of NR MBS in 38.321