3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2204907

Online, May 9 – 20, 2022

Agenda Item:	6.1.4

Source:	MediaTek Inc.

Title:	Discussion on mandatory ROHC support for MBS broadcast

Document for:	Discussion, Decision

Introduction

In the last RAN2 meeting, the ROHC capability have been discussed for MBS broadcast. The related agreement is listed below:

RAN2#117-e:

P12: RoHC is mandatory for UEs supporting MBS broadcast: • At least profiles 0x0000, 0x0001, 0x0002 are supported. FFS other profiles. • FFS how many RoHC context sessions the UE has to mandatorily support. The number between 2 and 16 should be chosen. - RoHC profile 0x0006 is not used / configurable for broadcast MRB.

In this paper, we further discuss the supporting ROHC context session and profiles for broadcast.

Discussion

In case that ROHC is agreed to be mandatory capability for MBS broadcast UEs, the capability requirement should be limited and applicable to all UEs supporting broadcast.

For ROHC context sessions, the default value of MAX_CID is 15, and it is also the maximum value of small CID. Considering the traffic type of broadcast service and limited UE performance in Idle/Inactive state, a smaller value should be used for the ROHC context session of broadcast.

Proposal 1: Define a smaller value of the ROHC context session (e.g. 2) to be supported for mandatory ROHC capability for broadcast

For ROHC profiles supported for broadcast, we suggest using a minimum set as profiles 0x0000, 0x0001 and 0x0002(which are mandatory at least for voice over IMS) to be supported for MBS.

Proposal 2: Only profiles 0x0000, 0x0001 and 0x0002 are supported for mandatory ROHC capability for broadcast.

Conclusion

In this contribution, the following proposals are made:

Proposal 1: Define a smaller value of the ROHC context session (e.g. 2) to be supported for mandatory ROHC capability for broadcast

Proposal 2: Only profiles 0x0000, 0x0001 and 0x0002 are supported for mandatory ROHC capability for broadcast.

References

R2_117-e Chair Notes EOM

R2-2203343 Report of: [Pre117-e][001][MBS] CP open Issues Input