3GPP TSG-RAN WG2 Meeting #118-e	R2-2204924

Online, May 9 – 20, 2022

Agenda Item:	6.15.2.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on latency bound for inter-UE coordination

Document for:	Discussion and Decision

- Introduction

The following agreements related to IUC were achieved in RAN2#117-e meeting [1]:

Agreement on IUC: 1: A standalone MAC CE for UE-A’s IUC information is transmitted through HARQ Feedback disabled MAC PDU. 2: When a MAC CE for IUC information is multiplexed with MAC SDU(s), the HARQ attribute of a MAC PDU is determined by following sl-HARQ-FeedbackEnabled being set to enabled or disabled for the highest priority logical channel included in the MAC PDU. 3: A standalone MAC CE for UE-B’s explicit request is transmitted through HARQ Feedback disabled MAC PDU. 4: When a MAC CE for explicit request is multiplexed with MAC SDU(s), the HARQ attribute of a MAC PDU is determined by following sl-HARQ-FeedbackEnabled being set to enabled or disabled for the highest priority logical channel included in the MAC PDU. 5: The priority order of a MAC CE for UE-B’s explicit request is between SL CSI reporting MAC CE and SL DRX command MAC CE (when priority of IUC REQ MAC CE is fixed as “1”). 6: The priority order of a IUC Information MAC CE is between SL CSI reporting MAC CE and SL DRX command MAC CE (when priority of IUC Information MAC CE is fixed as “1”). 7: Send LS to RAN1 to inform RAN2 understanding on the priority of IUC INFO/IUC REQ MAC CE and RAN2 preference to fix the priority of IUC INFO/IUC REQ MAC CE as “1”. 8: RAN2 introduces a mechanism of timer-based latency bound restriction for transmission of UE-A’s IUC information. 9: Timer-based latency bound restriction is applied for the explicit request based UE-A’s IUC information transmission. 10: RAN2 introduces the timer-based latency bound restriction on the transmission of UE-A’s IUC information for both preferred resource set and non-preferred resource set in explicit request-based IUC. 11: Working assumption: UE-B sets the timer value to UE-A through PC5 RRC signalling 12: RAN2 supports that UE-A starts the timer for the transmission of UE-A's IUC information in the explicit request-based IUC when receiving an explicit request from UE-B and deciding to trigger IUC information to be transmitted UE-B. 13: RAN2 supports that UE-A can stop the timer for the transmission of IUC information in explicit request-based IUC when an IUC information to UE-B is generated by the Multiplexing and Assembly procedure. 14: RAN2 supports that UE-A can cancel the transmission of IUC information in explicit request-based IUC if the timer for the triggered UE-A’s IUC information reporting expires. 15: RAN2 supports that UE-A can cancel the transmission of IUC information in explicit request-based IUC when an IUC information to UE-B is generated by the Multiplexing and Assembly procedure. 16: For determining preferred resource set in Scheme 1, PC5-RRC signalling from UE-B to UE-A for transmitting the parameters (i.e., prio_TX, L_subCH, P_rsvp_TX, n+T_1, n+T_2) is not supported when inter-UE coordination information transmission is triggered by a condition other than explicit request reception. 17: For inter-UE coordination information is triggered by UE-B’s request, RAN2 not further discuss PC5-RRC signaling from UE-B to UE-A to provide information on whether UE-B supports sensing/resource exclusion. 18: No special handling is needed to handle IUC REQ MAC CE latency bound.

In this contribution, we will discuss some remaining issues on latency bound for IUC.

- Discussion

In RAN2#117e-meeting, it was agreed that timer-based latency bound restriction is applied for the explicit request based UE-A’s IUC information transmission, and UE-B sets the timer value to UE-A through PC5 RRC signalling.

However, it is not clear whether such timer-based latency bound restriction is applied for condition based IUC information transmission. In Rel-16, SL-CSI reporting is always triggered by another UE via SCI, there is no condition based SL-CSI reporting transmission. Furthermore, regrading condition based IUC information, all operations are handled by UE-A, i.e., whether the condition to trigger an IUC transmission satisfied or not is determined by UE-A. Therefore, it is not necessary to introduce a timer to restrict the UE-A to start it when itself determines the condition is satisfied and decides to trigger the IUC transmission. In addition, the [T1, T2] window can only be up to UE-A’s implementation. Therefore, it does not make sense to let UE-B to set the latency bound to UE-A for condition based IUC information transmission. Based on the above analysis, it is suggested that RAN2 to confirm that timer-based latency bound restriction is not applied for the condition based UE-A’s IUC information transmission.

Proposal 1: RAN2 to agree that timer-based latency bound restriction is not applied for the condition based UE-A’s IUC information transmission.

Another issue is that how to handle the case when UE-A in mode 1 receiving latency bound timer value via PC5 RRC signalling from UE-B. It was agreed by RAN1 that inter-UE coordination is only feasible in mode 2 in Rel-17, thus in this case, if UE-A send PC5 RRCReconfiguration complete message to UE-B, UE-B may think UE-A is able to provide IUC information. However, since UE-A is in mode 1, it cannot provided any IUC information to UE-B, and UE-B may further request IUC information to UE-A if there is no any response from UE-A. Therefore, to avoid unnecessary signalling interaction and to improve the system performance of IUC mechanism, it is proposed that UE-A in mode 1 sends PC5 RRCReconfiguration Failure message to UE-B when receiving latency bound timer value via PC5 RRCReconfiguration message from UE-B, and an indication can be included in Failure message.

Proposal 2: For UE-A in mode 1, UE-A sends PC5 RRCReconfiguration Failure message to UE-B when receiving latency bound timer value via PC5Reconfiguation, and an indication can be included in such Failure message.

- Conclusion

This contribution further discuss some remaining issues on latency bound for IUC. The proposals are as follows:

Proposal 1: RAN2 to agree that timer-based latency bound restriction is not applied for the condition based UE-A’s IUC information transmission.

Proposal 2: For UE-A in mode 1, UE-A sends PC5 RRCReconfiguration Failure message to UE-B when receiving latency bound timer value via PC5Reconfiguation, and an indication can be included in such Failure message.

- Reference

- Report of 3GPP TSG RAN WG2 meeting #117-e, Online