3GPP TSG-RAN WG2 Meeting #118-e                                                    R2-2204946

Electronic, 9th – 20th May, 2022

Source:              CATT

Title:	Combination of SL DRX, Discovery and Relay-related Communication

Agenda Item:	6.15.2.3

Document for:	Discussion and Decision

Introduction

According to [1], the combination of SL DRX, discovery and relay has not been fully captured in spec. In this contribution, this issue will be discussed.

Discussion

Regarding to the combination of SL DRX and relay, the following agreements were reached in RAN2#116-e meeting:

Agreements on SL-DRX for ProSe:

1:	RAN2 confirm R17 SL-DRX design can support non-relay-related ProSe communication directly without additional specific solution discussion / specification effort.

2:	RAN2 confirm the R17 SL-DRX design can support non-relay-related ProSe discovery by reusing SL default-DRX configuration used for communication without further additional specific solution discussion / specification effort.

Agreements on SL-DRX for ProSe:

1:	RAN2 confirms Rel-17 SL-DRX design can be reused for relay-related ProSe communication in layer-3 relay without additional specific solution discussion/specification effort.

2:	Keep RAN2 previous agreement (prioritize the non-relay case without consideration of relay specific optimization in Rel-17) but we’re not going to make any conclusion if L2 relay-related ProSe communication is supported or not in Rel-17 now.

3:	RAN2 confirms Rel-17 SL-DRX design can be reused for L3 relay-related ProSe discovery without additional specific solution discussion/specification effort (by applying SL default-DRX configuration). No conclusion if L2 relay-related ProSe discovery is supported or not in Rel-17 now. RAN2 does not specify any restriction now.

The summary of the above agreements is shown in the following table:

Table-1 SL DRX support for discovery and relay communication

| Features |  | SL DRX support |
| --- | --- | --- |
| L3 U2N relay | Discovery | Yes |
|  | Communication | Yes |
| L2 U2N relay | Discovery | FFS |
|  | Communication | FFS |
| Non-relay discovery |  | Yes |

But according to the description in current published TS38.321 [1], how to support SL DRX for L2/L3 U2N relay and non-relay discovery has not been captured till now. For example, in TS38.321, when performing destination selection during LCP, for sidelink discovery and relay-related communication, active time is not considered:

###### 5.22.1.4.1.2 Selection of logical channels The MAC entity shall for each SCI corresponding to a new transmission: 1> if sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon is configured according to TS 38.331 [5]: 2> if the new transmission is associated with a sidelink grant in sl-DiscTxPoolSelected or sl-DiscTxPoolScheduling configured in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon: 3> select a Destination associated with sidelink discovery as specified in TS 23.304 [26], having at least one of the logical channel with the highest priority, among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI: 4> SL data is available for transmission; and 4> SBj > 0, in case there is any logical channel having SBj > 0; and 4> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 4> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant. 2> else: 3> select a Destination associated with one of unicast, groupcast and broadcast (excluding the Destination(s) associated with sidelink discovery as specified in TS 23.304 [26]), having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI: 4> SL data is available for transmission; and 4> SBj > 0, in case there is any logical channel having SBj > 0; and 4> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 4> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 4> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI. 1> else: 2> select a Destination associated to one of unicast, groupcast and broadcast, that is in the SL active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

Hence, RAN2 should first discuss whether the combination of SL DRX, discovery and relay-related communication should be reflected in the specs of Rel-17.

Proposal 1: Suggest RAN2 to discuss whether the combination of SL DRX, discovery and relay-related communication should be reflected in Rel-17 specs.

If RAN2 confirms the combination of SL DRX, discovery and relay-related communication should be reflected in RAN2 spec, it should further discuss how to handle the L2 U2N relay. Since there is divergence on whether the current SL DRX can be directly reused in L2 U2N relay discovery and communication based on the discussion in RAN#95 meeting, it is hard to reach common understanding on this point quickly. In order to make the spec complete, RAN2 should further make conclusion on this point.

Proposal 2: If the combination of SL DRX, discovery and relay-related communication should be reflected in Rel-17 specs, suggest RAN2 to discuss how to handle the L2 U2N relay when modifying the specs since there is no agreement on whether SL DRX can be supported for L2 U2N relay.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Suggest RAN2 to discuss whether the combination of SL DRX, discovery and relay-related communication should be reflected in Rel-17 specs.

Proposal 2: If the combination of SL DRX, discovery and relay-related communication should be reflected in Rel-17 specs, suggest RAN2 to discuss how to handle the L2 U2N relay when modifying the specs since there is no agreement on whether SL DRX can be supported for L2 U2N relay.

Reference

- TS38.321  NR; Medium Access Control (MAC) protocol specification (Release 17)