3GPP TSG-RAN WG2 Meeting #118-e                                                     R2-2204952

Electronic, 9th - 20th May, 2022

Source:	CATT

Title:	Open Issues of Inter-UE Coordination

Agenda Item:	6.15.2.4‎

Document for:	Discussion and Decision

Introduction

There are three open issues of inter-UE coordination should be discussed as following:

- Priority order between Inter-UE Coordination request MAC CE and Inter-UE Coordination Information MAC CE;

- Timer-based latency bound restriction for condition-based Inter-UE Coordination request MAC CE;

- Maximum number of resource combinations that can be included in Inter-UE Coordination Information MAC CE.

In this contribution, we will discuss the above issues.

Discussion

Priority order between Inter-UE Coordination request MAC CE and Inter-UE Coordination Information MAC CE

In [1], the priority order between Inter-UE Coordination request MAC CE and Inter-UE Coordination Information MAC CE was discussed. The options are below:

a)	Inter-UE Coordination request MAC CE has a higher priority than Inter-UE Coordination Information MAC CE;

b)	Inter-UE Coordination Information MAC CE has a higher priority than Inter-UE Coordination request MAC CE;

c)	Inter-UE Coordination request MAC CE and Inter-UE Coordination Information MAC CE have the same priority.

In RAN2#117-e meeting, there was no agreement on this issue. And in TS 38.321, option c) was captured.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - data from SCCH; - Sidelink CSI Reporting MAC CE; - Sidelink Inter-UE Coordination Request MAC CE and Sidelink Inter-UE Coordination Reporting MAC CE; - Sidelink DRX Command MAC CE; - data from any STCH.

Since Inter-UE Coordination Request MAC CE is sent by UE-B and Inter-UE Coordination Information MAC CE is sent by UE-A, it is unnecessary to differentiate the priority order of two MAC CEs. Even if a UE has both Inter-UE Coordination Request MAC CE and Inter-UE Coordination Information MAC CE available for transmission, the prioritization can be up to UE implementation.

Proposal 1: RAN2 confirms that Inter-UE Coordination request MAC CE and Inter-UE Coordination Information MAC CE have the same priority.‎

Timer-based latency bound restriction for condition-based Inter-UE Coordination Information MAC CE

In RAN2#117-e meeting, there was no agreement on whether the timer-based latency bound restriction is applied for the transmission of condition-based IUC information. In [1], most of companies supported it. Consideration on the content of Inter-UE Coordination Information MAC CE is sidelink resource(s), it is valid in a period of time. It is unnecessary to transmit it once the resource(s) included in Inter-UE Coordination Information MAC CE has passed.

Proposal 2: Timer-based latency bound restriction is applied for condition-based Inter-UE Coordination Information MAC CE.‎

Furthermore, if proposal 2 is agreed, the agreements for explicit request-based IUC can be applied for condition-based IUC:

Proposal 3: For condition-based IUC, the timer-based latency bound restriction for the transmission of UE-A’s IUC information only in UC.

Proposal 4: For condition-based IUC, the timer-based latency bound restriction for both preferred resource set and non-preferred resource set.

Proposal 5: UE-A starts the timer for the transmission of UE-A's IUC information in the condition-based IUC when UE-A decides to trigger IUC information to be transmitted to UE-B in the condition-based IUC.

Proposal 6: UE-A can stop the timer for the transmission of IUC information in condition-based IUC when an IUC information to UE-B is generated by the Multiplexing and Assembly procedure.

Proposal 7: UE-A can cancel the transmission of IUC information in condition-based IUC if the timer for the triggered UE-A’s IUC information reporting expires.

Proposal 8: UE-A can cancel the transmission of IUC information in condition-based IUC when an IUC information to UE-B is generated by the Multiplexing and Assembly procedure.

Since the timer value configured by UE-B is not feasible for condition-based IUC, it is proposed that the timer value is (pre)configured by the network.

Proposal 9: For condition-based IUC, timer value is configured based on (pre)configuration of the network.

Maximum number of resource combinations that can be included in Inter-UE Coordination Information MAC CE.

In TS 38.321 clause 6.1.3.53, the format of Inter-UE Coordination Information MAC CE is below:

Figure 6.1.3.53-1: Inter-UE Coordination Information MAC CE

The size of Inter-UE Coordination Information MAC CE is 6N+1 bytes. The length of the MAC SDU or variable-sized MAC CE is indicated by field L in the MAC subheader. The size of L is 8 bits or 16bits. Therefore, the maximum length of the MAC SDU or variable-sized MAC CE is 256bytes or 65536 bytes. The maximum value of N is 42 or 10922 corresponding to 8-bit L field or 16-bit L field. Consideration on latency, segmentation of Inter-UE Coordination Information MAC CE should be avoided. On the other hand, too-long MAC-CE should also be avoided. Hence, the maximum value of N can be restricted to 42 which can be indicated by 8-bit L field.

Proposal 10: Maximum number of resource combinations included in Inter-UE Coordination Information MAC CE can be 42 with 8-bit L field.

Conclusion

According to the above discussion, the proposals on the above issues are as follows:

Proposal 1: RAN2 confirms that Inter-UE Coordination request MAC CE and Inter-UE Coordination Information MAC CE have the same priority.‎

Proposal 2: Timer-based latency bound restriction is applied for condition-based Inter-UE Coordination Information MAC CE.‎

Proposal 3: For condition-based IUC, the timer-based latency bound restriction for the transmission of UE-A’s IUC information only in UC.

Proposal 4: For condition-based IUC, the timer-based latency bound restriction for both preferred resource set and non-preferred resource set.

Proposal 5: UE-A starts the timer for the transmission of UE-A's IUC information in the condition-based IUC when UE-A decides to trigger IUC information to be transmitted to UE-B in the condition-based IUC.

Proposal 6: UE-A can stop the timer for the transmission of IUC information in condition-based IUC when an IUC information to UE-B is generated by the Multiplexing and Assembly procedure.

Proposal 7: UE-A can cancel the transmission of IUC information in condition-based IUC if the timer for the triggered UE-A’s IUC information reporting expires.

Proposal 8: UE-A can cancel the transmission of IUC information in condition-based IUC when an IUC information to UE-B is generated by the Multiplexing and Assembly procedure.

Proposal 9: For condition-based IUC, timer value is configured based on (pre)configuration of the network.

Proposal 10: Maximum number of resource combinations included in Inter-UE Coordination Information MAC CE can be 42 with 8-bit L field.

TP corresponding to Proposal 9 is in Annex.

Reference

- R2-2203159	Summary of [POST116bis-e][707][V2X/SL] Open issues on IUC (LG)	LG

Annex

* * * Start of Text Proposal * * *

- –	SL-ConfigDedicatedNR

The IE SL-ConfigDedicatedNR specifies the dedicated configuration information for NR sidelink communication.

SL-ConfigDedicatedNR information element

-- ASN1START

-- TAG-SL-CONFIGDEDICATEDNR-START

SL-ConfigDedicatedNR-r16 ::=         SEQUENCE {

sl-PHY-MAC-RLC-Config-r16            SL-PHY-MAC-RLC-Config-r16                                              OPTIONAL,    -- Need M

sl-RadioBearerToReleaseList-r16      SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Uu-ConfigIndex-r16        OPTIONAL,    -- Need N

sl-RadioBearerToAddModList-r16       SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16       OPTIONAL,    -- Need N

sl-MeasConfigInfoToReleaseList-r16   SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIndex-r16     OPTIONAL,    -- Need N

sl-MeasConfigInfoToAddModList-r16    SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-MeasConfigInfo-r16       OPTIONAL,    -- Need N

t400-r16                             ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL,    -- Need M

...,

[[

sl-PHY-MAC-RLC-Config-v1700          SL-PHY-MAC-RLC-Config-v1700                                            OPTIONAL,    -- Need M

sl-DiscConfig-r17                    SetupRelease { SL-DiscConfig-r17}                                      OPTIONAL,    -- Need M

sl-RLC-ChannelToReleaseList-r17      SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelID-r17            OPTIONAL, -- Cond L2U2N

sl-RLC-ChannelToAddModList-r17       SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelConfig-r17        OPTIONAL  -- Cond L2U2N

]]

}

SL-DestinationIndex-r16  ::=             INTEGER (0..maxNrofSL-Dest-1-r16)

SL-PHY-MAC-RLC-Config-r16::=         SEQUENCE {

sl-ScheduledConfig-r16               SetupRelease { SL-ScheduledConfig-r16 }                                OPTIONAL,    -- Need M

sl-UE-SelectedConfig-r16             SetupRelease { SL-UE-SelectedConfig-r16 }                              OPTIONAL,    -- Need M

sl-FreqInfoToReleaseList-r16         SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-Freq-Id-r16               OPTIONAL,    -- Need N

sl-FreqInfoToAddModList-r16          SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16            OPTIONAL,    -- Need N

sl-RLC-BearerToReleaseList-r16       SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfigIndex-r16    OPTIONAL,    -- Need N

sl-RLC-BearerToAddModList-r16        SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16         OPTIONAL,    -- Need N

sl-MaxNumConsecutiveDTX-r16          ENUMERATED {n1, n2, n3, n4, n6, n8, n16, n32}                          OPTIONAL,    -- Need M

sl-CSI-Acquisition-r16               ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

sl-CSI-SchedulingRequestId-r16       SetupRelease {SchedulingRequestId}                                     OPTIONAL,    -- Need M

sl-SSB-PriorityNR-r16                INTEGER (1..8)                                                         OPTIONAL,    -- Need R

networkControlledSyncTx-r16          ENUMERATED {on, off}                                                   OPTIONAL     -- Need M

}

SL-PHY-MAC-RLC-Config-v1700 ::=      SEQUENCE {

sl-DRX-Config-r17                    SetupRelease { SL-DRX-Config-r17 }                                     OPTIONAL,    -- Need M

sl-LatencyBoundIUC-Report-r17        SL-LatencyBoundIUC-Report-r17                                          OPTIONAL,    -- Need M

...

}

SL-DiscConfig-r17::=                 SEQUENCE {

sl-RelayUE-Config-r17                SetupRelease { SL-RelayUE-Config-r17}                                  OPTIONAL, -- L2RelayUE

sl-RemoteUE-Config-r17               SetupRelease { SL-RemoteUE-Config-r17}                                 OPTIONAL  -- L2RemoteUE

}

-- TAG-SL-CONFIGDEDICATEDNR-STOP

-- ASN1STOP

SL-ConfigDedicatedNR field descriptions

sl-LatencyBoundIUC-Report Indicates the latency bound of SL Inter-UE coordination report from the associated SL Inter-UE coordination condition triggering in terms of number of slots.

sl-MeasConfigInfoToAddModList This field indicates the RSRP measurement configurations for unicast destinations to add and/or modify.

sl-MeasConfigInfoToReleaseList This field indicates the RSRP measurement configurations for unicast destinations to remove.

sl-PHY-MAC-RLC-Config This field indicates the lower layer sidelink radio bearer configurations.

sl-RadioBearerToAddModList This field indicates one or multiple sidelink radio bearer configurations to add and/or modify. This field is not configured to the PC5 connection used for L2 U2N relay operation.

sl-RadioBearerToReleaseList This field indicates one or multiple sidelink radio bearer configurations to remove. This field is not configured to the PC5 connection used for L2 U2N relay operation.

* * * End of Text Proposal * * *