3GPP TSG-RAN WG2 Meeting #118-e	R2-2204979

Online, 9–20 May 2022

# Agenda item:	6.12.2.2.2

# Source:	Samsung

# Title:	Cell reselection priority for RedCap (RIL#: S952)

# Document for:	Discussion and Agreement

# 1	Introduction

Even though RedCap is complete from functional point of view, the details on how to capture the agreement to the ASN.1 code still needs to be discussed.

# 2	Discussion

## 2.1	About redcapAccessRejected-r17

RAN2 confirm the following working assumption from RAN2#117 meeting [1], and it is captured to the RRC CR for RedCap [2] as follows:

Agreements [1]:

- The following working assumption is confirmed: “System information can provide information on which frequencies accept RedCap UE access (e.g., by considering whether supporting RedCap)”

### 6.3.1 System information blocks … #### – SIB4 … SIB4 information element -- ASN1START -- TAG-SIB4-START SIB4 ::= SEQUENCE { interFreqCarrierFreqList InterFreqCarrierFreqList, lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ interFreqCarrierFreqList-v1610 InterFreqCarrierFreqList-v1610 OPTIONAL -- Need R ]], [[ interFreqCarrierFreqList-v1700 InterFreqCarrierFreqList-v1700 OPTIONAL -- Need R ]] } InterFreqCarrierFreqList ::= SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo InterFreqCarrierFreqList-v1610 ::= SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610 InterFreqCarrierFreqList-v1700 ::= SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1700 InterFreqCarrierFreqInfo ::= SEQUENCE { dl-CarrierFreq ARFCN-ValueNR, … cellReselectionPriority CellReselectionPriority OPTIONAL, -- Need R cellReselectionSubPriority CellReselectionSubPriority OPTIONAL, -- Need R q-OffsetFreq Q-OffsetRange DEFAULT dB0, interFreqNeighCellList InterFreqNeighCellList OPTIONAL, -- Need R interFreqExcludedCellList InterFreqExcludedCellList OPTIONAL, -- Need R ... } … InterFreqCarrierFreqInfo-v1700 ::= SEQUENCE { interFreqNeighHSDN-CellList-r17 InterFreqNeighHSDN-CellList-r17 OPTIONAL, -- Need R highSpeedMeasInterFreq-r17 ENUMERATED {true} OPTIONAL, -- Need R redcapAccessRejected-r17 ENUMERATED {true} OPTIONAL -- Need R -- FFS: whether to change above to ‘redcapAccessAllowed’ and the relevant changes in the procedure. } …

As indicated in our contribution R2-2202937 [3] submitted for the previous meeting, the frequency allowing/excluding information for RedCap UEs can be provided as provided for the legacy UEs. In addition, cell-level control within the frequency can also be provided as for the legacy UEs, as there exists the deployment scenario that only subset of cells within a frequency support the RedCap UEs.

If it is agreeable, the following new fields can be provided instead of sending the field redcapAccessRejected:

cellReselectionPriorityRedCap-r17             CellReselectionPriority                                     OPTIONAL,   -- Need R

cellReselectionSubPriorityRedCap-r17          CellReselectionSubPriority                                  OPTIONAL,   -- Need R

interFreqNeighCellListRedCap-r17              InterFreqNeighCellList                                      OPTIONAL,   -- Need R

interFreqExcludedCellListRedCap-r17           InterFreqExcludedCellList                                   OPTIONAL,   -- Need R

Proposal 1:	Replace the field redcapAccessRejected-r17 with the following fields: cellReselectionPriorityRedCap-r17, cellReselectionSubPriorityRedCap-r17, interFreqNeighCellListRedCap-r17, and interFreqExcludedCellListRedCap-r17.

## 2.2	Cell reselection priority for 1RX and half duplex RedCap UEs

The RRC CR also has access control information in SIB1 for 1RX UEs and half-duplex RedCap UE (FFS) as shown below [2]:

#### – SIB1 … SIB1-v1700-IEs ::= SEQUENCE { … hyperSFN-r17 BIT STRING (SIZE (10)) OPTIONAL, -- Need R eDRX-Allowed-r17 ENUMERATED {true} OPTIONAL, -- Need R -- FFS whether eDRX-allowed is one or two bits, i.e. separate for idle/inactive halfDuplexRedCapAllowed-r17 ENUMERATED {true} OPTIONAL, -- Need R -- FFS whether halfDuplexRedCapAllowed is kept, remove also from related procedure cellBarredRedCap-r17 SEQUENCE { cellBarredRedCap1Rx-r17 ENUMERATED {barred, notBarred}, cellBarredRedCap2Rx-r17 ENUMERATED {barred, notBarred} } OPTIONAL, -- Need R intraFreqReselectionRedCap-r17 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need S nonCriticalExtension SEQUENCE {} OPTIONAL }

With this information, a RedCap UE with 1RX UE or HDD UE (FFS) may select the frequency that does not allow accessing the cell. Since UE has no clue for the supporting frequency/cell for those UEs, UE has to do trial and error for each frequency which would not be desirable especially from UE power saving perspective. Thus, it would be beneficial to provide a separate frequency/cell priority for 1RX UE and/or HDD UE (FFS), as in Proposal 1.

Proposal 2:	In SIB4, the following information may be provided for 1RX UE (and HDD UE, if agreed in SIB1) for each frequency: cellReselectionPriorityRedCap1Rx-r17, cellReselectionSubPriorityRedCap1Rx-r17, interFreqNeighCellListRedCap1Rx-r17, and interFreqExcludedCellListRedCap1Rx-r17; (cellReselectionPriorityRedCapHDD-r17, cellReselectionSubPriorityRedCapHDD-r17, interFreqNeighCellListRedCapHDD-r17, and interFreqExcludedCellListRedCapHDD-r17 (if HDD barring in SIB1 is agreed))

# 3	Conclusion

Proposal 1:	Replace the field redcapAccessRejected-r17 with the following fields: cellReselectionPriorityRedCap-r17, cellReselectionSubPriorityRedCap-r17, interFreqNeighCellListRedCap-r17, and interFreqExcludedCellListRedCap-r17.

Proposal 2:	In SIB4, the following information may be provided for 1RX UE (and HDD UE, if agreed in SIB1) for each frequency: cellReselectionPriorityRedCap1Rx-r17, cellReselectionSubPriorityRedCap1Rx-r17, interFreqNeighCellListRedCap1Rx-r17, and interFreqExcludedCellListRedCap1Rx-r17; (cellReselectionPriorityRedCapHDD-r17, cellReselectionSubPriorityRedCapHDD-r17, interFreqNeighCellListRedCapHDD-r17, and interFreqExcludedCellListRedCapHDD-r17 (if HDD barring in SIB1 is agreed))

# 4	References

[1]	RAN2-117-e - R17 NTN-REDCAP-CE_EOM.docx

[2]	R2-2204267	Introduction of RedCap	Ericsson

[3]	R2-2202937	Cell selection criterion for a RedCap UE with 1 Rx branch	Samsung