3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2205023

# Elbonia, 09 – 20 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0240 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on power saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - “using PEI parameters in system information” is reduncent since it was stated previously PEI is configured in system information. - “most recently” is not clear. - Typo in “pei-SearchSpace” - Wording for “PEI indicates the subgroup the UE belongs to to monitor” can be improved as is not clear with the two “to”s. - Discription for subgroupsNumForUEID and subgroupsNumForUEID should be from UE point of view other than from the cell point of view since different UEs might or might not apply the subgrouping based on the capability and configurations. - Editorial cleanup for PEI. - “to save power” is rather unclear and subjective statement for stage 3, should just say “for time and frequency tracking in RRC_IDLE and RRC_INACTIVE state for its paging reception” - TRS descriptions to be improved with TRS occasions configured via SIB and TRS availability on those TRS occasions via L1 signalling to align with RAN1 and stage 2 descriptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove “using PEI parameters in system information”. - “most recently” changed to “last” - pei-SearchSpace corrected. - Improve readability for PEI. - Descriptions for subgroupsNumForUEID and subgroupsNumForUEID changed to be from UE point of view. - Editorial cleanup for PEI. - Change “to save power” to “for time and frequency tracking in RRC_IDLE and RRC_INACTIVE state for its paging reception” - TRS descriptions clarified with TRS occasions configured via SIB and TRS availability on those TRS occasions via L1 signalling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplicated or unclear statements for PEI, subgrouping and TRS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1, 7.3.0, 7.3.1, 7.3.2, 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 7.2.1	Paging Early Indication reception

The UE may use Paging Early Indication (PEI) in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If PEI configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting PEI (except for the UEs expecting multicast session activation notification) can monitor PEI according to the procedure described below.

If lastUsedCellOnly is configured in system information of a cell, the UE monitors PEI only in the cell if the UE last entered RRC_IDLE or RRC_INACTIVE states in this cell. Otherwise, the UE monitors PEI in the camped cell regardless of which cell the UE last entered RRC_IDLE or RRC_INACTIVE states.

The UE monitors one PEI occasion per DRX cycle. A PEI occasion (PEI-O) is a set of PDCCH monitoring occasions (MOs) and can consist of multiple time slots (e.g. subframe or OFDM symbol) where PEI can be sent (TS 38.213 [4]).

The time location of PEI-O for UE's PO is determined by a reference point and an offset from the reference point to the start of the first PDCCH monitoring occasion of this PEI-O:

-	The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, provided by PEI-F_offset in SIB1;

-	The offset is a symbol-level offset from the reference point to the start of the first PDCCH MO of PEI-O, provided by firstPDCCH-MonitoringOccasionOfPEI-O in SIB1.

If one PEI-O is associated with POs of two PFs, the two PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N. The first PF of the PFs associated with the PEI-O is provided by ((SFN for PF) - floor (iPO/Ns)*T/N, where SFN for PF is determined in clause 7.1, iPO = ((UE_ID mod N) * Ns + i_s) mod po-NumPerPEI, and po-NumPerPEI is configured via SIB, T, Ns, and N are determined in clause 7.1.

The PDCCH monitoring occasions for PEI are determined according to pei-SearchSpace as specified in TS 38.213 [4], PEI-F_offset, firstPDCCH-MonitoringOccasionOfPEI-O and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pei-SearchSpace, the PDCCH monitoring occasions for PEI are same as for RMSI as defined in clause 13 in TS 38.213 [4]. UE determines first PDCCH MO for PEI-O based on PEI-F_offset and firstPDCCH-MonitoringOccasionOfPEI-O, as for the case with SearchSpaceId > 0 configured.

When SearchSpaceId = 0 is configured for pei-SearchSpace, the UE monitors the PEI-O according to searchSpaceZero. When SearchSpaceId other than 0 is configured for pei-SearchSpace, the UE monitors the PEI-O according to the SearchSpace of the configured SearchSpaceId.

A PEI occasion is a set of 'S*X' consecutive PDCCH monitoring occasions, where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for PEI in the PEI occasion corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PEI-O. When the UE detects a PEI within its PEI-O, the UE is not required to monitor the subsequent monitoring occasion(s) associated with the same PEI-O.

If the UE detects PEI and the PEI indicates to monitor its associated PO for the subgroup the UE belongs to, as specified in clause 10.4a in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1. If the UE does not detect PEI on the monitored PEI occasion or the PEI does not indicate to monitor its associated PO for the subgroup the UE belongs to, as specified in clause 10.4a in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1.

If the UE is unable to monitor the PEI occasion (i.e. all valid PDCCH monitoring occasions for PEI) corresponding to its PO, e.g. during cell re-selection, the UE monitors the associated PO according to clause 7.1.

Next Modified Subclause

### 7.3.0	General

If PEI and subgrouping are configured, UEs monitoring the same PO can be divided into one or more subgroups. With subgrouping, the UE monitors its PO if the corresponding bit for the subgroup the UE belongs to is set as 1 by PEI corresponding to its PO, as specified in clause 10.4a in TS 38.213 [4]. UE's subgroup can be either assigned by CN as specified in clause 7.3.1 or derived based on UE_ID as specified in clause 7.3.2:

-	If subgroupsNumForUEID is absent in subgroupConfig, the subgroup ID based on CN assigned subgrouping as specified in clause 7.3.1 is used in the cell for the UEs assigned with corresponding CN assigned subgroup IDs.

-	If both subgroupsNumPerPO and subgroupsNumForUEID are configured, and subgroupsNumForUEID has the same value as subgroupsNumPerPO, the subgroup ID based on UE_ID based subgrouping as specified in clause 7.3.2 is used in the cell for the UEs supporting PEI.

-	If both subgroupsNumPerPO and subgroupsNumForUEID are configured, and subgroupsNumForUEID < subgroupsNumPerPO, the UE applies CN assigned subgrouping if the UE is assigned subgroup ID as specified in clause 7.3.1 and if there is a corresponding indication in the PEI for its CN assigned subgroup ID; otherwise if the UE supports PEI and does not have CN assigned ID, the subgroup ID based on UE_ID based subgrouping as specified in clause 7.3.2 is used .

The following parameters are used for the determination of subgroup ID in the cell:

-	subgroupsNumPerPO: total number of subgroups for CN assigned subgrouping (if any) and UE_ID based subgrouping (if any) in a PO, which is broadcast in system information;

-	subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcast in system information.

If a UE has no CN assigned subgroup ID or does not support CN assigned subgrouping, and there is no configuration for subgroupsNumForUEID, the UE monitors paging in its associated PO as specified in clause 7.1.

Next Modified Subclause

### 7.3.1	CN assigned subgrouping

Paging with CN assigned subgrouping is used in the cell which supports at least CN assigned subgrouping, as described in clause 7.3.0. A UE supporting CN assigned subgrouping in RRC_IDLE or RRC_INACTIVE state can be assigned a subgroup ID (between 0 to 7) by AMF through NAS signalling. The UE belonging to the assigned subgroup ID with a corresponding indication in the PEI monitors its associated PEI which indicates the paged subgroup(s) as specified in clause 7.2.

Next Modified Subclause

### 7.3.2	UE_ID based subgrouping

Paging with UE_ID based subgrouping is used in the cell which supports UE_ID based subgrouping, as described in clause 7.3.0.

If the UE is not assigned with a CN assigned subgroup ID, or if the UE assigned with a CN assigned subgroup ID is in a cell which has only UE_ID based subgrouping enabled, the subgroup ID of the UE is determined by the formula below:

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID),

where:

N: number of total paging frames in T

Ns: number of paging occasions for a PF

UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192

subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

The UE belonging to the SubgroupID monitors its associated PEI which includes the paged subgroup(s) as specified in clause 7.2.

Next Modified Subclause

# 9	Tracking Reference Signal

The UE in RRC_IDLE and RRC_INACTIVE state may use Tracking Reference Signal (TRS) whose configurations are provided in system information for time and frequency tracking in RRC_IDLE and RRC_INACTIVE state for its paging reception. In a serving cell in which TRS occasions are configured for the UE in RRC_IDLE and RRC_INACTIVE states to use, the availability of configured TRS is informed to the RRC_IDLE and RRC_INACTIVE state UEs based on explicit L1 availability indication defined in TS 38.213 [4].

End of Changes