3GPP TSG-RAN WG2 Meeting #118 electronic                 	R2-2205058

Online, May 9th – May 20th, 2022

Source:	vivo

Title:	Discussion on MAC remaining issue

Agenda Item:	6.2.2

Document for:	Discussion and Decision

- Introduction

During the last RAN2 meeting, running MAC CR was agreed in [1]. There are three FFSs as following:

- FFS whether to capture "initiate a Random Access Procedure for beam failure recovery to carry BFR MAC CE" for SCG activation.

- FFS how to capture "Upon SCG activation, Bj values are zero", i.e., add "initialize Bj for each logical channel associated to SCG to zero" When SCG is activated or say "Bj are initialized to zero and remain to zero while the SCG is deactivated".

- Editor note: FFS if add new section as MAC reset for SCG deactivation or change the existing MAC reset section for SCG deactivation.

In this contribution, we give some analysis about the FFSs. CR can be found in [2]

- Discussion

  - MAC reset clause for SCG deactivation

For “Editor note: FFS if add new section as MAC reset for SCG deactivation or change the existing MAC reset section for SCG deactivation.”, some companies would like to change existing MAC reset section for SCG deactivation, however we think it is better to add new section as MAC reset for SCG deactivation because there still are lots of difference between existing MAC reset and MAC reset for SCG deactivation:

- existing MAC reset section also include Sidelink part

- different Bj initialization

- BFR handling

Changing the existing MAC reset for SCG deactivation will make the specification complex. We propose to remove the Editor note: FFS if add new section as MAC reset for SCG deactivation or change the existing MAC reset section for SCG deactivation directly.

Proposal1: Remove the Editor note: FFS if add new section as MAC reset for SCG deactivation or change the existing MAC reset section for SCG deactivation directly.

  - RACH for BFR MAC CE

For “FFS whether to capture "initiate a Random Access Procedure for beam failure recovery to carry BFR MAC CE" for SCG activation”, after BFR the UE will stop BFR procedure, the beam information may not be available, if the UE reports the old candidate beam information it will mess up the gNB. The normal RACH procedure with BRF MAC CE can still give some beam information e.g., based on RACH resource selection. We propose to remove the Editor note: FFS whether to capture "initiate a Random Access Procedure for beam failure recovery to carry BFR MAC CE" for SCG activation directly.

Proposal2: Remove the Editor note: FFS whether to capture "initiate a Random Access Procedure for beam failure recovery to carry BFR MAC CE" for SCG activation directly.

  - Initialize Bj

For “FFS how to capture "Upon SCG activation, Bj values are zero", i.e., add "initialize Bj for each logical channel associated to SCG to zero" When SCG is activated or say "Bj are initialized to zero and remain to zero while the SCG is deactivated”, one company commented that one change for Bj initialization can be initialized to zero and remain to zero while the SCG is deactivated, we think that the simple way is to initialize Bj for each logical channel associated to SCG to zero When SCG is activated. We propose to remove the Editor note: FFS how to capture "Upon SCG activation, Bj values are zero", i.e., add "initialize Bj for each logical channel associated to SCG to zero" When SCG is activated or say "Bj are initialized to zero and remain to zero while the SCG is deactivated directly.

Proposal3: Remove the Editor note: FFS how to capture "Upon SCG activation, Bj values are zero", i.e., add "initialize Bj for each logical channel associated to SCG to zero" When SCG is activated or say "Bj are initialized to zero and remain to zero while the SCG is deactivated directly.

- Conclusion

According to the above analysis, we have the following proposal:

Proposal1: Remove the Editor note: FFS if add new section as MAC reset for SCG deactivation or change the existing MAC reset section for SCG deactivation directly.

Proposal2: Remove the Editor note: FFS whether to capture "initiate a Random Access Procedure for beam failure recovery to carry BFR MAC CE" for SCG activation directly.

Proposal3: Remove the Editor note: FFS how to capture "Upon SCG activation, Bj values are zero", i.e., add "initialize Bj for each logical channel associated to SCG to zero" When SCG is activated or say "Bj are initialized to zero and remain to zero while the SCG is deactivated directly.

The CR can be found in [2].

- References

- R2-2204211 Introduction of eDCCA     vivo

- R2-2205057 MAC correction on eDCCA  vivo