3GPP TSG-RAN WG2 Meeting #118 electronic	R2-2205066

Online, May 09 – 20, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3038 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PC5 RLC channel configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In F1 interface, PC5 RLC channel ID can be allocated in scope of remote UE or relay UE. This depends on whether CU set remote UE ID in PC5 RLC channel configuration list of UE CONTEXT MODIFICATION REQUEST as shown in following. >PC5 RLC Channel to be Modified Item IEs; 1 .. <maxnoofPC5RLCChannels>. >>PC5 RLC Channel ID; M; 9.3.1.x. >>Remote UE local ID; O; 9.3.1.z. Therefore it is possible that PC5 RLC channel ID is allocated uniquely within the scope of remote UE. In other words, different remote UEs associated to one relay UE may share a same PC5 RLC channel ID. In this case, remote UE ID needs to be included in the PC5 RLC channel configuration, so that relay UE can identify the received PC5 RLC channel belongs to which remote UE. |
| Summary of change: |  | 1. Add the remote UE ID in PC5 RLC channel configuration. Impact analysis Impacted 5G architecture options: Standalone/Non-standalone Impacted functionality: NR sidelink communication configuration Inter-operability: - If the UE is implemented according to this CR but the network is not, no inter-operability issue foreseen. - If the network is implemented according to this CR but the UE is not, no inter-operability issue foreseen. - If one UE is implemented according to this CR but the other UE is not, no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relay UE may not identify the recevied RLC channel belongs to which remote UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

6.3.5	Sidelink information elements

–	SL-RLC-ChannelConfig

The IE SL-RLC-ChannelConfig specifies the SL RLC bearer configuration information for PC5 Relay RLC channel between L2 U2N Relay UE and L2 U2N Remote UE.

SL-RLC-ChannelConfig information element

-- ASN1START

-- TAG-SL-RLC-RLC-CHANNEL-CONFIG-START

SL-RLC-ChannelConfig-r17 ::=                  SEQUENCE {

sl-RLC-ChannelID-r17                          SL-RLC-ChannelID-r17,

sl-LocalIdentity-r17                         INTEGER (0..255),							 OPTIONAL,

sl-RLC-Config-r17                             SL-RLC-Config-r16                                 OPTIONAL,

sl-MAC-LogicalChannelConfig-r17               SL-LogicalChannelConfig-r16                       OPTIONAL,

sl-PacketDelayBudget-r17                      INTEGER (0..1023)                                 OPTIONAL,

...}

-- TAG-SL-RLC-CHANNEL-CONFIG-STOP

-- ASN1STOP

SL-RLC-ChannelConfig field descriptions

sl-MAC-LogicalChannelConfig The field is used to configure MAC SL logical channel paramenters.

sl-LocalIdentity Indicates the L2 U2N Remote UE.

sl-RLC-ChannelID Indicates the PC5 Relay RLC channel in the link between L2 U2N Relay UE and L2 U2N Remote UE.

sl-RLC-Config Determines the RLC mode (UM, AM) and provides corresponding parameters.

sl-PacketDelayBudget Indicates the Packet Delay Budget for a PC5 RLC bearer. Upper bound value for the delay that a packet may experience expressed in unit of 0.5ms.

End of the change