3GPP TSG-RAN WG2 Meeting #118 electronic	R2-2205114

Electronic Meeting, May. 09 – May 20, 2022                                                (was R2-2102543)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1254 | Rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reduction of some parts of selection of logical channels in SL Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  |  | Date: |  | 2022-04-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to introduce the support for the sidelink relay feature in NR. To increase readability, the duplicated part can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.22.1.4.1.2 - Remove the description duplicate description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the CR is not approved the description is redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.22.1.4.1.2	Selection of logical channels

The MAC entity shall for each SCI corresponding to a new transmission:

1>	if sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon is configured according to TS 38.331 [5]:

1>	if the new transmission is associated with a sidelink grant in sl-DiscTxPoolSelected or sl-DiscTxPoolScheduling configured in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon:

2>	select a Destination asssociated with sidelink disovery as specified in TS 23.304 [x1] , having at least one of the logical channel with the highest priority, among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI:

3>	SL data is available for transmission; and

3>	SBj > 0, in case there is any logical channel having SBj > 0; and

3>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

3>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

2>	else:

3>	select a Destination associated with one of unicast, groupcast and broadcast (excluding the Destinaiton(s) associated with sidelink discovery as specified in TS 23.304 [x1]) , having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

4>	SL data is available for transmission; and

4>	SBj > 0, in case there is any logical channel having SBj > 0; and

4>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

4>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

4>	sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

1>	else:

2>	select a Destination associated to one of unicast, groupcast and broadcast(excluding the Destinaiton(s) associated with sidelink discovery as specified in TS 23.304 [x1]), having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

3>	SL data is available for transmission; and

3>	SBj > 0, in case there is any logical channel having SBj > 0; and

3>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

3>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

3>	sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

NOTE 1:	If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE, which Destination is selected among them is up to UE implementation.

End of Changes