3GPP TSG-RAN WG2 Meeting #118 electronic	R2-2205122

# Online, 9th – 20th May, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MBS MAC subPDU discard |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc., Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-0x-xx |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In NR MBS, the network may not always be able to ensure that all MBS sessions transmitted via PTM transmission are of interest to all UEs decoding the transport block addressed to the corresponding G-RNTI. Therefore, UE’s MAC entity shall discard MBS MAC subPDU containing an LCID or an eLCID which is not configured. When determining whether to discard an MBS MAC subPDU or not, the PTP retransmission case is currently not taken into accout. Furthermore, it is not clear the UE’s MAC entity only discards MBS MAC subPDUs containing an LCID or an eLCID which is not configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a condition of discarding MBS MAC subPDU by considering PTP retransmission case and clarify that the MAC entity only discard MBS MAC subPDU containing an LCID or an eLCID which is not configured. Impact analysis Inter-operability: If the network is implemented according to the CR and the UE is not, the UE will not discard PDUs corresponding to (e)LCID that are not configured for MBS whenever PTP retransmissions are used. Depending on UE behaviour, this may limit the usage of PTP retransmissions.. If the UE is implemented according to the CR and the NW is not, there is no inter-operablity issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how UE’s MAC entity processes an MBS MAC PDU receiveid by PTP retransmission and whether UE’s MAC entity only discards MBS MAC sub PDUs containing an LCID or and eLCID which is not configured or not when an MBS MAC PDU is received. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Modified Subclause

5.3.3	Disassembly and demultiplexing

The MAC entity shall disassemble and demultiplex a MAC PDU as defined in clauses 6.1.2 and 6.1.5a.

When the MAC entity receives a MAC PDU for MAC entity’s G-RNTI or G-CS-RNTI, or C-RNTI or CS-RNTI for multicast transmission, or by the configured downlink assignment for MBS multicast, the MAC entity shall:

1>	discard the received subPDU containing an LCID or an eLCID which is not configured.

End of the modification