3GPP TSG-RAN WG2 Meeting #118 electronic	R2-2205129

Online, May 9 – 20, 2022

Agenda Item:	6.1.3.2

Source: 	ASUSTeK

Title:  	Handling of MAC PDU for MBS with Reserved LCID

Document for:	Discussion and Decision

- Introduction

Three new RNTIs are introduced for MBS [1]:

| G-CS-RNTI | Configured scheduled multicast transmission (activation, reactivation and retransmission) | DL-SCH | MTCH |
| --- | --- | --- | --- |
| G-RNTI | Dynamically scheduled MBS PTM transmission | DL-SCH | MTCH |
| MCCH-RNTI | Dynamically scheduled MCCH signalling and MCCH change notification | DL-SCH | MCCH |

This document discusses the handling of a Reserved LCID value received in a MAC PDU for MBS RNTIs.

- Discussion

According to the values of LCID [1], it is possible that a Reserved LCID value in Rel-17 will be used in future releases and a Rel-17 UE served by a future-release gNB may receive a MAC PDU for MBS broadcast/multicast containing a Reserved LCID value (for the purpose not known by the Rel-17 UE).

Table 6.2.1-1 Values of LCID for DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel of DCCH, DTCH and multicast MTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35–46 | Reserved |
| 47 | Recommended bit rate |
| […] | […] |
| 63 | Padding |

Table 6.2.1-1c Values of LCID for MBS broadcast on DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | MCCH |
| 1–32 | Identity of the logical channel of broadcast MTCH |
| 33–63 | Reserved |

For future compatibility, how to handle the MAC PDU containing a Reserved LCID value should be specified. Currently, when a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity would discard the received subPDU and any remaining subPDUs in the MAC PDU. The same handling could also be applied to the MAC PDU for G-RNTI, G-CS-RNTI, or MCCH-RNTI, i.e. discarding the received subPDU and any remaining subPDUs in the MAC PDU.

Proposal 1:	Handling of a MAC PDU for G-RNTI, G-CS-RNTI, or MCCH-RNTI, containing a Reserved LCID value is the same as that for a MAC PDU for C-RNTI or CS-RNTI.

- Conclusion

Handling of a Reserved LCID value received in a MAC PDU for MBS RNTIs is proposed as below:

Proposal 1:	Handling of a MAC PDU for G-RNTI, G-CS-RNTI, or MCCH-RNTI, containing a Reserved LCID value is the same as that for a MAC PDU for C-RNTI or CS-RNTI.

Corresponding text proposal for 38.321 is also provided as below.

- Reference

- TS 38.321 V17.0.0, NR MAC specification.

- Text proposal for 38.321

### 5.3.3	Disassembly and demultiplexing

The MAC entity shall disassemble and demultiplex a MAC PDU as defined in clauses 6.1.2 and 6.1.5a.

When a MAC entity receives a MAC PDU for MAC entity’s G-RNTI or G-CS-RNTI, or by the configured downlink assignment for MBS multicast containing an LCID or eLCID which is not configured, the MAC entity shall at least:

1>	discard the received subPDU.

5.13	Handling of unknown, unforeseen and erroneous protocol data

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, or MCCH-RNTI, or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity shall at least:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, the MAC entity shall at least:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for broadcast or groupcast, or an LCID value which is not configured, the MAC entity shall:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for unicast, the MAC entity shall:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.