3GPP TSG-RAN WG2 #118-e	R2-2205181

Electronical meeting, 9 May – 20 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | xxxx | rev | xxx | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR of TS 38.321 on SL grant reception in case of SL DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR introduces changes to 38.321 on SL grant reception. Issue 1: A Rel.17 UE served by a Rel.17 gNB may perform LTE sidelink transmission for V2X sidelink communication. When resource allocation mode 1 is configured, the gNB may use PDCCH to activate/deactivate the semi-persistently scheduled sidelink transmission. In this case PDCCH indicating activation of sidelink SPS shall be considered to indicate a new transmission similar as activation of UL SPS or activation of sidelink configured grant type 2. Issue 2: RAN2 agreed that For mode-1 DG and mode-2 grant, if the initial transmission occasion was dropped due to no Rx-UE in DRX active time, TX-UE can use re-transmission occasion for initial transmission. This agreement has been captured in clause 5.22.1.3.1. Therefore, below text in clause 5.22.1.1 (as highlited) is not needed. 2> if the TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check: …… 5> if selected resource for initial transmission occasion is not in the SL DRX Active time as specified in clause 5.28.1 of any destination that has data to be sent: 6> use retransmission occasion(s) for initial transmission of PSCCH and PSSCH. 3. Issue 3 The below RAN2 agreement For mode-1 DG and mode-2 grant, if the initial transmission occasion was dropped due to no Rx-UE in DRX active time, TX-UE can use re-transmission occasion for initial transmission. has been captured by the below texts as highlighted in clause 5.22.1.3.1 1> if the MAC entity determines that the sidelink grant is used for initial transmission as specified in clause 5.22.1.1; or 1> if the sidelink grant is a configured sidelink grant and no MAC PDU has been obtained in a sl-PeriodCG of the configured sidelink grant; or 1> if the sidelink grant is a dynamic sidelink grant or selected sidelink grant and no MAC PDU has been obtained in the previous sidelink grant when PSCCH duration(s) and 2nd stage SCI on PSSCH of the previous sidelink grant is not in SL DRX Active time as specified in clause 5.28.1 of the destination that has data to be sent: however, the wording is not accurate. The RAN2 agreement is only applicable to the case where the previous sidelink grant and the current sidelink grant are used for transmission of the same TB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issue 1: in section 5.7, the note “A PDCCH indicating activation of SPS, or configured grant type 2, or configured sidelink grant of configured grant Type 2 is considered to indicate a new transmission” includes the case “PDCCH indicating activation of sidelink SPS”. issue 2: remove the relevant text in 5.22.1.1. Issue 3: update the relevant text in 5.22.1.3.1 to capture that the previous sidelink grant and the current sidelink grant are used for transmission of the same TB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel.17 UE performing LTE sidelink transmission semi-persistently scheduled by Rel.17 gNB cannot use Uu DRX properly. Re-transmission occasion cannot be used for initial transmission properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7, 5.22.1.1, 5.22.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unmodified parts omitted >

### 5.7	Discontinuous Reception (DRX)

< Unmodified parts omitted >

2>	if the PDCCH indicates a new transmission (DL, UL or SL) on a Serving Cell in this DRX group:

3>	start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

NOTE 3a:	A PDCCH indicating activation of SPS, configured grant type 2, sidelink SPS, or configured sidelink grant of configured grant Type 2 is considered to indicate a new transmission.

< Unmodified parts omitted >

### 5.22.1.1	    SL Grant reception and SCI transmission

< Unmodified parts omitted >

2>	if the TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check:

3> if one or multiple SL DRX is configured in the destination UE(s) receiving SL-SCH data:

< Unmodified parts omitted >

3> if not configured by RRC, interUECoordinationScheme1Explicit or interUECoordinationScheme1Condition enabling reception of preferred resource set and non-prefererred resource set:

4> if transmission based on random selection is configured by upper layers:

5> randomly select the time and frequency resources for one transmission opportunity from the resource pool which occur within the SL DRX active time as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX active time above, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

< Unmodified parts omitted >

### 5.22.1.3.1	   SL Grant reception and SCI transmission

< Unmodified parts omitted >

For each sidelink grant, the Sidelink HARQ Entity shall:

1>	if the MAC entity determines that the sidelink grant is used for initial transmission as specified in clause 5.22.1.1; or

1>	if the sidelink grant is a configured sidelink grant and no MAC PDU has been obtained in a sl-PeriodCG of the configured sidelink grant; or

1>	if the sidelink grant is a dynamic sidelink grant or selected sidelink grant and no MAC PDU has been obtained in the previous sidelink grant when PSCCH duration(s) and 2nd stage SCI on PSSCH of the previous sidelink grant is not in SL DRX Active time as specified in clause 5.28.1 of the destination that has data to be sent, and the previous sidelink grant and the sidelink grant are used for transmission of the same TB:

NOTE 1:	Void.

2>	(re-)associate a Sidelink process to this grant, and for the associated Sidelink process: