3GPP TSG-RAN WG2 #118-e	R2- 2205189

Electronical meeting, 9 May – 20 May 2022

Agenda Item:	   6.20.1

Source:	   Ericsson

Title:	    RIL issue list of running RRC CR for 71 GHz

Document for:	Information

# 1	Introduction

This document aims at summarizing open RIL issues identified during ASN.1 review that need to be addressed for the 71 GHz Work Item by RAN2.

# 2		List of ASN.1 issues

| Issues | Relevant clauses/IEs in TS 38.331 | Conclusion / rapporteur’s suggestions on how to address |
| --- | --- | --- |
| E048 - Introduce changes to 36.331 and 38.331 for OverheatingAssistance in EN-DC scenarios. | Clause 5.7.4.3a | To be addressed in the RRC CR |
| E049 - PDCCH slot group configuration for FR2-2 is missing in PDCCH-ConfigCommon | Clause 6.3.2 PDCCH-ConfigCommon | To be discussed during online session of RAN2#118 |
| E133 – use the same or different RRC parameters for searchSpaceSwitchTimer. searchSpaceSwitchTimer need to be introduced for both 71GHz and ePowerSaving. Two parameters have the similiar intentions, therefore, discussions are neeeded to study it is feasible to use the same RRC parameter in the current R17 spec which has been introduced by power saving WI, for both WIs. | Clause 6.3.2 PDCCH-Config and PDCCH-ServingCellConfig | To be discussed during online session of RAN2#118 |
| E134 - RAN1 has defined new values for SCS 480 and 960 kHz in 38.213, therefore value ranges of searchSpaceSwitchDelay need to be extended for SCS 480 and 960 kHz. | Clause 6.3.2 SearchSpaceSwitchConfig | To be discussed during online session of RAN2#118 |
| E135 - value range of si-WindowLength in SI-SchedulingInfo needs to be extended for higher SCS e.g., 480 and 960 kHz. | Clause 6.3.2 SI-SchedulingInfo/si-WindowLength | To be discussed during online session of RAN2#118 |
| E136 - value range of drx-RetransmissionTimerUL/drx-RetransmissionTimerDL need to be extended for SCS 480 kHz and 960 kHz. | Clause 6.3.2 DRX-Config | To be discussed during online session of RAN2#118 |
| I017 - The procedural text implies that UE has to include both sub-fields. IF so, these sub-fields can be made mandatory. Else the procedural text has to be updated. | Clause 6.2.2 reducedMaxBW-FR2-2-r17 | To be addressed in the RRC CR |
| I027 - Using presence condition SetupRelease is problematic. Further the condition should not be based on the presence of the other field, but on whether it is configured. | Clause 6.3.2 MAC-CellGroupConfig/drx-ConfigExt-v1700 | To be addressed in the RRC CR |
| I034 - This is done as a critical extension. An NCE with just r17monitoringcapability would have been sufficient as the original field is also optional | clause 6.3.2 PDCCH-Config/ monitoringCapabilityConfig-r17 | To be addressed in the RRC CR |
| Z450 - For overheating assistance, reducedMaxBW-FR2-2 for (NG)EN-DC should also be considered similar to FR2-1. | Clause 5.7.4.3a | This issue is overlapping with E048. To be merged with E048 and addressed in the RRC running CR. |
| Z451 - For firstPDCCH-MonitoringOccasionOfPO, since SCS 480kHz and 960kHz is introduced, the corresponding value range should be extended. | Clause 6.3.2 PCCH-Config/firstPDCCH-MonitoringOccasionOfPO | To be addressed during online session of RAN2#118 |
| Z452 - According to the current agreement, for multiple PUSCH, each PDSCH/PUSCH is confined within one slot, so a restriction should be added for numberOfSlots-TBoMS-r17. | Clause 6.3.2 PUSCH-Allocation-r16/ numberOfSlots-TBoMS-r17 | To be addressed during online session of RAN2#118 |
| Z453 - According to the current agreement, 4 bit length should also be defined besides 8 bit length. | Clause 6.3.2 monitoringSlotsWithinSlotGroup-r17 | To be addressed in the RRC CR |
| Z454 - Some parameters from LS R1-2202759 are not captured in the current CR, such as cg-minDFI-Delay-v17, searchSpaceSwitchTimer-v17, duration-r17 and codeBlockGroupTransmission. | Clause 6.3.2 | To be addressed in the RRC CR |
| Z455 - Monitoring periodicity Periodicity values {32,64,128,5120,10240,20480} are added to the existing values in monitoringSlotPeriodicityAndOffset and the total list of supported periodicity values that can be configured for SCS 480kHz and 960kHz: {4,8,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480}. We need to capture the total periodicity list in the specification. According to the agreements made in RAN1#108-e meeting, for each periodicity value Xp, offset O values that can be configured:{0, 4, 8, …, 4(Xp-1)4} slots | Clause 6.3.2 SearchSpaceExt-v1700/ monitoringSlotPeriodicityAndOffset-r17 | To be addressed in the RRC CR |
| Q300 - Extended k0 in PDSCH-TimeDomainResourceAllocation The only difference between r16 and r17 seems to be the range of k0. Then it is unclear why -r17 needs to be introduced instead of simply adding NCE for k0-v1700 for the extende Easier to simply add k0-v1700 field in -r16 IE, unless a clear need for new IE is seen.d range. This also has impact on merging with MBS. Related to Q301 and Q302 | Clause 6.3.2 PDSCH-TimeDomainResourceAllocation-r17 | To be addressed in the RRC CR |
| H707 - This used IE is not correct here, shall be periodicityAndOffset-r17 . [Proposed Change]: If SchedulingRequestResourceConfigExt-v1700 periodicityAndOffset-r17 is present, any previously configured SchedulingRequestResourceConfigExtperiodicityAndOffset (without suffix) is released, and vice versa. | Clause 6.3.2 SchedulingRequestResourceConfigExt | To be addressed in the RRC CR |
| N109 - The outer SEQUENCE seems unnecessary and can be removed. This allows the IE to be used in multiple places. [Proposed Change]: Use the following IE structure and reuse that in places referring to SearchSpaceExt2. SearchSpaceType-r17 ::= SEQUENCE{ common-r17 SEQUENCE { dci-Format4-0-r17 SEQUENCE { ... } OPTIONAL, -- Need R dci-Format4-1-r17 SEQUENCE { ... } OPTIONAL, -- Need R dci-Format4-2-r17 SEQUENCE { ... } OPTIONAL, -- Need R dci-Format4-1-AndFormat4-2-r17 SEQUENCE { ... } OPTIONAL -- Need R } } | Clause 6.3.2 SearchSpaceExt2 | This issue is to be discussed together with E049. A CR is submitted and will be discussed in the main session. |
| N110 - Since these values have different maximum per SCS, it would be more appropriate to have CHOICE per SCS value to make that clearer instead of field description. It is also not clear what "offset is less than period duration" means, since the period is in ms and offset is in symbols. [Proposed Change]: Use the following: SemiStaticChannelAccessConfigUE-r17 ::= SEQUENCE { periodUE-r17 ENUMERATED {ms1, ms2, ms2dot5, ms4, ms5, ms10, spare2, spare1}, offsetUE-r17 CHOICE { scs-15kHz-r17 INTEGER (0..279) scs-30kHz-r17 INTEGER (0..559) scs-60kHz-r17 INTEGER (0..1119) } } and field description as: offsetUE Indicates the number of symbols from the beginning of the even indexed radio frame to the start of the first period within that radio frame that the UE can initiate a channel occupancy (see TS 37.213 [48], clause 4.3), based on the smallest SCS among the configured SCSs in the serving cell. Network sets the value of this field to be less than the value of periodUE in symbols. | Clause 6.3.2 SemiStaticChannelAccessConfigUE | This change is not introduced due to 71 GHz. Therefore, changes are handled by other topics (i.e., IOT). |
| N121 - The periodicity extension seems only required for 480 and 960 kHz, so why not provide it for only those cases? The field is fine as such, but it would make sense to restrict it to only the cases when it's necessary. For example, is it now allowed to include the Rel-16 and Rel-17 field, and what does UE do if it receives both? Since this is only in the field description, such cases can be very error-prone. [Proposed Change]: Use the following: periodicityExt-r17 INTEGER (1..40960) OPTIONAL, -- Cond FR2-2 | Clause 6.3.2 SPS-Config | It may be sufficient to add a restriction in the field description. To be addressed in the RRC CR |
| S626 - As per the current field description of drx-HARQ-RTT-TimerDL/UL, there is no drx-HARQ-RTT-TimerDL/UL value for BWP using legacy SCS (i.e., 15/30/60/120kHz) when the new field is configured. Thus, the description needs corrections. | Clause 6.3.2 DRX-Config | To be addressed in the RRC CR |

# References